3GPP TSG-CT3 Meeting #107	C3-195273

# Reno, USA, 11th – 15th  November 2019                                              (Revision of C3-195143)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.222 | CR | 0116 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for 3rd party API provider domain |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C3 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCAPIF |  |  |  |  |  | Date: |  |  | 2019-11-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TS 23.222 (Rel-16) specifies support for 3rd party API provider domain and its functions. For all the CAPIF APIs and the relevant service operations that the API provider domain functions consume, the API provider domain functions need to have sufficient permission. This is not currently addressed for 3rd party API provider domain functions in 29.222 in stage 3. This needs to be clarified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that CAPIF APIs and the service oprations can be consumed by 3rd party API provider domain functions with sufficient permissions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 3 details of CAPIF APIs are not clear for 3rd party API provider support. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.1.1, 5.6.2.4.2, 5.6.2.5.2, 5.8.1.1, 5.9.1.1, 5.10.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The CR does not impact any Open API specification. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

#### 5.3.1.1	Overview

The CAPIF publish service APIs, as defined in 3GPP TS 23.222 [2], allow API publishing function via CAPIF-4 reference point to publish and manage published service APIs at the CAPIF core function.

NOTE:	Functions from 3rd party API provider domain can also access this API with sufficient permissions.

2nd change

##### 5.6.2.4.2	Obtain API invoker's security information using Obtain_API_Invoker_Info service operation

To obtain authentication or authorization information from the CAPIF core function to authenticate or authorize an API invoker, the API exposing function shall send an HTTP GET message to that API invoker’s resource representation URI in the CAPIF core function with an indication to request authentication and/or authorization information, as specified in subclause 8.5.2.3.3.1.

Upon receiving the above described HTTP GET message, the CAPIF core function shall:

1.	determine the security information of API invoker for all the service API interfaces of the API exposing function; and

2.	return the security information in the response message.

NOTE:	Functions from 3rd party API provider domain can also access this service operation with sufficient permissions.

3rd change

##### 5.6.2.5.2	Invalidate authorization using Revoke_Authorization service operation

To invalidate authorization of an API invoker for all service APIs, the API exposing function shall send an HTTP DELETE message to that API invoker’s resource representation URI in the CAPIF core function using the API invoker ID as specified in subclause 8.5.2.3.3.2.

Upon receiving the HTTP DELETE message, the CAPIF core function shall delete the resource representation and shall notify the API invoker of the authorization invalidation using the Notification Destination URI received in the Obtain_Security_Method message.

The CAPIF core function shall also invalidate the previously assigned access token when the authorization of all service APIs are revoked for the API invoker.

To invalidate authorization of an API invoker for some service APIs, the API exposing function shall send an HTTP POST message to that API invoker’s "delete" custom resource representation URI in the CAPIF core function with a list of the service APIs that should be revoked.

Upon receiving the HTTP POST message, the CAPIF core function shall revoke the authorization of the API invoker for the indicated service APIs (e.g. it may update the list of unauthorized APIs locally); and shall notify the API invoker of the authorization invalidation using the Notification Destination URI received in the Obtain_Security_Method message.

In both alternatives, the CAPIF core function shall acknowledge the HTTP request from the API exposing function.

NOTE:	Functions from 3rd party API provider domain can also access this service operation with sufficient permissions.

4th change

#### 5.8.1.1	Overview

The Logging API invocations APIs, as defined in 3GPP TS 23.222 [2], allow API exposing functions via CAPIF-3 reference point to log the information related to service API invocations on the CAPIF core function.

NOTE:	Functions from 3rd party API provider domain can also access this API with sufficient permissions.

5th change

#### 5.9.1.1	Overview

The Auditing API, as defined in 3GPP TS 23.222 [2], allows API management functions via CAPIF-5 reference point to query the log information stored on the CAPIF core function.

NOTE:	Functions from 3rd party API provider domain can also access this API with sufficient permissions.

6th change

#### 5.10.1.1	Overview

The CAPIF access control policy APIs allow API exposing function via CAPIF-3 reference point to obtain the service API access policy from the CAPIF core function.

NOTE:	Functions from 3rd party API provider domain can also access this API with sufficient permissions.

End of changes