3GPP TSG-CT WG1 Meeting #122-e	C1-201029

# Electronic meeting, 20-28 February 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.301 | CR | 3338 | rev | 1 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Detach before attach for RLOS and Emergency |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | PARLOS, TEI16 |  |  |  |  |  | Date: |  |  | 2020-02-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | As per current specification 24.301 section: 5.2.3.2.3 LIMITED-SERVICE 5.2.3.2.4 PLMN-SEARCH UE may initiate ATTACH REQUEST for RLOS or emergency services. Since this are EMM-REGISTERED substate which in other words UE is already registered with network with valid context. There is no clarification provided whether the UE should perform detach by explicit signalling with network or locally before triggering attach for emergency services or attach for RLOS. As per 5.3.18 Restricted local operator services: If a UE attached for access to RLOS needs to initiate an emergency call, the UE shall first perform a local detach prior to initiating an attach procedure for emergency bearer services. It need to be clarified to perform local detach before initiating attach for emergency services when UE is attached for RLOS or normal services. These substates are entered wherein the UE cannot access normal service and RLOS attach and emergency attach can still be trigged. Since in these substates the UE cannot perform the detach procedure by explicit signalling it will delay the detach procedure and as a result it will delay the attach for RLOS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction to to detach before triggering attach for access to RLOS and emergency attach in substates EMM-REGISTERED.PLMN_SEARCH and EMM-REGISTERED.LIMITED-SERVICE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unneccessary delay for EPS RLOS attach and EPS Emergency attach from substates EMM-REGISTERED.PLMN_SEARCH and EMM-REGISTERED.LIMITED-SERVICE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.3.2.3, 5.2.3.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 5.2.3.2.3	LIMITED-SERVICE

The UE:

-	shall perform cell selection/reselection according to 3GPP TS 36.304 [21];

-	may respond to paging (with IMSI);

-	may detach locally and initiate attach for emergency bearer services;

-	may detach locally and initiate attach for access to RLOS; and

-	if configured for eCall only mode as specified in 3GPP TS 31.102 [17], shall perform the eCall inactivity procedure at expiry of timer T3444 or T3445 (see subclause 5.5.4).

***** Next change *****

##### 5.2.3.2.4	PLMN-SEARCH

The UE may enter this substate when it is in automatic network selection mode and the maximum allowed number of subsequently unsuccessful tracking area updating have been performed. The UE may also enter this substate as a result of a tracking area update rejected by the network (see subclause 5.5.3) or as a result of a service request rejected by the network (see subclause 5.6.1). If a new PLMN is selected, the UE shall reset the tracking area updating attempt counter and initiate the tracking area updating or combined tracking area updating procedure (see subclause 5.5.3).

If the selected cell is known not to be able to provide normal service:

-	the UE may detach locally and initiate attach for emergency bearer services; or

-	the UE may detach locally and initiate attach for access to RLOS.