3GPP TSG-CT Meeting #87-e	CP-200134

# Online, , 16th Mar 2020 - 18th Mar 2020	(was C4-201094)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.510 | CR | 0278 | rev | 3 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Data Type Descriptions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Work item code: | SBIProtoc16 |  |  |  |  |  | Date: |  |  | 2020-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Reusable components (data types) should have a "description" field in the OpenAPI spec. The absence of such field is usually highlighted by OpenAPI lint tools as a warning of a non-optimal API design. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a "description" field to all reusable data types under the "components" sections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The overall API description is not well described. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.1, 6.2.6.1, 6.3.5.1, 6.4.6.1, A.2, A.3, A.4, A.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backwards compatible new features to the Nnrf_NFManagement, Nnrf_NFDiscovery, Nnrf_AccessToken and Nnrf_Bootstrapping APIs. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev. 3 to CT#87e: Replace an underscore symbol by a whitespace in: UdrInfo: description:_Information of an UDR NF Instance |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Nnrf_NFManagement service-based interface protocol.

Table 6.1.6.1-1: Nnrf_NFManagement specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| NFProfile | 6.1.6.2.2 | Information of an NF Instance registered in the NRF. |
| NFService | 6.1.6.2.3 | Information of a given NF Service Instance; it is part of the NFProfile of an NF Instance. |
| DefaultNotificationSubscription | 6.1.6.2.4 | Data structure for specifying the notifications the NF service subscribes by default along with callback URI. |
| IpEndPoint | 6.1.6.2.5 | IP addressing information of a given NFService; it consists on, e.g. IP address, TCP port, transport protocol... |
| UdrInfo | 6.1.6.2.6 | Information of an UDR NF Instance. |
| UdmInfo | 6.1.6.2.7 | Information of an UDM NF Instance. |
| AusfInfo | 6.1.6.2.8 | Information of an AUSF NF Instance. |
| SupiRange | 6.1.6.2.9 | A range of SUPIs (subscriber identities), either based on a numeric range, or based on regular-expression matching. |
| IdentityRange | 6.1.6.2.10 | A range of GPSIs (subscriber identities), either based on a numeric range, or based on regular-expression matching. |
| AmfInfo | 6.1.6.2.11 | Information of an AMF NF Instance. |
| SmfInfo | 6.1.6.2.12 | Information of an SMF NF Instance. |
| UpfInfo | 6.1.6.2.13 | Information of an UPF NF Instance. |
| SnssaiUpfInfoItem | 6.1.6.2.14 | Set of parameters supported by UPF for a given S-NSSAI. |
| DnnUpfInfoItem | 6.1.6.2.15 | Set of parameters supported by UPF for a given DNN. |
| SubscriptionData | 6.1.6.2.16 | Information of a subscription to notifications to NRF events, included in subscription requests and responses. |
| NotificationData | 6.1.6.2.17 | Data sent in notifications from NRF to subscribed NF Instances. |
| NFServiceVersion | 6.1.6.2.19 | Contains the version details of an NF service. |
| PcfInfo | 6.1.6.2.20 | Information of a PCF NF Instance. |
| BsfInfo | 6.1.6.2.21 | Information of a BSF NF Instance. |
| Ipv4AddressRange | 6.1.6.2.22 | Range of IPv4 addresses. |
| Ipv6PrefixRange | 6.1.6.2.23 | Range of IPv6 prefixes. |
| InterfaceUpfInfoItem | 6.1.6.2.24 | Information of a given IP interface of an UPF. |
| UriList | 6.1.6.2.25 | Set of URIs following 3GPP hypermedia format (containing a "_links" attribute). |
| N2InterfaceAmfInfo | 6.1.6.2.26 | AMF N2 interface information. |
| TaiRange | 6.1.6.2.27 | Range of TAIs (Tracking Area Identities). |
| TacRange | 6.1.6.2.28 | Range of TACs (Tracking Area Codes). |
| SnssaiSmfInfoItem | 6.1.6.2.29 | Set of parameters supported by SMF for a given S-NSSAI. |
| DnnSmfInfoItem | 6.1.6.2.30 | Set of parameters supported by SMF for a given DNN. |
| NrfInfo | 6.1.6.2.31 | Information of an NRF NF Instance, used in hierarchical NRF deployments. |
| ChfInfo | 6.1.6.2.32 | Information of a CHF NF Instance. |
| ChfServiceInfo | 6.1.6.2.33 | Information of primary and secondary CHF services. |
| PlmnRange | 6.1.6.2.34 | Range of PLMN IDs. |
| SubscrCond | 6.1.6.2.35 | Condition to determine the set of NFs to monitor under a certain subscription in NRF. |
| NfInstanceIdCond | 6.1.6.2.36 | Subscription to a given NF Instance Id. |
| NfTypeCond | 6.1.6.2.37 | Subscription to a set of NFs based on their NF Type. |
| ServiceNameCond | 6.1.6.2.38 | Subscription to a set of NFs based on their support for a given Service Name. |
| AmfCond | 6.1.6.2.39 | Subscription to a set of AMFs, based on AMF Set Id and/or AMF Region Id. |
| GuamiListCond | 6.1.6.2.40 | Subscription to a set of AMFs, based on their GUAMIs. |
| NetworkSliceCond | 6.1.6.2.41 | Subscription to a set of NFs, based on the slices (S-NSSAI and NSI) they support . |
| NfGroupCond | 6.1.6.2.42 | Subscription to a set of NFs based on their Group Id. |
| NotifCondition | 6.1.6.2.43 | Condition (list of attributes in the NF Profile) to determine whether a notification must be sent by NRF. |
| PlmnSnssai | 6.1.6.2.44 | List of network slices (S-NSSAIs) for a given PLMN ID. |
| NwdafInfo | 6.1.6.2.45 | Information of a NWDAF NF Instance. |
| LmfInfo | 6.1.6.2.46 | Information of an LMF NF Instance. |
| GmlcInfo | 6.1.6.2.47 | Information of a GMLC NF Instance. |
| NefInfo | 6.1.6.2.48 | Information of an NEF NF Instance. |
| PfdData | 6.1.6.2.49 | List of Application IDs and/or AF IDs managed by a given NEF Instance. |
| AfEventExposureData | 6.1.6.2.50 | AF Event Exposure data managed by a given NEF Instance. |
| WAgfInfo | 6.1.6.2.51 | Information of the W-AGF endpoints. |
| TngfInfo | 6.1.6.2.52 | Information of the TNGF endpoints. |
| PcscfInfo | 6.1.6.2.53 | Information of a P-CSCF NF Instance. |
| NfSetCond | 6.1.6.2.54 | Subscription to a set of NFs based on their Set Id. |
| NfServiceSetCond | 6.1.6.2.55 | Subscription to a set of NFs based on their Service Set Id. |
| NfInfo | 6.1.6.2.56 | Information of a generic NF Instance. |
| HssInfo | 6.1.6.2.57 | Information of an HSS NF Instance. |
| ImsiRange | 6.1.6.2.58 | A range of IMSIs (subscriber identities), either based on a numeric range, or based on regular-expression matching. |
| InternalGroupIdRange | 6.1.6.2.59 | A range of Group IDs (internal group identities), either based on a numeric range, or based on regular-expression matching. |
| Fqdn | 6.1.6.3.2 | Fully Qualified Domain Name. |
| NefId | 6.1.6.3.2 | Identity of the NEF. |
| NFType | 6.1.6.3.3 | NF types known to NRF. |
| NotificationType | 6.1.6.3.4 | Types of notifications used in Default Notification URIs in the NF Profile of an NF Instance. |
| TransportProtocol | 6.1.6.3.5 | Types of transport protocol used in a given IP endpoint of an NF Service Instance. |
| NotificationEventType | 6.1.6.3.6 | Types of events sent in notifications from NRF to subscribed NF Instances. |
| NFStatus | 6.1.6.3.7 | Status of a given NF Instance stored in NRF. |
| DataSetId | 6.1.6.3.8 | Types of data sets stored in UDR. |
| UPInterfaceType | 6.1.6.3.9 | Types of User-Plane interfaces of the UPF. |
| ServiceName | 6.1.6.3.11 | Service names known to NRF. |
| NFServiceStatus | 6.1.6.3.12 | Status of a given NF Service Instance of an NF Instance stored in NRF. |

Table 6.1.6.1-2 specifies data types re-used by the Nnrf_NFManagement service-based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nnrf_NFManagement service-based interface.

Table 6.1.6.1-2: Nnrf_NFManagement re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| N1MessageClass | 3GPP 29.518 [6] | The N1 message type |
| N2InformationClass | 3GPP 29.518 [6] | The N2 information type |
| IPv4Addr | 3GPP TS 29.571 [7] |  |
| IPv6Addr | 3GPP TS 29.571 [7] |  |
| IPv6Prefix | 3GPP TS 29.571 [7] |  |
| Uri | 3GPP TS 29.571 [7] |  |
| Dnn | 3GPP TS 29.571 [7] |  |
| SupportedFeatures | 3GPP TS 29.571 [7] |  |
| Snssai | 3GPP TS 29.571 [7] |  |
| PlmnId | 3GPP TS 29.571 [7] |  |
| Guami | 3GPP TS 29.571 [7] |  |
| Tai | 3GPP TS 29.571 [7] |  |
| NfInstanceId | 3GPP TS 29.571 [7] |  |
| LinksValueSchema | 3GPP TS 29.571 [7] | 3GPP Hypermedia link |
| UriScheme | 3GPP TS 29.571 [7] |  |
| AmfName | 3GPP TS 29.571 [7] |  |
| DateTime | 3GPP TS 29.571 [7] |  |
| Dnai | 3GPP TS 29.571 [7] |  |
| ChangeItem | 3GPP TS 29.571 [7] |  |
| DiameterIdentity | 3GPP TS 29.571 [7] |  |
| AccessType | 3GPP TS 29.571 [7] |  |
| NfGroupId | 3GPP TS 29.571 [7] | Network Function Group Id |
| AmfRegionId | 3GPP TS 29.571 [7] |  |
| AmfSetId | 3GPP TS 29.571 [7] |  |
| PduSessionType | 3GPP TS 29.571 [7] |  |
| AtsssCapability | 3GPP TS 29.571 [7] | Capability to support procedures related to Access Traffic Steering, Switching, Splitting. |
| Nid | 3GPP TS 29.571 [7] |  |
| PlmnIdNid | 3GPP TS 29.571 [7] |  |
| NfSetId | 3GPP TS 29.571 [7] | NF Set ID (see clause 28.10 of 3GPP TS 23.003 [12]) |
| NfServiceSetId | 3GPP TS 29.571 [7] | NF Service Set ID (see clause 28.11 of 3GPP TS 23.003 [12]) |
| GroupId | 3GPP TS 29.571 [7] | Internal Group Identifier |
| EventId | 3GPP TS 29.520 [32] | Defined in Nnwdaf_AnalyticsInfo API. |
| NwdafEvent | 3GPP TS 29.520 [32] | Defined in Nnwdaf_EventsSubscription API. |
| ExternalClientType | 3GPP TS 29.572 [33] |  |
| AfEvent | 3GPP TS 29.517 [35] | Defined in Naf_EventExposure API |

* * * Next Change * * * *

#### 6.2.6.1	General

This clause specifies the application data model supported by the API.

Table 6.2.6.1-1 specifies the data types defined for the Nnrf_NFDiscovery service-based interface protocol.

Table 6.2.6.1-1: Nnrf_NFDiscovery specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| SearchResult | 6.2.6.2.2 | Contains the list of NF Profiles returned in a Discovery response. |
| NFProfile | 6.2.6.2.3 | Information of an NF Instance discovered by the NRF. |
| NFService | 6.2.6.2.4 | Information of a given NF Service Instance; it is part of the NFProfile of an NF Instance discovered by the NRF. |
| StoredSearchResult | 6.2.6.2.5 | Contains a complete search result (i.e. a number of discovered NF Instances), stored by NRF as a consequence of a prior search result. |
| PreferredSearch | 6.2.6.2.6 | Contains information on whether the returned NFProfiles match the preferred query parameters. |

Table 6.2.6.1-2 specifies data types re-used by the Nnrf_NFDiscovery service-based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nnrf_NFDiscovery service-based interface.

Table 6.2.6.1-2: Nnrf_NFDiscovery re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| Snssai | 3GPP TS 29.571 [7] |  |
| PlmnId | 3GPP TS 29.571 [7] |  |
| Dnn | 3GPP TS 29.571 [7] |  |
| Tai | 3GPP TS 29.571 [7] |  |
| SupportedFeatures | 3GPP TS 29.571 [7] |  |
| NfInstanceId | 3GPP TS 29.571 [7] |  |
| Uri | 3GPP TS 29.571 [7] |  |
| Gpsi | 3GPP TS 29.571 [7] |  |
| GroupId | 3GPP TS 29.571 [7] |  |
| Guami | 3GPP TS 29.571 [7] |  |
| IPv4Addr | 3GPP TS 29.571 [7] |  |
| IPv6Addr | 3GPP TS 29.571 [7] |  |
| UriScheme | 3GPP TS 29.571 [7] |  |
| Dnai | 3GPP TS 29.571 [7] |  |
| NfGroupId | 3GPP TS 29.571 [7] | Identifier of a NF Group |
| PduSessionType | 3GPP TS 29.571 [7] |  |
| AtsssCapability | 3GPP TS 29.571 [7] |  |
| PlmnIdNid | 3GPP TS 29.571 [7] |  |
| NfSetId | 3GPP TS 29.571 [7] |  |
| NfServiceSetId | 3GPP TS 29.571 [7] |  |
| EventId | 3GPP TS 29.520 [32] | Defined in Nnwdaf_AnalyticsInfo API. |
| NwdafEvent | 3GPP TS 29.520 [32] | Defined in Nnwdaf_EventsSubscription API. |
| ExtGroupId | 3GPP TS 29.503 [36] |  |
| DefaultNotificationSubscription | 3GPP TS 29.510 | See clause 6.1.6.2.4 |
| IPEndPoint | 3GPP TS 29.510 | See clause 6.1.6.2.5 |
| NFType | 3GPP TS 29.510 | See clause 6.1.6.3.3 |
| UdrInfo | 3GPP TS 29.510 | See clause 6.1.6.2.6 |
| UdmInfo | 3GPP TS 29.510 | See clause 6.1.6.2.7 |
| AusfInfo | 3GPP TS 29.510 | See clause 6.1.6.2.8 |
| SupiRange | 3GPP TS 29.510 | See clause 6.1.6.2.9 |
| AmfInfo | 3GPP TS 29.510 | See clause 6.1.6.2.11 |
| SmfInfo | 3GPP TS 29.510 | See clause 6.1.6.2.12 |
| UpfInfo | 3GPP TS 29.510 | See clause 6.1.6.2.13 |
| PcfInfo | 3GPP TS 29.510 | See clause 6.1.6.2.20 |
| BsfInfo | 3GPP TS 29.510 | See clause 6.1.6.2.21 |
| ChfInfo | 3GPP TS 29.510 | See clause 6.1.6.2.32 |
| ChfServiceInfo | 3GPP TS 29.510 | See clause 6.1.6.2.33 |
| NFServiceVersion | 3GPP TS 29.510 | See clause 6.1.6.2.19 |
| PlmnSnssai | 3GPP TS 29.510 | See clause 6.1.6.2.44 |
| NwdafInfo | 3GPP TS 29.510 | See clause 6.1.6.2.45 |
| NFStatus | 3GPP TS 29.510 | See clause 6.1.6.3.7 |
| DataSetId | 3GPP TS 29.510 | See clause 6.1.6.3.8 |
| ServiceName | 3GPP TS 29.510 | See clause 6.1.6.3.11 |
| NFServiceStatus | 3GPP TS 29.510 | See clause 6.1.6.3.12 |
| LmfInfo | 3GPP TS 29.510 | See clause 6.1.6.2.46 |
| GmlcInfo | 3GPP TS 29.510 | See clause 6.1.6.2.47 |
| NefInfo | 3GPP TS 29.510 | See clause 6.1.6.2.48 |
| PfdData | 3GPP TS 29.510 | See clause 6.1.6.2.49 |
| AfEventExposureData | 3GPP TS 29.510 | See clause 6.1.6.2.50 |
| PcscfInfo | 3GPP TS 29.510 | See clause 6.1.6.2.53 |
| HssInfo | 3GPP TS 29.510 | See clause 6.1.6.2.57 |
| ImsiRange | 3GPP TS 29.510 | See clause 6.1.6.2.58 |
| NefId | 3GPP TS 29.510 | See clause 6.1.6.3 |

* * * Next Change * * * *

#### 6.3.5.1	General

This clause specifies the application data model supported by the API.

Table 6.3.5.1-1 specifies the data types defined for the OAuth 2.0 Authorization Service API. The AccessTokenReq data structure shall be converted to the content type "application/x-www-urlencoded" when the OAuth 2.0 Access Token Request is invoked.

Table 6.3.5.1-1: OAuth 2.0 Authorization service specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| AccessTokenReq | 6.3.5.2.2 | Contains information related to the access token request. |
| AccessTokenRsp | 6.3.5.2.3 | Contains information related to the access token response. |
| AccessTokenClaims | 6.3.5.2.4 | The claims data structure for the access token. |
| Audience | 6.3.5.4.1 | Contains the audience claim of the access token. |

Table 6.3.5.1-2 specifies data types re-used by the OAuth 2.0 Authorization service from other specifications, including a reference to their respective specifications and when needed, a short description of their use.

Table 6.3.5.1-2: OAuth 2.0 Authorization service re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| NfInstanceId | 3GPP TS 29.571 [7] |  |
| PlmnId | 3GPP TS 29.571 [7] | PLMN ID |
| NFType | 3GPP TS 29.510 | See clause 6.1.6.3.3 |
| Snssai | 3GPP TS 29.571 [7] |  |

* * * Next Change * * * *

#### 6.4.6.1	General

This clause specifies the application data model supported by the API.

Table 6.4.6.1-1 specifies the data types defined for the Nnrf_Bootstrapping service-based interface protocol.

Table 6.4.6.1-1: Nnrf_Bootstrapping specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| BootstrappingInfo | 6.4.6.2.2 | Information returned by NRF in the bootstrapping response message. |
| Status | 6.4.6.3.2 | Overal status of the NRF. |

Table 6.4.6.1-2 specifies data types re-used by the Nnrf_Bootstrapping service-based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nnrf service-based interface.

Table 6.4.6.1-2: Nnrf_Bootstrapping re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| LinksValueSchema | 3GPP TS 29.571 [7] | 3GPP Hypermedia link |
| ProblemDetails | 3GPP TS 29.571 [7] |  |

* * * Next Change * * * *

## A.2	Nnrf_NFManagement API

openapi: 3.0.0

(… text not shown for clarity …)

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '/oauth2/token'

scopes:

nnrf-nfm: Access to the Nnrf_NFManagement API

schemas:

NFProfile:

description: Information of an NF Instance registered in the NRF

type: object

required:

- nfInstanceId

- nfType

- nfStatus

anyOf:

- required: [ fqdn ]

- required: [ ipv4Addresses ]

- required: [ ipv6Addresses ]

properties:

nfInstanceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

nfInstanceName:

type: string

nfType:

$ref: '#/components/schemas/NFType'

nfStatus:

$ref: '#/components/schemas/NFStatus'

heartBeatTimer:

type: integer

plmnList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

minItems: 1

snpnList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnIdNid'

minItems: 1

sNssais:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

minItems: 1

perPlmnSnssaiList:

type: array

items:

$ref: '#/components/schemas/PlmnSnssai'

minItems: 1

nsiList:

type: array

items:

type: string

minItems: 1

fqdn:

$ref: '#/components/schemas/Fqdn'

interPlmnFqdn:

$ref: '#/components/schemas/Fqdn'

ipv4Addresses:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

minItems: 1

ipv6Addresses:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Addr'

minItems: 1

allowedPlmns:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

minItems: 1

allowedSnpns:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnIdNid'

minItems: 1

allowedNfTypes:

type: array

items:

$ref: '#/components/schemas/NFType'

minItems: 1

allowedNfDomains:

type: array

items:

type: string

minItems: 1

allowedNssais:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

minItems: 1

priority:

type: integer

minimum: 0

maximum: 65535

capacity:

type: integer

minimum: 0

maximum: 65535

load:

type: integer

minimum: 0

maximum: 100

locality:

type: string

udrInfo:

$ref: '#/components/schemas/UdrInfo'

udrInfoExt:

type: array

items:

$ref: '#/components/schemas/UdrInfo'

minItems: 1

udmInfo:

$ref: '#/components/schemas/UdmInfo'

udmInfoExt:

type: array

items:

$ref: '#/components/schemas/UdmInfo'

minItems: 1

ausfInfo:

$ref: '#/components/schemas/AusfInfo'

ausfInfoExt:

type: array

items:

$ref: '#/components/schemas/AusfInfo'

minItems: 1

amfInfo:

$ref: '#/components/schemas/AmfInfo'

amfInfoExt:

type: array

items:

$ref: '#/components/schemas/AmfInfo'

minItems: 1

smfInfo:

$ref: '#/components/schemas/SmfInfo'

smfInfoExt:

type: array

items:

$ref: '#/components/schemas/SmfInfo'

minItems: 1

upfInfo:

$ref: '#/components/schemas/UpfInfo'

upfInfoExt:

type: array

items:

$ref: '#/components/schemas/UpfInfo'

minItems: 1

pcfInfo:

$ref: '#/components/schemas/PcfInfo'

pcfInfoExt:

type: array

items:

$ref: '#/components/schemas/PcfInfo'

minItems: 1

bsfInfo:

$ref: '#/components/schemas/BsfInfo'

bsfInfoExt:

type: array

items:

$ref: '#/components/schemas/BsfInfo'

minItems: 1

chfInfo:

$ref: '#/components/schemas/ChfInfo'

chfInfoExt:

type: array

items:

$ref: '#/components/schemas/ChfInfo'

minItems: 1

nefInfo:

$ref: '#/components/schemas/NefInfo'

nrfInfo:

$ref: '#/components/schemas/NrfInfo'

nwdafInfo:

$ref: '#/components/schemas/NwdafInfo'

pcscfInfo:

type: array

items:

$ref: '#/components/schemas/PcscfInfo'

minItems: 1

hssInfo:

type: array

items:

$ref: '#/components/schemas/HssInfo'

minItems: 1

customInfo:

type: object

recoveryTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

nfServicePersistence:

type: boolean

default: false

nfServices:

type: array

items:

$ref: '#/components/schemas/NFService'

minItems: 1

nfProfileChangesSupportInd:

type: boolean

default: false

writeOnly: true

nfProfileChangesInd:

type: boolean

default: false

readOnly: true

defaultNotificationSubscriptions:

type: array

items:

$ref: '#/components/schemas/DefaultNotificationSubscription'

lmfInfo:

$ref: '#/components/schemas/LmfInfo'

gmlcInfo:

$ref: '#/components/schemas/GmlcInfo'

nfSetIdList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSetId'

minItems: 1

servingScope:

type: array

items:

type: string

minItems: 1

NFService:

description: Information of a given NF Service Instance; it is part of the NFProfile of an NF Instance

type: object

required:

- serviceInstanceId

- serviceName

- versions

- scheme

- nfServiceStatus

properties:

serviceInstanceId:

type: string

serviceName:

$ref: '#/components/schemas/ServiceName'

versions:

type: array

items:

$ref: '#/components/schemas/NFServiceVersion'

minItems: 1

scheme:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UriScheme'

nfServiceStatus:

$ref: '#/components/schemas/NFServiceStatus'

fqdn:

$ref: '#/components/schemas/Fqdn'

interPlmnFqdn:

$ref: '#/components/schemas/Fqdn'

ipEndPoints:

type: array

items:

$ref: '#/components/schemas/IpEndPoint'

minItems: 1

apiPrefix:

type: string

defaultNotificationSubscriptions:

type: array

items:

$ref: '#/components/schemas/DefaultNotificationSubscription'

minItems: 1

allowedPlmns:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

minItems: 1

allowedSnpns:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnIdNid'

minItems: 1

allowedNfTypes:

type: array

items:

$ref: '#/components/schemas/NFType'

minItems: 1

allowedNfDomains:

type: array

items:

type: string

minItems: 1

allowedNssais:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

minItems: 1

priority:

type: integer

minimum: 0

maximum: 65535

capacity:

type: integer

minimum: 0

maximum: 65535

load:

type: integer

minimum: 0

maximum: 100

recoveryTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

chfServiceInfo:

$ref: '#/components/schemas/ChfServiceInfo'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

nfServiceSetIdList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfServiceSetId'

minItems: 1

NFType:

description: NF types known to NRF

anyOf:

- type: string

enum:

- NRF

- UDM

- AMF

- SMF

- AUSF

- NEF

- PCF

- SMSF

- NSSF

- UDR

- LMF

- GMLC

- 5G_EIR

- SEPP

- UPF

- N3IWF

- AF

- UDSF

- BSF

- CHF

- NWDAF

- PCSCF

- CBCF

- HSS

- UCMF

- type: string

Fqdn:

description: Fully Qualified Domain Name

type: string

NefId:

description: Identity of the NEF

type: string

IpEndPoint:

description: IP addressing information of a given NFService; it consists on, e.g. IP address, TCP port, transport protocol...

type: object

properties:

ipv4Address:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

ipv6Address:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Addr'

transport:

$ref: '#/components/schemas/TransportProtocol'

port:

type: integer

minimum: 0

maximum: 65535

SubscriptionData:

description: Information of a subscription to notifications to NRF events, included in subscription requests and responses

type: object

required:

- nfStatusNotificationUri

- subscriptionId

properties:

nfStatusNotificationUri:

type: string

reqNfInstanceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

subscrCond:

oneOf:

- $ref: '#/components/schemas/NfInstanceIdCond'

- $ref: '#/components/schemas/NfTypeCond'

- $ref: '#/components/schemas/ServiceNameCond'

- $ref: '#/components/schemas/AmfCond'

- $ref: '#/components/schemas/GuamiListCond'

- $ref: '#/components/schemas/NetworkSliceCond'

- $ref: '#/components/schemas/NfGroupCond'

- $ref: '#/components/schemas/NfSetCond'

- $ref: '#/components/schemas/NfServiceSetCond'

subscriptionId:

type: string

pattern: '^([0-9]{5,6}-)?[^-]+$'

readOnly: true

validityTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

reqNotifEvents:

type: array

items:

$ref: '#/components/schemas/NotificationEventType'

minItems: 1

plmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

nid:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Nid'

notifCondition:

$ref: '#/components/schemas/NotifCondition'

reqNfType:

$ref: '#/components/schemas/NFType'

reqNfFqdn:

$ref: '#/components/schemas/Fqdn'

reqSnssais:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

minItems: 1

reqPlmnList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

minItems: 1

NfInstanceIdCond:

description: Subscription to a given NF Instance Id

type: object

required:

- nfInstanceId

properties:

nfInstanceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

NfTypeCond:

description: Subscription to a set of NFs based on their NF Type

type: object

required:

- nfType

not:

required: [ nfGroupId ]

properties:

nfType:

$ref: '#/components/schemas/NFType'

ServiceNameCond:

description: Subscription to a set of NFs based on their support for a given Service Name

type: object

required:

- serviceName

properties:

serviceName:

$ref: '#/components/schemas/ServiceName'

AmfCond:

description: Subscription to a set of AMFs, based on AMF Set Id and/or AMF Region Id

type: object

anyOf:

- required: [ amfSetId ]

- required: [ amfRegionId ]

properties:

amfSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AmfSetId'

amfRegionId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AmfRegionId'

GuamiListCond:

description: Subscription to a set of AMFs, based on their GUAMIs

type: object

required:

- guamiList

properties:

guamiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Guami'

NetworkSliceCond:

description: Subscription to a set of NFs, based on the slices (S-NSSAI and NSI) they support

type: object

required:

- snssaiList

properties:

snssaiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

nsiList:

type: array

items:

type: string

NfGroupCond:

description: Subscription to a set of NFs based on their Group Id

type: object

required:

- nfType

- nfGroupId

properties:

nfType:

type: string

enum:

- UDM

- AUSF

- UDR

- PCF

nfGroupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfGroupId'

NotifCondition:

description: Condition (list of attributes in the NF Profile) to determine whether a notification must be sent by NRF

type: object

not:

required: [ monitoredAttributes, unmonitoredAttributes ]

properties:

monitoredAttributes:

type: array

items:

type: string

minItems: 1

unmonitoredAttributes:

type: array

items:

type: string

minItems: 1

UdrInfo:

description: Information of an UDR NF Instance

type: object

properties:

groupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfGroupId'

supiRanges:

type: array

items:

$ref: '#/components/schemas/SupiRange'

minItems: 1

gpsiRanges:

type: array

items:

$ref: '#/components/schemas/IdentityRange'

minItems: 1

externalGroupIdentifiersRanges:

type: array

items:

$ref: '#/components/schemas/IdentityRange'

minItems: 1

supportedDataSets:

type: array

items:

$ref: '#/components/schemas/DataSetId'

minItems: 1

SupiRange:

description: A range of SUPIs (subscriber identities), either based on a numeric range, or based on regular-expression matching

type: object

properties:

start:

type: string

pattern: '^[0-9]+$'

end:

type: string

pattern: '^[0-9]+$'

pattern:

type: string

IdentityRange:

description: A range of GPSIs (subscriber identities), either based on a numeric range, or based on regular-expression matching

type: object

properties:

start:

type: string

pattern: '^[0-9]+$'

end:

type: string

pattern: '^[0-9]+$'

pattern:

type: string

InternalGroupIdRange:

description: A range of Group IDs (internal group identities), either based on a numeric range, or based on regular-expression matching

type: object

properties:

start:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GroupId'

end:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GroupId'

pattern:

type: string

DataSetId:

description: Types of data sets stored in UDR

anyOf:

- type: string

enum:

- SUBSCRIPTION

- POLICY

- EXPOSURE

- APPLICATION

- type: string

UdmInfo:

description: Information of an UDM NF Instance

type: object

properties:

groupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfGroupId'

supiRanges:

type: array

items:

$ref: '#/components/schemas/SupiRange'

minItems: 1

gpsiRanges:

type: array

items:

$ref: '#/components/schemas/IdentityRange'

minItems: 1

externalGroupIdentifiersRanges:

type: array

items:

$ref: '#/components/schemas/IdentityRange'

minItems: 1

routingIndicators:

type: array

items:

type: string

pattern: '^[0-9]{1,4}$'

minItems: 1

internalGroupIdentifiersRanges:

type: array

items:

$ref: '#/components/schemas/InternalGroupIdRange'

minItems: 1

AusfInfo:

description: Information of an AUSF NF Instance

type: object

properties:

groupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfGroupId'

supiRanges:

type: array

items:

$ref: '#/components/schemas/SupiRange'

minItems: 1

routingIndicators:

type: array

items:

type: string

pattern: '^[0-9]{1,4}$'

minItems: 1

AmfInfo:

description: Information of an AMF NF Instance

type: object

required:

- amfSetId

- amfRegionId

- guamiList

properties:

amfSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AmfSetId'

amfRegionId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AmfRegionId'

guamiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Guami'

minItems: 1

taiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Tai'

minItems: 1

taiRangeList:

type: array

items:

$ref: '#/components/schemas/TaiRange'

minItems: 1

backupInfoAmfFailure:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Guami'

minItems: 1

backupInfoAmfRemoval:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Guami'

minItems: 1

n2InterfaceAmfInfo:

$ref: '#/components/schemas/N2InterfaceAmfInfo'

SmfInfo:

description: Information of an SMF NF Instance

type: object

required:

- sNssaiSmfInfoList

properties:

sNssaiSmfInfoList:

type: array

items:

$ref: '#/components/schemas/SnssaiSmfInfoItem'

minItems: 1

taiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Tai'

minItems: 1

taiRangeList:

type: array

items:

$ref: '#/components/schemas/TaiRange'

minItems: 1

pgwFqdn:

$ref: '#/components/schemas/Fqdn'

accessType:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

minItems: 1

priority:

type: integer

minimum: 0

maximum: 65535

SnssaiSmfInfoItem:

description: Set of parameters supported by SMF for a given S-NSSAI

type: object

required:

- sNssai

- dnnSmfInfoList

properties:

sNssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

dnnSmfInfoList:

type: array

items:

$ref: '#/components/schemas/DnnSmfInfoItem'

minItems: 1

DnnSmfInfoItem:

description: Set of parameters supported by SMF for a given DNN

type: object

required:

- dnn

properties:

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

UpfInfo:

description: Information of an UPF NF Instance

type: object

required:

- sNssaiUpfInfoList

properties:

sNssaiUpfInfoList:

type: array

items:

$ref: '#/components/schemas/SnssaiUpfInfoItem'

minItems: 1

smfServingArea:

type: array

items:

type: string

minItems: 1

interfaceUpfInfoList:

type: array

items:

$ref: '#/components/schemas/InterfaceUpfInfoItem'

minItems: 1

iwkEpsInd:

type: boolean

default: false

pduSessionTypes:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionType'

minItems: 1

atsssCapability:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AtsssCapability'

ueIpAddrInd:

type: boolean

default: false

taiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Tai'

minItems: 1

wAgfInfo:

$ref: '#/components/schemas/WAgfInfo'

tngfInfo:

$ref: '#/components/schemas/TngfInfo'

priority:

type: integer

minimum: 0

maximum: 65535

SnssaiUpfInfoItem:

description: Set of parameters supported by UPF for a given S-NSSAI

type: object

required:

- sNssai

- dnnUpfInfoList

properties:

sNssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

dnnUpfInfoList:

type: array

items:

$ref: '#/components/schemas/DnnUpfInfoItem'

minItems: 1

DnnUpfInfoItem:

description: Set of parameters supported by UPF for a given DNN

type: object

required:

- dnn

properties:

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

dnaiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

minItems: 1

pduSessionTypes:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionType'

minItems: 1

ipv4AddressRanges:

type: array

items:

$ref: '#/components/schemas/Ipv4AddressRange'

minItems: 1

ipv6PrefixRanges:

type: array

items:

$ref: '#/components/schemas/Ipv6PrefixRange'

minItems: 1

InterfaceUpfInfoItem:

description: Information of a given IP interface of an UPF

type: object

required:

- interfaceType

properties:

interfaceType:

$ref: '#/components/schemas/UPInterfaceType'

ipv4EndpointAddresses:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

minItems: 1

ipv6EndpointAddresses:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Addr'

minItems: 1

endpointFqdn:

$ref: '#/components/schemas/Fqdn'

networkInstance:

type: string

UPInterfaceType:

description: Types of User-Plane interfaces of the UPF

anyOf:

- type: string

enum:

- N3

- N6

- N9

- type: string

WAgfInfo:

description: Information of the W-AGF end-points

type: object

properties:

ipv4EndpointAddresses:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

minItems: 1

ipv6EndpointAddresses:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Addr'

minItems: 1

endpointFqdn:

$ref: '#/components/schemas/Fqdn'

TngfInfo:

description: Infomation of the TNGF endpoints

type: object

properties:

ipv4EndpointAddresses:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

minItems: 1

ipv6EndpointAddresses:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Addr'

minItems: 1

endpointFqdn:

$ref: '#/components/schemas/Fqdn'

PcfInfo:

description: Information of a PCF NF Instance

type: object

properties:

groupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfGroupId'

dnnList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

minItems: 1

supiRanges:

type: array

items:

$ref: '#/components/schemas/SupiRange'

minItems: 1

gpsiRanges:

type: array

items:

$ref: '#/components/schemas/IdentityRange'

minItems: 1

rxDiamHost:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DiameterIdentity'

rxDiamRealm:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DiameterIdentity'

BsfInfo:

description: Information of a BSF NF Instance

type: object

properties:

dnnList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

minItems: 1

ipDomainList:

type: array

items:

type: string

minItems: 1

ipv4AddressRanges:

type: array

items:

$ref: '#/components/schemas/Ipv4AddressRange'

minItems: 1

ipv6PrefixRanges:

type: array

items:

$ref: '#/components/schemas/Ipv6PrefixRange'

minItems: 1

ChfInfo:

description: Information of a CHF NF Instance

type: object

properties:

supiRangeList:

type: array

items:

$ref: '#/components/schemas/SupiRange'

minItems: 1

gpsiRangeList:

type: array

items:

$ref: '#/components/schemas/IdentityRange'

minItems: 1

plmnRangeList:

type: array

items:

$ref: '#/components/schemas/PlmnRange'

minItems: 1

Ipv4AddressRange:

description: Range of IPv4 addresses

type: object

properties:

start:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

end:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

Ipv6PrefixRange:

description: Range of IPv6 prefixes

type: object

properties:

start:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

end:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

DefaultNotificationSubscription:

description: Data structure for specifying the notifications the NF service subscribes by default along with callback URI

type: object

required:

- notificationType

- callbackUri

properties:

notificationType:

$ref: '#/components/schemas/NotificationType'

callbackUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

n1MessageClass:

$ref: 'TS29518_Namf_Communication.yaml#/components/schemas/N1MessageClass'

n2InformationClass:

$ref: 'TS29518_Namf_Communication.yaml#/components/schemas/N2InformationClass'

NfSetCond:

description: Subscription to a set of NFs based on their Set Id

type: object

required:

- nfSetId

properties:

nfSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSetId'

NfServiceSetCond:

description: Subscription to a set of NFs based on their Service Set Id

type: object

required:

- nfServiceSetId

properties:

nfServiceSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfServiceSetId'

NotificationType:

description: Types of notifications used in Default Notification URIs in the NF Profile of an NF Instance

anyOf:

- type: string

enum:

- N1_MESSAGES

- N2_INFORMATION

- LOCATION_NOTIFICATION

- DATA_REMOVAL_NOTIFICATION

- DATA_CHANGE_NOTIFICATION

- LOCATION_UPDATE_NOTIFICATION

- type: string

TransportProtocol:

description: Types of transport protocol used in a given IP endpoint of an NF Service Instance

anyOf:

- type: string

enum:

- TCP

- type: string

NotificationEventType:

description: Types of events sent in notifications from NRF to subscribed NF Instances

anyOf:

- type: string

enum:

- NF_REGISTERED

- NF_DEREGISTERED

- NF_PROFILE_CHANGED

- type: string

NotificationData:

description: Data sent in notifications from NRF to subscribed NF Instances

type: object

required:

- event

- nfInstanceUri

allOf:

#

# Condition: If 'event' takes value 'NF_PROFILE_CHANGED',

# then either 'nfProfile' or 'profileChanges' (but not both) must be present

#

- anyOf:

- not:

properties:

event:

type: string

enum:

- NF_PROFILE_CHANGED

- oneOf:

- required: [ nfProfile ]

- required: [ profileChanges ]

#

# Condition: If 'event' takes value 'NF_REGISTERED',

# then 'nfProfile' must be present

#

- anyOf:

- not:

properties:

event:

type: string

enum:

- NF_REGISTERED

- required: [ nfProfile ]

properties:

event:

$ref: '#/components/schemas/NotificationEventType'

nfInstanceUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

nfProfile:

allOf:

- $ref: '#/components/schemas/NFProfile'

- not:

required: [ interPlmnFqdn ]

- not:

required: [ allowedPlmns ]

- not:

required: [ allowedNfTypes ]

- not:

required: [ allowedNfDomains ]

- not:

required: [ allowedNssais ]

- properties:

nfServices:

type: array

items:

allOf:

- $ref: '#/components/schemas/NFService'

- not:

required: [ interPlmnFqdn ]

- not:

required: [ allowedPlmns ]

- not:

required: [ allowedNfTypes ]

- not:

required: [ allowedNfDomains ]

- not:

required: [ allowedNssais ]

profileChanges:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ChangeItem'

minItems: 1

NFStatus:

description: Status of a given NF Instance stored in NRF

anyOf:

- type: string

enum:

- REGISTERED

- SUSPENDED

- UNDISCOVERABLE

- type: string

NFServiceVersion:

description: Contains the version details of an NF service

type: object

required:

- apiVersionInUri

- apiFullVersion

properties:

apiVersionInUri:

type: string

apiFullVersion:

type: string

expiry:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

ServiceName:

description: Service names known to NRF

anyOf:

- type: string

enum:

- nnrf-nfm

- nnrf-disc

- nudm-sdm

- nudm-uecm

- nudm-ueau

- nudm-ee

- nudm-pp

- nudm-niddau

- nudm-mt

- namf-comm

- namf-evts

- namf-mt

- namf-loc

- nsmf-pdusession

- nsmf-event-exposure

- nausf-auth

- nausf-sorprotection

- nausf-upuprotection

- nnef-pfdmanagement

- npcf-am-policy-control

- npcf-smpolicycontrol

- npcf-policyauthorization

- npcf-bdtpolicycontrol

- npcf-eventexposure

- npcf-ue-policy-control

- nsmsf-sms

- nnssf-nsselection

- nnssf-nssaiavailability

- nudr-dr

- nlmf-loc

- n5g-eir-eic

- nbsf-management

- nchf-spendinglimitcontrol

- nchf-convergedcharging

- nnwdaf-eventssubscription

- nnwdaf-analyticsinfo

- ngmlc-loc

- nucmf-provisioning

- nucmf-uecapabilitymanagement

- nhss-sdm

- nhss-uecm

- nhss-ueau

- type: string

N2InterfaceAmfInfo:

description: AMF N2 interface information

type: object

properties:

ipv4EndpointAddress:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

minItems: 1

ipv6EndpointAddress:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Addr'

minItems: 1

amfName:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AmfName'

NFServiceStatus:

description: Status of a given NF Service Instance of an NF Instance stored in NRF

anyOf:

- type: string

enum:

- REGISTERED

- SUSPENDED

- UNDISCOVERABLE

- type: string

TaiRange:

description: Range of TAIs (Tracking Area Identities)

type: object

required:

- plmnId

- tacRangeList

properties:

plmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

tacRangeList:

type: array

items:

$ref: '#/components/schemas/TacRange'

minItems: 1

nid:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Nid'

TacRange:

description: Range of TACs (Tracking Area Codes)

type: object

properties:

start:

type: string

pattern: '^([A-Fa-f0-9]{4}|[A-Fa-f0-9]{6})$'

end:

type: string

pattern: '^([A-Fa-f0-9]{4}|[A-Fa-f0-9]{6})$'

pattern:

type: string

ChfServiceInfo:

description: Information of primary and secondary CHF services

type: object

not:

required: [ primaryChfServiceInstance, secondaryChfServiceInstance ]

properties:

primaryChfServiceInstance:

type: string

secondaryChfServiceInstance:

type: string

PlmnRange:

description: Range of PLMN IDs

type: object

properties:

start:

type: string

pattern: '^[0-9]{3}[0-9]{2,3}$'

end:

type: string

pattern: '^[0-9]{3}[0-9]{2,3}$'

pattern:

type: string

NrfInfo:

description: Information of an NRF NF Instance, used in hierarchical NRF deployments

type: object

properties:

servedUdrInfo:

type: object

additionalProperties:

$ref: '#/components/schemas/UdrInfo'

minProperties: 1

servedUdmInfo:

type: object

additionalProperties:

$ref: '#/components/schemas/UdmInfo'

minProperties: 1

servedAusfInfo:

type: object

additionalProperties:

$ref: '#/components/schemas/AusfInfo'

minProperties: 1

servedAmfInfo:

type: object

additionalProperties:

$ref: '#/components/schemas/AmfInfo'

minProperties: 1

servedSmfInfo:

type: object

additionalProperties:

$ref: '#/components/schemas/SmfInfo'

minProperties: 1

servedUpfInfo:

type: object

additionalProperties:

$ref: '#/components/schemas/UpfInfo'

minProperties: 1

servedPcfInfo:

type: object

additionalProperties:

$ref: '#/components/schemas/PcfInfo'

minProperties: 1

servedBsfInfo:

type: object

additionalProperties:

$ref: '#/components/schemas/BsfInfo'

minProperties: 1

servedChfInfo:

type: object

additionalProperties:

$ref: '#/components/schemas/ChfInfo'

minProperties: 1

servedNefInfo:

type: object

additionalProperties:

$ref: '#/components/schemas/NefInfo'

minProperties: 1

servedNwdafInfo:

type: object

additionalProperties:

$ref: '#/components/schemas/NwdafInfo'

minProperties: 1

servedPcscfInfo:

type: object

additionalProperties:

$ref: '#/components/schemas/PcscfInfo'

minProperties: 1

servedGmlcInfo:

type: object

additionalProperties:

$ref: '#/components/schemas/GmlcInfo'

minProperties: 1

servedLmfInfo:

type: object

additionalProperties:

$ref: '#/components/schemas/LmfInfo'

minProperties: 1

servedNfInfo:

type: object

additionalProperties:

$ref: '#/components/schemas/NfInfo'

minProperties: 1

servedHssInfo:

type: object

additionalProperties:

$ref: '#/components/schemas/HssInfo'

minProperties: 1

PlmnSnssai:

description: List of network slices (S-NSSAIs) for a given PLMN ID

type: object

required:

- plmnId

- sNssaiList

properties:

plmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

sNssaiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

minItems: 1

nid:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Nid'

NefInfo:

description: Information of an NEF NF Instance

type: object

properties:

nefId:

$ref: '#/components/schemas/NefId'

pfdData:

$ref: '#/components/schemas/PfdData'

afEeData:

$ref: '#/components/schemas/AfEventExposureData'

PfdData:

description: List of Application IDs and/or AF IDs managed by a given NEF Instance

type: object

properties:

appIds:

type: array

items:

type: string

minItems: 1

afIds:

type: array

items:

type: string

minItems: 1

NwdafInfo:

description: Information of a NWDAF NF Instance

type: object

properties:

eventIds:

type: array

items:

$ref: 'TS29520_Nnwdaf_AnalyticsInfo.yaml#/components/schemas/EventId'

minItems: 1

nwdafEvents:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/NwdafEvent'

minItems: 1

taiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Tai'

minItems: 1

taiRangeList:

type: array

items:

$ref: '#/components/schemas/TaiRange'

minItems: 1

LmfInfo:

type: object

properties:

servingClientTypes:

type: array

items:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/ExternalClientType'

minItems: 1

GmlcInfo:

type: object

properties:

servingClientTypes:

type: array

items:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/ExternalClientType'

minItems: 1

AfEventExposureData:

description: AF Event Exposure data managed by a given NEF Instance

type: object

required:

- afEvents

properties:

afEvents:

type: array

items:

$ref: 'TS29517_Naf_EventExposure.yaml#/components/schemas/AfEvent'

minItems: 1

afIds:

type: array

items:

type: string

minItems: 1

appIds:

type: array

items:

type: string

minItems: 1

PcscfInfo:

description: Information of a P-CSCF NF Instance

type: object

properties:

accessType:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

minItems: 1

dnnList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

minItems: 1

NfInfo:

description: Information of a generic NF Instance

type: object

properties:

nfType:

$ref: '#/components/schemas/NFType'

HssInfo:

description: Information of an HSS NF Instance

type: object

properties:

groupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfGroupId'

imsRanges:

type: array

items:

$ref: '#/components/schemas/ImsiRange'

minItems: 1

ImsiRange:

description: A range of IMSIs (subscriber identities), either based on a numeric range, or based on regular-expression matching

type: object

properties:

start:

type: string

pattern: '^[0-9]+$'

end:

type: string

pattern: '^[0-9]+$'

pattern:

type: string

* * * Next Change * * * *

## A.3	Nnrf_NFDiscovery API

openapi: 3.0.0

(… text not shown for clarity …)

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '/oauth2/token'

scopes:

nnrf-disc: Access to the Nnrf_NFDiscovery API

parameters:

searchId:

name: searchId

in: path

description: Id of a stored search

required: true

schema:

type: string

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/StoredSearchResult'

headers:

Cache-Control:

description: Cache-Control containing max-age, described in IETF RFC 7234, 5.2

schema:

type: string

ETag:

description: Entity Tag containing a strong validator, described in IETF RFC 7232, 2.3

schema:

type: string

schemas:

SearchResult:

description: Contains the list of NF Profiles returned in a Discovery response

type: object

required:

- nfInstances

properties:

validityPeriod:

type: integer

nfInstances:

type: array

items:

$ref: '#/components/schemas/NFProfile'

searchId:

type: string

numNfInstComplete:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

preferredSearch:

$ref: '#/components/schemas/PreferredSearch'

nrfSupportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

StoredSearchResult:

description: Contains a complete search result (i.e. a number of discovered NF Instances), stored by NRF as a consequence of a prior search result

type: object

required:

- nfInstances

properties:

nfInstances:

type: array

items:

$ref: '#/components/schemas/NFProfile'

NFProfile:

description: Information of an NF Instance discovered by the NRF

type: object

required:

- nfInstanceId

- nfType

- nfStatus

properties:

nfInstanceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

nfInstanceName:

type: string

nfType:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/NFType'

nfStatus:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/NFStatus'

plmnList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

minItems: 1

sNssais:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

minItems: 1

perPlmnSnssaiList:

type: array

items:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/PlmnSnssai'

minItems: 1

nsiList:

type: array

items:

type: string

minItems: 1

fqdn:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/Fqdn'

ipv4Addresses:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

minItems: 1

ipv6Addresses:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Addr'

minItems: 1

capacity:

type: integer

minimum: 0

maximum: 65535

load:

type: integer

minimum: 0

maximum: 100

locality:

type: string

priority:

type: integer

minimum: 0

maximum: 65535

udrInfo:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/UdrInfo'

udrInfoExt:

type: array

items:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/UdrInfo'

minItems: 1

udmInfo:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/UdmInfo'

udmInfoExt:

type: array

items:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/UdmInfo'

minItems: 1

ausfInfo:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/AusfInfo'

ausfInfoExt:

type: array

items:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/AusfInfo'

minItems: 1

amfInfo:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/AmfInfo'

amfInfoExt:

type: array

items:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/AmfInfo'

minItems: 1

smfInfo:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/SmfInfo'

smfInfoExt:

type: array

items:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/SmfInfo'

minItems: 1

upfInfo:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/UpfInfo'

upfInfoExt:

type: array

items:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/UpfInfo'

minItems: 1

pcfInfo:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/PcfInfo'

pcfInfoExt:

type: array

items:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/PcfInfo'

minItems: 1

bsfInfo:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/BsfInfo'

bsfInfoExt:

type: array

items:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/BsfInfo'

minItems: 1

chfInfo:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/ChfInfo'

chfInfoExt:

type: array

items:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/ChfInfo'

minItems: 1

nwdafInfo:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/NwdafInfo'

nefInfo:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/NefInfo'

pcscfInfo:

type: array

items:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/PcscfInfo'

minItems: 1

hssInfo:

type: array

items:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/HssInfo'

minItems: 1

customInfo:

type: object

recoveryTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

nfServicePersistence:

type: boolean

default: false

nfServices:

type: array

items:

$ref: '#/components/schemas/NFService'

minItems: 1

defaultNotificationSubscriptions:

type: array

items:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/DefaultNotificationSubscription'

lmfInfo:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/LmfInfo'

gmlcInfo:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/GmlcInfo'

snpnList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnIdNid'

minItems: 1

nfSetIdList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSetId'

minItems: 1

servingScope:

type: array

items:

type: string

minItems: 1

NFService:

description: Information of a given NF Service Instance; it is part of the NFProfile of an NF Instance discovered by the NRF

type: object

required:

- serviceInstanceId

- serviceName

- versions

- scheme

- nfServiceStatus

properties:

serviceInstanceId:

type: string

serviceName:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/ServiceName'

versions:

type: array

items:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/NFServiceVersion'

minItems: 1

scheme:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UriScheme'

nfServiceStatus:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/NFServiceStatus'

fqdn:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/Fqdn'

ipEndPoints:

type: array

items:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/IpEndPoint'

minItems: 1

apiPrefix:

type: string

defaultNotificationSubscriptions:

type: array

items:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/DefaultNotificationSubscription'

minItems: 1

capacity:

type: integer

minimum: 0

maximum: 65535

load:

type: integer

minimum: 0

maximum: 100

priority:

type: integer

minimum: 0

maximum: 65535

recoveryTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

chfServiceInfo:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/ChfServiceInfo'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

nfServiceSetIdList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfServiceSetId'

minItems: 1

PreferredSearch:

description: Contains information on whether the returned NFProfiles match the preferred query parameters

type: object

properties:

preferredTaiMatchInd:

type: boolean

default: false

* * * Next Change * * * *

## A.4	Nnrf_AccessToken API (NRF OAuth2 Authorization)

openapi: 3.0.0

(… text not shown for clarity …)

components:

headers:

cache-control:

required: true

schema:

type: string

enum:

- no-store

pragma:

required: true

schema:

type: string

enum:

- no-cache

schemas:

AccessTokenReq:

description: Contains information related to the access token request

type: object

required:

- grant_type

- nfInstanceId

- scope

properties:

grant_type:

type: string

enum:

- client_credentials

nfInstanceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

nfType:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/NFType'

targetNfType:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/NFType'

scope:

type: string

pattern: '^([a-zA-Z0-9_-]+)( [a-zA-Z0-9_-]+)*$'

targetNfInstanceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

requesterPlmn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

targetPlmn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

targetSnssaiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

minItems: 1

targetNsiList:

type: array

items:

type: string

minItems: 1

AccessTokenRsp:

description: Contains information related to the access token response

type: object

required:

- access_token

- token_type

properties:

access_token:

type: string

description: JWS Compact Serialized representation of JWS signed JSON object (AccessTokenClaims)

token_type:

type: string

enum:

- Bearer

expires_in:

type: integer

scope:

type: string

pattern: '^([a-zA-Z0-9_-]+)( [a-zA-Z0-9_-]+)*$'

AccessTokenClaims:

description: The claims data structure for the access token

type: object

required:

- iss

- sub

- aud

- scope

- exp

properties:

iss:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

sub:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

aud:

anyOf:

- $ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/NFType'

- type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

minItems: 1

scope:

type: string

pattern: '^([a-zA-Z0-9_-]+)( [a-zA-Z0-9_-]+)*$'

exp:

type: integer

consumerPlmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

producerPlmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

producerSnssaiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

minItems: 1

producerNsiList:

type: array

items:

type: string

minItems: 1

AccessTokenErr:

description: Error returned in the access token response message

type: object

required:

- error

properties:

error:

type: string

enum:

- invalid_request

- invalid_client

- invalid_grant

- unauthorized_client

- unsupported_grant_type

- invalid_scope

error_description:

type: string

error_uri:

type: string

* * * Next Change * * * *

## A.5	Nnrf_Bootstrapping API

openapi: 3.0.0

(… text not shown for clarity …)

components:

schemas:

BootstrappingInfo:

description: Information returned by NRF in the bootstrapping response message

type: object

required:

- _links

properties:

status:

$ref: '#/components/schemas/Status'

_links:

type: object

description: 'Map of link objects where the keys are the link relations defined in 3GPP TS 29.510 clause 6.4.6.3.3'

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/LinksValueSchema'

minProperties: 1

Status:

description: Overal status of the NRF

anyOf:

- type: string

enum:

- OPERATIVE

- NON_OPERATIVE

- type: string

* * * End of Changes * * * *