3GPP TSG-CT Meeting #88e	CP-201197

# E-Meeting, 29th June – 1st July 2020													Revision of C4-203198

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.518 | CR | 0357 | rev | 1 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PC5 policy container from PCF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XARC |  |  |  |  |  | Date: |  |  | 2020-06-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | As specified in 3GPP TS 23.287: In the roaming case, the H-PCF includes the PC5 QoS parameters used by NG-RAN in the Npcf_UEPolicyControl_Create Response message or Npcf_UEPolicyControl UpdateNotify Request message sent to V-PCF in an N2 PC5 policy container, and V-PCF relays this N2 PC5 policy container as the N2 container in the Namf_Communication_N1N2MessageTransfer message sent to AMF. In the non-roaming case, the PCF includes the PC5 QoS parameters used by NG-RAN in an N2 container in Namf_Communication_N1N2MessageTransfer message sent to AMF. As specified in 3GPP TS 29.525: - if the (V-)PCF determines that N2 PC5 policy needs to be provisioned, it shall use the Namf_Communication service specified in 3GPP TS 29.518 [14] to provision the N2 PC5 according to subclause 4.2.2.3. The data type of N2 PC5 policy is N2InfoContent defined in clause 5.6.2.2 of TS29.525. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new IE V2xInformation in N2InfoContainer; NgapIeType in N2InfoContent is extended to support the type of V2X; Define new type V2X in N2InformationClass; Define new type PC5_QOS_PARA in NgapIeType; Add new table for N2 Information content for class V2X. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PC5 QoS parameters can not be received. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.2.15, 6.1.6.2.27, 6.1.6.2.x(new), 6.1.6.3.4, 6.1.6.3.17, 6.1.6.4.3.2, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible corrections to the OpenAPI file for Namf_Communication API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: Add V2xInformation in Table 6.1.6.1-1. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Namf_Communication service based interface protocol.

Table 6.1.6.1-1: Namf_Communication specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| SubscriptionData | 6.1.6.2.2 | Information within AMFStatusChangeSubscribe |
| AmfStatusChangeNotification | 6.1.6.2.3 | Information within AMFStatusChangeNotify |
| AmfStatusInfo | 6.1.6.2.4 | Information within AMFStatusChangeNotify |
| AssignEbiData | 6.1.6.2.5 | Represents information needed for AMF to assign EBIs. |
| AssignedEbiData | 6.1.6.2.6 | Represents successful assignment of EBI(s). |
| AssignEbiFailed | 6.1.6.2.7 | Represents failed assignment of EBI(s) |
| UEContextRelease | 6.1.6.2.8 | Information within ReleaseUeContext |
| N2InformationTransferReqData | 6.1.6.2.9 | N2 information requested to be transferred to 5G AN. |
| NonUeN2InfoSubscriptionCreateData | 6.1.6.2.10 | Subscription information for non UE specific N2 information notification. |
| NonUeN2InfoSubscriptionCreatedData | 6.1.6.2.11 | The created subscription for non UE specific N2 information notification. |
| UeN1N2InfoSubscriptionCreateData | 6.1.6.2.12 | Subscription information for UE specific N1 and/or N2 information notification. |
| UeN1N2InfoSubscriptionCreatedData | 6.1.6.2.13 | The created subscription for UE specific N1 and/or N2 information notification. |
| N2InformationNotification | 6.1.6.2.14 | N2 information for notification. |
| N2InfoContainer | 6.1.6.2.15 | N2 information container. |
| N1MessageNotification | 6.1.6.2.16 | N1 message notification data structure. |
| N1MessageContainer | 6.1.6.2.17 | N1 Message Container |
| N1N2MessageTransferReqData | 6.1.6.2.18 | N1/N2 message container |
| N1N2MessageTransferRspData | 6.1.6.2.19 | N1/N2 message transfer response |
| RegistrationContextContainer | 6.1.6.2.20 | Registration Context Container used to send the UE context information, N1 message from UE, AN address etc during Registration with AMF re-allocation procedure. |
| AreaOfValidity | 6.1.6.2.21 | Area of validity information for N2 information transfer |
| UeContextTransferReqData | 6.1.6.2.23 | Represents to start transferring of an individual ueContext resource from old AMF to new AMF. |
| UeContextTransferRspData | 6.1.6.2.24 | Indicates the transferring of the individual ueContext resource is started successfully. |
| UeContext | 6.1.6.2.25 | Represents an individual ueContext resource |
| N2SmInformation | 6.1.6.2.26 | Represents the session management SMF related N2 information data part. |
| N2InfoContent | 6.1.6.2.27 | Represents a transparent N2 information content to be relayed by AMF. |
| NrppaInformation | 6.1.6.2.28 | Represents a NRPPa related N2 information data part. |
| PwsInformation | 6.1.6.2.29 | Represents a PWS related information data part. |
| N1N2MsgTxfrFailureNotification | 6.1.6.2.30 | N1/N2 Message Transfer Failure Notification |
| N1N2MessageTransferError | 6.1.6.2.31 | N1/N2 Message Transfer Error |
| N1N2MsgTxfrErrDetail | 6.1.6.2.32 | N1/N2 Message Transfer Error Details |
| N2InformationTransferRspData | 6.1.6.2.33 | Indicates a successful delivery of N2 Information to the AN. |
| MmContext | 6.1.6.2.34 | Represents a Mobility Management Context in UE Context |
| SeafData | 6.1.6.2.35 | Represents SEAF data derived from data received from AUSF |
| NasSecurityMode | 6.1.6.2.36 | Indicates the NAS Security Mode |
| PduSessionContext | 6.1.6.2.37 | Represents a PDU Session Context in UE Context |
| NssaiMapping | 6.1.6.2.38 | Represents a map of a S-NSSAI in serving PLMN to a S-NSSAI in home PLMN. |
| UeRegStatusUpdateReqData | 6.1.6.2.39 | Provides information on the UE registration completion at a target AMF. |
| AssignEbiError | 6.1.6.2.40 | Represents the details regarding EBI assignment failure. |
| UeContextCreateData | 6.1.6.2.41 | Indicates a request to create an individual ueContext resource |
| UeContextCreatedData | 6.1.6.2.42 | Indicates a successful creation of an individual ueContext resource |
| UeContextCreateError | 6.1.6.2.43 | Represents an error when creating a UE context |
| NgRanTargetId | 6.1.6.2.44 | Indicates a NG RAN as target of the handover |
| N2InformationTransferError | 6.1.6.2.45 | Error within NonUeN2MessageTransfer response |
| PWSResponseData | 6.1.6.2.46 | Represents the type of PWS |
| PWSErrorData | 6.1.6.2.47 | Represents the type of PWS error |
| NgKsi | 6.1.6.2.49 | Represents the ngKSI (see 3GPP TS 33.501 [27]) |
| KeyAmf | 6.1.6.2.50 | Represents the Kamf or K'amf. (see 3GPP TS 33.501 [27]). |
| ExpectedUeBehavior | 6.1.6.2.51 | Represents the expected UE behavior (e.g. UE moving trajectory) and its validity period. |
| UeRegStatusUpdateRspData | 6.1.6.2.52 | Provides the status of UE context transfer status update at a source AMF. |
| N2RanInformation | 6.1.6.2.53 | Represents the RAN related N2 information data part. |
| N2InfoNotificationRspData | 6.1.6.2.54 | N2 information notification response data |
| SmallDataRateStatusInfo | 6.1.6.2.55 | Represents the small data rate status |
| SmfChangeInfo | 6.1.6.2.56 |  |
| V2xContext | 6.1.6.2.57 | Represents the V2X services related parameters |
| V2xInformation | 6.1.6.2.59 | V2X related N2 information |
| EpsBearerId | 6.1.6.3.2 | EPS Bearer Identifier |
| Ppi | 6.1.6.3.2 | Paging Policy Indicator |
| NasCount | 6.1.6.3.2 | Represents a NAS COUNT |
| 5GMmCapability | 6.1.6.3.2 | Represents a 5GMM capability |
| UeSecurityCapability | 6.1.6.3.2 | Represents a UE Security Capability |
| S1UeNetworkCapability | 6.1.6.3.2 | Represents a S1 UE Network Capability |
| DrxParameter | 6.1.6.3.2 | Indicates the UE DRX Parameters |
| OmcIdentifier | 6.1.6.3.2 | Represents the OMC Identifier |
| MSClassmark2 | 6.1.6.3.2 | Indicates the MS Classmark 2 of a 5G SRVCC UE |
| SupportedCodec | 6.1.6.3.2 | Indicates the supported codec of a 5G SRVCC UE |
| StatusChange | 6.1.6.3.3 |  |
| N2InformationClass | 6.1.6.3.4 |  |
| N1MessageClass | 6.1.6.3.5 |  |
| N1N2MessageTransferCause | 6.1.6.3.6 |  |
| UeContextTransferStatus | 6.1.6.3.7 | Describes the status of an individual ueContext resource in UE Context Transfer procedures |
| N2InformationTransferResult | 6.1.6.3.8 | Describes the result of N2 information transfer by AMF to the AN. |
| CipheringAlgorithm | 6.1.6.3.9 | Indicates the supported Ciphering Algorithm |
| IntegrityAlgorithm | 6.1.6.3.10 | Indicates the supported Integrity Algorithm |
| SmsSupport | 6.1.6.3.11 | Indicates the supported SMS delivery of a UE. |
| ScType | 6.1.6.3.12 | Indicates the security context type. |
| KeyAmfType | 6.1.6.3.13 | Indicates the Kamf type. |
| TransferReason | 6.1.6.3.14 | Indicates UE Context Transfer Reason |
| PolicyReqTrigger | 6.1.6.3.15 | Policy Request Triggers |
| RatSelector | 6.1.6.3.16 | Indicates the RAT type for the transfer of N2 information |
| NgapIeType | 6.1.6.3.17 | Indicates the supported NGAP IE types |
| N2InfoNotifyReason | 6.1.6.3.18 | N2 Information Notify Reason |
| SmfChangeIndication | 6.1.6.3.19 | Indicates the I-SMF or V-SMF change or removal |
| SbiBindingLevel | 6.1.6.3.20 | SBI Binding Level |

Table 6.1.6.1-2 specifies data types re-used by the Namf service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Namf service based interface.

Table 6.1.6.1-2: Namf re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| Snssai | 3GPP TS 29.571 [6] |  |
| Arp | 3GPP TS 29.571 [6] |  |
| PduSesisonId | 3GPP TS 29.571 [6] |  |
| Guami | 3GPP TS 29.571 [6] | Globally Unique AMF Identifier |
| AmfName | 3GPP TS 29.571 [6] | The name of the AMF |
| Supi | 3GPP TS 29.571 [6] | Subscription Permanent Identifier |
| IndicationFlags | 3GPP TS 29.502 [16] | Indication Flags |
| Cause | 3GPP TS 29.571 [6] | 5G-AN Cause |
| ProblemDetails | 3GPP TS 29.571 [6] | Detailed problems in failure case |
| supportedFeatures | 3GPP TS 29.571 [6] | Supported Features |
| TimeZone | 3GPP TS 29.571 [6] |  |
| UserLocation | 3GPP TS 29.571 [6] |  |
| AccessType | 3GPP TS 29.571 [6] |  |
| AllowedNssai | 3GPP TS 29.531 [18] |  |
| NfInstanceId | 3GPP TS 29.571 [6] |  |
| Uri | 3GPP TS 29.571 [6] |  |
| Ecgi | 3GPP TS 29.571 [6] | EUTRA Cell Identifier |
| Ncgi | 3GPP TS 29.571 [6] | NR Cell Identifier |
| Uint16 | 3GPP TS 29.571 [6] |  |
| 5Qi | 3GPP TS 29.571 [6] | 5G QoS Identifier |
| CorrelationID | 3GPP TS 29.572 [25] | LCS Correlation ID |
| Pei | 3GPP TS 29.571 [6] |  |
| Dnn | 3GPP TS 29.571 [6] |  |
| Gpsi | 3GPP TS 29.571 [6] |  |
| GroupId | 3GPP TS 29.571 [6] |  |
| PlmnId | 3GPP TS 29.571 [6] |  |
| RfspIndex | 3GPP TS 29.571 [6] |  |
| EbiArpMapping | 3GPP TS 29.502 [16] | EBI - ARP mapping |
| NsiId | 3GPP TS 29.531 [18] |  |
| TraceData | 3GPP TS 29.571 [6] | Trace control and configuration parameters |
| ConfiguredSnssai | 3GPP TS 29.531 [18] |  |
| NgApCause | 3GPP TS 29.571 [6] | Represents the NG AP cause IE |
| Area | 3GPP TS 29.571 [6] |  |
| ServiceAreaRestriction | 3GPP TS 29.571 [6] |  |
| CoreNetworkType | 3GPP TS 29.571 [6] |  |
| Ambr | 3GPP TS 29.571 [6] |  |
| GlobalRanNodeId | 3GPP TS 29.571 [6] |  |
| NfGroupId | 3GPP TS 29.571 [6] | Network Function Group Id |
| DurationSec | 3GPP TS 29.571 [6] |  |
| StnSr | 3GPP TS 29.571 [6] | Session Transfer Number for SRVCC |
| CMsisdn | 3GPP TS 29.571 [6] | Correlation MSISDN |
| DateTime | 3GPP TS 29.571 [6] |  |
| SmallDataRateStatus | 3GPP TS 29.571 [6] |  |
| NfSetId | 3GPP TS 29.571 [13] | NF Set ID |
| NfServiceSetId | 3GPP TS 29.571 [13] | NF Service Set ID |
| LMFIdentification | 3GPP TS 29.572 [25] | LMF Identification |
| PlmnAssiUeRadioCapId | 3GPP TS 29.571 [6] |  |
| ManAssiUeRadioCapId | 3GPP TS 29.571 [6] |  |
| NrV2xAuth | 3GPP TS 29.571 [6] | NR V2X services authorized |
| LteV2xAuth | 3GPP TS 29.571 [6] | LTE V2X services authorized |
| BitRate | 3GPP TS 29.571 [6] | Bit Rate |
| Pc5QoSPara | 3GPP TS 29.571 [6] | PC5 QoS parameters |
| CnAssistedRanPara | 3GPP TS 29.502 [16] | SMF derived CN assisted RAN Parameters Tuning |
| MoExpDataCounter | 3GPP TS 29.571 [6] | MO Exception Data Counter |

* * * Next Change * * * *

6.1.6.2.15	Type: N2InfoContainer

Table 6.1.6.2.15-1: Definition of type N2InfoContainer

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n2InformationClass | N2InformationClass | M | 1 | This IE represents the class of N2 information to be transferred. |
| smInfo | N2SmInformation | C | 0..1 | This IE shall be present if session management N2 information is to be transferred. When present, it represents a session management SMF related N2 information data part. |
| ranInfo | N2RanInformation | C | 0..1 | This IE shall be present if RAN related N2 information is to be transferred (i.e. n2InformationClass is "RAN"). When present, it shall contain the RAN related N2 information data part. |
| nrppaInfo | NrppaInformation | C | 0..1 | This IE shall be present if location service related N2 information is to be transferred. When present, it represents a NRPPa related N2 information data part. |
| pwsInfo | PwsInformation | C | 0..1 | This IE shall be present if PWS related N2 information is to be transferred. |
| v2xInfo | V2xInformation | C | 0..1 | This IE shall be present if V2X related N2 information is to be transferred. |

* * * Next Change * * * *

6.1.6.2.27	Type: N2InfoContent

Table 6.1.6.2.27-1: Definition of type N2InfoContent

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| ngapMessageType | Uinteger | C | 0..1 | This IE shall be present if PWS related N2 information is to be transferred, or during the AMF planned removal procedure with UDSF deployed procedure to transfer a RAN N2 message. When present, it shall indicate the NGAP Message type of the ngapData as specified in clause 6.1.6.4.3.3. Its value equals the value of the Procedure Code defined in ASN.1 in clause 9.4.7 in 3GPP TS 38.413 [12]. |
| ngapIeType | NgapIeType | C | 0..1 | This IE shall be present if SM, RAN, V2X or NRPPa related N2 information is to be transferred. When present, it shall indicate the NGAP IE type of the ngapData as specified in clause 6.1.6.4.3.2. |
| ngapData | RefToBinaryData | M | 1 | This IE reference the N2 Information binary data corresponding to the N2 information class. See clause 6.1.6.4.3. |

* * * Next Change * * * *

6.1.6.2.x	Type: V2xInformation

Table 6.1.6.2.x-1: Definition of type V2xInformation

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n2Pc5Pol | N2InfoContent | C | 0..1 | This IE shall be present if N2 PC5 policy should be transferred. When present, the IE contains the NGAP V2X related IEs specified in clause 9.2.1.z of 3GPP TS 38.413 [12]. |

* * * Next Change * * * *

6.1.6.3.4	Enumeration: N2InformationClass

Table 6.1.6.3.4-1: Enumeration N2InformationClass

| Enumeration value | Description |
| --- | --- |
| "SM" | N2 SM information. |
| "NRPPa" | N2 NRPPa information. |
| "PWS" | N2 PWS information of PWS type. |
| "PWS-BCAL" | N2 Broadcast Completed Area List or the Broadcast Cancelled Area List. |
| "PWS-RF" | N2 Restart Indication or Failure Indication |
| "RAN" | N2 RAN related information. |
| "V2X" | N2 V2X information |

* * * Next Change * * * *

6.1.6.3.17	Enumeration: NgapIeType

Table 6.1.6.3.x-1: Enumeration NgapIeType

| Enumeration value | Description |
| --- | --- |
| "PDU_RES_SETUP_REQ" | PDU Session Resource Setup Request Transfer |
| "PDU_RES_REL_CMD" | PDU Session Resource Release Command Transfer |
| "PDU_RES_MOD_REQ" | PDU Session Resource Modify Request Transfer |
| "HANDOVER_CMD" | Handover Command Transfer |
| "HANDOVER_REQUIRED" | Handover Required Transfer |
| "HANDOVER_PREP_FAIL" | Handover Preparation Unsuccessful Transfer |
| "SRC_TO_TAR_CONTAINER" | Source to Target Transparent Container |
| "TAR_TO_SRC_CONTAINER" | Target to Source Transparent Container |
| "RAN_STATUS_TRANS_CONTAINER" | RAN Status Transfer Transparent Container |
| "SON_CONFIG_TRANSFER" | SON Configuration Transfer |
| "NRPPA_PDU" | NRPPa-PDU |
| "UE_RADIO_CAPABILITY" | UE Radio Capability |
| "RIM_INFO_TRANSFER" | RIM Information Transfer |
| "SECONDARY_RAT_USAGE" | Secondary RAT Data Usage Report Transfer |
| "PC5_QOS_PARA" | PC5 QoS Parameters |

* * * Next Change * * * *

6.1.6.4.3.2	NGAP IEs

For N2 information class SM, N2 Information may encode following NGAP SMF related IE specified in 3GPP TS 38.413 [12], as summarized in Table 6.1.6.4.3-1.

Table 6.1.6.4.3-1: N2 Information content for class SM

| NGAP IE | Reference (3GPP TS 38.413 [12]) | Related NGAP message |
| --- | --- | --- |
| PDU Session Resource Setup Request Transfer | 9.3.4.1 | PDU SESSION RESOURCE SETUP REQUEST |
| PDU Session Resource Release Command Transfer | 9.3.4.3 | PDU SESSION RESOURCE RELEASE COMMAND |
| PDU Session Resource Modify Request Transfer | 9.3.4.5 | PDU SESSION RESOURCE MODIFY REQUEST |
| Handover Command Transfer | 9.3.4.12 | HANDOVER COMMAND |
| Handover Required Transfer | 9.3.4.14 | HANDOVER REQUIRED |
| Handover Preparation Unsuccessful Transfer | 9.3.4.18 | HANDOVER COMMAND |
| Secondary RAT Data Usage Report Transfer | 9.3.4.23 | SECONDARY RAT DATA USAGE REPORT |

For N2 information class RAN, N2 Information may encode one of the following NGAP Transparent Container IEs specified in 3GPP TS 38.413 [12], as summarized in Table 6.1.6.4.3-2.

Table 6.1.6.4.3-2: N2 Information content for class RAN

| NGAP IE | Reference (3GPP TS 38.413 [12]) | Related NGAP message |
| --- | --- | --- |
| Source to Target Transparent Container | 9.3.1.20 | HANDOVER REQUIRED, HANDOVER REQUEST |
| Target to Source Transparent Container | 9.3.1.21 | HANDOVER COMMAND, HANDOVER REQUEST ACKNOWLEDGE |
| UE Radio Capability | 9.3.1.74 | UE CAPABILITY INFO INDICATION. (NOTE 1). |
| SON Configuration Transfer | 9.3.3.6 | UPLINK RAN CONFIGURATION TRANSFER, DOWNLINK RAN CONFIGURATION TRANSFER |
| RAN Status Transfer Transparent Container | 9.2.3.13, 9.2.3.14 | UPLINK RAN STATUS TRANSFER, DOWNLINK RAN STATUS TRANSFER |
| RIM Information Transfer | 9.3.3.28 | UPLINK RIM INFORMATION TRANSFER, DOWNLINK RIM INFORMATION TRANSFER |
| NOTE 1: The AMF receives the UE Radio Capability within a UE CAPABILITY INFO INDICATION message and then the AMF shall store the UE Radio Capability information and transfer it to the target AMF during an inter AMF mobility procedure. |  |  |

For N2 information class NRPPa, N2 Information may encode the following NGAP NRPPA Transport related IE specified in 3GPP TS 38.413 [12], as summarized in Table 6.1.6.4.3-3

Table 6.1.6.4.3-3: N2 Information content for class NRPPa

| NGAP IE | Reference (3GPP TS 38.413 [12]) | Related NGAP message |
| --- | --- | --- |
| NRPPa-PDU | 9.3.3.14 | DOWNLINK UE ASSOCIATED NRPPA TRANSPORT UPLINK UE ASSOCIATED NRPPA TRANSPORT DOWNLINK NON UE ASSOCIATED NRPPA TRANSPORT UPLINK NON UE ASSOCIATED NRPPA TRANSPORT |

For N2 information class V2X, N2 Information may encode the following V2X related IE specified in 3GPP TS 38.413 [12], as summarized in Table 6.1.6.4.3-4

Table 6.1.6.4.3-4: N2 Information content for class V2X

| NGAP IE | Reference (3GPP TS 38.413 [12]) | Related NGAP message |
| --- | --- | --- |
| PC5 QoS Parameters | 9.2.1.z | INITIAL CONTEXT SETUP REQUEST UE CONTEXT MODIFICATION REQUEST HANDOVER REQUEST PATH SWITCH REQUEST ACKNOWLEDGE |

* * * Next Change * * * *

A.2	Namf_Communication API

openapi: 3.0.0

info:

version: 1.1.0.alpha-4

title: Namf_Communication

description: |

AMF Communication Service

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

security:

- {}

- oAuth2ClientCredentials:

- namf-comm

[…]

N2InfoContainer:

type: object

properties:

n2InformationClass:

$ref: '#/components/schemas/N2InformationClass'

smInfo:

$ref: '#/components/schemas/N2SmInformation'

ranInfo:

$ref: '#/components/schemas/N2RanInformation'

nrppaInfo:

$ref: '#/components/schemas/NrppaInformation'

pwsInfo:

$ref: '#/components/schemas/PwsInformation'

v2xInfo:

$ref: '#/components/schemas/V2xInformation'

required:

- n2InformationClass

[…]

V2xContext:

type: object

properties:

nrV2xServicesAuth:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NrV2xAuth'

lteV2xServicesAuth:

$ref: 'TS29571_CommonData.yaml#/components/schemas/LteV2xAuth'

nrUeSidelinkAmbr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

lteUeSidelinkAmbr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

Pc5QoSPara:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pc5QoSPara'

V2xInformation:

type: object

properties:

n2Pc5Pol:

$ref: '#/components/schemas/N2InfoContent'

[…]

#

# ENUMERATIONS

#

StatusChange:

anyOf:

- type: string

enum:

- AMF_UNAVAILABLE

- AMF_AVAILABLE

- type: string

N2InformationClass:

anyOf:

- type: string

enum:

- SM

- NRPPa

- PWS

- PWS-BCAL

- PWS-RF

- RAN

- V2X

- type: string

[…]

RatSelector:

anyOf:

- type: string

enum:

- E-UTRA

- NR

- type: string

NgapIeType:

anyOf:

- type: string

enum:

- PDU_RES_SETUP_REQ

- PDU_RES_REL_CMD

- PDU_RES_MOD_REQ

- HANDOVER_CMD

- HANDOVER_REQUIRED

- HANDOVER_PREP_FAIL

- SRC_TO_TAR_CONTAINER

- TAR_TO_SRC_CONTAINER

- RAN_STATUS_TRANS_CONTAINER

- SON_CONFIG_TRANSFER

- NRPPA_PDU

- UE_RADIO_CAPABILITY

- RIM_INFO_TRANSFER

- SECONDARY_RAT_USAGE

- PC5_QOS_PARA

- type: string

[…]

* * * End of Changes * * * *