3GPP TSG-CT Meeting #88-e	CP-201289

# Online, , 28th Jun 2020 - 1st Jul 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.102 | CR | 0897 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | X | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Dedicated AID for USIM Applications with non-IMSI based SUPI Types |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | G+D MS, STMicroelectronics, Thales DIS |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2020-06-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The setting of Service #130 in EF_UST defines the SUPI Type used for a given USIM application (C6-200191 / CR#0884), meaning that: - if Service #130 is “available”, the SUPI Type shall be non-IMSI (i.e. NSI or GCI or GLI). - If Service #130 is “not available” the SUPI Type shall be IMSI. The setting configured in Service #130 is used by the ME to identify the SUPI Type (IMSI or non-IMSI) during USIM Initialization procedure. The fact that the Service #130 check is not done in an earlier stage than USIM Initialization procedure (i.e. Application selection) creates the following issues: Issue #1: pre-Rel16 MEs may select a USIM Application which is configured with a SUPI of type non-IMSI. And this shall not occur in any case. Issue #2: As a consequence of Issue#1, there is a backwards compatibility issue as the USIM initialization process will fail in pre-Rel16 MEs with a USIM Application configured with SUPI of type non-IMSI, in the same way as if the USIM Application is badly configured which is not correct and creates a misleading and bad user experience. Issue #3: Rel16 MEs cannot propose a good user experience by using a differentiation mechanism that identifies if the USIM Application is configured with a SUPI of type IMSI or type non-IMSI. Issue #4: Rel16 MEs cannot propose a good user experience when the UICC contains multiple USIMs and there is at least one USIM configured with a SUPI of type IMSI and at least one USIM configured with a SUPI of type non-IMSI. The CR proposed aims to fix the issues listed above. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new AID range for a USIM Application that is configured with a SUPI of type non-IMSI. This change makes not possible that Issue#1 occurs, and as a result Issue #2 shall not occur. Fixing Issue #3 and #4 improves the user experience. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Issue #1 will remain, which is not correct. Issue #2 will remain and as a consequence, Pre-Rel16 MEs will fail in a bad manner when USIM Application is configured with a SUPI of type non-IMSI. Issue #3 and #4 will remain deterioriating the user experience: - No way for the ME to know (or to inform the user) prior selection if the USIM Application is configured with a SUPI of type IMSI or type non-IMSI. - Difficult selection mechanism for a UICC which contains more than one USIM Application and there is at least one USIM configured with a SUPI of type IMSI and at least one USIM configured with a SUPI of type non-IMSI. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 31.101 CR #0095 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The ‘X’ in “Annex X of 3GPP TS 31.101 [11].” shall be done based on the implementation of TS 31.101 CR#0095. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** First change *****

Annex X (normative):
USIM supporting non-IMSI SUPI Type

## X.1	Introduction

IMSI based USIM is a USIM Application which is configured with a SUPI of type IMSI (i.e Service n°130 in the USIM Service Table shall not be "available").

Non-IMSI based USIM is a USIM Application which is configured with a SUPI of type non-IMSI (i.e Service n°130 in the USIM Service Table shall be "available"). Examples of non-IMSI type are: NSI, GLC and GLI.

Both USIM application types shall use different AID ranges as defined in Annex X of 3GPP TS 31.101 [11].

## X.2	Features of USIM supporting non-IMSI SUPI type

The non-IMSI based USIM may be implemented as a single USIM application on a UICC or as a secondary USIM application together with an IMSI based USIM on one UICC.

The non-IMSI based USIM is a regular USIM application and shall contain all mandatory EFs defined for a USIM application in the present document and may also include any of the optional EFs defined for a USIM application.

No additional features are supported by Non-IMSI based USIM.

## X.2	Application selection procedure

Application selection is performed according to the procedures defined in clause 5.1.1.1. The following provisions apply:

The method for selecting a non-IMSI based USIM is based on the presence of the corresponding application with the associated AID in EFDIR, as defined in the Annex X of 3GPP TS 31.101 [11].

***** End of changes *****