3GPP TSG-CT Meeting #88e	CP-201325

# E-Meeting, 29th June – 1st July 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.124 | CR | 0542 | rev | 3 | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | X | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Test Case Routing Indicator Data update via NAS message passed to USIM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Thales DIS |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Thales DIS |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1_UEConTest |  |  |  |  |  | Date: |  |  | 2020-05-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introducing a new Test case for Routing Indicator Data update NAS message on NG-RAN |  |  |  |  |  |  |  |  |
| Summary of change: |  | 5G NR new test case 27.22.xx for Routing Indicator Data update NAS message passed to USIM |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will not be Routing Indicator Data update NAS message test cases in 3GPP 31.124 for verifying NG-RAN Routing Indicator Data update NAS message properly passed to USIM using ENVELOPE SMS-PP Data Download |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.4, 27.22.XX, 27.22.XX.1, 27.22.XX.1.1, 27.22.XX.1.2, 27.22.XX.1.3, 27.22.XX.1.4, 27.22.XX.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| This CR's revision history: |  | was C6-200383 was C6-200343 was C6-200285 |  |  |  |  |  |  |  |  |

***** Next change *****

## 3.4	Applicability table

Table B.1: Applicability of tests

| Item | Description |  |  | Re-lease | Test sequence(s) |  | Rel 99 ME | Rel-4 ME | Rel-5 ME | Rel-6 ME | Rel-7 ME | Rel-8 ME | Rel-9 ME | Rel-10 ME | Rel-11 ME | Rel-12 ME | Rel-13 ME | Rel-14 ME | Rel-15 ME | Terminal Profile | Network Dependency | Sup-port | Additional test case execution parameter |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | PROFILE DOWNLOAD 27.22.1 |  |  | R99 | 1 |  | M | M | M | M | M | M | M | M | M | M | M | M | M | E.1/1 | No |  |  |
| 2 | Contents of the TERMINAL PROFILE command 27.22.2 |  |  | R99 |  |  | M | M | M | M | M | M | M | M | M | M | M | M | M | E.1/1 | No |  |  |
| 3 | Servicing of Proactive UICC Commands 27.22.3 |  |  | R99 |  |  | M | M | M | M | M | M | M | M | M | M | M | M | M |  | No |  |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 53 | Change eCall mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | REFRESH after change eCall mode, disable FDN in EFEST, E-UTRAN |  |  | Rel-8 | 1.1 |  |  |  |  |  |  |  |  |  |  |  |  | C190 | C190 | E.1/24 AND E.1/2 | E-UTRAN System Simulator only |  |  |
|  | REFRESH after change eCall mode, enable FDN in EFEST, E-UTRAN |  |  | Rel-8 | 1.2 |  |  |  |  |  |  |  |  |  |  |  |  | C190 | C190 | E.1/24 AND E.1/2 | E-UTRAN System Simulator only |  |  |
|  | REFRESH after changing eCall mode, disable FDN in EFEST, IMS Emergency Services in E-UTRAN |  |  | Rel-14 | 1.3 |  |  |  |  |  |  |  |  |  |  |  |  | C202 | C202 | E.1/24 AND E.1/2 | E-UTRAN System Simulator only |  |  |
|  | REFRESH after changing eCall mode, disable FDN in EFEST, UTRAN/GERAN |  |  | Rel-8 | 1.4 |  |  |  |  |  |  |  |  |  |  |  |  | C167 OR C184 | C167 OR C184 | E.1/24 AND E.1/2 | UMTS System Simulator or System Simulator only |  |  |
| AA | SMS-PP Data Download on NAS messages |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | SMS-PP Data Dowload after UE parameters update data (Routing Indicator Data) via DL NAS TRANSPORT message "acknowledgement not requested" and "re-registration not requested" |  |  | Rel-15 | 1.1 |  |  |  |  |  |  |  |  |  |  |  |  |  | C231 | E.1/24 AND E.1/2 | NG-SS only |  |  |
|  | SMS-PP Data Dowload after UE parameters update data (Routing Indicator Data) via DL NAS TRANSPORT message "acknowledgement not requested" and "re-registration requested" |  |  | Rel-15 | 1.2 |  |  |  |  |  |  |  |  |  |  |  |  |  | C231 | E.1/24 AND E.1/2 | NG-SS only |  |  |
|  | SMS-PP Data Dowload after UE parameters update data (Routing Indicator Data) via DL NAS TRANSPORT message "acknowledgement requested" and "re-registration requested" |  |  | Rel-15 | 1.3 |  |  |  |  |  |  |  |  |  |  |  |  |  | C231 | E.1/24 AND E.1/2 | NG-SS only |  |  |
|  | SMS-PP Data Dowload after UE parameters update data (Routing Indicator Data) via DL NAS TRANSPORT message "acknowledgement requested" and "re-registration not requested" |  |  | Rel-15 | 1.4 |  |  |  |  |  |  |  |  |  |  |  |  |  | C231 | E.1/24 AND E.1/2 | NG-SS only |  |  |
|  |  |  |  |  |  | NOTE: For Rel -13 if the UE supports NB-IoT, this test case shall be verified by accessing the NB System Simulator (NB-SS). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| C101 | IF A.1/1 THEN M ELSE N/A | -- O_Cap_Conf |
| --- | --- | --- |
| … |  |  |
| C230 | A.1/17 AND A.1/178 AND ((A.1/132 OR A.1/133) OR A.1/177) THEN M ELSE N/A | --O_UDP AND pc_Multiple_PDN AND ((pc_BIP_eFDD OR pc_BIP_eTDD) OR pc_BIP_NB) |
| C231 | IF A.1/187THEN M ELSE N/A | -- pc_NG_RAN |
| O.1 | IF A.1/zz tests x.yA M ELSE tests x.yB M (where zz corresponds to the option relating to the command being tested (e.g. A.1/90 if Display Text supports icons as defined in record 1 of EF(IMG)) and x.y is the expected sequence number value) |  |

***** Next change *****

##### 27.22.XX	ENVELOPE SMS-PP Data Dowload on NAS messages

#### 27.22.XX.1	Routing Indicator Data update via DL NAS TRANSPORT messages

##### 27.22.XX.1.1	Definition and applicability

See clause 3.2.2.

##### 27.22.XX.1.2	Conformance requirement

The ME shall support the Proactive UICC: SMS-PP Data Download facility as defined in the following technical specifications:

-	TS 31.111 [15] clause 5, clause 7.1, clause 8.1, clause 8.7, clause 8.13 and clause 11.

-	TS 31.115 [28] clause 4.

-	TS 23.038 [7] clause 4..

The ME shall support the Procedure for SMS-PP data download via DL NAS TRANSPORT messages as defined in the following technical specifications:

-	TS 31.111 [15] clause 7.1.1.1a.

##### 27.22.XX.1.3	Test purpose

To verify that when:

- the service "data download via SMS Point-to-point" is available in the USIM Service Table

and

- the ME receives a DL NAS TRANSPORT message that includes

  - a UE parameters update transparent container containing a UE parameters update data set with UE parameters update data set type with value "1"=Routing Indicator update data

  - containing a secure packet constructed as an SMS-Deliver (as specified in 3GPP TS 23.040  [5] with:

    - protocol identifier = SIM data download; and

    - data coding scheme = class 2 message

  - and the integrity check of the message was successful

then

- the ME shall

  - pass the message transparently to the UICC using the ENVELOPE (SMS-PP DOWNLOAD) command as defined in TS 31.111 [15] clause 7.1.1.2

  - the ME shall not display or alert the user

  - the secure packet is coded as a Command Packet formatted as Short Message Point to Point (as specified in 3GPP TS 31.115  [41]))

To verify that the ME interprets the UICC returns response correctly.

##### 27.22.XX.1.4	Method of Test

27.22.XX.1.4.1	Initial conditions

The ME is connected to the USIM Simulator and the NG-SS.

The ME is connected to NG-SS and it has performed the Registration procedure.

The default NG-RAN UICC and the following parameters.

The NG-RAN UICC parameters are:

- one OTA Key Set with:

  - Key Version:	01

    - 1st key

      - Key Index (Kic): 	01

      - Key value: 	000102030405060708090A0B0C0D0E0F

    - 2nd key

      - Key Index (Kid): 	02

      - Key value: 	000102030405060708090A0B0C0D0E0F

    - 3rd key

      - Key Index (Kik): 	03

      - Key value: 	000102030405060708090A0B0C0D0E0F

The default NG-RAN parameters and the following parameters are used:

The NG-RAN parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 01;

-	Tracking Area Code (TAC) = 000001;

-	NG-RAN Cell Id = 0001 (36 bits).

The elementary files are coded as USIM Application Toolkit default with the following exceptions:

- The "data download via SMS Point-to-point" service is available in the USIM Service Table.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.XX.1.4.2	Procedure

Expected Sequence 1.1 (SMS-PP Data Dowload after UE parameters update data (Routing Indicator Data) via DL NAS TRANSPORT message "acknowledgement not requested" and "re-registration not requested")

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | USER ME | The ME is switched on | ME will perform Profile Download and USIM initialisation |
| 2 | ME NG-SS | ME successfully REGISTER with NG-RAN cell. |  |
| 3 | NG-SS ME | NG-SS send to ME DL NAS TRANSPORT message 1.1.1 with acknowledgement not requested re-registration not requested | UE parameters update header with: - ACK set to "acknowledgement not requested" - REG set to "re-registration not requested" |
| 4 | ME UICC | ENVELOPE: SMS-PP DOWNLOAD 1.1.1 | the ME shall pass the message transparently to the UICC using the ENVELOPE (SMS-PP DOWNLOAD) command as specified in TS 31.111 [15] clause 7.1.1.1a |
| 5 | UICC ME | SW1/SW2 91 XX |  |
| 6 | UICC | EFRouting Indicator contents updated correctly | New EFRouting Indicator value "0055FFFF" |
| 7 | ME UICC | FETCH |  |
| 8 | UICC ME | PROACTIVE COMMAND: REFRESH 1.1.1 [File Change Notification] |  |
| 9 | ME UICC | TERMINAL RESPONSE RESPONSE: REFRESH 1.1.1A or TERMINAL RESPONSE: REFRESH 1.1.1B |  |
| 10 | ME UICC | PROACTIVE UICC SESSION ENDED |  |
| 11 | ME | Enter in 5GMM-IDLE |  |
| 12 | NG-SS ME | NG-SS shall send IDENTITY REQUEST for SUCI and verify if UE sends SUCI with newly updated Routing Indicator. |  |

DL NAS TRANSPORT message 1.1.1

Logically:

Message details (refereing to TS 24.501 Table 9.11.3.53A.1)

Payload container type IE:	"0110" (UE parameters update transparent container)

UE parameters update header:

UPU data type 	"0" (UE parameters update transparent container carries a UE parameters update list)

ACK:	"0" (acknowledgment not requested)

REG	"0" (re-registration not requested)

UE parameters update list:	includes one UE parameters update data set with UE parameters update data set type "0001" (Routing indicator update data)

Corresponding UE parameters update data set:

Secured packet	as specified in TS 31.111 [15] clause 7.1.1.1a – TPDU Command Packet

Coding:

| 40 | 00 | 91 | 7F | F6 | 00 | 00 | 00 | 00 | 00 | 00 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50 | 02 | 70 | 00 | 00 | 49 | 15 | 02 | 00 | 10 | 10 | B0 |
| 01 | 40 | 00 | 00 | 00 | 00 | 00 | 00 | 0F | 13 | 8E | 84 |
| E8 | D6 | F8 | 01 | AA | 31 | 22 | 07 | 00 | A4 | 00 | 04 |
| 02 | 5F | C0 | 22 | 07 | 00 | A4 | 00 | 04 | 02 | 4F | 0A |
| 22 | 07 | 00 | D6 | 00 | 00 | 02 | 00 | 55 | 81 | 14 | 81 |
| 03 | 01 | 01 | 01 | 82 | 02 | 81 | 82 | 12 | 09 | 01 | 3F |
| 00 | 7F | FF | 5F | C0 | 4F | 0A |  |  |  |  |  |

ENVELOPE: SMS-PP DOWNLOAD 1.1.1

Logically:

SMS-PP Download

Device identities

Source device:	Network

Destination device:	UICC

Address

TON	International number

NPI	"ISDN / telephone numbering plan"

Dialling number string	"1234"

SMS TPDU						Contents of Secured Patcket from DL NAS TRANSPORT message 1.1.1

Coding:

| BER-TLV: | D1 | 66 | 82 | 02 | 83 | 81 | 06 | 03 | 91 | 12 | 34 | 8B |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5B | 40 | 00 | 91 | 7F | F6 | 00 | 00 | 00 | 00 | 00 | 00 |
|  | 00 | 50 | 02 | 70 | 00 | 00 | 49 | 15 | 02 | 00 | 10 | 10 |
|  | B0 | 01 | 40 | 00 | 00 | 00 | 00 | 00 | 00 | 0F | 13 | 8E |
|  | 84 | E8 | D6 | F8 | 01 | AA | 31 | 22 | 07 | 00 | A4 | 00 |
|  | 04 | 02 | 5F | C0 | 22 | 07 | 00 | A4 | 00 | 04 | 02 | 4F |
|  | 0A | 22 | 07 | 00 | D6 | 00 | 00 | 02 | 00 | 55 | 81 | 14 |
|  | 81 | 03 | 01 | 01 | 01 | 82 | 02 | 81 | 82 | 12 | 09 | 01 |
|  | 3F | 00 | 7F | FF | 5F | C0 | 4F | 0A |  |  |  |  |

PROACTIVE COMMAND: REFRESH 1.1.1

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	File Change Notification

Device identities

Source device:	UICC

Destination device:	ME

File List:	EFRouting Indicator

Coding:

| BER-TLV: | D0 | 14 | 81 | 03 | 01 | 01 | 01 | 82 | 02 | 81 | 82 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 09 | 01 | 3F | 00 | 7F | FF | 5F | C0 | 4F | 0A |  |  |

TERMINAL RESPONSE: REFRESH 1.1.1A

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	File Change Notification

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 01 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

TERMINAL RESPONSE: REFRESH 1.1.1B

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	File Change Notification

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	REFRESH performed with additional EFs read

Coding:

| BER-TLV: | 81 | 03 | 01 | 01 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 03 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 1.2 (SMS-PP Data Dowload after UE parameters update data (Routing Indicator Data) via DL NAS TRANSPORT message "acknowledgement not requested" and "re-registration requested")

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | USER ME | The ME is switched on | ME will perform Profile Download and USIM initialisation |
| 2 | ME NG-SS | ME successfully REGISTER with NG-RAN cell. |  |
| 3 | NG-SS ME | NG-SS send to ME DL NAS TRANSPORT message 1.2.1 with acknowledgement not requested re-registration requested | UE parameters update header with: - ACK set to "acknowledgement not requested" - REG set to "re-registration requested" |
| 4 | ME UICC | ENVELOPE: SMS-PP DOWNLOAD 1.1.1 | the ME shall pass the message transparently to the UICC using the ENVELOPE (SMS-PP DOWNLOAD) command as specified in TS 31.111 [15] clause 7.1.1.1a |
| 5 | UICC ME | SW1/SW2 91 XX |  |
| 6 | UICC | EFRouting Indicator contents updated correctly | New EFRouting Indicator value "0055FFFF" |
| 7 | ME UICC | FETCH |  |
| 8 | UICC ME | PROACTIVE COMMAND: REFRESH 1.1.1 [File Change Notification] |  |
| 9 | ME UICC | TERMINAL RESPONSE RESPONSE: REFRESH 1.1.1A or TERMINAL RESPONSE: REFRESH 1.1.1B |  |
| 10 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 11 | ME | Enter in 5GMM-IDLE |  |
| 12 | ME | ME performs a de-registration procedure, delete its 5G-GUTI and initiate a registration procedure for initial registration Verify if UE sends SUCI with newly updated Routing Indicator in the REGISTRATION REQUEST. | As specified in TS 24.501 [70] clause 5.4.5.3.3 sub-clause i.1.i.C1 |

DL NAS TRANSPORT message 1.2.1

Logically:

Message details (refereing to TS 24.501 Table 9.11.3.53A.1)

Payload container type IE:	"0110" (UE parameters update transparent container)

UE parameters update header:

UPU data type 	"0" (UE parameters update transparent container carries a UE parameters update list)

ACK:	"0" (acknowledgment not requested)

REG	"1" (re-registration requested)

UE parameters update list:	includes one UE parameters update data set with UE parameters update data set type "0001" (Routing indicator update data)

Corresponding UE parameters update data set:

Secured packet	as specified in TS 31.111 [15] clause 7.1.1.1a – TPDU Command Packet

Coding:

| 40 | 00 | 91 | 7F | F6 | 00 | 00 | 00 | 00 | 00 | 00 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50 | 02 | 70 | 00 | 00 | 49 | 15 | 02 | 00 | 10 | 10 | B0 |
| 01 | 40 | 00 | 00 | 00 | 00 | 00 | 00 | 0F | 13 | 8E | 84 |
| E8 | D6 | F8 | 01 | AA | 31 | 22 | 07 | 00 | A4 | 00 | 04 |
| 02 | 5F | C0 | 22 | 07 | 00 | A4 | 00 | 04 | 02 | 4F | 0A |
| 22 | 07 | 00 | D6 | 00 | 00 | 02 | 00 | 55 | 81 | 14 | 81 |
| 03 | 01 | 01 | 01 | 82 | 02 | 81 | 82 | 12 | 09 | 01 | 3F |
| 00 | 7F | FF | 5F | C0 | 4F | 0A |  |  |  |  |  |

Expected Sequence 1.3 (SMS-PP Data Dowload after UE parameters update data (Routing Indicator Data) via DL NAS TRANSPORT message "acknowledgement requested" and "re-registration requested")

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | USER ME | The ME is switched on | ME will perform Profile Download and USIM initialisation |
| 2 | ME NG-SS | ME successfully REGISTER with NG-RAN cell. |  |
| 3 | NG-SS ME | NG-SS send to ME DL NAS TRANSPORT message 1.3.1 with acknowledgement requested re-registration requested | UE parameters update header with: - ACK set to "acknowledgement requested" - REG set to "re-registration requested" |
| 4 | ME UICC | ENVELOPE: SMS-PP DOWNLOAD 1.1.1 | the ME shall pass the message transparently to the UICC using the ENVELOPE (SMS-PP DOWNLOAD) command as specified in TS 31.111 [15] clause 7.1.1.1a |
| 5 | UICC ME | SW1/SW2 91 XX |  |
| 6 | UICC | EFRouting Indicator contents updated correctly | New EFRouting Indicator value "0055FFFF" |
| 7 | ME UICC | FETCH |  |
| 8 | UICC ME | PROACTIVE COMMAND: REFRESH 1.1.1 [File Change Notification] |  |
| 9 | ME UICC | TERMINAL RESPONSE RESPONSE: REFRESH 1.1.1A or TERMINAL RESPONSE: REFRESH 1.1.1B | ME shall have sent by now an acknowledgement in the Payload container IE of an UL NAS TRANSPORT message as specified in TS 24.501 [70] clause 5.4.5.3.3 sub-clause i.1.i.B |
| 10 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 11 | ME | Enter in 5GMM-IDLE |  |
| 12 | ME | ME performs a de-registration procedure, delete its 5G-GUTI and initiate a registration procedure for initial registration Verify if UE sends SUCI with newly updated Routing Indicator in the REGISTRATION REQUEST. | As specified in TS 24.501 [70] clause 5.4.5.3.3 sub-clause i.1.i.C1 |

DL NAS TRANSPORT message 1.3.1

Logically:

Message details (refereing to TS 24.501 Table 9.11.3.53A.1)

Payload container type IE:	"0110" (UE parameters update transparent container)

UE parameters update header:

UPU data type 	"0" (UE parameters update transparent container carries a UE parameters update list)

ACK:	"1" (acknowledgment requested)

REG	"1" (re-registration requested)

UE parameters update list:	includes one UE parameters update data set with UE parameters update data set type "0001" (Routing indicator update data)

Corresponding UE parameters update data set:

Secured packet	as specified in TS 31.111 [15] clause 7.1.1.1a – TPDU Command Packet

Coding:

| 40 | 00 | 91 | 7F | F6 | 00 | 00 | 00 | 00 | 00 | 00 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50 | 02 | 70 | 00 | 00 | 49 | 15 | 02 | 00 | 10 | 10 | B0 |
| 01 | 40 | 00 | 00 | 00 | 00 | 00 | 00 | 0F | 13 | 8E | 84 |
| E8 | D6 | F8 | 01 | AA | 31 | 22 | 07 | 00 | A4 | 00 | 04 |
| 02 | 5F | C0 | 22 | 07 | 00 | A4 | 00 | 04 | 02 | 4F | 0A |
| 22 | 07 | 00 | D6 | 00 | 00 | 02 | 00 | 55 | 81 | 14 | 81 |
| 03 | 01 | 01 | 01 | 82 | 02 | 81 | 82 | 12 | 09 | 01 | 3F |
| 00 | 7F | FF | 5F | C0 | 4F | 0A |  |  |  |  |  |

Expected Sequence 1.4 (SMS-PP Data Dowload after UE parameters update data (Routing Indicator Data) via DL NAS TRANSPORT message "acknowledgement requested" and "re-registration not requested")

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | USER ME | The ME is switched on | ME will perform Profile Download and USIM initialisation |
| 2 | ME NG-SS | ME successfully REGISTER with NG-RAN cell. |  |
| 3 | NG-SS ME | NG-SS send to ME DL NAS TRANSPORT message 1.4.1 with acknowledgement requested re-registration not requested | UE parameters update header with: - ACK set to "acknowledgement requested" - REG set to "re-registration not requested" |
| 4 | ME UICC | ENVELOPE: SMS-PP DOWNLOAD 1.1.1 | the ME shall pass the message transparently to the UICC using the ENVELOPE (SMS-PP DOWNLOAD) command as specified in TS 31.111 [15] clause 7.1.1.1a |
| 5 | UICC ME | SW1/SW2 91 XX |  |
| 6 | UICC | EFRouting Indicator contents updated correctly | New EFRouting Indicator value "0055FFFF" |
| 7 | ME UICC | FETCH |  |
| 8 | UICC ME | PROACTIVE COMMAND: REFRESH 1.1.1 [File Change Notification] |  |
| 9 | ME UICC | TERMINAL RESPONSE RESPONSE: REFRESH 1.1.1A or TERMINAL RESPONSE: REFRESH 1.1.1B | ME shall have sent by now an acknowledgement in the Payload container IE of an UL NAS TRANSPORT message as specified in TS 24.501 [70] clause 5.4.5.3.3 sub-clause i.1.i.B |
| 10 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 11 | ME | Enter in 5GMM-IDLE |  |
| 12 | NG-SS ME | NG-SS shall send IDENTITY REQUEST for SUCI and verify if UE sends SUCI with newly updated Routing Indicator. |  |

DL NAS TRANSPORT message 1.4.1

Logically:

Message details (refereing to TS 24.501 Table 9.11.3.53A.1)

Payload container type IE:	"0110" (UE parameters update transparent container)

UE parameters update header:

UPU data type 	"0" (UE parameters update transparent container carries a UE parameters update list)

ACK:	"1" (acknowledgment requested)

REG	"0" (re-registration not requested)

UE parameters update list:	includes one UE parameters update data set with UE parameters update data set type "0001" (Routing indicator update data)

Corresponding UE parameters update data set:

Secured packet	as specified in TS 31.111 [15] clause 7.1.1.1a – TPDU Command Packet

Coding:

| 40 | 00 | 91 | 7F | F6 | 00 | 00 | 00 | 00 | 00 | 00 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50 | 02 | 70 | 00 | 00 | 49 | 15 | 02 | 00 | 10 | 10 | B0 |
| 01 | 40 | 00 | 00 | 00 | 00 | 00 | 00 | 0F | 13 | 8E | 84 |
| E8 | D6 | F8 | 01 | AA | 31 | 22 | 07 | 00 | A4 | 00 | 04 |
| 02 | 5F | C0 | 22 | 07 | 00 | A4 | 00 | 04 | 02 | 4F | 0A |
| 22 | 07 | 00 | D6 | 00 | 00 | 02 | 00 | 55 | 81 | 14 | 81 |
| 03 | 01 | 01 | 01 | 82 | 02 | 81 | 82 | 12 | 09 | 01 | 3F |
| 00 | 7F | FF | 5F | C0 | 4F | 0A |  |  |  |  |  |

##### 27.22.XX.1.5	Test requirement

The ME shall operate in the manner defined in expected sequences 1.1 to 1.4.

…

***** End of changes *****