3GPP TSG-CT Meeting #97-e	CP-222181

# E-Meeting, 12th – 14th September 2022										(revision of C3-224060)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.549 | CR | 0095 | rev | 1 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Termination of reporting related updates |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Ericsson, Huawei |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL |  |  |  |  |  | Date: |  |  | 2022-09-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The data model of the SS_NetworkResourceMonitoring API needs to be organized and corrected with regards to termination of reporting as per the reply LS from SA6 in S6-221448 and S6-221404. In addition: - The related service description in clause 5.5.3.2.4.2 should also be updated. - Some editorial issues need to be fixed in the other service description clauses. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes to: - Define the data types to carry the reporting thresholds (for the case of threshold based reporting) and the threshold handling mode. - Introduce the threshold matching direction. - Update the definition of the ReportingRequirements data type and service description in clause 5.5.3.2.4.2 accordingly. - Apply the necessary editorial corrections to the other service description clauses. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Stage 2 provisions not fully specified in Stage 3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.5.3.2.2.2, 5.5.3.2.3.2, 5.5.3.2.4.1, 5.5.3.2.4.2, 7.4.2.4.1, 7.4.2.4.2.3, 7.4.2.4.2.5, 7.4.2.4.2.10 (new subclause added); 7.4.2.4.3.4 (new subclause added), A.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | - This CR implements changes according to SA6 LS reply S6-221448; - This CR introduces backwards compatible corrections to the OpenAPI description of the SS_NetworkResourceMonitoring API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | C3-224060 agreed in CT3#123e meeting is revised to remove the word “fetch” which was left by mistake in clause 5.5.3.2.2.2. |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

* * * * Start of changes * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[3]	3GPP TS 29.122: "T8 reference point for Northbound Application Programming Interfaces (APIs)".

[4]	IETF RFC 6455: "The Websocket Protocol".

[5]	IETF RFC 7230: "Hypertext Transfer Protocol (HTTP/1.1): Message Syntax and Routing".

[6]	IETF RFC 7231: "Hypertext Transfer Protocol (HTTP/1.1): Semantics and Content".

[7]	IETF RFC 7232: "Hypertext Transfer Protocol (HTTP/1.1): Conditional Requests".

[8]	IETF RFC 7233: "Hypertext Transfer Protocol (HTTP/1.1): Range Requests".

[9]	IETF RFC 7234: "Hypertext Transfer Protocol (HTTP/1.1): Caching".

[10]	IETF RFC 7235: "Hypertext Transfer Protocol (HTTP/1.1): Authentication".

[11]	Void

[12]	IETF RFC 7540: "Hypertext Transfer Protocol Version 2 (HTTP/2)".

[13]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[14]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[15]	Open API: "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[16]	3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs; Stage 3".

[17]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs; Stage 2".

[18]	3GPP TS 33.122: "Security Aspects of Common API Framework for 3GPP Northbound APIs".

[19]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[20]	3GPP TS 29.523: "5G System; Policy Control Event Exposure Service; Stage 3".

[21]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[22]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[23]	3GPP TS 29.468: "Group Communication System Enablers for LTE (GCSE_LTE); MB2 reference point; Stage 3".

[24]	3GPP TR 21.900: "Technical Specification Group working methods".

[25]	3GPP TS 33.210: "3G security; Network Domain Security (NDS); IP network layer security".

[26]	3GPP TS 33.434: "Service Enabler Architecture Layer for Verticals (SEAL); Security Aspects".

[27]	3GPP TS 29.486: "V2X Application Enabler (VAE) Services; Stage 3".

[28]	3GPP TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage 3".

[29]	IEEE 802.1Qcc-2018: "IEEE Standard for Local and Metropolitan Area Networks—Bridges and Bridged Networks".

[30]	3GPP TS 29.514: "5G System; Policy Authorization Service; Stage 3".

[31]	3GPP TS 29.572: "5G System; Location Management Services; Stage 3".

[32]	3GPP TS 29.508: "5G System; Session Management Event Exposure Service; Stage 3".

[33]	3GPP TS 29.520: "5G System; Network Data Analytics Services; Stage 3".

* * * * * Next changes * * * *

###### 5.5.3.2.2.2	VAL server subscribes for Unicast QoS Monitoring using Subscribe_Unicast_QoS_Monitoring

In order to subscribe to unicast QoS monitoring, the VAL server shall send an HTTP POST message to the NRM server targeting the URI of the "Unicast Monitoring Subscriptions" resource as specified in clause 7.4.2.2.2.3.1. The request body shall include the MonitoringSubscription data structure as defined in clause 7.4.2.4.2.8. The VAL server shall indicate within the ReportingRequirement data structure whether one-time reporting and/or immediate reporting is requested, i.e.:

-	the "immRep" attribute set to "true", if immediate reporting of the unicast QoS Monitoring data is requested; and/or

-	the "reportingMode" attribute set to "ONE_TIME" and the "immRep" attribute set to "true", if one-time reporting of the unicast QoS Monitoring data is requested via the Obtain_Unicast_QoS_Monitoring service operation.

Upon reception of the HTTP POST request message, the NRM server shall:

-	verify the identity of the VAL server and whether the VAL server is authorized to create a unicast QoS monitoring subscription at the NRM server;

-	if the VAL server is not authorized, the NRM server shall respond to the VAL server with an appropriate error status code;

-	if the VAL server is authorized:

-	if immediate reporting and one-time reporting are requested, the NRM server determines if it the requested data is available internally or not and whether to interact with the NEF to retrieve the data using the Nnef_AnalyticsExposure API (UE Communication Analytics Events and DN Performance Analytics) as defined in 3GPP TS 29.522 [28];

-	otherwise, the NRM server shall interact with the NEF to establish the associated QoS monitoring subscriptions by invoking the Nnef_AnalyticsExposure API (UE Communication Analytics Events and DN Performance Analytics) and AsSessionWithQoS API as defined in 3GPP TS 29.522 [28]. The NRM server determines the relevant NEF subscription procedures and the parameters for these subscriptions based on the inputs received from the VAL server;

-	upon reception of successful response(s) from the NEF or retrieval of the requested data internally:

-	if immediate reporting and one-time reporting are requested, an HTTP "200 OK" status code, with the response body including the MonitoringReport data structure containing the available requested Unicast QoS Monitoring data as defined in clause 7.4.2.4.2.2;

-	otherwise, the NRM server shall create a new "Individual Unicast Monitoring Subscription" resource and respond to the VAL server with:

-	an HTTP "201 Created" status code, including a Location header field containing the URI for the created "Individual Unicast Monitoring Subscription" resource and the response body including the MonitoringSubscription data structure containing a representation of the created resource as defined in clause 7.4.2.4.2.8; and

-	if immediate reporting was requested by the VAL server, the returned MonitoringSubscription data structure shall also contain the requested Unicast QoS Monitoring data within the "monRep" attribute, if the requested data is available, as defined in clause 7.4.2.4.2.2;

-	in the case of partial failure, i.e. the request fails for only a subset of the targeted VAL UE(s) or VAL Stream ID(s), the NRM server shall include the "failureRep" attribute within the returned MonitoringReport data structure indicating the list of VAL UE(s) or VAL Stream ID(s) for which the NRM server failed to obtain the requested data and the related failure reasons;

and

-	if the NRM server is unable to satisfy the request, the NRM server shall respond to the VAL server with an appropriate error status code.

* * * * * Next changes * * * *

###### 5.5.3.2.3.2	VAL server unsubscribes for Unicast QoS Monitoring using Unsubscribe_Unicast_QoS_Monitoring

In order to terminate a unicast QoS monitoring subscription, the VAL server shall send an HTTP DELETE request message to the NRM server, on the corresponding "Individual Unicast Monitoring Subscription" resource URI as specified in clause 7.4.2.2.3.3.1.

Upon reception of the HTTP DELETE request message, the NRM server shall:

1.	verify the identity of the VAL server and check if the VAL server is authorised to terminate the targeted "Individual Unicast Monitoring Subscription" associated with the resource URI;

2.	if the VAL server is authorized to unsubscribe from Unicast QoS Monitoring, interact with the NEF to terminate the related QoS monitoring subscription and delete the related "Individual Unicast Monitoring Subscription" subscription resource at the NRM server; and

3.	upon success, respond to the VAL server with a "204 No Content" status code.

* * * * * Next changes * * * *

###### 5.5.3.2.4.1	General

This service operation is used by the NRM server to notify the VAL server of unicast QoS monitoring data.

* * * * * Next changes * * * *

###### 5.5.3.2.4.2	NRM server notifies for Unicast QoS Monitoring using Notify_Unicast_QoS_Monitoring

The NRM server receives unicast QoS monitoring data by means of notifications provided by the NEF. The NRM server coordinates and aggregates the received information from the NEF notifications and determines whether to send a notification to the VAL server based on the VAL server subscription's reporting requirements. For event-triggered reporting, the NRM server notifies the VAL server when any given event is triggered. For a VAL group or a list of VAL UEs, the NRM server aggregates QoS monitoring data for each UE belonging to the group or the list; for a VAL stream, the NRM server aggregates the QoS monitoring data for the stream.

The NRM server stops reporting according to the VAL server subscription's termination of reporting requirements. In the case user-triggered termination of reporting is requested or no termination of reporting requirements are provided, the NRM server terminates the Unicast Monitoring Subscription after receiving an explicit request from the VAL Server as specified in clause 5.5.3.2.3. In the case of time-triggered or event-triggered termination of reporting, the NRM server shall stop the reporting and terminate the subscription when the provided conditions are met. The NRM server may also store internally the QoS monitoring data as needed for later retrieval.

In order to notify the VAL server about Unicast QoS Monitoring information updates, the NRM server shall send an HTTP POST request message to the VAL server targeting the notification URI provided during subscription creation as specified in clause 5.5.3.2.2.2.

Upon receiving the HTTP POST request message, the VAL server shall:

1. process the Unicast QoS Monitoring notification; and

2. upon success, respond to the NRM server with a "204 No Content" status code.

* * * * * Next changes * * * *

##### 7.4.2.4.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 6.2 apply to this API

Table 7.4.2.4.1-1 specifies the data types defined specifically for the SS_NetworkResourceMonitoring API service.

Table 7.4.2.4.1-1: SS_NetworkResourceMonitoring API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| FailureReport | 7.4.2.4.2.9 | Represents the failure report indicating the VAL UE(s) or VAL Stream ID(s) for which the NRM server failed to obtain the requested data. |  |
| FailureReason | 7.4.2.4.3.3 | Represents the failure reason. |  |
| MeasurementData | 7.4.2.4.2.3 | Presents the aggregated measurement data. |  |
| MeasurementDataType | 7.4.2.4.3.1 | Indicates the requested measurement data type. |  |
| MeasurementPeriod | 7.4.2.4.2.4 | Indicates the measurement time period. |  |
| MeasurementRequirements | 7.4.2.4.2.6 | Indicates the measurement requirements. |  |
| MonitoringReport | 7.4.2.4.2.2 | Indicates the monitoring report for VAL UEs list, VAL Group, or VAL Stream. |  |
| MonitoringSubscription | 7.4.2.4.2.7 | The monitoring subscription request. |  |
| ReportingRequirements | 7.4.2.4.2.5 | Indicates the requested requirements of reporting. |  |
| ReportingThreshold | 7.4.2.4.2.10 | Indicates a requested reporting threshold. |  |
| TerminationMode | 7.4.2.4.3.2 | Indicates the termination mode. |  |
| ThresholdHandlingMode | 7.4.2.4.3.4 | Indicates the multi-parameter threshold handling mode. |  |

Table 7.4.2.4.1-2 specifies data types re-used by the SS_NetworkResourceMonitoring API service.

Table 7.4.2.4.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| AverWindow | 3GPP TS 29.571 [21] |  |  |
| BitRate | 3GPP TS 29.571 [21] |  |  |
| DateTime | 3GPP TS 29.571 [21] |  |  |
| DurationSec | 3GPP TS 29.571 [21] |  |  |
| MatchingDirection | 3GPP TS 29.520 [33] |  |  |
| NotificationMethod | 3GPP TS 29.508 [32] |  |  |
| PacketLossRate | 3GPP TS 29.571 [21] |  |  |
| SupportedFeatures | 3GPP TS 29.571 [21] |  |  |
| Uinteger | 3GPP TS 29.571 [21] |  |  |
| Uri | 3GPP TS 29.571 [21] |  |  |
| ValTargetUe | clause 7.3.1.4.2.3 | Used to identify either a VAL User ID or a VAL UE ID. |  |
| WebsockNotifConfig | 3GPP TS 29.122 [3] |  |  |

* * * * * Next changes * * * *

###### 7.4.2.4.2.5	Type: ReportingRequirements

Table 7.4.2.4.2.5-1: Definition of type ReportingRequirements

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| reportingMode | NotificationMethod | M | 1 | The indication of the requested reporting option: one-time, periodic or event-triggered (i.e. "ON_EVENT_DETECTION") This attribute may be set to the value "ONE_TIME" only if the "immRep" attribute is provided and set to "true".. |  |
| reportingPeriod | DurationSec | C | 0..1 | Identifies the reporting time interval for the periodic reporting. (NOTE 1). |  |
| reportingThrs | array(ReportingThreshold) | C | 1..N | Identifies reporting threshold corresponding to the measurement index(es). (NOTE 2). |  |
| immRep | boolean | O | 0..1 | It indicates immediate reporting. When included and set to true, it indicates that immediate reporting of the subscribed event(s) is requested. |  |
| repTerminMode | TerminationMode | O | 0..1 | The indication of the requested reporting termination mode: time-triggered, event-triggered (number of reports reached), event-triggered (threshold reached) or user-triggered. If absent, user-triggered reporting is used as the default termination mode. |  |
| expirationTimer | DurationSec | C | 0..1 | Identifies the reporting time interval for the time triggered termination mode. (NOTE 3). |  |
| maxNumRep | Uinteger | C | 0..1 | Indicates the maximum number of reports (number of reports reached). (NOTE 4). |  |
| termThr | MeasurementData | C | 0..1 | Indicates the reporting termination threshold(s) corresponding to the measurement index(ex). (NOTE 5, NOTE 6, NOTE 7). |  |
| termThrMode | ThresholdHandlingMode | C | 0..1 | Indicates the multi-parameter threshold handling for the event-triggered threshold reached reporting termination. (NOTE 8). |  |
| NOTE 1: The "reportingPeriod" attribute shall be present only when the "reportingMode" attribute is set to "PERIODIC". NOTE 2: The "reportingThrs" attribute shall be present only when the "reportingMode" attribute is set to "EVENT_TRIGGERED". NOTE 3: The "expirationTimer" attribute shall be present only if the "repTerminMode" attribute is present and set to "TIME_TRIGGERED". NOTE 4: The "maxNumRep" attribute shall be present only if the "repTerminMode" attribute is present and set to "EVENT_TRIGGERED_NUM_REPORTS_REACHED". NOTE 5: The "termThr" attribute shall be present only when the "repTerminMode" attribute is present and set to "EVENT_TRIGGERED_MEAS_THR_REACHED". NOTE 6: The event-triggered reporting termination condition for the "dlDelay", "ulDelay", "rtDelay" and "avgPlr" attributes within the MeasurementData data structure is met when the measured value is greater than or equal to the given threshold. NOTE 7: The event-triggered reporting termination condition for the "avgDataRate", "maxDataRate", "avrDlTrafficVol" and "avrUlTrafficVol" attributes within the MeasurementData data structure is met when the measured value is less than or equal to the given threshold. NOTE 8: The "termThrMode" attribute shall be present only when the "repTerminMode" attribute is present and set to "EVENT_TRIGGERED_MEAS_THR_REACHED" and the "termThr" contains more than one threshold. |  |  |  |  |  |

* * * * * Next changes * * * *

###### 7.4.2.4.2.10	Type: ReportingThreshold

Table 7.4.2.4.2.10-1: Definition of type ReportingThreshold

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| measThrValues | MeasurementData | M | 1 | Indicates the value(s) for the measurement threshold index(es). |  |
| thrDirection | MatchingDirection | M | 1 | Indicates the threshold matching direction for the measurement threshold index(es) provided in the "measThrValues" attribute. |  |

* * * * * Next changes * * * *

###### 7.4.2.4.3.4	Enumeration: ThresholdHandlingMode

Table 7.4.2.4.3.4-1: Enumeration ThresholdHandlingMode

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| ALL_REACHED | The decision criterion is met when all the provided thresholds are reached. |  |
| ANY_REACHED | The decision criterion is met when any of the provided threshold(s) is reached. |  |

* * * * * Next changes * * * *

# A.10	SS_NetworkResourceMonitoring API

openapi: 3.0.0

info:

title: SS_NetworkResourceMonitoring

description: |

API for SEAL Network Resource Monitoring.

© 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.0.0"

externalDocs:

description: >

3GPP TS 29.549 V17.5.0 Service Enabler Architecture Layer for Verticals (SEAL);

Application Programming Interface (API) specification; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.549/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/ss-nrm/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549

paths:

/subscriptions:

post:

summary: Create individual unicast monitoring subscription resource or obtain unicast QoS monitoring data for VAL UEs, VAL Group, or VAL Streams.

operationId: SubscribeUnicastMonitoring

tags:

- Unicast Monitoring Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MonitoringSubscription'

responses:

'201':

description: >

The requested individual monitoring subscription resource is successfully created

and a representation of the created resource is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/MonitoringSubscription'

headers:

Location:

description: Contains the URI of the newly created individual monitoring resource.

required: true

schema:

type: string

'200':

description: The requested unicast QoS monitoring data is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/MonitoringReport'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

NotifyUnicastMonitoringData:

'{$request.body#/notifUri}':

post:

summary: Notify on updates of the individual monitoring resorce accoring the requested reporting settings.

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MonitoringReport'

responses:

'204':

description: The notification is successfully received.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

delete:

summary: Remove an existing individual unicast monitoring subscription resource according to the subscriptionId.

operationId: UnsubscribeUnicastMonitoring

tags:

- Individual Unicast Monitoring Subscription (Document)

parameters:

- name: subscriptionId

in: path

description: >

Represents the identifier of an individual unicast monitoring subscription resource.

required: true

schema:

type: string

responses:

'204':

description: >

The Individual Unicast Monitoring Subscription resource matching the

subscriptionId is deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

get:

summary: Read an existing individual unicast monitoring subscription resource according to the subscriptionId.

operationId: ReadUnicastMonitoringSubscription

tags:

- Individual Unicast Monitoring Subscription (Document)

parameters:

- name: subscriptionId

in: path

description: >

Represents the identifier of an individual unicast monitoring subscription resource.

required: true

schema:

type: string

responses:

'200':

description: The requested individual unicast monitoring subscription returned.

content:

application/json:

schema:

$ref: '#/components/schemas/MonitoringSubscription'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

MonitoringReport:

description: Indicates the monitoring information for VAL UEs list, VAL Group, or VAL Stream.

type: object

properties:

valUeIds:

type: array

minItems: 1

items:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

description: List of VAL UEs whose QoS monitoring data is requested.

valGroupId:

type: string

description: The VAL Group Id which QoS monitoring data is requested.

valStreamIds:

type: array

minItems: 1

items:

type: string

description: List of VAL streams for which QoS monitoring data is requested.

measData:

$ref: '#/components/schemas/MeasurementData'

failureRep:

type: array

items:

$ref: '#/components/schemas/FailureReport'

description: >

The failure report indicating the VAL UE(s) or VAL Stream ID(s) whose measurement

data is not obtained successfully.

timestamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

required:

- measData

- timestamp

oneOf:

- required: [valUeIds]

- required: [valGroupId]

- required: [valStreamIds]

MeasurementData:

description: Presents the aggregated measurement data.

type: object

properties:

dlDelay:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

ulDelay:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

rtDelay:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

avgPlr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketLossRate'

avgDataRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

maxDataRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

avrDlTrafficVol:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

avrUlTrafficVol:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

anyOf:

- required: [dlDelay]

- required: [ulDelay]

- required: [rtDelay]

- required: [avgPlr]

- required: [avgDataRate]

- required: [maxDataRate]

- required: [avrDlTrafficVol]

- required: [avrUlTrafficVol]

MeasurementPeriod:

description: >

Indicates the measurement time period.

type: object

properties:

measStartTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

measDuration:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

required:

- measStartTime

- measDuration

ReportingRequirements:

description: Indicates the requested frequency of reporting.

type: object

properties:

reportingMode:

$ref: 'TS29508_Nsmf_EventExposure.yaml#/components/schemas/NotificationMethod'

reportingPeriod:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

reportingThrs:

type: array

items:

$ref: '#/components/schemas/ReportingThreshold'

minItems: 1

immRep:

type: boolean

repTerminMode:

$ref: '#/components/schemas/TerminationMode'

expirationTimer:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

maxNumRep:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

termThr:

$ref: '#/components/schemas/MeasurementData'

termThrMode:

$ref: '#/components/schemas/ThresholdHandlingMode'

required:

- reportingMode

FailureReport:

description: >

Represents the failure report indicating the VAL UE(s) or VAL Stream ID(s)

for which the NRM server failed to obtain the requested data.

type: object

properties:

valUeIds:

type: array

minItems: 1

items:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

description: >

List of VAL UE(s) whose measurement data is not obtained successfully.

valStreamIds:

type: array

minItems: 1

items:

type: string

description: >

List of VAL stream ID(s) whose measurement data is not obtained successfully.

failureReason:

$ref: '#/components/schemas/FailureReason'

measDataType:

$ref: '#/components/schemas/MeasurementDataType'

required:

- measDataType

MeasurementRequirements:

description: Indicates the measurement requirements.

type: object

properties:

measDataTypes:

type: array

items:

$ref: '#/components/schemas/MeasurementDataType'

minItems: 1

description: Indicates the required the QoS measurement data types.

measAggrGranWnd:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AverWindow'

measPeriod:

$ref: '#/components/schemas/MeasurementPeriod'

required:

- measDataTypes

MonitoringSubscription:

description: The unicast monitoring subscription request.

type: object

properties:

valUeIds:

description: List of VAL UEs whose QoS monitoring data is requested.

type: array

minItems: 1

items:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

valGroupId:

type: string

description: The VAL Group Id which QoS monitoring data is requested.

valStreamIds:

type: array

minItems: 1

items:

type: string

description: List of VAL streams for which QoS monitoring data is requested.

measReqs:

$ref: '#/components/schemas/MeasurementRequirements'

monRep:

$ref: '#/components/schemas/MonitoringReport'

reportReqs:

$ref: '#/components/schemas/ReportingRequirements'

notifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

reqTestNotif:

type: boolean

wsNotifCfg:

$ref: 'TS29122_CommonData.yaml#/components/schemas/WebsockNotifConfig'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

ReportingThreshold:

description: >

Indicates the requested reporting termination threshold for the measurement index(es).

type: object

properties:

measThrValues:

$ref: '#/components/schemas/MeasurementData'

thrDirection:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/MatchingDirection'

required:

- measThrValues

- thrDirection

# Simple data types and Enumerations

MeasurementDataType:

anyOf:

- type: string

enum:

- DL_DELAY

- UL_DELAY

- RT_DELAY

- AVG_PLR

- AVG_DATA_RATE

- MAX_DATA_RATE

- AVG_DL_TRAFFIC_VOLUME

- AVG_UL_TRAFFIC_VOLUME

- type: string

description: >

Indicates the type of measurement data.

description: |

Possible values are:

- DL_DELAY: Downlink packet delay.

- UL_DELAY: Uplink packet delay.

- RT_DELAY: Round trip packet delay.

- AVG_PLR: Average packet loss rate.

- AVG_DATA_RATE: Average data rate.

- MAX_DATA_RATE: Maximum data rate.

- AVG_DL_TRAFFIC_VOLUME: Average downlink traffic volume.

- AVG_UL_TRAFFIC_VOLUME: Average uplink traffic volume.

TerminationMode:

anyOf:

- type: string

enum:

- TIME_TRIGGERED

- EVENT_TRIGGERED_NUM_REPORTS_REACHED

- EVENT_TRIGGERED_MEAS_THR_REACHED

- USER_TRIGGERED

- type: string

description: >

Indicates the termination mode

description: |

Possible values are:

- TIME_TRIGGERED: Time-triggered termination mode.

- EVENT_TRIGGERED_NUM_REPORTS_REACHED: Event-triggered termination number of reports reached mode.

- EVENT_TRIGGERED_MEAS_THR_REACHED: The event-triggered termination measurement index threshold reached mode.

- USER_TRIGGERED: User-triggered termination mode.

FailureReason:

anyOf:

- type: string

enum:

- USER_NOT_FOUND

- STREAM_NOT_FOUND

- DATA_NOT_AVAILABLE

- OTHER_REASON

- type: string

description: >

Represents the failure reason.

description: |

Possible values are:

- USER_NOT_FOUND: The user is not found.

- STREAM_NOT_FOUND: The stream is not found.

- DATA_NOT_AVAILABLE: The requested data is not available.

- OTHER_REASON: Other reason (unspecified).

ThresholdHandlingMode:

anyOf:

- type: string

enum:

- ALL_REACHED

- ANY_REACHED

- type: string

description: >

Indicates the multi-parameter threshold handling mode.

description: |

Possible values are:

- ALL_REACHED: The decision criterion is met when all the provided thresholds are reached.

- ANY_REACHED: The decision criterion is met when any of the provided threshold(s) is reached.

* * * * End of changes * * * *