3GPP TSG-CT Meeting #97-e	CP-222184

# E-Meeting, 12th – 14th September 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 4439 | rev | 6 | Current version: | 17.7.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NAS operation for network slice-based random access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Nokia, Nokia Shanghai Bell, Apple |  |  |  |  |  |  |  |  |  |
| Work item code: | NRslice |  |  |  |  |  | Date: |  |  | 2022-09-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | How the NAS operates for NSAG-aware random access needs to be specified. The stage 2 CR having formal linkage to this CR specifies the foloiwng requirement. For Network Slice based Random Access, different Random Access resources may be assigned to different NSAG(s). The UE determines Random Access configuration among NSAGs that are published in SIB for Random Access and that are associated to the S-NSSAIs triggering the access. If the signalling transaction triggering the access attempt is related to more than one network slice, and the S-NSSAIs of these network slices are associated with more than one NSAG for Random Access, the NSAG with the highest priority is selected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The NAS layer provides one or more S-NSSAIs associated with an access attempt. Backward compatibility analysis Backward compatible as it only impacts internal operation in the UE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Rel-17 WI NRslice is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.501 CR 3676 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 4.6.2.6	Provision of NSAG information to lower layers

NSAG information includes a list of NSAG IDs each of which is associated with:

a)	a list of S-NSSAIs, which shall be the ones included in the configured NSSAI;

b)	an NSAG area containing a list of TAIs which identify an area where the mapping between the S-NSSAI(s) in bullet a) and the NSAG ID is valid; and

c)	a priority value that is associated with each NSAG ID in the NSAG information.

If NSAG information is available, the UE NAS layer shall provide the lower layers with the most recent NSAG information.

NOTE 1:	Along with the NSAG information, the UE provides the lower layers with allowed NSSAI and requested NSSAI for the purpose of NSAG-aware cell reselection.

NOTE 2:	When an access attempt is made by a UE in 5GMM-IDLE mode or 5GMM-CONNECTED mode with RRC inactive indication, the UE implementation ensures that an NSAG with the highest priority with the S-NSSAI(s) of a control plane or user plane interaction triggering the access attempt is selected.

* * * End of Changes * * * *