3GPP TSG-CT Meeting #97-e	CP-222203

# E-Meeting, 12th – 14th September 2022

Source:	Huawei, HiSilicon

Title:	General description for TS 24.575

Spec:	3GPP TS 24.575 v0.1.0

Agenda item:	17.48

Document for:	Agreement

1. Introduction

This p-CR provides content of the General description for the 3GPP TS 24.575 specification related to the 5MBS work item.

2. Reason for Change

The general description of the 3GPP TS 24.575 specification needs to be defined.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 24.575 v0.1.0.

* * * First Change * * * *

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

MBS	Multicast/Broadcast Services

TMGI	Temporary Mobile Group Identity

USD	User Service Description

* * * Next Change * * * *

# 4	General description

A UE can support multicast and broadcast service (MBS) that is a point-to-multipoint service in which data is transmitted from a single source entity to multiple recipients either to all users in a broadcast service area (MBS broadcast communication), or to users in a multicast group (MBS multicast communication) as defined in 3GPP TS 23.247 [2]. The UE may support pre-configuration of information for broadcast communication services using MBS.

If the UE is pre-configured with information related to broadcast communication services using MBS, the UE can discover and receive data for broadcast communication services by using the provisioned configuration.

The UE pre-configuration contains a list of PLMNs in which for each PLMN, the following information is configured:

a)	PLMN ID of the PLMN for which the configuration applies;

b)	RAN information based on NR-ARFCN on which the broadcast communication service is available;

c)	list of TMGI, on which the broadcast communication service is available, along with the associated user service description (USD) information for the MBS broadcast service. If TMGI and USD information for the MBS user service is configured for the selected PLMN, the UE uses the information configured therein to acquire the broadcast communication service; and

d)-	list of TMGI, on which the service announcement for broadcast communication service is available along with the associated USD information for the MBS user service announcement service. If TMGI and USD information for MBS user service announcement is configured for a selected PLMN, use the information configured therein to acquire the service announcement for broadcast communication service.

* * * End of Changes * * * *