3GPP TSG-CT Meeting #98-e	CP-223bbb

# E-Meeting, 12th – 14th December 2022									(rev of CP-223214, C1-227041)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.302 | CR | 0737 | rev | 3 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for discovery of SNPNs with 5G connectivity support |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel, Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | OPPO, Intel, Lenovo |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN_Ph2 |  |  |  |  |  | Date: |  |  | 2022-12-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | SA2 has agreed in CR3714 to 23.501 (see S2-2209869) clause 5.30.2.Y that a non-3GPP access network may advertise (e.g. with ANQP), not only the PLMNs with which 5G connectivity is supported (as specified in clause 6.3.12.2), but also the SNPNs with which 5G connectivity is supported: “The UE initiates the access network selection procedure specified in clause 6.3.12.2 and constructs a list of available SNPNs. This list contains the SNPNs advertised by all discovered non-3GPP access networks. A non-3GPP access network may advertise (e.g. with ANQP), not only the PLMNs with which 5G connectivity is supported (as specified in clause 6.3.12.2), but also the SNPNs with which 5G connectivity is supported.” 23.501 clause 5.30.2.Y specifies for UE onboarding via Trusted non-3GPP the non-3GPP access network advertises (e.g. via ANQP) an Onboarding enabled indication, as specified in clause 5.30.2.10. “UE onboarding via Trusted non-3GPP access is supported as follows: - The non-3GPP access network advertises (e.g. via ANQP) an Onboarding enabled indication, as specified in clause 5.30.2.10.2.3. - The UE selects an SNPN advertising the Onboarding enabled indication following the network selection procedure specified in clause 5.30.2.10.2.5. - As part of UE registration via Trusted non-3GPP access, in Figure 4.12a.2.2-1, step 5 of TS 23.502 [3] the UE provides an onboarding indication inside the AN-Parameters.” [..] Accordingly, this CR proposes a new IE to advertise SNPNs with which 5G connectivity is supported. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a new IE to advertise SNPNs with which 5G connectivity is supported. The new IE includes indications to indicate access using credentials from a Credentials Holder is supported, the supported GINs, if registration attempts from UEs that are not explicitly configured to select the SNPN are allowed, if onboarding is enabled. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for a non-3GPP access network to advertise SNPNs with which 5G connectivity is supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | H.2.4.1, H.2.4.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.501 CR 3714 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1 - Added missing text in H.2.4.1 - Update information element identity enumeration to reserve a gap for additional PLMN related IEs - Removed Spare octets in Figure H.2.4.X-5 as there is already a length field. - Aligned octet numbering and its optionality - Added an EN that the coding of NID may need to be updated in alignment with the coding of the NID in TS 24.501 and TS 24.502. - Added additional co-source Rev2 of CR (submission to CT#98-e) - correcting decode in subclause H.2.4.1 and adjustment to avoid clash with changes brought in by 24.302CR0731 Rev 3 of CR (during CT#98-e) - remove the (double) addition of “00000101” |  |  |  |  |  |  |  |  |

*** First change ***

### H.2.4.1	Information Element Identity (IEI)

Indicates the information element identity. The following values for IEI are defined in this version of the specification:

00000000	PLMN List

00000001	PLMN List with S2a connectivity

00000010	PLMN List with trusted 5G connectivity

00000011	PLMN List with trusted 5G connectivity-without-NAS

00000100²

To

00011111	Reserved

00100000	SNPN List with trusted 5G connectivity

00100001

To

11111111	Reserved

*** Next change ***

### H.2.4.X	SNPN List with trusted 5G Connectivity IE

The SNPN List with trusted 5G Connectivity information element is used by the network to indicate the SNPNs that can be selected from the WLAN. The SNPN List with trusted 5G Connectivity information element optionally includes per SNPN access information, per SNPN supported GINs, and a GIN list. The format of the SNPN List with trusted 5G Connectivity information element coded according to 3GPP TS 24.007 [48] subclause 11.2.2.1 is shown in figure H.2.4.X-1.

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SNPN List with trusted 5G Connectivity IE |  |  |  |  |  |  |  | octet 1 |
| Length of SNPN List with trusted 5G Connectivity value contents |  |  |  |  |  |  |  | octet 2 |
| SNPN information List |  |  |  |  |  |  |  | octet 3 octet d |
| GIN list |  |  |  |  |  |  |  | octet d+1* octet Z* |

Figure H.2.4.X-1: SNPN List with trusted 5G Connectivity information element

The format of the SNPN information list according to 3GPP TS 24.007 [48] subclause 11.2.2.1 is shown in figure H.2.4.X-2:

| Length of SNPN information list |  | octet 3 |
| --- | --- | --- |
| SNPN information entry 1 |  | octet 4 octet b |
|  | .. | octet b+1* octet c-1* |
| SNPN information entry N |  | octet c* octet d* |

Figure H.2.4.X-2: SNPN information List

The format of the SNPN information entry according to 3GPP TS 24.007 [48] subclause 11.2.2.1 is shown in figure H.2.4.X-3:

| Length of SNPN information entry | octet 4 |
| --- | --- |
| SNPN identity | octet 5 octet 13 |
| SNPN access information | octet 14* octet 15* |
| Supported GINs | octet 16* octet b* |

Figure H.2.4.X-3: SNPN information entry

The format of the SNPN identity according to 3GPP TS 24.007 [48] subclause 11.2.2.1 is shown in figure H.2.4.X-4:

| 7 | 6 | 5 | 4 | 3 |  | 2 | 1 | 0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MCC digit 2 |  |  |  |  | MCC digit 1 |  |  |  | octet 5 |
| MNC digit 3 |  |  |  |  | MCC digit 3 |  |  |  | octet 6 |
| MNC digit 2 |  |  |  |  | MNC digit 1 |  |  |  | octet 7 |
| NID value digit 1 |  |  |  |  | NID assignment mode |  |  |  | octet 8 |
| NID value digit 3 |  |  |  |  | NID value digit 2 |  |  |  | octet 9 |
| NID value digit 5 |  |  |  |  | NID value digit 4 |  |  |  | octet 10 |
| NID value digit 7 |  |  |  |  | NID value digit 6 |  |  |  | octet 11 |
| NID value digit 9 |  |  |  |  | NID value digit 8 |  |  |  | octet 12 |
| Spare |  |  |  |  | NID value digit 10 |  |  |  | octet 13 |

Figure H.2.4.X-4: SNPN identity

Editor’s Note [WI: eNPN_Ph2, CR#0737]:	The coding of SNPN identity may need to be updated in alignment with the coding of the NID in 3GPP TS 24.501 and 3GPP TS 24.502.

The format of the SNPN access information according to 3GPP TS 24.007 [48] subclause 11.2.2.1 is shown in figure H.2.4.X-4:

| 7 | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of SNPN access information |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet 14 |
| Spare |  | Spare |  | Spare |  | Spare |  | Spare |  | OB |  | CHWC |  | CH | octet 15 |

Figure H.2.4.X-5: SNPN access information

The format of the Supported GINs according to 3GPP TS 24.007 [48] subclause 11.2.2.1 is shown in figure H.2.4.X-6:

| 7 | 6 | 5 | 4 | 3 |  | 2 | 1 | 0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of Supported GINs value |  |  |  |  |  |  |  |  | octet 16 |
| G(8) | G(7) | G(6) | G(5) |  | G(4) | G(3) | G(2) | G(1) | octet 17* |
| .. |  |  |  |  |  |  |  |  | octet 18* octet b-1* |
| G(n) | G(n-1) | G(n-2) | G(n-3) |  | G(n-4) | G(n-5) | G(n-6) | G(n-7) | octet b* |

Figure H.2.4.X-6: Supported GINs

The format of the GIN list according to 3GPP TS 24.007 [48] subclause 11.2.2.1 is shown in figure H.2.4.X 7:

| Length of GIN list | octet d+1 |
| --- | --- |
| GIN information entry 1 | octet d+2 octet e |
| .. | octet e+1* octet f-1* |
| GIN information entry N | octet f* octet Z* |

Figure H.2.4.X-7: GIN list

The format of the GIN information entry according to 3GPP TS 24.007 [48] subclause 11.2.2.1 is shown in figure H.2.4.X-8:

| 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of GIN information entry |  |  |  |  |  |  |  | octet d+2 |  |
| MCC digit 2 |  |  |  | MCC digit 1 |  |  |  | octet d+3 |  |
| MNC digit 3 |  |  |  | MCC digit 3 |  |  |  | octet d+4 |  |
| MNC digit 2 |  |  |  | MNC digit 1 |  |  |  | octet d+5 |  |
| NID list |  |  |  |  |  |  |  | octet d+6 octet e |  |

Figure H.2.4.X-8: GIN information entry

The format of the NID list according to 3GPP TS 24.007 [48] subclause 11.2.2.1 is shown in figure H.2.4.X-9:

| Length of NID list | octet d+6 |
| --- | --- |
| NID identity 1 | octet d+7 octet d+12 |
| .. | octet d+13* octet e-1* |
| NID identity N | octet e* octet e+5* |

Figure H.2.4.X-9: NID list

The format of the NID identity according to 3GPP TS 24.007 [48] subclause 11.2.2.1 is shown in figure H.2.4.X-10:

| 7 | 6 | 5 | 4 | 3 |  | 2 | 1 | 0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NID value digit 1 |  |  |  |  | NID assignment mode |  |  |  | octet d+7 |
| NID value digit 3 |  |  |  |  | NID value digit 2 |  |  |  | octet d+8 |
| NID value digit 5 |  |  |  |  | NID value digit 4 |  |  |  | octet d+9 |
| NID value digit 7 |  |  |  |  | NID value digit 6 |  |  |  | octet d+10 |
| NID value digit 9 |  |  |  |  | NID value digit 8 |  |  |  | octet d+11 |
| Spare |  |  |  |  | NID value digit 10 |  |  |  | octet d+12 |

Figure H.2.4.X-10: NID identity

Editor’s Note [WI: eNPN_Ph2, CR#0737]:	The coding of NID identity may need to be updated in alignment with the coding of the NID in 3GPP TS 24.501 and 3GPP TS 24.502.

Table H.2.4.X-1 shows the coding of SNPN List with trusted 5G Connectivity information element.

Table H.2.4.X-1: SNPN List with trusted 5G Connectivity information element

SNPN List with trusted 5G Connectivity information element (octets 1 to Z)

SNPN identity (octets 5 to 13) MCC (octet 5, octet 6 bits 0 to 3) or (octet d+3, octet d+4 bits 0 to 3) The MCC (Mobile country code) field is coded as in ITU-T Recommendation E.212 [63], Annex A MNC (octet 7, octet 6 bits 4 to 7) or (octet d+5, octet d+4 bits 4 to 7). The coding of the MNC (Mobile network code) field is the responsibility of each administration but BCD coding shall be used. The MNC shall consist of 2 or 3 digits. If a network operator decides to use only two digits in the MNC, bits 4 to 7 of octet 6 or octet d+4 shall be coded as "1111". NID assignment mode (octet 8 bits 0 to 3) or (octet d+7 bits 0 to 3) This field contains the binary encoding of the assignment mode of the NID as defined in 3GPP TS 23.003 [3]. NID value (octet 8 bits 4 to 7, octets 9 to 12, octet 13 bits 0 to 3) or (octet d+7 bits 4 to 7, octets d+8 to d+11, octet d+12 bits 0 to 3) This field contains the binary encoding of each hexadecimal digit of the NID value as defined in 3GPP TS 23.003 [3] Bits 4 to 7 of octet 13 or Bits 4 to 7 of octet d+12 are spare and shall be coded as zero.

SNPN access information (octets 14 to 15) CH (octet 15 bit 0) This bit indicates whether the SNPN supports access using credentials from a credentials holder. 0 access using credentials from a credentials holder supported. 1 access using credentials from a credentials holder not supported. CHWC (octet 15 bit 1) This bit indicates whether the SNPN allows registration attempts with credentials from a credentials holder from UEs that are not explicitly configured to select the SNPN. 0 registration attempts from UEs without configuration to select the SNPN not allowed. 1 registration attempts from UEs without configuration to select the SNPN allowed. OB (octet 15 bit 2) This bit indicates whether the SNPN allows onboarding. 0 onboarding is not allowed. 1 onboarding is allowed. octet 15 bits 3 to 7 are spare.

Supported GINs value (octets 16 to b) G(1) to G(n) is a bitmap indicating whether the SNPN identity in the SNPN information entry supports a corresponding GIN in the GIN list. G(1) bit corresponds to GIN index value one in the GIN list, G(2) corresponds to GIN index value two in the GIN list and so on. A bit set to 1 indicates that the GIN is supported by the SNPN and a bit set to 0 indicates that the GIN is not supported by the SNPN. The length of the Supported GINs value field in octets is the ceiling of the number of GINs in the GIN list divided by 8. The filling bits, if any, are set to 0.

GIN list (octets d+1 to Z) The GIN list contains one or more GIN information entries. Each GIN information entry contains either one GIN, which is identified by a PLMN ID (MCC, MNC) and a single NID (NID list with a single NID entry), or multiple GINs that share the same PLMN ID with each GIN identified by the shared PLMN ID and a NID entry in the NID list. The GIN index m is defined as d1+d2+…+d(n-1)+i for the GIN included in the n-th entry of the GIN list and the i-th entry of its corresponding GIN information entry, where d(k) is the number of GIN index values used in the k-th GIN information entry.

*** End change ***