3GPP TSG-CT WG1 Meeting #123-e	C1-202612

# Electronic meeting, 16-24 April 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.502 | CR | 0122 | rev | 1 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Inclusion of requested NSSAI in AN parameters |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Motorola Mobility, Lenovo, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC |  |  |  |  |  | Date: |  |  | 2020-03-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Subclause 4.12.2.2 in TS 23.502 lists the following text in 5. The UE shall send an IKE_AUTH request, which includes an EAP-Response/5G-NAS packet that contains the Access Network parameters (AN parameters) and a Registration Request message. The AN parameters contain information that is used by the N3IWF for selecting an AMF in the 5G core network. This information includes e.g. the GUAMI, the Selected PLMN ID, the Requested NSSAI and the Establishment cause. The Establishment cause provides the reason for requesting a signalling connection with 5GC. Whether and how the UE includes the Requested NSSAI as part of the AN parameters is dependent on the value of the Access Stratum Connection Establishment NSSAI Inclusion Mode parameter, as specified in clause 5.15.9 of TS 23.501 [2]. The highlighted text is new and needs to be addresses in the stage 3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added missing abbreviation for access network. A note is added to address that inclusion of the requested NSSAI as a part of access type specified in 3GPP TS 24.501. Notes are numbered by X and Y for the MCC to replace them with true values. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is not according to Stage 2 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 7.3.3.1A, 7.3A.2.3, 7A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*********************************** Next Change *****************************************

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GCN	5G Core Network

5GS	5G System

5G-AN	5G Access Network

5QI	5G QoS Identifier

AMF	Access and Mobility Management Function

AN	Access Network

ANDS	Access Network Discovery and Selection

ANDSP	Access Network Discovery and Selection Policy

AUSF	Authentication Server Function

CP	Control Plane

CRG	Cable Residential Gateway

DHCP	Dynamic Host Configuration Protocol

DL	Downlink

DNS	Domain Name System

DSCP	Differentiated Services Code Point

ePDG	Evolved Packet Data Gateway

ESP	Encapsulating Security Payload

FQDN	Fully Qualified Domain Name

H-PCF	A PCF in the HPLMN

IP	Internet Protocol

IPsec	Internet Protocol Security

N3AN	Non-3GPP Access Network

N3IWF	Non-3GPP InterWorking Function

N5CW	Non 5G Capable over WLAN

N5GC	Non-5G Capable

NAI	Network Access Identifier

NAS	Non Access Stratum

NID	Network Identifier

PCF	Policy control Function

PDU	Protocol Data Unit

QFI	QoS Flow Identifier

RQI	Reflective QoS Indicator

SA	Security Association

SNPN	Stand-alone Non-Public Network

SPI	Security Parameters Index

SUPI	Subscription Permanent Identifier

SUCI	Subscription Concealed Identifier

TCP	Transmission Control Protocol

TNAN	Trusted Non-3GPP Access Network

TNAP	Trusted Non-3GPP Access Point

TNGF	Trusted Non-3GPP Gateway Function

TWAN	Trusted WLAN Access Network

TWAP	Trusted WLAN Access Point

TWIF	Trusted WLAN Interworking Function

UL	Uplink

UP	User Plane

UPF	User Plane Function

V-PCF	A PCF in the VPLMN

WLAN	Wireless Local Area Network

WLANSP	WLAN Selection Policy

*********************************** Next Change *****************************************

#### 7.3.3.1A	EAP-5G session initiation

The UE and the N3IWF shall exchange EAP-5G messages within IKE_AUTH request and IKE_AUTH response messages. The N3IWF on reception of an IKE_AUTH request with no AUTH payload shall start an EAP-5G session by sending an EAP-Request/5G-Start message.

The UE acknowledges start of the EAP-5G session by sending an EAP-Response/5G-NAS message which shall include:

a)	a NAS-PDU field containing a NAS message, for example, a REGISTRATION REQUEST message; and

b)	an AN-parameters field containing access network parameters, such as GUAMI, selected PLMN ID, requested NSSAI, establishment cause and selected NID if the UE is accessing SNPN services via a PLMN (see 3GPP TS 23.502 [3]).

NOTE X:	If and how the UE includes the requested NSSAI as a part of the access type depends on the NSSAI inclusion mode IE as specified in 3GPP TS 24.501 [4].

The N3IWF, on reception of NAS messages from the UE within an EAP-Response/5G-NAS message, shall forward the NAS message to the AMF.

The N3IWF, on reception of NAS messages from the AMF, shall include the NAS message within an EAP-Request/5G-NAS message. The N3IWF shall transmit the EAP-Request/5G-NAS message to the UE.

NOTE Y:	The N3IWF is transparent to the NAS messages and as an intermediate network entity only conveys transparently the NAS messages between the UE and the AMF.

The EAP-Request/5G-NAS message shall include a NAS-PDU field that contains a NAS message.

Further NAS messages between the UE and the AMF, via the N3IWF, shall be inserted in NAS-PDU field of an EAP-Response/5G-NAS (UE to N3IWF direction) and EAP-Request/5G-NAS (N3IWF to UE direction) message.

*********************************** Next Change *****************************************

#### 7.3A.2.3	EAP-5G session initiation

The UE and the TNGF shall exchange EAP-5G messages. The TNGF on reception of the NAI by TNAP and passed on to TNGF, shall initiate EAP-5G session by sending an EAP-Request/5G-Start message. Upon reception of an EAP-Request/5G-Start message, the UE shall send an EAP-Response/5G-NAS message encapsulated in link layer protocol packets. In the EAP-Response/5G-NAS message, the UE shall include:

a)	a NAS-PDU field containing a NAS message, for example, a REGISTRATION REQUEST message; and

b)	an AN-parameters field containing access network parameters, such as GUAMI, selected PLMN ID, requested NSSAI and establishment cause, see 3GPP TS 23.502 [3].

NOTE X:	If and how the UE includes the requested NSSAI as a part of the access type depends on the NSSAI inclusion mode IE as especified in 3GPP TS 24.501 [4].

The TNGF on reception of EAP-Response/5G-NAS message, forwards the NAS message to the AMF.

NOTE Y:	The TNGF is transparent to the NAS messages and as an intermediate network entity only conveys transparently the NAS messages to the AMF.

The TNAN, on reception of the NAS messages from the AMF, shall send an EAP-Request/5G-NAS message encapsulated in the link layer protocol packets towards the UE via the TNAP. In the EAP-Request/5G-NAS message, the TNGF shall include:

a)	the NAS message from the AMF; and

b)	an AN-parameters field contain access network parameters, such as TNGF IPv4 contact information, TNGF IPv6 contact information, or both, see 3GPP TS 23.502 [3].

The EAP-Request/5G-NAS message shall include a NAS-PDU field that contains a NAS message. Further NAS messages between the UE and the AMF, via the TNGF, shall be inserted in NAS-PDU field of an EAP-Response/5G-NAS (UE to TNGF direction) and EAP-Request/5G-NAS (TNGF to UE direction) message.

The UE, on reception of the EAP-Request/5G-NAS message including a NAS-PDU field containing a NAS message e.g. for security establishment, shall send a response with EAP-Response/5G-NAS message including a NAS-PDU field containing a NAS message related to the NAS security context to the N3IWF.

*********************************** Next Change *****************************************

## 7A.2	EAP-5G session initiation

The 5G-RG and the W-AGF serving the 5G-RG shall exchange EAP-5G messages via W-CP connection. The W-AGF on reception of an W-CP connection establishment shall start an EAP-5G session by sending an EAP-Request/5G-Start message.

The 5G-RG acknowledges start of the EAP-5G session by sending an EAP-Response/5G-NAS message which shall include:

a)	a NAS-PDU field containing a NAS message, for example, a REGISTRATION REQUEST message; and

b)	an AN-parameters field containing access network parameters, such as GUAMI, selected PLMN ID, requested NSSAI and establishment cause, (see 3GPP TS 23.502 [3]).

NOTE X:	If and how the UE includes the requested NSSAI as a part of the access type depends on the NSSAI inclusion mode IE as especified in 3GPP TS 24.501 [4].

The W-AGF, on reception of NAS messages from the 5G-RG within an EAP-Response/5G-NAS message, shall forward the NAS message to the AMF.

The W-AGF, on reception of NAS messages from the AMF, shall include the NAS message within an EAP-Request/5G-NAS message. The W-AGF shall transmit the EAP-Request/5G-NAS message to the 5G-RG.

NOTE Y:	The W-AGF is transparent to the NAS messages and as an intermediate network entity only conveys transparently the NAS messages between the 5G-RG and the AMF.

The EAP-Request/5G-NAS message shall include a NAS-PDU field that contains a NAS message.

Further NAS messages between the 5G-RG and the AMF, via the W-AGF, shall be inserted in NAS-PDU field of an EAP-Response/5G-NAS (5G-RG to W-AGF direction) and EAP-Request/5G-NAS (W-AGF to 5G-RG direction) message.

*********************************** End of Change *****************************************