3GPP TSG-CT WG1 Meeting #126-e	C1-20xxxx

Electronic meeting, 15-23 October 2020												   was C1-205994

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.486 | CR | 0029 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to server procedure of V2X service discovery procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | V2XAPP |  |  |  |  |  | Date: |  |  | 2020-10-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The HTTP response message of the server procedure of V2X service discovery procedure is unclear, quote of Clause 6.6.2: 6.6.2 Server procedure Upon reception of an HTTP POST request message containing: a) a Content-Type header field set to "application/vnd.3gpp.vae-info+xml"; and b) an application/vnd.3gpp.vae-info+xml MIME body with a <service-discovery-info> element in the <VAE-info> root element, the VAE-S: 1) shall reply with a HTTP response with a <result> element of the <service-discovery-info> element set to a value "success" or "fail", and may include a <service-discovery-data> element which provides the V2X UE service discovery data. The HTTP response needs to be detailed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Specify the HTTP response message; |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The HTTP response message of the server procedure of V2X service discovery procedure is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

6.6.2	Server procedure

Upon reception of an HTTP POST request message containing:

a)	a Content-Type header field set to "application/vnd.3gpp.vae-info+xml"; and

b)	an application/vnd.3gpp.vae-info+xml MIME body with a <service-discovery-info> element in the <VAE-info> root element,

the VAE-S:

a)	shall generate an HTTP 200 (OK) response according to IETF RFC 2616 [19]. In the HTTP 200 (OK) response message, the VAE-S:

1)	shall include a Content-Type header field set to "application/vnd.3gpp.vae-info+xml"; and

2)	shall include an application/vnd.3gpp.vae-info+xml MIME body with a <service-discovery-info> element in the <VAE-info> root element:

i)	shall include a <result> child element set to the value "success" or "failure" indicating success or failure of getting the service discovery information; and

ii)	if <result> element is set to “success”, shall include a <service-discovery-data> element with one or more <V2X-service-mapping> elements, each <V2X-service-mapping> shall include:

A)	one or more <V2X-service-id> element(s) set to the identities of the available V2X service IDs; and

B)	a <V2X-AS-address> element set to the V2X AS address; and

b)	shall send the HTTP 200 (OK) response towards the VAE-C.

* * * End of Change * * * *