3GPP TSG-CT WG1 Meeting #126-e	C1-20xxxx

Electronic meeting, 15-23 October 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 27.007 | CR | 0705 | rev | 1 | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of 5GSM causes #37 and #59 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc., Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GProtoc17 |  |  |  |  |  | Date: |  |  | 2020-10-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |
| Reason for change: |  | Multiple 5GSM causes are introduced in TS 24.501, thus sub-clause 9.2.2.2.3 (Errors for 5GS) needs updates. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Three new 5GS errors are specified: - 137 5GS QoS not accepted (#37) - 181 Unsupported 5QI value (#59) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Muitple 5GSM causes are missing in the sub-clause 9.2.2.2.3 (Errors for 5GS). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.2.2.3, 10.1.49 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** Next change *****

##### 9.2.2.2.3	Errors for 5GS

Numeric	Text

126		Insufficient resources (#26)
127		Missing or unknown DNN (#27)
128		Unknown PDU session type (#28)
129		User authentication or authorization failed (#29)
131		Request rejected, unspecified (#31)
132		Service option not supported (#32)
133		Requested service option not subscribed (#33)
135		PTI already in use (#35)
136		Regular deactivation (#36)
137		5GS QoS not accepted (#37)
141		Semantic error in the TFT operation (#41)
142		Syntactical error in the TFT operation (#42)
143		Invalid PDU session identity (#43)
144		Semantic errors in packet filter(s) (#44)
145		Syntactical errors in packet filter(s) (#45)
172		Semantically incorrect message (#95)
173		Invalid mandatory information (#96)
174		Message type non-existent or not implemented (#97)
175		Conditional IE error (#100)
176		Protocol error, unspecified (#111)
177		Operator determined barring (#8)
181		Unsupported 5QI value (#59)
184		Invalid PTI value (#81)
186		Message not compatible with protocol state (#101)
190		Network failure (#38)
191		Reactivation requested (#39)
192		PDU session type IPv4 only allowed (#50)
193		PDU session type IPv6 only allowed (#51)
196		PDU session type IPv4v6 only allowed (#57)
197		PDU session type Unstructured only allowed (#58)
202		PDU session does not exist (#54)
205		Insufficient resources for specific slice and DNN (#67)
206		Not supported SSC mode (#68)
207		Insufficient resources for specific slice (#69)
208		Message type not compatible with protocol state (#98)
209		Information element non-existent or not implemented (#99)
213		Missing or unknown DNN in a slice (#70)
220		Out of LADN service area (#46)
221		PTI mismatch (#47)
222		Maximum data rate per UE for user-plane integrity protection is too low (#82)
223		Semantic error in the QoS operation (#83)
224		Syntactical error in the QoS operation (#84)
225		Invalid mapped EPS bearer identity (#85)
230		PDU session type Ethernet only allowed (#61)

NOTE:	Values in parentheses are 3GPP TS 24.501 [161] cause codes.

***** Next change *****

### 10.1.49	Define 5GS quality of service +C5GQOS

Table 10.1.49-1: +C5GQOS parameter command syntax

| Command | Possible Response(s) |
| --- | --- |
| +C5GQOS=[<cid>[,<5QI>[,<DL_GFBR>,<UL_GFBR>[,<DL_MFBR>,<UL_MFBR]]]] | +CME ERROR: <err> |
| +C5GQOS? | [+C5GQOS: <cid>,<5QI>[,<DL_GFBR>,<UL_GFBR>[,<DL_MFBR>,<UL_MFBR>]]] [<CR><LF>+C5GQOS: <cid>,<5QI>,[<DL_GFBR>,<UL_GFBR>[,<DL_MFBR>,<UL_MFBR>]] [...]] |
| +C5GQOS=? | +C5GQOS: (range of supported <cid>s),(list of supported <5QI>s),(list of supported <DL_GFBR>s),(list of supported <UL_GFBR>s),(list of supported <DL_MFBR>s),(list of supported <UL_MFBR>s) |

Description

The set command allows the TE to specify the 5GS Quality of Service parameters <cid>, <5QI>, [<DL_GFBR> and <UL_GFBR>] and [<DL_MFBR> and <UL_MFBR>] for a QoS flow (see 3GPP TS 23.501 [165] and 3GPP TS 24.501 [161]). Refer subclause 9.2 for possible <err> values.

A special form of the set command, +C5GQOS= <cid> causes the values for context number <cid> to become undefined.

The read command returns the current settings for each defined QoS.

The test command returns the ranges of the supported parameters as compound values.

Defined values

<cid>: integer type; specifies a particular QoS flow definition, EPS Traffic Flows definition and a PDP Context definition (see the +CGDCONT and +CGDSCONT commands).

<5QI>: integer type; specifies a class of 5GS QoS (see 3GPP TS 23.501 [165] and 3GPP TS 24.501 [161]).

0			5QI is selected by network

[1 – 4]			value range for guaranteed bit rate QoS flows

[71 – 76]			value range for guaranteed bit rate QoS flows

[5 – 9]			value range for non-guaranteed bit rate QoS flows

79, 80			values for non-guaranteed bit rate QoS flows

[82 – 85]			value range for delay critical guaranteed bit rate QoS flows

[128 – 254]		value range for Operator-specific 5QIs

The 5QI values 65, 66, 67, 69 and 70 are not allowed to be requested by the UE. If the TE requests a 5QI parameter 65, 66, 67, 69 or 70, the MT responds with result code +CME ERROR: 181 (unsupported 5QI value).

<DL_GFBR>: integer type; indicates DL GFBR in case of GBR 5QI. The value is in kbit/s. This parameter is omitted for a non-GBR 5QI (see 3GPP TS 24.501 [161]).

<UL_GFBR>: integer type; indicates UL GFBR in case of GBR 5QI. The value is in kbit/s. This parameter is omitted for a non-GBR 5QI (see 3GPP TS 24.501 [161]).

<DL_MFBR>: integer type; indicates DL MFBR in case of GBR 5QI. The value is in kbit/s. This parameter is omitted for a non-GBR 5QI (see 3GPP TS 24.501 [161]).

<UL_MFBR>: integer type; indicates UL MFBR in case of GBR 5QI. The value is in kbit/s. This parameter is omitted for a non-GBR 5QI (see 3GPP TS 24.501 [161]).

Implementation

Optional.