3GPP TSG-CT WG1 Meeting #127bis-e	C1-21xxxx

- Electronic meeting; 25-29 January 2021									Revision of C1-210133

Source:	China Mobile

Title:	Suggestion to KI#1-About unappropriate slice

Spec:	3GPP TR 23.700-10 v0.2.0

Agenda item:	17.3.3

Document for:	Agreement

1. Introduction

Key issue #1 Network Slicing and IMS addresses two scenarios and the following aspects:

a)	What information the UE needs to be able to select an appropriate slice.

b)	How the IMS network selects an appropriate slice.

In the case IMS services ask for dynamic PCC, it has a capability to verify whether the selected slice is appropriate or not. So whether the network needs to verify and indicate an unappropriate slice and indicate to the UE is to be considered.

This proposal suggests a further study about the above aspects.

In addition, considering that the slice is selected during PDU session establishment, IMS network does not need to select an appropriate slice. So it is suggested to remove the following aspect:

- How the IMS network selects an appropriate slice.

2. Reason for Change

It is suggested to remove the aspect “How the IMS network selects an appropriate slice.” and add the following aspect to Key issue #1:

-	Whether the network verifies an unappropriate slice and indicates to the UE.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-10 v0.2.0.

* * * First Change * * * *

## 5.1	Key issue 1: Network Slicing and IMS

In release 16, 5GC developed capabilities to divide the network into slices to better offer services to users and devices - in many cases the details of network slicing do not impact IMS, however if the IMS domain offers different services to different 5GC network slices there may be some impacts. This key issue investigates the impacts to the IMS domain due to 5GC network slicing, and how a IMS network can ensure a user or device with multiple 5GC network slices and multiple services offered by IMS from utilizing the appropriate 5GC network slice for each service on IMS and prevent inappropriate services been used from the incorrect 5GC network slice.

Examples of the use cases that solutions to this key issue will enable include:

Use Case A: Different 5GC slices optimized for different services on IMS

In this use case each 5GC slice supports a different services offered by IMS and the 5GC slice is optimized for the media type associated with the service on IMS. It is possible that either a shared or separate IMS network provides the services for each slice (scenario 1 or scenario 2 described below).

Figure 5.1.1 shows an example implementation of this use case where 3 5GC slices are used, one for IMS service #1, one for IMS service #2, and one for IMS service #3. The diagram shows the association of the services on IMS to their appropriate slice, the services on IMS may be supported by a shared IMS network (i.e. Scenario 2), in which case the 5GC slices will connect to through the common IMS network.

Figure 5.1.1: Example use case with different 5GC slices optimized for different services on IMS

Editor's note:	The definition of IMS service in the figure is FFS.

Use Case B: Different 3rd party 5GC slices with support for 3rd party services on IMS

In this use case separate 5GC slices are operated by different parties (some separate from the network operator); each 5GC slice is used to support services on IMS according to the 5GC slice operator's service needs; these services on IMS may be different or duplicated per slice according to service requirements. Additionally, some 5GC slices may provide non-IMS services. While it is possible that a shared IMS network provides the services for each slice (as per scenario 2 described below), this is an unlikely implementation and it is assumed each party has their own IMS network (as per scenario 1 described below).

Figure 5.1.2 shows an example implementation of this use case where 3 5GC slices are provided, one provided by an automobile manufacturer, one provided by an automotive entertainment service provider, and one provided by a network operator. A different mix of IMS and connectivity services are provided by each 5GC slice. The diagram shows the association of the services on IMS to their appropriate slice, however as each slice is provided by different parties it is unlikely that the services on IMS share an IMS network (as in Scenario 2); thus each 5GC slice would be associated with a different IMS network supporting the services on IMS associated with the 5GC slice.

Figure 5.1.2: Example use case with different 3rd party 5GC slices providing services on IMS

Editor's note:	The definition of IMS service in the figure is FFS.

Two scenarios are possible (and may be mixed when more than 2 slices are considered):

Scenario 1 - Each 5GC network slice is associated with a separate and distinct IMS network

Figure 5.1-3 UE connects to two IMS networks through two 5GC network slices

Scenario 2 - Each 5GC slice is associated with a common IMS network

Figure 5.1-4: UE connects to common IMS network through two 5GC network slices

Solutions may relate to one, or both, of these scenarios.

The following is a partial list of what aspects the solutions to this key issue need to address:

-	What information the UE needs to be able to select an appropriate slice.

-	How the IMS network handles unavailability of a slice.

-	When the IMS FEs and UE are in different PLMNs (i.e. roaming), how the two networks interact.

-	Whether the service offered by the IMS network is aware of the 5GC slice information.

-	If additional information is needed in the IMS signalling between the IMS network and the UE

-	Any changes to the P-CSCF discovery procedures due to 5GC slicing.

-	If any additional information is needed for the UE and IMS network to identify services supported by the IMS network.

-	IMS backward compatibility to Rel-16.

-	How an appropriate slice for a user initiating or upgrading to Multimedia Priority Service (MPS) is selected.

-	Whether the network verifies an unappropriate slice and indicates to the UE.

Editor's note:	It is FFS which scenario each aspect relates to.