3GPP TSG-CT WG1 Meeting #127bis-e	C1-210237

Electronic meeting, 25-29 January 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.483 | CR | 0088 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MOs to restrict MCVideo private communications |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMONASTERY2 |  |  |  |  |  | Date: |  |  | 2021-01-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |
| Reason for change: |  | Stage 2 requirements dictate that an MC service user shall only have private communications with those MC service users which are configured, whereas private communications with other MC service users shall not be allowed. For this purpose, two lists of users are introduced, one including the users that a user can call and a second one including the users from which a call can be received. MCVideo stage 3 specs do not have any of those private communication lists and hence limiting is not supported. In order to support that a user can receive calls from any user, a new option is used to indicate that the list should not be considered and any incoming call should be allowed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Updated MO Figures 2) Updated the parameters for the MCVideo user profile Management Object (MO). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | All incoming and outgoing MCVideo private calls are allowed, which contradicts stage 2 requirements |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.1, 13.2.38H1- 13.2.38H6 (new), 13.2.87B-13.2.87C4 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Added missing UserInfoID, and fixed typo |  |  |  |  |  |  |  |  |

## 13.1	General

The MCVideo user profile configuration Management Object (MO) is used to configure the MCVideo Client behaviour for the on-network or off-network MCVideo Service. The MCVideo user profile configuration parameters may be stored in the ME, or in the USIM as specified in 3GPP TS 31.102 [10], or in both the ME and the USIM. If both the ME and the USIM contain the same parameters, the values stored in the USIM shall take precedence.

The Management Object Identifier is: urn:oma:mo:ext-3gpp-MCVideo-user-profile:1.0.

Protocol compatibility: This MO is compatible with OMA OMA DM 1.2 [3].

The OMA DM ACL property mechanism (see OMA OMA-ERELD-DM-V1_2 [2]) may be used to grant or deny access rights to OMA DM servers in order to modify nodes and leaf objects of the MCVideo user profile MO.

The following nodes and leaf objects are possible under the MCVideo user profile node as described in figure 13.1.1, figure 13.1.2 and figure 13.1.3:

Figure 13.1.1: The MCVideo user profile MO (1 of 3)

Figure 13.1.2: The MCVideo user profile MO (2 of 3)

Figure 13.1.3: The MCVideo user profile MO (3 of 3)

*** Next change ***

### 13.2.38H1	/<x>/<x>/Common/PrivateCall/UserList

Table 13.2.38H1.1: /<x>/<x>/Common/PrivateCall/UserList

| <x>/Common/PrivateCall/UserList |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for a list of MCVideo users who can be called in an MCVideo private call. |  |  |  |  |

### 13.2.38H2	/<x>/<x>/Common/PrivateCall/UserList/<x>

Table 13.2.H2.1: /<x>/<x>/Common/PrivateCall/UserList/<x>

| <x>/Common/PrivateCall/UserList/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | OneOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more list of MCVideo users who can be called in an MCVideo private call. |  |  |  |  |

### 13.2.38H3	/<x>/<x>/Common/PrivateCall/UserList/<x>/Entry

Table 13.2.38H3.1: /<x>/<x>/Common/PrivateCall/UserList/<x>/Entry

| <x>/Common/PrivateCall/UserList/<x>/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more MCVideo users who can be called in an MCVideo private call. |  |  |  |  |

### 13.2.38H4	/<x>/<x>/Common/PrivateCall/UserList/<x>/Entry/MCVideoID

Table 13.2.38H4.1: /<x>/<x>/Common/PrivateCall/UserList/<x>/Entry/MCVideoID

| <x>/Common/PrivateCall/UserList/<x>/Entry/MCVideoID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates an MCVideo user identity (MCVideo ID) which is a globally unique identifier within the MCVideo service that represents the MCVideo user. |  |  |  |  |

The value is a "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 [4].

### 13.2.38H5	/<x>/<x>/Common/ PrivateCall/UserList/<x>/Entry/MCVideoIDKMSURI

Table 13.2.38H5.1: /<x>/<x>/Common/PrivateCall/UserList/<x>/Entry/MCVideoIDKMSURI

| <x>/Common/PrivateCall/UserList/<x>/Entry/MCVideoIDKMSURI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates the identity (URI) of the KMS associated with the MCVideoID. If the value is empty, the KMS leaf node present in the MCS UE initial configuration MO is used. |  |  |  |  |

The value is a URI as specified in 3GPP TS 23.003 [5].

### 13.2.38H6	/<x>/<x>/Common/PrivateCall/UserList/<x>/Entry/UserInfoID

Table 13.2.38H6.1: /<x>/<x>/Common/PrivateCall/UserList/<x>/Entry/UserInfoID

| <x>/Common/PrivateCall/UserList/<x>/Entry/UserInfoID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | int | Get, Replace |  |
|  | This leaf node indicates a ProSe user info ID as specified in 3GPP TS 23.303 [6]. |  |  |  |  |

*** Next change ***

### 13.2.87B	/<x>/<x>/OnNetwork/AuthorisedIncomingAny

Table 13.2.87B.1: /<x>/<x>/OnNetwork/AuthorisedIncomingAny

| <x>/OnNetwork/AuthorisedIncomingAny |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | Bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to receive a MCVideo private call from any MCVideo user. |  |  |  |  |

When set to "true" the MCVideo user is authorised to receive an MCVideo private call by any MCVideo user.

When set to "false" the MCVideo user is not authorised to receive an MCVideo private call by any MCVideo user, but only from the MCVideo users contained in the IncomingUserList.

### 13.2.87C	/<x>/<x>/OnNetwork/IncomingUserList

Table 13.2.87C.1: /<x>/<x>/OnNetwork/IncomingUserList

| <x>/ OnNetwork/IncomingUserList |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for a list of MCVideo user(s) who are authorised to initiate an MCVideo private call to the configured MCVideo user. |  |  |  |  |

### 13.2.87C1	/<x>/<x>/OnNetwork/IncomingUserList/<x>

Table 13.2.87C1.1: /<x>/<x>/OnNetwork/IncomingUserList/<x>

| <x>/OnNetwork/IncomingUserList/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | OneOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more list of MCVideo users who are authorised to initiate an MCVideo private call to the configured MCVideo user. |  |  |  |  |

### 13.2.87C2	/<x>/<x>/OnNetwork/IncomingUserList/<x>/Entry

Table 13.2.87C2.1: /<x>/<x>/OnNetwork/IncomingUserList/<x>/Entry

| <x>/OnNetwork/IncomingUserList/<x>/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more MCVideo users who are authorised to initiate an MCVideo private call to the configured MCVideo user. |  |  |  |  |

### 13.2.87C3	/<x>/<x>/OnNetwork/IncomingUserList/<x>/Entry/MCVideoID

Table 13.2.87C3.1: /<x>/<x>/OnNetwork/IncomingUserList/<x>/Entry/MCVideoID

| <x>/OnNetwork/IncomingUserList/<x>/Entry/MCVideoID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates an MCVideo user identity (MCVideo ID) which is a globally unique identifier within the MCVideo service that represents the MCVideo user. |  |  |  |  |

The value is a "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 [4].

### 13.2.87C4		/<x>/<x>/OnNetwork/IncomingUserList/<x>/Entry/MCVideoIDKMSURI

Table 13.2.87C4.1: /<x>/<x>/OnNetwork/IncomingUserList/<x>/Entry/MCVideoIDKMSURI

| <x>/OnNetwork/IncomingUserList/<x>/Entry/MCVideoIDKMSURI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates the identity (URI) of the KMS associated with the MCVideoID. |  |  |  |  |

The value is a URI as specified in 3GPP TS 23.003 [5].