3GPP TSG-CT WG1 Meeting #128-e	C1-21xxxx

- Electronic meeting; 25 February – 5 March 2021									rev of C1-210635

Source:	OPPO, Ericsson, Huawei, HiSilicon, TNO

Title:	KI#2, Update: Regulatory requirements and PLMN selection

Spec:	3GPP TR 24.821 v0.3.0

Agenda item:	17.2.4

Document for:	Agreement

1. Introduction

SA3-LI's TS 33.126, requires that the target or other users of a PLMN should not be aware or have knowledge of LI being performed, before, during or after LI has been performed on the target or other users. SA3-LI (in S3i200056) also requires enforceing the use of a Core Network of PLMN in the country where the UE is physically located.

2. Reason for Change

Given that a UE should not know about a PLMN's lawful interception requirements, it is wrong then that CT1 should be studying how in UE's PLMN selection procedure, an input to such selection is a PLMN's support of LI requirements. However, CT1 can study how to optimse PLMN selction such the UE is not getting too many rejects due to NW enforcing the use of a Core Network of PLMN in the country where the UE is physically located.

3. Conclusions

LI requirements should not play any part in UE's PLMN selection.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 24.821 v0.3.0.

* * * First Change * * * *

## 5.2	Key issue #2: Use of core network of PLMN in country where UE is located

In [8], SA3-LI has established the requirement to "enforce the use of a Core Network of PLMN in the country where the UE is physically located".

The core network location verification can take place during the registration procedure (by AS or NAS). However, from the protocol efficiency and resource utilization efficiency point of view, CT1 can study if there are ways to minimize the possibility of registration rejection due to selection onto PLMNs whose CNs are not in same physical country as the UE.

The following questions are expected to be studied within this key issue:

-	how the UE determines which PLMN(s) can be selected in determined country of its physical location (if it is determined that the UE is not in an international area);

-	what information could be available to the UE to help in selecting, in a given country, a PLMN whose CN is physically located in the same country as the UE

▪	pre-configured in the UE; and

▪	provided by PLMN; and

The regulatory requirement mentioned in [8] cannot apply to PLMN selection in international areas (e.g. international waters and airspace) as international areas are not under the jurisdiction of any country, see definition in subclause 3.1. Hence this key issue is not relevant for international areas.

This key issue is related to Key issue #1.

* * * End of Changes * * * *