3GPP TSG-CT WG1 Meeting #129-e	C1-212111

- Electronic meeting; 19 April – 23 April 2021

Source:	Lenovo, Motorola Mobility

Title:	Modification to Solution #57

Spec:	3GPP TR 24.811

Agenda item:	17.2.9

Document for:	Agreement

1. Introduction

In the last meeting, some question regarding the charging was raised and this pCR expanding on the charging aspect.

2. Reason for Change

The change expanding on the current text to address the charging issues and add missing steps in the registration procedures.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 24.811.

* * * First Change * * * *

## 6.57	Solution #57: Registration to PLMN providing Disaster Roaming service and to PLMN with Disaster Condition when no roaming interfaces are in place

### 6.57.1	Description

#### 6.57.1.1	Introduction

This solution is Key Issue #4 "Registration to the roaming PLMN without Disaster Condition in case of Disaster Condition".

This solution targets the scenario where there are no roaming interfaces in place between the PLMN without Disaster Condition (PLMN A) and the PLMN with Disaster Condition (PLMN D).

The following are the main features of the solution:

-	PLMN A is aware that disaster happened in PLMN D and configured to accept registration requests from disaster inbound roamers from PLMN D without primary network authentication.

-	PLMN D allows connectivity service to the disaster inbound roamers only to restricted destinations, e.g. PLMN D (or an N3IWF in PLMN D).

-	The UE is informed correspondingly, so that the UE initiates registration with PLMN D via the N3IWF.

-	The SMF/UPF may monitor the UP traffic to determine whether the registration with PLMN D is successful. If it is determined that the registration is not successful, the AMF initiates UE deregistration procedure.

#### 6.57.1.2	Detailed solution description

Figure 6.57.1.2-1: UE registering with PLMN D via N3IWF using connectivity provided from PLMN A

The steps of the solution are as follows:

1.	The UE applies the network selection procedure for disaster condition, e.g. as described in other Key Issues. As a result, the UE selects to camp on a cell from PLMN A.

2.	The UE initiates registration procedure with PLMN A. The UE indicates in the RRC signalling or in the NAS REGISTRATION REQUEST message that the registration is due to disaster conditions. The NAS REGISTRATION REQUEST message includes the UE's subscription concealed identifier (SUCI), which is based on the HPLMN SUPI and contains the identity of PLMN D. If PLMN D is not the HPLMN, the UE includes the identity of PLMN D in the message. The NAS REGISTRATION REQUEST message may also include an indication that the UE is capable of authentication for local registration, since the UE is not aware if PLMN A has a roaming agreetment with HPLMN and the primary authentication can be performed.

3.	The AMF determines that the registration is by the UE from PLMN D and is due to disaster conditions in PLMN D. Since the AMF cannot communicate with PLMN D due to missing roaming agreements, the AMF determines to apply:

a)	a registration without performing the primary authentication procedure PLMN A applies unauthenticated registration which is similar to Emergency registration with no USIM. The AMF may request the PEI of the UE; or

b)	a local registration, which requires to authenticate the UE by performing a local authentication based on credentials locally stored in the UE and PLMN A. The AMF continues with the registration procedure and perform a NAS security mode command (SMC) procedure and the UE transmits, in a secure way, its subscriber permanent identifier (SUPI) to the AMF, where it is stored to be used for signalling exchange with the SMF for e.g. charging purposes.

Editor's note:	The end-to-end details of security protocol for the NAS message exchange during the registration is to be determined by SA3. Potential security risks resulting from accepting registration requests from the disaster inbound roamers from PLMN D without primary network authentication are to be checked by SA3.

4.	The AMF sends REGISTRATION ACCEPT message to the UE including an allowed NSSAI having a single S-NSSAI identifying a network slice that provides disaster condition services in PLMN A. The AMF indicates also to the UE that only a single PDU session is allowed which can be used for communication with destination addresses.

NOTE 1:	The UE is supposed to establish PDU session with an N3IWF in PLMN D.

5.	The UE sends a UL NAS TRANSPORT message containing a PDU SESSION ESTABLISHMENT REQUEST message to the AMF. The AMF selects an SMF (e.g. by applying the disaster configuration data) and sends an Nsmf_PDUSession_CreateSMContext request to SMF requesting to establish a PDU session for the UE. The AMF may indicate that the requested PDU session is due to disaster condition and also to include PLMN A ID from the SUCI from step 2.

6.	Based on the local configuration data for disaster in the SMF and the additional indication from the AMF:

a)	The SMF decides to apply restricted data connectivity for the requested PDU session so that communication with destination addresses in PLMN D is only supported. The SMF restricts the bit rate of the PDU sessions and configures the UPF to perform deep packet inspection (DPI) to determine if the UE has successfully registered to PLMN D via the N3IWF.

b)	If the local registration is performed so the AMF stores the UE's SUPI and also transmits to the SMF, the SMF uses the SUPI as a key in the signalling exchange with the charging function (CHF) and to generate charging data records (CDRs) for this UE.

Editor's note:	SA5 needs to be consulted for the whether charging based on PEI or SUCI can be performed in PLMN A.

7.	The SMF sends a PDU SESSION ESTABLISHMENT ACCEPT message to the UE. This message indicates that data connectivity over the PDU session is limited to destination address of the N3IWF in PLMN D. The UE triggers registration to PLMN D via the N3IWF.

8. – 9.	The UE performs 5G registration via untrusted non-3GPP access via the N3IWF with PLMN D.

10.	The UPF determines whether the UE has successfully registered to PLMN D via the N3IWF, e.g. by performing deep-packet inspection (DPI) and analysing the traffic exchange via the PDU session. The UPF informs the SMF with the result and the SMF forwards this information to the AMF. The SMF decides to increase the bit rate of the PDU session.

11.	The AMF verifies whether the UE has successfully registered with PLMN D or the UE has registered to PLMN A in an attempt to misuse PLMN A. In the latter case, the AMF initiates deregistration procedure.

The SMF in PLMN A may collect offline charging data for the disaster inbound roamers using the PLMN D ID as key. PLMN D may also collect charging data for the UE.

NOTE 2:	Charging record is collected for all inbound disaster roamers as a group and not individually.

6.57.2	Impacts on existing nodes and functionality

Impacts to AMF:

-	Allowing unauthenticated registration for restricted service for disaster condition; and

-	initiating deregistration if the UE does not successfully register with PLMN D.

Impacts to SMF:

-	Establishing a PDU session for restricted connectivity to PLMN D; and

-	configuring the UPF to monitor if the UE misbehaves.

Impacts to CHF:

-	Collecting the charging information in PLMN A if the UE is not authenticated.

Impacts to UE:

-	After receiving an indication that the registration and PDU sessions are limited to destination to PLMN D, the UE triggers registration to PLMN D via the N3IWF.

* * * Next Change * * * *

<Proposed change in revision marks>

* * * Next Change * * * *

<Proposed change in revision marks>