3GPP TSG-CT WG1 Meeting #133e-bis	C1-220412_r2

- E-meeting, 17-21 January 2022

Source:	Huawei, HiSilicon

Title:	Structure for MSGin5G UE configuration

Spec:	3GPP TS 24.538 v0.2.1

Agenda item:	17.2.30

Document for:	Agreement

1. Introduction

This pCR is to propose the JSON Schema used in the MSGin5G UE configuration request.

2. Reason for Change

JSON format has been approved to be used in the MSGin5G UE configuration request, so the JSON Schema is necessary to be included.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.538 v0.2.1.

* * * First Change * * * *

#### 6.2.1.2	Procedure at MSGin5G Client

The MSGin5G UE should support the Configuration management client functionality as specified in 3GPP TS 23.546 [6] in or outside the MSGin5G Client. The configuration management client may be collocated with MSGin5G Client or it can be separate as per 3GPP TS 23.554 [2].

If the Configuration management client function is not collocated with the MSGin5G Client, the MSGin5G Client should use SEAL-C interface to interact with Configuration management client function for MSGin5G UE configuration.

The MSGin5G UE configuration procedures at the Configuration management client function is based on the procedures in clause 6.2.3.1 of 3GPP TS 24.546 [6], in the procedures:

a)	the Configuration management client function on the MSGin5G UE acts as SCM-C;

b)	the Configuration management server function at the server-side acts as SCM-S;

c)	in the Request-URI, the "XCAP Root" is the URI of Configuration management server function;

d)	in the Request-URI, the "auid" is the unique service identifier of MSGin5G service; and

e)	the parameters serialized into a JavaScript Object Notation (JSON) structure includes:

1)	MSGin5G UE ID as specified in 3GPP TS 23.554 [2]; and

2)	optionally, other related information (e.g. device type, device vendor, etc).

The corresponding JSON Schema used in step e) is defined in 7.3.Y.

Upon receiving the requested MSGin5G UE configuration data, the Configuration management client function shall submit the configuration data to MSGin5G Client by SEAL-C interface. The MSGin5G Client shall store the configuration data, including MSGin5G UE Service ID, the address of MSGin5G Server and other available MSGin5G Service specific informations.

* * * Next Change * * * *

## 7.3.Y	MSGin5G UE Configuration structure

The schema is based on JSON Schema Draft-07. For reducing the overhead of the message used in MSGin5G service, the properties are defined as shorten form and the relationship between the properties and IEs used in clause 6.2 are described in the description of the properties, The JSON schema is defined below:

{

"$schema": "http://json-schema.org/draft-07/schema#",

"$id": "http://www.3gpp.org/MSGin5G/MSGin5G_UE_Configuration_schema",

"title": "MSGin5G_UE_Configuration",

"type": "object",

"properties": {

"ueId": {

"type": "string",

"description": "Refer to MSGin5G UE ID"

},

"addInfos": {

"type": "array",

"description": "Refer to other related informations",

"items": {

"$ref": "#/$defs/AddInfo"

}

}

},

"required": ["ueId"],

"$defs": {

"AddInfo": {

"type": "object",

"properties": {

"name": {

"type": "string"

},

"value": {

"type": "string"

}

},

"required": ["name", "value"]

}

}

}

* * * Next Change * * * *

<Proposed change in revision marks>