3GPP TSG-CT WG1 Meeting #133-e	C1-216911

- E-meeting, 11-19 November 2021

Source:	Huawei, HiSilicon

Title:	General description on message topic subscription and unsubscription

Spec:	3GPP TS 24.538 v0.1.0

Agenda item:	17.2.30

Document for:	Agreement

1. Introduction

This pCR is to propose the description to declare that message topic subscription and unsubscription is based on the CoAP Observe option.

2. Reason for Change

As specified in TS 23.554, the MSGin5G UE may subscribe message topics.

If a UE subscribe a message topic, it should be possible for the UE to unsubcribe the message topic.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.538 v0.1.0.

* * * First Change * * * *

## 6.6	Messaging Topic Subscription

This clause covers the procedures in MSGin5G-1.

As specified in 3GPP TS 23.554 [2], an MSGin5G Client may subscribe one or more Messaging Topic(s) on the MSGin5G Server.

The message topic subscription is based on the CoAP Observe method as specified in IETF RFC 7641 [x1], the MSGin5G Client acts as an observer, the MSGin5G Server acts as a CoAP Server, the message topic is a resource to observe.

* * * Next Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.554: "Application architecture for MSGin5G Service; Stage 2;".

[3]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals".

[x1] 	IETF RFC 7641: "Observing Resources in the Constrained Application Protocol (CoAP)".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".