3GPP TSG-CT WG1 Meeting #133-e	C1-216912_revdraft1

- E-meeting, 11-19 November 2021

Source:	Huawei, HiSilicon

Title:	Message topic subscription procedures at MSGin5G Client

Spec:	3GPP TS 24.538 v0.1.0

Agenda item:	17.2.30

Document for:	Agreement

1. Introduction

This pCR is to propose the procedures on MSGin5G Client for message topic subscription.

2. Reason for Change

As specified in TS 23.554, the MSGin5G UE may subscribe message topics.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.538 v0.1.0.

* * * First Change * * * *

### 6.6.1	Messaging Topic Subscription Procedure at MSGin5G Client

Upon receiving a request to subscribe a messsage topic from an Application Client, MSGin5G Client shall send a CoAP GET request, as specified in IETF RFC 7641 [x1], to the MSGin5G Server. In the CoAP GET request, the MSGin5G Client:

a)	shall set the "T" field in the CoAP header to the value "0" to indicate this request is the type of Confirmable;

b)	shall include the MSGin5G Server host address in the Uri-Host Option;

c)	shall include the MSGin5G Server port number in the Uri-Port Option;

d)	shall include the message topic name in the Uri-Path Option (e.g. "\top");

e)	shall include the Observe Option with the value "0" which indicates the request is for observing a resource, i.e. for subscribing a message topic;

f)	shall include the Content-Format Option with the value "50" which indicate the format of the CoAP payload is "application/json" as specified in RFC 7252 [x2]; and

g)	shall include the CoAP Payload in JSON format, including the following information elements as specified in clause 8.8.1 of 3GPP TS 23.554 [2]:

1)	a "Originating UE Service ID" element set to the MSGin5G UE which requests the message topic subscription; and

2)	optionally, a "Expiration time" element which indicates the expiration time of the message topic subscription.

* * * Next Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.554: "Application architecture for MSGin5G Service; Stage 2;".

[3]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals".

[x1] 	IETF RFC 7641: "Observing Resources in the Constrained Application Protocol (CoAP)".

[x2] 	IETF RFC 7252: "The Constrained Application Protocol (CoAP)".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".