3GPP TSG-CT WG1 Meeting #133-e	C1-216918_revdraft1

- E-meeting, 11-19 November 2021

Source:	Huawei, HiSilicon

Title:	Message topic subscription and unscubscription procedures at MSGin5G Server

Spec:	3GPP TS 24.538 v0.1.0

Agenda item:	17.2.30

Document for:	Agreement

1. Introduction

This pCR is to propose the message topic subscription procedures on MSGin5G Server.

2. Reason for Change

As specified in TS 23.554, the MSGin5G UE may subscribe message topics.

If a UE subscribe a message topic, it should be possible for the UE to unsubcribe the message topic. The unsubscription procedures will be proposed after it is addressed in TS 23.554.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.538 v0.1.0.

* * * First Change * * * *

### 6.6.2	Procedure at MSGin5G Server

The MSGin5G Server should support parsing CoAP request as specified in RFC 7252 [x2] and RFC 7641 [x1].

Upon receiving a CoAP GET request from MSGin5G Client, the MSGin5G Server shall parse the CoAP headers, Options and Payload in the request to get:

a)	the value of Observe Option;

b)	the message topic from the Uri-Path Option;

c)	get the Originating UE Service ID from the Payload; and

d)	get the Expiration time from the Payload if exists in the Payload.

If the Observe Option is included in the CoAP GET request with a value "0" as specified in RFC 7461 [x1], the MSGin5G Server shall:

a)	if the message topic does not exist, create the message topic;

b)	if the Originating UE Service ID is not in the list of the subscribers of the message topic, add the Originating UE Service ID to the list of the subscribers of the topic, and recore its expiration time if exists;

c)	if an entry with a matching the Originating UE Service ID is already present in the list of the subscribers of the message topic, update the expiration time of the subscription of this UE;

d)	send a CoAP Notifications with a 2.05 (Content) response code to the MSGin5G Client; and

The MSGin5G Server shall remove the Originating UE Service ID from list of the subscribers of the message topic when the expiration time reached;

* * * Next Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.554: "Application architecture for MSGin5G Service; Stage 2;".

[3]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals".

[x1] 	IETF RFC 7641: "Observing Resources in the Constrained Application Protocol (CoAP)".

[x2] 	IETF RFC 7252: "The Constrained Application Protocol (CoAP)".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".