3GPP TSG-CT WG1 Meeting #133-e	C1-216772

- E-meeting, 11-19 November 2021

Title:	LS on the User Controlled PLMN Selector with Access Technology in Control plane solution for steering of roaming in 5GS

Response to:	-

Release:	Release 17

Work Item:	5GProtoc17

Source:	CT1

To:	SA1

Cc:	SA3

Contact person:

Name:	Lufeng Han

E-mail Address:	lufeng.han@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

SOR requirements in TS 22.011 states the following:

"If the UE is registered on a VPLMN that is present on the User Controlled PLMN List, the steering request shall be ignored. PLMNs contained on the User Controlled PLMN List shall have priority over the steered-to-PLMN."

Control plane solution for steering of roaming in 5GS was introduced in Rel-15 with a UE security check of the received SOR-information by the UE to help the UE to get the knowledge of anti-SOR performed by the VPLMN transferring the SOR information from the HPLMN to the UE. A successful security check leads to the UE sending an ACK to the HPLMN, if requested by the HPLMN,  to inform the HPLMN of the successful security check of the received SOR information by the UE.

CT1 is discussing the use case when the selected PLMN is a VPLMN which is in the "User Controlled PLMN Selector with Access Technology" list and the UE in automatic network selection mode receives SOR information, and the security check of the received SOR information fails in the UE.

CT1 would like to ask SA1, for the use case above, whether the VPLMN still has higher priority than the steered-to-PLMN or whether, given the failed security check, the VPLMN should be considered as lowest priority and the UE needs to obtain normal service in another available and allowable VPLMN as soon as possible.

Question:

CT1 kindly asks SA1 whether, in the use case above, the VPLMN still has higher priority than the steered-to-PLMN or the VPLMN should  be considered as lowest priority ?

# 2	Actions

To 3GPP SA1

ACTION: 	CT1 kindly asks SA1 to answer the questions above, and to update its specifications if deemed necessary.

# 3	Dates of next TSG CT WG1 meetings

CT1#133-bis-e	17th-21st January 2022	electronic meeting

CT1#134e	17th-25th February 2022	electronic meeting