3GPP TSG-CT WG1 Meeting #134-e	C1-22xxxx

- E-Meeting, 17th – 25th February 2022										Revision of C1-221421

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 4057 | rev | 1 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update the contents of an entry in the PLMN List for #78 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_ARCH-CT |  |  |  |  |  | Date: |  |  | 2022-02-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) ... Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | TS 24.501 subclause 4.23.2 defines a List of "PLMNs not allowed to operate at the present UE location" for NR satellite access, and also specifies its usage. For 3GPP satellite NG-RAN the UE shall store a list of "PLMNs not allowed to operate at the present UE location". Each entry consists of: a) the PLMN identity of the PLMN which sent a message including 5GMM cause value #78 "PLMN not allowed to operate at the present UE location" via satellite NG-RAN access technology; b) the geographical location, if known by the UE, where 5GMM cause value #78 was received on satellite NG-RAN access technology; and c) a UE implementation specific timer value which shall not be set to a value smaller than the value indicated by the network, if any. …… The UE shall not attempt to access a PLMN via satellite NG-RAN access technology which is part of the list of "PLMNs not allowed to operate at the present UE location" if: a) the current UE location is known, a geographical location is stored for the entry of this PLMN, and the distance to the current UE location is smaller than a UE implementation specific value. This UE implementation specific value shall not be set to a value smaller than the value indicated by the network, if any; or Editor's note: A minimum value can be optionally provided by the network in the same message as cause value #78, but IE naming and definition is FFS. b) the timer associated with the entry of this PLMN is running. Each entry contains a UE implementation specific timer value, but a UE implementation specific distance value is missing. If the UE is rejected by several PLMNs, all those PLMNs provide a reference value and different PLMNs provide different values, the UE needs to record the values within corresponding entries. It is suggested to add a UE implementation specific distance value to an entry with a geographical location in this PLMN List. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a UE implementation specific distance value to an entry with a geographical location in the List of "PLMNs not allowed to operate at the present UE location" for NR satellite access. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It cannot apply to the case that each PLMN provides a reference value and different PLMNs provide different values and the case that different areas or continents could have different values. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.23.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Update the description to separate the “distance/timer”and the related manatory requirments. |  |  |  |  |  |  |  |  |

***************************** NEXT CHANGE *************************************

### 4.23.2	List of "PLMNs not allowed to operate at the present UE location"

For 3GPP satellite NG-RAN the UE shall store a list of "PLMNs not allowed to operate at the present UE location". Each entry consists of:

a)	the PLMN identity of the PLMN which sent a message including 5GMM cause value #78 "PLMN not allowed to operate at the present UE location" via satellite NG-RAN access technology;

b)	the geographical location, if known by the UE, where 5GMM cause value #78 was received on satellite NG-RAN access technology;

c)	if the geographical location exists, a UE implementation specific distance value. The UE implementation specific distance value shall not be set to a value smaller than the value indicated by the network, if any; and

d)	a UE implementation specific timer value. The UE implementation specific timer value shall not be set to a value smaller than the value indicated by the network, if any.

Editor's note:	A minimum value can be optionally provided by the network in the same message as cause value #78, but IE naming and definition is FFS.

Before storing a new entry in the list, the UE shall delete any existing entry with the same PLMN identity.

The UE shall not attempt to access a PLMN via satellite NG-RAN access technology which is part of the list of "PLMNs not allowed to operate at the present UE location" if:

a)	the current UE location is known, a geographical location is stored for the entry of this PLMN, and the distance to the current UE location is smaller than a UE implementation specific value; or

Editor's note:	A minimum value can be optionally provided by the network in the same message as cause value #78, but IE naming and definition is FFS.

b)	the timer associated with the entry of this PLMN is running.

The list shall accommodate three or more entries. When the list is full and a new entry has to be inserted, the oldest entry shall be deleted.

Each entry shall be removed if for the entry:

a)	the UE successfully registers to the PLMN stored in the entry; or

b)	the timer instance associated with the entry expires.

When the UE is switched off, the UE shall keep the list of "PLMNs not allowed to operate at the present UE location" in its non-volatile memory. The UE shall delete the list of "PLMNs not allowed to operate at the present UE location" if the USIM is removed.

***************************** END of CHANGE ***************************