3GPP TSG-CT WG1 Meeting #134-e	C1-221822

# E-Meeting, 17th – 25th February 2022

# Title:	LS on Mapped NSSAI

# Release:	Rel-17

# Work Item:	5GProtoc17

Source:	CT1

To:	SA2

Cc:	---

Contact Person:

Name:	Robert Zaus

E-mail Address:	rzaus at apple dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	---

1. Overall Description:

CT1 has discussed UE requirements regarding the NSSAI mapping during the transfer of a PDU session between HPLMN and VPLMN, or upon a change of the Allowed NSSAI in the Serving PLMN. Specifically, CT1 has been discussing the following scenario:

- UE is registered in the HPLMN and receives Allowed NSSAI containing non-standard S-NSSAI_1.

- UE establishes PDU session in the HPLMN using S-NSSAI_1

- UE moves to a VPLMN, which provides Allowed S-NSSAI containing non-standard S-NSSAI_2 and no corresponding mapped S-NSSAI.

CT1 has been discussing the validity of the above scenario, in particular step 3, and the corresponding UE behaviour in relation to the PDU session.

In several places in TS 23.501, it is indicated that the mapping to HPLMN S-NSSAIs is optional. See, e.g., in clause 5.15.2.1:

The optional mapping of Serving PLMN S-NSSAIs to HPLMN S-NSSAIs contains Serving PLMN S-NSSAI values and corresponding mapped HPLMN S-NSSAI values.

CT1 understands that this optionality certainly applies to the non-roaming case. i.e., when the Serving PLMN is the HPLMN, then as a rule it will not provide a mapping because, by definition, the mapped S-NSSAI is the S-NSSAI in the HPLMN. When it comes to roaming, current CT1 specs are based on the assumption that step 3 above is valid, i.e. the optionality also applies to the roaming case, e.g. LBO case where VPLMN may locally deploy a slice not subject to SLA with the HPLMN.

To make sure that CT1 specs are fully in line with the stage 2 intention, CT1 would like to ask SA2 to clarify the following:

Question 1: Could SA2 confirm that it is indeed optional for VPLMN to provide the mapped S-NSSAI for a non-standard S-NSSAI in the VPLMN?

Question 2: If the answer to question 1 is yes, could SA2 further clarify if it is mandatory on the network to provide the mapped S-NSSAI to the UE if the mapped S-NSSAI is known to the network? ?

Question 3: If the answer to question 2 is no, could SA2 further clarify how the UE determines the mapped S-NSSAI in that case?

2. Actions:

To SA2 group.

ACTION: 	CT1 kindly asks SA2 to answer CT1's questions.

3. Date of Next CT4 Meetings:

CT1#135e	6th - 12th April 2022	electronic meeting

CT1#136e	12th - 20th May 2022	electronic meeting