3GPP TSG-CT WG1 Meeting #136-e	C1-224044

# E-Meeting, 12th – 20th May 2022

(revision of C1-223496)

3GPP TSG-CT WG4 Meeting #110-e	C4-223396

# E-Meeting, 12th – 20th May 2022

(revision of C4-223203)

# Source:	China Mobile, Nokia, Nokia Shanghai Bell, China Southern Power Grid Co.

# Title:	New WID on CT aspects of enhancement of RAN Slicing for NR

# Document for:	Approval

Agenda Item:	17.1.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: 	CT aspects of enhancement of RAN Slicing for NR

Acronym: NRslice

Unique identifier: 	TBD

Potential target Release: Rel-17

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No |  |  | X |  |  |
| Don't know | X |  |  |  | X |

2	Classification of the Work Item and linked work items

2.1	Primary classification

|  | Feature |
| --- | --- |
| X | Building Block |
|  | Work Task |
|  | Study Item |

2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| NR_Slice-Core | RAN2 | 911107 | Core part: Enhancement of RAN slicing for NR |

2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 860039 | Study on enhancement of RAN slicing for NR | RAN work item |

3	Justification

RAN2 carried out a Rel-17 work item on the enhancement of RAN Slicing for NR. And the LSs from RAN2 to CT1 (R2-2108928 and R2-2203807) pointed out the solution agreed in RAN2 for the normative phase has impacts on protocols and interfaces under CT WGs' responsibilities. Furthermore, the LS from SA2 (S2-2203597) and the attached agreed CRs on normative stage 2 work stated the requirements to enhance the NAS protocol.

This WID aims at carrying out the related work in CT WGs to complete the feature end to end.

4	Objective

The objective of the work item is to develop the specifications according to RAN Slicing for NR requirements from RAN WGs work item NR_Slice-Core. CT WGs will support and align to normative stage 2 updates in the technical specifications under SA2 responsibility for “RAN Slicing for NR” introduced under corresponding RAN WI code. Stage 3 work on the enhancement of the NAS protocol and the NF services shall be started only after the applicable normative stage 2 work is available. The following areas of work are expected to be covered (non-exhaustive):

CT1

Enhancement of the NAS to support signalling of network slice AS group information.

CT4

- Enhancement of NSSF services to support the network slice AS group management.

- Potential definition of common date type to support the feature in NR_Slice_Core.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 24.501 | Enhancement of the NAS to support signalling of network slice AS group information. | TSG#96 (June 2022) | CT1 |
| 29.531 | Enhancement of NSSF services to support the network slice AS group management. | TSG#96 (June 2022) | CT4 |
| 29.571 | Potential definition of common date type to support the feature in NR_Slice_Core. | TSG#96 (June 2022) | CT4 |

6	Work item Rapporteur(s)

Chen Xu, China Mobile, chenxu@chinamobile.com

7	Work item leadership

CT1

8	Aspects that involve other WGs

9	Supporting Individual Members

Supporting IM name

China Mobile

ZTE

Huawei

HiSilicon

vivo

CATT

China Telecom

OPPO

Xiaomi

Ericsson

China Unicom

Nokia

Nokia Shanghai Bell

China Southern Power Grid Co.

LG Electronics