3GPP TSG-CT WG1 Meeting #136-e	C1-223686

# E-Meeting, 12th – 20th May 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 27.007 | CR | 0781 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of RV QoE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Huawei, HiSilicon, Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17, NR_QoE-Core |  |  |  |  |  | Date: |  |  | 2022-04-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As indicated in LS in C1-223311, RAN2 has specified further requirements for RAN Visible QoE measurements. This requires the recently specified AT commands CAPPLEVMCNR and CAPPLEVMRNR to be extended with parameters as explained in the LS. <app-meas_service_type> was added to the description of CAPPLEVMCNR, but not in the description of the unsolicited result code. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added configuration and reporting related parameters as requested in the LS. <app-meas_service_type> added to CAPPLEVMCNR, in the description of the unsolicited result code. <meas_config_app_layer_id> is made mandatory in CAPPLEVMRNR |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RAN Visible QoE measurement will not be usable as the last link in the chain of the information flow is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.84, 8.85 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev-1: Added number of entries in lists Moved <meas_config_app_layer_id> |  |  |  |  |  |  |  |  |

*******

* * * First Change * * * *

## 8.84	Application level measurement configuration for NR +CAPPLEVMCNR

Table 8.84-1: +CAPPLEVMCNR parameter command syntax

| Command | Possible response(s) |
| --- | --- |
| +CAPPLEVMCNR=[<n>] | +CME ERROR: <err> |
| +CAPPLEVMCNR? | +CAPPLEVMCNR: <n> |
| +CAPPLEVMCNR=? | +CAPPLEVMCNR: (list of supported <n>s) |

Description

This command allows control of the application level measurement configuration according to 3GPP TS 38.331 [160]. The set command controls the presentation of the unsolicited result code +CAPPLEVMCNR: (list of [<CR><LF>],<meas_config_app_layer_id>,[<start-stop_measurement>,[<ran_visible_release_only>]],[<app-meas_config_file_length>,<app-meas_config-file>],[<transmission_of_session_start-end>],[<ran_visible_periodicity>],[<number_of_buffer_level_entries>],[<report_initial_playout_delay>],[<app-meas_service_type>]) providing data for the configuration. Refer clause 9.2 for possible <err> values.

Read command returns the current value of <n>.

Test command returns values supported as a compound value.

Defined values

<n>: integer type. Disable and enable presentation of the unsolicited result code +CAPPLEVMCNR to the TE.

0	Disable presentation of the unsolicited result code

1	Enable presentation of the unsolicited result code

<app-meas_service_type>: integer type. Contains the indication of what application that is target for the application level measurement configuration.

1	QoE measurement collection for streaming services

2	QoE measurement collection for MTSI services

3	QoE measurement collection for VR services

<start-stop_measurement>: integer type. Indicates the start and stop of the application level measurement reporting for the application indicated by the <app-meas_service_type>.

0	start the application level measurement

1	stop the application level measurement and release the application level measurement configuration

<app-meas_config_file_length>: integer type. Indicates the number of octets of the <app-meas_config-file> parameter.

<app-meas_config-file>: string of octets. Contains the application level measurement configuration file for the application indicated by the <app-meas_service_type>. The parameter shall not be subject to conventional character conversion as per +CSCS.

<meas_config_app_layer_id>: integer type. At QoE measurement configuration the <meas_config_app_layer_id> indicates an identity for the QoE  measurement configuration received in the <app-meas_config-file>. At QoE measurement configuration release, the <meas_config_app_layer_id> indicates the measurement to be released. The absence of this parameter indicates that all measurement configurations are released.

<transmission_of_session_start-end>: integer  type. Contains an indication of whether  session start-end is required.

0	Not required

1	Required

<ran_visible_periodicity>: integer  type.

0	120 ms

1	240 ms

2	480 ms

3	640 ms

4	1024 ms

<number_of_buffer_level_entries>: integer  type. Contains the number of buffer level entries.

1-8

<report_initial_playout_delay>: integer  type. Contains an indication of whether  report of initial playout delay is required.

0	Report of initial playout delay is not required

1	Report of initial playout delay is required

<ran_visible_release_only>: integer  type. Contains an indication of whether  all application level measurements for this <meas_config_app_layer_id> should be released or if only the RAN visible application level measurements should be released.

0	Release the RAN visible application level measurements for this <meas_config_app_layer_id>

1	Release all application level measurements for this <meas_config_app_layer_id>

Implementation

Optional.

## 8.85	Application level measurement report for NR +CAPPLEVMRNR

Table 8.85-1: +CAPPLEVMR action command syntax

| Command | Possible response(s) |
| --- | --- |
| +CAPPLEVMR=(list of [<CR><LF><app-meas_report_length>,<app-meas_report>],[<number-of-pdu-session_id-entries>,list of <pdu-session_id>],[<number_of_buffer_level_entries>,list of <application_layer_buffer-level>],[<qoe_measurement_status>],[<playout_delay_for_media_startup>],<meas_config_app_layer_id>) | +CME ERROR: <err> |
| +CAPPLEVMR=? |  |

Description

This command allows the MT to provide a list of application level measurement reports according to 3GPP TS 38.331 [160]. Refer clause 9.2 for possible <err> values.

Defined values

<qoe_measurement_status>: Indicates whether a session has started or ended.

0	started

1	ended

<app-meas_report_length>: integer type. Indicates the number of octets of the <app-meas_report> parameter.

<app-meas_report>: string of octets. Contains the application level measurement report for the application indicated by the <app-meas_service_type>. The parameter shall not be subject to conventional character conversion as per +CSCS.

<meas_config_app_layer_id>: integer type. Identifies the QoE measurement configuration associated with the report in the <app-meas_report> parameter.

<playout_delay_for_media_startup>: integer type. Indicates the application layer initial playout delay in ms.

0-30000

<number_of_buffer_level_entries>: integer  type. Contains the number of <application_layer_buffer-level> entries.

1-8

<application_layer_buffer-level>: integer type. Indicates the application layer buffer level in ms in steps of 10 ms.

0-30000

<number-of-pdu-session_id-entries>: integer type: Indicates the number of entries in the list of <pdu-session_id>

<pdu-session_id>: integer type. Identifies a PDU session ID.

0-255

Implementation

Optional.

* * * End of Changes * * * *