3GPP TSG-CT WG1 Meeting #136-e	C1-223847

# E-meeting, 12th -20th May 2022																(was_3124_2932)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 4238 | rev | 2 | Current version: | 17.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the confliction between the NSSRG information IE and the Configured NSSAI IE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph2 |  |  |  |  |  | Date: |  |  | 2022-04-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) ... Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The NSSRG information IE format has been fixed during CT1#134e, which contains the value of configured S-NSSAI(s). However, how to handle the issue in the UE has not been identified when the number or the contents of configured S-NSSAI(s) in the NSSRG information IE is different from that in the Configured NSSAI IE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The network has a responsibility to send the above two IE in the right format. If the mismatch happens, then the NSSRG informationwill be regarded as a syntactically incorrect optional IE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing statements for the conflict between the NSSRG IE and the Configured NSSAI IE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.4.2, 5.5.1.2.4, 5.5.1.3.4, 9.11.3.82 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | CT1#135e: In the above scenario, the UE shall consider the new configured S-NSSAI(s), which is included in both the NSSRG information IE and the configured NSSAI IE, is valid. In addition, the NSSRG value(s) associated with the valid configured S-NSSAI(s) are accepted. CT1#136e: The network has the responsibility to send the above two IE in the right format. If the mismatch happens, then the NSSARG information IE will be regarded as a syntactically incorrect optional IE. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 9.11.3.82	NSSRG information

The purpose of the NSSRG information information element is to identify one or more NSSRG values associated with each of the S-NSSAIs in a configured NSSAI.

The NSSRG information information element is coded as shown in figure 9.11.3.82.1, figure 9.11.3.82.2 and table 9.11.3.82.1.

The NSSRG information is a type 6 information element with minimum length of 7 octets and maximum length of 65538 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NSSRG information IEI |  |  |  |  |  |  |  | octet 1 |
| Length of NSSRG information contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| NSSRG values for S-NSSAI 1 |  |  |  |  |  |  |  | octet 4 octet m |
| NSSRG values for S-NSSAI 2 |  |  |  |  |  |  |  | octet m+1* octet n* |
| … |  |  |  |  |  |  |  | octet n+1* octet u* |
| NSSRG values for S-NSSAI x |  |  |  |  |  |  |  | octet u+1* octet v* |

Figure 9.11.3.82.1: NSSRG information information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of NSSRG values for S-NSSAI |  |  |  |  |  |  |  | octet 4 |
| S-NSSAI value |  |  |  |  |  |  |  | octet 5 octet w |
| NSSRG value 1 for the S-NSSAI |  |  |  |  |  |  |  | octet w+1 |
| NSSRG value 2 for the S-NSSAI |  |  |  |  |  |  |  | octet w+2* |
| … |  |  |  |  |  |  |  | octet w+3* octet m-1* |
| NSSRG value y for the S-NSSAI |  |  |  |  |  |  |  | octet m* |

Figure 9.11.3.82.2: NSSRG values for S-NSSAI

Table 9.11.3.82.1: NSSRG information information element

Value part of the NSSRG information information element (octet 4 to v) The value part of the NSSRG information information element consists of one or more NSSRG values for each S-NSSAI in the Configured NSSAI IE. S-NSSAI value (octet 5 to w) S-NSSAI value is coded as the length and value part of S-NSSAI information element as specified in subclause 9.11.2.8 starting with the second octet. (see NOTE 1)

NSSRG value for the S-NSSAI (octet w+1)

This field contains the 8 bit NSSRG value.

NOTE: The NSSRG information IE shall contain the complete set of (mapped) S-NSSAI(s) included in the configured NSSAI.

* * * End of Changes * * * *