Discussion on NSSRG procedure on pending NSSAI

Related CR

C1-223680: Handling of pending NSSAI in NSSRG procedure

Scenario:

UE is configured with Slice A, B (Subject to NSSAA), C and

NSSRG = {(A, B), (A, C)}

1) Registration request with requested NSSAI A+B.

2) Registration accept (allowed A and pending B).

3) UE gets trigger to additional slice C.

Question:  Shall the UE check whether C share a common NSSRG group with A+B?

Proposal in CR: Since a pending is implicit requested NSSAI then the UE will check whether A+B shall share the common NSSRG or not.

Detailed comparisons are given below between existing specification, solution in 223680 and proposal from Sung.

| Existing Specification All NSSAI of requested NSSAI shall share at least on NSSRG. Pending NSSAI is implicit Requested NSSAI. | Original proposal of C1-223680 Requested and pending NSSAIs shall share at least on NSSRG as the pending NSSAI is implicit requested NSSAI. | Proposed solution by Sung NSSRG restriction doesn’t apply for pending NSSAI. |
| --- | --- | --- |
| A, B and C are not subject to NSSAA. NSSRG = {(A, B), (A, C)} | A, B (NSSAA), C NSSRG = {(A, B), (A, C)} | A, B (NSSAA), C NSSRG = {(A, B), (A, C)} |
| 1) First registration trigger: Requested NSSAI A+B: A and B shall be in same NSSRG group. | 1) First registration trigger: Requested NSSAI A+B: A and B shall be in same NSSRG group. B pending. | 1) First registration trigger: Requested NSSAI A+B: A and B shall be in same NSSRG group. B pending. |
| 2) Second Registration trigger due to additional slice C, The UE checks if A+B+C is allowed or not, in this case A+B+C is not allowed because it doesn’t share common group. The UE doesn’t send registration request with requested NSSAI = A+B+C | 2) Second Registration trigger due to additional slice C, The UE checks if A+B (pending)+C is allowed or not, in this case A+B+C is not allowed because it doesn’t share common group. The UE doesn’t send registration request with requested NSSAI = A+B+C | 2) Second Registration trigger for additional slice C, UE excludes NSSRG check for B in second trigger, only check if A+C belongs to same NSSRG. New requested NSSAI = A+C which share common NSSRG, so it allowed. Why not checking A+B+C?? what is difference between requested NSSAI in the first column side (existing specificaition)? |
| 3) Network doesn’t get requested NSSAI which is out of sync in terms of NSSRG. no need to update. | 3) Network doesn’t get requested NSSAI which is out of sync in terms of NSSRG. no need to update. | 3) when the network receives A+C and NSSAA is completed successfully and B is allowed. then we have three allowed S-NSSAI A+B+C. A+B+C doesn’t belong to same group. What the network is going to do? accept A+C? or A+B? it needs to reject one S-NSSAI what is the cause value it needs to send to the UE for the rejected S-NSSAI? |