3GPP TSG-CT WG1 Meeting #137-e	C1-224888

# E-Meeting, 18th – 26th August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 4566 | rev | - | Current version: | 17.7.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Associate NSSRG values with HPLMN S-NSSAI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph2 |  |  |  |  |  | Date: |  |  | 2022-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The NSSRG values are associated with the HPLMN S-NSSAI. Hence, in non-roaming case, the NSSRG values are associated with each of the S-NSSAIs in a configured NSSAI; in roaming case, the NSSRG values are associated with the mapped S-NSSAI for each of the S-NSSAIs in a configured NSSAI. Current spec only captures the former case, see below: The purpose of the NSSRG information information element is to identify one or more NSSRG values associated with each of the S-NSSAIs in a configured NSSAI. The latter case is also needed to be captured. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Capture the case that the NSSRG values are associated with the mapped S-NSSAI for each of the S-NSSAIs in a configured NSSAI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The latter case above is not captured in the current spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.11.3.82 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*****First change *****

9.11.3.82	NSSRG information

The purpose of the NSSRG information information element is to identify one or more NSSRG values associated with each of the HPLMN S-NSSAIs in a configured NSSAI.

The NSSRG information information element is coded as shown in figure 9.11.3.82.1, figure 9.11.3.82.2 and table 9.11.3.82.1.

The NSSRG information is a type 6 information element with minimum length of 7 octets and maximum length of 65538 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NSSRG information IEI |  |  |  |  |  |  |  | octet 1 |
| Length of NSSRG information contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| NSSRG values for S-NSSAI 1 |  |  |  |  |  |  |  | octet 4 octet m |
| NSSRG values for S-NSSAI 2 |  |  |  |  |  |  |  | octet m+1* octet n* |
| … |  |  |  |  |  |  |  | octet n+1* octet u* |
| NSSRG values for S-NSSAI x |  |  |  |  |  |  |  | octet u+1* octet v* |

Figure 9.11.3.82.1: NSSRG information information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of NSSRG values for S-NSSAI |  |  |  |  |  |  |  | octet 4 |
| S-NSSAI value |  |  |  |  |  |  |  | octet 5 octet w |
| NSSRG value 1 for the S-NSSAI |  |  |  |  |  |  |  | octet w+1 |
| NSSRG value 2 for the S-NSSAI |  |  |  |  |  |  |  | octet w+2* |
| … |  |  |  |  |  |  |  | octet w+3* octet m-1* |
| NSSRG value y for the S-NSSAI |  |  |  |  |  |  |  | octet m* |

Figure 9.11.3.82.2: NSSRG values for S-NSSAI

Table 9.11.3.82.1: NSSRG information information element

Value part of the NSSRG information information element (octet 4 to v) The value part of the NSSRG information information element consists of one or more NSSRG values for each S-NSSAI in the Configured NSSAI IE. S-NSSAI value (octet 5 to w) (see NOTE 2) S-NSSAI value is coded as the length and value part of S-NSSAI information element as specified in subclause 9.11.2.8 starting with the second octet. See NOTE 1.

NSSRG value for the S-NSSAI (octet w+1)

This field contains the 8 bit NSSRG value.

NOTE 1: If a mapped HPLMN SST is included in a S-NSSAI value, then the NSSRG value(s) are associated with the Mapped HPLMN SST, and the Mapped HPLMN SD, if included. NOTE 2: The NSSRG information IE shall contain the complete set of S-NSSAI(s) included in the configured NSSAI.

***** End of changes *****