3GPP TSG-CT WG1 Meeting #138-e	C1-22XXXX

# E-Meeting, 10th – 14th October 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 27.007 | CR | 0794 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to NR QoE AT commands |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo, Ericsson, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17, NR_QoE-Core |  |  |  |  |  | Date: |  |  | 2022-10-10 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - +CAPPLEVMCNR: The definition of the codepoint <ran_visible_release_only> is not aligned with the RAN3 agreements made wrt the release of encapsulated QoE and RVQoE measurement configurations. RAN3 agreed the following cases: Case 1: If the gNB releases an encapsulated QoE measurement configuration, the associated RVQoE measurement configuration is released as well. Case 2: The gNB can release a list of RVQoE measurement configurations while not releasing the corresponding encapsulated QoE measurement configurations. Case 1 can be supported by +CAPPLEVMCNR by setting the codepoint <meas_config_app_layer_id> and the codepoint <start-stop_measurement> to value “1”. Furthermore, by setting the codepoint <start-stop_measurement> to value “1” and omitting the codepoint <meas_config_app_layer_id> all measurement configurations (incl. encapsulated QoE and RVQoE configurations) can be released at once. The codepoint <ran_visible_release_only> is only applicable for Case 2. Therefore, the definition of this codepoint needs to corrected to make it clear that it is only used for releasing RAN visible application level measurements. - +CAPPLEVMCNR: To be aligned with latest NR RRC specification in TS 38.331 the codepoint <report_initial_playout_delay> needs to be corrected to <report_playout_delay_for_media_startup>. - +CAPPLEVMRNR: In the definition of the codepoint <meas_config_app_layer_id> the part “associated with the report in the <app-meas_report> parameter” can be removed. Reason: an application level measurement report may contain only the QoE measurement status (session start/end indication) or the RAN-visible measurements due to the fact that reporting of RAN-visible measurements can be configured by network with a periodicity different from the encapsulated QoE measurements. Therefore, the QoE measurement configuration identity does not need to be linked with the <app-meas_report> parameter. - +CAPPLEVMRNR: The definition of the codepoint <playout_delay_for_media_startup> is not aligned with the codepoint, i.e. the part “initial playout delay” needs to be replaced by “playout delay for media startup”. - +CAPPLEVMRNR: In Table 8.85-1 the suffix “NR” is missing for the AT command name +CAPPLEVMR. - +CAPPLEVMRNR: In Table 8.85-1 the codepoint <meas_config_app_layer_id> should be separated from the group [<app-meas_report_length>,<app-meas_report>], since <meas_config_app_layer_id> identifies the QoE measurement configuration even if there is not any <app-meas_report>. Furthermore, the codepoint <meas_config_app_layer_id> should be defined as mandatory present to be aligned with ASN.1 signalling. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - +CAPPLEVMCNR: The definition of the codepoint <ran_visible_release_only> has been corrected to make it clear that it is only used for releasing RAN visible application level measurements indicated by the <meas_config_app_layer_id>. - +CAPPLEVMCNR: The codepoint <report_initial_playout_delay> has been be corrected to <report_playout_delay_for_media_startup>. - +CAPPLEVMRNR: In the definition of the codepoint <meas_config_app_layer_id> the part “associated with the report in the <app-meas_report> parameter” has been removed. - +CAPPLEVMRNR: In the definition of the codepoint <playout_delay_for_media_startup> the part “initial playout delay” has been replaced by “playout delay for media startup”. - +CAPPLEVMRNR: In Table 8.85-1 the suffix “NR” has been added to the AT command name +CAPPLEVMR. - +CAPPLEVMRNR: In Table 8.85-1 the codepoint <meas_config_app_layer_id> has been separated from the group [<app-meas_report_length>,<app-meas_report>] and defined as mandatory present. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The use of the codepoint <ran_visible_release_only> remains misaligned with its original purpose. Inconsistences remain in the definition of the NR QoE AT commands. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.84, 8.85 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******************* NEXT CHANGE ***********************

## 8.84	Application level measurement configuration for NR +CAPPLEVMCNR

Table 8.84-1: +CAPPLEVMCNR parameter command syntax

| Command | Possible response(s) |
| --- | --- |
| +CAPPLEVMCNR=[<n>] | +CME ERROR: <err> |
| +CAPPLEVMCNR? | +CAPPLEVMCNR: <n> |
| +CAPPLEVMCNR=? | +CAPPLEVMCNR: (list of supported <n>s) |

Description

This command allows control of the application level measurement configuration according to 3GPP TS 38.331 [160]. The set command controls the presentation of the unsolicited result code +CAPPLEVMCNR: (list of [<CR><LF>,<meas_config_app_layer_id>,[<start-stop_measurement>,[<ran_visible_release_only>]],[<app-meas_config_file_length>,<app-meas_config-file>],[<transmission_of_session_start-end>],[<ran_visible_periodicity>],[<number_of_buffer_level_entries>],[<report_playout_delay_for_media_startup>],[<app-meas_service_type>]]s) providing data for the configuration. Refer clause 9.2 for possible <err> values.

Read command returns the current value of <n>.

Test command returns values supported as a compound value.

Defined values

<n>: integer type. Disable and enable presentation of the unsolicited result code +CAPPLEVMCNR to the TE.

0	Disable presentation of the unsolicited result code

1	Enable presentation of the unsolicited result code

<app-meas_service_type>: integer type. Contains the indication of what application that is target for the application level measurement configuration.

1	QoE measurement collection for streaming services

2	QoE measurement collection for MTSI services

3	QoE measurement collection for VR services

<start-stop_measurement>: integer type. Indicates the start and stop of the application level measurement reporting for the application indicated by the <app-meas_service_type>.

0	start the application level measurement

1	stop the application level measurement and release the application level measurement configuration

<app-meas_config_file_length>: integer type. Indicates the number of octets of the <app-meas_config-file> parameter.

<app-meas_config-file>: string of octets. Contains the application level measurement configuration file for the application indicated by the <app-meas_service_type>. The parameter shall not be subject to conventional character conversion as per +CSCS.

<meas_config_app_layer_id>: integer type. At QoE measurement configuration the <meas_config_app_layer_id> indicates an identity for the QoE measurement configuration received in the <app-meas_config-file>. At QoE measurement configuration release, the <meas_config_app_layer_id> indicates the measurement to be released. The absence of this parameter indicates that all measurement configurations are released.

<transmission_of_session_start-end>: integer type. Contains an indication of whether session start-end is required.

0	Not required

1	Required

<ran_visible_periodicity>: integer type.

0	120 ms

1	240 ms

2	480 ms

3	640 ms

4	1024 ms

<number_of_buffer_level_entries>: integer type. Contains the number of buffer level entries.

1-8

<report_playout_delay_for_media_startup>: integer type. Contains an indication of whether report of playout delay for media startup is required.

0	Report of playout delay for media startup is not required

1	Report of playout delay for media startup is required

<ran_visible_release_only>: integer type. Indicates the RAN visible application level measurements to be released.

0	Release the RAN visible application level measurements for this <meas_config_app_layer_id>

Implementation

Optional.

******************* NEXT CHANGE ***********************

## 8.85	Application level measurement report for NR +CAPPLEVMRNR

Table 8.85-1: +CAPPLEVMRNR action command syntax

| Command | Possible response(s) |
| --- | --- |
| +CAPPLEVMRNR=(list of <CR><LF>,<meas_config_app_layer_id>,[<app-meas_report_length>,<app-meas_report>],[<number-of-pdu-session_id-entries>,(list of <pdu-session_id>s)],[<number_of_buffer_level_entries>,(list of <application_layer_buffer-level>s)],[<qoe_measurement_status>],[<playout_delay_for_media_startup>]s) | +CME ERROR: <err> |
| +CAPPLEVMRNR=? |  |

Description

This command allows the MT to provide a list of application level measurement reports according to 3GPP TS 38.331 [160]. Refer clause 9.2 for possible <err> values.

Defined values

<qoe_measurement_status>: Indicates whether a session has started or ended.

0	started

1	ended

<app-meas_report_length>: integer type. Indicates the number of octets of the <app-meas_report> parameter.

<app-meas_report>: string of octets. Contains the application level measurement report for the application indicated by the <app-meas_service_type>. The parameter shall not be subject to conventional character conversion as per +CSCS.

<meas_config_app_layer_id>: integer type. Identifies the QoE measurement configuration.

<playout_delay_for_media_startup>: integer type. Indicates the application layer playout delay for media startup in ms.

0-30000

<number_of_buffer_level_entries>: integer type. Contains the number of <application_layer_buffer-level> entries.

1-8

<application_layer_buffer-level>: integer type. Indicates the application layer buffer level in ms in steps of 10 ms.

0-30000

<number-of-pdu-session_id-entries>: integer type: Indicates the number of entries in the list of <pdu-session_id>

<pdu-session_id>: integer type. Identifies a PDU session ID.

0-255

Implementation

Optional.

******************* END OF CHANGE ***********************