3GPP TSG-CT WG1 Meeting #141e	C1-23xxxx

# Online 17– 21 April 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.554 | CR | 0297 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update 5G ProSe link modification messages for the L3 UE-to-UE relay reselection procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In S2-2303389 updates were agreed for the UE-to-UE reselection procedure and implemented in 23.304, clause 6.7.4.3. The proposed changes have impact on the 5G ProSe direct link modification procedure, which needs to be implemented in stage-3. Following updates are proposed in this CR: - Corresponding IEs added to the link modification messages - New PROSE DIRECT LINK MODIFICATION ACK message is added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 5G PROSE DIRECT LINK MODIFICATION ACCEPT, and 5G PROSE DIRECT LINK MODIFICATION REQUEST messages are updated, while 5G PROSE DIRECT LINK MODIFICATION ACK is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 2 updated of 5G ProSe UE-to-UE relay reselection procedure is not implemented |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.3.6.1,10.3.6.a, 10.3.6.b, 10.3.6.c, 10.3.6.d, 10.3.7.1, 10.3.7.a, 10.3.7.b, 10.3.7.c, 10.3.7.d,10.3.x.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** First change *****

#### 10.3.6.1	Message definition

This message is sent by the UE to another peer UE to initiate the direct link modification procedure. See table 10.3.6.1.1.

Message type:	PROSE DIRECT LINK MODIFICATION REQUEST

Significance:	dual

Direction:	UE to peer UE

Table 10.3.6.1.1: PROSE DIRECT LINK MODIFICATION REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | PROSE DIRECT LINK MODIFICATION REQUEST message identity | ProSe PC5 signalling message type 11.3.1 | M | V | 1 |
|  | Sequence number | Sequence number 11.3.2 | M | V | 1 |
|  | Link modification operation code | Link modification operation code 11.3.19 | M | V | 1 |
|  | QoS flow descriptions | PC5 QoS flow descriptions 11.3.5 | M | LV-E | 5-65537 |
| 7C | QoS rules | PC5 QoS rules 11.3.29 | O | TLV-E | 7-65538 |
| XY | Source end UE info | User info ID 11.3.xz | O | TLV | 3-257 |
| XZ | Target end UE info | User info ID 11.3.xz | O | TLV | 3-257 |
| ZZ | Target end UE layer-2 ID | Layer-2 ID 11.3.25 | O | TV | 4 |
| AA | Relay reselection indication | Relay reselection indication 11.3.xx | O | TV | 1 |
| AB | List of candidates U2U relay UE info | User info ID 11.3.xz | O | TLV | 3-257 |
| AC | Initiating UE IPv6 address | Link local IPv6 address 11.3.7 | O | TV | 17 |
| AD | List of target UE IPv6 address | Link local IPv6 address 11.3.7 | O | TLV | 17-257 |
| AF | List of candidates U2U relay UE layer-2 ID | Layer-2 ID 11.3.25 | O | TV | 4-257 |

#### 10.3.6.a	Relay reselection indication

The UE shall include this IE if the 5G ProSe direct link modification request is to trigger UE-to-UE relay reselection, and if:

a)	the UE acts as a source 5G ProSe end UE and the 5G ProSe direct link is between the source 5G ProSe end UE and the 5G ProSe UE-to-UE relay UE to indicate the Relay reselection indication to the 5G ProSe UE-to-UE relay UE; or

b)	the UE acts as a 5G ProSe UE-to-UE relay UE and the 5G ProSe direct link is between the 5G ProSe UE-to-UE relay UE and the target 5G ProSe end UE to indicate the Relay reselection indication to the target 5G ProSe end UE.

#### 10.3.6.b	List of candidates U2U relay UE user info

The UE shall include this IE if relay reselection is indicated, and if:

a)	the UE acts as a source 5G ProSe end UE and the 5G ProSe direct link is between the source 5G ProSe end UE and the 5G ProSe UE-to-UE relay UE to indicate the user info ID of the candidate 5G ProSe UE-to-UE relay UEs; or

b)	the UE acts as a 5G ProSe UE-to-UE relay UE and the 5G ProSe direct link is between the 5G ProSe UE-to-UE relay UE and the target 5G ProSe end UE to indicate the user info ID of the candidate 5G ProSe UE-to-UE relay UEs.

#### 10.3.6.c	List of target UE IPv6 address

The UE shall include this IE if relay reselection is indicated, and if:

a)	the UE acts as a source 5G ProSe end UE and the 5G ProSe direct link is between the source 5G ProSe end UE and the 5G ProSe UE-to-UE relay UE to indicate the list of link local IPv6 addresses of the target 5G ProSe end UEs.

#### 10.3.6.d	List of candidates U2U relay UE layer-2 ID

The UE shall include this IE if relay reselection is indicated, and if:

a)	the UE acts as a source 5G ProSe end UE and the 5G ProSe direct link is between the source 5G ProSe end UE and the 5G ProSe UE-to-UE relay UE to indicate the layer-2 ID of the 5G ProSe UE-to-UE relay UE; or

b)	the UE acts as a 5G ProSe UE-to-UE relay UE and the 5G ProSe direct link is between the 5G ProSe UE-to-UE relay UE and the target 5G ProSe end UE to indicate the layer-2 ID of the 5G ProSe UE-to-UE relay UE.

***** Next change *****

#### 10.3.7.1	Message definition

This message is sent by the UE to another peer UE to indicate that the link modification request is accepted. See table 10.3.7.1.1.

Message type:	PROSE DIRECT LINK MODIFICATION ACCEPT

Significance:	dual

Direction:	UE to peer UE

Table 10.3.7.1.1: PROSE DIRECT LINK MODIFICATION ACCEPT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | PROSE DIRECT LINK MODIFICATION ACCEPT message identity | ProSe PC5 signalling message type 11.3.1 | M | V | 1 |
|  | Sequence number | Sequence number 11.3.2 | M | V | 1 |
| 79 | QoS flow descriptions | PC5 QoS flow descriptions 11.3.5 | O | TLV-E | 6-65538 |
| 7C | QoS rules | PC5 QoS rules 11.3.29 | O | TLV-E | 7-65538 |
| XY | Source end UE info | User info ID 11.3.xz | O | TLV | 3-257 |
| XZ | Target end UE info | User info ID 11.3.xz | O | TLV | 3-257 |
| BA | Relay reselection indication | Relay reselection indication 11.3.xx | O | TV | 1 |
| BB | New 5G ProSe UE-to-UE relay UE info | User info ID 11.3.xz | O | TLV | 3-257 |
| BC | Initiating UE IPv6 address | Link local IPv6 address 11.3.7 | O | TV | 17 |
| BD | Target UE IPv6 address | Link local IPv6 address 11.3.7 | O | TV | 17 |

#### 10.3.7.a	Relay reselection indication

The UE shall include this IE if the 5G ProSe direct link modification request for UE-to-UE relay reselection is accepted, and if:

a)	the UE acts as a target 5G ProSe end UE and the 5G ProSe direct link is between the target 5G ProSe end UE and the 5G ProSe UE-to-UE relay UE to indicate the Relay reselection indication to the 5G ProSe UE-to-UE relay UE; or

b)	the UE acts as a 5G ProSe UE-to-UE relay UE and the 5G ProSe direct link is between the 5G ProSe UE-to-UE relay UE and the source 5G ProSe end UE to indicate the Relay reselection indication to the source 5G ProSe end UE.

#### 10.3.7.b	New 5G ProSe UE-to-UE relay UE info

The UE shall include this IE if relay reselection is indicated, and if:

a)	the UE acts as a target 5G ProSe end UE and the 5G ProSe direct link is between the target 5G ProSe end UE and the 5G ProSe UE-to-UE relay UE to indicate the user info ID of the new 5G ProSe UE-to-UE relay UE; or

b)	the UE acts as a 5G ProSe UE-to-UE relay UE and the 5G ProSe direct link is between the 5G ProSe UE-to-UE relay UE and the source 5G ProSe end UE to indicate the user info ID of the new 5G ProSe UE-to-UE relay UEs.

#### 10.3.7.c	Initiating UE IPv6 address

The UE shall include this IE if relay reselection is indicated, and if:

a)	the UE acts as a target 5G ProSe end UE and the 5G ProSe direct link is between the target 5G ProSe end UE and the 5G ProSe UE-to-UE relay UE to indicate the link local IPv6 address of the source 5G ProSe end UE; or

b)	the UE acts as a 5G ProSe UE-to-UE relay UE and the 5G ProSe direct link is between the 5G ProSe UE-to-UE relay UE and the source 5G ProSe end UE to indicate the link local IPv6 address of the target end UE.

#### 10.3.7.d	Target UE IPv6 address

The UE shall include this IE if relay reselection is indicated, and if:

a)	the UE acts as a target 5G ProSe end UE and the 5G ProSe direct link is between the target 5G ProSe end UE and the 5G ProSe UE-to-UE relay UE to indicate the link local IPv6 address of the target 5G ProSe end UE; or

b)	the UE acts as a 5G ProSe UE-to-UE relay UE and the 5G ProSe direct link is between the 5G ProSe UE-to-UE relay UE and the source 5G ProSe end UE to indicate the link local IPv6 address of the target end UE.

***** Next change *****

#### 10.3.x.1	Message definition

This message is sent by the UE to another peer UE to ack the direct link modification procedure. See table 10.3.x.1.1.

Message type:	PROSE DIRECT LINK MODIFICATION ACK

Significance:	dual

Direction:	UE to peer UE

Table 10.3.x.1.1: PROSE DIRECT LINK MODIFICATION ACK message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | PROSE DIRECT LINK MODIFICATION ACK message identity | ProSe PC5 signalling message type 11.3.1 | M | V | 1 |
|  | Sequence number | Sequence number 11.3.2 | M | V | 1 |
|  | Relay reselection indication | Relay reselection indication 11.3.xx | M | TV | 1 |
|  | Initiating UE IPv6 address | Link local IPv6 address 11.3.7 | M | TV | 17 |
|  | Responding UE IPv6 address | Link local IPv6 address 11.3.7 | M | TV | 17 |

***** End of changes *****