3GPP TSG-CT WG1 Meeting #141e	C1-232713

# Online 17– 21 April 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 5204 | rev | 2 | Current version: | 18.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Network Slice replacement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-03-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The current text for network slice replacement requires improvement to be consustent with stage 2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CR proposes: - Identifying whether the S-NSSAI is HPLMN S-NSSAI or VPLMN S-NSSAI. - Removal of EN and assing a note since it is now decided that it is one S-NSSAI at the time. - Correcting the format of clause 4.6.3.4so it does not have automatic numbering. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 3 is not well defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.2.7, 4.6.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------------------------------------- Next Change --------------------------------------

-------------------------------------- Next Change --------------------------------------

#### 4.6.2.7	Mobility management based network slice replacement

If the UE and network support network slice replacement, and the AMF determines that an S-NSSAI included in the allowed NSSAI needs to be replaced with an alternative S-NSSAI, the AMF provides the alternative S-NSSAI in the allowed NSSAI (if not included yet) and in the configured NSSAI (if not included yet) and the mapping information between the S-NSSAI to be replaced and the alternative S-NSSAI to the UE during UE configuration update procedure as follows:

a)	for non-roaming UE, the AMF provides the mapping information between the S-NSSAI included in the allowed NSSAI and the alternative S-NSSAI to the UE; and

b)	for roaming UE:

1)	if the S-NSSAI included in the allowed NSSAI needs to be replaced (i.e. the S-NSSAI to be replaced is part of the VPLMN S-NSSAIs), the AMF provides the mapping information between the S-NSSAI included in the allowed NSSAI and the alternative S-NSSAI to the UE; and

2)	if the S-NSSAI included in the mapped S-NSSAI(s) for the allowed NSSAI needs to be replaced (i.e. the S-NSSAI to be replaced is part of the HPLMN S-NSSAIs), the AMF provides the mapping information between the S-NSSAI and the alternative S-NSSAI to the UE.

NOTE:	The alternative S-NSSAI may be part of or not part of the subscribed S-NSSAI(s) in the UE subscription.

Editor's note:	(WI: eNS_Ph3, CR: 5069) Whether an S-NSSAI included in the allowed NSSAI can be replaced with more than one alternative S-NSSAI is FFS.

-------------------------------------- Next Change --------------------------------------

#### 4.6.3.4	Session management based network slice replacement

If:

a)	the UE and network support network slice replacement;

b)	the UE is provided with the mapping information between the S-NSSAI to be replaced and the alternative S-NSSAI; and

c)	the UE decides to establish a new PDU session with the S-NSSAI to be replaced,

the UE provides both the S-NSSAI to be replaced and the alternative S-NSSAI during PDU session establishment procedure. If the SMF receives both the S-NSSAI to be replaced and the alternative S-NSSAI during PDU session establishment procedure, the SMF proceeds with the PDU session establishment procedure with the alternative S-NSSAI. If the PDU session establishment request is accepted, the SMF includes the alternative S-NSSAI in the PDU session establishment accept message.

If the SMF receives from the AMF an alternative S-NSSAI for existing PDU session and:

a)	if the SMF decides to retain the existing PDU session (i.e. the SMF can serve both the alternative S-NSSAI and the S-NSSAI to be replaced with), the SMF sends the alternative S-NSSAI to the UE during network-requested PDU session modification procedure; or

b)	if the SMF decides to re-activate the existing PDU session and:

1)	if the SSC mode of the PDU session is SSC mode 3, the SMF initiates PDU session modification procedure to trigger PDU session reactivation by the UE; or

2)	if the SSC mode of the PDU session is SSC mode 1 or SSC mode 2, the SMF initiates PDU session release procedure to trigger PDU session reactivation by the UE.

-------------------------------------- End of Changes --------------------------------------