3GPP TSG-CT WG1 Meeting #146	C1-2401xx

# Online, 22– 26 January 2024													(revison of C1-240122)

Source:	Huawei, Hisilicon, China Mobile, China Southern Power Grid Co

Title:	Pseudo-CR on Update the procedure of MRF

Spec:	3GPP TS 24.186

Agenda item:	18.3.8

Document for:	Decision

1. Introduction

<Introduction part (optional)>

2. Reason for Change

The procedure at the MRF shall support not only bootstrap data channel but also application data channel.

The procedure of MF is outside the scope of this specification. So, the procedure at MF in terminating side shall be removed.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.186.

* * * First Change * * * *

9.3.2.3	Procedure at the MRF

MRF shall allocate data channel media resources based on the request from the IMS AS:

a)	create one bootstrap data channel media termination to be terminated locally;

b)	create one bootstrap data channel media termination to be offered to remote network; or

c)	update the data channel media terminations, i.e. add, modify, or delete the media streams in the termination;

and respond to the IMS AS with the allocated or updated data channel media resources information.

NOTE:	This is also applicable for the procedure at the MF.

Upon receipt the data channel media resources release message from IMS AS, MRF shall release the corresponding allocated data channel media resources for this IMS session.

* * * Next Change * * * *

9.3.3.3	Procedures at the MRF

The procedure defined in clause 9.3.2.3 applies.

* * * End of Changes * * * *