3GPP TSG-CT WG1 Meeting #146	C1-240164r1

# Online, 22– 26 January 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.380 | CR | 0366 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Reuse Inviting-MCPTT-User-Identity field to communicate callee information to the caller |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Motorola Solutions UK Ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | enh4MCPTT |  |  |  |  |  | Date: |  |  | 2024-01-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | <Release> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The reason for this change is Observation #1 mentioned in in ETSI MCX Plugtests#8 event Wiki. Inviting-MCPTT-User-Identity field is part of the Connect message, which is used for managing pre-established sessions. This field is used to communicate caller information to the callee. However, there is no suitable field to communicate callee information to the caller. Hence in case of First-to-answer call with pre-established session, the callee information is not available with the caller. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The name of the Inviting-MCPTT-User-Identity field is changed throughout the document to Peer-MCPTT-User-Identity so that it can be used to communicate peer user information with both the callee as well as the caller. Note: Changes related to sharing callee information with the caller during a First-to-answer call with pre-established session, are implemented in a separate CR which accompanies this one. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be no suitable field to communicate callee information to the caller. This impacts scenarios like First-to-answer-call using pre-established sessions, where callee information cannot be communicated to the caller. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.3.7, 8.3.4, 9.3.2.3.3, 9.3.2.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | None |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | C1-240164 |  |  |  |  |  |  |  |  |

* * * First Change * * *

#### 8.3.3.7	Peer MCPTT User Identity field

The Peer MCPTT User Identity field contains the MCPTT ID identifying the peer MCPTT user.

Table 8.3.3.7-1 describes the coding of the Peer MCPTT User Identity field.

Table 8.3.3.7-1: Peer MCPTT User Identity field coding

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|Peer MCPTT |Peer MCPTT |Peer MCPTT User Identity   |

|User Identity  |User Identity  |                               |

|field ID       |length         |                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+                               :

:                                             (Padding)         :

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

The <Peer MCPTT User Identity field ID> value is a binary value and shall be set according to table 8.3.3.1-2.

The <Peer MCPTT User Identity length> value is a binary value indicating the length in octets of the <MCPTT Group Identity> value item except padding.

The <Peer MCPTT User Identity> value contains the MCPTT ID of the peer MCPTT user. The <Peer MCPTT User Identity> value shall be coded as specified in the table 8.3.3.7-2. The MCPTT ID is specified in 3GPP TS 24.379 [2].

Table 8.3.3.7-2: ABNF syntax of string values of the <Peer MCPTT User Identity> value

peer-mcptt-user-identity = URI

If the length of the <Peer MCPTT User Identity> value is not (2 + multiple of 4) bytes, the <Peer MCPTT User Identity> value shall be padded to (2 + multiple of 4) bytes. The value of the padding bytes should be set to zero. The padding bytes shall be ignored.

* * * Next Change * * *

### 8.3.4	Connect message

The Connect message is sent by the participating MCPTT function on the originating side to the MCPTT client to confirm the establishment of an MCPTT call or sent on the terminating side to initiate an MCPTT call. The Connect message is only used in the on-network mode and only sent over the unicast bearer.

Table 8.3.4-1 shows the content of the Connect message.

Table 8.3.4-1: Connect message

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|V=2|P| Subtype |   PT=APP=204  |          Length               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|               SSRC of participating MCPTT function            |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                          name=MCPC                            |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                   MCPTT Session Identity field                |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                    MCPTT Group Identity field                 |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                      Media Streams field                      |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                      Warning Text field                       |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                      Answer State field                       |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                 Peer MCPTT User Identity field            |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                       PCK I_MESSAGE field                     |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

With the exception of the three first 32-bit words the order of the fields are irrelevant.

Subtype:

The subtype shall be coded according to table 8.3.2-1.

Length:

The length shall be coded as specified in to clause 8.1.2.

SSRC:

The SSRC field shall carry the SSRC of the participating MCPTT function.

The SSRC field shall be coded as specified in IETF RFC 3550 [3].

MCPTT Session Identity:

The MCPTT Session Identity field is coded as described in clause 8.3.3.3.

MCPTT Group Identity:

The MCPTT Group Identity field is coded as described in clause 8.3.3.5.

Media Streams:

The Media Streams field is coded as described in clause 8.3.3.2.

Warning Text:

The Warning Text field is coded as described in clause 8.3.3.4.

Answer State:

The Answer State field is coded as described in clause 8.3.3.6.

When the Answer State field is not included the value "confirmed" shall be assumed.

Peer MCPTT User Identity:

The Peer MCPTT User Identity field is coded as described in clause 8.3.3.5.

When the peer MCPTT user requested privacy, the <sip:anonymous@invalid.invalid> identity shall be used.

PCK I_MESSAGE:

The PCK I_MESSAGE is used to transport the PCK and PCK-ID for use in private call.

This field is used when the terminating participating MCPTT function sends the Connect message to the terminating client for a private call.

* * * Next Change * * *

##### 9.3.2.3.3	Receive SIP INVITE request (R: SIP INVITE)

Upon receiving a SIP INVITE request from the controlling MCPTT function, if in automatic answer mode, the participating MCPTT function:

1.	shall send the Connect message to the invited MCPTT client using the media plane control transport channel defined for this call. The Connect message:

a.	shall include the SIP URI received in the Contact header field of the SIP INVITE request in the <MCPTT Session Identity> value in the MCPTT Session Identity field;

b.	if the SIP INVITE request included an "application/vnd.3gpp.mcptt-info+xml" MIME body with the <session-type> element set to a value of "prearranged" or "chat":

i.	shall include the content of the <mcptt-calling-group-id> element in the MCPTT Group Identity field;

ii.	shall include the value 'prearranged' or 'chat' in the <Session Type> value in the MCPTT Session Identity field;

iii.	shall include the content of the <mcptt-calling-user-id> element in the Peer MCPTT User Identity field if available from the SIP INVITE message and privacy of the inviting MCPTT ID is not required; and

iv.	shall include 'anonymous@anonymous.invalid' in the Peer MCPTT User Identity field if the MCPTT ID of the inviting MCPTT user is not available from the SIP INVITE message or privacy of the inviting MCPTT ID is required;

c.	if the SIP INVITE request includes an "application/vnd.3gpp.mcptt-info+xml" MIME body with the <session-type> element set to a value of "private":

i.	shall include the content of the <mcptt-calling-user-id> element in the Peer MCPTT User Identity field if available from the SIP INVITE message and privacy of the inviting MCPTT ID is not required;

ii.	shall include 'anonymous@anonymous.invalid' in the Peer MCPTT User Identity field if the MCPTT ID of the inviting MCPTT user is not available from the SIP INVITE message or privacy of the inviting MCPTT ID is required;

iii.	shall include the value 'private' in the <Session Type> value in the MCPTT Session Identity field; and

iv.	if the MIKEY-SAKKE I_MESSAGE is included in the received SIP INVITE request, shall include the MIKEY-SAKKE I_MESSAGE as received, in the PCK I_MESSAGE field;

d.	shall include a Media Streams field, if the negotiated SDP in the pre-arranged session contains more media streams than required by the SDP in the SIP INVITE request; and

NOTE 1:	For example if the call is a private call without floor control the Media Streams field needs to be included with the <Control Channel> value set to '0'.

e.	shall set the first bit in the subtype of the Connect message to '1' (Acknowledgment is required) as described in clause 8.3.2;

2.	shall start timer T55 (Connect);

3.	shall initialize counter C55 (Connect) to 1;

4.	shall reserve the media channel resources if not yet reserved; and

5.	shall enter the 'G: Pre-established session in use' state.

NOTE 2: The case of manual answer is described in 3GPP TS 24.379 [2].

* * * Next Change * * *

##### 9.3.2.3.5	Receive SIP 200 (OK) response to the SIP re-INVITE request (R: 200 OK)

Upon receiving the SIP 200 (OK) response to the SIP re-INVITE request of the pre-established session a call as specified in 3GPP TS 24.379 [2] (call setup with manual answer with pre-established session) the participating MCPTT function:

1.	shall send the Connect message to the invited MCPTT client(s) using the media plane control transport channel defined for this call. The Connect message:

a.	shall include the SIP URI received in the Contact header field of the SIP re-INVITE request in the <MCPTT Session Identity> value in the MCPTT Session Identity field;

b.	if the SIP re-INVITE request included an "application/vnd.3gpp.mcptt-info+xml" MIME body with the <session-type> element set to a value of "prearranged" or "chat":

i.	shall include the content of the <mcptt-calling-group-id> element in the MCPTT Group Identity field;

ii.	shall include the value 'prearranged' or 'chat' in the <Session Type> value in the MCPTT Session Identity field;

iii.	shall include the content of the <mcptt-calling-user-id> element in the Peer MCPTT User Identity field if available from the SIP re-INVITE message and privacy of the inviting MCPTT ID is not required; and

iv.	shall include 'anonymous@anonymous.invalid' in the Peer MCPTT User Identity field if the MCPTT ID of the inviting MCPTT user is not available from the SIP re-INVITE message or privacy of the inviting MCPTT ID is required;

c.	if the SIP re-INVITE request included an "application/vnd.3gpp.mcptt-info+xml" MIME body with the <session-type> element set to a value of "private":

i.	shall include the content of the <mcptt-calling-user-id> element in the Peer MCPTT User Identity field, if available from the SIP re-INVITE message and privacy of the inviting MCPTT ID is not required;

ii.	shall include 'anonymous@anonymous.invalid' in the Peer MCPTT User Identity field if the MCPTT ID of the inviting MCPTT user is not available from the SIP re-INVITE message or privacy of the inviting MCPTT ID is required; and

iii.	shall include the value 'private' in the <Session Type> value in the MCPTT Session Identity field;

d.	shall include a Media Streams field, if the negotiated SDP in the pre-arranged session contains more media streams than required by the SDP in the SIP re-INVITE request; and

NOTE:	For example if the call is a private call without floor control the Media Streams field needs to be included with the <Control Channel> value set to '0'.

e.	may set the first bit in the subtype of the Connect message to '1' (Acknowledgment is required) as described in clause 8.3.2;

2.	shall start timer T55 (Connect);

3.	shall initialise counter C55 (Connect) to 1;

4.	shall reserve the media channel resources if not yet reserved; and

5.	shall enter the 'G: Pre-established session in use' state.

* * * End of Changes * * *