3GPP TSG-CT WG1 Meeting #146	C1-24XXXX

# Online , 22– 26 January 2024

Source:	Lenovo

Title:	Usage of HTTP, common API framework, and security

Spec:	3GPP TS 24.559V0.3.1

Agenda item:	18.2.29

Document for:	Agreement

1. Introduction

<Introduction part (optional)>

2. Reason for Change

ADEA service is a SEAL service as described in TS 23.346, thus:

-	the same TLS requirement as for SEAL-UU between the ADEA client and the ADEA server as for SEAL-UU; and

-	the same requirement for the ADAE server to use the common API framework as for SEAL server,

are valid.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.559V.0.3.1.

* * * First Change * * * *

### 7.1.2	Usage of HTTP

#### 7.1.2.1	General

For ADAE service configuration API, support of HTTP/1.1 (IETF RFC 7230 [11], IETF RFC 7231 [12], IETF RFC 7232 [13], IETF RFC 7233 [14], IETF RFC 7234 [15] and IETF RFC 7235 [16]) over TLS is mandatory and support of HTTP/2 (IETF RFC 7540 [17]) over TLS is recommended.

A functional entity desiring to use HTTP/2 shall use the HTTP upgrade mechanism to negotiate applicable HTTP version as described in IETF RFC 7540 [17].

#### 7.1.2.2	Content type

The bodies of HTTP request and successful HTTP responses shall be encoded in JSON format (see IETF RFC 8259 [18]).

The MIME media type that shall be used within the related Content-Type header field is "application/json", as defined in IETF RFC 8259 [18].

* * * Next Change * * * *

# 8	Usage of common API framework

## 8.1	General

Usage of common API framework shall be supported by the ADAES as described in clause 8 in 3GPP TS 29.549 [7].

* * * Next Change * * * *

# 9	Security

## 9.1	General

Usage of HTTP over TLS and the TLS profiles shall be as specified in clause 5.1.1.4 of 3GPP TS 33.434 [9].

* * * End of Changes * * * *