3GPP TSG-CT WG1 Meeting #146                                                       C1-240208

E-meeting , 22– 26 January 2024

Source:	CATT

Title:	Pseudo-CR on overview of message definitions and contents

Spec:	3GPP TS 24.572

Agenda item:	18.2.19

Document for:	Agreement

1. Introduction

TS 24.572 is a newly created specification to capture the LCS user plane protocol to support the LPP and LCS messages transfer between the UE and the LMF.

2. Reason for Change

1. It is proposed to update the description in clause 10.1 according to clauses 8.1 in TS 24.501.

2. LCS-UPP message has been defined in clause 10.2, so the editor’s note in clause 10.2 should be removed.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 24.572 v1.1.0.

* * * * First Change * * * *

10.1	Overview

This clause defines the structure of the messages of the Layer 3 (L3) protocols defined in the present document including LCS-UPP messages as defined in clause 10.2 and UPP-CM messages as defined in clause 10.3. These are standard L3 messages as defined in 3GPP TS 24.007 [7].

Each definition given in the present clause includes:

a)	a brief description of the message direction and use, including whether the message has:

1.	Local significance, i.e. relevant only on the originating or terminating access;

2.	Access significance, i.e. relevant in the originating and terminating access, but not in the network;

3.	Dual significance, i.e. relevant in either the originating or terminating access and in the network; or

4.	Global significance, i.e. relevant in the originating and terminating access and in the network.

b)	a table listing the Information Elements (IE) known in the message and the order of their appearance in the message. All IEs that may be repeated are explicitly indicated (The V, LV and LV-E formatted IEs, which compose the imperative part of the message, occur before the T, TV, TLV and TLV-E formatted IEs which compose the non-imperative part of the message, see 3GPP TS 24.007 [7]). In a (maximal) sequence of consecutive IEs with half octet length, the first IE with half octet length occupies bits 1 to 4 of octet N, the second IE bits 5 to 8 of octet N, the third IE bits 1 to 4 of octet N+1 etc. Such a sequence always has an even number of elements.

For each information element the table indicates:

1.	The Information Element Identifier (IEI), in hexadecimal notation, if the IE has format T, TV, TLV or TLV-E. If the IEI has half octet length, it is specified by a notation representing the IEI as a hexadecimal digit followed by a "-" (example: B-).

NOTE 1:	The same IEI can be used for different information element types in different messages of the same protocol.

NOTE 2:	If a message includes a Type 6 IE container information element, then the same IEI can be used for different information element types in the Type 6 IE container information element and in other parts of the same message.

2.	The name of the information element (which may give an idea of the semantics of the element). The name of the information element followed by "IE" or "information element" is used in this technical report as reference to the information element within a message.

3.	The name of the type of the information element (which indicates the coding of the value part of the IE), and generally, the referenced subclause of clause 11 of the present document describing the value part of the information element.

4.	The presence requirement indication (M, C, or O) for the IE as defined in 3GPP TS 24.007 [7].

5.	The format of the information element (T, V, TV, LV, TLV, LV-E or TLV-E) as defined in 3GPP TS 24.007 [7].

6.	The length of the information element (or permissible range of lengths), in octets, in the message, where "?" means that the maximum length of the IE is only constrained by link layer protocol. This indication is non-normative.

c)	subclauses specifying, where appropriate, conditions for IEs with presence requirement C or O in the relevant message which together with other conditions specified in the present document define when the information elements shall be included or not, what non-presence of such IEs means, and – for IEs with presence requirement C – the static conditions for presence or non-presence of the IEs or for both cases (see 3GPP TS 24.007 [12]).

10.2	LCS-UPP messages

* * * * End of Changes * * * *