3GPP TSG-CT WG1 Meeting #146                                                       C1-240210

E-meeting , 22– 26 January 2024

Source:	CATT, Xiaomi

Title:	Update the overview of the UPP-CM procedure

Spec:	3GPP TS 24.572

Agenda item:	18.2.19

Document for:	Agreement

1. Introduction

TS 24.572 is a newly created specification to capture the LCS user plane protocol to support the LPP and LCS messages transfer between the UE and the LMF. This contribution proposes editorial modifications to the overview of the UPP-CM procedure.

2. Reason for Change

There are two subclause (6.1 and 6.2.1) in clause 6 that are related to the overview of the UPP-CM procedure, so it is proposed to merge them together.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 24.572 v1.1.0.

* * * * First Change * * * *

6	Elementary procedures for UPP-CM

6.1	Overview

6.1.1	General

This clause defines the UPP-CM procedures (clause 6.2). The format and coding of the messages and information elements are specified in clause 10.3 and 11.3 respectively.

The user plane positioning connection management is to support the management of a secure user plane connection between the UE and the LMF.

The UPP-CM messages defined in this clause can be included in the UPP-CMI container of the UL NAS Transport messages and DL NAS Transport messages defined in 3GPP TS 24.501 [4].

6.1.2	Types of UPP-CM procedures

Two types of UPP-CM procedures can be distinguished:

a)	Procedures related to establish the user plane connection for LCS-UPP:

1)	Initiated by the network:

i)	user plane connection information provisioning procedure.

2)	Initiated by the UE:

i)	UE initiated user plane connection establishment request procedure.

Upon successful completion of the UE originated user plane connection establishment request procedure, the LMF shall initiate the user plane connection information provisioning procedure.

b)	Procedures related to release the user plane connection for LCS-UPP:

1)	Initiated by the network:

i)	user plane connection release procedure.

6.2	UPP-CM procedures

* * * * End of Changes * * * *