3GPP TSG-CT WG1 Meeting #146	C1-240304

# Online , 22– 26 January 2024

Source:	Lenovo

Title:	Service description and operations for UE-to-UE session performance analytics

Spec:	3GPP TS 24.559V0.3.1

Agenda item:	18.2.29

Document for:	Agreement

1. Introduction

<Introduction part (optional)>

2. Reason for Change

Detailed service description and service operations for UE-to-UE session performance analytics.

Corrected clause numbering.

Added missing reference number.

Miscellaneous corrections of UE-to-UE session performance analytics related text.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.559V0.3.1.

* * * First Change * * * *

## 6.3	UE-to-UE session performance analytics

### 6.3.1	Service description

#### 6.3.1.1	Overview

The ADAE_ServiceConfiguration API, as defined 3GPP TS 23.436 [3], allows the ADAES via ADAE-UU reference point, to obtain the UE-to-UE session performance analytics from the ADAEC.

### 6.3.2	Service Operations

#### 6.3.2.1	Introduction

The service operation defined for ADAE_ServiceConfiguration API for UE-to-UE session performance analytics is shown in the table 6.3.2.1-1.

Table 6.3.2.1-1: Operations for UE-to-UE session performance analytics

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Fetch_UE2UE_Session_Performance_Analytics | This service operation is used by ADAES to obtain the UE-to-UE session performance analytics. | ADAES |

#### 6.3.2.2	Fetch_UE2UE_Session_Performance_Analytics

##### 6.3.2.2.1	General

This service operation is used by the ADAES for obtaining the UE-to-UE session performance analytics from the ADAEC.

##### 6.3.2.2.2	Obtaining UE-to-UE session performance analytics using Fetch_UE2UE_Session_Performance_Analytics service operation

To obtain the UE-to-UE session performance analytics, the ADAES shall send an HTTP POST request with a Request-URI according to the pattern "{apiRoot}/adae-sc/<apiVersion>/ue2ue-session-performance/fetch" and with a body containing data type Ue2UePerfReq as defined in clause 7.1.5.2.2.

Upon receipt of the HTTP POST request, the ADAEC shall:

a)	verify the identity of the ADAES and determine if the ADAES is authorized to obtain the UE-to-UE session performance analytics; and

b)	if the ADAES:

1)	is not authorized, the ADAEC shall respond to the ADAES with an appropriate error status code; or

2)	is authorized, the ADAEC shall respond to the ADAES with an HTTP "200 OK" status code with the response body including the Ue2UePerfResp as defined in clause 7.1.3.3.4.2 with the following attributes:

i)	UE-to-UE session performance analytics;

ii)	one or more VAL UEs; and

iii)	identity of the UE-to-UE session performance analytics.

* * * Next Change * * * *

#### 7.1.3.4	Resource: UE-to-UE session performance analytics

##### 7.1.3.4.1	Description

This resource is used by the ADAES to request the ADAEC for the UE-to-UE session performance analytics.

##### 7.1.3.4.2	Resource definition

Resource URI: {apiRoot}/adae-sc/<apiVersion>/ue2ue-session-performance

This resource shall support the resource URI variables defined in the table 7.1.3.4.2-1.

Table 7.1.3.4.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.2.4 in 3GPP TS 29.122 [4] |

##### 7.1.3.4.3	Resource standard methods

None

##### 7.1.3.4.4	Resource custom operations

###### 7.1.3.4.4.1	Overview

Table 7.1.3.4.4.1-1: Custom operations

| Operation name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| Fetch | /ue2ue-session-performance/fetch | POST | Request for the UE-to-UE session performance analytics |

###### 7.1.3.4.4.2	Fetch

This custom operation is for the ADAES to request the ADAEC the UE-to-UE session performance analytics and shall support the URI query parameters specified in table 7.1.3.4.4.2-1.

Table 7.1.3.4.4.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This custom operation shall support the request data structures specified in table 7.1.3.4.4.2-2 and the response data structures and response codes specified in table 7.1.3.4.4.2-3.

Table 7.1.3.4.4.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| Ue2UePerfReq | M | 1 | ADAES requests ADAEC for the UE-to-UE session performance analytics |

Table 7.1.3.4.4.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| Ue2UePerfResp | M | 1 | 200 OK | ADAEC responses ADAES the UE-to-UE session performance analytics |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

* * * Next Change * * * *

##### 7.1.5.2.2	Type: Ue2UePerfReq

Table 7.1.5.2.2-1: Definition of type Ue2UePerfReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| serverId | string | M | 1 | Identity of the ADAES |  |
| analyticsId | string | M | 1 | Identity of the UE-to-UE session analytics |  |
| valUeIds | array(ValTargetUe) | M | 1..N | One or more VAL UEs, for which the UE-to-UE session performance analytics, is requested. |  |
| pc5Qos | Pc5QoSPara | M | 1 | The QoS attributes to be analyzed at the ADAEC during UE-to-UE session. |  |
| reportConfig | ReportingInformation | O | 0..1 | The configuration of UE-to-UE session performance analytics reporting. |  |
| area | LocationArea | O | 0..1 | The geographical or service area, for which the UE-to-UE session peformance analytics is requested. |  |
| timeWindow | DurationSec | O | 0..1 | The time window as the start time point and the end time point, for which the UE-to-UE session performance analytics is applied. |  |

* * * End of Changes * * * *