3GPP TSG-CT WG1 Meeting #146	C1-240219

# E-meeting, 22 – 26 January 2024

Source:	Xiaomi

Title:	Update on Located UE discovery and selection

Spec:	3GPP TS 24.514 v0.5.0

Agenda item:	18.2.23

Document for:	Agreement

1. Introduction

This p-CR provides update on Located UE discovery and selection.

2. Reason for Change

For located UE discovery and selection, the general description for Located UE discovery and selection was agreed captured in clause 6.4.1 3GPP TS 24.514 v0.5.0, based on the stage 2 description on located UE discovery and selection in clause 5.2.2 and clause 5.11 3GPP TS 23.586.

Furthermore, the following is indicated in clause 5.2.2 3GPP TS 23.586:

When LMF determines SL positioning for target UE and trigger the discovery of the Located UE, LMF can decide that LMF or target UE selects Located UEs. If the decision is LMF selecting Located UEs, Target UE sends the multiple discovered candidate Located UEs to the LMF for the selection. After the LMF determines the selected Located UE(s), the LMF sends the selected Located UE(s) to the Target UE.

And in clause 6.20.2 and clause 6.20.5 3GPP TS 23.273, the following is dscribed:

After LMF determines that the assistance of Located UE is needed for Target UE Positioning, LMF decides that Target UE or LMF selects Located UE, and SL-MT-LR request also includes the indication of Target UE/LMF selecting Located UE.

Additionally, the following item considered for located UE selection is covered in the UE discovery procedure, so it is removed from here.

-	Whether the serving PLMN of candidate located UE(s) is same with serving PLMN of target UE;

Therefore, the related general description and procedure description for Located UE discovery and selection should be covered in stage 3.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 24.514 v0.5.0.

* * * First Change * * * *

6.4	Located UE selection

6.4.1	General

When ranging and sidelink positioning service is applied for a target UE, located UE(s) is discovered by the target UE, which can be triggered by LMF or the application layer of the target UE.

When multiple candidate located UEs are discovered, the located UE(s) is selected by the LMF or the target UE from the candidate located UEs. The located UE(s) is selected based on the following:

-	Candidate list of located UE(s) received from the LMF, if available;

-	Capabilities of the candidate located UE(s), e.g. the supported Sidelink Positioning methods;

-	The required positioning QoS;

-	UE’s information including whether UE is in coverage or not, UE’s location, etc.

When LMF determines SL positioning for target UE and trigger the discovery of the Located UE, LMF can decide that LMF or target UE selects Located UEs, and the LMF sends the indication of target UE/LMF selecting located UE to the target UE. If LMF performs the located UEs selection, target UE sends the multiple discovered candidate located UEs to the LMF for the selection. After the LMF determines the selected located UE(s), the LMF sends the selected located UE(s) to the Target UE. If target performs the located UEs selection, target UE sends the selected candidate located UE(s) to the LMF.

* * * Next Change * * * *

6.4.2.1	Target UE selecting located UE

6.4.2.1.x1	General

The located UE selection is performed by target UE in following cases:

a)	when the LMF is not involved, UE-only operation for ranging and sidelink positioning is used (see clause 6.4.2.1.x2); and

b)	when the LMF is involved, network-based operation or network-assisted operation for ranging and sidelink positioning is used and the LMF determines the located UE selection is performed by the target UE (see clause 6.4.2.1.x3).

6.4.2.1.x2	Target UE selecting located UE for UE-only operation

The target UE selects the located UE(s) from the UE(s) which are discovered using:

a)	the monitoring procedure for UE discovery as specified in clause 6.2.2.1 or clause 6.2.2.3 when located UE acts as announcing UE; or

b)		the discoverer procedure for UE discovery as specified in clause 6.2.2.2 or clause 6.2.2.4 when located UE acts as discoveree UE.

The target UE selects the located UE(s) based on the received RSPP metadata and serving PLMN ID in the PROSE PC5 DISCOVERY message. A UE is selected as located UE by the target UE if:

a)	the UE role in the RSPP metadata indicates the UE supports UE role as an Anchor UE (see 3GPP TS 38.355); and

b)	the serving PLMN ID indicating the same PLMN as the serving PLMN of the target UE

6.4.2.1.x3	target UE selecting located UE for network-based operation or network-assisted operation

TBD

* * * Next Change * * * *

6.4.2.2	LMF selecting located UE

6.4.2.2.x1	General

The purpose of the LMF selecting located UE procedure is to enable a target UE for ranging and sidelink positioning to assist the LMF to select one or more suitable located UEs ranging and sidelink positioning for the ranging and sidelink positioning service.

6.4.2.2.x2	LMF selecting located UE procedure initiation

The target UE shall trigger the LMF selecting located UE procedure if the following conditions are met:

a)	the UE is authorised to act as a target UE as specified in clause 5.2.3;

b)	if the located UE selection is triggered by supplementary service message from the LMF and the LMF sends the indication of LMF selecting located UE to the target UE, the UE has obtained a list of located UE candidate(s) fulfilling with the following:

1)	the monitoring procedure for UE discovery as specified in clause 6.2.2.1 or clause 6.2.2.3 when located UE acts as announcing UE; or

Editor’s Note:	Whether and how the located UE can be triggered to perform the announcing procedure for UE discovery as specified in clause 6.2.2.1 or clause 6.2.2.3 is FFS.

2)	the discoverer procedure for UE discovery as specified in clause 6.2.2.2 or clause 6.2.2.4 when located UE acts as discoveree UE.

6.4.2.2.x3	LMF selecting located UE procedure completion

When the LMF selecting located UE procedure is initiated as described in clause 6.4.2.2.x2, and there is any located UE candidate satisfying the conditions in clause 6.4.2.2.x2, the target UE shall send the located UE candidate(s) to the LMF in the corresponding response supplementary service message for located UE selection.

* * * End of Changes * * * *