3GPP TSG-CT WG1 Meeting #147	C1-240948

Athens, Greece, 26 February-1 March 2024

Source:	vivo

Title:	Pseudo-CR on collision handling of the establishment and release procedure

Spec:	3GPP TS 24.572 v1.1.0

Agenda item:	18.2.19

Document for:	Approval

1. Introduction

TS 24.572 is a newly created specification to capture the UPP-CM protocol and LCS-UPP protocol.

2. Reason for Change

Both UE and LMF are enabled to initiate the user plane connection establishment procedure and user plane connection release procedure. How to handle the following collision cases are not specified:

- Collision of user plane connection establishment procedure

- Collision of user plane connection release procedure

Only LMF side’s collision needs to be specified. When the LMF receives a request message after sending the command message to the UE, the LMF shall ignore request message and proceed with command message directly.

There is no need to specify the collision on the UE side since the UE proceeds with request message and command message in the order.

3. Proposal

It is proposed to agree with the following changes to 3GPP TS 24.572.

* * * First Change * * * *

6.2.1.1.6	Abnormal cases on the network side

The following abnormal cases can be identified:

a)	Expiry of the timer T5012.

The network shall, on the first expiry of the timer T5012, retransmit the USER PLANE CONNECTION ESTABLISHMENT COMMAND message and shall reset and start timer T5012. This retransmission is repeated up to four times, i.e. on the fifth expiry of timer T5012, the LMF shall abort the procedure.

b)	Lower layer failure before the USER PLANE CONNECTION ESTABLISHMENT COMPLETE or USER PLANE CONNECTION ESTABLISHMENT COMMAND REJECT message is received.

The network shall abort the user plane connection information provisioning procedure.

c)	Collision of user plane connection information provisioning procedure and UE initiated user plane connection establishment procedure

When the LMF receives a USER PLANE CONNECTION ESTABLISHMENT REQUEST message after sending a USER PLANE CONNECTION ESTABLISHMENT COMMAND message to the UE, the LMF shall ignore the USER PLANE CONNECTION ESTABLISHMENT REQUEST message and proceed with the user plane connection information provisioning procedure.

* * * Next Change * * * *

6.2.1.2.4	Abnormal cases on the network side

The following abnormal cases can be identified:

a)	Expiry of the timer T5010

The LMF shall, on the first expiry of the timer T5010, retransmit the USER PLANE CONNECTION RELEASE COMMAND message and shall reset and start timer T5010. This retransmission is repeated up to four times, i.e. on the fifth expiry of timer T5010, the LMF shall abort the procedure and locally release the secured user plane connection between the UE and the LMF.

b)	Collision of network initiated user plane connection release procedure and UE initiated user plane connection release procedure

When the LMF receives a USER PLANE CONNECTION RELEASE REQUEST message after sending a USER PLANE CONNECTION RELEASE COMMAND message to the UE, the LMF shall ignore the USER PLANE CONNECTION RELEASE REQUEST message and proceed with the network initiated user plane connection release procedure.

Editor's note:	It is FFS to determine whether any abnormal cases in the UE need to be specified.

* * * End of Changes * * * *