| 3GPP TS 24.369 V0.2.0 (2025-05) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Ambient IoT Non-Access-Stratum (AIoT NAS) protocol for 5G System (5GS); Stage 3; (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Abbreviations	7

4	General	7

4.1	Overview	7

4.X	<Subtitle>	8

5	Elementary procedures for AIoT NAS protocol	8

5.1	General	8

5.2	Inventory procedure	8

5.2.1	General	8

5.2.2	Inventory procedure initiation	8

5.2.3	Inventory procedure completion	9

5.3	Command procedures	10

5.3.1	General	10

5.3.2	Read command procedure	10

5.3.3	Write command procedure	10

5.3.4	Disable command procedure	10

6	Handling of unknown, unforeseen, and erroneous protocol data	11

6.1	General	11

6.2	Message too short or too long	11

6.2.1	Message too short	11

6.2.2	Message too long	11

6.3	Unknown or unforeseen message type	11

6.4	Non-semantical mandatory information element errors	11

6.5	Unknown and unforeseen IEs in the non-imperative message part	12

6.5.1	IEIs unknown in the message	12

6.5.2	Out of sequence IEs	12

6.5.3	Repeated IEs	12

6.6	Non-imperative message part errors	12

6.6.1	General	12

6.6.2	Syntactically incorrect optional IEs	12

6.7	Messages with semantically incorrect contents	12

7	Encoding of AIoT NAS protocol	13

7.1	Message functional definitions and contents	13

7.1.1	General	13

7.1.2	Inventory report	13

7.2	Encoding of information elements	14

7.2.1	Message type	14

7.2.3	Security parameters	14

7.2.4	AIoT device identity	14

8	List of system parameters	14

8.1	General	14

8.2	<Subtitle>	14

For definitive guidance on drafting 3GPP TSs and TRs, see 3GPP TS 21.801 supplemented by the 3GPP web page http://www.3gpp.org/specifications-groups/delegates-corner/writing-a-new-spec.

Ensure all blue guidance text is removed before submitting the TS/TR to the TSG for approval.

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document specifies the ambient internet of things (AIoT) non-access-stratum (NAS) protocol to support the transport of signalling and AIoT data between AIoT device and the AIoT function (AIOTF) in the 5G system (5GS) as specified in 3GPP TS 23.369 [2].

Editor's note:	The scope should be aligned with the naming of the protocol and is FFS.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.369: "Architecture support for Ambient power-enabled Internet of Things; Stage 2".

[3]	3GPP TS 24.007: "Mobile radio interface signalling layer 3; General aspects".

[4]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[5]	3GPP TS 29.abc: "5G System; Ambient IoT Data Management (ADM) Services; Stage 3".

[6]	3GPP TS 33.369: "Security aspects of ambient IoT services in 5G".

[7]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[8]	3GPP TS 38.391: "Ambient IoT Medium Access Control Protocol specification".

Editor's note:	The TS name and TS number of 3GPP TS 29.abc should updated when it is assigned by CT plenary.

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.369 [2] apply:

AIoT Device

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AIoT	Ambient Internet of Things

AIOTF	AIoT Function

NAS	Non-Access-Stratum

# 4	General

## 4.1	Overview

The AIoT NAS protocol described in the present document provide signalling between the AIoT device and the AIOTF, for 5GS. The AIoT NAS as part of the protocol stacks for support of AIoT services in 5GS is described in 3GPP TS 23.369 [2].

The main function of the AIoT NAS protocol is to support the AIoT procedures (see 3GPP TS 23.369[2]):

a)	inventory procedure; and

b)	command procedure.

For the support of the above functions, the procedures are supplied within procedures for AIoT NAS, in clause 5.

The AIoT NAS messages are transported between the AIoT device and the AIOTF via AIOT1 reference point, and this transport does not involve N1 reference point. The AIoT NAS protocol specified in the present document is unrelated to the 5GMM NAS and 5GSM NAS protocols specified in 3GPP TS 24.501 [4].

Editor's note:	AIoT NAS security functions, e.g. integrity protection and ciphering of AIoT NAS signalling messages, is FFS and depends on SA3 progress and available normative requirements in 3GPP TS 33.369 [6].

The AIoT NAS protocol for 5GS follows the protocol architecture model for layer 3 as described in 3GPP TS 24.007 [3].

## 4.X	<Subtitle>

To be edited, use Normal style

# 5	Elementary procedures for AIoT NAS protocol

## 5.1	General

This clause describes the procedures used for AIoT NAS protocol performed between the AIoT device and the AIOTF. The format and coding of the messages and information elements are specified in clause 7.1 and 7.2 respectively.

The following AIoT NAS procedures can be distinguished:

a)	procedure related to network initiated inventory request; and

b)	procedure related to network initiated command request.

AIoT NAS messages are standard L3 messages according to 3GPP TS 24.007 [3] and error behaviour specified for L3 protocol according to 3GPP TS 24.007 [3] applies for AIoT unless otherwise specified in the present document.

Editor's note:	Whether AIoT NAS protocol supports conditional IEs and comprehension required IEs as specified in 3GPP TS 24.007 [3] is FFS.

Editor's note:	Whether AIoT NAS protocol supports TLV-E format is FFS.

## 5.2	Inventory procedure

### 5.2.1	General

The inventory procedure is used to discover one or more AIoT devices.

### 5.2.2	Inventory procedure initiation

To initiate the inventory procedure, the AIOTF requests the selected NG-RAN to perform AIoT paging as defined in 3GPP TS 38.391 [8].

NOTE:	How AIOTF selects NG-RAN is defined in 3GPP TS 23.369 [2].

Editor's note:	The reference to TS 38.391 may be updated according to RAN2 progress.

Editor's note:	Whether the AIOTF needs a timer handling for inventory procedure is FFS.

Figure 5.2.2-1 gives an example of inventory procedure for a AIoT device which sends the INVENTORY REPORT message to AIOTF.

Editor's note:	Figure 5.2.2-1 may further align with RAN2/RAN3 according to RAN2/RAN3 progress.

Figure 5.2.2-1: Inventory procedure

### 5.2.3	Inventory procedure completion

The AIoT device determines to respond to the AIoT paging:

a)	based on the indication from lower layer as specified in 3GPP TS 38.391 [8]; or

b)	based on the filtering information received from the lower layer that matches the AIoT device permanent identifier.

If the AIoT device determines to respond to the AIoT paging, the AIoT device shall initiate an INVENTORY REPORT message and send it to AIOTF. In the INVENTORY REPORT message, the AIoT device:

a)	shall include the AIoT device ID IE set to the AIoT device permanent identifier.

Editor's note:	Security parameters are FFS and should be aligned with SA3.

The AIoT device determines whether the filtering information matches the AIoT device permanent identifier as follows:

NOTE:	The filtering information is defined in 3GPP TS 29.abc [5], and also corresponds to paging ID as defined in 3GPP TS 38.391 [8]).

Editor's note:	The reference to filtering information (e.g. IE name) will be updated according to CT4 and RAN2 progress.

Editor's note:	Whether the filtering information covers the case of a specific AIoT device is FFS and based on the format of filtering information determined by CT4.

Editor's note:	How to determine whether the filtering information matches the AIoT device permanent identifier should be defined after the format of filtering information is determined by CT4.

Based on whether the filtering information matches the AIoT device permanent identifier:

a)	if the filtering information matches the AIoT device permanent identifier, the AIoT device shall indicate to the lower layer that the filtering information is matched; or

b)	otherwise, the AIoT device shall ignore the filtering information and determine not to respond to the AIoT paging.

Upon all the selected NG-RAN indicating the inventory procedure has been completed, the inventory procedure is considered as completed by the AIOTF as specified in 3GPP TS 23.369 [2].

NOTE:	How NG-RAN indicates the inventory procedure has been completed is defined in 3GPP TS 38.300 [7].

Editor's note:	The reference to TS 38.300 may be updated according to RAN2 progress.

## 5.3	Command procedures

### 5.3.1	General

### 5.3.2	Read command procedure

#### 5.3.2.1	General

#### 5.3.2.2	Read command procedure initiation

#### 5.3.2.3	Read command procedure accepted by AIoT device

#### 5.3.2.4	Read command procedure not accepted by AIoT device

#### 5.3.2.5	Abnormal cases in the AIoT device

#### 5.3.2.6	Abnormal cases in the AIOTF

### 5.3.3	Write command procedure

#### 5.3.3.1	General

#### 5.3.3.2	Write command procedure initiation

#### 5.3.3.3	Write command procedure accepted by AIoT device

#### 5.3.3.4	Write command procedure not accepted by AIoT device

#### 5.3.3.5	Abnormal cases in the AIoT device

#### 5.3.3.6	Abnormal cases in the AIOTF

### 5.3.4	Disable command procedure

#### 5.3.4.1	General

#### 5.3.4.2	Disable command procedure initiation

#### 5.3.4.3	Disable command procedure accepted by AIoT device

Editor's note:	Whether "disable command procedure not accepted by AIoT device" and "abnormal cases in AIoT device and AIOTF" need to be specified is FFS.

# 6	Handling of unknown, unforeseen, and erroneous protocol data

## 6.1	General

The procedures specified in the present document apply to those messages which pass the checks described in this clause.

This clause also specifies procedures for the handling of unknown, unforeseen, and erroneous protocol data by the receiving entity. These procedures are called "error handling procedures", but in addition to providing recovery mechanisms for error situations they define a compatibility mechanism for future extensions of the protocols.

Clauses 6.2 to 6.7 shall be applied in order of precedence.

Detailed error handling procedures in the network are implementation dependent and may vary from network to network. However, when extensions of this protocol are developed, networks will be assumed to have the error handling that is indicated in this clause as mandatory ("shall") and that is indicated as strongly recommended ("should").

Also, the error handling of the network is only considered as mandatory or strongly recommended when certain thresholds for errors are not reached during a dedicated connection.

For definition of semantical and syntactical errors see 3GPP TS 24.007 [3], clause 11.4.2.

Editor's note:	Whether AIoT NAS protocol supports conditional IEs and comprehension required IEs is FFS.

Editor's note:	Whether AIoT NAS protocol supports TLV-E format is FFS.

## 6.2	Message too short or too long

### 6.2.1	Message too short

When a message is received that is too short to contain a complete message type information element, that message shall be ignored, c.f. 3GPP TS 24.007 [3].

### 6.2.2	Message too long

Editor's note:	Maximum AIoT NAS message length is FFS and to be defined by RAN3.

## 6.3	Unknown or unforeseen message type

If the AIoT device or the network receives a AIoT NAS message with message type not defined for the AIoT NAS protocol or not implemented by the receiver, it shall ignore the AIoT NAS message.

NOTE:	A message type not defined for the AIoT NAS protocol in the given direction is regarded by the receiver as a message type not defined for the AIoT NAS protocol, see 3GPP TS 24.007 [3].

## 6.4	Non-semantical mandatory information element errors

When on receipt of a message,

a)	an "imperative message part" error; or

b)	a "missing mandatory IE" error;

is diagnosed or when a message containing a syntactically incorrect mandatory IE is received, the AIoT device shall ignore the AIoT NAS message and the network shall:

a)	try to treat the message (the exact further actions are implementation dependent); or

b)	ignore the message.

## 6.5	Unknown and unforeseen IEs in the non-imperative message part

### 6.5.1	IEIs unknown in the message

The AIoT device shall ignore all IEs with unknown IEI in a message.

The network shall take the same approach.

### 6.5.2	Out of sequence IEs

The AIoT device shall ignore all out of sequence IEs in a message.

The network should take the same approach.

### 6.5.3	Repeated IEs

If an information element with format T, TV or TLV is repeated in a message, the AIoT device shall handle only the contents of the information element appearing first and shall ignore all subsequent repetitions of the information element.

The network should follow the same procedures.

## 6.6	Non-imperative message part errors

### 6.6.1	General

This category includes syntactically incorrect optional IEs.

### 6.6.2	Syntactically incorrect optional IEs

The AIoT device shall treat all optional IEs that are syntactically incorrect in a message as not present in the message.

The network shall take the same approach.

## 6.7	Messages with semantically incorrect contents

When a message with semantically incorrect contents is received, the AIoT device shall perform the foreseen reactions of the procedural part of clause 5. If, however no such reactions are specified, the UE shall ignore the message.

The network should follow the same procedure.

# 7	Encoding of AIoT NAS protocol

## 7.1	Message functional definitions and contents

### 7.1.1	General

Within the AIoT NAS protocol defined in the present document, every message is a standard L3 message as defined in 3GPP TS 24.007 [3]. This means that the message consists of the following parts:

1)	message type;

2)	security parameters;

3)	other information elements, as required.

The organization of an AIoT NAS message is illustrated in the example shown in figure 7.1.1-1.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message type |  |  |  |  |  |  |  | octet 1 |
| Security parameters |  |  |  |  |  |  |  | octet 2 octet a |
|  |  |  |  |  |  |  |  | octet a+1 |
| Other information elements as required |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | octet n |

Figure 7.1.1-1: General message organization example for an AIoT NAS message

Editor's note:	Additional content of AIoT NAS general message organization is FFS and can be added as normative requirements are available.

### 7.1.2	Inventory report

##### 7.1.2.1	Message definition

The INVENTORY REPORT message is sent by the AIoT device to the AIOTF to report the AIoT device identity.

See table 7.1.2.1-1.

Message type:	INVENTORY REPORT

Significance:	dual

Direction:	AIoT device to AIOTF

Table 7.1.2.1-1: INVENTORY REPORT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Message type | Message type 7.2.2 | M | V | 1 |
|  | Security parameters | Security parameters 7.2.3 | M | V | n |
|  | AIoT device identity | AIoT device identity 7.2.x | M | V | n |

## 7.2	Encoding of information elements

### 7.2.1	Message type

The message type IE and its use are defined in 3GPP TS 24.007 [3]. Table 7.2.2.1 defines the value part of the message type IE used in the AIoT NAS protocol.

Table 7.2.1-1: Message types for AIoT NAS

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 |  | Inventory report |

Editor's note:	Whether the range for message type of 8 bits is needed or a shorter range is sufficient is FFS. If a shorter range is sufficient, remaining bits in the IE can be used for other mandatory parameters, e.g. part of the security parameters.

### 7.2.3	Security parameters

Editor's note:	This IE contains the mandatory security parameters in AIoT NAS messages and content is FFS awaiting available normative requirements in SA3. The IE can be renamed and split into multiple individual security IEs to better suit security requirements.

### 7.2.4	AIoT device identity

Editor's note:	This IE contains information provided by the AIoT device that can be translated by the network to a globally unique AIoT device permanent identifier. The information and coding of the IE is FFS awaiting available normative specification.

# 8	List of system parameters

## 8.1	General

This clause describes the definition of the timers for AIoT.

## 8.2	<Subtitle>

To be edited, use Normal style

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-04 | CT1#154 |  |  |  |  | TS skeleton draft. C1-252552, C1-252553. | 0.1.0 |
| 2025-05 | CT4#155 |  |  |  |  | Inclusion of pCRs agreed during CT1#155 – C1-253642, C1-253643, C1-253754, C1-253756, C1-254097, C1-254111, C1-254112, C1-254133. | 0.2.0 |