3GPP TS 29.163 V10.1920.0 (20152016-1206)
70
Release 10

3GPP TS 29.163 V10.20.0 (2016-06)
Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Core Network and Terminals;

Interworking between the IP Multimedia (IM) Core Network (CN)

subsystem and Circuit Switched (CS) networks

(Release 10)

[image: image1.jpg]



[image: image2.jpg]K oy




The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organisational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organisational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organisational Partners' Publications Offices.

Keywords

GSM, UMTS, LTE, BICC, ISUP, circuit mode, IP, network
3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org
Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2015, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.
UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association


Contents

15Foreword

1
Scope
16
2
References
16
3
Definitions and abbreviations
22
3.1
Definitions
22
3.2
Abbreviations
22
4
General
24
4.1
General interworking overview
24
5
Network characteristics
24
5.1
Key characteristics of ISUP/BICC based CS networks
24
5.2
Key characteristics of IM CN subsystem
25
6
Interworking with CS networks
25
6.1
Interworking reference model
25
6.1.1
Interworking reference points and interfaces
25
6.1.2
Interworking functional entities
26
6.1.2.1
Signalling Gateway Function (SGW)
26
6.1.2.2
Media Gateway Control Function (MGCF)
26
6.1.2.3
IP Multimedia - Media Gateway Function (IM-MGW)
26
6.2
Control plane interworking model
26
6.3
User plane interworking model
26
7
Control plane interworking
26
7.1
General
27
7.2
Interworking between CS networks supporting ISUP and the IM CN subsystem
27
7.2.1
Services performed by network entities in the control plane
28
7.2.1.1
Services performed by the SS7 signalling function
28
7.2.1.2
Services of the SGW
28
7.2.1.3
Services of the MGCF
28
7.2.1.4
Services of the SIP signalling function
28
7.2.2
Signalling interactions between network entities in the control plane
28
7.2.2.1
Signalling between the SS7 signalling function and MGCF
28
7.2.2.1.1
Signalling from MGCF to SS7 signalling function
29
7.2.2.1.2
Signalling from SS7 signalling function to MGCF
29
7.2.2.1.3
Services offered by SCTP and M3UA
29
7.2.2.2
Signalling between the MGCF and SIP signalling function
29
7.2.3
SIP-ISUP protocol interworking
29
7.2.3.1
Incoming call interworking from SIP to ISUP at I-MGCF
29
7.2.3.1.1
Sending of IAM
29
7.2.3.1.2
Coding of the IAM
31
7.2.3.1.2A
Coding of the IAM when Number Portability is supported
44
7.2.3.1.2B
Coding of the IAM for Carrier Routeing
47
7.2.3.1.3
Sending of COT
47
7.2.3.1.3A
Sending of SAM
48
7.2.3.1.4
Sending of 180 ringing
49
7.2.3.1.4A
Sending of 183 Session Progress for early media scenarios
52
7.2.3.1.4B
Sending of 181Call is being forwarded
54
7.2.3.1.4C
Sending of 183 Session Progress for overlap signalling using the in-dialog method
55
7.2.3.1.4D
Sending of 183 Session Progress to carry ISUP Cause
55
7.2.3.1.5
Sending of the 200 OK (INVITE)
56
7.2.3.1.6
Sending of the Release message (REL)
58
7.2.3.1.7
Coding of the REL
58
7.2.3.1.8
Receipt of the Release Message
59
7.2.3.1.9
Receipt of RSC, GRS or CGB (H/W oriented)
61
7.2.3.1.9a
Receipt of REFER
62
7.2.3.1.10
Autonomous Release at I-MGCF
62
7.2.3.1.11
Internal through connection of the bearer path
63
7.2.3.2
Outgoing Call Interworking from ISUP to SIP at O-MGCF
63
7.2.3.2.1
Sending of INVITE
63
7.2.3.2.1a
Sending of INVITE without determining the end of address signalling
66
7.2.3.2.2
Coding of the INVITE
68
7.2.3.2.2A
Coding of the INVITE when Number Portability is supported
80
7.2.3.2.2B
Coding of the INVITE for Carrier Routeing
82
7.2.3.2.2C
Coding of INVITE with instance-id in form of IMEI URN
82
7.2.3.2.3
Receipt of CONTINUITY
83
7.2.3.2.4
Sending of ACM and awaiting answer indication
83
7.2.3.2.5
Coding of the ACM
86
7.2.3.2.6
Sending of the Call Progress message (CPG)
89
7.2.3.2.7
Coding of the CPG
91
7.2.3.2.7a
Receipt of 200 OK(INVITE)
93
7.2.3.2.7b
Internal through connection of the bearer path
93
7.2.3.2.8
Sending of the Answer Message (ANM)
93
7.2.3.2.9
Coding of the ANM
93
7.2.3.2.10
Sending of the Connect message (CON)
95
7.2.3.2.11
Coding of the CON
95
7.2.3.2.11A
Receipt of a reINVITE request
95
7.2.3.2.12
Receipt of Status Codes 4xx, 5xx or 6xx
95
7.2.3 2.13
Receipt of a BYE
98
7.2.3.2.14
Receipt of the Release Message
98
7.2.3.2.15
Receipt of RSC, GRS or CGB (H/W oriented)
98
7.2.3.2.16
Autonomous Release at O-MGCF
99
7.2.3.2.17
Special handling of 580 precondition failure received in response to either an INVITE or UPDATE
99
7.2.3.2.18
Sending of CANCEL
99
7.2.3.2.19
Receipt of SIP redirect (3xx) response
100
7.2.3.2.20
Sending of INFO for overlap signalling using the in-dialog method
100
7.2.3.3
Timers
101
7.3
Interworking between CS networks supporting BICC and the IM CN subsystem
101
7.3.1
Services performed by network entities in the control plane
102
7.3.2
Signalling interactions between network entities in the control plane
102
7.3.2.1
Signalling between the SS7 signalling function and MGCF
102
7.3.2.1.1
Signalling from MGCF to SS7 signalling function
102
7.3.2.1.2
Signalling from SS7 signalling function to MGCF
103
7.3.2.1.3
Services offered by STC, SCTP and M3UA
103
7.3.2.2
Signalling between the MGCF and SIP signalling function
103
7.3.3
SIP-BICC protocol interworking
103
7.3.3.1
Incoming call interworking from SIP to BICC at I-MGCF
103
7.3.3.1.1
Sending of IAM
103
7.3.3.1.2
Coding of IAM
103
7.3.3.1.2A
Coding of the IAM when Number Portability is supported
104
7.3.3.1.2B
Coding of the IAM for Carrier Routeing
104
7.3.3.1.3
Sending of COT
104
7.3.3.1.3A
Sending of SAM
105
7.3.3.1.4
Sending of 180 Ringing
105
7.3.3.1.4A
Sending of 183 Session Progress for early media scenarios
105
7.3.3.1.4B
Sending of 181 Call is being forwarded
105
7.3.3.1.4C
Sending of 183 Session Progress for overlap signalling using the in-dialog method
105
7.3.3.1.4D
Sending of 183 Session Progress to carry ISUP Cause
105
7.3.3.1.5
Sending of the 200 OK (INVITE)
105
7.3.3.1.6
Sending of the Release message (REL)
105
7.3.3.1.7
Coding of the REL
106
7.3.3.1.8
Receipt of the Release Message
106
7.3.3.1.9
Receipt of RSC, GRS or CGB (H/W oriented)
106
7.3.3.1.9a
Receipt of REFER
106
7.3.3.1.9b
Autonomous Release at I-MGCF
106
7.3.3.1.10
Internal through connection of the bearer path
106
7.3.3.1.11
Out of Band DTMF
106
7.3.3.2
Outgoing Call Interworking from BICC to SIP at O-MGCF
106
7.3.3.2.1
Sending of INVITE
106
7.3.3.2.1a
Sending of INVITE without determining the end of address signalling
107
7.3.3.2.2
Coding of the INVITE
107
7.3.3.2.2A
Coding of the INVITE when number portability is supported
108
7.3.3.2.2B
Coding of the INVITE for Carrier Routeing
108
7.3.3.2.2C
Coding of INVITE with instance-id in form of IMEI URN
108
7.3.3.2.3
Sending of UPDATE
108
7.3.3.2.4
Sending of ACM and Awaiting Answer indication
109
7.3.3.2.5
Coding of the ACM
109
7.3.3.2.6
Sending of the Call Progress message (CPG)
109
7.3.3.2.7
Coding of the CPG
109
7.3.3.2.7a
Receipt of 200 OK (INVITE)
109
7.3.3.2.7b
Internal through connection of the bearer path
109
7.3.3.2.8
Sending of the Answer Message (ANM)
109
7.3.3.2.9
Coding of the ANM
109
7.3.3.2.10
Sending of the Connect message (CON)
109
7.3.3.2.11
Coding of the CON
110
7.3.3.2.11A
Receipt of re-INVITE requests
110
7.3.3.2.12
Receipt of Status Codes 4xx, 5xx or 6xx
110
7.3 3.2.13
Receipt of a BYE
110
7.3.3.2.14
Receipt of the Release Message
110
7.3.3.2.15
Receipt of RSC, GRS or CGB (H/W oriented)
110
7.3.3.2.16
Out of Band DTMF
110
7.3.3.2.17
Sending of CANCEL
110
7.3.3.2.18
Autonomous Release at O-MGCF
110
7.3.3.2.19
Special handling of 580 precondition failure received in response to either an INVITE or UPDATE
110
7.3.3.2.20
Receipt of SIP redirect (3xx) response
111
7.3.3.2.21
Sending of INFO for overlap signalling using the in-dialog method
111
7.3.3.3
Timers
111
7.4
Supplementary Services
111
7.4.1
Calling line identification presentation/restriction (CLIP/CLIR)
111
7.4.2
Connected line presentation and restriction (COLP/COLR)
111
7.4.2.0
General
111
7.4.2.1
Incoming Call Interworking from SIP to BICC/ISUP at the I-MGCF
111
7.4.2.1.1
INVITE to IAM interworking (SIP to ISUP/BICC calls)
111
7.4.2.1.2
ANM/CON to 200 OK (INVITE)
112
7.4.2.2
Outgoing Call Interworking from BICC/ISUP to SIP at O-MGCF
112
7.4.2.2.1
IAM to INVITE interworking (ISUP to SIP calls)
112
7.4.2.2.2
1XX to ANM or CON interworking
113
7.4.2.2.3
200 OK (INVITE) to ANM/CON interworking
113
7.4.3
Direct Dialling In (DDI)
114
7.4.4
Malicious call identification
114
7.4.5
Subaddressing (SUB)
114
7.4.5.1
General
114
7.4.5.2
Interworking at I-MGCF
114
7.4.5.3
Interworking at O-MGCF
115
7.4.6
Call Forwarding Busy (CFB)/ Call Forwarding No Reply (CFNR) / Call Forwarding Unconditional (CFU) / Call Deflection (CD)
116
7.4.6.1
General
116
7.4.6.2
Interworking at the O-MGCF
117
7.4.6.2.1
General
117
7.4.6.2.2
Interworking SIP to ISUP
117
7.4.6.2.3
Interworking ISUP to SIP
121
7.4.6.3
Interworking at the I-MGCF
123
7.4.6.3.1
General
123
7.4.6.3.2
Interworking from SIP to ISUP
123
7.4.6.3.3
Interworking from ISUP to SIP
126
7.4.7
Void
131
7.4.8
Explicit Call Transfer (ECT)
131
7.4.9
Call Waiting
131
7.4.10
Call Hold
131
7.4.10.1
Session hold initiated from the IM CN subsystem side
131
7.4.10.2
Session hold initiated from the CS network side
132
7.4.11
Call Completion on busy subscriber
133
7.4.12
Completion of Calls on No Reply (CCNR)
134
7.4.13
Terminal Portability (TP)
134
7.4.14
Conference calling (CONF) / Three-Party Service (3PTY)
134
7.4.15
Void
134
7.4.16
Closed User Group (CUG)
134
7.4.17
Multi-Level Precedence and Pre-emption (MLPP)
134
7.4.18
Global Virtual Network Service (GVNS)
134
7.4.19
International telecommunication charge card (ITCC)
134
7.4.20
Reverse charging (REV)
135
7.4.21
User-to-User Signalling (UUS)
135
7.4.21.0
General
135
7.4.21.1
User-to-User Signalling (UUS) service 1 (implicit)
135
7.4.21.1.0
General
135
7.4.21.1.1
Void
135
7.4.21.1.2
User-to-user information Interworking from SIP to ISUP
135
7.4.21.1.3
User-to-user information Interworking from ISUP to SIP
135
7.4.21.2
User-to-User Signalling (UUS) service 1 (explicit)
136
7.4.21.3
User-to-User Signalling (UUS) service 2 (explicit)
136
7.4.21.4
User-to-User Signalling (UUS) service 3 (explicit)
136
7.4.22
Multiple Subscriber Number (MSN)
136
7.4.23
Anonymous Call rejection
136
7.4.23.1
ISUP-SIP protocol interworking at the I-MGCF
136
7.4.23.2
SIP-ISUP protocol interworking at the O-MGCF
136
7.4.24
Customized Alerting Tones (CAT) in the 3GPP CS domain
136
7.5
IMS Supplementary Services
136
7.5.1
Originating Identification Presentation (OIP) and Originating Identification Restriction (OIR)
137
7.5.2
Terminating Identification Presentation (TIP) and Terminating Identification Restriction (TIR)
137
7.5.2.1
General
137
7.5.2.2
Interworking at the O-MGCF
137
7.5.2.3
Interworking at the I-MGCF
138
7.5.2.4
Timer
139
7.5.3
void
139
7.5.4
Communication Diversion (CDIV)
139
7.5.4.1
General
139
7.5.4.2
Interworking at the O-MGCF
140
7.5.4.2.1
General
140
7.5.4.2.2
Call forwarding within the ISUP Network appeared
145
7.5.4.3
Interworking at the I-MGCF
147
7.5.5
Communication Hold (HOLD)
155
7.5.6
Conference call (CONF)
155
7.5.6.1
General
155
7.5.6.2
Subscribing for the conference event package
155
7.5.6.3
Interworking the notification
155
7.5.7
 Anonymous Communication Rejection (ACR) and Communication Barring (CB)
156
7.5.8
 Message Waiting Indication (MWI)
156
7.5.9
 Malicious Communication Identification (MCID)
156
7.5.9.0
General
156
7.5.9.1
Interworking at the O-MGCF
157
7.5.9.1.0
General
157
7.5.9.1.1
Interworking of the MCID XML Request schema with the ISUP MCID request indicators
157
7.5.9.1.2
Interworking of the ISUP MCID response indicators with the MCID XML Response schema
157
7.5.9.1.3
Interworking of the ISUP Calling Party Number in an Identification Response with the OrigPartyIdentity within the MCID XML Response schema
158
7.5.9.1.4
Interworking of the ISUP Generic Number in an Identification Response with the GenericNumber within the MCID XML Response schema
158
7.5.9.2
Interworking at the I-MGCF
158
7.5.9.2.1
General
158
7.5.9.2.2
Interworking of identification Request
158
7.5.9.2.3
Interworking of identification Response
158
7.5.10
Closed User Group (CUG)
159
7.5.10.0
General
159
7.5.10.1
Interworking at the I-MGCF
159
7.5.10.2
Interworking at the O-MGCF
160
7.5.11
CCBS/CCNR
161
7.5.11.0
General
161
7.5.11.1
Interworking at the I-MGCF
161
7.5.11.2
Interworking at the O-MGCF
163
7.5.12
Communication Waiting (CW)
165
7.5.12.0
General
165
7.5.12.1
Interworking at the I-MGCF
165
7.5.12.2
Interworking at the O-MGCF
165
7.5.13
 Customized Alerting Tones (CAT)
165
7.5.13.1
General
165
7.5.13.2
Early session model
165
7.5.13.2.1
Interworking at the O-MGCF
165
7.5.13.2.2
MGCF – IM-MGW Interaction for CAT service
166
7.5.13.3
Forking model
170
7.5.13.4
Gateway model
170
8
User plane interworking
170
8.1
Interworking between IM CN subsystem and bearer independent CS network
170
8.1.1
Transcoder-less Mb to Nb Interworking
171
8.1.1.1
Initialisation
171
8.1.1.2
Time alignment
171
8.1.1.3
Rate control
171
8.1.1.4
Frame quality indication
172
8.1.1.5
Framing
172
8.1.1.6
Transcoding
172
8.1.1.7
Discontinuous transmission
172
8.1.1.8
Timing and sequence information
172
8.2
Interworking between IM CN subsystem and TDM-based CS network
173
8.3
Transcoding requirements
173
8.4
Diffserv code point requirements
173
8.5
DTMF handling
174
9
MGCF – IM-MGW Interaction
174
9.1
Overview
174
9.2
Mn signalling interactions
174
9.2.1
Network model
174
9.2.2
Basic IM CN subsystem originated session
175
9.2.2.1
BICC forward bearer establishment
175
9.2.2.1.1
IM-MGW selection
175
9.2.2.1.2
CS network side bearer establishment
175
9.2.2.1.3
IM CN subsystem side termination reservation
175
9.2.2.1.4
IM CN subsystem side session establishment
176
9.2.2.1.5
Through-connection
176
9.2.2.1.6
Codec handling
176
9.2.2.1.7
Failure handling in MGCF
176
9.2.2.1.8
Message sequence chart
177
9.2.2.2
BICC backward bearer establishment
179
9.2.2.2.1
IM-MGW selection
179
9.2.2.2.2
IM CN subsystem side termination reservation
179
9.2.2.2.3
IM CN subsystem side session establishment
179
9.2.2.2.4
CS network side bearer establishment
180
9.2.2.2.5
Through-connection
180
9.2.2.2.6
Codec handling
180
9.2.2.2.7
Failure handling in MGCF
180
9.2.2.2.8
Message sequence chart
180
9.2.2.3
ISUP
181
9.2.2.3.1
IM-MGW selection
181
9.2.2.3.2
IM CN subsystem side termination reservation
182
9.2.2.3.3
IM CN subsystem side session establishment
182
9.2.2.3.4
CS network side circuit reservation
182
9.2.2.3.5
Through-connection
183
9.2.2.3.6
Continuity check
183
9.2.2.3.7
Codec handling
183
9.2.2.3.8
Voice processing function
183
9.2.2.3.9
Failure handling in MGCF
183
9.2.2.3.10
Message sequence chart
183
9.2.3
Basic CS network originated session
185
9.2.3.1
BICC forward bearer establishment
185
9.2.3.1.1
IM-MGW selection
185
9.2.3.1.2
IM CN subsystem side termination reservation
185
9.2.3.1.3
IM CN subsystem side session establishment
185
9.2.3.1.4
CS network side bearer establishment
185
9.2.3.1.5
Called party alerting
186
9.2.3.1.6
Called party answer
186
9.2.3.1.7
Through-Connection
186
9.2.3.1.8
Codec handling
186
9.2.3.1.9
Failure handling in MGCF
186
9.2.3.1.10
Message sequence chart
186
9.2.3.2
BICC Backward bearer establishment
188
9.2.3.2.1
IM-MGW selection
188
9.2.3.2.2
CS network side bearer establishment
188
9.2.3.2.3
IM CN subsystem side termination reservation
188
9.2.3.2.4
IM CN subsystem side session establishment
189
9.2.3.2.5
Called party alerting
189
9.2.3.2.6
Called party answer
189
9.2.3.2.7
Through-Connection
189
9.2.3.2.8
Codec handling
189
9.2.3.2.9
Failure handling in MGCF
190
9.2.3.2.10
Message sequence chart
190
9.2.3.3
ISUP
192
9.2.3.3.1
IM-MGW selection
192
9.2.3.3.2
CS network side circuit reservation
192
9.2.3.3.3
IM CN subsystem side termination reservation
192
9.2.3.3.4
IM CN subsystem side session establishment
192
9.2.3.3.5
Called party alerting
193
9.2.3.3.6
Called party answer
193
9.2.3.3.7
Through-Connection
193
9.2.3.3.8
Continuity Check
193
9.2.3.3.9
Codec handling
193
9.2.3.3.10
Voice Processing function
194
9.2.3.3.11
Failure handling in MGCF
194
9.2.3.3.12
Message sequence chart
194
9.2.3.4
Handling of Forking
196
9.2.3.4.1
Detection of Forking
196
9.2.3.4.2
IM CN subsystem side session establishment
196
9.2.3.4.3
IM CN subsystem side session establishment completion
197
9.2.3.4.4
Message sequence chart
197
9.2.4
Session release initiated from IM CN subsystem side
200
9.2.4.1
BICC
200
9.2.4.1.1
Session release in the IM CN subsystem side
200
9.2.4.1.2
Session release in the CS network side
200
9.2.4.1.3
Message sequence chart
200
9.2.4.2
ISUP
201
9.2.4.2.1
Session release in the IM CN subsystem side
201
9.2.4.2.2
Session release in the CS network side
201
9.2.4.2.3
Message sequence chart
201
9.2.5
Session release initiated from CS network side
201
9.2.5.1
BICC
201
9.2.5.1.1
Session release in the CS network side
201
9.2.5.1.2
Session release in the IM CN subsystem side
202
9.2.5.1.3
Message sequence chart
202
9.2.5.2
ISUP
202
9.2.5.2.1
Session release in the CS network side
202
9.2.5.2.2
Session release in the IM CN subsystem side
203
9.2.5.2.3
Message sequence chart
203
9.2.6
Session release initiated by MGCF
203
9.2.6.1
BICC
203
9.2.6.1.1
Session release in the CS network side
203
9.2.6.1.2
Session release in the IM CN subsystem side
203
9.2.6.1.3
Message sequence chart
204
9.2.6.2
ISUP
204
9.2.6.2.1
Session release in the CS network side
204
9.2.6.2.2
Session release in the IM CN subsystem side
204
9.2.6.2.3
Message sequence chart
204
9.2.7
Session release initiated by IM-MGW
205
9.2.7.1
BICC
205
9.2.7.1.1
Session release in the CS network side
205
9.2.7.1.2
Session release in the IM CN subsystem side
205
9.2.7.1.3
Message sequence chart
205
9.2.7.2
ISUP
206
9.2.7.2.1
Session release in the CS network side
206
9.2.7.2.2
Session release in the IM CN subsystem side
206
9.2.7.2.3
Message sequence chart
207
9.2.8
Handling of RTP telephone events
207
9.2.8.1
Sending DTMF digits out-of-band to CS CN (BICC)
208
9.2.8.2
Sending and receiving DTMF digits inband to/from CS CN (ISUP or BICC)
210
9.2.8.3
Receiving DTMF digits out-of-band from CS CN (BICC)
211
9.2.9
Session hold initiated from IM CN subsystem
212
9.2.10
Session hold initiated from CS network
214
9.2.11
Explicit Congestion Notification Support
215
9.2.11.1
General
215
9.2.11.1a
Incoming Call Interworking from SIP to ISUP/BICC at I-MGCF
215
9.2.11.1b
Outgoing Call Interworking from ISUP/BICC to SIP at O-MGCF
216
9.2.11.1c
Detection of ECN failures by IM-MGW
216
9.2.11.2
Message sequence chart
216
9.3
Mn Signalling procedures
217
9.3.1
Procedures related to terminations towards the IM CN Subsystem
217
9.3.1.1
Reserve IMS connection point
218
9.3.1.2
Configure IMS resources
219
9.3.1.3
Reserve IMS Connection point and configure remote resources
220
9.3.1.4
Release IMS termination
221
9.3.1.5
Detect IMS RTP Tel event
221
9.3.1.6
Notify IMS RTP Tel event
221
9.3.1.7
Void
221
9.3.1.8
Send IMS RTP Tel event
221
9.3.1.9
Stop IMS RTP Tel event
221
9.3.1.10
Termination heartbeat indication
222
9.3.1.11
IMS Bearer Released
222
9.3.1.12
End IMS RTP Tel event
222
9.3.1.13
IMS Send Tone
222
9.3.1.14
IMS Stop Tone
222
9.3.1.15
IMS Tone Completed
222
9.3.1.16
ECN Failure Indication
223
9.3.2
Procedures related to a termination towards an ISUP network
223
9.3.2.1
Reserve TDM circuit
223
9.3.2.2
Change TDM through-connection
224
9.3.2.3
Activate TDM voice-processing function
224
9.3.2.4
Send TDM tone
224
9.3.2.5
Stop TDM tone
224
9.3.2.6
Play TDM announcement
224
9.3.2.7
TDM announcement completed
224
9.3.2.8
Stop TDM announcement
224
9.3.2.9
Continuity check
224
9.3.2.10
Continuity check verify
225
9.3.2.11
Continuity check response
225
9.3.2.12
Release TDM termination
226
9.3.2.13
Termination Out-of-Service
226
9.3.2.14
Termination heartbeat indication
226
9.3.2.15
Bearer Released
226
9.3.2.16
TDM tone completed
226
9.3.3
Procedures related to a termination towards a BICC network
227
9.3.4
Non-call related procedures
227
9.3.5
Multiple IP Realms
228
Annex A (informative):
 Void
230
Annex B (normative):
Codec Negotiation between a BICC CS network and the IM CN subsystem
231
B.1
Introduction
231
B.2
Control plane interworking
231
B.2.1
Incoming call interworking from SIP to BICC at I-MGCF
231
B.2.1.1
Sending of IAM
231
B.2.1.2
Sending of SDP answer
231
B.2.2
Outgoing call interworking from BICC to SIP at O-MGCF
232
B.2.2.1
Sending of INVITE
232
B.2.2.2
Responding to serving node initiating codec negotiation
232
B.2.3
Mid-call interworking from SIP to BICC at I-MGCF or O-MGCF
232
B.2.3.1
Receipt of SDP offer
232
B.2.3.2
Generating SDP answer
233
B.2.4
Mid-call interworking from BICC to SIP at I-MGCF or O-MGCF
233
B.2.4.1
Receipt of mid-call codec negotiation request
233
B.2.4.2
Responding to serving node initiating mid-call codec negotiation
233
B.2.4.3
Receipt of codec modification request
234
B.2.5
Codec parameter translation between BICC CS network and the IM CN subsystem
234
B.2.5.1
Codec parameters for 3GPP AMR-NB codecs
235
B.2.5.2
Codec parameters for 3GPP AMR-WB codecs
237
B.2.5.3
Codec parameters for 3GPP non-AMR codecs
238
B.2.5.4
Codec parameters for ITU-T codecs
238
B.3
MGCF – IM-MGW interaction during interworking of codec negotiation
239
B.3.1
Basic IM CN subsystem originated session
239
B.3.1.1
BICC forward bearer establishment
240
B.3.1.1.1
IM-MGW selection
240
B.3.1.1.2
CS network side bearer establishment
240
B.3.1.1.3
IM CN subsystem side session establishment
240
B.3.1.1.4
Through-connection
240
B.3.1.1.5
Codec handling
241
B.3.1.1.6
Failure handling in MGCF
241
B.3.1.1.7
Message sequence chart
241
B.3.2
Basic CS network originated session
243
B.3.2.1
BICC forward bearer establishment
243
B.3.2.1.1
IM-MGW selection
243
B.3.2.1.2
IM CN subsystem side termination reservation
243
B.3.2.1.3
IM CN subsystem side session establishment
243
B.3.2.1.4
CS network side bearer establishment
243
B.3.2.1.5
Called party alerting
244
B.3.2.1.6
Called party answer
244
B.3.2.1.7
Through-Connection
244
B.3.2.1.8
Codec handling
244
B.3.2.1.9
Failure handling in MGCF
244
B.3.2.1.10
Message sequence chart
244
B.3.3
CS network initiated mid-call codec negotiation
247
B.3.4
IM CN subsystem initiated mid-call codec negotiation
248
Annex C (normative):
Interworking of CPC parameter
249
C.1
Interworking SIP to ISUP
249
C.2
Interworking ISUP to SIP
250
Annex D:
Void
250
Annex E (normative):
Multimedia interworking between the IP Multimedia Core Network (CN) Subsystem (IMS) and Circuit Switched (CS) networks
251
E.1
Basic Multimedia calls interworking between the IMS and CS Networks scenarios
251
E.2
Control plane interworking
252
E.2.1
General
252
E.2.2
Functionalities required in the MGCF for multimedia calls support
252
E.2.3
IM CN subsystem originated session
252
E.2.3.1
Preconditions used at IMS side
252
E.2.3.1.1
Interactions between H.245 or MONA and SIP/SDP
252
E.2.3.2
Preconditions not used at IMS side
254
E.2.3.2.1
Interactions between H.245 or MONA and SIP/SDP
254
E.2.3.3
Fallback to speech at session establishment
256
E.2.4
CS network originated session
256
E.2.4.1
Interactions between SIP/SDP and H.245 or MONA
256
E.2.4.1.1
Normal Call setup
256
E.2.4.1.2
Call setup if multimedia call can not be recognized in an unambiguous manner
258
E.2.4.1.3
Fallback to speech during call setup
258
E.2.4.2
CS originated - IM CN transit - CS terminated
259
E.2.5
Service change
260
E.2.5.2.1
SCUDIF
260
E.2.5.2.1.0
General
260
E.2.5.2.1.1
IM CN subsystem originated change
260
E.2.5.2.1.2
CS network originated change
262
E.2.5.2.2
Non-SCUDIF case (ISUP or BICC without SCUDIF)
263
E.2.5.2.2.1
Change from multimedia to audio
263
E.2.5.2.2.2
Change from speech to multimedia
264
E.2.6
Call release
265
E.2.6.1
Call release initiated from the IM CN subsystem side
265
E.2.6.2
Call release initiated from the CS network side
266
E.2.6.3
Call release initiated from the interworking node
266
E.3
User plane interworking
267
E.3.1
Functionalities required in the IM-MGW for multimedia calls support
267
E.4
MGCF and IM-MGW interactions
267
E.4.1
Introduction
267
E.4.2
Mn signalling interactions
268
E.4.2.1
Introduction
268
E.4.2.2
H.248 Context Model
268
E.4.2.3
Transport of H.245 messages between the MGCF and IM-MGW
269
E.4.2.3.1
General
269
E.4.2.3.2
Transport from MGCF to IM-MGW
269
E.4.2.3.3
Transport from IM-MGW to MGCF
269
E.4.2.4
Call establishment procedure
270
E.4.2.5
Handling of H.245 indication message
272
E.4.2.5.1
Overview
272
E.4.2.5.2
Function Not Understood / Function Not Supported message
272
E.4.2.5.3
User Input Indication message
273
E.4.2.6
Handling of H.245 Command message
273
E.4.2.6.1
Overview
273
E.4.2.6.2
Flow control command
273
E.4.2.6.3
End Session Command
274
E.4.2.6.4
videoFastUpdatePicture
274
E.4.2.7
Mn Signalling Interactions to support the Media Oriented Negotiation Acceleration (MONA)
275
E.4.2.7.1
Overview
275
E.4.2.7.2
Mn Interactions for MONA preference messages
277
E.4.2.7.3
Mn Interactions for MONA MPCs
279
E.4.2.7.4
Mn Interactions for MONA SPCs
281
E.4.2.7.4.1
General
281
E.4.2.7.4.2
Transport from MGCF to IM-MGW
281
E.4.2.7.4.3
Transport from IM-MGW to MGCF
282
E.4.2.7.4.4
Termination of SPC procedure
283
E.4.2.7.5
Mn Interactions for fallback from MONA procedures to standard H.324 setup
284
E.4.2.8
Interworking between RTCP messages and H.245 messages
285
E.4.2.8.1
Overview
285
E.4.2.8.2
IM CN subsystem originated RTCP messages
285
E.4.2.8.3
CS network originated H.245 messages
286
E.4.3
Mn Signalling procedures
287
E.4.3.1
Overview
287
E.4.3.2
Add Multiplex Termination
288
E.4.3.3
Configure Multiplex Termination
289
E.4.3.4
Signal H245 Message
290
E.4.3.5
Notify H245 Message
290
E.4.3.6
Notify MONA Preference Reception
291
E.4.3.7
Notify MONA Preference Completed
291
E.4.3.8
Signal SPC
292
E.4.3.9
Notify SPC
292
E.4.3.10
Notify MPC
293
E.4.3.11
Notify Detection of Legacy Interworking
293
E.4.3.12
Request RTCP-Interworking
294
E.4.3.13
Notify RTCP-Interworking
294
E.4.3.14
Signal-H.245-Interworking
295
E.4.3.15
Stop MPC
295
E.4.3.16
Stop SPC
296
E.4.3.17
Stop MONA Negotiation
297
Annex F (normative):
PSTN XML Scheme
298
F.1
Scope
298
F.2
MIME type
298
F.3
XML Schema definition
298
F.4
IANA registration template
304
Annex G (informative):
Void
306
Annex H (normative):
Interworking of Originating Line Information (OLI) parameter (network option)
307
H.1
Interworking SIP to ISUP
307
H.2
Interworking ISUP to SIP
307
Annex I (normative):
GTT interworking between the IP Multimedia Core Network (CN) Subsystem (IMS) and Circuit Switched (CS) networks
308
I.1
Overview of GTT interworking between the IMS and Circuit Switched (CS) networks
308
I.2
Control plane interworking
309
I.2.1
General
309
I.2.2
Functionalities required in the MGCF for GTT calls support
309
I.2.3
IM CN subsystem originated session
309
I.2.3.1
Initial INVITE with an SDP offer including a text media line
309
I.2.4
CS network originated session
310
I.2.4.1
General
310
I.2.4.2
Initial INVITE with an SDP offer including audio only
310
I.2.5
Subsequent SDP offer/answer exchange adding text to an audio session
311
I.3
User plane interworking
311
I.3.1
Functionalities required in the IM-MGW for GTT support
311
I.3.2
Monitoring of text/modem signals on the CS side
311
I.3.3
Multiplexing between the CS and IMS streams
312
I.3.4
Conversion between text/modem and Real-Time Text over RTP
312
I.4
MGCF and IM-MGW interactions
312
I.4.1
Introduction
312
I.4.2
Mn signalling interactions
313
I.4.2.1
Introduction
313
I.4.2.2
H.248 Context Model
313
I.4.2.3
Specific Mn signalling for GTT
313
I.4.2.4
IM CN subsystem originated session between the MGCF and IM-MGW
314
I.4.2.4.1
Initial INVITE with an SDP offer including Real-Time Text
314
I.4.2.5
CS network originated session
316
I.4.2.5.1
Initial INVITE with an SDP offer only including audio
316
I.4.3
Mn Signalling procedures
317
I.4.3.1
Overview
317
Annex J (informative):
Call Flow for Customized Alerting Tone (CAT) interworking between CS network and IMS
318
J.1
Introduction
318
J.2
IMS CAT provided by the terminating IMS user using gateway model towards user served in CS networks
318
J.2.1
Callflow without using precondition mechanism
318
Annex K (informative):
Change history
320


Foreword
This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x
the first digit:

1
presented to TSG for information;

2
presented to TSG for approval;

3
or greater indicates TSG approved document under change control.

y
the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z
the third digit is incremented when editorial only changes have been incorporated in the document.

1
Scope

The present document specifies the principles of interworking between the 3GPP IM CN subsystem and BICC/ISUP based legacy CS networks, in order to support IM basic voice, data and multimedia calls.

The present document addresses the areas of control and user plane interworking between the IM CN subsystem and CS networks through the network functions, which include the MGCF and IM-MGW. For the specification of control plane interworking, areas such as the interworking between SIP and BICC or ISUP are detailed in terms of the processes and protocol mappings required for the support of both IM originated and terminated voice and multimedia calls.

Other areas addressed encompass the transport protocol and signalling issues for negotiation and mapping of bearer capabilities and QoS information.

The present document specifies the interworking between 3GPP profile of SIP (as detailed according to TS 24.229 [9]) and BICC or ISUP, as specified in ITU-T Recommendations Q.1902.1 to Q.1902.6 [30] and ITU-T Q761 to Q764 [4] respectively.

The present document also specifies the interworking between circuit switched multimedia telephony service, as described in TS 26.110 [78] TS 26.111 [79], and ITU-T Recommendation H.324 [81] and packet switched multimedia services, as described in TS 26.235 [80] and TS 26.236 [32], in particular and the interworking between the 3GPP profile of SIP and the inband control protocols for multimedia communication as specified in ITU-T Recommendations H.245 [82] and H.324 Annex K [81].

The present document addresses two interworking scenarios with respect to the properties of the CS network:

-
The CS network does not use any 3GPP specific additions.

-
The CS network uses 3GPP specific additions.

2
References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

· References are either specific (identified by date of publication, edition number, version number, etc.) or non‑specific.

· For a specific reference, subsequent revisions do not apply.

· For a non-specific reference, the latest version applies.In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]
ITU-T Recommendation G.711 (11/88): "Pulse Code Modulation (PCM) of voice frequencies".

[2]
ITU-T Recommendation H.248.1 (05/02): "Gateway control protocol: Version 2".

[3]
ITU-T Recommendation Q.701 (03/93), Q.702 (11/88), Q.703 (07/96), Q.704 (07/96), Q.705 (03/93), Q.706 (03/93), Q.707 (11/88), Q.708 (03/99), Q.709 (03/93): "Functional description of the message transfer part (MTP) of Signalling System No. 7".

[4]
ITU-T Recommendations Q.761to Q.764 (12/99): "Specifications of Signalling System No.7 ISDN User Part (ISUP)".

[5]
Void.

[6]
3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[7]
Void.

[8]
3GPP TS 24.228: "Signalling flows for the IP multimedia call control based on SIP and SDP".

[9]
3GPP TS 24.229: "IP Multimedia Call Control Protocol based on SIP and SDP".

[10]
3GPP TS 23.002: "Network Architecture".

[11]
3GPP TS 22.228: "Service requirements for the IP Multimedia Core Network Subsystem".

[12]
3GPP TS 23.228: "IP Multimedia subsystem (IMS)".

[13]
Void.

[14]
3GPP TS 29.205: "Application of Q.1900 series to Bearer Independent CS Network architecture; Stage 3".

[15]
3GPP TS 29.332: "Media Gateway Control Function (MGCF) – IM-Media Gateway (IM-MGW) interface, Stage 3".

[16]
IETF RFC 791: "Internet Protocol".

[17]
IETF RFC 768: "User Datagram Protocol".

[18]
IETF RFC 2960: "Stream Control Transmission Protocol".

[19]
IETF RFC 3261: "SIP: Session Initiation Protocol".

[20]
3GPP TS 29.202: "Signalling System No. 7 (SS7) signalling transport in core network; Stage 3".

[21]
IETF RFC 2474: "Definition of the Differentiated Services Field (DS Field) in the IPv4 and IPv6 Headers".

[22]
IETF RFC 2475: "An Architecture for Differentiated Services".

[23]
IETF RFC 4867: "RTP Payload Format and File Storage Format for the Adaptive Multi-Rate (AMR) and Adaptive Multi-Rate Wideband (AMR-WB) Audio Codecs".

[24]
IETF RFC 793: "Transmission Control Protocol".

[25]
3GPP TS 29.414: "Core network Nb data transport and transport signalling".

[26]
3GPP TS 29.415: "Core network Nb interface user plane protocols".

[27]
3GPP TS 23.205: "Bearer-independent circuit-switched core network; Stage 2".

[28]
Void.

[29]
ITU-T Recommendation Q.2150.1: "Signalling transport converter on MTP3 and MTP3b".

[30]
ITU-T Recommendations Q.1902.1 to Q.1902.6 (07/01): "Bearer Independent Call Control".

[31]
Void.
[32]
3GPP TS 26.236: "Packet switched conversational multimedia applications; Transport protocols".

[33]
Void.
[34]
Void.
[35]
ITU-T Recommendation Q.765.5: "Signalling system No. 7 – Application transport mechanism: Bearer Independent Call Control (BICC)".

[36]
IETF RFC 3264: "An Offer/Answer Model with the Session Description Protocol (SDP)".

[37]
IETF RFC 3312: "Integration of Resource Management and Session Initiation Protocol (SIP)".

[38]
ITU-T Recommendation Q.850 (05/1998) including Amendment 1 (07/2001): "Usage of cause and location in the Digital Subscriber Signalling System No. 1 and the Signalling System No. 7 ISDN User Part".

[39]
IETF RFC 2460: "Internet Protocol, Version 6 (IPv6) Specification".
[40]
IETF RFC 3323: "A Privacy Mechanism for the Session Initiation Protocol (SIP)".

[41]
IETF RFC 3325: "Private Extensions to the Session Initiation Protocol (SIP) for Asserted Identity within Trusted Networks".

[42]
ITU-T Recommendation Q.730 (12/99), Q.731.1 (07/96), Q.731.3 (03/93), Q.731.4 (03/93), Q.731.5 (03/93), Q.731.6 (03/93), Q.731.7 (06/97), Q.731.8 (02/92), Q.732.2-5 (12/99), Q.732.7 (07/96), Q.733.1 (02/92), Q.733.2 (03/93), Q.733.3 (06/97), Q.733.4 (03/93), Q.733.5 (12/99), Q.734.1 (03/93), Q.734.2 (07/96), Q.735.1 (03/93), Q.735.3 (03/93), Q.735.6 (07/93), Q.736.1 (10/95), Q.736.3 (10/95), Q.737.1 (06/97): "ISDN user part supplementary services".

[43]
ITU-T Recommendation I.363.5 (1996): "B-ISDN ATM Adaptation Layer specification: Type 5 AAL".

[44]
ITU-T Recommendation Q.2110 (1994): "B-ISDN ATM adaptation layer - Service Specific Connection Oriented Protocol (SSCOP)".

[45]
ITU-T Recommendation Q.2140 (1995): "B-ISDN ATM adaptation layer - Service specific coordination function for signalling at the network node interface (SSCF AT NNI)".

[46]
ITU-T Recommendation Q.2210 (1996): "Message transfer part level 3 functions and messages using the services of ITU-T Recommendation Q.2140".

[47]
3GPP TS 23.221: "Architectural requirements".

[48]
ITU-T Recommendation E.164 (05/1997): "The international public telecommunication numbering plan".

[49]
ITU-T Recommendation Q.1912.5 (03/04): "Interworking between Session Initiation Protocol (SIP) and Bearer Independent Call Control Protocol or ISDN User Part".

[50]
3GPP TS 26.102: "Adaptive Multi-Rate (AMR) speech codec; Interface to Iu and Uu".

[51]
IETF RFC 3550: "RTP: A Transport Protocol for Real-Time Applications".

[52]
IETF RFC 3551: "RTP Profile for Audio and Video Conferences with Minimal Control".

[53]
IETF RFC 3555: "MIME Type Registration of RTP Payload Formats".

[54]
IETF RFC 3262: "Reliability of provisional responses".

[55]
IETF RFC 3311: "SIP UPDATE method".

[56]
IETF RFC 4566: "SDP: Session Description Protocol".

[57]
3GPP TS 26.103: "Speech Codec List for GSM and UMTS".

[58]
3GPP TS 28.062: "Inband Tandem Free Operation (TFO) of speech codecs".
[59]
IETF RFC 3556: "Session Description Protocol (SDP) Bandwidth Modifiers for RTP Control Protocol (RTCP) bandwidth".

[60]
3GPP TS 24.604: "Communication Diversion (CDIV) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".
[61]
3GPP TS 24.605: "Conference (CONF) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".
[62]
3GPP TS 24.606: "Message Waiting Indication (MWI) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".
[63]
3GPP TS 24.607: "Originating Identification Presentation (OIP) and Originating Identification Restriction (OIR) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".
[64]
3GPP TS 24.608: "Terminating Identification Presentation (TIP) and Terminating Identification Restriction (TIR) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".
[65]
3GPP TS 24.610: "Communication HOLD (HOLD) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".
[66]
Void.
[67]
3GPP TS 24.611: "Anonymous Communication Rejection (ACR) and Communication Barring (CB) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".
[68]
Void.
[69]
IETF RFC 4040: "RTP Payload Format for a 64 kbit/s Transparent Call".
[70]
ETSI EN 300 356-1 (V4.2.1): "Integrated Services Digital Network (ISDN); Signalling System No.7 (SS7); ISDN User Part (ISUP) version 4 for the international interface; Part 1: Basic services [ITU-T Recommendations Q.761 to Q.764 (1999) modified]".

[71]
ETSI EN 300 356-21 (V4.2.1): "Integrated Services Digital Network (ISDN); Signalling System No.7 (SS7); ISDN User Part (ISUP) version 4 for the international interface; Part 21: Anonymous Call Rejection (ACR) supplementary service".
[72]
ITU-T Recommendation T.38 (06/98): "Procedures for real-time Group 3 facsimile communication over IP networks".
[73]
IETF RFC 3362: "Real-time Facsimile (T.38) - image/t38 MIME Sub-type Registration".
[74]
3GPP TS 23.003: "Numbering, addressing and identification".

[75]
IETF RFC 3515: "The Session Initiation Protocol (SIP) REFER method".
[76]
Void.
[77]
IETF RFC 5079: "Rejecting Anonymous Requests in the Session Initiation Protocol (SIP)".
[78]
3GPP TS 26.110: "Codec for circuit switched multimedia telephony service; General description".
[79]
3GPP TS 26.111: "Codec for Circuit switched Multimedia Telephony Service; Modifications to H.324".

[80]
3GPP TS 26.235: "Packet switched conversational multimedia applications; Default codecs".

[81]
ITU-T Recommendation H.324 (04/09): "Terminal for low bitrate multimedia communication".

[82]
ITU-T Recommendation H.245: "Control protocol for multimedia communication".

[83]
ITU-T Recommendation H.261 (03/93): "Video codec for audiovisual services at p x 64 kbit/s".

[84]
ITU-T Recommendation H.263 (01/05): "Video coding for low bitrate communication".

[85]
Void.
[86]
Void.
[87]
Void.

[88]
3GPP TS 24.173: "IMS Multimedia Telephony Communication Service and Supplementary Services, stage 3".

[89]
IETF RFC 5009: "Private Header (P-Header) Extension to the Session Initiation Protocol (SIP) for Authorization of Early Media".

[90]
IETF RFC 2663: "IP Network Address Translator (NAT) Terminology and Considerations".

[91]
IETF RFC 4244: "An extension to the Session Initiation Protocol (SIP) for Request History Information".

[92]
ITU-T Recommendation Q.769.1 (12/99): "Signalling system No. 7 – ISDN user part enhancements for the support of number portability".

[93]
IETF RFC 4694: "Number portability parameters for the "tel" URI".

[94]
Void.
[95]
Void.

[96]
ETSI EN 300 403‑1 (V1.3.2): "Integrated Services Digital Network (ISDN); Digital Subscriber Signalling System No. one (DSS1) protocol; Signalling network layer for circuit-mode basic call control; Part 1: Protocol specification [ITU-T Recommendation Q.931 (1993), modified]".

[97]
IETF RFC 3966: "The tel URI for Telephone Numbers".

[98]
Void.
[99]
IETF RFC 7433: "A Mechanism for Transporting User-to-User Call Control Information in SIP".
[99A]
IETF RFC 7434: "Interworking ISDN Call Control User Information with SIP".
[100]
IETF RFC 4575: "A Session Initiation Protocol (SIP) Event Package for Conference State".

[101]
3GPP TS 24.654: "Closed User Group (CUG) using IP Multimedia (IM) Core Network (CN) subsystem, Protocol Specification".
[102]
3GPP TS 24.616: "Malicious Communication Identification (MCID) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".
[103]
3GPP TS 23.014: "Technical Specification Group Core Network; Support of Dual Tone Multi-Frequency (DTMF) signalling".

[104]
3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[105]
IETF RFC 4733: "RTP Payload for DTMF Digits, Telephony Tones and Telephony Signals".
[106]
IETF RFC 6910: "Completion of Calls for the Session Initiation Protocol (SIP)".
[107]
IETF RFC 7462: "URNs for the Alert-Info Header Field of the Session Initiation Protocol (SIP)".
[108]
IETF RFC 4715: "The Integrated Services Digital Network (ISDN) Subaddress Encoding Type for tel URI".
[109]
IETF RFC 4585: "Extended RTP Profile for Real-time Transport Control Protocol (RTCP)-Based Feedback (RTP/AVPF)".
[110]
IETF RFC 5104: "Codec Control Messages in the RTP Audio-Visual Profile with Feedback (AVPF)".
[111]
3GPP TS 24.615: "Communication Waiting (CW) using IP Multimedia (IM) Core Network (CN) subsystem, Protocol Specification".

[112]
3GPP TS 24.642: "Completion of Communications to Busy Subscriber (CCBS) Completion of Communications by No Reply (CCNR) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".
[113]
IETF RFC 4458: "Session Initiation Protocol (SIP) URIs for Applications such as Voicemail and Interactive Voice Response (IVR)".
[114]
IETF RFC 5993: "RTP Payload format for Global System for Mobile Communications Half Rate (GSM-HR)".
[115]
IETF RFC 6432: "Carrying Q.850 Codes in Reason Header Fields in SIP (Session Initiation Protocol) Responses".
[116]
IETF RFC 3326: "The Reason Header Field for the Session Initiation Protocol (SIP)".

[117]
ANSI Standard ATIS-1000113.2005(R2010) (07/2005): "Signalling System No. 7 (SS7) – Integrated Services Digital Network (ISDN) User Part".

[118]
3GPP TS 23.237: "IP Multimedia subsystem (IMS) Service Continuity; Stage 2".

[119]
IETF RFC 5626: "Managing Client Initiated Connections in the Session Initiation Protocol (SIP)".

[120]
IETF RFC 7254: "A Uniform Resource Name Namespace for the Global System for Mobile Communications Association (GSMA) and the International Mobile station Equipment Identity (IMEI)".

[121]
3GPP TS 23.172: "Technical realization of Circuit Switched (CS) multimedia service UDI/RDI fallback and service modification; Stage 2".
[122]
3GPP TS 23.226: "Global Text Telephony (GTT); stage 2".

[123]
ITU‑T Recommendation T.140 (02/98): "Text conversation presentation protocol".

[124]
IETF RFC 4103: "RTP Payload for Text Conversation".

[125]
ITU‑T Recommendation V.18 (11/00): "Operational and interworking requirements for DCEs operating in the text telephone mode" including V.18 (2000) Ammendment 1 (11/02): "Harmonization with ANSI TIA/EIA-825 (2000) text phones".

[126]
3GPP TS 26.226: "Cellular text telephone modem; General description".
[127]
Void.

[128]
Void.
[129]
IETF RFC 3168: "The Addition of Explicit Congestion Notification (ECN) to IP".

[130]
IETF RFC 6679: "Explicit Congestion Notification (ECN) for RTP over UDP".

[131]
IETF RFC 3959: "The Early Session Disposition Type for the Session Initiation Protocol (SIP)".

[132]
3GPP TS 24.182: "IP Multimedia Subsystem (IMS) Customized Alerting Tones (CAT); Protocol specification".
[133]
IETF RFC 6086: "Session Initiation Protocol (SIP) INFO Method and Package Framework".
[134]
IETF RFC 3023: "XML Media Types".
[135]
IETF RFC 3265: "Session Initiation Protocol (SIP)-Specific Event Notification".
[136]
ITU-T Recommendation Q.931 (05/98): "ISDN user-network interface layer 3 specification for basic call control".
3
Definitions and abbreviations

3.1
Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [6], ITU-T Recommendation E.164 [48] and the following apply:
Carrier textphone mode: a mode for text communication, where continuous signals (i.e. a carrier tone) are present on the connection irrespective of whether text is being exchanged or not.

Carrierless textphone mode: a mode for text communication, where signals are only present on the connection when text is being exchanged. E.g.: Baudot
SS7 signalling function: function in the CS network, which has the capabilities to transport the SS7 MTP-User parts ISUP and BICC+STCmtp
SIP signalling function: function in the IM CN subsystem, which has the capabilities to transport SIP

Incoming or Outgoing: used in the present document to indicate the direction of a call (not signalling information) with respect to a reference point.

Incoming MGCF (I‑MGCF): entity that terminates incoming SIP calls from the IMS side and originates outgoing calls towards the CS side using the BICC or ISUP protocols.

Outgoing Interworking Unit (O‑MGCF): entity that terminates incoming BICC or ISUP calls from the CS side and originates outgoing calls towards the IMS using SIP.

Root Termination: refers to Media Gateway as an entity in itself, rather than a Termination within it. A special TerminationID, "Root" is reserved for this purpose. See ITU-T Recommendation H.248.1. [2]
Signalling Transport Converter (STC): function that converts the services provided by a particular Signalling Transport to the services required by the Generic Signalling Transport Service.

STCmtp: Signalling Transport Converter on MTP. See ITU-T Recommendation Q.2150.1 [29].

BICC+STCmtp: this terminology means that BICC signaling always need to be used on top of STCmtp sublayer.

diverting URI: the URI of the node by which the existing communication was last diverted/forwarded.

diverted-to URI: the URI to which the existing communication is diverted/forwarded to.

3.2
Abbreviations

For the purposes of the present document, the abbreviations as defined in TR 21.905 [6] and the following apply:

3PTY
Three Party

AAL1
ATM Adaptation Layer type 1

ACR
Anonymous Call Rejection

ANM
ANswer Message

APM
Application Transport Message

APRI
Address Presentation Restriction Indicator

ATP
Access Transport Parameter

BC
Bearer Capability

BCI
Backward Call Indicators

BGCF
Breakout Gateway Control Function

BICC
Bearer Independent Call Control
CAT
Customized Alerting Tones
CB
Communication Barring

CCNR
Call Completion on No Reply
CCSS
Call Completion Service Set-up
CDa
Call Deflection Alerting

CDi
Call Deflection Immediate

CDIV
Communication Diversion

CdPN
Called Party Number

CFB
Call Forwarding Busy

CFNR
Call Forwarding No Reply

CGB
Circuit Group Blocking

CgPN
Calling Party Number

CIC
Carrier Identification Code

CMR
Codec Mode Request

CON
Connect

CONF
Conference

COT
Continuity

CPC
Calling Party's Category

CPG
Call ProGress message

CSI
Carrier Selection Information

DSCP
DiffServ Code Point
ECN
Explicit Congestion Notification

ECN-CE
ECN Congestion Experienced
FAC
Facility

FQC
Frame Quality Classification

GN
Generic Number

GRS
Group Reset

GVNS
Global Virtual Network Service

H/W
Hardware

IDR
Identification Request

IEPS
International Emergency Preference Scheme

I-MGCF
Incoming MGCF

IM-MGW
IP Multimedia Media Gateway Function

INF
Information

INR
Information Request

IRS
Identification Response

ITCC
International Telecommunication Charge Card

ITU-T
International Telecommunication Union – Telecommunication Standardization Sector

MCID
Malicious Communication Identification

M3UA
MTP-L3 User Adaptation layer

MLPP
Multi-Level Precedence and Pre-emption

MONA
Media Orientation Negotiation Acceleration

MPC
Media Preconfigured Channel

MRFP
Media Resource Function Processor

MSN
Multiple Subscriber Number

MSU
Message Signalling Unit

MWI
Message Waiting Indication

NOA
Nature Of Address

NPDI
Number Portability Database Dip Indicator

OIP
Originating Identification Presentation

OIR
Originating Identification Restriction

OLI
Originating Line Information

O-MGCF
Outgoing MGCF

PI
Progress Indicator

PIDF
Presence Information Data Format

REV
Reverse Charging

RLC
Release Complete

RSC
Reset Circuit

RTCP
RTP Control Protocol

SAM
Subsequent Address Message

SCTP
Stream Control Transmission Protocol

SGW
Signalling Gateway

SPC
Signalling Preconfigured Channel

ST
Sending Terminated

TCAP
Transaction Capabilities Application Part

TDM
Time Division Multiplex

TIP
Terminating Identification Presentation

TIR
Terminating Identification Restriction

TMR
Transmission Medium Requirement

TMU
Transmission Medium Used

TNL
Transport Network Layer

TNS
Transit Network Selection

TP
Terminal Portability

UA
User Agent

UAC
User Agent Client

UDI
Unrestricted Digital Information

UDI-TA
Unrestricted Digital Information with Tones/Announcemnets

URI
Uniform Resource Identifier

USI
User Service Information

UUS
User-to-User Signalling

XML
eXtensible Markup Language

4
General

4.1
General interworking overview

The IM CN subsystem shall interwork with BICC and ISUP based legacy CS networks, e.g. PSTN, ISDN, CS PLMNs, in order to provide the ability to support basic voice calls (see TS 22.228 [11]), between a UE located in the IM CN subsystem and user equipment located in a CS network.

For the ability to support the delivery of basic voice calls between the IM CN subsystem and CS networks, basic protocol interworking between SIP (as specified in TS 24.229 [9]) and BICC or ISUP (as specified in ITU-T Recommendations Q.1902.1-6 [30] and ITU-T Recommendations Q761 to Q764 [4] respectively) has to occur at a control plane level, in order that call setup, call maintenance and call release procedures can be supported. The MGCF shall provide this protocol mapping functionality within the IM CN subsystem.

User plane interworking between the IM CN subsystem and CS network bearers (e.g. 64k TDM, ATM/AAL2 circuit or IP bearer) are supported by the functions within the IM-MGW. The IM-MGW resides in the IM CN subsystem and shall provide the bearer channel interconnection. The MGCF shall provide the call control to bearer setup association.

The IM CN subsystem shall interwork, at the control and user plane, with BICC and ISUP based legacy CS networks. The support of supplementary services shall be as defined in TS 22.228 [11]. The MGCF and IM-MGW shall support the interworking of the IM CN subsystem to an external ISUP based CS network. They may also support interworking to a BICC based CS network where no 3GPP specific extension is applied. The MGCF and the IM-MGW may also support interworking to a BICC based CS network where 3GPP specific extensions in accordance with TS 29.205 [14] are applied.

5
Network characteristics

5.1
Key characteristics of ISUP/BICC based CS networks

This signalling interface to a PSTN is either based on BICC Capability Set 2 as specified in ITU-T Recommendations Q.1902.1 to Q.1902.6 [30], or on ISUP (see ITU-T Recommendations Q.761 to Q.764 [4]).

The interface towards a CS-PLMN may either be one of the interfaces mentioned in the paragraph above or a signalling interface based on BICC with 3GPP specific extensions, as specified for the 3GPP Nc interface in TS 29.205 [14], and the IM-MGW may support the 3GPP Nb interface, as specified in TS 29.414 [25] and TS 29.415 [26]. If the 3GPP Nc interface is applied as signalling interface, the 3GPP Nb interface is used as user plane interface and the Nb UP Framing protocol is applied.

5.2
Key characteristics of IM CN subsystem

The IM CN subsystem uses SIP to manage IP multimedia sessions in a 3GPP environment, it also uses IPv6, as defined in RFC 2460 [39], as the transport mechanism for both SIP session signalling and media transport. The 3GPP profile of SIP defining the usage of SIP within the IM CN subsystem is specified in TS 24.229 [9]. Example callflows are provided in TS 24.228 [8].

6
Interworking with CS networks

6.1
Interworking reference model

Figure 1 details the reference model required to support interworking between the 3GPP IM CN subsystem and CS networks for IM basic voice calls.


[image: image3.wmf] 

(Note 3)

 

CSCF

 

CS network

 

IM

-

 

MGW

 

MGCF

 

SGW

 

Mb

 

CS channels

 

e.g. PCM

 

BICC/ISUP

 

over MTP

 

Mn

 

User Plane

 

Control Plane

 

BICC/ISUP over 

SCTP/IP

 

Mg

 

BGCF

 

Mj

 

BICC over SCTP/IP

 


NOTE 1:
The logical split of the signalling and bearer path between the CS network and the IM CN subsystem is as shown, however the signalling and bearer may be logically directly connected to the IM-MGW.

NOTE 2:
The SGW may be implemented as a stand-alone entity or it may be located in another entity either in the CS network or the IM-MGW. The implementation options are not discussed in the present document.

NOTE 3:
The IM-MGW may be connected via the Mb to various network entities, such as a UE (via a GTP Tunnel to a GGSN), an MRFP, or an application server.

NOTE 4:
A SGW function is not required for certain signalling transports, where M3UA+SCTP+IP is used in CS network and IM-MGCF.

Figure 1: IM CN subsystem to CS network logical interworking reference model

6.1.1
Interworking reference points and interfaces

The reference points and network interfaces shown in figure 1 are as described:

Protocol for Mg reference point: The single call control protocol applied across the Mg reference point (i.e. between CSCF and MGCF) will be based on the 3GPP profile of SIP as defined in accordance with TS 24.229 [9].

Protocol for Mn reference point: The Mn reference point describes the interfaces between the MGCF and IM-MGW, and has the properties as detailed in TS 29.332 [15].

Protocol for Mj reference point: The single call control protocol applied across the Mj reference point (i.e. between BGCF and MGCF) will be based on the 3GPP profile of SIP as defined in accordance with TS 24.229 [9].

Protocol for Mb reference point: The Mb reference point is defined in accordance with TS 23.002 [10] and is IPv6 based.

6.1.2
Interworking functional entities

6.1.2.1
Signalling Gateway Function (SGW)

This component performs the call related signalling conversion to or from BICC/ISUP based MTP transport networks to BICC/ISUP based SCTP/IP transport networks, and forwards the converted signalling to or from the MGCF. The functionality within SGW shall be in accordance with TS 23.002 [10].

6.1.2.2
Media Gateway Control Function (MGCF)

This is the component within the IM CN subsystem, which controls the IM-MGW, and also performs SIP to BICC or SIP to ISUP call related signalling interworking.

The functionality defined within MGCF shall be defined in accordance with TS 23.002 [10].

6.1.2.3
IP Multimedia - Media Gateway Function (IM-MGW)

This is the component within the IM CN subsystem, which provides the interface between the PS domain and the CS domain, and it shall support the functions as defined in accordance with TS 23.002 [10].

6.2
Control plane interworking model

Within the IM CN subsystem, the 3GPP profile of SIP is used to originate and terminate IM sessions to and from the UE.

External CS networks use BICC or ISUP to originate and terminate voice calls to and from the IM CN subsystem.

Therefore, in order to provide the required interworking to enable inter network session control, the control plane protocols shall be interworked within the IM CN subsystem. This function is performed within the MGCF (see clause 6.1.2).

6.3
User plane interworking model

Within the IM CN subsystem, IPv6, and framing protocols such as RTP, are used to transport media packets to and from the IM CN subsystem entity like UE or MRFP.

External legacy CS networks use circuit switched bearer channels like TDM circuits (e.g. 64 kbits PCM), ATM/AAL2 circuit or IP bearers to carry encoded voice frames, to and from the IM CN subsystem.

Other CN networks use ATM/AAL 1 or AAL 2 or IP as a backbone, with different framing protocols.

Therefore, in order to provide the required interworking to enable media data exchange, the user plane protocols shall be translated within the IM CN subsystem. This function is performed within the IM-MGW (see clause 6.1.2).

7
Control plane interworking

Signalling from CS networks to or from IM CN subsystem, where the associated supported signalling protocols are SS7/M3UA+ SCTP+IP and M3UA+SCTP+IP respectively, requires a level of interworking between the nodes across the Control Plane, i.e. the SS7 signalling function, SGW (if applicable), MGCF and SIP signalling function. This interworking is required in order to provide a seamless support of a user part, i.e. SIP and BICC+STCmtp or SIP and ISUP.

The transport of SS7 signalling protocol messages of any protocol layer that is identified by MTP level 3, in SS7 terms, as a user part (MTP3-user) shall be accomplished in accordance with the protocol architecture defined in the following clauses. For the present document these protocol layers include, but are not limited to, Bearer Independent Call Control (BICC)+STCmtp and ISDN User Part (ISUP).

7.1
General

The following clauses define the signalling interworking between the Bearer Independent Call Control (BICC) or ISDN User Part (ISUP) protocols and Session Initiation Protocol (SIP) with its associated Session Description Protocol (SDP) at a MGCF. The MGCF shall act as a Type A exchange (ITU-T Recommendation Q.764 [4]) for the purposes of ISUP and BICC Compatibility procedures. The services that can be supported through the use of the signalling interworking are limited to the services that are supported by BICC or ISUP and SIP based network domains.

BICC is the call control protocol used between Nodes in a network that incorporates separate call and bearer control. The BICC/ISUP capabilities or signalling information defined for national use is outside the scope of the present document. It does not imply interworking for national-specific capabilities is not feasible.

The capabilities of SIP and SDP that are interworked with BICC or ISUP are defined in TS 24.229 [9].
Services that are common in SIP and BICC or ISUP network domains will seamlessly interwork by using the function of the MGCF. The MGCF will originate and/or terminate services or capabilities that do not interwork seamlessly across domains according to the relevant protocol recommendation or specification.

Table 1 lists the services seamlessly interworked and therefore within the scope of the present document.

Table 1: Interworking Capabilities between BICC/ISUP and SIP profile for 3GPP

	Service

	Speech/3.1 kHz audio

	CS data Calls (optional)

	En bloc address signalling

	Overlap address signalling from the CS side towards the IMS

	Out of band transport of DTMF tones and information. (BICC only)

	Inband transport of DTMF tones and information. (BICC and ISUP)

	Direct-Dialling-In (DDI)

	Multiple Subscriber Number (MSN)

	Calling Line Identification Presentation (CLIP)

	Calling Line Identification Restriction (CLIR)

	Connected line presentation (COLP)

	Connected line restriction (COLR)

	Carrier routeing


7.2
Interworking between CS networks supporting ISUP and the IM CN subsystem

The control plane between CS networks supporting ISUP and the IM CN subsystem supporting SIP, where the underlying network is SS7 and IP respectively is as shown in figure 2.


[image: image4.wmf] 

 

SS7 signalling

 

function

 

Signalling g

ateway

 

function

 

Media gateway

 

control function

 

SS7

 

SIP signalling

 

function

 

SIP

 

IP

 

IP

 

SCTP

 

M3UA

 

ISUP

 

TCP / 

 

UDP /

 

SCTP

 

IP

 

L1

 

SCTP

 

M3UA

 

MTP2

 

MTP3

 

IP

 

TCP /

 

UDP /

 

SCTP

 

SIP

 

L1

 

MTP2

 

MTP3

 

ISUP

 

IP

 

IP

 

ISUP

 

SIP

 


Figure 2: Control plane interworking between CS networks supporting ISUP
and the IM CN subsystem

7.2.1
Services performed by network entities in the control plane

7.2.1.1
Services performed by the SS7 signalling function

The SS7 signalling function provides the capabilities to deliver or receive SS7 MTP3-User information (e.g. ISUP or BICC+STCmtp) across the SS7 signalling network. The functional interface of the MTP, the MTP User parts and the signalling network are as detailed in ITU-T Recommendations Q.701 to Q.709 [3].

7.2.1.2
Services of the SGW

The SGW shall perform the functions as described in TS 23.002 [10].

In order to support the seamless operation of the MTP3-User part information between networks incorporating SS7 and IP (either IPv4, see RFC 791 [16], or IPv6, see RFC 2460 [39]), the SGW shall support the services of MTP as well as the services of the M3UA (see TS 29.202 [20]) and SCTP (see RFC 2960 [18]).

7.2.1.3
Services of the MGCF

The session handling and session control of the MGCF shall be as detailed in TS 24.229 [9].

The MGCF shall provide the interaction, through the use of its interworking function, between the SS7 MTP3-User part information, e.g. ISUP, and SIP. The MGCF interworking function shall also provide the translation between the SS7 MTP3-User part information and SIP, where the interworking of SIP to ISUP and BICC+STCmtp are detailed below.

7.2.1.4
Services of the SIP signalling function

The SIP signalling function is a logical entity that provides the capabilities to deliver or receive multimedia session information across the IM CN subsystem signalling system.

7.2.2
Signalling interactions between network entities in the control plane

7.2.2.1
Signalling between the SS7 signalling function and MGCF

The SGW shall enable the signalling interaction between the SS7 signalling function and the MGCF.

7.2.2.1.1
Signalling from MGCF to SS7 signalling function

For signalling from the MGCF to the SS7 signalling function, the SGW shall terminate the SCTP and M3UA protocol layers and deliver the MTP3-User protocol messages, e.g. ISUP messages, towards the SS7 signalling function. The SGW transmits and receives SS7 Message Signalling Units (MSUs) to and from the SS7 signalling function over standard SS7 network interfaces, using MTP to provide reliable transport of the messages.

7.2.2.1.2
Signalling from SS7 signalling function to MGCF

For signalling from the SS7 signalling function to the MGCF, the SGW shall terminate SS7 MTP2 and MTP3 protocol layers and deliver MTP3-User part information messages, e.g. ISUP, towards the MGCF. In order to direct messages received from the SS7 MTP3 network to the appropriate IP destination, e.g. MGCF, the SGW shall perform a message distribution function using the information received from the MTP3-User message. Message distribution at the SGW shall be performed in accordance with TS 29.202 [20].

7.2.2.1.3
Services offered by SCTP and M3UA

The SGW internal protocol mapping and transportation between BICC or ISUP messages and IP encapsulated BICC or ISUP messages respectively is supported by the services of the M3UA adaptation layer and the underlying SCTP layer. The SGW shall allow for the transfer of MTP3-User signalling messages, e.g. BICC or ISUP, to and from an MGCF, where the peer MTP3-User protocol exists.

7.2.2.1.3.1
Services offered by SCTP

SCTP offers the ability to reliably transfer the SCTP User applications, e.g. M3UA, between the SCTP User application peers. The initialization procedure used for an association between two SCTP end-to-end peers, and the initialization to the SCTP User applications shall be performed as detailed in RCF 2960 [18].

7.2.2.1.3.2
Services offered by M3UA

When an association between two SCTP peers has been established, the use of M3UA shall provide the transport service in accordance with MTP (see ITU-T Recommendations Q.701 to Q.709 [3]) to the MTP3-User, e.g. ISUP.

7.2.2.2
Signalling between the MGCF and SIP signalling function

Signalling between the SIP signalling function and the MGCF uses the services of IP (RFC 2460 [39]), and transport protocol such as TCP (RFC 793 [24]) or UDP (RFC 768 [17]) or SCTP (RFC 2960 [18]) (see TS 24.229 [9]), and SIP.

The naming and addressing concepts between the MGCF and SIP signalling function shall be detailed in accordance with TS 23.228 [12]. The issues of general IP address management are discussed in TS 23.221 [47].

7.2.3
SIP-ISUP protocol interworking

When a coding of a parameter value is omitted it implies that it is not affected by the interworking and the values are assigned by normal protocol procedures.

7.2.3.1
Incoming call interworking from SIP to ISUP at I-MGCF

7.2.3.1.1
Sending of IAM

On reception of a SIP INVITE requesting a session, the I‑MGCF shall send an IAM message. The allowed sessions are given in clause 7.2.3.1.2.5.
An I-MGCF shall support both incoming INVITE requests containing SIP preconditions and 100rel extensions in the SIP Supported or Require header fields, and INVITE requests not containing these extensions, unless the Note below applies.

NOTE:
If the I-MGCF is deployed in an IMS network that by local configuration serves no user requiring preconditions, the MGCF may not support incoming requests requiring preconditions.

The I-MGCF shall interwork forked INVITE requests with different request URIs.

If the SIP precondition extension is not included in the Supported or Require header field, the I-MGCF shall send an IAM immediately after the reception of the INVITE, as shown in figure 3. The I-MGCF shall set the continuity indicators to "Continuity check not required".

If a Continuity Check procedure is supported in the ISUP network and SIP precondition extension are included in the SIP Supported or Require header field, the I-MGCF shall send the IAM immediately after the reception of the INVITE, as shown in figure 3. If the received SDP indicates that precondition is fulfilled the I-MGCF shall set the continuity indicators to "continuity check is not required". If the received SDP indicates that precondition is not fulfilled the I-MGCF shall set the continuity indicators to "continuity check performed on a previous circuit". The procedure in figure 3 applies when the value of the continuity indicator is either set to "continuity check required", "continuity check performed on a previous circuit" or "continuity check not required". If the continuity indicator is set to "continuity check required" the corresponding procedures at the Mn interface described in clause 9.2.2.3 also apply.


[image: image5.wmf] 

INVITE

 

IAM

 

I

-

MGCF

 


Figure 3: Receipt of an INVITE request (continuity procedure supported in the ISUP network)

If Continuity Check procedure is not supported in the ISUP network, and the SDP in the received INVITE request contains preconditions not met, the I-MGCF shall delay sending the IAM until the SIP preconditions are met and set the continuity indicators in the resulting IAM to "Continuity check not required".

[image: image6.wmf] 

I

-

MGCF

 

INVITE

 

SDP indicating 

pre

-

conditions 

met

 

IAM

 


Figure 4: Receipt of an INVITE request (continuity procedure not supported in the ISUP network)

The I-MGCF shall reject an INVITE request for a session only containing unsupported media types by sending a status code 488 "Not Acceptable Here". If several media streams are contained in a single INVITE request, and if the I-MGCF does not support multimedia interworking according to Annex E, then the I-MGCF shall select one of the supported media streams, reserve the codec(s) for that media stream, and reject the other media streams and unselected codecs in the SDP answer, as detailed in IETF RFC 3264 [36]. If supported audio media stream(s) and supported non-audio media stream(s) are contained in a single INVITE request, an audio stream should be selected.

The I-MGCF shall include a To tag in the first backward non-100 provisional response, in order to establish an early dialog as described in IETF RFC 3261 [19].

If an MGCF discovers an emergency call it shall, depending on national requirements, map that to appropriate indication in ISUP.
According to IETF RFC 3261 [19] and IETF RFC 3264 [36], if an INVITE message is received without an SDP offer, then the I-MGCF sends an SDP offer in the first reliable non-failure message.
7.2.3.1.2
Coding of the IAM

7.2.3.1.2.0
General

The following ISDN user part parameters description can be found in ITU-T Recommendation Q.763 [4].

7.2.3.1.2.1
Called party number

The E.164 address encoded in the Request-URI shall be mapped to the called party number parameter of the IAM message.

Table 2: Coding of the called party number
	INVITE(
	IAM(

	Request-URI
	Called Party Number

	E.164 address

(format +CC NDC SN)

(e.g. as User info portion of a SIP URI with user=phone, or as tel URI)
	Address Signal:

Analyse the information contained in received E.164 address.

If CC is country code of the network in which the next hop terminates, then remove "+CC" and use the remaining digits to fill the Address signals.

If CC is not the country code of the network in which the next hop terminates, then remove "+" and use the remaining digits to fill the Address signals.
(NOTE 2)

	
	Odd/even indicator: set as required

	
	Nature of address indicator:

Analyse the information contained in received E.164 address.

If CC is country code of the network in which the next hop terminates, then set Nature of Address indicator to "National (significant) number.

If CC is not the country code of the network in which the next hop terminates, then set Nature of Address indicator to "International number".

(NOTE 1)

	
	Internal Network Number Indicator:

1
routing to internal network number not allowed

	
	Numbering plan Indicator:

001 ISDN (Telephony) numbering plan (Rec. E.164)

	national operator option for service numbers:

Non E.164 numbers

(as a local-number with a phone-context in the User Info portion in a SIP URI with user=phone, or as a local number with a phone-context in a tel URI)
	(NOTE 3)
	Address Signal:

use received non E.164 number to fill the Address signals with national significant number.

	
	
	Odd/even indicator: set as required

	
	
	Nature of address indicator:

"National (significant) number". (NOTE 1)

	
	
	Internal Network Number Indicator:

1
routing to internal network number not allowed

	
	
	Numbering plan Indicator:

001 ISDN (Telephony) numbering plan (Rec. E.164)

	
	(NOTE 3)
	Address Signal:

Use received non E.164 number to fill the Address signals.

	
	
	Odd/even indicator: set as required

	
	
	Nature of address indicator:

set Nature of Address indicator to "network-specific number" or to "reserved for national use".

	
	
	Internal Network Number Indicator:

1
routing to internal network number not allowed

	
	
	Numbering plan Indicator:

Based on operator policy other numbering plan indicators than "001 ISDN (Telephony) numbering plan (Rec. E.164)" can be used e.g. depending on phone context value.

	NOTE 1:
The usage of "nature of address indicator" value "unknown" is allowed but the mapping is not specified in the present specification.

NOTE 2:
If PSTN XML and ISUP Sending Terminated (ST) signal are supported as a network option, then the PSTN XML sendingCompleteIndication, if present, is mapped to the sending terminated digit (hexadecimal digit F) in the address signals field of the Called Party Number parameter.
NOTE 3:
Depending on configuration, network-specific numbers (identified by phone context according to operator policy) should be translated into "ISDN (Telephony) numbering plan numbers (Rec. E.164)" unless such a mapping is not possible and local operator's policy requires keeping them in local format.


7.2.3.1.2.2
Nature of connection indicators

bits
BA

Satellite indicator

0 0


no satellite circuit in the connection
bits
DC

Continuity check indicator

0 0


continuity check not required, if the continuity check procedure is not supported in the succeeding

network (figure 4).

0 1
continuity check required, if a continuity check shall be carried out on the succeeding circuit.


(figure 3)

1 0
continuity check performed on a previous circuit otherwise, if the continuity check procedure is


supported in the succeeding network, but shall not be carried out on the succeeding circuit otherwise.

(figure 3)

bit

E

Echo control device indicator

1


outgoing echo control device included, for speech calls, e.g., TMR is "3.1KHz audio".
0


outgoing echo control device not included, for known data calls, e.g., TMR "64 kBit/s unrestricted"

or HLC "Facsimile Group 2/3".

7.2.3.1.2.3
Forward call indicators

bits
CB

End-to-end method indicator
0 0


no end-to-end method available (only link-by-link method available)
bit

D

Interworking indicator

1


interworking encountered
As a network operator option, the value D = 0 "No interworking encountered" is used if the TMR = 64 kBit/s unrestricted is used.

NOTE:
This avoids sending of a progress indicator with progress information 0 0 0 0 0 0 1 "Call is not end-to-end ISDN; further call progress information may be available in‑band ", so the call will not be released for that reason by an ISDN terminal.

bit

E

End-to-end information indicator (national use)

0


no end-to-end information available
bit

F

ISDN user part/BICC indicator

0


ISDN user part/BICC not used all the way
As a network operator option, the value F = 1 "ISDN user part/BICC used all the way" is used if the TMR = 64 kBit/s unrestricted is used.

NOTE:
This avoids sending of a progress indicator with progress information 0 0 0 0 0 0 1 "Call is not end-to-end ISDN; further call progress information may be available in‑band ", so the call will not be released for that reason by an ISDN terminal.

bits
HG
ISDN user part/BICC preference indicator


if any used supplementary service requires ISUP or BICC all the way, depending on operator policy:




0 0

ISDN user part/BICC preferred all the way, or




1 0

ISDN user part/BICC required all the way;



Otherwise:
   0 1   ISDN user part/BICC not required all the way
bit

I

ISDN access indicator

0


originating access non-ISDN
As a network operator option, the value I = 1 "originating access ISDN" is used if the TMR = 64 kBit/s unrestricted is used.

NOTE:
This avoids sending of a progress indicator with progress information 0 0 0 0 1 1 "Originating access is non-ISDN", so the call will not be released for that reason by an ISDN terminal.
bits
KJ
SCCP method indicator

0 0
no indication
If the PSTN XML is supported as a network option, the Forward Call indicators derived as shown in table 02a shall take precedence.

Table 02a: Mapping of PSTN XML elements to Forward call indicators parameter

	INVITE (
	IAM(

	PSTN XML
	Forward call indicators parameter 

	PSTN XML with Progress indicator with Progress Description value 6

(Meaning: originating access ISDN)
	bit
D
Interworking Indicator



0
"no interworking encountered (No. 7 signalling all the way)"
bit
F
ISDN User Part indicator



1
"ISDN User Part used all the way"
bit
I
ISDN access indicator



1
"originating access ISDN"

	NOTE:
Progress Indicator with Progress Description value "6" shall not be included in an ATP within the IAM.


7.2.3.1.2.4
Calling party's category

See ANNEX C for the normative interworking of the CPC parameter.
7.2.3.1.2.4A
Originating Line Information

The ISUP Originating Line Information parameter is defined by ANSI Standard ATIS-1000113 [117], Chapter 3.

See Annex H for the normative interworking of the OLI parameter as a network option.
7.2.3.1.2.5
Transmission medium requirement

The I-MGCF may either transcode the selected codec(s) to the codec on the PSTN side or it may attempt to interwork the media without transcoding. If the I-MGCF transcodes, it shall select the TMR parameter to "3.1 kHz audio". If the I-MGCF does not transcode, it should map the TMR, USI and Access Transport parameters from the selected codec according to table 2a. However, if the I-MGCF supports the PSTN XML body as a network option, and if a PSTN XML body is received in the INVITE request and the I-MGCF selects media encoded in any of the formats in table 2a (G.711, Clearmode or t38) among the offered media, the I-MGCF shall derive these parameters from the XML body instead, as detailed in table 2b. The I-MGCF should only apply the mapping in table 2b if the TMR and USI values derived from the selected codec according to table 2a are equivalent with the values within the first Bearer Capability element in the PSTN XML, and otherwise the I-MGCF should apply the mapping according to table 2a.
The support of any of the media listed in table 2a is optional.
If no SDP is received from the remote peer (as described in clause 7.2.3.1.1), then the TMR parameter should be set to "3.1 kHz audio". Transcoding shall be applied as required.
Table 2a: Coding of TMR/USI/HLC from SDP: SIP to ISUP

	
	m= line
	
	b= line (NOTE 4)
	a= line
	TMR parameter
	USI parameter (optional) (NOTE 1)
	HLC IE in the ATP parameter (optional)

	<media>
	<transport>
	<fmt-list>
	<modifier>:<bandwidth-value>


(NOTE 5)
	rtpmap:<dynamic-PT> <encoding name> <clock rate>[<encoding parameters>]
	TMR codes
	Information Transfer Capability
	User Information Layer 1 Protocol Indicator
	High Layer Characteristics Identification

	audio
	RTP/AVP
	0
	N/A or AS: up to (64 kbit/s + RTP/UDP/IP overhead)
	N/A
	"3.1KHz audio"
	
	
	(NOTE 3)

	audio
	RTP/AVP
	Dynamic PT
	N/A or AS: up to (64 kbit/s + RTP/UDP/IP overhead)
	rtpmap:<dynamic-PT> PCMU/8000
	"3.1KHz audio"
	
	
	(NOTE 3)

	audio
	RTP/AVP
	8
	N/A or AS: up to (64 kbit/s+ RTP/UDP/IP overhead)
	N/A
	"3.1KHz audio"
	
	
	(NOTE 3)

	audio
	RTP/AVP
	Dynamic PT
	N/A or AS: up to (64 kbit/s + RTP/UDP/IP overhead)
	rtpmap:<dynamic-PT> PCMA/8000
	"3.1KHz audio"
	
	
	(NOTE 3)

	audio
	RTP/AVP
	Dynamic PT
	AS: (64 kbit/s+ RTP/UDP/IP overhead)


	rtpmap:<dynamic-PT> CLEARMODE/8000

(NOTE 2)
	"64 kbit/s unrestricted"
 or

"64 kbit/s preferred"
(NOTE 7)
	"Unrestricted digital information"
(NOTE 6)
	
	

	image
	Udptl [73]
	t38 [73]
	N/A or AS: up to (64 kbit/s + UDP/IP overhead)
	Based on ITU-T T.38 [72]
	"3.1 KHz audio"
	"3.1 KHz audio"
	
	"Facsímile
Group 2/3"

	image
	tcp
	t38 [73]
	N/A or AS: up to (64 kbit/s + TCP/IP overhead)
	Based on ITU-T T.38 [72]
	"3.1 KHz audio"
	"3.1 KHz audio"
	
	"Facsímile
Group 2/3"

	NOTE 1:
In this table the codec G.711 is used only as an example. Other codecs are possible.

NOTE 2:
CLEARMODE is specified in RFC4040 [69].

NOTE 3:
HLC is normally absent in this case. It is possible for HLC to be present with the value "Telephony", although 6.3.1/Q.939 indicates that this would normally be accompanied by a value of "Speech" for the Information Transfer Capability element.

NOTE 4:
The MGCF should return an b:AS bandwidth modifier with a bandwidth of 64kbit/s + RTP/UDP/IP overhead in the SDP answer to request that the peer does not send with a higher bandwidth. If the received b=line indicates a bandwidth greater than 64kbit/s + RTP/UDP/IP overhead, the MGCF should also accept the incoming call.
NOTE 5:
<bandwidth value> for <modifier> of AS is in units of kbit/s.

NOTE 6:
In the case where the Clearmode codec appears together with speech codecs in the same m-line, the value "Unrestricted digital inf. w/tones/ann" is applicable but is mapped into the USI prime parameter (see clause 7.2.3.1.2.5a).

NOTE 7:
The value "64 k/bits preferred" should only be used if the Clearmode codec appears together with speech codecs in the same m-line and two PSTN XML Bearer Capability elements appear in the initial INVITE request as described in the clause 7.2.3.1.2.5a.


Table 2b: Mapping of PSTN XML elements into ISUP Parameters

	INVITE ( 
	IAM (

	PSTN XML 
	Value
	ISUP Parameter
	Content

	HighLayerCompatibility
	
	Access Transport Parameter
	High layer compatibility (NOTE 1)

	LowLayerCompatibility
	
	
	Low layer compatibility

	BearerCapability (NOTE 2)
	
	User Service Information
	

	HighLayerCompatibility
	
	User Tele Service
	High layer compatibility

	BearerCapability (InformationTransferCapability) (NOTE 2)
	Speech
	TMR
	Speech

	
	3.1 kHz audio
	
	3.1 kHz audio

	
	Unrestricted digital information
	
	64 kbit/s unrestricted

	
	Unrestricted digital information with tones/announcements
	
	64 kbit/s unrestricted

	NOTE 1:
If two high layer compatibility information elements are received, they shall be transferred in the same order as received in the PSTN XML body in the INVITE message.
NOTE 2:
If there are two BCs present, see clause 7.2.3.1.2.5a.

NOTE 3:
The above mapping assumes that there is only a single BearerCapability present.


7.2.3.1.2.5a
Transmission medium requirement prime and USI prime (optional)

The procedures to support UDI-TA Fallback mechanism described in the present clause shall only apply if two PSTN XML Bearer Capability elements appear within the INVITE Request and the MGCF supports the PSTN XML body as a network option.

When all the following conditions apply:

-
The INVITE request includes SDP with one m-line with at least two formats, and with the coding of the first two formats appearing in table 2a;

-
the TMR and USI prime values derived from the first format in the m-line according to table 2a are equivalent with the values within the second Bearer Capability element in the PSTN XML;
-
the TMR prime and USI values derived from the second format in the m-line according to table 2a are equivalent with the values within the first Bearer Capability element in the PSTN XML; and.

-
the I-MGCF supports forwarding fallback signalling.

then the I-MGCF shall


-
if TMR "64 kBit/s preferred" is supported at the succeeding trunk:

-
map the first XML Bearer Capability element into the "USI" within the IAM;

-
map the the first PSTN XML BearerCapability (InformationTransferCapability) into the "TMR prime" within the IAM, applying the same mapping rules as specified for the mapping into the "TMR" in table 2b;

-
map the second XML Bearer Capability element (InformationTransferCapability) into the USI prime within the IAM;

-
set the TMR within the IAM to "64 kBit/s preferred";

-
configure the IM-MGW; and

-
store those values;


-
if TMR "64 kBit/s preferred" is not supported at the succeeding trunk:

-
apply the procedures as described within clause 7.2.3.1.2.5, using the first Bearer Capability element in the PSTN XMLand the second format in the m-line;


discard the second Bearer Capability element in the PSTN XML;

-
select the second format in the m-line within the SDP answer; and

-
configure the IM-MGW.
Otherwise (i.e. if some Bearer Capability element in the PSTN XML did not match the SDP), the I-MGCF shall:

-
discard the XML Bearer Capability elements;

-
if the I-MGCF received at least two formats within the m-line, select one of those formats, exept for the CLEARMODE codec, within the SDP answer;

-
apply the mapping for the selected format according to table 2a; and

-
configure the IM-MGW accordingly

7.2.3.1.2.6
Calling party number

The SIP "Privacy" header is defined within IETF RFC 3323 [40]. The SIP "P-Asserted-Identity" header is defined in IETF RFC 3325 [41].

Table 3: Mapping of SIP From/P-Asserted-Identity/Privacy headers to CLI parameters

	Has a "P-Asserted-Identity" header field (NOTE 2, NOTE 5, NOTE 6) been received?
	Has a "From" header field (NOTE 3) containing a URI that encodes an E.164 address been received (NOTE 6)?
	Calling Party Number parameter

Address signals
	Calling Party Number parameter

 APRI
	Generic Number (additional calling party number) address signals
	Generic Number parameter APRI

	No
	No
	Network option to either include a network provided E.164 number (See table 4) or omit the Address signals.
(NOTE 4)
	Network option to set APRI to "presentation restricted" or "presentation allowed"
(NOTE 4)
(See table 5)

As a network option the APRI "presentation restricted by network" (NOTE 7) can be used instead of the APRI "presentation restricted"
	Parameter not included


	Not applicable



	No
	Yes
	Network option to either include a network provided E.164 number (See table 4) or omit the Address signals.
(NOTE 4)
	Network option to set APRI to "presentation restricted" or "presentation allowed"
(NOTE 4)

(See table 5)

As a network option the APRI "presentation restricted by network" (NOTE 7) can be used instead of the APRI "presentation restricted""
	Network option to either omit the parameter (if CgPN has been omitted) or derive from the "From" header
(NOTE 1)

(See table 6)
	APRI = "presentation restricted" or "presentation allowed" depending on SIP Privacy header.

(See table 6)



	Yes
	No
	Derived from

P-Asserted-Identity

(See table 5)


	APRI = "presentation restricted" or "presentation allowed" depending on SIP Privacy header.

(See table 5)
	Not included
	Not applicable

	Yes
	Yes
	Derived from

P-Asserted-Identity

(See table 5)
	APRI = "presentation restricted" or "presentation allowed" depending on SIP Privacy header.

(See table 5)
	Network option to either omit the parameter or derive from the "From" header
(NOTE 1)

(See table 6)
	APRI = "presentation restricted" or "presentation allowed" depending on SIP Privacy header.

(see table 6)

	NOTE 1:
This mapping effectively gives the equivalent of Special Arrangement to all SIP UAC with access to the I‑MGCF.

NOTE 2:
It is possible that the P-Asserted-Identity header field includes both a tel URI and a SIP URI. In this case, either the tel URI or the SIP URI with user="phone" and a specific host portion, as selected by operator policy, may be used.
NOTE 3:
The "From" header may contain an "Anonymous User Identity". An "Anonymous User Identity" includes information that does not point to the calling party. IETF RFC 3261 recommends that the display-name component contain "Anonymous". That the Anonymous User Identity will take the form defined in TS 23.003 [74]. The Anonymous User Identity indicates that the calling party desired anonymity. The From header may also contain an Unavailable User Identity as defined in TS 23.003 [74], that indicates that the calling party is unknown.
NOTE 4:
A national option exists to set the APRI to "Address not available".
NOTE 5:
TS 24.229 [9] guarantees that the received number is an E.164 number formatted as an international number, with a "+" sign as prefix.

NOTE 6:
The E.164 numbers considered within the present document are composed by a Country Code (CC), followed by a National Destination Code (NDC), followed by a Subscriber Number (SN). On the IMS side, the numbers are international public telecommunication numbers ("CC"+"NDC"+"SN") and are prefixed by a "+" sign. On the CS side, it is a network option to omit the CC.

NOTE 7:
This is an ETSI specific value described within ETSI EN 300 356-1 [70].


Table 4: Setting of the network-provided BICC/ISUP calling party number parameter with a CLI (network option)
	BICC/ISUP CgPN Parameter field
	Value

	Screening Indicator
	"network provided"

	Number Incomplete Indicator
	"complete"

	Number Plan Indicator
	ISDN/Telephony (E.164)

	Address Presentation Restricted Indicator
	Presentation allowed/restricted

As a network option the APRI value "presentation restricted by network" (NOTE) can be used instead of the APRI value "presentation restricted"

	Nature of Address Indicator
	If next BICC/ISUP node is located in the same country set to "National (Significant) number" else set to "International number"

	Address signals
	If NOA is "national (significant) number" no country code should be included. If NOA is "international number", then the country code of the network-provided number should be included. 

	NOTE :
This is an ETSI specific value described within ETSI EN 300 356-1 [70]


Table 5: Mapping of P-Asserted-Identity and privacy headers to the ISUP/BICC calling party number parameter

	SIP Component
	Value
	BICC/ISUP Parameter / field
	Value

	P-Asserted-Identity header field (NOTE)
	 E.164 number
	Calling Party Number
	

	
	Number incomplete indicator
	"Complete"

	
	Numbering Plan Indicator
	"ISDN/Telephony (E.164)"

	
	Nature of Address Indicator
	If CC encoded in the URI is equal to the CC of the country where MGCF is located AND the next BICC/ISUP node is located in the same country then

set to "national (significant) number" 

else set to "international number"

	
	Screening indicator 
	Network Provided

	Addr-spec
	"CC" "NDC" "SN" from the URI


	Address signal
	If NOA is "national (significant) number" then set to

"NDC" + "SN" 

If NOA is "international number" then set to "CC"+"NDC"+"SN"

	Privacy header field is not present
	
	APRI
	"presentation allowed"

	Privacy header field (IETF RFC 3323 [40])
	priv-value
	APRI
	If the Privacy header field contains the values "id" (see IETF RFC 3325 [41]) and/or "header", the APRI shall be set to "presentation restricted". Otherwise, the APRI shall be set to "presentation allowed".

	NOTE:
It is possible that a P-Asserted-Identity header field includes both a tel URI and a SIP URI. In this case, either the tel URI or the SIP URI with user="phone" and a specific host portion, as selected by operator policy, may be used.


7.2.3.1.2.7
Generic number

Table 6: Mapping of SIP From header field to BICC/ISUP generic number (additional calling party number) parameter (network option)

	SIP component
	Value
	BICC/ISUP parameter / field
	Value

	From header field


	name-addr or addr-spec
	Generic number

Number Qualifier Indicator
	"additional calling party number"

	
	Nature of Address Indicator
	If CC encoded in the URI is equal to the CC of the country where MGCF is located AND the next BICC/ISUP node is located in the same country then

set to "national (significant) number"
else set to "international number.

	
	Number incomplete indicator
	"Complete"

	
	Numbering Plan Indicator
	"ISDN/Telephony (E.164)"

	
	Screening indicator
	"user provided not verified"

	Addr-spec
	"CC" "NDC" + "SN" from the URI
	Address signal
	If NOA is "national (significant) number" then set to

"NDC" + "SN"
If NOA is "international number" then set to "CC"+"NDC"+"SN"

	Privacy header field is not present
	
	APRI
	"presentation allowed"

	Privacy header field (IETF RFC 3323 [40])
	priv-value
	APRI
	If the Privacy header field contains the value "user", the APRI shall be set to "presentation restricted". Otherwise, the APRI shall be set to "presentation allowed".


7.2.3.1.2.8
User service information

For coding of the USI see 7.2.3.1.2.5.
7.2.3.1.2.9
Hop Counter (National option)

The I-MGCF shall perform the following interworking procedure if the Hop Counter procedure is supported in the CS network.

At the I‑MGCF the Max-Forwards SIP header shall be used to derive the Hop Counter parameter if applicable. Due to the different default values (that are based on network demands/provisions) of the SIP Max-Forwards header and the Hop Counter, a factor shall be used to adapt the Max Forwards to the Hop Counter at the I‑MGCF. For example, the following guidelines could be applied:
1)
Max-Forwards for a given message should be monotone decreasing with each successive visit to a SIP entity, regardless of intervening interworking, and similarly for Hop Counter.

2)
The initial and successively mapped values of Max-Forwards should be large enough to accommodate the maximum number of hops that may be expected of a validly routed call.

Table 7 shows the principle of the mapping:

Table 7: Max forwards -- hop counter

	Max-Forwards
	= X
	Hop Counter
	= INTEGER part of (X /Factor) =Y

	NOTE:
The Mapping of value X to Y should be done with the used (implemented) adaptation mechanism.


The Principle of adoption could be implemented on a basis of the network provision, trust domain rules and bilateral agreement.
7.2.3.1.2.10
Progress Indicator

If the I-MGCF supports the PSTN XML body as a network option and an INVITE containing a PSTN XML body is received, an available "ProgressIndicator" element in the PSTN XML body shall be mapped into a Progress Indicator in the Access Transport Parameter of the sent IAM as shown in table 7.2.3.1.2.10.1.

Table 7.2.3.1.2.10.1: Contents of the Access Transport Parameter

	INVITE (
	IAM(

	PSTN XML
	Access Transport Parameter

	Progress indicator
	Progress indicator (NOTE)

	NOTE:
A ProgressIndicator with Progress Description value 6 shall not be included into the ISUP ATP, and is mapped instead to Forward call indicators parameter according to table 02a. 


7.2.3.1.2.11
Location Number

Location Number is defined in section 3.30 of ITU-T Q.763 [4].

If the received INVITE message contains a P-Access-Network-Info header and the P-Access-Network-Info header contains an access-type with the value "GSTN" and a gstn-location parameter, the I-MGCF shall include an ISUP Location Number parameter in the outgoing IAM.

Table 7.2.3.1.2.11.1: Contents of the location number parameter

	INVITE (
	IAM(

	
	Location Number parameter

	P-Access-Network-Info with access-type="GSTN" and gstn-location parameter
	Binary value derived from the gstn-location parameter of the P-Access-Network-Info

	NOTE:
The binary value shall be obtained by removing the quotes and converting each pair of consecutive character strings into an octet that has the equivalent hexadecimal representation


Table 7.2.3.1.2.11.2: Mapping of P-Access-Network-Info to Location Number

	INVITE(
	IAM(

	P-Access-Network-Info
	Location Number

	access type="GSTN"
gstn-location=value
	Address Signal:

Copied from octet 3 to n of the binary representation of the gstn-location field

	
	Odd/even indicator:

Copied from bit 8 octet 1 of the the binary representation of the gstn-location field

	
	Nature of address indicator:

Copied from bit 7 to 1 of octet 1 of the the binary representation of the gstn-location field

	
	Internal Network Number Indicator:

Copied from bit 8 of octet 2 of the the binary representation of the gstn-location field

	
	Numbering plan Indicator:

Copied from bit 7 to 5 of octet 2 of the the binary representation of the gstn-location field

	
	Address presentation restricted indicator:
If the SIP Privacy header field = header, APRI is set to "01 (presentation restricted)" otherwise APRI is copied from bit 4 and 3 of octet 2 of the binary representation of the gstn-location field

	
	Screening indicator:

If the np parmater is present in the P-Access-Network-Info header field, set to "11 (network provided)" otherwise set from bit 2 and 1 of octet 2 of the binary representation of the gstn-location field


7.2.3.1.2A
Coding of the IAM when Number Portability is supported

This clause describes optional coding procedures when Number Portability is supported.

7.2.3.1.2A.1
Coding of the IAM when a Number Portability Routing Number is available

ITU-T Q.769.1 [92] describes three possible addressing methods for signalling of the Called Party E.164 address and Number Portability Routing Number (ITU-T Q.769.1 [92] uses the terms directory number and network routing number respectively). The choice of these methods is based on network operator and national requirements.

The following clauses describe how the IAM is populated, based on these methods, when a Number Portability Routing Number is available in the Request URI in the form of a Tel URI "rn=" parameter.

When the optional Number Portability Routing Number is available and supported, these procedures take precedence over procedures for coding of the Called Party Number described in clause 7.2.3.1.2.1.

If the Number Portability Database Dip Indicator is present within the Request-URI the procedures described in clause 7.2.3.1.2A.2 apply. When a Number Portability Routing Number is not available, the Called Party Number parameter is populated as described in clause 7.2.3.1.2.1.

7.2.3.1.2A.1.1
Separate Directory Number Addressing Method

Table 7a.0a: Coding of the called party number and called directory number with Number Portability Separate Directory Number Addressing Method

	INVITE(
	IAM(

	Request-URI
	Called Party Number
	Called Directory Number

	Called Party E.164 address

(format +CC NDC SN)

(e.g. as User info in SIP URI with user=phone, or as tel URL) plus Number Portability Routing Number (format +CC NDC SN) (e.g. as Tel URI rn= parameter) plus Number Portability Database Dip Indicator as defined in IETF RFC 4694 [93] (e.g. as Tel URI npdi parameter)
	Address Signal:

Analyse the information contained in received Number Portability Routing Number. If the Number Portability Routing number contains an E.164 address, then remove "+CC" and use the remaining digits to fill the Address signal. Otherwise, use the digits of the Number Portability Routing number to fill the Address signal.
(NOTE)
	Address Signal:

Remove "+CC" from the Called Party E.164 address and use the remaining digits to fill the Address signals.



	
	Odd/even indicator: set as required
	Odd/even indicator: set as required

	
	Nature of address indicator:

Set Nature of Address indicator to "Network routing number in national (significant) number fornat". "National (significant) number" and "Network routing number in network specific number format" may alternately be chosen as described in ITU-T Q.769.1 [92]
	Nature of address indicator:

Set Nature of Address indicator to "National (significant) number".



	
	Internal Network Number Indicator:

1
routing to internal network number not allowed
	Internal Network Number Indicator:

1
routing to internal network number not allowed

	
	Numbering plan Indicator:

001 ISDN (Telephony) numbering plan (Rec. E.164)
	Numbering plan Indicator:

001 ISDN (Telephony) numbering plan (Rec. E.164)

	NOTE:
If PSTN XML and ISUP Sending Terminated (ST) signal are supported as a network option, then the PSTN XML sendingCompleteIndication, if present, is mapped to the sending terminated digit (hexadecimal digit F) in the address signals field of the Called Party Number parameter.


7.2.3.1.2A.1.2
Concatenated Addressing Method

Table 7a.0b: Coding of the called party number with Number Portability Concatenated Addressing Method
	INVITE(
	IAM(

	Request-URI
	Called Party Number

	Called Party E.164 address

(format +CC NDC SN)

(e.g. as User info in SIP URI with user=phone, or as tel URL) plus

Number Portability Routing Number (format +CC NDC SN) (e.g. as Tel URI rn= parameter) plus

Number Portability Database Dip Indicator as defined in IETF RFC 4694 [93] (e.g. as Tel URI npdi parameter)
	Address Signal:

Analyse the information contained in received Number Portability Routing Number. If the Number Portability Routing number contains an E.164 address, then remove "+CC" and use the remaining digits to fill the Address signal. Otherwise, use the digits of the Number Portability Routing number to fill the Address signal.

Remove the "+CC" from the Called Party E.164 address and append the remaining digits to the Address signal.

(NOTE)

	
	Odd/even indicator: set as required

	
	Nature of address indicator:

set Nature of Address indicator to "Network routing number concatenated with called directory number" or "National (significant) number" as described in ITU-T Q.769.1 [92]

	
	Internal Network Number Indicator:

1
routing to internal network number not allowed

	
	Numbering plan Indicator:

001 ISDN (Telephony) numbering plan (Rec. E.164)

	NOTE:
If PSTN XML and ISUP Sending Terminated (ST) signal are supported as a network option, then the PSTN XML sendingCompleteIndication, if present, is mapped to the sending terminated digit (hexadecimal digit F) in the address signals field of the Called Party Number parameter.


7.2.3.1.2A.1.3
Separate Network Routing Number Addressing Method

Table 7a.0c: Coding of the network routing number and called party number with Number Portability Separate Network Routing Number Addressing Method
	INVITE(
	IAM(

	Request-URI
	Network Routing Number
	Called Party Number

	Called Party E.164 address

(format +CC NDC SN)

(e.g. as User info in SIP URI with user=phone, or as tel URL) plus Number Portability Routing Number (format +CC NDC SN) (e.g. as Tel URI rn= parameter) plus Number Portability Database Dip Indicator as defined in IETF RFC 4694 [93] (e.g. as Tel URI npdi parameter)
	Address Signal:

Analyse the information contained in received Number Portability Routing Number. If the Number Portability Routing number contains an E.164 address, then remove "+CC" and use the remaining digits to fill the Address signal. Otherwise, use the digits of the Number Portability Routing number to fill the Address signal.
	Address Signal:

Remove "+CC" from the Called Party E.164 address and use the remaining digits to fill the Address signals.

(NOTE)

	
	Odd/even indicator: set as required
	Odd/even indicator: set as required

	
	Nature of address indicator:

Set Nature of Address indicator to "Network routing number in national (significant) number format".

"Network routing number in network specific number format" may alternately be chosen as described in ITU-T Q.769.1 [92]
	Nature of address indicator:

Set Nature of Address indicator to "National (significant) number".

	
	Numbering plan Indicator:

001 ISDN (Telephony) numbering plan (Rec. E.164)
	Internal Network Number Indicator:

1
routing to internal network number not allowed

	
	
	Numbering plan Indicator:

001 ISDN (Telephony) numbering plan (Rec. E.164)

	NOTE:
If PSTN XML and ISUP Sending Terminated (ST) signal are supported as a network option, then the PSTN XML sendingCompleteIndication, if present, is mapped to the sending terminated digit (hexadecimal digit F) in the address signals field of the Called Party Number parameter.


7.2.3.1.2A.2
Number Portability Forward Information

Network Operator or National policy may allow forward transfer of number portability status information, as described in ITU-T Q.769.1 [92]. In this case, the following coding applies.

Table 7a.0d: Coding of the number portability forward information

	INVITE(
	IAM(

	Request-URI
	Number Portability Forward Information

	Called Party E.164 address

(format +CC NDC SN)

(e.g. as User info in SIP URI with user=phone, or as tel URL) plus Number Portability Routing Number (format +CC NDC SN) (e.g. as Tel URI rn= parameter) plus Number Portability Database Dip Indicator as defined in IETF RFC 4694 [93] (e.g. as Tel URI npdi parameter)
	If the Number Portability Database Dip Indicator is present, and there is no Number Portability Routing Number, set to "number portability query done for called number, non-ported called subscriber".

If the Number Portability Database Dip Indicator is present, and a Number Portability Routing Number is present, set to "number portability query done for called number, ported called subscriber".

If there is no Number Portability Database Dip Indicator, set to "number portability query not done for called number"


7.2.3.1.2B
Coding of the IAM for Carrier Routeing

This clause describes optional coding procedures for carrier-based routeing. The interworking of the CIC parameter is defined.

7.2.3.1.2B.1
Coding of the IAM when a Carrier Identification Code (CIC) is present

The procedures followed in clause 7.2.3.1.2.1 apply with the following addition.

Based on network configuration, if the tel-URI parameter in a tel-URI or the userinfo part of a SIP URI with user=phone in the Request-URI of an initial INVITE request, contains a "cic=" parameter, as defined in IETF RFC 4694 [93], the I-MGCF may extract the carrier identification code from the "cic=" field for routeing the call. If the outgoing IAM message contains the Transit Network Selection (TNS) parameter, as defined in ITU-T Q.763 [4], based on network configuration the TNS may be populated using the carrier identification code from the "cic=" field. The format of the "cic" parameter (e.g. global-cic and local-cic) shall be compliant to IETF RFC 4694 [93].

7.2.3.1.2B.2
Void
7.2.3.1.3
Sending of COT


[image: image7.wmf] 

SDP indicating 

 

preconditions met

 

 and

/or

 

 

active media

 

COT

 

I

-

MGCF

 


Figure 5: Sending of COT

If the IAM has already been sent, the Continuity message shall be sent indicating "continuity check successful", when all of the following conditions have been met:

-
The requested remote preconditions (if any) in the IMS network have been met.
-
The media stream previously set to inactive mode is set to active (as specified in IETF RFC 4566 [56]).
-
Local preconditions have been fulfilled.
-
A possible outstanding continuity check procedure is successfully performed on the outgoing circuit.
7.2.3.1.3A
Sending of SAM
7.2.3.1.3A.1
General

The procedures in the present clause are only applicable if the I-MGCF supports the network option of overlap signalling, using either the in-dialog method or the multiple INVITEs method. Within one IMS only a single of those methods shall be used.

After the ISUP IAM message has been sent the I-MGCF can receive additional digits. The additional digits may either, as a network option, be received in:

-
in-dialog SIP INFO requests ("legacy" mode of usage of the INFO method as defined in IETF RFC 6086 [133]), in-dialog method as specified in clause 7.2.3.1.3A.2 or
-
in additional SIP INVITE requests, multiple INVITEs method as specified in 7.2.3.1.3A.3.


[image: image8.wmf] 

 SIP message

 

carrying 

additional digits    

 

SAM

 

I

-

MGCF

 


Figure 5a: Sending of SAM

7.2.3.1.3A.2
Additional digits received in in-dialog SIP INFO requests

If interworking of overlap signalling using the in-dialog method is supported by the I-MGCF, the ISUP IAM message has already been sent, but no ISUP ACM or REL message has been received, and a SIP INFO request carrying additional digits is received, then the I-MGCF shall generate a SAM and pass it to outgoing BICC/ISUP procedures. The SAM shall contain in its Subsequent Number parameter only the additional digits received in this SIP INFO request.

7.2.3.1.3A.3
Additional digits received in SIP INVITE requests

If interworking of overlap signalling using the multiple INVITEs method is supported by the I-MGCF, the ISUP IAM message has already been sent, but no ISUP ACM or REL message has been received, and the I-MGCF receives an INVITE with the same Call‑ID and From tag as a previous INVITE which was associated with a BICC/ISUP call/bearer control instance currently existing on the BICC/ISUP side, then:

a)
If the number of digits in the Request‑URI is greater than the number of digits already received for the communication, the I-MGCF shall generate a SAM and pass it to outgoing BICC/ISUP procedures. The SAM shall contain in its Subsequent Number parameter only the additional digits received in this Request‑URI compared with the digits already received for the communication. The I-MGCF shall reply to any earlier INVITE with a 484 Address Incomplete response if this has not already been done.

b)
If the number of digits in the Request‑URI is equal or less than the number of digits already received for the communication, then the I-MGCF shall immediately send a 484 Address Incomplete response for this INVITE. In this case, no SAM shall be sent to BICC/ISUP procedures.


[image: image9.emf]I-MGCF

1. INVITE (1)

2. IAM

3. INVITE (2)

4. SAM

5. 484 (1)

6. INVITE (3)

7. SAM

8. 484 (2)

9.ACM

10. 180 (3)


Figure 5b: Receipt of subsequent INVITE for multiple INVITEs method overlap signalling

On sending of a 484 Address Incomplete message for an INVITE transaction, the I-MGCF considers any offer/answer exchange initiated by the INVITE to be terminated. The new INVITE initiates a new offer/answer exchange. However, if resources have already been reserved and they can be reused within the new offer/answer exchange, the precondition signalling shall reflect the current status of the affected preconditions.
7.2.3.1.4
Sending of 180 ringing

7.2.3.1.4.0
General

The I-MGCF shall send the SIP 180 Ringing when receiving any of the following messages:

-
ACM with Called party's status indicator set to subscriber free.


[image: image10.wmf] 

180 Ringing

 

P

-

Early

-

Media

 (1)

 

ACM 

(Subscriber Free)

 

I

-

MGCF

 

Ring tone

 


NOTE:
The P-Early-Media header field is a included if the received INVITE contains a P-Early-Media header field with the "supported" parameter as described in IETF RFC 5009 [89].

Figure 6: The receipt of ACM

-
CPG with Event indicator set to alerting


[image: image11.wmf] 

180 Ringing

 

P

-

Early

-

Media

 (1)

 

CPG (Alerting)

 

I

-

MGCF

 

Ring tone

 


NOTE:
The P-Early-Media header field is a included if the received INVITE contains a P-Early-Media header field with the "supported" parameter as described in IETF RFC 5009 [89].

Figure 7: Receipt of CPG (Alerting)

For a speech call, if the INVITE request includes the P-Early-Media header field, the I-MGCF shall include in the SIP 180 Ringing response a P-Early-Media header field authorizing early media, except when

-
the I-MGCF has already sent a reliable provisional response (see IETF RFC 3262 [54]) including a P-Early-Media header, as defined in IETF RFC 5009 [89], and

-
the most recently sent P-Early-Media header field authorized early media.

NOTE:
If the I-MGCF signals the P-Early-Media header field authorizing early media, then the IMS can expect tones or announcements to the calling party to flow from the CS network via an MGW controlled by the I-MGCF. In particular, once the I-MGCF sends the 180 Ringing response, ringback is expected in media from the CS network.
As a network option, an I-MGCF may generate a Call-Info header field, or an Alert-Info header field according to rules and procedures of IETF RFC 3261 [19] to provide media instead of the in-band media received from the PSTN.
7.2.3.1.4.0a
PSTN XML body

The procedures in the present clause apply only if the I-MGCF supports the PSTN XML body as a network option. The I-MGCF shall map the Access Transport parameter received in the CPG or ACM into PSTN XML elements as shown in table 7a.0f and include this PSTN XML body in the 180 Ringing.

Table 7a.0f: Mapping of ISUP ATP Parameter into PSTN XML elements

	(18x 
	(CPG or ACM 

	PSTN XML 
	ISUP Parameter
	Content

	ProgressIndicator
	Access Transport Parameter
	Progress indicator

	HighLayerCompatibility (NOTE 2)
	
	High layer compatibility 

	LowLayerCompatibility (NOTE 2)
	
	Low layer compatibility

	BearerCapability (NOTE 1, NOTE 2)
	
	Bearer Capability

	BearerCapability (NOTE 1, NOTE 2)
	Transmission medium used parameter (NOTE 1)
	

	NOTE 1:
see clause 7.2.3.1.4.1 Transmission Medium Used parameter (TMU)
NOTE 2:
The I-MGCF shall only provide this IE if it interworks media encoded in any of the formats in table 2a (G.711, Clearmode, or t38) without transcoding. If both TMU and a BC in the ATP have been received, the BC in the ATP shall be mapped.


The I-MGCF shall map a possibly available "Progress indicator" element in the ATP parameter within the ACM or CPG into a ProgressIndicator in the PSTN XML body of the 180 Ringing. In addition, the I-MGCF shall map the Backward call indicators parameter and the Optional backward call indicators parameter (if present) within the ACM or CPG into other ProgressIndicator(s) in the PSTN XML body of the 180 Ringing as shown in table 7a.0g.
NOTE:
The order of ProgressIndicators within the same PSTN XML body is irrelevant.
Table 7a.0g: Mapping of ISUP BCI and optional BCI parameters into PSTN XML ProgressIndicator
	( 180 Ringing 
	(ACM or CPG

	 PSTN XML body with ProgressIndicator with "Progress Description" value No (Value of PI)
	Backward call indicators parameter

Optional backward call indicators parameter

	No. 1

("Call is not end-to-end ISDN: further call progress information may be available in-band")
	Backward call indicators parameter


ISDN User Part indicator



0
"ISDN User Part not used all the way"

	No. 2

("Destination address is non-ISDN")
	Backward call indicators parameter


ISDN User Part indicator



1
"ISDN User Part used all the way"

ISDN access indicator



0
"Terminating access non-ISDN"

	No. 7
("Terminating access ISDN")
	Backward call indicators parameter


ISDN User Part indicator



1
"ISDN User Part used all the way"

ISDN access indicator



1
"Terminating access ISDN"

	No. 8

("In-band information or an appropriate pattern is now available")
	Optional backward call indicators parameter


In-band information indicator



"in-band information or an appropriate pattern is
now available"

	NOTE:
The ProgressIndicator "Coding Standard" parameter shall be set to "00 (ITU-T standardized coding)". The default value for the ProgressIndicator "Location" parameter is "0011 (Transit Network)".


7.2.3.1.4.0b
Fallback by I-MGCF

If the I-MGCF supports the PSTN XML body as a network option and the I-MGCF received two PSTN XML Bearer Capability elements within the INVITE Request, but the succeeding trunk does not support TMR "64 kBit/s preferred" ( as described in clause 7.2.3.1.2.5a), and if no corresponding elements were received in the Access Transport Parameter (see table 7a.0f) , then the I-MGCF shall create a PSTN XML body containing the following elements, and include it in the 18x Response :

-
a BearerCapability element, which shall becopied from the first BearerCapability element received in the PSTN XML in the INVITE;

-
if two HighLayerCompatibility elements were present in the PSTN XML body in the INVITE, then a HighLayerCompatibility element, which shall be copied from the first HighLayerCompatibility element received in the PSTN XML in the INVITE; and

-
a ProgressIndicator element with "Progress Description" value 5 ("Interworking has occurred and has resulted in a telecommunication service change"),"Coding Standard" value "00 (ITU-T standardized coding)", and default value "0011 (Transit Network)" for the "Location" parameter.
7.2.3.1.4.1
Fallback in a succeeding network: Transmission Medium Used parameter (TMU) received
The procedures in the present clause apply only if the I-MGCF supports the PSTN XML body as a network option and receives a Transmission Medium Used parameter (TMU).

NOTE:
The I-MGCF will only receive a TMU parameter if it has applied the Fallback related procedures in clause 7.2.3.1.2.5a, including both a TMR and TMR Prime in the IAM, and fallback to the bearer capability identified in TMR Prime and USI occurred in a succeeding network.
If the I-MGCF receives a Transmission Medium Used parameter (TMU) , the I-MGCF shall:

-
If an BC is not available in the ATP in the Address Complete Message (ACM) or Call Progress Message (CPG), map the TMU value (Speech or 3.1 kHz audio) to the PSTN XML BearerCapability;

-
If an BC is available in the ATP in the Address Complete Message (ACM) or Call Progress Message (CPG), include the received BC value the PSTN XML BearerCapability;

-
configure the IM-MGW to use the second format in the m-line in the SDP that has been received in the INVITE as codec at the IMS termination; and

-
send SDP selecting the second format in the m-line of the SDP that has been received in the INVITE as soon as allowed according to SIP rules.
Table 7.2.3.1.4.1.1: Sending of BC fallback indication

	( 180 Ringing or 183 Session Progress
	(ACM/CPG

	PSTN XML BearerCapability = "Speech"

	TMU

"Speech"
ATP

No BC

	PSTN XML BearerCapability = "3.1 kHz audio"

	TMU

"3.1 kHz audio"
ATP

No BC

	PSTN XML BearerCapability received in the ATP
("speech" or "3.1 kHz audio")
	TMU

"Speech" or "3.1 kHz audio"
ATP

BC ("speech" or "3.1 kHz audio")


7.2.3.1.4A
Sending of 183 Session Progress for early media scenarios

If SIP preconditions are used, the first 183 Session Progress will be sent after the reception of the INVITE request, before any ISUP message has been received from the CS network. The I-MGCF shall not include the P-Early-Media header field in any SIP message before it receives an ISUP ACM.

For a speech call upon receipt of one of the following messages and if the I-MGCF has received the P-Early-Media header field in the INVITE request, and has not already sent a provisional response including a P-Early-Media header field with parameters indicating authorization of early media, then the I-MGCF shall send the 183 Session Progress response with a P-Early-Media header field authorizing early media:

-
ACM with the value of the called party's status indicator "no indication" and one of the options described in table 7.2.3.1.4A.1 If the I-MGCF supports the PSTN XML body as a network option, the I-MGCF shall map parameters within the ACM into the PSTN XML body within the 183 as indicated in table 7.2.3.1.4A.1. Based on local configuration, the I-MGCF may also send a 183 Session Progress response with a P-Early-Media header field authorizing early media if it receives an ACM with other parameters than described in table 7.2.3.1.4A.1.

[image: image12.wmf] 

18

3 Progress

 

P

-

Early

-

Media

 

ACM 

 

(

No indication

)

 

I

-

MGCF

 

appropriate announc

e

ment

 


Figure 7c: Receipt of ACM "No indication"
Table 7.2.3.1.4A.1: ACM Parameters that trigger the 183 Session Progress response
	(183 Session Progress
	(ACM

	P-Early-Media header field authorizing early media, if not already sent
PSTN XML with ProgressIndicator with "Progress Description" value No. 8

("In-band information or appropriate pattern is now available") (NOTE)
	Optional backward call indicators parameter



In-band information indicator


1  "In-band info or an appropriate pattern is now 
       available"

	PSTN XML with ProgressIndicator with "Progress Description" value No. 1

("Call is not end-to-end ISDN: further call progress information may be available in-band") (NOTE)
	Backward call indicators parameter



ISDN User Part indicator

   0
"ISDN User Part not used all the way"

	PSTN XML with ProgressIndicator with "Progress Description" value No. 2 ("Destination address is non-ISDN") (NOTE)
	Backward call indicators parameter



ISDN User Part indicator


0
"ISDN User Part used all the way"


ISDN access indicator


0
"Terminating access non-ISDN"

	PSTN XML with ProgressIndicator with "Progress Description" value No. 7

("Terminating access ISDN") (NOTE)
	Backward call indicators parameter


ISDN User Part indicator



1
"ISDN User Part used all the way"



ISDN access indicator



1
"Terminating access ISDN"

	NOTE:
The ProgressIndicator "Coding Standard" parameter shall be set to "00 (ITU-T standardized coding)". The default value for the ProgressIndicator "Location" parameter is "0011 (Transit Network)".


NOTE 1:
As a network option the I-MGCF can also map ACM into 183 in other cases than those described in table 7.2.3.1.4A.1.
-
CPG message, when:

1.
Event indicator is set to "in-band information or an appropriate pattern is now available", or

2.
Event indicator is set to "Progress" and one of the options described in table 7.2.3.1.4A.2.


[image: image13.wmf] 

CPG

 

 (

in

-

band informa

tion available

)

 

18

3 Progress

 

P

-

Early

-

Media

 

I

-

MGCF

 

appropriate announcment

 


Figure 7d: Receipt of CPG (in-band information available)
Table 7.2.3.1.4A.2: CPG Parameters that trigger the 183 Session Progress response
	(183 Session Progress
	(CPG

	P-Early-Media header field authorizing early media, if not already sent

PSTN XML with ProgressIndicator with "Progress Description" value No. 8

("In-band information or appropriate pattern is now available") (NOTE 3)
	Event indicator

000 0010

(progress)


Optional backward call indicators parameter


In-band information indicator


1
"In-band info or an appropriate pattern is now
available"

	PSTN XML with ProgressIndicator with "Progress Description" value No. 1

(Call is not end-to-end ISDN: further progress information may be available in-band") (NOTE 3)
	Event indicator

000 0010

(progress)

Backward call indicators parameter

ISDN User Part indicator


0
"ISDN User Part not used all the way"

	PSTN XML with ProgressIndicator with "Progress Description" value No. 2 ("Destination address is non-ISDN") (NOTE 3)
	Backward call indicators parameter


ISDN User Part indicator


"ISDN User Part used all the way"

ISDN access indicator


0
"Terminating access non-ISDN"

	PSTN XML with ProgressIndicator with "Progress Description" value No. 7

("Terminating access ISDN") (NOTE 3)
	Backward call indicators parameter


ISDN User Part indicator


"ISDN User Part used all the way"

ISDN access indicator


1
"Terminating access ISDN"

	NOTE 1:
The mapping of the contents in the CPG message is only relevant if the information received in the message is different compared to earlier received information, e.g., in the ACM message or a CPG message received prior to this message.

NOTE 2:
183 Session Progress message including a P-Early-Media header authorizing early media may only be sent for a speech call.
NOTE 3:
The ProgressIndicator "Coding Standard" parameter shall be set to "00 (ITU-T standardized coding)". The default value for the ProgressIndicator "Location" parameter is "0011 (Transit Network)".


NOTE 2:
As a network option the I-MGCF can also map CPG into 183 in other cases than those described in table 7.2.3.1.4A.1.
If the I-MGCF supports the PSTN XML body as a network option, the I-MGCF shall map the Access Transport Parameter received in the CPG or ACM into PSTN XML elements as shown in table 7a.0f and include this PSTN XML body in the 183 Session Progress. The I-MGCF shall include both a ProgressIndicator mapped from possibly received Progress indicator element in the Access Transport Parameter and ProgressIndicators derived according to table 7.2.3.1.4A.1 or table 7.2.3.1.4A.2 in the PSTN XML body.
NOTE 3:
The order of ProgressIndicators within the same PSTN XML body is irrelevant.
If the I-MGCF has applied UDI-TA fallback related procedures in clause 7.2.3.1.2.5a, the I-MGCF shall also apply the procedures in clauses 7.2.3.1.4.0b and 7.2.3.1.4.1, including the PSTN XML body in the 183 Session Progress.
As a network option, an I-MGCF may generate a Call-Info header field, or an Alert-Info header field according to rules and procedures of IETF RFC 3261 [19] to provide media instead of the in-band media received from the PSTN.
If the ACM or CPG contains an ISUP Cause, the MGCF may add a Reason header field containing the received Cause Value to the SIP 183 Session Progress provisional response as a network option. The mapping of the Cause Indicators parameter to the Reason header as shown in table 9a shall be applied. IETF RFC 6432 [115] describes the use of the Reason header field in responses. The Reason header field itself is described in IETF RFC 3326 [116].
7.2.3.1.4B
Sending of 181Call is being forwarded

The I-MGCF shall send the SIP 181 Call is being forwarded when receiving any of the following messages:

-
ACM with call diversion information not indicating that presentation is not allowed and optional backward call indicators indicate that in-band information is available.


[image: image14.wmf] 

181

 Call is being 

forwarded

 

P

-

Early

-

Media

 (1)

 

ACM( call diver

-

sion information)

 

I

-

MGCF

 

Audio

 


NOTE 1:
The P-Early-Media header field is a included if the received INVITE contains a P-Early-Media header field with the "supported" parameter as described in IETF RFC 5009 [89].

Figure 7c: The receipt of ACM (call diversion information)

-
CPG with call diversion information not indicating that presentation is not allowed and optional backward call indicators indicate that in-band information is available.


[image: image15.wmf] 

181 Call is being 

forwarded

 

P

-

Early

-

Media

 (1)

 

CPG(Call diversion 

information

)

 

I

-

MGCF

 

Audio

 


NOTE 1:
The P-Early-Media header field is a included if the received INVITE contains a P-Early-Media header field with the "supported" parameter as described in IETF RFC 5009 [89].

Figure 7d: Receipt of CPG (Call diversion information)

For a speech call, and if the INVITE request includes the P-Early-Media header, the I-MGCF shall include in the SIP 181 Call is being forwarded response a P‑Early-Media header authorizing early media, except when

-
the I-MGCF has already sent a reliable provisional response including a P-Early-Media header field, as defined in IETF RFC 5009 [89], or a 180 Ringing response; and

-
the most recently sent P-Early-Media header field authorized early media.

NOTE:
If the I-MGCF signals the P-Early-Media header field authorizing early media, then the IMS can expect tones or announcements to the calling party to flow from the CS network via an MGW controlled by the I‑MGCF.

7.2.3.1.4C
Sending of 183 Session Progress for overlap signalling using the in-dialog method

If the I-MGCF supports the network option of overlap signalling using the in-dialog method, and the SIP INVITE request contained an indication that the 100rel extension is supported or required, the I-MGCF shall send a reliable 183 (Session Progress) response immediately after the reception of an INVITE request.

NOTE:
If the INVITE request does not contain an indication that the 100rel extension is supported or required, it is assumed that overlap is not used, and that no further digits will be received.

7.2.3.1.4D
Sending of 183 Session Progress to carry ISUP Cause

If the I-MGCF receives an ACM or CPG message containing an ISUP Cause, and if the I-MGCF does not send any SIP provisional response due to interworking procedures described in clauses 7.2.3.1.4A, for the ACM or CPG message, the I-MGCF may send a SIP 183 Session Progress with a Reason header field containing the received Cause Value as a network option. The mapping of the Cause Indicators parameter to the Reason header as shown in table 9a shall be applied. IETF RFC 6432 [115] describes the use of the Reason header field in responses. The Reason header field itself is described in IETF RFC 3326 [116].


[image: image16.wmf] 

18

3

 

Session Progress

 

(Reason(Cause=xxx))

 

ACM

 or CPG

 

(

Cause=xxx

)

 

I

-

MGCF

 


Figure 7.2.3.1.4D.1: The receipt of ACM or CPG with ISUP Cause
7.2.3.1.5
Sending of the 200 OK (INVITE)

The following cases are possible trigger conditions for sending the 200 OK (INVITE):

-
The reception of the ANM.


[image: image17.wmf] 

ANM

 

200 OK  (INVITE)

 

I

-

MGCF

 


Figure 8: Receipt of ANM

-
The reception of the CON message.


[image: image18.wmf] 

I

-

MGCF

 

CON

 

200 OK (INVITE)

 


Figure 9: Receipt of CON
If the I-MGCF supports the PSTN XML body as a network option, the I-MGCF shall map the Access Transport Parameter received in the ANM or CON into PSTN XML elements as shown in table 7.2.3.1.5.1 and include this PSTN XML body in the 200 OK (INVITE).

On receipt of an ANM/CON message containing the ATP including the Bearer Capability set to "unrestricted digital information with tones/announcement" without TMU parameter the 200 OK message shall contain the PSTN XML Bearer Capability "unrestricted digital information with tones/announcement".
If the I-MGCF supports the PSTN XML body as a network option, the I-MGCF shall map an available BCI element in the ANM or CON into a ProgressIndicator in the PSTN XML body as shown in table 7.2.3.1.5.2; the I-MGCF shall include both a ProgressIndicator mapped from possibly received Progress indicator element in the Access Transport Parameter and ProgressIndicators derived according to table 7.2.3.1.5.2 in the PSTN XML.
NOTE 1:
The order of ProgressIndicators within the same PSTN XML body is irrelevant.
Table 7.2.3.1.5.1: Mapping of ISUP ATP Parameter into PSTN XML elements

	(200 OK
	(ANM or CON 

	PSTN XML 
	ISUP Parameter
	Content

	ProgressIndicator
	Access Transport Parameter
	Progress indicator

	HighLayerCompatibility (NOTE 2)
	
	High layer compatibility 

	LowLayerCompatibility (NOTE 2)
	
	Low layer compatibility

	BearerCapability (NOTE 1, NOTE 2)
	
	Bearer Capability

	BearerCapability (NOTE 1, NOTE 2)
	Transmission medium used parameter (NOTE 1)
	

	NOTE 1:
see clause 7.2.3.1.4.1 Transmission Medium Used parameter (TMU)
NOTE 2:
The I-MGCF shall only provide this IE if it interworks media encoded in any of the formats in table 2a (G.711, Clearmode, or t38) without transcoding. If both TMU and a BC in the ATP have been received, the BC in the ATP shall be mapped.


Table 7.2.3.1.5.2: Mapping of ISUP BCI and optional BCI parameters into PSTN XML ProgressIndicator
	( 200 OK 
	(ANM or CON

	 PSTN XML body with ProgressIndicator with "Progress Description" value No (Value of PI) (NOTE)
	Content
Backward call indicators parameter

Optional backward call indicators parameter

	No. 1

("Call is not end-to-end ISDN: further call progress information may be available in-band")
	Backward call indicators parameter


ISDN User Part indicator



0
"ISDN User Part not used all the way"

	No. 2

("Destination address is non-ISDN")
	Backward call indicators parameter


ISDN User Part indicator



1
"ISDN User Part used all the way"

ISDN access indicator



0
"Terminating access non-ISDN"

	No. 7

("Terminating access ISDN")
	Backward call indicators parameter


ISDN User Part indicator



1
"ISDN User Part used all the way"

ISDN access indicator



1
"Terminating access ISDN"

	No. 8

("In-band information or an appropriate pattern is now available")
	Optional backward call indicators parameter


In-band information indicator



 1  "in-band information or an appropriate 
          pattern is now available"

	NOTE:
The ProgressIndicator "Coding Standard" parameter shall be set to "00 (ITU-T standardized coding)". The default value for the ProgressIndicator "Location" parameter is "0011 (Transit Network)".


If the I-MGCF supports the PSTN XML body and receives a Transmission Medium Used (TMU) parameter,

NOTE 2:
The I-MGCF will only receive a TMU parameter if it has applied the Fallback related procedures in clause 7.2.3.1.2.5a, including both a USI and TMR Prime parameter in the IAM, and fallback to the bearer capability identified in USI and TMR Prime occurred at the terminating side.

then the I-MGCF shall:

-
if a BC is not available in the ATP in the ANM or CON, map the TMU value (Speech or 3.1 kHz audio) to the PSTN XML BearerCapability element;

-
if a BC is available in the ATP in the ANM or CON, include the received BC in the PSTN XML BearerCapability element;

-
configure the IM-MGW to use the second format in the m-line in the SDP that has been received in the INVITE as codec at the IMS termination; and

-
send SDP selecting the second format in the m-line of the SDP that has been received in the INVITE as soon as allowed according to SIP rules.
If the I-MGCF supports the PSTN XML body, has applied the Fallback related procedures in clause 7.2.3.1.2.5a, including both a TMR and TMR Prime in the IAM, and did not receive TMU in the ANM, CON, or any previous ISUP message,

NOTE 3:
Fallback to the bearer capability identified in TMR did not occurre at the terminating side.

then the I-MGCF shall:

-
configure the IM-MGW to use the first format in the m-line in the SDP offer that has been received in the INVITE as codec at the IMS termination; and

-
send SDP selecting the first format in the m-line in the SDP offer at the first possibility according to SIP rules.
7.2.3.1.6
Sending of the Release message (REL)
The following are possible triggers for sending the Release message:

-
Receipt of the BYE method

[image: image19.wmf] 

BYE

 

REL

 

I

-

MGCF

 


Figure 10: Receipt of the Bye method

-
Receipt of the CANCEL method


[image: image20.wmf] 

I

-

MGCF

 

CANCEL

 

REL

 


Figure 11: Receipt of Cancel method

Additional triggers are contained in table 10.

7.2.3.1.7
Coding of the REL

If the Reason header field with Q.850 Cause Value is included in the BYE or CANCEL request, then the Cause Value shall be mapped to the ISUP Cause Value field in the ISUP REL. The mapping of the Reason header to the Cause Indicators parameter is shown in table 8a. Table 8 shows the coding of the Cause Value in the REL if it is not available from the Reason header field. In both cases, the Location Field shall be set to "network beyond interworking point".
Table 8: Coding of REL

	SIP Message (
	REL (

	Request
	cause indicators parameter

	BYE
	Cause value No. 16 (normal clearing)

	CANCEL
	Cause value No. 16 (normal clearing) 


Table 8a: Mapping of SIP Reason header fields 
into Cause Indicators parameter

	Component of SIP Reason header field
	Component value
	BICC/ISUP Parameter field
	Value 

	Protocol
	"Q.850"
	Cause Indicators parameter
	–

	protocol‑cause
	"cause = XX" (NOTE 1)
	Cause Value
	"XX" (NOTE 1)

	–
	–
	Location
	"network beyond interworking point"

	NOTE 1:
"XX" is the Cause Value as defined in ITU-T Recommendation Q.850 [38].


Editor's Note: The mapping of reason headers towards the ISDN may be misused due to possible user creation of the reason header since there is no screening in IMS.
If the I-MGCF supports this PSTN XML body as a network option and the I-MGCF interworks media encoded in any of the formats in table 2a (G.711, Clearmode or t38) without transcoding, and if a PSTN XML body is received in the BYE or CANCEL request, the I-MGCF shall derive the Access Transport Parameter in the REL message from the PSTN XML body as shown in table 8b.

Table 8b: Mapping of PSTN XML elements into ISUP Parameters

	BYE or CANCEL ( 
	REL (

	PSTN XML 
	ISUP Parameter
	Content

	HighLayerCompatibility
	Access Transport Parameter
	High layer compatibility 

	LowLayerCompatibility
	
	Low layer compatibility


7.2.3.1.8
Receipt of the Release Message

If the REL message is received and a final response (i.e. 200 OK (INVITE)) has already been sent, the I-MGCF shall send a BYE message.

NOTE:
According to SIP procedures, in the case that the REL message is received and a final response (e.g. 200 OK (INVITE)) has already been sent (but no ACK request has been received) on the incoming side of the I-MGCF then the I-MGCF does not send a 487 Request terminated response and instead waits until the ACK request is received before sending a BYE message.

If the REL message is received and the final response (i.e. 200 OK (INVITE)) has not already been sent, the I-MGCF shall send a Status-Code 4xx (Client Error), 5xx (Server Error) or 6xx (Global Failure) response. The Status code to be sent is determined by examining the Cause value received in the REL message. Table 9 specifies the mapping of the cause values, as defined in ITU-T Recommendation Q.850 [38], to SIP response status codes. Cause values not appearing in the table shall have the same mapping as the appropriate class defaults according to ITU-T Recommendation Q.850 [38].

Table 9: Receipt of the Release message (REL)

	(SIP Message
	( REL

	Status code
	Cause indicators parameter

	404 Not Found
	Cause value No 1 (Unallocated (unassigned) number)

	604 Does not exist anywhere
	Cause value No 2 (No route to specified transit network)

	604 Does not exist anywhere
	Cause value No 3 (No route to destination)

	500 Server Internal error
	Cause value No 4 (Send special information tone)

	404 Not Found
	Cause value No 5 (Misdialled trunk prefix)

	486 Busy Here
	Cause value No 17 (User busy)

	480 Temporarily unavailable
	Cause value No 18 (No user responding)

	480 Temporarily unavailable
	Cause value No 19 (No answer from user (user alerted))

	480 Temporarily unavailable
	Cause value No 20 (Subscriber absent)

	603 Decline IF location field is set to user ELSE 403 Forbidden
	Cause value No 21 (Call rejected)

	410 Gone
	Cause value No 22 (Number changed)

	410 Gone
	Cause value No 23 (Redirection to new destination)

	433 Anonymity Disallowed (NOTE 1)
	Cause value No 24 (Call rejected due to ACR supplementary service)

	483 Too Many Hops
	Cause value No 25 (Exchange routing error)

	480 Temporarily unavailable
	Cause value No 26 (Non-selected user clearing)

	502 Bad Gateway
	Cause value No 27 (Destination out of order)

	484 Address Incomplete
	Cause value No 28 (Invalid number format (address incomplete))

	501 (Not Implemented)
	Cause value No 29 (Facility rejected)

	480 Temporarily unavailable
	Cause value No 31 (Normal, unspecified)
(class default) (NOTE 2)

	486 Busy here if Diagnostics indicator includes the (CCBS indicator = CCBS possible)

else 503 Service Unavailable (NOTE 3)
	Cause value No 34 (No circuit/channel available)

	500 Server Internal error
	Cause value No 38 (Network out of order)

	503 Service Unavailable (NOTE 3)
	Cause value No 41 (Temporary failure)

	503 Service Unavailable (NOTE 3)
	Cause value No 42 (Switching equipment congestion)

	500 Server Internal error
	Cause value No 43 (Access information discarded)

	503 Service Unavailable (NOTE 3)
	Cause value No 44 (Requested channel not available)

	500 Server Internal error
	Cause value No 46 (Precedence call blocked)

	503 Service Unavailable (NOTE 3)
	Cause value No 47 (Resource unavailable, unspecified)
(class default)

	488 Not acceptable here
	Cause value No 50 (Requested facility not subscribed)

	603 Decline
	Cause value No 55 (Incoming class barred within Closed User Group (CUG))

	603 Decline
	Cause value No 57 (Bearer capability not authorised)

	503 Service Unavailable (NOTE 3)
	Cause value No 58 (Bearer capability not presently available)

	501 (Not Implemented)
	Cause value No 63 (Service option not available, unspecified)
(class default)

	500 Server Internal error
	Cause value No 65 (Bearer capability not implemented)

	501 Not Implemented
	Cause value No 69 (Requested facility not implemented)

	501 Not Implemented
	Cause value No 70 (Only restricted digital information capability is available)

	501 Not Implemented
	Cause value No 79 (Service or option not implemented, unspecified)
(class default)

	403 Forbidden
	Cause value No 87 (User not member of Closed User Group (CUG))

	606 Not Acceptable
	Cause value No 88 (Incompatible destination)

	403 Forbidden
	Cause value No 90 (Non existing Closed User Group (CUG))

	500 Server Internal error
	Cause value No 91 (Invalid transit network selection)

	513 Message too large
	Cause value No 95 (Invalid message, unspecified)
(class default)

	501 Not Implemented
	Cause value No 97 (Message type non-existent or not implemented)

	501 Not Implemented
	Cause value No 98 (Message not compatible with call state or message type non-existent or not implemented)

	501 Not Implemented
	Cause value No 99 (Information element/parameter non-existent or not implemented)

	504 Server timeout
	Cause value No 102 (Recovery on timer expiry)

	501 Not Implemented
	Cause value No 103 (Parameter non-existent or not implemented, passed on)

	501 Not Implemented
	Cause value No 110 (Message with unrecognised parameter, discarded)

	400 Bad Request
	Cause value No 111 (Protocol error, unspecified)
(class default)

	500 Server Internal error
	Cause value No 127 (Interworking, unspecified)
(class default)

	NOTE 1:
Anonymity Disallowed, IETF RFC 5079 [77] refers

NOTE 2:
Class 0 and class 1 have the same default value.
NOTE 3:
No Retry-After header field shall be included.


A Reason header field containing the received (Q.850) Cause Value of the REL shall be added to the SIP final response or BYE request sent as a result of this subclause. The mapping of the Cause Indicators parameter to the Reason header is shown in table 9a. IETF RFC 6432 [115] describes the use of the Reason header field in responses. The Reason header field itself is described in IETF RFC 3326 [116].
Table 9a: Mapping of Cause Indicators parameter into SIP Reason header fields

	Cause indicators parameter field
	Value of parameter field
	component of SIP Reason header field
	component value

	–
	–
	protocol
	"Q.850"

	Cause Value
	"XX" (NOTE 1)
	protocol‑cause
	"cause = XX" (NOTE 1)

	–
	–
	reason‑text
	Should be filled with the definition text as stated in ITU-T Recommendation Q.850 [38].
(NOTE 2)

	NOTE 1:
"XX" is the Cause Value as defined in ITU-T Recommendation Q.850 [38].
NOTE 2:
Due to the fact that the Cause Indicators parameter does not include the definition text as defined in table 1/Q.850 [38], this is based on provisioning in the I-MGCF.


As a network option, an I-MGCF may generate an Error-Info header field according to rules and procedures of IETF RFC 3261 [19] to provide media instead of the in-band media received from the PSTN.
If the I-MGCF supports the PSTN XML body as a network option, the I-MGCF shall map the Access Transport Parameter received in the REL into PSTN XML elements as shown in table 9aa and include this PSTN XML body in the SIP final response or BYE.
Table 9aa: Mapping of ISUP Parameters into PSTN XML elements

	(4xx,5xx,6xx or BYE
	(REL

	PSTN XML 
	ISUP Parameter
	Content

	ProgressIndicator
	Access Transport Parameter
	Progress indicator

	HighLayerCompatibility (NOTE)
	
	High layer compatibility

	LowLayerCompatibility (NOTE)
	
	Low layer compatibility

	NOTE:
The I-MGCF shall only provide this IE if it interworks media encoded in any of the formats in table 2a (G.711, Clearmode, or t38) without transcoding,


7.2.3.1.9
Receipt of RSC, GRS or CGB (H/W oriented)

Upon receipt of a RSC, GRS or CGB (H/W oriented) message the following applies independently for each affected circuit:

NOTE:
For the RSC message, the circuit identified by the CIC is affected.
For the GRS message, the affected circuits are identified by the CIC and the Range subfield of the Range and Status parameter.
For the CGB message, the affected circuits are identified by the CIC and the Range and Status parameter.

If an initial address message has been sent for the affected circuit and after at least one backward message relating to that call has been received then:

-
If the final response (i.e. 200 OK (INVITE)) has already been sent, the I-MGCF shall send a BYE message.

-
If the final response (i.e. 200 OK (INVITE)) has not already been sent, the I-MGCF shall send a SIP response with Status-Code 480 Temporarily Unavailable.
A Reason header field containing the (Q.850) Cause Value of the REL message generated by the ISUP procedures shall be added to the SIP message (BYE or 480 response) to be sent by the SIP side of the I‑MGCF.

7.2.3.1.9a
Receipt of REFER


[image: image21.emf]MGCF

REFER

To: B

P-Asserted ID: A

Refer to: C

Method: INVITE

IM CN Subsystem CS Network

403 Forbidden


Figure 11a: Receipt of REFER method

Upon receipt of a REFER request at the MGCF, the default behaviour of the I-MGCF is to reject the REFER request with a 403 Forbidden response.

NOTE:
The I-MGCF may also decide for example to execute the REFER request as specified in IETF RFC 3515 [75] as an operator option, but such handling is outside of the scope of the present document.

7.2.3.1.10
Autonomous Release at I-MGCF

Table 10 shows the trigger events at the MGCF and the release initiated by the MGCF when the call is traversing from SIP to ISUP/BICC.
A Reason header field containing the (Q.850) Cause Value of the REL message sent by the I‑MGCF shall be added to the SIP Message (BYE request or final response) sent by the SIP side of the I‑MGCF.
Editor's Note: It is FFS whether to indicate the cause value for internal error in the network to the user.

Table 10: Autonomous Release at I‑MGCF

	( SIP

Response
	Trigger event
	REL (

	
	
	cause parameter

	484 Address Incomplete
	Determination that insufficient digits received.
	Not sent.

	480 Temporarily Unavailable
	Congestion at the MGCF/Call is not routable.
	Not sent.

	BYE
	ISUP/BICC procedures result in release after answer
	According to ISUP/BICC procedures.

	BYE
	SIP procedures result in release after answer.
	127 (Interworking unspecified)

	500 Server Internal error
	Call release due to the ISUP/BICC compatibility procedure (NOTE)
	According to ISUP/BICC procedures.

	484 Address Incomplete
	Call release due to expiry of T7 within the ISUP/BICC procedures
	According to ISUP/BICC procedures.

	480 Temporarily Unavailable
	Call release due to expiry of T9 within the BICC/ISUP procedures
	According to BICC/ISUP procedures.

	480 Temporarily Unavailable.
	Other BICC/ISUP procedures result in release before answer.
	According to BICC/ISUP procedures.

	NOTE:
MGCF receives unrecognized ISUP or BICC signalling information and determines that the call needs to be released based on the coding of the compatibility indicators, refer to ITU-T Recommendation Q.764 [4] and ITU-T Q.1902.4 [30].


7.2.3.1.11
Internal through connection of the bearer path

The through connection procedure is described in clause 9.2.2.3.5.
7.2.3.2
Outgoing Call Interworking from ISUP to SIP at O-MGCF

7.2.3.2.1
Sending of INVITE

7.2.3.2.1.1
General

[image: image22.wmf] 

IAM

 

INVITE 

 

O

-

MGCF

 


Figure 11b: Receipt of an IAM

Upon reception of an IAM message, the O-MGCF shall send a SIP INVITE request, as further detailed in the clauses below.

An O-MGCF shall support both the SIP preconditions and 100 rel extensions and indicate the support of the SIP preconditions and 100rel extensions in the INVITE request, unless the Note below applies.

NOTE:
If the O-MGCF is deployed in an IMS network that by local configuration serves no user requiring preconditions, it may send the INVITE request without indicating support of preconditions.

7.2.3.2.1.2
Interaction with continuity check

If the Continuity Check indicator in the Nature of Connection Indicators parameter in the incoming IAM is set to indicate either "continuity check required on this circuit" or "continuity check performed on previous circuit", the O-MGCF should defer sending the INVITE request until receiving a COT message.
NOTE:
If the Continuity Check indicator in the Nature of Connection Indicators parameter in the incoming IAM is set to indicate either "continuity check required on this circuit" or "continuity check performed on previous circuit" and the O-MGCF sends the INVITE request before receiving a COT message, the following considerations apply: If the receiving terminal is not supporting the SIP precondition and the SIP UPDATE method, clipping may occur. Furthermore, if the MGCF sets the SDP "inactive" attribute in the initial INVITE request and the receiving terminal is not supporting the SIP precondition, the interworking procedures within the present specification do not describe all necessary signalling interactions required to set up a call, in particular with respect to the sending of the re-INVITE that may also cause additional delay in the call setup. In addition, the interworking of the ringing indication might not be possible if the peer sends the ringing indication only as response to a re-INVITE.

[image: image23.wmf] 

IAM

 

INVITE 

 

O

-

MGCF

 

COT 

(NOTE)

 


NOTE:
Waiting for the COT is recommended, if the Continuity Check indicator in the Nature of Connection Indicators parameter in the incoming IAM is set to indicate either "continuity check required on this circuit" or "continuity check performed on previous circuit"
Figure 12: Receipt of an IAM (Waiting for the COT message)

7.2.3.2.1.3
IAM without calling party number

If no calling party number is received in the incoming IAM message, as a network option, the O-MGCF may send an INR message to request the calling party number and not send the INVITE request until receiving an INF message with calling party number. If no calling party number is received in the INF message, O-MGCF may reject or continue the call based on local configuration.


[image: image24.wmf] 

IAM

 

INR

 

INVITE

 

O

-

MGCF

 

INF

 


Figure 12a: Receipt of an IAM (Request for calling party number)

7.2.3.2.1.4
Terminating overlap signalling at MGCF


[image: image25.wmf] 

IAM

 

SAM

 

INVITE 

 

O

-

MGCF

 


Figure 13: Receipt of an IAM (Overlap signalling in CS network)

After initiating the normal incoming BICC/ISUP call establishment procedures, determining the end of address signalling and selecting to route the call to the IMS domain, the O‑MGCF shall send the initial INVITE.

The end of address signalling shall be determined by the earlier of the following criteria:

a)
by receipt of an end-of-pulsing (ST) signal; or

b)
by receipt of the maximum number of digits used in the national numbering plan; or

c)
by analysis of the called party number to indicate that a sufficient number of digits has been received to route the call to the called party; or

d)
by observing that timer Ti/w1 has expired after the receipt of the latest address message and the minimum number of digits required for routing the call have been received.
If the end of the address signalling is determined in accordance with criteria a) b) or c), the timer Ti/w2 is started when INVITE is sent. Also, if the PSTN XML body is supported as a network option, the O-MGCF shall insert the PSTN XML sendingCompleteIndication.
7.2.3.2.1.5
Fallback (optional)

The Fallback mechanism described in the present clause shall only apply if the O-MGCF supports the PSTN XML body as a network option, and propagates UDI-TA fallback signalling into the IMS as a network option.

NOTE:
If the Fallback related signalling is not forwarded according to the procedures in the present sublause by the O-MGCF, the O-MGCF will apply the ISUP Fallback procedures when the IAM includes a TMR and a TMR prime parameter and a USI and USI Prime parameter.
When the IAM includes a TMR and a TMR prime parameter and a USI and USI Prime parameter then the O-MGCF shall:

-
map the "USI Prime" into the "InformationTransferCapability" of the second BearerCapability element in the PSTN XML body;

-
include SDP with one m-line of "audio" media type in the INVITE;
-
map the "TMR" and "USI Prime" into a first offerered format in the SDP m-line according to table 10b;
-
map the "USI" into the first Bearer Capability element in the PSTN XML body;

-
map the "TMR prime" into the "InformationTransferCapability" of the first BearerCapability element in the PSTN XML body; and
-
map the TMR prime and USI into a second offerered format in the SDP m-line according to table 10b.

7.2.3.2.1a
Sending of INVITE without determining the end of address signalling
7.2.3.2.1a.1
General

As a network option, the O-MGCF is not required to determine the end of address signalling. In this case the O-MGCF shall send an initial INVITE when a preconfigured number of digits has been reached.

When the MGCF receives ISUP SAM messages the additional digits received in the SAMs may either be sent using the in-dialog overlap method as specified in clause 7.2.3.2.1a.2 or using the multiple INVITEs overlap method as specified in 7.2.3.2.1a.3. It depends on the network configuration which of these methods is applied. However, within one IMS only a single method shall be used.

7.2.3.2.1a.2
Additional digits sent with in-dialog overlap method


[image: image26.wmf] 

IAM

 

SAM

 

INVITE 

 

O

-

MGCF

 

INFO

 

18x

 

200 (INFO)

 

SAM

 

INVITE

 

404/484

 


Figure 14: Overlap signalling using in-dialog INFOs in CS an IMS network
If the O-MGCF sends an initial SIP INVITE request before the end of address signalling is determined, the O-MGCF shall:

-
use the SIP precondition extension within the SIP INVITE request;

-
start timer Ti/w2;

-
be prepared to process SAM as described below;

-
be prepared to handle incoming SIP 18x provisional responses, establishing early dialogs; and

-
be prepared to handle incoming SIP 404 or 484 error responses as detailed in clause 7.2.3.2.12.1.

NOTE:
A SIP INVITE request with incomplete address information can be rejected with a SIP 404 or 484 error response.

On receipt of a SAM from the BICC/ISUP side, unless the O-MGCF has received a SIP 180 (Ringing) response for the call, the following O-MGCF procedures apply:

-
The O-MGCF shall stop timer Ti/w3 (if it is running).
-
If no response has been received for the previous INVITE request of the same call, the O-MGCF shall wait for the response and then apply the procedures in the next bullets to transfer the digits received in the SAM. If additional SAMs are received while the O-MGCF is waiting for the response for the previous SIP INVITE request, the digits within shall be combined with the digits of the previous SAMs.

-
If an early dialog has not been established, and a SIP 404 or 484 error response has been received for the last previous SIP INVITE request for the same call, the O-MGCF shall send a SIP INVITE request complying to the following:

-
The SIP INVITE request shall use the SIP preconditions extension.

-
The SIP INVITE request shall include all digits received so far for this call in the Request‑URI.

-
The SIP INVITE request shall include the same Call‑ID and From tag as the previous SIP INVITE request for the call.

-
If an early dialog has been established, and a response has been received for any previously sent SIP INFO request ("legacy" mode of usage of the INFO method as defined in IETF RFC 6086 [133]), the O-MGCF shall send an in-dialog SIP INFO request complying the following:

-
The SIP INFO request shall only include the digits received since the previous SIP request with digits was sent (see Note).

-
If no response has been received for the previous SIP INFO request, the O-MGCF shall wait for the response and then apply the procedures in the previous bullet to transfer digits received in the SAM. If additional SAMs are received while the O-MGCF is waiting for the response for the previous SIP INFO request, the digits within shall be combined with the digits of the previous SAMs.

-
Restart Ti/w2.
If timer Ti/w2 has expired, or the O-MGCF has received a SIP 180 (Ringing) response for the call, the O‑MGCF shall ignore subsequent SAMs received.

NOTE:
The encoding of the digits within the SIP INFO request is described in clause 7.2.3.2.20.2.

Editor's note:
It needs to be verified whether the timer procedures associated with the in-dialog method are correct.
7.2.3.2.1a.3
Additional digits sent using the multiple INVITEs overlap method


[image: image27.wmf] 

IAM

 

SAM

 

INVITE 

 

O

-

MGCF

 

SAM

 

INVITE 

 

INVITE 

 

4

0

4

/484

 

4

0

4

/484

 


Figure 14a: Overlap signalling using multiple INVITEs in CS an IMS network
If the O-MGCF sends an SIP INVITE request before the end of address signalling is determined, the O-MGCF shall:

-
use the SIP precondition extension within the SIP INVITE request;

-
start timer Ti/w2; and

-
be prepared to process SAM as described below; and
-
be prepared to handle incoming SIP 404 or 484 error responses as detailed in clause 7.2.3.2.12.1.

NOTE:
An SIP INVITE request with incomplete address information will be rejected with a SIP 404 or 484 error response.

On receipt of a SAM from the BICC/ISUP side, unless the O-MGCF has received a SIP 180 (Ringing) response for the call, the O‑MGCF shall:

-
stop timer Ti/w3 (if it is running);

-
send an INVITE request complying to the following:

-
The SIP INVITE request shall use the SIP preconditions extension.

-
The SIP INVITE request shall include all digits received so far for this call in the Request‑URI.
-
The SIP INVITE request shall include the same Call‑ID and From tag as the previous SIP INVITE request for the call.

-
restart Ti/w2;

If timer Ti/w2 has expired, or the O-MGCF has received a SIP 180 (Ringing) response for the call, the O‑MGCF shall ignore subsequent SAMs received.

7.2.3.2.2
Coding of the INVITE

7.2.3.2.2.0
Overview

Table 10aa provides a summary of how the header fields within the outgoing INVITE message are populated.

Table 10aa: Interworked contents of the INVITE message

	IAM(
	INVITE(

	Called Party Number 
	Request-URI

To

	Calling Party Number
	P-Asserted-Identity 

	
	Privacy

	
	From

	Generic Number ("additional calling party number")
	From

	
	Privacy

	Hop Counter
	Max-Forwards

	TMR/USI 
	Message Body (application/SDP)

	Location Number
	P-Access-Network-Info


7.2.3.2.2.1
REQUEST URI Header

The called party number parameter of the IAM message is used to derive Request URI of the INVITE Request. The Request URI is a tel URI or SIP URI with "user=phone" and shall contain:

-
an E.164 International public telecommunication number prefixed by a "+" sign (e.g. tel:+4911231234567) , or

-
a non E.164 number (national operator option for service numbers), expressed as a local number as per IETF RFC 3966 [97].

Table 10a: Mapping ISUP Called Party Number to
SIP Request-URI and To header field
	IAM
	INVITE

	BICC/ISUP Parameter / field
	Value
	SIP component
	Value

	Called Party Number (NOTE 3) 
	
	Request-URI and To header field
	display-name (optional) and addr-spec derived from Called Party Number parameter address signals 

	Nature of Address Indicator (NOTE 2) (NOTE 6)
	"national (significant) number"
	Tel URI or SIP URI


	Insert "+CC" before the Address signals (NOTE 1)

	
	"international number"
	
	Insert "+" before the Address signals

	
	"Network-specific number" or
"reserved for national use"
	
	according to local policies should either be:

- a global number (+CC), if the called party number may be converted into an E.164 address

OR, depending on operator's requirements may be converted into

- local number (with a phone-context parameter) (NOTE 4) (NOTE 5)

	NOTE 1:
CC = Country Code of the network in which the O-MGCF is located.

NOTE 2:
The usage of "Nature of address indicator" value "unknown" is allowed but the mapping is not specified in the present specification.

NOTE 3:
If the address signals received in the ISUP Called Party Number contain a sending terminated signal (hexadecimal digit F), then this shall be discarded or if the O-MGCF supports the PSTN XML body as a network option then the PSTN XML sendingCompleteIndication shall be set.
NOTE 4:
Mapping between nature of address indicator values and phone-context values is provisioned in the MGCF. Setting of value of phone-context is depending on local operator's policies.

NOTE 5:
Network-specific number or reserved for national use shall be translated into E.164 format numbers except if local operator's policy requires keeping in local format (e.g. for national reasons E.164 numbers cannot be used for such purpose). In the later case the mapping shall be done as indicated in the table.

NOTE 6:
The values "Network routing number in national (significant) number format", "Network routing number in network specific number format" or "Network routing number concatenated with called directory number" are used when number portability is supported. For the mapping see section 7.2.3.2.2A. 


7.2.3.2.2.2
SDP Media Description

If the O-MGCF indicates support of the SIP preconditions in the initial INVITE request and local preconditions have not been met, the SDP media description shall contain precondition information as per TS 24.229 [9]. Depending on the coding of the continuity indicators different precondition information (IETF RFC 3312 [37]) is included. If the continuity indicator indicates "continuity performed on a previous circuit" or "continuity required on this circuit", and the INVITE is sent before receiving a COT, message (which is not recommended according to clause 7.2.3.2.1.1), then the O-MGCF shall indicate that the preconditions are not met. Otherwise the O-MGCF shall indicate whether the preconditions are met, dependent on thestatus of the local resource reservation. If the local preconditions are not met the O-MGCF should set the media stream to inactive mode (by including an attribute "a=inactive"). If the local configuration indicates that O-MGCF is deployed in the IMS network that serves users supporting SIP precondition mechanism, the attribute "a=inactive" may be omitted when the initial SDP offer indicates local preconditions are not met. If the initial SDP offer indicates local preconditions are fulfilled, the O-MGCF shall not set the media stream to inactive mode.
If the O-MGCF determines that a speech call is incoming, the O-MGCF shall include the AMR codec transported according to IETF RFC 4867 [23] with the options listed in clause 5.1.1 of TS 26.236 [32] in the SDP offer, unless the Note below applies. Within the SDP offer, the O-MGCF should also provide SDP RR and RS bandwidth modifiers specified in IETF RFC 3556 [59] to disable RTCP, as detailed in clause 7.4 of TS 26.236 [32]. The O-MGCF may include other codecs according to operator policy.

NOTE:
If the O-MGCF is deployed in an IMS network that by local configuration serves no user equipment that implements the AMR codec, then the AMR codec may be excluded from the SDP offer.

To avoid transcoding or to support non-speech services, the O-MGCF may add media derived from the incoming ISUP information according to table 10b. The support of the media listed in table 10b is optional. If the O-MGCF supports the PSTN XML body as a network option and adds media derived from the incoming ISUP information according to table 10b, the O-MGCF shall also map the media related ISUP information into the XML body as shown in table 7.2.3.2.2.7.1.
Table 10b: Coding of SDP media description lines from TMR/USI: ISUP to SIP

	TMR parameter 
	USI parameter (Optional)
	HLC IE in ATP (Optional)
	m= line
	b= line
	a= line

	TMR codes
	Information Transfer-Capability
	User Information Layer 1 Protocol Indicator
	High Layer Characteristics Identification
	<media>
	<transport>
	<fmt-list>
	<modifier>:
<bandwidth-value>
	rtpmap:<dynamic-PT> <encoding name>/<clock rate>[/encoding parameters>

	"speech"
	"Speech"
	"G.711 μ-law"
	Ignore
	audio
	RTP/AVP
	0 (and possibly 8) (NOTE 1)
	AS: (64 + RTP/UDP/IP overhead)
	rtpmap:0 PCMU/8000

(and possibly rtpmap:8 PCMA/8000)

(NOTE 1)

	"speech"
	"Speech"
	"G.711 μ-law"
	Ignore
	audio
	RTP/AVP
	Dynamic PT (and possibly a second Dynamic PT)

(NOTE 1)
	AS: (64 + RTP/UDP/IP overhead)
	rtpmap:<dynamic-PT> PCMU/8000

(and possibly rtpmap:<dynamic-PT> PCMA/8000)

(NOTE 1)

	"speech"
	"Speech"
	"G.711 A-law"
	Ignore
	audio
	RTP/AVP
	8
	AS: (64 + RTP/UDP/IP overhead)
	rtpmap:8 PCMA/8000

	"speech"
	"Speech"
	"G.711 A-law"
	Ignore
	audio
	RTP/AVP
	Dynamic PT
	AS:(64 + RTP/UDP/IP overhead)
	rtpmap:<dynamic-PT> PCMA/8000

	"3.1 KHz audio"
	USI Absent
	
	Ignore
	audio
	RTP/AVP
	8
	AS:(64 + RTP/UDP/IP overhead)
	rtpmap:8 PCMA/8000

	"3.1 KHz audio"
	"3.1 KHz audio"
	"G.711 μ-law"
	(NOTE 3)
	audio
	RTP/AVP
	0 (and possibly 8)

(NOTE 1)
	AS:(64 + RTP/UDP/IP overhead)
	rtpmap:0 PCMU/8000

(and possibly rtpmap:8 PCMA/8000)

(NOTE 1)

	"3.1 KHz audio"
	"3.1 KHz audio"
	"G.711 A-law"
	(NOTE 3)
	audio
	RTP/AVP
	8
	AS:(64 + RTP/UDP/IP overhead)
	rtpmap:8 PCMA/8000

	"3.1 KHz audio"
	"3.1 KHz audio"
	
	"Facsimile Group 2/3"
	image
	Udptl [73]
	t38[73]
	AS:(64 + UDP/IP overhead)
	Based on ITU-T T.38 [72].

	"3.1 KHz audio"
	"3.1 KHz audio"
	
	"Facsimile Group 2/3"
	image
	Tcp
(NOTE 7)
	t38[73]
	AS:(64 + TCP/IP overhead)
	Based on ITU-T T.38 [72].

	"64 kbit/s preferred"
	"Speech/ 3.1KHz audio" (NOTE 6)
	N/A
	Ignore
	audio
	RTP/AVP
	Dynamic PT
	AS:(64 + RTP/UDP/IP overhead)
	rtpmap:<dynamic-PT> CLEARMODE/8000

(NOTE 2)(NOTE 4)

	"64 kbit/s unrestricted"
	"Unrestricted digital information"
	N/A 
	Ignore
	audio
	RTP/AVP
	Dynamic PT
	AS:(64 + RTP/UDP/IP overhead)
	rtpmap:<dynamic-PT> CLEARMODE/8000

(NOTE 2)(NOTE 5)

	NOTE 1:
Both PCMA and PCMU could be required.
NOTE 2:
CLEARMODE is specified in IETF RFC 4040 [69].

NOTE 3:
HLC is normally absent in this case. It is possible for HLC to be present with the value "Telephony", although 6.3.1/ITU-T Q.939 indicates that this would normally be accompanied by a value of "Speech" for the Information Transfer Capability element.

NOTE 4:
After the CLEARMODE codec, additional speech codecs such as AMR and/or G.722 and/or G.711 available via transcoding or reframing should be offered in the same m-line.

NOTE 5:
As alternative or in addition to the m-line containing the CLEARMODE codec, an MGCF supporting the multimedia interworking detailed in Annex E may add an m-line for speech codecs and an m-line for video codecs as detailed in this Annex.
NOTE 6:
In this case, the USI prime parameter will also be present and will indicate "Unrestricted Digital Information with tones/announcements".
NOTE 7:
This value is not recommended in a network supporting MTSI clients as it is not supported by an MTSI client (see TS 24.173 [88]).


7.2.3.2.2.3
P-Asserted-Identity, From and Privacy header fields

Table 12: Mapping BICC/ISUP CLI parameters to SIP header fields
	Has a Calling Party Number parameter with complete E.164 number, and with Screening Indicator = UPVP or NP (NOTE 1) been received?
	Calling Party Number APRI
	Has a Generic Number (ACgPN) with a complete E.164 number and with Screening Indicator = UPNV been received?
	Generic Number APRI
	P-Asserted-Identity header field
	From header field
	Privacy header field

	N
	-
	N
	-
	Header field not included
	SIP URI with addr-spec of Unavailable User Identity (NOTE 2) (NOTE 6)
	Header field not included

	N
	-
	Y (NOTE 3)
	"presentation allowed"
	Header field not included
	addr-spec derived from Generic Number (ACgPN) address signals

or network provided value (NOTE 6)
	Header field not included

	N
	-
	Y (NOTE 3)
	"presentation restricted"
	Header field not included
	SIP URI with addr-spec of Unavailable User Identity (NOTE 2) (NOTE 6)
	Header field not included

	
	
	
	
	
	Derived from the Generic Number parameter address signals (see table 13) (NOTE 8)
	priv-value =: "user"
(See table 16).

	Y
	"presentation allowed"
	N
	-
	Derived from Calling Party Number parameter address signals

(See table 14)
	Tel URI or SIP URI derived from Calling Party Number parameter address signals (See table 15) 
	Privacy header is not included or if included, "id" is not included (See table 16)

	Y
	"presentation allowed"
	Y
	"presentation allowed"
	Derived from Calling Party Number parameter address signals

(See table 14)
	Derived from Generic Number (ACgPN) address signals

(See table 13)

(NOTE 6)
	Privacy header is not included or if included, "id" and "user" are not included

(See table 16 )

	Y
	"presentation allowed"
	Y
	"presentation restricted"
	Derived from Calling Party Number parameter address signals

(See table 14)
	Tel URI or SIP URI derived from Calling Party Number parameter address signals (See table 15)

(NOTE 9)
	Privacy header is not included or if included, "id" is not included

(See table 16 )

	
	
	
	
	
	SIP URI with addr-spec of Anonymuous URI (NOTE 6) (NOTE 7) (NOTE 10)
	Privacy header is not included or if included, "id" is not included

(See table 16).

	
	
	
	
	
	Derived from the Generic Number parameter address signals (see table 13) (NOTE 8)
	priv-value =: "user".

"id" is not included

(See table 16).

	Y
	"presentation restricted"
	N
	-
	Derived from Calling Party Number parameter address signals

(See table 14)
	SIP URI with addr-spec of Anonymous URI (NOTE 7) (NOTE 6)
	priv-value =: "id". (See table 16)

	Y
	"presentation restricted"
	Y
	"presentation allowed"
	Derived from Calling Party Number parameter address signals

(See table 14)
	Derived from Generic Number (ACgPN) address signals

(See table 13)

(NOTE 6)
	priv-value =: "id".

	Y
	"presentation restricted"
	Y
	"presentation restricted"
	Derived from Calling Party Number parameter address signals

(See table 14)
	SIP URI with addr-spec of Anonymous URI (NOTE 7) (NOTE 6) (NOTE 10)
	priv-value =: "id".

	
	
	
	
	
	Derived from the Generic Number parameter address signals (see table 13) (NOTE 8)
	priv-value =: "id", "user".

(See table 16).

	Y
	"presentation restricted by network"
(NOTE 4)
	N
	-
	Header field not included.
	addr-spec is set to "unavailable@hostportion" (NOTE 5)
	Privacy header is not included or if included, "id" is not included (See table 16)

	Y
	"presentation restricted by network"
	Y
	"presentation allowed"
	Header field not included.
	Derived from Generic Number (ACgPN) address signals

(See table 13)

(NOTE 6)
	Privacy header is not included or if included, "id" and "user" are not included

(See table 16 )

	Y
	"presentation restricted by network"
	Y
	"presentation restricted"
	Header field not included.
	addr-spec is set to "unavailable@hostportion" (NOTE 5)
	Privacy header is not included or if included, "id" is not included

(See table 16 )

	
	
	
	
	
	Derived from the Generic Number parameter address signals (see table 13) (NOTE 8)
	priv-value =: "user".

"id" is not included

(See table 16).



	NOTE 1:
A Network Provided CLI in the CgPN parameter may occur on a call to IMS. Therefore in order to allow the "display" of this Network Provided CLI at a SIP UAS it shall be mapped into the SIP From header. It is also considered suitable to map into the P-Asserted-Identity header since in this context it is a fully authenticated CLI related exclusively to the calling line, and therefore as valid as a User Provided Verified and Passed CLI for this purpose.

NOTE 2:
The "From" header may contain an "Unavailable User Identity". An "Unavailable User Identity" includes information that does not point to the calling party and indicates that the caller's identity is unknown. The encoding of the "Unavailable User Identity" shall be as defined in TS 23.003 [74].

NOTE 3:
This combination of CgPN and ACgPN is an error case but is shown here to ensure consistent mapping across different implementations.

NOTE 4:
This is an ETSI specific value described within ETSI EN 300 356-1 [70].

NOTE 5:
The setting of the hostportion is according to operator policy.

NOTE 6:
In accordance with IETF RFC 3261 [19] procedures, a tag shall be added to the "From" header.

NOTE 7:
The "From" header may contain an "Anonymous User Identity". An "Anonymous User Identity" includes information that does not point to the calling party and indicates that the caller has withheld their identity. The encoding of the "Anonymous User Identity" shall be as defined in TS 23.003 [74].

NOTE 8:
As a network option, the "From" header may be derived from the Generic Number parameter address signals (see table 13). This option is only recommended to use within a trusted domain where an entity such as a TAS is configured to be inserted into the call path that is able to change the "From" Header content to an anonymous user identity (NOTE 7).

NOTE 9:
As a network option, the "From" header may be derived from the Calling Party Number parameter address signals (see table 15). This option is only recommended to use within a trusted domain where an entity such as a TAS is configured to be inserted into the call path that is able to change the "From" Header content to an anonymous user identity (NOTE 7).
NOTE 10: Network option.


Table 13: Mapping of generic number (additional calling party number) to SIP from header fields

	BICC/ISUP parameter / field
	Value
	SIP component
	Value

	Generic Number

Number Qualifier Indicator
	"additional calling party number"
	From header field
	display-name (optional) and addr-spec

	Nature of Address Indicator
	"national (significant) number"

	Tel URI or SIP URI
	Add CC (of the country where the MGCF is located) to GN address signals to construct E.164 number in URI. Prefix number with "+".

	
	"international number"

	
	Map complete GN address signals to E.164 number in URI. Prefix number with "+".

	Address signal
	if NOA is "national (significant) number" then the format of the address signals is:

NDC+ SN

If NOA is "international number"
 then the format of the address signals is:

CC + NDC + SN
	
	

	
	
	Tel URI or SIP URI 
	CC+NDC+SN as E.164 number in URI. Prefix number with "+".


Table 14: Mapping of calling party number parameter to SIP P-Asserted-Identity header fields

	BICC/ISUP Parameter / field
	Value
	SIP component
	Value

	Calling Party Number
	
	P-Asserted-Identity header field
	

	Nature of Address Indicator
	"national (significant) number"

	Tel URI or SIP URI
	Add CC (of the country where the MGCF is located) to CgPN address signals to construct E.164 number in URI. Prefix number with "+". 

	
	"international number"
 
	
	Map complete CgPN address signals to E.164 number in URI. Prefix number with "+".

	Address signal
	If NOA is "national (significant) number" then the format of the address signals is:

NDC + SN

If NOA is "international number"
then the format of the address signals is:

CC + NDC + SN
	
	


Table 15: Mapping of BICC/ISUP Calling Party Number parameter to SIP From header fields

	BICC/ISUP parameter / field
	Value
	SIP component
	Value

	Calling Party Number
	
	From header field
	

	Nature of Address Indicator
	"national (significant) number"
	Tel URI or SIP URI

(NOTE)
	Add CC (of the country where the MGCF is located) to CgPN address signals then map to construct E.164 number in URI. Prefix number with "+".

	
	"international number"

	
	Map complete CgPN address signals to construct E.164 number in URI. Prefix number with "+".

	Address signal
	If NOA is "national (significant) number" then the format of the address signals is:

NDC + SN

If NOA is "international number"
 then the format of the address signals is:

CC + NDC + SN
	Tel URI or SIP URI
(NOTE) 
	CC+NDC+SN as E.164 number in URI. Prefix number with "+". 

	NOTE:
A tel URI or a SIP URI with "user=phone" is used according to operator policy. 


Table 16: Mapping of BICC/ISUP APRIs into SIP privacy header fields

	BICC/ISUP parameter / field
	Value
	SIP component
	Value

	Calling Party Number


	
	Privacy header field
	priv-value

	APRI
	"presentation restricted"
	Priv-value
	"id"
("id" included only if the P-Asserted-Identity header is included in the SIP INVITE)

	
	"presentation allowed" or "presentation restricted by network"
 
	Priv-value

 
	omit Privacy header

or Privacy header without "id" if other privacy service is needed

	Generic Number (ACgPN)
	
	Privacy header field
	priv-value

	APRI (NOTE)
	"presentation restricted"
	Priv-value
	"user"

	
	"presentation allowed"
	Priv-value
	omit Privacy header

or Privacy header without "user" if other privacy service is needed

	NOTE:
Mapping of Generic Number APRI is only applicable, if the Generic Number is mapped to the "From" header field (see table 12).


7.2.3.2.2.3A
"cpc" URI Parameter in P-Asserted-Identity Header

See Annex C for normative interworking of a Calling party's category to a "cpc" URI parameter within P-Asserted-Identity header field.

7.2.3.2.2.3B
"oli" URI Parameter in P-Asserted-Identity Header

See Annex H for normative interworking of the "oli" URI parameter as a network option.

7.2.3.2.2.4
Max Forwards header

If the Hop Counter procedure is supported in the CS network, the O‑MGCF shall use the Hop Counter parameter to derive the Max-Forwards SIP header. Due to the different default values (that are based on network demands/provisions) of the SIP Max-Forwards header and the Hop Counter, an adaptation mechanism shall be used to adopt the Hop Counter to the Max Forwards at the O‑MGCF. For example, the following guidelines could be applied.

a)
Max-Forwards for a given message should be monotone decreasing with each successive visit to a SIP entity, regardless of intervening interworking, and similarly for Hop Counter.

b)
The initial and successively mapped values of Max-Forwards should be large enough to accommodate the maximum number of hops that may be expected of a validly routed call.

The table 17 shows the principle of the mapping:

Table 17: Hop counter-Max forwards

	Hop Counter
	= X
	Max-Forwards
	= Y = Integer part of (X * Factor)

	NOTE:
The Mapping of value X to Y should be done with the used (implemented) adaptation mechanism.


The factor used to map from Hop Counter to Max‑Forwards for a given call will depend on call origin, and will be provisioned at the O‑MGCF based on network topology, trust domain rules, and bilateral agreement.

The Principle of adaptation could be implemented on a basis of the network provision, trust domain rules and bilateral agreement.

7.2.3.2.2.5
IMS Communication Service Identifier

For speech and video calls, the O-MGCF shall insert an IMS Communication Service Identifier, indicating the IMS Multimedia Telephony Communication Service.
The IMS Communication Service Identifier for the IMS Multimedia Telephony Communication Service is defined in TS 24.173 [88].

7.2.3.2.2.6
P-Early-Media header field
For a speech call the O-MGCF may include the P-Early-Media header field with the "supported" parameter as described in IETF RFC 5009 [89] in each outgoing INVITE request.
NOTE:
The P-Early-Media header with the "supported" parameter can be ignored by terminating devices when not supporting the procedures described in IETF RFC 5009 [89].
7.2.3.2.2.7
PSTN XML elements

If the O-MGCF supports the PSTN XML body as a network option, the O-MGCF shall map ISUP information into the XML body as shown in table 7.2.3.2.2.7.1 and 7.2.3.2.2.8.1.

Table 7.2.3.2.2.7.1: Mapping of ISUP Parameters with PSTN XML elements

	IAM (
	INVITE ( 

	ISUP Parameter
	Content
	PSTN XML 

	Access Transport Parameter

	High layer compatibility
	HighLayerCompatibility (NOTE 1, NOTE 2, NOTE 3)

	
	Low layer compatibility
	LowLayerCompatibility (NOTE 3)

	User Service Information
	
	Bearer Capability (NOTE 3, NOTE 4)

	User Teleservice Information
	High layer compatibility
	HighLayerCompatibility (NOTE 2, NOTE 3)

	Called Party Number 
	Sending terminated signal (hexadecimal digit F) 
	sendingCompleteIndication

	NOTE 1:
If two high layer compatibility information elements are received in the ATP of the IAM, they shall be transferred in the same order as received into the PSTN XML body within the INVITE.

NOTE 2:
In the normal case, the High layer compatibility information in the ATP is equal to the High layer compatibility in the User Teleservice Information parameter. In the PSTN XML body, no two identical High layer compatibility information shall be present. If an HLC is available both in the ATP and in the User Teleservice information, the HLC from the ATP should be mapped.
NOTE 3:
The O-MGCF shall only map this information element if the O-MGCF offers media formats which can be transferred by the IM-MGW without transcoding and are derived from the incoming ISUP information according to table 10b.
NOTE 4:
See clause 7.2.3.2.1.5. 


7.2.3.2.2.8
Progress indicator

If the O-MGCF supports the PSTN XML body as a network option, the Forward call indicators parameter and an available "Progress indicator" element in the IAM shall be mapped into a ProgressIndicator in the PSTN XML body of the INVITE as shown in table 7.2.3.2.2.8.
Table 7.2.3.2.2.8: Coding of PSTN XML ProgressIndicator
	IAM(
	INVITE (

	Forward call indicators parameter
	Access transport parameter
	 PSTN XML body with ProgressIndicator with "Progress Description" value No. (Value of PI)

	ISDN User Part indicator
	ISDN access indicator
	
	

	0
("ISDN User Part
not used all the way")
	
Value non-significant
	
Value non-significant
	
No. 1 (NOTE 1)

	1
("ISDN User Part
used all the way")
	0
("originating access non - ISDN")
	Value non-significant
	No. 3 (NOTE 1)

	1
("ISDN User Part
used all the way")
	1
("originating access ISDN")
	Progress indicator No. (Value of PI)
	ProgressIndicator mapped from Progress indicator received in the ATP (NOTE 2) and additional ProgressIndicator with "Progress Description" value No.6 (NOTE 1, NOTE 3)

	1
("ISDN User Part
used all the way")
	1
("originating access ISDN")
	Not present
	No. 6 (NOTE 1)

	NOTE 1:
The ProgressIndicator "Coding Standard" parameter shall be set to "00 (ITU-T standardized coding)". The default value for the ProgressIndicator "Location" parameter is "0011 (Transit Network)".

NOTE 2:
The entire Progress indicator, including the "Progress Description", "Coding Standard" and "Location" parameters shall be copied.
NOTE 3:
The order of ProgressIndicators within PSTN XML body is irrelevant.


7.2.3.2.2.9
P-Access-Network-Info

If the IAM message includes a location number ISDN user part parameter, the O-MGCF shall include a P-Access-Network-Info header. The P-Access-Network-Info shall be populated as shown in table 7.2.3.2.2.9.1.

Table 7.2.3.2.2.9.1: Coding of the P-Access-Network-Info header fields

	BICC/ISUP parameter / field
	SIP component
	Value

	Location Number
	access-type
	"GSTN"

	
	gstn-location
	value of Location Number, in quotes


Table 7.2.3.2.2.9.2: Mapping ISUP Location Number to SIP P-Access-Network-Info

	IAM
	INVITE

	Location Number
	P-Access-Network-Info

	Parameter name
	not mapped

	Parameter length 
	not mapped 

	Parameter content 
	gstn-location set to the hexadecimal representation of the ISUP parameter content, encoded as a text string between quotes

	NOTE 1:
As specified in ITU-T Q.763 [4], the parameter content includes both the header fields (octets 1 and 2) and the address signals.

NOTE 2:
The parameter content includes the address presentation restricted indicator. This field is not mapped to the Privacy header field. It is upon network operator responsibility to remove the P-Access-Network-Info header field when leaving the trust domain, as specified in TS 24.229 [9].
NOTE 3:
If the screening indicator is set to network provided, a np parameter is added to the P-Access-Network-Info header field value.


7.2.3.2.2A
Coding of the INVITE when Number Portability is supported

This clause describes optional coding procedures when Number Portability is supported.

7.2.3.2.2A.1
REQUEST URI and To Header

When Number Portability is supported, the method used for signalling of the Called Party E.164 address and the Number Portability Routing Number determines the parameters of the IAM message used to derive the Request URI of the INVITE Request.
The number portability information (rn and npdi) shall not be mapped into the To header field.
ITU-T Q.769.1 [92] describes three possible addressing methods for signalling of the Called Party E.164 address and Number Portability Routing Number (ITU-T Q.769.1 [92] uses the terms directory number and network routing number respectively). The choice of these methods is based on network operator and national requirements.

The following clauses describe how the Request URI and To header fields are populated, based on these methods, when a Number Portability Routing Number is available in the IAM.

When the optional Number Portability Routing Number is available and supported, these procedures take precedence over procedures for coding of the Request URI and To header fields described in clause 7.2.3.2.2.1.

When a Number Portability Routing Number is not available, the Request URI and To Header fields are populated as described in clause 7.2.3.2.2.1, with the following addition: If a Number Portability Forward Information Parameter parameter is present in the IAM, containing a value of "number portability query done for called number, non-ported called subscriber", a tel URI npdi parameter [93] is added.
For the following clauses, the Request URI is a tel URI or SIP URI with "user=phone" and shall contain an International public telecommunication number prefixed by a "+" sign (e.g. tel:+4911231234567).

7.2.3.2.2A.1.1
Separate Directory Number Addressing Method

Table 17a: Mapping ISUP to SIP Request-URI and To header field with Number Portability Separate Directory Number Addressing Method
	IAM(
	INVITE

	Called Party Number
	Called Directory Number
	Request-URI and To Header Field

	Address Signal:

Nature of address indicator:

"Network routing number in national (significant) number format" or "National (significant) number" or "Network routing number in network specific number format" as described in ITU-T Q.769.1 [92]
(NOTE 2)


	Address Signal:

Nature of address indicator:

"National (significant) number".
	The "telephone-subscriber" is populated from the Called Directory Number as follows:

Insert "+CC" before the Address signals (NOTE 1)

The Tel URI rn= parameter is populated from the Called Party Number as follows:

Insert "+CC" before the Address signals (NOTE 1)

and is added only to the Request-URI.
Use of the local form of the rn= parameter is out of the scope of the present specification.

Tel URI npdi parameter as defined in IETF RFC 4694 [93] is added only to the Request-URI.

	NOTE 1:
CC = Country Code of the network in which the O-MGCF is located.

NOTE 2:
If the address signals received in the ISUP Called Party Number contain a sending terminated signal (hexadecimal digit F), then this shall be discarded or if the PSTN XML is supported then the sendingCompleteIndication shall be included.


7.2.3.2.2A.1.2
Concatenated Addressing Method

Table 17b: Mapping ISUP to SIP Request-URI and To header field with Number Portability Concatenated Number Addressing Method
	IAM(
	INVITE

	Called Party Number
	Request-URI and To Header Field

	Address Signal:

Nature of address indicator:

"Network routing number concatenated with called directory number" or

"National (significant) number" as described in ITU-T Q.769.1 [92]
(NOTE 2)
	The "telephone-subscriber" is populated from the Called Party Number as follows:

Remove the prefix representing the Number Portability Routing Number

or the prefix prior to the directory number (NOTE3).

Insert "+CC" before the Address signals (NOTE 1).

The Tel URI rn= parameter is populated from the Called Party Number as follows and is added only to the Request-URI:

Use all address digits contained within the Called Party Number or

remove the digits that follow the prefix representing the Number Portability Routing Number

Insert "+CC" before the Address signals (NOTE 1)

Use of the local form of the rn= parameter is out of the scope of the present specification.

Tel URI npdi parameter as defined in IETF RFC 4694 [93] is added only to the Request-URI.

	NOTE 1:
CC = Country Code of the network in which the O-MGCF is located.

NOTE 2:
If the address signals received in the ISUP Called Party Number contain a sending terminated signal (hexadecimal digit F), then this shall be discarded or if the PSTN XML is supported then the sendingCompleteIndication shall be included.
NOTE 3:
Based on national policy the whole Number Portability Routing number includes the Called Party Number and a prefix. In such cases only the Prefix has to be removed. Normally the Nature of address indicator indicates if the Number Portability Routing Number contains a Called Party Number and a prefix.


7.2.3.2.2A.1.3
Separate Network Routing Number Addressing Method

Table 17c: Mapping ISUP to SIP Request-URI and To header field with Number Portability Separate Network Routing Number Addressing Method
	IAM(
	INVITE

	Network Routing Number
	Called Party Number
	Request-URI and To Header Field

	Address Signal:

Nature of address indicator:

"Network routing number in national (significant) number format" or "Network routing number in network specific number format" as described in ITU-T Q.769.1 [92]
(NOTE 2)
	Address Signal:

Nature of address indicator:

"National (significant) number".
	The "telephone-subscriber" is populated from the Called Party Number as follows:

Insert "+CC" before the Address signals (NOTE 1)

The Tel URI rn= parameter is populated from the Network Routing Number as follows and is added only to the Request-URI:

Insert "+CC" before the Address signals (NOTE 1)

Use of the local form of the rn= parameter is out of the scope of the present specification.

Tel URI npdi parameter as defined in IETF RFC 4694 [93] is added only to the Request-URI.

	NOTE 1:
CC = Country Code of the network in which the O-MGCF is located.

NOTE 2:
If the address signals received in the ISUP Called Party Number contain a sending terminated signal (hexadecimal digit F), then this shall be discarded or if the PSTN XML is supported then the sendingCompleteIndication shall be included.


7.2.3.2.2B
Coding of the INVITE for Carrier Routeing

This clause describes optional coding procedures for carrier-based routeing.

7.2.3.2.2B.1
Mapping of "cic" in REQUEST URI Header

The procedures followed in clause 7.2.3.2.2.1 apply with the following addition.

If the Transit Network Selection parameter, defined according to ITU-T Q.761 [4], is included in the IAM message the O-MGCF, based on network configuration, may send the transit network selection information to the SIP network. In such a case the "cic=" parameter as defined in IETF RFC 4694 [93] is included in the SIP-Request URI and configured according to the table below.
Table 17d: Mapping of ISUP "Transit Network Selection" (TNS) to SIP "Carrier Identification Code" (CIC)

	ISUP parameter/field
	Value
	SIP Component
	Value

	Transit Network Selection
	Digits
	Carrier id code in Userinfo of Request URI


	"cic=carrier ID code" as defined in IETF RFC 4694 [93]


7.2.3.2.2B.2
Void
7.2.3.2.2C
Coding of INVITE with instance-id in form of IMEI URN

An Emergency Access Transfer Function (EATF) that provides IMS-based mechanisms for enabling service continuity of IMS emergency sessions is described in TS 23.237 [118]. A correlation of the call legs at the EATF is based on the equipment identifier.

A Mobile Equipment Identifier (MEI) parameter of the Mobile Service Transport (MST) Application Transport Parameter is defined in TS 29.205 [14].

An instance-id is a SIP Contact header field parameter defined in IETF RFC 5626 [119]. When an IMEI is available, the instance-id shall take the form of an International Mobile station Equipment Identity (IMEI) URN as specified in IETF RFC 7254 [120].

When an O-MGCF receives the Mobile Service Transport (MST) Application Transport Parameter containing the Mobile Equipment Identifier (MEI) parameter within the IAM the O-MGCF shall perform the mapping to the "+sip.instance" Contact header field parameter according to table 7.2.3.2.2C.1.

Table
7.2.3.2.2C.1:
Mapping of ISUP/BICC to SIP

	( IAM
	( INVITE

	ISUP Parameter
	Value
	SIP Component
	Value

	MST Application Transport Parameter
	Mobile Equipment Identifier: IMEI
	Contact header

containing "+sip.instance" parameter in the form of IMEI URN
	gmsa urn set to "imei" namespace

NOTE 1

NOTE 2



	
	Mobile Equipment Identifier: IMEISV
	
	

	NOTE 1:
The gsma-specifier "imei" is generated as:

            urn:gsma:imei:tac-snr-spare
            where 
     "tac" represents 8 digits type allocation code (TAC), "snr" represents 4 digits serial number (SNR), 
     and "spare" represents spare decimal digit as specified in TS 23.003 [74].
NOTE 2:
The Software Version Number is not interworked and thus the "svn" parameter is not included within the gsma urn.


7.2.3.2.3
Receipt of CONTINUITY
This clause only applies if the O-MGCF has sent the INVITE request without waiting for an outstanding COT message (see clause 7.2.3.2.1).

[image: image28.wmf] 

 SDP indicating  

pre

-

conditions  

met     

 

COT(success)

 

‚9

 

O

-

MGCF

 


Figure 14a: Receipt of COT (success)

When the requested local preconditions (if any) have been met and if possible outstanding continuity procedures have successfully been completed (COT with the Continuity Indicators parameter set to "continuity check successful" is received), a SDP offer (e.g. a SIP UPDATE request, as defined in IETF RFC 3311 [55]) shall be sent for each early SIP dialogue for which the received provisional response indicated support of preconditions confirming that all the required local preconditions have been met. If the O-MGCF previously offered inactive media stream it shall set the media stream to active mode.

NOTE:
This procedure applies regardless of whether the early SIP dialog existed prior to the preconditions being fulfilled or is subsequently created.
7.2.3.2.4
Sending of ACM and awaiting answer indication

If the Address Complete Message (ACM) has not yet been sent, the following cases are possible trigger conditions that shall lead to the sending the address complete message (ACM):
-
the detection of end of address signalling by the expiry of Timer T i/w1 or,


[image: image29.wmf] 

IAM

 

ACM (no indication)

 

O

-

MGCF

 

T i/w

1 

running

 

T i/w

1 

running

 

T i/w

1 

elapses

 

SAM

 

SAM

 

INVITE 

 


Figure 15: Sending of ACM T i/w1 elapses

-
the reception of the first 180 Ringing. An O-MGCF initiates the sending of an awaiting answer indication only if according to IETF RFC 5009 [89] backward early media is not authorized (the most recently received P-Early-Media header field is received does not authorize the backward early media or the P-Early-Media header field has not yet been received).

[image: image30.wmf] 

180 Ringing

 

ACM (Subscriber Free)

 

Ring tone

 

O

-

MGCF

 


Figure 16: Sending of ACM (Receipt of first 180 Ringing and backward early media is not authorized)

Based on local knowledge that the call is transited to a PSTN network, the O-MGCF may decide not to generate the awaiting answer indication when receiving the 180 Ringing message and backward early media is not authorized according to IETF RFC 5009 [89].

[image: image31.emf] 

O - MGCF  

ACM    (Subscriber Free)  

Ring tone  

180 Ringing     [P - Early - Media header:  backward early media  authorized]  


Figure 16a: Sending of ACM (Receipt of first 180 Ringing and backward early media is authorized)

-
the reception of the first 181 Call is Being Forwarded.

[image: image32.emf]   

O - MGCF  

ACM    (Call Diversion  information)  181 Call is Being  Forwarded   


Figure 16b: Sending of ACM (Receipt of first 181 Call is Being Forwarded and backward early media is not authorized)


[image: image33.emf] 

O - MGCF  

ACM     (Call Diversion  information, OBCI: in - band info available)  

Audio  

181 Call is Being  Forwarded     [P - Early - Media  header: backward early  media authorized]  


Figure 16c: Sending of ACM (Receipt of first 181 Call is Being Forwarded that includes authorization of early media)
-
At an O-MGCF once all the following sub-conditions have been met: {1} the reception of the first 183 Session Progress that includes a P-Early-Media header field authorizing backward early media, and {2} SDP preconditions are not used, or applicable SDP preconditions have been met.

[image: image34.emf] 

O - MGCF  

ACM   (No indication,  OBCI: in - band info  available)  

Appropriate    announcement  

183 Session Progress     [P - Early - Media header:  backward early media  authorized]  


Figure 16d: Sending of ACM (Receipt of first 183 Session Progress that includes authorization of early media)

-
As a network option, reception of 183 containing a SIP reason header with an Q.850 Cause Value.


[image: image35.wmf] 

18

3

 

Session Progress

 

(Reason(Cause=xxx))

 

ACM

 

(

 

No indication

 

Cause=xxx

)

 

O

-

MGCF

 


Figure 16e: Sending of ACM (Receipt of 183 Session Progress containing SIP Reason header)

-
Ti/w 2 expires after the initial INVITE is sent.


[image: image36.wmf] 

IAM

 

INVITE

 

T i/w 2

 

ACM (no indication)

 

O

-

MGCF

 


Figure 17: Sending of ACM (Ti/w2 elapses)

The sending of an awaiting answer indication is described in clause 9.2.3.3.
When a 180 (Ringing) response is received with the Alert-Info header field at an O-MGCF supporting capabilities associated with the Alert-Info header field an O-MGCF may instruct the IM-MGW to play out early media available at the associated URL to the PSTN leg of the communication.

If the O-MGCF receives a 18x response with a P-Early-Media header field that changes the authorization of early media, the O-MGCF terminates the sending of the awaiting answer indication if the header field authorizes backward early media, and initiates the sending of the awaiting answer indication if the header field removes authorization of backward early media and if the O-MGCF has received the 180 Ringing response.
7.2.3.2.5
Coding of the ACM

7.2.3.2.5.0
General

The description of the following ISDN user part parameters can be found in ITU-T Recommendation Q.763 [4].

7.2.3.2.5.1
Backward call indicators

bits
AB

Charge indicator Contributors

1 0


charge
bits
DC

Called party's status indicator

01


subscriber free if the 180 Ringing has been received.

00


no indication otherwise
bits
FE

Called party's category indicator

0 0


no indication
bits
HG
End-to-end method indicator

00


no end-to-end method available
bit

I

Interworking indicator

1


interworking encountered
As a network operator option, the value I = 0 "no interworking encountered" is used for TMR = 64 kBit/s unrestricted

NOTE:
This avoids sending of a progress indicator with Progress information 0 0 0 0 0 0 1 "Call is not end-to-end ISDN; further call progress information may be available in‑band", so the call will not be released for that reason by an ISDN terminal.

bit

J

End-to-end information indicator

0


no end-to-end information available
bit

K

ISDN user part/BICC indicator

0


ISDN user part not used all the way
As a network operator option, the value K = 1 "ISDN user part/BICC used all the way" is used for TMR = 64 kBit/s unrestricted

NOTE:
This avoids sending of a progress indicator with progress information 0 0 0 0 0 0 1 "Call is not end-to-end ISDN; further call progress information may be available in‑band", so the call will not be released for that reason by an ISDN terminal.

bit

L

Holding indicator (national use)

0


holding not requested
bit

M

ISDN access indicator

0


terminating access non-ISDN
As a network operator option, the value M = 1 "terminating access ISDN" is used for TMR = 64 kBit/s unrestricted.

NOTE:
This avoids sending of a progress indicator with progress information 0 0 0 0 0 1 0 "Destination access is non-ISDN", so the call will not be released for that reason by an ISDN terminal.

bit

N

Echo control device indicator

1


incoming echo control device included, for speech calls, e.g., TMR is "3.1KHz audio".

0


incoming echo control device not included, for known data calls, e.g., TMR "64 kBit/s unrestricted"

or
HLC "Facsimile Group 2/3".
If the PSTN XML body is supported as a network option, the Backward Call indicators parameters derived as shown in table 7.2.3.2.5.1.1 shall take precedence over the above Backward Call indicators parameter setting.

Table 7.2.3.2.5.1.1: Derivation of Backward Call Indicators from PSTN XML body
	(ACM
	( 180 Ringing or 183 Session Progress

	Backward call indicators parameter

Optional backward call indicators parameter
	PSTN XML body with Progress indicator with "Coding Standard" value "00 (ITU-T standardized coding)" and with "Progress Description" No (Value of PI)

	Backward call indicators parameter



ISDN User Part indicator

0
"ISDN User Part not used all the way"
	No. 1

("Call is not end-to-end ISDN: further call progress information may be available in-band")

	Backward call indicators parameter



ISDN User Part indicator

0
"ISDN User Part used all the way"


ISDN access indicator


0
"Terminating access non-ISDN"
	No. 2 
("Destination address is non-ISDN")

	Backward call indicators parameter



Interworking indicator

       0   "no interworking encountered"

ISDN User Part indicator



1
"ISDN User Part used all the way"



ISDN access indicator



1
"Terminating access ISDN"
	No. 7

("Terminating user ISDN")

	Optional backward call indicators parameter


In-band information indicator



1  "in-band information or an appropriate pattern is
   now available"
	No. 8

("In-band information or an appropriate pattern is now available")


7.2.3.2.5.2
Optional Backward call indicators

Bit A
1
"in-band information or an appropriate pattern is now available" shall be set if 183 Session Progress or 
      181 Call is Being Forwarded response is received and according to IETF RFC 5009 [89] backward early 
      media is authorized.
Table 7a0.4: Sending criteria of Optional backward call indicators parameter

	(ACM
	( 183 Session Progress or 181 Call is Being Forwarded

	Optional backward call indicators parameter


In-band information indicator



in-band information or an appropriate pattern is now available 
	P-Early-Media header authorizing backward early media


7.2.3.2.5.3
Access Transport Parameter, Transmission medium used parameter
If the O-MGCF supports the PSTN XML body as a network option and if a PSTN XML body is received within the 180 ringing or 183 session progress, the O-MGCF shall store the received PSTN XML elements, replacing any previously stored PSTN XML elements on that dialog.
NOTE:
Multiple 18x responses can be received, both within a single dialog and in multiple dialogs. The PSTN XML bodies are stored on a per-dialog basis to be mapped to the ATP/TMU parameters on receipt of the 200 OK (see clause 7.2.3.2.9.2).

Table 7.2.3.2.5.3.1: Void
7.2.3.2.5.4
Progress indicator

If the O-MGCF supports the PSTN XML body as a network option and receives it in the 180 or 183, the O-MGCF shall store a "ProgressIndicator" element from the PSTN XML body on a per dialog basis and shall add itionaly map it into a Progress Indicator in the ACM as shown in table 7.2.3.2.5.4.1.
Table 7.2.3.2.5.4.1: Handling of the progress indicator

	(ACM
	(180/183

	Access transport parameter
	 PSTN XML body with Progress indicator with "Coding Standard" value "00 (ITU-T standardized coding)" and with "Progress Description" No. (Value of PI)

	Progress indicator (NOTE)
	Progress indicator No. 1 / 2

	Progress indicator (NOTE)
	Progress indicator No. 8

	NOTE:
The entire Progress Indicator, including the "Progress Description", "Coding Standard" and "Location" parameters shall be copied.


Table 7.2.3.2.5.4.2: Void
7.2.3.2.5.5
Cause Value

If the O-MGCF sends the ACM upon reception of a SIP 183 provisional response containing a SIP reason header with a Q.850 Cause value, the IO-MGCF may include the received Cause value within the ACM as a network option. The mapping of the Cause Indicators parameter to the Reason header as shown in table 8a shall be applied. IETF RFC 6432 [115] describes the use of the Reason header field in responses. The Reason header field itself is described in IETF RFC 3326 [116].

7.2.3.2.6
Sending of the Call Progress message (CPG)

7.2.3.2.6.0
General

If the Address Complete Message (ACM) has already been sent, the O-MGCF shall send the Call Progress message (CPG) in the following cases:

-
Upon receipt of the SIP 180 Ringing provisional response. An O-MGCF should initiate the sending of an awaiting answer indication only if according to IETF RFC 5009 [89] backward early media is not authorized (the most recently received P-Early-Media header field does not authorize the backward early media or the P-Early-Media header field has not yet been received).
[image: image37.wmf] 

180 Ringing

 

CPG (Alerting)

 

O

-

MGCF

 

R

ing tone

 


Figure 18: Sending of CPG(Alerting) (Receipt of 180 Ringing response and backward early media is not authorized)


[image: image38.emf] 

O - MGCF  

CPG     (Alerting)  

Ring tone  

180 Ringing     [P - Early - Media header:  backward early media  authorized]  


Figure 18a: Sending of CPG(Alerting) (Receipt of 180 Ringing response with authorization of early media)

Based on local knowledge that the call is transited to a PSTN network, the O-MGCF may decide not to generate the awaiting answer indication when receiving the 180 Ringing message and backward early media is not authorized according to IETF RFC 5009 [89].
-
At an O-MGCF upon receipt of a 183 Session Progress that includes the first P-Early-Media header field authorizing backward early media.

[image: image39.emf] 

O - MGCF  

CPG     (In - band information  available)  

Appropriate  announcement  

183 Session Progress     [P - Early - Media header:  backward early media  authorized]  


Figure 18b: Sending of CPG(in-band information available)
-
Upon receipt of the 181 Call is Being Forwarded provisional response.


[image: image40.emf] 

O - MGCF  

CPG     (Progress, OBCI: in - band info available)  

Audio  

181 Call is Being  Forwarded   [P - Early - Media header:  backward early media  authorized]  


Figure 18c: Sending of CPG(Progress)

-
As a network option, reception of 183 containing a SIP reason header with a Q.850 Cause Value.


[image: image41.wmf] 

18

3

 

Session Progress

 

(Reason(Cause=xxx))

 

CPG

 

(

in

-

band 

information available

 

Cause=xxx

)

 

O

-

MGCF

 


Figure 18d: Sending of CPG (Receipt of 183 Session Progress containing SIP Reason header)
If the O-MGCF receives a 18x response with P-Early-Media header that changes the authorization of early media, the O-MGCF terminates the sending of the awaiting answer indication if the header authorizes backward early media and initiates the sending of the awaiting answer indication if the header removes authorization of backward early media and if the O-MGCF has received the 180 Ringing response.
7.2.3.2.6.1
Handling of the progress indicator

If the O-MGCF supports the PSTN XML body as a network option and receives it in the 180 or 183, any "ProgressIndicator" element in the PSTN XML body shall be stored on a per-dialog basis as well as mapped as shown in tables 7.2.3.2.6.1.1 and 7.2.3.2.6.1.3.

Table 7.2.3.2.6.1.1: Mapping of progress indicator in PSTN XML body into ATP
	(CPG 
	(180/183 

	Access transport parameter
	 PSTN XML body with Progress indicator X

	Progress indicator (NOTE 1, NOTE 3)
	Progress indicator No. 1 / 2

	Progress indicator (NOTE 2, NOTE 3)
	Progress indicator No. 4

	Progress indicator No. 4 (NOTE 2, NOTE 4)
	Progress indicator No. 7

	NOTE 1:
Values 1 ("call is not end-to-end ISDN: further call progress information may be available in-band") or 2 ("destination address is non-ISDN") shall be sent if Value 4 ("Call has returned to the ISDN") has been sent since value 1 or 2 was previously sent or if no value 1 or 2 was previously sent.

NOTE 2:
Value 4 ("Call has returned to the ISDN") shall be sent if value 1 ("call is not end-to-end ISDN: further call progress information may be available in-band") or 2 ("destination address is non-ISDN") was sent previously and no value 4 has been signalled since.
NOTE 3:
The entire Progress Indicator, including the "Progress Description", "Coding Standard" and "Location" parameters shall be copied.

NOTE 4:
The Progress Indicator "Coding Standard" parameter shall be set to "00 (ITU-T standardized coding)". The default value for the Progress Indicator "Location" parameter is "0100 (Public Network serving remote user)".


Table 7.2.3.2.6.1.2: Void
Table 7.2.3.2.6.1.3: Mapping of progress indicator in PSTN XML body into Event Indicator
	(CPG 
	(180/183 

	Event indicator 
	 PSTN XML body with Progress indicator with "Coding Standard" value "00 (ITU-T standardized coding)" and with Progress Description" value No. X

	"In-band information or appropriate pattern is now available"
	No. 8

"In-band information or appropriate pattern is now available"


7.2.3.2.7
Coding of the CPG

7.2.3.2.7.0
General

The description of the following ISDN user part parameters can be found in ITU-T Recommendation Q.763 [4].

7.2.3.2.7.1
Event information

bits
G-A


Event indicator

0 0 0 0 0 0 1
alerting if 180 Ringing response received
0 0 0 0 0 1 0
progress, if 181 Call is Being Forwarded response received
0 0 0 0 0 1 1
in-band information or an appropriate pattern is now available, if the received 183 Session 




Progress response and most recently received P-Early-Media header authorizes backward early



media
NOTE:
In national networks other values of the Event indicator may be used.
If the O-MGCF supports the PSTN XML body as a network option and receives in the 180 or 183 a "ProgressIndicator" element in the PSTN XML body with a "Coding Standard" value "00 (ITU-T standardized coding)" and with Progress Description" value No. 8, instead of the mapping above, the O-MGCF shall map this "Progress Indicator" into the "Event Indicator" within the CPG as shown in table 7.2.3.2.6.1.3.
7.2.3.2.7.2
Access Transport Parameter

If the O-MGCF supports the PSTN XML body as a network option and if a PSTN XML body is received within the 180 ringing or 183 session progress, the O-MGCF shall store the contained information as described in clause 7.2.3.2.5.3, and additionally shall map this "Progress Indicator" into the ATP within the CPG as shown in table 7.2.3.2.6.1.1.

NOTE:
Multiple 18x responses can be received, both within a single dialog and in multiple dialogs. The PSTN XML bodies are stored on a per-dialog basis to be mapped to the ATP parameter on receipt of the 200 OK (see clause 7.2.3.2.9.2).
7.2.3.2.7.3
Void
Table 17h: Void
Table 17j: Void
7.2.3.2.7.4
Handling of Backward Call indicators
The Backward Call indicator shall be derived as shown in section 7.2.3.2.5.1. The Backward Call Indicators parameter is optional in the CPG message and shall only be included if any indicators have changed from those previously sent.
7.2.3.2.7.5
Optional Backward call indicators

Bit A
1
"in-band information or an appropriate pattern is now available" shall be set if 181 Call is Being 
      Forwarded response is received and according to IETF RFC 5009 [89] the backward early media is 
      authorized.
Table 17k: Sending criteria of Optional backward call indicators parameter

	(CPG
	( 181 Call is Being Forwarded

	Optional backward call indicators parameter


In-band information indicator



in-band information or an appropriate pattern is 
      now available 
	P-Early-Media header authorizing backward early media


7.2.3.2.7.6
Cause Value

If the O-MGCF sends the CPG upon reception of a SIP 183 provisional response containing a SIP reason header with a Q.850 Cause value, the O-MGCF may include the received Cause value within the ACM as a network option. The mapping of the Cause Indicators parameter to the Reason header as shown in table 8a shall be applied. IETF RFC 6432 [115] describes the use of the Reason header field in responses. The Reason header field itself is described in IETF RFC 3326 [116].

7.2.3.2.7a
Receipt of 200 OK(INVITE)

Upon receipt of the first 200 OK (INVITE), the O-MGCF shall send an Answer Message (ANM) or Connect message (CON) as described in clauses 7.2.3.2.8 to 7.2.3.2.11.

The O-MGCF shall not progress any further early dialogues to established dialogues. Therefore, upon the reception of a subsequent final 200 (OK) response for any further dialogue for an INVITE request (e.g., due to forking), the O-MGCF shall:

1)
acknowledge the response with an ACK request; and

2)
send a BYE request to this dialog in order to terminate it.
7.2.3.2.7b
Internal through connection of the bearer path

The through connection procedure is described in clause 9.2.3.3.7.
7.2.3.2.8
Sending of the Answer Message (ANM)

Upon receipt of the first 200 OK (INVITE), if the Address Complete Message (ACM) has already been sent, the O-MGCF shall send the Answer Message (ANM) to the preceding exchange.

NOTE: Through connection and the stop of awaiting answer indication are described in clause 9.2.3.3


[image: image42.wmf] 

200 OK (INVITE)

 

ANM

 

O

-

MGCF

 


Figure 19: Sending of ANM

7.2.3.2.9
Coding of the ANM

7.2.3.2.9.1
Backwards Call Indicators

If Backwards Call Indicators are included in the ANM, then the coding of these parameters shall be as described in clause 7.2.3.2.5.1. The Backward Call Indicators parameter is optional in the ANM message and shall only be included if any indicators have changed from those previously sent.
7.2.3.2.9.2
Access Transport Parameter

If the O-MGCF supports the PSTN XML body as a network option and if a PSTN XML body is received within the 200 OK(INVITE) or has been previously stored from a 18x message, the O-MGCF shall map the most recently received information for the established dialog (i.e. the dialog for which the first 200 OK has been received) into the ANM as shown in table 7.2.3.2.9.2.1 except Progress Indicator value No. 3 or No. 8.

Table 7.2.3.2.9.2.1: Mapping of PSTN XML elements into ISUP Parameters

	( ANM
	( 200 OK / stored information from previous 18x

	ISUP Parameter
	Content
	PSTN XML (NOTE 9)

	Access Transport Parameter
	Progress indicator (NOTE 5, NOTE 8)
	ProgressIndicator with "Coding Standard" value "00 (ITU-T standardized coding)" and with "Progress Description" value No. 1 / 2

	
	Progress indicator with "Progress Description" value No. 4 (NOTE 4, NOTE 6, NOTE 9)
	ProgressIndicator with "Coding Standard" value "00 (ITU-T standardized coding)" and with "Progress Description" value No. 7 

	
	Progress indicator (NOTE 6, NOTE 8)
	ProgressIndicator with "Coding Standard" value "00 (ITU-T standardized coding)" and with "Progress Description" value No. 4

	
	Progress indicator (NOTE 7, NOTE 8)
	ProgressIndicator with "Coding Standard" value "00 (ITU-T standardized coding)" and with "Progress Description" value No. 5

	
	High layer compatibility (NOTE 1)
	HighLayerCompatibility

	
	Bearer Capability
	BearerCapability (NOTE 2)

	
	Bearer Capability ("UDI-TA")
	BearerCapability ("UDI-TA") (NOTE 3)

	NOTE 1:
This information element shall only be mapped if the O-MGCF transfers media types listed in table 10b without transcoding.

NOTE 2:
Applicable if the O-MGCF has not propagated UDI fallback signalling according to clause 7.2.3.2.1.5.

NOTE 3:
Applicable if the O-MGCF has propagated UDI fallback signalling according to clause 7.2.3.2.1.5. Only the value "UDI-TA" within the PSTN XML BC shall be mapped. Other values within the PSTN XML BC are mapped to TMU as described in clause 7.2.3.2.9.3.
NOTE 4:
ProgressIndicator No. 7 is not mapped into the ISUP ATP. However, it may be mapped into PI=4.

NOTE 5:
Values 1 ("call is not end-to-end ISDN: further call progress information may be available in-band") or 2 ("destination address is non-ISDN") shall be sent if Value 4 ("Call has returned to the ISDN") has been sent since value 1 or 2 was previously sent or if no value 1 or 2 was previously sent.

NOTE 6:
Value 4 ("Call has returned to the ISDN") shall be sent if value 1 ("call is not end-to-end ISDN: further call progress information may be available in-band") or 2 ("destination address is non-ISDN") was sent previously and no value 4 has been signalled since.

NOTE 7:
This value indicates a bearer service change and is present with an associated BearerCapability and indicates that fallback has occurred (i.e. TMR and TMR Prime present in IAM and the destination ISDN user has accepted the BearerCapability equating to TMR Prime).
NOTE 8:
The entire Progress Indicator, including the "Progress Description", "Coding Standard" and "Location" parameters shall be copied.

NOTE 9:
The Progress Indicator "Coding Standard" parameter shall be set to "00 (ITU-T standardized coding)". The default value for the Progress Indicator "Location" parameter is "0100 (Public Network serving remote user)".


7.2.3.2.9.3
Transmission Medium Used parameter (TMU)

The procedures in the present clause shall only apply if the O-MGCF supports the PSTN XML body, and has propagated UDI-TA fallback signalling according to clause 7.2.3.2.1.5.

If a Bearer Capability element within a PSTN XML body is received within the first 200 OK(INVITE) or has been previously stored from a 18x message relating to the now established dialog (i.e. the dialogue for which the first 200 OK has been received), the O-MGCF shall map the most recently received information (if any) into a TMU within the ANM as shown in table 7.2.3.2.9.3.1. If the most recently received PSTN XML BearerCapability is "UDI-TA", it shall be mapped into an ISUP Access Transport Parameter Bearer Capability (see clause 7.2.3.2.9.2).
NOTE:
The TMU is only included if both the TMR and TMR Prime were received in the ISUP IAM and fallback has occurred.

Table 7.2.3.2.9.3.1 – Mapping to TMU parameter

	( ANM
	( 200 OK / stored information from previous 18x

	TMU
	PSTN XML BearerCapability

	TMU = "Speech"
	PSTN XML BearerCapability = "Speech"

	TMU= "3.1 kHz audio"
	PSTN XML BearerCapability = "3.1 kHz audio"

	No mapping (fallback has not occurred)
	PSTN XML BearerCapability = "UDI-TA"

	TMU = "3.1 kHz audio"
	PSTN XML BearerCapability not present


7.2.3.2.10
Sending of the Connect message (CON)

Upon receipt of the first 200 OK (INVITE), if the Address Complete Message (ACM) has not yet been sent, the O-MGCF shall send the Connect message (CON) to the preceding exchange.


[image: image43.wmf] 

200 OK (INVITE)

 

CON

 

O

-

MGCF

 


Figure 20: Sending of CON

7.2.3.2.11
Coding of the CON

7.2.3.2.11.0
General

The description of the following ISDN user part parameters can be found in ITU-T Recommendation Q.763 [4].

7.2.3.2.11.1
Backward call indicators

The Called Party's status indicator (Bit DC) of BCI parameter shall be set to "no indication". The other BCI indicators shall be set as described in clause 7.2.3.2.5.1
7.2.3.2.11.2
Access Transport Parameter
The O-MGCF shall apply the same procedure as described for the ANM in clause 7.2.3.2.9.2.
7.2.3.2.11.3
Transmission medium used parameter

The O-MGCF shall apply the same procedure as described for the ANM in clause 7.2.3.2.9.3.
7.2.3.2.11A
Receipt of a reINVITE request

When a reINVITE request is received from the network containing a Call-Info header field the MGCF may instruct the IM-MGW to send media available at the associated URL to the PSTN leg of the communication.
7.2.3.2.12
Receipt of Status Codes 4xx, 5xx or 6xx


[image: image44.wmf] 

REL

 

Status Code 

4xx, 5xx or 6xx

 

O

-

MGCF

 


Figure 21: Receipt of Status codes 4xx, 5xx or 6xx

If a Reason header as described in IETF RFC 6432 [115] is included in a 4xx, 5xx, 6xx response, then the Cause Value of the Reason header shall be mapped to the ISUP Cause Value field in the ISUP REL message. The Reason header field itself is described in IETF RFC 3326 [116]. The mapping of the Reason header to the Cause Indicators parameter is shown in table 8a (see clause 7.2.3.1.7). Otherwise coding of the Cause value field in the REL message is derived from the SIP Status code received according to table 18. The Cause Indicators Parameter Values are defined in ITU-T Recommendation Q.850 [38].
In all cases where SIP itself specifies additional SIP side behaviour related to the receipt of a particular INVITE response these procedures should be followed in preference to the immediate sending of a REL message to BICC/ISUP.

If there are no SIP side procedures associated with this response, the REL shall be sent immediately.

NOTE
Depending upon the SIP side procedures applied at the O-MGCF it is possible that receipt of certain 4xx/5xx/6xx responses to an INVITE may in some cases not result in any REL message being sent to the BICC/ISUP network. For example, if a 401 Unauthorized response is received and the O-MGCF successfully initiates a new INVITE containing the correct credentials, the call will proceed.

Table 18: 4xx/5xx/6xx Received on SIP side of O-MGCF

	(REL (cause value)
	(4xx/5xx/6xx SIP Message

	Cause value No 111 (Protocol error, unspecified)
	400 Bad Request

	Cause value No 127 (Interworking, unspecified)
	401 Unauthorized

	Cause value No 127 (Interworking, unspecified)
	402 Payment Required

	Cause value No 79 (Service or option not implemented, unspecified)
	403 Forbidden

	Cause value No 1 (Unallocated (unassigned) number)
	404 Not Found

	Cause value No 127 (Interworking, unspecified)
	405 Method Not Allowed

	Cause value No 127 (Interworking, unspecified)
	406 Not Acceptable

	Cause value No 127 (Interworking, unspecified)
	407 Proxy authentication required

	Cause value No 102 (Recovery on timer expiry)
	408 Request Timeout

	Cause value No 22 (Number changed)
	410 Gone

	Cause value No 127 (Interworking, unspecified)
	413 Request Entity too long

	Cause value No 111 (Protocol error, unspecified)
	414 Request-URI too long

	Cause value No 127 (Interworking, unspecified)
	415 Unsupported Media type

	Cause value No 111 (Protocol error, unspecified)
	416 Unsupported URI scheme

	Cause value No 79 (Service or option not implemented, unspecified)
	417 Unknown Resource-Priority

	Cause value No 111 (Protocol error, unspecified)
	420 Bad Extension

	Cause value No 111 (Protocol error, unspecified)
	421 Extension required

	Cause value No 31 (Normal, unspecified)
	422 Session Interval Too Small

	Cause value No 127 (Interworking, unspecified)
	423 Interval Too Brief

	Cause value No 24 (Call rejected due to ACR supplementary service)
	433 Anonymity Disallowed (NOTE 1)

	Cause value No 127 (Interworking, unspecified)
	440 Max-Breadth Exceeded

	Cause value No 20 (Subscriber absent)
	480 Temporarily Unavailable

	Cause value No 127 (Interworking, unspecified)
	481 Call/Transaction does not exist

	Cause value No 127 (Interworking, unspecified)
	482 Loop detected

	Cause value No 25 (Exchange routing error)
	483 Too many hops

	Cause value No 28 (Invalid number format (address incomplete))
	484 Address Incomplete

	Cause value No 1 (Unallocated (unassigned) number)
	485 Ambiguous

	Cause value No 17 (User busy)
	486 Busy Here

	Cause value No 127 (Interworking, unspecified) or not interworked. (NOTE 2)
	487 Request terminated

	Cause value No 50 (Requested facility not subscribed)
	488 Not acceptable here

	Cause value No 127 (Interworking, unspecified)
	493 Undecipherable

	Cause value No 127 (Interworking, unspecified)
	500 Server Internal error

	Cause value No 79 (Service or option not implemented, unspecified)
	501 Not implemented

	Cause value No 27 (Destination out of order)
	502 Bad Gateway

	Cause value No 41 (Temporary failure)
	503 Service Unavailable

	Cause value No 102 (Recovery on timer expiry)
	504 Server timeout

	Cause value No 127 (Interworking, unspecified)
	505 Version not supported

	Cause value No 95 (Invalid message, unspecified)
	513 Message too large

	Cause value No 127 (Interworking, unspecified)
	580 Precondition failure

	Cause value No 17 (User busy)
	600 Busy Everywhere

	Cause value No 21 (Call rejected)
	603 Decline

	Cause value No 2 (No route to specified transit network)
	604 Does not exist anywhere

	Cause value No 88 (Incompatible destination)
	606 Not Acceptable

	NOTE 1:
Anonymity Disallowed, IETF RFC 5079 [77] refers.

NOTE 2:
No interworking if the O-MGCF previously issued a CANCEL request for the INVITE.

NOTE 3:
The 4xx/5xx/6xx SIP responses that are not covered in this table are not interworked.


If the O-MGCF supports the PSTN XML body as a network option and if a PSTN XML body is received within the 4xx/5xx/6xx, the O-MGCF shall map the contained information into the Access Transport Parameter of the REL as shown in clause 7.2.3.2.9.2.
When a 4xx, 5xx or 6xx SIP response to an INVITE request is received from the network containing an Error-Info header field, the O-MGCF, supporting the capabilities associated with the Error-Info header field, may instruct the IM-MGW to play out media available at the associated URL towards PSTN.
7.2.3.2.12.1
Special handling of 404 Not Found and 484 Address Incomplete responses after sending of INVITE without determining the end of address signalling

This clause is only applicable when the network option of Sending of INVITE without determining the end of address signalling is being used (see clause 7.2.3.2.1.a).

On receipt of a 404 Not Found or 484 Address Incomplete response while Ti/w2 is running, the O-MGCF shall start timer Ti/w3, if there are no other pending INVITE transactions for the corresponding call.

At the receipt of a SAM, a SIP 1xx provisional response or a SIP 200 OK (INVITE), the O-MGCF shall stop Ti/w2 and Ti/w3.

The O-MGCF shall send a REL message with Cause Value 28 towards the BICC/ISUP network if Ti/w3 expires.

7.2.3 2.13
Receipt of a BYE


[image: image45.wmf] 

REL

 

O

-

MGCF

 

BYE

 


Figure 22: Receipt of BYE method

If a Reason header field with Q.850 Cause Value is included in the BYE request, then the Cause Value shall be mapped to the ISUP Cause Value field in the ISUP REL as shown in table 8a (see clause 7.2.3.1.7). Otherwise, the O-MGCF sends a REL message with Cause Code value 16 (Normal Call Clearing).
If the O-MGCF supports the PSTN XML body as a network option and if a PSTN XML body is received within the BYE, the O-MGCF shall map the contained information into the Access Transport Parameter of the REL as shown in clause 7.2.3.2.9.2.
7.2.3.2.14
Receipt of the Release Message

In the case that the REL message is received and a final response (i.e. 200 OK (INVITE)) has already been received the O-MGCF shall send a BYE request. If the final response (i.e. 200 OK (INVITE)) has not already been received the O-MGCF shall send a CANCEL method.
A Reason header field containing the received (Q.850) Cause Value of the REL message shall be added to the CANCEL or BYE request. The mapping of the Cause Indicators parameter to the Reason header is shown in table 9a (see clause 7.2.3.1.8).
If the O-MGCF supports the PSTN XML body as a network option, the O-MGCF shall map the contained information into a PSTN XML body within the BYE or CANCEL as shown in table 9aa.
7.2.3.2.15
Receipt of RSC, GRS or CGB (H/W oriented)

Upon receipt of a RSC, GRS or CGB (H/W oriented) message the following applies independently for each affected circuit:

NOTE:
For the RSC message, the circuit identified by the CIC is affected.
For the GRS message, the affected circuits are identified by the CIC and the Range subfield of the Range and Status parameter.
For the CGB message, the affected circuits are identified by the CIC and the Range and Status parameter.

If a final response (i.e. 200 OK (INVITE) has already been received, the O-MGCF shall send a BYE method. If a final response (i.e. 200 OK (INVITE)) has not already been received, the O-MGCF shall send a CANCEL method.

A Reason header field containing the (Q.850) Cause Value of the REL message generated by the ISUP procedures shall be added to the SIP message (BYE or CANCEL request) to be sent by the SIP side of the O‑MGCF.

Editor's Note: It is FFS whether to indicate the cause value for internal error in the network to the user.
7.2.3.2.16
Autonomous Release at O-MGCF

If the O-MGCF determines due to internal procedures that the call shall be released then the MGCF shall send

-
A BYE method if the ACK has been sent.

-
A CANCEL method before 200 OK (INVITE) has been received.

NOTE:
The MGCF shall send the ACK method before it sends the BYE, if 200 OK (INVITE) is received.

A Reason header field containing the (Q.850) Cause Value of the REL message sent by the O‑IWU shall be added to the SIP Message (BYE or CANCEL request) to be sent by the SIP side of the O‑IWU.
Editor's Note: It is FFS whether to indicate the cause value for internal error in the network to the user.
Table 18a: Autonomous Release at O-MGCF

	REL (
Cause parameter
	Trigger event
	( SIP

	As determined by BICC/ISUP procedure.
	COT received with the Continuity Indicators parameter set to "continuity check failed" (ISUP only) or the BICC/ISUP timer T8 expires.
	CANCEL or BYE according to the rules described in this subclause.

	REL with cause value 47 (resource unavailable, unspecified).
	Internal resource reservation unsuccessful
	As determined by SIP procedure

	As determined by BICC/ISUP procedure.
	BICC/ISUP procedures result in generation of autonomous REL on BICC/ISUP side.
	CANCEL or BYE according to the rules described in this subclause. 

	Depending on the SIP release reason.
	SIP procedures result in a decision to release the call.
	As determined by SIP procedure.


7.2.3.2.17
Special handling of 580 precondition failure received in response to either an INVITE or UPDATE

A 580 Precondition failure response may be received as a response either to an INVITE or to an UPDATE request.

7.2.3.2.17.1
580 Precondition failure response to an INVITE

Release with cause code as indicated in table 18 is sent immediately to the BICC/ISUP network.

7.2.3.2.17.2
580 Precondition failure response to an UPDATE within an early dialog

A BYE request is sent for the early dialog within which the UPDATE was sent.
If all the early dialogs that were generated from the INVITE request have answered the respective UPDATE request with 580 Precondition failure response then the O-MGCF shall send the Release message with Cause Code '127 Interworking' to the ISUP network.
7.2.3.2.18
Sending of CANCEL


[image: image46.wmf] 

COT (failure)

 

CANCEL

 

O

-

MGCF

 


Figure 23: Receipt of COT (failure).

CANCEL shall be sent if the Continuity message is received with the Continuity Indicators parameter set to "continuity check failed" or the ISUP (or BICC) timer T8 expires.
A Reason header field containing the (Q.850) Cause Value 41 Temporary Failure shall be added to the CANCEL request to be sent by the SIP side of the O‑MGCF.
7.2.3.2.19
Receipt of SIP redirect (3xx) response


[image: image47.wmf] 

REL

 

(Cause value 127)

 

SIP response 

code 3xx

 

O

-

MGCF

 


Figure 24: Receipt of SIP response code 3xx

When receiving a SIP response with a response code 3xx, the default behaviour of the O-MGCF is to release the call with a cause code value 127 (Interworking unspecified).

NOTE:
The O-MGCF may also decide for example to redirect the call towards the URIs in the Contact header field of the response as an operator option, but such handling is outside of the scope of the present document.
7.2.3.2.20
Sending of INFO for overlap signalling using the in-dialog method
7.2.3.2.20.1
General

SIP INFO request ("legacy" mode of usage of the INFO method as defined in IETF RFC 6086 [133]) is used to carry additional digits when overlap signalling is sent using the in-dialog method as described in clause 7.2.3.2.1a.2, which is a network option. clause 7.2.3.2.1a.2 also describes when the O-MGCF sends a SIP INFO request.

7.2.3.2.20.2
Encoding of the INFO

Table 18b provides a summary of how the header fields within the outgoing INFO messages are populated when in-dialog SIP INFO requests are used for overlap dialling.

Table 18b: Interworked contents of the INFO message

	SAM(
	INFO(

	Digits
	See TS 24.229 [9] clause 7.11.1.2


7.2.3.3
Timers

Table 19: Timers for interworking

	Symbol
	Time-out value
	Cause for initiation
	Normal termination
	At expiry
	Reference

	Ti/w1
	4 s to 6 s (default of 4 s)
	When last address message is received and the minimum number of digits required for routing the call have been received.
	At the receipt of fresh address information.
	Send INVITE, send the address complete message
	7.2.3.2.1
7.2.3.2.4
(NOTE 1)

	Ti/w2
	4 s to 20 s (default of 4 s)
	When INVITE is sent unless the ACM has already been sent.
	On reception of 180 Ringing , or 183 Session Progress and P-Early-Media header authorizing backward early media, or 181 Call is Being Forwarded, or 404 Not Found or 484 Address Incomplete for an INVITE transaction for which Ti/w3 is running, or 200 OK (INVITE).
	Send ACM (no indication)
	7.2.3.2.4
7.2.3.2.1

(NOTE 2)

	Ti/w3
	4‑6 seconds (default of 4 seconds)


	On receipt of 404 Not Found or 484 Address Incomplete if there are no other pending INVITE transactions for the corresponding call.
	At the receipt of SAM
	Send REL with Cause Value 28 to the BICC/ISUP side.
	7.2.3.2.1A, 7.2.3.2.12.1

(NOTE 3)

	NOTE 1:
This timer is used when overlap signalling is received from BICC/ISUP network and converted to en-block signalling at the MGCF.

NOTE 2:
This timer is used to send an early ACM if a delay is encountered in receiving a response from the subsequent SIP network.

NOTE 3:
This timer is known as the "SIP dialog protection timer". This timer is only used where the O‑MGCF is configured to send INVITE before end of address signalling is determined.


7.3
Interworking between CS networks supporting BICC and the IM CN subsystem

The control plane between CS networks supporting BICC and the IM CN subsystem supporting SIP, where the underlying network is SS7 and IP respectively is as shown in figures 25, 26 and 27.


[image: image48.wmf] 

 

SS7 signalling

 

function

 

Signallin

g gateway

 

function

 

Media gateway

 

control function

 

SS7

 

SIP signalling

 

function

 

IP

 

L1

 

SCTP

 

M3UA

 

MTP2

 

MTP3

 

IP

 

TCP / 

 

UDP /

 

SCTP

 

SIP

 

IP

 

IP

 

L1

 

MTP2

 

MTP3

 

BICC

 

STC

 

SIP

 

IP

 

IP

 

SCTP

 

M3UA

 

TCP / 

 

UDP /

 

SCTP

 

BICC

 

STC

 

BICC

 

SIP

 


Figure 25: Control Plane interworking between CS networks supporting BICC over MTP3 and the IM CN subsystem


[image: image49.wmf] 

 

SS7 signalling

 

function

 

Signalling gateway

 

function

 

Media gateway

 

control function

 

AAL

5

 

SIP signalling

 

function

 

IP

 

SSCOP

 

SCTP

 

M3UA

 

SSCF

 

MTP3B

 

IP

 

TCP /

 

UDP /

 

SCTP

 

SIP

 

IP

 

IP

 

SSCOP

 

SSCF

 

MTP3B

 

BICC

 

STC

 

SIP

 

IP

 

IP

 

SCTP

 

M3UA

 

TCP /

 

UDP /

 

SCTP

 

BICC

 

STC

 

BICC

 

SIP

 

AAL5

 

AAL5

 


Figure 26: Control Plane interworking between CS networks supporting BICC over MTP3B over AAL5 and the IM CN subsystem


[image: image50.wmf] 

 

SS7 signalling

 

function

 

Media gateway

 

control function

 

SIP signallin

g

 

function

 

IP

 

TCP /

 

UDP /

 

SCTP

 

SIP

 

IP

 

IP

 

IP

 

SCTP

 

M3UA

 

BICC

 

STC

MTP

 

SIP

 

IP

 

IP

 

SCTP

 

M3UA

 

TCP /

 

UDP / 

 

SCTP

 

BICC

 

STC

MTP

 

BICC

 

SIP

 


Figure 27: Control Plane interworking between CS networks supporting BICC
over STC and M3UA and the IM CN subsystem

7.3.1
Services performed by network entities in the control plane

Services offered by the network entities in the control plane are as detailed in clause 7.2.1.

If ATM transport is applied between the SS7 Signalling function and the Signalling Gateway Function, they shall apply MTP3B (ITU-T Recommendation Q.2210 [46]) over SSCF (ITU-T Recommendation Q.2140 [45]) over SSCOP (ITU-T Recommendation Q.2110 [44]) over AAL5 (ITU-T Recommendation I.363.5 [43]) as depicted in figure 26.

If IP transport is applied between the SS7 Signalling function and the MGCF, they shall support and apply M3UA, SCTP and IP (either IPv4, see RFC 791 [16], or IPv6, see RFC 2460 [39]), as depicted in figure 27.

7.3.2
Signalling interactions between network entities in the control plane

7.3.2.1
Signalling between the SS7 signalling function and MGCF

See clause 7.2.2.1.

7.3.2.1.1
Signalling from MGCF to SS7 signalling function

See clause 7.2.2.1.1.

7.3.2.1.2
Signalling from SS7 signalling function to MGCF

See clause 7.2.2.1.2.

7.3.2.1.3
Services offered by STC, SCTP and M3UA

See clause 7.2.2.1.3.

7.3.2.1.3.1
Services offer by SCTP

See clause 7.2.2.1.3.1.

7.3.2.1.3.2
Services offered by M3UA

See clause 7.2.2.1.3.2.

7.3.2.1.3.3
Services offered by STC

STC provides the services for the transparent transfer of STC user information, e.g. BICC, between STC users, i.e. the SS7 signalling function and the MGCF (see TS 29.205 [14]).

STC performs the functions of data transfer service availability reporting and congestion reporting to the STC user and User part availability control. See ITU-T Recommendation Q.2150.1 [29].
7.3.2.2
Signalling between the MGCF and SIP signalling function

See clause 7.2.2.2.

7.3.3
SIP-BICC protocol interworking

7.3.3.1
Incoming call interworking from SIP to BICC at I-MGCF

7.3.3.1.1
Sending of IAM

Clause 7.2.3.1.1 is applicable with the clarification that the Continuity Check procedure applies.

Figure 28: Void
7.3.3.1.2
Coding of IAM

The description of the following ISDN user part parameters can be found in ITU-T Recommendation Q.763 [4].

7.3.3.1.2.1
Called party number

See clause 7.2.3.1.2.1.

7.3.3.1.2.2
Nature of connection indicators

bits
BA

Satellite indicator

0 0


no satellite circuit in the connection
bits
DC

Continuity indicator (BICC)
0 0


no COT to be expected, if the received SDP does not contain precondition information or




indicates that all preconditions are fulfilled, and all local resource reservation is completed.
1 0


COT to be expected, if the received SDP indicates that precondition is not fulfilled or any local


resource reservation is not completed.
bit

E

Echo control device indicator

1


outgoing echo control device included
7.3.3.1.2.3
Forward call indicators

See clause 7.2.3.1.2.3.

7.3.3.1.2.4
Calling party's category

See clause 7.2.3.1.2.4.

7.3.3.1.2.4A
Originating Line Information

See clause 7.2.3.1.2.4A

7.3.3.1.2.5
Transmission medium requirement

See clause 7.2.3.1.2.5.

7.3.3.1.2.6
Calling party number

See clause 7.2.3.1.2.6.

7.3.3.1.2.7
Generic number

See clause 7.2.3.1.2.7.

7.3.3.1.2.8
User service information

See clause 7.2.3.1.2.8.

7.3.3.1.2.9
Hop counter (National option)

See clause 7.2.3.1.2.9.
7.3.3.1.2.10
Location Number

See clause 7.2.3.1.2.11.
7.3.3.1.2A
Coding of the IAM when Number Portability is supported

See clause 7.2.3.1.2A.

7.3.3.1.2B
Coding of the IAM for Carrier Routeing

See clause 7.2.3.1.2B.
7.3.3.1.3
Sending of COT


[image: image51.wmf] 

COT

 

SDP indi

cating 

 

preconditions met 

 

and

/or

 

 

a

ctive media

 

I

-

MGCF

 


Figure 29: Sending of COT

If the IAM has already been sent, then the Continuity message shall be sent indicating "continuity check successful", when all of the following conditions have been met.
-
When the requested remote preconditions (if any) in the IMS have been met.

-
The media stream previously set to inactive mode is set to active (as specified in IETF RFC 4566 [56]).

-
Local preconditions have been fulfilled.
7.3.3.1.3A
Sending of SAM

See clause 7.2.3.1.3A.
7.3.3.1.4
Sending of 180 Ringing

See clause 7.2.3.1.4.
7.3.3.1.4A
Sending of 183 Session Progress for early media scenarios

See clause 7.2.3.1.4A.
7.3.3.1.4B
Sending of 181 Call is being forwarded

See clause 7.2.3.1.4B.
7.3.3.1.4C
Sending of 183 Session Progress for overlap signalling using the in-dialog method

See clause 7.2.3.1.4C.
NOTE:
If the network option of overlap signalling using the in-dialog method is applied at the I-MGCF in combination with the optional codec negotiation interworking with BICC in clause B.2., in order to meet the requirement in clause B.2.1 that the I-MGCF suspends the SDP answer procedure until it receives backward codec information from BICC, the 183 session progress response will not contain the SDP answer.

7.3.3.1.4D
Sending of 183 Session Progress to carry ISUP Cause

See clause 7.2.3.1.4D.
7.3.3.1.5
Sending of the 200 OK (INVITE)

See clause 7.2.3.1.5.

7.3.3.1.6
Sending of the Release message (REL)

See clause 7.2.3.1.6.

7.3.3.1.7
Coding of the REL

See clause 7.2.3.1.7.

7.3.3.1.8
Receipt of the Release Message

See clause 7.2.3.1.8.

7.3.3.1.9
Receipt of RSC, GRS or CGB (H/W oriented)

See clause 7.2.3.1.9.
7.3.3.1.9a
Receipt of REFER

See clause 7.2.3.1.9a.

7.3.3.1.9b
Autonomous Release at I-MGCF

See clause 7.2.3.1.10.

7.3.3.1.10
Internal through connection of the bearer path

The through connection procedure is described in clauses 9.2.2.1.5 and 9.2.2.2.5.

7.3.3.1.11
Out of Band DTMF

If a SIP UA sends DTMF tones to the IM-MGW, the IM-MGW may report this information via the Mn interface to the MGCF. The MGCF shall send to the BICC network the APM message with the following values for the different parameters:

-
Action indicator in accordance with the requested DTMF transport function;
-
Signal in accordance with which DTMF digit to send;
-
Duration in accordance with the required duration of the DTMF digit.

If the BICC network sends an APM message with DTMF signal, duration and action indicator to the MGCF, the MGCF may send this information to the IM-MGW via the Mn interface. The IM-MGW shall send the corresponding DTMF signal and duration information on the user plane of the IM CN subsystem according to RFC 4733 [105].

The interactions with the IM-MGW are shown in clause 9.2.8.

7.3.3.2
Outgoing Call Interworking from BICC to SIP at O-MGCF

7.3.3.2.1
Sending of INVITE

The following particularities apply for a BICC IAM received case, with regard to the already specified in clause 7.2.3.2.1.
The O-MGCF should defer sending the INVITE request until the BICC bearer setup and any local resource reservation is completed.

NOTE:
If the O-MGCF sends the INVITE request before the BICC bearer setup and any local resource reservation is completed, the following considerations apply: If the receiving terminal is not supporting the SIP precondition, clipping may occur. Furthermore, if the MGCF sets the SDP "inactive" attribute in the initial INVITE request and the receiving terminal is not supporting the SIP precondition and the SIP UPDATE method, the interworking procedures within the present specification do not describe all necessary signalling interactions required to set up a call, in particular with respect to the sending of the re-INVITE that may also cause additional delay in the call setup. In addition, the interworking of the ringing indication might not be possible if the peer sends the ringing indication only as response to a re-INVITE.
The BICC bearer setup is completed when one of the following conditions is met:
-
The event Bearer Set-up indication – for the forward bearer set-up case where the incoming Connect Type is "notification not required", which indicate successful completion of bearer set-up, is received by the Incoming bearer set-up procedure (ITU-T Recommendation Q.1902.4 [30] clause 7.5);
-
Bearer Set-up Connect indication for the backward call set-up case, which indicate successful completion of bearer set-up, is received by the Incoming bearer set-up procedure (ITU-T Recommendation Q.1902.4 [30] clause 7.5);
-
BNC set-up success indication for cases using bearer control tunnelling which indicate successful completion of bearer set-up, is received by the Incoming bearer set-up procedure (ITU-T Recommendation Q.1902.4 [30] clause 7.5).
7.3.3.2.1a
Sending of INVITE without determining the end of address signalling

See clause 7.2.3.2.1a.

7.3.3.2.2
Coding of the INVITE

7.3.3.2.2.1
REQUEST URI Header

See clause 7.2.3.2.2.1

7.3.3.2.2.2
SDP Media Description

If the O-MGCF sends the INVITE request without waiting for the BICC bearer setup and any local resource reservation to complete, (which is not recommended according to clause 7.3.3.2.1) it shall indicate that SIP preconditions are not met when the initial INVITE request indicates support of the SIP preconditions.

The SDP media description shall contain precondition information as per TS 24.229 [9]. If the local preconditions are not met the O-MGCF should set the media stream to inactive mode (by including an attribute "a=inactive"). If the local configuration indicates that O-MGCF is deployed in the IMS network that serves users supporting SIP precondition mechanism, the attribute "a=inactive" may be omitted when the initial SDP offer indicates local preconditions are not met. If the initial SDP offer indicates local preconditions are fulfilled, the O-MGCF shall not set the media stream to inactive mode.
The O-MGCF shall include the AMR codec transported according to IETF RFC 4867 [23] with the options listed in clause 5.1.1 of TS 26.236 [32] in the SDP offer. Within the SDP offer, the O-MGCF should also provide SDP RR and RS bandwidth modifiers specified in IETF RFC 3556 [59] to disable RTCP, as detailed in clause 7.4 of TS 26.236 [32].
7.3.3.2.2.3
P-Asserted-Identity and privacy header fields

See clause 7.2.3.2.2.3

7.3.3.2.2.3A
"cpc" URI Parameter in P-Asserted-Identity Header

See clause 7.2.3.2.2.3A.

7.3.3.2.2.3B
"oli" URI Parameter in P-Asserted-Identity Header

See clause 7.2.3.2.2.3B.

7.3.3.2.2.4
Max Forwards header

See clause 7.2.3.2.2.4

7.3.3.2.2.5
IMS Communication Service Identifier

For speech and video calls, the O-MGCF shall insert an IMS Communication Service Identifier, indicating the IMS Multimedia Telephony Communication Service.
The IMS Communication Service Identifier for the IMS Multimedia Telephony Communication Service is defined in TS 24.173 [88].
7.3.3.2.2.6
P-Access-Network-Info

See clause 7.2.3.2.2.9.

7.3.3.2.2A
Coding of the INVITE when number portability is supported

See clause 7.2.3.2.2A.

7.3.3.2.2B
Coding of the INVITE for Carrier Routeing

See clause 7.2.3.2.2B.

7.3.3.2.2C
Coding of INVITE with instance-id in form of IMEI URN

See clause 7.2.3.2.2C.
7.3.3.2.3
Sending of UPDATE

This clause only applies if the O-MGCF sends the INVITE request before SIP preconditions are met (see clause 7.3.3.2.1) .

[image: image52.wmf] 

COT (success)

 

UPDATE

 

O

-

MGCF

 


Figure 30: Receipt of COT (success)

The UPDATE request with a new SDP offer shall be sent for each early SIP dialogue for which the received provisional response indicated support of preconditions confirming that all the required local preconditions have been met when all the following conditions are met.

1.
A Continuity message, with the Continuity Indicators parameter set to "continuity check successful" shall be received.

2.
The reservation of requested local resources has been completed.

In addition, depending on which bearer set-up procedure used for the call one of the following condition shall be met:
-
The event Bearer Set-up indication – for the forward bearer set-up case where the incoming Connect Type is "notification not required", which indicate successful completion of bearer set-up, is received by the Incoming bearer set-up procedure (ITU-T Recommendation Q.1902.4 [30] clause 7.5);
-
Bearer Set-up Connect indication for the backward call set-up case, which indicate successful completion of bearer set-up, is received by the Incoming bearer set-up procedure (ITU-T Recommendation Q.1902.4 [30] clause 7.5);
-
BNC set-up success indication for cases using bearer control tunnelling which indicate successful completion of bearer set-up, is received by the Incoming bearer set-up procedure, (ITU-T Recommendation Q.1902.4 [30] clause 7.5).
If the O-MGCF previously offered inactive media stream it shall set the media stream to active mode.

NOTE:
This procedure applies regardless of whether the early SIP dialog existed prior to the preconditions being fulfilled or is subsequently created.
7.3.3.2.4
Sending of ACM and Awaiting Answer indication

See clause 7.2.3.2.4

The sending of an awaiting answer indication is described in clause 9.2.3.1 and clause 9.2.3.2.

7.3.3.2.5
Coding of the ACM

7.3.3.2.5.1
Backward call indicators

See clause 7.2.3.2.5.1

7.3.3.2.6
Sending of the Call Progress message (CPG)

See clause 7.2.3.2.6.

7.3.3.2.7
Coding of the CPG

7.3.3.2.7.1
Event information

See clause 7.2.3.2.7.1.
7.3.3.2.7.2
Optional Backward call indicators

See clause 7.2.3.2.7.5.
7.3.3.2.7a
Receipt of 200 OK (INVITE)

See clause 7.2.3.2.7a.
7.3.3.2.7b
Internal through connection of the bearer path

The through connection procedure is described in clauses 9.2.3.1.7 and 9.2.3.2.7.
7.3.3.2.8
Sending of the Answer Message (ANM)

See clause 7.2.3.2.8.

7.3.3.2.9
Coding of the ANM

See clause 7.2.3.2.9.

7.3.3.2.10
Sending of the Connect message (CON)

See clause 7.2.3.2.10.

7.3.3.2.11
Coding of the CON

See clause 7.2.3.2.11.

7.3.3.2.11.1
Void
7.3.3.2.11A
Receipt of re-INVITE requests

See clause 7.2.3.2.11A.
7.3.3.2.12
Receipt of Status Codes 4xx, 5xx or 6xx

See clause 7.2.3.2.12.

7.3 3.2.13
Receipt of a BYE

See clause 7.2.3.2.13.

7.3.3.2.14
Receipt of the Release Message

See clause 7.2.3.2.14.

7.3.3.2.15
Receipt of RSC, GRS or CGB (H/W oriented)

See clause 7.2.3.2.15.

7.3.3.2.16
Out of Band DTMF

If a SIP UA sends DTMF tones to the IM-MGW, the IM-MGW may report this information via the Mn interface to the MGCF. The MGCF shall send to the BICC network the APM message with the following values for the different parameters:
-
Action indicator in accordance with the requested DTMF transport function;
-
Signal in accordance with which DTMF digit to send;
-
Duration in accordance with the required duration of the DTMF digit.

If the BICC network sends an APM message with DTMF signal, duration and action indicator to the MGCF, the MGCF may send this information to the IM-MGW via the Mn interface. The IM-MGW shall send the corresponding DTMF signal and duration information on the user plane of the IM CN subsystem according to 4733 [105].

The interaction with the IM-MGW is shown in clause 9.2.8.

7.3.3.2.17
Sending of CANCEL

See clause 7.2.3.2.18.

7.3.3.2.18
Autonomous Release at O-MGCF

See clause 7.2.3.2.16.

7.3.3.2.19
Special handling of 580 precondition failure received in response to either an INVITE or UPDATE

See clause 7.2.3.2.17.

7.3.3.2.20
Receipt of SIP redirect (3xx) response

See clause 7.2.3.2.19.
7.3.3.2.21
Sending of INFO for overlap signalling using the in-dialog method
See clause 7.2.3.2.20.

7.3.3.3
Timers

See clause 7.2.3.3.

7.4
Supplementary Services
The following clauses describe the MGCF behaviour related to supplementary services as defined in ITU-T Recommendations Q.730 to ITU-T Q.737 [42] when interworking with an IMS which does not use a Multimedia Telephony Application Server (MTAS) providing supplementary services according to 3GPP according to TS 24.173 [88]. The support of these supplementary services is optional. If the supplementary services are supported, the procedures described within this clause shall be applied.
7.4.1
Calling line identification presentation/restriction (CLIP/CLIR)

The inter working between the Calling Party Number parameter and the P-Asserted-ID header and vice versa used for the CLIP-CLIR service is defined in the clauses 7.2.3.1.2.6 and 7.2.3.2.2.3. This inter working is essentially the same as for basic call and differs only in that if the CLIR service is invoked the "Address Presentation Restriction Indicator (APRI)" (in the case of ISUP to SIP calls) or the "priv value" of the "calling" Privacy header field (in the case of SIP to ISUP calls) is set to the appropriate "restriction/privacy" value.

In the specific case of ISUP originated calls, use of the CLIP service additionally requires the ability to determine whether the number was network provided or provided by the access signalling system. Due to the possible SIP indication of the P-Asserted-Identity the Screening indicator is set to network provided as default. For the CLIP-CLIR service the mapping of the APRI from privacy header at the O-MGCF is described within table 16 in clause 7.2.3.2.2.3.
At the O-MGCF the presentation restricted indication shall be mapped to the Privacy header field values "id" and "header". This is described in table 12 in clause 7.2.3.2.2.3.

7.4.2
Connected line presentation and restriction (COLP/COLR)

7.4.2.0
General
The COLP/COLR services are only to be interworked between trusted nodes - that is before passing any COLP/COLR information over the SIP-BICC/ISUP boundary the MGCF shall satisfy itself that the nodes on the BICC/ISUP side to which the information is to be passed are trusted.
The procedure within clause 7.5.2 for the generic number ("additional connected number") mapping shall apply.
7.4.2.1
Incoming Call Interworking from SIP to BICC/ISUP at the I-MGCF

7.4.2.1.1
INVITE to IAM interworking (SIP to ISUP/BICC calls)

In the case of SIP to ISUP/BICC calls the I-MGCF may invoke the COLP service as an operator option by setting the "Connected Line Identity Request indicator" field of the "Optional forward call indicators" parameter of the IAM to "requested".

NOTE: This implies that all outgoing calls will invoke the COLP service.

7.4.2.1.2
ANM/CON to 200 OK (INVITE)

Tables 20 and 21 specify the interworking required in the case when the COLP has been automatically requested on behalf of the originating SIP node. The table also indicates the inter workings required if the COLP service has been invoked and the called party has or has not invoked the COLR service.

Table 20: Mapping to P-Asserted-Identity and Privacy Header Fields

	SIP Component
	Setting

	P-Asserted-Identity
	See table 21

	Privacy
	See table 22


Table 21: Mapping of connected number parameter to SIP P-Asserted-Identity header fields

	BICC/ISUP parameter / field
	Value
	SIP component
	Value

	Connected Number 
	
	P-Asserted-Identity header field
	

	Nature of Address Indicator
	 "national (significant) number"

	Tel URI or SIP URI

(NOTE 1)
	Add CC (of the country where the MGCF is located) to Connected PN address signals to construct E.164 number in URI. Prefix number with "+". 

	
	"international number"
 
	
	Map complete Connected address signals to construct E.164 number in URI. Prefix number with "+".

	Address signal
	If NOA is "national (significant) number" then the format of the address signals is:

NDC + SN

If NOA is "international number"
 then the format of the address signals is:

CC + NDC + SN
	Tel URI or SIP URI

(NOTE 1)
	CC+NDC+SN as E.164 number in URI. Prefix number with "+".

	NOTE 1:
A tel URI or a SIP URI with "user=phone" is used according to operator policy.


Table 22: Mapping of BICC/ISUP APRIs into SIP privacy header fields

	BICC/ISUP parameter / field
	Value
	SIP component
	Value

	Connected Number


	
	Privacy header field
	priv-value

	APRI

(See to determine which APRI to use for this mapping)
	"presentation restricted"
	Priv-value
	 "id"
("id" included only if the P-Asserted-Identity header is included in the SIP INVITE)

	
	 "presentation allowed"
 
	Priv-value

 
	omit Privacy header

or Privacy header without "id" if other privacy service is needed


7.4.2.2
Outgoing Call Interworking from BICC/ISUP to SIP at O-MGCF

7.4.2.2.1
IAM to INVITE interworking (ISUP to SIP calls)

The O-MGCF determines that the COLP service has been requested by the calling party by parsing the "Optional Forward Call Indicators" field of the incoming IAM. If the "Connected Line Identity Request indicator" is set to "requested" then the BICC/ISUP to SIP interworking node shall ensure that any backwards "connected party" information is interworked to the appropriate parameters of the ISUP ANM or CON message sent backwards to the calling party as detailed within this subclause.

The O-MGCF has to store the status of the "Connected Line Identity Request indicator".

7.4.2.2.2
1XX to ANM or CON interworking

If the P-Asserted-Identity header field is included within a 1XX SIP response, the identity shall be stored within the O-MGCF together with information about the SIP dialogue of the 1XX SIP response and shall be included within the ANM or CON message if required by the procedures described in clause 7.4.2.2.3. In accordance with ISUP procedures, a connected number shall not be included within the ACM message. The mapping of the of the P-Asserted-Identity and Privacy header fields is shown in tables 23 and 24.

7.4.2.2.3
200 OK (INVITE) to ANM/CON interworking

Tables 23 and 24 specify the interworking required in the case when the calling party has invoked the COLP service. The tables also indicate the interworking procedures required if the calling party has invoked the COLP service and the called party has or has not invoked the COLR service.

If no P-Asserted-Identity header field is provided within the 200 OK (INVITE) message, the stored information previously received in the last provisional 1xx response of the same SIP dialogue shall be used.

NOTE: Due to forking, other P-Asserted-Identities might have been received in different SIP dialogues.

If the Calling Party has requested the COLP service (as indicated by the stored request status) but the 200 OK (INVITE) and previous 1XX provisional responses do not include a P-Asserted-Identity header field, the O-MGCF shall set up a network provided Connected Number with an Address not Available indication.

If the P-Asserted-Identity is available then the Connected number has to be setup with the screening indication network provided. The mapping of the P-Asserted-Identity and Privacy (if available) is shown in table 24.

Table 23: Connected number parameter mapping

	( ANM/CON
	( 200 OK INVITE

	Connected Number (Network Provided)
	P-Asserted-ID

	Address Presentation Restricted Indicator
	Privacy Value Field


Table 24: Mapping of P-Asserted-Identity and privacy headers to the ISUP/BICC connected number parameter

	SIP component
	Value
	BICC/ISUP parameter / field
	Value

	P-Asserted-Identity header field (NOTE)
	 E.164 number
	Connected Number
	

	
	Number incomplete indicator 
	 "Complete" 

	
	Numbering Plan Indicator
	"ISDN/Telephony (E.164)"

	
	Nature of Address Indicator
	If CC encoded in the URI is equal to the CC of the country where MGCF is located AND the next BICC/ISUP node is located in the same country then

set to "national (significant) number"
else set to "international number"

	
	Address Presentation Restricted Indicator (APRI)
	Depends on priv-value in Privacy header.

	
	Screening indicator 
	Network Provided

	Addr-spec


	"CC" "NDC" "SN" from the URI


	Address signal
	if NOA is "national (significant) number" then set to "NDC" + "SN".
If NOA is "international number"
then set to "CC"+"NDC"+"SN".

	Privacy header field is not present
	
	APRI
	Presentation allowed

	Privacy header field
	priv-value
	APRI
	"Address Presentation Restricted Indicator"

	priv-value


	"header"
	APRI 
	Presentation restricted

	
	"user"
	APRI
	Presentation restricted

	
	"none" 
	APRI
	Presentation allowed

	
	"id" 
	APRI
	Presentation restricted

	NOTE:
It is possible that a P-Asserted-Identity header field includes both a tel URI and a SIP URI. In this case, either the tel URI or the SIP URI with user="phone" and a specific host portion, as selected by operator policy, may be used.


7.4.3
Direct Dialling In (DDI)

A direct dialling in call is a basic call and no additional treatment is required by the MGCF.

7.4.4
Malicious call identification

The actions of the MGCF at the ISUP/BICC side are described in ITU-T Recommendation Q.731.7 [42] under the clause "Interactions with other networks".

7.4.5
Subaddressing (SUB)

7.4.5.1
General

The ISDN Subaddress in ISUP is transported within the Access Transport Parameter. The Coding of the Subaddress parameter within the Access Transport Parameter is described within ETSI EN 300 403‑1 [96]. The isdn-subaddress parameter carried within a tel or sip URI is defined within RFC 3966 [97]. The isdn-subaddress encoding type carried within a tel or sip URI is defined within RFC 4715 [108].
7.4.5.2
Interworking at I-MGCF

The mapping in table 24ba of the isdn-subaddress parameter received within a tel or sip URI to the ISUP Access Transport Parameter encapsulating the Subaddress shall be applied.

The mapping in table 24bb of the Subaddress received within an ANM Message containing the ISUP Access Transport Parameter to the isdn-subaddress of a tel or sip URI to be sent within a 200 OK (INVITE) shall be applied.
Table 24ba: Mapping of the Subaddress received in an initial INVITE to the Subaddress sent in the IAM

	SIP Message INVITE
	ISUP Message IAM

	Source SIP header field and component
	Source component value
	ISUP Parameter field
	Derived value of parameter field

	To header field including the isdn-subaddress
(NOTE)
	"isub=" 1*uric
 
"uric" containing the Subaddress digits
	isub-encoding not present
	Access Transport Parameter



	called party Subaddress 
	Type of Subaddress = "NSAP" (000)

	
	
	"isub-encoding=nsap-ia5"
	
	
	

	
	
	"isub-encoding=nsap-bcd"
	
	
	Type of Subaddress = "NSAP" (000)

	
	
	"isub-encoding=nsap"
	
	
	Type of Subaddress = "NSAP" (000)

	
	"isub=" 1*uric ("uric" containing the Subaddress digits) and isub-encoding does not contain nsap value
	No mapping

	P-Asserted-Identity header Field

including the isdn-subaddress
	";isub=" 1*uric

"uric" containing the Subaddress digits
	isub-encoding not present
	Access Transport Parameter



	calling party Subaddress


	Type of Subaddress = "NSAP" (000)

	
	
	"isub-encoding=nsap-ia5"
	
	
	

	
	
	"isub-encoding=nsap-bcd"
	
	
	Type of Subaddress = "NSAP" (000)

	
	
	"isub-encoding=nsap"
	
	
	Type of Subaddress = "NSAP" (000)

	
	"isub=" 1*uric ("uric" containing the Subaddress digits) and isub-encoding does not contain nsap value
	No mapping

	NOTE:
As an operator option, an isdn-subaddress within the Request-URI may also be mapped into the ISUP Access Transport parameter.


Table 24bb: Mapping of the Subaddress received in an ANM to the Subaddress sent in the 200 OK (INVITE)

	ISUP Message ANM
	SIP Message 200 (OK)

	ISUP Parameter field
	Source component value 
	Source SIP header field and component
	Derived value of parameter field

	Access Transport Parameter
	connected Subaddress and Type of Subaddress = "NSAP" (000)
	P-Asserted-Identity including the isdn-subaddress
	";isub=" 1*uric and "isub-encoding=nsap-ia5"
The Subaddress digits included into the "uric" shall be derived from the Access Transport Parameter

	
	connected Subaddress

and Type of Subaddress ≠ "NSAP" (000)
	No mapping


7.4.5.3
Interworking at O-MGCF

The mapping in table 24bc of the subaddress information received in the Access Transport Parameter of the IAM message to the isdn-subaddress parameters of a tel or SIP URI to be sent within the initial INVITE request shall be applied.

The mapping in table 24bd of the isdn-subaddress information received within a tel or SIP URI of a 200 OK (INVITE) to the subaddress information of the ISUP Access Transport Parameter to be sent within the ANM message shall be applied.
Table 24bc: Mapping of the Subaddress received in an IAM to the Subaddress sent in the INVITE

	ISUP IAM Message 
	SIP INVITE Message

	ISUP Parameter field
	Source component value 
	Source SIP header field and component
	Derived value of parameter field

	Access Transport Parameter




	called party Subaddress and Type of Subaddress = "NSAP" (000)
	To header field including the isdn-subaddress, and, as an operator option, Request URI 
	";isub=" 1*uric and "isub-encoding=nsap-ia5"
The Subaddress digits included into the "uric" shall be derived from the Access Transport Parameter

	
	called party Subaddress and Type of Subaddress ≠ "NSAP" (000)
	No mapping

	
	calling party Subaddress and Type of Subaddress = "NSAP" (000)
	P-Asserted-Identity header field including the isdn-subaddress 
	";isub=" 1*uric and "isub-encoding=nsap-ia5"
The Subaddress digits included into the "uric" shall be derived from the Access Transport Parameter

	
	calling party Subaddress and Type of Subaddress ≠ "NSAP" (000)
	No mapping


Table 24bd: Mapping of the Subaddress received in a 200OK to the Subaddress sent in the ANM
	SIP Message 200 (OK)
	ISUP Message ANM

	Source SIP header field and component
	Source component value
	ISUP Parameter field
	Derived value of parameter field

	P-Asserted-Identity header Field

including the isdn-subaddress
	"isub=" 1*uric


"uric" containing the Subaddress digits
	isub-encoding not present
	Access Transport Parameter 
	connected Subaddress


	Type of Subaddress = "NSAP" (000)

	
	
	"isub-encoding=nsap-ia5"
	
	
	

	
	
	"isub-encoding=nsap-bcd"
	
	
	Type of Subaddress = "NSAP" (000)

	
	
	"isub-encoding=nsap"
	
	
	Type of Subaddress = "NSAP" (000)

	
	"isub=" 1*uric ("uric" containing the Subaddress digits) and isub-encoding does not contain nsap value
	No mapping


7.4.6
Call Forwarding Busy (CFB)/ Call Forwarding No Reply (CFNR) / Call Forwarding Unconditional (CFU) / Call Deflection (CD)
7.4.6.1
General

A MGCF within an IMS network applying the Communication Diversion service uses the procedures defined in clause 7.5.4.
This clause describes additional interworking of call forwarding service information between a PSTN/PLMN network and an IMS network. This can also be used when interworking between SIP networks (e.g., IMS network interworking with enterprise networks making use of the History-Info header field with an escaped Reason header field as described in IETF RFC 4244 [91]). The procedures support the interworking of the diversion reason within the History-Info header field using an escaped Reason header field defined by IETF RFC 3326 [116] as described in IETF RFC 4244 [91] and also support the diversion cause using the "cause" URI parameter as described by IETF RFC 4458 [113].

If the MGCF is supporting the interworking of Call Forwarding and also applying the Communication Diversion services as defined by TS 24.604 [60], it uses both IETF RFC 4244 [91] and IETF RFC 4458 [113] to signal the diversion reason. An IMS network supporting the interworking of Call Forwarding and not applying the IMS Communication Diversion supplementary service according to TS 24.604 [60] uses IETF RFC 4244 [91] and can also use IETF RFC 4458 [113] based on network option.

When interworking the SIP History-Info header field to ISUP, and both the diversion reason based on IETF RFC 4244 [91] and IETF RFC 4458 [113] are present for the same diversion instance, the MGCF should use the "cause" URI parameter per IETF RFC 4458 [113] for deriving the ISUP. When the "cause" URI parameter per IETF RFC 4458 [113] is used, the corresponding interworking as described in clause 7.5.4 shall be applied for that diversion instance. Otherwise, the interworking as defined in the present clause should be applied.

When interworking from ISUP to the History-Info header field, to signal the diversion reason the MGCF should use the escaped Reason header field from IETF RFC 4244 [91] and apply the interworking procedures in the present subclause. The MGCF should also apply the interworking procedures described in clause 7.5.4 to use the "cause" URI parameter  from IETF RFC 4458 [113] to signal the diversion reason.
In the event that the interworking procedures described in this clause as well as the interworking procedures in clause 7.5.4 are not applied, the actions of the MGCF at the ISUP/BICC side as described in ITU-T Recommendation Q.732.2-5 [42] under the clause "Interactions with networks not providing any call diversion information" may be applied as a network option.
7.4.6.2
Interworking at the O-MGCF

7.4.6.2.1
General

This clause describes the optional mapping of Call Forwarding information at the O-MGCF to the protocol-cause specified in IETF RFC 3326 [116].

7.4.6.2.2
Interworking SIP to ISUP

Table 7.4.6.2.2.1: Mapping of SIP messages to ISUP messages

	(Message sent to ISUP
	(Message Received from SIP
	

	ACM indicating call forwarding
	181 (Call Is Being Forwarded) response
	See table 7.4.6.2.2.6

	CPG indicating call forwarding (see NOTE)
	181 (Call Is Being Forwarded) response
	See table 7.4.6.2.2.7

	ACM indicating ringing
	180 (Ringing) response
	See table 7.4.6.2.2.8

	CPG indicating alerting (see NOTE)
	180 (Ringing) response
	See table 7.4.6.2.2.9

	ANM
	200 (OK) response
	See table 7.4.6.2.2.10

	CON
	200 (OK) response (Neither a 181 (Call Is Being Forwarded) response nor a 180 (Ringing) response was received)
	See table 7.4.6.2.2.10

	NOTE:
A CPG will be sent if an ACM was already sent.


Table 7.4.6.2.2.2: Mapping of History-Info header field to ISUP Redirection number

	Source SIP header field and component
	Source Component value
	Redirection number
	Derived value of parameter field

	hi-targeted-to-uri of the hi-entry following the last hi-entry containing a Reason "headers" component as defined in IETF RFC 4244 [91] with cause value as listed in table 7.4.6.2.2.4.
Appropriate global number portion of the hi-targeted-to-uri, assumed to be in form 
"+" CC + NDC + SN.  (NOTE)
	CC
	Nature of address indicator
	If CC is equal to the country code of the country where O‑MGCF is located AND the next ISUP node is located in the same country, then set to "national (significant) number" else set to "international number".

	
	CC, NDC, SN
	Address signals
	If NOA is "national (significant) number" then set to
NDC + SN.

If NOA is "international number"
then set to CC + NDC + SN.

	NOTE:
If the SIP URI doesn't contain "user=phone", mapping to redirection number is impossible, therefore no need to generate Redirection number and Redirection number restriction parameter (per table 7.4.6.2.2.3), Notification subscription options can't be set as "presentation allowed with redirection number".


Table 7.4.6.2.2.3: Mapping of History-Info header field to ISUP Redirection number restriction

	Source SIP header field and component
	Source Component value
	Redirection number restriction 
	Derived value of parameter field

	Privacy header field, or/and Privacy "headers" component of the hi-entry following the last hi-entry containing a Reason "headers" component as defined in IETF RFC 4244 [91] with cause value as listed in table 7.4.6.2.2.4
	"history" or "session" or "header"
	Presentation restricted indicator
	"Presentation restricted"

	
	Privacy "headers" component of the hi-targeted-to-uri and Privacy header field absent or "none"
	
	"Presentation allowed" or absent


Table 7.4.6.2.2.4: Mapping of hi-targeted-to-uri to ISUP Call Diversion Information

	Source SIP header field and component
	Source Component value
	Call Diversion Information
	Derived value of parameter field

	Privacy "headers" component of the hi-targeted-to-uri or/and Privacy header field
	
	Notification subscription options
	If the priv-value "history" or "session" or "header" is received within the Privacy header field or the priv-value "history" is received within the "headers" component of the hi-targeted-to-uri representing the diverting URI(s) and within the hi-targeted-to-uri representing diverted-to URI then "presentation not allowed" shall be set.
Otherwise, if the priv-value "history" is received only within the "headers" component of the hi-targeted-to-uri representing the diverted-to URI then "presentation allowed without redirection number" shall be set. 
(NOTE 1, NOTE 2)
Otherwise, "presentation allowed with redirection number" shall be set.

	hi-targeted-to-uri; Reason "headers" component as defined in IETF RFC 4244 [91] with cause parameter
	Cause parameter value
	Redirecting reason
	

	
	302
	
	Deflection immediate response

	
	486
	
	User busy 

	
	408
	
	No reply

	
	503
	
	Mobile subscriber not reachable 

	
	all other cause values
	
	Unknown

	NOTE 1:
diverting URI corresponds to the hi-targeted-to-uri of the last hi-entry containing a Reason "headers" component with cause parameter.

NOTE 2:
diverted-to URI corresponds to the hi-targeted-to-uri of the hi-entry following the last hi-entry containing a Reason "headers" component with cause parameter and is mapped to the Redirection number, see table 7.4.6.2.2.2.


Table 7.4.6.2.2.5: Void
Table 7.4.6.2.2.6: Mapping of 181 (Call Is Being Forwarded) ( ACM when ACM was not previously sent
	Source SIP header field and component
	Source Component value
	ISUP Parameter


	Derived value of parameter field

	181 (Call Is Being Forwarded)
	
	ACM
	

	
	
	Generic notification indicator
Notification indicator
	Call is diverting

	History-Info header field
	See table 7.4.6.2.2.2 
	Redirection number
	See table 7.4.6.2.2.2 

	Privacy "headers" component of the hi-targeted-to-uri or/and Privacy header field
	See table 7.4.6.2.2.3 
	Redirection number restriction 
	See table 7.4.6.2.2.3



	Privacy "headers" component of the hi-targeted-to-uri or/and Privacy header field
	See table 7.4.6.2.2.4 
	Call diversion information Notification subscription options
	See table 7.4.6.2.2.4



	hi-targeted-to-uri; Reason "headers" component as defined in IETF RFC 4244 [91] with cause parameter
	See table 7.4.6.2.2.4
	Call diversion information 

Redirecting reason
	See table 7.4.6.2.2.4


Table 7.4.6.2.2.7: Mapping of 181 (Call Is Being Forwarded) ( CPG if ACM was already sent

	Source SIP header field and component
	Source Component value
	ISUP Parameter


	Derived value of parameter field

	181 (Call Is Being Forwarded) response
	
	CPG
	

	
	
	Generic notification indicator
Notification indicator
	Call is diverting

	hi-targeted-to-uri; Reason "headers" component as defined in IETF RFC 4244 [91] with cause parameter
	486
	Event information

Event indicator
	CFB (national use)

	
	408 (see NOTE)
	
	CFNR (national use)

	
	all other values, or if appropriate "national use" value (CFB, CFNR or CFU) is not used in a network, or if no hi-targeted-to-uri "cause" URI parameter is contained in the SIP 181 (Call Is Being Forwarded) response.
	
	Progress 

	History-Info header field
	See table 7.4.6.2.2.2 
	Redirection number
	See table 7.4.6.2.2.2 

	Privacy "headers" component of the hi-targeted-to-uri or/and Privacy header field
	See table 7.4.6.2.2.3


	Redirection number restriction 
	See table 7.4.6.2.2.3



	Privacy "headers" component of the hi-targeted-to-uri or/and Privacy header field
	See table 7.4.6.2.2.4 
	Call diversion information Notification subscription options
	See table 7.4.6.2.2.4 

	hi-targeted-to-uri; Reason "headers" component as defined in IETF RFC 4244 [91] with cause parameter
	See table 7.4.6.2.2.4


	Call diversion information Redirecting reason
	See table 7.4.6.2.2.4

	NOTE:
This appears in the cases of CFNR.


Table 7.4.6.2.2.8: Mapping of 180 (Ringing) ( ACM when ACM was not previously sent
	Source SIP header field and component
	Source Component value
	ISUP Parameter


	Derived value of parameter field

	180 (Ringing) response
	
	ACM
	

	History-Info header field
	If hi-targeted-to-uri of at least one History-Info hi-entry contains a Reason "headers" component as defined in IETF RFC 4244 [91].
	Generic notification indicator
Notification indicator
	Call is diverting

	History-Info header field
	See table 7.4.6.2.2.2
	Redirection number (NOTE)
	See table 7.4.6.2.2.2

	Privacy "headers" component of the hi-targeted-to-uri or/and Privacy header field
	See table 7.4.6.2.2.3
	Redirection number restriction (NOTE)
	See table 7.4.6.2.2.3

	Privacy "headers" component of the hi-targeted-to-uri or/and Privacy header field
	See table 7.4.6.2.2.4
	Call diversion information Notification subscription options (NOTE)
	See table 7.4.6.2.2.4

	hi-targeted-to-uri; Reason "headers" component as defined in IETF RFC 4244 [91] with cause parameter
	See table 7.4.6.2.2.4
	Call diversion information Redirecting reason (NOTE)
	See table 7.4.6.2.2.4

	NOTE:
Parameter shall only be supplied if hi-targeted-to-uri contains a Reason "headers" component, as defined in IETF RFC 4244 [91] with cause value as listed in table 7.4.6.2.2.4.


The mapping described within table 7.4.6.2.2. 8 can only appear if the communication has already undergone a Communications diversion in the IMS and the 180 (Ringing) is the first provisional response sent in backward direction.

The O-MGCF can indicate the call diversion information in the mapping of 180 (Ringing) provisional response to a CPG message in fact if the response before was a 181 (Call Is Being Forwarded).

Table 7.4.6.2.2.9: Mapping of 180 (Ringing) ( CPG if ACM was already sent
	Source SIP header field and component
	Source Component value
	ISUP Parameter


	Derived value of parameter field

	180 (Ringing) response
	
	CPG
	

	
	
	Generic notification indicator
Notification indicator
	Call is diverting

	History-Info header field
	
	Event information

Event indicator
	ALERTING

 

	History-Info header field
	See table 7.4.6.2.2.2
	Redirection number (NOTE)
	See table 7.4.6.2.2.2

	Privacy "headers" component of the hi-targeted-to-uri or/and Privacy header field
	See table 7.4.6.2.2.3
	Redirection number restriction (NOTE)
	See table 7.4.6.2.2.3

	Privacy "headers" component of the hi-targeted-to-uri or/and Privacy header field
	See table 7.4.6.2.2.4
	Call diversion information Notification subscription options (NOTE)
	See table 7.4.6.2.2.4

	hi-targeted-to-uri; Reason "headers" component as defined in IETF RFC 4244 [91] with cause parameter
	See table 7.4.6.2.2.4
	Call diversion information Redirecting reason (NOTE)
	See table 7.4.6.2.2.4

	NOTE:
Parameter shall only be supplied if hi-targeted-to-uri contains a Reason "headers" component, as defined in IETF RFC 4244 [91] with cause value as listed in table 7.4.6.2.2.4.


The mapping in table 7.4.6.2.2.9 appears when a 181 (Call Is Being Forwarded) provisional response previously was mapped to an ACM. Therefore the state machine of the O-MGCF knows that a CDIV is in progress.

Table 7.4.6.2.2.10: Mapping of 200 (OK) response

	Source SIP header field and component
	Source Component value
	ISUP Parameter


	Derived value of parameter field

	200 (OK) response 
	
	ANM/CON (NOTE)
	

	History-Info header field
	See table 7.4.6.2.2.2
	Redirection number
	See table 7.4.6.2.2.2

	Privacy "headers" component of the hi-targeted-to-uri or/and Privacy header field
	See table 7.4.6.2.2.3
	Redirection number restriction
	See table 7.4.6.2.2.3



	NOTE:
Redirection number shall only be supplied if 200 (OK) response is mapped to ANM message.


7.4.6.2.3
Interworking ISUP to SIP

For the interworking of 180 (Ringing) response and 200 (OK) response (to the INVITE request) to the regarding ISUP messages and parameters no additional procedures beyond the basic call procedures are needed.

To interwork the redirecting number at the O-MGCF it can be needed to create placeholder History-Info hi-entries. Such a History-Info hi-entry has to provide a hi-targeted-to-uri with a placeholder value "unknown@unknown.invalid", a Reason "headers" component with a cause parameter and a hi-index as described within table 7.4.6.2.3.1.

Table 7.4.6.2.3.1: Mapping of IAM to SIP INVITE request

	ISUP Parameter or IE 
	Derived value of parameter field
	SIP component


	Value

	IAM
	
	INVITE request
	

	Redirecting number
	
	History-Info header field
	IF Redirection counter exceeds 1


hi-targeted-to-uri of the

penultimate created

hi-entry (NOTE 8)


IF Redirecting number is


available



set to the value of the



Redirecting number


ELSE



set to the



placeholder value



"unknown



@unknown.invalid"

ELSE


no mapping (NOTE 7) 

	Nature of address indicator:
	"national (significant) number"

	hi-targeted-to-uri 
	Add CC (of the country where the MGCF is located) to Redirecting number Address Signals to construct E.164 number in URI.

	
	"international number"
	
	Map complete Redirecting number Address Signals to E.164 number in URI.

	Address Signals
	If NOA is "national (significant) number" then the format of the Address Signals is:

NDC + SN

If NOA is "international number"
then the format of the Address Signals is:

CC + NDC + SN 
	hi-targeted-to-uri 
	Addr-spec

"+" CC NDC SN mapped to userinfo portion of SIP URI.

(NOTE 5)
Add "user=phone".

	Redirecting number
	APRI
	Privacy "headers" component of  the penultimate hi-targeted-to-uri entry in the History-Info header
	Priv-value

	
	"presentation restricted"
	
	"history"

	
	"presentation allowed"
	
	Privacy header field absent or "none" (NOTE 3)

	Redirection Information
	Redirecting indicator
	Privacy "headers" component of the penultimate hi-targeted-to-uri entry in the History-Info header
	Priv-value

	
	Call diverted
	
	"none" (NOTE 4)

	
	Call diverted, all redirection information presentation restricted
	
	"history"

	Redirection Information
	Redirection counter

1
	hi-index
	Number of diversions is shown to the number of levels in hi-index.
Index for Original called number = 1

Index for Called party number = 1.1



	
	2
	
	Index for Original called number = 1

Index for Redirecting number = 1.1

Index for Called party number = 1.1.1



	
	N
	
	Index for Original called number = 1

Placeholder History-Info hi-entry with Index = 1.1

…

Fill up

…

Index for Redirecting number = 1.[(N-1)*".1"]

Index for Called party number= 1.N* ".1" (e.g. N=3 ( 1.1.1.1)

	Redirection Information
	Redirecting Reason and

Original Redirection Reason

(NOTE 1)
	hi-targeted-to-uri; Reason "headers" component as defined in IETF RFC 4244 [91] with cause parameter.

For a placeholder History-Info hi-entry the value "404" shall be taken (NOTE 2).

Cause parameter for redirecting reason will be put in the entry of redirecting number, and cause parameter for original redirection reason will be put in the entry of original called number. 
	Cause parameter value

	
	unknown/not available
	
	404

	
	unconditional
	
	302

	
	User Busy 
	
	486

	
	No reply
	
	408

	
	Deflection during alerting
	
	302

	
	Deflection immediate response 
	
	302

	
	Mobile subscriber not reachable
	
	503



	Called Party Number
	See Redirecting number 
Nature of address indicator and Address signal
	History-Info header field see hi-targeted-to-uri
	URI of the last hi-targeted-to-uri entry of History-Info header field
(NOTE 6)

	Original called number
	See Redirecting number 
Nature of address indicator and Address signal
	History-Info header field see hi-targeted-to-uri 
	URI of first hi-targeted-to-uri entry of History-Info header field
 (NOTE 5, NOTE 8)
IF the Original called number is available:


set to the value of the


Original called number

ELSE


IF the Redirection counter


equals 1 AND the


Redirecting number is


available,



set to the value of



the Redirecting



number


ELSE



set to the



placeholder value



"unknown



@unknown.invalid"

	Original called number
	APRI
	Privacy "headers" component  of the first hi-targeted-to-uri entry of History-Info header 
	Priv-value

	
	"presentation restricted"
	
	"history" 

	
	"presentation allowed"
	
	"none"

	NOTE 1:
Original Redirection Reason contains only the "unknown/not available " parameter

NOTE 2:
For all History-Info hi-entries except the last one a cause parameter in Reason "headers" component as defined in IETF RFC 4244 [91] has to be included.
NOTE 3:
If the Redirecting indicator has the value "Call diverted, all redirection information presentation restricted", the privacy value "history" shall be set.

NOTE 4:
If the redirecting number APRI has the value "presentation restricted", the privacy value "history" shall be set.
NOTE 5:
Used URI scheme shall be SIP URI. The Reason "headers" component with a cause parameter cannot be added if hi-targeted-to-uri is a tel URI.

NOTE 6:
The used URI scheme can be tel or SIP since the last hi-targeted-to-uri entry of the History-Info header field does not contain the Reason "headers" component with a cause parameter.
NOTE 7:
If the Original called number parameter is not available and if the value of the Redirecting counter is equal to one, the Redirecting number parameter is used for the first hi-targeted-to-uri entry of the History-Info header field.

NOTE 8:
If a further SIP to ISUP interworking occurs, parameters not present in the original message can then be included.


7.4.6.3
Interworking at the I-MGCF

7.4.6.3.1
General

This clause describes the interworking of the Call Forwarding information at the I-MGCF.

7.4.6.3.2
Interworking from SIP to ISUP

Table 7.4.6.3.2.1: Mapping of SIP to ISUP messages

	
(Message received from SIP
	(Message send to BICC/ISUP 

	INVITE request
	IAM


Table 7.4.6.3.2.2: Mapping of History-Info header field to ISUP Redirecting number

	Source SIP header field and component
	Source Component value
	Redirecting number
	Derived value of parameter field

	latest History-Info header field entry containing a Reason "headers" component as defined in IETF RFC 4244 [91] with cause parameter value as listed in the 
cause parameter row in 
table 7.4.6.2.2.4 (Note 1)
	
	Redirecting number
	

	hi-targeted-to-uri

appropriate global number portion of the URI, assumed to be in form 
"+" CC + NDC + SN
	CC
	Nature of address indicator
	If CC is equal to the country code of the country where MGCF is located AND the next ISUP node is located in the same country, then set to "national (significant) number" else set to "international number"

	
	CC, NDC, SN
	Address signals
	If NOA is "national (significant) number" then set to
NDC + SN.

If NOA is "international number"
then set to CC + NDC + SN

	Privacy header field or/and Privacy "headers" component of the hi-entry
in History-Info header field as specified in this table (NOTE 2)
	
	APRI
	If the priv-value "history" or "session" or "header" is received within the Privacy header field or if the priv-value "history" is received within the "headers" component of the hi-targeted-to-uri in the latest hi-entry containing a Reason "headers" component as defined in IETF RFC 4244 [91] with cause parameter then "presentation restricted" shall be set.
Otherwise, "presentation allowed" shall be set.

	
	
	
	

	NOTE 1:
If the SIP URI doesn't contain "user=phone", mapping to redirecting number is impossible, therefore no need to generate Redirecting number.
NOTE 2:
It is possible that an entry of the History-Info header field itself is marked as restricted or the whole History-Info header.


Table 7.4.6.3.2.3: Mapping of History-Info header to ISUP Redirection Information

	Source SIP header field and component
	Source Component value
	Redirection Information
	Derived value of parameter field

	Privacy header field, and in History-Info header field in the Privacy "headers" component of the last hi-targeted-to-uri containing a Reason "headers" component as defined in IETF RFC 4244 [91] with cause parameter value as listed in the cause parameter row in this table 
	"history" or "session" or "header"
for the Privacy SIP header or for the hi-targeted-to-uri entry
	Redirecting indicator
	Call diverted, all redirection information presentation restricted

	
	Privacy header field and the privacy component of the hi-targeted-to-uri entry either absent

or

set to "none"

	
	Call diverted

	
	
	Original redirection reason
	Unknown/not available

	Cause parameter in the last hi-targeted-to-uri containing a Reason "headers" component as defined in IETF RFC 4244 [91] 
	Cause parameter value
	Redirecting Reason
	

	
	302
	
	Deflection immediate response

	
	486
	
	User busy

	
	408
	
	No reply

	
	503
	
	Mobile subscriber not reachable

	
	All other values
	
	unknown

	Hi-index
	
	Redirection counter
	number of History-Info hi-entry containing a Reason "headers" component with a cause parameter with values as listed in the cause parameter  row in this table (NOTE)

	NOTE:
If the determined number of redirection in SIP exceeds the ISUP maximum parameter value, the MGCF shall set the Redirection counter to its maximum value. For instance, in ISUP ITU-T Q.763 [4], the Redirection counter parameter cannot exceed 5.


Table 7.4.6.3.2.4: Mapping of History-Info header field to ISUP Original Called number

	Source SIP header field and component
	Source Component value
	Original called number
	Derived value of parameter field

	
	
	Numbering Plan Indicator
	"ISDN (Telephony) numbering plan (Recommendation E.164)"

	hi-targeted-to-uri of 1st hi-targeted-to-uri containing a Reason "headers" component as defined in IETF RFC 4244 [91] with cause parameter;

appropriate global number portion of the URI, assumed to be in form 
"+" CC + NDC + SN
 (NOTE 1)
	CC
	Nature of address indicator
	If CC is equal to the country code of the country where MGCF is located AND the next ISUP node is located in the same country, then set to "national (significant) number" else set to "international number"

	
	CC, NDC, SN
	Address signals
	If NOA is "national (significant) number" then set to
NDC + SN.

If NOA is "international number"
then set to CC + NDC + SN

	Privacy "headers" component

in History-Info header field of the History-Info header field entry as defined above in this table (NOTE 2)
	"history" or "session" or "header"
	APRI
	"presentation restricted"

	
	Privacy header field absent or "none"
	
	"presentation allowed"

	NOTE 1:
If it is SIP URI and doesn't contain "user=phone", mapping to Original Called number is impossible, therefore no need to generate Original Called number.
NOTE 2:
It is possible that an entry of the History-Info header field itself is marked as restricted or the whole History-Info header.


Table 7.4.6.3.2.5: Mapping of INVITE to IAM

	INVITE
	
	IAM
	

	History-Info header field
	See table 7.4.6.3.2.2
	Redirecting number 
	See table 7.4.6.3.2.2

	History-Info header field
	See table 7.4.6.3.2.3
	Redirection Information
	See table 7.4.6.3.2.3

	History-Info header field 
	See table 7.4.6.3.2.4


	Original Called Number
	See table 7.4.6.3.2.4




7.4.6.3.3
Interworking from ISUP to SIP

Table 7.4.6.3.3.1: Mapping of ISUP to SIP Messages

	(Message sent to SIP
	(Message Received from BICC/ISUP
	

	181 (Call Is Being Forwarded)
	ACM no indication with Redirection number and Call diversion information parameters (CFU, CFB, CDi)
	See table 7.4.6.3.3.3

	180 (Ringing)
	ACM indicating ringing, OBCI: "Call diversion may occur" (CFNR, CDa)
	See clause 7.2.3.1.4

	181 (Call Is Being Forwarded)
	CPG indicating progress or subsequent diversion indicated in the CPG with Redirection number and Call diversion information parameters (CFNR, CDa)
	See table 7.4.6.3.3.4

	180 (Ringing)
	CPG indicating ringing and Redirection number restriction parameter
	See table 7.4.6.3.3.5

	200 (OK)
	ANM and Redirection number restriction parameter
	See table 7.4.6.3.3.6


Table 7.4.6.3.3.2: Mapping of ISUP Redirection Number Restriction to History-Info header field

	Redirection Number Restriction 
	Derived value of parameter field
	SIP component


	Value

	Presentation restricted indicator
	"Presentation restricted"
	Privacy "headers" component of the hi-targeted-to-uri
	"History"

	
	"Presentation allowed" or absent AND a previously received notification subscription options was NOT "presentation not allowed" OR was NOT "presentation allowed without redirection number"
	
	Privacy header field absent

or

"none"


Table 7.4.6.3.3.3: Mapping of ACM ( 181 (Call Is Being Forwarded) response

	ISUP Parameter 
	Derived value of parameter field
	SIP component


	Value

	Generic notification indicator
Notification indicator
	Call is diverting
	
	

	
	
	1st hi-entry of History-info header having an index = 1 (NOTE 1)
	Placeholder URI "unknown@unknown.invalid"

	Redirection number
	
	2nd hi-entry of History-Info header field having an index 1.1
	hi-targeted-to-uri:

	Nature of address indicator:
	"national (significant) number"

	hi-targeted-to-uri
	Add CC (of the country where the MGCF is located) to Redirection number Address Signals to construct E.164 number in URI.

	
	"international number"
	
	Map complete Redirection number Address Signals to E.164 number in URI.

	Address Signals
	If NOA is "national (significant) number" then the format of the Address Signals is:

NDC + SN

If NOA is "international number"
then the format of the Address Signals is:

CC + NDC + SN
	hi-targeted-to-uri
	Addr-spec

"+" CC NDC SN mapped to userinfo portion of SIP URI 
(NOTE 3)
Add "user=phone".


	Call diversion information
	Redirecting Reason
	Reason "headers" component as defined in IETF RFC 4244 [91] with cause parameter in the penultimate hi-entry
(NOTE 2)
	Cause parameter value

	
	Unknown
	
	404

	
	Unconditional
	
	302

	
	User busy
	
	486

	
	No reply
	
	408

	
	Deflection immediate response 
	
	302

	
	Deflection during alerting
	
	302

	
	Mobile subscriber not reachable
	
	503

	
	Notification subscription options
	Privacy "headers" component associated with Redirection number hi-targeted-to-uri
 (NOTE 2)
	

	
	unknown
	
	Escaped Privacy value is set according to the rules of TS 24.604 [60] clause 4.5.2.6.4 item c

	
	presentation not allowed
	
	A 181 (Call Is Being Forwarded) shall not be sent

	
	presentation allowed with redirection number
	
	Escaped Privacy value is set according to the rules of TS 24.604 [60] clause 4.5.2.6.4 item c

	
	presentation allowed without redirection number
	
	Escaped Privacy value is set according to the rules of TS 24.604 [60] clause 4.5.2.6.4 item c

	NOTE 1:
It is necessary to create two History-Info header entries to carry both Redirection number and Call diversion information. Since the original called number is not available from the ISUP message a placeholder URI is included in the first entry. Only two entries are provided because the number of diversions is not available.

NOTE 2:
Needs to be stored for a possible inclusion into subsequent messages.


Table 7.4.6.3.3.4: Mapping of CPG ( 181 (Call Is Being Forwarded) response

	ISUP Parameter
	Derived value of parameter field
	SIP component


	Value

	Event information
Event Indicator
	Progress
	
	

	Generic notification indicator
Notification indicator
	Call is diverting
	
	

	
	
	1st hi-entry of History-Info header having an index =1 (NOTE 1)
	Placeholder URI "unknown@unknown.invalid"

	Redirection number
	
	2nd hi-entry of History-Info header field having an index = 1.1
	hi-targeted-to-uri:

	Nature of address indicator
	"national (significant) number"

	hi-targeted-to-uri
	Add CC (of the country where the MGCF is located) to Redirection number Address Signals to construct E.164 number in URI.

	
	"international number"
	hi-targeted-to-uri
	Map complete Redirection number Address Signals to E.164 number in URI.

	Address Signals
	If NOA is "national (significant) number" then the format of the Address Signals is:

NDC + SN

If NOA is "international number"
then the format of the Address Signals is:

CC + NDC + SN
	hi-targeted-to-uri
	Addr-spec

"+" CC NDC SN mapped to userinfo portion of SIP URI 
(NOTE 3)
Add "user=phone".

	Call diversion information
	Redirecting Reason
	Reason "headers" component as defined in IETF RFC 4244 [91] with cause parameter in the penultimate hi-entry
 (NOTE 2)
	Cause parameter value

	
	Unknown
	
	404

	
	Unconditional
	
	302 

	
	User busy
	
	486

	
	No reply
	
	408

	
	Deflection immediate response 
	
	302

	
	Deflection during alerting
	
	302

	
	Mobile subscriber not reachable
	
	503

	
	Notification subscription options
	Privacy "headers" component associated with Redirection number hi-targeted-to-uri (NOTE 2)
	

	
	unknown
	
	Escaped Privacy value is set according to the rules of TS 24.604 [60] clause 4.5.2.6.4 item c

	
	presentation not allowed
	
	A 181 (Call Is Being Forwarded) shall not be sent

	
	presentation allowed with redirection number
	
	Escaped Privacy value is set according to the rules of TS 24.604 [60] clause 4.5.2.6.4 item c

	
	presentation allowed without redirection number
	
	Escaped Privacy value is set according to the rules of TS 24.604 [60] clause 4.5.2.6.4 item c

	NOTE 1:
It is necessary to create two History-Info header entries to carry both Redirection number and Call diversion information. Since the original called number is not available from the ISUP message a placeholder URI is included in the first entry. Only two entries are provided because the number of diversions is not available.

NOTE 2:
Needs to be stored for a possible inclusion into subsequent messages.


Table 7.4.6.3.3.5 addresses two separate conditions: the CPG message is received from the diverting exchange in which case the Call diversion information parameter is included; and the CPG message is received from the diverted-to exchange in which case the Call diversion information parameter is not included. Interworking for both conditions is shown.
Table 7.4.6.3.3.5: Mapping of CPG ( 180 (Ringing) response

	ISUP Parameter 
	Derived value of parameter field
	SIP component


	Value

	Event information
Event Indicator
	Alerting
	
	

	
	
	1st hi-entry of History-Info header having an index = 1
(NOTE 1)
	Placeholder URI "unknown@unknown.invalid"

	Redirection number 
	
	2nd hi-entry of History-Info header field having an index = 1.1
	See table 7.4.6.3.3.3

	Call diversion information
	Redirecting Reason
	Reason "headers" component as defined in IETF RFC 4244 [91] with cause parameter in the penultimate hi-entry
 (NOTE 2)
	Cause parameter value

	
	Unknown
	
	404

	
	Unconditional
	
	302 

	
	User busy
	
	486

	
	No reply
	
	408

	
	Deflection immediate response 
	
	302

	
	Deflection during alerting
	
	302

	
	Mobile subscriber not reachable
	
	503

	
	Notification subscription options
	Privacy "headers" component associated with Redirection number hi-targeted-to-uri (NOTE 2)
	

	
	unknown
	
	Escaped Privacy value is set according to the rules of TS 24.604 [60] clause 4.5.2.6.4 item c

	
	presentation not allowed
	
	The 180 (Ringing) response shall be sent without the the History-Info header field included

	
	presentation allowed with redirection number
	
	Escaped Privacy value is set according to the rules of TS 24.604 [60] clause 4.5.2.6.4 item c

	
	presentation allowed without redirection number
	
	Escaped Privacy value is set according to the rules of TS 24.604 [60] clause 4.5.2.6.4 item c

	If no Call diversion information parameter is present
	
	Reason "headers" component as defined in IETF RFC 4244 [91] with cause parameter in the penultimate hi-entry
	Value stored from a previous received ACM or CPG. See tables 7.4.6.3.3.3 and 7.4.6.3.3.4

	
	
	Privacy "headers" component associated with Redirection number hi-targeted-to-uri
	Value stored from a previous received ACM or CPG. See tables 7.4.6.3.3.3 and 7.4.6.3.3.4

	Redirection number restriction
	
	
	See table 7.4.6.3.3.2

	NOTE 1:
It is necessary to create two History-Info header entries to carry both Redirection number and Redirecting reason. Since the original called number is not available from the ISUP message a placeholder URI is included in the first entry. Only two entries are provided because the number of diversions is not available.
NOTE 2:
Needs to be stored for a possible inclusion into subsequent messages.


Table 7.4.6.3.3.6: Mapping of ANM ( 200 (OK) response (to INVITE request)

	ISUP Parameter
	Derived value of parameter field
	SIP component


	Value

	
	
	1st hi-entry of History-Info header having an index = 1 (NOTE)
	Placeholder URI "unknown@unknown.invalid"

	Redirection number 
	
	2nd hi-entry of History-Info header field having an index = 1.1
	See table 7.4.6.3.3.3

	
	
	Reason "headers" component as defined in IETF RFC 4244 [91] with cause parameter in the penultimate hi-entry
	Value stored from a previously received ACM or CPG. See tables 7.4.6.3.3.3 and 7.4.6.3.3.4

	Redirection number restriction
	
	
	See table 7.4.6.3.3.2

	NOTE:
It is necessary to create two hi-entries of History-Info header field to carry both Redirection number and Redirecting reason. Since the original called number is not available from the ISUP message a placeholder URI is included in the first entry. Only two entries are provided because the number of diversions is not available.


7.4.7
Void
7.4.8
Explicit Call Transfer (ECT)

When the MGCF receives a FAC message with Generic notification indicator coded as "Call transfer active" or "call transfer alerting" and a CPG with Generic notification indicator coded as "Remote hold" was received previously for the current communication, the action described in table 24be applies. In all other cases the actions of the MGCF at the ISUP/BICC side are described in ITU-T Recommendation Q.732.7 [42] under the clause "Interactions with other networks".

Table 24be: Mapping between ISUP and SIP for the Explicit Communication Transfer supplementary service

	ISUP message
	Mapping

	FAC with a "call transfer, active" or "call transfer, alerting" Generic notification indicator
	As described for CPG message with a "remote retrieval" Generic notification indicator in clause 7.4.10.2


7.4.9
Call Waiting

The actions of the MGCF at the ISUP/BICC side are described in ITU-T Q.733.1 [42] under the clause "Interactions with other networks".

7.4.10
Call Hold

The service is interworked as indicated in TS 23.228 [12].

7.4.10.1
Session hold initiated from the IM CN subsystem side

The IMS network makes a hold request by sending an UPDATE or re-INVITE message with an "inactive" or a "sendonly" SDP attribute (refer to RFC 3264 [36]), depending on the current state of the session. Upon receipt of the hold request from the IMS side, the MGCF shall send a CPG message to the CS side with a "remote hold" Generic notification indicator. To resume the session, the IMS side sends an UPDATE or re-INVITE message with a "recvonly" or "sendrecv" SDP attribute, depending on the current state of the session. Upon receipt of the resume request from the IMS side, the MGCF shall send a CPG message to the CS side with a "remove retrieval" Generic notification indicator. However, the I-MGCF shall not send a CPG message upon reception of SDP containing "inactive" media within an initial INVITE request establishing a new SIP dialogue and upon reception of the first subsequent SDP activating those media.

The user plane interworking of the hold/resume request is described in the clause 9.2.9.

[image: image53.emf]MGCF

2. 

BICC/ISUP

:  CPG  (Hold) 

3. 

SIP

:  200  OK  [SDP]

1 . 

SIP

:  UPDATE  [SDP,  a=sendonly/

inactive]

4. 

SIP

:  UPDATE  [SDP,  a=sendrecv/

recvonly]

5. 

BICC/ISUP

:  CPG  (Retrieve)

6. 

SIP

:  200  OK  [SDP]

 


Figure 30a: Session hold/resume initiated from the IM CN subsystem side

7.4.10.2
Session hold initiated from the CS network side

If an MGCF receives a CPG message with "remote hold" and there is no dialog established towards the UE the MGCF shall send an UPDATE or re-INVITE request containing an SDP offer with "sendonly" or "inactive" media, as described in IETF RFC 3264 [36], when the first dialog is established. For an early dialog only UPDATE shall be used.
When an MGCF receives a CPG message with a "remote hold" Generic notification indicator and the media on the IMS side are "sendrecv" or "recvonly", the MGCF shall forward the hold request by sending an UPDATE request on the early dialog which was last established containing an SDP offer with "sendonly" if the stream was previously "sendrecv" or "inactive" if the stream was previously "recvonly" media, as described in IETF RFC 3264 [36].
If an additional early dialog is established during the "remote hold" condition the MGCF shall send an UPDATE request containing an SDP offer with "sendonly" or "inactive" media on the new early dialog, as described in IETF RFC 3264 [36].
If an UPDATE request with an SDP offer is received on one of the early dialogs for a call in the "remote hold" condition the MGCF shall send an appropriate SDP answer followed by a new UPDATE request including SDP with "sendonly" or "inactive" media on the dialog , as described in IETF RFC 3264 [36].
If an MGCF receives a 200 OK (INVITE) response on an early dialog for which the call is in a "remote hold" condition the MGCF shall send an UPDATE or re-INVITE request containing an SDP offer with "sendonly" or "inactive" media on the dialog where 200 OK (INVITE) was received, as described in IETF RFC 3264 [36].

If the MGCF receives a CPG with Generic Notification Indicator "remote retrieval" and there is an early dialog on IMS side then a SIP UPDATE request (indicating call retrieval) shall be sent if the call hold service had been invoked on the early dialog before. For each subsequent early dialog for which the MGCF receives an 18x response or an UPDATE request with an SDP offer, the MGCF shall send SIP UPDATE indicating call retrieval after a possible SDP answer to the SDP offer, if that dialog had received a call hold indication before.

If the MGCF receives a CPG with Generic Notification Indicator "remote retrieval" and there is a confirmed dialog on IMS side then a SIP re-INVITE or UPDATE request (according to implementation option) shall be sent for this dialog only if the call hold service had been invoked for this dialog before.

When an MGCF receives a CPG message with a "remote retrieval" Generic notification indicator and the media on the IMS side are "sendonly" or "inactive", the MGCF shall forward the resume request by sending an UPDATE or re-INVITE message containing an SDP offer with "sendrecv" if the stream was previously "sendonly" or "recvonly" if the stream was previously "inactive" media, as described in IETF RFC 3264 [36].

If the MGCF receives a CPG with "remote hold" or "remote retrieval" before answer, it shall forward the request using an UPDATE message. If the MGCF receives a CPG with "remote hold" or "remote retrieval" after answer, it should forward the request using re-INVITE but may use UPDATE.

If link aliveness information is required at the IM-MGW while the media are on hold, the O-MGCF should provide modified SDP RR and RS bandwidth modifiers specified in IETF RFC 3556 [59] within the UPDATE or re-INVITE messages holding and retrieving the media to temporarily enable RTCP while the media are on hold, as detailed in clause 7.4 of TS 26.236 [32]. If no link aliveness information is required at the IM-MGW, the O-MGCF should provide the SDP RR and RS bandwidth modifiers previously used.

The interworking does not impact the user plane, unless the MGCF provides modified SDP RR and RS bandwidth modifiers within the UPDATE or re-INVITE messages. If the MGCF provides modified SDP RR and RS bandwidth modifiers to the IMS side, the MGCF shall also provide modified SDP RR and RS bandwidths to the IM-MGW, as described in the clause 9.2.10.

[image: image54.emf]MGCF

1 . 

BICC/ISUP

:  CPG  (Hold) 

3. 

SIP

:  200  OK  [SDP]

2. 

SIP

:  UPDATE  [SDP,  a=sendonly/

inactive]

5. 

SIP

:  UPDATE  [SDP,  a=sendrecv/

recvonly]

4. 

BICC/ISUP

:  CPG  (Retrieve)

6. 

SIP

:  200  OK  [SDP]

 


Figure 30b: Session hold/resume initiated from the CS network side

7.4.11
Call Completion on busy subscriber

The actions of the MGCF at the ISUP/BICC side are described in ITU-T Recommendation Q.733.3 [42] under the clause "Interactions with other networks".

7.4.12
Completion of Calls on No Reply (CCNR)

The actions of the MGCF at the ISUP/BICC side are described in ITU-T Recommendation Q.733.5 [42] under the clause "Interactions with other networks".

7.4.13
Terminal Portability (TP)

Terminal Portability is defined as an ISUP supplementary service within ITU-T Rec. Q.733.4. [42].
A Suspend message containing the Suspend/Resume indicators set to "ISDN subscriber initiated" shall be treated like a CPG with "remote hold" in Section 7.4.10 Resume message containing the Suspend/Resume indicators set to "ISDN subscriber initiated" shall be treated like a CPG with "remote retrieval" in Section 7.4.10.
7.4.14
Conference calling (CONF) / Three-Party Service (3PTY)
The default behaviour of the MGCF at the ISUP/BICC side is described in ITU-T Recommendation Q.734.1[42] under the clause "Interactions with other networks". In addition, the MGCF may apply the interworking from ISUP to SIP described in table 24aa.
Alternatively, the MGCF may apply the interworking to the Conference supplementary service described in clause 7.5.6.

Table 24aa: Mapping between ISUP and SIP for the Conference Calling (CONF) and Three-Party Service (3PTY) supplementary service

	ISUP message
	Mapping

	CPG with a "Conference established" Generic notification indicator
	As described for CPG message with a "remote retrieval" Generic notification indicator in clause 7.4.10.2

	CPG with a "Conference disconnected" Generic notification indicator
	As described for CPG message with a "remote retrieval" Generic notification indicator in clause 7.4.10.2

	CPG with an "isolated" Generic notification indicator
	As described for CPG message with a "remote hold" Generic notification indicator in clause 7.4.10.2

	CPG with a "reattached" Generic notification indicator
	As described for CPG message with a "remote retrieval" Generic notification indicator in clause 7.4.10.2


7.4.15
Void
7.4.16
Closed User Group (CUG)

The actions of the MGCF at the ISUP/BICC side are described in ITU-T Recommendation Q.735.1[42] under the clause 1.5.2.4.2 "Exceptional procedures".

7.4.17
Multi-Level Precedence and Pre-emption (MLPP)

The actions of the MGCF at the ISUP/BICC side are described in ITU-T Recommendation Q.735.3 [42] under the clause "Interactions with other networks".

7.4.18
Global Virtual Network Service (GVNS)

The actions of the MGCF at the ISUP/BICC side are described in ITU-T Recommendation Q.735.6 [42] under the clause "Interactions with other networks".

7.4.19
International telecommunication charge card (ITCC)

An International Telecommunication charge card call is a basic call and no additional treatment is required by the MGCF.

7.4.20
Reverse charging (REV)

The actions of the MGCF at the ISUP/BICC side are described in ITU-T Recommendation Q.736.3 [42] under the clause "Interactions with other networks".

7.4.21
User-to-User Signalling (UUS)

7.4.21.0
General

Procedures for the ISDN supplementary service "User-to-user signalling" are specified in ITU T Recommendation Q.737.1 [42].
7.4.21.1
User-to-User Signalling (UUS) service 1 (implicit)

7.4.21.1.0
General
The coding of the User-user information element is described within ITU-T Recommendation Q.931 [136]. The User-user information element is carried within the ISDN user part parameter user-to-user information, as defined in ITU-T Recommendation Q.763 [4]. The User-to-User header field is defined within IETF RFC 7433 [99]. A package for interworking user-to-user information with the ISDN is defined by IETF RFC 7434 [99A].
7.4.21.1.1
Void
7.4.21.1.2
User-to-user information Interworking from SIP to ISUP

On the receipt of a header field with the "purpose" header field parameter set to "isdn-uui", or a User-to-User header field without a "purpose" parameter, with "encoding" header field parameter set to "hex" or without an "encoding" parameter, with "content" header field parameter set to "isdn-uui" or without a "content" parameter, that is valid as defined by IETF RFC 7434 [99A]. the MGCF shall map the content of the "uui-data" field to the "protocol discriminator" and "user information" parameters of the User-user information element.

The "length of user-user contents" parameter shall be set by the MGCF according to the normal procedures.

The MGCF maps the messages transporting the user-to-user information according to the normal interworking procedures (see table 24ab).

Table 24ab: Mapping of the User-to-User header field to the ISUP user-to-user information parameter

	SIP parameter (
	( ISUP parameter

	SIP header field 
	Source component value
	ISUP parameter name
	ISUP parameter field

	User-to-User 
	uui-data
	User-to-user
	Protocol discriminator and user information 


7.4.21.1.3
User-to-user information Interworking from ISUP to SIP

On the receipt of the user-user information parameter the MGCF shall map the protocol discriminator and user information parameter fields to the uui-data field of the User-to-User header field (see table 24ac).

If sent, the "purpose", "content" and "encoding" header field parameters are not mapped and are set in accordance with IETF RFC 7434 [99A].
The MGCF maps the messages transporting the user-to-user information parameters according to the normal interworking procedures.

Table 24ac: Mapping of the ISUP user-to-user information parameter to the User-to-User header field
	( ISUP parameter
	( SIP parameter 

	ISUP parameter name
	ISUP parameter field
	SIP header field
	Source component value

	User-to-user
	Protocol discriminator and user information
	User-to-User
	uui-data (NOTE)

	NOTE:
The MGCF shall always send uui-data as a token (see IETF RFC 7433 [99]). The letters used for the hex digits shall always be capital form.


7.4.21.2
User-to-User Signalling (UUS) service 1 (explicit)

The actions of the MGCF at the ISUP/BICC side are described in ITU-T Recommendation Q.737.1 [42] under the clause "Interaction with other networks".

7.4.21.3
User-to-User Signalling (UUS) service 2 (explicit)

The actions of the MGCF at the ISUP/BICC side are described in ITU-T Recommendation Q.737.1 [42] under the clause "Interaction with other networks".

7.4.21.4
User-to-User Signalling (UUS) service 3 (explicit)

The actions of the MGCF at the ISUP/BICC side are described in ITU-T Recommendation Q.737.1 [42] under the clause "Interaction with other networks".
7.4.22
Multiple Subscriber Number (MSN)

A MSN call is a basic call and no additional treatment is required by the MGCF.

7.4.23
Anonymous Call rejection

This section describes the interworking of the ETSI ACR service as described ETSI EN 300 356-21 [71].

7.4.23.1
ISUP-SIP protocol interworking at the I-MGCF

If ISUP Cause Value field in the ISUP REL includes Cause Value 24 "call rejected due to ACR supplementary service" the I-MGCF shall map this to a 433 (Anonymity Disallowed) as described in RFC 5079 [77].
7.4.23.2
SIP-ISUP protocol interworking at the O-MGCF
If the response is a 433 (Anonymity Disallowed) response, then this response shall be mapped to the ISUP Cause Value field 24 "call rejected due to ACR supplementary service" in the ISUP REL.
7.4.24
Customized Alerting Tones (CAT) in the 3GPP CS domain

The Customized Alerting Tones (CAT) in the 3GPP CS domain service is described in TS 23.205 [27].

To pass CAT early media from the CS network towards the IM CN subsystem, no special interworking procedures beyond basic call at the I-MGCF are required. The procedure to supply the "P-Early-Media" header field is described in clause 7.2.3.1.4, clause 7.2.3.1.4A and clause 7.2.3.1.4B.
7.5
IMS Supplementary Services
The following clauses describe the MGCF behaviour related to supplementary services as defined in ITU-T Recommendations Q.730 to ITU-T Q.737 [42] when interworking with an IMS which uses a Multimedia Telephony Application Server (MTAS) providing supplementary services according to TS 24.173 [88]. The support of the related procedures is optional.
7.5.1
Originating Identification Presentation (OIP) and Originating Identification Restriction (OIR)

The mapping of Originating Identification Presentation (OIP) and Originating Identification Restriction (OIR); supplementary service with the CLIP/CLIR PSTN/ISDN Supplementary Service is the same mapping as described in clause 7.4.1. The Service itself is described within TS 24.607 [63].
7.5.2
Terminating Identification Presentation (TIP) and Terminating Identification Restriction (TIR)

7.5.2.1
General

The protocol specification of the Terminating Identification Presentation and Terminating Identification Restriction supplementary services is described in TS 24.608 [64].
The procedures for mapping of the connected number described within clause 7.4.2 shall apply.
7.5.2.2
Interworking at the O-MGCF

For the mapping of IAM to the INVITE request:

If an Optional forward call indicators parameter in the IAM is received where the bit H Connected line identity request indicator is set to "requested", then the option tag "from-change" shall be add to the Supported header field. See table 7.5.2.2.1.

Table 7.5.2.2.1: Mapping of ISUP IAM to SIP INVITE request

	ISUP Parameter
	Derived value of parameter field
	Source SIP header field and component
	Source Component value

	Optional forward call indicator
	Connected line identity request indicator is set to "requested"
	Supported
	"from-change"


NOTE:
The presence of "from-change" enables the reception of Generic Number with Number qualifier parameter field set to additional connected, if available. As per TS 24.608 [64] the presence of "from-change" tag is not a criterion for TIP service.
If a provisional or final response including the option tag "from-change" is received, then the O-MGCF shall:

-
if a 200 (OK) response to the INVITE request is received, start timer TTIR1; and

-
store the 200 (OK) response, without interworking it.

Otherwise the 200 (OK) response (to the INVITE request) shall be mapped as described in clause 7.2.3.2.7a.

If an UPDATE request is received containing a changed From header field before the timer TTIR1 expired, then the O‑MGCF shall:

-
stop timer TTIR1;
-
map the From header field received in the UPDATE request to the Generic number in the ANM as shown in table 7.5.2.2.2 and table 7.5.2.2.3;

-
if the UPDATE request includes a P-Asserted-Identity header field that is different from the one within the stored 200 (OK) response, the latest received P-Asserted-Identity header field shall be mapped to the connected number as described table 24; and

-
map the parameters needed to be mapped of the stored 200 (OK) response to an ANM as described in clause 7.4.2.2.3, modified by the changed mapping steps of the From and P-Asserted-Identity header fields.

When TTIR1 expires, then the stored 200 (OK) response (to the INVITE request) response shall be mapped as described in clause 7.4.2.2.3.

Table 7.5.2.2.2: Mapping of SIP UPDATE request to ISUP ANM/CON

	ANM/CON
	UPDATE

	Generic number
	From header field


Table 7.5.2.2.3: Mapping of SIP From header field to ISUP Generic Number 
("additional connected number") parameter
	Source SIP header field and component
	Source component value
	Generic Number parameter field
	Derived value of parameter field

	–
	–
	Number Qualifier Indicator
	"additional connected number"

	From, userinfo component of URI assumed to be in form 
"+" CC + NDC + SN
	CC
	Nature of Address Indicator
	If CC is equal to the country code of the country where I‑MGCF is located AND the next ISUP node is located in the same country, then set to "national (significant) number" else set to "international number"

	–
	–
	Number Incomplete Indicator
	"complete"

	–
	–
	Numbering Plan Indicator
	"ISDN (Telephony) numbering plan (Recommendation E.164)"

	Privacy, priv‑value component
	Privacy header field absent
	Address Presentation Restricted Indicator (APRI)
	"presentation allowed"

	
	"none"
	
	"presentation allowed"

	
	"header"
	
	"presentation allowed"

	
	"user"
	
	"presentation restricted"

	
	"id"
	
	"presentation allowed"

	–
	–
	Screening Indicator
	"user provided, not verified"

	From, userinfo component assumed to be in form 
"+" CC + NDC + SN
	CC, NDC, SN
	Address Signals
	If NOA is "national (significant) number" then set to

NDC + SN.

If NOA is "international number"
then set to CC + NDC + SN


7.5.2.3
Interworking at the I-MGCF

For the mapping of INVITE request to IAM:

-
the bit H Connected line identity request indicator of the Optional forward call indicators parameter in the IAM shall be set to "requested".

Table 7.5.2.3.1: Void
If a received ISUP ANM includes an ISUP Generic Number ("additional connected number") parameter, then the 
I-MGCF shall send a 200 (OK) response (to the INVITE request) including an option tag "from-change" If the initial INVITE was received and the Supported header field contains the "from-change" tag, the 200 (OK) response is followed by an UPDATE request, containing the "additional connected number" copied into the From header field as shown in table 7.5.2.3.2.

The To header field of the UPDATE request is derived from the P-Asserted-Identity header field received within the initial INVITE request.

Table 7.5.2.3.2: Mapping of ANM Generic Number ("additional connected number") 
to SIP From header field in a SIP UPDATE request
	ISUP Parameter/field
	Value
	SIP component
	Value

	Generic Number

Number Qualifier Indicator
	"additional connected number"
	From header field
	display‑name (optional) and addr‑spec

	Nature of Address Indicator
	"national (significant) number" 
	Addr‑spec
	Add "+" CC (of the country where the IWU is located) to Generic Number Address Signals then map to user portion of URI scheme used

	
	"international number" 
	
	Map complete Generic Number Address Signals used prefixed with a "+" to user portion of URI scheme used

	Address Presentation restriction indicator (APRI)
	"presentation allowed"
	
	No Privacy header fieldor not "user"

	
	"presentation restricted"
	
	"user"

	Address Signals
	if NOA is "national (significant) number" then the format of the address signals is:

NDC + SN

If NOA is "international number"
then the format of the address signals is:

CC + NDC + SN
	Display‑name (optional)
	display‑name shall be mapped from Address Signals, if network policy allows it

	
	
	Addr‑spec 
	"+" CC NDC SN mapped to user portion of URI scheme used 


A received connected number in an ANM shall be mapped to the P-Asserted-Identity header field as shown in table 21 of the UPDATE request.
7.5.2.4
Timer

Table 7.5.2.4.1 TIR timer definition

	Symbol
	Timeout
value
	Cause for initiation
	Normal termination
	At expiry

	TTIR1
	100 – 2000 milliseconds (default 100 milliseconds)
	On receipt of provisional or final response including the option tag "from-change"
	At the receipt of an UPDATE
	map the received 200OK to an ANM 


7.5.3
void
7.5.4
Communication Diversion (CDIV)
7.5.4.1
General

The protocol specification of the Communication Diversion supplementary service is described in TS 24.604 [60]. The mapping of Communication Diversion supplementary service with Call Diversion services PSTN/ISDN supplementary service including the mapping of the optional History-Info header field as defined in IETF RFC 4244 [91] is described.

In case of interworking with networks which do not provide any notification of the communication diversion or communication redirection information (e.g. redirection counter) in the signalling system, the communication continues according to the basic call procedures.

7.5.4.2
Interworking at the O-MGCF

7.5.4.2.1
General

For the mapping of IAM to the INVITE request no additional procedures beyond the basic call and interworking procedures are needed unless Call forwarding within the ISUP Network appeared.
With regard to the backward messages the following mapping is valid.

Table 7.5.4.2.1.1: Mapping of SIP messages to ISUP messages

	(Message sent to ISUP
	(Message Received from SIP
	

	ACM indicating call forwarding
	181 (Call Is Being Forwarded) response
	See table 7.5.4.2.1.6

	CPG indicating call forwarding (see NOTE)
	181 (Call Is Being Forwarded) response
	See table 7.5.4.2.1.7

	ACM indicating ringing
	180 (Ringing) response
	See table 7.5.4.2.1.8

	CPG indicating Alerting (see NOTE)
	180 (Ringing) response
	See table 7.5.4.2.1.9

	ANM
	200 (OK) response
	See table 7.5.4.2.1.10

	CON
	200 (OK) response (Neither a 181 (Call Is Being Forwarded) response nor a 180 (Ringing) response was received)
	See table 7.5.4.2.1.10

	NOTE:
A CPG will be sent if an ACM was already sent.


Table 7.5.4.2.1.2: Mapping of History-Info header field to ISUP Redirection number

	Source SIP header field and component
	Source Component value
	Redirection number
	Derived value of parameter field

	hi-targeted-to-uri of the last History-Info hi-entry containing a "cause" URI parameter, as defined in IETF RFC 4458 [113].
The global number portion of the hi-targeted-to-uri is assumed to be in form 
"+" CC + NDC + SN.(NOTE)
	CC
	Nature of address indicator
	If CC is equal to the country code of the country where O‑MGCF is located AND the next ISUP node is located in the same country, then set to "national (significant) number" else set to "international number".

	
	CC, NDC, SN
	Address signals
	If NOA is "national (significant) number" then set to NDC + SN.

If NOA is "international number"
then set to CC + NDC + SN.

	NOTE:
If the SIP URI doesn't contain "user=phone", mapping to redirection number is impossible, therefore no need to generate Redirection number and Redirection number restriction indicator (per table 7.5.4.2.1.3), Notification subscription options can't be set as "presentation allowed with redirection number".


Table 7.5.4.2.1.3: Mapping of History-Info header field to ISUP Redirection number restriction

	Source SIP header field and component
	Source Component value
	Redirection number restriction 
	Derived value of parameter field

	Privacy "headers" component of the hi-targeted-to-uri or/and Privacy header field
	"history" or "session" or "header"
	Presentation restricted indicator
	"Presentation restricted"

	
	Privacy "headers" component of the hi-targeted-to-uri and Privacy header field absent

or "none"
	
	"Presentation allowed" or absent


Table 7.5.4.2.1.4: Mapping of hi-targeted-to-uri to ISUP Call Diversion Information

	Source SIP header field and component
	Source Component value
	Call Diversion Information
	Derived value of parameter field

	Privacy "headers" component of the hi-targeted-to-uri or/and Privacy header field
	
	Notification subscription options
	If the priv-value "history" or "session" or "header" is received within the Privacy header field or the priv-value "history" is received within the "headers" component of the hi-targeted-to-uri representing the diverting URI(s) and within the hi-targeted-to-uri representing diverted-to URI then "presentation not allowed" shall be set.
Otherwise, if the priv-value "history" is received only within the "headers" component of the hi-targeted-to-uri representing the diverted-to URI then "presentation allowed without redirection number" shall be set. (NOTE 1, NOTE 2)

Otherwise, "presentation allowed with redirection number" shall be set.

	"cause" URI parameter, as defined in IETF RFC 4458 [113] of the last History-Info hi-entry containing hi-targeted-to-uri with "cause" URI parameter. 
	cause value
	Call diversion information 
	Redirecting Reason

	
	404
	
	Unknown 

	
	302
	
	Unconditional

	
	486
	
	User busy 

	
	408
	
	No reply

	
	480
	
	Deflection immediate 

	
	503
	
	Mobile subscriber not reachable 

	
	487
	
	Deflection during alerting

	NOTE 1:
diverting URI corresponds to the hi-targeted-to-uri of the hi-entry before the last hi-entry containing "cause" URI parameter.

NOTE 2:
diverted-to URI corresponds to the hi-targeted-to-uri of the last hi-entry containing "cause" URI parameter and is mapped to the Redirection number, see table 7.5.4.2.1.2.


Table 7.5.4.2.1.5: Void
Table 7.5.4.2.1.6: Mapping of 181 (Call Is Being Forwarded) ( ACM if no ACM was sent before
	Source SIP header field and component
	Source Component value
	ISUP Parameter


	Derived value of parameter field

	181 (Call Is Being Forwarded)
	
	ACM
	

	
	
	Generic notification indicators
	Call is diverting

	History-Info header field
	See table 7.5.4.2.1.2 
	Redirection number
	See table 7.5.4.2.1.2 

	Privacy "headers" component of the hi-targeted-to-uri or/and Privacy header field
	See table 7.5.4.2.1.3 
	Redirection number restriction 
	See table 7.5.4.2.1.3



	Privacy "headers" component of the hi-targeted-to-uri or/and Privacy header field
	See table 7.5.4.2.1.4 
	Call diversion information Notification subscription options
	See table 7.5.4.2.1.4



	hi-targeted-to-uri;"cause" URI parameter as defined in IETF RFC 4458 [113] of the last History-Info hi-entry containing such "cause" URI parameter.
	See table 7.5.4.2.1.4
	Call diversion information 
	Redirecting Reason

See table 7.5.4.2.1.4


Table 7.5.4.2.1.7: Mapping of 181 (Call Is Being Forwarded)( CPG if ACM was already sent

	Source SIP header field and component
	Source Component value
	ISUP Parameter


	Derived value of parameter field

	181 (Call Is Being Forwarded) response
	
	CPG
	

	
	
	Generic notification indicators
	Call is diverting

	"cause" URI parameter, as defined in IETF RFC 4458 [113] of the last History-Info hi-entry containing hi-targeted-to-uri withsuch "cause" URI parameter.
	486
	Event indicator
	CFB (national use)

	
	408 (see NOTE)
	
	CFNR (national use)

	
	302
	
	CFU (national use)

	
	Any other value, or if appropriate national use value CFB, CFNR or CFU is not used in a network. or if no agreement exists between operators to use theses values, or if no hi-targeted-to-uri with "cause" URI parameter is contained in the SIP 181.
	
	PROGRESS 

	History-Info header field
	See table 7.5.4.2.1.2 
	Redirection number
	See table 7.5.4.2.1.2 

	Privacy "headers" component of the hi-targeted-to-uri or/and Privacy header field
	See table 7.5.4.2.1.3


	Redirection number restriction 
	See table 7.5.4.2.1.3



	Privacy "headers" component of the hi-targeted-to-uri or/and Privacy header field
	See table 7.5.4.2.1.4 
	Call diversion information Notification subscription options
	See table 7.5.4.2.1.4 

	hi-targeted-to-uri; "cause" URI parameter, as defined in IETF RFC 4458 [113] of the last History-Info hi-entry containing such "cause" URI parameter.
	See table 7.5.4.2.1.4


	Call diversion information Redirecting Reason
	See table 7.5.4.2.1.4

	NOTE:
This appears in the cases of CFNR.


Table 7.5.4.2.1.8: Mapping of 180 (Ringing) ( ACM if no ACM was sent before
	Source SIP header field and component
	Source Component value
	ISUP Parameter


	Derived value of parameter field

	180 (Ringing) response
	
	ACM
	

	History-Info header field
	If hi-targeted-to-uri of at least one History-Info hi-entry contains a "cause" URI parameter, as defined in IETF RFC 4458 [113].
	Generic notification indicators
	Call is diverting

	History-Info header field
	See table 7.5.4.2.1.2
	Redirection number (NOTE)
	See table 7.5.4.2.1.2

	Privacy "headers" component of the hi-targeted-to-uri or/and Privacy header field
	See table 7.5.4.2.1.3
	Redirection number restriction (NOTE)
	See table 7.5.4.2.1.3

	Privacy "headers" component of the hi-targeted-to-uri or/and Privacy header field
	See table 7.5.4.2.1.4
	Call diversion information Notification subscription options (NOTE)
	See table 7.5.4.2.1.4

	hi-targeted-to-uri; "cause" URI parameter, as defined in IETF RFC 4458 [113] of the last History-Info hi-entry containing such "cause" URI parameter.
	See table 7.5.4.2.1.4
	Call diversion information Redirecting Reason (NOTE)
	See table 7.5.4.2.1.4

	NOTE:
Parameter shall only be supplied if hi-targeted-to-uri of at least one History-Info hi-entry contains a "cause" URI parameter, as defined in IETF RFC 4458 [113].


The mapping described within table 7.5.4.2.1.8 can only appear if the communication has already undergone a Call Forwarding in the ISDN/PSTN and the 180 is the first provisional response sent in backward direction.

The IWU can indicate the call diversion in the mapping of 180 (Ringing) to CPG in fact if the response before was a 181 (Call is being forwarded).

Table 7.5.4.2.1.9: Mapping of 180 (Ringing) ( CPG if ACM was already sent
	Source SIP header field and component
	Source Component value
	ISUP Parameter


	Derived value of parameter field

	180 (Ringing) response
	
	CPG
	

	History-Info header field
	If hi-targeted-to-uri of at least one History-Info hi-entry contains a "cause" URI parameter, as defined in IETF RFC 4458 [113].
	Generic notification indicators
	Call is diverting

	
	
	Event indicator
	ALERTING

 

	History-Info header field
	See table 7.5.4.2.1.2
	Redirection number
(NOTE)
	See table 7.5.4.2.1.2

	Privacy "headers" component of the hi-targeted-to-uri or/and Privacy header field
	See table 7.5.4.2.1.3
	Redirection number restriction (NOTE)
	See table 7.5.4.2.1.3

	Privacy "headers" component of the hi-targeted-to-uri or/and Privacy header field
	See table 7.5.4.2.1.4
	Call diversion information Notification subscription options (NOTE)
	See table 7.5.4.2.1.4

	hi-targeted-to-uri; "cause" URI parameter, as defined in IETF RFC 4458 [113] of the last History-Info hi-entry containing such "cause" URI parameter.
	See table 7.5.4.2.1.4
	Call diversion information Redirecting Reason (NOTE)
	See table 7.5.4.2.1.4

	NOTE:
Parameter shall only be supplied if hi-targeted-to-uri of at least one History-Info hi-entry contains a "cause" URI parameter, as defined in IETF RFC 4458 [113].


The mapping in table 7.5.4.2.1.9 appears when a 181 previously was mapped to an ACM. Therefore the state machine of the MGCF knows that a CDIV is in Progress.

Table 7.5.4.2.1.10: Mapping of 200 (OK) response

	Source SIP header field and component
	Source Component value
	ISUP Parameter


	Derived value of parameter field

	200 (OK) response 
	
	ANM/CON
	

	History-Info header field
	See table 7.5.4.2.1.2
	Redirection number (NOTE)
	See table 7.5.4.2.1.2

	Privacy "headers" component of the hi-targeted-to-uri or/and Privacy header field
	See table 7.5.4.2.1.3
	Redirection number restriction
	See table 7.5.4.2.1.3



	NOTE:
Redirection number shall only be supplied if 200 (OK) response is mapped to ANM message.


7.5.4.2.2
Call forwarding within the ISUP Network appeared

The following scenario shows if a Call Forwarding appears in the ISUP/PSTN and the diverted-to number is within the SIP network. Table 7.5.4.2.2.1 should be seen as an example.

For the mapping of 180 (Ringing) response and 200 (OK) response to the regarding ISUP messages and parameters no additional procedures beyond the basic call procedures are needed.

To interwork the redirecting number at the O-MGCF it can be needed to create placeholder History entries. Such a History entry has to provide a hi-targeted-to-uri with a placeholder value "unknown@unknown.invalid", a "cause" URI parameter and a hi-index as described within table 7.5.4.2.2.1.

Table 7.5.4.2.2.1: Mapping of IAM to SIP INVITE request

	ISUP Parameter or IE 
	Derived value of parameter field
	SIP component


	Value

	IAM
	
	INVITE request
	

	Redirecting number
	
	History-Info header field
	IF Redirection counter exceeds 1


hi-targeted-to-uri of the

penultimate created hi-entry

(NOTE 8)


IF Redirecting number is 
available



set to the value of the



Redirecting number


ELSE



set to the placeholder value



"unknown@unknown.invalid"

ELSE


no mapping (NOTE 7)

	Nature of address indicator:
	"national (significant) number"

	hi-targeted-to-uri 
	Add CC (of the country where the MGCF is located) to Redirecting number Address Signals to construct E.164 number in URI.

	
	"international number"
	
	Map complete Redirecting number Address Signals to E.164 number in URI.

	Address Signals
	If NOA is "national (significant) number" then the format of the Address Signals is: 

NDC + SN 

If NOA is "international number" 

then the format of the Address Signals is: 

CC + NDC + SN 
	hi-targeted-to-uri 
	Addr-spec

"+" CC NDC SN mapped to userinfo portion of SIP URI. (NOTE 5)

Add "user=phone".

	Redirecting number
	APRI
	Privacy header field that

corresponds to the penultimate hi-targeted-to-uri entry in the History-Info header
	Priv-value

	
	"presentation restricted"
	
	"history"

	
	"presentation allowed"
	
	Privacy header field absent or "none" (NOTE 3)

	Redirection Information
	Redirecting indicator
	Privacy header field that

corresponds to the penultimate hi-targeted-to-uri entry in the History-Info header
	Priv-value

	
	Call diverted
	
	"none" (NOTE 4)

	
	Call diverted, all redirection info presentation restricted
	
	"history"

	Redirection Information
	Redirection counter

1
	hi-index
	Number of diversions is shown due to the number of hi-index Entries

Index for original called number = 1

Address Signals (CdPN)

Number = 1.1

	
	2
	
	Index for original called number = 1

Index for Redirecting number

with Index = 1.1

Address Signals (CdPN)

Number = 1.1.1

	
	N
	
	Index for original called number = 1

Placeholder History entry with Index = 1.1

…

Fill up

…

Index for Redirecting Number

with = 1.[(N-1)*".1"]

Index for Address Signals (CdPN)

= 1.N* ".1" (e.g. N=3 ( 1.1.1.1)

	Redirection Information
	Redirecting Reason and

Original Redirection Reason 

(NOTE 1)
	hi-targeted-to-uri; "cause" URI parameter, as defined in IETF RFC 4458 [113].

The Redirecting Reason shall be mapped to the last hi-targeted-to-uri.

If the redirection counter is 2 or higher, the Original Redirection Reason shall be mapped to the second hi-targeted-to-uri.

If the redirection counter is 3 or higher, for each hi-targeted-to-uri following a placeholder History entry the value "404" shall be taken (NOTE 2)
	cause value

	
	unknown 
	
	404

	
	unconditional
	
	302 

	
	User Busy 
	
	486

	
	No reply
	
	408

	
	Deflection during alerting
	
	487

	
	Deflection immediate response 
	
	480

	
	Mobile subscriber not reachable
	
	503



	Called Party Number
	See Redirecting number
	History-Info header field see hi-targeted-to-uri
	URI of the last hi-targeted-to-uri entry of History-Info header field

	Original called number
	See Redirecting number
	History-Info header field see hi-targeted-to-uri 
	URI of first hi-targeted-to-uri entry of History-Info header field (NOTE 6, NOTE 8)
IF the Original called number is available:


set to the value of Original


called number

ELSE


IF the Redirection counter equals


1 AND the Redirecting number is


available,



set to the value of the



Redirecting number


ELSE



set to the placeholder value



"unknown@unknown.invalid"

	Original called number
	APRI
	Privacy header field of the first hi-targeted-to-uri entry of History-Info header 
	Priv-value

	
	"presentation restricted"
	
	"history" 

	
	"presentation allowed"
	
	"none"

	NOTE 1:
Original Redirection Reason contains only the "unknown" parameter

NOTE 2:
For all History entries except the first one a "cause" URI parameter as defined in IETF RFC 4458 [113] has to be included.

NOTE 3:
If the Redirecting Indicator has the value "Call diverted, all redirection info presentation restricted", the privacy value "history" shall be set.

NOTE 4:
If the Redirecting Number APRI has the value "presentation restricted", the privacy value "history" shall be set.

NOTE 5:
Used URI scheme shall be SIP URI. The "cause" URI parameter cannot be added if hi-targeted-to-uri is a tel URI.

NOTE 6:
The used URI scheme can be tel URI or SIP URI since the first hi-targeted-to-uri entry of the History-Info header field does not contain the "cause" URI parameter.
NOTE 7:
If the Original called number parameter is not available and if the value of the Redirecting counter is equal to one, the Redirecting number parameter is used for the first hi-targeted-to-uri entry of the History-Info header field.

NOTE 8:
If a further SIP to ISUP interworking occurs, parameters not present in the original message can then be included.


7.5.4.3
Interworking at the I-MGCF

Table 7.5.4.3.1: Mapping of SIP to ISUP messages

	
(Message received from SIP
	(Message send to BICC/ISUP 

	INVITE request
	IAM


Table 7.5.4.3.2: Mapping of History-Info header field to ISUP Redirecting number

	Source SIP header field and component
	Source Component value
	Redirecting number
	Derived value of parameter field

	In History-Info SIP header field, hi-targeted-to-uri in hi-entry before last hi-entry containing a "cause" URI parameter, as defined in IETF RFC 4458 [113].(NOTE 1)
	
	Redirecting number
	

	hi-targeted-to-uri

appropriate global number portion of the URI, assumed to be in form 
"+" CC + NDC + SN
	CC
	Nature of address indicator
	If CC is equal to the country code of the country where MGCF is located AND the next ISUP node is located in the same country, then set to "national (significant) number" else set to "international number"

	
	CC, NDC, SN
	Address signals
	If NOA is "national (significant) number" then set to
NDC + SN. 

If NOA is "international number" 

then set to CC + NDC + SN

	Privacy header field or/and priv‑value component of the hi-entry in History-Info header field specified in this table (NOTE 2)
	
	APRI
	If the priv-value "history" or "session" or "header" is received within the Privacy header field or if the priv-value "history" is received within the "headers" component of the hi-targeted-to-uri in hi-entry before last hi-entry containing a "cause" URI parameter then "presentation restricted" shall be set.
Otherwise, "presentation allowed" shall be set.

	NOTE 1:
If the SIP URI doesn't contain "user=phone", mapping to redirecting number is impossible, therefore no need to generate Redirecting number.
NOTE 2:
It is possible that an entry of the History-Info header field itself is marked as restricted or the whole History-Info header.


Table 7.5.4.3.3: Mapping of History-Info header to ISUP Redirection Information

	Source SIP header field and component
	Source Component value
	Redirection Information
	Derived value of parameter field

	Privacy header field and priv‑value of hi-entry before the last hi-entry containing a "cause" URI parameter as defined in IETF RFC 4458 [113] of the History-Info header field. 
	"history" or "session" or "header"
for the Privacy header field or for the hi-targeted-to-uri entry
	Redirecting indicator
	Call diverted, all redirection info presentation restricted

	
	Privacy header field and the privacy component of the hi-targeted-to-uri entry either absent

or

"none"

	
	Call diverted

	
	
	Original redirection reason
	Unknown

	Cause value in the last hi-targeted-to-uri containing a "cause" URI parameter as defined in IETF RFC 4458 [113] 
	cause value
	Redirecting Reason
	Redirecting Reason

	
	404
	
	Unknown/not available 

	
	302
	
	Unconditional

	
	486
	
	User busy

	
	408
	
	No reply

	
	480
	
	Deflection immediate response 

	
	487
	
	Deflection during alerting

	
	503
	
	Mobile subscriber not reachable

	Hi-index
	
	Redirection counter
	number of History entries containing a "cause" URI parameter with value as listed in the "cause" URI parameter row in this table (NOTE)

	NOTE:
If the determined number of redirection in SIP exceeds the ISUP maximum parameter value, the MGCF shall set the Redirection counter to its maximum value. For instance, in ISUP ITU-T Q.763 [4], the Redirection counter parameter cannot exceed 5.


Table 7.5.4.3.4: Mapping of History-Info header field to ISUP Original Called number

	Source SIP header field and component
	Source Component value
	Original called number
	Derived value of parameter field

	
	
	Numbering Plan Indicator
	"ISDN (Telephony) numbering plan (Recommendation E.164)"

	hi-targeted-to-uri of hi-entry preceding the 1st hi-targeted-to-uri containing a "cause" URI parameter, as defined in IETF RFC 4458 [113]; the

global number portion of the URI, is assumed to be in form 
"+" CC + NDC + SN

(NOTE 1)
	CC
	Nature of address indicator
	If CC is equal to the country code of the country where MGCF is located AND the next ISUP node is located in the same country, then set to "national (significant) number" else set to "international number"

	
	CC, NDC, SN
	Address signals
	If NOA is "national (significant) number" then set to
NDC + SN. 

If NOA is "international number" 

then set to CC + NDC + SN

	priv‑value component

in History-Info header field of the History-Info header field entry as defined above in this table (NOTE 2)
	"history" or "session" or "header"
	APRI
	"presentation restricted"

	
	Privacy header field absent or "none"
	
	"presentation allowed"

	NOTE 1:
If it is SIP URI and doesn't contain "user=phone", mapping to Original Called number is impossible, therefore no need to generate Original Called number 

NOTE 2:
It is possible that an entry of the History-Info header field itself is marked as restricted or the whole History-Info header.


Table 7.5.4.3.5: Mapping of INVITE to IAM

	INVITE
	
	IAM
	

	History-Info header field
	See table 7.5.4.3.2
	Redirecting number 
	See table 7.5.4.3.2

	History-Info header field
	See table 7.5.4.3.3
	Redirection Information
	See table 7.5.4.3.3

	History-Info header field 
	See table 7.5.4.3.4


	Original Called Number
	See table 7.5.4.3.4




Table 7.5.4.3.6: Mapping of ISUP to SIP Messages

	(Message sent to SIP
	(Message Received from BICC/ISUP
	

	181 (Being forwarded)
	ACM no indication with Redirection number and call diversion information (CFU, CFB, CDi)
	See table 7.5.4.3.8

	180 (Ringing)
	ACM indicating ringing, oBCi: Call diversion may occur (CFNR, CDa)
	See clause 7.2.3.1.4

	181 (Being forwarded)
	CPG indicating progress or subsequent diversion indicated in the CPG with Redirection number and call diversion information (CFNR, CDa)
	See table 7.5.4.3.9

	180 (Ringing)
	CPG indicating ringing and Redirection number restriction parameter
	See table 7.5.4.3.10

	200 (OK)
	ANM and Redirection number restriction parameter
	See table 7.5.4.3.11


Table 7.5.4.3.7: Mapping of ISUP Redirection Number Restriction to History-Info header field

	Redirection Number Restriction 
	Derived value of parameter field
	SIP component


	Value

	Presentation restricted indicator
	"Presentation restricted"
	privacy "headers" component of the hi-targeted-to-uri
	"History"

	
	"Presentation allowed" or absent AND any previous received notification subscription option was NOT "presentation not allowed" AND was NOT "presentation allowed without redirection number"
	
	Privacy header field absent

or

"none"


Table 7.5.4.3.8: Mapping of ACM ( 181 (Call Is Being Forwarded) response

	ISUP Parameter 
	Derived value of parameter field
	SIP component


	Value

	Generic notification indicators
	Call is diverting
	
	

	Redirection number
	
	History-Info header field with one hi-entry
	hi-targeted-to-uri:

	Nature of address indicator:
	"national (significant) number"

	hi-targeted-to-uri
	Add CC (of the country where the MGCF is located) to Redirection number Address Signals to construct E.164 number in URI.

	
	"international number"
	
	Map complete Redirection number Address Signals to E.164 number in URI.

	Address Signals
	If NOA is "national (significant) number" then the format of the Address Signals is: 

NDC + SN 

If NOA is "international number" 

then the format of the Address Signals is: 

CC + NDC + SN
	hi-targeted-to-uri
	Addr-spec

"+" CC NDC SN mapped to userinfo portion of SIP URI. (NOTE 2)

Add "user=phone".

	Call diversion information
	Redirecting Reason
	IETF RFC 4458 [113] "cause" URI parameter in the hi-targeted-to-uri (NOTE 1) 
	cause value

	
	Unknown/not available 
	
	404

	
	Unconditional
	
	302 

	
	User busy
	
	486

	
	No reply
	
	408

	
	Deflection immediate response 
	
	480

	
	Deflection during alerting
	
	487

	
	Mobile subscriber not reachable
	
	503

	
	Notification subscription option
	privacy "headers" component of the hi-targeted-to-uri (NOTE 1)
	Roles

	
	unknown
	
	Escaped Privacy value is set according to the rules of TS 24.604 [60] subclause 4.5.2.6.4 item c

	
	presentation not allowed
	
	A 181 Being Forwarded shall not be sent

	
	presentation allowed with redirection number
	
	Escaped Privacy value is set according to the rules of TS 24.604 [60] subclause 4.5.2.6.4 item c

	
	presentation allowed without redirection number
	
	Escaped Privacy value is set according to the rules of TS 24.604 [60] subclause 4.5.2.6.4 item c

	NOTE 1:
Needs to be stored for a possible inclusion into subsequent messages.

NOTE 2:
Used URI scheme shall be SIP URI. The "cause" URI parameter cannot be added if hi-targeted-to-uri is a tel URI.


Table 7.5.4.3.9: Mapping of CPG ( 181 (Call Is Being Forwarded) response

	ISUP Parameter
	Derived value of parameter field
	SIP component


	Value

	Event Indicator
	Progress
	
	

	Generic notification indicators
	Call is diverting
	
	

	Redirection number
	
	History-Info header field with one hi-entry
	hi-targeted-to-uri:

	Nature of address indicator
	"national (significant) number"

	hi-targeted-to-uri
	Add CC (of the country where the MGCF is located) to Redirection number Address Signals to construct E.164 number in URI.

	
	"international number"
	hi-targeted-to-uri
	Map complete Redirection number Address Signals to E.164 number in URI.

	Address Signals
	If NOA is "national (significant) number" then the format of the Address Signals is: 

NDC + SN 

If NOA is "international number" 

then the format of the Address Signals is: 

CC + NDC + SN
	hi-targeted-to-uri
	Addr-spec

"+" CC NDC SN mapped to userinfo portion of SIP URI. (NOTE 2)

Add "user=phone".

	Call diversion information
	Redirecting Reason
	IETF RFC 4458 [113] "cause" URI parameter in the hi-targeted-to-uri (NOTE 1)
	cause value

	
	Unknown/not available 
	
	404

	
	Unconditional
	
	302 

	
	User busy
	
	486

	
	No reply
	
	408

	
	Deflection immediate response 
	
	480

	
	Deflection during alerting
	
	487

	
	Mobile subscriber not reachable
	
	503

	
	Notification subscription option
	privacy "headers" component of the hi-targeted-to-uri (NOTE 1)
	Roles

	
	unknown
	
	Escaped Privacy value is set according to the rules of TS 24.604 [60] subclause 4.5.2.6.4 items c

	
	presentation not allowed
	
	A 181 Being Forwarded shall not be sent

	
	presentation allowed with redirection number
	
	Escaped Privacy value is set according to the rules of TS 24.604 [60] subclause 4.5.2.6.4 items c

	
	presentation allowed without redirection number
	
	Escaped Privacy value is set according to the rules of TS 24.604 [60] subclause 4.5.2.6.4 items c

	NOTE 1:
Needs to be stored for a possible inclusion into subsequent messages.

NOTE 2:
Used URI scheme shall be SIP URI. The "cause" URI parameter cannot be added if hi-targeted-to-uri is a tel URI.


Table 7.5.4.3.10 addresses two separate conditions: the CPG is received from the diverting exchange in which case the Call diversion information is included; and the CPG is received from the diverted-to exchange in which case the Call diversion information is not included. Interworking for both conditions is shown.
Table 7.5.4.3.10: Mapping of CPG ( 180 (Ringing) response

	ISUP Parameter 
	Derived value of parameter field
	SIP component


	Value

	Event Indicator
	Alerting
	
	

	Redirection number 
	
	History-Info header field with one hi-entry
	See table 7.5.4.3.8

	Call diversion information
	Redirecting Reason
	IETF RFC 4458 [113] 

"cause" URI parameter in the hi-targeted-to-uri (NOTE) 
	cause value

	
	Unknown/not available 
	
	404

	
	Unconditional
	
	302 

	
	User busy
	
	486

	
	No reply
	
	408

	
	Deflection immediate response 
	
	480

	
	Deflection during alerting
	
	487

	
	Mobile subscriber not reachable
	
	503

	
	Notification subscription option
	privacy "headers" component of the hi-targeted-to-uri (NOTE)
	Roles

	
	unknown
	
	Escaped Privacy value is set according to the rules of TS 24.604 [60] subclause 4.5.2.6.4 item c

	
	presentation not allowed
	
	The 180 Ringing response shall be sent without the the History-Info header field included

	
	presentation allowed with redirection number
	
	Escaped Privacy value is set according to the rules of TS 24.604 [60] subclause 4.5.2.6.4 item c

	
	presentation allowed without redirection number
	
	Escaped Privacy value is set according to the rules of TS 24.604 [60] subclause 4.5.2.6.4 item c

	If no Call diversion information parameter is present
	
	IETF RFC 4458 [113] "cause" URI parameter in the hi-targeted-to-uri
	Value stored from a previous received ACM or CPG. See tables 7.5.4.3.8 and 7.5.4.3.9.

	
	
	privacy "headers" component of the hi-targeted-to-uri
	Value stored from a previous received ACM or CPG. See tables 7.5.4.3.8 and 7.5.4.3.9.

	Redirection number restriction
	
	
	See table 7.5.4.3.7

	NOTE:
Needs to be stored for a possible inclusion into subsequent messages.


Table 7.5.4.3.11: Mapping of ANM ( 200 (OK) response (to INVITE request)

	ISUP Parameter
	Derived value of parameter field
	SIP component


	Value

	Redirection number 
	
	History-Info header field with one hi-entry
	See table 7.5.4.3.8

	
	
	IETF RFC 4458 [113] "cause" URI parameter in the hi- targeted-to-uri
	cause value= as stored from a previous received ACM or CPG. See tables 7.5.4.3.8 and 7.5.4.3.9.

	Redirection number restriction
	
	
	See table 7.5.4.3.7


7.5.5
Communication Hold (HOLD)

The mapping of Communication Hold supplementary service with Call Hold PSTN/ISDN Supplementary Service is the same mapping as described in clause 7.4.10. The Service itself is described within TS 24.610 [65].
7.5.6
Conference call (CONF)
7.5.6.1
General

The protocol description of the CONF supplementary service is described in TS 24.605 [61]. In this clause the interworking from the conference event package RFC 4575 [100] to the messages of the PSTN/ISDN CONF supplementary service is described. Note that an interworking from the PSTN/ISDN to the IMS is out of scope.

7.5.6.2
Subscribing for the conference event package

Based on local policy, the MGCF may subscribe for the conference event package on behalf of the PSTN/ISDN participant after the participant joins or is added to a conference.

When the conference event package option is implemented, and one of the following events occurs at the MGCF:

-
a 200 (OK) response is received as a response to an initial INVITE request originated by the MGCF, where the Contact header field contains an "isfocus" parameter; or

-
an ACK message is received which acknowledges a 200 (OK) response to the initial INVITE request, and the initial INVITE request is originated by the conferencing AS and contains an "isfocus" parameter in the Contact header field;
then the following steps shall be performed:

1)
a SUBSCRIBE request shall be created according to RFC 4575 [100];

2)
the Request URI is set to the Contact address of the conferencing AS;

3)
the P-Asserted-Identity header field, the From header field and the Privacy header field are set with the same value as:

-
the P-Asserted-Identity header field, the From header field and the Privacy header field in the initial INVITE request originated by the MGCF; or

-
the P-Asserted-Identity header field, the To header field and the Privacy header field in a 1xx or 2xx response sent by the MGCF to the initial INVITE request from the conferencing AS.

7.5.6.3
Interworking the notification

NOTE:
There is a need to differentiate between the procedures of interworking for a full and a partial type of notification.

When a full type of notification is received a check is made of the content. If the changes with respect a previous version of the notification have not been sent on to the PSTN/ISDN for this session, the MGCF shall perform an ISUP interaction towards the PSTN/ISDN. If the changes with respect a previous version of the notification have been sent to the PSTN/ISDN for this session, the MGCF shall not perform an ISUP interaction towards the PSTN/ISDN.

When a partial notification is received then it is assumed that a value of a received notification has changed, so the MGCF performs an ISUP interaction towards the PSTN/ISDN, as follows:

-
Conference established:


Upon the receipt of a conference information document with the <conference-state-type> element active is set to "true", the MGCF shall send a CPG message to the PSTN/ISDN with a notification "conference established".

-
Participant added:


Upon the receipt of a conference information document with the <endpoint-type> and the element status of endpoint‑status-type is set to "connected" and it was not set to "on-hold" before and the Contact URI in the element entity is not the address of the served PSTN/ISDN participant, the MGCF shall send a CPG message to the PSTN/ISDN with a notification "other party added".

-
Served PSTN/ISDN participant isolated:


Upon the receipt of a conference information document with the <endpoint-type> and the element status of endpoint‑status-type is set to "on-hold" and it was set to "connected" before and the Contact URI in the element entity is the address of the served PSTN/ISDN participant, the MGCF shall send a CPG message to the PSTN/ISDN with a notification "isolated".

-
Other participant isolated:


Upon the receipt of a conference information document with the <endpoint-type> and the element status of endpoint‑status-type is set to "on-hold" and it was set to "connected" before and the Contact URI in the element entity is not the address of the served PSTN/ISDN participant, the MGCF shall send a CPG message to the PSTN/ISDN with a notification "other party isolated".

-
Served PSTN/ISDN participant reattached:


Upon the receipt of a conference information document with the <endpoint-type> and the element status of endpoint‑status-type is set to "connected" and it was set to "on-hold" before and the Contact URI in the element entity is the address of the served PSTN/ISDN participant, the MGCF shall send a CPG message to the PSTN/ISDN with a notification "reattached".

-
Other participant reattached:


Upon the receipt of a conference information document with the <endpoint-type> and the element status of endpoint‑status-type is set to "connected" and it was set to "on-hold" before and the Contact URI in the element entity is not the address of the served PSTN/ISDN participant, the MGCF shall send a CPG message to the PSTN/ISDN with a notification "other party reattached".

-
Other party disconnected:


Upon the receipt of a conference information document with the <endpoint-type> and the element status of endpoint‑status-type is set to "disconnected" and the element joining-method of joining-type is not set to "focus-owner", the MGCF shall send a CPG message to the PSTN/ISDN with a notification "other party disconnected".
7.5.7

Anonymous Communication Rejection (ACR) and Communication Barring (CB)

The Anonymous Communication rejection (ACR) and Communication Barring (CB) services are described within TS 24.611 [67].

The mapping of Anonymus Communication Rejection supplementary service with Anonymus Call Rejection PSTN/ISDN Supplementary Service is described in clause 7.4.23.
The mapping for Communication Barring is in accordance with the basic call procedures as described in clauses 7.2.3.1.8 and 7.2.3.2.12.
7.5.8

Message Waiting Indication (MWI)

The Message Waiting Indication supplementary service is described within TS 24.606 [62].
7.5.9

Malicious Communication Identification (MCID)
7.5.9.0
General

The protocol specification of the Malicious Communication Identification supplementary service is described in TS 24.616 [102]. The XML MCID body used in related SIP messages is also specified in TS 24.616 [102].
7.5.9.1
Interworking at the O-MGCF

7.5.9.1.0
General

If the MGCF supports the interworking of the MCID service the O-MGCF shall map a SIP INFO request ("legacy" mode of usage of the INFO method as defined in IETF RFC 6086 [133]) containing a XML mcid body with MCID XML Request schema to an Identification Request (IDR) message and an Identification response (IRS) message to a SIP INFO request containing a XML mcid body with MCID XML Response schema in accordance with table 7.5.9.1.1.

The IDR message shall be generated upon reception of the SIP INFO request containing a XML mcid body with MCID XML Request schema.
The SIP INFO request containing a XML mcid body with MCID XML Response schema shall be generated upon reception of the IRS message.
Table 7.5.9.1.1 Mapping between ISUP IDR and IRS and SIP messages
	ISUP Message
	SIP Message

	IDR
	INFO containing a XML mcid body with MCID XML Request schema 

	IRS
	INFO containing a XML mcid body with MCID XML Response schema


7.5.9.1.1
Interworking of the MCID XML Request schema with the ISUP MCID request indicators

If the MGCF supports the interworking of the MCID service O-MGCF shall map the codes in the MCID XML elements to MCID request indicator and holding indicator parameter fields in accordance with table 7.5.9.1.1.1.

Table 7.5.9.1.1.1 Mapping between ISUP MCID request and holding indicators and MCID XML elements
	ISUP Parameter
	XML Element

	bit
A:
	MCID request indicator
	McidRequestIndicator

	
0
	MCID not requested
	type=0

	
1
	MCID requested
	type=1

	
	
	

	bit
B:
	Holding indicator (national use)
	HoldingIndicator

	
0
	holding not requested
	type=0

	
1
	holding requested
	type=1


7.5.9.1.2
Interworking of the ISUP MCID response indicators with the MCID XML Response schema

If the MGCF supports the interworking of the MCID service the O-MGCF shall map the codes in the MCID response indicator and hold provided indicator parameter field to the MCID XML elements in accordance with table 7.5.9.1.2.1.
Table 7.5.9.1.2.1 Mapping between ISUP MCID response and hold provided indicators and MCID XML elements
	ISUP Parameter
	XML Element

	bit
A:
	MCID response indicator
	McidResponseIndicator

	
0
	MCID not included
	type=0

	
1
	MCID included
	type=1

	
	
	

	bit
B:
	Hold provided indicator (national use)
	HoldingProvidedIndicator

	
0
	holding not provided
	type=0

	
1
	holding provided
	type=1


7.5.9.1.3
Interworking of the ISUP Calling Party Number in an Identification Response with the OrigPartyIdentity within the MCID XML Response schema

If the O-MGCF supports the interworking of the MCID service and receives an ISUP Identification Response containing a Calling Party Number with the screening indicator set to "user provided, verified and passed" or "network provided", the O-MGCF shall map the Calling Party Number to the MCID XML Response schema OrigPartyIdentity element applying the same mapping procedures as described in table 14 for the mapping into the SIP P-Asserted-Identity header and shall map the Calling Party Number APRI to the MCID XML Response schema OrigPartyPresentationRestriction element. If the Calling Party Number APRI has a value of "presentation allowed" then the MCID XML Response schema OrigPartyPresentationRestriction element shall be set to "false", otherwise it shall be set to "true".

7.5.9.1.4
Interworking of the ISUP Generic Number in an Identification Response with the GenericNumber within the MCID XML Response schema

If the O-MGCF supports the interworking of the MCID service and receives an ISUP Identification Response containing a Generic Number with the screening indicator set to "user provided, verified and passed", or "user provided, not verified", or "network provided", the O-MGCF shall map the Generic Number to the MCID XML Response schema GenericNumber element applying the same mapping procedures as described in table 13 for the mapping into the SIP From header and shall map the Generic Number APRI to the MCID XML Response schema GenericNumberPresentationRestriction element. If the Generic Number APRI has a value of "presentation allowed" then the MCID XML Response schema GenericNumberPresentationRestriction element shall be set to "false", otherwise it shall be set to "true".
7.5.9.2
Interworking at the I-MGCF

7.5.9.2.1
General

If the MGCF supports the interworking of the MCID service the I-MGCF shall map an Identification Request (IDR) message to a SIP INFO request ("legacy" mode of usage of the INFO method as defined in IETF RFC 6086 [133]) containing a XML mcid body with MCID XML Request schema and a SIP INFO request containing a XML mcid body with MCID XML Response schema to an Identification response (IRS) message in accordance with table 7.5.9.1.1.

7.5.9.2.2
Interworking of identification Request

The SIP INFO request containing a XML mcid body with MCID XML Request schema shall be generated upon reception of the IDR message. The I-MGCF shall map the codes in the MCID request indicator and holding indicator parameter fields to the MCID XML elements in accordance with table 7.5.9.1.1.1.

7.5.9.2.3
Interworking of identification Response

The IRS message shall be generated upon reception of the SIP INFO request containing a XML mcid body with MCID XML Response schema. The I-MGCF shall map the codes in the MCID XML elements to the MCID response indicator and hold provided indicator parameter fields in accordance with table 7.5.9.1.2.1.
If the received MCID XML Response schema contains an OrigPartyIdentity element, the I-MGCF shall map the OrigPartyIdentity to the Calling Party Number within the IRS applying the same mapping procedure as described in table 5 for the mapping from the SIP P-Asserted-Identity header, with the exception that the I-MGCF shall map the MCID XML Response schema OrigPartyPresentationRestriction element to the Calling Party Number APRI. If the MCID XML Response schema OrigPartyPresentationRestriction element has the value "true", the Calling Party Number APRI shall be set to "presentation restricted", and otherwise the Calling Party Number APRI shall be set to "presentation allowed".
If the received MCID XML Response schema contains an GenericNumber element, the I-MGCF shall map the GenericNumber to the Generic Number within the IRS applying the same mapping procedure as described in table 6 for the mapping from the From header, with the exception that the I-MGCF shall map the MCID XML Response schema GenericNumber PresentationRestriction element to the Generic Number APRI. If the MCID XML Response schema GenericNumberPresentationRestriction element has the value "true", the Generic Number APRI shall be set to "presentation restricted", and otherwise the Generic Number APRI shall be set to "presentation allowed".
7.5.10
Closed User Group (CUG)
7.5.10.0
General

The protocol specification of the Closed User Group supplementary service is described in TS 24.654 [101].
7.5.10.1
Interworking at the I-MGCF

If the I-MGCF supports the interworking of CUG supplementary service, the I-MGCF shall map between the SIP and ISUP messages in accordance with table 7.5.10.1.1.

Table 7.5.10.1.1: Mapping of SIP messages to ISUP messages

	SIP Message
	ISUP Message

	INVITE request containing a XML cug body with CUG XML schema 
	IAM containing the Closed user group interlock code Parameter and the closed user group call indicator of the Optional Forward Call Indicator Parameter


If the MGCF supports the interworking of CUG supplementary service, the I-MGCF shall interwork the CUG XML schema with the ISUP Closed user group interlock code parameter and the Closed user group call indicator of the optional forward call indicator parameter in accordance with tables 7.5.10.1.2 and 7.5.10.1.3.

Table 7.5.10.1.2: Mapping of the SIP XML CUG Element to the ISUP closed usergroup interlock code parameter

	CUG XML Element
	Source component value 
	ISUP Closed user group interlock code Parameter
	derived value of parameter field

	networkIndicator
	networkIdentityType = 4 hexbinary coded digits (NOTE)
	"Network Identity"
	Octet 1 & Octet 2 including 4 binary coded digits derived from XML Network Identity 

	cugInterlockBinaryCode
	sixteenbitType = 16 bit coded value 
	"Binary Code"
	Octet 3 & Octet 4 including a 16 bit Binary Code derived from the XML Binary Code

	NOTE:
ISUP Closed user group interlock code Parameter Octet 1 contains "Network Identity" (NI) digits 1 & 2 and Octet 2 contains "Network Identity" digits 3 & 4.The networkidentityType is filled with "Octet1 & Octet 2" = "NI digit 1, NI digit 2, NI digit 3, NI digit 4". Example: Digit 1=0, Digit 2=4, Digit 3=9, Digit 4=0 so the networkidentityType is encoded with "0490".


Table 7.5.10.1.3: Mapping of the SIP XML CUG Element to the ISUP closed user group call indicator included in the optional Forward Call Indicator Parameter

	CUG XML Element
	Source component value
	ISUP "Optional Forward Call Indicator" Parameter
	derived value of parameter field

	cugCommunicationIndicator
	Type=00
	"closed user group call" indicator
	non-CUG call

	
	Type=01
	
	spare

	
	Type=10
	
	closed user group call, outgoing access allowed

	
	Type=11
	
	closed user group call, outgoing access not allowed


If the I-MGCF supports the interworking of CUG supplementary service, then if an INVITE request with the MIME including a cug XML element is received and the terminating network is not supporting CUG, the I-MGCF shall behave as shown in table 7.5.10.1.4.

Table 7.5.10.1.4: Action at the I-MGCF with a PSTN/ISDN network without CUG capability

	cugCommunicationIndicator in INVITE request
	Action at the I-MGCF

	Type=11 (CUG without outgoing access)
	Release the communication with 403

	Type=10 (CUG with outgoing access)
	Treat the communication as an ordinary call (NOTE)

	Non-CUG
	Treat the communication as an ordinary call

	NOTE:
The cugCommunicationIndicator shall not be mapped or if appropriate the CUG call indicator of the optional forward call indicator shall be set to non-CUG call.


7.5.10.2
Interworking at the O-MGCF

If the MGCF supports the interworking of CUG supplementary service, the O-MGCF shall map between the SIP and ISUP messages in accordance with table 7.5.10.2.1

Table 7.5.10.2.1: Mapping of ISUP messages to SIP messages

	ISUP Message
	SIP Message

	IAM containing the Closed user group interlock code Parameter and the closed user group call indicator of the Optional Forward Call Indicator Parameter
	INVITE request containing a XML cug body with CUG XML schema 


If the MGCF supports the interworking of CUG supplementary service, the MGCF shall interwork the CUG XML schema with the ISUP Closed user group interlock code parameter and the Closed user group call indicator of the optional forward call indicator parameter in accordance with tables 7.5.10.2.2 and table 7.5.10.2.3.

Table 7.5.10.2.2: Mapping of the ISUP closed usergroup interloccode to SIP XML CUG element

	ISUP Closed user group interlock code Parameter
	Source component value
	CUG XML Element
	derived value of parameter field

	"Network Identity"
	Octet 1 & Octet 2 including 4 binary coded digits
	networkIndicator
	networkIdentityType = 4 hexbinary coded digits derived from Network Identity (NOTE)

	"Binary Code"
	Octet 3 & Octet 4 including a 16 bit Binary Code 
	cugInterlockBinaryCode
	sixteenbitType = 16 bit coded value derived from Binary Code

	NOTE:
ISUP Closed user group interlock code Parameter Octet 1 contains "Network Identity" (NI) digits 1 & 2 and Octet 2 contains "Network Identity" digits 3 & 4.The networkidentityType shall be filled with "Octet1 & Octet 2" = "NI digit 1, NI digit 2, NI digit 3, NI digit 4". Example: Digit 1=0, Digit 2=4, Digit 3=9, Digit 4=0 so the networkidentityType is encoded with "0490".


Table 7.5.10.2.3: Mapping of the ISUP Closed user group call indicator to SIP XML CUG element

	ISUP Optional Forward Call Indicator Parameter
	Source component value
	CUG XML Element
	derived value of parameter field

	closed user group call indicator
	non-CUG call
	cugCommunicationIndicator
	Type=00

	
	spare
	
	Type=01

	
	closed user group call, outgoing access allowed
	
	Type=10

	
	closed user group call, outgoing access not allowed
	
	Type=11


If the MGCF supports the interworking of CUG supplementary service, but the IMS is not supporting CUG, the procedures described in ITU Q.735.1 [42] shall apply if an INVITE request with the MIME body including a cug XML element is sent and the O-MGCF supports CUG supplementary service.
7.5.11
CCBS/CCNR
7.5.11.0
General

The protocol specification of the Completion of Communications to Busy Subscriber and Completion of Communications by No Reply supplementary services is described in TS 24.642 [112].
SIP specific Event Notifications shall be used in accordance with IETF RFC 3265 [135] including as referenced in IETF RFC 6910 [106].
7.5.11.1
Interworking at the I-MGCF

If the I-MGCF supports the interworking of CCBS/CCNR supplementary services, the I-MGCF shall map between the SIP and ISUP messages in accordance with table 7.5.11.1.1.

Table 7.5.11.1.1: Mapping of ISUP and SIP messages
	SIP Message
	Parameter
	ISUP Message
	Parameter

	← 180 Ringing


	Call-Info header field with purpose header field parameter set to "call-completion" and "m" header field parameter set to "NR" (no reply)
(NOTE 1)
	← ACM
	Called party's status indicator set to "Subscriber free" and

CCNR possible indicator set to "CCNR possible"
(NOTE 2)

	← 180 Ringing


	Call-Info header field with purpose header field parameter set to "call-completion and "m" header field parameter set to "NR" (no reply)
(NOTE 1)
	← CPG
	Event indicator set to "Alerting" and

CCNR possible indicator set to "CCNR possible"
(NOTE 2)

	← 486 Busy here


	Call-Info header field with purpose header field parameter set to "call-completion" and "m" header field parameter set to "BS" (busy subscriber)

(NOTE 1)
	← REL
	Cause Indicator
cause #17 with Diagnostic (CCBS indicator set to "CCBS possible")
(NOTE 3)

	→ INVITE
	Request URI contains "m" SIP URI parameter or Call-Info header field contains "purpose" header field parameter set to "call-completion" and "m" header field parameter. 
(NOTE 1)
	→ IAM
	CCSS parameter set to "CCSS call"
(NOTE 2) (NOTE 4)

	NOTE 1:
The coding shall be in accordance with IETF RFC 6910 [106].

NOTE 2:
The coding shall be in accordance with ITU-T recommendation Q. 763 [4].

NOTE 3:
The coding shall be in accordance with ITU-T recommendation Q. 850 [38].

NOTE 4:
CCSS parameter set to the value "CCSS call" is included in the IAM if Request-URI contains the SIP URI parameter "m" i.e. creation of the CCSS parameter does not depend on the received value of the SIP URI parameter "m".


If the I-MGCF supports the interworking of CCBS/CCNR supplementary services, the I-MGCF shall map between the SIP and TCAP messages in accordance with table 7.5.11.1.2 and table 7.5.11.1.3.
Table 7.5.11.1.2: Mapping of SIP messages to TCAP messages

	SIP Message 
	Parameter
	TCAP Message
	Parameter

	SUBSCRIBE with m-parameter in Request URI set to "BS" or containing Call-Info header field with "purpose" parameter set to "call-completion" and m-parameter set to "BS"
(NOTE 1)
	Request URI (NOTE 5)
	TC-Begin CCBS REQUEST (invoke)
	CalledPartyNumber (NOTE 3)

	
	P-Asserted-Identity
	
	CallingPartyNumber (NOTE 4)

	
	
	
	RetainSupported (NOTE 2)

	SUBSCRIBE with m-parameter in Request URI set to "NR" or containing Call-Info header field with "purpose" parameter set to "call-completion" and m-parameter set to "NR"
(NOTE 1)
	Request URI (NOTE 5)
	TC-Begin CCNR REQUEST (invoke)
	CalledPartyNumber (NOTE 3)

	
	P-Asserted-Identity
	
	CallingPartyNumber (NOTE 4)

	
	
	
	RetainSupported (NOTE 2)

	PUBLISH with m-parameter in Request URI or m-parameter in Call-Info header field set to "BS" and body containing PIDF basic status set to "closed".
	
	TC-Cont CCBS SUSPEND
	

	PUBLISH with m-parameter in Request URI or m-parameter in Call-Info header field set to "BS" and body containing PIDF basic status set to "open".
	
	TC-Cont CCBS RESUME
	

	PUBLISH with m-parameter in Request URI or m-parameter in Call-Info header field set to "NR" and body containing PIDF basic status set to "closed".
	
	TC-Cont CCBS SUSPEND
	

	PUBLISH with m-parameter in Request URI or m-parameter in Call-Info header field set to "NR" and body containing PIDF basic status set to "open".
	
	TC-Cont CCBS RESUME
	

	SUBSCRIBE with m-parameter in Request URI or m-parameter in Call-Info header field set to "BS" and with Expires header set to "zero".
	
	TC-End CCBS CANCEL
	

	SUBSCRIBE with m-parameter in Request URI or m-parameter in Call-Info header field set to "NR" and with Expires header set to "zero".
	
	TC-End CCBS CANCEL
	

	NOTE 1:
Expires header defines subscription duration / CC service duration timer (CC-T3).

NOTE 2:
Parameter is set by default, as retention option is supported in IMS by default.

NOTE 3:
Mapping of the Request URI header field to the CalledPartyNumber is done according to table 2 in clause 7.2.3.1.2.1.

NOTE 4:
Mapping of the P-Asserted Identity header field to the CallingPartyNumber is done according to table 5 in clause 7.2.3.1.2.6.
NOTE 5:
If URI of the MGCF was returned in the Call-Info header field in the 180 Ringing or 486 Busy here messages described in table 7.5.11.1.1, then the MGCF needs to remember the Request URI of the original INVITE for mapping to the CalledPartyNumber. 


Table 7.5.11.1.3: Mapping of TCAP messages to SIP messages

	TCAP Message
	Parameter
	SIP Message
	Parameter

	TC-Cont CCBS REQUEST (return result)
	RetainSupported
	NOTIFY with cc-state parameter set to "queued"
	cc-service-retention

	TC-End CCBS/CCNR REQUEST (error result)
	ShortTermDenial
	480 Temporarily unavailable
	

	TC-End CCBS/CCNR REQUEST (error result)
	LongTermDenial
	403 Forbidden
	

	TC-End CCBS CANCEL
	
	NOTIFY with the "reason" Subscription-State header field parameter set to "noresource"
	

	TC-Cont REMOTE USER FREE
	
	NOTIFY with cc-state set to "ready" (NOTE 1)
	

	NOTE 1:
This does not terminate the subscribtion AS-AS.


7.5.11.2
Interworking at the O-MGCF

If the O-MGCF supports the interworking of CCBS/CCNR supplementary services, the O-MGCF shall map between the ISUP and SIP messages in accordance with table 7.5.11.2.1.

Table 7.5.11.2.1: Mapping of SIP and ISUP messages
	ISUP Message
	Parameter
	SIP Message
	Parameter

	← ACM


	Called party's status indicator set to "Subscriber free" and

CCNR possible indicator set to "CCNR possible"
(NOTE 2)
	← 180 Ringing


	Call-Info header field with purpose header field parameter set to "call-completion" 
(NOTE 1)



	← CPG


	Event indicator set to "Alerting" and

CCNR possible indicator set to "CCNR possible"
(NOTE 2) (NOTE 4)
	
	

	← REL


	Cause Indicator
cause #17 or #34 with Diagnostic (CCBS indicator set to "CCBS possible")
(NOTE 3)
	← 486 Busy here
	Call-Info header field with purpose header field parameter set to "call-completion" 
(NOTE 1)

	→ IAM 
	CCSS parameter set to "CCSS call"
(NOTE 2)
	→ INVITE
	Request URI contains m-parameter and a Call-Info header field field, with purpose header field parameter set to "call-completion", and an m-parameter
(NOTE 1) (NOTE 5)

	NOTE 1:
The coding shall be in accordance with IETF RFC 6910 [106].

NOTE 2:
The coding shall be in accordance with ITU-T recommendation Q. 763 [4].

NOTE 3:
The coding shall be in accordance with ITU-T recommendation Q. 850 [38].

NOTE 4:
A CPG will be sent if an ACM was already sent.

NOTE 5:
Based on the operator policy the "m" SIP URI parameter in the Request-URI and m-parameter in Call-Info header field is set to the value "BS" or "NR".


If the O-MGCF supports the interworking of CCBS/CCNR supplementary services, the O-MGCF shall map between the TCAP and SIP messages in accordance with table 7.5.11.2.2 and table 7.5.11.2.3.

Table 7.5.11.2.2: Mapping of TCAP messages to SIP messages

	TCAP Message
	Parameter
	SIP Message
	Parameter

	TC-Begin CCBS REQUEST (invoke)
	CalledPartyNumber 
	SUBSCRIBE with m-parameter in Request URI and m-parameter in Call-Info header field set to "BS" (NOTE 1)
	To header (NOTE 2)

	
	
	
	Request-URI (NOTE 2)

	
	CallingPartyNumber
	
	From header (NOTE 3)

	
	
	
	P-Asserted-Identity (NOTE 4)

	TC-Begin CCNR REQUEST (invoke)
	CalledPartyNumber
	SUBSCRIBE with m-parameter in Request URI and m-parameter in Call-Info header field set to "NR" (NOTE 1)
	To header (NOTE 2)

	
	
	
	Request-URI (NOTE 2)

	
	CallingPartyNumber
	
	From header (NOTE 3)

	
	
	
	P-Asserted-Identity (NOTE 4)

	TC-Cont CCBS SUSPEND
	
	PUBLISH with m-parameter in Request URI and m-parameter in Call-Info header field set to "BS" and body containing PIDF basic status set to "closed".
	

	
	
	PUBLISH with m-parameter in Request URI and m-parameter in Call-Info header field set to "NR" and body containing PIDF basic status set to "closed".
	

	TC-Cont CCBS RESUME
	
	PUBLISH with m-parameter in Request URI and m-parameter in Call-Info header field set to "BS" and body containing PIDF basic status set to "open".
	

	
	
	PUBLISH with m-parameter in Request URI and m-parameter in Call-Info header field set to "NR" and body containing PIDF basic status set to "open".
	

	TC-End CCBS CANCEL


	
	SUBSCRIBE with m-parameter in Request URI and m-parameter in Call-Info header field set to "BS" and with Expires header set to "0"
	

	
	
	SUBSCRIBE with m-parameter in Request URI and m-parameter in Call-Info header field set to "NR" and with Expires header set to "0"
	

	NOTE 1:
Expires header defines subscription duration / CC service duration timer (CC-T3).

NOTE 2:
For the mapping of the CalledPartyNumber to the To header and Request-URI see table 10a in clause 7.2.3.2.2.1.

NOTE 3:
For the mapping of the CallingPartyNumber to the From header see table 15 in clause 7.2.3.2.2.3. If no CallingPartyNumber is available, the From header shall be set to a SIP URI with addr-spec of Unavailable User Identity as defined in TS 23.003 [74].
NOTE 4:
For the mapping of the CallingPartyNumber to the P-Asserted-Identity header see table 14 in clause 7.2.3.2.2.3. If no CallingPartyNumber is available, a P-Asserted-Identity header shall not be inserted.


Table 7.5.11.2.3: Mapping of SIP messages to TCAP messages

	SIP Message
	Parameter
	TCAP Message
	Parameter

	NOTIFY with cc-state parameter set to "queued"
	cc-service-retention
	TC Cont CCBS/CCNR REQUEST (return result)
	RetainSupported

	480 Temporarily unavailable 
	
	TC-End CCBS/CCNR REQUEST (error result)
	ShortTermDenial

	403 Forbidden 
	
	TC-End CCBS/CCNR REQUEST (error result)
	LongTermDenial

	NOTIFY with the Subscription-State header field set to "terminated" and the "reason" parameter set to "noresource"
	
	TC-End CCBS CANCEL
	

	NOTIFY with cc-state set to "ready" (NOTE)
	
	TC-Cont REMOTE USER FREE
	

	NOTE:
This does not terminate the subscription AS-AS.


7.5.12
Communication Waiting (CW)

7.5.12.0
General

The protocol specification of the Communication Waiting supplementary service is described in TS 24.615 [111].
7.5.12.1
Interworking at the I-MGCF

With regard to the backward messages of the Call Waiting PSTN/ISDN supplementary service, the following mapping is valid:

Table 7.5.12.1: Mapping of ISUP messages to SIP Massages

	(Message Received from BICC/ISUP
	(Message sent to SIP

	ACM or CPG with generic notification indicator "Call is a waiting call" (NOTE 1).
	180 Ringing with an Alert-Info header field set to "urn:alert:service:call-waiting" (NOTE 2).

	NOTE 1:
The coding shall be in accordance with ITU-T recommendation Q.733 [42].

NOTE 2:
The coding shall be in accordance with IETF RFC 7462 [107].


7.5.12.2
Interworking at the O-MGCF

With regard to the backward messages of the Communication Waiting service, the following mapping is valid:

Table 7.5.12.2: Mapping of SIP messages to ISUP messages

	(Message sent to ISUP
	(Message Received from SIP

	ACM or CPG with generic notification indicator "Call is a waiting call" (NOTE 1).
	180 Ringing with an Alert-Info header field set to "urn:alert:service:call-waiting" (NOTE 2).

	NOTE 1:
The coding shall be in accordance with ITU-T recommendation Q733 [42].

NOTE 2:
The coding shall be in accordance with IETF RFC 7462 [107].


7.5.13

Customized Alerting Tones (CAT)
7.5.13.1
General

The Customized Alerting Tones supplementary service is described in TS 24.182 [132].

The interworking procedures in this clause enable the transport of customized alerting tones provided by an AS in the IM CN subsystem towards a caller in a CS network.

Based on operator options, one of three different models described in TS 24.182 [132].is used in an IM CN subsystem to provide the CAT service. The CAT interworking procedures at the O-MGCF differ between these models and are described seperately for each model in the subsequent clauses. Support of the interworking procedures for each of those models is optional.

I-MGCF procedures are not affected.
7.5.13.2
Early session model

7.5.13.2.1
Interworking at the O-MGCF

When CAT service is provided by using early session model as specificed in TS 24.182 [132], and if the O-MGCF supports early-sesssion as a network option, then it shall include the option tag "early-session", as defined in IETF RFC 3959 [131], in Supported header field in each outgoing initial SIP INVITE request.

After receiving the first SIP 18x response including an early-session SDP, then the O-MGCF shall send the address complete message (ACM) or Call Progress message (CPG) with the Optional backward call indicators parameter set to "in-band information or an appropriate pattern is now available". If the SIP 18x response is the SIP 180 Ringing response, then additionally the Called party's status indicator in the ACM message shall be set to "subscriber free" or the Event information in the CPG message shall be set to "ALERTING".

If the O-MGCF applies codec interworking procedures according to Annex B, it shall interwork the "session" disposition SDP (rather than the "early session" disposition SDP).
7.5.13.2.2
MGCF – IM-MGW Interaction for CAT service

7.5.13.2.2.1
General

When the IMS CAT is provided by using early session model as described in TS 24.182 [132], the procedures described in clauses 9.2.3.1 to 9.2.3.3 shall be applied with the following additions.

7.5.13.2.2.2
IM CN subsystem side Early Session establishment

If the MGCF supports early-session as a network option, upon receipt of a SIP 18x response with an early-session SDP, the MGCF shall use the Configure IMS Resources procedure (signals 10 and 11 in figure 7.5.13.2.2.5-1, signals 7 and 8 in figure 7.5.13.2.2.5-2) to provide configuration data (derived from early session SDP received in signal 8 in figure 7.5.13.2.2.5-1 and local configuration data) to the IM-MGW as detailed below:

-
The MGCF shall indicate the remote IP address and UDP port, i.e. the destination IP address and UDP port for data sent in the user plane towards the IM CN subsystem,

-
The MGCF shall indicate the remote codec(s), i.e. the speech codec(s) for data sent in the user plane towards the IM CN subsystem.

-
The MGCF should select the same codec(s) for early-session SDP with the codec(s) used for normal session SDP.

-
The MGCF may indicate the local codec(s) and the local IP address and UDP port. The MGCF shall indicate the local codec(s) if a change is required.

-
The MGCF may indicate that IP interface type is for MboIP.
The IM-MGW shall reply with the selected remote codec(s) and reserve resources for these codec(s). If local codec(s) were received, the IM-MGW shall also reply with the selected local codec(s) and reserve the corresponding resources.

The MCGF shall send the local codec(s), UDP port and IP address in the early-session SDP to the IMS in the PRACK messages (signal 12 in figure 7.5.13.2.2.5-1 and 7.5.13.2.2.5-2).

7.5.13.2.2.3
IM CN subsystem side normal Session establishment

When the MGCF receives a SIP 200(OK) response to the SIP INVITE request (signal 14 in figure 7.5.13.2.2.5-1, signal 15 in figure 7.5.13.2.2.5-2), it shall request the IM-MGW use the Configure IMS Resources procedure for normal session SDP (signals 15 and 16 in figure 7.5.13.2.2.5-1, signal 16 and 17 in figure 7.5.13.2.2.5-2) to provide configuration data (derived from SDP received in signal 8 in figure 7.5.13.2.2.5-1 and local configuration data) to the IM-MGW as detailed below:

-
The MGCF shall indicate the remote IP address and UDP port, i.e. the destination IP address and UDP port for data sent in the user plane towards the IM CN subsystem,

-
The MGCF shall indicate the remote codec(s), i.e. the speech codec(s) for data sent in the user plane towards the IM CN subsystem.

-
The MGCF may indicate the local codec(s) and the local IP address and UDP port. The MGCF shall indicate the local codec(s) if a change is required.

-
The MGCF may indicate that IP interface type is for MboIP.
The IM-MGW shall reply with the selected remote codec(s) and reserve resources for these codec(s). If local codec(s) were received, the IM-MGW shall also reply with the selected local codec(s) and reserve the corresponding resources.

7.5.13.2.2.4
Called party alerting

If the MGCF supports early-session as a network option, upon receipt of a SIP 18x provisional response with an early-session SDP, the MGCF shall not request the IM-MGW to provide an awaiting answer indication (ringing tone) to the calling party.

Editor's Note:
The interaction with other supplementary services are FFS.

7.5.13.2.2.5
Message sequence chart

Figure 7.5.13.2.2.5-1 shows an example message sequence chart for an CS network originating Session Setup with ISUP, where IMS CAT is provided by early session model.


[image: image55.emf]IM-

MGW

MGCF

6. SIP: INVITE 

(Session SDP(IP1,p1)

7. SIP: 100 Trying

1. ISUP: IAM 

2. H.248: ADD.req [Context ID = ?, Termination ID = ?]

3. H.248: ADD.resp [Context ID = C1, Termination ID = T2] 

4. H.248: ADD.req [Context ID = C1, Termination ID=?]

5. H.248: ADD.resp [Context ID = C1, Termination ID = T1, 

IP1,p1]

8. SIP:180Ringing

(Session SDP and Early 

Session SDP)

12. SIP: PRACK

(Early Session SDP(IP1,p1))

13. SIP:200 OK (PRACK) 

10. H.248: MOD.req [Context ID = C1,Termination ID = T1],

Early session SDP

11.H.248: MOD.resp [Context ID = C1, Termination ID = T1 ] 

Reserve TDM Circuit,

Change Through-

Connection  = both  

Reserve IMS Connection Point, 

Change IMS Through-

Connection  = backward 

Configure IMS  Resources

9.ISUP: ACM 

18. SIP: ACK

15. H.248:  MOD.req [Context ID=C1, Termination ID = T1, 

esssion SDP]

16. H.248: MOD.resp [Context ID = C1, Termination ID = T1] 

17. ISUP: ANM 

CALL IN ACTIVE STATE

Configure IMS 

Resources,

Change IMS Through-

Connection  = both

14. SIP:200 OK (INVITE)


Figure 7.5.13.2.2.5-1: CS Network Originating Session with Early Session SDP, ISUP (message sequence chart)

Figure 7.5.13.2.2.5-2 shows an example message sequence chart for an CS network originating Session Setup with BICC, where IMS CAT is provided by early session model.


[image: image56.emf]IM-

MGW

MGCF

4. SIP: INVITE 

(Session SDP offer)

5. SIP: 100 Trying

1. BICC: IAM 

6. SIP:180Ring

(Session SDP answer and 

Early Session SDP offer)

12. SIP: PRACK

(Early Session SDP answer)

13. SIP:200 OK (PRACK) 

19. SIP: ACK

16. H.248:  MOD.req [Context ID=C1, Termination ID = T1]

17. H.248: MOD.resp [Context ID = C1, Termination ID = T1] 

18. BICC: ANM 

CALL IN ACTIVE STATE

Configure IMS 

Resources,

Change IMS Through-

Connection  = both

15. SIP:200 OK (INVITE)

2. H.248: ADD.req  [Context ID = ?, Termination ID=?]

3. H.248: ADD.resp [Context ID = C1, Termination ID = T1]

Reserve IMS Connection Point, 

Change IMS Through-

Connection = backward 

7. H.248: MOD.req Context ID = C1,Termination ID = T1]

8. H.248: MOD.resp [Context ID = C1,Termination ID = T1] 

Configure IMS  Resources

9. H.248: ADD.req [Context ID = C1, Termination ID = ?]

10. H.248: ADD.resp [Context ID = C1, Termination ID = T2] 

11. BICC: APM 

Prepare Bearer,

Change Through-

Connection= both

14. BICC: ACM

START CAT MEDIA


Figure 7.5.13.2.2.5-2: CS Network Originating Session with Early Session SDP, BICC (message sequence chart)
7.5.13.3
Forking model
Interworking at O-MGCF follows the basic call interworking procedures using the P-Early-Media header field and related procedures as described in clause 7.2.3.2.1, 7.2.3.2.2, 7.2.3.2.4, 7.2.3.2.5, 7.2.3.2.6, 7.2.3.2.7, 7.2.3.2.8, and 7.2.3.2.9.
The O-MGCF shall apply the procedures in clause 9.2.3.4 and shall use the P-Early-Media header field that authorizes early media as trigger to configure the IM-MGW as described in clause 9.2.3.4.2.
7.5.13.4
Gateway model
Interworking at O-MGCF follow the basic call interworking procedures using the P-Early-Media header field and related procedures as described in clause 7.2.3.2.1, 7.2.3.2.2, 7.2.3.2.4, 7.2.3.2.5, 7.2.3.2.6, 7.2.3.2.7, 7.2.3.2.8, and 7.2.3.2.9.
Example callflows are shown in annex J

8
User plane interworking

8.1
Interworking between IM CN subsystem and bearer independent CS network

When the IM CN subsystem interworks with the bearer independent CS networks (e.g. CS domain of a PLMN, TS 29.414 [25], TS 29.415 [26], TS 23.205 [27]), the Transport Network Layer (TNL) of the bearer independent CS network can be based e.g. on IP/UDP/RTP, or IP/UDP/RTP/IuFP, or ATM/AAL2/ framing protocol (e.g. Iu framing) transport techniques. Figure 31 shows the user plane protocol stacks for the IM CS subsystem and bearer independent CS network interworking. If the same AMR configuration is used on the CS network side as on the IMS side, transcoding is not required. However, there is still a need to interwork between RTP/UDP/IP/L2/LI to TNL/LI.


[image: image57.wmf]AMR

IM-MGW

L1

L2

IP

 UDP

 RTP

voice

codec

L1

TNL

Transcoding

Mb


Figure 31/1: IM CN subsystem to bearer independent CS network user plane protocol stack

8.1.1
Transcoder-less Mb to Nb Interworking


[image: image58.wmf] 

Transcoder

-

less 

user plane 

interworking

 

IM

-

MGW

 

L1

 

L2

 

IP

 

 UDP

 

 RTP

 

L1

 

Nb FP

 

 

Mb

 

 

L2

 

L3

 

Nb

 


Figure 31/2: IM CN subsystem to bearer independent CS network user plane protocol stack (optional in the event the codecs on both sides are the same)

If no transcoder is inserted, the IM-MGW shall interwork the following procedures between the Nb and Mb interfaces.

8.1.1.1
Initialisation

There is no need to interwork initialisation procedures between Nb and Mb interfaces see TS 29.415 [26].

8.1.1.2
Time alignment

The purpose of the time alignment procedure on the Nb interface is to minimise the buffer delay in the RNC for downlink transmissions by adjusting the vocoder time reference within the network. No such procedure exists on the Mb interface, so the IM-MGW shall return NACK indication time alignment not supported according to TS 29.415 [26].

8.1.1.3
Rate control

The rate control procedure signals to the peer entity the maximum rate among the currently allowed rates at which it can receive codec frames. Rate control only applies to AMR family codec configurations with multiple active modes. On the Nb interface, IuFP provides for rate control via the exchange of RATE CONTROL and RATE CONTROL ACK PDUs. On the Mb interface, IETF RFC 4867 [23] provides for in-band rate control via the Codec Mode Request (CMR) field of every codec frame.

Interworking of rate control procedures at an IM-MGW between an Mb interface and a corresponding Nb interface only applies when the IM-MGW bridges compatible codec configurations between the interfaces without applying a transcoding function. An IM-MGW receiving a CMR from an Mb interface shall initiate the IuFP rate control procedure on the corresponding Nb interface. An IM-MGW receiving a rate control request on an Nb interface shall adjust the CMR field of outgoing speech frames on the corresponding Mb interface.
A IM-MGW may support the MTSI service defined in TS 24.173 [88] and may then support codecs according to TS 26.114 [104].

NOTE 1:
Alternative codec specifications for the MTSI service are listed in TS 24.173 [88].

If the IM-MGW supports the MTSI service with the codecs according to TS 26.114 [104] the IM-MGW should support reception of RTCP-APP-CMR messages as specified in the clause 10.2.1 of TS 26.114 [104] which can be sent by a MTSI client in the terminal. If the IM-MGW is in TrFO then RTCP-APP-CMR messages should be translated to the appropriate rate control method on the CS side (e.g. IuFP rate control or RTP based CMR) in the same way as for an RTP based CMR, described in the preceding paragraph.

NOTE 2:
When the IM-MGW is transcoding at the Mb interface then any RTCP-APP-CMR requests received from the MTSI client in the terminal when the IM-MGW is transcoding will result in a change to to DL codec mode towards the MTSI client terminal.
8.1.1.4
Frame quality indication

The Nb interface signals frame quality with the Frame Quality Classification (FQC) field of each speech frame PDU. See TS 26.102 [50] and TS 29.415 [26] for details. The FQC may have possible values: 0=frame_good; 1=frame_bad; 2=frame_bad_due_to_radio; and 3=spare. The Mb interface signals frame quality with the Q bit (frame quality indicator) field of each speech frame, as defined in IETF RFC 4867 [23]. The Q bit may have values: 1=speech_good; and 0=speech_bad or sid_bad.

Tables 24a and 24b provide the mapping between Mb and Nb interfaces.

Table 24a: Mapping of Mb (Q bit) onto Nb (FQC)

	Mb - Qbit
	Mb - FT
	Nb - FQC

	1
	X
	0

	0
	X
	1


Table 24b: Mapping Nb onto Mb

	Nb - FQC
	Mb - Qbit
	Mb – FT

	0
	1
	NC

	1
	0
	NO_DATA

	2
	0
	NC


8.1.1.5
Framing

Even when the IM-MGW bridges compatible codec configurations between the Nb and Mb interfaces, the IM-MGW shall perform translation between the frame formats defined for the two interfaces, since all codec configurations have different framing procedures for the two interfaces. The framing details for Nb are defined in TS 26.102 [50] and TS 29.415 [26], although they do not describe the framing for ITU-T codecs other than G.711. The framing details for Mb are defined in IETF RFC 4867 [23], IETF RFC 3550 [51], IETF RFC 3551 [52] and IETF RFC 3555 [53].

8.1.1.6
Transcoding

Transcoding at the IM-MGW is avoided when the IM-MGW bridges compatible codec configurations between the Nb and Mb interfaces. Otherwise transcoding is necessary, which eliminates the need to interwork other user plane procedures between the interfaces.

8.1.1.7
Discontinuous transmission

When the IM-MGW bridges compatible codec configurations between the Nb and Mb interfaces, the DTX procedures are normally interworked transparently by translating between the framing formats on the interfaces. All the ITU-T and AMR family codecs have configurations that are compatible between the Mb and Nb interfaces.

8.1.1.8
Timing and sequence information
The IM-MGW shall always correct out-of-sequence delivery between Nb and Mb interfaces, either by re-ordering frames, or by dropping frames that are out of sequence.

When the IuFP frame numbers are based on time and if the IM-MGW bridges compatible codec configurations between the Nb and Mb interfaces, it shall either correct jitter before forwarding PDUs or interwork the RTP timestamp (see RFC 3550 [51]) with the IuFP Frame Number (see TS 29.415 [26]) so that both the RTP timestamp and IuFP frame number similarly reflect the nominal sampling instant of the user data in the packet.

NOTE:
Correcting jitter may cause additional delay.

The RTP sequence number (see RFC 3550 [51]) is handled independently on Mb, i.e. it is not interworked with the IuFP Frame Number (see TS 29.415 [26]).

8.2
Interworking between IM CN subsystem and TDM-based CS network

It shall be possible for the IM CN subsystem to interwork with the TDM based CS networks (e.g. PSTN, ISDN or CS domain of a PLMN). Figure 32 describes the user plane protocol stack to provide the particular interworking.


[image: image59.wmf]AMR

IM-MGW

L1

L2

IP

 UDP

 RTP

G.711

PCM

Coding

L1

TDM

 CS

Bearer

Channel

Transcoding

Mb


Figure 32: IM CN subsystem to TDM-based CS network user plane protocol stack
8.3
Transcoding requirements

The IM CN subsystem supports the AMR codec as the native codec for basic voice services. For IM CN subsystem terminations, the IM MGW shall support the transport of AMR over RTP according to IETF RFC 4867 [23]. The MGCF shall support the options of IETF RFC 4867 [23] listed within clause 5.1.1 of TS 26.236 [32].

It shall be possible for the IM CN subsystem to interwork with the CS networks (e.g. PSTN, ISDN or a CS domain of a PLMN) by supporting AMR to G.711 transcoding (see ITU-T Recommendation G.711 [1]) in the IM-MGW. The IM-MGW may also perform transcoding between AMR and other codec types supported by CS networks.
8.4
Diffserv code point requirements

The IM-MGW shall perform DiffServ Code Point (DSCP) markings (see RFC 2474 [21]) on the IP packets sent towards the IM CN subsystem entity like UE or MRFP across the Mb interface to allow DiffServ compliant routers and GGSNs to schedule the traffic accordingly.

The IETF Differentiated Services architecture (see RFC 2475 [22]) shall be used to provide QoS for the external bearer service.

The DSCP shall be operator configurable.
8.5
DTMF handling

When sending DTMF inband towards the CS network, the MGW shall comply with the encoding requirements in TS 23.014 [103]; in particular the requirements for the minimum length of a tone and for the minimum gap between two subsequent tones shall be ensured.
When detecting DTMF digits arriving from the CS side, the MGW shall comply with TS 23.014 [103] (by checking that a valid digit with minimum duration and minimum gap has been received) before initiating an RTP Telephony Event to the IMS interface.
When sending DTMF towards the IMS side according to the IETF RFC 4733 [105] RTP Payload format, the MGW shall comply with the DTMF encoding requirements of Annex G.2 of TS 26.114 [104], in particular the minimum duration of 65ms shall be ensured. It is optional if the RTP Telephony Event is sent as a number of "RTP Events" with interim durations (e.g. every 20ms or 40ms in line with the speech packetisation time) or as a single "RTP Event" with the at least 65ms duration.
9
MGCF – IM-MGW Interaction

9.1
Overview

The MGCF shall control the functions of the IM-MGW, which are used to provide the connection between media streams of an IP based transport network and bearer channels from a CS network.

The MGCF shall interact with the IM-MGW across the Mn reference point. The MGCF shall terminate the signalling across the Mn interface towards the IM-MGW and the IM-MGW shall terminate the signalling from the MGCF.

The signalling interface across the Mn reference point shall be defined in accordance with ITU-T Recommendation H.248.1 [2] and shall conform to 3GPP specific extensions as detailed in TS 29.332 [15].

The present specification describes Mn signalling procedures and their interaction with BICC/ISUP and SIP signalling in the control plane, and with user plane procedures. TS 29.332 [15] maps these signalling procedures to H.248 messages and defines the required packages and parameters.

9.2
Mn signalling interactions

The following paragraphs describe the Mn interface procedures triggered by SIP and BICC signalling relayed in MGCF.

The SIP signalling occurring at the MGCF is described in TS 24.229 [9].

All message sequence charts in this clause are examples.

9.2.1
Network model

Figure 33 shows the network model, applicable to BICC and ISUP cases. The broken line represents the call control signalling. The dotted line represents the bearer control signalling (if applicable) and the user plane. The MGCF uses one context with two terminations in the IM-MGW. The termination T1 is used towards the IM CN subsystem entity and the bearer termination T2 is used for the bearer towards the succeeding CS network element.


[image: image60.wmf]    

 

IM

-

MGW

 

M

GCF

 

C

1

 

T1

 

T2

 

CS 

N

ETWORK

 


Figure 33: Network model

9.2.2
Basic IM CN subsystem originated session

9.2.2.1
BICC forward bearer establishment

9.2.2.1.1
IM-MGW selection

The MGCF shall select an IM-MGW for the bearer connection before it performs the CS network side bearer establishment. This may happen either before sending the IAM or after receiving the APM message (signal 5 or signal 6 in figure 34). In the latter case, the IM-MGW selection may be based on a possibly received MGW-id from the succeeding node.

9.2.2.1.2
CS network side bearer establishment

The MGCF shall either select bearer characteristics or request the IM-MGW to select and provide the bearer characteristics for the CS network side bearer connection before sending the IAM. In the latter case the MGCF shall use the Prepare Bearer procedure, not shown in figure 34, to request the IM-MGW to select the bearer characteristics. After the succeeding node has provided a bearer address and a binding reference in the APM, the MGCF shall use the Establish Bearer procedure to request the IM-MGW to establish a bearer towards the destination CS-MGW. The MGCF shall provide the IM-MGW with the bearer address, the binding reference and the bearer characteristics (signal 7 in figure 34).

9.2.2.1.3
IM CN subsystem side termination reservation

On receipt of an initial INVITE (signal 1 in figure 34) the MGCF shall initiate the Reserve IMS Connection Point and Configure Remote Resources procedure (signal 3 and 4 in figure 34). From the received SDP and local configuration data the MGCF:

-
Shall send the appropriate remote codec(s), the remote UDP port and the remote IP address to the IM-MGW. The remote UDP port and IP address refer to the destination of user plane data sent towards the IM CN subsystem. The remote codec(s) are the codec(s) the IM-MGW may select for user plane data sent towards the IM CN subsystem.

-
Shall indicate to the IM-MGW the appropriate local codec(s) and request a local IP address and UDP port. The local IP address and UDP port are used by the IM-MGW to receive user plane data from the IM CN subsystem. The local codec(s) are the codec(s) the IM-MGW may select to receive user plane data from the IM CN subsystem.

-
If DTMF support together with speech support is required, the reserve value indicator shall be set to "true".
-
May indicate that IP interface type is for MboIP.
The IM-MGW

-
Shall reply to the MGCF with the selected local codec(s) and the selected remote codec(s) and the selected local UDP port and IP address.

-
Shall reserve resources for those codec(s).

The MCGF shall send the local codec(s), UDP port and IP address to the IMS in the Session Progress (signal 9 in figure 34).

9.2.2.1.4
IM CN subsystem side session establishment

Dependent on what the MGCF receives in the PRACK message (signal 10 in figure 34), the MGCF may initiate the Configure IMS Resources procedure. If no SDP is received, or if the received SDP does not contain relevant changes compared to the previous SDP sent to the IMS in signal 9 in figure 34, the procedure is not invoked. Otherwise the MGCF shall use the Configure IMS Resources procedure to provide to the IM-MGW:
-
The appropriate remote codec(s), the remote UDP port and the remote IP address.

-
Optionally the appropriate local codec(s), UDP port and IP address.

-
If DTMF support together with speech support is required, the reserve value indicator shall be set to "true".
-
May indicate that IP interface type is for MboIP.
The IM-MGW shall:

-
Reply to the MGCF with the selected remote codec(s),

-
Reply to the MGCF with the selected local codec(s)if the MGCF supplied local codec(s),

-
Update the codec reservation and remote IP address and remote UDP port in accordance with the received information.

The MGCF shall include the selected codec(s) and UDP port and IP address in a 200 OK (PRACK) (signal 11 in figure 34) sent back to the IMS.

9.2.2.1.5
Through-connection

During the Prepare Bearer and Establish Bearer procedures, the MGCF shall either use the Change Through-Connection procedure to request the IM-MGW to backward through-connect the BICC terminations, or the MGCF shall use this procedure to both-way through-connect the BICC termination already on this stage (signal 7 in figure 34). During the Reserve IMS Connection Point procedure, the MGCF shall use the Change IMS Through-Connection procedure to request the IM-MGW to backward through-connect the IMS termination (signal 3 in figure 34).

When the MGCF receives the BICC:ANM answer indication, it shall request the IM-MGW to both-way through-connect the termination using the Change Through-Connection or Change IMS Through-Connection procedures (signal 22 in figure 34), unless those terminations are already both-way through-connected.

9.2.2.1.6
Codec handling

The IM-MGW may include a speech transcoder based upon the speech coding information provided to each termination.

9.2.2.1.7
Failure handling in MGCF

If any procedure between the MGCF and the IM-MGW is not completed successfully the default action by the MGCF is to release the session, as described in clause 9.2.6. If the MGCF receives a Bearer Released procedure from the IM-MGW the default action by the MGCF is to release the session as described in clause 9.2.7.

NOTE:
As an implementation option the MGCF may also decide for example to only release the resources in the IM-MGW that caused the failure, possibly select a new IM-MGW for the connection and continue the call establishment using new resources in the selected IM-MGW but such handling is outside of the scope of the present document.

9.2.2.1.8
Message sequence chart

Figure 34 shows the message sequence chart for the IM CN subsystem originating session with BICC forward bearer establishment where the selection of IM-MGW is done before the sending of the IAM. In the chart the MGCF requests the seizure of an IM CN subsystem side termination. When the APM is received from the succeeding node, the MGCF requests the seizure of a CS network side bearer termination and the establishment of the bearer. When the MGCF receives an answer indication, it requests the IM-MGW to both-way through-connect the terminations. Dashed lines represent optional or conditional messages.

[image: image61.wmf]MGCF

IM

-

MGW

1

. 

SIP

: 

INVITE 

2

. 

SIP

: 

100 

Trying

5

. 

BICC

: 

IAM 

6

. 

BICC

: 

APM 

7

. 

H

.

248

: 

ADD

.

req 

[

Context ID 

= 

C

1

, 

Termination ID 

= 

?

]

8

. 

H

.

248

: 

ADD

.

resp 

[

Context ID 

= 

C

1

, 

Termination ID 

= 

T

2

] 

3

. 

H

.

248

: 

ADD

.

req  

[

Context ID 

= 

?, 

Termination ID

=

?

]

4

. 

H

.

248

: 

ADD

.

resp 

[

Context ID 

= 

C

1

, 

Termination ID 

= 

T

1

]

9

. 

SIP

:

183 

Session Progress 

10

. 

SIP

: 

PRACK

11

. 

SIP

 

:

200 

OK 

(

PRACK

) 

12

. 

H

.

248

: 

MOD

.

req 

[

Context ID 

= 

C

1

,

Termination ID 

= 

T

1

]

13

. 

H

.

248

: 

MOD

.

resp 

[

Context ID 

= 

C

1

, 

Termination ID 

= 

T

1 

] 

Bearer Establishment

14

. 

SIP

: 

UPDATE

15

. 

SIP

 

:

200 

OK 

(

UPDATE

)

16

. 

BICC

: 

COT 

17

.

 

BICC

: 

ACM 

18

. 

SIP

 

:

180 

Ringing

22

. 

H

.

248

:  

MOD

.

req 

[

Context ID

=

C

1

, 

Termination ID 

= 

T

1

]

23

. 

H

.

248

: 

MOD

.

resp 

[

Context ID 

= 

C

1

, 

Termination ID 

= 

T

1

] 

21

.

 

BICC

: 

ANM 

19

. 

SIP

: 

PRACK

20

. 

SIP

 

:

200 

OK 

(

PRACK

)

24

. 

SIP

 

:

200 

OK 

(

INVITE

)

25

. 

SIP

: 

ACK

CALL IN ACTIVE STATE

Reserve IMS Connection 

Point and Configure Remote 

Resourcest

,

Change IMS Through

-

Connection 

= 

backward 

Establish Bearer

, 

Change

 Through

-

Connection 

= 

both 

Configure IMS 

Resources

Change IMS 

Through

-

Connection 

= 

both 

Only if 

SIP 

Precon

-

ditions 

are 

used

Only if BICC 

continuity check 

and

/

or SIP 

Preconditions 

are used

Optional

if SIP 

100 

rel 

is  

used

if SIP 

100 

rel 

is  used


Figure 34: Basic IM CN Subsystem originating session, BICC forward bearer establishment
(message sequence chart)
9.2.2.2
BICC backward bearer establishment

9.2.2.2.1
IM-MGW selection

The MGCF shall select an IM-MGW for the bearer connection before it performs the IM CN subsystem session establishment or the CS network side bearer establishment, and before it sends the IAM (signal 8 in figure 35).

9.2.2.2.2
IM CN subsystem side termination reservation

On receipt of an initial INVITE (signal 1in figure 35) the MGCF shall initiate the Reserve IMS Connection Point and Configure Remote Resources procedure (signal 3 and 4 in figure 35). From the received SDP and local configuration data the MGCF:

-
Shall send the appropriate remote codec(s), the remote UDP port and the remote IP address to the IM-MGW. The remote UDP port and IP address refer to the destination of user plane data sent towards the IM CN subsystem. The remote codec(s) are the codec(s) the IM-MGW may select for user plane data sent towards the IM CN subsystem.

-
Shall indicate to the IM-MGW the appropriate local codec(s) and request a local IP address and UDP port. The local UDP port and IP address are used by the IM-MGW to receive user plane data from the IM CN subsystem. The local codec(s) are the codec(s) the IM-MGW may select to receive user plane data from the IM CN subsystem.

-
If DTMF support together with speech support is required, the reserve value indicator shall be set to "true".
-
May indicate that IP interface type is for MboIP.
The IM-MGW shall

-
Reply to the MGCF with the selected local codec(s) and the selected remote codec(s) and the selected local UDP port and IP address.

-
Reserve resources for those codec(s).

The MCGF shall send the local codec(s), UDP port and IP address to the IMS in the Session Progress (signal 5 in figure 35).

9.2.2.2.3
IM CN subsystem side session establishment

Dependent on what the MGCF receives in the PRACK message (signal 9 in figure 35) the MGCF may initiate the Select Configure IMS Resources procedure (signals 10 and 11 in figure 35). If no SDP is received, or if the received SDP does not contain relevant changes compared to the previous SDP the procedure is not invoked. Otherwise the MGCF shall use the Configure IMS Resources procedure to provide to the IM-MGW:
-
the appropriate remote codec(s), the remote UDP port and the remote IP address.

-
optionally if DTMF support together with speech support is required, the reserve value indicator shall be set to "true".
-
May indicate that IP interface type is for MboIP.
The IM-MGW shall:

-
Reply to the MGCF with the selected remote codec(s).

-
Reply to the MGCF with the selected local codec(s), if the MGCF supplied local codec(s).

-
Update the codec reservation and remote IP address and remote UDP port in accordance with the received information.

The MGCF shall include the selected codec(s), IP address and UDP port in an 200 OK (PRACK) (signal 12 in figure 35) sent back to the IMS

9.2.2.2.4
CS network side bearer establishment

The MGCF shall request the IM-MGW to prepare for the CS network side bearer establishment using the Prepare Bearer procedure before sending the IAM to the succeeding node. Within this procedure, the MGCF shall request the IM-MGW to provide a bearer address and a binding reference, and the MGCF shall either provide the IM-MGW with the preferred bearer characteristics or it shall request the IM-MGW to select and provide the bearer characteristics (signal 6 in figure 35). After the IM-MGW has replied with the bearer address, the binding reference and the bearer characteristics (if requested), the MGCF sends the IAM to the succeeding node (signal 8 in figure 35).

9.2.2.2.5
Through-connection

During the Prepare Bearer procedure, the MGCF shall either use the Change Through-Connection procedure to request the IM-MGW to backward through-connect the BICC termination, or the MGCF shall use this procedure to both-way through-connect the BICC termination already on this stage (signal 6 in figure 35). During the Reserve IMS Connection Point procedure, the MGCF shall use the Change IMS Through-Connection procedure to request the IM-MGW to backward through-connect the IMS termination (signal 3 in figure 35).

When the MGCF receives the BICC:ANM answer indication, it shall request the IM-MGW to both-way through-connect the terminations using the Change Through-Connection or Change IMS Through-Connection procedures (signal 21 in figure 35), unless those terminations are already both-way through-connected.

9.2.2.2.6
Codec handling

The IM-MGW may include a speech transcoder based upon the speech coding information provided to each termination.

9.2.2.2.7
Failure handling in MGCF

If any procedure between the MGCF and the IM-MGW is not completed successfully the default action by the MGCF is to release the session as described in clause 9.2.6. If the MGCF receives a Bearer Released procedure from the IM-MGW the default action by the MGCF is to release the session, as described in clause 9.2.7.

NOTE:
As an implementation option the MGCF may also decide for example to only release the resources in the IM-MGW that caused the failure, possibly select a new IM-MGW for the connection and continue the call establishment using new resources in the selected IM-MGW but such handling is outside of the scope of the present document.

9.2.2.2.8
Message sequence chart

Figure 35 shows the message sequence chart for the IM CN subsystem originating session with BICC backward bearer establishment. In the chart the MGCF requests the seizure of an IM CN subsystem side termination and a CS network side bearer termination. When the MGCF receives an answer indication, it requests the IM-MGW to both-way through-connect the terminations. Dashed lines represent optional or conditional messages.

[image: image62.wmf]MGCF

IM

-

MGW

1

. 

SIP

: 

INVITE 

2

. 

SIP

: 

100 

Trying

8

. 

BICC

: 

IAM 

6

. 

H

.

248

: 

ADD

.

req 

[

Context ID 

= 

C

1

, 

Termination ID 

= 

?

]

7

. 

H

.

248

: 

ADD

.

resp 

[

Context ID 

= 

C

1

, 

Termination ID 

= 

T

2

] 

3

. 

H

.

248

: 

ADD

.

req 

[

Context ID 

= 

?, 

Termination ID

=

?

]

4

. 

H

.

248

: 

ADD

.

resp 

[

Context ID 

= 

C

1

, 

Termination ID 

= 

T

1

]

5

. 

SIP

:

183 

Session 

Progress 

9

. 

SIP

: 

PRACK

12

. 

SIP

 

:

200 

OK  

(

PRACK

)

 10

. 

H

.

248

: 

MOD

.

req 

[

Context ID 

= 

C

1

,

Termination ID 

= 

T

1

]

11

. 

H

.

248

: 

MOD

.

resp 

[

Context ID 

= 

C

1

, 

Termination ID 

= 

T

1 

] 

Bearer Establishment

13

. 

SIP

: 

UPDATE

14

. 

SIP

 

:

200 

OK 

(

UPDATE

)

15

. 

BICC

: 

COT 

16

.

 

BICC

: 

ACM 

17

. 

SIP

 

:

180 

Ringing

21

. 

H

.

248

:  

MOD

.

req  

[

Context ID

=

C

1

, 

Termination ID 

= 

T

1

]

22

. 

H

.

248

: 

MOD

.

resp 

[

Context ID 

= 

C

1

, 

Termination ID 

= 

T

1

] 

20

.

 

BICC

: 

ANM 

18

. 

SIP

: 

PRACK

19

. 

SIP

 

:

200 

OK 

(

PRACK

)

25

. 

SIP

 

:

200 

OK 

(

INVITE

)

26

. 

SIP

: 

ACK

CALL IN ACTIVE STATE

Reserve IMS Connection Point

and Configure Remote Resources

,

Change IMS Through

-

Connection 

= 

backward 

)

Configure IMS Resources

Prepare Bearer

,

Change Through

-

Connection 

= 

both

Change IMS Through

-

Connection 

= 

both

Only if 

SIP 

Precon

-

ditions 

are 

used

Only if BICC 

continuity check 

and

/

or SIP 

Preconditions 

are used

Optional

if SIP 

100 

rel 

is  used

if SIP 

100 

rel 

is  used


Figure 35: Basic IM CN Subsystem originating session, BICC backward bearer establishment (message sequence chart)

9.2.2.3
ISUP

9.2.2.3.1
IM-MGW selection

The MGCF shall select an IM-MGW with circuits to the given destination in the CS domain before it performs the IM CN subsystem session establishment and before it sends the IAM (signal 8 in figure 36).

9.2.2.3.2
IM CN subsystem side termination reservation

On receipt of an initial INVITE (signal 1 in figure 36) the MGCF shall initiate the Reserve IMS Connection Point and Configure Remote Resources procedure (signal 3 and 4 in figure 36). From the received SDP and local configuration data the MGCF:
-
shall send the appropriate remote codec(s), the remote UDP port and the remote IP address to the IM-MGW. The remote UDP port and IP address refer to the destination of user plane data sent towards the IM CN subsystem. The remote codec(s) are the codec(s) the IM-MGW may select for user plane data sent towards the IM CN subsystem.

-
shall indicate to the IM-MGW the appropriate local codec(s) and request a local IP address and UDP port. The local IP address and UDP port are used by the IM-MGW to receive user plane data from the IM CN subsystem. The local codec(s) are the codec(s) the IM-MGW may select to receive user plane data from the IM CN subsystem.

-
If DTMF support together with speech support is required, the reserve value indicator shall be set to "true".
-
May indicate that IP interface type is for MboIP.
The IM-MGW shall

-
reply to the MGCF with the selected local codec(s) and the selected remote codec(s) and the selected local UDP port and IP address.

-
reserve resources for those codec(s).

The MCGF shall send selected local codec(s) and the selected remote codec and the selected local UDP port and IP address to the IMS in the Session Progress (signal 5 in figure 36)

9.2.2.3.3
IM CN subsystem side session establishment

Dependent on what the MGCF receives in the PRACK message (signal 9 in figure 35) the MGCF may initiate the Configure IMS Resources procedure. If no SDP is received, or if the received SDP does not contain relevant changes compared to the previous SDP, the procedure is not invoked. Otherwise the MGCF shall use the Configure IMS Resources procedure to provide to the IM-MGW:
-
the appropriate remote codec(s), the remote UDP port and the remote IP address.

-
optionally the appropriate local codec(s), UDP port and IP address.

-
If DTMF support together with speech support is required, the reserve value indicator shall be set to "true".
-
May indicate that IP interface type is for MboIP.
The IM-MGW shall:

-
reply to the MGCF with the selected remote codec.

-
reply to the MGCF with the selected local codec(s), if the MGCF supplied local codec(s).

-
update the codec reservation and remote IP address and UDP port in accordance with the received information.

The MGCF shall include the selected codec(s) UDP port and IP address in 200 OK (PRACK) (signal 12 in figure 36) sent back to the IMS.

9.2.2.3.4
CS network side circuit reservation

The MGCF shall request the IM-MGW to reserve a circuit using the Reserve TDM Circuit procedure. The MGCF sends the IAM to the succeeding node including the reserved circuit identity.

9.2.2.3.5
Through-connection

During the Reserve TDM Circuit and Reserve IMS Connection Point procedures, the MGCF shall either use the Change TDM Through-Connection procedure to request the IM-MGW to backward through-connect the termination, or the MGCF shall use this procedure to both-way through-connect the TDM termination already on this stage (signal 6 in figure 36). During the Reserve IMS connection Point procedure, the MGCF shall use the Change IMS through-connection procedure to request the IM-MGW to backward through-connect the IMS termination (signal 3 in figure 36).

When the MGCF receives the ISUP:ANM answer indication, it shall request the IM-MGW to both-way through-connect the terminations using the Change IMS Through-Connection or Change TDM Through-Connection procedures (signal 21 in figure 36), unless those terminations are already both-way through-connected.

9.2.2.3.6
Continuity check

The MGCF may request a continuity check on the connection towards the CS network within the IAM message. In this case, the MGCF shall use the Continuity Check procedure towards the IM-MGW to request the generation of a continuity check tone on the TDM termination. The IM-MGW shall then use the Continuity Check Verify procedure to notify the MGCF of an incoming continuity check tone on the corresponding circuit. In addition to other conditions detailed in Section 7, the MGCF shall wait until receiving this notification before sending the COT. (Not depicted in figure 36)

9.2.2.3.7
Codec handling

The IM-MGW may include a speech transcoder based upon the speech coding information provided to each termination.

9.2.2.3.8
Voice processing function

A voice processing function located on the IM-MGW may be used to achieve desired acoustic quality on the terminations. If the voice processing function is used, the MGCF shall request the activation of it in the termination towards the CS network using the Activate TDM Voice Processing Function procedure (signal 23 in figure 36).

9.2.2.3.9
Failure handling in MGCF

If any procedure between the MGCF and the IM-MGW is not completed successfully session shall be released as described in clause 9.2.6. If the MGCF receives a Bearer Released procedure from the IM-MGW the default action by the MGCF is to release the session as described in clause 9.2.7.
9.2.2.3.10
Message sequence chart

Figure 36 shows the message sequence chart for the IM CN subsystem originating session. In the chart the MGCF requests the seizure of an IM CN subsystem side termination and a CS network side bearer termination. When the MGCF receives an answer indication, it requests the IM-MGW to both-way through-connect the terminations. The MGCF requests the possible activation of the voice processing functions for the bearer terminations. Dashed lines represent optional or conditional messages.

[image: image63.wmf]MGCF

IM

-

MGW

1

. 

SIP

: 

INVITE 

2

. 

SIP

: 

100 

Trying

8

. 

ISUP

: 

IAM 

6

. 

H

.

248

: 

ADD

.

req 

[

Context ID 

= 

C

1

, 

Termination ID 

= 

T

2

]

7

. 

H

.

248

: 

ADD

.

resp 

[

Context ID 

= 

C

1

, 

Termination ID 

= 

T

2

] 

3

. 

H

.

248

: 

ADD

.

req 

[

Context ID 

= 

?, 

Termination ID

=

?

]

4

. 

H

.

248

: 

ADD

.

resp 

[

Context ID 

= 

C

1

, 

Termination ID 

= 

T

1

]

5

. 

SIP

:

183 

Session 

Progress 

9

. 

SIP

: 

PRACK

12

. 

SIP

 

:

200 

OK  

(

PRACK

)

 10

. 

H

.

248

: 

MOD

.

req 

[

Context ID 

= 

C

1

,

Termination ID 

= 

T

1

]

11

. 

H

.

248

: 

MOD

.

resp 

[

Context ID 

= 

C

1

, 

Termination ID 

= 

T

1 

] 

13

. 

SIP

: 

UPDATE

14

. 

SIP

 

:

200 

OK 

(

UPDATE

)

15

. 

ISUP

: 

COT 

16

.

 

ISUP

: 

ACM 

17

. 

SIP

 

:

180 

Ringing

21

. 

H

.

248

:  

MOD

.

req  

[

Context ID

=

C

1

, 

Termination ID 

= 

T

1

]

22

. 

H

.

248

: 

MOD

.

resp 

[

Context ID 

= 

C

1

, 

Termination ID 

= 

T

1

] 

20

.

 

ISUP

: 

ANM 

18

. 

SIP

: 

PRACK

19

. 

SIP

 

:

200 

OK 

(

PRACK

)

25

. 

SIP

 

:

200 

OK 

(

INVITE

)

26

. 

SIP

: 

ACK

CALL IN ACTIVE STATE

23

. 

H

.

248

:  

MOD

.

req 

) [

Context ID

=

C

1

, 

Termination ID 

= 

T

2

]

24

. 

H

.

248

: 

MOD

.

resp 

[

Context ID 

= 

C

1

, 

Termination ID 

= 

T

2

] 

Reserve IMS Connection

Point

, 

and Configure

Remote Resources

Change IMSThrough

-

Connection 

= 

backward 

Configure IMS 

Resources

Reserve TDM Circuit

,

Change  TDM Through

-

Connection

= 

both

Change IMS Through

-

Connection 

= 

both

Activate TDM Voice 

Prosessing Function

Only if 

SIP 

Precon

-

ditions 

are 

used

Only if BICC 

continuity check 

and

/

or SIP 

Preconditions 

are used

Optional

if SIP 

100 

rel 

is  used

if SIP 

100 

rel 

is  used


Figure 36: Basic IM CN Subsystem originating session, ISUP (message sequence chart)

9.2.3
Basic CS network originated session

9.2.3.1
BICC forward bearer establishment

9.2.3.1.1
IM-MGW selection

The MGCF shall select an IM-MGW for the bearer connection before it performs the IM CN subsystem session establishment or the CS network side bearer establishment.

9.2.3.1.2
IM CN subsystem side termination reservation

The MGCF shall derive from configuration data one or several appropriate local codec(s) the IM-MGW may use to receive user plane data from the IM CN subsystem. The MGCF shall use the Reserve IMS Connection Point procedure (signals 2 and 3 in figure 37). Within this procedure, the MGCF shall indicate the local codec(s) and request a local IP address and UDP port from the IM-MGW. The local IP address and UDP port are used by the IM-MGW to receive user plane data from the IM CN subsystem. If DTMF support together with speech support is required, or if the resources for multiple speech codecs shall be reserved at this stage, the reserve value indicator shall be set to "true".

The MGCF may also indicate that the IP interface type is for MboIP.

The IM-MGW shall reply to the MGCF with the selected local codec(s) and the selected local IP address and UDP port.

The MGCF shall send this information in the INVITE (signal 4 in figure 37) to the IM CN subsystem.

9.2.3.1.3
IM CN subsystem side session establishment

The MGCF shall use the Configure IMS Resources procedure (signals 7 and 8 or 23a and 23b in figure 37) to provide configuration data (derived from SDP received in signal 6 in figure 37 and local configuration data) to the IM-MGW as detailed below:

-
The MGCF shall indicate the remote IP address and UDP port, i.e. the destination IP address and UDP port for data sent in the user plane towards the IM CN subsystem,

-
The MGCF shall indicate the remote codec(s), i.e. the speech codec(s) for data sent in the user plane towards the IM CN subsystem.

-
The MGCF may indicate the local codec(s) and the local IP address and UDP port. The MGCF shall indicate the local codec(s) if a change is required.

-
IF DTMF support together with speech support is required, the reserve value indicator shall be set to "true".
-
The MGCF may indicate that IP interface type is for MboIP.
The IM-MGW shall reply with the selected remote codec(s) and reserve resources for these codec(s). If local codec(s) were received, the IM-MGW shall also reply with the selected local codec(s) and reserve the corresponding resources.

If the selected local codec(s) differ from the codec(s) received in the SDP of signal 6 in figure 37 (if any), the MGCF shall send the local reserved codec(s), and the local IP address and UDP port in the PRACK (signal 9 in figure 37) to the IMS.
If the selected local codec(s) differ from the codec(s) received in the SDP of signal 23 in figure 37 (if any), the MGCF shall send the local reserved codec(s), and the local IP address and UDP port in an re-INVITE or UPDATE (not depicted in figure 37) to the IMS.

9.2.3.1.4
CS network side bearer establishment

The MGCF shall request the IM-MGW to prepare for the CS network side bearer establishment using the Prepare Bearer procedure (signals 11 and 12 in figure 37). Within this procedure, the MGCF shall request the IM-MGW to provide a bearer address, a binding reference and optionally notify when the bearer is established. The MGCF shall also provide the IM-MGW with the bearer characteristics that was received from the preceding node in the IAM. After the IM-MGW has replied with the bearer address and the binding reference, the MGCF provides the APM message (signals 13 in figure 37) to the preceding node. The MGCF may also provide the IM-MGW-id in the APM message.

9.2.3.1.5
Called party alerting

The MGCF shall request the IM-MGW to provide an awaiting answer indication (ringing tone) to the calling party using the Send Tone procedure (signals 20 and 21 in figure 37) , when the following condition is satisfied:

-
the MGCF receives the first 180 Ringing message, unless this message or some previous SIP provisional response contained a P-Early-Media header field that authorizes early media.
9.2.3.1.6
Called party answer

When the MGCF receives a 200 OK message (signal 23 in figure 34), it shall request the IM-MGW to stop providing the ringing tone to the calling party using the Stop Tone procedure (signals 26 and 27 in figure 37).

9.2.3.1.7
Through-Connection

During the Prepare Bearer procedure, the MGCF shall either use the Change Through-Connection procedure to request the IM-MGW to backward through-connect the BICC termination, or the MGCF shall use this procedure to both-way through-connect the BICC termination already on this stage (signals 11 and 12 in figure 37). During the Reserve IMS Connection Point procedure, the MGCF shall use the Change IMS Through-Connection procedure to request the IM-MGW to backward through-connect the IMS termination (signals 2 and 3 in figure 37).

When the MGCF receives the SIP 200 OK(INVITE) (signal 23 in figure 37), it requests the IM-MGW to both-way through-connect the terminations using the Change IMS Through-Connection or Change Through-Connection procedures (signals 28 and 29 in figure 37), unless those terminations are already both-way through-connected.

9.2.3.1.8
Codec handling

The IM-MGW may include a speech transcoder based upon the speech coding information provided to each termination.

9.2.3.1.9
Failure handling in MGCF

If any procedure between the MGCF and the IM-MGW is not completed successfully, the default action by the MGCF is to release the session as described in clause 9.2.6. If the MGCF receives a Bearer Released procedure from the IM-MGW the default action by the MGCF is to release the session, as described in clause 9.2.7.

NOTE:
As an implementation option the MGCF may also decide for example to only release the resources in the IM-MGW that caused the failure, possibly select a new IM-MGW for the connection and continue the call establishment using new resources in the selected IM-MGW but such handling is outside of the scope of the present document.

9.2.3.1.10
Message sequence chart

Figure 37 shows the message sequence chart for the CS network originating session with BICC forward bearer establishment. In the chart the MGCF requests the seizure of the IM CN subsystem side termination and CS network side bearer termination. When the MGCF receives an answer indication, it requests the IM-MGW to both-way through-connect the terminations. Dashed lines represent optional or conditional messages.

[image: image64.wmf]IM

-

MGW

MGCF

4

. 

SIP

: 

INVITE 

5

. 

SIP

: 

100 

Trying

1

. 

BICC

: 

IAM 

11

. 

H

.

248

: 

ADD

.

req 

[

Context ID 

= 

C

1

, 

Termination ID 

= 

?

]

12

. 

H

.

248

: 

ADD

.

resp 

[

Context ID 

= 

C

1

, 

Termination ID 

= 

T

2

] 

2

. 

H

.

248

: 

ADD

.

req  

[

Context ID 

= 

?, 

Termination ID

=

?

]

3

. 

H

.

248

: 

ADD

.

resp 

[

Context ID 

= 

C

1

, 

Termination ID 

= 

T

1

]

6

. 

SIP

:

183 

Session 

Progress 

9

. 

SIP

: 

PRACK

10

. 

SIP

 

:

200 

OK  

(

PRACK

)

7

. 

H

.

248

: 

MOD

.

req Context ID 

= 

C

1

,

Termination ID 

= 

T

1

]

  8

. 

H

.

248

: 

MOD

.

resp 

[

Context ID 

= 

C

1

, 

Termination ID 

= 

T

1 

] 

Bearer Establishment

15

. 

SIP

: 

UPDATE

16

. 

SIP

 

:

200 

OK 

(

UPDATE

)

14

. 

BICC

: 

COT 

17

. 

SIP

 

:

180 

Ringing

18

. 

SIP

: 

PRACK

22

. 

SIP

 

:

200 

OK 

(

PRACK

)

13

. 

BICC

: 

APM 

20

. 

H

.

248

: 

MOD

.

req  

[

Context ID 

= 

C

1

, 

Termination ID 

= 

T

2

]

21

. 

H

.

248

: 

MOD

.

resp 

[

Context ID 

= 

C

1

, 

Termination ID 

= 

T

2

] 

19

.

 

BICC

: 

ACM 

Reserve IMS Connection Point

, 

Change IMS Through

-

Connection 

= 

backward 

Configure IMS  Resources

Prepare Bearer

,

Change Through

-

Connection

= 

both

Send Tone

if SIP 

Precon

-

ditions  

and

/

or 

100 

rel 

are used

if SIP 

Precon

-

ditions  

and

/

or 

100 

rel 

are used

Optional

if 

continuity  

check is 

used


Figure 37/1: Basic CS Network Originating Session, Forward Bearer Establishment (message sequence chart)


[image: image65.wmf]IM

-

MGW

MGCF

24

. 

H

.

248

:  

MOD

.

req

 [

Context ID

=

C

1

, 

Termination ID 

= 

T

2

]

25

. 

H

.

248

: 

MOD

.

resp 

[

Context ID 

= 

C

1

, 

Termination ID 

= 

T

2

] 

29

. 

SIP

: 

ACK

26

. 

H

.

248

:  

MOD

.

req  

[

Context ID

=

C

1

, 

Termination ID 

= 

T

1

]

27

. 

H

.

248

: 

MOD

.

resp 

[

Context ID 

= 

C

1

, 

Termination ID 

= 

T

1

] 

28

.

 

BICC

: 

ANM 

23

. 

SIP

 

:

200 

OK 

(

INVITE

)

CALL IN ACTIVE STATE

Stop 

Tone 

Change IMS 

Through

-

Connection  

= 

both

23

a

. 

H

.

248

: 

MOD

.

req 

[

Context ID 

= 

C

1

,

Termination ID 

= 

T

1

]

  

23

b

. 

H

.

248

: 

MOD

.

resp

[

Context ID 

= 

C

1

, 

Termination ID 

= 

T

1 

] 

Configure 

IMS  

Resources

if SIP 

Precon

-

ditions  

and

100 

rel 

are not 

used

If  tone 

was 

inserted


Figure 37/2: Basic CS Network Originating Session, Forward Bearer Establishment
 (message sequence chart continue)

9.2.3.2
BICC Backward bearer establishment

9.2.3.2.1
IM-MGW selection

The MGCF shall select an IM-MGW for the bearer connection before it performs the IM CN subsystem session establishment or the CS network side bearer establishment.

9.2.3.2.2
CS network side bearer establishment

The MGCF shall request the IM-MGW to establish a bearer using the Establish Bearer procedure (signals 2 and 3 in figure 38). The MGCF provides the IM-MGW with the bearer address, the binding reference and the bearer characteristics that were received from the preceding node in the IAM (signal 1 in figure 38).

9.2.3.2.3
IM CN subsystem side termination reservation

The MGCF shall derive from configuration data one or several appropriate local codec(s) the IM-MGW may use to receive user plane data from the IM CN subsystem. The MGCF shall use the Reserve IMS Connection Point procedure (signals 2 and 3 in figure 38). Within this procedure, the MGCF shall indicate the local codec(s) and request a local IP address and UDP port from the IM-MGW. The local IP address and UDP port are used by the IM-MGW to receive user plane data from the IM CN subsystem. If DTMF support together with speech support is required, or if the resources for multiple speech codecs shall be reserved at this stage, the reserve value indicator shall be set to "true".

The MGCF may also indicate that the IP interface type is for MboIP.

The IM-MGW shall reply to the MGCF with the selected local codec(s) and the selected local IP address and UDP port.

The MGCF shall send this information in the INVITE (signal 6 in figure 38) to the IM CN subsystem.
9.2.3.2.4
IM CN subsystem side session establishment

The MGCF shall use the Configure IMS Resources procedure (signals 9 and 10 or 22a and 22b in figure 38) to provide configuration data (derived from SDP received in signal 8 in figure 38 and local configuration data) to the IM-MGW as detailed below:

-
The MGCF shall indicate the remote IP address and UDP port, i.e. the destination IP address and UDP port for data sent in the user plane towards the IM CN subsystem

-
The MGCF shall indicate the remote codec(s), i.e. the speech codec(s) for data sent in the user plane towards the IM CN subsystem.

-
The MGCF may indicate the local codec(s) and the local IP address and UDP port. The MGCF shall indicate the local codec(s) if a change is required.

-
If DTMF support together with speech support is required, the reserve value indicator shall be set to "true".
-
The MGCF may indicate that IP interface type is for MboIP.
The IM-MGW shall reply with the selected remote codec(s) and reserve resources for this codec. If local codec(s) were received, the IM-MGW shall also reply with the selected local codec(s) and reserve the corresponding resources.

If the selected local codec(s) differ from the codec(s) received in the SDP of signal 8 in figure 38 (if any), the MGCF shall send the reserved speech codec(s), and the local IP address and UDP port in the PRACK (signal 11 in figure 38) to the IMS.
If the selected local codec(s) differ from the codec(s) received in the SDP of signal 22 in figure 38 (if any), the MGCF shall send the local reserved codec(s), and the local IP address and UDP port in an re-INVITE or UPDATE (not depicted in figure 38) to the IMS.
9.2.3.2.5
Called party alerting

The MGCF shall request the IM-MGW to provide an awaiting answer indication (ringing tone) to the calling party using the Send Tone procedure (signals 19 and 20 in figure 38) , when the following conditions is satisfied:

-
the MGCF receives the first 180 Ringing message, unless this message or some previous SIP provisional response contained a P-Early-Media header field that authorizes early media.
9.2.3.2.6
Called party answer

When the MGCF receives a 200 OK message (signal 22 in figure 38), it shall request the IM-MGW to stop providing the ringing tone to the calling party using the Stop Tone procedure (signals 23 and 24 in figure 38).

9.2.3.2.7
Through-Connection

During the Establish Bearer procedure, the MGCF shall either use the Change Through-Connection procedure to request the IM-MGW to backward through-connect the BICC termination, or the MGCF shall use this procedure to both-way through-connect the BICC termination already on this stage (signals 2 and 3 in figure 38). During the Reserve IMS Connection Point procedure, the MGCF shall use the Change IMS Through-Connection procedure to request the IM-MGW to backward through-connect the IMS termination (signals 4 and 5 in figure 38).

When the MGCF receives the SIP 200 OK(INVITE) (signal 22 in figure 38), it shall request the IM-MGW to both-way through-connect the bearer using the Change IMS Through-Connection or Change Through-Connection procedure (signals 25 and 26 in figure 38), unless those terminations are already both-way through-connected.

9.2.3.2.8
Codec handling

The IM-MGW may include a speech transcoder based upon the speech coding information provided to each termination.

9.2.3.2.9
Failure handling in MGCF

If any procedure between the MGCF and the IM-MGW is not completed successfully, the default action by the MGCF is to release the session as described in clause 9.2.6. If the MGCF receives a Bearer Released procedure from the IM-MGW the default action by the MGCF is to release the session as described in clause 9.2.7.

NOTE:
As an implementation option the MGCF may also decide for example to only release the resources in the IM-MGW that caused the failure, possibly select a new IM-MGW for the connection and continue the call establishment using new resources in the selected IM-MGW but such handling is outside of the scope of the present document.

9.2.3.2.10
Message sequence chart

Figure 38 shows the message sequence chart for the CS network originating session with BICC backward bearer establishment. In the chart the MGCF requests seizure of the IM CN subsystem side termination and CS network side bearer termination. When the MGCF receives an answer indication, it requests the IM-MGW to both-way through-connect the terminations. Dashed lines represent optional or conditional messages.

[image: image66.wmf]IM

-

MGW

MGCF

6

. 

SIP

: 

INVITE 

7

. 

SIP

: 

100 

Trying

1

. 

BICC

: 

IAM 

2

. 

H

.

248

: 

ADD

.

req  

[

Context ID 

= 

?, 

Termination ID 

= 

?

]

3

. 

H

.

248

: 

ADD

.

resp 

[

Context ID 

= 

C

1

, 

Termination ID 

= 

T

2

] 

4

. 

H

.

248

: 

ADD

.

req 

[

Context ID 

= 

C

1

, 

Termination ID

=

?

]

5

. 

H

.

248

: 

ADD

.

resp 

[

Context ID 

= 

C

1

, 

Termination ID 

= 

T

1

]

8

. 

SIP

:

183 

Session 

Progress 

11

. 

SIP

: 

PRACK

12

. 

SIP

 

:

200 

OK 

(

PRACK

) 

9

. 

H

.

248

: 

MOD

.

req 

[

Context ID 

= 

C

1

,

Termination ID 

= 

T

1

]

10

.

H

.

248

: 

MOD

.

resp 

[

Context ID 

= 

C

1

, 

Termination ID 

= 

T

1 

] 

Bearer Establishment

14

. 

SIP

: 

UPDATE

15

. 

SIP

 

:

200 

OK 

(

UPDATE

)

13

. 

BICC

: 

COT 

16

. 

SIP

 

:

180 

Ringing

17

. 

SIP

: 

PRACK

21

. 

SIP

 

:

200 

OK 

(

PRACK

)

19

. 

H

.

248

: 

MOD

.

req  

[

Context ID 

= 

C

1

, 

Termination ID 

= 

T

2

]

20

. 

H

.

248

: 

MOD

.

resp 

[

Context ID 

= 

C

1

, 

Termination ID 

= 

T

2

] 

18

.

 

BICC

: 

ACM 

Establish Bearer

, 

Change

Through

-

Connection  

= 

both 

Reserve IMS Connection Point

,

Change IMSThrough

-

Connection  

= 

backward 

Configure IMS Resources

Send Tone

if SIP 

Precon

-

ditions  

and

/

or 

100 

rel 

are used

if SIP 

Precon

-

ditions  

and

/

or 

100 

rel 

are used

Optional

if 

continuity  

check is 

used


Figure 38/1: Basic CS Network Originating Session, BICC Backward Bearer Establishment (message sequence chart)


[image: image67.wmf]IM

-

MGW

MGCF

23

. 

H

.

248

:  

MOD

.

req 

 [

Context ID

=

C

1

, 

Termination ID 

= 

T

2

]

24

. 

H

.

248

: 

MOD

.

resp 

[

Context ID 

= 

C

1

, 

Termination ID 

= 

T

2

] 

28

. 

SIP

: 

ACK

25

. 

H

.

248

:  

MOD

.

req 

 [

Context ID

=

C

1

, 

Termination ID 

= 

T

1

]

26

. 

H

.

248

: 

MOD

.

resp 

[

Context ID 

= 

C

1

, 

Termination ID 

= 

T

1

] 

27

.

 

BICC

: 

ANM 

22

. 

SIP

 

:

200 

OK 

(

INVITE

)

CALL IN ACTIVE STATE

Stop 

Tone

Change IMS Through

-

Connection  

= 

both

22

a

. 

H

.

248

: 

MOD

.

req 

[

Context ID 

= 

C

1

,

Termination ID 

= 

T

1

]

  

22

b

. 

H

.

248

: 

MOD

.

resp

[

Context ID 

= 

C

1

, 

Termination ID 

= 

T

1 

] 

if SIP 

Precon

-

ditions  

and

100 

rel 

are not 

used

If  tone 

was 

inserted

Configure 

IMS  

Resources


Figure 38/2: Basic CS Network Originating Session, BICC Backward Bearer Establishment
(message sequence chart continue)

9.2.3.3
ISUP

9.2.3.3.1
IM-MGW selection

The MGCF selects the IM-MGW based on the received circuit identity in the IAM.

9.2.3.3.2
CS network side circuit reservation

The MGCF shall request the IM-MGW to reserve a circuit using the Reserve TDM Circuit procedure.

9.2.3.3.3
IM CN subsystem side termination reservation

The MGCF shall derive from configuration data one or several appropriate local codec(s) the IM-MGW may use to receive user plane data from the IM CN subsystem. The MGCF shall use the Reserve IMS Connection Point procedure (signals 2 and 3 in figure 39). Within this procedure, the MGCF shall indicate the local codec(s) and request a local IP address and UDP port from the IM-MGW. The local IP address and UDP port are used by the IM-MGW to receive user plane data from the IM CN subsystem. If DTMF support together with speech support is required, or if the resources for multiple speech codecs shall be reserved at this stage, the reserve value indicator shall be set to "true".

The MGCF may also indicate that the IP interface type is for MboIP.

The IM-MGW shall reply to the MGCF with the selected local codec(s) and the selected local IP address and UDP port.

The MGCF shall send this information in the INVITE (signal 6 in figure 39) to the IM CN subsystem.

9.2.3.3.4
IM CN subsystem side session establishment

The MGCF shall use the Configure IMS Resources procedure (signals 9 and 10 or 22a and 22b in figure 39) to provide configuration data (derived from SDP received in signal 8 in figure 39 and local configuration data) as detailed below:

-
The MGCF shall indicate the remote IP address and UDP port, i.e. the destination IP address and UDP port for data sent in the user plane towards the IM CN subsystem.

-
The MGCF shall indicate the remote codec(s), i.e. the speech codec(s) for data sent in the user plane towards the IM CN subsystem.

-
The MGCF may indicate the local codec(s) and the local IP address and UDP port. The MGCF shall indicate the local codec(s) if a change is required.

-
If DTMF support together with speech support is required, the reserve value indicator shall be set to "true".
-
The MGCF may indicate that IP interface type is for MboIP.
The IM-MGW shall reply with the selected remote codec(s) and reserve resources for these codec(s). If local codec(s) were received, the IM-MGW shall also reply with the selected local codec(s) and reserve the corresponding resources.

If the selected local codec(s) differ from the codec(s) received in the SDP of signal 8 in figure 39 (if any), the MGCF shall send the reserved speech codec(s), and the local IP address and UDP port in the PRACK (signal 11 in figure 39) to the IMS.
If the selected local codec(s) differ from the codec(s) received in the SDP of signal 22 in figure 39 (if any), the MGCF shall send the local reserved codec(s), and the local IP address and UDP port in an re-INVITE or UPDATE (not depicted in figure 39) to the IMS.
9.2.3.3.5
Called party alerting

The MGCF shall request the IM-MGW to provide an awaiting answer indication (ringing tone) to the calling party using the Send TDM Tone procedure (signals 19 and 20in figure 39), when the following condition is satisfied:

-
the MGCF receives the first 180 Ringing message, unless this message or some previous SIP provisional response contained a P-Early-Media header field that authorizes early media.
9.2.3.3.6
Called party answer

When the MGCF receives a 200 OK message (signal 22 in figure 39), it shall request the IM-MGW to stop providing the ringing tone to the calling party using the Stop TDM Tone procedure (signals 23 and 24 in figure 39).

9.2.3.3.7
Through-Connection

Within the Reserve TDM Circuit procedure, the MGCF shall either use the Change TDM Through-Connection procedure to request the IM-MGW to backward through-connect the TDM termination, or the MGCF shall use this procedure to both-way through-connect the TDM termination already on this stage (signals 2 and 3 in figure 39). During the Reserve IMS Connection Point procedure, the MGCF shall use the Change IMS Through-Connection procedure to request the IM-MGW to backward through-connect the IMS termination (signals 4 and 5 in figure 39).

When the MGCF receives the SIP 200 OK(INVITE) message, it shall request the IM-MGW to both-way through-connect the terminations using the Change IMS Through-Connection or Change TDM Through-Connection procedure (signals 25 and 26 in figure 39), unless those terminations are already both-way through-connected.

9.2.3.3.8
Continuity Check

If a continuity check on the connection towards the CS network is requested in the IAM message, the MGCF shall use the Continuity Check Response procedure towards the IM-MGW to request loop-back of a received continuity check tone on the TDM circuit. Upon reception of the COT message, the MGCF shall use the Continuity Check Response procedure towards the IM-MGW to request the removal of the loop-back. (Not depicted in figure 39)

9.2.3.3.9
Codec handling

The IM-MGW may include a speech transcoder based upon the speech coding information provided to each termination.

9.2.3.3.10
Voice Processing function

A voice processing function located on the IM-MGW may be used to achieve desired acoustic quality on the terminations. If the voice processing function is used, the MGCF shall request the activation of it in the termination towards the CS network using the Activate TDM Voice Processing Function procedure (signal 23 in figure 39).

9.2.3.3.11
Failure handling in MGCF

If any procedure between the MGCF and the IM-MGW is not completed successfully, the session shall be released as described in clause 9.2.6. If the MGCF receives a Bearer Released procedure from the IM-MGW the default action by the MGCF is to release the session as described in clause 9.2.7.
9.2.3.3.12
Message sequence chart

Figure 39 shows the message sequence chart for the CS network originating Session with ISUP. In the chart the MGCF requests seizure of the IM CN subsystem side termination and CS network side bearer termination. When the MGCF receives an answer indication, it requests the IM-MGW to both-way through-connect the terminations. The MGCF may request the possible activation of the voice processing functions for the terminations. Dashed lines represent optional or conditional messages.

[image: image68.wmf]IM

-

MGW

MGCF

6

. 

SIP

: 

INVITE 

7

. 

SIP

: 

100 

Trying

1

. 

ISUP

: 

IAM 

2

. 

H

.

248

: 

ADD

.

req 

[

Context ID 

= 

?, 

Termination ID 

= 

?

]

3

. 

H

.

248

: 

ADD

.

resp 

[

Context ID 

= 

C

1

, 

Termination ID 

= 

T

2

] 

4

. 

H

.

248

: 

ADD

.

req 

[

Context ID 

= 

C

1

, 

Termination ID

=

?

]

5

. 

H

.

248

: 

ADD

.

resp 

[

Context ID 

= 

C

1

, 

Termination ID 

= 

T

1

]

8

. 

SIP

:

183 

Session 

Progress 

11

. 

SIP

: 

PRACK

12

. 

SIP

 

:

200 

OK 

(

PRACK

) 

9

. 

H

.

248

: 

MOD

.

req 

[

Context ID 

= 

C

1

,

Termination ID 

= 

T

1

]

10

.

H

.

248

: 

MOD

.

resp 

[

Context ID 

= 

C

1

, 

Termination ID 

= 

T

1 

] 

14

. 

SIP

: 

UPDATE

15

. 

SIP

 

:

200 

OK 

(

UPDATE

)

13

. 

ISUP

: 

COT 

16

. 

SIP

 

:

180 

Ringing

17

. 

SIP

: 

PRACK

21

. 

SIP

 

:

200 

OK 

(

PRACK

)

19

. 

H

.

248

: 

MOD

.

req  

[

Context ID 

= 

C

1

, 

Termination ID 

= 

T

2

]

20

. 

H

.

248

: 

MOD

.

resp 

[

Context ID 

= 

C

1

, 

Termination ID 

= 

T

2

] 

18

.

 

ISUP

: 

ACM 

Reserve TDM Circuit

,

Change Through

-

Connection  

= 

both  

Reserve IMS Connection Point

, 

Change IMS Through

-

Connection  

= 

backward 

Configure IMS  Resources

Send TDM Tone

if SIP 

Precon

-

ditions  

and

/

or 

100 

rel 

are used

if SIP 

Precon

-

ditions  

and

/

or 

100 

rel 

are used

Optional

if 

continuity  

check is 

used


Figure 39/1: Basic CS Network Originating Session, ISUP (message sequence chart)


[image: image69.wmf]IM

-

MGW

MGCF

23

. 

H

.

248

:  

MOD

.

req  

[

Context ID

=

C

1

, 

Termination ID 

= 

T

2

]

24

. 

H

.

248

: 

MOD

.

resp 

[

Context ID 

= 

C

1

, 

Termination ID 

= 

T

2

] 

28

. 

SIP

: 

ACK

25

. 

H

.

248

:  

MOD

.

req 

[

Context ID

=

C

1

, 

Termination ID 

= 

T

1

]

26

. 

H

.

248

: 

MOD

.

resp 

[

Context ID 

= 

C

1

, 

Termination ID 

= 

T

1

] 

27

. 

ISUP

: 

ANM 

22

. 

SIP

 

:

200 

OK 

(

INVITE

)

CALL IN ACTIVE STATE

Stop TDM 

Tone 

,

Activate 

TDM Voice 

Processing 

Function

Change IMS Through

-

Connection  

= 

both

22

a

. 

H

.

248

: 

MOD

.

req 

[

Context ID 

= 

C

1

,

Termination ID 

= 

T

1

]

  

22

b

. 

H

.

248

: 

MOD

.

resp

[

Context ID 

= 

C

1

, 

Termination ID 

= 

T

1 

] 

if SIP 

Preconditions  

and

100 

rel are not 

used

If  tone 

was 

inserted or 

for voive 

processing

Configure 

IMS  

Resources


Figure 39/2: Basic CS Network Originating Session, ISUP (message sequence chart continue)

9.2.3.4
Handling of Forking

The procedures described in clauses 9.2.3.1 to 9.2.3.3 shall be applied with the following additions.

9.2.3.4.1
Detection of Forking

According to SIP procedures, the O-MGCF inspects the tags in the "to" SIP header fields of provisional and final responses to identify the SIP dialogue the response belongs to. If responses belonging to different dialogues are received (signals 8 and 13 in figure 39a), the INVITE request (signal 6 in figure 39a) has been forked.

9.2.3.4.2
IM CN subsystem side session establishment

If SDP is received in a provisional response and more than one SIP dialogue exists (signal 13 in figure 39a), the MGCF may either refrain from reconfiguring the IM-MGW, or it may use the Configure IMS Resources procedure (signals 14 and 15 in figure 39a) as detailed below:

-
If the MGCF receives a SIP provisional response containing a P-Early-Media header field that authorizes early media, the MGCF shall provide the remote IP address and UDP port, and the remote codec selected from the received SDP and local configuration data, corresponding to the SIP dialogue of the SIP provisional response containing a P-Early-Media header field that authorizes early media. The IM-MGW may be configured to use the remote IP address and port information as source filter for incoming packages to prevent that early media from other early SIP dialogues interfere. The MGCF may also provide an IP address and port source filter that disallows early media from other early dialogues without an early media authorization in order to prevent that such unauthorized early media interfere with the authorized early media. (NOTE 1, NOTE 2)

-
If the MGCF did not receive any SIP provisional response containing a P-Early-Media header field that authorizes early media, the MGCF may compare the selected local codecs of the different dialogues (which the MGCF selects due to the received SDP answer and local configuration data). If different local codecs are selected for the different dialogues, the MGCF may include all these codecs in the "local IMS resources", and set the "reserve value" to indicate that resources for all these codecs shall be reserved. Alternatively, the MGCF may only include the codecs received in the last SDP in the "local IMS resources".

-
If the MGCF did not receive any SIP provisional response containing a P-Early-Media header field that authorizes early media, the MGCF may update the "remote IMS resources" with the information received in the latest SDP. The MGCF should provide the remote IP address and UDP port, and the remote codec selected from the received SDP and local configuration data. (NOTE 3)
NOTE 1:
The O-MGCF can use the P-Early-Media header field (IETF RFC 5009 [89]) to determine whether the media associated with a forked dialog is authorized and thus eligible for a through connection. In the presence of early media for multiple dialogs due to forking, if the IM-MGW is able to identify the media associated with a dialog (i.e., if symmetric RTP is used by the peer and the IM-MGW can use the remote SDP information to determine the source of the media), then the O-MGCF/IM-MGW can selectively establish a through-connection for an authorized early media flow.

NOTE 2:
The behaviour of an O-MGCF upon the possible reception of SIP provisional responses containing P-Early-Media header fields that authorize early media for several early dialogues is left unspecified.

NOTE 3:
The behaviour in the third bullet is beneficial if forking is applied in a sequential manner.
9.2.3.4.3
IM CN subsystem side session establishment completion

Upon reception of the first final 2xx response (signal 32 in figure 39a), the MGCF shall use the Configure IMS Resources procedure (signals 35 and 36 in figure 39a) as detailed below unless the IM-MGW is already configured accordingly:

-
If the remote IMS resources configured at the IM-MGW do not match the remote resources selected for the established dialogue of the final response, the MGCF shall provide the remote IP address and UDP port from the latest received SDP of this established dialogue, and the remote codec selected from the latest received SDP of this established dialogue and local configuration data within the "remote IMS resources".
-
If the local IMS resources configured at the IM-MGW contain more codecs than selected for the established dialogue of the final response, the MGCF should update the "local IMS resources" with the selected local codec derived from the latest SDP of this established dialogue and local configuration data. The "reserve value" may be cleared unless it is required for DTMF.

-
The IM-MGW may be configured to use the remote IP address and port information as source filter for incoming packages to prevent that early media from other early SIP dialogues interfere with the media of the established dialogue. The MGCF may also provide an IP address and port source filter that disallows early media from other early dialogues in order to prevent that such early media interfere with the media of the established dialogue. If the MGCF has provided a source filter selecting media of another SIP early dialogue, it shall remove or update this source filter.

9.2.3.4.4
Message sequence chart

Figure 39a shows an example message sequence chart for a CS network originating Session Setup with ISUP, where forking occurs.


[image: image70.wmf]IM-MGW

MGCF

6. 

SIP

: INVITE

7. 

SIP

: 100 Trying

1. 

ISUP

: 

IAM

2. 

H.248

: ADD.req [Context ID = ?, Termination ID = ?]

3. 

H.248

: ADD.resp [Context ID = C1, Termination ID = T2]

4. 

H.248

: ADD.req [Context ID = C1, Termination ID=?]

5. 

H.248

: ADD.resp [Context ID = C1, Termination ID = T1]

8. 

S

IP

:183 Session

Progress

(to:xxx, tagA)

11. 

SIP

: PRACK

(to:xxx, tagA)

12. 

S

IP

 :

200 OK (PRACK)

(to:xxx, tagA)

9. 

H.248

: MOD.req [Context ID = C1,Termination ID = T1]

10.

H.248

: MOD.resp [Context ID = C1, Termination ID = T1 ]

19. 

SIP

: UPDATE

(to:xxx, tagA)

21. 

S

IP

 :

200 OK (UPDATE)

(to: xxx,, tagA)

18. 

ISUP

: 

COT

Reserve TDM Circuit,

Change Through-

Connection  = both

Reserve IMS Connection Point,

Change IMS Through-

Connection  = backward

Configure IMS  Resources

13. 

S

IP

:183 Session

Progress

(to:xxx, tagB)

16. 

SIP

: PRACK

(to:xxx, tagB)

17. 

S

IP

 :

200 OK (PRACK)

(to:xxx, tagB)

14. 

H.248

: MOD.req [Context ID = C1,Termination ID = T1]

15.

H.248

: MOD.resp [Context ID = C1, Termination ID = T1 ]

Configure IMS  Resources

20. 

SIP

: UPDATE

(to:xxx, tagB)

22. 

S

IP

 :

200 OK (UPDATE)

(to: xxx,, tagB)


Figure 39a/1: CS Network Originating Session with forking, ISUP (message sequence chart)


[image: image71.wmf]IM-MGW

MGCF

33. 

H.248

:  MOD.req  [Context ID=C1, Termination ID = T2]

34. 

H.248

: MOD.resp [Context ID = C1, Termination ID = T2]

38. 

SIP

: ACK

35. 

H.248

:  MOD.req [Context ID=C1, Termination ID = T1]

36. 

H.248

: MOD.resp [Context ID = C1, Termination ID = T1]

37. 

ISUP

: 

ANM

32. 

S

IP

 :

200 OK (INVITE)

(to:xxx, tagA)

CALL IN ACTIVE STATE

Stop TDM Tone ,

Activate TDM Voice

Processing Function

Configure IMS

Resources,

Change IMS Through-

Connection  = both

29. 

S

IP

 :180 Ringing

(to:xxx, tagA)

30. 

SIP

: PRACK

(to:xxx, tagA)

31. 

S

IP

 :

200 OK (PRACK)

(to:xxx, tagA)

23. 

S

IP

 :180 Ringing

(to:xxx, tagB)

24. 

SIP

: PRACK

(to:xxx, tagB)

25. 

S

IP

 :

200 OK (PRACK)

(to:xxx, tagB)

27. 

H.248

: MOD.req  [Context ID = C1, Termination ID = T2]

28. 

H.248

: MOD.resp [Context ID = C1, Termination ID = T2]

26.

 ISUP

: 

ACM

Send TDM Tone


Figure 39a/2: CS Network Originating Session with forking, ISUP (message sequence chart continue)

9.2.4
Session release initiated from IM CN subsystem side

9.2.4.1
BICC

9.2.4.1.1
Session release in the IM CN subsystem side

When the MGCF has received a BYE message from the IM CN subsystem side, the MGCF shall release resources in the IM-MGW serving the relevant Mb interface connection by using the "Release IMS Termination" procedure (signals 5 and 6 in figure 40). After receiving the BYE message, the MGCF shall also send a 200 OK [BYE] message towards the IM CN subsystem (signal 2 in figure 40).

9.2.4.1.2
Session release in the CS network side

When the MGCF has received a BYE message from the IM CN subsystem side, the MGCF shall send a REL message to the succeeding node (signal 3 in figure 40). Once the succeeding node has responded with the RLC message (signal 6 in figure 40), the MGCF shall release the resources for the CS network side in the IM-MGW. If any resources were seized in the IM-MGW, the MGCF shall use the "Release Bearer", "Change Through-Connection" and "Release Termination" procedures (signals 7 to 10 in figure 40) to indicate to the IM-MGW that the CS network side bearer termination shall be removed and the bearer shall be released towards the succeeding MGW.

9.2.4.1.3
Message sequence chart

Figure 40 shows the message sequence chart for the session release initiated from the IM CN subsystem side.


[image: image72.wmf]MGCF

IM-MGW

1. 

SIP

: BYE

3. 

BICC

: 

REL

5. 

H.248

: SUB.req [Context ID = C1, Termination ID = T1]

6. 

H.248

: SUB.resp [Context ID = C1, Termination ID = T1]

9. 

H.248

: SUB.req [Context ID = C1,Termination ID = T2]

10. 

H.248

: SUB.resp [Context ID = C1, Termination ID = T2]

7. 

H.248

: MOD.req [Context ID = C1, Termination ID = T2]

8. 

H.248

: MOD.resp [Context ID = C1, Termination ID = T2]

4. 

BICC

: RLC

2. 

SIP

: 200 OK [BYE]

Release IMS Termination

Release Bearer,

Change Through-

Connection = backward

Release Termination


Figure 40: Session release from IM CN subsystem side for BICC (message sequence chart)

9.2.4.2
ISUP

9.2.4.2.1
Session release in the IM CN subsystem side

When the MGCF has received a BYE message from the IM CN subsystem side, the MGCF shall release resources in the IM-MGW serving the relevant Mb interface connection by using the "Release IMS Termination" procedure (signals 4 and 5 in figure 41). After receiving the BYE message, the MGCF shall also send a 200 OK [BYE] message towards the IM CN subsystem (signal 2 in figure 41).

9.2.4.2.2
Session release in the CS network side

When the MGCF has received a BYE message from the IM CN subsystem side, the MGCF shall send a REL message to the succeeding node (signal 3 in figure 41). After sending the REL message, the MGCF shall expect a RLC message (signal 8 in figure 41) from the succeeding node. The MGCF shall also release the resources for the CS network side in the IM-MGW. If any resources were seized in the IM-MGW, the MGCF shall use the "Release TDM Termination" procedure (signals 6 to 7 in figure 41) to indicate to the IM-MGW that the CS network side bearer termination can be released.

9.2.4.2.3
Message sequence chart

Figure 41 shows the message sequence chart for the session release initiated from the IM CN subsystem side.


[image: image73.wmf]MGCF

IM-MGW

1. 

SIP

: BYE

4. 

H.248

: SUB.req [Context ID = C1, Termination ID = T1]

5. 

H.248

: SUB.resp [Context ID = C1, Termination ID = T1]

2. 

SIP

: 200 OK [BYE]

Release IMS Termination

6. 

H.248

: SUB.req [Context ID = C1, Termination ID = T2]

7. 

H.248

: SUB.resp [Context ID = C1, Termination ID = T2]

Release TDM Termination

3. 

ISUP

: 

REL

8. 

ISUP

: RLC


Figure 41: Session release from IM CN subsystem side for ISUP (message sequence chart)

9.2.5
Session release initiated from CS network side

9.2.5.1
BICC

9.2.5.1.1
Session release in the CS network side

When the MGCF receives a REL message from the preceding node (signal 1 in figure 42), the MGCF shall release resources for the CS network side in the IM-MGW. If any resources were seized in the IM-MGW, the MGCF shall use the "Release Bearer", "Change Through-Connection" and "Release Termination" procedures to indicate to the IM-MGW that the CS network side bearer termination shall be removed and the bearer shall be released towards the preceding MGW (signal 3 to 6 in figure 42). After completion of resource release, the MGCF shall send a RLC message towards the preceding node.

9.2.5.1.2
Session release in the IM CN subsystem side

When the MGCF receives a REL message from the preceding node (signal 1 in figure 42), the MGCF shall send a BYE message to the IM CN subsystem (signal 2 in figure 42) and the MGCF shall release the resources in the IM-MGW serving the relevant Mb interface connection by using the "Release IMS Termination" procedure (signals 7 and 8 in figure 42). The MGCF shall also expect to receive a 200 OK [BYE] message from the IM CN subsystem side (signal 10 in figure 42).

9.2.5.1.3
Message sequence chart

Figure 42 shows the message sequence chart for the session release initiated from the CS network side.


[image: image74.wmf]1.

BICC

: REL

2. 

SIP

: BYE

7. 

H.248

: SUB.req [Context ID = C1, Termination ID = T1]

8. 

H.248

: SUB.resp [Context ID = C1, Termination ID = T1]

5. 

H.248

: SUB.req [Context ID = C1,Termination ID = T2]

6. 

H.248

: SUB.resp [Context ID =C1, Termination ID =T2 ]

3. 

H.248

: MOD.req [Context ID = C1, Termination ID = T2]

4. 

H.248

: MOD.resp [Context ID =C1, Termination ID =T2]

MGCF

IM-MGW

9. 

BICC

: RLC

10. 

SIP

: 200 OK [BYE]

Release Bearer,

Change Through-

Connection = backward

Release Termination

Release IMS

Termination


Figure 42: Session release from CS network side for BICC (message sequence chart)

9.2.5.2
ISUP

9.2.5.2.1
Session release in the CS network side

When the MGCF receives a REL message from the preceding node (signal 1 in figure 43), the MGCF shall release resources for the CS network side in the IM-MGW. If any resources were seized in the IM-MGW, the MGCF shall use the "Release TDM Termination procedures" to indicate to the IM-MGW that the CS network side bearer termination can be released (signal 3 to 4 in figure 43). After completion of resource release, the MGCF shall send a RLC message towards the preceding node.

9.2.5.2.2
Session release in the IM CN subsystem side

When the MGCF receives a REL message from the preceding node (signal 1 in figure 43), the MGCF shall send a BYE message to the IM CN subsystem (signal 2 in figure 43) and the MGCF shall release the resources in the IM-MGW serving the relevant Mb interface connection by using the "Release IMS Termination" procedure (signal 5 to 6 in figure 43). The MGCF shall also expect to receive a 200 OK [BYE] message from the IM CN subsystem side (signal 8 in figure 43).

9.2.5.2.3
Message sequence chart

Figure 43 shows the message sequence chart for the session release initiated from the CS network side.


[image: image75.wmf]1.

ISUP

: REL

2. 

SIP

: BYE

5. 

H.248

: SUB.req [Context ID = C1, Termination ID = T1]

6. 

H.248

: SUB.resp [Context ID =C1, Termination ID =T1]

3. 

H.248

: SUB.req [Context ID = C1,Termination ID = T2]

4. 

H.248

: SUB.resp [Context ID = C1, Termination ID = T2]

MGCF

IM-MGW

7. 

ISUP

: RLC

8. 

SIP

: 200 OK [BYE]

Release TDM

Termination

Release IMS

Termination


Figure 43: Session release from CS network side for ISUP (message sequence chart)

9.2.6
Session release initiated by MGCF

9.2.6.1
BICC

9.2.6.1.1
Session release in the CS network side

The MGCF shall send a REL message to the succeeding node on the CS network side (signal 1 in figure 44) Once the succeeding node has responded with the RLC message (signal 3 in figure 44), the MGCF shall release the resources for the CS network side in the IM-MGW. If any resources were seized in the IM-MGW, the MGCF shall use the "Release Bearer", "Change Through-Connection" and "Release Termination" procedures to indicate to the IM-MGW that the CS network side bearer termination shall be removed and the bearer shall be released towards the succeeding MGW (signal 4 to 7 in figure 44).

9.2.6.1.2
Session release in the IM CN subsystem side

The MGCF shall sends a BYE message to the IM CN subsystem side (signal 2 in figure 44) and the MGCF shall release the resources in the IM-MGW serving the relevant Mb interface connection by using the "Release IMS Termination" procedure (signals 8 and 9 in figure 44). The MGCF shall also expect to receive a 200 OK [BYE] message is received from the IM CN subsystem side (signal 10 in figure 44).

9.2.6.1.3
Message sequence chart

Figure 44 shows the message sequence chart for the session release initiated by the MGCF.


[image: image76.wmf]MGCF

IM-MGW

2. 

SIP

: BYE

1. 

BICC

: 

REL

3. 

BICC

: RLC

8. 

H.248

: SUB.req [Context ID = C1, Termination ID = T1]

9. 

H.248

: SUB.resp [Context ID = C1, Termination ID = T1]

6. 

H.248

: SUB.req [Context ID = C1,Termination ID = T2]

7. 

H.248

: SUB.resp [Context ID = C1, Termination ID = T2 ]

4. 

H.248

: MOD.req [Context ID = C1, Termination ID = T2]

5. 

H.248

: MOD.resp [Context ID = C1, Termination ID = T2]

10. 

SIP

: 200 OK [BYE]

Release Bearer,

Change Through-

Connection = backward

Release Termination

Release IMS

Termination


Figure 44: Session release initiated by MGCF for BICC (message sequence chart)

9.2.6.2
ISUP

9.2.6.2.1
Session release in the CS network side

The MGCF shall send a REL message to the succeeding node on the CS network side (signal 2 in figure 45) and the MGCF shall release the resources for the CS network side in the IM-MGW. If any resources were seized in the IM-MGW, the MGCF shall use the "Release TDM Termination" procedure to indicate to the IM-MGW that the CS network side termination shall be released (signal 5 to 6 in figure 45). The MGCF shall also expect to receive a RLC message from the succeeding node on the CS network side (signal 7 in figure 45).

9.2.6.2.2
Session release in the IM CN subsystem side

The MGCF shall send a BYE message to the IM CN subsystem side (signal 1 in figure 45) and the MGCF shall release the resources in the IM-MGW serving the relevant Mb interface connection by using the "Release IMS Termination" procedure (signal 5 to 6 in figure 45). The MGCF shall also expect to receive a 200 OK [BYE] message from the IM CN subsystem side (signal 8 in figure 45).

9.2.6.2.3
Message sequence chart

Figure 45 shows the message sequence chart for the session release initiated by the MGCF.


[image: image77.wmf]MGCF

IM-MGW

1. 

SIP

: BYE

8. 

SIP

: 200 OK [BYE]

7. 

ISUP

: RLC

2. 

ISUP

: 

REL

3. 

H.248

: SUB.req [Context ID = C1, Termination ID = T1]

4. 

H.248

: SUB.resp [Context ID = C1, Termination ID = T1]

Release IMS

Termination

5. 

H.248

: SUB.req [Context ID = C1,Termination ID = T2]

6. 

H.248

: SUB.resp [Context ID = C1, Termination ID = T2 ]

Release TDM

Termination


Figure 45: Session release initiated by MGCF for ISUP (message sequence chart)

9.2.7
Session release initiated by IM-MGW

9.2.7.1
BICC

9.2.7.1.1
Session release in the CS network side

Upon receiving from the IM-MGW a "Bearer Released" procedure (signal 1a and 2a in figure 46) or "IMS Bearer Released" procedure (signal 1b and 2b in figure 46) or a "MGW Out-of-Service" procedure indicating an immediate release (H248 ServiceChangeMethod="Forced") (not depicted in figure 46), the MGCF shall send a REL message to the succeeding node on the CS network side (signal 3 in figure 46). Once the succeeding node has responded with the RLC message (signal 5 in figure 46), the MGCF shall release the resources for the CS network side in the IM-MGW, unless the "MGW Out-of-Service" procedure was received. If any resources were seized in the IM-MGW, the MGCF shall use the "Release Termination" procedure to indicate to the IM-MGW that the CS network side bearer termination shall be removed (signals 6 and 7 in figure 46).

NOTE:
Other actions related to MGW Out-Of-Service procedure is defined in TS 23.205 [27].
9.2.7.1.2
Session release in the IM CN subsystem side

Upon receiving from the IM-MGW a "Bearer Released" procedure (signals 1a and 2a in figure 46) or "IMS Bearer Released" procedure (signal 1b and 2b in figure 46) or a "MGW Out-of-Service" procedure indicating an immediate release (H248 ServiceChangeMethod="Forced") (not depicted in figure 46), the MGCF shall send a BYE/CANCEL message to the IM CN subsystem side (signal 4 in figure 46) Upon receiving from the IM-MGW a "Bearer Released" procedure or "IMS Bearer Released" procedure, the MGCF shall also release the resources in the IM-MGW serving the relevant Mb interface connection by using the "Release IMS Termination" procedure (signals 8 and 9 in figure 46). The MGCF shall also expect to receive a 200 OK [BYE] message from the IM CN subsystem side (signal 10 in figure 46).

NOTE:
Other actions related to MGW-Out-Of-Service procedure is defined in TS 23.205 [27]

9.2.7.1.3
Message sequence chart

Figure 46 shows the message sequence chart for the session release initiated by the IM-MGW.


[image: image78.wmf]MGCF

IM

-

MGW

4

. 

SIP

: 

BYE

3

. 

BICC

: 

REL

5

. 

BICC

: 

RLC

8

. 

H

.

248

: 

SUB

.

req 

[

Context ID 

= 

C

1

, 

Termination ID 

= 

T

1

]

9

. 

H

.

248

: 

SUB

.

resp 

[

Context ID 

= 

C

1

, 

Termination ID 

= 

T

1

]

6

. 

H

.

248

: 

SUB

.

req 

[

Context ID 

= 

C

1

,

Termination ID 

= 

T

2

]

7

. 

H

.

248

: 

SUB

.

resp 

[

Context ID 

= 

C

1

, 

Termination ID 

= 

T

2 

] 

10

. 

SIP

: 

200 

OK 

[

BYE

]

1

a

. 

H

.

248

: 

Notify

.

req 

[

Context ID 

= 

C

1

, 

Termination ID 

= 

T

2

]

2

a

. 

H

.

248

: 

Notify

.

resp 

[

Context ID 

= 

C

1

, 

Termination ID 

= 

T

2

]

Bearer Released

Release Termination

Release IMS Termination

1

b

. 

H

.

248

: 

Notify

.

req 

[

Context ID 

= 

C

1

, 

Termination ID 

= 

T

1

]

2

b

. 

H

.

248

: 

Notify

.

resp 

[

Context ID 

= 

C

1

, 

Termination ID 

= 

T

1

]

IMS Bearer 

Released

or


Figure 46: Session release initiated by the IM-MGW for BICC (message sequence chart)

9.2.7.2
ISUP

9.2.7.2.1
Session release in the CS network side

Upon receiving from the IM-MGW a "Termination Out-of-Service" procedure indicating an immediate release (signals 1a and 2a in figure 47), a "Bearer Released" procedure (signal 1b and 2b in figure 47), a "IMS Bearer Released" procedure (signal 1c and 2c in figure 47) or a "MGW Out-of-Service procedure" (not depicted in figure 47) indicating an immediate release (H248 ServiceChangeMethod="Forced") the MGCF shall send a REL message to the succeeding node (signal 3 in figure 47). Upon receiving from the IM-MGW a "Termination Out-of-Service" message procedure indicating an immediate release or a "Bearer Released" procedure, the MGCF shall also release the resources for the corresponding CS network side termination(s) in the IM-MGW. If any resources were seized in the IM-MGW, the MGCF shall use the "Release TDM Termination" procedure to indicate to the IM-MGW that the CS network side bearer termination can be removed (signals 7 and 8 in figure 47). The MGCF also expects to receive a RLC message on the CS network side (signal 9 in figure 47) before the circuit is reselectable.

NOTE:
Other actions related to "MGW-Out-Of-Service" procedure are defined in TS 23.205 [27].

9.2.7.2.2
Session release in the IM CN subsystem side

Upon receiving from the IM-MGW a "Termination Out-of-Service" procedure indicating an immediate release (signal 1a and 2a in figure 47) on the CS termination in the context, a "Bearer Released" procedure (signal 1b and 2b in figure 47), an "IMS Bearer Released" procedure (signal 1c and 2c in figure 47) or a "MGW Out-of-Service procedure" (not depicted in figure 47) indicating an immediate release, (H248 ServiceChangeMethod="Forced") the MGCF shall send a BYE/CANCEL message to the IM CN subsystem side (signal 4 in figure 47). Upon receiving from the IM-MGW a "Termination Out-of-Service" procedure indicating an immediate release on the CS termination in the context, a "Bearer Released" procedure or an "IMS Bearer Released" procedure, the MGCF shall also release the resources in the IM-MGW for the corresponding terminations towards the IM CN subsystem using the "Release IMS Termination" procedure (signals 5 and 6 in figure 47). The MGCF also expects to receive a 200 OK [BYE] message from the IM CN subsystem side (signal 10 in figure 47).

NOTE:
Other actions related to "MGW-Out-Of-Service" procedure are defined in TS 23.205 [27].

9.2.7.2.3
Message sequence chart

Figure 47 shows the message sequence chart for the session release initiated by the IM-MGW.


[image: image79.wmf]MGCF

IM

-

MGW

4

. 

SIP

: 

BYE

10

. 

SIP

: 

200 

OK 

[

BYE

]

1

a

. 

H

.

248

: 

ServiceChange

.

req

[

Context ID 

= 

C

1

, 

Termination ID 

= 

T

2

]

2

a

. 

H

.

248

: 

ServiceChange

.

resp 

[

Context ID 

= 

C

1

, 

Termination ID 

= 

T

2

]

Termination 

Out

-

of

-

Service

9

. 

ISUP

: 

RLC

3

. 

ISUP

: 

REL

5

. 

H

.

248

: 

SUB

.

req 

[

Context ID 

= 

C

1

, 

Termination ID 

= 

T

1

]

6

. 

H

.

248

: 

SUB

.

resp 

[

Context ID 

= 

C

1

, 

Termination ID 

= 

T

1

]

7

. 

H

.

248

: 

SUB

.

req 

[

Context ID 

= 

C

1

,

Termination ID 

= 

T

2

]

8

. 

H

.

248

: 

SUB

.

resp 

[

Context ID 

= 

C

1

, 

Termination ID 

= 

T

2 

] 

Release IMS 

Termination

Release TDM 

Termination

1

b

. 

H

.

248

: 

Notify

.

req 

[

Context ID 

= 

C

1

, 

Termination ID 

= 

T

2

]

2

b

. 

H

.

248

: 

Notify

.

resp 

[

Context ID 

= 

C

1

, 

Termination ID 

= 

T

2

]

Bearer Released

or

1

c

. 

H

.

248

: 

Notify

.

req 

[

Context ID 

= 

C

1

, 

Termination ID 

= 

T

1

]

2

c

. 

H

.

248

: 

Notify

.

resp 

[

Context ID 

= 

C

1

, 

Termination ID 

= 

T

1

]

or

IMS Bearer 

Released


Figure 47: Session release initiated by the IM-MGW for ISUP (message sequence chart)
9.2.8
Handling of RTP telephone events

DTMF digits, telephony tones and signals (telephone events) can be transferred using different mechanisms. For the IM CN Subsystem, TS 24.229 [9] defines the usage of the RTP payload format defined for DTMF Digits, Telephony Tones and Telephony Signals in RFC 4733 [105]. When BICC signalling is used in the CS network, telephony signals may be sent either inband or out-of-band as defined in ITU-T Recommendation Q.1902.4 [30] and in ITU-T Recommendation Q.765.5 [35]. If ISUP signalling is used the DTMF tones are sent inband. The following paragraphs describe the Mn interface procedures to transfer DTMF between RTP format defined in RFC 4733 [105] and the CS CN.

Before the actual usage of the telephony signals can occur the sending/receiving of telephone events need to be agreed with the SDP offer-answer mechanism defined in RFC 3264 [36]. The outcome of the negotiation can be e.g. that no telephone events are sent in RTP payload, telephone events are sent only in one direction or in both directions. If the outcome of the negotiation is that RTP payload telephone-events are sent in both directions, the IM-MGW may nevertheless be configured to interwork only mobile originated telephone-events.

When the offer-answer mechanism based session parameters negotiation results in an agreement that telephone events are sent in the RTP payload and the needed preconditions are fulfilled, telephone events can be sent in RTP payload. This negotiation can be done at call control signalling phase or during an ongoing call.

If the MGCF and IM-MGW support the reception and/or transmission of the RTP MIME type "telephone event" (as defined in RFC 4733 [105]) with the IMS, the following applies:

-
For CS Network Originating Sessions, the MGCF shall include the MIME type "telephone events" with default events in the first SDP offer. After the usage of telephone events is agreed in the subsequent offer-answer parameter exchanges and the needed preconditions defined in RFC 3312 [37] are fulfilled, telephone events can be sent as RTP payload.

-
In case of IM CN Subsystem Originating Sessions, the MGCF shall accept the MIME type "telephone events" with default events in any SDP answer when it received such an offer.

9.2.8.1
Sending DTMF digits out-of-band to CS CN (BICC)

For the IM CN subsystem terminated session, the MGCF shall use the "Configure IMS Resources" procedure as described in clause 9.2.3. For the IM CN subsystem originating session, the MGCF shall use the "Reserve IMS Connection Point and Configure Remote Resources" procedure as described in clause 9.2.2. If DTMF is supported, the MGCF shall include "telephone event" along with the selected speech codecs within the "local IMS resources" Parameter of these procedures. The same termination shall be used to receive and transmit DTMF and speech of the same call.

Furthermore, the MGCF shall use the "Detect IMS RTP Tel Signal" procedure to request the MGW to detect incoming telephone events from the IMS and notify the MGCF about the detected events. The MGW shall use the "Notify IMS RTP Tel Event" procedure for this notification. The termination used to receive DTMF shall be placed in the same context used for the speech of the same call. The MGCF shall request to be notified when the MGW detects the end of a digit and may also request to be notified when the MGW detects the start of a digit. An IM-MGW not supporting the notification about the detection of the start of a digit may ignore the request to provide this notification. If the IM-MGW received a "Detect IMS RTP Tel Event" procedure for a termination, the IM-MGW shall not forward inband to the CS network any DTMF received at this termination.

Figure 48 shows an example message sequence chart when a DTMF digit is received from the IM CN subsystem in the RTP payload and the MGCF has requested to be notified only about the detection of the end of a digit. Figure 48a shows an example message sequence chart when a DTMF digit is received from the IM CN subsystem in the RTP payload and the MGCF has requested to be notified about the detection of the start and the end of a digit.

[image: image80.wmf] 

10. 

H.248

 Notify.ind 

 

 [Context ID = C1, Termination ID = T1, Tone

-

ID,

 

  Even

t=”End tone detected”, Duration]

 

8. 

RTP

 encoded new DTMF event

 

             (tone

-

event, end=0, duration

=

x

)

 

9. 

RTP

 encoded continued DTMF event

 

                (tone

-

event, end=1, duration

=

 

Duration

)

 

 

1. 

H.248

 Add/Mod.req

 

              [Context ID = C1, Termination ID = T1,

 

               Codec = “telephone event, AMR”,

 

               Notify IMS RTP Tel Event (”End tone detected”)  ]                                                   

  

 

2. 

H.248

 Add/Mod.resp [Context ID = C1, T

ermination ID =T1]

 

IM

-

MGW

 

MGCF

 

11. 

H.248

 Notify.resp[Context ID = C1, Termination ID = T1]

 

12. 

BICC

 APM (ActInd=

 

Start signal notify, signal

-

type, duration)

 

13. 

BICC 

APM (ActInd=Start signal ack)

 

Configure IMS Resources

 

or

 

Reserve IMS Conection Point 

and configure Remote 

Resources,

 

Detect_IMS_RTP_Tel_Event

 

 

Notify_IMS_RTP_Tel_Event

 

…

 

1 Digit

 


Figure 48: Activation of notification of DTMF digits received in RTP and examples of sending the digits out-of-band to CS CN, a whole digit received by IM-MGW before sending further (message sequence chart)

[image: image81.wmf] 

5. 

H.248

 Notify.resp[Context ID = C1, Termination ID = T1]

 

6. 

BICC

 APM (ActInd=Start signal notify, signal

-

type)

 

7. 

BICC 

APM (ActIn

d=Start signal ack)

 

4. 

H.248

  Notify.ind 

 

               [ Context ID = C1, Termination ID = T1, Tone_ID,

 

 Event=”Start tone detected”]

 

10. 

H.248

 Notify.ind 

 

 [Context ID = C1, Termination ID = T1, Tone

-

ID,

 

  Event=”End tone detected”, Duration]

 

3. 

RTP

 encoded new DTMF event

 

             (tone

-

event, end=0, duration

=

x

)

 

9. 

RTP

 encoded continued DTMF event

 

                (tone

-

event, end=1, duration

=Duration

)

 

 

1. 

H.248

 Add/Mod.req

 

              [Context ID = C1, Termination ID = T1,

 

               Codec = “telephone event, AMR”,

 

               Notify IMS RTP Tel Event (”Start tone detected”,

 

                                                          ”End t

one detected”)  ]

 

2. 

H.248

 Add/Mod.resp [Context ID = C1, Termination ID

 =T1]

 

IM

-

MGW

 

MGCF

 

1

1

. 

H.248

 Notify.resp[Context ID = C1, Termination ID = T1]

 

12. 

BICC

 APM (ActInd=Stop signal notify)

 

13. 

BICC 

APM (ActInd=Stop signal ack)

 

Configure IMS Resources

 

or

 

Reserve IMS Conection Point 

and configure Remote 

Resources,

 

Detect_IMS_RTP_Tel_E

vent

 

 

Notify_IMS_RTP_Tel_Event

 

 

Notify_IMS_RTP_Tel_Event

 

8. 

RTP

 encoded new DTMF event

 

             (tone

-

event, end=0, dura

tion

=

y

)

 

…

 

1 Digit

 


Figure 48a: Activation of notification of DTMF digits received in RTP and examples of sending the digits out-of-band to CS CN, IM-MGW starts sending the digit further when the start of the digit is recognized (message sequence chart)

9.2.8.2
Sending and receiving DTMF digits inband to/from CS CN (ISUP or BICC)

For the IM CN subsystem terminated session, the MGCF shall use the "Configure IMS Resources" procedure as described in clause 9.2.3. For the IM CN subsystem originating session, the MGCF shall use the "Reserve IMS Connection Point and Configure Remote Resources" procedure as described in clause 9.2.2. If DTMF is supported and the MGCF wants to configure the IM_MGW to send and receive DTMF to/from the CS network side, the MGCF shall include "telephone event" along with the selected speech codecs within the "local IMS resources" parameter of these procedures to request the MGW to detect incoming telephone events and transform them into speech signals on the CS side and shall not apply the "Detect IMS RTP Tel Event" procedure. When receiving this configuration, an MGW supporting DTMF shall detect DTMF encoded according as RTP Tel Event and transform this into DTMF tones encoded within the speech codec used at the CS CN network and may in addition optionally detect incoming telephone events received inband from the CS CN network and transform them into telephone events on the IMS side. The same termination shall be used to receive and transmit DTMF and speech of the same call.

Figure 49 shows the message sequence chart to configure the IM-MGW to receive DTMF detection on the IMS side and transfer the DTMF inband on the CS side. When receiving this configuration, the IM-MGW may in addition optionally detect DTMF inband on the CS side and transmit DTMF on the IMS side.

[image: image82.wmf] 

1. 

H.248

 Add/Mod.req 

 

[Context ID = C1, Termination ID = T1,

 

Codec = “telephone event, AMR”,]

 

IM

-

MGW

 

MGCF

 

2. 

H.248

 Add/Mod.resp [Context ID = C1, Termination ID = T1]

 

Configure IMS Resources

 

Or

 

Reserve IMS Conection Point 

and configure Remote

 

Resources

 


Figure 49: Activation of processing of DTMF digits received in RTP for sending the digits inband to CS CN (message sequence chart)

9.2.8.3
Receiving DTMF digits out-of-band from CS CN (BICC)

For the IM CN subsystem terminated session, the MGCF shall use the "Configure IMS Resources" procedure as described in clause 9.2.3. For the IM CN subsystem originating session, the MGCF shall use the "Reserve IMS Connection Point and Configure Remote Resources" procedure as described in clause 9.2.2. If DTMF is supported, the MGCF shall include "telephone event" along with the selected speech codecs within the "local IMS resources" Parameter of these procedures. The same termination shall be used to receive and transmit DTMF and speech of the same call.

Furthermore, the MGCF shall use the "Send IMS RTP Tel Event" and may use the "Stop IMS RTP Tel Event" procedures to request the MGW to play out DTMF to the IM CN subsystem whenever it receives out-of-band DTMF indications from the BICC network.

Figure 49a shows an example message sequence chart when DTMF digits are transmitted to the IM CN subsystem in the RTP payload. For the first digit, the received APM message contains all information including the duration and only a single notification is received. For the second digit, the start and the end of the DTMF digit are notified separately.


[image: image83.wmf] 

…

 

5. 

H.2

48

 Mod.ind

 

 [Context ID = C1, Termination ID = T1, Tone

-

Id,

 

  

SignalType=TimeOut, 

Duration]

 

3. 

BICC 

APM(ActInd=Start signal notify, signal

-

type, duration)

 

4. 

BICC

 APM (ActInd=Start signal ack)

 

11. 

H.248

 Mod.resp[Context ID = C1, Termination ID = T1]

 

8. 

BICC

 APM (ActInd=Start signal notify, signal

-

type)

 

9. 

BI

CC 

APM (ActInd=Start signal ack)

 

10. 

H.248

  Mod.ind 

 

               [ Context ID = C1, Termination ID = T1, Tone_ID,

 

 

SignalType=On/Off

]

 

1

6

. 

H.248

 Mod.ind 

 

 [Context ID = C1, Termination ID = T1]

 

Digit 1

 

Digit 2

 

7. 

RTP

 

encoded new DTMF event

 

            (tone

-

event, end=

0

, duration

=

x

)

 

1

3

. 

RTP

 encoded new DTMF event

 

             (tone

-

event, end=0, duration

=

y

)

 

1

8

7. 

RTP

 encoded continued DTMF event

 

                (tone

-

event, end=1, duration)

 

 

1. 

H.248

 Add/Mod.req

 

              [Context ID = C1, Termination ID = T1,

 

               Codec = “telephone event, AMR”,

 

               Notify IMS RTP Tel Event (”Start tone detected

”,

 

                                                          ”End tone detected”)  ]

 

2. 

H.248

 Add/Mod.resp [Context ID = C1, Termination ID =T1]

 

IM

-

MGW

 

M

GCF

 

6. 

H.248

 Mod.resp[Context ID = C1, Termination ID = T1]

 

1

7

. 

H.248

 Mod.resp[Context ID = C1, Termination ID = T1]

 

1

4

. 

BICC

 APM (ActInd=Stop signal notify)

 

1

5

. 

BICC 

APM (ActInd=Stop signal ack)

 

Configure IMS Resources

 

or

 

Reserve IMS Conection Point 

and configure Remote 

Resources,

 

Detect_IMS_RTP_Tel_Event

 

 

Send_IMS_RTP_Tel_Event

 

 

Send_IMS_RTP_Tel_Event

 

 

Stop_IMS_RTP_Tel_Event

 

12. 

RTP

 encoded new DTMF event

 

             (tone

-

event, end=0, duration=

x

)

 

7. 

RTP

 encoded new DTMF event

 

            (tone

-

event, end=1, duration)

 

…

 


Figure 49a: Examples of receiving DTMF digits out-of-band from the CS CN
and transmitting them in RTP (message sequence chart)

9.2.9
Session hold initiated from IM CN subsystem

The network model in the clause 9.2.1 shall apply here.

Hold request

When the IMS network makes a hold request by sending an UPDATE or re-INVITE message (signal 1 of figure 50), the MGCF shall request the IM-MGW to suspend sending media towards the IMS side by changing the through-connection of the IM CN subsystem side termination to "not through-connected" (signal 2 of figure 50). If the IMS side provides modified SDP RR or RS bandwidth modifiers, as specified in IETF RFC 3556 [59], within the hold request, the MGCF shall use the Configure IMS Resources Mn procedure to forward this information to the IM-MGW (not depicted in figure 50, but may be combined with signal 2). The MGCF shall send a CPG (Hold) message to the succeeding CS network node to indicate that the session is on hold (signal 4 of figure 50). Simultaneously a SIP message acknowledging the Hold request is sent to the IMS side (signal 7 of figure 50, acknowledged by signal 7.a if the INVITE method is used). Announcements may be applied to the party on hold, depending on the held party's status, using the Play Announcement procedure (for BICC) or the Play TDM Announcement procedure (for ISUP, signal 5 in figure 50). The hold operation shall not block RTCP flows.

Resume request

When the IMS network makes a request to retrieve the session on hold by sending an UPDATE or re-INVITE message (signal 8 of figure 50), the MGCF shall request the IM-MGW to re-establish communication towards the IMS network by changing the through-connection of the IM CN subsystem side termination to both-way through-connected (signal 11 of figure 50). If the IMS side provides modified SDP RR or RS bandwidth modifiers, as specified in IETF RFC 3556 [59], within the retrieve request, the MGCF shall use the Configure IMS Resources Mn procedure to forward this information to the IM-MGW (not depicted in figure 50, but may be combined with signal 11). Possible announcements to the party on hold shall be stopped using the Stop Announcement procedure (for BICC) or the Stop TDM Announcement procedure (for ISUP, signal 9 in figure 50). The MGCF shall send a CPG (Retrieve) message to the succeeding CS network node to indicate that the session is retrieved (signal 13 of figure 50).

Message sequence chart

Figure 50 shows the message sequence chart for the call hold and retrieval procedures.


[image: image84.emf]MGCF IM-MGW

4. 

BICC/ISUP

:  CPG  (Hold) 

2. 

H.248

:  MOD.req   

[Context ID  =  C1 ,  Termination ID  =  T1 ]

3. 

H.248

:  MOD.resp 

[Context ID  =  C1 ,  Termination ID  =  T1 ]

11 . 

H.248

:  MOD.req  

[Context ID  =  C1 ,  Termination ID  =  T1 ]

12. 

H.248

:  MOD.resp 

[Context ID  =  C1 ,  Termination ID  =  T1 ]

7. 

SIP

:  200  OK  [SDP]

1 . 

SIP

:  UPDATE/INVITE  [SDP, 

a=sendonly/inactive]

8. 

SIP

:  UPDATE/INVITE  [SDP, 

a=sendrecv/recvonly]

13. 

BICC/ISUP

:  CPG  (Retrieve)

14. 

SIP

:  200  OK  [SDP]

7.a 

SIP

:  ACK  (if INVITE is used)

14.a 

SIP

:  ACK  (if INVITE is used)

5. 

H.248

:  MOD.req 

[Context ID  =  C1 ,  Termination ID  =  T2]

9. 

H.248

:  MOD.req   

[Context ID  =  C1 ,  Termination ID  =  T2]

Play TDM 

Announcement

Stop TDM 

Announcement

Change Through-

Connection=Inactive

Change Through-

Connection=Both

6. 

H.248

:  MOD.resp 

[Context ID  =  C1 ,  Termination ID  =  T2]

10. 

H.248

:  MOD.resp 

[Context ID  =  C1 ,  Termination ID  =  T2]

 


Figure 50 Session hold from IM CN subsystem

9.2.10
Session hold initiated from CS network

When an MGCF receives a CPG message with a "remote hold" Generic notification indicator (signal 1 of figure 51), the MGCF forwards the hold request by sending an UPDATE or re-INVITE message containing SDP with "sendonly" or "inactive" media (signal 4 of figure 51).

When an MGCF receives a CPG message with a "remote retrieval" Generic notification indicator (signal 6 of figure 51), the MGCF forwards the resume request by sending an UPDATE or re-INVITE message containing SDP with "sendrecv" or "recvonly" media (signal 9 of figure 51).

If the MGCF receives a CPG with "remote hold" or "remote retrieval" before answer, it shall forward the request using an UPDATE message. If the MGCF receives a CPG with "remote hold" or "remote retrieval" after answer, it should forward the request using re-INVITE but may use UPDATE.

If link aliveness information is required at the IM-MGW while the media are on hold, the MGCF should provide to the modified SDP RR and RS bandwidth modifiers specified in IETF RFC 3556 [59] within the SDP offers in the UPDATE or re-INVITE messages holding and retrieving the media to temporarily enable RTCP while the media are on hold, as detailed in clause 7.4 of TS 26.236 [32]. If no link aliveness information is required at the IM-MGW, the MGCF should provide the SDP RR and RS bandwidth modifiers previously used.

The interworking does not impact the user plane, unless the MGCF provides modified SDP RR and RS bandwidth modifiers in the UPDATE or re-INVITE messages. If the MGCF provides modified SDP RR and RS bandwidth modifiers in the UPDATE or re-INVITE messages, the MGCF shall also provide modified SDP RR and RS bandwidths to the IM-MGW using the Configure IMS Resoureces procedures (signals 2-3 and 7-8 of figure 51).

Message sequence chart

Figure 51 shows the message sequence chart for the call hold and retrieval procedures.


[image: image85.emf]MGCF IM-MGW

1 . 

BICC/ISUP

:  CPG  (Hold) 

2. 

H.248

:  MOD.req   

[Context ID  =  C1 ,  Termination ID  =  T1 ]

3. 

H.248

:  MOD.resp 

[Context ID  =  C1 ,  Termination ID  =  T1 ]

5. 

SIP

:  200  OK  [SDP]

4. 

SIP

:  UPDATE/INVITE  [SDP,  a=sendonly/inactive]

6. 

BICC/ISUP

:  CPG  (Retrieve)

Configure IMS 

Resources

(optional)

7. 

H.248

:  MOD.req   

[Context ID  =  C1 ,  Termination ID  =  T1 ]

8. 

H.248

:  MOD.resp 

[Context ID  =  C1 ,  Termination ID  =  T1 ]

10. 

SIP

:  200  OK  [SDP]

9. 

SIP

:  UPDATE/INVITE  [SDP,  a=sendrecv/recvonly]

Configure IMS 

Resources

(optional)

5.a 

SIP

:  ACK  (if INVITE is used)

10.a 

SIP

:  ACK  (if INVITE is used)

 


Figure 51 Session hold from CS network

9.2.11
Explicit Congestion Notification Support

9.2.11.1
General

A MGCF and IM-MGW may support Multimedia Telephony using Explicit Congestion Notification (see IETF RFC 3168 [129], IETF RFC 6679 [130], and TS 26.114 [104]), and may then act as an ECN endpoint to enable ECN with a local ECN-capable terminal within a local network that properly handles ECN-marked packets.
9.2.11.1a
Incoming Call Interworking from SIP to ISUP/BICC at I-MGCF
If the MGCF receives an SDP Offer containing ECN negotiation, see IETF RFC 6679 [130], and if all of the following statements are true:

-
the MGCF supports ECN according to TS 26.114 [104];

-
the IM-MGW supports ECN according to TS 26.114 [104];

-
the IM-MGW knows (via configuration) that the preceding (IMS) network supports ECN according to TS 26.114 [104]; and

-
the IM-MGW supports at least some of the initialisation methods offered within the "a=ecn-capable-rtp",

NOTE 1: Only the "leap" initialisation method is supported over the Mn interface in this release.
the MGCF shall:
-
act as an end point for ECN;
-
select an initialisation method supported by the IM-MGW;

-
determine if application specific feedback or ECN feedback messages shall be used, taking into account whether the IM-MGW supports ECN feedback messages, and the negotiation procedures in TS 26.114 [104];

-
determine if ECN XR summary reports can be used, taking into account whether they are supported at the IM-MGW and the negotiation procedures in TS 26.114 [104];

-
return an SDP Answer according to TS 26.114 [104] and the capabilities of the IM-MGW, containing the "a=ecn-capable-rtp" attribute; and
NOTE 2:
ECN XR summary reports and RTCP AVPF ECN feedback message are not supported in this release.
-
indicate to the IM-MGW that it shall apply the ECN procedures (according to TS 26.114 [104]) and act as an ECT endpoint.
The IM-MGW should not send RTCP XR ECN summary reports.
9.2.11.1b
Outgoing Call Interworking from ISUP/BICC to SIP at O-MGCF

When creating the SDP Offer the MGCF may initiate ECN negotiation (in accordance with TS 26.114 [104]) , indicating the capabilities of the IM-MGW (e.g. supported initialisation methods, support of ECN feedback message, and support of ECN XR summary reports).
NOTE:
ECN XR summary reports and RTCP AVPF ECN feedback message are not supported in this release.
If the MGCF receives the SDP Answer also containing ECN attributes (indicating successful ECN negotiation) then it shall indicate to the IM-MGW that it shall apply the ECN procedures (according to TS 26.114 [104]) and act as an ECT endpoint.
The IM-MGW should not send RTCP XR ECN summary reports.
9.2.11.1c
Detection of ECN failures by IM-MGW

If the IM-MGW detects an ECN-related error case, for example non-ECT in the received packets when ECT(0) was expected or detecting a very high packet loss rate when ECN is used, the IM-MGW shall notify the MGCF. The MGCF should then initiate a session re-negotiation to disable ECN.
9.2.11.2
Message sequence chart

Figure 9.2.11.2.1 shows the message sequence chart example for requesting Explicit Congestion Notification.

[image: image86.emf]MGCF

IM-

MGW

H.248:ADD.req[

Termination X,

Transport = RTP/AVPF, 

ECN_enable=True, 

{Notify_ECN_Error}]

H.248:ADD.resp[Termination X]

Context(Cx)

Context(Cx)

Reserve IMS 

Connection Point and 

Configure Remote 

Resources


Figure 9.2.11.2.1: Procedure to Request ECN
Upon receipt of a request to apply Explicit Congestion Notification the IM-MGW shall set the ECN field of the IP header in accordance with TS 26.114 [104] when sending any data packets.

Upon receipt of any IP headers indicating ECN Congestion Experienced (ECN-CE) the IM-MGW shall trigger rate adaptation in accordance with TS 26.114 [104].
Figure 9.2.11.2.2 shows the message sequence chart example for an ECN Failure Event.

[image: image87.emf]MGCF

IM-

MGW

H.248:NOTIFY.req(ECN Failure) 

[Termination =x, cause=ecn_cause]

H.248:NOTIFY.resp(ECN Failure)

[Termination x]

Context(Cx)

Context(Cx)

Report ECN Failure


Figure 9.2.11.2.2: Procedure to Report ECN Failure
When the MGCF receives a Notification indicating that a failure has occurred, the MGCF may trigger a new SDP offer to dissable ECN.
9.3
Mn Signalling procedures

This clause describes of logical signalling procedures (i.e. message identifiers are not part of the protocol) between the MGCF and IM-MGW. The procedures within this clause are intended to be implemented using the standard H.248 procedure as defined in] ITU recommendation H.248.1 [2] with appropriate parameter combinations.

9.3.1
Procedures related to terminations towards the IM CN Subsystem

A mapping of the procedures defined here to H.248 procedures and parameters is provided in TS 29.332 [15].

9.3.1.1
Reserve IMS connection point

This procedure is used to reserve local connection addresses and local resources.

Table 25: Procedures toward the IM Subsystem: Reserve IMS connection point

	Procedure
	Initiated
	Information element name
	Information element required
	Information element description

	Reserve IMS Connection Point

 
	MGCF
	Context/Context Request
	M
	This information element indicates the existing context or requests a new context for the bearer termination.

	
	
	IMS Termination Request
	M
	This information element requests a new IMS termination for the bearer to be established.

	
	
	Local IMS Resources/
	M
	This information element indicates the resource(s) (i.e. codecs) for which the IM-MGW shall be prepared to receive user data, 

	
	
	IP Interface Type
	O
	This information element indicates the used interface type

	
	
	ReserveValue
	O
	This information element indicates if multiple local IMS resources are to be reserved.

	
	
	Local Connection Addresses Request
	M
	This information element requests an IP address and port number on the IM-MGW that the remote end can send user plane data to.

	
	
	Notify termination heartbeat
	M
	This information element requests termination heartbeat indications.

	
	
	Notify Released Bearer
	O
	This information element requests a notification of a released bearer.

	
	
	IP Realm Identifier
	O
	This information element indicates the IP realm of the IP termination.

	
	
	ECN Enable
	O
	This information element requests the IM-MGW to apply ECN procedures

	
	
	ECN Initiation Method
	C
	This information element specifies the ECN Initiation method and requests the IM-MGW to perform IP header settings as an ECN endpoint. It may be included only if ECN is enabled.

	
	
	Notify ECN Failure Event
	C
	This information element requests a notification if an ECN failure occurs. It may only be supplied if ECN is enabled.

	Reserve IMS Connection Point Ack
	IM-MGW
	Context
	M
	This information element indicates the context where the command was executed.

	
	
	IMS Termination
	M
	This information element indicates the IMS termination where the command was executed.

	
	
	Local IMS Resources
	M
	This information element indicates the resources that the IM-MGW has reserved to receive the user plane data from the IMS.

	
	
	Local Connection Addresses
	M
	This information element indicates the IP address and port on the IM-MGW that shall receive user plane data from IMS.


9.3.1.2
Configure IMS resources

This procedure is used to select multimedia-processing resources for an Mb interface connection.
Table 26: Procedures toward the IM Subsystem: Select Local, 
Select Remote IMS Processing Resource

	Procedure
	Initiated
	Information element name
	Information element required
	Information element description

	Configure IMS Resources
	MGCF
	Context
	M
	This information element indicates the existing context.

	
	
	IMS Termination
	M
	This information element indicates the existing bearer termination.

	
	
	Local IMS Resources
	O
	This information element indicates the resources (i.e. codec) that the IM-MGW may use on the reception of user plane data.

	
	
	Remote IMS Resources
	M
	This information element indicates the resources (i.e. codec) that the IM-MGW may send user plane data to.

	
	
	Local Connection Addresses
	O
	This information element indicates the IP address and port on the IM-MGW that the IMS user can send user plane data to.

	
	
	Remote Connection Addresses
	M
	This information element indicates the IP address and port that the IM-MGW can send user plane data to. 

	
	
	IP Interface Type
	O
	This information element indicates the used interface type

	
	
	Reserve Value
	O
	This information element indicates if multiple IMS resources are to be reserved.

	
	
	Remote Connection Addresses Source Filter
	O
	This information element indicates an optional source filter restricting the IP addresses and ports that the IM-MGW shall accept as source for incoming user plane data. If this information element is set, the IM-MGW shall silently discard incoming user plane data from disallowed sources.

	
	
	ECN Enable
	O
	This information element requests the IM-MGW to apply ECN procedures.

	
	
	ECN Initiation Method
	C
	This information element specifies the ECN Initiation method and requests the IM-MGW to perform IP header settings as an ECN endpoint. It may be included only if ECN is enabled.

	
	
	Notify ECN Failure Event
	C
	This information element requests a notification if an ECN failure occurs. It may only be supplied if ECN is enabled.

	Configure IMS Resources

Ack
	IM-MGW
	Context
	M
	This information element indicates the context where the command was executed.

	
	
	IMS Termination
	M
	This information element indicates the IMS termination where the command was executed.

	
	
	Local IMS Resource
	O
	This information element indicates the resources that the IM-MGW has reserved to receive the user plane data from the far end.

	
	
	Remote IMS Resource
	M
	This information element indicates the resource (i.e. codec) that the IM-MGW shall use to send user data to.

	
	
	Local Connection Address
	O
	This information element indicates the IP address and port on the IM-MGW that the remote end can send user plane data to.

	
	
	Remote Connection Address
	M
	This information element indicates the IP address and port that the IM-MGW can send user plane data to. 


9.3.1.3
Reserve IMS Connection point and configure remote resources

This procedure is used to reserve multimedia-processing resources for an Mb interface connection.

Table 27: Procedures toward the IM Subsystem: reserve local, reserve remote IMS connection point

	Procedure
	Initiated
	Information element name
	Information element required
	Information element description

	 Reserve IMS Connection Point and Configure Remote Resources
	MGCF
	Context/Context Request
	M
	This information element indicates the existing context or requests a new context for the bearer termination.

	
	
	IMS Termination/IMS Termination Request
	M
	This information element indicates the existing bearer termination or requests a new IMS termination for the bearer to be established.

	
	
	Local IMS Resources
	M
	This information element indicates the resource(s) (i.e. codecs) for which the IM-MGW shall be prepared to receive user data, 

	
	
	Remote IMS Resources
	M
	This information element indicates the resources (i.e. codec) that the IM-MGW shall use to send user data in the IMS.

	
	
	IP Interface Type
	O
	This information element indicates the used interface type

	
	
	Reserve Value
	O
	This information element indicates if multiple IMS resources are to be reserved.

	
	
	Local Connection Address request
	M
	This information element requests an IP address and a port number on the IM-MGW that the remote end can send user plane data to.

	
	
	Remote Connection Addresses
	M
	This information element indicates the IP address and ports at an IMS user that the IM-MGW can send user plane data to. 

	
	
	Notify termination heartbeat
	M
	This information element requests termination heartbeat indications.

	
	
	Notify Released Bearer
	O
	This information element requests a notification of a released bearer.

	
	
	IP Realm Identifier
	O
	This information element indicates the IP realm of the IP termination.

	
	
	ECN Enable
	O
	This information element requests the IM-MGW to apply ECN procedures and IP header settings as an ECN endpoint.

	
	
	ECN Initiation Method
	C
	This information element specifies the ECN Initiation method and requests the IM-MGW to perform IP header settings as an ECN endpoint. It may be included only if ECN is enabled.

	
	
	Notify ECN Failure Event
	C
	This information element requests a notification if an ECN failure occurs. It may only be supplied if ECN is enabled.

	Reserve IMS Connection Point and Configure Remote Resources Ack
	IM-MGW
	Context
	M
	This information element indicates the context where the command was executed.

	
	
	IMS Termination
	M
	This information element indicates the IMS termination where the command was executed.

	
	
	Local IMS Resources
	M
	This information element indicates the resources that the IM-MGW has reserved to receive the user plane data from IMS.

	
	
	Remote IMS Resources
	M
	This information element indicates the resource (i.e. codec) that the IM-MGW shall use to send user data.

	
	
	Local Connection Addresses 
	M
	This information element indicates the IP address on the IM-MGW that shall receive user plane data from the IMS.


9.3.1.4
Release IMS termination

This procedure is used by the MGCF to release a termination towards the IMS and free all related resources.

Table 28: Release IMS termination

	Procedure
	Initiated
	Information element name
	Information element required
	Information element description

	Release IMS Termination
	MGCF
	Context
	M
	This information element indicates the context for the bearer termination.

	
	
	Bearer Termination
	M
	This information element indicates the bearer termination to be released.

	Release IMS Termination Ack
	IM-MGW
	Context
	M
	This information element indicates the context where the command was executed.

	
	
	Bearer Termination
	M
	This information element indicates the bearer termination where the command was executed.


9.3.1.5
Detect IMS RTP Tel event

This procedure is used by the MGCF to request from the MGW the detection of telephony events signalled within RTP according to RFC 4733 [105] and the notification of received telephony events. This procedure is the same as that is defined in the clause "Detect DTMF"" in TS 23.205 [27].

Table 29: VOID
9.3.1.6
Notify IMS RTP Tel event

This procedure is used by the MGW to notify the MGCF about the detection of telephony events signalled within RTP according to RFC 4733 [105]. This procedure is the same as that defined in the clause "Report DTMF" in TS 23.205 [27].

Table 30: VOID
9.3.1.7
Void
9.3.1.8
Send IMS RTP Tel event

This procedure is used by the MGCF to request from the MGW to signal a telephone event within RTP according to RFC 4733 [105]. This procedure is the same as that defined in the clause "Send DTMF" in TS 23.205 [27].

9.3.1.9
Stop IMS RTP Tel event

This procedure is used by the MGW to request from the MGW to stop signalling a telephone event within RTP according to RFC 4733 [105]. This procedure is the same as that defined in the clause "Stop DTMF" in TS 23.205 [27].

9.3.1.10
Termination heartbeat indication

This procedure is used to report indication of hanging termination.

Table 30a: Procedures between (G)MSC server and MGW: Hanging termination indication

	Procedure
	Initiated
	Information element name
	Information element required
	Information element description

	Termination heartbeat indication
	MGW
	Context
	M
	This information element indicates the context for the bearer termination.

	
	
	Bearer Termination
	M
	This information element indicates the bearer termination for which the termination heartbeat is reported. 

	
	
	Termination heartbeat
	M
	Hanging Termination event, as defined in TS 29.332 [15].

	Termination heartbeat indication Ack
	(G)MSC-S
	Context
	M
	This information element indicates the context where the command was executed.


9.3.1.11
IMS Bearer Released

This procedure is used by the IM-MGW to indicate towards the MGCF that an error occurred on an IMS termination which requires the release of the termination. This procedure is the same as that defined in the clause "Bearer Released" in TS 23.205 [27].

9.3.1.12
End IMS RTP Tel event

This procedure is used by the MGCF to indicate to the IM-MGW to stop detection of telephony events signalled within RTP according to IETF RFC 4733 [105]. This procedure is the same as that is defined in the clause "Stop Detect DTMF" in TS 23.205 [27].

9.3.1.13
IMS Send Tone

This procedure is used by the MGCF to order the IM-MGW to generate a tone at termination towards IMS. This procedure is the same as that defined in the clause "Send Tone" in TS 23.205 [27].

9.3.1.14
IMS Stop Tone

This procedure is used by the MGCF to order the IM-MGW to stop generating a tone at a termination towards IMS. This procedure is the same as that defined in the clause "Stop Tone" in TS 23.205 [27].

9.3.1.15
IMS Tone Completed

This procedure is used by the IM-MGW to indicate to the MGCF that a tone has finished being generated at a termination. This procedure is the same as that defined in the clause "Tone Completed" in TS 23.205 [27].
9.3.1.16
ECN Failure Indication

This procedure is used to report ECN related errors (see clause 9.2.11.1c).

Table 9.3.1.16.1: Procedures toward the IM Subsystem: ECN Failure Indication

	Procedure
	Initiated
	Information element name
	Information element required
	Information element description

	ECN Failure Indication
	IM-MGW
	Context
	M
	This information element indicates the context for the bearer termination.

	
	
	Bearer Termination
	M
	This information element indicates the bearer termination for which the ECN failure is reported. 

	
	
	ECN Failure Indication
	M
	This IE indicates an ECN failure event.

	ECN Failure Indication Ack
	MGCF
	Context
	M
	This information element indicates the context where the command was executed.

	
	
	Bearer Termination
	M
	This information element indicates the bearer termination where the command was executed


9.3.2
Procedures related to a termination towards an ISUP network

A mapping of the procedures defined here to H.248 procedures and parameters is provided in TS 29.332 [15].

9.3.2.1
Reserve TDM circuit

This procedure is used by the MGCF to reserve a TDM circuit in the IM-MGW towards the preceding/succeeding CS network element.

Table 31: Reserve TDM circuit procedure

	Procedure
	Initiated
	Information element name
	Information element required
	Information element description

	Reserve TDM Circuit
	MGCF
	Context/Context Request
	M
	This information element indicates the existing context or requests a new context for the bearer termination.

	
	
	Bearer Termination
	M
	This information element indicates the physical bearer termination for the TDM circuit.

	
	
	Bearer Service Characteristics
	M
	This information element indicates the bearer service requested by the user. 

	
	
	Notify termination heartbeat
	O
	This information element requests termination heartbeat indications.

	
	
	Notify Released Bearer
	O
	This information element requests a notification of a released bearer

	Reserve Circuit Ack
	IM-MGW
	Context
	M
	This information element indicates the context where the command was executed.


9.3.2.2
Change TDM through-connection

This procedure is used by the MGCF to modify the through-connection (forward, backward, both-way, inactive) of a TDM termination at the IM-MGW towards the PSTN.

This procedure is the same as Change Through Connection in TS 23.205 [27].

9.3.2.3
Activate TDM voice-processing function

This procedure is used by the MGCF to activate or de-activate a voice processing function of a TDM termination at the IM-MGW towards the PSTN. This voice processing function may include a cancellation for electronic echoes.

This procedure is the same as Activate Voice Processing Function in TS 23.205 [27].

9.3.2.4
Send TDM tone

This procedure is used by the MGCF to order the IM-MGW to generate a tone at a TDM termination towards the PSTN.

This procedure is the same as Send Tone in TS 23.205 [27].

9.3.2.5
Stop TDM tone

This procedure is used by the MGCF to order the IM-MGW to stop generating a tone at a TDM termination towards the PSTN.

This procedure is the same as Stop tone in TS 23.205 [27].

9.3.2.6
Play TDM announcement

This procedure is used by the MGCF to order the IM-MGW to generate an announcement at a TDM termination towards the PSTN. The MGCF may request a notification that the announcement is completed. This procedure is the same as Play Announcement in TS 23.205 [27]. This procedure is optional.

9.3.2.7
TDM announcement completed

This procedure is used by the IM-MGW to notify the MGCF that an announcement at a TDM termination towards the PSTN is completed. This procedure is the same as Announcement Completed in TS 23.205 [27]. This procedure is optional.

9.3.2.8
Stop TDM announcement

This procedure is the same as Stop Announcement TS 23.205 [27]. This procedure is used by the MGCF to order the IM-MGW to stop generating an announcement at a TDM termination towards the PSTN. This procedure is optional.

9.3.2.9
Continuity check

This procedure is used by the MGCF to order the IM-MGW to generate a continuity check tone at a TDM termination towards the PSTN and to inform the MGCF about the result of the continuity check as soon as the continuity check tone is received or a time-out occurs. This procedure is optional.

Table 32: Continuity check procedure

	Procedure
	Initiated
	Information element name
	Information element required
	Information element description

	Continuity check
	MGCF
	Context/Context Request
	M
	This information element indicates the existing context or requests a new context for the bearer termination.

	
	
	TDM Termination
	M
	This information element indicates the existing bearer termination

	
	
	Request for continuity tone sending
	M
	This information request the IM-MGW to apply the continuity check procedure on the indicated TDM termination

	
	
	Request for continuity check tone detection


	M
	This information request the IM-MGW to inform e continuity check procedure on the indicated TDM termination


9.3.2.10
Continuity check verify

This procedure is used by the IM-MGW to indicate towards the MGCF that the continuity check at a TDM termination towards the PSTN has been completed and to return the result of the check: success or failure. This procedure is optional.

Table 33: Continuity check verify procedure

	Procedure
	Initiated
	Information element name
	Information element required
	Information element description

	Continuity check

Verify
	IM-MGW
	Context/t
	M
	This information element indicates the context where the command was executed.

	
	
	TDM Termination
	M
	This information element indicates the TDM termination involved in the procedure

	
	
	Outcome of the continuity check
	M
	This information element indicates the outcome of the continuity check (successful/unsuccessful)


9.3.2.11
Continuity check response

This procedure is used by the MGCF to order the IM-MGW to loop back an incoming continuity check tone at a TDM termination towards the PSTN. This procedure is optional.

Table 34: Continuity check response procedure

	Procedure
	Initiated
	Information element name
	Information element required
	Information element description

	Continuity check response
	MGCF
	Context/Context Request
	M
	This information element indicates the existing context or requests a new context for the bearer termination.

	
	
	TDM Termination
	M
	This information element indicates the existing bearer termination

	
	
	Request for loop back of the continuity tone
	M
	This information request the IM-MGW to loop back the continuity check tone on the indicated TDM termination


9.3.2.12
Release TDM termination

This procedure is used by the MGCF to release a TDM termination at the IM-MGW towards the PSTN and free all related resources.

Table 35: Release TDM termination procedure

	Procedure
	Initiated
	Information element name
	Information element required
	Information element description

	Release TDM Termination
	MGCF
	Context
	M
	This information element indicates the context for the bearer termination.

	
	
	Bearer Termination
	M
	This information element indicates the bearer termination to be released.

	Release TDM Termination Ack
	IM-MGW
	Context
	M
	This information element indicates the context where the command was executed.

	
	
	Bearer Termination
	M
	This information element indicates the bearer termination where the command was executed.


9.3.2.13
Termination Out-of-Service

This procedure is used by the IM-MGW to indicate towards the MGCF that one or several physical termination(s) will go out of service. This procedure is the same as Termination Out-of-Service in TS 23.205 [27].

9.3.2.14
Termination heartbeat indication

This procedure is used to report indication of hanging termination.

Table 35a: Procedures between (G)MSC server and MGW: Hanging termination indication

	Procedure
	Initiated
	Information element name
	Information element required
	Information element description

	Termination heartbeat indication
	MGW
	Context
	M
	This information element indicates the context for the bearer termination.

	
	
	Bearer Termination
	M
	This information element indicates the bearer termination for which the termination heartbeat is reported. 

	
	
	Termination heartbeat
	M
	Hanging termination event, as defined in TS 29.332 [15].

	Termination heartbeat indication Ack
	(G)MSC-S
	Context
	M
	This information element indicates the context where the command was executed.


9.3.2.15
Bearer Released

This procedure is used by the IM-MGW to indicate towards the MGCF that an error occurred on a physical termination which requires the release of the termination. This procedure is the same as Bearer Released in TS 23.205 [27].

9.3.2.16
TDM tone completed

This procedure is used by the IM-MGW to MGCF to indicate that a tone has finished being generated at a TDM termination.

This procedure is the same as Tone Completed in TS 23.205 [27].
9.3.3
Procedures related to a termination towards a BICC network

The call related procedures detailed in table 36 shall be supported. Those procedures are defined in TS 29.332 [15].
Table 36: Required procedures defined in TS 29.332 [15]
	Procedure defined in TS 29.332 [15]
	Remarks

	Establish Bearer
	

	Prepare Bearer
	

	Change Through-Connection
	

	Release Bearer 
	

	Release Termination
	

	Bearer Established
	

	Bearer Released
	

	Send Tone
	

	Stop Tone
	

	Tone completed
	

	Play Announcement
	Optional

	Stop Announcement
	Optional

	Announcement Completed
	Optional

	Confirm Char
	Optional

	Modify Bearer Characteristics
	Optional

	Reserve Char
	Optional

	Bearer Modified
	Optional

	Activate Voice Processing Function
	Optional

	Tunnel Information Down
	Conditional: For IP Transport at BICC termination

	Tunnel Information Up
	Conditional: For IP Transport at BICC termination

	Termination Out-of-Service
	

	Termination heartbeat indication
	


9.3.4
Non-call related procedures

The procedures from TS 23.205 [27] detailed in table 37 shall be applied for the IM‑MGW handling component of the Mn interface.

Table 37: Non-call related procedures

	Procedure defined in

 TS 29.332 [15]
	Corresponding Procedure defined in TS 23.205 [27]
	Remarks

	IM-MGW Out of service
	MGW Out of Service
	

	IM-MGW Communication Up
	MGW Communication Up
	

	IM-MGW Restoration
	MGW Restoration
	

	IM-MGW Register
	MGW Register
	

	IM-MGW Re-register
	MGW Re-register
	

	MGCF Ordered Re-register
	(G)MSC Server Ordered Re-register
	

	MGCF Restoration
	(G)MSC Server Restoration
	

	MGCF Out of Service
	(G)MSC Server Out of Service
	

	Termination Out-of-Service
	Termination Out-of-Service
	The "Termination Out-of-Service procedure" is used as call-related H.248 command as well

	Termination Restoration
	Termination Restoration
	

	Audit Value
	Audit Value
	

	Audit Capability
	Audit Capability
	

	Command Rejected
	Command Rejected
	The "Command Rejected" procedure may be used in response both to call-related and non-call-related H.248 Commands.

	IM-MGW Capability Change
	Capability Update
	

	IM-MGW Resource Congestion Handling - Activate
	MGW Resource Congestion Handling - Activate
	

	IM-MGW Resource Congestion Handling - Indication
	MGW Resource Congestion Handling - Indication
	

	Control association monitoring
	Control association monitoring
	

	Inactivity timeout activation
	Inactivity timeout activation
	

	Inactivity timeout indication
	Inactivity timeout indication
	

	Hanging termination detection
	Hanging termination detection
	

	Hanging termination detection
	Hanging termination detection
	


9.3.5
Multiple IP Realms

The procedures to support multiple IP realms in the present clause are optional.

Figure 9.3.5.1 shows a scenario where multiple IP realms are applied.


[image: image88.emf] 

IM - MGW  

 

IM CN 1  

Termination 1   Termination 2  

 

CS  CN 1  


Figure 9.3.5.1 Multiple IP realms scenario

Shown in Figure 9.3.5, the IM CN1 and CS CN1 represent separate IP realms. The definition of IP realm is specified in IETF RFC 2663[90].

The termination1 and termination2 are connected with different IP realms in the IM-MGW separately.

For establishing session when multiple IP realms are used in the IM-MGW, the MGCF may indicate the IP realm identifier to the IM-MGW. The IM-MGW shall assign the IP termination with the indicated IP realm.

A default IP realm may be configured in IM-MGW such that if the IM-MGW has not received the IP realm identifier and the IM-MGW supports multiple IP realms then the default IP realm shall be used.

If the IM-MGW does not support the option to indicate an IP realm then it is free to select any IP port.
Annex A (informative): 
Void
Annex B (normative):
Codec Negotiation between a BICC CS network and the IM CN subsystem

B.1
Introduction

This annex describes optional procedures for interworking of codec negotiation between a BICC CS network and the IM CN subsystem.

B.2
Control plane interworking

The following optional procedures apply in addition to the procedures of clause 7.3 when both the BICC CS network and the IM CN subsystem support codec negotiation. All five variations of the bearer set-up procedures defined in clauses 7.4 and 7.5 of ITU-T Q.1902.4 [30] are supported. The codec negotiation procedures are also independent of the procedures for interworking between continuity procedures and SDP preconditions.

B.2.1
Incoming call interworking from SIP to BICC at I-MGCF

B.2.1.1
Sending of IAM

When the I-MGCF receives an INVITE with SDP offer, the I-MGCF shall follow the procedures of clause B.2.5 to convert the list of codecs in the SDP offer into a Supported Codec List for transmission in the outgoing IAM, according to clause 8.3.1 of ITU-T Q.1902.4 [30], and deleting those codecs not supported at the IM-MGW. When generating the Supported Codec List, the I-MGCF should add to the SDP offer all codec configurations for which it can provide transcoding. The I-MGCF shall allocate any IM-MGW resources as necessary to support the chosen bearer set-up procedures towards the BICC CS network.

When the I-MGCF receives an INVITE without SDP offer, the I-MGCF shall continue call establishment without interworking of codec negotiation procedures. The mid-call interworking procedures of clause B.2.3 and clause B.2.4 may still apply.

B.2.1.2
Sending of SDP answer

The I-MGCF shall suspend the SDP answer procedure until it receives backward codec information from the BICC serving node terminating codec negotiation. When the I-MGCF receives the backward codec information, it shall select a codec configuration for use on the bearer interface to the IM CN subsystem from the codecs in the SDP offer, format an SDP answer based on this selected codec, send the SDP answer to the offerer in the appropriate SIP message (e.g., a reliable 18x response), and complete bearer establishment procedures. To avoid allocating a transcoder at the IM-MGW, the I-MGCF should preferably select a codec for the IM CN subsystem by converting the Selected Codec from the BICC CS network into an SDP answer according to the procedures of clause B.2.5, if allowed by the SDP offer/answer rules. Otherwise the I-MGCF should select the highest priority codec from the codecs in the received SDP offer supported by the IM-MGW for insertion in the SDP answer. Note that the I-MGCF stores the Available Codec List and does not send it to the offerer in the SDP answer. Codec negotiation is complete so it is not necessary for the offerer to begin a second phase offer/answer exchange using the PRACK request.

B.2.2
Outgoing call interworking from BICC to SIP at O-MGCF

B.2.2.1
Sending of INVITE

When the O-MGCF receives an IAM, the O-MGCF shall follow the procedures of clause B.2.5 to convert the Supported Codec List from the IAM into an SDP offer for transmission in the outgoing INVITE request, according to RFC 3264, deleting those codecs not supported at the IM-MGW. When generating the SDP offer, the O-MGCF should include all codec configurations for which it can provide transcoding in addition to those converted from the Supported Codec List. The O-MGCF shall include at least one AMR codec configuration in the SDP offer. The O-MGCF shall allocate any IM-MGW resources as necessary to support the inclusion of session address information in the SDP offer towards the IM CN subsystem.

B.2.2.2
Responding to serving node initiating codec negotiation

The O-MGCF shall suspend the incoming bearer set-up procedure while waiting for receipt of the SDP answer from the IM CN subsystem. When the O-MGCF receives the SDP answer while suspending the incoming bearer set-up procedure, it shall select a codec configuration for use on the bearer interface to the IM CN subsystem from the codecs in the SDP answer, construct the Available Codec List for the BICC CS network from the list of codecs received in the Supported Codec List by removing codecs not supported at the IM-MGW, choose the Selected Codec for the BICC CS network from the codecs in the Available Codec List, initiate the second SDP offer/answer exchange with the IM CN subsystem using the codec selected for the IM CN subsystem, if necessary, and resume the incoming bearer set-up procedure in the BICC CS network. The O-MGCF should select codecs for the bearer interfaces to the BICC CS network and IM CN subsystem in such a way as to avoid transcoding at the IM-MGW and minimize speech degradation, if possible, according to clause B.2.5. Otherwise the O-MGCF should choose the highest priority codec from the Available Codec List as the Selected Codec for the BICC CS network, and the highest priority codec from the codecs in the SDP answer as the codec for the IM CN subsystem. If the SDP answer only included a single voice codec, then there is no need for a second SDP offer/answer exchange, and the codec selected for the IM CN subsystem is the codec in the SDP answer.

Certain BICC timers or events can force completion of the incoming bearer set-up procedure before the O-MGCF receives the SDP answer from the IM CN subsystem. In this case, the O-MGCF shall perform the terminating codec negotiation procedure according to clause 8.3.3 of ITU-T Q.1902.4 [30], including all supported codecs in the Available Codec List, and shall resume the incoming bearer set-up procedure without waiting any longer for the SDP answer.

When an SDP answer arrives from the IM CN subsystem in response to the SDP offer in an INVITE request after the BICC incoming bearer set-up procedure has started, the O-MGCF shall select a codec configuration for use on the bearer interface to the IM CN subsystem from the codecs in the SDP answer, choose a new Selected Codec for the BICC CS network from the codecs in the Available Codec List constructed during incoming bearer set-up, and initiate the second SDP offer/answer exchange with the IM CN subsystem using the codec selected for the IM CN subsystem, if necessary. The O-MGCF should select codecs for the bearer interfaces to the BICC CS network and IM CN subsystem in such a way as to avoid transcoding at the IM-MGW and minimize speech degradation, if possible, according to clause B.2.5. Otherwise the O-MGCF should select the highest priority codecs from the available options for the two bearer interfaces. If the SDP answer only included a single voice codec, then there is no need for a second SDP offer/answer exchange, and the codec selected for the IM CN subsystem is the codec in the SDP answer. When the call in the BICC CS network enters a state capable of supporting codec modification, if the new Selected Codec is different from the Selected Codec chosen during the incoming bearer set-up procedure for the BICC CS network, the O-MGCF should initiate the codec modification procedure towards the BICC CS network using the new Selected Codec, according to clause 10.4.1 of ITU-T Q.1902.4 [30].

B.2.3
Mid-call interworking from SIP to BICC at I-MGCF or O-MGCF

B.2.3.1
Receipt of SDP offer

When the MGCF receives a SIP message (e.g. UPDATE request or re-INVITE request) with an SDP offer that is not associated with incoming call bearer establishment or preconditions, if the call is in a state capable of supporting BICC codec negotiation, the MGCF shall follow the procedures of clause B.2.5 to convert the list of codecs in the SDP offer into a Supported Codec List, delete those codecs in the Supported Codec List not supported at the IM-MGW, and initiate the mid-call codec negotiation procedure according to clause 10.4.4 of ITU-T Q.1902.4 [30], by sending an APM with the Supported Codec List and an Action indicator set to "mid-call codec negotiation". When generating the Supported Codec List, the MGCF should add to the SDP offer all codec configurations for which it can provide transcoding.

When the MGCF receives a SIP message with an SDP offer that is not associated with incoming call bearer establishment or preconditions, if the call is not in a state capable of supporting BICC codec negotiation, the MGCF shall respond to the SDP offer with existing procedures for the IM CN subsystem. When the call is in a state capable of supporting BICC codec negotiation, the MGCF may send a re-INVITE request without SDP towards the IM CN subsystem, soliciting a response with an SDP offer, thereby restarting the codec negotiation interworking procedure.

B.2.3.2
Generating SDP answer

After initiating a BICC codec negotiation procedure towards the BICC CS network in response to receipt of a SIP message with an SDP offer from the IM CN subsystem, the MGCF shall suspend the SDP answer procedure until it receives codec information from the succeeding BICC serving node. If the succeeding serving node returns a successful response, the MGCF shall select a codec configuration for use on the bearer interface to the IM CN subsystem from the codecs in the SDP offer, format an SDP answer based on this selected codec, send the SDP answer to the offerer in the appropriate SIP message (e.g. 200 OK (UPDATE) or 200 OK (INVITE)), send an APM to the succeeding serving node with an Action indicator set to "successful codec modification", and complete bearer establishment procedures. To avoid allocating a transcoder at the IM-MGW, the MGCF should preferably select a codec for the IM CN subsystem by converting the Selected Codec from the BICC CS network into an SDP answer according to the procedures of clause B.2.5, if allowed by the SDP offer/answer rules. Otherwise the MGCF should select the highest priority codec from the codecs in the received SDP offer supported by the IM-MGW for insertion in the SDP answer. Note that the MGCF stores the Available Codec List and does not send it to the offerer in the SDP answer.

If the succeeding serving node returns an Action indicator set to "mid-call codec negotiation failure", the MGCF either should send a 488 response to the SDP offerer indicating rejection of the initial SDP offer, or should select the highest priority codec from the codecs in the received SDP offer supported by the IM-MGW, format an SDP answer based on this selected codec, and send the SDP answer to the offerer in the appropriate SIP message. If the MGCF sends a 488 response to the SDP offerer, it should continue the call with the bearer configuration in place before initiating this codec negotiation procedure.

B.2.4
Mid-call interworking from BICC to SIP at I-MGCF or O-MGCF

B.2.4.1
Receipt of mid-call codec negotiation request

When the MGCF receives an APM with an Action indicator set to "mid-call codec negotiation", the MGCF shall follow the procedures of clause B.2.5 to convert the Supported Codec List from the APM into an SDP offer for transmission in an appropriate SIP message (e.g. re-INVITE request) towards the IM CN subsystem, according to RFC 3264 [36], deleting those codecs not supported at the IM-MGW. When generating the SDP offer, the MGCF should include all codec configurations for which it can provide transcoding in addition to those converted from the Supported Codec List. The MGCF shall include at least one AMR codec configuration in the SDP offer.

B.2.4.2
Responding to serving node initiating mid-call codec negotiation

The MGCF shall delay responding to the mid-call codec negotiation from the BICC CS network until it receives a response to the SDP offer from the IM CN subsystem. If the MGCF receives an SDP answer, it shall construct the Available Codec List for the BICC CS network from the list of codecs received in the Supported Codec List by removing codecs not supported at the IM-MGW, choose the Selected Codec for the BICC CS network from the codecs in the Available Codec List, and complete the mid-call codec negotiation procedure towards the preceding serving node according to clause 10.4.5 of ITU-T Q.1902.4 [30]. The MGCF should choose the Selected Codec for the BICC CS network in such a way as to avoid transcoding at the IM-MGW and minimize speech degradation, if possible, according to clause B.2.5. Otherwise the MGCF should choose the highest priority codec from the Available Codec List for the Selected Codec for the BICC CS network. If the MGCF receives an APM from the preceding serving node with an Action indicator set to "codec modification failure", then the MGCF may initiate a new SDP offer/answer exchange towards the IM CN subsystem in an attempt to recreate the bearer configuration in place before this codec negotiation procedure began.

If the MGCF receives a 488 response or other failure response (e.g. 3xx-6xx) to the SDP offer, either it should reject the mid-call codec negotiation from the BICC CS network by sending an APM with an Action indicator set to "mid-call codec negotiation failure" towards the preceding serving node, or it should continue as if it received an SDP answer with no change in codec selected for the IM CN subsystem. If the MGCF sends an APM with an Action indicator set to "mid-call codec negotiation failure", it should continue the call with the bearer configuration in place before initiating this codec negotiation procedure.

B.2.4.3
Receipt of codec modification request

If the MGCF receives an APM from a BICC CS network that includes an Action indicator set to "modify codec" with no change in the selected codec, it shall act as a serving node terminating codec modification, according to clause 10.4.2 of ITU-T Q.1902.4 [30], without interworking the procedure with the IM CN subsystem.

If the MGCF receives an APM from a BICC CS network that includes an Action indicator set to "modify codec" and the new selected codec in the message is different from the Selected Codec at the IM-MGW bearer interface to the BICC CS network, the MGCF either may act as a serving node terminating codec modification, according to clause 10.4.2 of ITU-T Q.1902.4 [30], without interworking the procedure with the IM CN subsystem, or may follow the procedures of clause B.2.5 to convert the new Available Codec List (with new priority order) from the APM into an SDP offer for transmission in an appropriate SIP message (e.g. re-INVITE request) towards the IM CN subsystem, according to RFC 3264 [36], deleting those codecs not supported at the IM-MGW. When generating the SDP offer, the MGCF should include all codec configurations for which it can provide transcoding in addition to those converted from the new Available Codec List. The MGCF shall include at least one AMR codec configuration in the SDP offer.

If the MGCF sends a SIP message with an SDP offer towards the IM CN subsystem in response to receipt of a BICC codec modification request, then it shall delay responding to the BICC codec modification request until it receives a response to the SDP offer from the IM CN subsystem. When the MGCF receives either an SDP answer or a rejection of the SDP offer within the appropriate SIP message (e.g. 200 OK (INVITE)) from the IM CN subsystem, it shall decide whether to accept or reject the BICC codec modification procedure and complete the procedure for a BICC serving node terminating codec modification, according to clause 10.4.2 of ITU-T Q.1902.4 [30].

If the MGCF sends an APM with an Action indicator set to "codec modification failure" in response to receipt of a codec modification request, the preceding BICC serving node may retry the request with a mid-call codec negotiation using an APM including an Action indicator set to "mid-call negotiation" and a Supported Codec List with a new priority order encouraging selection of a new codec.
B.2.5
Codec parameter translation between BICC CS network and the IM CN subsystem

The IM CN subsystem uses the Session Description Protocol (SDP, defined in IETF RFC 4566 [56]) to select and potentially re-negotiate the codec type and configuration and associated bearer format attributes to be used in the user plane. IETF RFC 3550 [51] defines the Real Time Protocol (RTP) for framing of all codecs in the user plane, IETF RFC 3551 [52] and IETF RFC 3555 [53] define the framing details for many of the ITU-T codecs, and IETF RFC 4867 [23] defines framing details for the AMR family of codecs. This clause will focus only on codec-specific SDP parameters not already constrained by clause 5.1.1 of TS 26.236 [32]. The signalling plane of the IM CN subsystem uses SDP offer/answer procedures defined in IETF RFC 3264 [36] to select the desired codec type and configuration for the user plane from a prioritized list of codec types and configurations and to re-negotiate the user plane attributes as necessary.

The bearer independent CS network uses the Single Codec and Codec List information elements of the Application Transport Mechanism (APM) defined in ITU-T Recommendation Q.765.5 [35] to negotiate (offer and select) and potentially re-negotiate the codec type and configuration and associated bearer format attributes to be used in the user plane. TS 29.414 [25] and TS 29.415 [26] define the IuFP framing protocol for all codecs in the user plane for both ATM and IP transport, and TS 26.102 [50] provides the framing details for AMR and PCM family codecs. The Codec List information element of the APM comprises multiple instances of the Single Codec information element in priority order, as shown in Figure 13 of ITU-T Recommendation Q.765.5 [35]. Figure 14 of ITU-T Recommendation Q.765.5 [35] defines the Single Codec information element. clause 11.1.7.2 of ITU-T Recommendation Q.765.5 [35] defines the encoding of the Single Codec information element for the ITU-T codecs. TS 26.103 [57] defines the encoding of the Single Codec information element for the 3GPP codecs, and table 7.11.3.1.3-2 of TS 28.062 [58] defines the preferred configurations of the narrowband AMR codecs (Config-NB-Code) for interoperation with TFO. The signalling plane of the bearer independent CS network uses the APM to negotiate the desired codec type and configuration for the user plane from the prioritized list of codec types and to re-negotiate the user plane attributes as necessary.

The following clauses define the translations between the SDP payload format parameters of the IM CN subsystem and the corresponding subfields of the Single Codec information element of the bearer independent CS network for certain 3GPP and ITU-T codecs. Following these translation rules will in many cases allow the IM-MGW to perform interworking between the framing protocols on the bearer interfaces to the BICC CS network and the IM CN subsystem without transcoding. Implementations may signal other codec types not listed herein or other codec configurations of codec types listed herein. Implementations may also choose to perform transcoding between codec configurations signalled separately for the bearer interfaces to the networks, if necessary, but voice quality may suffer.

B.2.5.1
Codec parameters for 3GPP AMR-NB codecs

Table B.1 shows the correspondence between the codec format parameters in the Single Codec information element (TS 26.103 [57]) and the SDP for the 3GPP narrowband AMR codecs (IETF RFC 4867 [23]).

Table B.1: Mapping between Single Codec subfields and SDP parameters for 3GPP AMR-NB codecs

	Single Codec information element
	SDP payload format parameters

	Codec IDentification
	ACS, SCS, OM, MACS
	Payload Type number
	Encoding name
	Other Parameters
(NOTE 1) (NOTE 2)

	FR_AMR or OHR_AMR or
HR_AMR
	OM=0 or

Selected Codec Type
	dynamic
	AMR
	mode-set=values corresponding to ACS (NOTE 3)

	FR_AMR or OHR_AMR or
HR_AMR
	(OM=1 or OM not present) and

(Supported Codec List or Available Codec List)
	dynamic
	AMR
	mode-set=select from values corresponding to ACS, SCS and MACS (NOTE 3)



	UMTS_AMR
	OM=0 or

Selected Codec Type
	dynamic
	AMR
	mode-set=values corresponding to ACS

	UMTS_AMR
	(OM=1 or OM not present) and

(Supported Codec List or Available Codec List)
	dynamic
	AMR
	mode-set=select from values corresponding to ACS, SCS and MACS (NOTE 4)


	UMTS_AMR_2
	OM=0 or

Selected Codec Type
	dynamic
	AMR
	mode-set=values corresponding to ACS (NOTE 5)

	UMTS_AMR_2
	(OM=1 or OM not present) and

(Supported Codec List or Available Codec List)
	dynamic
	AMR
	mode-set=select from values corresponding to ACS, SCS and MACS (NOTE 3) (NOTE 5)

	NOTE 1:
Table 1 of IETF RFC 4867 [23] provides the correspondence between codec rates and AMR modes for use when generating the "mode-set" parameter. When all modes are selected for use, the "mode-set" parameter shall not be included in SDP.

NOTE 2:
SDP payload format configurations in this table with only one value in the "mode-set" parameter shall not include the "mode-change-period" and "mode-change-neighbor" parameters.

NOTE 3:
Payload types for FR_AMR, OHR_AMR and HR_AMR with more than one value in the "mode-set" parameter shall include the "mode-change-period=2" and "mode-change-capability=2" parameters and should include the "mode-change-neighbor=1" parameter.

NOTE 4:
IETF RFC 4867 [23] does not currently provide a mechanism to signal the SCS, MACS or OM parameters in SDP, nor does it distinguish between the different AMR-NB codec types. Each AMR-NB codec type in the Supported Codec List or the Available Codec List with OM=1 should be translated into a list of SDP payload formats in priority order, where each includes a "mode-set" parameter with a unique value derived from the ACS, SCS and MACS. Each "mode-set" should correspond to a codec configuration that is compatible with the given codec type according to the compatibility rules defined in clauses 11 and 12 of TS 28.062 [58].
NOTE 5:
Payload types for UMTS_AMR_2 should include the "mode-change-period=2", "mode-change-capability=2" and "mode-change-neighbor=1" parameters, normally used for signalling GSM AMR codecs, to assure end-to-end interoperability with OoBTC and TFO. Its actual capabilities would otherwise be signalled without these parameters.


Definitions:

Supported Codec List: contains the offered Codec Types and Configuration-possibilities of the node initiating codec negotiation in BICC (see also TS 23.153). The Supported Codec List is sent from the initiating node forward to the terminating node. The Supported Codec List corresponds to an SDP offer during codec negotiation.
Available Codec List: contains the offered Codec Types and Configuration-possibilities of the contiguous portion of the connection between initiating and terminating BICC nodes, including all intermediate nodes through the BICC network(s). The Available Codec List is sent from the BICC node terminating codec negotiation backward to the initiating node. The Available Codec List corresponds to information sometimes available in a first-round SDP answer. The Available Codec List might not represent an end-to-end view of the available Codec Types and Configuration-possibilities when traversing both BICC and SIP networks.

Selected Codec Type: is determined by the node terminating codec negotiation. It specifies exactly the Codec Type and one unique Codec Configuration for the call. The Selected Codec Type corresponds to the final SDP answer.

When translating from a Single Codec information element to the equivalent SDP payload format parameters, where either OM=0 (in the Supported or Available Codec List) or the information element is the Selected Codec Type, the SDP shall include a single payload type and any associated parameters from the corresponding row in table B.1. When translating from a Single Codec information element to the equivalent SDP payload format parameters, where OM=1 in the Supported or Available Codec List, the SDP shall only include payload formats corresponding to Codec Configurations compatible with the offered ACS, SCS and MACS, according to table B.1. Since the number of compatible payload formats can be large, implementations should select a reasonable subset of the higher-priority payload formats for inclusion in the SDP. When translating a list of Single Codec information elements into SDP, duplicate payload types (matching on all parameters) shall be removed.

The following guidelines shall apply when translating from an SDP payload format specification to a Single Codec information element:

-
If there is no "mode-set" parameter for a payload format in the SDP and the SDP is to be translated into a Supported or Available Codec List, then the corresponding Single Codec subfields shall be OM=1, MACS=8, all SCS modes offered, and ACS modes offered. Alternatively it is sufficient to specify only the Codec Type (see below) and omit the other parameters.

-
If there is no "mode-set" parameter for a payload format in an SDP answer that is to be translated into a Selected Codec Type, then the corresponding Single Codec subfields shall be derived from the payload type in the SDP offer (to which the SDP answer was sent in response).

-
If there is a "mode-set" parameter for a payload format in the SDP, then the corresponding Single Codec subfields shall be OM=0 and ACS modes selected according to the value of "mode-set". The SCS shall be set identical to the ACS and MACS shall be set to the number of modes in the ACS. If this "mode-set" does not represent a valid configuration for the Codec Type (determined by OoBTC procedures), then the payload format shall not be translated.

-
If a payload format in an SDP offer that is to be translated into a Supported Codec List includes "mode-change-period=2" parameter or includes "mode-change-capability=2" parameter, then the Codec IDentification value for the corresponding Single Codec shall be FR_AMR.

-
If a payload format in an SDP answer that is to be translated into a Selected Codec Type or Available Codec List includes "mode-change-period=2", then the Codec IDentification value for the corresponding Single Codec shall be one of FR_AMR, HR_AMR, OHR_AMR or UMTS_AMR_2, if offered in the Supported Codec List.

-
If a payload format in an SDP offer that is to be translated into a Supported Codec List does not include "mode-change-period=2" parameter and does not include "mode-change-capability=2" parameter, then the Codec IDentification value for the corresponding Single Codec shall be UMTS_AMR.

-
If a payload format in an SDP answer that is to be translated into a Selected Codec Type or Available Codec List does not include "mode-change-period=2" parameter and does not include "mode-change-capability=2" parameter, then the Codec IDentification value for the corresponding Single Codec shall be one of UMTS_AMR_2, FR_AMR, HR_AMR, OHR_AMR or UMTS_AMR, if offered in the Supported Codec List.

B.2.5.2
Codec parameters for 3GPP AMR-WB codecs

Table B.2 shows the correspondence between the codec format parameters in the Single Codec information element (TS 26.103 [57]) and the SDP for the 3GPP wideband AMR codecs (IETF RFC 4867 [23]).

Table B.2: Mapping between Single Codec subfields and SDP parameters for 3GPP AMR-WB codecs

	Single Codec information element
	SDP payload format parameters

	Codec IDentification
	Config-WB-Code
	Payload Type number
	Encoding name
	Other Parameters

(NOTE 1)

	FR_AMR-WB or
OHR_AMR-WB
	0
	dynamic
	AMR-WB
	mode-set=0,1,2 


	OFR_AMR-WB or
UMTS_AMR-WB
	0
	dynamic
	AMR-WB
	mode-set=0,1,2
(NOTE 2)

	OFR_AMR-WB or
UMTS_AMR-WB 
	1
	dynamic
dynamic
dynamic
	AMR-WB
AMR-WB
AMR-WB
	mode-set=0,1,2
mode-set=0,1,2,8
mode-set=0,1,2,4
(NOTE 2)

	OFR_AMR-WB or
UMTS_AMR-WB 
	2
	dynamic
	AMR-WB
	mode-set=0,1,2,4
(NOTE 2)

	OFR_AMR-WB or
UMTS_AMR-WB 
	3
	dynamic
dynamic
dynamic
	AMR-WB
AMR-WB
AMR-WB
	mode-set=0,1,2,4
mode-set=0,1,2,8
mode-set=0,1,2
(NOTE 2)

	OFR_AMR-WB or
UMTS_AMR-WB 
	4
	dynamic
	AMR-WB
	mode-set=0,1,2,8
(NOTE 2)

	OFR_AMR-WB or
UMTS_AMR-WB 
	5
	dynamic
dynamic
dynamic
	AMR-WB
AMR-WB
AMR-WB
	mode-set=0,1,2,8
mode-set=0,1,2,4
mode-set=0,1,2
(NOTE 2)

	NOTE 1:
Payload types for FR_AMR-WB, OHR_AMR-WB and OFR_AMR-WB shall include the "mode-change-period=2" and "mode-change-capability=2" parameters and should include the "mode-change-neighbor=1" parameter.

NOTE 2:
Payload types for UMTS_AMR-WB should include the "mode-change-period=2", "mode-change-capability=2" and "mode-change-neighbor=1" parameters, normally used for signalling GSM AMR-WB codecs, to assure end-to-end interoperability with OoBTC and TFO. Its actual capabilities would otherwise be signalled without these parameters. 


When translating from a Single Codec information element to the equivalent SDP payload format parameters, the SDP shall include a distinct payload type and any associated parameters for each row in the table that matches the Config-WB-Code parameter. For example, OFR_AMR-WB with Config-WB-Code=3 can generate three SDP payload types for AMR-WB, each including the "mode-change-period=2" parameter, the "mode-change-neighbor=1" parameter, and the "mode-set" parameter with value sets "0,1,2,4", "0,1,2,8", and "0,1,2", respectively. When translating a list of Single Codec information elements into SDP, duplicate payload types (matching on all parameters) shall be removed.

The following guidelines shall apply when translating from one or more SDP payload format specifications to a Single Codec information element:

-
Payload formats that match except for different values of "mode-set" shall be represented with the fewest values of Config-WB-Code, while retaining the priority represented by the order of the payload formats in the SDP. For example, three SDP payload types for AMR-WB, each including the "mode-change-period=2" parameter, the "mode-change-neighbor=1" parameter, and the "mode-set" parameter with value sets "0,1,2,4", "0,1,2,8", and "0,1,2", respectively, will generate Config-WB-Code=3.

-
If there is no "mode-set" parameter for a payload format in the SDP and the SDP is to be translated into a Supported or Available Codec List, then the corresponding Single Codec shall have a Config-WB-Code value of 1.

-
If there is no "mode-set" parameter for a payload format in an SDP answer that is to be translated into a Selected Codec Type, then the corresponding Config-WB-Code value shall be derived from the payload type in the SDP offer (to which the SDP answer was sent in response).

-
If a payload format in an SDP offer that is to be translated into a Supported Codec List includes "mode-change-period=2" parameter or includes "mode-change-capability=2" parameter, then the Codec IDentification value for the corresponding Single Codec shall be OFR_AMR-WB.

-
If a payload format in an SDP answer is to be translated into a Selected Codec Type or Available Codec List, then the Codec IDentification value for the corresponding Single Codec shall be one of OFR_AMR-WB, FR_AMR-WB, OHR_AMR-WB or UMTS_AMR-WB, if offered in the Supported Codec List.

-
If a payload format in an SDP offer that is to be translated into a Supported Codec List does not include "mode-change-period=2" parameter and does not include "mode-change-capability=2" parameter, then the payload format shall not be translated.
B.2.5.3
Codec parameters for 3GPP non-AMR codecs

Table B.3 shows the correspondence between the codec format parameters in the Single Codec information element (TS 26.103 [57]) and the SDP for the 3GPP non-AMR codecs (IETF RFC 4867 [23], IETF RFC 3551 [52], IETF RFC 3555 [53] and IETF RFC 5993 [114]).

Table B.3: Mapping between Single Codec subfields and SDP parameters for 3GPP non-AMR codecs

	Single Codec information element
	SDP payload format parameters

	Codec IDentification
	Payload Type number
	Encoding name
	Other Parameters

	GSM FR
	3
	GSM
	

	GSM HR
	dynamic
	GSM-HR-08
	

	GSM EFR (NOTE 1)
	dynamic
	GSM-EFR
	

	GSM EFR (NOTE 2)
	dynamic
	AMR
	mode-set=7

	TDMA EFR (NOTE 2)
	dynamic
	AMR
	mode-set=4

	PDC EFR (NOTE 2)
	dynamic
	AMR
	mode-set=3

	NOTE 1:
This translation for GSM EFR (GSM-EFR) is preferred to the alternative (AMR mode-set=7) if it is supported by the IM-MGW.
NOTE 2:
AMR DTX is not compatible with the DTX schemes for any of the codecs in this list. The IM-MGW may support these configurations without transcoding by providing interworking between the DTX procedures and frame encodings on the bearer interfaces to the BICC CS network and the IM CN subsystem.


B.2.5.4
Codec parameters for ITU-T codecs

Table B.4 shows the correspondence between the codec format parameters in the Single Codec information element (Clause 11.1.7 of ITU-T Q.765.5 [35]) and the SDP for the ITU-T codecs (Table 4 of RFC 3551 [52], and RFC 3555 [53]).

Table B.4: Mapping between Single Codec subfields and SDP parameters for ITU-T codecs

	Single Codec information element
	SDP payload format parameters

	Codec Type subfield
	Codec Name
	Codec Configuration subfield (dcba)
	Payload Type number
	Encoding name
	Other Parameters

	00000001
	G.711 64 kbit/s A-law
	N/A
	8
	PCMA
	

	00000010
	G.711 64 kbit/s (-law
	N/A
	0
	PCMU
	

	00000011
	G.711 56 kbit/s A-law
	N/A
	N/A
	N/A
	

	00000100
	G.711 56 kbit/s (-law
	N/A
	N/A
	N/A
	

	00000101
	G.722 (SB-ADPCM)
	N/A
	9
	G722
	

	00000110
	G.723.1
	N/A
	4
	G723
	annexa=no

	00000111
	G.723.1 Annex A (silence suppression)
	N/A
	4
	G723
	

	00001000
	G.726 (ADPCM)
	xxx1
xx1x
x1xx
1xxx
	dynamic
dynamic
dynamic
dynamic
	G726-16
G726-24
G726-32
G726-40
	

	00001001
	G.727 (Embedded ADPCM)
	xxxx
	N/A
	N/A
	

	00001010
	G.728
	111
(subsets of defined rates not supported)
	15
	G728
	

	00001011
	G.729 (CS-ACELP)
	xx1
x1x
1xx
	dynamic
18
dynamic
	G729D
G729
G729E
	annexb=no
annexb=no
annexb=no

	00001100
	G.729 Annex B (silence suppression)
	xx1
x1x
1xx
	dynamic
18
dynamic
	G729D
G729
G72"E"

	NOTE:
An "x" in a bit position of the Codec Configuration subfield indicates a "don't care" value. The SDP payload description for each listed codec includes a clock rate of 8000 Hz. TS 26.102 [50] only describes the BICC CS network framing for the PCM codecs. 


When translating from a Single Codec information element to the equivalent SDP payload format parameters, the SDP shall include a distinct payload type and any associated parameters for each matching instance of the Codec Configuration subfield. For example, G.726 (ADPCM) with Codec Configuration subfield "0101" shall generate SDP payload types for G726-32 and G726-16.

When translating from an SDP payload format specification to the Single Codec information element, each SDP payload type should be represented by one matching Single Codec information element. For example, SDP payload types for G729 and G729E may generate one Single Codec information element for "G.729 Annex B" with Codec Configuration subfield "110". The G729 and G729E codecs may alternately be represented by two Single Codec information elements for "G.729 Annex B" with Codec Configuration subfields "100" and "010", respectively, if it is necessary to indicate preference between them.

B.3
MGCF – IM-MGW interaction during interworking of codec negotiation

B.3.1
Basic IM CN subsystem originated session

This clause shows an example of the interworking of codec negotiation between an IM CN subsystem and a BICC CS network during session establishment for an IM CN subsystem originated session. The example applies to BICC forward bearer establishment. Similar procedures apply to the other four versions of bearer establishment procedure applicable to the BICC CS network. The exchange of codec information is identical in all five cases, but there are differences in the sequence of operations associated with bearer establishment within the BICC CS network.

B.3.1.1
BICC forward bearer establishment

B.3.1.1.1
IM-MGW selection

The MGCF shall select an IM-MGW for the bearer connection before it performs the CS network side bearer establishment. This may happen either before sending the IAM or after receiving the APM message (signal 3 or signal 4 in figure B.1). In the latter case, the IM-MGW selection may be based on a possibly received MGW-id from the succeeding node.

B.3.1.1.2
CS network side bearer establishment

The MGCF shall either select bearer characteristics or request the IM-MGW to select and provide the bearer characteristics for the CS network side bearer connection before sending the IAM. In the latter case the MGCF shall use the Prepare Bearer procedure, not shown in figure B.1, to request the IM-MGW to select the bearer characteristics. After the succeeding node has provided a bearer address and a binding reference in the APM, the MGCF shall use the Establish Bearer procedure to request the IM-MGW to establish a bearer towards the destination CS-MGW. The MGCF shall provide the IM-MGW with the bearer address, the binding reference and the bearer characteristics (signal 5 in figure B.1).

B.3.1.1.3
IM CN subsystem side session establishment

When the MGCF receives the Selected Codec from the succeeding serving node in the CS network (signal 4 in figure B.1) and selects a codec for use in the IM CN subsystem, the MGCF shall initiate the Reserve IMS Connection Point and Configure Remote Resources procedure (signal 7 and 8 in figure B.1). From the received SDP and selected configuration data the MGCF:

-
Shall send the appropriate remote codec(s), the remote UDP port and the remote IP address to the IM-MGW. The remote UDP port and IP address refer to the destination of user plane data sent towards the IM CN subsystem. The remote codec(s) are the codec(s) the IM-MGW may select for user plane data sent towards the IM CN subsystem.

-
Shall indicate to the IM-MGW the appropriate local codec(s) and request a local IP address and UDP port. The local IP address and UDP port are used by the IM-MGW to receive user plane data from the IM CN subsystem. The local codec(s) are the codec(s) the IM-MGW may select to receive user plane data from the IM CN subsystem.

-
If DTMF support together with speech support is required, the reserve value indicator shall be set to "true".

The IM-MGW

-
Shall reply to the MGCF with the selected local codec(s) and the selected remote codec(s) and the selected local UDP port and IP address.

-
Shall reserve resources for those codec(s).

The MCGF shall send the local codec(s), UDP port and IP address to the IMS in the Session Progress (signal 9 in figure B.1).

B.3.1.1.4
Through-connection

During the Prepare Bearer and Establish Bearer procedures, the MGCF shall either use the Change Through-Connection procedure to request the IM-MGW to backward through-connect the BICC terminations, or the MGCF shall use this procedure to both-way through-connect the BICC termination already on this stage (signal 5 in figure B.1). During the Reserve IMS Connection Point procedure, the MGCF shall use the Change IMS Through-Connection procedure to request the IM-MGW to backward through-connect the IMS termination (signal 7 in figure B.1).

When the MGCF receives the BICC:ANM answer indication, it shall request the IM-MGW to both-way through-connect the termination using the Change Through-Connection or Change IMS Through-Connection procedures (signal 20 in figure B.1), unless those terminations are already both-way through-connected.

B.3.1.1.5
Codec handling

The IM-MGW may include a speech transcoder based upon the speech coding information provided to each termination.

B.3.1.1.6
Failure handling in MGCF

If any procedure between the MGCF and the IM-MGW is not completed successfully the default action by the MGCF is to release the session, as described in clause 9.2.6. If the MGCF receives a Bearer Released procedure from the IM-MGW the default action by the MGCF is to release the session as described in clause 9.2.7.

NOTE:
As an implementation option the MGCF may also decide for example to only release the resources in the IM-MGW that caused the failure, possibly select a new IM-MGW for the connection and continue the call establishment using new resources in the selected IM-MGW but such handling is outside of the scope of the present document.

B.3.1.1.7
Message sequence chart

Figure B.1 shows the message sequence chart for the IM CN subsystem originating session with BICC forward bearer establishment where the selection of IM-MGW is done after receipt of the APM. The MGCF then requests the seizure of a CS network side bearer termination and the establishment of the bearer. When the MGCF receives an answer indication, it requests the IM-MGW to both-way through-connect the terminations.


[image: image89.wmf]MGCF

IM-MGW

1. 

SIP

: INVITE

[supported codec list 1]

2. 

SIP

: 100 Trying

3. 

BICC

: 

IAM  [supported codec list 2]

4. 

BICC

: 

APM

 [selected codec 2]

7. 

H.248

: ADD.req [Context ID = C1, Termination ID = ?]

8. 

H.248

: ADD.resp [Context ID = C1, Termination ID = T1]

5. 

H.248

: ADD.req  [Context ID = ?, Termination ID=?]

6. 

H.248

: ADD.resp [Context ID = C1, Termination ID = T2]

9. 

S

IP

:183 [selected codec 1]

10. 

SIP

: PRACK

11. 

S

IP

 :

200 OK (PRACK)

12. 

SIP

: UPDATE

13. 

S

IP

 :

200 OK (UPDATE)

14. 

BICC

: 

COT

15.

 

BICC

: 

ACM

16. 

S

IP

 :180 Ringing

20. 

H.248

:  MOD.req [Context ID=C1, Termination ID = T1]

21. 

H.248

: MOD.resp [Context ID = C1, Termination ID = T1]

19.

 

BICC

: 

ANM

17. 

SIP

: PRACK

18. 

S

IP

 :

200 OK (PRACK)

22. 

S

IP

 :

200 OK (INVITE)

23. 

SIP

: ACK

CALL IN ACTIVE STATE

Change IMS

Through-Connection = both

Reserve IMS Connection Point

and Configure Remote

Resources, Change IMS

Through-Connection = backward

Establish Bearer, Change

Through-Connection = both

Bearer Establishment


Figure B.1: Basic IM CN Subsystem originating session, BICC forward bearer establishment
(message sequence chart)

B.3.2
Basic CS network originated session

This clause shows an example of the interworking of codec negotiation between a BICC CS network and an IM CN subsystem during session establishment for a BICC CS network originated session. The example applies to BICC forward bearer establishment. Similar procedures apply to the other four versions of bearer establishment procedure applicable to the BICC CS network. The exchange of codec information is identical in all five cases, but there are differences in the sequence of operations associated with bearer establishment within the BICC CS network.

B.3.2.1
BICC forward bearer establishment

B.3.2.1.1
IM-MGW selection

The MGCF shall select an IM-MGW for the bearer connection before it performs the IM CN subsystem session establishment or the CS network side bearer establishment.

B.3.2.1.2
IM CN subsystem side termination reservation

The MGCF shall derive from the codec negotiation procedure one or several appropriate local codec(s) the IM-MGW may use to receive user plane data from the IM CN subsystem. The MGCF shall use the Reserve IMS Connection Point procedure (signals 2 and 3 in figure B.2/1). Within this procedure, the MGCF shall indicate the local codec(s) and request a local IP address and UDP port from the IM-MGW. The local IP address and UDP port are used by the IM-MGW to receive user plane data from the IM CN subsystem. If DTMF support together with speech support is required, or if the resources for multiple speech codecs shall be reserved at this stage, the reserve value indicator shall be set to "true".

The IM-MGW shall reply to the MGCF with the selected local codec(s) and the selected local IP address and UDP port.

The MGCF shall send this information in the INVITE (signal 4 in figure B.2/1) to the IM CN subsystem.

B.3.2.1.3
IM CN subsystem side session establishment

The MGCF shall use the Configure IMS Resources procedure (signals 7 and 8 in figure B.2/1) to provide configuration data (derived from SDP received in signal 6 in figure B.2/1 and the codec negotiation procedure) to the IM-MGW as detailed below:

-
The MGCF shall indicate the remote IP address and UDP port, i.e. the destination IP address and UDP port for data sent in the user plane towards the IM CN subsystem,

-
The MGCF shall indicate the remote codec(s), i.e. the speech codec(s) for data sent in the user plane towards the IM CN subsystem.

-
The MGCF may indicate the local codec(s) and the local IP address and UDP port. The MGCF shall indicate the local codec(s) if a change is required.

-
IF DTMF support together with speech support is required, the reserve value indicator shall be set to "true".
The IM-MGW shall reply with the selected remote codec(s) and reserve resources for these codec(s). If local codec(s) were received, the IM-MGW shall also reply with the selected local codec(s) and reserve the corresponding resources.

If the selected local codec(s) differ from the codec(s) received in the SDP of signal 6 in figure B.2/1, the MGCF shall send the local reserved codec(s), and the local IP address and UDP port in the PRACK (signal 9 in figure B.2/1) to the IMS.

B.3.2.1.4
CS network side bearer establishment

The MGCF shall request the IM-MGW to prepare for the CS network side bearer establishment using the Prepare Bearer procedure (signals 11 and 12 in figure B.2/1). Within this procedure, the MGCF shall request the IM-MGW to provide a bearer address, a binding reference and optionally notify when the bearer is established. The MGCF shall also provide the IM-MGW with the bearer characteristics determined by the codec negotiation procedure. After the IM-MGW has replied with the bearer address and the binding reference, the MGCF provides the APM message (signal 13 in figure B.2/1) to the preceding node. The MGCF may also provide the IM-MGW-id in the APM message.

B.3.2.1.5
Called party alerting

The MGCF shall request the IM-MGW to provide an awaiting answer indication (ringing tone) to the calling party using the Send Tone procedure (signals 21 and 22 in figure B.2/1), when the following condition is satisfied:

-
the MGCF receives the first 180 Ringing message

B.3.2.1.6
Called party answer

When the MGCF receives a 200 OK message (signal 23 in figure B.2/2), it shall request the IM-MGW to stop providing the ringing tone to the calling party using the Stop Tone procedure (signals 26 and 27 in figure B.2/2).

B.3.2.1.7
Through-Connection

During the Prepare Bearer procedure, the MGCF shall either use the Change Through-Connection procedure to request the IM-MGW to backward through-connect the BICC termination, or the MGCF shall use this procedure to both-way through-connect the BICC termination already on this stage (signals 11 and 12 in figure B.2/1). During the Reserve IMS Connection Point procedure, the MGCF shall use the Change IMS Through-Connection procedure to request the IM-MGW to backward through-connect the IMS termination (signals 2 and 3 in figure B.2/1).

When the MGCF receives the SIP 200 OK(INVITE) (signal 23 in figure B.2/2), it requests the IM-MGW to both-way through-connect the terminations using the Change IMS Through-Connection or Change Through-Connection procedures (signals 28 and 29 in figure B.2/2), unless those terminations are already both-way through-connected.

B.3.2.1.8
Codec handling

The IM-MGW may include a speech transcoder based upon the speech coding information provided to each termination.

B.3.2.1.9
Failure handling in MGCF

If any procedure between the MGCF and the IM-MGW is not completed successfully, the default action by the MGCF is to release the session as described in clause 9.2.6. If the MGCF receives a Bearer Released procedure from the IM-MGW the default action by the MGCF is to release the session, as described in clause 9.2.7.

NOTE:
As an implementation option the MGCF may also decide for example to only release the resources in the IM-MGW that caused the failure, possibly select a new IM-MGW for the connection and continue the call establishment using new resources in the selected IM-MGW but such handling is outside of the scope of the present document.

B.3.2.1.10
Message sequence chart

Figures B.2/1 and B.2/2 show the message sequence chart for the CS network originating session with BICC forward bearer establishment.

[image: image90.wmf]IM-MGW

MGCF

4. 

SIP

: INVITE

[supported codec list 2]

5. 

SIP

: 100 Trying

1. 

BICC

: 

IAM [supported codec list 1]

11. 

H.248

: ADD.req [Context ID = C1, Termination ID = ?]

12. 

H.248

: ADD.resp [Context ID = C1, Termination ID = T2]

2. 

H.248

: ADD.req  [Context ID = ?, Termination ID=?]

3. 

H.248

: ADD.resp [Context ID = C1, Termination ID = T1]

6. 

S

IP

:183

[available codec list 2]

9. 

SIP

: PRACK

[selected codec 2]

10. 

S

IP

 :

200 OK  (PRACK)

[selected codec 2]

7. 

H.248

: MOD.req Context ID = C1,Termination ID = T1]

  8. 

H.248

: MOD.resp [Context ID = C1, Termination ID = T1 ]

15. 

SIP

: UPDATE

16. 

S

IP

 :

200 OK (UPDATE)

14. 

BICC

: 

COT

17. 

S

IP

 :180 Ringing

18. 

SIP

: PRACK

19. 

S

IP

 :

200 OK (PRACK)

13. 

BICC

: 

APM

 [selected codec 1, available codec list 1]

21. 

H.248

: MOD.req  [Context ID = C1, Termination ID = T2]

22. 

H.248

: MOD.resp [Context ID = C1, Termination ID = T2]

20.

 

BICC

: 

ACM

Reserve IMS Connection Point,

Change IMS Through-

Connection = backward

Configure IMS  Resources

Prepare Bearer,

Change Through-

Connection= both

Send Tone

Bearer Establishment


Figure B.2/1: Basic CS Network Originating Session, BICC forward bearer establishment
 (message sequence chart)


[image: image91.wmf]IM-MGW

MGCF

24. 

H.248

:  MOD.req [Context ID=C1, Termination ID =

T2]

25. 

H.248

: MOD.resp [Context ID = C1, Termination ID = T2]

29. 

SIP

: ACK

26. 

H.248

:  MOD.req  [Context ID=C1, Termination ID =

T1]

27. 

H.248

: MOD.resp [Context ID = C1, Termination ID = T1]

28.

 

BICC

: 

ANM

23. 

S

IP

 :

200 OK (INVITE)

CALL IN ACTIVE STATE

Stop Tone

Change IMS

Through-Connection  = both


Figure B.2/2: Basic CS Network Originating Session, BICC forward bearer establishment
 (message sequence chart continued)

B.3.3
CS network initiated mid-call codec negotiation

Figure B.3 shows the CS network initiated mid-call codec negotiation procedure interworking with the IM CN subsystem. When the MGCF selects the codecs for the CS network and the IM CN subsystem (after signal 3 in figure B.3), the MGCF shall modify the CS network termination and the IM CN subsystem termination on the IM-MGW to conform to the newly selected configuration data on the two interfaces. The MGCF may perform bearer operations (not shown) at the IM-MGW before interworking the initial codec modification request (signal 2 in figure B.3) to determine new connection information, if necessary, or to verify resource availability.


[image: image92.wmf]IM-MGW

MGCF

2. 

SIP

: re-INVITE

[supported codec list 2]

1. 

BICC

: 

APM [supported codec list 1]

6. 

H.248

: MOD.req [Context ID = C1, Termination ID = T2]

7. 

H.248

: MOD.resp [Context ID = C1, Termination ID = T2]

3. 

S

IP

:200 OK (INVITE)

[selected codec 2]

8. 

SIP

: ACK

4. 

H.248

: MOD.req Context ID = C1,Termination ID = T1]

  5. 

H.248

: MOD.resp [Context ID = C1, Termination ID = T1 ]

10. 

BICC

: 

APM [action = successful codec modification]

9. 

BICC

: 

APM

 [selected codec 1]

Configure IMS  Resources

Modify Bearer Characteristics


Figure B.3: CS network initiated mid-call codec negotiation
 (message sequence chart)

B.3.4
IM CN subsystem initiated mid-call codec negotiation

Figure B.4 shows the IM CN subsystem initiated mid-call codec negotiation procedure interworking with a BICC CS network. When the MGCF selects the codecs for the CS network and the IM CN subsystem (after signal 3 in figure B.4), the MGCF shall modify the CS network termination and the IM CN subsystem termination on the IM-MGW to conform to the newly selected configuration data on the two interfaces. The MGCF may perform bearer operations (not shown) at the IM-MGW before interworking the initial codec modification request (signal 2 in figure B.3) to determine new connection information, if necessary, or to verify resource availability. The MGCF may also perform bearer operations (not shown) at the IM-MGW after sending the final APM (signal 8 in figure B.4) to modify transport bandwidth, if necessary.


[image: image93.wmf]IM-MGW

MGCF

1. 

SIP

: re-INVITE

[supported codec list 1]

2. 

BICC

: 

APM [supported codec list 2]

6. 

H.248

: MOD.req [Context ID = C1, Termination ID = T2]

7. 

H.248

: MOD.resp [Context ID = C1, Termination ID = T2]

9. 

S

IP

:200 OK (INVITE)

[selected codec 1]

10. 

SIP

: ACK

4. 

H.248

: MOD.req Context ID = C1,Termination ID = T1]

  5. 

H.248

: MOD.resp [Context ID = C1, Termination ID = T1 ]

8. 

BICC

: 

APM [action = successful codec modification]

3. 

BICC

: 

APM [supported codec list 2]

Configure IMS  Resources

Modify Bearer Characteristics


Figure B.4: IM CN subsystem initiated mid-call codec negotiation
 (message sequence chart)

Annex C (normative):
Interworking of CPC parameter

The CPC extension to tel-URI Parameter is defined in clause 7.2A.12 of TS 24.229 [9]. The SIP "Accept-Language" header is defined in IETF RFC 3261 [19].
C.1
Interworking SIP to ISUP

Table C.1.1 shows the mapping of a "cpc" URI parameter received within tel URI or the userinfo part of SIP URI with user="phone" in a P-Asserted-Identity header in the initial INVITE request to the Calling party's category parameter in the ISUP IAM. When the "cpc" URI parameter value "operator" is received the I-MGCF shall use an Accept-Language header field to determine the value of the Calling party's category parameter.
Table C.1.1: Mapping of the CPC parameter to the ISUP Calling party's category parameter

	SIP Parameters
	ISUP Parameters

	"cpc" URI parameter in
P-Asserted-Identity (NOTE 2)
	Accept-Language
	Calling party's category 

	operator
	fr
	operator, language French

	operator
	en
	operator, language English

	operator
	de
	operator, language German

	operator
	ru
	operator, language Russian

	operator
	es
	operator, language Spanish

	ordinary
	
	ordinary calling subscriber

	test
	
	test call

	payphone
	
	payphone

	unknown
	
	calling party's category unknown at this time (national use)

	mobile-hplmn
	
	mobile terminal located in the home PLMN

	mobile-vplmn
	
	mobile terminal located in a visited PLMN

	emergency
	
	emergency service call per ANSI Standard ATIS-1000113.2005 [117] (NOTE 1)

	NOTE 1:
This is a national/regional specific value. Interworking shall only occur when interconnecting with indicated national network.
NOTE 2:
In case the "cpc" URI parameter is absent or contains values that are not mapped per this table then the ISUP shall contain the default CPC value "ordinary calling subscriber".


In case the Accept-Language header field is not received or contains values that are not in this table then based on operator policy the Calling party's category parameter shall contain the CPC value "operator, language X" (where X is one of the following languages: French, English, German, Russian or Spanish) or national/regional specific value.
C.2
Interworking ISUP to SIP

Table C.2.1 shows the mapping of a Calling party's category received in an ISUP IAM to a "cpc" URI parameter within tel URI or the userinfo part of SIP URI with user="phone" in the P-Asserted-Identity header. When the Calling party's category parameter value "operator, language x" is received the O-MGCF shall generate an Accept-Language header field with the value that corresponds to language x.
Table C.2.1: Mapping of the ISUP Calling party's category parameter to the CPC parameter

	ISUP Parameter
	SIP Parameters

	calling party's category 
	"cpc" URI parameter in P-Asserted-Identity
	Accept-Language

	operator, language French
	operator
	fr

	operator, language English
	operator
	en

	operator, language German
	operator
	de

	operator, language Russian
	operator
	ru

	operator, language Spanish
	operator
	es

	ordinary calling subscriber
	ordinary
	-

	test call
	test
	

	payphone
	payphone
	

	calling party's category unknown at this time (national use)
	unknown
	

	mobile terminal located in the home PLMN
	mobile-hplmn
	

	mobile terminal located in a visited PLMN
	mobile-vplmn
	

	emergency service call per ANSI Standard ATIS-1000113.2005 [117] (NOTE 1)
	emergency
	

	NOTE 1:
This is a national/regional specific value. Interworking shall only occur when interconnecting with indicated national network.
NOTE 2:
In case the calling party's category contains values that are not in this table then based on operator policy the "cpc" URI parameter may be omitted or may contain national/regional specific value.


Annex D:
Void

Annex E (normative):
Multimedia interworking between the IP Multimedia Core Network (CN) Subsystem (IMS) and Circuit Switched (CS) networks

E.1
Basic Multimedia calls interworking between the IMS and CS Networks scenarios

The Interworking between Circuit switched multimedia telephony service, as described in TS 26.110 [78] and TS 26.111 [79], and packet switched multimedia services, as described in TS 26.235 [80] and TS 26.236 [32] is addressed.

[image: image94.wmf] 

3G

PP

 TS 26.111

 

Video I/O 

Equipment

 

Video Codec

 

H.263, [MPEG

-

4, H.261 …]

 

Audio I/O 

Equipment

 

Speech Codec 

3GPP

-

AMR, 

[G.723.1 …]

 

Optional 

Receive Path 

Delay

 

Multiplex/

 

Demultiplex

 

H.223,

 

H

.223 Annex 

A, H.223 

Annex B, 

[H.223 Annex 

C, H.223 

Annex D]

 

User Data 

Applications

 

[T.120, …]

 

Data Protocols

 

[V.14, LAPM, …]

 

System 

Control

 

H.245

 

3GPP 

Network

 

CCSR

L

 

NSRP

[LAP

M/V.42]

 

Call Set

-

up

 

Optional

Multilink

 

 H.324 

Anne

x H

 


Figure E.1.1.1 Overview of relevant CS-Domain Protocols (from TS 26.110 [78])

E.2
Control plane interworking

E.2.1
General

In addition to the control plane Interworking between SIP and ISUP or BICC, interactions between the H.245 signalling at the CS side and SIP/SDP signalling are described.

The establishment of the H.223 multiplexing protocol and the subsequent H.245 signalling procedures take place after the set-up and both-way through-connection of the CS bearer.

E.2.2
Functionalities required in the MGCF for multimedia calls support

In addition to the control plane Interworking between SIP and ISUP or BICC, the MGCF needs to mediate interactions between the H.245 signalling or MONA (Media Oriented Negotiation Acceleration) procedures at the CS side and SIP/SDP signalling at the IMS side. The interactions between H.245 signalling or MONA procedures and SIP/SDP signalling should aim at avoiding media transcoding by selecting the same codec for the CS side and the PS side.

NOTE:
Detailed procedures for the mapping between SDP parameters and H.245 parameters are not specified in the present release.

E.2.3
IM CN subsystem originated session

E.2.3.1
Preconditions used at IMS side

E.2.3.1.1
Interactions between H.245 or MONA and SIP/SDP

Figure E.2.3.1.1.1 shows examples of interactions between H.245 or MONA procedures and SIP/SDP for IM CN subsystem originated session. Most SIP and ISUP or BICC messages are intentionally omitted, since the SDP may be embedded in various SIP messages and since the in‑band H.245 Messages are not tightly coupled with out-of-band ISUP or BICC messages.

Figure E.2.3.1.1.1 assumes that the IMS peer uses the SIP precondition extension to indicate that preconditions have not yet been met.


[image: image95.wmf]Interworking Node

 (

MGCF

+ 

IM MGW

)

1

. 

SIP

: 

INVITE 

[

SDP offer

:

,

m 

= 

H

.

263

, 

MP

4

V

-

ES

, 

H

.

261

;

 m

= 

AMR

,

Preconditions not met

]

3

. 

SIP

: 

[

SDP answer

:

m 

= 

H

.

263

;

 m

= 

AMR

,

Preconditions not met 

]

CALL IN ACTIVE STATE

2

.  

IA

M

8

. 

H

.

245

: 

Terminal Capability Set

[

H

.

261

, 

MP

4

V

-

ES

;

AMR

, 

G

.

711

]

4

. 

Bearer Establishment

7

. 

H

.

245

: 

Terminal Capability Set

[

H

.

263

, 

MP

4

V

-

ES

; 

H

.

261

AMR

]

9

. 

H

.

245

: 

Open Logical Channels

[

MP

4

V

-

ES

, 

AMR

]

CS Network

IMS

11

. 

SIP

:

[

SDP offer

; 

m 

=: 

MP

4

V

-

ES

;

 

a

= 

3

gpp

_

MaxRecvSDUSize 

600

; 

m

= 

AMR

 ]

12

. 

SIP

 

:

[

SDP answer

; 

m 

= 

MP

4

V

-

ES

, 

m

= 

AMR

]

5

. 

ANM

10

. 

SIP

: 

200 

OK 

(

INVITE

)

6

. 

H

.

223 

Bearer Setup

 including possible MONA 

Channel 

establishment method negotiation

Alternative to 

7

-

9

:

 

MONA messages


Figure E.2.3.1.1.1: Interactions between H.245 and SIP/SDP for IM CN subsystem originated session
IMS peer indicates unmet local preconditions

Upon receipt of a SIP INVITE request containing speech and video Codecs (signal 1 in figure E.2.3.1.1.1) the Interworking Node (consisting of MGCF and IM-MGW) starts the call set-up for multimedia call at the CS side by sending an IAM requesting an UDI bearer (signal 2 in figure E.2.3.1.1.1).

If SDP local preconditions, which are not yet met, are contained in signal 1, the Interworking node should immediately send an SDP answer to allow for the IMS-side bearer set-up to progress. The Interworking node selects codecs supported by the IM-MGW and likely to be supported within the CS network and communicates the selected codecs towards the IMS side within an SDP answer message (signal 3 in figure E.2.3.1.1.1). If theses codecs are contained in the SDP offer, the Interworking Node should select the H.263 codec and may select other codec from the SDP offer in addition. The interworking node should include a b:AS bandwidth modifier with a bandwidth suitable for the selected codec(s), but not higher than 64kbit/s plus RTP/UDP/IP overhead, in the SDP answer to request that the peer does not send media with a higher bandwidth.
The Interworking Node shall engage in an H.223 bearer setup (step 6 in figure E.2.3.1.1.1). If the interworking Node supports MONA (Media Oriented Negotiation Acceleration), it shall first attempt a MONA Channel establishment method negotiation according to Annex K of ITU-T Recommendation H.324 [81]. If the interworking node does not support MONA, it shall use the multiplexing level negotiation procedures of Annex C of H.324 [81]. If the Interworking Node supports MONA, but the remote peer does not do so, a fallback to the multiplexing level negotiation procedures of Annex C of H.324 [81] will occur.

If both the Interworking Node and the remote CS terminal support MONA procedures, the MONA procedures as per ITU-T Recommendation H.324 Annex K [81] may be used to replace the H.245 negotiation (signals 7 – 9) as shown in figure E.2.3.1.1.1.

If MONA procedures are not used, the following applies:

-
After the completion of the H.223 bearer setup at the CS side, the Interworking Node shall send a Terminal Capability Set message describing its own capabilities (signal 7 in figure E.2.3.1.1.1). Unless the Interworking Node supports transcoding, the Interworking Node shall only send codecs that have been offered at the IM CN subsystem side (as received in signal 1 in figure E.2.3.1.1.1) within this message.

-
The Interworking Node will receive an H.245 Terminal Capability Set message describing the supported Codecs at the peer's side (signal 8 in figure E.2.3.1.1.1).

-
The codecs contained both in the sent and received terminal capability set messages may be selected at the CS side. The final decision of the selected codecs at the CS side is taken when the H.245 open logical Channels message (signal 9 in figure E.2.3.1.1.1) is sent or received. The direction of this message is determined by the H.245 master-slave determination procedure.

If the Interworking Node does not transcode, it should indicate the codecs selected within the H.245 negotiation (signal 11 in figure E.2.3.1.1.1) or within the MONA procedures and enable any media that have previously been put on hold at the IMS side after the completion of the H.245 negotiation or MONA procedures.
The interworking node should include in step 11 of figure E.2.3.1.1.1 the SDP "a" attribute "3gpp_MaxRecvSDUSize" indicating the maximum SDU size of the application data that can be transmitted to the receiver without segmentation, as specified in clause 12.2.4.6 of TS 26.114 [104].

E.2.3.2
Preconditions not used at IMS side

E.2.3.2.1
Interactions between H.245 or MONA and SIP/SDP

Figure E.2.3.2.1.1 shows examples of interactions between H.245 or MONA procedures and SIP/SDP for IM CN subsystem originated session. Most SIP and ISUP or BICC messages are intentionally omitted, since the SDP may be embedded in various SIP messages and since the in‑band H.245 Messages are not tightly coupled with out-of-band ISUP or BICC messages.

Figure E.2.3.2.1.1 assumes that the IMS peer does not use the SIP precondition extension.


[image: image96.wmf]Interworking Node

 (

MGCF

+ 

IM MGW

)

1

. 

SIP

: 

INVITE 

[

SDP offer

:

,

m 

= 

H

.

263

, 

MP

4

V

-

ES

, 

H

.

261

;

 m

= 

AMR

]

9

. 

SIP

: 

200

OK 

(

INVITE

)

[

SDP answer

:

m 

= 

MP

4

V

-

ES

;

 a

= 

3

gpp

_

MaxRecvSDUSize 

600

;

 m

= 

AMR

]

CALL IN ACTIVE STATE

2

.  

IA

M

7

. 

H

.

245

: 

Terminal Capability Set

[

H

.

261

, 

MP

4

V

-

ES

;

AMR

, 

G

.

711

]

3

. 

Bearer Establishment

6

. 

H

.

245

: 

Terminal Capability Set

[

MP

4

V

-

ES

;

 

AMR

]

8

. 

H

.

245

: 

Open Logical Channels

[

MP

4

V

-

ES

, 

AMR

]

CS Network

IMS

5

. 

H

.

223 

Bearer Setup

 including possible

MONA 

Channel establishment method negotiation

4

. 

ANM

Alternative to 

6

-

8

: 

MONA messages


Figure E.2.3.2.1.1 Interactions between H.245 and SIP/SDP for IM CN subsystem originated session
IMS peer does not use SIP preconditions.

Upon receipt of a SIP INVITE request containing speech and video Codecs (signal 1 in figure E.2.3.2.1.1) the Interworking Node (consisting of MGCF and IM-MGW) starts the call set-up for multimedia call at the CS side by sending an IAM requesting an UDI bearer (signal 2 in figure E.2.3.2.1.1).

If no unmet local SDP preconditions are contained in signal 1, the Interworking node should defer sending an SDP answer until the H.245 negotiation or MONA procedures is/are completed.
The Interworking Node shall engage in an H.223 bearer setup (step 5 in figure E.2.3.2.1.1). If the interworking Node supports MONA (Media Oriented Negotiation Acceleration), it shall first attempt a MONA Channel establishment method negotiation according to Annex K of ITU-T Recommendation H.324 [81]. If the interworking node does not support MONA, it shall use the multiplexing level negotiation procedures of Annex C of H.324 [81]. If the Interworking Node supports MONA, but the remote peer does not do so, a fallback to the multiplexing level negotiation procedures of Annex C of H.324 [81] will occur.

If both the Interworking Node and the remote CS terminal support MONA procedures, the MONA procedures as per ITU-T Recommenation H.324 [81] Annex K may be used to replace the H.245 negotiation (signals 6 – 8) as shown in figure E.2.3.2.1.1.

If MONA procedures are not used, the following applies:
-
After the completion of the H.223 bearer setup at the CS side, the Interworking Node shall send a Terminal Capability Set message describing its own capabilities (signal 6 in figure E.2.3.2.1.1). Unless the Interworking Node supports transcoding, the Interworking Node shall only send codecs that have been offered at the IM CN subsystem side (as received in signal 1 in figure E.2.3.2.1.1) within this message.

-
The Interworking Node will receive an H.245 Terminal Capability Set message describing the supported Codecs at the peer's side (signal 7 in figure E.2.3.2.1.1).

-
The codecs contained both in the sent and received terminal capability set message may be selected at the CS side. The final decision of the selected codecs at the CS side is taken when the H.245 open logical Channels message (signal 8 in figure E.2.3.2.1.1) is sent or received. The direction of this message is determined by the H.245 master-slave determination procedure.

If the Interworking Node does not transcode, it shall send an SDP answer (signal 9 in figure E.2.3.2.1.1) indicating the codecs selected within the H.245 negotiation or within the MONA procedures after the completion of the H.245 negotiation or MONA procedures. The interworking node should include a b:AS bandwidth modifier with a bandwidth suitable for the selected codec(s), but not higher than 64kbit/s plus RTP/UDP/IP overhead, in the SDP answer to request that the peer does not send media with a higher bandwidth.
The interworking node should include in Step 9 of figure E.2.3.2.1.1 the SDP "a" attribute "3gpp_MaxRecvSDUSize" indicating the maximum SDU size of the application data that can be transmitted to the receiver without segmentation, as specified in clause 12.2.4.6 of TS 26.114 [104].

E.2.3.3
Fallback to speech at session establishment

If SCUDIF Fallback (see TS 23.172 [121]) occurs on the CS side, the APM message contains a speech codec as "Selected Codec". The MGCF shall then disable the video "m-line" in the first SDP answer, if not yet sent, and complete the call-setup in the same way as for a normal speech call. If the SDP answer was already sent (precondition used), the MGCF shall update the SIP session to speech by sending a SIP UPDATE message with a relevant SDP.

If in a non-SCUDIF case (ISUP or BICC without SCUDIF) the called CS terminal or network rejects the video call setup by sending a REL message to the MGCF, the MGCF releases the CS video call being established, re-establishes the CS call in a speech only mode sending a new IAM with a speech BCIE to the CS network and updates the IM CN leg codecs to a speech only codec. Then the call/session continues as in a speech only case. If the interworking node does not support the fallback, it shall release the session.

E.2.4
CS network originated session

E.2.4.1
Interactions between SIP/SDP and H.245 or MONA
E.2.4.1.1
Normal Call setup

Figure E.2.4.1.1 shows examples of interactions between H.245 or MONA and SIP/SDP for the CS network originated session. Most SIP and ISUP or BICC messages are intentionally omitted, since the SDP may be embedded in various SIP messages and since the in‑band H.245 Messages are not tightly coupled with out-of-band ISUP or BICC messages.


[image: image97.wmf]Interworking Node

(

MGCF

+ 

IM MGW

)

2

. 

SIP

: 

INVITE 

[

SDP offer

:

,

m 

= 

H

.

263

, 

MP

4

V

-

ES

;

 m

= 

AMR

1

.  

IA

M

3

. 

SIP

: 

[

SDP answer

:

m 

= 

H

.

263

, 

MP

4

V

-

ES

;

 m

= 

AMR

]

8

. 

H

.

245

: 

Terminal Capability Set

[

H

.

263

, 

H

.

261

AMR

, 

G

.

711

]

4

. 

Bearer Establishment

CALL IN ACTIVE STATE

9

. 

SIP

:

[

SDP offer

m 

=: 

H

.

263

, 

H

.

261

;

 m

= 

AMR

]

10

. 

SIP

 

:

[

SDP answer

m 

= 

H

.

263

, 

H

.

261

;

 m

= 

AMR

]

11

. 

H

.

245

: 

Terminal Capability Set

[

H

.

263

, 

H

.

261

AMR

]

12

. 

H

.

245

: 

Open Logical Channels

[

H

.

263

, 

AMR

]

13

. 

SIP

:

[

SDP offer

m 

=: 

H

.

263

;

 a

= 

3

gpp

_

MaxRecvSDUSize 

600

;

 m

= 

AMR

]

1

4

. 

SIP

 

:

[

SDP answer

m 

= 

H

.

263

;

 m

= 

AMR

]

CS Network

IMS

5

. 

SIP

: 

200

OK 

(

INVITE

)

7

. 

H

.

223 

Bearer Setup

 

in

cluding possible

MONA 

Channel establishment method negotiation

6

. 

ANM

Alternative to 

8

-

12

: 

MONA messages


NOTE:
Messages 3 and 5 may be combined in some scenarios.

Figure E.2.4.1.1: Interactions between H.245 and SIP/SDP for CS network originated session

Upon receipt of an IAM request for a multimedia Call (signal 1 in figure E.2.4.1.1) the Interworking Node (consisting of MGCF and IM-MGW) starts the call set-up for multimedia call at the IM CN subsystem side by sending an INVITE request (signal 2 in figure E.2.4.1.1). For the INVITE request, the Interworking Node selects codecs supported by the IM-MGW and likely to be supported within the CS Network. The Interworking Node should select the H.263 codec and may select other codec in addition. The interworking node should add a b:AS bandwidth modifier with a bandwidth suitable for the selected codec(s), but not higher than 64kbit/s plus RTP/UDP/IP overhead, in the SDP offer to request that the peer does not send media with a higher bandwidth.
NOTE:
The SDP coding to express that either a combined voice and video call, or a voice call, or a Clearmode codec, or some other data call is desired is not defined in the present release.

The Interworking Node shall engage in an H.223 bearer setup (step 7 in figure E.2.4.1.1). If the interworking Node supports MONA (Media Oriented Negotiation Acceleration), it shall first attempt a MONA Channel establishment method negotiation according to Annex K of ITU-T Recommendation H.324 [81]. If the interworking node does not support MONA, it shall use the multiplexing level negotiation procedures of Annex C of H.324 [81]. If the Interworking Node supports MONA, but the remote peer does not do so, a fallback to the multiplexing level negotiation procedures of Annex C of H.324 [81] will occur.

If both the Interworking Node and the remote CS terminal support MONA procedures, the MONA procedures as per ITU-T Recommendation H.324 [81] Annex K may be used to replace the H.245 negotiation (signals 8, 11 and 12) as shown in figure E.2.4.1.1. Furthermore, the SIP codec renegotiation in signals 9 and 10 is then also not applicable.

If MONA procedures are not used, the following applies:

-
After the completion of the H.223 bearer setup at the CS side the Interworking Node will receive a H.245 Terminal Capability Set message describing the supported Codecs at the peer's side (signal 8 in figure E.2.4.1.1).

-
Due to information received in a Terminal Capability Set message (signal 8 in figure E.2.4.1.1), the Interworking node may send an SDP offer at the IMS side (signal 9 in figure E.2.4.1.1), to offer additional codecs supported at the CS side but not contained in the first SDP offer (signal 2 in figure E.2.4.1.1), or to restrict the selected codecs at the IMS side to codecs which are available at the CS side.

NOTE:
It is not clear if the addition of codecs not included in previous SDP exchange has any impacts on IMS procedures, e.g. resource reservation related procedures.

-
The Interworking Node shall send a Terminal Capability Set message describing its own capabilities (signal 11 in figure E.2.4.1.1). Unless the Interworking Node supports transcoding, the Interworking node shall only send codecs that are also negotiated at the IM CN subsystem side (as received in signal 3 in figure E.2.4.1.1) within this message. The Interworking Node may defer sending the Terminal Capability Set message for some time to attempt to receive the peer's Terminal Capability set message and perform a possible IMS-side codec re-negotiation. However, to avoid blocking situations, the Interworking Node shall not defer sending the Terminal Capability Set message for an excessive period of time.

-
The codecs contained both in the sent and received Terminal Capability Set message may be selected at the CS side. The final decision of the selected codecs at the CS side is taken when the H.245 open logical Channels message (signal 12 in figure E.2.4.1.1) is sent or received. The direction of this message is determined by the H.245 master-slave determination procedure.

If the Interworking Node does not transcode, it should indicate the codecs selected within the H.245 negotiation or within MONA procedures after the completion of the H.245 negotiation (signal 13 in figure E.2.4.1.1) or MONA procedures.
The interworking node should include in Step 11 of figure E.2.4.1.1 the SDP "a" attribute "3gpp_MaxRecvSDUSize" indicating the maximum SDU size of the application data that can be transmitted to the receiver without segmentation, as specified in clause 12.2.4.6 of TS 26.114 [104].
E.2.4.1.2
Call setup if multimedia call can not be recognized in an unambiguous manner

If the Interworking Node is not able to determine from the information within the IAM request whether a multimedia call or some other type of data call is requested (for example, if only TMR=UDI but no BC IE is contained in the IAM), the Interworking Node may also include appropriate codecs for other possible types of data call it supports in the INVITE request. If video and audio codecs are contained in the first SDP answer (signal 3), the Interworking Node should continue to attempt to set up a multimedia call as described in clause E.2.4.1.1. Otherwise, calls are set up as described in clause 7.2.3.2.

E.2.4.1.3
Fallback to speech during call setup

The called party can reject the video component in the SDP offer (signal 2 in figure E.2.4.1.1) by returning an SDP answer where the video related m-line is disabled (in signal 3 in figure E.2.4.1.1).

If the MGCF receives an SDP answer during call setup where the video related m-line is disabled, it shall react as follows:

-
If the MGCF has received a SCUDIF call setup from the CS side (MuMe and speech codecs in IAM message, see TS 23.172 [121]), the MGCF shall apply a SCUDIF fallback to speech on the CS side. The MGCF shall use a speech codec as Selected Codec and exclude the MuMe codec from the available codec list.

-
If the MGCF has received a multimedia call setup without SCUDIF from the CS side, and the CS network supports CS fallback, the MGCF should terminate the call.

NOTE 1:
A call termination can trigger the caller's device to set up a new speech call.

NOTE 2:
Other procedures to maintain the CS call, e.g. disabling the video component on the CS side or providing a suitable video announcement and discarding received video media from the CS side, are FFS.

E.2.4.2
CS originated - IM CN transit - CS terminated

Figure E.2.4.2.1 describes ISUP and SIP/SDP interactions in a CS originated - IM CN transit - CS terminated case with a clear channel through the IM CN. An interworking node A receives an IAM message with a UDI H.223 & H.245 video call request (message 1). If the interworking node A supports both CS/IMS video interworking and a clearmode codec / clear channel, it may send both audio and video codecs and a clearmode codec and a UDI & H.223 & H.245 video indication in the INVITE message towards the IMS (message 2). The message is received by an interworking node B. The interworking node B sends an IAM message with a UDI & H.223 & H.245 video call request to the terminating CS network (message 3). If the interworking node B supports a clearmode codec / clear channel, it may send a SIP response with a clearmode codec towards the calling side to indicate that a clear channel can be established between the IMS interworking nodes (message 5). After the called party answers, the interworking node B sends a SIP 200 OK (Invite) with the clearmode codec to the calling node to indicate that a clear channel can be established (message 11). After the called party has answered the call, either MONA procedures are performed and the H.223 bearer is established or the H.223 bearer is established and the H.245 signalling is performed (step 14 in figure E.2.4.2.1).

If the interworking node A does not support CS-IMS video interworking, but supports a clearmode codec / clear channel, it sends the INVITE message with a clearmode codec and UDI & H.223 & H.245 indication, but without a video codec, to allow the establishment of a CS video call through a clear channel. The interworking node A may also send an audio codec (alone or with a clearmode codec) to allow a fallback to speech. The interworking node B either accepts the clearmode and sends the corresponding IAM message with a UDI & H.223 & H.245 request (message 3) and SIP response with a clearmode codec (message 5 or 11), or accepts the speech mode and sends the corresponding IAM message with a speech request (message 3) and SIP response with a speech codec (messages 5, 11), or rejects the INVITE message if the requested codec(s) cannot be supported.
The format of the indication of UDI & H.223 & H.245 from interworking node A to interworking node B is defined in Annex F "PSTN XML Scheme". UDI & H.223 & H.245 indication is conveyed in PSTN XML body within "BearerCapabilty" ("InformationTransferCapability" and "UserInfoLayerProtocol") and "LowLayerCompatibility" elements. Elements are defined in ETSI EN 300 403‑1 (V1.3.2) [96] section 4.5.5 and in ISUP transported in the ATP is in Q.761-Q.764 [4] section 3.5.

[image: image98.wmf]Interworking no

d

e

 

A

 

(

MGCF

+ 

IM MGW

)

1

. 

IAM 

[

UDI

, 

H

.

223 

& 

H

.

245

]

1

1

. 

SIP

:

200 

OK

(

INVITE

)

 

8

. 

SIP

 

:

180 

Ringing

1

3

. 

SIP

: 

ACK

3

. 

IAM 

[

UDI

, 

H

.

223 

& 

H

.

245

]

7

. 

ACM 

10

. 

ANM 

CS Network

IMS

Interworking no

d

e

 

B

 

(

MGCF

+ 

IM MGW

)

CS Network

2

. 

SIP

: 

INVITE

  

[

m 

= 

H

.

263

, 

 

m

= 

AMR

;

m

=

clearmode

]

 

[

PSTN XML body

] 

4

. 

TDM circuit reservation

5

. 

SIP

 

: 

Response 

[

m

=

audio

, 

a

=

clearmode

]

6

. 

TDM circuit reservation

9

. 

ACM 

12

. 

ANM 

14

. 

Clear channel

. 

Either MONA messages 

+ 

H

.

223 

bearer setup 

or H

.

223 

bearer setup and H

.

245 

signaling between CS terminals

CALL IN ACTIVE STATE


Figure E.2.4.2.1: ISUP and SIP/SDP interactions in a CS originated - IM CN transit
- CS terminated case with a clear channel through the IM CN

E.2.5
Service change

E.2.5.2.1
SCUDIF

E.2.5.2.1.0
General

SCUDIF is standardised in 3GPPP TS 23.172 [121].

E.2.5.2.1.1
IM CN subsystem originated change

E.2.5.2.1.1.1
Change from multimedia to speech

Figure E.2.5.2.1.1.1.1 shows an IM CN subsystem originated modification from multimedia to speech during an ongoing session when the CS leg supports BICC. The interworking node receives an INVITE message that indicates the dropping of the video media from the session, message 1. The interworking node can only accept the dropping of the media component and sends a corresponding codec modification request to the BICC network, message 2, and acknowledges the INVITE with a 200 OK message. The BICC network indicates a successful codec modification, message 5.


[image: image99.wmf]Interworking Node 

(

MGCF

+ 

IM MGW

)

1

. 

SIP

: 

INVITE 

[

m

= 

AMR

]

4

. 

SIP

: 

ACK

3

. 

SIP

 

:

200 

OK 

(

INVITE

) [

m

= 

AMR

]

Audio 

+ 

Video

2

. 

Modify Codec 

(

Selected Codec

=

AMR

)

5

. 

Successful Codec Modification

CS Network

IMS

Speech


Figure E.2.5.2.1.1.1.1: IM CN subsystem originated modification
from multimedia to speech when the CS leg supports BICC

Editor's note:
Handling of a case, where a codec is received in BICC negotiation but not included in the available codec list negotiated previously, is ffs.

E.2.5.2.1.1.2
Change from speech to multimedia

Figure E.2.5.2.1.1.2.1 shows an IM CN subsystem originated modification from speech to multimedia during an ongoing session when the CS leg supports BICC. The interworking node receives an INVITE message that offers the adding of a video media to the ongoing speech session, message 1. The interworking node accepts the offer and sends a corresponding codec modification request to the BICC network, message 2. The BICC network indicates a successful codec modification, message 3. The interworking node acknowledges the INVITE with a 200 OK message after the MONA or H.245 in‑band negotiation in step 4 is completed.

If the codec modification is not successful in the BICC network, the interworking node responds to the INVITE message with the speech codec in the 200 OK message to retain the speech only session.


[image: image100.wmf]Interworking Node 

(

MGCF

+ 

IM MGW

)

1

. 

SIP

: 

INVITE 

[

m

= 

AMR

; 

m

=

H

.

263

]

6

. 

SIP

: 

ACK

5

. 

SIP

 

:

200 

OK 

(

INVITE

) [

m

= 

AMR

; 

m

=

H

.

263

]

Speech

2

. 

Modify Codec 

(

Selected Codec

=

MuMe

)

3

. 

Successful Codec Modification

CS Network

IMS

Audio 

+ 

Video

4

. 

MONA or H

.

245 

inband negotiation


Figure E.2.5.2.1.1.2.1: IM CN subsystem originated modification
from speech to multimedia when the CS leg supports BICC

E.2.5.2.1.2
CS network originated change

E.2.5.2.1.2.1
Change from multimedia to speech

Figure E.2.5.2.1.2.1.1 shows a CS network originated modification from multimedia to speech during an ongoing session when the CS leg supports BICC. The interworking node receives a Modify Codec message that indicates the dropping of the video media from the session, message 1. The interworking node accepts the dropping of the video component and sends a corresponding INVITE message to the IM CN subsystem, message 2, and acknowledges the codec modification request to the BICC network, message 3. The IM CN subsystem acknowledges the INVITE dropping the video media with a 200 OK, message 4.


[image: image101.wmf]Interworking Node 

(

MGCF

+ 

IM MGW

)

5

. 

SIP

:

 

A

CK

4

. 

SIP

: 

200 

OK 

(

INVITE

) [

m

= 

AMR

]

2

. 

SIP

: 

INVITE 

[

m

= 

AMR

]

Audio 

+ 

Video

1

. 

Modify Codec 

(

Selected Codec

=

AMR

)

CS Network

IMS

Speech

3

. 

Successful Codec Modification


Figure E.2.5.2.1.2.1.1: CS network originated modification
from multimedia to speech when the CS leg supports BICC

E.2.5.2.1.2.2
Change from speech to multimedia

Figure E.2.5.2.1.2.2.1 shows a CS network originated modification from speech to multimedia during an ongoing session when the CS leg supports BICC. The interworking node receives a Modify Codec message that indicates the adding of a video media to the ongoing speech session, message 1. The interworking node accepts the offer and sends a corresponding INVITE message to the IM CN subsystem, message 2. The IM CN subsystem acknowledges the INVITE adding the video media with a 200 OK, message 3, and acknowledges the codec modification request to the BICC network, message 4. The interworking node may have to update the codecs, messages 7 and 8, after the MONA or H.245 in‑band negotiation in step 6.

If the IM CN subsystem does not accept the addition of the video media to the session, the interworking node rejects the modify codec request to retain the speech only session.


[image: image102.wmf]Interworking Node 

(

MGCF

+ 

IM MGW

)

5

. 

SIP

:

 

A

CK

3

. 

SIP

: 

200 

OK 

(

INVITE

) [

m

= 

AMR

; 

m

=

H

.

263

]

2

. 

SIP

: 

INVITE 

[

m

= 

AMR

; 

m

=

H

.

263

]

Speech

1

. 

Modify Codec 

(

Selected Codec

=

MuMe

)

CS Network

IMS

Audio 

+ 

Video

4

. 

Successful Codec Modification

6

. 

MONA or H

.

245 

inband negotiation

7

. 

SIP

: 

INVITE 

[

m

= 

AMR

; 

m

=

H

.

263

]

8

. 

SIP

: 

200 

OK 

(

INVITE

) [

m

= 

AMR

; 

m

=

H

.

263

]


Figure E.2.5.2.1.2.2.1: CS network originated modification
from speech to multimedia when the CS leg supports BICC

E.2.5.2.2
Non-SCUDIF case (ISUP or BICC without SCUDIF)

E.2.5.2.2.1
Change from multimedia to audio

Figure E.2.5.2.2.1.1 shows an IM CN subsystem originated modification from multimedia to audio during an ongoing session when the CS leg supports ISUP or BICC without SCUDIF. The interworking node receives an INVITE message that indicates the dropping of the video media from the session, message 1. The interworking node can only accept the dropping of the media component and acknowledges the INVITE with a 200 OK, message 2. There are three alternative ways to handle the issue:

-
The video component stays on in the CS leg. The interworking node may use the video component to send an announcement to the CS terminal to inform the user about the change of the end-to-end connection to audio only. Refer to figure E.2.5.2.2.1.1.

-
The interworking node initiates an H.245 in‑band negotiation to close the video channel.

-
The interworking node terminates the session.


[image: image103.wmf]Interworking Node 

(

MGCF

+ 

IM MGW

)

1

. 

SIP

: 

INVITE 

[

m

= 

AMR

]

3

. 

SIP

: 

ACK

2

. 

SIP

 

:

200 

OK 

(

INVITE

) [

m

= 

AMR

]

Audio 

+ 

Video

CS Network

IMS

Audio

Video

Video announcement


Figure E.2.5.2.2.1.1: IM CN subsystem originated modification
from multimedia to speech when the CS leg supports ISUP or BICC without SCUDIF

E.2.5.2.2.2
Change from speech to multimedia

Figure E.2.5.2.2.2.1 shows an IM CN subsystem originated attempt to change from speech to multimedia during an ongoing session when the CS leg supports ISUP or BICC without SCUDIF. The interworking node receives an INVITE message that offers the adding of a video media to the ongoing speech session, message 1. The interworking node turns down the offer and responds to the INVITE message with the speech codec in the 200 OK message to retain the speech only session, message 2.


[image: image104.wmf]Interworking Node 

(

MGCF

+ 

IM MGW

)

1

. 

SIP

: 

INVITE 

[

m

= 

AMR

; 

m

=

H

.

263

]

3

. 

SIP

: 

ACK

2

. 

SIP

 

:

200 

OK 

(

INVITE

) [

m

= 

AMR

]

Speech

CS Network

IMS

Speech


Figure E.2.5.2.2.2.1: IM CN subsystem originated modification
from speech to multimedia when the CS leg supports ISUP or BICC without SCUDIF

E.2.6
Call release

E.2.6.1
Call release initiated from the IM CN subsystem side

When the MGCF has received a BYE message (signal 1 in figure E.2.6.1.1) from the IM CN subsystem side, the MGCF may end the H.245 session between the IM-MGW and the CS network side (signal 3 in figure E.2.6.1.1) firstly.

NOTE:
A call release using only ISUP/BICC signalling at the CS side proceeds faster. H.324 terminals can handle situations where they do not receive H.245 call release signalling (see clause 7.5.2 of ITU-T Recommendation H.324 [81]), and this scenario also occurs e.g. at loss of coverage or when a node transporting H.223 transparently releases the call.

The procedure of ending the H.245 session is defined in the clause E.4. After receiving the BYE message, the MGCF shall also send a 200 OK [BYE] message (signal 2 in figure E.2.6.1.1) towards the IM CN subsystem.

After ending the H.245 session, the MGCF shall send a REL message (signal 4 in figure E.2.6.1.1) to the succeeding node. If the IM CN subsystem interworks with ISUP based CS network, the interworking node shall release the resources for the IMS side and the CS network side (signal 6 in figure E.2.6.1.1) after sending the REL message. If the IM CN subsystem interworks with BICC based CS network, the interworking node shall release the resources for the IMS side and the CS network side (signal 6 in figure E.2.6.1.1) upon receiving the RLC message (signal 5 in figure E.2.6.1.1) from the CS network side. The procedures of releasing the resources for the IMS side and the CS network side are specified in the present TS in clause 7.

Figure E.2.6.1.1 shows the message sequence chart for the multimedia call release initiated from the IM CN subsystem side.


[image: image105.emf]Interworking node

(MGCF+ IM-MGW)

4. 

BICC or ISUP

: REL

1. 

SIP

: BYE

CS   Network IMS

3.

 H.245 Signalling to end the H.245 session

between the IM-MGW and the CS network side

5. 

BICC

: RLC

2. 

SIP

: 200 OK [BYE]

6.

 Release the resources for the IMS side

and the CS network side

7. 

ISUP

: RLC


NOTE:
Signal 7 is omitted when IM CN subsystem interworks with BICC based CS network and Signal 5 is omitted when IM CN subsystem interworks with ISUP based CS network.


Figure E.2.6.1.1: Call release initiated from the IM CN subsystem side

E.2.6.2
Call release initiated from the CS network side

If the CS network side initiates the call release, it possibly ends the H.245 session with explicit signalling (signal 1 in figure E.2.6.2.1). The CS network side sends a REL message (signal 2 in figure E.2.6.2.1) towards the IM CN subsystem. The procedure of ending the H.245 session is defined in the clause E.4.

When the MGCF receives a REL message (signal 2 in figure E.2.6.2.1) from the preceding node, the MGCF sends a BYE message (signal 3 in figure E.2.6.2.1) to the IM CN subsystem. After receiving the REL message, the interworking node also releases the resources for the IMS side and the CS network side (signal 4 in figure E.2.6.2.1). The procedure of the releasing the resources for the IMS side and the CS network side are specified in the present TS in clause 7. After completion of resource release, the MGCF sends a RLC message (signal 5 in figure E.2.6.2.1) towards the preceding node. Figure E.2.6.2.1 shows the message sequence chart for the multimedia call release initiated from the CS network side.


[image: image106.emf] 

Interworking node  

(MGCF+ IM - MGW)  

2.   BICC  or ISUP  :  REL  

3.   SIP  : BYE  

CS   Network   IMS  

1.   

H.245 Signalling to end the H.245 session  

(optional)  

5 .  BICC or ISUP : RLC  

6.  SIP   : 200 OK [BYE]    

4 .   

Release the resources for the IMS side  

and the CS network side  


Figure E.2.6.2.1: Call release initiated from the CS network side

E.2.6.3
Call release initiated from the interworking node

The interworking node may end the H.245 session between the IM-MGW and the CS network side (signal 1 in figure E.2.6.3.1) firstly. The procedure of ending the H.245 session is defined in the clause E.4.

NOTE:
A Call Release using only SIP and ISUP/BICC signalling may proceed faster. H.324 terminals can handle situations where they do not receive H.245 call release signalling (see clause 7.5.2 of ITU-T Recommendation H.324 [81]), and this scenario also occurs e.g. at loss of coverage or when a node transporting H.223 transparently releases the call.

To release the call, the MGCF shall send a REL message (signal 2 in figure E.2.6.3.1) to the succeeding node on the CS network side. The MGCF shall also send a BYE message (signal 3 in figure E.2.6.3.1) to the IM CN subsystem side.

If the IM CN subsystem interworks with ISUP based CS network, the interworking node shall release the resources for the IMS side and the CS network side (signal 5 in figure E.2.6.3.1) after sending the REL message. If the IM CN subsystem interworks with BICC based CS network, the interworking node shall release the resources for the IMS side and the CS network side upon receiving the RLC message (signal 4 in figure E.2.6.3.1) from the CS network side. The procedures of releasing the resources for the IMS side and the CS network side are specified in the present TS in clause 7. Figure E.2.6.3.1 shows the message sequence chart for the multimedia call release initiated from the interworking node.

[image: image107.emf]Interworking node

(MGCF+ IM-MGW)

2. 

BICC or ISUP

: REL

3. 

SIP

: BYE

CS   Network IMS

7. 

SIP

: 200 OK [BYE]

1. 

H.245 Signalling to end the H.245 session

between the IM-MGW and the CS network side

4. 

BICC

: RLC

5. 

Release the resources for the IMS side

and the CS network side

6. 

ISUP

: RLC


NOTE:
Signal 6 is omitted when IM CN subsystem interworks with BICC based CS network and Signal 4 is omitted when IM CN subsystem interworks with ISUP based CS network.

Figure E.2.6.3.1: Call release initiated from the interworking node

E.3
User plane interworking

E.3.1
Functionalities required in the IM-MGW for multimedia calls support

To enable a multimedia Interworking, the IM-MGW needs to support the reframing of the H.263 video codec and the AMR audio codec between CS transport and PS transport as a minimum. The IM-MGW may also support the reframing of other codecs and the transcoding of audio and/or video codecs.

At the CS side, the IM-MGW needs to terminate the H.223 protocol and multiplex / de-multiplex audio, video and H.245 signalling. How H.245 related information (e.g. H.245 messages or extracted information) is communicated between the MGCF and the IM-MGW is described in clause E.4.

E.4
MGCF and IM-MGW interactions

E.4.1
Introduction

This clause describes requirements for extensions to the Mn interface protocol in TS 29.332 [15] needed to support the Interworking of multimedia calls. ITU-T Recommendation H.248.1 [2] is used at the Mn interface.

The H.245 signalling shall be handled by the MGCF. Upon reception of the H.245 messages from the CS side at the IM-MGW, the IM-MGW shall forward those H.245 messages as binary data within H.248 messages over the Mn interface towards the MGCF. Upon reception of encapsulated binary H.245 messages within H.248 messages, the IM-MGW shall forward those H.245 messages towards the CS side.

NOTE:
Procedures to support MONA (see Annex K of ITU-T Recommendation H.324 [81]) over the Mn interface are not defined in the present Release. Furthermore, the signalling flows in clause E.2 may not show MONA related signalling in sufficient detail for MONA related Mn interface interactions.

E.4.2
Mn signalling interactions

E.4.2.1
Introduction

The following clauses describe the Mn interface procedures triggered by H.245 signalling received in the IM-MGW and SIP and RTCP and BICC or ISUP signalling received in the MGCF.

All message sequence charts in these clauses are examples.

E.4.2.2
H.248 Context Model

The H.248 context model depicted in figure E.4.2.2.1 shall be applied for Multimedia Interworking.


[image: image108.emf] 

Context C1  

Termination T2  

Stream 1  

Stream  3  

Stream  2  

Termination T 3  

Termination T1  

CS - Domain   IMS  

Termination:   T1 CS - Domain (CS - Bearer (BS30) f or  H.245 control,  Speech , Video)   T2 Multiplexing (H.245 control,  Speech , V ideo)   T3 Video ( own  RTP - stream )   + S peech (own RTP - stream)     Stream:   Stream1  (between T1 a nd T2)   data  (H.245 control,  speech , Video)   Stream 2  ( between T2 a nd T3)  Video   Stream 3   ( between  T2 a nd T 3 )  Sp eech  


Figure E.4.2.2.1: H.248 Context Model for Multimedia Interworking

E.4.2.3
Transport of H.245 messages between the MGCF and IM-MGW

E.4.2.3.1
General

H.245 messages shall be transported between the IM-MGW and MGCF over the Mn interface using H.248 Events (from the IM‑MGW towards the MGCF) and H.248 Signals (from the MGCF towards the IM-MGW). The Events/Signals shall contain the following information:

-
H.245 message (binary).

E.4.2.3.2
Transport from MGCF to IM-MGW


[image: image109.emf]IM MGW

.

MGCF

 1. H.248: MOD.req

[C=C1,T=T2,

signal=H245msgout

{h245mc=XXXX}

]

2. H.248: MOD.resp

3. send H.245 message XXXX in H.223 stream

Signal 

H245 

Message


Figure E.4.2.3.2.1: Mn signalling interactions for sending H.245 message

In Signal 1, the MGCF requests the IM-MGW to send an H.245 message to the CS side. To request the IM-MGW to send a H.245 message to the CS side, the MGCF shall sent an H.248 signal to the IM-MGW with the complete H.245 message content.

NOTE:
In order for this command to succeed, Termination T1 towards the CS side needs to be configured. If a sending of an H.245 message and a removal of termination T1 is desired, the MGCF needs to apply signal 1 before removing T1.

Upon reception of this signal, the IM-MGW shall send the encapsulated H.245 message within the H.248 signal, through the H.245 control channel to the CS side (signal 3).

E.4.2.3.3
Transport from IM-MGW to MGCF


[image: image110.emf]IM MGW

.

MGCF

1. 

H.248: ADD.req

[C=C1,T=?... ,Event=H245msgin]

2. H.248: ADD.resp

3. receive H.245 message XXXX in H.223

4. 

H.248

[C=C1,T=T2

,

Event=H245msgin

{h245mc=XXXX}]

5. H.248

: Notify.resp

Notify 

H245 

Message

: Notify.req

Add 

Multiplex 

Termination


Figure E.4.2.3.3.1: Mn signalling interactions for receiving H.245 message

In signal 1, the MGCF requests the IM-MGW to detect received H.245 message from the CS side and forward them to the MGCF. To request the IM-MGW to detect and forward a H.245 message to the CS side, the MGCF shall send a suitable H.248 event to the IM-MGW. The event may be indicated through an H.248 ADD command.

In signal 3, the IM-MGW receives an H.245 message from the CS side. Upon reception of an H.245 message from the CS side, the IM-MGW shall de-multiplex the H.245 message from the H.223 stream and forward the H.245 message to the MGCF within an H.248 Notify command (signal 4).

E.4.2.4
Call establishment procedure

The following information shall be provided from the MGCF towards the IM-MGW:

-
Properties to start H.223 Negotiation.

-
Request for events for Notification of H.245 message received by the IM-MGW.

-
Signals to provide H.245 messages that the IM-MGW shall send towards the CS side.

-
Properties to provide incoming and outgoing H.223 multiplex table.

-
Properties to provide H.223 logical channel parameters.

-
Property to provide remote H.223 capabilities.

The Highest Multiplexing Level shall be predefined in the IM-MGW.

[image: image111.wmf]IM MGW

6

. 

H

.

248

: 

ADD

.

req

[

C

=

C

1

, 

T

=

?,  

Mux

=

H

223

,

T

1

,

LocalControl

{ 

muxlv

=

2

} 

Event

=

H

245

msgin 

]

9

. 

H

.

245

: 

Terminal Capability Set

,

[

AMR

,

MP

4

V

-

ES

,

H

263

]

Master

-

Slave Determination

5

. 

Bearer Establishment

13

. 

H

.

245

: 

Open Logical Channel

[

H

245 

logical Channel 

= 

LC

1

]

MGCF

8

. 

H

.

223

: 

Multiplexing Level Negotiation

[

level 

=

2

]

10

. 

H

.

245

: 

Terminal Capability Set Ack

,

Master

-

Slave Determination Ack

11

. 

H

.

245

: 

Terminal Capability Set

,

[

H

.

261

,

H

.

263

,

AMR

]

Master

-

Slave Determination

12

. 

H

.

245

: 

Terminal Capability Set Ack

, 

Master

-

Slave Determination Ack

14

. 

H

.

245

: 

Open Logical Channel Ack

21

. 

H

.

248

: 

MOD

.

req

[

C

=

C

1

, 

T

=

T

2

, 

Stream

=

1

{

LocalCont

{

h

223

capr

,

muxtbl

_

in

,

muxtbl

_

out

}}

stream

=

2

{

LCN

=

LC

1

, 

Codec

=

H

263

}

stream

=

3

{

LCN

=

LC

2

, 

Codec

=

AMR

}

]

22

. 

H

.

248

: 

MOD

.

resp

1

. 

H

.

248

: 

ADD

.

req

[

C

=

?, 

T

=

?, 

stream

=

1

]

2

. 

H

.

248

: 

ADD

.

resp

[

C

=

C

1

, 

T

=

T

1

]

7

. 

H

.

248

: 

ADD

.

resp

[

C

=

C

1

, 

T

=

T

2

]

23

. 

H

.

248

: 

MOD

.

req

[

C

=

C

1

, 

T

=

3

, 

stream

=

2

{

Codec

=

H

263

}, 

stream

=

3

{

Codec

=

AMR

]

24

. 

H

.

248

: 

MOD

.

resp

[

C

=

C

1

, 

T

=

T

3

]

3

. 

H

.

248

: 

ADD

.

req

[

C

=

C

1

, 

T

=

?, 

stream

=

2

{

Codec

=

H

.

263

}, 

stream

=

3

{

Codec

=

AMR

}

]

4

. 

H

.

248

: 

ADD

.

resp

[

C

=

C

1

, 

T

=

T

3

]

17

. 

H

.

245

: 

Multiplex Entry Send

18

. 

H

.

245

: 

Multipex Entry Send Ack

20

. 

H

.

245

: 

Multiplex Entry Send Ack

19

. 

H

.

245

: 

Multipex Entry Send

Configure 

Multiplex 

Termination

Add 

Multiplex 

Termination

15

. 

H

.

245

: 

Open Logical Channel

[

H

245 

logical Channel 

= 

LC

2

]

16

. 

H

.

245

: 

Open Logical Channel Ack

Reserve IMS 

Connection Point

 or 

Reserve IMS 

Connection Point

and Configure 

Remote Resources

Configure

 IMS 

Ressources 

Prepare Bearer or 

Establish Bearer or 

Reserve TDM Circuit


NOTE 1:
All H.245 messages (from Signal 9 to Signal 18) are transported through the IM-MGW between the MGCF and the CS side, using the procedures described in clause E.4.2.3

NOTE 2:
Signals 21 and 22 are omitted if the same codec information has already been provisioned in signal 3.

Figure E.4.2.4.1: Mn signalling interactions for H.245 termination at the MGCF

The MGCF shall request terminations towards the CS network (Signal 1 and 2) and towards the IMS (Signal 3 and 4). For the terminations towards the IMS, the MGCF provides an estimate about the applicable codecs in the required information elements "Local IMS Resources" (for both "Reserve IMS Connection Point" procedure and "Reserve IMS Connection Point and Configure Remote Resources" procedure) and possibly "Remote IMS Resources" (only for "Reserve IMS Connection Point and Configure Remote Resources" procedure).

The MGCF shall request that the H.223 stream is (de-)multiplexed at the MUX termination T2, and that the H.245 control in H.223 Logical channel 0 is separated (signal 6). Furthermore, the MGCF shall request that the H.223 negotiation is started, and shall request to be notified about all H.245 messages received by the IM-MGW.
The IM-MGW shall start the H.223 Multiplexing Level Negotiation after receipt of the corresponding request from the MGCF and CS bearer establishment (Signal 8).

Upon reception of a H.245 Terminal Capability Set message (Signal 9), the MGCF sends a H.245 Acknowledgment message (Signal 10).

The MGCF shall know the H.324 related capabilities of the IM-MGW before starting the H.245 capability negotiation with the CS side, e.g. through configuration. The H.245 Terminal Capability Set message send by the MGCF (Signal 11) should include the codecs which are supported by both the IMS side and the IM-MGW, and the codecs which could be transcoded by the IM-MGW from the codecs supported by the IMS side.

The MGCF may defer sending the Terminal Capability Set message (Signal 11) for some time to wait for codec information from the CS peer's Terminal Capability Set message and perform a possible IMS-side codec re-negotiation. To avoid blocking situations, the MGCF shall not defer sending the signal for an excessive period of time.

To avoid the CS side selecting the codecs that need to be transcoded at the IM-MGW, the MGCF should aim to be the master in the H.245 master-slave determination procedure (Signals 9 to 12). The MGCF shall set the Terminal Type parameter as a number larger than 128 in the H.245 Master Slave Determination message. The H.245 master-slave determination procedure could be combined with the messages used for the H.245 capability exchange.

The codecs contained both in the sent and received terminal capability set message may be selected at the CS side. The final decision of the selected codecs at the CS side is taken with the H.245 open logical channel procedure (Signals 13 to 16).

After the completion of the H.245 multiplex table exchange procedure (Signals 17 to 20), the MGCF shall configure the multiplexing termination T2 by indicating to the IM-MGW the contents of the incoming and outgoing multiplex tables (Signal 21).

If codec information needs to be changed compared to what has been provisioned in signal 3, the MGCF shall also configure T3 with the appropriate video and/or speech codec(s) (signal 23).

The call is in the active state.

E.4.2.5
Handling of H.245 indication message

E.4.2.5.1
Overview

The MGCF shall support the following H.245 indication messages: Function Not Understood Indication / Function Not Supported Indication, Jitter Indication. The MGCF may support the H.245 User Input Indication message. All these H.245 messages are conveyed between the MGCF and the CS side through the IM-MGW, as described in clause E.4.2.3.

E.4.2.5.2
Function Not Understood / Function Not Supported message

This indication message is used to return requests, responses and commands that are not understood back to the transmitter.

If the MGCF receives a Function Not Understood or Function Not Supported message from the CS side, the MGCF may choose to attempt simpler H.245 interaction than the previous H.245 interaction that caused this indication. If this is not possible, the MGCF may release the call.

If the MGCF receives a H.245 request, response or command that can not be understood, the MGCF shall send H.245 Function Not Supported indication message to the CS side.

E.4.2.5.3
User Input Indication message

The User-Input-Indication message is used e.g. to transport the in-band DTMF information in the H.324 system.

The MGCF and IM-MGW may support transporting the DTMF information both from the CS side to the IMS side, and from the IMS side to the CS side.

Upon Receipt of a H.245 User-Input-Indication message, the MGCF may apply the procedures in clause 9.2.8.3 to request the IM-MGW to send corresponding RTP telephone-event(s) towards the IMS side.

Upon receipt of RTP telephone events from the IMS, the MGCF will be notified by the IM-MGW using the procedures in clause 9.2.8.1, if the MGCF has previously configured the IM-MGW as also described in this subclause. Upon receipt of the notification, the MGCF may send a H.245 User-Input-Indication message to the CS side.

E.4.2.6
Handling of H.245 Command message

E.4.2.6.1
Overview

The MGCF shall support the End Session command message. The MGCF may support the Flow Control command message and the videoFastUpdatePicture message. All these H.245 messages are conveyed between the MGCF and the CS side by the IM-MGW, as described in clause E.4.2.3.

E.4.2.6.2
Flow control command

The flow control command is used to restrict the upper limit of bit rate of either a single logical channel or the whole multiplex stream. The MGCF may support the flow control command received from the CS side.
NOTE:
It is very unlikely that the MGCF will receive a Flow control command, as the capability of controlling the video bit rate during a video call was neither implemented nor tested for UEs offering 3G-324M since R99, due to the lack of such a necessity in WCDMA where a fixed 64 kbps bearer is maintained to continuously transport 3G-324M PDUs.
[image: image112.emf]Call In Active State

7.H.248: Modify.rsp

1. H.245: Flow Control Command

8.H.245: Flow Control indication

4. H.245: Close Logical Channel{Old LCN}

                      Open Logical Channel {New LCN}

9.SIP:reINVITE

10.SIP:200 OK 11.H.248: Modify.req

C=C1,T=T3,stream{codec}

12.H.248: Modify.rsp

5. H.245: Close Logical Channel ACK {Old LCN}

              Open Logical Channel ACK{New LCN}

2.H.248: Modify.req

C=C1,T=T2,stream{LCN}, stream mode=Recieve Only

 3.H.248: Modify.rsp

6.H.248: Modify.req

C=C1,T=T2,stream{New LCN, codec}, stream mode = sendreceive

IM MGW MGCF


Figure E.4.2.6.2.1: Mn procedure of Flow control command

In Signal 1, the MGCF receives the Flow Control Command from the CS side.

If the minimum bitrate of the current codec is larger than the bitrate requested by the H.245 Flow Control Command message, the MGCF indicates the IM-MGW to stop the transmission of the media stream over the logical channel (signal 2). Then the MGCF may select another codec that can satisfy the requested bitrate limit. In signal 4, the MGCF closes the old logical channel and opens a new logical channel with new codec to satisfy the bitrate limit in the CS side. In signal 6, the MGCF indicates the IM-MGW to modify the LCN, codec and stream mode of the multiplexing termination. In signal 8, the MGCF sends flow control indication message to CS side with the current maximum bitrate. If the MGCF chooses to change the CS-side codec, the MGCF may also adjust the codec at the IMS side accordingly. To do so, the MGCF may need to re-negotiate the codec at the IMS side using a SIP re-INVITE message (signals 9 and 10). In addition, the MGCF may modify the codec of IMS termination accordingly (signal 11).
The MGCF may also apply interworking procedure towards RTCP at the IMS side as described in clause E.4.2.8 when receiving a H.245 Flow Control Command.
E.4.2.6.3
End Session Command

The end session command is used to close the H.245 control channel after all the logical channels have been closed.

The MGCF may send an end session command to the CS side through the IM-MGW to release a call.

If the MGCF receives an end session command from the CS side, it shall release the call if the call is in the active state.
E.4.2.6.4
videoFastUpdatePicture

The MGCF may apply interworking procedure towards RTCP as described in clause E.4.2.8 when receiving a H.245 videoFastUpdatePicture message. The MGCF may also be triggered by the procedures in clause E.4.2.8 to send a H.245 videoFastUpdatePicture message.

When receiving frequent H.245 videoFastUpdatePicture messages for a SCUDIF call (see TS 23.172 [121]), an MGCF should initiate a service-change from multimedia to speech and remove the IMS MTSI video component with SIP/SDP signalling. When receiving frequent H.245 videoFastUpdatePicture messages for a non-SCUDIF call, an MGCF should disable the CS video and remove the IMS MTSI video component with SIP/SDP signalling.

NOTE:
In a congestion situation at the IMS side, frequent packet losses might occur and dropping the video component may serve as a counter-measure to reduce the packet loss frequency. However, CS terminals might not be able to initiate such a switch-over to voice, but might rather react with repeated H.245 videoFastUpdatePicture message.
E.4.2.7
Mn Signalling Interactions to support the Media Oriented Negotiation Acceleration (MONA)

E.4.2.7.1
Overview

Media Oriented Negotiation Acceleration (MONA), as specified in ITU-T H.324[81] provides simplified procedures that allow for a faster call set-up of a H.324 Multimedia call than standard H.324 procedures, and also allow for a fallback to standard H.324 procedures if either party does not support the enhanced procedures.

The support of MONA is optional for an IM-MGW and MGCF supporting multimedia interworking, as no call failure but only a fallback to standard H.324 setup procedures will occur if the procedures are not supported.

MONA "preference message" signalling is used instead of H.324 Multiplexing level negotiation. Should standard H.324 Multiplexing level stuffing flags be received, a fallback to standard H.324 procedures is triggered. The sending of MONA preference messages is repeated by each MONA capable H.324 terminal until a reception is acknowledged by the peer. During this phase, two PDU types may optionally be attached by MONA terminals to these preference messages:

-
Media Preconfigured Channel (MPC) PDUs: MONA defines a small number of preconfigured H.223 channels for the most widespread audio and video codecs (AMR, AMR-WB, H.264 MPEG4 and H.263). Media PDUs for these codecs may be attached to the MONA "preference message" during the call setup.

-
Signalling Preconfigured Channel (SPC) PDUs: These PDUs are H.245 generic request messages with special parameters defined by MONA. These PDUs may also be attached to MONA preference messages.

According to MONA, each MONA capable terminal shall support at least one of these PDU types. The MONA capability of the IM-MGW can be audited by the MGCF.

The MONA preference message exchange in combination with attached MPC or SPC PDUs may result in the establishment of the desired media channels without further H.245 signalling. Otherwise, H.245 will be used after the MONA preference message exchange is acknowledged to negotiate media channels, but MONA defines some accelerated H.245 procedures (ACP) to speed up these H.245 procedures.

The design of Mn procedures to support MONA is guided by the following considerations:

-
The H.245 handling should be performed in the MGCF to keep procedures aligned as far as possible with standard Mn procedures to support H.324 interworking.

-
The MGCF should also control MONA preference message exchange procedures in order to maintain the agreed architectural work split between MGCF and IM-MGW in analogy to the H.245 handling.

-
However, the IM-MGW needs to understand the MONA preference messages at least to a sufficient degree to de-encapsulate the possibly attached MPC and SBC PDUs.

-
Furthermore, the frequent retransmissions of MONA preference messages required by MONA procedures are to be performed by the IM-MGW autonomously to avoid unnecessary load at the Mn interface and the MGCF.

-
Furthermore, for resource reservation at the IM-MGW, it is assumed that the IM-MGW has knowledge about supported predefined Media Preconfigured Channel configurations as specified in clause K.9.2 of H.324 [81] in order to limit the amount of information transferred in the Mn interface when establishing MPC channels. The offered channel resources are reserved by the IM-MGW.

-
In order to avoid increasing call establishment time when interworking with legacy terminals and at the same time avoid unnecessary load at the Mn interface, the MGCF may initiate both MONA and legacy H.245 procedures simultaneously in parallel. The MGCF shall in this case arm a legacy detection event with an embedded signal descriptor including the initial legacy H.245 message. The IM-MGW will only send the signal in case a fallback condition to legacy is detected.

Editor's note: It is FFS whether the mux-level-indication event can be used as a legacy detection event, and whether it is more feasible than the mechanism for legacy detection described.

E.4.2.7.2
Mn Interactions for MONA preference messages


[image: image113.wmf]IM MGW

6

. 

H

.

248

: 

ADD

.

req

[

C

=

C

1

, 

T

=

?,  

Mux

=

H

223

,

T

1

,

LocalControl

{

MONA preferences

, 

muxlv

=

2

} 

Event

=

legacyDetected

(

h

245

msgout

[

octet string

])

, 

MONA

-

preference

-

reception

,

 

MONA

-

preference

-

completed

, 

H

245

msgin

, 

[

MPC

-

reception

, 

SBC

-

reception

] ]

5

. 

Bearer Establishment

MGCF

8

. 

MONA preferences

[

ack

=

00

]

19

. 

H

.

248

: 

MOD

.

req

[

C

=

C

1

, 

T

=

T

2

, 

Stream

=

1

{

LocalCont

{

h

223

capr

,

muxtbl

_

in

,

muxtbl

_

out

}}

stream

=

2

{

LCN

=

LC

1

, 

Codec

=

H

263

}

stream

=

3

{

LCN

=

LC

2

, 

Codec

=

AMR

}

]

20

. 

H

.

248

: 

MOD

.

resp

1

. 

H

.

248

: 

ADD

.

req

[

C

=

?, 

T

=

?, 

stream

=

1

]

2

. 

H

.

248

: 

ADD

.

resp

[

C

=

C

1

, 

T

=

T

1

]

7

. 

H

.

248

: 

ADD

.

resp

[

C

=

C

1

, 

T

=

T

2

]

21

. 

H

.

248

: 

MOD

.

req

[

C

=

C

1

, 

T

=

3

, 

stream

=

2

{

Codec

=

H

263

}, 

stream

=

3

{

Codec

=

AMR

]

22

. 

H

.

248

: 

MOD

.

resp

[

C

=

C

1

, 

T

=

T

3

]

3

. 

H

.

248

: 

ADD

.

req

[

C

=

C

1

, 

T

=

?, 

stream

=

2

{

Codec

=

H

.

263

}, 

stream

=

3

{

Codec

=

AMR

}

]

4

. 

H

.

248

: 

ADD

.

resp

[

C

=

C

1

, 

T

=

T

3

]

Configure 

Multiplex 

Termination

Add 

Multiplex 

Termination

Reserve IMS 

Connection Point

 or 

Reserve IMS 

Connection Point

and Configure 

Remote Resources

Configure

 IMS 

Ressources 

Prepare Bearer or 

Establish Bearer or 

Reserve TDM Circuit

10

. 

MONA preferences

[

ack

=

01

]

8

. 

MONA preferences

[

ack

=

00

]

9

. 

MONA preferences

[

ack

=

00

]

9

. 

MONA preferences

[

ack

=

00

]

11

.

H

.

248

[

C

=

C

1

,

T

=

T

2

,

Event

=

MONA

-

preference

-

receptiom

 

{

MONA preferences

}

]

12

.

H

.

248

: 

Notify

.

resp

: 

Notify

.

req

Notify 

MONA 

prefrence 

reception

10

. 

MONA preferences

[

ack

=

01

]

14

. 

MONA preferences

[

ack

=

10

]

13

. 

MONA preferences

[

ack

=

01

]

Event

=

MONA

-

preference

-

completed

]

17

.

H

.

248

: 

Notify

.

resp

Notify 

MONA 

prefrence 

completed

16

.

H

.

248

[

C

=

C

1

,

T

=

T

2

,

: 

Notify

.

req

14

. 

MONA preferences

[

ack

=

10

]

13

. 

MONA preferences

[

ack

=

01

]

15

. 

MONA preferences

[

ack

=

10

]

18

. 

Accelerated H

.

245 

call setup procedures

Optional Procedures for reception of SPC PDU

, 

sending of SPC PDU

, 

reception of MPC 

PDU

, 

sending of MPC PDU


NOTE 1:
MONA preference messages are repeated several times. One repetition is shown for each such message, with the same signal number as the first message.

NOTE 2:
The Context model in figure E.4.2.2.1 is assumed in this call flow.

Figure E.4.2.7.2.1: Mn signalling interactions for MONA preference messages

The MGCF shall request terminations towards the CS network (Signal 1 and 2) and towards the IMS (Signal 3 and 4). For the terminations towards the IMS, the MGCF provides an estimate about the applicable codecs in the required information elements "Local IMS Resources" (for both "Reserve IMS Connection Point" procedure and "Reserve IMS Connection Point and Configure Remote Resources" procedure) and possibly "Remote IMS Resources" (only for "Reserve IMS Connection Point and Configure Remote Resources" procedure).

The MGCF shall request that the H.223 stream is (de-)multiplexed at the MUX termination T2 (signal 6). Furthermore, the MGCF shall request that MONA preferences negotiation is started, and shall provision the MONA preferences to be indicated by the IM-MGW. The MGCF shall encode the MONA preferences as described in clause K.6 of ITU-T H.324 [81]. The MGCF shall take the H.324 related capabilities of the IM-MGW into account in the MONA preferences. The MGCF can know these capabilities by configuration. The IM-MGW will only support symmetric codec usage. If several codec alternatives are offered for MPC, it is the responsibility of the MGCF to ensure that symmetric codecs are established by not selecting transmit codec until the receive channel has been opened by MPC media. The MGCF shall also request to be notified about the reception of the remote MONA preferences and about the completion of the MONA preference exchange, or an H.245 message on the H.223 control channel. The MGCF may also initiate standard H.245 signalling in parallel in order to minimize the time for a legacy interworking fallback. This is done by arming a legacy detection event including an embedded signal descriptor. The embedded signal is the initial H.245 message out signal (including H.245 TCS+MSD) to send in case fallback to legacy interworking is detected. The IM-MGW will only send the embedded signal in case it detects H.223 related indications of a legacy interworking as specified in clause K.7.1.2 in H.324 [81]. Upon receiving the legacy detected event, the MGCF continues with standard H.245 call setup procedures waiting for the reception of a remote H.245 TCS as well as acknowledgements on the sent H.245 TCS+MSD. If the MGCF indicates the capability to receive SPC PDUs within the MONA preferences, it shall also request to be notified about incoming SPC PDUs, as detailed in clause E.4.2.7.4. If the MGCF indicates the capability to receive any MPC PDUs within the MONA preferences, it shall also request to be notified about incoming MPC PDUs, as detailed in clause E.4.2.7.3.
The IM-MGW shall start sending MONA preference messages after receipt of the corresponding request from the MGCF and CS bearer establishment (Signal 8). The IM-MGW shall repeat sending those messages and increment the acknowledgment bits of sent MONA preference messages when receiving incoming MONA preference messages according to MONA procedures (signals 8, 10, 14).

After sending at least 10 MONA preference messages, while the IM-MGW continues to send and receive MONA preference messages, it shall attach MPC or SPC PDUs if requested to do so by the MGCF as described in clauses E.4.2.7.3 and E.4.2.7.4, respectively. If the IM-MGW receives preference messages with an attachment, it shall inspect the first octet of that attachment that will contain a MUX code according to table K.15 of ITU-T H.324 [81] that identifies the attached PDU as either a MPC PDU of one of the predefined channels or a SPC PDU. The IM-MGW shall handle the attached MPC or SPC PDUs as described in clause E.4.2.7.3 and E.4.2.7.4, respectively.

After sending at least 10 MONA preference messages, the IM-MGW should insert stuffing flags indicating the multiplexing level received from the MGCF between MONA preference messages as described in clause K.7.1.1 of ITU-T H.324 [81].

The IM-MGW shall notify the MGCF when receiving the first incoming MONA preference message (Signals 11 and 12) and forward the received information. Subsequent incoming MONA preference message will be identical apart from possible increments in the acknowledgement bits. The IM-MGW shall not notify the MGCF about these messages. Upon reception of the notification of a MONA preference message, the MGCF shall compare the received MONA preferences message with the preferences message it sent and react as described in clause 7.1 of ITU-T H.324 [81].

When receiving an incoming MONA preference message with acknowledgment bits 10 or receiving the first non-empty H.223 MUX PDU, the IM-MGW shall notify the MGCF about the completion of the MONA preference exchange procedure (signals 16 and 17). The notification shall be only triggered by the one of the two events which occurs first. If the IM-MGW is not configured to send and detect SPCs, the IM-MGW shall then stop sending MONA preference messages and, if a MONA preference message with acknowledgment bits 10 has been received, it shall also stop receiving MONA preference messages. If the IM-MGW is configured to send and detect SPCs, the IM-MGW shall continue sending and receiving MONA Preference messages encapsulating the SPC/MOS messages until the MGCF configures the IM-MGW stop sending and detecting SPCs.
NOTE:
In the unlikely case that a first non-empty H.223 MUX-PDU is received before the first MONA Preference Message has been received, the IM-MGW will continue the detection of incoming MONA Preference Messages and will apply the Notify-MONA-preference-Reception-procedure after the Notify-MONA-preference-completed procedure.
After receiving the notification about the completion of the MONA preference exchange procedure, and a completion of the possible subsequent accelerated H.245 setup procedures, the MGCF shall configure the multiplexing termination T2 by indicating to the IM-MGW the contents of the incoming and outgoing multiplex tables (Signal 19), and may modify the selected codecs at both the MUX and the IMS side (signal 21).

E.4.2.7.3
Mn Interactions for MONA MPCs


[image: image114.emf]IM MGW MGCF

1. 

H.248: ADD.req

[C=C1,T=?

... ,Event=MPC-reception, 

Signal =MPC MUX Code]

}

2. H.248: ADD.resp

6H.248

[C=C1,T=T2

,

Event=MONA-preference-receptiom

{MONA preferences},]

7H.248

: Notify.resp

: Notify.req

Add 

Multiplex 

Termination

8. H.248: MOD.req

[C=C1, T=T2, 

Stream=1{LocalCont{,muxtbl_out}}]

9. H.248: MOD.resp

Configure 

Multiplex 

Termination

Notify 

MONA 

prefrence 

reception,

.

3

MONA preference message,

Mux Code

.

4

MONA preference message,

Mux Code, MPC PDU

.

5

MONA preference message,

Mux Code

.

11.

MONA preference message,

Mux Code

.

12.

MONA preference message,

Mux Code, MPC PDU

13. H.248

[C=C1,T=T2

,

Event=MPCin

{muxcode=Mux Code}]

14 H.248

: Notify.resp

: Notify.req

Notify 

MPC

10.

PDU in MPC 

15. H.248: MOD.req

[C=C1, T=T2, 

Stream=1{LocalCont{,muxtbl_in}}]

16. H.248: MOD.resp

Configure 

Multiplex 

Termination


NOTE 1:
MONA preference messages are repeated several times. One repetition is shown for each such message, with the same signal number as the first message.

NOTE 2:
The Context model in figure E.4.2.2.1 is assumed in this call flow.
Figure E.4.2.7.3.1: Mn interactions for MONA MPCs
If the MGCF indicates the ability to receive any predefined MPCs channel types in the MONA preferences messages, the MGCF shall request the IM-MGW to report the channel type of received MPC PDUs (Signal 1 in figure E.4.2.7.3.1, Event "MPC Reception").

If the MGCF intends to use MPCs for sending media during the MONA setup, the MGCF shall request the IM-MGW to send available media encoded according to the media predefined channel types defined by MONA (signal 1 in figure E.4.2.7.3.1, Signal "MPC MUX Code" ) while the MONA preference exchange described in clause E.4.2.7.2 is ongoing. The MGCF should select channel types for codecs which are supported by both the IMS side and the IM-MGW, and/or for codecs which could be transcoded by the IM-MGW from the codecs supported by the IMS side. The MGCF shall only include one channel type per each desired media type (audio, video) in the "MPC MUX Code" signal. The MGCF may also configure the MGW to receive these channels at the same time by supplying the Incoming Multiplex Table IE in the Add Multiplex Termination Procedure.

Upon reception of this request, the IM-MGW shall forward any media received from the IMS side in MPC PDUs of the corresponding predefined channel type attached to MONA preference messages, transcoding the media if required (Signal 4).
According to the procedures in clause E.4.2.7.2, the IM-MGW will notify the MGCF (signal 6) when receiving the first incoming MONA preference message (Signals 5). The MGCF shall then analyse the Media Preconfigured Channel Receive bits within the MONA preference message and configure the MGW accordingly:

-
If MPC(s) that the MGCF has previously configured within the "MPC MUX Code" signal are supported according to these bits, the MGCF shall configure the IM-MGW to send these MPC using the predefined channels (signal 10) by supplying the corresponding Outgoing Multiplex Table in the Configure Multiplex Termination procedure (signal 8).

-
If some of the MPC(s) that the MGCF has previously configured within the "MPC MUX Code" signal are not supported according to these bits, the MGCF shall configure the IM-MGW to terminate the media on those MPC(s) by modifying the MPC Mux Code signal using the Configure Multiplex Termination procedure. In addition, the MGCF may configure the IM-MGW to send media on other supported MPC(s) by supplying the corresponding Outgoing Multiplex Table in the Configure Multiplex Termination procedure (not shown in figure E.4.2.7.3.1).
-
If all of the MPC(s) that the MGCF has previously configured within the "MPC MUX Code" signal are not supported according to these bits, the MGCF shall configure the IM-MGW to terminate MPC operations in those unsupported channels. The MGCF shall either remove the MPC Mux Code signal using the Stop MPC procedure or configure the IM-MGW to send media on other supported MPC(s) by supplying the corresponding Outgoing Multiplex Table in the Configure Multiplex Termination procedure (not shown in figure E.4.2.7.3.1).

When the MGCF provides an Outgoing Multiplex Table, the IM-MGW shall terminate sending any MPC PDUs as attachments to MONA preference message.

NOTE:
Any previously provisioned "MPC MUX Code" signal becomes irrelevant.

When being notified about the receipt of the first incoming MONA preference message, the MGCF may also analyse the Media Preconfigured Channel Receive bits within the MONA preference message and configure the MGW with a suitable Incoming Multiplex Table using the Configure Multiplex Termination procedure (not shown in figure E.4.2.7.3.1).
If the IM-MGW receives the first MONA preference message with attached MPC PDU of a given predefined channel type (signal 12), or if the IM-MGW receives the first non-empty H.223 MUX PDU of a given predefined channel type which has been configured with the Incoming Multiplex Table, and the MGCF has requested a notification about such an event, the IM-MGW shall notify the MGCF about the received channel type (signal 13). The IM-MGW shall not notify the MGCF about subsequent receptions of MPC PDUs of the same channel type.

Upon reception of such a notification, if the IM-MGW supports the indicated channel type and has not yet been configured to receive media of that channel type, and if the MGCF has previously indicated the capability to receive MPCs of that channel type within MONA preference messages, the MGCF shall configure the IM-MGW to receive media of that channel type and forward them to the IMS side by supplying the Incoming Multiplex Table IE in the Configure Multiplex Termination procedure (Signal 15).
If the MGCF determines (e.g. after receipt of signal 5 or signal 12) that some desired media channels can not be established using the MPC procedures, the MGCF should use accelerated H.245 procedures as defined by MONA to set up media channels. Corresponding H.245 messages shall be transported transparently between the IM-MGW and the MGCF using the "Signal H.245 message" and "Notify H.245 message" procedures.
The MGCF can receive a forwarded MONA preference message from the IM-MGW when the MGCF has already configured the IM-MGW to send media on an MPC. If the processing of this message in the MGCF leads to the conclusion that MPC procedures need to be aborted because SPC preferred (SPP) is determined, the MGCF shall configure the IM-MGW to terminate MPC operations by removing the MPC related event MPC-reception and MPC Mux Code signal using the Stop MPC procedure.
E.4.2.7.4
Mn Interactions for MONA SPCs

E.4.2.7.4.1
General

H.245 PDUs for SPC shall be transported between the IM-MGW and MGCF over the Mn interface using H.248 Events (from the IM‑MGW towards the MGCF) and H.248 Signals (from the MGCF towards the IM-MGW). The Events/Signals shall contain the following information:

-
H.245 message (binary).

The related procedures are distinct from the procedures in clause E.4.2.3, since the PDUs are received or sent by the IM-MGW using the SPC, i.e. as attachment to MONA preference messages.

If the MGCF supports SPCs, it shall comply with the SPC procedures in clause K.8 of ITU-T H.324 [81]. The repetition of sending the same SPCs will be handled by the IM-MGW. When the MGCF receives an acknowledgement from the CS side it shall request the IM-MGW to stop the repetition sending of the SPCs,

Within the sent SPC PDUs, the MGCF should include the codecs which are supported by both the IMS side and the IM-MGW, and the codecs which could be transcoded by the IM-MGW from the codecs supported by the IMS side.

E.4.2.7.4.2
Transport from MGCF to IM-MGW


[image: image115.emf]IM MGW

.

MGCF

 1. H.248: MOD.req

[C=C1,T=T2,

signal=SPCout

{h245mc=XXXX}

]

2. H.248: MOD.resp

3. 

send H.245 message XXXXattached to 

MONA prefrence request

Signal 

SPC

.

3. 

send H.245 message XXXXattached to 

MONA prefrence request


NOTE 1:
MONA preference messages are repeated several times. One repetition is shown for each such message, with the same signal number as the first message.

NOTE 2:
The Context model in figure E.4.2.2.1 is assumed in this call flow.
Figure E.4.2.7.4.2.1: Mn interactions for sending MONA SPCs

In Signal 1, the MGCF requests the IM-MGW to send an H.245 message to the CS side. To request the IM-MGW to send a H.245 message to the CS side, the MGCF shall sent an H.248 signal to the IM-MGW with the complete H.245 message content.

Upon reception of this signal, the IM-MGW shall send the encapsulated H.245 message within the H.248 signal, as attachment to a MONA preference message as described in clause K.9.4 of ITU-T H.324 [81] (signal 3). It should repeat sending this H.245 message as attachment to subsequent MONA preference messages.

E.4.2.7.4.3
Transport from IM-MGW to MGCF


[image: image116.emf]IM MGW

.

MGCF

1. 

H.248: ADD.req

[C=C1,T=? ... ,Event=SPC-reception]

2. H.248: ADD.resp

3. 

receive H.245 message attached to 

MONA preference message

4. 

H.248

[C=C1,T=T2

,

Event=SPCin

{h245mc=XXXX}]

5. H.248

: Notify.resp

Notify 

SPC

: Notify.req

Add 

Multiplex 

Termination


NOTE 1:
MONA preference messages are repeated several times. One repetition is shown for each such message, with the same signal number as the first message.

NOTE 2:
The Context model in figure E.4.2.2.1 is assumed in this call flow.
Figure E.4.2.7.4.3.1: Mn interactions for receiving MONA SPCs

In signal 1, the MGCF requests the IM-MGW to detect received H.245 message from the CS side in SPC PDUs attached to MONA preference messages and forward them to the MGCF. To request the IM-MGW to detect and forward these H.245 message, the MGCF shall send a suitable H.248 event to the IM-MGW. The event may be indicated through an H.248 ADD command.

In signal 3, the IM-MGW receives an H.245 message from the CS side attached as SPC PDU to a MONA preference message. Upon reception of such an H.245 message from the CS side, the IM-MGW may check, based on bitwise comparison of the previously received H.245 message, if it has already forwarded the same H.245 message to the MGCF, in which case the IM-MGW may choose not to forward the same H.245 message to the MGCF. Otherwise the IM-MGW shall forward the H.245 message to the MGCF within an H.248 Notify command (signal 4).

NOTE:
According to H.324 [81] a MOS request Ack message shall be sent to every received MOS request. If the IM-MGW chooses, based on the bitwise comparison, not to forward the received H.245 message to the MGCF, no MOS request Ack message will be generated. However, the MGCF will request the IM-MGW to automatically retransmit the MOS request Ack message generated by the MGCF itself.

If the IM-MGW does not support forwarding SPC PDUs or has not been requested by the MGCF to forward these PDUs, it shall discard received SPC PDUs.
E.4.2.7.4.4
Termination of SPC procedure


[image: image117.emf]IM MGW

.

MGCF

1 receive final H.245 message attached 

to MONA preference message

2H.248

[C=C1,T=T2

,

Event=SPCin

{h245mc=XXXX}]

3H.248

: Notify.resp

Notify 

SPC

: Notify.req

4

H.248: MOD.req

[C=C1,T=T2Remove Signal-SPC+ SPC-reception]

5H.248: MOD.resp

Stop SPC


NOTE 2:
The Context model in figure E.4.2.2.1 is assumed in this call flow.

Figure E.4.2.7.4.4.1: Mn interactions for terminating SPC procedure.

If the processing of an incoming MONA Preference Message in the MGCF leads to the conclusion that SPC procedures need to be aborted as a result of the test of the SPC bits, the MGCF shall configure the IM-MGW to terminate SPC operations by removing the SPC related event SPC-reception and SPCout signal using the Stop SPC procedure. In this case, H.245 signalling may be started simultaneously.

If the MGCF determines the completion of the SPC procedures according to clause K.8 of ITU-T H.324 [81] (based on incoming signal 1 in figure E.4.2.7.4.4.1), the MGCF shall configure the IM-MGW to stop sending and detecting SPCs (signal 4 in figure E.4.2.7.4.4.1).
E.4.2.7.5
Mn Interactions for fallback from MONA procedures to standard H.324 setup


[image: image118.wmf]IM MGW

6

. 

H

.

248

: 

ADD

.

req

[

C

=

C

1

, 

T

=

?,  

Mux

=

H

223

,

T

1

,

LocalControl

{

MONA preferences

, 

muxlv

=

2

}

,

 

Event

=

legacyDetected

(

h

245

msgout

[

octet string

])

, 

MONA

-

preference

-

reception

,

 

MONA

-

preference

-

completed

, 

H

245

msgin

, 

MPC

-

reception

, 

SBCin 

]

5

. 

Bearer Establishment

MGCF

8

. 

MONA preferences

[

ack

=

00

]

1

. 

H

.

248

: 

ADD

.

req

[

C

=

?, 

T

=

?, 

stream

=

1

]

2

. 

H

.

248

: 

ADD

.

resp

[

C

=

C

1

, 

T

=

T

1

]

7

. 

H

.

248

: 

ADD

.

resp

[

C

=

C

1

, 

T

=

T

2

]

3

. 

H

.

248

: 

ADD

.

req

[

C

=

C

1

, 

T

=

?, 

stream

=

2

{

Codec

=

H

.

263

}, 

stream

=

3

{

Codec

=

AMR

}

]

4

. 

H

.

248

: 

ADD

.

resp

[

C

=

C

1

, 

T

=

T

3

]

Add 

Multiplex 

Termination

Reserve IMS 

Connection Point

 or 

Reserve IMS 

Connection Point

and Configure 

Remote Resources

Prepare Bearer or 

Establish Bearer or 

Reserve TDM Circuit

8

. 

MONA preferences

[

ack

=

00

]

9

. 

Legacy interworking condition

1

1

.

H

.

248

[

C

=

C

1

,

T

=

T

2

,

Event

=

legacyDetected

1

2

.

H

.

248

: 

Notify

.

resp

: 

Notify

.

req

Notify

 detection of 

legacy interworking

1

3

. 

standard H

.

245 

call setup procedures

 continuation

, 

Figure 

E

.

4

.

2

.

4

.

1

 starting with step 

9

10

. 

octet string 

(

H

.

245 

TCS

+

MSD

)


NOTE 1:
MONA preference messages are repeated several times. One repetition is shown for each such message, with the same signal number as the first message.

NOTE 2:
The Context model in figure E.4.2.2.1 is assumed in this call flow.
Figure E.4.2.7.5.1: Mn signalling interactions for fallback from MONA procedures to standard H.324 setup

When the MGCF requests that the MONA preferences negotiation is started, the MGCF may also initiate standard H.245 signalling towards the IM-MGW that shall only be sent in case the IM-MGW detects legacy interworking. The MGCF arms an event to detect legacy interworking with an embedded signal descriptor including an H.245 message out signal. The embedded H.245 signal is the initial H.245 TCS+MSD signal to send in case fallback to legacy interworking is detected. The MGCF shall also request to be notified about a H.245 message on the H.223 control channel. The MGCF shall also provision a multiplexing level which will be advertised by the IM-MGW (Signal 6).
If the IM-MGW detects a legacy interworking condition (signal 9), it shall stop sending MONA preference messages, including those MONA messages used to encapsulate MPC or SPC. The IM-MGW shall engage in normal H.324 multiplexing level negotiations. In case the MGCF armed a legacy detection event, the embedded H.245 signal shall be sent by the IM-MGW (signal 10). The legacy detection event is sent to the MGCF (signal 11).

The MGCF shall upon detection of legacy interworking stop MONA procedures and continue with standard H.245 call set up procedures, as depicted in figure E.4.2.4.1 starting with step 9.

If the IM-MGW receives a normal H.245 message (not depicted), it shall also forward this message to the MGCF. If the MGCF receives such a H.245 message during the MONA call setup, and this H.245 message is a normal Terminal Capability Set message, the MGCF shall also stop MONA procedures and continue with standard H.245 call set up procedures, as depicted in figure E.4.2.4.1 starting with step 9.
To stop MONA related procedures at the IM-MGW, the MGCF shall remove the MONA related signals that were active (MONA Preferences, SPCout, MPC Mux Code; note that MONA Preference and SPCout causes the MGW to send the MONA Preference message or the contained H.245 message several times) and events (MonaPreferenceReception, MonaPreferenceCompleted, SPC-reception and MPC-reception, using the Stop MONA Negotiation procedure.
E.4.2.8
Interworking between RTCP messages and H.245 messages

E.4.2.8.1
Overview

The following clauses describe the interworking procedures between the RTCP messages listed in table E.4.2.8.1.1, which are used to enhance the quality of the media distribution by MTSI terminals at the IMS side, and the corresponding H.245 messages in 3G-324M at the CS side listed in Table E.4.2.8.1.1.

As the H.245 protocol is terminated at the MGCF and RTCP is received at the IM-MGW, Mn procedures to support the transfer of RTCP related information are defined in the following clauses to support this interworking.

Table E.4.2.8.1.1: RTCP and H.245 messages that may be interworked
	RTCP Messages
	H.245 messages
	Remarks

	Picture Loss Indication (PLI) ([109])
	VideoFastUpdatePicture
	

	Temporary Maximum Media Stream Bit Rate Request (TMMBR) ([110])
	Flow Control Command/ Flow Control Indication
	This interworking procedure is not recommended.
The reception of TMMBR at the IMS side is expected to be more frequent than the reception of Flow Control Command at the CS side.


NOTE 1:
It is very unlikely that the MGCF will receive a Flow control command, as the capability of controlling the video bit rate during a video call was neither implemented nor tested for UEs offering 3G-324M since R99, due to the lack of such a necessity in WCDMA where a fixed 64 kbps bearer is maintained to continuously transport 3G-324M PDUs.

NOTE 2:
Terminals are expected to support receiving the FlowControlCommand, as this is a mandatory feature in clause 6.4.3 in H.324. But most terminals would not really follow the received FlowControlCommand, but rather ignore it. However, for older 3G-324M terminal lock-up or system crash was not uncommon during IOTs. Therefore safe handling of this command is not guaranteed absolutely.
The interworking shown in Table E.4.2.8.1.1 is only applicable if video is relayed without transcoding.
If transcoding is applied, the IM-MGW should adopt its transcoder accordingly if receiving the RTCP messages listed in Table E.4.2.8.1.1, and the MGCF should configure the transcoder in the IM-MGW accordingly if it receives the H.245 messages in Table E.4.2.8.1.1.

For an MGCF and IM-MGW that support video interworking as specified in the present Annex E, the support of the procedures in the present clause E.4.2.8.2 and E.4.2.8.3 is optional.

E.4.2.8.2
IM CN subsystem originated RTCP messages

The MGCF may configure the IM-MGW to detect specific received RTCP packets and forward information derived from it by providing a bit pattern(s) for bits 3-15 of incoming RTCP packets as filter criterion in an event descriptor of event "RTCP-interworking". Bits 3-7 in the RTCP packet represent the Feedback message type (FMT), and bits 8-15 in the RTCP packet represent the Payload type (PT) RTCP header fields, see IETF RFC 4585 [109]. For instance, to request that an AVPF Picture Loss Indication (PLI) packet is detected for interworking, the MGCF needs to configure a combination of PT=206 (Payload-specific FB message) and FMT=1 (see IETF RFC 4585 [109]). To request that a AVPF Temporary Maximum Media Bit-rate Request (TMMBR) packet is detected for interworking, the MGCF needs to configure a combination of PT=205 (transport layer feedback message) and FMT=3 (see IETF RFC 5104 [110]).

A IM-MGW configured in this way shall check after receiving an incoming RTCP Packet if it is of a desired type, as expressed by a bit pattern received from the MGCF. An incoming RTCP message can be of compound format and contain several RTCP packets. The IM-MGW shall then perform the check separately for each packet. If the IM-MGW determines that the received RTCP packet is of a desired type, the IM-MGW shall send information derived from the RTCP message as ObservedEventsDescriptor Parameter(s) of event "RTCP-interworking" in an H.248 "Notify" message to the MGCF.

The information derived from the RTCP message to be sent to the MGCF is listed in table E.4.2.8.2.1. Information elements in table E.4.2.8.2.1 shall only be sent if the corresponding RTCP message has been received.

Table E.4.2.8.2.1: RTCP information elements to be notified over Mn

	RTCP message
	Information elements transported over Mn interface
	Remarks

	Picture Loss Indication (PLI) ([109])
	UpdatePicture
	

	Temporary Maximum Media Stream Bit Rate Request (TMMBR) ([110]) 
	MaxBitRate
	This information indicates the maximum media stream bit rate as received within the TMMBR.


Figure E.4.2.8.2.2.1 shows examples of interactions between RTCP and H.245 for IM CN subsystem originated feedback on the quality of the media distribution.

[image: image119.emf]IM MGW

3. receive RTCP 

feedback messages

4.  send RTCP 

feedback Ack messages

CALL IN ACTIVE STATE

8. receive H.245 message response

7. send H.245 message

CS Network

IMS

MGCF

1. H.248: MOD.req [C=C1, T=T1

…

, 

Event=RTCP-Interworking]

2. H.248: MOD.resp

6. H.248: Notify.resp

5. H.248: Notify.req [C=C1, T=T1, 

Event=RTCP-Interworking]


Figure E.4.2.8.2.2.1: Interactions between RTCP and H.245 for IM CN subsystem originated RTCP feedback on the quality of the media distribution
In signal 1, the MGCF requests the IM-MGW to detect received RTCP feedback message from the IMS side and notify the feedback information elements to the MGCF when the interworking is required. To request the IM-MGW to detect and notify these feedback information elements, the MGCF shall send the "RTCP-interworking" H.248 event to the IM-MGW. The event may be indicated through an H.248 MOD command.
In signal 3, the IM-MGW receives a RTCP feedback message from the IMS side.

In signal5, the IM-MGW notifies the feedback information elements from RTCP message to the MGCF.

In signal7, the MGCF send the corresponding H.245 message to the CS side to request for the media adaption.

E.4.2.8.3
CS network originated H.245 messages

Figure E.4.2.8.3.2.1 shows examples of interactions between H.245 and RTCP for CS network originated feedback on the quality of the media distribution.


[image: image120.emf]IM MGW

6. receive RTCP 

feedback Ack message

5.  send RTCP 

feedback message

CALL IN ACTIVE STATE

CS Network

IMS

MGCF

3. H.248: MOD.req [C=C1, T=T1

…

, 

signal=H.245-Interworking]

4. H.248: MOD.resp

1. receive H.245 message to 

feedback on the 

quality of the data distribution

2. send H.245 message response


Figure E.4.2.8.3.2.1: Interactions between H.245 and RTCP for CS network originated H.245 feedback on the quality of the media distribution
In signal 1, the MGCF receives an H.245 feedback message from the CS side.

In signal3, the MGCF requests the IM_MGW to send an RTCP message provisioning the signal "H.245 interworking" with parameters identifying information to be transported within this RTCP message.

In signal5, the IM-MGW send the corresponding RTCP message to the IMS side to request for the media adaption.

To request the IM-MGW to send an RTCP message, the MGCF shall send the corresponding information elements listed in table E.4.2.8.3.2.1 as parameters of the "H.245-interworking signal" over the Mn interface.
Table E.4.2.8.3.2.1: Feedback information elements to be indicated

	RTCP message
	Information elements transported over Mn interface
	Remarks

	Picture Loss Indication (PLI) ([109])
	UpdatePicture
	

	Temporary Maximum Media Stream Bit Rate Request (TMMBR) ([110])
	MaxBitRate
	This information indicates the maximum media stream bit rate to be sent within the TMMBR.


E.4.3
Mn Signalling procedures

E.4.3.1
Overview

This clause describes the logical signalling procedures (i.e. message identifiers are not part of the protocol) between the MGCF and IM-MGW. The procedures within this clause are intended to be implemented using the standard H.248 procedure as defined in ITU recommendation H.248.1 [2] with appropriate parameter combinations.

E.4.3.2
Add Multiplex Termination

This procedure is used to add a termination to multiplex/demultiplex H.223. This procedure containing the MuxDescriptor with H.223 value enables the IM-MGW to start the H.324 Multiplexing Level Negotiation.
Table E.4.3.2.1: Add Multiplex Termination Procedure

	Procedure
	Initiated
	Information element name
	Information element required
	Information element description

	Add Multiplex Termination
	MGCF
	Context
	M
	This information element indicates the existing context.

	
	
	Termination
	M
	This information element requests a new termination

	
	
	MuxDescriptor
	M
	This information element indicates that data multiplexed according to H223 shall be received, and from which termination.

	
	
	Notify Termination Heartbeat
	M
	This information element requests termination heartbeat indications

	
	
	Notify Released Bearer
	O
	This information element requests a notification of a released bearer.

	
	
	Incoming H.245 message Notification Request
	M
	This Event shall indicate that a Notification about H.245 messages received by the IM-MGW is requested by the MGCF

	
	
	MONA Preferences
	O
	This information element requests the MGW to start a MONA negotiation and provisions MONA preferences to be indicated by the IM-MGW, encoded as described in clause K.6 of ITU-T H.324 [81]

	
	
	MONA Preference Reception
	O
	This information element requests a notification of the reception of the first MONA preference message.

	
	
	MONA Preference Completed
	O
	This information element requests a notification of the reception of the first MONA preference message with indication that the MONA preference negotiation is completed.

	
	
	Legacy Detected
	O
	This information element requests a notification of a legacy interworking condition. The Event Descriptor also embeds a signal descriptor including an H.245 message out signal which shall be send when legacy interworking is detected

	
	
	MPC Reception
	O
	This information element requests a notification of the reception of the first MONA preference message with attached MPC.

	
	
	SPC Reception
	O
	This information element requests a notification of the reception of a MONA preference message with attached SPC.

	
	
	Incoming Multiplex Table
	O
	This information element indicates the value of the H245 MultiplexEntrySend message and allows the MGW to identify incoming H.223 MUX PDUs of the channel types given in the table.

	
	
	MPC Mux Code
	O
	This information element requests the MGW to send available media in MPCs and indicates the channel type to use for sending media.

	Add Multiplex Termination

Ack
	IM-MGW
	Context
	M
	This information element indicates the context where the command was executed.

	
	
	Termination
	M
	This information element indicates the new termination where the command was executed.


E.4.3.3
Configure Multiplex Termination

This procedure is used to configure a termination to multiplex/demultiplex H.223. It is also used to exclude some of the outgoing MPC channel types due to the MONA Preference received from the peer.
Table E.4.3.3.1: Configure Multiplex Termination Procedure

	Procedure
	Initiated
	Information element name
	Information element required
	Information element description

	Configure Multiplex Termination
	MGCF
	Context
	M
	This information element indicates the existing context.

	
	
	Termination
	M
	This information element indicates the termination

	
	
	Remote H223 Capability
	O
	This information element indicates the remote H223 capabilities, as received by the MGCF. The element shall not be included if the MPC Mux Code IE is included.

	
	
	Incoming Multiplex Table
	O
	This information element indicates the value of the H245 MultiplexEntrySend message, as received by the MGCF from the remote H.245 peer. Either this element or the MPC Mux Code IE shall be included.

	
	
	Outgoing Multiplex Table
	O
	This information element indicates the value of the H245 MultiplexEntrySend message, as sent by the MGCF towards the remote H.245 peer. Either this element or the MPC Mux Code IE shall be included.

	
	
	MPC Mux Code
	O
	This information element requests the MGW to continue sending available media in MPC and indicates the channel type to use for sending media. It is used in this procedure only to restrict the list of channel types indicated previously in the Add Multiplex Termination procedure. Either this element or the Incoming and Outgoing Multiplex Table elements shall be included.

	Configure Multiplex Termination
Ack
	IM-MGW
	Context
	M
	This information element indicates the context where the command was executed.

	
	
	Termination
	M
	This information element indicates the termination where the command was executed.


E.4.3.4
Signal H245 Message

This procedure is used to send a H245 message to the IM-MGW that the IM-MGW shall forward towards the CS side within H.223.

Table E.4.3.4.1: Signal H245 Message Procedure

	Procedure
	Initiated
	Information element name
	Information element required
	Information element description

	Signal H245 Message
	MGCF
	Context
	M
	This information element indicates the existing context.

	
	
	Termination
	M
	This information element indicates the termination

	
	
	Signal
	M
	This information element indicates the signal to request forwarding of a H245 message towards the CS side within H.223

	
	
	H.245 message
	M
	This information element indicates the H.245 message to be forwarded

	Signal H245 Message Ack
	IM-MGW
	Context
	M
	This information element indicates the context where the command was executed.

	
	
	Termination
	M
	This information element indicates the termination where the command was executed.


E.4.3.5
Notify H245 Message

This procedure is used by the IM-MGW to notify the MGCF that the IM-MGW has received a H245 message from the CS side within H.223.

Table E.4.3.5.1: Notify H245 Message Procedure

	Procedure
	Initiated
	Information element name
	Information element required
	Information element description

	Notify H245 Message
	IM-MGW
	Context
	M
	This information element indicates the existing context.

	
	
	Termination
	M
	This information element indicates the termination

	
	
	Event
	M
	This information element indicates the event to indicate that a H245 message has been received from the CS side within H.223

	
	
	H.245 message
	M
	information element indicates the received H.245 message

	Notify H245 Message Ack
	MGCF
	Context
	M
	This information element indicates the context where the command was executed.

	
	
	Termination
	M
	This information element indicates the termination where the command was executed.


E.4.3.6
Notify MONA Preference Reception

This procedure is used by the IM-MGW to notify the MGCF that the IM-MGW has received the first MONA preference Message from the CS side.

Table E.4.3.6.1: Notify MONA Preference Reception Procedure

	Procedure
	Initiated
	Information element name
	Information element required
	Information element description

	Notify H245 Message
	IM-MGW
	Context
	M
	This information element indicates the existing context.

	
	
	Termination
	M
	This information element indicates the termination

	
	
	Event
	M
	This information element indicates the event to indicate that a MONA Preference Message has been received 

	
	
	MONA Preferences
	M
	This information element indicates the received MONA preference message

	Notify H245 Message Ack
	MGCF
	Context
	M
	This information element indicates the context where the command was executed.

	
	
	Termination
	M
	This information element indicates the termination where the command was executed.


E.4.3.7
Notify MONA Preference Completed

This procedure is used by the IM-MGW to notify the MGCF that the IM-MGW has received the first MONA preference Message with indication that the MONA preference negotiation is completed.

Table E.4.3.7.1: Notify MONA Preference Completed Procedure

	Procedure
	Initiated
	Information element name
	Information element required
	Information element description

	Notify H245 Message
	IM-MGW
	Context
	M
	This information element indicates the existing context.

	
	
	Termination
	M
	This information element indicates the termination

	
	
	Event
	M
	This information element indicates the event to indicate that the first MONA preference Message with indication that the MONA preference negotiation is completed received 

	Notify H245 Message Ack
	MGCF
	Context
	M
	This information element indicates the context where the command was executed.

	
	
	Termination
	M
	This information element indicates the termination where the command was executed.


E.4.3.8
Signal SPC

This procedure is used to send a H245 message to the IM-MGW that the IM-MGW shall forward towards the CS side using the SPC, i.e. as attachment to MONA preference messages.
Table E.4.3.8.1: Signal SPC Procedure

	Procedure
	Initiated
	Information element name
	Information element required
	Information element description

	Signal H245 Message
	MGCF
	Context
	M
	This information element indicates the existing context.

	
	
	Termination
	M
	This information element indicates the termination

	
	
	Signal
	M
	This information element indicates the signal to request forwarding of a H245 message towards the CS side using the SPC, i.e. as attachment to MONA preference messages

	
	
	H.245 message
	M
	This information element indicates the H.245 message to be forwarded

	Signal H245 Message Ack
	IM-MGW
	Context
	M
	This information element indicates the context where the command was executed.

	
	
	Termination
	M
	This information element indicates the termination where the command was executed.


E.4.3.9
Notify SPC

This procedure is used by the IM-MGW to notify the MGCF that the IM-MGW has received a H245 message from the CS side in SPC PDUs attached to MONA preference messages.

Table E.4.3.9.1: Notify SPC Procedure

	Procedure
	Initiated
	Information element name
	Information element required
	Information element description

	Notify H245 Message
	IM-MGW
	Context
	M
	This information element indicates the existing context.

	
	
	Termination
	M
	This information element indicates the termination

	
	
	Event
	M
	This information element indicates the event to indicate that a H245 message has been received from the CS side in SPC PDUs attached to MONA preference messages

	
	
	H.245 message
	M
	information element indicates the received H.245 message

	Notify H245 Message Ack
	MGCF
	Context
	M
	This information element indicates the context where the command was executed.

	
	
	Termination
	M
	This information element indicates the termination where the command was executed.


E.4.3.10
Notify MPC

This procedure is used by the IM-MGW to notify the MGCF that the IM-MGW has received the first MONA preference message with attached MPC from the CS side.

Table E.4.3.10.1: Notify MPC Procedure

	Procedure
	Initiated
	Information element name
	Information element required
	Information element description

	Notify H245 Message
	IM-MGW
	Context
	M
	This information element indicates the existing context.

	
	
	Termination
	M
	This information element indicates the termination

	
	
	Event
	M
	This information element indicates the event to indicate that the first MONA preference message with attached MPC has been received.

	
	
	MuxCode
	M
	information element that indicates the received Channel Type

	Notify H245 Message Ack
	MGCF
	Context
	M
	This information element indicates the context where the command was executed.

	
	
	Termination
	M
	This information element indicates the termination where the command was executed.


E.4.3.11
Notify Detection of Legacy Interworking

This procedure is used by the IM-MGW to notify the MGCF that the IM-MGW detects a legacy interworking condition that causes it to abort MONA procedures.

Table E.4.3.11.1: Notify Detection of Legacy Interworking Procedure

	Procedure
	Initiated
	Information element name
	Information element required
	Information element description

	Notify H245 Message
	IM-MGW
	Context
	M
	This information element indicates the existing context.

	
	
	Termination
	M
	This information element indicates the termination

	
	
	Event
	M
	This information element indicates the event to indicate that a legacy interworking condition has been detected.

	Notify H245 Message Ack
	MGCF
	Context
	M
	This information element indicates the context where the command was executed.

	
	
	Termination
	M
	This information element indicates the termination where the command was executed.


E.4.3.12
Request RTCP-Interworking

This procedure is used by the MGCF to request the IM-MGW to detect the RTCP feedback message on the quality of the media distribution requiring interworking.

Table E.4.3.12.1: Request RTCP-Interworking

	Procedure
	Initiated
	Information element name
	Information element required
	Information element description

	Request RTCP-Interworking
	MGCF
	Context
	M
	This information element indicates the existing context.

	
	
	Termination
	M
	This information element indicates an existing

	
	
	RTCPfilter
	M
	This information element provides one or several bit pattern(s) for bits 3-15 of an RTCP packet. Information from RTCP packets that match those patterns shall be notified to the MGCF.

	
	
	RTCP-Interworking
	M
	This information element requests a notification about information derived from the incoming RTCP packet types denoted by RTCPfilter.

	Request RTCP-Interworking Ack
	IM-MGW
	Context
	M
	This information element indicates the context where the command was executed.

	
	
	Termination
	M
	This information element indicates the new termination where the command was executed.


E.4.3.13
Notify RTCP-Interworking

This procedure is used by the IM-MGW to notify the MGCF the feedback information elements when the IM-MGW detects a RTCP feedback message on the quality of the media distribution requiring interworking.

Table E.4.3.13.1: Notify of RTCP-Interworking

	Procedure
	Initiated
	Information element name
	Information element required
	Information element description

	Notify RTCP-Interworking
	IM-MGW
	Context
	M
	This information element indicates the existing context.

	
	
	Termination
	M
	This information element indicates the termination

	
	
	UpdatePicture
	C
	This information element shall be included upon reception of a RTCP PLI message. It indicates the request of sending of full intra-pictures. 

	
	
	MaxBitRate
	C
	This information element shall be included upon reception of a RTCP TMMBR message. It shall contain the bandwidth as indicated in the RTCP TMMBR message excluding the overhead indicated in the RTCP TMMBR message.

	Notify RTCP-Interworking Ack
	MGCF
	Context
	M
	This information element indicates the context where the command was executed.

	
	
	Termination
	M
	This information element indicates the termination where the command was executed.


E.4.3.14
Signal-H.245-Interworking

This procedure is used by the MGCF to indicate the IM-MGW the feedback information elements when a H.245 feedback message on the quality of the media distribution requiring interworking is received.

Table E.4.3.14.1: Signal H.245-Interworking

	Procedure
	Initiated
	Information element name
	Information element required
	Information element description

	Signal H.245-Interworking
	MGCF
	Context
	M
	This information element indicates the existing context.

	
	
	Termination
	M
	This information element indicates the termination

	
	
	UpdatePicture
	C
	This information element indicates the request to send a RTCP PLI message.

	
	
	MaxBitRate
	C
	This information element indicates the request to send a RTCP TMBR message. It shall contain the bandwidth as indicated in the RTCP TMMBR message excluding the overhead indicated in the RTCP TMMBR message.

	Signal H.245-Interworking Ack
	IM-MGW
	Context
	M
	This information element indicates the context where the command was executed.

	
	
	Termination
	M
	This information element indicates the termination where the command was executed.


E.4.3.15
Stop MPC

This procedure is used by the MGC to indicate to the MG that it is no longer allowed to send MPC media encapsulated in MONA Preference messages and that it shall stop MPC detection.

Table E.4.3.15.1: Stop MPC

	Procedure
	Initiated
	 Information element 
name
	Information element required
	Information element description

	Stop MPC
	MGCF
	Context
	M
	This information element indicates the existing context.

	
	
	Termination
	M
	This information element indicates the termination

	
	
	Stop MPC Reception
	M
	This information element indicates that the reception of MPC encapsulated in MONA Preference Messages shall be stopped.

	
	
	Stop MPC Sending
	M
	This information element requests that the sending of the MPC shall be stopped.

	Stop MPC Ack
	IM-MGW
	Context
	M
	This information element indicates the context where the command was executed.

	
	
	Termination
	M
	This information element indicates the termination where the command was executed.


E.4.3.16
Stop SPC

This procedure is used by the MGC to indicate to the MG that it is no longer allowed to send MPC media encapsulated in MONA Preference messages and that it shall stop SPC detection.

Table E.4.3.16.1: Stop MPC

	Procedure
	Initiated
	 Information element 
name
	Information element required
	Information element description

	Stop SPC
	MGCF
	Context
	M
	This information element indicates the existing context.

	
	
	Termination
	M
	This information element indicates the termination

	
	
	Stop SPC Sending
	M
	This information element requests that the sending of SPC encapsulated in MONA Preference messages shall be stopped.

	
	
	Stop SPC Detection
	M
	This information element requests that the detection of received SPC encapsulated in MONA Preference Message shall be stopped.

	Stop SPC Ack
	IM-MGW
	Context
	M
	This information element indicates the context where the command was executed.

	
	
	Termination
	M
	This information element indicates the termination where the command was executed.


E.4.3.17
Stop MONA Negotiation

This procedure is used by the MGC to request the MG to stop all MONA nogotiation related procedures.

Table E.4.3.17.1: Stop MONA Negotiation

	Procedure
	Initiated
	 Information element 
name
	Information element required
	Information element description

	Stop MONA Negotiation
	MGCF
	Context
	M
	This information element indicates the existing context.

	
	
	Termination
	M
	This information element indicates the termination

	
	
	Stop MONA Preference Message Sending
	M
	This information element requests that the sending of MONA Preference messages be stopped.

	
	
	Stop MONA Detection
	M
	This information element requests that the detection of first MONA Preference Message and of the MONA negotiation completion be stopped.

	
	
	Stop SPC Sending
	C
	This information element requests that the sending of SPC encapsulated in MONA Preference messages be stopped.

Required only if SPC sending was previously configured.

	
	
	Stop SPC Detection
	C
	This information element requests that the detection of received SPC encapsulated in MONA Preference Message be stopped.

Required only if SPC reception was previously configured.

	
	
	Stop MPC Reception
	C
	This information element indicates that the reception of MPC encapsulated in MONA Preference Messages shall be stopped.

Required only if MPC reception was previously configured.

	
	
	Stop MPC Sending
	C
	This information element requests that the sending of the MPC shall be stopped.

Required only if MPC sending was previously configured.

	Stop MONA Negotiation Ack
	IM-MGW
	Context
	M
	This information element indicates the context where the command was executed.

	
	
	Termination
	M
	This information element indicates the termination where the command was executed.


Annex F (normative):
PSTN XML Scheme
F.1
Scope

This section defines the PSTN XML Schema to be used for providing the BearerCapability, Low Layer Compatibility, High Layer Compatibility and Progress indicator embedded as body in SIP messages.

The support of this PSTN XML Schema is a network option.

F.2
MIME type

The XML schema defined in the present Annex is registered at IANA as "application/vnd.etsi.pstn+xml" MIME type.

If the XML scheme is embedded in SIP messages as body, the Content-Type header shall be set to "application/vnd.etsi.pstn+xml" and the Content-Disposition shall be set to "signal" with the "handling" parameter set to "optional".

F.3
XML Schema definition

<?xml version="1.0" encoding="UTF-8"?>
<xs:schema xmlns:xs="http://www.w3.org/2001/XMLSchema" xmlns="http://uri.etsi.org/ngn/params/xml/simservs/pstn" xmlns:ns1="http://uri.etsi.org/ngn/params/xml/simservs/pstn" targetNamespace="http://uri.etsi.org/ngn/params/xml/simservs/pstn" elementFormDefault="qualified">

<xs:annotation>


<xs:documentation>XML Schema definition for mapping of some PSTN into SIP MIME Bodies</xs:documentation>

</xs:annotation>

<!--Definition of simple types-->

<xs:simpleType name="OneBitType">


<xs:restriction base="xs:string">



<xs:pattern value="[0-1]"/>


</xs:restriction>

</xs:simpleType>

<xs:simpleType name="TwoBitType">


<xs:restriction base="xs:string">



<xs:pattern value="[0-1][0-1]"/>


</xs:restriction>

</xs:simpleType>

<xs:simpleType name="ThreeBitType">


<xs:restriction base="xs:string">



<xs:pattern value="[0-1][0-1][0-1]"/>


</xs:restriction>

</xs:simpleType>

<xs:simpleType name="FourBitType">


<xs:restriction base="xs:string">



<xs:pattern value="[0-1][0-1][0-1][0-1]"/>


</xs:restriction>

</xs:simpleType>

<xs:simpleType name="FiveBitType">


<xs:restriction base="xs:string">



<xs:pattern value="[0-1][0-1][0-1][0-1][0-1]"/>


</xs:restriction>

</xs:simpleType>

<xs:simpleType name="SixBitType">


<xs:restriction base="xs:string">



<xs:pattern value="[0-1][0-1][0-1][0-1][0-1][0-1]"/>


</xs:restriction>

</xs:simpleType>

<xs:simpleType name="SevenBitType">


<xs:restriction base="xs:string">



<xs:pattern value="[0-1][0-1][0-1][0-1][0-1][0-1][0-1]"/>


</xs:restriction>

</xs:simpleType>

<!--Definition of complex types-->

<!--Definition of BearerCapability Octets-->

<xs:complexType name="BCOctet3Type">


<xs:sequence>



<xs:element name="CodingStandard" type="TwoBitType"/>



<xs:element name="InformationTransferCapability" type="FiveBitType"/>


</xs:sequence>

</xs:complexType>

<xs:complexType name="BCOctet4Type">


<xs:sequence>



<xs:element name="TransferMode" type="TwoBitType"/>



<xs:element name="InformationTransferRate" type="FiveBitType"/>


</xs:sequence>

</xs:complexType>

<xs:complexType name="BCOctet4-1Type">


<xs:sequence>



<xs:element name="RateMultiplier" type="SevenBitType"/>


</xs:sequence>

</xs:complexType>

<xs:complexType name="BCOctet5Type">


<xs:sequence>



<xs:element name="Layer1Identification" type="TwoBitType"/>



<xs:element name="UserInfoLayer1Protocol" type="FiveBitType"/>


</xs:sequence>

</xs:complexType>

<xs:complexType name="BCOctet5aType">


<xs:sequence>



<xs:element name="SynchronousAsynchronous" type="OneBitType"/>



<xs:element name="Negotiation" type="OneBitType"/>



<xs:element name="UserRate" type="FiveBitType"/>


</xs:sequence>

</xs:complexType>

<xs:complexType name="BCOctet5bV110Type">


<xs:sequence>



<xs:element name="IntermediateRate" type="TwoBitType"/>



<xs:element name="NIConTX" type="OneBitType"/>



<xs:element name="NIConRX" type="OneBitType"/>



<xs:element name="FlowControlOnTX" type="OneBitType"/>



<xs:element name="FlowControlOnRX" type="OneBitType"/>


</xs:sequence>

</xs:complexType>

<xs:complexType name="BCOctet5bV120Type">


<xs:sequence>



<xs:element name="RateAdaptionHeader" type="OneBitType"/>



<xs:element name="MultipleFrameEstablishmentSupport" type="OneBitType"/>



<xs:element name="ModeOfOperation" type="OneBitType"/>



<xs:element name="LogicalLinkIdentifier" type="OneBitType"/>



<xs:element name="Assignor" type="OneBitType"/>



<xs:element name="InbandOutbandNegotiation" type="OneBitType"/>


</xs:sequence>

</xs:complexType>

<xs:complexType name="BCOctet5cType">


<xs:sequence>



<xs:element name="NumberOfStopBits" type="TwoBitType"/>



<xs:element name="NumberOfDataBits" type="TwoBitType"/>



<xs:element name="Parity" type="ThreeBitType"/>


</xs:sequence>

</xs:complexType>

<xs:complexType name="BCOctet5dType">


<xs:sequence>



<xs:element name="DuplexMode" type="OneBitType"/>



<xs:element name="ModemType" type="SixBitType"/>


</xs:sequence>

</xs:complexType>

<xs:complexType name="BCOctet6Type">


<xs:sequence>



<xs:element name="Layer2Identification" type="TwoBitType"/>



<xs:element name="UserInfoLayer2Protocol" type="FiveBitType"/>


</xs:sequence>

</xs:complexType>

<xs:complexType name="BCOctet7Type">


<xs:sequence>



<xs:element name="Layer3Identification" type="TwoBitType"/>



<xs:element name="UserInfoLayer3Protocol" type="FiveBitType"/>


</xs:sequence>

</xs:complexType>

<xs:complexType name="BCOctet7aType">


<xs:sequence>



<xs:element name="AdditionalLayer3Info" type="FourBitType"/>


</xs:sequence>

</xs:complexType>

<xs:complexType name="BCOctet7bType">


<xs:sequence>



<xs:element name="AdditionalLayer3Info" type="FourBitType"/>


</xs:sequence>

</xs:complexType>

<!--Definition of High Layer Compatibility Octets-->

<xs:complexType name="HLOctet3Type">


<xs:sequence>



<xs:element name="CodingStandard" type="TwoBitType"/>



<xs:element name="Interpretation" type="ThreeBitType"/>



<xs:element name="PresentationMethod" type="TwoBitType"/>


</xs:sequence>

</xs:complexType>

<xs:complexType name="HLOctet4Type">


<xs:sequence>



<xs:element name="HighLayerCharacteristics" type="SevenBitType"/>


</xs:sequence>

</xs:complexType>

<xs:complexType name="HLOctet4aMaintenanceType">


<xs:sequence>



<xs:element name="HighLayerCharacteristics" type="SevenBitType"/>


</xs:sequence>

</xs:complexType>

<xs:complexType name="HLOctet4aAudioType">


<xs:sequence>



<xs:element name="VideoTelephonyCharacteristics" type="SevenBitType"/>


</xs:sequence>

</xs:complexType>

<!--Definition of Low Layer Compatibility Octets-->

<xs:complexType name="LLOctet3Type">


<xs:sequence>



<xs:element name="CodingStandard" type="TwoBitType"/>



<xs:element name="InformationTransferCapability" type="FiveBitType"/>


</xs:sequence>

</xs:complexType>

<xs:complexType name="LLOctet3aType">


<xs:sequence>



<xs:element name="NegotiationIndicator" type="OneBitType"/>


</xs:sequence>

</xs:complexType>

<xs:complexType name="LLOctet4Type">


<xs:sequence>



<xs:element name="TransferMode" type="TwoBitType"/>



<xs:element name="InformationTransferRate" type="FiveBitType"/>


</xs:sequence>

</xs:complexType>

<xs:complexType name="LLOctet4-1Type">


<xs:sequence>



<xs:element name="RateMultiplier" type="SevenBitType"/>


</xs:sequence>

</xs:complexType>

<xs:complexType name="LLOctet5Type">


<xs:sequence>



<xs:element name="Layer1Identification" type="TwoBitType"/>



<xs:element name="UserInfoLayer1Protocol" type="FiveBitType"/>


</xs:sequence>

</xs:complexType>

<xs:complexType name="LLOctet5aType">


<xs:sequence>



<xs:element name="SynchronousAsynchronous" type="OneBitType"/>



<xs:element name="Negotiation" type="OneBitType"/>



<xs:element name="UserRate" type="FiveBitType"/>


</xs:sequence>

</xs:complexType>

<xs:complexType name="LLOctet5bV110Type">


<xs:sequence>



<xs:element name="IntermediateRate" type="TwoBitType"/>



<xs:element name="NIConTX" type="OneBitType"/>



<xs:element name="NIConRX" type="OneBitType"/>



<xs:element name="FlowControlOnTX" type="OneBitType"/>



<xs:element name="FlowControlOnRX" type="OneBitType"/>


</xs:sequence>

</xs:complexType>

<xs:complexType name="LLOctet5bV120Type">


<xs:sequence>



<xs:element name="RateAdaptionHeader" type="OneBitType"/>



<xs:element name="MultipleFrameEstablishmentSupport" type="OneBitType"/>



<xs:element name="ModeOfOperation" type="OneBitType"/>



<xs:element name="LogicalLinkIdentifier" type="OneBitType"/>



<xs:element name="Assignor" type="OneBitType"/>



<xs:element name="InbandOutbandNegotiation" type="OneBitType"/>


</xs:sequence>

</xs:complexType>

<xs:complexType name="LLOctet5cType">


<xs:sequence>



<xs:element name="NumberOfStopBits" type="TwoBitType"/>



<xs:element name="NumberOfDataBits" type="TwoBitType"/>



<xs:element name="Parity" type="ThreeBitType"/>


</xs:sequence>

</xs:complexType>

<xs:complexType name="LLOctet5dType">


<xs:sequence>



<xs:element name="DuplexMode" type="OneBitType"/>



<xs:element name="ModemType" type="SixBitType"/>


</xs:sequence>

</xs:complexType>

<xs:complexType name="LLOctet6Type">


<xs:sequence>



<xs:element name="Layer2Identification" type="TwoBitType"/>



<xs:element name="UserInfoLayer2Protocol" type="FiveBitType"/>


</xs:sequence>

</xs:complexType>

<xs:complexType name="LLOctet6aHDLCType">


<xs:sequence>



<xs:element name="Mode" type="TwoBitType"/>


</xs:sequence>

</xs:complexType>

<xs:complexType name="LLOctet6aUserSpecificType">


<xs:sequence>



<xs:element name="UserSpecificLayer2Information" type="SevenBitType"/>


</xs:sequence>

</xs:complexType>

<xs:complexType name="LLOctet6bType">


<xs:sequence>



<xs:element name="WindowSize" type="SevenBitType"/>


</xs:sequence>

</xs:complexType>

<xs:complexType name="LLOctet7Type">


<xs:sequence>



<xs:element name="Layer3Identification" type="TwoBitType"/>



<xs:element name="UserInfoLayer3Protocol" type="FiveBitType"/>


</xs:sequence>

</xs:complexType>

<xs:complexType name="LLOctet7aUserSpecificType">


<xs:sequence>



<xs:element name="OptionalLayer3Information" type="SevenBitType"/>


</xs:sequence>

</xs:complexType>

<xs:complexType name="LLOctet7aX25Type">


<xs:sequence>



<xs:element name="Mode" type="TwoBitType"/>


</xs:sequence>

</xs:complexType>

<xs:complexType name="LLOctet7bX25Type">


<xs:sequence>



<xs:element name="DefaultPacketSize" type="FourBitType"/>


</xs:sequence>

</xs:complexType>

<xs:complexType name="LLOctet7cType">


<xs:sequence>



<xs:element name="PacketWindowSize" type="SevenBitType"/>


</xs:sequence>

</xs:complexType>

<xs:complexType name="LLOctet7aTR9577Type">


<xs:sequence>



<xs:element name="AdditionalLayer3Info" type="FourBitType"/>


</xs:sequence>

</xs:complexType>

<xs:complexType name="LLOctet7bTR9577Type">


<xs:sequence>



<xs:element name="AdditionalLayer3Info" type="FourBitType"/>


</xs:sequence>

</xs:complexType>

<xs:complexType name="DispOctet3Type">


<xs:sequence>



<xs:element name="DisplayInformation" type="SevenBitType"/>


</xs:sequence>

</xs:complexType>

<!--Definition of the information elements-->

<xs:complexType name="BearerCapabilityType">


<xs:sequence>



<xs:element name="BCoctet3" type="BCOctet3Type"/>



<xs:element name="BCoctet4" type="BCOctet4Type"/>



<xs:element name="BCoctet4-1" type="BCOctet4-1Type" minOccurs="0"/>



<xs:element name="BCoctet5" type="BCOctet5Type" minOccurs="0"/>



<xs:element name="BCoctet5a" type="BCOctet5aType" minOccurs="0"/>



<xs:element name="BCoctet5bV110" type="BCOctet5bV110Type" minOccurs="0"/>



<xs:element name="BCoctet5bV120" type="BCOctet5bV120Type" minOccurs="0"/>



<xs:element name="BCoctet5c" type="BCOctet5cType" minOccurs="0"/>



<xs:element name="BCoctet5d" type="BCOctet5dType" minOccurs="0"/>



<xs:element name="BCoctet6" type="BCOctet6Type" minOccurs="0"/>



<xs:element name="BCoctet7" type="BCOctet7Type" minOccurs="0"/>



<xs:element name="BCoctet7a" type="BCOctet7aType" minOccurs="0"/>



<xs:element name="BCoctet7b" type="BCOctet7bType" minOccurs="0"/>


</xs:sequence>

</xs:complexType>

<xs:complexType name="HighLayerCompatibilityType">


<xs:sequence>



<xs:element name="HLOctet3" type="HLOctet3Type"/>



<xs:element name="HLOctet4" type="HLOctet4Type"/>



<xs:element name="HLOctet4aMaintenance" type="HLOctet4aMaintenanceType" minOccurs="0"/>



<xs:element name="HLOctet4Audio" type="HLOctet4aAudioType" minOccurs="0"/>


</xs:sequence>

</xs:complexType>

<xs:complexType name="LowLayerCompatibilityType">


<xs:sequence>



<xs:element name="LLOctet3" type="LLOctet3Type"/>



<xs:element name="LLOctet3a" type="LLOctet3aType" minOccurs="0"/>



<xs:element name="LLOctet4" type="LLOctet4Type"/>



<xs:element name="LLOctet4-1" type="LLOctet4-1Type" minOccurs="0"/>



<xs:element name="LLOctet5" type="LLOctet5Type" minOccurs="0"/>



<xs:element name="LLOctet5a" type="LLOctet5aType" minOccurs="0"/>



<xs:element name="LLOctet5bV110" type="LLOctet5bV110Type" minOccurs="0"/>



<xs:element name="LLOctet5bV120" type="LLOctet5bV120Type" minOccurs="0"/>



<xs:element name="LLOctet5c" type="LLOctet5cType" minOccurs="0"/>



<xs:element name="LLOctet5d" type="LLOctet5dType" minOccurs="0"/>



<xs:element name="LLOctet6" type="LLOctet6Type" minOccurs="0"/>



<xs:element name="LLOctet6aHDLC" type="LLOctet6aHDLCType" minOccurs="0"/>



<xs:element name="LLOctet6aUserSpecific" type="LLOctet6aUserSpecificType" minOccurs="0"/>



<xs:element name="LLOctet6b" type="LLOctet6bType" minOccurs="0"/>



<xs:element name="LLOctet7" type="LLOctet7Type" minOccurs="0"/>



<xs:element name="LLOctet7aUserSpecific" type="LLOctet7aUserSpecificType" minOccurs="0"/>



<xs:element name="LLOctet7aX25" type="LLOctet7aX25Type" minOccurs="0"/>



<xs:element name="LLOctet7bX25" type="LLOctet7bX25Type" minOccurs="0"/>



<xs:element name="LLOctet7c" type="LLOctet7cType" minOccurs="0"/>



<xs:element name="LLOctet7aTR9577" type="LLOctet7aTR9577Type" minOccurs="0"/>



<xs:element name="LLOctet7bTR9577" type="LLOctet7bTR9577Type" minOccurs="0"/>


</xs:sequence>

</xs:complexType>

<xs:complexType name="DisplayType">


<xs:sequence>



<xs:element name="DispOctet3" type="DispOctet3Type"/>


</xs:sequence>

</xs:complexType>

<!--Definition of progress indicator-->

<xs:complexType name="ProgressOctet3Type">


<xs:sequence>



<xs:element name="CodingStandard" type="TwoBitType"/>



<xs:element name="Location" type="FourBitType "/>


</xs:sequence>

</xs:complexType>

<xs:complexType name="ProgressOctet4Type">


<xs:sequence>



<xs:element name="ProgressDescription" type="SevenBitType"/>


</xs:sequence>

</xs:complexType>

<xs:complexType name="ProgressIndicatorType">


<xs:sequence>



<xs:element name="ProgressOctet3" type="ProgressOctet3Type"/>



<xs:element name="ProgressOctet4" type="ProgressOctet4Type"/>


</xs:sequence>

</xs:complexType>

<!--Definition of document structure-->

<xs:element name="PSTN">


<xs:complexType>



<xs:sequence>




<xs:element name="BearerCapability" type="BearerCapabilityType" minOccurs="0" maxOccurs="2"/>




<xs:element name="HighLayerCompatibility" type="HighLayerCompatibilityType" minOccurs="0" maxOccurs="2"/>




<xs:element name="LowLayerCompatibility" type="LowLayerCompatibilityType" minOccurs="0"/>




<xs:element name="ProgressIndicator" type="ProgressIndicatorType" minOccurs="0" maxOccurs="unbounded"/>




<xs:element name="Display" type="DisplayType" minOccurs="0" maxOccurs="unbounded"/>




<xs:element name="sendingCompleteIndication" minOccurs="0"/>



</xs:sequence>


</xs:complexType>

</xs:element>
</xs:schema>
F.4
IANA registration template

Within the present subclause, information required for an IANA registration at http://www.iana.org/cgi-bin/mediatypes.pl is provided.

1. Media Type Name

Application

2. Subtype name

"vnd.etsi.pstn+xml" (Vendor Tree)

3. Required parameters

none

4. Optional parameters

none

5. Encoding considerations

binary

6. Security considerations

The security considerations for XML in RFC 3023 apply.

Modifications of the MIME body by a man-in-the-middle can have severe consequences:

The information derived from ISUP that can be transported with this MIME body influence the call handling of the SIP session that is being setup.

However, this MIME body is used only attached to SIP messages, and modifications of other parts of the SIP signalling will lead to comparable consequences. Protection of the SIP signalling will also protect the present MIME body. The SIP signalling should be protected as described in RFC 3261.

7. Interoperability considerations

None

8. Published specification

TS 29.163

9. Applications which use this media type

This MIME type is used as a message body within SIP messages.

It is used for sending information derived from ISUP or DSS1 signalling with no SIP equivalent.

10. Additional information

Magic number(s): none (see RFC 3023)

File extension(s): ".xml" (see RFC 3023)

Macintosh File Type Code(s): "Text" (see RFC 3023)

Object Identifier(s) or OID(s): none

11. Intended usage

Limited Use

12. Other Information/General Comment

The content of this MIME type is UTF-8 encoded. The charset parameter is not used.
Annex G (informative):
Void
NOTE:
The former contents of this annex are now contained in TS 24.229 [9].
Annex H (normative):
Interworking of Originating Line Information (OLI) parameter (network option)

H.1
Interworking SIP to ISUP

The "oli" URI parameter received within tel URI or the userinfo part of SIP URI with user="phone" (as defined in clause 7.2A.12 of TS 24.229 [9]) received in a P-Asserted-Identity header in the initial INVITE request shall be used to set the ISUP IAM OLI parameter. In case the P-Asserted-Identity URI "oli" parameter is absent then the ISUP IAM OLI parameter shall be omitted.

H.2
Interworking ISUP to SIP

The ISUP IAM OLI parameter shall be used to set the "oli" URI parameter within tel URI or the userinfo part of SIP URI with user="phone" parameter (as defined in clause 7.2A.12 of TS 24.229 [9]) of a P-Asserted-Identity header in the initial INVITE request. In case the ISUP IAM OLI parameter is absent then the P-Asserted-Identity URI "oli" parameter shall be omitted from the initial INVITE request.
Annex I (normative):
GTT interworking between the IP Multimedia Core Network (CN) Subsystem (IMS) and Circuit Switched (CS) networks

I.1
Overview of GTT interworking between the IMS and Circuit Switched (CS) networks

The support of Global Text Telephony (GTT) is optional, but may be required by national regulatory requirements. If GTT is supported, the procedures described within this Annex shall be applied.

Global Text Telephony (GTT) offers real time conversation in text, optionally combined with voice. GTT is mainly used for distant conversation with hearing or speech impaired users.

GTT is supported in IMS via the Real-Time Text protocol over RTP, using IETF SIP/SDP for the negotiation of the text media and IETF RFC 4103 [124] RTP-text for transport, with text coded according to ITU‑T Recommendation T.140 [123]. See TS 23.226 [122], TS 26.114 [104], TS 26.235 [80] and TS 26.236 [32].

In PSTN, different specified systems for text telephony exist and are used in different regions, e.g. Baudot (in US), EDT, V.21, Bell103, Minitel and V.18. They all use different modem technologies within PCM and different character coding for the transmission of text. They are described in the annexes of ITU-T Recommendation V.18 [125]. ITU-T Recommendation V.18 [125] is an international text telephone modem standard with an automoding mechanism that enables communication with all the different kinds of PSTN text telephone systems. Any party of a GTT call may at any time initiate text or send voice. Speech and text may be used in an alternating manner during a conversation on the PSTN. It is also possible that speech is transferred in one direction and text in the opposite direction. However, speech and text can not be used in the same direction at the same time in most sub-modes of V.18.

In the 3GPP CS radio interface, a dedicated CTM modem is used (see TS 26.226 [126]), which is terminated within the CS domain and interworked to PTSN inband text telephony format.

Interworking between Real-Time Text over RTP within IMS and PSTN text telephony is provided at the MGCF / IM-MGW by the MGCF triggering the insertion of an Interworking (conversion) function within the IM-MGW which then behaves in accordance with ITU‑T Recommendation V.18 [125] or any of its specific sub-modes.

The support of this Interworking function between IP-based Real-Time Text over RTP and modem based transmission of real-time text is optional both at the MGCF and IM-MGW, but can be required by national regulatory bodies. If this Interworking function is supported, the procedures described within this Annex shall be applied.

The Interworking function in the IM-MGW shall support the detection of text modem signals on the CS side and the conversion between text/modem signals and Real-Time Text over RTP.

The IM_MGW shall support at least one of the sub-modes listed in ITU-T Recommendation V.18 [125] (e.g. Baudot). The support of any of the sub-modes is optional.

The procedures to detect and convert text/modem involve expensive MGW resources. The present GTT interworking procedures intend to allow cost effective implementations by avoiding additional load or resources in MGW for calls not using text telephony (which represent most of the calls).

It is assumed that IMS terminals supporting text media will not automatically offer text media, but that this will be instead governed by terminal configuration options and user interactions to suit the communication preferences and abilities of the user. However, an IMS terminal desiring to set up a GTT call will offer Real-Time Text media, possibly in parallel to voice media. The IMS-MGW shall then provide the conversion between Real-Time Text over RTP and text/modem signals. On the contrary, if the mobile does not request Real-Time Text support, no Interworking function is necessary. An IMS Multimedia terminal configured to use Real-Time Text Telephony but receiving an SDP offer for voice-only media will accept this offer and then send an own subsequent SDP offer adding text media. When receiving such a subsequent offer for text media, the IMS-MGW shall provide the conversion between Real-Time Text over RTP and text/modem signals at the CS interface. On the contrary, if the mobile does not offer Real-Time Text, no Interworking function is necessary.

I.2
Control plane interworking

I.2.1
General

Before text conversation can begin, a call shall first be established, from PSTN to IMS or vice-versa. The ISUP or BICC signalling to/from the PSTN indicates a "speech" call, without any GTT indication.

The IMS user may request a text connection from the beginning of a call, or add request for Real-Time Text media at a later stage in a call that was originally established with audio only.

I.2.2
Functionalities required in the MGCF for GTT calls support

In addition to the control plane interworking between SIP and ISUP or BICC (see clause 7), the MGCF shall support the negotiation of the Real-Time Text payload type (T.140 Text Conversation MIME media type as specified by IETF RFC 4103 [124]) in a distinct SDP m-line.

I.2.3
IM CN subsystem originated session

I.2.3.1
Initial INVITE with an SDP offer including a text media line

Figure I.2.3.1.1 shows an example call flow where the IMS terminal requests Real-Time Text telephony by sending an SDP offer including one audio line and one text media line within an initial INVITE message.


[image: image121.emf]Interworking Node

 (MGCF )

1. SIP: INVITE 

[SDP offer: Audio + Text]

2.  IAM (speech)

CS Network

IMS

3. SIP: 18x or 200 OK

[SDP answer: Audio + Text]

Audio / Text


Figure I.2.3.1.1: IM CN subsystem originated session - Initial INVITE offering audio and text

Upon receipt of a SIP INVITE request offering text media (possibly combined with audio media) (signal 1 in figure I.2.3.1.1) the Interworking Node (MGCF) starts the call set-up at the CS side by sending an IAM requesting a speech with G.711 codec only or 3.1 Khz bearer (signal 2 in figure I.2.3.1.1), and completes the call setup on IMS and CS side using the procedures specified in clause 7, but returning an SDP answer including Real-Time Text media (possibly combined with audio media if audio media has been offered) (signal 3 in figure I.2.3.1.1).

The MGCF triggers the insertion an Interworking function in the IM-MGW for the duration of the call if a Real-Time Text media stream is established.

The MGCF reserves corresponding Real-Time Text media resources in the IM-MGW and activate the Interworking function, and if resources were available, return an SDP answer with audio and Real-Time Text.

I.2.4
CS network originated session

I.2.4.1
General

When starting the session setup signalling from a CS based network towards the IMS, the MGCF has no knowledge whether the CS side terminal supports and will accept to use text telephony.

I.2.4.2
Initial INVITE with an SDP offer including audio only

The MGCF offers only audio media when setting up a call towards the IMS and waits for IMS terminals desiring Real-Time Text media to send a new offer adding Real-Time Text media attribute prior to inserting an Interworking function in the IM-MGW. If the MGCF receives a new offer adding Real-Time Text media and applies signalling with OoBTC on the CS CN, the MGCF shall apply appropriate OoBTC procedures to only select G.711 audio codec on the CS call leg.

Figure I.2.4.2.1 shows an example call flow.


[image: image122.emf]Interworking Node

 (MGCF)

2. SIP: INVITE 

[SDP offer: Audio]

12. SIP: 200OK (INVITE)

1.  IAM (speech)

Bearer Establishment

IMS

CS Network

9. ACM

13. ANM

8. SIP: 180 Ringing

3. SIP: 183 Session Progress

[SDP answer: Audio]

6. SIP: UPDATE [SDP offer: Audio + Text]

7. SIP: 200 UPDATE

[SDP answer: Audio + Text]

Audio / Text

4. SIP: PRACK

5. SIP: 200 OK (PRACK)

10. SIP: PRACK

11. SIP: 200 OK (PRACK)


Figure I.2.4.2.1: CS originated session - Initial INVITE offering audio only

Upon receipt of an IAM request for a speech or 3.1 Khz audio call (signal 1 in figure I.2.4.2.1) the Interworking Node (MGCF) starts the call set-up at the IM CN subsystem side by sending an INVITE request (signal 2 in figure I.2.4.2.1) offering audio media applying the interworking procedures in clause 7.

IMS terminals supporting GTT and configured to use it will send a new SDP offer including an audio and a Real-Time Text media line within a subsequent UPDATE or re-INVITE request.

When GTT interworking between IMS and CS networks is required, the MGCF shall reserve corresponding Real-Time Text media resources in the IM-MGW and thereby request the insertion of the Interworking function, and if resources were available, return an SDP answer with audio and Real-Time Text media attributes.

I.2.5
Subsequent SDP offer/answer exchange adding text to an audio session

If only audio media has been offered in the initial SDP offer, the IMS terminal can also request GTT support by sending a new SDP offer including audio and Real-Time Text when a SIP dialogue (early or confirmed) has already been established.

The MGCF shall then trigger the insertion of an Interworking function in the IM-MGW following the same principles as those specified in clause I.2.3.14. However, if OoBTC signalling is supported and a codec other than PCM (default G.711) has been selected at the CS network, then a codec modification is required to G.711. Otherwise No additional ISUP or BICC signalling towards the CS network is required.

I.3
User plane interworking

I.3.1
Functionalities required in the IM-MGW for GTT support

An IM-MGW supporting GTT interworking between IMS and CS network shall support:

-
Real-Time Text as specified in TS 26.235 [80], TS 26.236 [32] and TS 26.114 [104] for a Multimedia Telephony Service Indicator (MTSI) compliant IM-MGW; this includes:

-
requirement to support ITU-T Recommendation T.140 [123] Text Conversation MIME media type and RTP-text transport as specified by IETF RFC 4103 [124] and ITU-T Recommendation T.140 [123] Text Conversation presentation coding;

-
recommended support of redundancy coding variant specified in IETF RFC 4103 [124] for error resilience;

-
PSTN based text telephony using ITU‑T Recommendation V.18 [4] or any of its specific sub-modes.

NOTE:
One specific text telephone protocol of the ITU-T Recommendation V.18 standard, called "Baudot Code" (Baudot modulation at 45.45 baud), is particularly important for support of emergency calls in US.

I.3.2
Monitoring of text/modem signals on the CS side

When the IMS session is setup with text media (possibly combined with audio media), the MGCF thereby requests the IM-MGW to monitor the CS termination for possible receipt of text telephone signals, in order to detect whether the CS user wishes to transit from voice to text during a voice call. The IM-MGW shall be provisioned with the text telephone mode(s) for which the termination should be monitored. The presence of the text media in the IMS session is an explicit indication that monitoring of text telephone signals is required.

When monitoring for text telephony signals the IM-MGW should listen on the CS side voice line for text telephone signals, and shall not transmit modem tones from the CS side termination to IMS side termination until one of the following occurs:

1)
If the PSTN terminal is a carrier-based device and transmits answer tone. The IM-MGW shall then resolve the text telephone mode according to V.18 procedures and then operate in that mode;

2)
If the PSTN terminal is a carrier-less device, the end user starts sending characters such that these may be detected by the IM-MGW. The IM-MGW shall then detect the text telephone mode and then operate in that mode.

If the IMS terminal transmits text characters first the IM-MGW shall initiate a text connection toward the CS network and resolve the textphone type, so that communication can continue in text mode.

See ITU-T Recommendation V.18 [125] for a detailed description of those procedures.
I.3.3
Multiplexing between the CS and IMS streams

The IM-MGW should remain in speech mode until such time that text is detected from either user. The IM-MGW shall detect modem signals on the CS termination and decide based on this detection if the media received from the CS side is interworked to the speech or the text media stream on the IMS side. In the media direction towards the CS network, the IM-MGW needs to multiplex both media streams into the PCM signal.

The following procedures provide further information on the procedures to be followed by the IM-MGW, for normative behaviour the ITU-T Recommendation V.18 [125] shall be followed.

-
Carrier text phone:
-
After a carrier mode text connection is established, loss of carrier can be taken as the indication that the audio stream on the IMS termination shall be connected with the (PCM) interface of the line on the CS termination.

-
When the text carrier reappears on the CS termination, or text is received from the text media stream on the IMS termination, the IM-MGW connects the text stream on the IMS termination with the (PCM) interface of the line on the CS termination.
-
Carrierless text phone:
-
When the V.18 modem detects text, the textphone CS termination stops feeding the audio stream of the IMS termination, and instead inserts the detected and T.140 converted characters into the text stream of the IMS termination. This mode is continued as long as characters keep coming from the PSTN textphone. When no more characters arrive, and no textphone signal is received within 1 second, the audio channel is again fed to the audio stream of the IMS termination. If new text comes from the V.18 side, the process is repeated.

-
If text is received from the text stream of the IMS termination when V.18 is not actively receiving text, no media shall be applied to the IMS audio stream, and the characters are sent to the V.18 modem for transmission. When all text is transmitted and no more is received for two seconds, the audio channels are enabled again. Since the carrierless systems are one-way alternate transmission systems, transmission of characters is possible only in one direction at a time. Once started, reception is given priority. Characters received from text stream of the IMS termination while V.18 is busy receiving should be buffered (up to a reasonable limit).

I.3.4
Conversion between text/modem and Real-Time Text over RTP

The legacy PSTN textphone modes have limited character sets. For all legacy modes, the text received through the V.18 modem shall be converted if necessary in the RTP/T.140 format for the text stream on the IMS termination, according to the rules in ITU-T Recommendation. T.140 [123] and IETF RFC 4103 [124], and vice-versa.

Redundancy may be used as specified in IETF RFC 4103 [124]. Use or not of redundancy is derived from the text media description configured on the IMS termination (RTP/RED/T.140).

I.4
MGCF and IM-MGW interactions

I.4.1
Introduction

This clause describes requirements for extensions to the Mn interface protocol in TS 29.332 [15] needed to support the Interworking of Real-Time Text Telephony media in IMS with text/modem calls in CS domain.
I.4.2
Mn signalling interactions

I.4.2.1
Introduction

The following clauses describe the Mn interface procedures.

All message sequence charts in these clauses are examples.

I.4.2.2
H.248 Context Model

The H.248 context model depicted in figure I.4.2.2.1 shall be applied for interworking Real-Time Text media in IMS to text/modem in CS.

[image: image123.emf] 

CS network  

Context C1  

Termination T2  

Real - Time  T ext /RTP  stream  

Termination T1  

A udio /RTP   stream  

IMS  

2 - way topology    (audio + V.18) stream  


Figure I.4.2.2.1: H.248 Context Model for GTT support

The H.248 context contains one IP termination (IMS side) with two streams (one stream for Real-Time Text/RTP, another stream for audio/RTP), plus a TDM, AAL2 or IP termination (CS side) with one single stream carrying both voice via PCM and V.18 text telephony.

I.4.2.3
Specific Mn signalling for GTT

The following information shall be provided from the MGCF towards the IM-MGW:

-
SDP with audio and Real-Time Text m-lines for the IMS termination;

-
PCM (G.711) codec for the CS side termination.

This shall trigger the IM-MGW to insert an IWF to support RTT to PSTN text telephony conversion. If the topology is incoming (or bothway) from the CS network the IM-MGW shall monitor for text telephony and convert to RTT towards the IMS network. If the topology is outgoing (or bothway) to the CS network the IM-MGW shall multiplex RTT payload into the PCM stream.

I.4.2.4
IM CN subsystem originated session between the MGCF and IM-MGW

I.4.2.4.1
Initial INVITE with an SDP offer including Real-Time Text


[image: image124.emf]MGCF IM-MGW

1. SIP: INVITE 

[SDP offer: audio + Real-

Time Text]

2. SIP: 100 Trying

7. ISUP: IAM 

5. H.248: ADD.req [Context ID = C1, Termination ID = T2]

6. H.248: ADD.resp [Context ID = C1, Termination ID = T2] 

3. H.248: ADD.req [Context ID = ?, Termination ID=?, 

stream=1{audio media},stream=2{Real-Time Text media}]

4. H.248: ADD.resp [Context ID = C1, Termination ID = T1]

CALL IN ACTIVE STATE

Reserve IMS Connection

Point, and Configure

Remote Resources

Change IMSThrough-

Connection = backward 

Reserve TDM Circuit,

Change  TDM Through-

Connection= both, 

Monitor Text Call

Next steps of basic  call establishment (as specified in subclause 9.2.2.3), returning an SDP Answer with audio + text

9. H.248:  MOD.req  [Context ID=C1, Termination ID = T1]

10. H.248: MOD.resp [Context ID = C1, Termination ID = T1] 

8.ISUP: ANM 

13. SIP:200 OK (INVITE)

14. SIP: ACK

11. H.248:  MOD.req ) [Context ID=C1, Termination ID = T2]

12. H.248: MOD.resp [Context ID = C1, Termination ID = T2] 

Change IMS Through

-Connection = both

Activate TDM Voice 

Prosessing Function

15. Text/modem 

16. Text/RTP

CS Network

IMS


Figure I.4.2.4.1.1: Example Mn signalling interactions for IMS originated session with audio and text

The Mn signalling interactions specified in clause 9.2.2.3 shall apply, with the following differences.

When reserving the IMS termination, the MGCF shall configure two streams; one for the audio and one for the Real-Time Text media line, with their respective media description (signal 3 in figure I.4.2.4.1.1). Monitoring of text telephony at the CS side termination is indicated by the inclusion of Real-Time Text media at the IMS side termination and by the topology being bothway or incoming from the CS side.

The text telephone mode(s) for which the CS side termination is monitored shall be provisioned in the IM-MGW.

I.4.2.5
CS network originated session

I.4.2.5.1
Initial INVITE with an SDP offer only including audio


[image: image125.emf]IM-MGW MGCF

1. ISUP: IAM 

2. H.248: ADD.req [Context ID = ?, Termination ID = ?]

3. H.248: ADD.resp [Context ID = C1, Termination ID = T2] 

4. H.248: ADD.req [Context ID = C1, Termination ID=?, 

stream=1{audio media}]

5. H.248: ADD.resp [Context ID = C1, Termination ID = T1]

13. H.248: MOD.req [Context ID = C1,Termination ID = T1, 

stream=1{audio media},stream=2{Real-Time Text 

media}]

14.H.248: MOD.resp [Context ID = C1, Termination ID = T1 ] 

Reserve IMS Connection Point, 

Change IMS Through-

Connection  = backward 

Configure IMS  Resources

CS Network IMS

Reserve TDM Circuit,

Change  TDM Through-

Connection= both

Monitor Text Call

Next steps of basic  call establishment (as specified in subclause 9.2.3.3)

CALL IN ACTIVE STATE

16. Text/modem 

17. Real-Time Text/RTP

6. SIP: INVITE 

[SDP offer: Audio]

7. SIP: 183 Session Progress

[SDP answer: Audio]

12. SIP: UPDATE [SDP offer: Audio + Real-

Time Text]

15. SIP: 200 UPDATE

[SDP answer: Audio + Real-Time Text]

10. SIP: PRACK

11. SIP: 200 OK (PRACK)

8. H.248: MOD.req [Context ID = C1,Termination ID = T1]

9.H.248: MOD.resp [Context ID = C1, Termination ID = T1 ] 

Configure IMS  Resources


Figure I.4.2.5.1.1: Mn signalling interactions for CS originated session – Initial INVITE with audio only

The Mn signalling interactions specified in clause 9.2.3.3 shall apply, with the following differences.

Upon receipt of a subsequent SDP offer from the IMS terminal including the audio and Real-Time Text media lines, the MGCF shall add an additional text stream to the IMS termination and configure it with the remote media description (signal 13 in figure I.4.2.5.1.1).
I.4.3
Mn Signalling procedures

I.4.3.1
Overview

This clause describes the logical signalling procedures (i.e. message identifiers are not part of the protocol) between the MGCF and IM-MGW. The procedures within this clause are intended to be implemented using the standard H.248 procedure as defined in ITU recommendation H.248.1 [2] with appropriate parameter combinations.

The procedures "Reserve IMS connection point" and "Configure IMS resources" and "Reserve IMS Connection point and configure remote resources" shall allow the configuration of two media streams; one for audio and one for Real-Time Text on the IMS termination. See clause 9.3.1.

Annex J (informative):
Call Flow for Customized Alerting Tone (CAT) interworking between CS network and IMS

J.1
Introduction

This annex describes procedures for interworking Customized Alerting Tone (CAT) between the IMS and CS Networks.

J.2
IMS CAT provided by the terminating IMS user using gateway model towards user served in CS networks

Callflows for providing IMS CAT provided by the terminating IMS user towards user served in CS networks are based on procedures specified in TS 24.182 [xx], with procedures for MGCF as shown in the following clauses.

J.2.1
Callflow without using precondition mechanism

The following callflow shows the case when precondition mechanism is not used, and 100rel extension is used.

[image: image126.emf]MGCF

1. IAM

2. INVITE (SDP_O)

3. 183 (Session Progress) 

(SDP_A_cat)

4. ACM

5. PRACK

6. 200 (OK) (PRACK)

7. UPDATE

(SDP_O_Regular)

8. 200 (OK) (UPDATE)

(SDP_A)

10. ANM

9. 200 (OK) (INVITE)

11. ACK


Figure J.2.1: CAT gateway model callflow without using precondition mechanism

The content of the SDP message body described in figure J.2.1 is meant to be as follows:

-
SDP_O in step 2 indicates the SDP offer inserted by the MGCF;

-
SDP_A_cat in step 3 indicates the SDP answer received by the MGCF, which in general is the SDP answer generated by the CAT AS based on the content for CAT media;

-
SDP_O_Regular in step 7 indicates the SDP offer received by the MGCF, which in general is the SDP offer sent by the CAT AS based on the SDP answer received by the terminating UE; and

-
SDP_A in step 8 indicates the SDP answer generated by the MGCF.

Upon receiving IAM (see figure J.2.1 step 1), MGCF sends SIP INVITE request containing the SDP offer to terminating IMS (see figure J.2.1 step 2).

Upon receiving 183 (Session Progress) response to the SIP INVITE request containing SDP answer (see figure J.2.1 step 3), MGCF sends ACM to the originating CS network (see figure J.2.1 step 4). Also, MGCF sends a PRACK request towards terminating IMS (see figure J.2.1 step 5). A 200 (OK) response will be received later as a result of the PRACK request (see figure J.2.1 step 6).

Upon receiving SIP UPDATE request from terminating IMS with a SDP offer (see figure J.2.1 step 7), MGCF sends a 200 (OK) response to the terminating IMS with an SDP answer (see figure J.2.1 step 8).

Upon receiving 200 (OK) response to the INVITE request sent in figure J.2.1 step 2 (see figure J.2.1 step 9), MGCF sends an ANM to the originating CS network (see figure J.2.1 step 10).
Annex K (informative):
Change history
	Change history

	Date
	TSG #
	TSG Doc.
	CR
	Rev
	Subject/Comment
	Old
	New

	2010-12
	CP#50
	CP-100787
	434
	1
	Support of ECN
	9.4.0
	10.0.0

	2011-03
	CP#51
	CP-110105
	446
	2
	Clarification of I-MGCF behaviour when SIP preconditions are used
	10.0.0
	10.1.0

	2011-03
	CP#51
	CP-110105
	450
	1
	Clarification of O-MGCF behaviour when SIP preconditions are used
	10.0.0
	10.1.0

	2011-03
	CP#51
	CP-110109
	453
	
	IETF Reference Update: RFC 5993
	10.0.0
	10.1.0

	2011-03
	CP#51
	CP-110108
	456
	2
	CCBS/CCNR correction of SUBSCRIBE request
	10.0.0
	10.1.0

	2011-03
	CP#51
	CP-110105
	460
	1
	New Cause Code mappings of CV 23,38- 47and 65-79
	10.0.0
	10.1.0

	2011-03
	CP#51
	CP-110105
	464
	1
	Change of Cause code mapping
	10.0.0
	10.1.0

	2011-03
	CP#51
	CP-110110
	467
	2
	Missing Timer definition for TIR
	10.0.0
	10.1.0

	2011-03
	CP#51
	CP-110105
	471
	1
	Mapping of ISUP Cause Value 88 to Response 606 and vice versa
	10.0.0
	10.1.0

	2011-03
	CP#51
	CP-110126
	472
	5
	Adding function for supporting early session
	10.0.0
	10.1.0

	2011-03
	CP#51
	CP-110105
	476
	1
	Mapping of SIP Response 604 to ISUP Cause Value 2
	10.0.0
	10.1.0

	2011-03
	CP#51
	CP-110110
	479
	1
	IETF Reference Update
	10.0.0
	10.1.0

	2011-03
	CP#51
	CP-110108
	482
	3
	CC timer expiry correction
	10.0.0
	10.1.0

	2011-03
	CP#51
	CP-110105
	486
	1
	correction BCI mapping
	10.0.0
	10.1.0

	2011-03
	CP#51
	CP-110110
	489
	1
	rn parameter mapping
	10.0.0
	10.1.0

	2011-03
	CP#51
	CP-110130
	493
	1
	Reference update: RFC 6086
	10.0.0
	10.1.0

	2011-03
	CP#51
	CP-110128
	497
	4
	Alignment of Rate Adaptation procedures with TS 26.114 for MTSI.
	10.0.0
	10.1.0

	2011-03
	CP#51
	CP-110127
	499
	2
	Alignment of FAX transport with MTSI
	10.0.0
	10.1.0

	2011-03
	CP#51
	CP-110119
	500
	2
	ECN Updates
	10.0.0
	10.1.0

	2011-03
	CP#51
	CP-110122
	501
	4
	Clarification of the interworking of IMS CAT using Gateway model
	10.0.0
	10.1.0

	2011-03
	CP#51
	CP-110122
	502
	2
	CAT supplementary service skeleton
	10.0.0
	10.1.0

	2011-03
	CP#51
	CP-110110
	503
	1
	Handling of non-E.164 format numbers on NNI
	10.0.0
	10.1.0

	2011-03
	CP#51
	
	
	
	Editorial changes made by MCC
	10.1.0
	10.1.1

	2011-06
	CP#52
	CP-110404
	505
	2
	Align IMEI interworking with latest IETF I-D
	10.1.1
	10.2.0

	2011-06
	CP#52
	CP-110395
	509
	
	Removal of reference IETF draft-patel-dispatch-cpc-oli-parameter
	10.1.1
	10.2.0

	2011-06
	CP#52
	CP-110414
	511
	2
	Editors note clarification when CAT service interacting with other services
	10.1.1
	10.2.0

	2011-06
	CP#52
	CP-110410
	518
	3
	Adapting ECN IEs to agreed ECN H.248 package
	10.1.1
	10.2.0

	2011-06
	CP#52
	CP-110412
	519
	1
	IMS CAT procedures for forking model
	10.1.1
	10.2.0

	2011-06
	CP#52
	CP-110399
	522
	1
	Removal of dial around indicator
	10.1.1
	10.2.0

	2011-06
	CP#52
	
	
	
	Editorial improvements made by MCC
	10.2.0
	10.2.1

	2011-09
	CP#53
	CP-110604
	527
	2
	INVITE with multiple m-lines
	10.2.1
	10.3.0

	2011-09
	CP#53
	CP-110603
	531
	1
	Interactions of " ISDN user part/BICC preference indicator" in the Forward Call Indicator with Supplementary services.
	10.2.1
	10.3.0

	2011-09
	CP#53
	CP-110720
	534
	1
	PSTN XML handling
	10.2.1
	10.3.0

	2011-09
	CP#53
	CP-110607
	537
	
	CCBS correction PSTN CLIR
	10.2.1
	10.3.0

	2011-09
	CP#53
	CP-110613
	544
	1
	Updating IMEI URN draft reference
	10.2.1
	10.3.0

	2011-12
	CP#54
	CP-110823
	551
	1
	Reference update: Reason header in SIP responses
	10.3.0
	10.4.0

	2011-12
	CP#54
	CP-110826
	560
	1
	Correction of interworking for UUS
	10.3.0
	10.4.0

	2011-12
	CP#54
	CP-110831
	565
	
	Clarification on interworking from ISUP to SIP
	10.3.0
	10.4.0

	2011-12
	CP#54
	CP-110823
	569
	2
	PSTN XML handling for UDI fallback
	10.3.0
	10.4.0

	2012-03
	CP#55
	CP-120189
	556
	8
	Updating of UUS references
	10.4.0
	10.5.0

	2012-03
	CP#55
	CP-120057
	573
	1
	History-Info correction of SIP Privacy location
	10.4.0
	10.5.0

	2012-03
	CP#55
	CP-120065
	576
	
	Reference update: draft-ietf-avtcore-ecn-for-rtp
	10.4.0
	10.5.0

	2012-03
	CP#55
	CP-120057
	583
	1
	HOLD interworking correction
	10.4.0
	10.5.0

	2012-03
	CP#55
	CP-120057
	587
	2
	Correction on mapping of the called party Subaddress between a PSTN/PLMN network and an IMS network.
	10.4.0
	10.5.0

	2012-03
	CP#55
	CP-120057
	590
	2
	Add missing Call Diversion Information interworking for CPG(alerting)
	10.4.0
	10.5.0

	2012-03
	CP#55
	CP-120062
	593
	
	Updating IMEI URN draft reference
	10.4.0
	10.5.0

	2012-03
	CP#55
	CP-120057
	597
	3
	Wrong mapping and no note binding in Table 10a
	10.4.0
	10.5.0

	2012-03
	CP#55
	CP-120057
	604
	3
	Mapping of rn parameter into to header field
	10.4.0
	10.5.0

	2012-06
	CP#56
	CP-120337
	616
	3
	Redirection counter 29.163
	10.5.0
	10.6.0

	2012-06
	CP#56
	CP-120333
	622
	1
	Reference update: draft-salud-alert-info-urns
	10.5.0
	10.6.0

	2012-06
	CP#56
	CP-120337
	626
	1
	History-Info mapping correction for CDIV
	10.5.0
	10.6.0

	2012-06
	CP#56
	CP-120333
	631
	2
	NetworkIdentity interworking correction
	10.5.0
	10.6.0

	2012-06
	CP#56
	CP-120344
	644
	1
	Reference update: draft-ietf-avtcore-ecn-for-rtp
	10.5.0
	10.6.0

	2012-06
	CP#56
	CP-120331
	648
	
	Updating of UUS references
	10.5.0
	10.6.0

	2012-06
	CP#56
	CP-120341
	659
	1
	Privacy for PANI 29.163
	10.5.0
	10.6.0

	2012-09
	CP#57
	CP-120516
	607
	4
	TIP TIR interworking
	10.6.0
	10.7.0

	2012-09
	CP#57
	CP-120512
	613
	2
	Mapping of CANCEL request
	10.6.0
	10.7.0

	2012-09
	CP#57
	CP-120513
	636
	6
	P-Early-Media Header is Mandatory for MGCF
	10.6.0
	10.7.0

	2012-09
	CP#57
	CP-120515
	663
	3
	Correction of mapping for COLP-TIP second-first identity
	10.6.0
	10.7.0

	2012-09
	CP#57
	CP-120516
	672
	
	Reference alignment for RTP Payload Format for AMR and AMR-WB Audio Codecs
	10.6.0
	10.7.0

	2012-09
	CP#57
	CP-120515
	680
	1
	PSTN XML schema Bearer Capability correction
	10.6.0
	10.7.0

	2012-09
	CP#57
	CP-120515
	683
	2
	XML schema defintion correction
	10.6.0
	10.7.0

	2012-12
	CP#58
	CP-120828
	688
	
	Reference update: RFC 6679
	10.7.0
	10.8.0

	2012-12
	CP#58
	CP-120825
	691
	1
	Updating IMEI URN draft reference
	10.7.0
	10.8.0

	2013-03
	CP#59
	CP-130062
	701
	
	Correction of CDIV-related interworking
	10.8.0
	10.9.0

	2013-03
	CP#59
	CP-130064
	705
	
	Clarification on Progress Indicator mapping
	10.8.0
	10.9.0

	2013-03
	CP#59
	CP-130177
	709
	2
	Renaming overlap message body to comply with IETF conventions
	10.8.0
	10.9.0

	2013-03
	CP#59
	CP-130067
	712
	1
	IANA registration template for Overlap message body
	10.8.0
	10.9.0

	2013-03
	CP#59
	CP-130062
	716
	2
	IANA registration template for PSTN XML message body
	10.8.0
	10.9.0

	2013-03
	CP#59
	CP-130163
	719
	5
	Generic Number APRI to Privacy
	10.8.0
	10.9.0

	2013-03
	CP#59
	CP-130066
	722
	
	APRIs to Privacy cleaning
	10.8.0
	10.9.0

	2013-06
	CP#60
	CP-130318
	725
	
	From and Privacy mapping to Generic Number and APRI clarification
	10.9.0
	10.10.0

	2013-06
	CP#60
	CP-130375
	732
	2
	Reference Update: RFC 6910
	10.9.0
	10.10.0

	2013-06
	CP#60
	CP-130311
	745
	
	Internal reference correction, ch. 7.4.1
	10.9.0
	10.10.0

	2013-06
	CP#60
	CP-130311
	752
	2
	New Cause Code mappings
	10.9.0
	10.10.0

	2013-06
	CP#60
	CP-130375
	756
	2
	Consolidation of DTMF info package definition in 3GPP TS 24.229
	10.9.0
	10.10.0

	2013-09
	CP#61
	CP-130546
	759
	1
	Updating IMEI URN draft reference
	10.10.0
	10.11.0

	2013-09
	CP#61
	CP-130540
	765
	1
	Interworking of ISDN subaddress information at O-MGCF
	10.10.0
	10.11.0

	2013-09
	CP#61
	CP-130538
	771
	1
	Erroneous reference to Table 17
	10.10.0
	10.11.0

	2013-09
	CP#61
	CP-130545
	779
	1
	Mapping Location Number to P-Access-Network-Info header field
	10.10.0
	10.11.0

	2013-09
	CP#61
	CP-130538
	786
	1
	UDI & H.223 & H.245 indication
	10.10.0
	10.11.0

	2013-12
	CP#62
	CP-130662
	794
	1
	Reference Update: draft-ietf-salud-alert-info-urns
	10.11.0
	10.12.0

	2013-12
	CP#62
	CP-130667
	798
	
	Updating IMEI URN draft reference
	10.11.0
	10.12.0

	2014-03
	CP#63
	CP-140067
	811
	1
	Updating IMEI URN draft reference
	10.12.0
	10.13.0

	2014-06
	CP#64
	CP-140355
	818
	1
	Correction on the ISUP parameter name in the mapping table
	10.13.0
	10.14.0

	2014-09
	CT-65
	CP-140527
	823
	-
	Updating IMEI URN draft reference to RFC 7254
	10.14.0
	10.15.0

	2015-03
	CT-67
	CP-150095
	858
	1
	Reference update for UUSIW
	10.15.0
	10.16.0

	2015-03
	CT-67
	CP-150097
	852
	2
	Correction of APRI Mapping for Generic Number
	10.15.0
	10.16.0

	2015-06
	CT-68
	CP-150336
	0870
	-
	Reference Update: RFC7462 (alert-info urns)
	10.16.0
	10.17.0

	2015-06
	CT-68
	CP-150335
	0883
	2
	User-to-user information interworking
	10.16.0
	10.17.0

	2015-06
	CT-68
	CP-150338
	0890
	1
	APRI Mapping for mutiple Privacy header values
	10.16.0
	10.17.0

	2015-06
	CT-68
	CP-150334
	0898
	3
	URI scheme in History-Info header field
	10.16.0
	10.17.0

	2015-09
	CT-69
	CP-150462
	0917
	5
	Sending of IAM at I-MGCF for SIP-BICC-interworking
	10.17.0
	10.18.0

	2015-12
	CT-70
	CP-150670
	0928
	1
	URI scheme in History-Info header field
	10.18.0
	10.19.0

	2015-12
	CT-70
	CP-150670
	0943
	1
	Removal for the unsupported SIPS URI scheme
	10.18.0
	10.19.0


	Change history

	Date
	TSG #
	TSG Doc.
	CR
	Rev
	Cat
	Subject/Comment
	New

	2016-06
	CT#72
	CP-160257
	0950
	5
	A
	Interworking of History-Info Header Field when Original Called Number is not present
	10.20.0








3GPP


_1200029140.doc
 


IP







IP







SIP







TCP /



UDP /



SCTP







IP







IP







IP







M3UA







SCTP







IP







SIP











MTP3







MTP2











MTP3







ISUP







MTP2







L1







TCP / 



UDP /



SCTP







SIP







ISUP







M3UA







SCTP







ISUP











L1



















SS7







Media gateway



control function







Signalling gateway



function







SS7 signalling



function







SIP signalling



function












_1301951078.doc

[image: image1]

O-MGCF







ACM �(Subscriber Free)







Ring tone







180 Ringing �[P-Early-Media header: backward early media authorized]












_1326906730.vsd
Process Name


SignalName


SignalName


2.  IAM (speech)


Interworking Node
 (MGCF )


1. SIP: INVITE  [SDP offer: Audio + Text]


CS Network


IMS


3. SIP: 18x or 200 OK
[SDP answer: Audio + Text]


Audio / Text



_1347273334.doc


O-MGCF







INVITE 







SAM







IAM







SAM











INVITE 







INVITE 











404/484







404/484




















_1357687095.doc


I-MGCF







COT







SDP indicating �preconditions met  and/or �active media












_1360190780.vsd
Process Name


SignalName


SignalName


SignalName


SignalName


IM-MGW


MGCF


6. SIP: INVITE  (Session SDP(IP1,p1)


7. SIP: 100 Trying


1. ISUP: IAM 


2. H.248: ADD.req [Context ID = ?, Termination ID = ?]


3. H.248: ADD.resp [Context ID = C1, Termination ID = T2] 


4. H.248: ADD.req [Context ID = C1, Termination ID=?]


5. H.248: ADD.resp [Context ID = C1, Termination ID = T1, IP1,p1]


8. SIP:180 Ringing
(Session SDP and Early Session SDP)


12. SIP: PRACK
(Early Session SDP(IP1,p1))


13. SIP :200 OK (PRACK) 


10. H.248: MOD.req [Context ID = C1,Termination ID = T1],
Early session SDP


11.H.248: MOD.resp [Context ID = C1, Termination ID = T1 ] 


9. ISUP: ACM 


Reserve TDM Circuit,
Change Through-
Connection  = both  


Reserve IMS Connection Point, Change IMS Through-
Connection  = backward 


Configure IMS  Resources


14. SIP :200 OK (INVITE)


18. SIP: ACK


15. H.248:  MOD.req [Context ID=C1, Termination ID = T1, esssion SDP]


16. H.248: MOD.resp [Context ID = C1, Termination ID = T1] 


17. ISUP: ANM 


CALL IN ACTIVE STATE


Configure IMS Resources,
Change IMS Through-
Connection  = both



_1366782081.vsd
MGCF


IM-MGW


H.248:NOTIFY.req(ECN Failure) [Termination =x, cause=ecn_cause] 


H.248:NOTIFY.resp(ECN Failure)
[Termination x]


Context(Cx)


Context(Cx)


Report ECN Failure



_1436082063.vsd
Process Name


SignalName


SignalName


SignalName


SignalName


12. ANM 


Interworking node A 
(MGCF+ IM MGW)


14. Clear channel. Either MONA messages + H.223 bearer setup 
or H.223 bearer setup and H.245 signaling between CS terminals


CALL IN ACTIVE STATE


3. IAM [UDI, H.223 & H.245]


9. ACM 


8. SIP :180 Ringing


7. ACM 


1. IAM [UDI, H.223 & H.245]


2. SIP: INVITE  [m = H.263,  m= AMR;
m=clearmode] [PSTN XML body] 


6. TDM circuit reservation


10. ANM 


4. TDM circuit reservation


5. SIP : Response [m=audio, a=clearmode]


11. SIP:200 OK(INVITE) 


13. SIP: ACK


CS Network


IMS


Interworking node B 
(MGCF+ IM MGW)


CS Network



_1366710315.vsd
MGCF


IM-MGW


H.248:ADD.req[
Termination X,
Transport = RTP/AVPF, ECN_enable=True, {Notify_ECN_Error}]


H.248:ADD.resp[Termination X]


Context(Cx)


Context(Cx)


Reserve IMS Connection Point and Configure Remote Resources



_1359951088.vsd
Process Name


SignalName


SignalName


SignalName


SignalName


SignalName


SignalName


IM-MGW


MGCF


4. SIP: INVITE  (Session SDP offer)


5. SIP: 100 Trying


1. BICC: IAM 


6. SIP:180 Ring
(Session SDP answer and Early Session SDP offer)


12. SIP: PRACK
(Early Session SDP answer)


13. SIP :200 OK (PRACK) 


19. SIP: ACK


16. H.248:  MOD.req [Context ID=C1, Termination ID = T1]


17. H.248: MOD.resp [Context ID = C1, Termination ID = T1] 


18. BICC: ANM 


CALL IN ACTIVE STATE


Configure IMS Resources,
Change IMS Through-
Connection  = both


15. SIP :200 OK (INVITE)


2. H.248: ADD.req  [Context ID = ?, Termination ID=?]


3. H.248: ADD.resp [Context ID = C1, Termination ID = T1]


Reserve IMS Connection Point, Change IMS Through-
Connection = backward 


7. H.248: MOD.req Context ID = C1,Termination ID = T1]


 8. H.248: MOD.resp [Context ID = C1,Termination ID = T1] 


Configure IMS  Resources


9. H.248: ADD.req [Context ID = C1, Termination ID = ?]


10. H.248: ADD.resp [Context ID = C1, Termination ID = T2] 


11. BICC: APM 


Prepare Bearer,
Change Through-Connection= both


14. BICC: ACM 


START CAT MEDIA



_1359186083.vsd
MGCF


1. IAM


2. INVITE (SDP_O)


3. 183 (Session Progress) (SDP_A_cat)


4. ACM


5. PRACK


6. 200 (OK) (PRACK)


7. UPDATE (SDP_O_Regular)


8. 200 (OK) (UPDATE)
(SDP_A)


10. ANM


9. 200 (OK) (INVITE)


11. ACK



_1357687031.doc


I-MGCF







COT







SDP indicating �preconditions met  and/or �active media












_1327342003.vsd
SignalName


IM MGW


.


MGCF


 1. 


H.248


: ADD.req


[C=


C1


,T=


?


... ,Event=MPC-reception, Signal =MPC MUX Code]}


4


2. 


H.248


: ADD.resp


Notify MONA prefrence reception,


MONA preference message, Mux Code, MPC PDU


6


H.248


[C=


C1


,T=


T2


,


Event=MONA-preference-receptiom  {MONA preferences},]


.


5


7


H.248


: Notify.resp


: Notify.req


Add Multiplex Termination


MONA preference message, Mux Code


.


8. H.248: MOD.req [C=C1, T=T2, 
Stream=1{LocalCont{,muxtbl_out}}]


11.


9. H.248: MOD.resp


Configure Multiplex Termination


.


3


MONA preference message, Mux Code


MONA preference message, Mux Code


.


12.


MONA preference message, Mux Code, MPC PDU


13.


H.248


[C=


C1


,T=


T2


,


Event=MPCin  {muxcode=Mux Code}]


14


H.248


: Notify.resp


: Notify.req


Notify MPC


10.


PDU in MPC 


15. H.248: MOD.req [C=C1, T=T2, 
Stream=1{LocalCont{,muxtbl_in}}]


16. H.248: MOD.resp


Configure Multiplex Termination



_1328488585.vsd
Process Name


SignalName


SignalName


SignalName


MGCF


IM-MGW


Next steps of basic  call establishment (as specified in subclause 9.2.2.3), returning an SDP Answer with audio + text


9. H.248:  MOD.req  [Context ID=C1, Termination ID = T1]


10. H.248: MOD.resp [Context ID = C1, Termination ID = T1] 


8. ISUP: ANM 


13. SIP :200 OK (INVITE)


1. SIP: INVITE 
[SDP offer: audio + Real-Time Text]


2. SIP: 100 Trying


7. ISUP: IAM 


14. SIP: ACK


11. H.248:  MOD.req ) [Context ID=C1, Termination ID = T2]


12. H.248: MOD.resp [Context ID = C1, Termination ID = T2] 


15. Text/modem 


16. Text/RTP


CALL IN ACTIVE STATE


Change IMS Through
-Connection = both


Reserve IMS Connection
Point, and Configure
Remote Resources
Change IMSThrough-Connection = backward 


Activate TDM Voice 
Prosessing Function


5. H.248: ADD.req [Context ID = C1, Termination ID = T2]


6. H.248: ADD.resp [Context ID = C1, Termination ID = T2] 


Reserve TDM Circuit,
Change  TDM Through-Connection= both, Monitor Text Call


3. H.248: ADD.req [Context ID = ?, Termination ID=?, stream=1{audio media}, stream=2{Real-Time Text media}]


4. H.248: ADD.resp [Context ID = C1, Termination ID = T1]


CS Network


IMS



_1328499489.vsd
Process Name


SignalName


SignalName


SignalName


SignalName


SignalName


SignalName


IM-MGW


MGCF


Monitor Text Call


Next steps of basic  call establishment (as specified in subclause 9.2.3.3)


CALL IN ACTIVE STATE


16. Text/modem 


6. SIP: INVITE  [SDP offer: Audio]


7. SIP: 183 Session Progress
[SDP answer: Audio]


1. ISUP: IAM 


Reserve IMS Connection Point, Change IMS Through-
Connection  = backward 


Configure IMS  Resources


17. Real-Time Text/RTP


12. SIP: UPDATE [SDP offer: Audio + Real-Time Text]


15. SIP: 200 UPDATE
[SDP answer: Audio + Real-Time Text]


10. SIP: PRACK


11. SIP: 200 OK (PRACK)


2. H.248: ADD.req [Context ID = ?, Termination ID = ?]


3. H.248: ADD.resp [Context ID = C1, Termination ID = T2] 


8. H.248: MOD.req [Context ID = C1,Termination ID = T1]


9. H.248: MOD.resp [Context ID = C1, Termination ID = T1 ] 


4. H.248: ADD.req [Context ID = C1, Termination ID=?, 
                stream=1{audio media}]


5. H.248: ADD.resp [Context ID = C1, Termination ID = T1]


Configure IMS  Resources


13. H.248: MOD.req [Context ID = C1,Termination ID = T1, 
                stream=1{audio media}, stream=2{Real-Time Text media}]


14. H.248: MOD.resp [Context ID = C1, Termination ID = T1 ] 


CS Network


IMS


Reserve TDM Circuit,
Change  TDM Through-Connection= both



_1328277174.doc

[image: image1]

Context C1







Termination T1







Termination T2











2-way topology











Audio/RTP stream







Real-Time Text/RTP stream







CS network







IMS











 (audio + V.18) stream












_1327299360.vsd
Text


The height of the text box and its associated braces increases or decreases as you add text. To change the width of the comment, drag one of the side handles.


Text


Select note and type your message!


text


Process Name


SignalName


SignalName


Figure 4a – CS Orig, 183 & 180 response, pMGW generated media (MGCF)


Figure 4b – CS Orig, 183 & 180 response, pMGW generated media (MGCF/MSC)


IMS


MSC


INVITE (P-EM: supported) [SDP] 


MGCF


IAM


ACM (sub free)


180 (P-EM: inactive) [SDP*]


IMS


INVITE (P-EM: supported) 


MGCF/MSC


180 (P-EM: inactive) [SDP*]


183 (P-EM: sendonly/sendrecv) [SDP*#]


183 (P-EM: sendonly/sendrecv) [SDP*#]


CPG (inband)


IMS


MSC


INVITE (P-EM: supported) [SDP] 


MGCF


IAM


ACM (inband)


IMS


MSC


INVITE (P-EM: supported) [SDP] 


MGCF


IAM


183 (P-EM: inactive/recvonly) [SDP*]


IMS


INVITE (P-EM: supported) [SDP] 


MGCF/MSC


183 (P-EM: inactive/recvonly) [SDP*]


180 [SDP**]


180 [SDP**]


ACM (sub free)


183 (P-EM: sendonly, sendrecv) [SDP*]


IMS


INVITE (P-EM: supported) [SDP]


MGCF/MSC


183 (P-EM: sendonly, sendrecv) [SDP*]


180 [SDP*#]


180 [SDP*#]


CPG (alerting)


Figure 5a – CS Orig, 183 & 180 response, IMS generated media (MGCF)


Figure 5b – CS Orig, 183 & 180 response, IMS generated media (MGCF)


Cut through media path


Figure 6a – CS Orig, switch of media authorization from LCP to RCP (MGCF)


Figure 6b – CS Orig, switch of media authorization from LCP to RCP (MGCF/MSC)


MGCF


IMS


ACM


18x (P-EM: sendonly) [SDP]


IMS


IMS


INVITE [SDP] 


MGCF/MSC


MSC


Figure 1a – CS Term (MGCF)


INVITE (P-EM: supported) [SDP] 


18x (P-EM: sendonly) [SDP]


MSC


IAM


MGCF


IAM


ACM (?)


18x (P-EM: sendonly/sendrecv) [SDP]


SDP* - SDP may or may not be present.
SDP** - SDP may or may not be present. Can’t be present if previously provided reliably.
SDP*# - SDP may or may be present. Expected if not previously provided reliably.


IMS


IMS


INVITE (P-EM: supported) 


MGCF/MSC


Early media


MSC


INVITE (P-EM: supported) [SDP] 


18x (P-EM: sendonly/sendrecv) [SDP]


When early media is shown passing  through the MGCF or MGCF/MSC, then cut through must have occur.


MGCF


IAM


ACM (sub free)


180 (P-EM: note) [SDP*]


IMS


INVITE (P-EM: supported)  [SDP]


MGCF/MSC


180 (P-EM: note) [SDP*]


Note: P-Early-Media header set to recvonly, inactive, or P-EM header not provided.


Note: P-Early-Media header set to recvonly, inactive, or P-EM header not provided.


Figure 1b – CS Term (MGCF/MSC)


INVITE [SDP] 


Figure 2a – CS Orig, IMS generated media (MGCF)


Figure 2b – CS Orig, IMS generated media (MGCF/MSC)


Figure 3a – CS Orig, 180 resonse, pMGW generated media (MGCF)


Figure 3b – CS Orig, 180 response, uMGW generated media (MGCF/MSC)


Note: content of ACM is dependent on type of 18x


IMS


MSC


INVITE (P-EM: supported) [SDP] 


MGCF


IAM


ACM (inband)


183 (P-EM: sendonly/sendrecv) [SDP]


IMS


INVITE (P-EM: supported) [SDP]


MGCF/MSC


183 (P-EM: sendonly/sendrecv) [SDP]


180 (P-EM: inactive/recvonly)


180 (P-EM: inactive/recvonly)


CPG (alerting)


IMS


MSC


INVITE (P-EM: supported) [SDP] 


MGCF


IAM


ACM (inband)


183 (P-EM: sendonly/sendrecv) [SDP]


IMS


INVITE (P-EM: supported) [SDP]


MGCF/MSC


183 (P-EM: sendonly/sendrecv) [SDP]


180


180


CPG (alerting)


183 (P-EM: inactive/recvonly)


183 (P-EM: inactive/recvonly)


X


X


X – through connect blocked, not backward media allowed.


Figure 8a – CS Orig, switch of media authorization from RCP to LCP, wait for 180 (MGCF)


Figure 8b – CS Orig, switch of media authorization from RCP to LCP, wait for 180 (MGCF/MSC)


Figure 7a – CS Orig, switch of media authorization from RCP to LCP (MGCF)


Figure 7b – CS Orig, switch of media authorization from RCP to LCP (MGCF/MSC)


IMS


MSC


MGCF


IAM


ACM


18x (P-EM: sendonly) [SDP]


IMS


INVITE 


MGCF/MSC


18x (P-EM: sendonly) [SDP]


Figure 10a – CS Term INVITE w/o SDP (MGCF)


Figure 10b – CS Term INVITE w/o SDP  (MGCF/MSC)


INVITE


PRACK [SDP]


200 OK (PRACK)


PRACK [SDP]


200 OK (PRACK)


IMS


MSC


INVITE (P-EM: supported) [SDP] 


MGCF


IAM


183 [SDP]


IMS


INVITE (P-EM: supported)  [SDP]


MGCF/MSC


183 [SDP]


180


180


ACM (sub free)


Figure 9a – CS Orig, switch of implicit media authorization from RCP to LCP, wait for 180 (MGCF)


Figure 9b – CS Orig, switch of implicit media authorization from RCP to LCP, wait for 180 (MGCF/MSC)


IMS


MSC


MSC


INVITE [SDP]


18x (P-EM*) [SDP]


18x (P-EM) [SDP]


IMS


IMS


MSC


INVITE [SDP2]


Figure 11 – Call Forwarding Immediate G/V-MSC when no MGW has been allocated


Figure 13a – Call Forwarding when MGW has been allocated, INVITE w/ SDP


INVITE [SDP]


18x (P-EM*) [SDP]


18x (P-EM) [SDP]


PRACK


200 OK (PRACK)


INVITE [SDP1]


PRACK


PRACK


IMS


IMS


MSC


INVITE


18x (P-EM*) [SDP]


18x (P-EM) [SDP]


Figure 12 – Call Forwarding G/V-MSC when no MGW has been allocated, INVITE w/o SDP


INVITE


PRACK [SDP]


200 OK (PRACK)


PRACK [SDP]


200 OK (PRACK)


Figure 13a – Call Forwarding when MGW has been allocated, INVITE w/o SDP


IMS


IMS


MSC


INVITE [SDP2]


18x (P-EM*) [SDP]


18x (P-EM) [SDP]


200 OK (PRACK)


200 OK (PRACK)


PRACK


200 OK (PRACK)


18x (P-EM)


PRACK


200 OK (PRACK)


INVITE


PRACK [SDP1]


200 OK (PRACK)


PRACK


200 OK (PRACK)


18x (P-EM)


PRACK


200 OK (PRACK)


PEM* - P-Early-Media header may or may not be present.
SDP1/SDP2 – the two SDPs are indenpend. SDP2 is generated based on current agreed codec(s) on ingress interface and with IP address set to far ingress IP address (IMS IP address).


UPDATE [SDP]


200 OK (UPDATE) [SDP]


UPDATE [SDP]


200 OK (UPDATE) [SDP]


Figure 14 – Call Forwarding, first response w/o SDP


IMS


IMS


MSC


INVITE [SDP2]


18x (P-EM*)


18x (P-EM) [SDP]


INVITE [SDP1]


PRACK


200 OK (PRACK)


PRACK


200 OK (PRACK)


18x (P-EM)


PRACK


200 OK (PRACK)


18x (P-EM*) [SDP]


PRACK


200 OK (PRACK)


18x (P-EM)


PRACK


200 OK (PRACK)


UPDATE [SDP]


200 OK (UPDATE) [SDP]


Interworking Node
 (MGCF)


2. SIP: INVITE  [SDP offer: Audio]


12. SIP: 200 OK (INVITE)


1.  IAM (speech)


Bearer Establishment


IMS


CS Network


9. ACM


13. ANM


8. SIP: 180 Ringing


3. SIP: 183 Session Progress
[SDP answer: Audio]


6. SIP: UPDATE [SDP offer: Audio + Text]


7. SIP: 200 UPDATE
[SDP answer: Audio + Text]


Audio / Text


4. SIP: PRACK


5. SIP: 200 OK (PRACK)


10. SIP: PRACK


11. SIP: 200 OK (PRACK)



_1301952370.doc

[image: image1]

O-MGCF







ACM 



(Call Diversion information, OBCI: in-band info available)







Audio







181 Call is Being Forwarded �[P-Early-Media header: backward early media authorized]












_1319633342.doc


I-MGCF







ACM or CPG�(Cause=xxx)







183 Session Progress



(Reason(Cause=xxx))
















_1321164947.doc


BYE







O-MGCF







REL












_1319555462.doc


O-MGCF







CPG (in-band information available Cause=xxx)







183 Session Progress



(Reason(Cause=xxx))
















_1319555500.doc


O-MGCF







ACM (�No indication Cause=xxx)







183 Session Progress



(Reason(Cause=xxx))
















_1301952456.doc


ACM 



(Call Diversion information)







181 Call is Being Forwarded 







O-MGCF












_1301951241.doc

[image: image1]

O-MGCF







CPG 



(In-band information available)







Appropriate announcement







183 Session Progress �[P-Early-Media header: backward early media authorized]












_1301952240.doc

[image: image1]

O-MGCF







ACM�(No indication, OBCI: in-band info available)







Appropriate 



announcement







183 Session Progress �[P-Early-Media header: backward early media authorized]












_1301951322.doc

[image: image1]

O-MGCF







CPG 



(Progress, OBCI: in-band info available)







Audio







181 Call is Being Forwarded�[P-Early-Media header: backward early media authorized]












_1301951157.doc

[image: image1]

O-MGCF







CPG �(Alerting)







Ring tone







180 Ringing �[P-Early-Media header: backward early media authorized]












_1240045971.doc


I-MGCF







ACM (Subscriber Free)







180 Ringing



P-Early-Media (1)







Ring tone












_1255193203.doc


13. BICC APM (ActInd=Stop signal ack)







12. BICC APM (ActInd=Stop signal notify)







11. H.248 Notify.resp[Context ID = C1, Termination ID = T1]











MGCF







IM-MGW







2. H.248 Add/Mod.resp [Context ID = C1, Termination ID =T1]







1. H.248 Add/Mod.req



              [Context ID = C1, Termination ID = T1,



               Codec = “telephone event, AMR”,�               Notify IMS RTP Tel Event (”Start tone detected”,



                                                          ”End tone detected”)  ]







9. RTP encoded continued DTMF event



                (tone-event, end=1, duration=Duration)











3. RTP encoded new DTMF event



             (tone-event, end=0, duration=x)











8. RTP encoded new DTMF event



             (tone-event, end=0, duration=y)











10. H.248 Notify.ind 



 [Context ID = C1, Termination ID = T1, Tone-ID,



  Event=”End tone detected”, Duration]







4. H.248  Notify.ind 



               [ Context ID = C1, Termination ID = T1, Tone_ID,



 Event=”Start tone detected”]







7. BICC APM (ActInd=Start signal ack)







6. BICC APM (ActInd=Start signal notify, signal-type)







5. H.248 Notify.resp[Context ID = C1, Termination ID = T1]







…







1 Digit











Configure IMS Resources



or



Reserve IMS Conection Point and configure Remote Resources,



Detect_IMS_RTP_Tel_Event















Notify_IMS_RTP_Tel_Event











Notify_IMS_RTP_Tel_Event












_1279100394.vsd
Process Name


SignalName


SignalName


SignalName


SignalName


2.  IAM


Interworking Node
 (MGCF+ IM MGW)


7. H.245: Terminal Capability Set
[H.261, MP4V-ES;
AMR, G.711]


3. Bearer Establishment


6. H.245: Terminal Capability Set
[MP4V-ES; AMR]


8. H.245: Open Logical Channels
[MP4V-ES, AMR]


1. SIP: INVITE  [SDP offer:,
m = H.263, MP4V-ES, H.261;
 m= AMR]


9. SIP: 200OK (INVITE) [SDP answer:
m = MP4V-ES; a= 3gpp_MaxRecvSDUSize 600;
 m= AMR]


CALL IN ACTIVE STATE


CS Network


IMS


5. H.223 Bearer Setup including possible
MONA Channel establishment method negotiation


4. ANM


Alternative to 6-8: MONA messages



_1280840899.doc


I-MGCF







ACM( call diver-sion information)







181 Call is being forwarded



P-Early-Media (1)







Audio












_1285026465.doc


O-MGCF







INVITE 







SAM







IAM











404/484







INFO











INVITE







18x







200 (INFO)







SAM




















_1299485614.vsd
IM MGW


.


MGCF


4


H.248


: MOD.req


[C=


C1


,T=


T2


Remove Signal-SPC+ SPC-reception]


5


H.248


: MOD.resp


Stop SPC


1


receive final H.245 message attached to MONA preference message


2


H.248


[C=


C1


,T=


T2


,


Event=SPCin  {h245mc=XXXX}]


3


H.248


: Notify.resp


Notify SPC


: Notify.req



_1284042281.vsd
Process Name


SignalName


SignalName


IM MGW


3. receive RTCP feedback messages 


4.  send RTCP feedback Ack messages 


CALL IN ACTIVE STATE


6. H.248: Notify.resp


8. receive H.245 message response


5.  H.248: Notify.req [C=C1, T=T1, Event=RTCP-Interworking]


7. send H.245 message


CS Network


IMS


MGCF


1. H.248: MOD.req [C=C1, T=T1…, Event=RTCP-Interworking]


2.  H.248: MOD.resp



_1284127308.vsd
Process Name


SignalName


SignalName


1. receive H.245 message to feedback on the quality of the data distribution


IM MGW


6. receive RTCP feedback Ack message 


5.  send RTCP feedback message 


CALL IN ACTIVE STATE


CS Network


IMS


MGCF


3. H.248: MOD.req [C=C1, T=T1…, signal=H.245-Interworking]


4.  H.248: MOD.resp


2.  send H.245 message response



_1279538796.doc

[image: image1]

I-MGCF







181 Call is being forwarded



P-Early-Media (1)







CPG(Call diversion information)



















Audio












_1279100646.vsd
Process Name


SignalName


SignalName


SignalName


SignalName


Interworking Node
(MGCF+ IM MGW)


12. H.245: Open Logical Channels
[H.263, AMR]


13. SIP:
[SDP offer
m =: H.263; a= 3gpp_MaxRecvSDUSize 600;
 m= AMR]


11. H.245: Terminal Capability Set
[H.263, H.261
AMR]


14. SIP :
[SDP answer
m = H.263;
 m= AMR]


2. SIP: INVITE  [SDP offer:,
m = H.263, MP4V-ES;
 m= AMR


1.  IAM


4. Bearer Establishment


3. SIP:  [SDP answer:
m = H.263, MP4V-ES;
 m= AMR]


8. H.245: Terminal Capability Set
[H.263, H.261
AMR, G.711]


CALL IN ACTIVE STATE


9. SIP:
[SDP offer
m =: H.263, H.261;
 m= AMR]


10. SIP :
[SDP answer
m = H.263, H.261;
 m= AMR]


CS Network


IMS


5. SIP: 200OK (INVITE)


7. H.223 Bearer Setup including possible
MONA Channel establishment method negotiation



6. ANM


Alternative to 8-12: MONA messages



_1270660335.vsd
IM MGW


.


MGCF


 1. 


H.248


: ADD.req


[C=


C1


,T=


?


... ,Event=SPC-reception]


2. 


H.248


: ADD.resp


3. 


receive H.245 message attached to MONA preference message


 4. 


H.248


[C=


C1


,T=


T2


,


Event=SPCin  {h245mc=XXXX}]


5. 


H.248


: Notify.resp


Notify SPC


: Notify.req


Add Multiplex Termination



_1279100338.vsd
Process Name


SignalName


SignalName


SignalName


SignalName


2.  IAM


Interworking Node
 (MGCF+ IM MGW)


8. H.245: Terminal Capability Set
[H.261, MP4V-ES;
AMR, G.711]


4. Bearer Establishment


7. H.245: Terminal Capability Set
[H.263, MP4V-ES; H.261
AMR]


9. H.245: Open Logical Channels
[MP4V-ES, AMR]


1. SIP: INVITE  [SDP offer:,
m = H.263, MP4V-ES, H.261;
 m= AMR,
Preconditions not met]


3. SIP:  [SDP answer:
m = H.263;
 m= AMR,
Preconditions not met ]


CALL IN ACTIVE STATE


CS Network


IMS


11. SIP:
[SDP offer; m =: MP4V-ES; a= 3gpp_MaxRecvSDUSize 600; m= AMR ]


12. SIP :
[SDP answer; m = MP4V-ES, m= AMR]


6. H.223 Bearer Setup including possible MONA Channel establishment method negotiation


5. ANM


10. SIP: 200 OK (INVITE)


Alternative to 7-9: MONA messages



_1270660296.vsd
Process Name


SignalName


SignalName


SignalName


SignalName


IM MGW


9. MONA preferences
[ack=00]


11.


19. H.248: MOD.req [C=C1, T=T2, 
Stream=1{LocalCont{h223capr,muxtbl_in,muxtbl_out}}
stream=2{LCN=LC1, Codec=H263}
stream=3{LCN=LC2, Codec=AMR}]


6. H.248: ADD.req [C=C1, T=?,  Mux=H223,T1,
LocalControl{MONA preferences, muxlv=2} 
Event=legacyDetected(h245msgout[octet string]),  MONA-preference-reception, MONA-preference-completed, H245msgin,  [MPC-reception, SBC-reception] ]


MGCF


20. H.248: MOD.resp


1. H.248: ADD.req [C=?, T=?, stream=1]


2. H.248: ADD.resp
[C=C1, T=T1]


7. H.248: ADD.resp
[C=C1, T=T2]


21. H.248: MOD.req [C=C1, T=3, stream=2{Codec=H263}, stream=3{Codec=AMR]


5. Bearer Establishment


10. MONA preferences
[ack=01]


8. MONA preferences
[ack=00]


9. MONA preferences
[ack=00]


H.248


[C=


C1


,T=


T2


,


Event=MONA-preference-receptiom  {MONA preferences}]


22. H.248: MOD.resp
[C=C1, T=T3]


12.


H.248


: Notify.resp


: Notify.req


Notify MONA prefrence reception


10. MONA preferences
[ack=01]


14. MONA preferences
[ack=10]


13. MONA preferences
[ack=01]


8. MONA preferences
[ack=00]


Event=MONA-preference-completed]


3. H.248: ADD.req [C=C1, T=?, stream=2{Codec=H.263}, stream=3{Codec=AMR}]


4. H.248: ADD.resp
[C=C1, T=T3]


17.


H.248


Configure Multiplex Termination


Add Multiplex Termination


: Notify.resp


Reserve IMS Connection Point or Reserve IMS Connection Point
and Configure Remote Resources


Configure IMS Ressources 


Prepare Bearer or Establish Bearer or Reserve TDM Circuit


Notify MONA prefrence completed


16.


H.248


[C=


C1


,T=


T2


,


: Notify.req


14. MONA preferences
[ack=10]


13. MONA preferences
[ack=01]


15. MONA preferences
[ack=10]


18. Accelerated H.245 call setup procedures


Optional Procedures for reception of SPC PDU, sending of SPC PDU, reception of MPC PDU, sending of MPC PDU



_1270657978.vsd
Process Name


SignalName


SignalName


SignalName


SignalName


IM MGW


11.


6. H.248: ADD.req [C=C1, T=?,  Mux=H223,T1,
LocalControl{MONA preferences, muxlv=2},  Event=legacyDetected(h245msgout[octet string]), MONA-preference-reception, MONA-preference-completed, H245msgin,  MPC-reception, SBCin ]


MGCF


1. H.248: ADD.req [C=?, T=?, stream=1]


2. H.248: ADD.resp
[C=C1, T=T1]


7. H.248: ADD.resp
[C=C1, T=T2]


5. Bearer Establishment


8. MONA preferences
[ack=00]


9. Legacy interworking condition


H.248


[C=


C1


,T=


T2


,


Event=legacyDetected


12.


H.248


: Notify.resp


: Notify.req


Notify detection of legacy interworking


8. MONA preferences
[ack=00]


3. H.248: ADD.req [C=C1, T=?, stream=2{Codec=H.263}, stream=3{Codec=AMR}]


4. H.248: ADD.resp
[C=C1, T=T3]


Add Multiplex Termination


Reserve IMS Connection Point or Reserve IMS Connection Point
and Configure Remote Resources


Prepare Bearer or Establish Bearer or Reserve TDM Circuit


13. standard H.245 call setup procedures continuation, Figure E.4.2.4.1 starting with step 9


10. octet string (H.245 TCS+MSD)



_1255189131.doc


13. BICC APM (ActInd=Start signal ack)







12. BICC APM (ActInd= Start signal notify, signal-type, duration)







11. H.248 Notify.resp[Context ID = C1, Termination ID = T1]











MGCF







IM-MGW







2. H.248 Add/Mod.resp [Context ID = C1, Termination ID =T1]







1. H.248 Add/Mod.req



              [Context ID = C1, Termination ID = T1,



               Codec = “telephone event, AMR”,�               Notify IMS RTP Tel Event (”End tone detected”)  ]                                                     







9. RTP encoded continued DTMF event



                (tone-event, end=1, duration= Duration)











8. RTP encoded new DTMF event



             (tone-event, end=0, duration=x)











…











10. H.248 Notify.ind 



 [Context ID = C1, Termination ID = T1, Tone-ID,



  Event=”End tone detected”, Duration]



































Configure IMS Resources



or



Reserve IMS Conection Point and configure Remote Resources,



Detect_IMS_RTP_Tel_Event







1 Digit















Notify_IMS_RTP_Tel_Event












_1255189550.doc


15. BICC APM (ActInd=Stop signal ack)







14. BICC APM (ActInd=Stop signal notify)







17. H.248 Mod.resp[Context ID = C1, Termination ID = T1]







6. H.248 Mod.resp[Context ID = C1, Termination ID = T1]







MGCF







IM-MGW







2. H.248 Add/Mod.resp [Context ID = C1, Termination ID =T1]







1. H.248 Add/Mod.req



              [Context ID = C1, Termination ID = T1,



               Codec = “telephone event, AMR”,�               Notify IMS RTP Tel Event (”Start tone detected”,



                                                          ”End tone detected”)  ]







187. RTP encoded continued DTMF event



                (tone-event, end=1, duration)











13. RTP encoded new DTMF event



             (tone-event, end=0, duration=y)







7. RTP encoded new DTMF event



            (tone-event, end=0, duration=x)







Digit 2







Digit 1







16. H.248 Mod.ind 



 [Context ID = C1, Termination ID = T1]







10. H.248  Mod.ind 



               [ Context ID = C1, Termination ID = T1, Tone_ID,



 SignalType=On/Off]







9. BICC APM (ActInd=Start signal ack)







8. BICC APM (ActInd=Start signal notify, signal-type)







11. H.248 Mod.resp[Context ID = C1, Termination ID = T1]







4. BICC APM (ActInd=Start signal ack)







3. BICC APM(ActInd=Start signal notify, signal-type, duration)







5. H.248 Mod.ind



 [Context ID = C1, Termination ID = T1, Tone-Id,



  SignalType=TimeOut, Duration]







Configure IMS Resources



or



Reserve IMS Conection Point and configure Remote Resources,



Detect_IMS_RTP_Tel_Event











Stop_IMS_RTP_Tel_Event











Send_IMS_RTP_Tel_Event











Send_IMS_RTP_Tel_Event







12. RTP encoded new DTMF event



             (tone-event, end=0, duration=x)







7. RTP encoded new DTMF event



            (tone-event, end=1, duration)







…







…












_1248176385.vsd
Process Name


SignalName


SignalName


SignalName


SignalName


IM MGW


13. H.245: Open Logical Channel
[H245 logical Channel = LC1]


14. H.245: Open Logical Channel Ack


21. H.248: MOD.req [C=C1, T=T2, 
Stream=1{LocalCont{h223capr,muxtbl_in,muxtbl_out}}
stream=2{LCN=LC1, Codec=H263}
stream=3{LCN=LC2, Codec=AMR}]


6. H.248: ADD.req [C=C1, T=?,  Mux=H223,T1,
LocalControl{ muxlv=2} 
Event=H245msgin ]


MGCF


22. H.248: MOD.resp


1. H.248: ADD.req [C=?, T=?, stream=1]


2. H.248: ADD.resp
[C=C1, T=T1]


7. H.248: ADD.resp
[C=C1, T=T2]


23. H.248: MOD.req [C=C1, T=3, stream=2{Codec=H263}, stream=3{Codec=AMR]


5. Bearer Establishment


10. H.245: Terminal Capability Set Ack,
Master-Slave Determination Ack


11. H.245: Terminal Capability Set,
[H.261,H.263,AMR]
Master-Slave Determination


12. H.245: Terminal Capability Set Ack, Master-Slave Determination Ack


24. H.248: MOD.resp
[C=C1, T=T3]


9. H.245: Terminal Capability Set, [AMR,MP4V-ES,H263]
Master-Slave Determination


8. H.223: Multiplexing Level Negotiation
[level =2]


3. H.248: ADD.req [C=C1, T=?, stream=2{Codec=H.263}, stream=3{Codec=AMR}]


4. H.248: ADD.resp
[C=C1, T=T3]


17. H.245: Multiplex Entry Send


18. H.245: Multipex Entry Send Ack


20. H.245: Multiplex Entry Send Ack


19. H.245: Multipex Entry Send


Configure Multiplex Termination


Add Multiplex Termination


15. H.245: Open Logical Channel
[H245 logical Channel = LC2]


16. H.245: Open Logical Channel Ack


Reserve IMS Connection Point or Reserve IMS Connection Point
and Configure Remote Resources


Configure IMS Ressources 


Prepare Bearer or Establish Bearer or Reserve TDM Circuit



_1249207566.doc


O-MGCF







INVITE







INR







IAM







INF
















_1243337094.doc

[image: image1]

CS CN1















Termination2







Termination1







IM CN1















IM-MGW












_1248086239.vsd
IM MGW


.


MGCF


 1. 


H.248


: MOD.req


[C=


C1


,T=


T2


,


signal=SPCout
{h245mc=XXXX}


]


2. 


H.248


: MOD.resp


3. 


send H.245 message XXXX attached to MONA prefrence request


 


Signal SPC


.


3. 


send H.245 message XXXX attached to MONA prefrence request



_1231863464.doc


I-MGCF







183 Progress



P-Early-Media







CPG



 (in-band information available)







appropriate announcment







appropriate announcment







appropriate announcment







appropriate announcment












_1236684701.vsd
Process Name


SignalName


SignalName


SignalName


SignalName


Audio + Video


Interworking Node (MGCF+ IM MGW)


2. Modify Codec (Selected Codec=MuMe)


3. Successful Codec Modification


1. SIP: INVITE [m= AMR; m=H.263]


6. SIP: ACK


5. SIP :200 OK (INVITE) [m= AMR; m=H.263]


Speech


CS Network


IMS


4. MONA or H.245 inband negotiation



_1237863108.doc

[image: image1]

Context C1







Termination T1







Termination T2











Termination T3







Stream 1







Stream 3







Stream 2







CS-Domain







IMS











Termination:



T1 CS-Domain (CS-Bearer (BS30) for H.245 control, Speech, Video)



T2 Multiplexing (H.245 control, Speech, Video)



T3 Video (own RTP-stream) + Speech (own RTP-stream)







Stream:



Stream1 (between T1 and T2) data (H.245 control, speech, Video)



Stream2 (between T2 and T3) Video



Stream3 (between T2 and T3) Speech












_1240045830.doc

[image: image1]

I-MGCF







180 Ringing



P-Early-Media (1)







CPG (Alerting)



















Ring tone












_1236976165.vsd
IM MGW


.


MGCF


 1. 


H.248


: MOD.req


[C=


C1


,T=


T2


,


signal=H245msgout
{h245mc=XXXX}


]


2. 


H.248


: MOD.resp


3. 


send H.245 message XXXX in H.223 stream


 


Signal H245 Message



_1237800680.vsd
IM MGW


.


MGCF


 1. 


H.248


: ADD.req


[C=


C1


,T=


?


... ,Event=H245msgin]


2. 


H.248


: ADD.resp


3. 


receive H.245 message XXXX in H.223


 4. 


H.248


[C=


C1


,T=


T2


,


Event=H245msgin  {h245mc=XXXX}]


5. 


H.248


: Notify.resp


Notify H245 Message


: Notify.req


Add Multiplex Termination



_1236508894.doc


O-MGCF











ACM (no indication)







T i/w 2







INVITE







IAM












_1236684700.vsd
Process Name


SignalName


SignalName


SignalName


SignalName


Audio + Video


Interworking Node (MGCF+ IM MGW)


1. Modify Codec (Selected Codec=MuMe)


5. SIP: ACK


3. SIP: 200 OK (INVITE) [m= AMR; m=H.263]


2. SIP: INVITE [m= AMR; m=H.263]


Speech


CS Network


IMS


4. Successful Codec Modification


6. MONA or H.245 inband negotiation


7. SIP: INVITE [m= AMR; m=H.263]


8. SIP: 200 OK (INVITE) [m= AMR; m=H.263]



_1232364397.doc

[image: image1]

and the CS network side







Release the resources for the IMS side







4. 















6. SIP : 200 OK [BYE]



















5. BICC or ISUP: RLC







(optional)







H.245 Signalling to end the H.245 session







1. 







IMS







CS   Network







: BYE







SIP







3. 







: REL







BICC or ISUP







2. 







(MGCF+ IM-MGW)







Interworking node












_1236508823.doc


T i/w1 elapses







T i/w1 running







T i/w1 running







O-MGCF







ACM (no indication)







IAM











SAM







SAM







INVITE 












_1232304702.vsd
Process Name


SignalName


SignalName


SignalName


SignalName


Audio


Interworking Node (MGCF+ IM MGW)


Video


Video announcement


1. SIP: INVITE [m= AMR]


3. SIP: ACK


2. SIP :200 OK (INVITE) [m= AMR]


Audio + Video


CS Network


IMS



_1218416460.vsd
MGCF


IM CN Subsystem


REFER
To: B
P-Asserted ID: A
Refer to: C
Method: INVITE


403 Forbidden


CS Network



_1225109985.vsd
Process Name


SignalName


SignalName


SignalName


SignalName


Speech


Interworking Node (MGCF+ IM MGW)


1. Modify Codec (Selected Codec=AMR)


5. SIP: ACK


4. SIP: 200 OK (INVITE) [m= AMR]


2. SIP: INVITE [m= AMR]


Audio + Video


CS Network


IMS


3. Successful Codec Modification



_1231158551.vsd

_1231863431.doc


I-MGCF







ACM 



(No indication)







183 Progress



P-Early-Media







appropriate announcement












_1225110117.vsd
Process Name


SignalName


SignalName


SignalName


SignalName


Speech


Interworking Node (MGCF+ IM MGW)


1. SIP: INVITE [m= AMR; m=H.263]


3. SIP: ACK


2. SIP :200 OK (INVITE) [m= AMR]


Speech


CS Network


IMS



_1225109855.vsd
Process Name


SignalName


SignalName


SignalName


SignalName


Speech


Interworking Node (MGCF+ IM MGW)


2. Modify Codec (Selected Codec=AMR)


5. Successful Codec Modification


1. SIP: INVITE [m= AMR]


4. SIP: ACK


3. SIP :200 OK (INVITE) [m= AMR]


Audio + Video


CS Network


IMS



_1208694026.vsd
Process Name


SignalName


SignalName


MGCF


IM-MGW


Bearer Released


Release Termination


10. SIP: 200 OK [BYE]


1a. H.248: Notify.req [Context ID = C1, Termination ID = T2]


2a. H.248: Notify.resp [Context ID = C1, Termination ID = T2]


Release IMS Termination


4. SIP: BYE


3. BICC: REL


5. BICC: RLC


8. H.248: SUB.req [Context ID = C1, Termination ID = T1]


9. H.248: SUB.resp [Context ID = C1, Termination ID = T1]


6. H.248: SUB.req [Context ID = C1,Termination ID = T2]


7. H.248: SUB.resp [Context ID = C1, Termination ID = T2 ] 


1b. H.248: Notify.req [Context ID = C1, Termination ID = T1]


2b. H.248: Notify.resp [Context ID = C1, Termination ID = T1]


IMS Bearer Released


or



_1208694324.vsd
Process Name


SignalName


SignalName


MGCF


IM-MGW


9. ISUP: RLC


Termination 
Out-of-Service


10. SIP: 200 OK [BYE]


1a. H.248: ServiceChange.req
[Context ID = C1, Termination ID = T2]


2a. H.248: ServiceChange.resp 
[Context ID = C1, Termination ID = T2]


3. ISUP: REL


4. SIP: BYE


5. H.248: SUB.req [Context ID = C1, Termination ID = T1]


6. H.248: SUB.resp [Context ID = C1, Termination ID = T1]


7. H.248: SUB.req [Context ID = C1,Termination ID = T2]


8. H.248: SUB.resp [Context ID = C1, Termination ID = T2 ] 


Release IMS Termination


Release TDM Termination


1b. H.248: Notify.req [Context ID = C1, Termination ID = T2]


2b. H.248: Notify.resp [Context ID = C1, Termination ID = T2]


Bearer Released


or


1c. H.248: Notify.req [Context ID = C1, Termination ID = T1]


2c. H.248: Notify.resp [Context ID = C1, Termination ID = T1]


or


IMS Bearer Released



_1200029813.doc
 


IP







IP







SIP







TCP /



UDP /



SCTP







IP







IP







IP



















STCMTP







BICC















SIP







M3UA







BICC







SCTP







IP







TCP /



UDP / 



SCTP







SIP







BICC







M3UA







SCTP







STCMTP































Media gateway



control function











SS7 signalling



function







SIP signalling



function












_1107613990.doc


O-MGCF







INVITE 







SAM







IAM












_1162169915.doc


O-MGCF







COT(success)



‚9







 SDP indicating  pre-conditions  met     












_1195890909.vsd
Process Name


SignalName


SignalName


MGCF


IM-MGW


14. SIP: UPDATE


15. SIP :200 OK (UPDATE)


16. BICC: COT 


17. BICC: ACM 


18. SIP :180 Ringing


22. H.248:  MOD.req [Context ID=C1, Termination ID = T1]


1. SIP: INVITE 


2. SIP: 100 Trying


5. BICC: IAM 


23. H.248: MOD.resp [Context ID = C1, Termination ID = T1] 


21. BICC: ANM 


19. SIP: PRACK


20. SIP :200 OK (PRACK)


24. SIP :200 OK (INVITE)


25. SIP: ACK


CALL IN ACTIVE STATE


Reserve IMS Connection Point and Configure Remote Resourcest,
Change IMS Through- Connection = backward 


Bearer Establishment


6. BICC: APM 


7. H.248: ADD.req [Context ID = C1, Termination ID = ?]


8. H.248: ADD.resp [Context ID = C1, Termination ID = T2] 


Establish Bearer, Change
 Through-Connection = both 


Configure IMS Resources


3. H.248: ADD.req  [Context ID = ?, Termination ID=?]


4. H.248: ADD.resp [Context ID = C1, Termination ID = T1]


Change IMS 
Through-Connection = both 


9. SIP:183 Session Progress 


10. SIP: PRACK


11. SIP :200 OK (PRACK) 


12. H.248: MOD.req [Context ID = C1,Termination ID = T1]


13. H.248: MOD.resp [Context ID = C1, Termination ID = T1 ] 


Only if SIP 
Precon-
ditions 
are used


Only if BICC continuity check and/or SIP 
Preconditions 
are used


Optional


if SIP 
100 rel is  used


if SIP 
100 rel is  used



_1199974927.doc
 


IP







IP







SIP







TCP / 



UDP /



SCTP







IP







IP







IP







M3UA







SCTP







IP







STC







BICC







MTP3







MTP2







SIP







MTP3







BICC







MTP2







L1







TCP / 



UDP /



SCTP







SIP







BICC







M3UA







SCTP







STC











L1



















SS7







Media gateway



control function







Signalling gateway



function







SS7 signalling



function







SIP signalling



function












_1199974961.doc
 


IP







IP







SIP







TCP /



UDP /



SCTP







IP







IP







IP







M3UA







SCTP







IP







STC







BICC







MTP3B







SSCF







SIP







MTP3B







BICC







SSCF







SSCOP







TCP /



UDP /



SCTP







SIP







BICC







M3UA







SCTP







STC







AAL5







SSCOP











AAL5











AAL5







Media gateway



control function







Signalling gateway



function







SS7 signalling



function







SIP signalling



function












_1195891134.vsd
Process Name


SignalName


SignalName


MGCF


IM-MGW


13. SIP: UPDATE


14. SIP :200 OK (UPDATE)


15. BICC: COT 


16. BICC: ACM 


17. SIP :180 Ringing


21. H.248:  MOD.req  [Context ID=C1, Termination ID = T1]


1. SIP: INVITE 


2. SIP: 100 Trying


8. BICC: IAM 


22. H.248: MOD.resp [Context ID = C1, Termination ID = T1] 


20. BICC: ANM 


18. SIP: PRACK


19. SIP :200 OK (PRACK)


25. SIP :200 OK (INVITE)


26. SIP: ACK


CALL IN ACTIVE STATE


Reserve IMS Connection Point
and Configure Remote Resources,
Change IMS Through
-Connection = backward )


Bearer Establishment


Configure IMS Resources


6. H.248: ADD.req [Context ID = C1, Termination ID = ?]


7. H.248: ADD.resp [Context ID = C1, Termination ID = T2] 


Prepare Bearer,
Change Through-Connection = both


Change IMS Through-
Connection = both


3. H.248: ADD.req [Context ID = ?, Termination ID=?]


4. H.248: ADD.resp [Context ID = C1, Termination ID = T1]


5. SIP:183 Session Progress 


9. SIP: PRACK


12. SIP :200 OK  (PRACK)


 10. H.248: MOD.req [Context ID = C1,Termination ID = T1]


11. H.248: MOD.resp [Context ID = C1, Termination ID = T1 ] 


Only if SIP 
Precon-
ditions 
are used


Only if BICC continuity check and/or SIP 
Preconditions 
are used


Optional


if SIP 
100 rel is  used


if SIP 
100 rel is  used



_1191684916.doc


O-MGCF







INVITE 







IAM







COT (NOTE)












_1192628822.vsd
Process Name


SignalName


SignalName


MGCF


IM-MGW


13. SIP: UPDATE


14. SIP :200 OK (UPDATE)


15. ISUP: COT 


16. ISUP: ACM 


17. SIP :180 Ringing


21. H.248:  MOD.req  [Context ID=C1, Termination ID = T1]


1. SIP: INVITE 


2. SIP: 100 Trying


8. ISUP: IAM 


22. H.248: MOD.resp [Context ID = C1, Termination ID = T1] 


20. ISUP: ANM 


18. SIP: PRACK


19. SIP :200 OK (PRACK)


25. SIP :200 OK (INVITE)


26. SIP: ACK


CALL IN ACTIVE STATE


23. H.248:  MOD.req ) [Context ID=C1, Termination ID = T2]


Reserve IMS Connection
Point, and Configure
Remote Resources
Change IMSThrough-Connection = backward 


Configure IMS Resources


6. H.248: ADD.req [Context ID = C1, Termination ID = T2]


7. H.248: ADD.resp [Context ID = C1, Termination ID = T2] 


24. H.248: MOD.resp [Context ID = C1, Termination ID = T2] 


Reserve TDM Circuit,
Change  TDM Through-Connection= both


3. H.248: ADD.req [Context ID = ?, Termination ID=?]


4. H.248: ADD.resp [Context ID = C1, Termination ID = T1]


Change IMS Through
-Connection = both


Activate TDM Voice 
Prosessing Function


5. SIP:183 Session Progress 


9. SIP: PRACK


12. SIP :200 OK  (PRACK)


 10. H.248: MOD.req [Context ID = C1,Termination ID = T1]


11. H.248: MOD.resp [Context ID = C1, Termination ID = T1 ] 


Only if SIP 
Precon-
ditions 
are used


Only if BICC continuity check and/or SIP 
Preconditions 
are used


Optional


if SIP 
100 rel is  used


if SIP 
100 rel is  used



_1195889998.vsd
Process Name


SignalName


SignalName


IM-MGW


MGCF


14. SIP: UPDATE


15. SIP :200 OK (UPDATE)


13. BICC: COT 


18. BICC: ACM 


16. SIP :180 Ringing


Establish Bearer, Change
Through-Connection  = both 


6. SIP: INVITE 


7. SIP: 100 Trying


1. BICC: IAM 


Reserve IMS Connection Point,
Change IMSThrough-
Connection  = backward 


Configure IMS Resources


17. SIP: PRACK


21. SIP :200 OK (PRACK)


Send Tone


Bearer Establishment


2. H.248: ADD.req  [Context ID = ?, Termination ID = ?]


3. H.248: ADD.resp [Context ID = C1, Termination ID = T2] 


4. H.248: ADD.req [Context ID = C1, Termination ID=?]


5. H.248: ADD.resp [Context ID = C1, Termination ID = T1]


19. H.248: MOD.req  [Context ID = C1, Termination ID = T2]


20. H.248: MOD.resp [Context ID = C1, Termination ID = T2] 


8. SIP:183 Session Progress 


11. SIP: PRACK


12. SIP :200 OK (PRACK) 


9. H.248: MOD.req [Context ID = C1,Termination ID = T1]


10.H.248: MOD.resp [Context ID = C1, Termination ID = T1 ] 


if SIP 
Precon-ditions  and/or 
100 rel are used


if SIP 
Precon-ditions  and/or 
100 rel are used


Optional


if continuity  check is used



_1195890333.vsd
Process Name


SignalName


SignalName


IM-MGW


MGCF


14. SIP: UPDATE


15. SIP :200 OK (UPDATE)


13. ISUP: COT 


18. ISUP: ACM 


16. SIP :180 Ringing


Reserve TDM Circuit,
Change Through-
Connection  = both  


6. SIP: INVITE 


7. SIP: 100 Trying


1. ISUP: IAM 


Reserve IMS Connection Point, Change IMS Through-
Connection  = backward 


Configure IMS  Resources


17. SIP: PRACK


21. SIP :200 OK (PRACK)


Send TDM Tone


2. H.248: ADD.req [Context ID = ?, Termination ID = ?]


3. H.248: ADD.resp [Context ID = C1, Termination ID = T2] 


4. H.248: ADD.req [Context ID = C1, Termination ID=?]


5. H.248: ADD.resp [Context ID = C1, Termination ID = T1]


19. H.248: MOD.req  [Context ID = C1, Termination ID = T2]


20. H.248: MOD.resp [Context ID = C1, Termination ID = T2] 


8. SIP:183 Session Progress 


11. SIP: PRACK


12. SIP :200 OK (PRACK) 


9. H.248: MOD.req [Context ID = C1,Termination ID = T1]


10.H.248: MOD.resp [Context ID = C1, Termination ID = T1 ] 


if SIP 
Precon-ditions  and/or 
100 rel are used


if SIP 
Precon-ditions  and/or 
100 rel are used


Optional


if continuity  check is used



_1195889426.vsd
Process Name


SignalName


SignalName


IM-MGW


MGCF


15. SIP: UPDATE


16. SIP :200 OK (UPDATE)


14. BICC: COT 


19. BICC: ACM 


17. SIP :180 Ringing


Reserve IMS Connection Point, Change IMS Through-
Connection = backward 


4. SIP: INVITE 


5. SIP: 100 Trying


1. BICC: IAM 


Configure IMS  Resources


Prepare Bearer,
Change Through-Connection= both


18. SIP: PRACK


22. SIP :200 OK (PRACK)


Send Tone


Bearer Establishment


11. H.248: ADD.req [Context ID = C1, Termination ID = ?]


12. H.248: ADD.resp [Context ID = C1, Termination ID = T2] 


2. H.248: ADD.req  [Context ID = ?, Termination ID=?]


3. H.248: ADD.resp [Context ID = C1, Termination ID = T1]


13. BICC: APM 


20. H.248: MOD.req  [Context ID = C1, Termination ID = T2]


21. H.248: MOD.resp [Context ID = C1, Termination ID = T2] 


6. SIP:183 Session Progress 


9. SIP: PRACK


10. SIP :200 OK  (PRACK)


7. H.248: MOD.req Context ID = C1,Termination ID = T1]


  8. H.248: MOD.resp [Context ID = C1, Termination ID = T1 ] 


if SIP 
Precon-ditions  and/or 
100 rel are used


if SIP 
Precon-ditions  and/or 
100 rel are used


Optional


if continuity  check is used



_1192628038.vsd
Process Name


SignalName


SignalName


SignalName


SignalName


IM-MGW


MGCF


23. H.248:  MOD.req 

 [Context ID=C1, Termination ID = T2]


24. H.248: MOD.resp [Context ID = C1, Termination ID = T2] 


28. SIP: ACK


25. H.248:  MOD.req 
 [Context ID=C1, Termination ID = T1]


26. H.248: MOD.resp [Context ID = C1, Termination ID = T1] 


27. BICC: ANM 


22. SIP :200 OK (INVITE)


CALL IN ACTIVE STATE


Stop Tone


Change IMS Through-Connection  = both


22a. H.248: MOD.req 
[Context ID = C1,Termination ID = T1]


  22b. H.248: MOD.resp
[Context ID = C1, Termination ID = T1 ] 


if SIP 
Precon-ditions  and
100 rel are not used


If  tone was inserted


Configure IMS  Resources



_1192628234.vsd
Process Name


SignalName


SignalName


SignalName


SignalName


IM-MGW


MGCF


23. H.248:  MOD.req  [Context ID=C1, Termination ID = T2]


24. H.248: MOD.resp [Context ID = C1, Termination ID = T2] 


28. SIP: ACK


25. H.248:  MOD.req [Context ID=C1, Termination ID = T1]


26. H.248: MOD.resp [Context ID = C1, Termination ID = T1] 


27. ISUP: ANM 


22. SIP :200 OK (INVITE)


CALL IN ACTIVE STATE


Stop TDM Tone ,
Activate TDM Voice Processing Function


Change IMS Through-
Connection  = both


22a. H.248: MOD.req 
[Context ID = C1,Termination ID = T1]


  22b. H.248: MOD.resp
[Context ID = C1, Termination ID = T1 ] 


if SIP 
Preconditions  and
100 rel are not used


If  tone was inserted or for voive processing


Configure IMS  Resources



_1179665514.doc


[image: image1.wmf]MGCF


1


. 


BICC


/


ISUP


: 


CPG 


(


Hold


) 


3


. 


SIP


: 


200 


OK 


[


SDP


]


2


. 


SIP


: 


UPDATE 


[


SDP


, 


a


=


sendonly


/


inactive


]


5


. 


SIP


: 


UPDATE 


[


SDP


, 


a


=


sendrecv


/


recvonly


]


4


. 


BICC


/


ISUP


: 


CPG 


(


Retrieve


)


6


. 


SIP


: 


200 


OK 


[


SDP


]




_1174295882.vsd

Process Name



SignalName



SignalName



MGCF



3. SIP: 200 OK [SDP]



5. SIP: UPDATE [SDP, a=sendrecv/recvonly]



4. BICC/ISUP: CPG (Retrieve)



2. SIP: UPDATE [SDP, a=sendonly/inactive]



1. BICC/ISUP: CPG (Hold) 



6. SIP: 200 OK [SDP]






_1179666048.doc


[image: image1.wmf]MGCF


IM


-


MGW


1


. 


BICC


/


ISUP


: 


CPG 


(


Hold


) 


2


. 


H


.


248


: 


MOD


.


req   


[


Context ID 


= 


C


1


, 


Termination ID 


= 


T


1


]


3


. 


H


.


248


: 


MOD


.


resp 


[


Context ID 


= 


C


1


, 


Termination ID 


= 


T


1


]


5


. 


SIP


: 


200 


OK 


[


SDP


]


4


. 


SIP


: 


UPDATE


/


INVITE 


[


SDP


, 


a


=


sendonly


/


inactive


]


6


. 


BICC


/


ISUP


: 


CPG 


(


Retrieve


)


Configure IMS 


Resources


(


optional


)


7


. 


H


.


248


: 


MOD


.


req   


[


Context ID 


= 


C


1


, 


Termination ID 


= 


T


1


]


8


. 


H


.


248


: 


MOD


.


resp 


[


Context ID 


= 


C


1


, 


Termination ID 


= 


T


1


]


10


. 


SIP


: 


200 


OK 


[


SDP


]


9


. 


SIP


: 


UPDATE


/


INVITE 


[


SDP


, 


a


=


sendrecv


/


recvonly


]


Configure IMS 


Resources


(


optional


)


5


.


a 


SIP


: 


ACK 


(


if INVITE is used


)


10


.


a 


SIP


: 


ACK 


(


if INVITE is used


)




_1176019897.vsd

Process Name



SignalName



SignalName



MGCF



IM-MGW



7. H.248: MOD.req   
[Context ID = C1, Termination ID = T1]



8. H.248: MOD.resp 
[Context ID = C1, Termination ID = T1]



5. SIP: 200 OK [SDP]



10. SIP: 200 OK [SDP]



6. BICC/ISUP: CPG (Retrieve)



9. SIP: UPDATE/INVITE [SDP, a=sendrecv/recvonly]



4. SIP: UPDATE/INVITE [SDP, a=sendonly/inactive]



1. BICC/ISUP: CPG (Hold) 



Configure IMS Resources
(optional)



Configure IMS Resources
(optional)



2. H.248: MOD.req   
[Context ID = C1, Termination ID = T1]



3. H.248: MOD.resp 
[Context ID = C1, Termination ID = T1]



5.a SIP: ACK (if INVITE is used)



10.a SIP: ACK (if INVITE is used)






_1191678822.vsd
Process Name


SignalName


SignalName


SignalName


SignalName


IM-MGW


MGCF


24. H.248:  MOD.req
 [Context ID=C1, Termination ID = T2]


25. H.248: MOD.resp [Context ID = C1, Termination ID = T2] 


29. SIP: ACK


26. H.248:  MOD.req  [Context ID=C1, Termination ID = T1]


27. H.248: MOD.resp [Context ID = C1, Termination ID = T1] 


28. BICC: ANM 


23. SIP :200 OK (INVITE)


CALL IN ACTIVE STATE



Stop Tone 


Change IMS 
Through-Connection  = both


23a. H.248: MOD.req 
[Context ID = C1,Termination ID = T1]


  23b. H.248: MOD.resp
[Context ID = C1, Termination ID = T1 ] 


Configure IMS  Resources


if SIP 
Precon-ditions  and
100 rel are not used


If  tone was inserted



_1179665798.doc


[image: image1.wmf]MGCF


IM


-


MGW


4


. 


BICC


/


ISUP


: 


CPG 


(


Hold


) 


2


. 


H


.


248


: 


MOD


.


req   


[


Context ID 


= 


C


1


, 


Termination ID 


= 


T


1


]


3


. 


H


.


248


: 


MOD


.


resp 


[


Context ID 


= 


C


1


, 


Termination ID 


= 


T


1


]


11


. 


H


.


248


: 


MOD


.


req  


[


Context ID 


= 


C


1


, 


Termination ID 


= 


T


1


]


12


. 


H


.


248


: 


MOD


.


resp 


[


Context ID 


= 


C


1


, 


Termination ID 


= 


T


1


]


7


. 


SIP


: 


200 


OK 


[


SDP


]


1


. 


SIP


: 


UPDATE


/


INVITE 


[


SDP


, 


a


=


sendonly


/


inactive


]


8


. 


SIP


: 


UPDATE


/


INVITE 


[


SDP


, 


a


=


sendrecv


/


recvonly


]


13


. 


BICC


/


ISUP


: 


CPG 


(


Retrieve


)


14


. 


SIP


: 


200 


OK 


[


SDP


]


7


.


a 


SIP


: 


ACK 


(


if INVITE is used


)


14


.


a 


SIP


: 


ACK 


(


if INVITE is used


)


5


. 


H


.


248


: 


MOD


.


req 


[


Context ID 


= 


C


1


, 


Termination ID 


= 


T


2


]


9


. 


H


.


248


: 


MOD


.


req   


[


Context ID 


= 


C


1


, 


Termination ID 


= 


T


2


]


Play TDM 


Announcement


Stop TDM 


Announcement


Change Through


-


Connection


=


Inactive


Change Through


-


Connection


=


Both


6


. 


H


.


248


: 


MOD


.


resp 


[


Context ID 


= 


C


1


, 


Termination ID 


= 


T


2


]


10


. 


H


.


248


: 


MOD


.


resp 


[


Context ID 


= 


C


1


, 


Termination ID 


= 


T


2


]




_1175280662.vsd

Process Name



SignalName



SignalName



MGCF



IM-MGW



11. H.248: MOD.req  
[Context ID = C1, Termination ID = T1]



12. H.248: MOD.resp 
[Context ID = C1, Termination ID = T1]



7. SIP: 200 OK [SDP]



8. SIP: UPDATE/INVITE [SDP, a=sendrecv/recvonly]



13. BICC/ISUP: CPG (Retrieve)



5. H.248: MOD.req 
[Context ID = C1, Termination ID = T2]



1. SIP: UPDATE/INVITE [SDP, a=sendonly/inactive]



9. H.248: MOD.req   
[Context ID = C1, Termination ID = T2]



4. BICC/ISUP: CPG (Hold) 



14. SIP: 200 OK [SDP]



7.a SIP: ACK (if INVITE is used)



14.a SIP: ACK (if INVITE is used)



Play TDM 
Announcement



Stop TDM 
Announcement



Change Through-Connection=Inactive



Change Through-Connection=Both



6. H.248: MOD.resp 
[Context ID = C1, Termination ID = T2]



10. H.248: MOD.resp 
[Context ID = C1, Termination ID = T2]



2. H.248: MOD.req   
[Context ID = C1, Termination ID = T1]



3. H.248: MOD.resp 
[Context ID = C1, Termination ID = T1]






_1179665289.doc


[image: image1.wmf]MGCF


2


. 


BICC


/


ISUP


: 


CPG 


(


Hold


) 


3


. 


SIP


: 


200 


OK 


[


SDP


]


1


. 


SIP


: 


UPDATE 


[


SDP


, 


a


=


sendonly


/


inactive


]


4


. 


SIP


: 


UPDATE 


[


SDP


, 


a


=


sendrecv


/


recvonly


]


5


. 


BICC


/


ISUP


: 


CPG 


(


Retrieve


)


6


. 


SIP


: 


200 


OK 


[


SDP


]




_1174717731.vsd

Process Name



SignalName



SignalName



MGCF



3. SIP: 200 OK [SDP]



4. SIP: UPDATE [SDP, a=sendrecv/recvonly]



5. BICC/ISUP: CPG (Retrieve)



1. SIP: UPDATE [SDP, a=sendonly/inactive]



2. BICC/ISUP: CPG (Hold) 



6. SIP: 200 OK [SDP]






_1123499430.doc


O-MGCF







UPDATE







COT (success)












_1127921368.vsd

_1145945660.vsd

_1145946234.vsd

_1145946297.vsd

_1145946106.vsd

_1142340211.doc
[image: image1.wmf] 


Mb


 




Transcoder-less user plane interworking







IM-MGW







L1







L2







IP







 UDP







 RTP







Nb











L3











L1







Nb FP



















L2







�












_1123572103.vsd

_1127825164.doc


2. H.248 Add/Mod.resp [Context ID = C1, Termination ID = T1]







MGCF







IM-MGW







1. H.248 Add/Mod.req 



[Context ID = C1, Termination ID = T1,�Codec = “telephone event, AMR”,]







Configure IMS Resources



Or



Reserve IMS Conection Point and configure Remote Resources












_1127921285.vsd

_1123572347.vsd

_1123499536.doc


O-MGCF







CANCEL







COT (failure)












_1107615237.doc


O-MGCF







Status Code 4xx, 5xx or 6xx







REL












_1122715052.vsd

_1122716402.vsd

_1122716648.vsd

_1122716190.vsd

_1122186000.doc


O-MGCF







SIP response code 3xx







REL



(Cause value 127)












_1112523121.vsd

_1107614972.doc


O-MGCF







ANM







200 OK (INVITE)












_1107615119.doc


O-MGCF







CON







200 OK (INVITE)












_1107614491.doc


O-MGCF







Ring tone







ACM (Subscriber Free)







180 Ringing












_1107613350.doc


200 OK  (INVITE)







ANM







I-MGCF












_1107613595.doc


I-MGCF







REL







BYE












_1107613739.doc


REL







CANCEL







I-MGCF












_1107613404.doc


200 OK (INVITE)







CON







I-MGCF












_1099482568.doc


AMR







IM-MGW







L1







L2







IP







 UDP







 RTP







voice











codec











L1







TNL



















Transcoding







Mb












_1107612327.doc






I-MGCF







IAM







INVITE












_1107612598.doc


IAM







SDP indicating pre-conditions met







INVITE







I-MGCF












_1106662881.doc


CSCF







CS network







BICC over SCTP/IP







IM-



MGW







MGCF







SGW











Mb







CS channels







e.g. PCM







BICC/ISUP



over MTP







(Note 3)







Mn







User Plane







Control Plane







BICC/ISUP over SCTP/IP







Mj











BGCF







Mg












_144825468.vsd
3. INVITE (2)


4. SAM


1. INVITE (1)


I-MGCF


2. IAM


5. 484 (1)


6. INVITE (3)


7. SAM


8. 484 (2)


9.ACM


10. 180 (3)



_1099463145.doc


AMR







IM-MGW







L1







L2







IP







 UDP







 RTP







G.711







PCM







Coding











L1







TDM







 CS







Bearer







Channel







Transcoding







Mb












_1097476123.unknown

_144492612.doc


I-MGCF







SAM







 SIP message carrying additional digits    












