3GPP TS 29.213 V14.0.0 (2016-06)
Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Core Network and Terminals;

Policy and Charging Control signalling flows and Quality of Service (QoS) parameter mapping
(Release 14)

[image: image1.jpg]



[image: image2.png]=

A GLOBAL INITIATIVE




The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners’ Publications Offices.

Keywords

UMTS, LTE, QoS, Charging, Policy
3GPP

Postal address

3GPP support office address

650 Route des Lucioles – Sophia Antipolis

Valbonne – France
Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org
Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2016, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).
All rights reserved.
UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI currently being registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association


Contents

9Foreword

1
Scope
10
2
References
10
3
Definitions and abbreviations
12
3.1
Definitions
12
3.2
Abbreviations
13
3a
Reference architecture
13
4
Signalling Flows over Gx, Gxx, Rx, Sd, Sy, Np, Nt, St and S9
17
4.0
General
17
4.1
IP-CAN Session Establishment
18
4.2
IP-CAN Session Termination
22
4.2.1
UE-Initiated
22
4.2.1.1
AF located in the HPLMN
22
4.2.1.2
AF located in the VPLMN
25
4.2.2
PCEF-Initiated
27
4.2.2.1
AF located in the HPLMN
27
4.2.2.2
AF located in the VPLMN
29
4.2.3
PCRF-Initiated
30
4.2.3.1
AF located in the HPLMN
30
4.2.3.2
AF located in the VPLMN
32
4.3
IP-CAN Session Modification
34
4.3.1
Network-Initiated IP-CAN Session Modification
34
4.3.1.1
Interactions between BBERF, PCEF, TDF, OCS, TSSF and PCRF(PCC/QoS/ADC Rule Provisioning in PUSH mode)
34
4.3.1.2
Interactions between PCRF, AF and SPR
39
4.3.1.2.1
AF Session Establishment
39
4.3.1.2.1.1
AF located in HPLMN
39
4.3.1.2.1.2
AF located in VPLMN
40
4.3.1.2.2
AF session modification
42
4.3.1.2.2.2
AF located in the VPLMN
43
4.3.1.2.3
AF session termination
44
4.3.2
PCEF –Initiated IP-CAN Session Modification (PCC Rule Provisioning in PULL Mode)
46
4.3.2.1
PCEF-initiated IP-CAN Session Modification. AF located in HPLMN.
46
4.3.2.2
PCEF-initiated IP-CAN Session Modification, AF located in the VPLMN
51
4.4
Gateway Control Session Procedures
52
4.4.1
Gateway Control Session Establishment
53
4.4.2
Gateway Control and QoS Rules Request
57
4.4.2.1
Non-Roaming and Home Routed cases
57
4.4.2.2
Visited access cases
59
4.4.3
Gateway Control and QoS Rules Provision
60
4.4.4
Gateway Control Session Termination
61
4.4.4.1
BBERF-Initiated Gateway Control Session Termination
61
4.4.4.2
PCRF-Initiated Gateway Control Session Termination
63
4.5
Multiple BBERF Signalling Flows
64
4.5.1
Non-Roaming and Home Routed cases
64
4.5.1.1
New Gateway Control Session Establishment
64
4.5.1.2
PCEF IP-CAN session modification – Handover
67
4.5.1.3
PCEF IP-CAN session modification – S2c-based IP flow mobility
68
4.5.1.4
Gateway Control Session Establishment and PCEF IP-CAN session modification – S2c-based IP flow mobility
70
4.5.2
Visited access case
71
4.5.2.1
New Gateway Control Session Establishment
71
4.5.2.2
PCEF-Initiated IP-CAN session modification-Handover
73
4.5.2.3
PCEF-Initiated IP-CAN session modification - S2c-based IP flow mobility
75
4.5.2.4
Gateway Control Session Establishment and PCEF IP-CAN session modification – S2c-based IP flow mobility
77
4.6
Application Detection and Enforcement Procedures
79
4.6.1
TDF Session Establishment in case of solicited reporting
79
4.6.1A
TDF Session Establishment in case of unsolicited reporting
80
4.6.2
TDF Session termination
80
4.6.3
TDF Session modification
82
4.6.3.1
Application Detection, Reporting and Control Rules Request
82
4.6.3.2
Application Detection and Control Rules Provision
83
4.7
Spending limits Procedures over Sy reference point
84
4.7.1
Initial Spending Limit Report Request
84
4.7.2
Intermediate Spending Limit Report Request
84
4.7.3
Final Spending Limit Report Request
85
4.7.4
Spending Limit Report
86
4.8
Call flows for User Plane Congestion Management
86
4.8.1
General
86
4.8.2
RUCI reporting
87
4.8.2.1
Non-aggregated RUCI report procedure
87
4.8.2.2
Aggregated RUCI report procedure
87
4.8.3
Np reporting restriction provisioning
88
4.8.4
UE mobility between RCAFs
88
4.8.5
Removal of UE context
89
4.9
Traffic Steering Control Procedures over St reference point
89
4.9.1
St Session Establishment
89
4.9.2
St Session Modification
90
4.9.3
St Session Termination
91
4.9.4
St notification initiated by the TSSF
91
4.10
Negotiation for future background data transfer procedure over Nt reference point
92
5
Binding Mechanism
92
5.1
Overview
92
5.2
Session Binding
93
5.3
PCC Rule Authorization and QoS Rule Generation
94
5.4
Bearer Binding
95
6
QoS Parameters Mapping
96
6.1
Overview
96
6.1.1
UE-Initiated IP-CAN bearers
97
6.1.2
Network-Initiated IP-CAN bearers
99
6.2
QoS parameter mapping Functions at AF
100
6.3
QoS parameter mapping Functions at PCRF
107
6.4
QoS parameter mapping Functions at PCEF
114
6.4.1
GPRS
114
6.4.1.1
Authorized IP QoS parameters per PDP Context to Authorized UMTS QoS parameters mapping in GGSN
114
6.4.1.2
Comparing UMTS QoS Parameters against the Authorized UMTS QoS parameters in GGSN for UE initiated PDP context
116
6.4.2
3GPP- EPS
116
6.4.2.1
Authorized IP QoS parameters per PDP Context to Authorized UMTS QoS parameters mapping in P-GW.
116
6.4.2.2
Comparing UMTS QoS Parameters against the Authorized UMTS QoS parameters in P-GW for UE initiated PDP context
118
6.5
QoS parameter mapping Functions at UE for a UE-initiated GPRS PDP Context
118
6.5.1
SDP to UMTS QoS parameter mapping in UE
120
6.5.2
SDP parameters to Authorized UMTS QoS parameters mapping in UE
120
7
PCRF addressing
124
7.1
General
124
7.2
DRA Definition
124
7.3
DRA Procedures
124
7.3.1
General
124
7.3.2
DRA Information Storage
124
7.3.3
Capabilities Exchange
125
7.3.4
Redirect DRA
125
7.3.4.1
Redirecting Diameter Requests
125
7.3.4.2
DRA binding removal
126
7.3.5
Proxy DRA
126
7.3.6
PCRF selection by BBERF/PCEF (non-roaming case)
126
7.3.7
PCRF selection by AF
127
7.3.8
PCRF selection in a roaming scenario
127
7.3.9
PCRF selection by TDF for unsolicited application reporting
128
7.3.10
PCRF selection by RCAF
128
7.4
DRA flows
128
7.4.1
Proxy DRA
128
7.4.1.1
Establishment of Diameter Sessions
128
7.4.1.1.1
Non-roaming cases
128
7.4.1.1.2
Roaming cases
129
7.4.1.2
Modification of Diameter Sessions
130
7.4.1.2.1
Non-roaming cases
130
7.4.1.2.1.1
Client-initiated
130
7.4.1.2.1.2
PCRF-initiated
131
7.4.1.2.2
Roaming cases
132
7.4.1.2.2.1
V-PCRF initiated
132
7.4.1.2.2.2
H-PCRF initiated
133
7.4.1.3
Termination of Diameter Sessions
134
7.4.1.3.1
Non-roaming cases
134
7.4.1.3.2
Roaming cases
135
7.4.2
Redirect DRA
136
7.4.2.1
Establishment of Diameter Sessions
136
7.4.2.1.1
Non-roaming cases
136
7.4.2.1.2
Roaming cases
137
7.4.2.2
Modification of Diameter sessions
138
7.4.2.3
Termination of Diameter Sessions
138
7.4.2.3.1
Non-roaming cases
138
7.4.2.3.2
Roaming cases
139
8
Diameter race condition handling
141
8.1
Overview
141
8.2
Procedures for Gx, Gxx, Sd and S9
141
Annex A (informative):
Examples of deriving the Maximum Authorized parameters from the SDP parameters
143
Annex B (normative):
Signalling Flows for IMS
144
B.0
General
144
B.1
Subscription to Notification of Signalling Path Status at IMS Registration
144
B.1a
Subscription to Notification of Change of IP-CAN Type at IMS Registration
145
B.1b
Provisioning of SIP signalling flow information at IMS Registration
146
B.2
IMS Session Establishment
147
B.2.1
Provisioning of service information at Originating P-CSCF and PCRF
147
B.2.2
Provisioning of service information at terminating P-CSCF and PCRF
150
B.3
IMS Session Modification
154
B.3.1
Provisioning of service information
154
B.3.2
Enabling of IP Flows
158
B.3.3
Disabling of IP Flows
159
B.3.4
Media Component Removal
160
B.4
IMS Session Termination
161
B.4.1
Mobile initiated session release / Network initiated session release
161
B.4.2
IP-CAN Bearer Release/Loss
163
B.5
P-CSCF Restoration
164
Annex C (normative):
NAT Related Procedures
166
C.1
Support for media traversal of NATs using ICE
166
C.2
P-CSCF procedures
166
C.2.1
General
166
C.2.2
Deriving the Ues IP address
167
C.2.3
Deriving flow descriptions
167
C.2.4
Gating control
167
C.2.5
Bandwidth impacts
167
C.3
PCRF procedures
168
C.3.1
General
168
C.3.2
Deriving additional flow descriptions
168
C.3.3
Gating control
168
C.3.4
Bandwidth impacts
168
C.4
P_CSCF procedures to support media traversal through hosted NAT without ICE
168
Annex D (normative):
Access specific procedures for GPRS
170
D.1
General
170
D.2
Binding Mechanisms
170
D.3
PCC Procedures
171
D.3.1
IP-CAN Session Modification
171
D.3.1.1
Network-initiated IP-CAN Session Modification
171
D.3.1.2
PCEF-initiated IP-CAN Session Modification
171
D.3.1.2.1
UE-initiated IP-CAN Bearer Establishment or IP-CAN Bearer Modification
171
D.3.1.2.2
UE-initiated IP-CAN Bearer Termination
174
Annex E (normative):
Fixed Broadband Access Interworking with EPC
177
E.1
General
177
E.2
Definitions and abbreviations
177
E.2.1
Definitions
177
E.2.2
Abbreviations
177
E.3
Binding Mechanisms
177
E.3.1
EPC-routed traffic
177
E.3.2
NSWO traffic
177
E.4
PCC Procedures
178
E.4.1
Introduction
178
E.4.2
IP-CAN Session Establishment
179
E.4.2.1
IP-CAN Session Establishment for EPC- routed traffic
179
E.4.2.2
IP-CAN Session Establishment for NSWO traffic
183
E.4.3
IP-CAN Session Termination
185
E.4.3.1
IP-CAN Session Termination for EPC- routed traffic
185
E.4.3.2
IP-CAN Session Termination for NSWO traffic
189
E.4.3.2.1
BPCF-initiated IP-CAN Session Termination for NSWO traffic
189
E.4.3.2.2
PCRF-initiated IP-CAN Session Termination for NSWO traffic
191
E.4.4
IP-CAN Session Modification
192
E.4.4.1
IP-CAN Session Modification for EPC-routed traffic
192
E.4.4.1.1
PCRF-initiated IP-CAN Session Modification
192
E.4.4.1.2
BPCF-initiated IP-CAN Session Modification
193
E.4.4.1.3
PCEF-initiated IP-CAN Session Modification
195
E.4.4.1.4
BBERF-initiated IP-CAN Session Modification
196
E.4.4.2
IP-CAN Session Modification for NSWO traffic
198
E.4.4.2.1
PCRF-initiated IP-CAN Session Modification
198
E.4.4.2.2
BPCF-initiated IP-CAN Session Modification
200
E.5
3GPP HNB Procedures – CS Support
202
E.5.1
S9a CS Session Establishment
202
E.5.2
PCRF initiated S9a CS Session Modification
203
E.5.2a
BPCF initiated S9a CS Session Modification
204
E.5.3
S9a CS Session Termination
204
E.6
PCRF Addressing
205
E.6.1
General
205
E.6.2
DRA Definition
206
E.6.3
DRA Procedure
206
E.6.3.1
DRA Information Storage
206
E.6.3.2
Capabilities Exchange
206
E.6.3.3
Redirect DRA
207
E.6.3.4
Proxy DRA
207
E.6.3.5
PCRF selection by BPCF
207
E.6.3.6
PCRF selection by AF and TDF in Unsolicited application reporting mode for NSWO traffic
208
E.6.3.7
PCRF selection in a roaming scenario
208
E.6.3.8
PCRF selection for the HNB CS Service
208
E.6.4
DRA flows
209
E.6.4.1
General
209
E.6.4.2
Proxy DRA
209
E.6.4.2.1
S9 session establishment trigger
209
E.6.4.2.2
S9 session termination notification
210
E.6.4.3
Redirect DRA
211
E.6.4.3.1
S9 session establishment trigger
211
E.6.4.3.2
S9 session termination notification
211
E.7
BPCF Addressing
212
E.7.1
General
212
E.8
Session Linking Function
212
Annex F (normative):
Access specific aspects, Fixed Broadband Access network convergence
213
F.1
General
213
F.2
Definitions and abbreviations
213
F.2.1
Definitions
213
F.2.2
Abbreviations
213
F.3
Binding Mechanisms
213
F.3.1
NSWO traffic
213
F.3.2
Traffic from fixed devices
214
F.4
PCC procedures
214
F.4.1
Introduction
214
F.4.2
IP-CAN Session Establishment
214
F.4.3
IP-CAN Session Termination
214
F.4.3.1
UE-Initiated
214
F.4.3.2
PCEF-Initiated
215
F.4.3.3
PCRF-Initiated
215
F.4.4
IP-CAN Session Modification
215
F.4.4.1
PCRF-Initiated IP-CAN Session Modification
215
F.4.4.2
PCEF-Initiated IP-CAN Session Modification
215
F.5
PCRF Addressing
215
F.5.1
General
215
F.5.2
DRA Definition
215
F.5.3
DRA Procedure
215
F.5.3.1
Redirect DRA
215
F.5.3.2
Proxy DRA
215
F.5.3.3
PCRF selection by AF and TDF in unsolicited application reporting mode
216
F.5.3.4
PCRF selection in a roaming scenario
216
F.5.4
DRA flows
216
Annex G (normative):
Diameter overload control mechanism
217
G.1
General
217
G.2
Reporting Node
217
G.3
Reacting Node
217
G.4
DRA Diameter Overload Behavior
217
G.4.1
DRA reacting to Host Reports
217
Annex H (normative):
Access specific procedures for 3GPP EPS
219
H.1
General
219
H.2
Binding Mechanisms
219
Annex I (normative):
APN matching procedures
220
Annex J (normative):
Diameter message priority mechanism
220
J.1
General
220
J.2
PCC functional element behaviour
220
J.3
Interactions
220
Annex K (informative):
Change history
221


Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x
the first digit:

1
presented to TSG for information;

2
presented to TSG for approval;

3
or greater indicates TSG approved document under change control.

Y
the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z
the third digit is incremented when editorial only changes have been incorporated in the document.

1
Scope

The present specification adds detailed flows of Policy and Charging Control (PCC) over the Diameter-based Rx, Gx, Gxx, Sd, Sy, S9, Nt, Diameter-based St and Np reference points and their relationship with the bearer level signalling flows over the Gn/Gp, S4, S5/S8, S2a and S2c interfaces.

The calls flows depicted in this Technical Specification represent usual cases, i.e. not all situations are covered. Detailed information provided in 3GPP TS 29.212 [9], 3GPP TS 29.214 [10], 3GPP TS 29.215 [22], 3GPP TS 29.217 [36] , 3GPP TS 29.154 [56] and 3GPP TS 29.219 [28] shall be taken into consideration.
The present specification also describes the binding and the mapping of QoS parameters among SDP, UM TS QoS parameters, and QoS authorization parameters.
The present specification also describes the PCRF addressing using DRA.
The present specification also describes Diameter race condition handling for Gx based applications, i.e Gx, Gxx, Sd and S9.
2
References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

· References are either specific (identified by date of publication and/or edition number or version number) or non‑specific.

· For a specific reference, subsequent revisions do not apply.

· For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]
3GPP TR 21.905: “Vocabulary for 3GPP Specifications”.

[2]
3GPP TS 23.203: “Policy Control and charging architecture”.

[3]
3GPP TS 23.060: “General Packet Radio Service (GPRS); Service description; Stage 2”.

[4]
3GPP TS 23.107: “Quality of Service (QoS) concept and architecture”.

[5]
3GPP TS 24.229: “IP Multimedia Call Control Protocol based on SIP and SDP; Stage 3”.
[6]
3GPP TS 26.234: “End-to-end transparent streaming service; Protocols and codecs”.

[7]
Void.

[8]
Void
[9]
3GPP TS 29.212: “Policy and Charging Control (PCC); Reference points”.
[10]
3GPP TS 29.214: “Policy and Charging Control over Rx reference point”.

[11]
IETF RFC 2327: “SDP: Session Description Protocol”.

[12]
IETF RFC 3264: “An Offer/Answer model with the Session Description Protocol (SDP)”.
[13]
IETF RFC 3556: “Session Description Protocol (SDP) Bandwidth Modifiers for RTP Control Protocol (RTCP) Bandwidth”.

[14]
IETF RFC 3588: “Diameter Base Protocol”.
[15]
IETF RFC 5245: “Interactive Connectivity Establishment (ICE): A Protocol for Network Address Translator (NAT) Traversal for Offer/Answer Protocols”.

[16]
IETF RFC 4145: “TCP-Based Media Transport in the Session Description Protocol (SDP)”.

[17]
IETF RFC 4975: “The Message Session Relay Protocol (MSRP)”.
[18]
3GPP2 C.S0046-0 v1.0: “3G Multimedia Streaming Services”.
[19]
3GPP2 C.S0055-A v1.0: “Packet Switched Video Telephony Services (PSVT/MCS)”.

[20]
Void
[21]
3GPP TS 23.402: “Architecture Enhancements for non-3GPP accesses”.
[22]
3GPP TS 29.215: “Policy and Charging Control over S9 reference point”.
[23]
IETF RFC 3890: “A Transport Independent Bandwidth Modifier for the Session Description Protocol (SDP) “.
[24]
3GPP TS 24.292: “IP Multimedia (IM) Core Network (CN) subsystem Centralized Services (ICS); Stage 3”.
[25]
3GPP TS 23.335: “User Data Convergence (UDC); Technical realization and information flows; Stage 2”.

[26]
3GPP TS 29.335: “User Data Convergence (UDC); User Data Repository Access Protocol over the Ud interface; Stage 3”.
[27]
3GPP TS 23.216: “Single Radio Voice Call Continuity (SRVCC); Stage 2”.
[28]
3GPP TS 29.219: “Policy and Charging Control over Sy reference point”.
[29]
3GPP TS 26.114: “IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction”
[30]
3GPP TS 26.247: “Transparent end-to-end Packet-switched Streaming Service (PSS) Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH) “.
[31]
Void.
[32]
Broadband Forum WT-134: “Policy Control Framework” (work in progress).

[33]
IETF RFC 7683: "Diameter Overload Indication Conveyance".
[34]
3GPP TS 23.468: "Group Services and System Aspects; Group Communication System Enablers for LTE (GCSE LTE).

[35]
3GPP TS 23.380: "IMS Restoration Procedures".
[36]
3GPP TS 29.217: “Policy and Charging Control: Congestion Reporting over Np reference point”.
[37]
3GPP TS 23.003: "Numbering, addressing and identification".
[38]
3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".
[39]
IETF RFC 5761: "Multiplexing RTP Data and Control Packets on a Single Port".
[40]
IETF draft-ietf-dime-drmp-02: "Diameter Routing Message Priority".
Editor's note:
The above document cannot be formally referenced until it is published as an RFC. 
[51]
3GPP TS 23.335: "User Data Convergence (UDC); Technical realization and information flows; Stage 2".

[52]
3GPP TS 29.335: "User Data Convergence (UDC); User Data Repository Access Protocol over the Ud interface; Stage 3".
[53]
3GPP TS 29.201: "Representational State Transfer (REST) reference point between Application Function (AF) and Protocol Converter (PC)".

[54]
3GPP TS 29.155: "Traffic Steering Control; Representational State Transfer (REST) over St reference point".

[55]
3GPP TS 32.240: "Telecommunication management; Charging management; Charging architecture and principles".
[56]
3GPP TS 29.154: "Service Capability Exposure Function over Nt reference point".

[57]
3GPP TS 23.179: "Functional architecture and information flows to support mission critical communication services".
[58]
3GPP TS 22.153: "Multimedia Priority Service".
[59]
3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".
3
Definitions and abbreviations

3.1
Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply:
DRA binding: The PCRF routing information stored per UE or per PDN in the DRA, which include the user identity (UE NAI), the UE Ipv4 address and/or Ipv6 prefix, the APN (if available) and the selected PCRF identity for a certain IP-CAN Session.
Gateway Control Session: An association between a BBERF and a PCRF (when GTP is not used in the EPC), used for transferring access specific parameters, BBERF events and QoS rules between the PCRF and BBERF. In the context of this specification this is implemented by use of the Gxx procedures.
IP-CAN session: association between a UE and an IP network.
The association is identified by one or more UE Ipv4 addresses/ and/or Ipv6 prefix together with a UE identity information, if available, and a PDN represented by a PDN ID (e.g. an APN). An IP-CAN session incorporates one or more IP-CAN bearers. Support for multiple IP-CAN bearers per IP-CAN session is IP-CAN specific. An IP-CAN session exists as long as the related UE Ipv4 address and/or Ipv6 prefix are assigned and announced to the IP network.
Policy counter: A mechanism within the OCS to track spending applicable for a subscriber.

Policy counter status: A label whose values are not standardized and that is associated with a policy counter’s value relative to the spending limit(s) (the number of possible policy counter status values for a policy counter is one greater than the number of thresholds associated with that policy counter, i.e policy counter status values describe the status around the thresholds). This is used to convey information relating to subscriber spending from OCS to PCRF. Specific labels are configured jointly in OCS and PCRF.
RAN user plane congestion: RAN user plane congestion occurs when the demand for RAN resources exceeds the available RAN capacity to deliver the user data for a prolonged period of time.

NOTE:
Short-duration traffic bursts is a normal condition at any traffic load level, and is not considered to be RAN user plane congestion. Likewise, a high-level of utilization of RAN resources (based on operator configuration) is considered a normal mode of operation and might not be RAN user plane congestion.
Spending limit: A spending limit is the usage limit of a policy counter (e.g. monetary, volume, duration) that a subscriber is allowed to consume.

Spending limit report: a notification, containing the current policy counter status generated from the OCS to the PCRF via the Sy reference point.
TDF session: An association between an IP-CAN session and the assigned TDF for the purpose of application detection and control by the PCRF . The association is identified by one UE Ipv4 address and/or Ipv6 prefix together with optionally a PDN represented by a PDN ID and a set of ADC rules to be applied by the TDF.
3.2
Abbreviations

For the purpose of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply:

ADC
Application Detection and Control
AF
Application Function
ARA
Aggregated RUCI Report Answer
ARP
Allocation and Retention Priority

ARR
Aggregated RUCI Report Request
AVP
Attribute-Value Pair
BBERF
Bearer Binding and Event Reporting Function

CSG
Closed Subscriber Group

CSG ID
Closed Subscriber Group Identity
CoA
Care of AddressDRA
Diameter Routing Agent
DRMP
Diameter Routing Message Priority

GBR
Guaranteed Bitrate
GCS
Group Communication Service

GCS AS
Group Communication Service Application Server

H-AF
Home AF

H-DRA
Home DRA

H-PCRF
Home PCRF

HPLMN
Home PLMN

MBR
Maximum Bitrate
MUA
Modify UE context Answer
MUR
Modify UE context Request
MPS
Multimedia Priority Service
NBIFOM
Network-based IP flow mobility

NRA
Non-Aggregated RUCI Report Answer
NRR
Non-Aggregated RUCI Report Request
PA
Proxy Agent

PCC
Policy and Charging Control

PCEF
Policy and Charging Enforcement Function

PCRF
Policy and Charging Rule Function
PGW
PDN-Gateway

QCI
QoS Class Identifier

RCAF
RAN Congestion Awareness Function

RUCI
RAN User Plane Congestion Information
SCEF
Service Capability Exposure Function

SDF
Service Data Flow
SLA
Spending Limit Answer
SLR
Spending Limit Request
SNA
Spending-Status Notification Answer
SNR
Spending-Status Notification Request
STA
Session Termination Answer
TSSF
Traffic Steering Support Function
STR
Session Termination Request
TDF
Traffic Detection Function
UDC
User Data Convergence

UDR
User Data Repository
V-AF
Visited AF

V-DRA
Visited DRA

V-PCRF
Visited PCRF

VPLMN
Visited PLMN
3a
Reference architecture
The PCC functionality is provided by the functions of the Policy and Charging Rules Function (PCRF), the Policy and Charging Enforcement Function (PCEF), the Bearer Binding and Event Reporting Function (BBERF), the Application Function (AF), the Traffic Detection Function (TDF), the Traffic Steering Support Function (TSSF), the RAN Congestion Awareness Function (RCAF), the Service Capability Exposure Function (SCEF), the Online Charging System (OCS), the Offline Charging System (OFCS) and the Subscription Profile Repository (SPR) or the User Data Repository (UDR). UDR replaces SPR when the UDC architecture as defined in 3GPP TS 23.335 [51] is applied to store PCC related subscription data. In this deployment scenario Ud interface as defined in 3GPP TS 29.335 [52] between PCRF and UDR is used to access subscription data in the UDR. 3GPP TS 23.203 [2] specifies the PCC stage 2 functionality.

[image: image3.emf] 

Offline   

Charging   

System   

(OFCS)  

Online Charging  

System    

(OCS)  

Gx  

Policy and Charging Rules Function   

(PCRF)  

Bearer  Binding and  Event  Reporting  Function   (BBERF)  

Policy and  Charging  Enforcement  Function   (PCEF)  

S y  

Gy  

Rx  

Sp /Ud  

RAN Congestion  Awareness  Function    (RCAF)  

Np  

Traffic   Detection   Function   (TDF)  

Gateway  

Gz  

Gy n  

Gz n  

Application     Function          (AF)  

Subscription Profile   Repository   (SPR) /   User Data   Repository   (UDR)  

AN - Gateway  

Sd  

Policy and Charging Rules Function (PCRF)  

Gx x  

T raffic  Steering  Support   Function   ( TSSF )  

St  

Service  Capability  Exposure  Fu nction         ( SCEF )  

Nt  


Figure 3a.1: Overall PCC architecture for non-roaming scenario


[image: image4.emf] 

Gy  

Sd  

Policy and  Charging  Enforcement  Function   (PCEF)  

Policy and  Charging Rules  Function   (V - PCR F)  

Bearer  Binding and  Event  Reporting  Function   (BBERF)  

Gateway  

AN - Gateway  

Gz  

Offline  Charging  System   (OFCS)  

   

Subscription  Profile  Repository   (SPR) /    User Data  Repository   (UDR)  

Rx  

Application  Function   (AF)  

Sp /Ud  

Policy and  Charging Rules  Function   ( H - PCRF)   S9  

VPLMN   HPLMN  

Gxx  

Traffic  Detection  Function   (TDF)  

Gx  

Sy  

 

Gyn  

Gzn  

RAN Congestion     Awareness Function         ( RCAF )  

Np  

Online   Charging  System   (OCS)  

T raffic  Steering  Support   Function   ( TSSF )  

St  

Service  Capability  Exposure  Function   ( SCEF )  

Nt  


Figure 3a.2: Overall PCC architecture for roaming with home routed access scenario

[image: image5.emf] 

Gy  

Rx  

Application  Function   (AF)  

Sd  

Policy and  Charging  Rules  Function   (V - PCRF)  

Gx  

Bearer  Binding and  E vent  Reporting  Function   (BBERF)  

Gateway  

AN - Gateway  

Gz  

Offline  Charging  System   (OFCS)  

   

Subscription  Profile  Repository   (SPR) /   User Data  Repository  (UDR)  

Rx  

Application  Function   (AF)  

Sp /Ud  

Policy and  Charging Rules  Function   (H - PCRF)   S9  

VPLMN   HPLMN  

Traffic Detection  Function   (TDF)  

Gxx  

Sy  

Online  Charging  System   (OCS)  

Policy and  Charging  Enforcement  Function   (PCEF)  

Gzn  

Gyn  

RAN Congestion     Awareness Function         ( RCAF )  

Np  

Service  Capability  Exposure  Function   ( SCEF )  

Nt  


Figure 3a.3: Overall PCC architecture for roaming with visited access scenario
NOTE 1:
The Sp reference point is located between PCRF and SPR, The Ud interface is located between PCRF and UDR.

NOTE 2:
The UDC Application Informational Model related to the PCRF is not specified in this Release.
NOTE 3:
AF can be located in both VPLMN and HPLMN for the visited access.
NOTE 4:  The H-PCRF can optionally send the addresses of the OCS to the V-PCRF in the visited access.
NOTE 5: 
Use of TSSF in roaming scenarios is in this release only specified for the home routed access.
NOTE 6: 
The SCEF acts as an AF (using Rx) in some service capability exposure use cases as described in 3GPP TS 23.682 [38].
The protocols for the Gx reference point, the Gxx reference point and the Sd reference point are specified in 3GPP TS 29.212 [9].

The protocols for the St reference point are specified in 3GPP TS 29.212 [2] and 3GPP TS 29.155 [54].
The protocols for the Rx reference point are specified in 3GPP TS 29.214 [10] and 3GPP TS 29.201 [53].
The protocol for the S9 reference point is specified in 3GPP TS 29.215 [22].
The protocol for the Sy reference point is specified in 3GPP TS 29.219 [28].
The protocol for the Np reference point is specified in 3GPP TS 29.217 [36]. 
The protocols for the Nt reference point is specified in 3GPP TS 29.154 [56].
More information about the protocols on the Gy reference point, the Gyn refrence point, the Gz refrence point and the Gzn refrence point are provided in 3GPP TS 32.240 [55].

The details associated with the Sp reference point are not specified in this Release. The SPR's relation to existing subscriber databases is not specified in this Release.
The UDR replaces the SPR when the UDC architecture as defined in 3GPP TS 23.335 [51] is applied to store PCC related subscription data. In this deployment scenario Ud interface as defined in 3GPP TS 29.335 [52] between PCRF and UDR is used to access subscription data in the UDR.
4
Signalling Flows over Gx, Gxx, Rx, Sd, Sy, Np, Nt, St and S9
4.0
General

There are three distinct network scenarios for an IP-CAN Session:

Case 1. No Gateway Control Session is required, no Gateway Control Establishment occurs at all (e.g. 3GPP Access where GTP-based S5/S8 are employed, and Non-3GPP access where GTP-based S2a or GTP-based S2b is employed).

Case 2. A Gateway Control Session is required. There are two subcases:

2a)
The BBERF assigns a Care of Address (CoA) to the UE and establishes a Gateway Control Session prior to any IP-CAN session establishment that will apply for all IP-CAN sessions using that CoA.

2b) At IP-CAN session establishment a Gateway Control Session is required before the PCEF announces the IP-CAN Session to the PCRF. At BBERF change and pre-registration the Gateway Control Session shall match an IP-CAN session that the PCEF has already announced. Each IP-CAN session is handled in a separate Gateway Control Session.


The PCRF shall select whether case 2a or case 2b applies based on the information received in the Gateway Control Session Establishment. For a user identified with a Subscription-Id AVP, when the PDN identifier included as part of the Called-Station-Id AVP is received, case 2b applies. If not received, case 2a applies.

The following considerations shall be taken into account when interpreting the signalling flows:
-
V-PCRF is included to also cover the roaming scenarios.
-
H-PCRF will act as a PCRF for non-roaming Ues.
-
The steps numbered as “number+letter” (e.g. “3a”) will be executed, for the roaming case, instead of steps numbered as “number” (e.g. “3”), as indicated in the explanatory text below the signalling flows.
-
Emergency services are handled locally in the serving network, therefore the S9 reference point does not apply.

NOTE: For the Visited Access case, the operator can by roaming agreement decide not to use S9 reference point.

The procedure to detect that the Gx session or a Gateway Control Session is restricted to Emergency Services is described in 3GPP TS 29.212 [9].
-
Subscription-related information is not relevant for Emergency Sessions; therefore Sp reference point does not apply.
-
With the UDC-based architecture as defined in 3GPP TS 23.203 [2] and 3GPP TS 23.335 [25], SPR, whenever mentioned in the present specification, refers to UDR. The Ud interface as defined in 3GPP TS 29.335 [26] is the interface between the PCRF and the UDR.
-
When monitoring event reporting via PCRF applies as defined in 3GPP TS 23.682 [38], the SCEF is acting as an AF. In this case, only the flows where the AF is located in the home network apply. Support of this functionality is detailed in 3GPP TS 29.214 [10], Annex D.

-
If the PCEF/BBERF/TDF needs to send an IP-CAN session/ Gateway Control Session/ TDF session modification request towards a PCRF which is known to have restarted since the IP-CAN session/ Gateway Control Session/ TDF session establishment, the PCEF/BBERF/TDF shall follow the PCRF Failure and Restoration procedure as defined in 3GPP TS 29.212 [9].
NOTE:
Only the interaction with OCS for spending limits 
vailabil over Sy interface is introduced in this document.
4.1
IP-CAN Session Establishment

This clause is applicable if a new IP-CAN Session is being established.

[image: image6.emf] 

VISITED   

ACCESS   

CASE  

VISITED   

ACCESS   

CASE  

PCEF  

TSSF  

SPR   BBREF   H - PCRF   TDF  

2  .    Establish IP   -   CAN Session   

Request  

3  .    Diameter CCR  

.   

3a. Diameter CC R  

3b. Store Information  

3c. Diameter CCR  

4. Store Information  

.   

5.  Profile Request  

6.  Profile Response  

8  .    PCC Rules Decision  ;   

Policy Decision   

10   .    ADC Rules Decision ;   Store   

ADC Rules  ;  

9   .    Store PCC Rules  

1  3   .    Diameter CCA  

1  4  .    Gateway Control and QoS Rules Provision    (   case    2  a and    2  b  )  

15. Install PCC Rules ,  

Policy Enforcement  

1 3  c  .    Store    ADC   Rules  ;   

Policy Decision  

13a. Diameter CCA  

1 3  d  

.   

Diameter   

CCR  

1  3  

e  

Diameter   

CCA  

.    13g. Diameter   CCA    

11  .   TDF session establishment   .  

1 3  b  .   

Policy Enforcement    

   

.   

16. Establish IP - CAN session Response  

-  

1 3  f  .    TDF session establishment   .  

OCS  

.   

.     7. Initial / intermediate Spending Limit Report Request  

V - PCRF  

1  .    Gateway Control Session Establishment     (  case    2  a and    2  b  )  

12.   St   s ession establishment  

.  


Figure 4.1.1: IP-CAN Session Establishment

1.
The BBERF may initiate a Gateway Control Session Establishment procedure as defined in 4.4.1 (applicable for cases 2a during initial attach and 2b, as defined in clause 4.0), if appropriate. In this step, the PCRF determines whether the cases 2a or 2b applies, as defined in clause 4.0.
2.
The PCEF receives an Establish IP-CAN Session Request. The form of the Establish IP-CAN Session Request depends upon the type of the IP-CAN.
3.
For the non-roaming case, and for the case when the UE is roaming in a Home Routed scenario, the PCEF informs the H-PCRF of the IP-CAN Session establishment. The PCEF starts a new Gx session by sending a CCR to the H-PCRF using the CC-Request-Type AVP set to the value INITIAL_REQUEST. The PCEF provides UE identity information, PDN identifier, the UE Ipv4 address and/or UE Ipv6 prefix and, if available, the PDN connection identifier, IP-CAN type, RAT type and/or the default charging method and additional charging parameters as defined in clause 4.5.1 of 3GPP TS 29.212 [9] and may send charging characteristics if available. The PCEF provides, when available, the Default-EPS-Bearer-QoS and the APN-AMBR to the PCRF. The PCEF may provide the applicable TDF routing information in TDF-Information AVP. If the UE has declared support for the extended TFT filter format and the PCEF does not prevent the use thereof, the PCE F shall indicate that the support for extended TFT filters is available in the IP-CAN session. For types of IP-CAN, where the H-PCRF can be in control of IP-CAN Bearers, e.g. GPRS, the PCEF also provides a new bearer identifier and information about the requested bearer, such as QoS. If applicable for the IP-CAN type, it will also provide information to indicate whether NW-initiated bearer control procedures are supported, if available. The PCRF links the Gx session for the new IP-CAN session with the corresponding Gateway Control Session as defined in clause 4.0. The PCRF maintains aligned set of PCC and QoS rules in the PCEF and BBERF(s) as applicable for the case. For case 2a and if IP flow mobility is supported, the PCEF provides, when available, the IP flow mobility routing rules. For case 1 and if NBIFOM is supported, the PCEF provides, when available, the NBIFOM support and NBIFOM mode information. If available, the PCEF provides a timestamp corresponding to the time at which the request was initiated by the originating node, and additionally the PCEF can include the maximum wait time as defined in subclause 4.5.1 of 3GPP TS 29.212 [9].
For the case when the UE is roaming in a Visited Access scenario, steps 3a~3c are executed instead of step 3.
3a.
The PCEF informs the V-PCRF of the establishment of the IP-CAN session. The PCEF starts a new Gx session by sending a CCR to the V-PCRF with the CC-Request-Type AVP set to the value INITIAL_REQUEST. The parameters for CCR as listed in step 3 are applicable here.

3b.
The V-PCRF determines that the request is for a roaming user and concludes the IP-CAN session uses visited access. V-PCRF stores the received information.
3c.
If there is not an already established S9 session for this roaming user, the V-PCRF sends a CCR to the H-PCRF with the CC-Request-Type AVP set to the value INITIAL_REQUEST. The V-PCRF includes the Subsession-Enforcement-Info AVP within the CCR with a new S9 subsession identifier assigned by the V-PCRF to this IP-CAN session within the Subsession-Id AVP, and the Subsession-Operation AVP set to the value ESTABLISHMENT.
If there is an already established S9 session for this roaming user, the V-PCRF sends a CCR to the H-PCRF with the CC-Request-Type AVP set to the value UPDATE_REQUEST. The V-PCRF includes the Subsession-Enforcement-Info AVP within the CCR with a new S9 subsession identifier assigned by the V-PCRF to this IP-CAN session within the Subsession-Id AVP, and the Subsession-Operation AVP set to the value ESTABLISHMENT.

4.
The H-PCRF stores the information received in the CCR. For cases 2a and 2b, the H-PCRF links the Gx session with the Gateway Control Session(s).

NOTE 1:
In the case 2a, when an additional PDN connection is established, the Gx session is linked with the already established Gateway Control Session.
5.
If the H-PCRF requires subscription-related information and does not have it, the H-PCRF sends a request to the SPR in order to receive the information.

6.
The SPR replies with the subscription related information containing the information about the allowed service(s), QoS information, PCC Rules information and may include MPS EPS Priority, MPS Priority Level and IMS Signalling Priority for establishing a PS session with priority.

NOTE 2:
For steps 5 and 6: The details associated with the Sp reference point are not specified in this Release. The SPR’s relation to existing subscriber databases is not specified in this Release.
7.
If the PCRF determines that the policy decision depends on the status of the policy counters available at the OCS and no Sy session yet has been established for this subscriber, the PCRF sends an Initial Spending Limit Report Request as defined in clause 4.7.1. If the Sy sesson is already established for this subscriber, the PCRF may send, if required, an Intermediate Spending Limit Report Request as defined in clause 4.7.2.
8.
The H-PCRF selects or generates PCC Rule(s) to be installed. The H- PCRF may also make a policy decision by deriving an authorized QoS and by deciding whether service flows described in the PCC Rules are to be enabled or disabled. If MPS EPS Priority, MPS Priority Level, and IMS Signalling Priority are present for the user, the PCRF takes the information into account. If NBIFOM is supported, the PCRF determines whether the NBIFOM applies to the IP-CAN session and selects the NBIFOM mode.
9.
The H-PCRF stores the selected PCC Rules. The H-PCRF selects the Bearer Control Mode that will apply during the IP-CAN session if applicable for the particular IP-CAN. If the H-PCRF controls the binding of IP-CAN Bearers, the H-PCRF stores information about the IP-CAN Bearer to which the PCC Rules have been assigned. If the BBERF/PCEF controls the binding of IP-CAN bearers, the H-PCRF may derive the QoS information per QCI applicable to that IP-CAN session for non-GBR bearers.
10.
When user profile configuration indicates that Application Detection and Control function is enabled, the H-PCRF makes the policy decision for the application detection and control. For the non-roaming case, or for the case when the UE is roaming in a Home Routed scenario, the H-PCRF selects the applicable PCC Rules to be provided for application detection and control for the PCEF supporting ADC feature, or the applicable ADC rules for the solicited application reporting with a TDF. For the case when the UE is roaming in a Visited Access scenario, the H-PCRF selects the applicable PCC Rules to be provided for application detection and control. For solicited application reporting with a TDF, the H-PCRF finds the TDF by using the TDF-Information AVP received from the PCEF in step 3, or, if not received, using a pre-configured TDF address.

11.
Only applicable for non-roaming case, and for the case when the UE is roaming in a home routed case, In case of solicited application reporting with a TDF, the PCRF initiates a TDF Session Establishment procedure, according to clause 4.6.1, with the selected TDF. 
12.
If traffic steering control over St applies andthe H-PCRF determines that traffic steering control information is needed for the IP-CAN session, the PCRF initiates an St session establishment procedure according to subclause 4.9.1 with the TSSF. The TSSF identifier is pre-configured on the PCRF per e.g. PCEF.
13.
For the non-roaming case, and for the case when the UE is roaming in a Home Routed scenario, the H-PCRF provisions the PCC Rules to the PCEF using CCA. The H-PCRF also provides the selected Bearer Control Mode if applicable for the particular IP-CAN and if available, the QoS information per QCI. If the PCEF has indicated that the support for extended TFT filters is available in the IP-CAN session, then the PCRF may, by indicating the PCRF support for extended TFT filters, enable the use of the extended TFT filter format in the IP-CAN session. The PCRF may also provide event triggers listing events for which the PCRF desires PCC Rule Requests. Furthermore, the PCRF may provide authorized QoS including the APN-AMBR and the Default-EPS-Bearer-QoS, User Location Information, user CSG information (if received from the BBERF) and Presence Area Information. If usage monitoring is enabled, the H-PCRF may provide the applicable thresholds for usage monitoring control at PCEF within the Usage-Monitoring-Information AVP. If NBIFOM is supported, the H-CRF provides the decision on whether the NBIFOM is supported and the selected NBIFOM mode if applicable.

For types of IP-CAN, where the PCRF controls IP-CAN Bearers, e.g. GPRS, the PCRF indicates the IP-CAN Bearer where the PCC Rules are to be installed and that the authorized QoS refers to. Otherwise, the PCRF operates without any further reference to any specific bearer.

If the PCEF supports Application Detection and Control feature, the PCRF provisions the applicable PCC Rules to the PCEF for the corresponding IP-CAN session.

If online charging is applicable then the PCEF requests credit information from the OCS over the Gy interface. If the PCEF receives credit re-authorisation triggers from the OCS then, for case 2b, it requests the PCRF via a CCR message to provision the triggers at the BBERF. The triggers to be provisioned are specified in the Event-Report-Indication AVP in the CCR message.
For the case when the UE is roaming in a Visited Access scenario, steps 13a -13g are executed.
13a.
The PCC Rules, if they were selected in step 9, are provisioned by the H-PCRF to the V-PCRF by using a CCA. The H-PCRF includes PCC Rules in the Subsession-Decision-Info AVP of the CCA, along with the S9 subsession identifier as received in step 3c within the Subsession-Id AVP. Other parameters listed in step 9 are also applicable here.
13b.
The V-PCRF enforces visited operator policies regarding QoS authorization requested by the H-PCRF as indicated by the roaming agreements.
13c. In case of TDF, if Application Detection and Control function is enabled for the IP-CAN session, the V-PCRF extracts ADC rules from the received PCC rules from the H-PCRF and stores the ADC rules.
13d.
The V-PCRF informs the H-PCRF when a request has been denied and may provide the acceptable QoS Information for the service.

13e.
The H-PCRF acknowledges the CCR and may additionally include new or modified PCC rules to the V-PCRF. When user profile configuration indicates that Application Detection and Control function is enabled, some of those PCC Rules may be dedicated for application detection and control.
13f.
In case of solicited application reporting with a TDF, the V-PCRF initiates a TDF Session Establishment procedure, according to clause 4.6.1, with the selected TDF and provides ADC Rules extracted from the corresponding PCC Rules.
13g.
The V-PCRF provisions PCC rules received from the H-PCRF to the PCEF by using CCA. The parameters listed in step 11a are applicable here, User Location Information and user CSG information (if received from the BBERF).
NOTE 3:
From this point and onward, the PCRF is responsible for keeping the active PCC, ADC and QoS rules aligned.
14.
If case 2a or 2b applies, the PCRF aligns the set of QoS rules at the BBERF with the set of active rules at the PCEF.

15.
The PCEF installs the received PCC Rules. The PCEF also enforces the authorized QoS and enables or disables service flows according to the flow status of the corresponding PCC Rules. If QoS information is received per QCI, PCEF sets the upper limit accordingly for the MBR that the PCEF assigns to the non-GBR bearer(s) for that QCI. In case of PCEF supporting Application Detection and Control feature, the PCEF enforces the application detection and control.
16. The PCEF sends a response to the Establish IP-CAN Session Request.
For GPRS, the GGSN accepts the PDP Context Request based on the results of the authorisation policy decision enforcement. If the requested QoS parameters do not correspond to the authorized QoS, the GGSN adjusts (downgrades /upgrades) the requested UMTS QoS parameters to the authorized values.
NOTE 4:
The PCRF can reject the IP-CAN session establishment, e.g. the PCRF cannot obtain the subscription-related information from the SPR and the PCRF cannot make the PCC rule decisions, as described in 3GPP TS 29.212 [9].

PCEF can also enforce active preconfigured PCC rules which are not know to the PCRF.

The PCEF can also reject the IP-CAN session establishment, e.g. there is no active PCC rule for the IP-CAN session as specified in 3GPP TS 23.203 [2].
4.2
IP-CAN Session Termination

4.2.1
UE-Initiated

4.2.1.1
AF located in the HPLMN

This clause is applicable if an IP-CAN Session is being released by the UE and the AF is located in the HPLMN.

[image: image7.emf]BBERF PCEF TSSF H-PCRF H-AF SPR

14.  Gateway Control and Qos Rules provision (case 2a)

Or

PCRF-initiated Gateway Control Session Termination (case 2a)

2. BBERF-initiated Gateway Control Session Termination (case2b)

4. Identify AF Sessions 

bound to the removed 

IP-CAN Session  

5b. removes the 

information related to the 

terminated IP-CAN Session

3. Diameter CCR

3a. Diameter CCR

3d. Diameter CCR

5. Diameter CCA 

5a.  Diameter CCA

5c. Diameter CCA

9. Diameter ASR

10. Diameter ASA

11. Diameter STR

12. Diameter STA

 

For each 

affected 

AF session

(if any)

15. Cancel Subcribed Notification request

16. Cancel Subcribed Notification Response

Conditional 

Mandatory

Legend:

1. Remove IP-CAN Session Request

6. Remove IP-CAN Session 

Response

V

I

S

I

T

E

D

 

A

C

C

E

S

S

 

C

A

S

E

V

I

S

I

T

E

D

 

A

C

C

E

S

S

 

C

A

S

E

OCS RCAF TDF

3b. TDF session termination for roaming 

UE with visited access

7. TDF session termination for roaming UE with home-

routed access

8. UE context removal

13. Final Spending Limit Report Request/ 

Intermediate Spending Limit Report Request

3c. UE context removal

V-PCRF

17. St session termination


Figure 4.2.1.1.1: UE-Initiated IP-CAN Session Termination – AF located in the HPLMN
In the following procedures, the V-PCRF is included to depict the roaming scenarios. H-PCRF acts as the PCRF for non-roaming UEs.
1.
If case 2b applies (as defined in clause 4.0), the BBERF receives a request to remove the IP-CAN session. In case 2a, the request goes transparently through the BBERF. In all cases, the PCEF receives a request to remove the IP-CAN Session. The form of the Remove IP-CAN Session Request depends upon the type of the IP-CAN.
2.
If case 2b applies (as defined in clause 4.0), the BBERF-initiated Gateway Control Session Termination procedure as defined in clause 4.4.4 (BBERF-Initiated Gateway Control Session Termination) is initiated.
3.
For the non-roaming case, and for the case when the UE is roaming in a Home Routed scenario, the PCEF sends a CCR to the H-PCRF, indicating the IP-CAN Session termination. The PCEF requests the termination of the Gx session using the CC-Request-Type AVP set to the value TERMINATION_REQUEST. If the usage monitoring is enabled, the PCEF informs the H-PCRF about the resources that have been consumed by the user since the last report. If the Required-Access-Info AVP is included in any PCC Rule, the PCEF informs the H-PCRF about the access network information. If NAS-RAN-Cause feature is supported the PCEF informs the H-PCRF about the access network information and the failure cause(s), if available.
For the case when the UE is roaming in a Visited Access scenario, steps 3a~3b are executed instead of step 3:
3a.
The PCEF sends a CCR to the V-PCRF, indicating the IP-CAN Session termination. The PCEF requests the termination of the Gx session using the CC-Request-Type AVP set to the value TERMINATION_REQUEST. If the usage monitoring is enabled, the PCEF informs the V-PCRF about the resources that have been consumed by the user since the last report. If the Required-Access-Info AVP is included in any PCC Rule, the PCEF informs the V-PCRF about access network information.
3b
If there is an active TDF session between the TDF and the V-PCRF, for roaming UE with visited access, the TDF Session termination is initiated as defined in Section 4.6.2. For this case, the PCRF described in Section 4.6.2 acts as a V-PCRF.
3c.
If congestion reporting is active for the user id and PDN id, for roaming UE with visited access, the V-PCRF initiates a UE context removal as defined in clause 4.8.5.
3d.
The V-PCRF sends the CCR to the H-PCRF. If case 2b or case 1 applies and this is the last subsession associated with the S9 session, the V-PCRF sends a CCR to the H-PCRF to request the termination of the S9 session using the CC-Request-Type AVP set to the value TERMINATION_REQUEST. Otherwise, the V-PCRF sends a CCR to the H-PCRF with a CC-Request-Type AVP set to the value UPDATE_REQUEST and a Subsession-Enforcement-Info within which the Subsession-Operation AVP set to value TERMINATION to request the termination of the conresponding S9 subsession.
NOTE 1:
If the usage monitoring is enabled on PCEF and/or TDF, the V-PCRF gathers the reports and provides them all to H-PCRF in the single CCR message.
4.
The H-PCRF identifies the AF sessions that are bound to IP flows of the removed IP-CAN Session.

5.
For the non-roaming case, and for the case when the UE is roaming in a Home Routed scenario, the H-PCRF acknowledges the Gx session termination by sending a CCA to the PCEF.

For the case when the UE is roaming in a Visited Access scenario, steps 5a~5c are executed instead of step 5:
5a. The H-PCRF acknowledges the S9 session or subsession termination by sending a CCA to the V-PCRF.

5b. The V-PCRF removes the information related to the terminated IP-CAN Session.

5c. The V-PCRF acknowledges the Gx session termination by sending a CCA to the PCEF.
6.
The PCEF sends a response to the Remove IP-CAN Session Request. The form of the Remove IP-CAN Session Response depends upon the type of the IP-CAN. Step 6 may be executed in parallel with step 3 or 3a (as applicable).
7. If there is an activeTDF session between the TDF and the H-PCRF, for non-roaming UE/roaming UE with home routed access, the TDF Session termination is initiated as defined in Section 4.6.2.
NOTE 2: Step 7 can occur anytime after step 3.

For each AF session identified in step 4 as bound to the IP-CAN Session being removed, steps 7-11 are executed:
8.
If congestion reporting is active for the user id and PDN id, for non-roaming UE or UE is roaming in home routed case, the H-PCRF initiates a UE context removal as defined in clause 4.8.5.

NOTE 3:
Step 8 can occur anytime after step 3.
9.
The H-PCRF indicates the session abort to the H-AF by sending an ASR to the H-AF.

10.
The H-AF responds by sending an ASA to the H-PCRF.

11.
The H-AF sends an STR to the H-PCRF to indicate that the session has been terminated.

12.
The H-PCRF responds by sending an STA to the H-AF. If the provided PCC rules are related to an AF session associated with a sponsor, usage thresholds were provided by the H-AF earlier, and the H-PCRF has usage data that has not yet been reported to the H-AF, the H-PCRF informs the H-AF about the resources that have been consumed by the user since the last report. If the BBERF in step 2 or PCEF in step 3 reports the access network information and if the AF requested the H-PCRF to report access network information previously and/or the RAN-NAS-Cause feature is supported, the H-PCRF informs the H-AF about the access network information and/or RAN/NAS/TWAN/Untrusted WLAN failure cause(s) if available.
13.
If this is the last IP-CAN session for this subscriber the Final Spending Limit Report Request as defined in clause 4.7.3 is sent. If any existing IP-CAN sessions for this subscriber require policy counter status reporting, the Intermediate Spending Limit Report Request as defined in clause 4.7.2 can be sent to alter the list of subscribed policy counters.
14.
If case 2a applies (as defined in clause 4.0), the Gateway Control and QoS Rules Provision procedure as defined in clause 4.4.3 (Gateway Control and QoS Rules Provision) may be initiated to remove the QoS rules associated with the IP‑CAN session being terminated. This applies e.g. in case the Gateway Control Session remains to serve other IP‑CAN sessions.


Alternatively, if UE acquires a care of address (CoA) that is used for the S2c reference point and the H-PCRF determines that all QoS rules are to be removed and the Gateway Control Session to be terminated, the PCRF-initiated Gateway Control Session Termination procedure as defined in clause 4.4.4 (PCRF-Initiated Gateway Control Session Termination) is initiated. This applies e.g. in case the UE is detached and the CoA acquired by the UE is not used for any other IP‑CAN session.
15. The H-PCRF sends a cancellation notification request to the SPR if it has subscribed such notification. The H-PCRF stores the remaining usage allowance in the SPR if all IP-CAN sessions of the user to the same APN are terminated. Step 14 may be initiated any time after step 5 or 5a (as applicable).
16.
The SPR sends a response to the H-PCRF.
NOTE 4:
For steps 14 and 15: The details associated with the Sp reference point are not specified in this Release. The SPR’s relation to existing subscriber databases is not specified in this Release.
17.
If the H-PCRF has provided traffic steering control information to the TSSF for the IP-CAN session, the H-PCRF initiates the St session termination procedure to the TSSF to remove the traffic steering control information associated to the UE IPv4 address and/or to the UE IPv6 prefix for the terminated IP-CAN session according to the subclause 4.9.3.

NOTE 5:
Step 17 can be initiated any time after step 5.
4.2.1.2
AF located in the VPLMN

This clause is applicable only for the Visited Access scenario for the case when an IP-CAN Session is being released by the UE and the AF is located in the VPLMN.

[image: image8.emf] 

BBERF PCEF V- PCRF H- PCRF V-AF SPR

. 

For each 

affected 

AF session

(if any)

2.  Diameter ASR

3 .  Diameter ASR

4.  Diameter ASA

5.  Diameter ASA

6.  Diameter STR

7.  Diameter STR

8.  Diameter STA

9.  Diameter STA

Conditional 

Mandatory

Legend:

1  .    Perform  Step  1  through Step  8:  as specified in Figure  4.2.1 .1 .1  : 

UE Initiated IP-CAN Session Termination  - AF Located in the HPLMN 

V

I

S

I

T

E

D

 

A

C

C

E

S

S

 

C

A

S

E

10.      Perform Step  13 through Step 1 6:   as specified in in Figure  4.2.1 .1 .1  : 

UE Initiated IP-CAN Session Termination  - AF Located in the HPLMN 


Figure 4.2.1.2.1: UE-Initiated IP-CAN Session Termination – AF located in the VPLMN
If the AF resides in the VPLMN, the V‑PCRF proxies AF session signalling over S9 between the V-AF and the H‑PCRF.
1
In order to perform UE initiated IP-CAN Session Termination Procedures, step 1 through step 8: as specified in Figure 4.2.1.1.1: UE Initiated IP-CAN Session Termination – AF Located in the HPLMN are executed.


For each AF session identified in step 4 (Figure 4.2.1.1.1) as bound to the IP-CAN Session being removed steps 2-10 are executed:
2.
The H-PCRF indicates the session abort to the V-AF in VPLMN by sending an ASR to the V-PCRF.
3.
The V-PCRF proxies the ASR to the V-AF.
4.
The V-AF responds by sending an ASA to the V-PCRF.
5.
The V-PCRF proxies the ASA to the H-PCRF.
6.
The V-AF sends an STR to the V-PCRF to indicate that the session has been terminated.
7.
The V-PCRF proxies the STR to the H-PCRF.
8.
The H-PCRF responds by sending an STA to the V-PCRF.
9.
The V-PCRF proxies the STA to the V-AF.
10.
Step 13 through step 16: as specified in Figure 4.2.1.1.1: UE Initiated IP-CAN Session Termination – AF Located in the HPLMN are executed, as needed.

NOTE:
For steps 15 and 16: the details associated with the Sp reference point are not specified in this Release. The SPR’s relation to existing subscriber databases is not specified in this Release.

4.2.2
PCEF-Initiated

4.2.2.1
AF located in the HPLMN

This clause is applicable if an IP-CAN Session is being released by the PCEF and the AF is located in the HPLMN.

[image: image9.emf]TSSF

H-PCRF H-AF SPR PCEF

BBERF

1.  Detection that IP-CAN 

Session Termination is required

5. Diameter CCR

V

I

S

I

T

E

D

 

A

C

C

E

S

S

 

C

A

S

E

6.  Identify AF Sessions bound 

to the removed IP-CAN 

Session 

5a.Diameter CCR

5d. Diameter CCR

7. Diameter CCA

7c. Diameter CCA

7b.  Removes the 

information related to the 

terminated IP-CAN Session

7a. DiameterCCA

Foreach 

affected 

AF 

session

(if any)

10. Diameter ASR

11. Diameter ASA

12. Diameter STR

13. Diameter STA

.  15.   Gateway Control and QoS Rules provision (case 2a)

or

PCRF-initiated Gateway Control Session Termination(case 2a)

16. Cancel Subscribed Notification request

17. Cancel Subcribed Notification Response

Legend:

Mandatory

Conditional 

V

I

S

I

T

E

D

 

A

C

C

E

S

S

 

C

A

S

E

2.  Remove IP-CAN Session Request

4. Remove IP-CAN Session Response -

5b. TDF session termination for roaming UE 

with visited access

PCEF TDF

8. TDF session termination for non-roaming UE/

roaming UE with home routed access

RCAF

3.  BBERF-initiated Gateway Control Session Termination (case 2b)

V-PCRF

5c. UE context removal

OCS

9. UE context removal

14. Final Spending Limit Report Request

18. St session termination


Figure 4.2.2.1.1 : PCEF-initiated IP-CAN Session Termination – AF located in the HPLMN

In the following procedures, the V-PCRF is included to depict the roaming scenarios. H-PCRF acts as the PCRF for non-roaming Ues.
1.
The PCEF detects that the terminat
ion of an IP-CAN Session or bearer is required.

2.
If case 2b applies (as defined in clause 4.0), PCEF sends the Remove IP-CAN Session Request to the BBERF. If case 2a applies (as defined in clause 4.0), the request goes transparently through the BBERF. In all cases, the PCEF sends a Remove IP-CAN Session Request to remove the IP-CAN Session. The form of the Remove IP-CAN Session Request depends upon the type of the IP-CAN. It can consist of separate requests for each IP-CAN Bearer within an IP-CAN Session.
3
If case 2b applies (as defined in clause 4.0), the BBERF-initiated Gateway Control Session Termination
procedure as defined in clause 4.4.4 (BBERF-Initiated Gateway Control Session Termination) is initiated.
4.
The PCEF receives a response to the Remove IP-CAN Session Request.

5 – 7.
Same as Steps 3~5 in figure 4.2.1.1.1.
8 – 18.Same as Steps 7~17 in figure 4.2.1.1.1.
NOTE 1:
Steps 2 and 5 may be executed in parallel.
4.2.2.2
AF located in the VPLMN

This clause is applicable only for the Visited Access scenario for the case when an IP-CAN Session is being released by the PCEF and the AF is located in the VPLMN


[image: image10.emf]BBERF PCEF V- PCRF H- PCRF V-AF SPR

. 

For each 

affected 

AF session

(if any)

2.  Diameter ASR

3 .  Diameter ASR

4.  Diameter ASA

5.  Diameter ASA

6.  Diameter STR

7.  Diameter STR

8.  Diameter STA

9.  Diameter STA

Conditional 

Mandatory

Legend:

1  .    Perform  Step  1  through Step  9:  as specified in Figure  4.2. 2.1. 1  : 

PCEF Initiated IP-CAN Session Termination - AF Located in the HPLMN 

V

I

S

I

T

E

D

 

A

C

C

E

S

S

 

C

A

S

E

10.      Perform Step  14 through Step  1 7:   as specified in in Figure  4.2.2.1 .1  : 

PCEF Initiated IP-CAN Session Termination - AF Located in the HPLMN 

 


Figure 4.2.2.2.1: PCEF-Initiated IP-CAN Session Termination – AF located in the VPLMN
If the AF resides in the VPLMN, the V‑PCRF proxies AF session signalling over S9 between the V-AF and the H‑PCRF.
1.
In order to perform PCEF initiated IP-CAN Session Termination Procedures, step 1 through step 9: as specified in Figure 4.2.2.1.1: PCEF Initiated IP-CAN Session Termination – AF Located in the HPLMN are executed.


For each AF session identified in step 6 (Figure 4.2.2.1.1) as bound to the IP-CAN Session being removed, steps 2-9 are executed:

2.
The H-PCRF indicates the session abort to the V-AF in VPLMN by sending an ASR to the V-PCRF.
3.
The V-PCRF proxies the ASR to the V-AF.
4.
The V-AF responds by sending an ASA to the V-PCRF.
5.
The V-PCRF proxies the ASA to the H-PCRF.
6.
The V-AF sends an STR to the V-PCRF to indicate that the session has been terminated.
7.
The V-PCRF proxies the STR to the H-PCRF.
8.
The H-PCRF responds by sending an STA to the V-PCRF.

9.
The V-PCRF proxies the STA to the V-AF.
10.
Step 14 through step 17: as specified in Figure 4.2.2.1.1: PCEF Initiated IP-CAN Session Termination – AF Located in the HPLMN are executed, as needed.

NOTE:
For steps 16 and 17: the details associated with the Sp reference point are not specified in this Release. The SPR’s relation to existing subscriber databases is not specified in this Release.

4.2.3
PCRF-Initiated

4.2.3.1
AF located in the HPLMN

This clause is applicable if an IP-CAN Session is being released by the PCRF and the AF is located in the HPLMN.

[image: image11.emf]TSSF H-PCRF H-AF SPR PCEF

BBERF

V

I

S

I

T

E

D

 

A

C

C

E

S

S

 

C

A

S

E

Foreach 

affected 

AF 

session

(if any)

Legend

:

Mandatory

Conditional 

V

I

S

I

T

E

D

 

A

C

C

E

S

S

 

C

A

S

E

1. Detection that IP-CAN 

Session termination is 

required

2. Diameter RAR

2a. Diameter RAR

2b. Diameter RAR

3. Removal of all 

PCC Rules

4. Diameter RAA

4a. Diameter RAA

4b. Diameter RAA

V

I

S

I

T

E

D

 

A

C

C

E

S

S

 

C

A

S

E

V

I

S

I

T

E

D  

A

C

C

E

S

S

 

C

A

S

E

8. Diameter CCR

8a. Diameter CCR

8d. Diameter CCR

9. Identify AF Sessions 

bound to the removed IP-

CAN Session

10. Diameter CCA

10a. Diameter CCA

10c. Diameter CCA

13. Diameter ASR

14. Diameter ASA

15. Diameter STR

16. Diameter STA

19. Cancel Subcribed Notification request

20. Cancel Subcribed Notification response

5. Remove IP-CAN 

Session Request

7. Remove IP-CAN 

Session Response

OCS RCAF TDF

6. BBERF-initiated Gateway Control Session Termination(case 2b)

10b. removes the 

information related to 

the terminated IP-CAN 

Session

18.  Gateway Control and Qos Rules provision (case 2a)

Or

PCRF-initiated Gateway Control Session Termination(case 2a)

8b. TDF session termination for roaming 

UE with visited access

11. TDF session termination for roaming UE 

with visited access

12. UE context removal

8c. UE context removal

17. Final Spending Limit Report Request

V-PCRF

21. St session termination


Figure 4.2.3.1.1: PCRF-initiated IP-CAN Session Termination – AF located in HPLMN

In the following procedures, the V-PCRF is included to depict the roaming scenarios. H-PCRF acts as the PCRF for non-roaming Ues.
1.
The H-PCRF detects that the termination of an IP-CAN Session is required.

2.
For the non-roaming case, and for the case when the UE is roaming in a Home Routed scenario, the H-PCRF sends a RAR including the Session-Release-Cause AVP to request that the PCEF terminates the IP CAN session.
For the case when the UE is roaming in a Visited Access scenario, steps 2a~2b are executed instead of step 2:
2a.
If case 2b or case 1 applies and the subsession being terminated is the last subsession over S9, the H-PCRF sends a RAR including the Session-Release-Cause AVP to the V-PCRF to indicate the termination of the S9 session. Otherwise, the H-PCRF sends a RAR to the V-PCRF including the Subsession-Decision-Info AVP with the Session-Release-Cause AVP to indicate the request for terminating the S9 subsession corresponding to the IP-CAN session.
2b. The V-PCRF sends a RAR including the Session-Release-Cause AVP to the PCEF.
3.
The PCEF removes all the PCC Rules which are applied to the IP-CAN session.

4.
For the non-roaming case, and for the case when the UE is roaming in a Home Routed scenario, the PCEF sends a RAA to acknowledge the RAR.

For the case when the UE is roaming in a Visited Access scenario, steps 4a~4b are executed instead of step 4:
4a.
The PCEF sends a RAA to the V-PCRF.

4b. The V-PCRF sends a RAA to the H-PCRF and acknowledges the request for terminating the S9 session or the S9 subsession corresponding to the IP-CAN session.
5.
The PCEF applies IP CAN specific procedures to terminate the IP CAN session.

6. – 21.
Same as Steps 3-18 in figure 4.2.2.1.1.
4.2.3.2
AF located in the VPLMN

This clause is applicable only for the Visited Access scenario for the case when an IP-CAN Session is being released by the PCRF and the AF is located in the VPLMN


[image: image12.emf]BBERF PCEF V- PCRF H- PCRF V-AF SPR

. 

For each 

affected 

AF session

(if any)

2.  Diameter ASR

3 .  Diameter ASR

4.  Diameter ASA

5.  Diameter ASA

6.  Diameter STR

7.  Diameter STR

8.  Diameter STA

9.  Diameter STA

Conditional 

Mandatory

Legend:

1  .    Perform  Step  1  through Step  12:  as specified in Figure  4.2.3 .1 .1  : 

PCRF Initiated IP-CAN Session Termination  - AF Located in the HPLMN 

V

I

S

I

T

E

D

 

A

C

C

E

S

S

 

C

A

S

E

10.      Perform Step  17 through Step  20:   as specified in in Figure  4.2.3.1 .1  : 

PCRF Initiated IP-CAN Session Termination - AF Located in the HPLMN 

   


Figure 4.2.3.2.1: PCRF-Initiated IP-CAN Session Termination  – AF located in the VPLMN
If the AF resides in the VPLMN, the V‑PCRF proxies AF session signalling over S9 between the V-AF and the H‑PCRF.
1.
In order to perform PCRF initiated IP-CAN Session Termination Procedures, step 1 through step 12: asspecified in Figure 4.2.3.1.1: PCRF Initiated IP-CAN Session Termination – AF Located in the HPLMN areexecuted.


For each AF session identified in step 6 (Figure 4.2.3.1.1) as bound to the IP-CAN Session being removed, steps 2-9 are executed:

2.
The H-PCRF indicates the session abort to the V-AF in VPLMN by sending an ASR to the V-PCRF.

3.
The V-PCRF proxies the ASR to the V-AF.
4.
The V-AF responds by sending an ASA to the V-PCRF.
5.
The V-PCRF proxies the ASA to the H-PCRF.
6.
The V-AF sends an STR to the V-PCRF to indicate that the session has been terminated.
7.
The V-PCRF proxies the STR to the H-PCRF.
8.
The H-PCRF responds by sending an STA to the V-PCRF.
9.
The V-PCRF proxies the STA to the V-AF.
10.
Step 17 through step 20: as specified in Figure 4.2.3.1.1: PCRF Initiated IP-CAN Session Termination – AF Located in the HPLMN are executed, as needed.

NOTE:
For steps 19 and 20: the details associated with the Sp reference point are not specified in this Release. The SPR’s relation to existing subscriber databases is not specified in this Release.

4.3
IP-CAN Session Modification

4.3.1
Network-Initiated IP-CAN Session Modification
4.3.1.1
Interactions between BBERF, PCEF, TDF, OCS, TSSF and PCRF(PCC/QoS/ADC Rule Provisioning in PUSH mode)

This flow shows the provisioning of PCC/QoS/ADC Rules and/or authorized QoS triggered by an event in the PCRF.

[image: image13.emf].

BBERF

8a. Diameter RAA

PCEF

TSSF

H-PCRF

8. Diameter RAA

5. Gateway Control & QoS Rule Provision (case 2a &2b)

8b. Diameter RAA

V

I

S

I

T

E

D

 

A

C

C

E

S

S

C

A

S

E

8c. Store Result

12. IP -CAN session modification 

1. Trigger

3. Store PCC and ADC 

Rules

4. Timer

7. Install, modify or remove 

PCC rules

8d, Diameter RAA

13. IP -CAN sessionsignalling

6. Diameter RAR

6d. Diameter RAR

6e. Install, modify or 

remove QoS rules

6f. Diameter RAR

6a. Diameter RAR

2. PCC Rules decision,

Policy Decision

V

I

S

I

T

E

D

 

A

C

C

E

S

S

C

A

S

E

TDF

6c. TDF session establishment/ 

modification/ termination

9. TDF session establishment/ modification/ 

termination for non-roaming/HR

OCS

14. IP -CAN session modification 

Legend

Mandatory

Conditional 

11. Initial/ Intermediate/ Final

Spending Limit Report Request

V-PCRF

6b. PCC Rules validation,

Local Policy Decision

10.St establishment /modification/termination for 

non-roaming/HR


Figure 4.3.1.1.1: Interactions between BBERF, PCEF, TDF, OCS, TSSF and PCRF for PCRF-Initiated IP-CAN Session Modification
1.
The H-PCRF receives an internal or external trigger to re-evaluate PCC Rules and policy decision for an IP-CAN Session. Possible external trigger events are described in clause 4.3.1.2. In addition, this procedure is triggered by
-
PCEF subscribed event
-
SPR subscription data modification (e.g. change in MPS EPS Priority, MPS Priority Level and/or IMS Signalling Priority, or change in user profile configuration indicating whether supporting application detection and control).
-
OCS status of a policy counter identifier(s) has changed and the PCRF requested notification for spending limits apply.
-
Application detection information report from TDF.
2.
The H-PCRF selects the PCC Rule(s) to be installed, modified or removed for the IP-CAN Session. In case of PCEF supporting Application Detection and Control feature, some of those PCC rules may be used for application detection and control. The H-PCRF may also update the policy decision by defining an authorized QoS and enable or disable the service flow(s) of PCC Rules. If the PCEF controls the binding of IP-CAN bearers, the H-PCRF may add or change QoS information per QCI applicable to that IP-CAN session. In case of TDF, the H-PCRF selects the applicable ADC rules for the solicited application reporting with a TDF. If NBIFOM applies to the IP-CAN session and Network-initiated mode is selected, the H-PCRF determines the access that is to be used for the transfer of traffic and include the allowed access type in the PCC rules. If NBIFOM applies to the IP-CAN session, the H-PCRF determines one access is to be removed from multi access IP-CAN session.
3.
The H-PCRF stores the updated PCC Rules, and ADC rules if available.

4.
Step 4 is only applicable if the Bearer Control Mode (BCM) selected is UE-only or, for UE/NW the H-PCRF determines that UE mode applies for the affected PCC Rules, and the PCRF receives an external trigger from the AF.
The PCRF may start a timer to wait for a UE requested bearer resource initiation, modification or removal procedure initiated by the UE, as depicted in figure 4.3.2.1.1.

If a UE requested bearer resource initiation, modification or termination procedure initiated by the terminal is received for the affected PCC rules while the timer is running, all subsequent steps in the present figure shall not be executed and, for case 1, the steps in figure 4.3.2.1.1 (on provisioning based on PULL procedure at PCEF –initiated IP-CAN session modification when the AF is located in the HPLMN), 4.3.2.2.1 (on provisioning based on PULL procedure at PCEF-initiated IP-CAN session modification when the AF is located in the VPLMN) and for cases 2a and 2b, the steps in figure 4.4.2.1.1 (Home Routed case) or 4.4.2.2.1 (Visited Access case) shall be executed instead.


Otherwise, if the BCM selected is UE/NW, the PCRF shall proceed with the subsequent steps (provisioning based on PUSH procedure) in the present figure after timer expiry.
NOTE 1:
For IMS Emergency session step 4 is not applicable.
5.
For case 2a and 2b, if Gxx applies for the IP-CAN session and the user is not roaming, or the user is roaming in a Home Routed scenario or a Visited Access scenario for case 2a when the available QoS rule are not related to any IP-CAN session, the H-PCRF may initiate Gateway Control and QoS rules provisioning procedures described in clause 4.4.3.
NOTE 2: This step is not executed if this procedure is triggered by PCEF subscribed events and/or credit re-authorisation triggers reported by the BBERF.
6.
The H-PCRF sends a Diameter RAR to request that the PCEF installs, modifies or removes PCC Rules and updates the policy decision, or to report PCEF subscribed events reported by the BBERF. The report includes the User Location Information and the User CSG Information (If received from the BBERF). If the provided PCC rules are related to an AF session associated with a sponsor that has requested sponsoring the service, the H-PCRF includes, in the Charging-Rule-Definition AVP, the Sponsor-Identity AVP and the Application-Service-Provider-Identity AVP that it received from the AF if the Reporting-Level AVP is set to the value SPONSORED_CONNECTIVITY_LEVEL and, if AF provided the application usage thresholds, the Usage-Monitoring-Information AVP. If the AF requests the access network information, the H-PCRF includes Required-Access-Info AVP in the Charging-Rule-Definition AVP and/or Charging-Rule-Remove AVP. In addition, the H-PCRF includes the Event-Trigger set to the value "ACCESS_NETWORK_INFO_REPORT" if not provided yet. If the Sharing-Key-UL AVP and/or Sharing-Key-DL AVP are provided within the Media-Component-Description AVP, the PCRF may apply the mechanisms for resource sharing as specified at 3GPP TS 29.212 [9]. The PCRF includes the Removal-Of-Access AVP set to the value REMOVAL_OF_ACCESS (0) and the IP-CAN-Type AVP set to the value of removed access to remove one access from the multi access IP-CAN session.

When the UE is roaming in a Visited Access scenario, steps 6a ~ 6e are executed instead of step 6:

6a.
The H-PCRF sends a Diameter RAR to the V-PCRF to request that the PCEF installs, modifies or removes PCC Rules and updates the policy decision. In the case of VPLMN supporting Application Detection and Control feature for solicited application reporting, some of those PCC Rules may be used for application detection and control. If the provided PCC rules are related to an AF session associated with a sponsor that has requested sponsoring the service, the H-PCRF includes, in the Charging-Rule-Definition AVP, the Sponsor-Identity AVP and the Application-Service-Provider-Identity AVP that it received from the AF if the Reporting-Level AVP is set to the value SPONSORED_CONNECTIVITY_LEVEL and, if AF provided the application usage thresholds, the Usage-Monitoring-Information AVP. If the AF requests the access network information, the H-PCRF includes Required-Access-Info AVP in the Charging-Rule-Definition AVP and/or Charging-Rule-Remove AVP. In addition, the H-PCRF includes the Event-Trigger set to the value "ACCESS_NETWORK_INFO_REPORT" if not provided yet.
6b. The V-PCRF enforces visited operator policies regarding PCC rules requested by the H-PCRF based on roaming agreements or locally configured policy. In case of TDF, V-PCRF extracts the ADC rules from the PCC Rules received from the H-PCRF and stores them.
NOTE 3:
If the V-PCRF rejects provisioned PCC rules received from the H-PCRF, the remaining steps in this call flow are not followed. Instead, the V-PCRF shall notify the H-PCRF by sending a Diameter RAA, including the Experimental-Result-Code AVP set to the value PCC_RULE_EVENT, identify the failed PCC rules as specified in 3GPP TS 29.212 [9], and additionally may provide the acceptable QoS Information for the service.
6c. In case of TDF, solicited application reporting, for roaming UE with visited access, the V-PCRF initiates the TDF session establishment, modification, or termination. If the last ADC rule is deactivated, the V-PCRF requests the TDF to terminate the TDF session toward the V-PCRF as defined in clause 4.6.2. If there is no active TDF session between the TDF and the V-PCRF, the V-PCRF requests the TDF to establish the TDF session towards V-PCRF and provides ADC Rules to the TDF as defined in clause 4.6.1. If there is already an active TDF session between the TDF and the V-PCRF, the V-PCRF provides the ADC rules to the TDF as defined in clause 4.6.3.2.
6d.
For case 2a and 2b, V-PCRF will derive the QoS rules from the PCC rules. The V-PCRF will initiate a Gateway Control and QoS Rule procedure as described in clause 4.4.3 to install, modify or remove QoS rules and optionally subscribe to new events in the BBERF.

6e.
The BBERF installs, modifies or removes the identified QoS Rules. The BBERF also enforces the authorized QoS of the corresponding QoS Rules.

6f. The V-PCRF sends a Diameter RAR to request that the PCEF installs, modifies or removes PCC Rules.
7.
The PCEF installs, modifies or removes the identified PCC Rules. The PCEF also enforces the authorized QoS and enables or disables service flow according to the flow status of the corresponding PCC Rules. If QoS information is received per QCI, PCEF shall set/update the upper limit for the MBR that the PCEF assigns to the non-GBR bearer for that QCI. In the case of PCEF supporting Application Detection and Control feature, the PCEF may also enforce application detection and control. The PCEF may perform resource sharing among PCC rules if requested from the PCRF. If allowed access type information is included in the PCC rules, the PCEF generates the NBIFOM routing rules based on the PCC rules.

The following applies for emergency sessions only:


When the PCEF receives an IP-CAN Session Modification Request from the PCRF requesting the removal of the PCC rules of an emergency session, the PCEF starts an inactivity timer to enable the PSAP to request a callback session with the UE.


When the timer expires, the PCEF initiates an IP-CAN Session Termination Request (per clause 4.2.2.1) to terminate the emergency session.


If, before the timer expires, the PCEF receives an IP-CAN Session Modification Request from the PCRF with PCC rules for an emergency session the PCEF cancels the timer.
8.
The PCEF sends a Diameter RAA to acknowledge the RAR. The PCEF informs the H-PCRF about the outcome of the PCC rule operation
When the UE is roaming in a Visited Access scenario, steps 8a ~ 8d are executed instead of step 8:
8a.
The BBERF informs the V-PCRF about the outcome of the operation by sending a Diameter RAA command.

8b.
The PCEF informs the V-PCRF about the outcome of the PCC rule operation by sending a Diameter RAA command.

8c.
The V-PCRF stores the received information.

8d.
The V-PCRF informs the H-PCRF about the outcome of the operation by sending a Diameter RAA command.
9.
In case of TDF, solicited application reporting, for non-roaming UE/roaming UE with home routed access, H-PCRF initiates the TDF session establishment, modification, or termination. If the last ADC rule is deactivated, the H-PCRF requests the TDF to terminate the TDF session toward the H-PCRF as defined in clause 4.6.2. If there is no active TDF session between the TDF and the H-PCRF, the H-PCRF requests the TDF to establish the TDF session towards H-PCRF and provides ADC Rules to the TDF as defined in clause 4.6.1. If there is already an active TDF session between the TDF and the H-PCRF, H-PCRF provides the ADC rules to the TDF as defined in clause 4.6.3.2.

NOTE 4: Step 9 can occur anytime after step 6.
10.
If traffic steering control over St applies, if the PCRF determines that the traffic steering control information needs to be provisioned for the IP-CAN session; the PCRF initiates the St session establishment procedure according to the subclause 4.9.1. If the PCRF determines that the traffic steering control information provisioned to the TSSF needs to be updated, the PCRF initiates the St session modification procedure according to the subclause 4.9.2. If the PCRF determines that the traffic steering control information is not needed for the IP-CAN session any more, the PCRF initiates the St session termination procedure according to the subclause 4.9.3.
11.
In case of spending limits, for non-roaming/roaming UE with both home routed and visited access, H-PCRFinitiates Initial/ Intermediate/ Final Spending Limit Report Request. The H-PCRF sends an Initial Spending Limit Report Request if this is the first time a status notification of policy counter is requested for the user as defined in clause 4.7.1. If the H-PCRF decides to modify the list of subscribed policy counters the H-PCRF sends an Intermediate Spending Limit Report Request as defined in clause 4.7.2. If the H-PCRF decides to unsubscribe any future status notification of policy counters, it sends a Final Spending Limit Report Request to cancels the request for reporting the change of the status of the policy counters available at the OCS as defined in clause 4.7.3.

NOTE 5: Step 11 can occur anytime after step 2.
12.
If usage monitoring is enabled and the H-PCRF either removed the last PCC rule applicable for certain monitoring key, or disabled usage monitoring or requested usage report, the PCEF shall initiate an IP-CAN session modification procedure.
13.
When Gxx does not apply for the IP-CAN session, IP-CAN bearer signalling is executed separately for each IP-CAN bearer under the following conditions:

-
If all PCC rules bound to a bearer have been removed or deactivated (bearer deactivation is applicable)

-
If one or more bearers have to be modified

-
If the PCEF needs to establish a new bearer (bearer establishment is applicable).
-
If the PCEF needs to request access network information.

-
if the PCEF needs to send the NBIFOM routing rules to the UE in the corresponding access.

14.
If the AF, in step 1, has requested notifications from the PCRF, e.g. in the case of access network information the PCEF initiates an IP-CAN session modification procedure to provide the requested information as described in clause 4.3.2.

NOTE 6:
If the conditions of both step 14 and step 12 apply the PCEF can make use of only one IP-CAN session modification procedure in step 14.
4.3.1.2
Interactions between PCRF, AF and SPR

4.3.1.2.1
AF Session Establishment
4.3.1.2.1.1
AF located in HPLMN


[image: image14.wmf] 

H

-

PCRF

 

PCEF

 

AF

 

Legend

:

 

 

       

 

Mandatory

 

               

 

 

Conditional 

 

V

-

PCRF

 

 BBERF

 

 

 

2. Diameter AAR

 

 

 

7. Diameter AAA

 

3. 

 

Store Service 

Information

 

SPR

 

4. Profile Request

 

5. Profile Response

 

If the PCRF 

needs 

subscription

-

related 

information 

and does 

not have it 

 

 

8

. 

 

Interactions in Figure 4.3.1.1.1

 

6. 

 

Identify affected IP CAN 

session(s)

 

1

. 

 

Trigger

 


Figure 4.3.1.2.1.1.1: AF session establishment triggers PCRF-Initiated IP-CAN Session Modification (AF in HPLMN)
1.
The AF receives an internal or external trigger to set-up a new AF session and provides Service Information. The AF identifies the Service Information needed (e.g. IP address of the IP flow (s), port numbers to be used, information on media types, etc).

2.
The AF provides the Service Information to the H-PCRF by sending a Diameter AAR for a new Rx Diameter session. If this AF session is associated with a sponsor, Sponsor-Identity AVP, optionally the Sponsoring-Action AVP set to applicable value and the Application-Service-Provider-Identity AVP are included in Sponsored-Connectivity-Data AVP. If usage thresholds are to be associated with this sponsored AF session, then Granted-Service-Unit AVP is included in Sponsored-Connectivity-Data AVP. The AF can request access network information within the AAR by adding Required-Access-Info AVP(s) and Specific-Action AVP set to the value "ACCESS_NETWORK_INFO_REPORT". If the Service can be subject to resource sharing, the AF includes the Sharing-Key-UL AVP and/or Sharing-Key-DL AVP within the Media-Component-Description AVP. If the service has negotiated the background data transfer policy, the AF includes the reference id of the transfer policy within the Reference-Id AVP.
3.
The H-PCRF stores the received Service Information.

4.
If the H-PCRF requires subscription related information and does not have it, the PCRF sends a request to the SPR in order to receive the information. If the Reference-Id AVP is received, the PCRF retrieves the corresponding transfer policy from the SPR and derives the PCC rules for the background data transfer according to the transfer policy.
5.
The SPR replies with the subscription related information containing the information about the allowed service(s), QoS information and PCC Rules information.
NOTE:
For steps 4 and 5: The details associated with the Sp reference point are not specified in this Release. The SPR’s relation to existing subscriber databases is not specified in this Release.
6.
If the AF session is associated with a sponsor and sponsored service is enabled,
-
if the UE is in the non-roaming case or UE is roaming with the home routed case and operator policies allow accessing the sponsored data connectivity with this roaming case, the H-PCRF authorizes the request based on sponsored data connectivity profile obtained from the SPR;

-
if the UE is roaming with the home routed case and operator policies do not allow accessing the sponsored data connectivity with this roaming case or the UE is roaming with the visited access case, the H-PCRF rejects the request.


The H-PCRF identifies the affected established IP-CAN Session(s) using the information previously received from the PCEF/V-PCRF and the Service Information received from the AF.
7.
The H-PCRF sends a Diameter AAA to the AF. The PCRF indicates whether the support for UE IP address/mask in the TFT filter is available in the IP-CAN session.
8.
The H-PCRF interacts with the PCEF/BBERF/V-PCRF according to figure 4.3.1.1.1 (Interactions between BBERF/PCEF and PCRF for PCRF-Initiated IP-CAN Session Modification).
4.3.1.2.1.2
AF located in VPLMN

[image: image15.wmf] 

H

-

PCRF

 

PCEF

 

AF

 

V

-

PCRF

 

 BBERF

 

 

ROAMING CASE

 

9

. Diameter AAA

 

10. Diameter AAA

 

4. 

Diameter AA

R

 

5.

 

Store Service 

information 

Policy 

Decision

 

SPR

 

6. Profile Request

 

7. Profile Response

 

If the PCRF 

needs 

subscription

-

related 

information 

and does 

not have it 

 

 

8

.

 

Identify affected IP CAN 

session(s)

 

11

. 

 

Interactions in Figure 4.3.1.1.1

 

2. Diameter AAR

 

3.

Store Service Information

 

Lege

nd

:

 

 

       

 

Mandatory

 

               

 

 

Conditional 

 

1. Trigger

 


Figure 4.3.1.2.1.2.1: AF session establishment triggers PCRF-Initiated IP-CAN Session Modification (AF in VPLMN)
1.
The AF receives an internal or external trigger to set-up a new AF session and provides Service Information. The AF identifies the Service Information needed (e.g. IP address of the IP flow (s), port numbers to be used, information on media types, etc).
2.
The AF provides the Service Information to the V-PCRF by sending a Diameter AAR for a new Rx Diameter session. If the AF session is associated with a sponsor, Sponsor-Identity AVP and Application-Service-Provider-Identity are included in Sponsored-Connectivity-Data AVP. If usage thresholds are to be associated with this sponsored AF session, then Granted-Service-Unit AVP is included in Sponsored-Connectivity-Data AVP. The AF can request access network information within the AAR by adding Required-Access-Info AVP(s) and Specific-Action AVP set to the value "ACCESS_NETWORK_INFO_REPORT". If the Service can be subject to resource sharing, the AF includes the Sharing-Key-UL AVP and/or Sharing-Key-DL AVP within the Media-Component-Description AVP.
3.
The V-PCRF stores the Service Information.
NOTE: 
The V-PCRF may employ operator policies and reject the AAR from the AF if the provided Service Information is not acceptable. If this happens, the V-PCRF replies immediately to the AF, includes an unsuccessful Result-Code or Experimental-Result-Code in the AAA, and the remaining steps of this call flow are not carried out.
4.
The V-PCRF forwards the Diameter AAR to the H-PCRF.
5.
The H-PCRF stores the received Service Information.
6. If the H-PCRF requires subscription-related information and does not have it, the H-PCRF sends a request to the SPR in order to receive the information.

7.
The SPR replies with the subscription related information containing the information about the allowed service(s), QoS information and PCC Rules information.
NOTE:
For steps 6 and 7: The details associated with the Sp reference point are not specified in this Release. The SPR’s relation to existing subscriber databases is not specified in this Release.
8.
If the AF session is associated with a sponsor, the H-PCRFrejects the request. Otherwise, the H-PCRF stores the Service Information and identifies the affected established IP-CAN Session (s) using the information previously received from the PCEF via the V-PCRF and the Service Information received from the AF.
9.
The H-PCRF responds to the V-PCRF with a Diameter AAA. The H- PCRF indicates whether the support for UE IP address/mask in the TFT filter is available in the IP-CAN session.
10. The V-PCRF forwards the Diameter AAA to the AF.

11. The H-PCRF interacts with the PCEF/BBERF via the V-PCRF according to figure 4.3.1.1.1 (Interactions between BBERF/PCEF and PCRF for PCRF-Initiated IP-CAN Session Modification).
4.3.1.2.2
AF session modification
4.3.1.2.2.1
AF located in the HPLMN
[image: image16.wmf] 

6. 

 

Diameter AAA

 

3. 

 

Diameter AAR

 

AF

 

H

-

PCRF

 

PC

EF

 

1. 

 

Trigger

 

 

2.

 

Define service 

information

 

 

4. 

 

Store Service 

Information

 

5.

 

Identify affected IP CAN 

session(s)

 

V

-

PCRF

 

BBERF

 

7. 

 

Interactions in Figure 4.3.1.1.1

 


Figure 4.3.1.2.2.1.1: AF session modification triggers PCRF-Initiated IP-CAN Session Modification (AF in HPLMN)
1.
The AF receives an internal or external trigger to modify an existing AF session and provide related Service Information.

2.
The AF identifies the Service Information needed (e.g. IP address of the IP flow(s), port numbers to be used, information on media types, etc.).

3.
The AF provides the Service Information to the H-PCRF by sending a Diameter AAR for the existing Rx Diameter session corresponding to the modified AF session. If this AF session is associated with a sponsor, Sponsor-Identity AVP, optionally if the AF wants to enable sponsored dataconnectivity the Sponsoring-Action AVP set to the value "ENABLE_SPONSORING" and Application-Service-Provider-Identity are included in Sponsored-Connectivity-Data AVP. If application usage thresholds are to be associated with this sponsored AF session, then Granted-Service-Unit AVP is included in Sponsored-Connectivity-Data AVP. If this AF session is associated with a sponsor and the AF wants to disable sponsored data connectivity the AF includes the Sponsoring-Action AVP set to the value "DISABLE_SPONSORING" within the Sponsored-Connectivity-Data AVP. The AF can request access network information within the AAR by adding Required-Access-Info AVP(s) and Specific-Action AVP set to the value "ACCESS_NETWORK_INFO_REPORT". If resource sharing conditions have been changed, the AF includes the Sharing-Key-UL AVP and/or Sharing-Key-DL AVP within the Media-Component-Description AVP. If the service has negotiated the background data transfer policy, the AF includes the reference id of the transfer policy within the Reference-Id AVP.
4.
The H-PCRF stores the received Service Information. If the Reference-Id AVP is received, the PCRF retrieves the corresponding transfer policy from the SPR and derives the PCC rules for the background data transfer according to the transfer policy.
5.
If the AF session is associated with a sponsor and sponsored service is enabled,

-
if the UE is in the non-roaming case or UE is roaming with the home routed case and operator policies allow accessing the sponsored data connectivity with this roaming case, the H-PCRF authorizes the request based on sponsored data connectivity profile obtained from the SPR;

-
if the UE is roaming with the home routed case and operator policies do not allow accessing the sponsored data connectivity with this roaming case or the UE is roaming with the visited access case, the H-PCRF rejects the request.

The H-PCRF identifies the affected established IP-CAN Session(s) using the information previously received from the PCEF/V-PCRF and the Service Information received from the AF.

6.
The H-PCRF sends a Diameter AAA to the AF.

7.
The H-PCRF interacts with the BBERF/PCEF/V-PCRF according to figure 4.3.1.1.1.

4.3.1.2.2.2
AF located in the VPLMN


[image: image17.emf] 

ROAMING CASE  

8.  Diameter AAA

5.  Diameter AAR

3.  Diameter AAR

AF

H-PCRF

PCEF V-PCRF BBERF

10.  Interactions in Figure 4.3.1.1.1

4.  Store Service 

Information

9.  Diameter AAA

7. Identify affected IP CAN 

session(s)

1.  Trigger

2. Define service 

information

6.  Store Service 

Information


Figure 4.3.1.2.2.2.1 AF session modification triggers PCRF-Initiated IP-CAN Session Modification (AF in VPLMN)

1.
The AF receives an internal or external trigger to modify an existing AF session and provide related Service Information.

2.
The AF identifies the Service Information needed (e.g. IP address of the IP flow(s), port numbers to be used, information on media types, etc.). If resource sharing conditions have been changed, the AF includes the Sharing-Key-UL AVP and/or Sharing-Key-DL AVP within the Media-Component-Description AVP.
3.
The AF provides the Service Information to the V-PCRF by sending a Diameter AAR for the existing Rx Diameter session corresponding to the modified AF session. If this AF session is associated with a sponsor, Sponsor-Identity AVP and Application-Service-Provider-Identity AVP are included in Sponsored-Connectivity-Data AVP. If usage thresholds are to be associated with this sponsored AF session, then Granted-Service-Unit AVP is included in Sponsored-Connectivity-Data AVP. The AF can request access network information within the AAR by adding Required-Access-Info AVP(s) and Specific-Action AVP set to the value "ACCESS_NETWORK_INFO_REPORT".
4.
The V-PCRF stores the received Service Information.

NOTE: The V-PCRF may employ operator policies and reject the AAR from the AF if the provided Service Information is not acceptable. If this happens, the V-PCRF replies immediately to the AF, includes an unsuccessful Result-Code or Experimental-Result-Code in the AAA, and the remaining steps of this call flow are not carried out.

5.
The V-PCRF forwards the Diameter AAR to the H-PCRF.

6.
The H-PCRF stores the received Service Information.

7.
The H-PCRF identifies the affected established IP-CAN Session(s) using the information previously received from the PCEF/V-PCRF and the Service Information received from the AF.

8.
The H-PCRF responds with a Diameter AAA.

9.
The V-PCRF forwards the Diameter AAA to the AF.

10.
The H-PCRF interacts with the BBERF/PCEF via the V-PCRF according to figure 4.3.1.1.1.
4.3.1.2.3
AF session termination
4.3.1.2.3.1
AF located in the HPLMN

[image: image18.emf]4a. Diameter STA

2.  Diameter STR

AF

H-PCRF

PCEF

1.  Trigger

3.

Identify affected IP CAN 

session where PCC/QoS 

rules need to be 

removed.

V-PCRF BBERF

5.  Interactions in Figure 4.3.1.1.1

Diameter STA 4b.


Figure 4.3.1.2.3.1.1: Removal of PCC/QoS Rules at AF session release (AF in HPLMN)
1.
The AF receives an internal or external trigger for a session release.

2.
The AF sends a session termination request, Diameter STR, to the H-PCRF to request the removal of the session. The AF can request access network information within the STR by adding a Required-Access-Info AVP.
3.
The H-PCRF identifies the affected IP-CAN Session where PCC Rules and, if available, QoS Rules for the IP flow(s) of this AF session are installed. These PCC/QoS Rules need to be removed.
If the AF did not request access network information, and if no usage thresholds due to an AF session associated with a sponsor were provided that relate to the installed PCC rules and RAN-NAS-Cause feature is not supported, step 4a applies. If AF requested access network information, or usage thresholds due to an AF associated with a sponsor were provided that relate to the installed PCC rules,step 4b applies. If RAN-NAS-Cause is supported but Enh-RAN-NAS-Cause is not supported over Gx and the previous conditions do not apply , step 4a applies.. Otherwise step 4b applies.
4a.
The H-PCRF sends Diameter STA, session termination answer, to the AF.
4b.
The H-PCRF sends Diameter STA, session termination answer, to the AF and includes access network information and/or information about the resources that have been consumed by the user since the last report or RAN/NAS/TWAN/Untrusted WLAN release cause(s) if available obtained in step 5.
5.
The H-PCRF interacts with the BBERF/PCEF/V-PCRF according to figure 4.3.1.1.1.
4.3.1.2.3.2
AF located in the VPLMN


[image: image19.emf] 

ROAMING CASE  

5a.  Diameter STA

3. Diameter STR

2.  Diameter STR

AF

H-PCRF PCEF

V-PCRF

BBERF

7.  Interactions in Figure 4.3.1.1.1 

6a.  Diameter STA

1.  Trigger

4.  Identify affected IP 

CAN session where 

PCC/QoS rules need

to be removed

5bDiameter STA

6bDiameter STA


Figure 4.3.1.2.3.2.1: Removal of PCC/QoS Rules at AF session release (AF in VPLMN)

1.
The AF receives an internal or external trigger for a session release.

2.
The AF sends a session termination request, Diameter STR, to the V-PCRF to request the removal of the session. The AF can request access network information within the STR by adding a Required-Access-Info AVP.
3.
The V-PCRF forwards the Diameter STR to the H-PCRF.

4.
The H-PCRF identifies the affected IP-CAN Session where PCC Rules and, if available, QoS Rules for the IP flow(s) of this AF session are installed. These PCC/QoS Rules need to be removed.
Steps 5a and 6a apply if the AF did not request access network information
5a.
The H-PCRF sends Diameter STA, session termination answer, to the V-PCRF.

6a.
The V-PCRF forwards the Diameter STA to the AF.

7.
The H-PCRF interacts with the BBERF/PCEF via the V-PCRF according to figure 4.3.1.1.1.
Steps 5b and 6b apply if the AF requested access network information

5b.
The H-PCRF sends Diameter STA, session termination answer, to the V-PCRF and includes access network information obtained in step 7.

6b.
The V-PCRF forwards the Diameter STA to the AF.
4.3.2
PCEF –Initiated IP-CAN Session Modification (PCC Rule Provisioning in PULL Mode)

4.3.2.1
PCEF-initiated IP-CAN Session Modification. AF located in HPLMN.
This flow shows the provisioning of PCC Rules and/or authorized QoS triggered by the PCEF.

[image: image20.emf]V

I

S

I

T

E

D

 

A

C

C

E

S

S

C

A

S

E

BBERF PCEF TDF V-PCRF H-PCRF AF SPR OCS

3a. Diameter CCR

V

I

S

I

T

E

D

 

A

C

C

E

S

S

C

A

S

E

TSSF

1. Gateway control and QoS Rule request (case 2a & case 2b)

2. IP-CAN Session Modification

3. Diameter CCR

3b. Store 

Information

3c. Diameter CCR

4. Profile Request

5. Profile Response

6. Diameter RAR

7. Diameter RAA

8. Diameter ASR

9. Diameter ASA

10. Diameter STR

11. Diameter STA

If a specific action 

is triggered for the 

AF session

If all session info 

for an AF session 

is decided

Initial/Intermediate/Final Spending limit Report Request

12. PCC/ADC Rule Decision

13. Diameter CCA

13a. Diameter CCA

13b. PCC/ADC Rules 

validation

13c. Diameter CCA

13d. TDF session modification

14. TDF session modification

15. Install, modify or 

remove PCC/ADC Rules

17. IP-CAN Session modification signalling

18. Resource allocation confirmation (case 1)

19. Gateway control & QoS Rule Provision (case 2a & case 2b)

16. St session 

establishment/modification/termination


Figure 4.3.2.1.1: PCEF-initiated IP-CAN Session Modification. AF in HPLMN.

1.
For case 2a and 2b, the BBERF may initiate Gateway Control and QoS rules request procedure described in subclause 4.4.2.

2.
The PCEF may receive a request for IP‑CAN Session modification. The IP-CAN session modification can be initiated upon receiving UE-initiated resource modification request (case 1), a new IP-CAN bearer establishment signalling (case 1), due to a specific event (e.g. UE requested PDN connectivity in all cases) or an internal trigger(e.g. if the PCEF supports Application Detection and Control feature, the start/stop of application traffic event that matches with one or more activated PCC rules for application detection and control that do not contain the Mute-Notification AVP has been detected by the PCEF).
3.
The PCEF informs the H-PCRF about the IP-CAN session modification for non-roaming case and Home Routed roaming scenario. The PCEF sends a CCR command to the H-PCRF including the CC-Request-Type AVP set to the value “UPDATE_REQUEST”. For an IP-CAN Session modification where an existing IP-CAN bearer is modified, the PCEF supplies the specific event that caused the IP-CAN Session modification within the Event-Trigger AVP and the PCC rule name(s) and their status within the Charging-Rule-Report AVP. For an IP-CAN Session modification where an existing IP-CAN bearer is terminated, the PCEF supplies the affected PCC rule name(s), their status set to inactive, the rule failure code and, if available, the RAN/NAS/TWAN/Untrusted WLAN cause(s) within the Charging-Rule-Report AVP. In the case where the UE initiates a resource modification request procedure, the PCEF includes the Packet-Filter-Information AVP, Packet-Filter-Operation AVP and QoS-Information AVP, if applicable. In the case of PCEF supporting Application Detection and Control feature, when the start or stop of the application’s traffic, identified by TDF-Application-Identifier, is detected, if PCRF has previously subscribed to the APPLICATION_START/APPLICATION_STOP Event-Triggers, the PCEF shall report the information regarding the detected application’s traffic in the Application-Detection-Information AVP in the CCR command. If NBIFOM applies to the IP-CAN session, if one of the conditions specified in subclause 4.5.25.2 of 3GPP TS 29.212 [9] applies, the PCEF informs the PCRF accordingly making use of the applicable event trigger, i.e. "ADDITION_OF_ACCESS", " REMOVAL_OF_ACCESS", "UNAVAILIBILITY_OF_ACCESS", "AVAILIBILITY_OF_ACCESS" or "ROUTING_RULE_CHANGE".
When the UE is roaming in a Visited Access case, steps 3a ~ 3c are executed instead of step 3:

3a.
The PCEF sends a Diameter CCR to the V-PCRF to request PCC/ADD Rules for the roaming user. The parameters listed in step 3 are applicable here.

3b. The V-PCRF stores the information received in the Diameter CCR from the PCEF.

3c. The V-PCRF sends a CCR command with the CC-Request-Type AVP set to “UPDATE_REQUEST” to the H-PCRF. The V-PCRF includes the Subsession-Enforcement-Info AVP and the assigned S9 subsession identifier within Subsession-Id AVP. The Subsession-Operation AVP is set to the value “MODIFICATION”.

4.
If the H-PCRF requires subscription-related information and does not have it, the PCRF sends a request to the SPR in order to receive the information.
5.
The SPR replies with the subscription related information containing the information about the allowed service(s) and PCC Rules information.

NOTE 1:
For steps 4 and 5: The details associated with the Sp reference point are not specified in this Release. The SPR’s relation to existing subscriber databases is not specified in this Release.

6.
If the AF requested a notification of the corresponding event, the H-PCRF sends a Diameter RAR with the Specific-Action AVP set to indicate the event that caused the request. If the session modification affected a sponsored data flow and the H-PCRF detects that the usage threshold provided by the AF has been reached, this message includes the accumulated usage in the Used-Service-Unit AVP within the Sponsored-Connectivity-Data AVP and the Specific-Action AVP set to the value USAGE_REPORT.
7.
If step 6 takes place, the AF may take the application specific procedure (e.g. for IMS refer to 3GPP TS 23.228 [59], replies with a Diameter RAA and may provide updated service information within. Additionally, the AF may terminate the Rx session as per subclause 4.3.1.2.3.

8-11.
If all service data flows for an AF session are deleted, the AF session is terminated. If the session modification affected a sponsored data flow and the H-PCRF detects the UE is roaming with home routed case, the H-PCRF initiates the AF session termination.
 If the IP-CAN session is associated with a sponsor, usage thresholds were provided by the AF earlier, and the H-PCRF has usage data that has not yet been reported to the AF, the H-PCRF informs the AF, in step 11, about the resources that have been consumed by the user since the last report.
If RAN-NAS-Cause feature is supported and RAN/NAS/TWAN/Untrusted WLAN cause(s) and/or access network information were received in step 3, the H-PCRF sends this information to the AF in step 11.
NOTE 2:
Initial/intermediate/Final Spending Limit Report Request can be triggered at any time after this if PCRF, based on policy decisions, find the need to initialize, modify, or deactivate spending limit reporting for the subscriber according to subclause 4.7.1/2/3 respectively.

12. 
The H-PCRF selects or generates PCC Rule(s) to be installed. The H-PCRF may also identify existing PCC rules that need to be modified or removed. In the case of VPLMN supporting Application Detection and Control feature for solicited application reporting, some of those PCC Rules may be used for application detection and control. The PCC Rules may relate to any of the matching AF sessions or may exist in the PCRF without matching to any AF session. The H-PCRF may also make a policy decision by deriving an authorized QoS and by deciding whether service data flows described in the PCC Rules are to be enabled or disabled. The H-PCRF may also update the ADC decisions and select the ADC rules to be installed, modified or removed for the IP-CAN session in the non-roaming case. If the NBIFOM applies to the IP-CAN session, the PCRF can also make a decision of NBIFOM as defined in subclause 4.5.25.2 of 3GPP TS 29.212 [9].
13. For the non-roaming case, and for the case when the UE is roaming in a Home Routed scenario, the H-PCRF provisions the PCC Rules to the PCEF using CCA command. The H-PCRF also provides the selected Bearer Control Mode, if changed and applicable for the IP-CAN type. The PCRF may also provide a new list of event triggers for which the PCRF requires to be notified. The PCRF may provide QoS information within the APN-AMBR AVP and the Default-EPS-Bearer-QoS AVP. In the case of PCEF supporting Application Detection and Control feature, the H-PCRF may provision the PCC rules for application detection and control to the PCEF.
When the UE is roaming in a Visited Access, steps 13a ~13c are executed instead of step 13:

13a. The H-PCRF sends a Diameter CCA to the V-PCRF including the PCC Rules to be provisioned within the Subsession-Decision AVP, along with the S9 subsession identifier as received in step 3b within the Subsession-Id AVP. Other parameters listed in step 9 are also applicable here.

13b. The V-PCRF validates the QoS parameters requested within the PCC Rules and enforces visited operator policies regarding QoS authorization requested by the H-PCRF as indicated by the roaming agreements. In case of TDF, the V-PCRF extracts and validates the ADC rules from the PCC rules received from the H-PCRF according to the local policy and roaming agreements if provided by the H-PCRF.
NOTE:
If the V-PCRF rejects provisioned PCC rules received from the H-PCRF, the remaining steps in this call flow are not followed. Instead, the V-PCRF shall notify the H-PCRF by sending a Diameter CCR, including the Experimental-Result-Code AVP set to the value PCC_RULE_EVENT, identify the failed PCC rules as specified in 3GPP TS 29.215 [22], and additionally may provide the acceptable QoS Information for the service.

13c. The V-PCRF provisions PCC rules to the PCEF by using CCA command. The parameters listed in step 13a are applicable here.
13d. In case of TDF, solicited application reporting, the V-PCRF provisions the ADC rules to the TDF as defined in subclause 4.6.3.2. In case of TDF, unsolicited application reporting, the V-PCRF initiates the TDF session termination as defined in clause 4.6.2 if the PCEF reported the UE_IP_ADDRESS_RELEASE to the V-PCRF and there is an active Ipv4 address related TDF session for that IP-CAN session.
14. In case of TDF, solicited application reporting, the H-PCRF provisions the ADC rules to the TDF as defined in subclause 4.6.3.2. In case of TDF, unsolicited application reporting, the H-PCRF initiates the TDF session termination as defined in subclause 4.6.2 if the PCEF reported the UE_IP_ADDRESS_RELEASE to the H-PCRF and there is an active Ipv4 address related TDF session for that IP-CAN session.
15. The PCEF installs, modifies or removes the provided PCC Rules. The PCEF also enforces the authorized QoS and enables or disables service flows according to the flow status of the corresponding PCC Rules. In the case of PCEF supporting Application Detection and Control feature, the PCEF enforces application detection and control.
16. If traffic steering control over St applies, if the PCRF determines that the traffic steering control information needs to be provisioned for the IP-CAN session; the PCRF initiates the St session establishment procedure according to the subclause 4.9.1. If the PCRF determines that the traffic steering control information provisioned to the TSSF needs to be updated, the PCRF initiates the St session modification procedure according to the subclause 4.9.1. If the PCRF determines that the traffic steering control information is not needed for the IP-CAN session any more, the PCRF initiates the St session termination procedure according to the subclause 4.9.2. 
17.
The PCEF may initiate IP-CAN session signalling or acknowledges any IP‑CAN Session signalling for IP-CAN Session modification received in step 2.

18.
If the PCRF requested to confirm that the resources associated to a PCC Rule have been successfully allocated or the resource release procedure has concluded, the PCEF-initiated IP-CAN session modification procedure is performed again starting from step 3.

19.
For case 2a and 2b, the PCRF may initiate Gateway Control and QoS rules Provision procedure described in subclause 4.4.3.
4.3.2.2
PCEF-initiated IP-CAN Session Modification, AF located in the VPLMN

[image: image21.emf] 

BBERF   PCEF   V - PCRF   V - AF   H - PCRF  

SPR  

If all session  info for an AF  session is  deleted  

11. Diameter RAR  

4. Diameter AS A  

5. Diameter ASA  

10. Diameter RAR  

2. Diameter ASR  

12. Diameter RAA  

9. Diameter STA  

3. Diameter ASR  

13 . Diameter  RAA  

7. Diameter STR  

6. Diameter STR  

8. Diameter STA  

If  a specific  action is  triggered for  the AF  session     

Legend :                 Mandatory                       Conditional   

TDF  

1.  Steps 1  to 5 as specified in figure 4.3.2.1.1 .   

14.  Steps 1 2 to 1 8  as specified in figure 4.3.2.1.1 .   


Figure 4.3.2.2.1: PCEF-initiated IP-CAN Session Modification, AF in VPLMN. If the AF resides in the VPLMN, the V‑PCRF proxies the AF session signalling over S9 between the V-AF and the H‑PCRF.
04.�.� Steps 1 to 5 in figure 4.3.2.1.1 are executed.


 When all PCC Rules related to a particular AF session are removed, the H-PCRF initiates the AF session termination procedure. For each AF session bound to the modified IP-CAN session that is being removed, steps 2-9 are executed instead of steps 10-13:

2.
The H-PCRF indicates the session abort to the V-PCRF by sending a Diameter ASR to the V-PCRF.
3.
The V-PCRF proxies the Diameter ASR command to the V-AF.
4.
The V-AF responds by sending a Diameter ASA command to the V-PCRF.
5.
The V-PCRF proxies the ASA command to the H-PCRF.
6.
The V-AF sends a Diameter STR command to the V-PCRF to indicate that the session has been terminated.
7.
The V-PCRF proxies the Diameter STR command to the H-PCRF.
8.
The V-PCRF proxies the Diameter STA command to the V-AF.
9.
The H-PCRF responds by sending a Diameter STA command.

When the H-PCRF receives event triggers related to specific actions that the AF has subscribed to, the H-PCRF initiates the AF session modification procedure to notify the AF of these specific actions. For each Afsession bound to the modified IP-CAN session that has subscribed to these specific actions, steps 10-13 are executed instead of steps 2-9:

10. If the H-PCRF is notified of an event in the access network that has to be notified to the V-AF for an AF session, the H-PCRF informs of the event by sending a RAR command to the V-PCRF.

11. The V-PCRF proxies the RAR command to the V-AF.

12.
The V-AF responds by sending a RAA command to the V-PCRF.
13. The V-PCRF proxies the RAA to the H-PCRF.
14.
Steps 12 to 18 in figure 4.3.2.1.1 are executed.
4.4
Gateway Control Session Procedures

There are two kinds of Gateway Control (GC) sessions:

· A Gateway Control session that serves a single IP-CAN session (e.g. S-GW/BBERF connecting to PDN-GW using S5/S8 PMIP according to 3GPP TS 23.402 [21]).

· A Gateway Control session that serves all the IP-CAN sessions from the same Care-of address of the UE (e.g. a UE connecting to PDN-GW using S2c according to 3GPP TS 23.402 [21]).

These Gateway Control sessions are initiated in connection with IP-CAN session establishment and Initial Attach respectively. For the first case, the PCRF will identify that the GC session serves a single IP-CAN session based on the PDN Identifier received in the request.
An access network may support mobility with BBERF change. The new BBERF shall establish new Gateway Control sessions according to the procedures defined for the new access type and the PCRF shall correlate those sessions with ongoing IP-CAN sessions as part of the handover procedure.

These scenarios are shown separately in different flows.

In the following procedures, the V-PCRF is included to depict the roaming scenarios. H-PCRF will act as a PCRF for non-roaming Ues. The procedure to detect that the IP-CAN Session is restricted to Emergency Services is described in 3GPP TS 29.212 [9].
4.4.1
Gateway Control Session Establishment


[image: image22.wmf] 

H

-

PCRF

 

PCEF

 

8. 

 

Diameter CCA

 

6. 

 

PCC Rules Decision,

 

Policy Decision

 

3

. 

Store Information

 

7. 

 

Store PCC Rules 

 

SPR

 

4. 

 

Profile Request

 

5. 

 

Profile Response

 

If the PCRF needs 

subscription

-

related 

information and do

es 

not have it 

 

 

1. 

 

Establish Gateway 

Control 

Session Request

 

10. 

 

Establish Gateway Control

 

Session

 Response

 

Legend

:

 

 

       

 

Mandatory

 

               

 

 

Conditional 

 

V

-

PCRF

 

2b

.

 

Store Information

 

BBERF

 

 

8b

. 

 

Store PCC Rules. Policy 

decision

 

8

e

. 

 

Diameter CCA

 

ROAMING CASE

 

2c. 

Diameter CCR

 

2. 

 

Diameter CCR

 

8c

. 

 

Diameter CCR

 

8d

. 

 

Diameter CCA

 

2a. 

 

Diameter CCR

 

ROAMING CASE

 

9. 

 

Install 

QoS

 Rules. 

Policy enforcement

 

8a. 

 

Diameter CCA

 


Figure 4.4.1.1 Gateway Control Session Establishment.

1.
The BBERF receives a message or indication that it needs to establish a Gateway Control session.
For case 2a, as defined in clause 4.0, the BBERF detects that a UE has been assigned a Local IP address that the UE may use as a Care-of Address in MIP registrations (see 3GPP TS 23.402 [21], clause 6.3).
For case 2b, as defined in clause 4.0, the BBERF detects that the UE requests an IP-CAN session to be established (see 3GPP TS 23.402 [21], clauses 4.5.2 and 5.6.1) or, at BBERF relocation, to be resumed with a certain APN (see 3GPP TS 23.402 [21], clauses 5.7.1 and 5.7.2) or the UE requests a pre-registration with this BBERF (see 3GPP TS 23.402 [21], clause 9.3.1).
2.
For the non-roaming case, the BBERF initiates a Gateway Control session with the H-PCRF by sending a CCR to the H-PCRF with the CC-Request-Type AVP set to the value INITIAL_REQUEST. The BBERF provides UE identity information and the IP-CAN type, User Location Information, User CSG Information (if received from the access network) and the indication of the BBERF support for the extended TFT filters.
For case 2a, as defined in clause 4.0, the BBERF provides the CoA assigned to the UE.
For case 2b, as defined in clause 4.0,the BBERF provides the PDN identifier and PDN connection identifier, if multiple PDN connections for the same APN are supported and, if applicable, a Session-Linking-Indicator to indicate if the session linking has to be deferred. The BBERF provides, when available, the APN-AMBR and Default-EPS-Bearer-QoS.
NOTE:
The BBERF support is a prerequisite for the PCRF enabling the possibility for usage of the extended TFT filter in the IP-CAN session(s).

If applicable for the IP-CAN type, the BBERF additionally provides Network-Request-Support AVP to indicate whether NW-initiated procedures are supported.

When the UE is roaming, the steps 2a-2c are executed instead of step 2:

2a.
The BBERF initiates a Gateway Control session with the V-PCRF by sending a CCR to the V-PCRF with the CC-Request-Type AVP set to the value INITIAL_REQUEST. The BBERF provides UE identity information and the IP-CAN type, User Location Information and User CSG Information (if received from the access network).
For case 2a, as defined in clause 4.0, the BBERF provides the CoA assigned to the UE.
For case 2b, as defined in clause 4.0, the BBERF provides the PDN identifier and, if applicable, a Session-Linking-Indicator AVP to indicate if the session linking has to be deferred. The BBERF provides, when available, the APN-AMBR and Default-EPS-Bearer-QoS.

If applicable for the IP-CAN type, the BBERF additionally provides Network-Request-Support AVP to indicate whether NW-initiated procedures are supported.

2b.
The V-PCRF determines based on the UE identity information that the request is for a roaming user. The V-PCRF checks whether the V-PCRF needs to send the CCR to the H-PCRF based on the roaming agreements. For the Visited Access case, the V-PCRF does not send the CCR to the H-PCRF if the Session-Linking-Indicator AVP was received indicating that the session linking has to be deferred.
NOTE:
If the V-PCRF does not send the CCR to the H-PCRF, the PCRF may generate QoS rules based on VPLMN roaming agreements.

2c.
For case 2a:

-
If there is not an already established S9 session for this roaming user, the V-PCRF sends a CCR to the H-PCRF with the CC-Request-Type AVP set to the value INITIAL_REQUEST. The V-PCRF includes in the CCR the information received in step 2a.
-
If there is an already established S9 session for this roaming user, the V-PCRF sends a CCR to the H-PCRF with the CC-Request-Type AVP set to the value UPDATE_REQUEST. The V-PCRF includes in the CCR the information received in step 2a.

For case 2b and for the visited case or for the home routed case and if the Session-Linking-Indicator AVP is not received or it indicates SESSION_LINKING_IMMEDIATE, the following procedures apply:

-
If there is not an already established S9 session for this roaming user, the V-PCRF sends a CCR to the H-PCRF with the CC-Request-Type AVP set to the value INITIAL_REQUEST. The V-PCRF includes the Subsession-Enforcement-Info AVP within the CCR with a new S9 subsession identifier assigned by the V-PCRF to this Gateway Control Session within the Subsession-Id AVP, and the Subsession-Operation AVP set to the value ESTABLISHMENT.
-
If there is an already established S9 session for this roaming user and not an already established S9 subsession for the PDN connection corresponding to the Gateway Control Session, the V-PCRF sends a CCR to the H-PCRF with the CC-Request-Type AVP set to the value UPDATE_REQUEST. The V-PCRF includes the Subsession-Enforcement-Info AVP within the CCR with a new S9 subsession identifier assigned by the V-PCRF to this Gateway Control Session within the Subsession-Id AVP, and the Subsession-Operation AVP set to the value ESTABLISHMENT.
-
If there is an already established S9 session for this roaming user and an already established S9 subsession for the PDN connection corresponding to the Gateway Control Session, the V-PCRF sends a CCR to the H-PCRF with the CC-Request-Type AVP set to the value UPDATE_REQUEST. The V-PCRF includes the Subsession-Enforcement-Info AVP within the CCR command with the S9 subsession identifier assigned by the V-PCRF for this Gateway Control Session within the Subsession-Id AVP, the Subsession-Operation AVP set to the value MODIFICATION, and the BBERF identity within AN-GW-Address AVP.
For case 2b and for the home routed case and if the Session-Linking-Indicator AVP was received indicating that the session linking has to be deferred, following procedure applies:

-
The V-PCRF sends a CCR to the H-PCRF with the CC-Request-Type AVP set to the value UPDATE_REQUEST. The V-PCRF includes the Subsession-Enforcement-Info AVP within the CCR with a new S9 subsession identifier assigned by the V-PCRF to this Gateway Control Session within the Subsession-Id AVP, the Subsession-Operation AVP set to the value ESTABLISHMENT and the Session-Linking-Indicator AVP set to the value “SESSION_LINKING_DEFERRED”.
3.
The H-PCRF stores the information received in the CCR. The H-PCRF determines the network scenario that applies (case 2a or 2b) as described in clause 4.0.

For case 2a, the H-PCRF may correlate the UE identity information with already established Gx sessions for the same UE.
For case 2b, for non roaming case, the H-PCRF links the Gateway Control session with the already established Gx Session and acts as follows:

-
if the Session-Linking-Indicator was received indicating that the session linking has to be deferred, defers the session linking till the associated IP-CAN session establishment or modification is received.

-
if the Session-Linking-Indicator was not received or indicates that the session linking has to be performed immediately, links the Gateway Control session with the already established Gx Session.
4.
If the H-PCRF requires subscription-related information and does not have it, the H-PCRF sends a request to the SPR in order to receive the information.

5.
The SPR replies with the subscription related information containing the information about the allowed service(s), QoS information, PCC Rules information and may include MPS EPS Priority, MPS Priority Level and IMS Signalling Priority of establishment a PS session with priority.
NOTE:
For steps 4 and 5: The details associated with the Sp reference point are not specified in this Release. The SPR’s relation to existing subscriber databases is not specified in this Release.
6.
For case 2a, the H-PCRF may prepare for the installation of QoS rules if available;
For case 2b, the H-PCRF may
-
At IP-CAN session establishment, if the session linking was not deferred, select or generate and store PCC Rule(s) in preparation for the anticipated Gx session and derive the QoS rules from them. If MPS EPS Priority, MPS Priority Level, and IMS Signalling Priority are present for the user, the PCRF takes the information into account. If the session linking was deferred, the PCC rules are not generated;

-
At BBERF relocation and at pre-registration, if the Session-Linking-Indicator was not received or indicates that the session linking has to be performed immediately, prepare for the installation of QoS rules, derived from the active PCC rules, at the target BBERF;
7.
The H-PCRF stores the selected QoS Rules and PCC Rules. If applicable the H-PCRF selects the Bearer Control Mode that will apply during the Gateway Control session.

8.
For the non-roaming case, the H-PCRF acknowledges the Gateway Control Session by sending a CCA to the BBERF. The H-PCRF includes:
-
The selected BCM, if applicable for the IP-CAN type
-
If NW-initiated procedures are available, the available QoS rules
-
If BCM is UE-only, the QoS rules that correspond to the request from the BBERF
-
Default-EPS-Bearer-QoS and APN-AMBR when applicable
-
The event triggers


When the UE is roaming, the steps 8a-8e are executed instead of step 8:

8a. The H-PCRF acknowledges the Gateway Control Session by sending a CCA to the V-PCRF. The H-PCRF includes

-
The selected BCM, if applicable for the IP-CAN type
-
If NW-initiated procedures are available, the available QoS rules for the home routed case or the available PCC rules for the visited access case
-
If BCM is UE-only, the QoS rules that correspond to the request from the V-PCRF for the home routed case or the PCC rules that correspond to the request from the V-PCRF for the visited access case

-
For the case 2a, the QoS rules when the available QoS rules are not related to any IP-CAN session
-
Default-EPS-Bearer-QoS and APN-AMBR when applicable
-
Event triggers

8b.
The V-PCRF enforces visited operator policies regarding QoS authorization requested by the H-PCRF as indicated by the roaming agreements.
8c.
If the V-PCRF denies an authorization, it informs the H-PCRF and may provide the acceptable QoS Information for the service.

8d.
The H-PCRF may provide new or modified QoS rules to the V-PCRF

8e.
If V-PCRF receives the PCC rules from the H-PCRF, the V-PCRF extracts the QoS rules from the PCC rules. The V-PCRF acknowledges the Gateway Control Session establishment by sending a CCA to the BBERF. The V-PCRF includes the selected BCM if applicable for the IP-CAN type, any applicable QoS rules and event triggers.

9.
The BBERF installs and enforces the received QoS Rules.
10.
The BBERF sends an Establish Gateway Session Control Response to ack the Gateway Control Session Request.
4.4.2
Gateway Control and QoS Rules Request
4.4.2.1
Non-Roaming and Home Routed cases

[image: image23.emf]R

o

a

m

i

n

g

 

c

a

s

e

(

h

o

m

e

-

r

o

u

t

e

d

)

R

o

a

m

i

n

g

 

c

a

s

e

(

h

o

m

e

 

r

o

u

t

e

d

)

BBERF

2. Diameter CCR

PCEF V-PCRF H-PCRF

2a. Diameter CCR

2c. Diameter CCR

4b. Extract QoS rules

3. Store information and 

update QoS rules

4. Diameter CCA

4a. Diameter CCA

4c. Diameter CCA

1. Trigger to request QoS 

rules or report event

2b. Store information

5. Install and update QoS 

rules


Figure 4.4.2.1.1: Gateway Control and QoS Rules Request for non-roaming and home routed
1.
The BBERF is triggered to either report an event or obtain QoS rules or both for a gateway control session.

2.
The BBERF sends a Diameter CCR to the H-PCRF with the CC-Request-Type AVP set to the value UPDATE_REQUEST to report event or request QoS rules.

When the UE is roaming (home routed traffic), steps 2a ~ 2c are executed instead of step 2:

2a. The BBERF sends a Diameter CCR to the V-PCRF with the CC-Request-Type AVP set to the value UPDATE_REQUEST to report event or request QoS rules.

2b. The V-PCRF stores the information received.

2c. For case 2a, The V-PCRF sends a Diameter CCR to the H-PCRF within the information received in step 2a at command level.


For case 2b, The V-PCRF sends a Diameter CCR to the H-PCRF within the information received in step 2a at Subsession-Enforcement-Info AVP.
3.
The H-PCRF stores the received information in the Diameter CCR and derives updated QoS rules and event triggers.

4.
The H-PCRF provisions the updated QoS rules and event triggers to the BBERF using Diameter CCA. The CCA may also only acknowledge that the event report has been received successfully.

When the UE is roaming (home routed traffic), steps 4a ~ 4c are executed instead of step 4:

4a.
The H-PCRF sends the updated QoS rules and event triggers to the V-PCRF using Diameter CCA. The CCA may also only acknowledge that the event report has been received successfully.

4b. The V-PCRF may also perform further authorization of the rules based on local policies.

4c. The V-PCRF sends the updated QoS rules and event triggers to the BBERF using Diameter CCA.

5.
The BBERF installs the received QoS Rules and event triggers. This may result in bearer binding being performed according to the rules. The BBERF also enables or disables service flow according to the flow status of the corresponding QoS Rules. The result of the QoS rule activation may trigger the BBERF to send an additional Diameter CCR as described above to the PCRF, for example, to indicate that QoS rule activation has failed.
4.4.2.2
Visited access cases

[image: image24.emf]BBERF PCEF V-PCRF H-PCRF

2. Diameter CCR

3. Store information and 

update QoS rules

1. Trigger to request QoS 

rules or report event

5. Install and update QoS 

rules

4. Diameter CCA

3a. Diameter CCR

3c. Diameter CCA

R

o

a

m

i

n

g

 

c

a

s

e

(

V

i

s

i

t

e

d

 

a

c

c

e

s

s

)

Legend:

Mandatory

Conditional 

3b. Store information 

and update PCC rules

3d. Store information 

and Extract QoS rules


Figure 4.4.2.2.1: Gateway Control and QoS Rules Request for visited access

1.
The BBERF is triggered to either report an event or obtain QoS rules or both for a gateway control session.

2.
The BBERF sends a Diameter CCR to the V-PCRF with the CC-Request-Type AVP set to the value UPDATE_REQUEST to report event or request QoS rules.
3.
The V-PCRF stores the information received in the Diameter CCR and derives updated QoS rules and event triggers according to local policies and roaming agreements.

When the report event is subscribed by H-PCRF, the steps 3a~3d are executed instead of step3:

3a. The V-PCRF sends a Diameter CCR to the H-PCRF with the CC-Request-Type AVP set to the value UPDATE_REQUEST to report event.

For case 2a, the information received in step 2 is send to H-PCRF at command level.

For case 2b, the CCR is send with the information provided at subsession level within the Subsession-Enforcement-Info AVP.
3b. The H-PCRF stores the received information in the Diameter CCR and derives event triggers.

3c.
The H-PCRF sends the event triggers to the V-PCRF using Diameter CCA. The CCA may also only acknowledge that the event report has been received successfully.

3d. The V-PCRF may also perform further authorization of the rules based on local policies.
4.
The V-PCRF provisions the updated QoS rules and event triggers to the BBERF using Diameter CCA.
5.
The BBERF installs the received QoS Rules and event triggers. This may result in bearer binding being performed according to the rules. The BBERF also enables or disables service flow according to the flow status of the corresponding QoS Rules. The result of the QoS rule activation may trigger the BBERF to send an additional Diameter CCR as described above to the PCRF, for example, to indicate that QoS rule activation has failed.
4.4.3
Gateway Control and QoS Rules Provision

Since the PCRF is required to keep QoS rules aligned with the active PCC rules for a certain IP-CAN session, it shall initiate the Gateway Control and QoS Rules Provision whenever there is a change to the corresponding PCC rules for a Gx session that is linked with the Gateway Control Session.

[image: image25.emf]R

o

a

m

i

n

g

 

c

a

s

e

R

o

a

m

i

n

g

 

c

a

s

e

BBERF

2. Diameter RAR

PCEF V-PCRF H-PCRF

2c. Diameter RAR

2a. Diameter RAR

4. Diameter RAA

4b. Diameter RAA

4a. Diameter RAA

1. Trigger to update QoS rules

2b. Identify the gateway 

control session and perform 

authorization

3. Install/Update QoS rules

5. Access specific procedures 

to modify bearers


Figure 4.4.3.1: Gateway Control and QoS Rules Provision

1.
The H-PCRF receives an internal or external trigger to update QoS Rules and event triggers for a gateway control session. If the trigger is from the AF and the AF requests the access network information, the H-PCRF applies the procedure as defined in clause 4a.5.16 of 3GPP TS 29.212 [9] to request the access network information. If the trigger is from the AF and the AF provided the Sharing-Key-UL AVP and/or Sharing-Key-DL AVP within the Media-Component-Description AVP, the H-PCRF may apply the procedures as defined in clause 4a.5.10.6 of 3GPP TS 29.212 [9] to request the resource sharing.
2.
The H-PCRF sends a Diameter RAR to request that the BBERF installs, modifies or removes QoS Rules and/or updates the event triggers.

If the UE is roaming, then steps 2a ~ 2c are executed instead of step 2:

2a.
The H-PCRF sends a Diameter RAR to the V-PCRF to provision updated QoS Rules and updated event triggers.
For case 2a, the RAR provides the updated QoS Rules and updated event triggers with the information included at command level.


For case 2b, The H-PCRF sends a Diameter RAR to the V-PCRF within the information at Subsession-Decision-Info AVP.
2b. The V-PCRF identifies the gateway control session if needed and performs local authorization of the updated QoS rules when necessary.

2c. The V-PCRF sends a Diameter RAR to the BBERF to provision updated QoS rules and updated event triggers.

3.
The BBERF installs, modifies or removes the identified QoS Rules. The BBERF also enforces the authorized QoS and enables or disables service flow according to the flow status of the corresponding QoS Rules.

4.
The BBERF sends RAA to the H-PCRF to acknowledge the RAR and informs the H-PCRF about the outcome of the QoS rule operation. If network initiated resource allocation procedures apply for the QoS rules and the corresponding IP-CAN bearer can not be established or modified to satisfy the bearer binding, then the BBERF rejects the activation of a PCC rule.

If the UE is roaming, then steps 4a ~ 4b are executed instead of step 4:

4a.
The BBERF sends RAA to the V-PCRF to acknowledge the RAR and informs the V-PCRF about the outcome of the QoS rule operation. If network initiated resource allocation procedures apply for the QoS rules and the corresponding IP-CAN bearer can not be established or modified to satisfy the bearer binding, then the BBERF rejects the activation of a PCC rule.

4b. The V-PCRF forwards the RAA to the H-PCRF to acknowledge the RAR and informs the H-PCRF about the outcome of the QoS rule operation.

5.
If needed, the BBERF initiates the access specific procedures to create or modify existing IP-CAN bearers. When the procedure in step 5 is completed and requires of notifications from the BBERF to the PCRF, e.g. in the case of access network information, the steps described as in clause 4.4.2 are additionally executed.
4.4.4
Gateway Control Session Termination

4.4.4.1
BBERF-Initiated Gateway Control Session Termination
This procedure applies for case 2b, as defined in clause 4.0, whenever the BBERF detects a request for a PDN disconnection, mobility to other access or the termination of a pre-registration at the BBERF.

[image: image26.emf]BBERF

1. Remove Gxx 

Session Request

2. Diameter CCR

5. Remove Gxx 

Session Reply

PCEF V-PCRF H-PCRF

2a. Diameter CCR

2b. Diameter CCR

4b. Identify related IP-CAN 

session and perform update if 

needed

3. Identify related IP-CAN 

session and perform update 

as needed.

4. Diameter CCA

4a. Diameter CCA

4c. Diameter CCA

R

o

a

m

i

n

g

 

c

a

s

e

R

o

a

m

i

n

g

 

c

a

s

e


Figure 4.4.4.1.1: BBERF-Initiated Gateway Control Session Termination

1.
The BBERF is requested to terminate its gateway control session. The form of the request to remove the gateway control session depends upon the type of the IP-CAN.

2.
The BBERF sends a Diameter CCR message to the H-PCRF, indicating the gateway control session termination. The BBERF requests the termination of the DCC session using the CC-Request-Type AVP set to the value TERMINATION_REQUEST.

If the UE is roaming, then steps in 2a ~ 2b are executed instead of step 2:

2a.
The BBERF sends a Diameter CCR message to the V-PCRF, indicating the gateway control session termination. The BBERF requests the termination of the DCC session using the CC-Request-Type AVP set to the value TERMINATION_REQUEST.

2b. For the case 2a or if this is the last subsession associated with the S9 session for the case 2b, the V-PCRF sends a Diameter CCR message to the H-PCRF to request the termination of the S9 session. Otherwise, if the gateway control session is locally handled at the V-PCRF, the V-PCRF continues from step 4b; if the gateway control session has a corresponding S9 subsession, then the V-PCRF sends a Diameter CCR message to the H-PCRF to request the termination of the corresponding S9 subsession.
3. The H-PCRF identifies the related IP-CAN session and performs update as necessary.


4. The H-PCRF acknowledges the session termination by sending a Diameter CCA message.

If the UE is roaming, then steps 4a ~ 4c are executed instead of step 4:

4a. If the H-PCRF receives the Diameter CCR message in step 2b, the H-PCRF acknowledges the session or subsession termination request by sending a Diameter CCA message to the V-PCRF.

4b. The V-PCRF identifies the related IP-CAN session and performs update as necessary.

4c. The V-PCRF acknowledges the session termination by sending a Diameter CCA message to the BBERF.
5.
The BBERF sends a reply to the request to remove the gateway control session. The form of the reply depends upon the type of the IP-CAN.
4.4.4.2
PCRF-Initiated Gateway Control Session Termination
This procedure applies for case 2a, as defined in clause 4.0, when the PCRF detects that there is no remaining IP-CAN session at the PCRF.

[image: image27.emf]BBERF

1. Remove Gxx 

Session Request

2. Diameter RAR

PCEF V-PCRF H-PCRF

2a. Diameter RAR

2b. Diameter RAR

3. Diameter RAA

3b. Diameter RAA

3a. Diameter RAA

R

o

a

m

i

n

g

 

c

a

s

e

R

o

a

m

i

n

g

 

c

a

s

e

4. The BBERF terminates the Gateway control session based on the BBERF-

initiated termination procedures.


Figure 4.4.4.2.1: PCRF-Initiated Gateway Control Session Termination

1.
The H-PCRF is requested to terminate the gateway control session.

2.
The H-PCRF sends a Diameter RAR message to the BBERF including a Session-Release-Cause AVP to indicate request for terminating the gateway control session.


If the UE is roaming, then steps in 2a ~ 2b are executed instead of Step 2:

2a. The H-PCRF sends a Diameter RAR message to the V-PCRF to indicate the termination of the S9 session including the Session-Release-Cause AVP.


2b. The V-PCRF sends a Diameter RAR message to the BBERF based on the termination request received from the H-PCRF to indicate the gateway control session termination.

3.
The BBERF acknowledges the gateway control session termination request by sending a Diameter RAA message.


If the UE is roaming, then steps 3a ~ 3b are executed instead of Step 3:

3a. The BBERF acknowledges the gateway control session termination request by sending a Diameter RAA message to the V-PCRF.

3b. The V-PCRF sends a Diameter RAA message to the H-PCRF and acknowledges the request for terminating the S9 session corresponding to the gateway control session.

4.
The BBERF follows the BBERF-initiated gateway control session termination procedures described in clause 4.4.4.1 to terminate the gateway control session.
4.5
Multiple BBERF Signalling Flows

4.5.1
Non-Roaming and Home Routed cases

4.5.1.1
New Gateway Control Session Establishment

The following signalling flow describes an example of a new BBERF initiating a GW control session establishment associated with an existing IP-CAN session.


[image: image28.wmf] 

H

-

P

CRF

 

Target 

BBERF

 

7

. 

 

Diameter CCA

 

3. Store Information

 

6

. 

 

Store PCC

/QoS

 Rules 

 

1. 

 

Establish Gateway

 

Control 

Session 

Request

 

9

. 

 

Establish Gateway 

Control 

Session Response

 

Legend

:

 

 

       

 

Mandatory

 

               

 

 

Conditional 

 

V

-

PCRF

 

2b. Store Information

 and 

identify home routed 

access

 

Source 

BBERF

 

 

7

b. 

 

Store PCC

/QoS

 Rules. 

Policy decision

 

7

e. 

 

Diameter CCA

 

ROA

MING CASE

 

2

c

. Diameter CCR

 

2. 

 

Diameter CCR

 

7

c. 

 

Diameter CCR

 

7

d. 

 

Diameter CCA

 

2a. 

 

Diameter CCR

 

ROAMING CASE

 

7

a. 

 

Diameter CCA

 

PCEF

 

8

. 

 

Install 

QoS

 Rules. 

Policy enforcement

 

4. 

 

Identify 

multiple

 

BBERF(s)

 

 

5. 

 

PCC/QoS Rules 

decision based on 

BCM 

of GW control session

 


Figure 4.5.1.1.1: Gateway Control Session Establishment during BBERF relocation
1.
The target BBERF receives a message or indication to establish a Gateway Control session

2.
The target BBERF initiates a Gateway Control session with the H-PCRF by sending a CCR using the CC-Request-Type AVP set to the value INITIAL_REQUEST to the H-PCRF. The target BBERF provides information as detailed in clause 4a.5.1 of 3GPP TS 29.212 [9].
When the UE is roaming, the following steps are executed instead of step 2:

2a. The target BBERF initiates a Gateway Control session with the V-PCRF by sending a CCR using the CC-Request-Type AVP set to the value INITIAL_REQUEST to the V-PCRF. The target BBERF provides information as detailed in clause 4a.5.1 of 3GPP TS 29.212 [9].

2b.
The V-PCRF determines based on the UE identity information that the request is for a roaming user. The V-PCRF checks whether the V-PCRF is required to send the request to the H-PCRF based on the roaming agreements.

2c.
For case 2a:

-
The V-PCRF sends a CCR to the H-PCRF with the CC-Request-Type AVP set to the value UPDATE_REQUEST. The V-PCRF includes in the CCR the information received in step 2a. The V-PCRF includes the BBERF identity by including the AN-GW-Address AVP at command level.

For case 2b:

- If the Session-Linking-Indicator AVP is not received, or it indicates SESSION_LINKING_IMMEDIATE, the V-PCRF sends the CCR command to the H-PCRF with the CC-Request-Type AVP set to the value UPDATE_REQUEST. The V-PCRF includes the Subsession-Enforcement-Info AVP within the CCR command with allocated S9 subsession identifier assigned by the V-PCRF for this PDN connection within the Subsession-Id AVP, the Subsession-Operation AVP set to the value MODIFICATION, the BBERF identity within AN-GW-Address AVP.
- If the Session-Linking-Indicator AVP is received indicating that the session linking has to be deferred, the V-PCRF sends a CCR to the H-PCRF with the CC-Request-Type AVP set to the value UPDATE_REQUEST. The V-PCRF includes the Subsession-Enforcement-Info AVP within the CCR with a new S9 subsession identifier assigned by the V-PCRF to this Gateway Control Session within the Subsession-Id AVP, the Subsession-Operation AVP set to the value ESTABLISHMENT and the Session-Linking-Indicator AVP set to the value “SESSION_LINKING_DEFERRED”.

3.
The H-PCRF stores the information received in the Diameter CCR.

4.
If the Session-Linking-Indicator AVP was received indicating that the session linking has to be deferred, the linking between the Gateway Control Session and the Gx session shall be deferred. Otherwise, based on the information received the H-PCRF identifies multiple BBERF sessions for a particular IP-CAN session.

5.
The H-PCRF derives applicable PCC/QoS rules based on the BCM mode as defined in clause 4a.5.7 of 3GPP TS 29.212 [9].
6.
The H-PCRF stores the selected QoS Rules and PCC Rules. For non-roaming users the H-PCRF selects the Bearer Control Mode that will apply during the Gateway Control session.

7.
The H-PCRF acknowledges the Gateway Control Session by sending a Diameter CCA. The H-PCRF includes the selected BCM if applicable, the QoS rules and event triggers.

When the UE is roaming, the following steps are executed instead of step 7:

7a. The H-PCRF acknowledges the Gateway Control Session by sending a Diameter CCA to the V-PCRF. The H-PCRF includes applicable QoS rules and also event triggers. The H-PCRF also includes the AN-GW-Address AVP if the QoS rules are applicable for a single BBERF.

7b.
The V-PCRF enforces visited operator policies regarding QoS authorization requested by the H-PCRF as indicated by the roaming agreements.

7c.
If the V-PCRF denies an authorization, it informs the H-PCRF and may provide the acceptable QoS Information for the service.

7d.
The H-PCRF may provide new or modified QoS rules to the V-PCRF.
7e.
The V-PCRF acknowledges the Gateway Control Session and provisions, when applicable, the selected BCM, policy decisions and event triggers to the target BBERF.

8.
The BBERF installs the received QoS Rules.

9.
The target BBERF establishes an indication for a Gateway control session response.
4.5.1.2
PCEF IP-CAN session modification – Handover

The following signalling flow describe the case when an indication of handover is received by the PCEF and the H-PCRF derives QoS rules based on the type of BBERF (primary/non-primary).

[image: image29.wmf] 

H

-

PCRF

 

Target 

BBERF

 

7. 

 

Diameter CCA

 

3. Store Information

 

6. 

 

Store PCC/QoS Rules 

 

1. 

 

Indication of handover

 

Lege

nd

:

 

 

       

 

Mandatory

 

               

 

 

Conditional 

 

V

-

PCRF

 

Source 

BBERF

 

 

8b. 

 

Store PCC/QoS Rules. 

Policy decision

 

8e. 

 

Diameter RAR

 

2. 

 

Diameter CCR

 

8c. 

 

Diameter RAA

 

8d. 

 

Diameter RAR

 

ROAMING CASE

 

8a. 

 

Diameter RAR

 

PCEF

 

9. 

 

Install QoS Rules. 

Policy enforcement

 

4. 

 

Derive 

primary/non

-

primary 

BBERF

(s) 

 

5. 

 

PCC/QoS Rules 

decision based on BCM 

of GW control session

 

8. 

 

Diameter RAR

 

10a

. 

 

Diameter RAA

 

10b

. 

 

Diameter RAA

 

10

. 

 

Diameter RAA

 

ROAMING 

 

CASE

 


Figure 4.5.1.2.1: PCEF IP-CAN session modification – Handover
1.
The PCEF receives a message or indication that a handover occurred

2.
The PCEF initiates an IP-CAN Session Modification procedure by sending a CCR using the CC-Request-Type AVP set to the value UPDATE_REQUEST to the H-PCRF. The PCEF includes the AN_GW_CHANGE event trigger, and if applicable the IP-CAN_CHANGE event trigger as well, to indicate that handover has occurred.

3.
The H-PCRF stores the information received in the Diameter CCR.

4.
If there is a pending gateway control session to be linked to a Gx session, the H-PCRF shall perform the session linking according to clause 4a.5.6 of 3GPP TS 29.212 [9] for the non-roaming case. Based on the information received the H-PCRF reclassifies primary/non-primary BBERFs according to the procedures defined in clause 4a.5.7 of 3GPP TS 29.212 [9].
5.
The H-PCRF derives PCC rules for the PCEF, and QoS rules for the new reclassified primary BBERF, based on the BCM mode of the GW control session as defined in clause 4a.5.7 of 3GPP TS 29.212 [9].
6.
The H-PCRF stores the selected QoS Rules and PCC Rules.

7.
The H-PCRF acknowledges the IP-CAN session modification request by sending a Diameter CCA to the PCEF. The H-PCRF includes updated PCC rules and event triggers (if applicable).
8.
The H-PCRF initiates a Gateway Control and QoS Rules Provision procedure by sending a Diameter RAR. The H-PCRF includes the selected BCM if applicable, the QoS rules and event triggers.

When the UE is roaming, the following steps are executed instead of step 8:

8a. The H-PCRF initiates a Gateway Control and QoS Rules Provision procedure to the V-PCRF by sending a Diameter RAR to the V-PCRF. The H-PCRF sends applicable QoS rules based on the BBERF type (primary/non-primary) and BCM mode selected as defined in clause 4a.5.9 of 3GPP TS 29.212 [9]. The H-PCRF includes the AN-GW-Address AVP if the QoS rules are applicable only for a single BBERF. If the QoS rules are applicable for all BBERF sessions this AVP is omitted.

8b.
The V-PCRF enforces visited operator policies regarding QoS authorization requested by the H-PCRF as indicated by the roaming agreements.

8c.
If the V-PCRF denies an authorization, it informs the H-PCRF and may provide the acceptable QoS Information for the service by including in the RAA command the QoS-Rule-Report AVP to indicate the QoS Rules that were not accepted, the Rule-Failure-Code AVP set to UNSUCCESSFUL-QoS-VALIDATION, and the QoS-Information AVP.

8d.
The H-PCRF may provide new or modified QoS rules to the V-PCRF.
8e.
The V-PCRF initiates the Gateway Control Session and QoS rules provisions, when applicable, the selected BCM, policy decisions and event triggers to the target BBERF.

9.
The BBERF installs the received QoS Rules.

10. The target BBERF acknowledges the RAR command by sending a Diameter RAA command to the PCRF.
When the UE is roaming, the following steps are executed instead of step 10:

10a.The BBERF acknowledges the Gateway Control and QoS Rules Provision request by sending a Diameter RAA to the V-PCRF.
10b. The V-PCRF acknowledges the Gateway Control and QoS Rules Provision request by sending a Diameter RAA to the H-PCRF.
4.5.1.3
PCEF IP-CAN session modification – S2c-based IP flow mobility

The following signalling flow describes an example of S2c-based IP flow mobility. In this case, the H-PCRF receives an IP flow mobility event by the PCEF and derives QoS rules based on the IP flow mobility routing rules.


[image: image30.wmf] 

H

-

PCRF

 

BBERF

 

#

2

 

6

. 

 

Diameter CCA

 

3. Store Information

 

5

. 

 

Store PCC/QoS Rules 

 

1. 

 

Indication of 

IP flow mobility

 

Legend

:

 

 

       

 

Mandatory

 

               

 

 

Conditional 

 

V

-

PCRF

 

BBERF

 

#1

 

 

7

b. 

 

Store PCC/QoS Rules. 

Policy decision

 

7

e. 

 

Diameter RAR

 

2. 

 

Diameter CCR

 

7

c. 

 

Diameter RAA

 

7

d. 

 

Diameter RAR

 

ROAMING CASE

 

7

a. 

 

Diameter

 

RAR

 

PCEF

 

8

. 

 

Perform Step 

9

-

10

b

: as specified in Figure 4.5.

1

.

2

.1: PCEF

-

initiated IP

-

CAN session modification 

–

 

Handover.

 

4

. 

 

PCC/QoS Rules 

decision based on BCM 

of GW control session

 

and r

eceived IP flow 

mobility routing rules

 

7

. 

 

Diameter RAR

 


Figure 4.5.1.3.1: PCEF IP-CAN session modification – S2c-based IP flow mobility

1.
The PCEF receives a message or indication that an IP flow mobility event occurred

2.
The PCEF initiates an IP-CAN Session Modification procedure by sending a CCR using the CC-Request-Type AVP set to the value UPDATE_REQUEST to the H-PCRF. The PCEF includes the ROUTING_RULE_CHANGE event trigger to indicate that a change in the IP flow mobility routing rules has occurred. The PCEF includes in the CCR the Routing-Rule-Install and/or Routing-Rule-Removal AVPs.

3.
The H-PCRF stores the information received in the Diameter CCR.

4.
The H-PCRF derives PCC rules for the PCEF, and QoS rules for the BBERF(s), based on the BCM mode of the GW control session and the IP flow mobility routing rules as defined in clause 4a.5.7 of 3GPP TS 29.212 [9]
5.
The H-PCRF stores the selected PCC/QoS Rules.

6.
The H-PCRF acknowledges the IP-CAN session modification request by sending a Diameter CCA to the PCEF. The H-PCRF includes updated PCC rules and event triggers (if applicable).

7.
The H-PCRF initiates a Gateway Control and QoS Rules Provision procedure by sending a Diameter RAR. The H-PCRF includes the QoS rules and event triggers.

When the UE is roaming, the following steps are executed instead of step 7:

7a. The H-PCRF initiates a Gateway Control and QoS Rules Provision procedure to the V-PCRF by sending a Diameter RAR to the V-PCRF. The H-PCRF sends applicable QoS rules based on BCM mode selected as defined in clause 4a.5.9 of 3GPP TS 29.212 [9]. The H-PCRF includes the AN-GW-Address AVP identifying the BBERF involved in the exchange of the IP flows described by the received IP flow mobility routing rules

7b.
The V-PCRF enforces visited operator policies regarding QoS authorization requested by the H-PCRF as indicated by the roaming agreements.

7c.
If the V-PCRF denies an authorization, it informs the H-PCRF and may provide the acceptable QoS Information for the service by including in the RAA command the QoS-Rule-Report AVP to indicate the QoS Rules that were not accepted, the Rule-Failure-Code AVP set to UNSUCCESSFUL-QoS-VALIDATION, and the QoS-Information AVP.

7d.
The H-PCRF may provide new or modified QoS rules to the V-PCRF.

7e.
The V-PCRF initiates the Gateway Control Session and QoS rules provisions, when applicable, policy decisions and event triggers to BBERF.

8
Step 9 through step 10b: as specified in Figure 4.5.1.2.1: PCEF-initiated IP-CAN session modification‑ Handover are executed, as needed. If the IP flows were moved from one access to another (e.g. 3GPP to WLAN, or vice versa), the PCRF may also initiate a RAR command towards BBERF#1 to modify or release the related resources associated with the flows that were moved to BBERF#2.
4.5.1.4
Gateway Control Session Establishment and PCEF IP-CAN session modification – S2c-based IP flow mobility

The following signalling flow describes an example of S2c-based IP flow mobility. In this case, the H-PCRF receives a Gateway Control session establishment by a BBERF and an IP flow mobility event by the PCEF. H-PCRF associates the IP‑CAN session to multiple Gateway Control Sessions and derives QoS rules based on the IP flow mobility routing rules.


[image: image31.wmf] 

H

-

PCRF

 

BBE

RF

 

#

2

 

4

. 

 

Store Information

. 

 

2

. 

 

Indication of 

IP flow mobility

 

Legend

:

 

 

       

 

Mandatory

 

 

V

-

PCRF

 

BBERF

 

#1

 

 

3

. 

 

Diameter CCR

 

PCEF

 

5

. 

 

PCC/QoS Rules 

decision based on 

BCM of GW control 

session

 

and received 

IP flow mobility routing 

rules

 

1

. 

 

Perform Step 

1

-

10

: as specified in Figure 

Figure 4.4.1.1 Gateway Control Session Establis

hment

.

 

 

6

. 

 

Perform Step 

5

-

8

: as specified in Figure 4.5.

1

.

x

.1: PCEF

-

initiated IP

-

CAN session modification 

–

 

IP 

Flow mobility

.

 

 


Figure 4.5.1.4.1: Gateway Control Session Establishment and PCEF IP-CAN session modification – S2c-based IP flow mobility

1.
Step 1 through step 10: as specified in Figure 4.4.1.1: Gateway Control Session Establishment are executed, as needed. The Gateway Control Session is established for BBERF #2. The Gateway Control Session for BBERF #1 was previously established.

NOTE:
If the Gateway Control Session is established for WLAN access, only case 2a is possible.

2.
The PCEF receives a message or indication that an IP flow mobility event occurred.

3.
The PCEF initiates an IP-CAN Session Modification procedure by sending a CCR using the CC-Request-Type AVP set to the value UPDATE_REQUEST to the H-PCRF. The PCEF includes the ROUTING_RULE_CHANGE event trigger to indicate that a change in the IP flow mobility routing rules has occurred. The PCEF includes in the CCR the Routing-Rule-Install and/or Routing-Rule-Removal AVPs.

4.
The H-PCRF stores the information received in the Diameter CCR.

5.
The H-PCRF does not differentiate between primary and non-primary BBFs. H-PCRF derives PCC rules for the PCEF, and QoS rules for the BBERF(s), based on the BCM mode of the GW control session and the IP flow mobility routing rules as defined in clause 4a.5.7 of 3GPP TS 29.212 [9]
6.
Step 5 through step 8: as specified in Figure 4.5.1.3.1: PCEF-initiated IP‑CAN session modification‑ IP Flow mobility are executed, as needed.
4.5.2
Visited access case
4.5.2.1
New Gateway Control Session Establishment

The following signalling flow describes an example of a new BBERF initiating a GW control session establishment associated with an existing IP-CAN session.

[image: image32.wmf] 

H

-

PCRF

 

Target 

BBERF

 

1. 

 

Establish Gateway Cont

rol 

Session Request

 

V

-

PCRF

 

3

. 

 

Store Information and 

identify 

visited 

access

 

Source 

BBERF

 

 

2. 

 

Diameter CCR

 

PCEF

 

4. 

 

Identify multiple BBERF(s)

.

 

D

erive 

and 

Store

 

QoS Rules

. 

Policy decision

 

ROAMING CASE

 

(

Visited

 access

)

 

5

. Diameter 

CCR

 

7

. Diameter 

CC

 

A

 

6

. Store Information

, 

PCC Rules 

decision 

and 

store

 

 

11. 

 

Establish Gateway Control 

Session Response

 

9

. 

 

Diameter CCA

 

1

0

. 

 

Install QoS Rules. 

Policy enforcement

 

8

. 

 

Store PCC/QoS Rules. 

Policy decision

 

8a.

 Diameter 

CCR

 

8b

. Diameter 

CC

 

A

 


Figure 4.5.2.1.1: Gateway Control Session Establishment during BBERF relocation
1.
The target BBERF receives a message or indication to establish a Gateway Control Session.

2.
The target BBERF initiates a Gateway Control Session with the V-PCRF by sending a CCR using the CC-Request-Type AVP set to the value INITIAL_REQUEST to the V-PCRF. The target BBERF provides information as detailed in clause 4a.5.1 of 3GPP TS 29.212 [9].
3.
The V-PCRF stores the information received in the Diameter CCR and determines based on the UE identity information that the request is for a roaming user. The V-PCRF checks whether the V-PCRF is required to send the request to the H-PCRF based on the roaming agreements. The V-PCRF does not send the CCR to the H-PCRF to update the S9 session immediately if the Session-Linking-Indicator AVP was received indicating that the session linking has to be deferred.
4.
If the Session-Linking-Indicator AVP was received indicating that the session linking has to be deferred, the linking between the Gateway Control Session and the Gx session shall be deferred. Otherwise, based on the information received the V-PCRF identifies multiple BBERF sessions for a particular IP-CAN session. The V-PCRF derives applicable QoS rules according to local policies and stores them.

For case 2a or if either the AN_GW_CHANGE or the IP-CAN_CHANGE event is subscribed by H-PCRF and this event trigger is received steps 5~8 are executed. Otherwise steps 5~8 are skipped.
5.
The V-PCRF initiates an IP-CAN Session Modification procedure by sending a CCR to the H-PCRF with the CC-Request-Type AVP set to the value UPDATE_REQUEST. The V-PCRF includes in the CCR the information received in step 2.

6.
The H-PCRF stores the information received in the Diameter CCR. And the H-PCRF decides PCC rules for the BBERF and stores PCC rules.

7.
The H-PCRF sends a Diameter CCA to the V-PCRF to provide the PCC rules. The H-PCRF sends applicable PCC rules. The H-PCRF includes the AN-GW-Address AVP if the PCC rules are applicable only for a single BBERF. If the PCC rules are applicable for all BBERF sessions this AVP is omitted.
8.
If the steps 5~7 are executed, the V-PCRF enforces visited operator policies regarding QoS authorization requested by the H-PCRF as indicated by the roaming agreements.
Steps 8a and 8b are executed if the V-PCRF denies authorisation for one or more PCC rules.

8a. If V-PCRF denies authorization, it informs the H-PCRF by sending a CCR command including the Charging-Rule-Report AVP to indicate the PCC Rules that were not accepted, the Rule-Failure-Code AVP set to UNSUCCESSFUL-QoS-VALIDATION, and the acceptable QoS Information for the service.

8b. The H-PCRF may provide new modified PCC rules to the V-PCRF.
9.
The V-PCRF acknowledges the Gateway Control Session and provisions policy decisions and event triggers to the target BBERF.
10. The BBERF installs the received QoS rules.

11. The target BBERF responds to the Gateway control session establishment request in step 1.
4.5.2.2
PCEF-Initiated IP-CAN session modification-Handover

The following signalling flow describe the case when an indication of handover is received by the PCEF and the H-PCRF derives QoS rules based on the type of BBERF (primary/non-primary)


[image: image33.wmf] 

H

-

PCRF

 

Target 

BBERF

 

7

. Store Information

 

1. 

 

Indication of handover

 

Legend

:

 

 

       

 

Man

datory

 

               

 

 

Conditional 

 

V

-

PCRF

 

Source 

BBERF

 

 

10

. 

 

Store PCC/QoS Rules. 

Policy decision

 

12

. 

 

Diameter 

RAR

 

2. Diameter CCR

 

ROAMING CASE

 

（

Visited access

）

 

9

. 

 

Diameter 

CC

 

A

 

PCEF

 

13

. 

 

Install QoS Rules 

Policy enforcement

 

5

. 

 

Derive 

primary/non

-

primary 

BBERF(s)

 

8

. 

 

PCC Rules decision 

and 

store

 

14

. Diameter 

RA

A

 

3. Store Information

 

6

. Diameter 

CCR

 

11

 Diameter CCA

 

4. 

 

Identify multiple 

BBERF(s) 

 

10

a.

 Diameter 

CCR

 

10

b. 

Diameter 

CC

 

A

 


Figure 4.5.2.2.1: PCEF-initiated IP-CAN session modification – Handover
1.
The PCEF receives a message or indication that a handover occurred.

2.
The PCEF initiates an IP-CAN Session Modification procedure by sending a CCR using the CC-Request-Type AVP set to the value UPDATE_REQUEST to the V-PCRF. The PCEF includes the AN_GW_CHANGE event trigger, and if applicable the IP-CAN_CHANGE event trigger as well, to indicate that handover has occurred.

3.
The V-PCRF stores the information received in the Diameter CCR.
4.
If there is a pending Gateway Control Session to be linked to a Gx session, the V-PCRF links Gateway Control Session with the Gx session according to clause 4a.5.6 of 3GPP TS 29.212 [9]. Otherwise based on the information received the V-PCRF identifies multiple BBERF sessions for a particular IP-CAN session.
5.
Based on the information received the V-PCRF reclassifies primary/non-primary BBERFs according to the procedures defined in clause 4a.5.7 of 3GPP TS 29.212 [9].
If either the AN_GW_CHANGE or the IP-CAN_CHANGE event is subscribed by H-PCRF and this event trigger is received steps 6~9 are executed. Otherwise steps 6~9 are skipped.
6.
The V-PCRF initiates an IP-CAN Session Modification procedure by sending a CCR to the H-PCRF with the CC-Request-Type AVP set to the value UPDATE_REQUEST. The V-PCRF includes in the CCR the information received in step 2.

7.
The H-PCRF stores the information received in the Diameter CCR.

8.
The H-PCRF decides PCC rules for the PCEF and stores PCC rules.

9.
The H-PCRF sends a Diameter CCA to the V-PCRF to provide the PCC Rules. The H-PCRF sends applicable PCC rules. The H-PCRF includes the AN-GW-Address AVP if the PCC rules are applicable only for a single BBERF. If the PCC rules are applicable for all BBERF sessions this AVP is omitted.
10.
If the steps 6~9 are executed, the V-PCRF enforces visited operator policies regarding QoS authorization requested by the H-PCRF as indicated by the roaming agreements.
Steps 10a and 10b are executed if the V-PCRF denies authorisation for one or more PCC rules.

10a. If V-PCRF denies authorization, it informs the H-PCRF by sending a CCR command including the Charging-Rule-Report AVP to indicate the PCC Rules that were not accepted, the Rule-Failure-Code AVP set to UNSUCCESSFUL-QoS-VALIDATION, and the acceptable QoS Information for the service.

10b. The H-PCRF may provide new modified PCC rules to the V-PCRF.
11. The V-PCRF acknowledges the IP-CAN session modification request by sending a Diameter CCA to the PCEF. The V-PCRF includes updated PCC rules and event triggers (if applicable)
12. The V-PCRF initiates the Gateway Control Session and QoS rules provisions by sending a Diameter RAR to the BBERF policy decisions and event triggers to the target BBERF.
13. The BBERF installs the received QoS Rules.

14. The BBERF acknowledges the Gateway Control and QoS Rules Provision request by sending a Diameter RAA to the V-PCRF.
4.5.2.3
PCEF-Initiated IP-CAN session modification - S2c-based IP flow mobility

The following signalling flow describes an example of S2c-based IP flow mobility. In this case, the H-PCRF receives an IP flow mobility event by the PCEF and derives QoS rules based on the IP flow mobility routing rules.

[image: image34.wmf] 

 

H

-

PCRF

 

BBERF

 

#2

 

6

. Store Information

 

1. 

 

Indication of 

IP flow mobil

ity

 

Legend

:

 

 

       

 

Mandatory

 

               

 

 

Conditional 

 

V

-

PCRF

 

BBERF

 

#1

 

 

10

. 

 

Perform 

Step

 

1

1

-

1

4: as specified in Figure 4.5.2.

2

.1: PCEF

-

initiated IP

-

CAN session 

modification 

–

 

Handover.

 

2. Diameter CCR

 

ROAMING CASE

 

（

Visited access

）

 

PCEF

 

7

. 

 

PCC Rules decision 

and 

store

 

3. Store Information

 

4. 

 

Change in IP flow 

 

mobility 

routing

 

rules

 

9

. 

 

E

x

tract

 

QoS Rules 

based on

 

PCC rules and 

IP flow mobility rules

 

5

. Diameter 

CCR

 

8

. 

 

Diameter 

CC

A

 

9a

. 

 

Diameter 

CC

R

 

9b

. 

 

Diameter 

CC

A

 


Figure 4.5.2.3.1: PCEF-initiated IP-CAN session modification – S2c-based IP flow mobility
1.
The PCEF receives a message or indication that a handover occurred.

2.
The PCEF initiates an IP-CAN Session Modification procedure by sending a CCR using the CC-Request-Type AVP set to the value UPDATE_REQUEST to the V-PCRF. The PCEF includes the ROUTING_RULE_CHANGE event trigger to indicate that a change in the IP flow mobility routing rules has occurred. The PCEF includes in the CCR Routing-Rule-Install and/or Routing-Rule-Removal AVPs.

3.
The V-PCRF stores the information received in the Diameter CCR.
4.
Based on the information received the V-PCRF determines that there is a change in the IP flow routing in the multiple BBERF(s) as described in clause 4a.5.7 of 3GPP TS 29.212 [9].

If either the AN_GW_CHANGE or the IP-CAN_CHANGE event is subscribed by H-PCRF and this event trigger is received steps 5~8 are executed. Otherwise steps 5~8 are skipped.
5.
The V-PCRF initiates an IP-CAN Session Modification procedure by sending a CCR to the H-PCRF with the CC-Request-Type AVP set to the value UPDATE_REQUEST. The V-PCRF includes in the CCR the information received in step 2.

6.
The H-PCRF stores the information received in the Diameter CCR.

7.
The H-PCRF decides PCC rules for the PCEF and stores PCC rules. Based on the IP flow mobility routing rule as defined in clause 4a.5.7 of 3GPP TS 29.212 [9]
8.
The H-PCRF sends a Diameter CCA to the V-PCRF to provide the PCC Rules. The H-PCRF sends applicable PCC rules.

9.
If the steps 5~8 are executed, the V-PCRF establishes QoS Rules based on received IP flow mobility routing rules and enforces visited operator policies regarding QoS authorization requested by the H-PCRF as indicated by the roaming agreements.
Steps 9a and 9b are executed if the V-PCRF denies authorisation for one or more PCC rules.

9a. If V-PCRF denies authorization, it informs the H-PCRF by sending a CCR command including the Charging-Rule-Report AVP to indicate the PCC Rules that were not accepted, the Rule-Failure-Code AVP set to UNSUCCESSFUL-QoS-VALIDATION, and the acceptable QoS Information for the service.

9b. The H-PCRF may provide new modified PCC rules to the V-PCRF.
10.
Step 11 through step 14: as specified in Figure 4.5.2.2.1: PCEF-initiated IP-CAN session modification‑ Handover are executed, as needed. If the IP flows were moved from one access to another (e.g. 3GPP to WLAN, or vice versa), the PCRF may also initiate a RAR command towards BBERF#1 to modify or release the related resources associated with the flows that were moved to BBERF#2.
4.5.2.4
Gateway Control Session Establishment and PCEF IP-CAN session modification – S2c-based IP flow mobility

The following signalling flow describes an example of IP flow mobility. In this case, the V-PCRF receives a Gateway Control session establishment by a BBERF and an IP flow mobility event by the PCEF. V-PCRF associates the IP‑CAN session to multiple Gateway Control Sessions and derives QoS rules based on the IP flow mobility routing rules.

[image: image35.wmf] 

 

H

-

PCRF

 

BBERF

 

#2

 

2

. 

 

Indication of 

IP flow mobility

 

V

-

PCRF

 

BBERF

 

#1

 

 

10

. 

 

Perform 

Step

 

9

-

10

: as specified in Figure 4.5.2.

x

.1: PCEF

-

initiated IP

-

CAN session modification 

–

 

IP flow mobility

.

 

3

. Diameter CCR

 

ROAMING CASE

 

（

Visited access

）

 

PCEF

 

4

. 

 

Store Information

 

 

5

. 

 

Evaluates 

 

IP flow 

 

mobility 

routing

 

rules

 

1. 

 

Perform Step 1

-

10

: as specified in Figure 

Figure 4.4.1.1 Gateway Control Session Establishment

.

 

 

7

. Store Information

 

8

. 

 

PCC Rules decision 

and 

store

 

6

. Diameter 

CCR

 

9

. 

 

Diameter 

CC

A

 

Legend

:

 

 

       

 

Mandato

ry

 

               

 

 

Conditional 

 


Figure 4.5.2.4.1: Gateway Control Session Establishment and PCEF IP-CAN session modification – S2c-based IP flow mobility
1.
Step 1 through step 10: as specified in Figure 4.4.1.1: Gateway Control Session Establishment are executed, as needed. The Gateway Control Session is established for BBERF #2. The Gateway Control Session for BBERF #1 was previously established.

NOTE:
If the Gateway Control Session is established for WLAN access, only case 2a is possible.

2.
The PCEF receives a message or indication that an IP flow mobility event occurred.

3.
The PCEF initiates an IP-CAN Session Modification procedure by sending a CCR using the CC-Request-Type AVP set to the value UPDATE_REQUEST to the V-PCRF. The PCEF includes the ROUTING_RULE_CHANGE event trigger to indicate that a change in the IP flow mobility routing rules has occurred. The PCEF includes in the CCR Routing-Rule-Install and/or Routing-Rule-Removal AVPs.

4.
The V-PCRF stores the information received in the Diameter CCR.
5.
Based on the received information, the V-PCRF does not differentiate between primary and non-primary BBFs and determines that there are IP flows routed through multiple BBERF(s) as described in clause 4a.5.7 of 3GPP TS 29.212 [9].

If either the AN_GW_CHANGE or the IP-CAN_CHANGE event is subscribed by H-PCRF and this event trigger is received steps 6~9 are executed. Otherwise steps 6~9 are skipped.
6.
The V-PCRF initiates an IP-CAN Session Modification procedure by sending a CCR to the H-PCRF with the CC-Request-Type AVP set to the value UPDATE_REQUEST. The V-PCRF includes in the CCR the information received in step 2.

7.
The H-PCRF stores the information received in the Diameter CCR.

8.
The H-PCRF decides PCC rules for the PCEF and stores PCC rules. Based on the IP flow mobility routing rule as defined in clause 4a.5.7 of 3GPP TS 29.212 [9].
9.
The H-PCRF sends a Diameter CCA to the V-PCRF to provide the PCC Rules. The H-PCRF sends applicable PCC rules.

10. Step 9 through step 10: as specified in Figure 4.5.2.3.1: PCEF-initiated IP‑CAN session modification‑ IPFlow mobility are executed, as needed.
4.6
Application Detection and Enforcement Procedures

4.6.1
TDF Session Establishment in case of solicited reporting
In the following procedure, the PCRF is the H-PCRF for the roaming UE with home routed access and the V-PCRF for the roaming UE with visited access.
As part of the IP-CAN Session Establishment or Modification procedure, in case of solicited application reporting with a TDF, the PCRF initiates a TDF Session Establishment with the selected TDF. The TDF is selected based on data received from the PCEF or a local configuration at the PCRF.


[image: image36.emf]PCRF

TDF

1. Diameter TSR

1a.  Credit session 

establishment with the OCS.

2. Diameter TSA


Figure 4.6.1.1: TDF Session Establishment in case of solicited reporting
1
PCRF initiates a session towards the TDF. The PCRF provisions the applicable ADC Rules for the corresponding TDF session by sending a Diameter TS-Request to the TDF, including user identity information, the UE Ipv4 address and/or UE Ipv6 prefix and, if available, PDN identifier, IP-CAN type, RAT type and additional parameters as defined in clause 4b.5.1.1 of 3GPP TS 29.212 [9]. PCRF may also subscribe to the Event Triggers (e.g. APPLICATION_START and APPLICATION_STOP).
NOTE:
For PDN type Ipv4v6, in case the UE Ipv4 address is not available in the PCRF, the PCRF initiates the TDF session establishment providing the UE Ipv6 prefix, and will subsequently provide UE Ipv4 address to the TDF using Event-Report-Indication AVP to the TDF.
1a.
This step applies to the IP-CAN Session Establishment procedure. If online charging is applicable for the TDF, and at least one ADC rule with charging parameters was activated, then the TDF requests credit information from the OCS over the Gyn interface. If the TDF receives credit re-authorisation triggers from the OCS then it requests the PCRF via a TSA message to provision the triggers at the PCEF and/or BBERF. The triggers to be provisioned are specified in the Event-Report-Indication AVP in the TSA message.
2.
The TDF acknowledges the session establishment by sending a Diameter TS-Answer. The TDF may include Event-Report-Indication in the response.
4.6.1A
TDF Session Establishment in case of unsolicited reporting

In the following procedure, the PCRF is the H-PCRF for the roaming UE with home routed access and the V-PCRF for the roaming UE with visited access.
When the TDF detects for an Ipv4 address or Ipv6 address the first application start, the TDF shall initiate the TDF Session Establishment procedure with the PCRF.


[image: image37.emf] 

PCRF   TDF  

1 . Diameter  CCR  

2 . Diameter  CCA  


Figure 4.6.1A.1: TDF Session Establishment in case of unsolicited reporting
1. The TDF initiates a session by sending a CCR to the PCRF using the CC-Request-Type AVP set to the value INITIAL_REQUEST. The TDF provides the full UE IP address using either Framed-IP-Address AVP or Framed-Ipv6-Prefix AVP and, if available, the PDN identifier. The TDF also includes the TDF-Application-Identifier AVP, the Flow-Information AVP of the detected application when service data flow descriptions are deducible, within the Application-Detection-Information AVP and sets the event trigger value with APPLICATION_START. If Flow-Information AVP is included, the TDF-Application-Instance-Identifier shall also be included within the Application-Detection-Information AVP in order to allow correlation of APPLICATION_START.


2
The PCRF stores the information and acknowledges the session establishment by sending a CCA. The PCRF may include the Ipv6 prefix within the Framed-Ipv6-Prefix AVP if the established TDF session is Ipv6 address related.
NOTE 1:
The TDF handles each Ipv4 address and Ipv6 prefix within a separate TDF session.
NOTE 2:
In the scenario where the TDF performs initial Application Detection on 
vailabi simultaneous traffic flows for the same Ipv6 prefix (i.e. two or more from Ipv6 addresses of the same IP-CAN session) the TDF could not be aware that those flows belong to the same IP-CAN session until a response is received from the PCRF, containing the Ipv6 prefix. This leads to using separate TDF sessions for the Ipv6 addresses for the same IP-CAN session. The TDF reports new application detection information related to that Ipv6 prefix via any of the TDF sessions at a later stage.
4.6.2
TDF Session termination
In the following procedures, the PCRF is the H-PCRF for the roaming UE with home routed access and the V-PCRF for the roaming UE with visited access.
This procedure applies in any of the following cases:

·  the corresponding IP-CAN session is terminated;
·  the Ipv4 address of a dual stack IP-CAN session is released and there is an active Ipv4 address related TDF session for the IP-CAN session (only for unsolicited application reporting);

· at any point of time when the PCRF decides that the session with TDF is to be terminated (e.g. subscriber profile changes).

[image: image38.emf]PCRF

TDF

1. Diameter RAR

2.  Removal/deactivation of 

all ADC Rules.

3. Diameter RAA

4. Diameter CCR

5. Diameter CCA


Figure 4.6.2.1: TDF Session Termination
1.
The PCRF sends a RAR including the Session-Release-Cause AVP to request that the TDF terminates the TDF session.
2.
For the solicited application reporting, the TDF removes/deactivates all the ADC Rules which are applied to the TDF session.

3.
The TDF sends a RAA to acknowledge the RAR.
4.
The TDF sends a CCR to the PCRF, indicating the TDF Session termination. The TDF requests the termination of the Sd session using the CC-Request-Type AVP set to the value TERMINATION_REQUEST. For solicited application reporting, if the usage monitoring is enabled, the TDF informs the PCRF about the resources that have been consumed by the user since the last report in the same request.
5.
The PCRF acknowledges the TDF session termination by sending a CCA to the TDF.
4.6.3
TDF Session modification

4.6.3.1
Application Detection, Reporting and Control Rules Request

In the following procedure, the PCRF is the H-PCRF for the roaming UE with home routed access and the V-PCRF for the roaming UE with visited access.

[image: image39.emf]PCRF

TDF

2. Diameter CCR

5.  Installation, modification 

and removal of the ADC 

Rules.

4. Diameter CCA

1.  Trigger to report the 

events.

3.  makes the ADC 

decisions

3. Makes the 

PCC/QoS/ADC decisions


Figure 4.6.3.1.1 Application Detection, Reporting and Control Rules Request

1.
TDF is triggered to report an event(s) (e.g. The TDF detects the start/stop of an application traffic that matches with one or more activated ADC rules that do not contain the Mute-Notification AVP) for a TDF session. For the start of traffic detection, in case the enforcement actions were provided as a part of ADC rules, the TDF enforces corresponding actions for solicited application reporting.

2.
The TDF sends a Diameter CCR to the PCRF with the CC-Request-Type AVP set to the value UPDATE_REQUEST to report an event. For the start of traffic detection, if PCRF has previously subscribed to the APPLICATION_START/APPLICATION_STOP Event-Triggers, the TDF includes TDF-Application-Identifier AVP, the Flow-Information AVP of the detected application when service data flow descriptions are deducible, within the Application-Detection-Information AVP and sets the event trigger value with APPLICATION_START. If Flow-Information AVP is included, the TDF-Application-Instance-Identifier shall also be included within the Application-Detection-Information AVP in order to allow correlation of APPLICATION_START. For the stop of traffic detection, if PCRF has previously subscribed to the APPLICATION_START/APPLICATION_STOP Event-Triggers, the TDF includes TDF-Application-Identifier AVP, the TDF-Application-Instance-Identifier AVP, if provided in the report of the start of application traffic detection within the Application-Detection-Information AVP and sets the event trigger value with APPLICATION_STOP. For the solicited application reporting, if usage monitoring is enabled and the usage threshold is reached or the PCRF removes the last ADC rule applicable for certain monitoring key or disables usage monitoring or requests usage report, the TDF may inform the PCRF about the corresponding usage that have been consumed by the user since the last report.

3.
The PCRF stores the information, received in the Diameter CCR and makes the PCC/QoS and – in the solicited reporting case – ADC decisions.
4.
The PCRF acknowledges to the TDF by sending a Diameter CCA. For the solicited application reporting, the PCRF may provide a new ADC decisions by including the ADC-Rule-Install AVP and/or ADC-Rule-Remove AVP to the TDF within this acknowledge.
5.
For the solicited application reporting, the TDF installs, modifies and removes the ADC rules according the new ADC decisions provided in step 4.
NOTE:
If the installation or modification of one or more ADC rules fails, the TDF reports the failure to the PCRF as defined in clause 4b.5.5 of 3GPP TS 29.212 [9].
4.6.3.2
Application Detection and Control Rules Provision

In the following procedure, the PCRF is the H-PCRF for the roaming UE with home routed access and the V-PCRF for the roaming UE with visited access. This procedure is applicable only for the solicited application reporting.

[image: image40.emf]PCRF

TDF

2. Diameter RAR

3.  Installation, modification 

and removal of the ADC 

Rules.

4. Diameter RAA

1.  Trigger to update the 

ADC decisions.


Figure 4.6.3.2.1 Application Detection and Control Rules Provision

1.
The PCRF receives an internal or external trigger (e.g. the subscriber’s profile configuration is changed) to update the ADC rule or notify the event occurred at the PCEF/BBERF for a TDF session.

2.
The PCRF sends a Diameter RAR to provide a new ADC decision by including the ADC-Rule-Install AVP and/or ADC-Rule-Remove AVP or notify the event occurred at the PCEF/BBERF by including the Event-Report-Indication AVP.
3.
The TDF stores the information, received in the Diameter RAR. The TDF installs, modifies and removes the ADC rules according the new ADC decisions provided in step 2.

4.
The TDF acknowledges to the PCRF by sending a Diameter RAA to inform the PCRF about the outcome of the actions related to the decision(s).
4.7
Spending limits Procedures over Sy reference point

4.7.1
Initial Spending Limit Report Request
In the following procedure, the signalling flow for the H-PCRF to request the status of the policy counters available at the OCS, and to subscribe to updates of these policy counters by the OCS. If the H-PCRF provides the list of policy counter identifier(s), the OCS returns the policy counter status per policy counter identifier provided by the PCRF. If the H-PCRF does not provide the list of policy counter identifier(s), the OCS returns the policy counter status for all policy counter identifier(s), which are available for this subscriber.

[image: image41.emf]1. Policy decisiondepends 

on spendinginfo

H-PCRF

OCS

2. Diameter SLR

3. Diameter SLA 


Figure 4.7.1: Initial Spending Limit Report Request

1.
The H-PCRF retrieves subscription information that indicates that policy decisions depend on policy counter(s) held at the OCS and optionally the list of policy counter identifier(s).

2.
The H-PCRF sends a Diameter SLR command if no Sy session yet has been established for this subscriber. The Diameter SLR command includes the Subscription-Id AVP (e.g. IMSI) and optionally the list of policy counter identifier(s) within Policy-Counter-Identifier AVPs. The request also includes the SL-Request-Type AVP which set to the value INITIAL_REQUEST (0).

3.
The OCS sends a Diameter SLA command to the PCRF. The Diameter SLA includes a Policy-Counter-Status-Report AVP for each requested policy counter identifier containing the policy counter identifier and the current status value, optionally pending policy counter statuses with the activation times, and Result-Code AVP contains the result of the operation. When no policy counter identifier(s) was provided, the Diameter SLA includes a Policy-Counter-Status-Report AVP for all policy counter identifiers applicable to the subscriber. The OCS stores the H-PCRF’s subscription to changes in the status of all policy counter identifiers provided to the H-PCRF in the Diameter SLA.

4.7.2
Intermediate Spending Limit Report Request

This clause describes the signalling flow for the H-PCRF to request the status of additional policy counters available at the OCS or to remove the request for the status of policy counters available at OCS. If the H-PCRF provides the list of policy counter identifier(s), the OCS returns the policy counter status per policy counter identifier provided by the PCRF.

[image: image42.emf]H-PCRF

OCS

1 . Additional or fewer policy 

counters are determined to be 

relevant for policy decision

4. Policy Decision

2. Diameter SLR

3. Diameter SLA 


Figure 4.7.2: Intermediate Spending Limit Report Request

1.
The H-PCRF decides to modify the list of subscribed policy counters, e.g. PCRF determines that policy decisions depend on additional policy counter identifier(s) held at the OCS or that notifications of policy counter status changes for some policy counters are no longer required.

2.
The H-PCRF sends a Diameter SLR command, optionally including the list of policy counter identifier(s) within Policy-Counter-Identifier(s) AVPs. The request also includes the SL-Request-Type AVP which set to the value INTERMEDIATE_REQUEST (1).
3.
The OCS sends the Diameter SLA command to the PCRF including Policy-Counter-Status-Report AVP(s) containing the policy counter identifier, the current status value and optionally pending policy counter statuses with the activation times. Result-Code contains the result of the operation is also included in the response.

4.
The PCRF makes the policy decision based on the information provided by the OCS.

4.7.3
Final Spending Limit Report Request

This clause describes the signalling flow for the H-PCRF to unsubscribe to any future updates of policy counters for a given subscriber by the OCS. It cancels the request for reporting the change of the status of the policy counters available at the OCS.

[image: image43.emf]H-PCRF

OCS

2. Diameter STR

3. Diameter STA 

1. Decision to unsubscribe to 

changes in the Policy-

Counter

-

Status


Figure 4.7.3: Final Spending Limit Report Request

1.
The PCRF decides that policy decisions for a given user no longer depend on policy counter(s) to which the PCRF has existing subscriptions for status change notification.

2.
The H-PCRF sends the Diameter STR command to the OCS to cancel the notification request from the OCS on policy counter status. The request includes the Termination-Cause which contains the reason why the session was terminated set to “DIAMETER_LOGOUT”.

3.
The OCS sends the Diameter STA command to the H-PCRF with Result-Code contains the result of the operation.

4.7.4
Spending Limit Report

This clause describes the signalling flow for the OCS to notify the changes of the status of a subscribed policy counter(s) available at the OCS for that subscriber. Alternatively, the signalling flow can be re-used by the OCS to provide one or more pending statuses for a subscribed policy counter together with the time that have to be applied.
[image: image44.emf]H-PCRF

OCS

2. Diameter SNR 

3. Diameter SNA

1.  Policy Counter Status 

change (e.g. Threshold 

reached)


Figure 4.7.4: Spending Limit Report

1.
The OCS detects that status of a policy counter identifier(s) has changed and the PCRF requested notification of changes in the status of a policy counter(s). Alternatively, if the OCS detects a policy counter status will change at a future point in time, the OCS shall be able to instruct the PCRF to apply one or more pending statuses for a requested policy counter.

2.
When the status of a specific policy counter changes, or the OCS detects that a policy counter status will change at a future point in time and decides to instruct the PCRF to apply one or more pending statuses for a requested policy counter, the OCS shall determine the Sy sessions impacted by the change (i.e. those Sy sessions that have subscribed to status change notifications for the changed policy counter) and send the Diameter SNR command to the PCRF associated with each affected Sy session including one Policy-Counter-Status-Report AVP. If several counters change status at the same time, the OCS may group the status change notifications into a single Spending Limit Report request to the PCRF by sending multiple Policy-Counter-Status-Report AVPs in the request.
NOTE:
Sy session is per UE. And more than one Sy session will exist when the UE has IP-CAN session connection with more than one PCRF.

3.
The H-PCRF acknowledges the request by sending a Diameter SNA command with a Result-Code AVP set to DIAMETER_SUCCESS and use the status of the received policy counter(s) as input to its policy decision to apply operator defined actions, e.g. downgrade the QoS. The PCRF shall ignore an unknown policy counter status report for all unknown policy counter identifiers in the SNR from the OCS.

4.8
Call flows for User Plane Congestion Management

4.8.1
General
The call flows for User Plane Congestion Management (UPCON) over Np reference point are specified as follows:

· RAN User Plane Congestion Information (RUCI) reporting, in which non-aggregated RUCI reporting and aggregated RUCI reporting are included.
· Reporting restriction provisioning
· UE mobility between RAN Congestion Awareness Functions (RCAFs)
· Removal of UE context
4.8.2
RUCI reporting

4.8.2.1
Non-aggregated RUCI report procedure


[image: image45.emf]PCRF

RCAF

1. Diameter NRR

2. Diameter NRA


Figure 4.8.2.1.1: Non-aggregated RUCI reporting
1.
The RCAF sends a diameter NR-Request command to the PCRF, including the user id, PDN ID, the congestion level info and additional parameters as defined in clause 4.4.1.2 of 3GPP TS 29.217 [36].

2.
The PCRF stores the related information and acknowledges the RUCI reporting by sending a Diameter NR-Answer including the PCRF id.
4.8.2.2
Aggregated RUCI report procedure


[image: image46.emf]PCRF

RCAF

1. Diameter ARR

2. Diameter ARA


Figure 4.8.2.2.1: Aggregated RUCI reporting
1. The RCAF sends a Diameter AR-Request command to the PCRF, including one or more Aggregated-RUCI-Report AVP and additional parameters as defined in clause 4.4.1.3 of 3GPP TS 29.217 [36]. The PCRF is the common destination PCRF for a set of user id and PDN ID and identified by the PCRF id returned in the previous non-aggregating RUCI reporting.
2.
The PCRF stores the related information and acknowledges the RUCI reporting by sending a Diameter AR-Answer back to the RCAF.
4.8.3
Np reporting restriction provisioning

The Np reporting restriction provisioning is used by the PCRF to stop RUCI reporting for a specific user id and PDN ID. It is also used by the PCRF to specify, modify or disable restrictions for RUCI reporting.

[image: image47.emf]RCAF PCRF

1. Diameter MUR

2. Diameter MUA


Figure 4.8.3.1: Np reporting restriction provisioning

1.
The PCRF sends a Diameter MUR command to the RCAF for the provisioning of RUCI reporting restriction including the parameters as defined in clause 4.4.2 of 3GPP TS 29.217 [36].
2.
The RCAF responds a Diameter MUA command to the PCRF including the parameters as defined in clause 4.4.2 of 3GPP TS 29.217 [36].

4.8.4
UE mobility between RCAFs

This signalling flow describes the mobility handling of a UE from one RCAF to another RCAF. The UE is affected by congestion for both RCAFs.


[image: image48.emf]RCAF2 PCRF

1 .  Diameter NRR

2.  Diameter NRA

RCAF1

     

UE moves to RCAF2

3.  Diameter NRR

4.  Diameter NRA

5.  Removal of UE context in RCAF1

 


Figure 4.8.4.1: UE mobility from RCAF1 to RCAF2 (the UE is affected by congestion at RCAF2 after the UE was earlier affected by congestion at RCAF1)

1.
The UE is located in the RCAF1 which is affected by congestion. The RCAF1 sends a Diameter NRR command to the PCRF for reporting RUCI including the parameters as defined in clause 4.4.1.2 of 3GPP TS 29.217 [36].
2.
The PCRF stores the identity of RCAF1 for the given UE and responds a Diameter NRA command to the RCAF1 including the parameters as defined in clause 4.4.1.2 of 3GPP TS 29.217 [36].
3.
The UE moves to the RCAF2 which is also affected by congestion. The RCAF2 sends a Diameter NRR command to the PCRF for reporting RUCI of the same UE including the parameters as defined in clause 4.4.1.2 of 3GPP TS 29.217 [36].
4.
The PCRF stores the identity of RCAF2 for the given UE and responds a Diameter NRA command to the RCAF2 including the parameters as defined in clause 4.4.1.2 of 3GPP TS 29.217 [36].
5.
The PCRF initiates a removal of the UE context to RCAF1 to release the context of the given user id and PDN ID, including the parameters as defined in clause 4.4.4 of 3GPP TS 29.217 [36].
4.8.5
Removal of UE context
This signalling flow describes the removal of a UE context from the RCAF.


[image: image49.emf]RCAF PCRF

1. Diameter MUR

2. Diameter MUA


Figure 4.8.5.1: UE context removal in RCAF
1.
The PCRF sends a Diameter MUR command to the RCAF to release the context of the given user id and PDN ID, including the parameters as defined in clause 4.4.4 of 3GPP TS 29.217 [36].
2.
The RCAF responds with a Diameter MUA command to the PCRF including the parameters as defined in clause 4.4.4 of 3GPP TS 29.217 [36]. The RCAF releases the context corresponding to the given user id and PDN ID, including any reporting restriction.
4.9
Traffic Steering Control Procedures over St reference point

4.9.1
St Session Establishment
In the following procedure, the PCRF initiates the St session establishment procedure and provides the traffic steering control informaiton to the TSSF due to the PCRF determines that the traffic steering control is needed for the IP-CAN session.


[image: image50.emf]TSSF

PCRF

1. Diameter TSR

2. Diameter TSA


Figure 4.9.1.1: St Session Establishment

1 The PCRF initiates session between the PCRF and TSSF and provisions the applicable ADC Rules for the corresponding IP-CAN session by sending a Diameter TSR to the TSSF, including the Request-Type AVP set to the value "0" and other parameters as defined in subclause 4c.4.1 of 3GPP TS 29.212 [9].
2 The TSSF installs and/or activates the ADC rules and acknowledges to the PCRF by sending a Diameter TSA command. 
4.9.2
St Session Modification
In the following procedure, the PCRF updates the traffic steering control informaiton at the TSSF due to the PCRF determines that the traffic steering control information needs to be update for the IP-CAN session (e.g. Subscription is changed) or notifies the TSSF that the IPv4 address is allocated/released.


[image: image51.emf]TSSF

PCRF

1. Diameter TSR

2. Diameter TSA


Figure 4.9.2.1: St Session Modification

1 The PCRF provisions the new, updated and/or requests to remove ADC rules for the corresponding IP-CAN session or notifies the TSSF that the IPv4 address is allocated/released by sending a Diameter TSR command to the TSSF, including the Request-Type AVP set to the value "1" and other parameters as defined in subclause 4c.4.1 or 4c.4.4 of 3GPP TS 29.212 [9].
2 The TSSF installs, modifies and/or removes the ADC rules or activates and/or deactivates the ADC rules and acknowledges to the PCRF by sending a Diameter TSA commands.
4.9.3
St Session Termination
In the following procedure, the the PCRF initiates the St session termination procedure and removes all the traffic steering control informaiton to the TSSF due to the corresponding IP-CAN session is terminated or the H-PCRF determines that the traffic steering control information is not needed for the IP-CAN session any longer (e.g. Subscription has changed).

[image: image52.emf]TSSF

PCRF

1. Diameter STR

3. Diameter STA

2. Removes/deactivates 

all the ADC rules


Figure 4.9.3.1: St Session Termination

1.
The PCRF sends an STR to the TSSF to terminate the session between the PCRF and the TSSF.
2.
The TSSF removes/deactivates all the ADC Rules which are applied to the session.

3.
The TSSF sends an STA to acknowledge the STR.
4.9.4
St notification initiated by the TSSF

In the following procedure, the TSSF reports the ADC rule error when the ADC rule can’t be enforced any longer.


[image: image53.emf]PCRF

TSSF

1. Diameter TNR

2. Diameter TNA


Figure 4.9.x.1: St notification initiated by the TSSF

1 The TSSF reports the ADC rules error by sending a Diameter TNR command to the PCRF as defined in subclause 4c.4.3 of 3GPP TS 29.212 [9].
2 The PCRF acknowledges to the TSSF by sending a Diameter TNA command.
4.10
Negotiation for future background data transfer procedure over Nt reference point
The following procedure shows the signaling flow for the SCEF to request the transfer policy for future background data transfer determined by the H-PCRF. 


[image: image54.emf]SCEF

H-PCRF

1. Diameter BTR

3. Diameter BTA

4. Diameter BTR

5. Diameter BTA

2.H-PCRF determines 

the transfer policies


Figure 4.x.1: Negotiation for future background data transfer
1.
Based on the SCS/AS request, the SCEF sends a Diameter BTR command to the H-PCRF for the request of  transfer policy including the parameters as defined in subclause 4.4.1 of 3GPP TS 29.154 [56]. 

2.
The H-PCRF determines one or more transfer policies based on the information received from the SCEF and other available information (e.g. network policy, existing transfer policies).

3.
The H-PCRF sents a Diameter BTA command to the SCEF including the parameters as defined in subclause 4.4.1 of 3GPP TS 29.154 [56].

4.
If there is more than one transfer policy provided from the H-PCRF to the SCEF in the BTA command, when the SCEF receives the selected transfer policy from the SCS/AS, the SCEF sends a Diameter BTR command to the H-PCRF with the selected transfer policy.

5.
The H-PCRF acknowledges the BTR command by sending BTA command.
5
Binding Mechanism
5.1
Overview

The binding mechanism associates the session information with the IP-CAN bearer that is intended to carry the service data flow.
For PCC rules with application identifier, and for certain IP-CAN types, up-link traffic can be received on other/additional IP-CAN bearers than the one determined by the binding mechanism (further details provided in clause 6.2.2.2 and the IP-CAN specific annexes of 3GPP TS 23.203 [2]).
The binding mechanism includes three steps as defined in 3GPP TS 23.203 [2]:

1.
Session binding.
2.
PCC Rule authorization and QoS Rule generation.
3.
Bearer binding.
The Session Binding function receives the Session Information and determines the relevant IP-CAN session. With this information the PCC Rule Authorization and QoS Rule generation function runs the policy rules and constructs the PCC rule(s) and if applicable, the QoS rule(s) if the authorization is granted. Finally the Bearer Binding function selects the IP-CAN bearer where the PCC rule(s) or QoS rule(s) should be installed within the IP-CAN session already known.
PCC Rule Authorization and QoS Rule generation function and Bearer Binding function can take place without Session Binding at certain IP-CAN Session events (e.g. IP-CAN Session Establishment).

5.2
Session Binding

Session binding is the association of the AF session information to an IP-CAN session.
When the PCRF accepts an AA-Request from the AF over the Rx interface with service information, the PCRF shall perform session binding and associate the described service IP flows within the AF session information (and therefore the applicable PCC rules) to one and only one existing IP-CAN session. When the PCRF receives an AA-Request from the P-CSCF acting as an AF over the Rx interface for P-CSCF Restoration, the PCRF shall perform session binding and associate the request to the existing IMS PDN connection and perform P-CSCF Restoration for that impacted IMS PDN connection. This association is done comparing the user IP address received via the Rx interface in either the Frame-IP-Address AVP or the Framed-Ipv6-Prefix AVP with the Ipv4 address or Ipv6 prefix received via the Gx interface. The UE Identity if present in the Subscription-Id AVP and the PDN information if available in the Called-Station-Id AVP may also assist on this association. The Domain identity if available in the IP-Domain-Id AVP may also assist on this association, e.g. if other user identity information than the UE IP address is unavailable and the UE IP addresses is not unique for a certain APN.

NOTE 1:
The UE IP address is unique per Domain identity.

NOTE 2:
The IP-Domain-Id AVP is helpful in the following scenario: Within a PLMN, there are several separate IP address domains, with PCEF(s) that allocate Ipv4 IP addresses out of the same private address range to Ues. The same IP address can thus be allocated to Ues served by PCEFs in different address domains. If one PCRF controls several PCEFs in different IP address domains, the UE IP address is thus not sufficient for the session binding. An AF can serve Ues in different IP address domains, either by having direct IP interfaces to those domains, or by having interconnections via NATs in the user plane between PCEFs and the AF. If a NAT is used, the AF obtains the IP address allocated to the UE via application level signalling and supplies it for the session binding as Framed-IP-Address to the PCRF. The AF supplies an IP-Domain-Id value denoting the IP address domain behind the NAT in addition. The AF can derive the appropriate value from the source address (allocated by the NAT) of incoming user plane packets.

NOTE 3：When the scenario described in NOTE 2 applies and the AF is a P-CSCF it is assumed that the P-CSCF has direct IP interfaces to the different IP address domains and that no NAT is located between P-GW and P-CSCF. How a non-IMS AF obtains the UE private IP address to be provided to the PCRF is out of scope of the present release; it is unspecified how to support applications that use a protocol that does not retain the original UE’s private IP address.
If the Domain identity is not used to assist association, the PCRF will determine that the UE has an IP-CAN session if the IP address (Ipv4 or Ipv6) received over the Rx interface matches the Ipv4 address or Ipv6 prefix received via one or more of the following interfaces: Gx interface and S9 interface, and if the UE identity is used to assist the association, the UE identity received over the Rx interface matches the UE identity received via one or more of the following interfaces: Gx interface and S9 interface.
If the Domain identity is used to assist association, the PCRF will determine that the UE has an IP-CAN session if the Domain identity received over the Rx interface matches the PCEF Identity received via the Gx interface, and the IP address (Ipv4) received over the Rx interface matches the Ipv4 address received via the Gx interface.
For the roaming scenario, the Domain identity may be used for session binding when the AF and PCEF are located in same PLMN, i.e. for the home routed case, the Domain identity may be used by the (H-)PCRF for session binding, and for the visited case, Domain identity could be used by the V-PCRF for session binding.
For an IP-CAN session associated to a dedicated APN for the purpose of offering services to remote UEs via a ProSe UE-to-network relay UE, the PCRF shall use the UE IPv6 prefix alone in the session binding.
NOTE 4:
In case the UE identity in the IP-CAN and the application level identity for the user are of different kinds, the PCRF needs to maintain, or have access to, the mapping between the identities. Such mapping is not subject to specification within this TS.
NOTE 5:
An Ipv6 address provided over Rx matches an Ipv6 prefix provided over Gx or S9 if the Ipv6 address belongs to the Ipv6 (sub-)network prefix.
NOTE 6:
The PCRF derives the PCEF identity from the Origin-Host AVP of the CCR command received from the PCEF. In order to correlate the PCEF Identity and the domain identity received over the Rx interface, the PCRF uses configured mapping between those identities. The Domain Identity is useful for assistance in session binding if the Origin-Host of the Gx CCR command is not modified by intermediate Diameter proxies deployed between the PCEF and the PCRF.
NOTE 7:
The PCEF identity is not transported over S9 reference point in the present release. So for the visited access with AF located in HPLMN case, session binding assisted with Domain identity is not supported in the present release. Session binding is still possible based on other available information in addition to the IP address when provided by the AF, according to the current procedures.
If the PCRF uses the Called-Station-Id AVP to assist the association, the PCRF shall apply the procedures in Annex I to match APN information.

As a result from the session binding function, the PCRF identifies what IP-CAN session the current AF session is related with. If the PCRF is not capable of executing the Session Binding, the PCRF shall issue an AA-Answer command to the AF with a negative response.
5.3
PCC Rule Authorization and QoS Rule Generation
The PCRF shall perform the PCC rule authorization and QoS rule generation when the PCRF receives session information from an AF over Rx interface, when the PCRF receives notification of IP-CAN session events (e.g. establishment, modification) from the PCEF over Gx or S9 interface, when the PCRF receives IP-CAN events from the BBERF over Gxa/Gxc interface, or the PCRF receives a notification from the SPR that calls for a policy decision. The PCRF shall also perform PCC Rule Authorization and QoS Rule generation for dynamic PCC Rules already provisioned to the PCEF and dynamic QoS rules already provisioned to the BBERF due to internal PCRF triggers (e.g. policies are included or modified within PCRF).
If the PCRF receives any traffic mapping information from the BBF that does not match any service data flow filter, the PCRF shall also perform PCC and/or QoS rule authorization when the UE’s subscriber profile allows subscription based authorization. In this case, the PCRF shall treat the received traffic mapping information as if it is service data flow filter information.

If the PCRF receives from the AF a reference ID together with the AF session information, the PCRF may provide it to the SPR in order to retrieve the corresponding transfer policy. The PCRF shall derive the PCC rules for the background data transfer according to the retrieved transfer policy.

NOTE 1:
A transfer policy is only valid within its time window. The removal of outdated transfer policies from the SPR is up to implementation.

If the PCRF receives information from the SPR for the invocation/revocation of a Priority EPS Bearer service (i.e. the MPS EPS Priority is set/removed or the MPS Priority Level changes while the MPS EPS Priority is set) , then the PCRF should change the QCI/ARP of the dynamic PCC/QoS rules that have the same QCI/ARP as for the present default EPS bearer with the new QCI/ARP assigned to the default EPS bearer. If there are active non-MPS services, the QCI/ARP of the PCC/QoS rules related to the non-MPS services shall also be changed.

If the PCRF receives information from the SPR that invokes/revokes the IMS Signalling Priority or changes the MPS Priority Level while IMS Signaling Priority is enabled, then the PCRF should

· change the ARP of the dynamic PCC/QoS rules applicable to the IM CN signalling

· replace the predefined PCC rules applicable to the IM CN signalling by predefined rules that apply when the IMS Signalling Priority is set according to operator policies

When IMS Signalling Priority is set, the PCRF should keep the ARP assigned to the default EPS bearer higher than the ARP related to the IM CN signalling bearer.

NOTE 2.
IMS Signalling Priority only applies to an APN enabled for IMS.

The PCRF assigns appropriate QoS parameters (QCI, ARP, GBR, MBR, etc.) to each PCC or QoS rule. The PCRF takes the information received over Rx into account for determining the appropriate QCI/ARP. If the Rx authorization indicates IMS Multimedia Priority Service, then the PCRF shall allow the prioritization of the MPS session according to clauses 4.5.19.1.3 and 4a.5.14.1.3 in 3GPP TS 29.212 [9] for Gx/Gxx respectively. If the Rx authorization indicates Group Communication Service prioritization as described in 3GPP TS 23.468 [34], then the PCRF shall allow the prioritization of the Group Communication session according to clause 4.5.19 in 3GPP TS 29.212 [9].
The PCRF authorizes the affected PCC rules and /or QoS rules after successful Session Binding. By the authorization process the PCRF will determine whether the user can have access to the requested services and under what constraints. If so, the PCC rules and QoS rules are created or modified. If the Session Information is not authorized, a negative answer shall be issued to the AF by sending an AA-Answer command.
The PCRF assigns an appropriate QCI to each PCC or QoS rule. IP-CAN specific restrictions and other information available to the PCRF (e.g. users subscription information, operator policies) shall be taken into account. Each PCC or QoS rule shall receive a QCI that can be supported by the IP-CAN. The PCRF shall ensure consistency between the QoS rules and PCC rules authorized for the same service data flow when QoS rules are derived from corresponding PCC rules.
If PrioritySharing feature is supported over Rx reference point as described in 3GPP TS 29.214[10], the PCRF shall apply the priority sharing procedures as described in 3GPP TS 29.212 [9], subclauses 4.5.27 and 4a.5.17, if applicable.

In roaming scenarios, the V-PCRF may further authorize the rules received from the H-PCRF based on local operator policy. Depending on the local policy, the V-PCRF may change the authorized QoS for the affected rules. If local authorization of the rules fails, the V-PCRF shall issue a negative answer to the H-PCRF.
5.4
Bearer Binding

The Bearer Binding function is responsible for associating a PCC rule and QoS rule (if applicable) to an IP-CAN bearer within the IP-CAN session. The QoS demand in the rule, as well as the service data flow template, is input to the bearer binding. The selected bearer shall have the same QCI and ARP as the one indicated by the PCC or QoS rule.
NOTE 1:
The PCRF provides the appropriate ARP/QCI for both PCC Rules and default bearer QoS so that the PCEF can perform a valid bearer binding.
The Bearer Binding Function (BBF) is located either at the BBERF or at the PCEF.
The PCRF shall supply the PCC rules to be installed, modified or removed over Gx interface to PCEF. If there are gateway controls sessions associated with the Gx session, the PCRF shall also supply the QoS rules to be installed, modified, or removed over Gxa/Gxc interface to the BBERF.

The BBF shall then check the QoS class identifier and ARP indicated by the rule and bind the rule with an IP-CAN bearer that has the same QoS class identifier and ARP. The BBF shall evaluate whether it is possible to use one of the existing IP-CAN bearers or not and, if applicable, whether to initiate IP-CAN bearer modification or not. If none of the existing bearers are possible to use, the BBF should initiate the establishment of a suitable IP-CAN bearer. The BBF should not bind rules with the PS to CS session continuity indicator to the same bearer as the rules without the PS to CS session continuity indicator.
NOTE 2:
For an IP-CAN, limited to a single IP-CAN bearer per IP-CAN session, the bearer is implicit, so finding the IP-CAN session is sufficient for successful bearer binding.

NOTE 3:
The handling of a rule with MBR>GBR is up to operator policy (e.g. an independent IP-CAN bearer may be maintained for that SDF to prevent unfairness between competing SDFs).
NOTE 4:
The QCI and ARP (including the Priority-Level, Pre-emption-Capability and Pre-emption-Vulnerability) are used for the bearer binding. Depending on operator policy, only the QCI and ARP Priority-Level can be used for bearer binding. In such a case, it is left to operator policy to determine whether different PCC rules with the same QCI and ARP Priority-Level but different Pre-emption-Capability and Pre-emption-Vulnerability can be bound to the same bearer.
For an IP-CAN, where the BBF gains no information on what IP-CAN bearer the UE selects to send an uplink IP flow on, the binding mechanism shall assume that, for bi-directional service data flows, both downlink and uplink packets travel on the same IP-CAN bearer.

Whenever the service data flow template, the QoS authorization of a PCC/QoS rule or the negotiated traffic mapping information change, the existing bearer bindings shall be re-evaluated. The re-evaluation may, for a service data flow, require a new binding with another IP-CAN bearer. The BBF should, if the PCRF requests the same change to the ARP/QCI for all PCC/QoS Rules bound to the same bearer, modify the bearer ARP/QCI as requested.
NOTE 5:
A QoS change of the default EPS bearer causes the bearer binding for PCC/QoS rules previously bound to the default EPS bearer to be re-evaluated.

During PCC/QoS rules enforcement, if packet filters are provided to the UE, the BBF shall provide packet filters with the same content as that in the SDF template filters received over the Gx/Gxx interface from the PCRF within the Flow-Description or the Flow-Information AVP. The representation/format of the packet filters provided by the network to the UE is access-system dependent and may vary between accesses and also may be different from that of the SDF template filter on the Gx/Gxx interface. The PCRF may control the provisioning of packet filters to the UE, i.e. which filters are required to be sent to the UE, as described in 3GPP TS 29.212 [9].
If PCC rule(s) with application identifier(s) are the only PCC rule(s) that are bound to a bearer which requires traffic mapping information, the PCEF shall derive traffic mapping information based on implementation specific logic (e.g. traffic mapping information that effectively disallows any useful packet flows in uplink direction as described in clause 15.3.3.4 of 3GPP TS 23.060 [3]) and provides it to the UE.
NOTE 6:
For GPRS and EPS, the state of TFT packet filters, as defined in 3GPP TS 23.060 [3], for an IP-CAN session requires that there is at most one bearer with no TFT packet filter for the uplink direction.
Requirements specific for each type of IP-CAN are defined in the IP-CAN specific Annex. The Bearer Binding Function may also be located in the PCRF (e.g. as specified in Annex D for GPRS running UE only IP-CAN bearer establishment mode). Selection of the Bearer Binding location shall be based on the Bearer Control Mode selected by the PCRF.

6
QoS Parameters Mapping

6.1
Overview

Several QoS parameters mapping functions are needed during PCC interaction. These functions are located at the AF, PCRF, PCEF and UE. The main purpose of these mapping functions is the conversion of QoS parameters from one format to another. Examples of QoS information are:

-
Parts of a session description language (SDI), e.g. SDP, MPD.

-
IP QoS parameters.

-
Access specific QoS parameters.

One QoS mapping function is located at the AF, which maps the application specific information into the appropriate AVPs that are carried over the Rx interface. The AF derives information about the service from the SDI or from other sources. The mapping is application specific. If SDP (IETF RFC 2327 [11]) is used as SDI, the AF should apply the mapping described in clause 6.2. If MPD (3GPP TS 26.247 [30]) is used, the AF may apply the mapping described in Annex X in 3GPP TS 26.247 [30]. For IMS, the mapping rules in clause 6.2 shall be used at the P-CSCF. The AF passes service information to the PCRF over the Rx interface. Subclause 6.2 specifies the QoS parameter mapping functions at the AF applicable for all IMS P-CSCFs regardless of the access technology.
One QoS mapping function is located at the PCRF, which maps the service information received over the Rx interface into IP QoS parameters (e.g. QCI, GBR, MBR, ARP, …). This mapping is access independent. Subclause 6.3 specifies the QoS mapping functions at the PCRF applicable for all accesses.

The other mapping functions located at PCEF, BBERF, and UE are implementation dependent and are not specified within this specification except for GPRS case.

The PCRF notes and authorizes the IP flows described within this service information by mapping from service information to Authorized IP QoS parameters for transfer to the PCEF/BBERF via the Gx/Gxx interface. Both the PCEF and BBERF will map from the Authorized IP QoS parameters to the access specific QoS parameters. For GPRS, the GGSN acting as PCEF will map from the Authorized IP QoS parameters to the Authorized UMTS QoS parameters.
The general QoS mapping framework is shown in figure 6.1.1.

[image: image55.wmf] 

 

 

 

 

GW

 

PCEF

/BBERF

 

 

PCRF

 

(2)

 

IP BS 

Manager

 

Translation / 

Mapping 

fu

nction

 

(4

)

 

 Access

-

 

Specific BS 

Manager

 

(5)

 

UE

 

(3)

 

 

 Translation / 

Mapping 

function

 

Access

-

Specific  BS 

Manager

 

IP BS Manager

 

 Application

 

Gx

/Gxx

 

AF session signalling , 

possibly with SDI

 

Service 

information

 

Rx

 

 AF

 

(1)

 

SDI 

Mapping 

function

 

Signalling path

 

LEGEND

 

Policy 

Engine

 

Authz IP QoS 

parameters

 

Access

-

specific 

QoS parameter

s

 


Figure 6.1.1: Framework for QoS mapping

NOTE 1:
The AF can derive the Service information from the AF session signalling.

NOTE 2:
Service Information on Rx interface to Authorized IP QoS parameters mapping.

NOTE 3:
For the UE initiated bearer setup, the UE may derive IP QoS parameters, requested Access-Specific QoS parameters mapping and Authorized Access-Specific QoS parameters from the AF session signalling.

NOTE 4:
Authorized IP QoS parameters to Authorized Access-Specific QoS parameters mapping.

NOTE 5:
Access Specific QoS parameters with Authorized Access-Specific QoS parameters comparison.

6.1.1
UE-Initiated IP-CAN bearers

This clause covers the case where the UE is capable to initiate/modify the IP-CAN bearers sending requests to the PCEF/BBERF. When a UE desires to establish/modify an IP-CAN bearer the following steps are followed:

1.
The AF can map from SDI within the AF session signalling to service information passed to the PCRF over the Rx interface. (see clause 6.2 if SDP is used as SDI).

2.
The PCRF shall map from the service information received over the Rx interface to the Authorized IP QoS parameters that shall be passed to the PCEF/BBERF via the Gx/Gxx interface. The mapping is performed for each IP flow. Upon a request from the PCEF/BBERF, the PCRF combines per direction the individual Authorized IP QoS parameters per flow (see clause 6.3).

3.
The UE derives access specific QoS parameters, e.g. UMTS QoS parameters, and, if an IP BS manager is present, IP QoS parameters from the AF session signalling in an application specific manner. The IP and access specific QoS parameters should be generated according to application demands.

For GPRS, the recommendations for conversational (3GPP TS 26.114 [29]) or streaming applications (3GPP TS 26.234 [6]) should also be taken into account when the UE derives the IP and UMTS QoS parameters. If SDP is used as SDI, e.g. for IMS, the UE should apply clause 6.5.1.and should also apply mapping rules for the authorised QoS parameters in clause 6.5.2 to derive the maximum values for the different requested bit rates and traffic classes. In case the UE multiplexes several IP flows onto the same PDP Context, it has to combine their IP and UMTS QoS parameters. If an IP BS manager is present, the Translation/Mapping function maps the IP QoS parameters to the corresponding UMTS QoS parameters.

4.
The PCEF/BBERF shall map from the Authorized IP QoS parameters received from PCRF to the Authorized access specific QoS parameters.
For GPRS. The GGSN shall map to the Authorized UMTS QoS parameters (see clause 6.4.1.1).

5.
The PCEF/BBERF shall compare the requested access specific QoS parameters against the authorized access specific QoS parameters.
For GPRS, the GGSN shall compare the UMTS QoS parameters of the PDP context against the Authorized UMTS QoS parameters (see clause 6.4.1.2).

The mapping that takes place in the UE and the network should be compatible in order to ensure that the PCEF will be able to correctly authorize the session.
Figure 6.1.1.1 shows the different kind of QoS parameters in the different points of QoS mapping figure. Due to the UE requests, there are bidirectional flows between the UE and the PCRF.


[image: image56.wmf] 

 

 

 

 

GW

 

PCEF

/BBERF

 

 

PCRF

 

 

IP BS 

Manager

 

4) Translat

ion 

/ Mapping 

function

 

5) Access

-

 

Specific BS 

Manager

 

3)

 UE

 

 

 Translation / 

Mapping 

function

 

Access

-

Specific  BS 

Manager

 

IP BS 

Manager

 

 Application

 

Gx

/Gxx

 

AF session signalling , 

possibly with SDI

 

Service 

information

 

Rx

 

 AF

 

1) 

SDI 

Mapping 

function

 

Signalling path

 

LEGEND

 

2) 

Policy 

Engine

 

Authorized  IP 

QoS parameters

 

Access

-

specific 

QoS parameters

 

IP QoS 

parameters

 

Authorized  IP 

QoS paramet

ers

 

IP QoS 

parameters

 

Authorized  

 

Access

-

Specific

 

QoS parameters

 

Access

-

Specific

 

QoS parameters

 

Authorized

 

Access

-

specific 

QoS parameters

 

Access

-

Specific

 

QoS parameters

 

IP QoS 

parameters

 

Authorized  IP 

Q

oS parameters

 

Authorized  

 

Access

-

Specific

 

QoS parameters

 


Figure 6.1.1.1: QoS mapping for UE initiated IP CAN bearers

6.1.2
Network-Initiated IP-CAN bearers

When the IP-CAN session supports Network-Initiated bearers, the network sets up IP CAN bearer(s) with a suitable QoS. If the type of IP CAN supports such an indication, the network indicates to the terminal the QoS characteristics of those IP-CAN bearer(s). Therefore the flow of QoS related information will be unidirectional as indicated in the figure 6.1.2.1.



Figure 6.1.2.1: QoS mapping for network initiated IP CAN bearers

1.
The AF can map from SDI within the AF session signalling to service information passed to the PCRF over the Rx interface (see subclause 6.2 if SDP is used as SDI).
2.
The PCRF shall map from the service information received over the Rx interface to the Authorized IP QoS parameters that shall be passed to the PCEF/BBERF via the Gx/Gxx interface. The mapping is performed for each IP flow. Upon a request from the PCEF/BBERF, the PCRF combines per direction the individual Authorized IP QoS parameters per flow (see subclause 6.3).

3.
The PCEF/BBERF shall map from the Authorized IP QoS parameters received from PCRF to the access specific QoS parameters. For GPRS, the GGSN shall map to the UM TS QoS parameters (see clause 6.4.1.1).

6.2
QoS parameter mapping Functions at AF
The mapping described in this clause is mandatory for the P-CSCF and should also be applied by other AFs, if the SDI is SDP.

When a session is initiated or modified the P-CSCF shall use the mapping rules in table 6.2.1 for each SDP media component to derive a Media-Component-Description AVP from the SDP Parameters. The mapping shall not apply to media components where the SDP payload is proposing to use a circuit-switched bearer (i.e. " c=" line set to "PSTN" and an "m=" line set to "PSTN ", refer to 3GPP TS 24.292 [24]). Circuit-switched bearer related media shall not be included in the service information sent to the PCRF.

Table 6.2.1: Rules for derivation of service information within
Media-Component-Description AVP from SDP media component

	service information per Media-Component-Description AVP

(see notes 1 and 7)
	Derivation from SDP Parameters
(see Note 2)

	Media-Component-Number
	ordinal number of the position of the "m=" line  in the SDP

	AF-Application-Identifier
	The AF-Application-Identifier AVP may be supplied or omitted, depending on the application.

For IMS, if the AF-Application-Identifier AVP is supplied, its value should not demand application specific bandwidth or QoS class handling unless the IMS application is capable of handling a QoS downgrading.

	Media-Type
	The Media Type AVP shall be included with the same value as supplied for the media type in the "m=" line.

	Flow-Status
	IF port in m-line = 0 THEN

    Flow-Status:= REMOVED;

ELSE

  IF Transport in m-line is "TCP" or “"TCP/MSRP" THEN (NOTE 9)

     Flow-Status := ENABLED;

  ELSE /* UDP or RTP/AVP transport
   IF a=rtcp-mux is negotiated THEN

     Flow-Status :=ENABLED; (NOTE 12 and 13)

   ELSE
      IF a=recvonly THEN

        IF <SDP direction> = UE originated (NOTE 8) THEN

          Flow-Status := ENABLED_DOWNLINK; (NOTE 4)

        ELSE /* UE terminated (NOTE 8) */

          Flow-Status := ENABLED_UPLINK; (NOTE 4)

        ENDIF;

      ELSE

        IF a=sendonly THEN

          IF <SDP direction> = UE originated (NOTE 8) THEN

            Flow-Status := ENABLED_UPLINK; (NOTE 4)

          ELSE /* UE terminated (NOTE 8) */

            Flow-Status := ENABLED_DOWNLINK; (NOTE 4)

          ENDIF;

        ELSE

          IF a=inactive THEN

               Flow-Status :=DISABLED;

          ELSE /* a=sendrecv or no direction attribute */

               Flow-Status := ENABLED (NOTE 4)

          ENDIF;

        ENDIF; 
      ENDIF;
    ENDIF;

  ENDIF;

ENDIF;
(NOTE 5)

	Max-Requested-Bandwidth-UL
	IF <SDP direction> = UE terminated (NOTE 8) THEN

  IF Transport in m-line is "TCP" or "TCP/MSRP" THEN (NOTE 9)

    IF a=recvonly or a=sendrecv or no direction attribute THEN

      IF b=AS:<bandwidth> is present and

            ( b=TIAS:<Tibandwidth> is not

              present or is present but not supported ) THEN

        Max-Requested-Bandwidth-UL:= <bandwidth> * 1000; /* Unit bit/s
      ELSE

        IF b=TIAS:<Tibandwidth> is present and supported THEN

          Max-Requested-Bandwidth-UL:= <Transport-dependent bandwidth>
            (NOTE 11) /* Unit bit/s
        ELSE

          Max-Requested-Bandwidth-UL:= <Operator specific setting>;

        ENDIF;
      ENDIF; 
    ELSE

      Max-Requested-Bandwidth-UL:= <Operator specific setting>,
        (NOTE 10)

    ENDIF;

  ELSE /* UDP or RTP/AVP transport

    IF b=AS:<bandwidth> is present and
        ( b=TIAS:<Tibandwidth> is not

          present or is present but not supported ) THEN
      IF a=rtcp-mux is negotiated(NOTE 13) THEN

        IF b=RR:<rrbandwidth> is present 
            OR b=RS:<rsbandwidth> is present THEN
          Max-Requested-Bandwidth-UL:= <bandwidth> * 1000 +

            <rrbandwidth> + <rsbandwidth>; (NOTE 3; NOTE 6)
        ELSE

          Max-Requested-Bandwidth-UL:= <bandwidth> * 1050; 
           /* Unit is bit/s
        ENDIF

      ELSE
   Max-Requested-Bandwidth-UL:= <bandwidth> * 1000; 
          /* Unit is bit/s
      ENDIF;
    ELSE

      IF b=TIAS:<Tibandwidth> is present and supported THEN

        IF a=rtcp-mux is negotiated (NOTE 13) THEN

          IF b=RR:<rrbandwidth> is present 
              OR b=RS:<rsbandwidth> is present THEN
            Max-Requested-Bandwidth-UL:= 
              <Transport-dependent bandwidth> (NOTE 11) +

              <rrbandwidth> + <rsbandwidth>; (NOTE 3; NOTE 6)
          ELSE

            Max-Requested-Bandwidth-UL:= 
              <Transport-dependent bandwidth>
              * 1.05 (NOTE 11) /* Unit bit/s
          ENDIF

        ELSE

          Max-Requested-Bandwidth-UL:= <Transport-dependent bandwidth>
           (NOTE 11) /* Unit bit/s
        ENDIF;

      ELSE

        Max-Requested-Bandwidth-UL:= <Operator specific setting>,

        or AVP not supplied;

      ENDIF;

    ENDIF;

  ENDIF

ELSE

  Consider SDP in opposite direction
ENDIF

	Max-Requested-Bandwidth-DL
	IF <SDP direction> = UE originated (NOTE 8) THEN

  IF Transport in m-line is "TCP" or "TCP/MSRP" THEN (NOTE 9)

    IF a=recvonly or a=sendrecv or no direction attribute THEN

      IF b=AS:<bandwidth> is present and 
         ( b=TIAS:<Tibandwidth> is notpresent or
           is present but not supported ) THEN

        Max-Requested-Bandwidth-DL:= <bandwidth> * 1000; /* Unit bit/s
      ELSE
        IF b=TIAS:<Tibandwidth> is present and supported THEN

          Max-Requested-Bandwidth-DL:= <Transport-dependant bandwidth>
             /* Unit bit/s (see NOTE 11) 
          OR Operator specific setting
        ELSE

          Max-Requested-Bandwidth-DL:= <Operator specific setting>;

        ENDIF;

      ELSE

        Max-Requested-Bandwidth-DL:= <Operator specific setting>,
          (NOTE 10)

      ENDIF;

  ELSE /* UDP or RTP/AVP transport

    IF b=AS:<bandwidth> is present and b=TIAS:<Tibandwidth> is not

            present THEN
      IF a=rtcp-mux is negotiated(NOTE 13) THEN

        IF b=RR:<rrbandwidth> is present 
            OR b=RS:<rsbandwidth> is present THEN
          Max-Requested-Bandwidth-DL:= <bandwidth> * 1000 +

            <rrbandwidth> + <rsbandwidth>; (NOTE 3; NOTE 6)
        ELSE

          Max-Requested-Bandwidth-DL:= <bandwidth> * 1050; 
             /* Unit is bit/s
        ENDIF

      ELSE
         Max-Requested-Bandwidth-DL:= <bandwidth> * 1000
          ;/* Unit is bit/s
      ENDIF;
    ELSE

      IF b=TIAS:<Tibandwidth> is present THEN
        IF a=rtcp-mux is negotiated (NOTE 13) THEN

          IF b=RR:<rrbandwidth> is present 
              OR b=RS:<rsbandwidth> is present THEN
            Max-Requested-Bandwidth-DL:= 
              <Transport-dependent bandwidth> (NOTE 11) +

              <rrbandwidth> + <rsbandwidth>; (NOTE 3; NOTE 6)
          ELSE

            Max-Requested-Bandwidth-DL:= 
              <Transport-dependent bandwidth>
              * 1.05 (NOTE 11) /* Unit bit/s
          ENDIF

        ELSE

          Max-Requested-Bandwidth-DL:= <Transport-dependent bandwidth> 
           (NOTE 11) /* Unit bit/s
        ENDIF;

      ELSE

        Max-Requested-Bandwidth-DL:= <Operator specific setting>,

        or AVP not supplied;

      ENDIF;

    ENDIF;
  ENDIF

ELSE

  Consider SDP in opposite direction
ENDIF

	Max-Supported-Bandwidth-UL 
	IF a=bw-info is present and includes MaxSupBw: <bandwidth> and direction: recv (UE terminated) or send (UE originated) or sendrecv (NOTE 14) THEN     Max-Supported-Bandwidth-UL:= [supplied <bandwidth>] * 1000 /Unit bit/s/
(NOTE 16)
ELSE /* a=bw-info is not present or is present but MaxSupBw is not  

          included or direction is the opposite

       AVP not supplied

ENDIF;

(NOTE 15)

	Max-Supported-Bandwidth-DL
	IF a=bw-info is present and includes MaxSupBw : <bandwidth> and direction: send (UE terminated) or recv (UE originated) or sendrecv (NOTE 14)THEN

     Max-Supported-Bandwidth-DL:= [supplied <bandwidth>] * 1000 /Unit bit/s/

(NOTE 16)ELSE /* a=bw-info is not present or is present but MaxSupBw is not  

          included or direction is the opposite

       AVP not supplied

ENDIF;   

(NOTE 15) 

	Min-Desired-Bandwidth-UL


	IF a=bw-info is present and includes MinDesBw : <bandwidth> and direction: recv (UE terminated) or send (UE originated) or sendrecv (NOTE 14) THEN

     Min-Desired-Bandwidth-UL:= supplied <bandwidth> * 1000 /Unit bit/s/

(NOTE 16)ELSE /* a=bw-info is not present or is present but MinDesBw is not  

          included or direction is the opposite

       AVP not supplied

ENDIF;

	Min-Desired-Bandwidth-DL


	IF a=bw-info is present and includes MinDesBw : <bandwidth> and direction: send (UE terminated) or recv (UE originated) or sendrecv (NOTE 14) THEN

     Min-Desired-Bandwidth-DL:= [supplied <bandwidth>] * 1000 /Unit bit/s/
(NOTE 16)
ELSE /* a=bw-info is not present or is present but MinDesBw is not  

          included or direction is the opposite

       AVP not supplied

ENDIF;     

	RR-Bandwidth
	IF b=RR:<bandwidth> is present THEN

   RR-Bandwidth:= <bandwidth>;

ELSE

   AVP not supplied

ENDIF;

(NOTE 3; NOTE 6)

	RS-Bandwidth
	IF b=RS:<bandwidth> is present THEN

   RS-Bandwidth:= <bandwidth>;

ELSE

   AVP not supplied

ENDIF;

(NOTE 3: NOTE 6)

	Media-Sub-Component
	Supply one AVP for bidirectional combination of two corresponding IP flows, if available, and for each single IP flow without a corresponding IP flow in opposite direction. 
If a media component comprises separate IP flows for RTP and RTCP, they are described in two separate Media-Sub-Component AVPs. However, if a=rtcp-mux is negotiated, RTP and RTCP use the same IP flow and shall be described in a single Media-Sub-Component AVP.
The encoding of the AVP is described in Table 6.2.2

	Reservation-Priority
	The AF may supply or omit this AVP.

	Codec-Data
	Codec Data AVP(s) are provisioned as specified in clause 5.3.16 of 3GPP TS 29.214 [10], including the codec-related information detailed in clause 5.3.7 of 3GPP TS 29.214 [10].

	NOTE 1:
The encoding of the service information is defined in 3GPP TS 29.214 [10].

NOTE 2:
The SDP parameters are described in IETF RFC 2327 [11].

NOTE 3:
The "b=RS:" and "b=RR:" SDP bandwidth modifiers are defined in IETF RFC 3556 [13].

NOTE 4:
As an operator policy to disable forward and/or backward early media, for media with UDP as transport protocol only the Flow-Status AVP may be downgraded by using the gate control procedures defined in the annex A of 3GPP TS 29.214 [10] before a SIP confirmed dialogue is established, i.e. until a 200 (OK) response to an INVITE request is received.
NOTE 5:
If the SDP answer is available when the session information is derived, the direction attributes and port number from the SDP answer shall be used to derive the flow status. However, to enable interoperability with SIP clients that do not understand the inactive SDP attribute, if "a=inactive" was supplied in the SDP offer, this shall be used to derive the flow status. If the SDP answer is not available when the session information is derived, the direction attributes from the SDP offer shall be used.

NOTE 6:
Information from the SDP answer is applicable, if available.

NOTE 7:
The AVPs may be omitted if they have been supplied in previous service information and have not changed, as detailed in 3GPP TS 29.214 [10].

NOTE 8:
"Uplink SDP indicates that the SDP was received from the UE and sent to the network. This is equivalent to <SDP direction> = UE originated.

"Downlink SDP" indicates that the SDP was received from the network and sent to the UE. This is equivalent to <SDP direction> = UE terminated.

NOTE 9:
Support for TCP at a P-CSCF acting as AF is only required if services with TCP transport are used in the corresponding IMS system.

As an operator policy to disable forward and/or backward early media, for media with TCP as transport protocol, the Max-Requested-Bandwidth-UL/DL values may be downgraded before a SIP confirmed dialogue is established, i.e. until a 200 (OK) response to an INVITE request is received. Only a small bandwidth in both directions is required in this case in order for TCP control packets to flow.

NOTE 10:

TCP uses IP flows in the directionality opposite to the transferred media for feedback. To enable these flows, a small bandwidth in this direction is required.
NOTE 11:
TIAS is defined in IETF RFC 3890 [23]. IETF RFC 3890 clause 6.4 provides procedures for converting TIAS to transport-dependant values. This procedure relies on the presence of maxprate (also defined in IETF RFC 3890). 
NOTE 12:
Multiplexed RTP/RTCP flows need to have Flow-Status set to “ENABLED” in order to always permit the RTCP traffic. 

NOTE 13:
RTP/RTCP multiplexing is defined in IETF RFC 5761 [39].

NOTE 14:
This AVP is derived from the SDP answer information and is omitted if E2EQOSMTSI feature is not supported. 

NOTE 15:
When both "b =" line and "a=bw-info" including MaxSupBw are present when sending the SDP, it is expected that the values are aligned.
NOTE 16:
When the supplied bandwidth does not correspond to the bandwidth applicable to the IP version used by the UE, the AF shall re-compute it considering the IP version used by the UE as defined in 3GPP TS 26.114 [29].


Table 6.2.2: Rules for derivation of Media-Sub-Component AVP from SDP media component

	service information per Media-Sub-Component AVP

(see notes 1 and 5)
	Derivation from SDP Parameters
(see Note 2)

	Flow-Number
	The AF shall assign a number to the media-subcomponent AVP that is unique within the surrounding media component AVP and for the entire lifetime of the AF session. The AF shall select the ordinal number of the IP flow(s) within the "m=" line assigned in the order of increasing downlink destination port numbers, if downlink destination port numbers are available. For uplink or inactive unicast media IP flows, a downlink destination port number is nevertheless available, if SDP offer-answer according to IETF RFC 3264 is used.

The AF shall select the ordinal number of the IP flow(s) within the "m=" line assigned in the order of increasing uplink destination port numbers, if no downlink destination port numbers are available.

	Flow-Status
	AVP not supplied

	Max-Requested-Bandwidth-UL
	AVP not supplied

	Max-Requested-Bandwidth-DL
	AVP not supplied

	Flow-Description
	For uplink and downlink direction, a Flow-Description AVP shall be provided unless no IP Flows in this direction are described within the media component.

If UDP is used as transport protocol, the SDP direction attribute (NOTE 4) indicates the direction of the media IP flows within the media component as follows:

   IF a=recvonly THEN (NOTE 3)

      IF <SDP direction> = UE originated (NOTE 7) THEN

         Provide only downlink Flow-Description AVP

      ELSE /* UE terminated (NOTE 7) */

         Provide only uplink Flow-Description AVP

      ENDIF;

   ELSE

      IF a=sendonly THEN (NOTE 3)

         IF <SDP direction> = UE originated (NOTE 7) THEN

            Provide only uplink Flow-Description AVP

         ELSE /* UE terminated (NOTE 7) */

            Provide only downlink Flow-Description AVP

         ENDIF;

      ELSE /* a=sendrecv or a=inactive or no direction attribute */

         Provide uplink and downlink Flow-Description AVPs

      ENDIF;

   ENDIF;

However, for RTCP and RTP/RTCP multiplexed IP flows uplink and downlink Flow-Description AVPs shall be provided irrespective of the SDP direction attribute.

If TCP is used as transport protocol (NOTE 8), IP flows in uplink and downlink direction are described in SDP irrespective of the SDP direction attribute, as TCP uses an IP flow for feedback even if contents are transferred only in the opposite direction. Thus, both uplink and downlink Flow-Description AVPs shall be provided.
The uplink destination address shall be copied from the "c=" line of downlink SDP. (NOTE 6) (NOTE 7)
The uplink destination port shall be derived from the "m=" line of downlink SDP. (NOTE 6) (NOTE 7) However, for TCP transport the uplink destination port shall be wildcarded, if the local UE is the passive endpoint (NOTE 9)
The downlink destination address shall be copied from the "c=" line of uplink SDP. (NOTE 6) However, a P-CSCF acting as AF and applying NAT traversal procedures in Annex C shall derive the downlink destination address using those procedures.
The downlink destination port shall be derived from the "m=" line of uplink SDP. (NOTE 6) (NOTE 7) However, for TCP transport the downlink destination port shall be wildcarded, if the local UE is the active endpoint (NOTE 9). A P-CSCF acting as AF and applying NAT traversal procedures in Annex C shall derive the downlink destination port using those procedures.

For IPv6, uplink and downlink source addresses shall either be derived from the prefix of the destination address or be wildcarded by setting to "any", as specified in 3GPP TS 29.214 [10]. However, a P-CSCF acting as AF and applying NAT traversal procedures in Annex C shall derive the uplink source address using those procedures.

If IPv4 is being utilized, the uplink source address shall either be set to the address contained in the "c=" line of the uplink SDP or be wildcared, and the downlink source address shall either be set to the address contained in the "c=" line of the downlink SDP or be wildcarded. However, for TCP transport, if the local UE is the passive endpoint (NOTE 9), the uplink source address shall not be wildcarded. If the local UE is the active endpoint (NOTE 9), the downlink source address shall not be wildcarded. A P-CSCF acting as AF and applying NAT traversal procedures in Annex C shall derive the uplink source address using those procedures.
Source ports shall not be supplied. However, for TCP transport, if the local UE is the passive end point (NOTE 9), the uplink source port shall be derived from the "m=" line of the uplink SDP. If the local UE is the active end point (NOTE 9), the downlink source port shall be derived from the "m=" line of the downlink SDP. A P-CSCF acting as AF and applying NAT traversal procedures in Annex C shall derive the downlink source ports using those procedures.
Proto shall be derived from the transport of the "m=" line. For “RTP/AVP” proto is 17(UDP). For "TCP", as defined in IETF RFC 4145 [16], or "TCP/MSRP", as defined in IETF RFC 4975 [17], proto is 6(TCP).

	Flow-Usage
	The Flow-Usage AVP shall be supplied with value "RTCP" if the IP flow(s) described in the Media-Sub-Component AVP are used to transport RTCP only. Otherwise the Flow-Usage AVP shall not be supplied. IETF RFC 2327 [11] specifies how RTCP flows are described within SDP.

If the IP flows(s) are used to transport signalling  the value should be “AF-SIGNALLING”


	NOTE 1:
The encoding of the service information is defined in 3GPP TS 29.214 [10].

NOTE 2:
The SDP parameters are described in IETF RFC 2327 [11].

NOTE 3:
If the SDP direction attribute for the media component negotiated in a previous offer-answer exchange was sendrecv, or if no direction attribute was provided, and the new SDP direction attribute sendonly or recvonly is negotiated in a subsequent SDP offer-answer exchange, uplink and downlink Flow-Description AVPs shall be supplied.

NOTE 4:
If the SDP answer is available when the session information is derived, the direction attributes from the SDP answer shall be used to derive the flow description. However, to enable interoperability with SIP clients that do not understand the inactive SDP attribute, if "a=inactive" was supplied in the SDP offer, this shall be used. If the SDP answer is not available when the session information is derived, the direction attributes from the SDP offer shall be used.
NOTE 5:
The AVPs may be omitted if they have been supplied in previous service information and have not changed, as detailed in 3GPP TS 29.214 [10].
NOTE 6:
If the session information is derived from an SDP offer, the required SDP may not yet be available. The corresponding Flow Description AVP shall nethertheless be included and the unavailable fields (possibly all) shall be wildcarded.

NOTE 7:
"Uplink SDP" indicates that the SDP was received from the UE and sent to the network. This is equivalent to <SDP direction> = UE originated.

"Downlink SDP" indicates that the SDP was received from the network and sent to the UE. This is equivalent to <SDP direction> = UE terminated.

NOTE 8:
Support for TCP at a P-CSCF acting as AF is only required if services with TCP transport are used in the corresponding IMS system.

NOTE 9:
For TCP transport, the passive endpoints is derived from the SDP "a:setup" attribute according to the rules in IETF RFC 4145 [16], or, if that attribute is not present, from the rules in IETF RFC 4975 [17].


6.3
QoS parameter mapping Functions at PCRF

The QoS authorization process consists of the derivation of the parameters Authorized QoS Class Identifier (QCI), Allocation and Retention Priority (ARP), and Authorized Maximum/Guaranteed Data Rate UL/DL.

When a session is initiated or modified the PCRF shall derive Authorized IP QoS parameters (i.e. QCI, Authorized Maximum/Guaranteed Data Rate DL/UL, ARP) from the service information. If the selected Bearer Control Mode (BCM) is UE-only this process shall be performed according to the mapping rules in table 6.3.1 to avoid undesired misalignments with the UE QoS parameters mapping.

In the case of forking, the various forked responses may have different QoS requirements for the IP flows of the same media component. Each Authorized IP QoS Parameter should be set to the highest value requested for the IP flow(s) of that media component by any of the active forked responses.

Table 6.3.1: Rules for derivation of the Maximum Authorized Data Rates, Authorized Guaranteed Data Rates
and Maximum Authorized QoS Class per IP flow or bidirectional combination of IP flows in the PCRF

	Authorized IP QoS Parameter 
	Derivation from service information
(see Note 4)

	Maximum Authorized Data Rate DL (Max_DR_DL) and UL (Max_DR_UL) 
	IF operator special policy exists THEN

  Max_DR_UL:= as defined by operator specific algorithm;

  Max_DR_DL:= as defined by operator specific algorithm;

ELSE

  IF AF-Application-Identifier AVP demands application specific data rate

  handling THEN

    Max_DR_UL:= as defined by application specific algorithm;

    Max_DR_DL:= as defined by application specific algorithm;

  ELSE IF Codec-Data AVP provides Codec information for a codec that is

  supported by a specific algorithm THEN

    Max_DR_UL:= as defined by specific algorithm;

    Max_DR_DL:= as defined by specific algorithm;

  ELSE

    IF not RTCP flow(s) according to Flow-Usage AVP THEN

      IF Flow-Status = REMOVED THEN

        Max_DR_UL:= 0;

        Max_DR_DL:= 0;

      ELSE

        IF uplink Flow Description AVP is supplied THEN
          IF Max-Supported-Bandwidth-UL is present and supported THEN

             Max_DR_UL:= Max-Supported-Bandwidth-UL;
         ELSE IF Max-Requested-Bandwidth-UL is present THEN

            Max_DR_UL:= Max-Requested-Bandwidth-UL ;

          ELSE

            Max_DR_UL:= as set by the operator;

          ENDIF;

        ELSE

          Max_DR_UL:= 0;

        ENDIF;

        IF downlink Flow Description AVPs is supplied THEN
          IF Max-Supported-Bandwidth-DL is present and supported THEN

            Max_DR_DL:= Max-Supported-Bandwidth-DL;
          ELSE IF Max-Requested-Bandwidth-DL is present THEN
            Max_DR_DL:= Max-Requested-Bandwidth-DL;

          ELSE

            Max_DR_DL:= as set by the operator;

          ENDIF;

        ELSE

          Max_DR_DL:= 0;

        ENDIF;

      ENDIF;

    ELSE /* RTCP IP flow(s) */

      IF RS-Bandwidth is present and RR-Bandwidth is present THEN 
        Max_DR_UL:= (RS-Bandwidth + RR-Bandwidth);
        Max_DR_DL:= (RS-Bandwidth + RR-Bandwidth);
      ELSE
        IF Max-Requested-Bandwidth-UL is present THEN

          IF RS-Bandwidth is present and RR-Bandwidth is not present THEN

            Max_DR_UL:= MAX[0.05 * Max-Requested-Bandwidth-UL,RS-Bandwidth];

          ENDIF;

          IF RS-Bandwidth is not present and RR-Bandwidth is present THEN

            Max_DR_UL:= MAX[0.05 * Max-Requested-Bandwidth-UL,RR-Bandwidth];

          ENDIF;

          IF RS-Bandwidth and RR-Bandwidth are not present THEN

            Max_DR_UL:= 0.05 * Max-Requested-Bandwidth_UL ;

          ENDIF;

        ELSE

          Max_DR_UL:= as set by the operator;

        ENDIF;

        IF Max-Requested-Bandwidth-DL is present THEN

          IF RS-Bandwidth is present and RR-Bandwidth is not present THEN

            Max_DR_DL:= MAX[0.05 * Max-Requested-Bandwidth-DL,RS-Bandwidth];

          ENDIF;

          IF RS-Bandwidth is not present and RR-Bandwidth is present THEN

            Max_DR_DL:= MAX[0.05 * Max-Requested-Bandwidth-DL,RR-Bandwidth];

          ENDIF;

          IF RS-Bandwidth and RR-Bandwidth are not present THEN

            Max_DR_DL:= 0.05 * Max-Requested-Bandwidth-DL;
          ENDIF;

        ELSE

          Max_DR_DL:= as set by the operator;

        ENDIF;

      ENDIF;

    ENDIF;

  ENDIF;

ENDIF;

IF SIP-Forking-Indication AVP indicates SEVERAL_DIALOGUES THEN

   Max_DR_UL = MAX[Max_DR_UL, previous Max_DR_UL]
   Max_DR_DL = MAX[Max_DR_DL, previous Max_DR_DL]

ENDIF;



	Authorized Guaranteed Data Rate DL (Gua_DR_DL) and UL (Gua_DR_UL)
(see NOTE 11, 13, 15, 16)
	IF operator special policy exists THEN

  Gua_DR_UL:= as defined by operator specific algorithm;

  Gua_DR_DL:= as defined by operator specific algorithm;

ELSE

  IF AF-Application-Identifier AVP demands application specific data rate

  handling THEN

    Gua_DR_UL:= as defined by application specific algorithm;

    Gua_DR_DL:= as defined by application specific algorithm;

  ELSE IF Codec-Data AVP provides Codec information for a codec that is

  supported by a specific algorithm (NOTE 5)THEN

    Gua_DR_UL:= as defined by specific algorithm;

    Gua_DR_DL:= as defined by specific algorithm;

  ELSE

       IF uplink Flow-Description AVP is supplied THEN
          IF Min-Desired-Bandwidth-UL is present and supported THEN

            Gua_DR_UL:= Min-Desired-Bandwidth-UL ;
          ELSE IF Min-Requested-Bandwidth-UL is present THEN

            Gua_DR_UL:= Min-Requested-Bandwidth-UL ;

          ELSE

            Gua_DR_UL:= as set by the operator;

          ENDIF;

       ELSE

          Gua_DR_UL:= Max DR UL;

       ENDIF;

       IF downlink Flow-Description AVP is supplied THEN
          IF Min-Desired-Bandwidth-DL is present and supported THEN

            Gua_DR_DL:= Min-Desired-Bandwidth-DL
          ELSE IF Min-Requested-Bandwidth-DL is present THEN

            Gua_DR_DL:= Min-Requested-Bandwidth-DL ;

          ELSE

            Gua_DR_DL:= as set by the operator;

          ENDIF;

       ELSE

          Gua_DR_DL:= Max DR DL;

       ENDIF;

  ENDIF;

ENDIF;

IF SIP-Forking-Indication AVP indicates SEVERAL_DIALOGUES THEN

   Gua_DR_UL = MAX[Gua_DR_UL, previous Gua_DR_UL]
   Gua_DR_DL = MAX[Gua_DR_DL, previous Gua_DR_DL]

ENDIF;



	Authorized QoS Class Identifier [QCI]

(see NOTE 1, 2, 7, 12 and 14)
	IF an operator special policy exists THEN

  QCI:= as defined by operator specific algorithm;

ELSE IF MPS-Identifier AVP demands MPS specific QoS Class handling THEN

     QCI:= as defined by MPS specific algorithm (NOTE 18);

ELSE IF GCS-Identifier AVP demands Group Communication specific handling THEN

     QCI:= as defined by GCS specific algorithm (NOTE 17);

ELSE IF AF-Application-Identifier AVP demands application specific QoS Class handling THEN

     QCI:= as defined by application specific algorithm;

ELSE IF Codec-Data AVP provides Codec information for a codec that is supported by a specific algorithm THEN

     QCI:= as defined by specific algorithm; (NOTE 5)

ELSE
     /* The following QCI derivation is an example of how to obtain the QCI

         values in a GPRS network */
     IF Media-Type is present THEN

          /* for GPRS: streaming */

          IF (only uplink Flow Description AVPs are supplied for all IP

          flows of the AF session, which have media type “audio” or “video”
          and no flow usage “RTCP”, or

          only downlink Flow Desription AVPs are supplied for all IP

          flows of the AF session, which have media type “audio” or “video”
          and no flow usage “RTCP”) THEN

             CASE Media-Type OF

               “audio”:        MaxClassDerivation := 3 OR 4; (NOTE 9)

               “video”:        MaxClassDerivation := 4

             END;

           /* for GPRS: conversational */

           ELSE

             CASE Media-Type OF

               “audio”:        MaxClassDerivation:= 1 OR 2; (NOTE 6)

               “video”:        MaxClassDerivation:= 2

             END;

           ENDIF;

           CASE Media-Type OF

             “audio”:        QCI := MaxClassDerivation

             “video”:        QCI := MaxClassDerivation

             “application”:  QCI := 1 OR 2; (NOTE 6)

                /*e.g. for GPRS: conversational*/

             “data”:         QCI := 6 OR 7 OR 8; (NOTE 8)

                /*e.g. for GPRS: interactive with prio 1, 2 AND 3
                   respectively*/

             “control”:      QCI := 6;

                /*e.g. for GPRS: interactive with priority 1*/

/* NOTE: include new media types here */

             OTHERWISE:      QCI := 9;

                /*e.g. for GPRS: background*/

           END;
   ENDIF;
ENDIF;

IF SIP-Forking-Indication AVP indicates SEVERAL_DIALOGUES THEN

   QCI = MAX[QCI, previous QCI](NOTE 10)

ENDIF ;


	NOTE 1:
The QCI assigned to a RTCP IP flow is the same as for the corresponding RTP media IP flow.

NOTE 2:
When audio or video IP flow(s) are removed from a session, the parameter MaxClassDerivation shall keep the originally assigned value.

NOTE 3:
When audio or video IP flow(s) are added to a session, the PCRF shall derive the parameter MaxClassDerivation taking into account the already existing media IP flow(s) within the session.

NOTE 4:
The encoding of the service information is defined in TS 29.214 [10]. If AVPs are omitted within a Media-Component-Description AVP or Media-Sub-Component AVP of the service information, the corresponding information from previous service information shall be used, as specified in TS 29.214 [10].

NOTE 5:
TS 26.234 [6], TS 26.114 [29], 3GPP2 C.S0046 [18], and 3GPP2 C.S0055 [19] contain examples of QoS parameters for codecs of interest. The support of any codec specific algorithm in the PCRF is optional.

NOTE 6:
The final QCI value will depend on the value of SSID (speech/unknown) according to TS 23.107 [4]. If the PCRF is not able to determine the SSID, it should use the QCI value 2 that corresponds to SSID unknown. For UE-init and mixed mode, the PCRF may derive from the requested QoS of an IP CAN bearer which SSID is applicable.

NOTE 7:
The numeric value of the QCI are based on TS 29.212 [9].

NOTE 8:
The QCI value also encodes the traffic handling priority for GPRS. If the PCRF is not able to determine a traffic handling priority, it should choose QCI 8 that corresponds to priority 3. Also, for UE-initiated bearers the PCRF should only use QCI 8 in order to have the same mapping rules in both UE and PCRF.

NOTE 9:
The final QCI value will depend on the value of SSID (speech/unknown) according to TS 23.107 [4]. If the PCRF is not able to determine the SSID, it should use the QCI value 4 that corresponds to SSID unknown. For UE-init and mixed mode, the PCRF may derive from the requested QoS of an IP CAN bearer which SSID is applicable.

NOTE 10:
The Max function shall use the following precedence order for the QCI values:  2 > 1 > 4 > 3 > 5 > 6 > 7 > 8 > 9
NOTE 11: Authorized Guaranteed Data Rate DL and UL shall not be derived for QCI values 5, 6, 7, 8 and 9.
NOTE 12: Recommended QCI values for standardised QCI characteristics are shown in table 6.1.7 in TS 23.203 [2].
NOTE 13: The PCRF may be configured with operator specific preconditions for setting the Authorized Guaranteed Data Rate lower than the corresponding Maximum Authorized Data Rate.
NOTE 14: In a network where SRVCC is enabled, the QCI=1 shall be used for IMS services in accordance to TS 23.216 [27]. Non-IMS services using QCI=1 may suffer service interruption and/or inconsistent service experience if SRVCC is triggered. Triggering SRVCC for WebRTC IMS session will cause service interruption and/or inconsistent service experience when using QCI=1. Operator policy (e.g. use of specific AF application identifier) may be used to avoid using QCI 1 for a voice service, e.g. WebRTC IMS session.
NOTE 15: For certain services (e.g. DASH services according to TS 26.247 [30]), the AF may also provide a minimum required bandwidth so that the PCRF can derive an Authorized Guaranteed Data Rate lower than the Maximum Authorized Data Rate.
NOTE 16: For GPRS and EPS, the PCRF shall assign an Authorized Guaranteed Data Rate UL/DL value within the limit supported by the serving network.
NOTE 17:
The GCS specific algorithm shall consider various inputs, including the received Reservation-Priority AVP, for deriving the QCI.
NOTE 18:
The MPS specific algorithm shall consider various inputs, including the received MPS-Identifier AVP and Reservation-Priority AVP, for deriving the QCI.


The PCRF should per ongoing session store the Authorized IP QoS parameters per for each IP flow or bidirectional combination of IP flows (as described within a Media Subcomponent AVP).

If the PCRF provides a QoS-Information AVP within a Charging-Rule-Definition AVP it may apply the rules in table 6.3.2 to combine the Authorized QoS per IP flow or bidirectional combination of IP flows (as derived according to table 6.3.1) for all IP flows described by the corresponding PCC rule.

If the PCRF provides a QoS-Information AVP for an entire IP CAN bearer (for a UE-initiated IP-CAN bearer in the GPRS case) or IP CAN session, it may apply the rules in table 6.3.2 to combine the Authorized QoS per IP flow or bidirectional combination of IP flows (as derived according to table 6.3.1) for all IP flows allowed to be transported within the IP CAN bearer or session. It is recommended that the rules in table 6.3.2 are applied for all IP flows with corresponding AF session. The PCRF may increase the authorized QoS further to take into account the requirements of predefined PCC rules without ongoing AF sessions.
NOTE:
QoS-Information AVP provided at IP-CAN session level is not derived based on mapping tables, but based on subscription and operator specific policies.
NOTE:
Allocation-Retention-Priority AVP is always calculated at PCC rule level according to table 6.3.2.
For a UE initiated PDP context within GPRS, the PCRF applies the binding mechanism described in clause 5 to decide which flows are allowed to be transported within the IP CAN bearer.

Table 6.3.2: Rules for calculating the Maximum Authorized/Guaranteed Data Rates,
QCI and ARP in the PCRF

	Authorized IP QoS Parameter 
	Calculation Rule

	Maximum Authorized Data Rate DL and UL 
	Maximum Authorized Data Rate DL/UL is the sum of all Maximum Authorized Data Rate DL/UL for all the IP flows or bidirectional combinations of IP flows (as according to table 6.3.1).

IF Network = GPRS AND Maximum Authorized Data Rate DL/UL > 256 Mbps THEN
   Maximum Authorized Data Rate DL/UL = 256 Mbps  /* See TS 23.107 [4] */
ENDIF;



	Guaranteed Authorized Data Rate DL and UL
(see NOTE 3)
	Guaranteed Authorized Data Rate DL/UL is the sum of all Guaranteed Authorized Data Rate DL/UL for all the IP flows or bidirectional combinations of IP flows (as according to table 6.3.1).



	QCI 
	QCI = MAX [needed QoS parameters per IP flow or bidirectional combination of IP flows (as operator’s defined criteria) among all the IP flows or bidirectional combinations of IP flows.]



	ARP
(see NOTE 1)
	IF an operator special policy exists THEN

  ARP:= as defined by operator specific algorithm;

ELSE IF MPS-Identifier AVP demands MPS specific ARP handling THEN

    ARP:= as defined by MPS specific algorithm (NOTE 2);
ELSE IF GCS-Identifier AVP demands Group Communication Service specific ARP handling THEN

    ARP:= as defined by GCS specific algorithm (NOTE 4);

ELSE IF MCPTT-Identifier AVP demands MCPTT specific ARP handling THEN

    ARP:= as defined by MCPTT specific algorithm (NOTE 5);

ELSE  IF AF-Application-Identifier AVP demands application specific ARP

    handling THEN

      ARP:= as defined by application specific algorithm;

ELSE IF Reservation-Priority AVP demands application specific ARP handling THEN

        ARP:= as defined by application specific algorithm;
ENDIF;



	NOTE 1:
The ARP priority levels 1-8 should only be assigned to resources for services that are authorized to receive prioritized treatment within an operator domain.
NOTE 2:
The MPS specific algorithm shall consider various inputs, including the received Reservation-Priority AVP, for deriving the ARP.
NOTE 3:  For GPRS and EPS, the PCRF may check that the Guaranteed Authorized Data Rate DL/UL does not exceed the limit supported by the serving network to minimize the risk of rejection of the bearer by the serving network.

NOTE 4:
The GCS specific algorithm shall consider various inputs, including the received Reservation-Priority AVP, for deriving the ARP.
NOTE 5:
The MCPTT specific algorithm shall consider the value of the MCPTT-Identifier AVP and the value of the Reservation-Priority AVP.


6.4
QoS parameter mapping Functions at PCEF
6.4.1
GPRS

6.4.1.1
Authorized IP QoS parameters per PDP Context to Authorized UMTS QoS parameters mapping in GGSN

The Translation/Mapping function in the GGSN shall derive the Authorized UMTS QoS parameters from the Authorized IP QoS parameters received from the PCRF according to the rules in table 6.4.1.

Table 6.4.1: Rules for derivation of the Authorized UMTS QoS Parameters per PDP context
from the Authorized IP QoS Parameters in GGSN
	Authorized UMTS QoS Parameter per PDP context
	Derivation from Authorized IP QoS Parameters

	Maximum Authorized Bandwidth DL and UL per PDP context
(see NOTE 2)
	Maximum Authorized Bandwidth DL/UL per PDP context = Maximum Authorized Data Rate DL/UL 

	Guaranteed Authorized Data Rate DL and UL per PDP context
	Guaranteed Authorized Data Rate DL/UL per PDP context = Guaranteed Authorized Data Rate DL/UL

	Maximum Authorized Traffic Class per PDP context
	IF QCI = 1 OR 2 THEN
     Maximum Authorized Traffic Class = “Conversational”

ELSEIF QCI = 3 OR 4 THEN
     Maximum Authorized Traffic Class = “Streaming”

ELSEIF QCI = 5 OR 6 OR 7 OR 8 THEN
     Maximum Authorized Traffic Class = “Interactive”;

ELSE Maximum Authorized Traffic Class = “Background”
ENDIF ;

	Traffic Handling Priority
	IF QCI = 5 OR 6 THEN

     Maximum Authorized Traffic Handling Priority = “1”;

ELSE IF QCI = 7 THEN

     Maximum Authorized Traffic Handling Priority = “2”;

ELSE IF QCI = 8 THEN

     Maximum Authorized Traffic Handling Priority = “3”;

ELSE the GGSN shall not derive Traffic Handling Priority

ENDIF ;

	Signalling Indication
	IF QCI = 5 THEN

     Signalling Indication = “Yes”;

ELSE IF QCI = 6 OR 7 OR 8 THEN

     Signalling Indication = “No”;

ELSE the GGSN shall not derive Signalling Indication

ENDIF ;



	Source Statistics Descriptor
	IF QCI = (1 OR 3) THEN

     Source Statistics Descriptor = “speech”;

ELSE IF QCI = 2 OR 4 THEN

     Source Statistics Descriptor = “unknown”;

ELSE the GGSN shall not derive Source Statistics Descriptor

ENDIF ;



	Evolved Allocation/Retention Priority

(see NOTE 1)
	Evolved Allocation/Retention Priority  = Allocation-Retention-Priority as follows :

      PL := Priority-Level ;

      PVI := Pre-emption-Vulnerability ;

      PCI := Pre-emption-Capability ;


	APN-AMBR UL and DL


	For non-GBR PDP Contexts, APN-AMBR DL/UL = APN-Aggregate-Max-Bitrate DL/UL



	NOTE 1:   Evolved Allocation/Retention Priority is derived only if supported by the SGSN.
NOTE 2:   When APN-AMBR is supported in GPRS and the PCEF performs the bearer binding, the MBR for non-GBR

           PDP-Contexts is not derived


6.4.1.2
Comparing UMTS QoS Parameters against the Authorized UMTS QoS parameters in GGSN for UE initiated PDP context

Upon receiving a PDP context activation, the GGSN requests PCC rules from the PCRF (see 3GPP TS 29.212 [9] for details). The PCRF may supply Authorized IP QoS Parameters per PDP context together with the PCC rules. The GGSN maps the Authorized IP QoS parameters per PDP Context to Authorized UMTS QoS parameters according to subclause 6.4.1.1 and then compares the requested UMTS QoS parameters against the corresponding Authorized UMTS QoS parameters. The following criteria shall be fulfilled:

-
If the requested Guaranteed Bitrate DL/UL (if the requested Traffic Class is Conversational or Streaming) is equal to the Authorized Guaranteed data rate DL/UL; and
-
if received, the requested Maximum Bitrate DL/UL (if the requested Traffic Class is Interactive or Background) is equal to Maximum Authorized data rate DL/UL; and

-
the requested Traffic Class is equal to Maximum Authorized Traffic Class; and.
-
if received, the requested Evolved Allocation/Retention Priority is equal to Allocation-Retention-Priority.
-
if received, the requested APN-AMBR DL/UL is equal to the APN-Aggregate-Max-Bitrate DL/UL.
Then, the GGSN shall accept the PDP context activation or modification with the UE requested parameters.Otherwise, the GGSN is adjusted (downgrade or upgrade) the requested UMTS QoS parameters to the values that were authorized.
6.4.2
3GPP- EPS

6.4.2.1
Authorized IP QoS parameters per PDP Context to Authorized UMTS QoS parameters mapping in P-GW.

This Translation/Mapping function in the P-GW applies when the P-GW interacts with a Gn/Gp SGSN.

The Translation/Mapping function in the P-GW shall derive the Authorized UM TS QoS parameters from the Authorized IP QoS parameters derived for the bearer applying the rules in table 6.4.2.

Table 6.4.2: Rules for derivation of the Authorized UMTS QoS Parameters per PDP context
from the Authorized IP QoS Parameters in P-GW.
	Authorized UMTS QoS Parameter per PDP context
	Derivation from Authorized IP QoS Parameters

	Maximum Authorized Bandwidth DL and UL per PDP context

(see NOTE 2)
	For non-GBR bearers, Maximum Authorized Bandwidth DL/UL per PDP context = APN-Aggregate-Max-Bitrate DL/UL

For GBR bearers, Maximum Authorized Bandwidth DL/UL per PDP context = Sum of Maximum Authorized Data Rate DL/UL for all PCC Rules bound to that bearer 

	Guaranteed Authorized Data Rate DL and UL per PDP context
	Guaranteed Authorized Data Rate DL/UL per PDP context = Sum of Guaranteed Authorized Data Rate DL/UL for all PCC Rules bound to that bearer

	Maximum Authorized Traffic Class per PDP context
	IF QCI = 1 OR 2 OR 3 THEN
     Maximum Authorized Traffic Class = “Conversational”

ELSEIF QCI = 4 THEN
     Maximum Authorized Traffic Class = “Streaming”

ELSEIF QCI = 5 OR 6 OR 7 OR 8 THEN
     Maximum Authorized Traffic Class = “Interactive”;

ELSE Maximum Authorized Traffic Class = “Background”
ENDIF ;

	Traffic Handling Priority
	IF QCI = 5 OR 6 THEN

     Maximum Authorized Traffic Handling Priority = “1”;

ELSE IF QCI = 7 THEN

     Maximum Authorized Traffic Handling Priority = “2”;

ELSE IF QCI = 8 THEN

     Maximum Authorized Traffic Handling Priority = “3”;

ELSE the P-GW shall not derive Traffic Handling Priority

ENDIF ;

	Signalling Indication
	IF QCI = 5 THEN

     Signalling Indication = “Yes”;

ELSE IF QCI = 6 OR 7 OR 8 THEN

     Signalling Indication = “No”;

ELSE the P-GW shall not derive Signalling Indication

ENDIF ;



	Source Statistics Descriptor
	IF QCI = 1 THEN

     Source Statistics Descriptor = “speech”;

ELSE IF QCI = 2 OR 3 OR 4 THEN

     Source Statistics Descriptor = “unknown”;

ELSE the P-GW shall not derive Source Statistics Descriptor

ENDIF ;



	APN-AMBR DL and UL

(see NOTE 3)
	For non-GBR bearers, APN-AMBR = APN-Aggregate-Max-Bitrate DL/UL



	Transfer Delay

(see NOTE 1)
	IF QCI = 2 THEN

     Transfer Delay = 150 ms

ELSE IF QCI = 3 THEN

     Transfer Delay >= 80 ms

ELSE IF QCI = 1 OR 4 the P-GW shall set the Transfer Delay as the Packet Delay Budget for that QCI

ELSE the P-GW shall not derive Transfer Delay.

ENDIF ;



	Evolved Allocation/Retention Priority

(see NOTE 4)
	Evolved Allocation/Retention Priority  = Allocation-Retention-Priority as follows :

      PL := Priority-Level ;

      PVI := Pre-emption-Vulnerability ;

      PCI := Pre-emption-Capability ;


	NOTE 1:   Recommended Packet Delay Budget values for the different QCI values are defined in subclause 
6.1.7,3GPP TS 23.203 [2].

NOTE 2:   For non-GBR bearers, applicable only if APN-AMBR is not supported by the SGSN.

NOTE 3:   Applicable to all non-GBR PDP-Contexts when supported by the SGSN.
NOTE 4:   Evolved Allocation/Retention Priority is derived only if supported by the SGSN.


6.4.2.2
Comparing UMTS QoS Parameters against the Authorized UMTS QoS parameters in P-GW for UE initiated PDP context

Upon receiving a PDP context activation, the P-GW requests PCC rules from the PCRF (see 3GPP TS 29.212 [9] for details). The PCRF may supply Authorized IP QoS Parameters applicable for the provided PCC rules. The P-GW calculates the Authorized IP QoS parameters per bearer and maps the Authorized IP QoS parameters per bearer to Authorized UMTS QoS parameters according to subclause 6.4.2.1 and then compares the requested UMTS QoS parameters against the corresponding Authorized UMTS QoS parameters. The following criteria shall be fulfilled:

-
If the requested Guaranteed Bitrate DL/UL (if the requested Traffic Class is Conversational or Streaming) is equal to the Authorized Guaranteed data rate DL/UL; and

-
the requested Maximum Bitrate DL/UL (if the requested Traffic Class is Interactive or Background) is equal to Maximum Authorized data rate DL/UL; and

-
the requested Traffic Class is equal to Maximum Authorized Traffic Class; and

-
if received, the requested Evolved Allocation/Retention Priority is equal to Allocation-Retention-Priority.

Then, the P-GW shall accept the PDP context activation with the UE requested parameters. Otherwise, the P-GW shall accept the request and adjust (downgrade or upgrade) the requested UMTS QoS parameters to the values that were authorized.
6.5
QoS parameter mapping Functions at UE for a UE-initiated GPRS PDP Context

Figure 6.5.1 indicates the entities participating in the generation of the requested QoS parameters when the UE activates or modifies a PDP Context. The steps are:

1.
The Application provides the UMTS BS Manager, possibly via the IP BS Manager and the Translation/Mapping function, with relevant information to perform step 2 or step 4. (Not subject to standardization within 3GPP).

2.
If needed, information from step 1 is used to access a proper set of UMTS QoS Parameters. See 3GPP TS 26.114 [29] for Conversational Codec Applications and 3GPP TS 26.234 [6] for Streaming Codec Applications.

3.
If SDP is available then the SDP Parameters should give guidance for the UMTS BS Manager (possibly via the IP Manager and the Translation/Mapping function) ,according to the rules in clause 6.5.1, to set the Maximum Bitrate UL/DL and the Guaranteed Bitrate UL/DL. Furthermore the Maximum Authorized Bandwidth UL/DL and Maximum Authorised Traffic Class should be derived according to the rules in clause 6.5.2.
4.
A set of UMTS QoS Parameters values from step 2 (or directly from step 1) is possibly merged together with the Maximum Bitrate UL/DL and the Guaranteed Bitrate UL/DL from step 3. The result should constitute the requested UMTS QoS Parameters. The UE should check that the requested Guaranteed Bitrate UL/DL or requested Maximum Bitrate UL/DL (depending on the requested Traffic Class) does not exceed the Maximum Authorized Bandwidth UL/DL derived in step 3. Furthermore, if the UE has implemented the mapping rule for Maximum Authorized Traffic Class, as defined in clause 6.5.2, the UE should check that the requested Traffic Class does not exceed the Maximum Authorised Traffic Class derived in step 3.

[image: image58.wmf] 

 

Application

 

SDP Handler

 

UMTS BS 

Manager

 

Session

 

Manager

 

SGSN

 

Session

 

Manager

 

PDP Contex Activation and Modification

 

(SDP)

 

UE

 

UMTS

 

QoS

 

Param.

 

Per

 

Applic 

Type

 

1)

 

2)

 

3)

 

4)

 

Translation /

 

Mapping

 

IP BS Manager

 


Figure 6.5.1: Framework for generating requested QoS parameters in the UE

6.5.1
SDP to UMTS QoS parameter mapping in UE

If SDP Parameters are available, then before activating or modifying a PDP Context the UE should check if the SDP Parameters give guidance for setting the requested UMTS QoS Parameters. The UE should use the mapping rule in table 6.5.1.1 to derive the Maximum and Guaranteed Bitrate DL/UL from the SDP Parameters.
Table 6.5.1.1: Recommended rules for derivation of the requested Maximum
and Guaranteed Bitrate DL/UL per media component in the UE
	UMTS QoS Parameter per media component 
	Derivation from SDP Parameters

	Maximum Bitrate DL/UL and
Guaranteed Bitrate DL/UL per media component
	/* Check if the media use codec(s) */
IF [(<media> = (“audio” or “video”)) and (<transport> = “RTP/AVP”)] THEN

    /* Check if Streaming */ 
    IF a= (“sendonly” or “recvonly”) THEN
        Maximum Bitrate DL/UL and Guaranteed Bitrate DL/UL per media component as specified in reference [6] ;
    /* Conversational as default !*/ 
    ELSE
        Maximum Bitrate DL/UL and Guaranteed Bitrate DL/UL per media component as specified in reference [7] ;
    ENDIF ;
 
/* Check for presence of bandwidth attribute for each media component */
ELSEIF b=AS:<bandwidth-value> is present THEN
    IF media stream only downlink THEN

        Maximum Bitrate DL = Guaranteed Bitrate DL =<bandwidth-value >;

    ELSEIF mediastream only uplink THEN

        Maximum Bitrate UL = Guaranteed Bitrate UL =<bandwidth-value >;

    ELSEIF mediastreams both downlink and uplink THEN

        Maximum Bitrate DL = Guaranteed Bitrate DL =<bandwidth-value >;

        Maximum Bitrate UL = Guaranteed Bitrate UL =<bandwidth-value >;

    ENDIF;

ELSE

/* SDP does not give any guidance ! */
Maximum Bitrate DL/UL and Guaranteed Bitrate DL/UL per media component as specified by the UE manufacturer; 
ENDIF ;


6.5.2
SDP parameters to Authorized UMTS QoS parameters mapping in UE

If the PDP Context is activated or modified the UE should use the mapping rules in table 6.5.2.1 for all applications using SDP to derive the Maximum Authorized Bandwidth UL/DL per IP flow or bidirectional combinations of IP flows.

Table 6.5.2.1 also has a mapping rule for derivation of Maximum Authorized Traffic Class per IP flow or bidirectional combinations of IP flows which applies for session initiation and modification.

In future releases this mapping rule may change.

In the case of forking, the various forked responses may have different QoS requirements for the same IP flows of a media component. When the Authorized UMTS QoS Parameters are used by the UE, they shall be set equal to the highest values requested for the IP flows of that media component by any of the active forked responses. The UE should use the mapping rule in table 6.5.2.1 for each forked response.

Table 6.5.2.1: Rules for derivation of the Maximum Authorized Bandwidth DL/UL and
the Maximum Authorized Traffic Class per IP flow or bidirectional combination of IP flows in the UE
	Authorized UMTS QoS Parameter 
	Derivation from SDP Parameters
(see Note 4)

	Maximum Authorized Bandwidth DL (Max_BW_DL) and UL (Max_BW_UL)
(see NOTE 5)
	IF a=recvonly THEN

  IF <SDP direction> = mobile originated THEN

    Direction:= downlink;

  ELSE /* mobile terminated */

             Direction:= uplink;

  ENDIF;

ELSE /* a!= recvonly */

  IF a=sendonly THEN

    IF <SDP direction> = mobile originated THEN

      Direction: = uplink;

    ELSE /* mobile terminated */

      Direction:= downlink;

    ENDIF;

  ELSE /*sendrecv, inactive or no direction attribute*/

    Direction:=both;

  ENDIF;

ENDIF;

/* Max_BW_UL and Max_BW_DL */

IF media IP flow(s) THEN

  IF bAS=AS:<bandwidth> is present and

     ( bTIAS=TIAS:<Tibandwidth> is not present or

       is present but not supported ) THEN

    IF Direction=downlink THEN

      Max_BW_UL:= 0;

      Max_BW_DL:= bAS;

    ELSE

      IF Direction=uplink THEN

        Max_BW_UL := bAS ;

        Max_BW_DL := 0 ;

      ELSE /*Direction=both*/

        Max_BW_UL:= bAS;

        Max_BW_DL := bAS ;

      ENDIF ;

    ENDIF;
  ELSE

    IF bTIAS=TIAS:<Tibandwidth> is present and supported THEN

      bTDBW= bTIAS + transport-overhead; (NOTE 6)

      IF Direction=downlink THEN

        Max_BW_UL:= 0;

        Max_BW_DL:= bTDBW; (NOTE 6)

      ELSE

        IF Direction=uplink THEN

          Max_BW_UL:= bTDBW; (NOTE 6)

          Max_BW_DL:= 0;

        ELSE /*Direction=both*/

          Max_BW_UL:= bTDBW; (NOTE 6)

          Max_BW_DL:= bTDBW; (NOTE 6)

        ENDIF;

      ENDIF;
    ELSE /* bTIAS=TIAS:<Tibandwidth> is NOT present or is present but not 
          supported*/
      bw:= as set by the UE manufacturer;

      IF Direction=downlink THEN

        Max_BW_UL:= 0;

        Max_BW_DL:= bw;

      ELSE

        IF Direction=uplink THEN

          Max_BW_UL:= bw;

          Max_BW_DL:= 0;

        ELSE /*Direction=both*/

          Max_BW_UL:= bw;

          Max_BW_DL:= bw;

        ENDIF;

      ENDIF;
    ENDIF;
  ENDIF;

ELSE /* RTCP IP flow(s) */

  IF bRS=RS:<bandwidth> and bRR=RR:<bandwidth> is present THEN 
    Max_BW_UL:= (bRS + bRR) / 1000;
    Max_BW_DL:= (bRS + bRR) / 1000;
  ELSE
    IF bAS=AS:<bandwidth> is present and

       ( bTIAS=TIAS:<Tibandwidth> is not present or
         is present but not supported ) THEN

      IF bRS=RS:<bandwidth> is present and bRR=RR:<bandwidth> is not present

      THEN

        Max_BW_UL := MAX[0.05 * bAS, bRS / 1000] ;
        Max_BW_DL := MAX[0.05 * bAS, bRS / 1000] ;

      ENDIF;

      IF bRS=RS:<bandwidth> is not present and bRR=RR:<bandwidth> is present

      THEN

        Max_BW_UL:= MAX[0.05 * bAS, bRR / 1000];
        Max_BW_DL:= MAX[0.05 * bAS, bRR / 1000];

      ENDIF;

        IF bRS=RS:<bandwidth> and bRR=RR:<bandwidth> is not present THEN

          Max_BW_UL := 0.05 * bAS ;
          Max_BW_DL := 0.05 * bAS ;

        ENDIF;

    ELSE

      IF bTIAS=TIAS:<Tibandwidth> is present and supported THEN

        bTDBW= bTIAS + transport-overhead; (NOTE 6)

        IF bRS=RS:<bandwidth> is present and

           bRR=RR:<bandwidth> is not present THEN          Max_BW_UL:= MAX[0.05 * bTDBW, bRS]/1000;
          Max_BW_DL:= MAX[0.05 * bTDBW, bRS]/1000;

        ENDIF;

        IF bRS=RS:<bandwidth> is not present and

           bRR=RR:<bandwidth> is present THEN

          Max_BW_UL:= MAX[0.05 * bTDBW, bRR]/1000;
          Max_BW_DL:= MAX[0.05 * bTDBW, bRR]/1000;

        ENDIF;

        IF bRS=RS:<bandwidth> and

           bRR=RR:<bandwidth> is not present THEN

          Max_BW_UL:= 0.05 * bTDBW /1000;
          Max_BW_DL:= 0.05 * bTDBW /1000;

        ENDIF;

      ELSE

         Max_BW_UL:= as set by the UE manufacture;

         Max_BW_DL:= as set by the UE manufacture;

      ENDIF;

    ENDIF;
  ENDIF;

ENDIF;


	Maximum Authorized Traffic Class [MaxTrafficClass] (see NOTE 1, 2 and3)
	IF (all media IP flows of media type “audio”  or  “video” for the session are

    unidirectional and have the same direction) THEN

            MaxService:= streaming;

ELSE

            MaxService:= conversational;

ENDIF;

CASE <media> OF

         “audio”:         MaxTrafficClass:= MaxService;

         “video”:         MaxTrafficClass:= MaxService;

         “application”:   MaxTrafficClass:=conversational;

         “data”:          MaxTrafficClass:=interactive with priority 3;

         “control”:       MaxTrafficClass:=interactive with priority 1;

         /*new media type*/

         OTHERWISE:       MaxTrafficClass:=background;

END;

	NOTE 1:
The Maximum Authorized Traffic Class for a RTCP IP flow is the same as for the corresponding RTP media IP flow.

NOTE 2:
When audio or video IP flow(s) are removed from a session, the parameter MaxService shall keep the originally assigned value.

NOTE 3:
When audio or video IP flow(s) are added to a session, the UE shall derive the parameter MaxService  taking into account the already existing media IP flows within the session.

NOTE 4:
The SDP parameters are described in RFC 2327 [11].

NOTE 5:
The ‘b=RS:’ and ‘b=RR:’ SDP bandwidth modifiers are defined in RFC 3556 [13].
NOTE 6:
TIAS is defined in IETF RFC 3890 [23]. RFC 3890 clause 6.4 provides procedures for converting TIAS to transport-dependant values. This procedure relies on the presence of maxprate (also defined in RFC 3890).


The UE should per ongoing session store the Authorized UMTS QoS parameters per IP flow or bidirectional combination of IP flows.

Before activating or modifying a PDP context the UE should check that the requested Guaranteed Bitrate UL/DL (if the Traffic Class is Conversational or Streaming) or the requested Maximum Bitrate UL/DL (if the Traffic Class is Interactive or Background) does not exceed the Maximum Authorized Bandwidth UL/DL per PDP context (calculated according to the rule in table 6.5.2.2). If the requested Guaranteed Bitrate UL/DL or the requested Maximum Bitrate UL/DL exceeds the Maximum Authorized Bandwidth UL/DL per PDP context, the UE should reduce the requested Guaranteed Bitrate UL/DL or the requested Maximum Bitrate UL/DL to the Maximum Authorized Bandwidth UL/DL per PDP context. Furthermore, if the rule in table 6.5.2.1 for calculating Traffic Class per IP flow or bdirectional combination of IP flows is implemented, the UE should check that the requested UMTS QoS parameter Traffic Class does not exceed the Maximum Authorized Traffic Class per PDP context (calculated according to the rule in table 6.5.2.2). If the requested UMTS QoS parameter Traffic Class exceeds the Maximum Authorized Traffic Class per PDP context, the UE should reduce the requested UMTS QoS parameter Traffic Class to the Maximum Authorized Traffic Class per PDP context.

Table 6.5.2.2: Rules for calculating the Maximum Authorized Bandwidths
and Maximum Authorized Traffic Class per PDP Context in the UE

	Authorized UMTS QoS Parameter per PDP Context
	Calculation Rule

	Maximum Authorized Bandwidth DL and UL per PDP Context
	Maximum Authorized Bandwidth DL/UL per PDP Context is the sum of all Maximum 
Authorized Bandwidth DL/UL for all the IP flows or bidirectional combinations

of IP flows (as derived according to table 6.5.2.1) associated with that PDP

Context ;

IF Maximum Authorized Bandwidth DL/UL per PDP Context > 256 Mbps THEN
   Maximum Authorized Bandwidth DL/UL per PDP Context = 256 Mbps
   /* See  ref [4] */
END;


	Maximum Authorized Traffic Class per PDP Context
	Maximum Authorised Traffic Class per PDP Context = MAX [Maximum Authorized QoS Class per IP flow or bidirectional combination of IP flows (as derived according to table 6.5.2.1) among all the IP flows or bidirectional combinations of IP flows associated with that PDP Context] ;

(The MAX function ranks the possible Maximum Authorised Traffic Class values as follows: Conversational > Streaming > Interactive with priority 1 > Interactive with priority 2 > Interactive with priority 3 > Background)


7
PCRF addressing
7.1
General
The PCRF discovery procedures are needed where more than one PCRF is present in an operator’s network realm. Within such a deployment, an additional functional element, called DRA, is needed. PCRF discovery procedures include all the procedures that involve a DRA functional element.

Routing of Diameter messages from a network element towards the right Diameter realm in a PLMN is based on standard Diameter realm-based routing, as specified in IETF RFC 3588 [14] using the UE-NAI domain part. If PLMN is separated into multiple realms based on PDN information or IP address range (if applicable); the PDN information available in the Called-Station-Id AVP, or the UE’s Ipv4 address available in the Framed-IP-Address AVP or the UE’s Ipv6 address or prefix provided within the Framed-Ipv6-Prefix AVP may be used to assist routing PCC message to the appropriate Diameter realm.
The DRA keeps status of the assigned PCRF for a certain UE and IP-CAN session across all reference points, e.g. Gx, Gxx, S9, Rx and for unsolicited application reporting, the Sd interfaces.

The DRA shall support the functionality of a proxy agent and a redirect agent as defined in RFC 3588 [14]. The mode in which it operates (i.e. proxy or redirect) shall be based on operator’s requirements.

Diameter clients of the DRA, i.e. AF, PCEF, BBERF, RCAF and PCRF in roaming scenarios shall support all procedures required to properly interoperate with the DRA in both the proxy and redirect modes.
NOTE: The proxy mode includes two variants:

PA1:
Proxy agent based on the standard Diameter proxy agent functionality. All the messages need to go through the DRA.

PA2:
Proxy agent based on the standard Diameter proxy agent functionality. Session establishment messages always go through the DRA. Gx, Gxx and S9 session termination messages always go through the DRA. All other messages bypass the DRA.
7.2
DRA Definition
The DRA (Diameter Routing Agent) is a functional element that ensures that all Diameter sessions established over the Gx, S9, Gxx, Rx interfaces and for unsolicited application reporting, the Sd interface for a certain IP-CAN session reach the same PCRF when multiple and separately addressable PCRFs have been deployed in a Diameter realm. The DRA also ensures that the NRR commands over Np interface reach the same PCRF for a certain IP-CAN session. The DRA is not required in a network that deploys a single PCRF per Diameter realm.
7.3
DRA Procedures
7.3.1
General

A DRA implemented as a Diameter Redirect Agent or a Diameter Proxy Agent shall be compliant to IETF RFC 3588 [14], except when noted otherwise in this document.

7.3.2
DRA Information Storage

The DRA shall maintain PCRF routing information per IP-CAN session or per UE-NAI, depending on the operator’s configuration.
The DRA shall select the same PCRF for all the Diameter sessions established for the same UE in case 2a.
As there’s only one S9 session per UE, the V-DRA/H-DRA shall select the same V-PCRF/H-PCRF respectively for the same UE in the roaming case.
The DRA has information about the user identity (UE NAI), the UE Ipv4 address and/or Ipv6 prefix, the APN (if available), the PCEF identity (if available) and the selected PCRF identity for a certain IP-CAN Session.
NOTE 1:
The DRA derives the PCEF identity from the Origin-Host AVP of the CCR command received from the PCEF.
The DRA finds the correct PCRF by matching the user identity (if available), IPv4 address or IPv6 address/prefix (if available) and APN (if available) received in the message from the BBERF/PCEF/AF/TDF/ RCAF/V-PCRF with the corresponding information stored the DRA.

NOTE 2:
If the DRA does not use the IP address to find the PCRF and the user identity in the IP‑CAN and the application level identity for the user are of different kinds (e.g. user identity is the IP-CAN is IMSI and application level identity for the user is SIPURI), the DRA needs to maintain, or have access to, the mapping between the identities. Such mapping is not subject to specification within this TS.

NOTE 3:
An IPv6 address provided over Rx matches an IPv6 prefix stored in the DRA binding if the IPv6 address belongs to the IPv6 (sub-)network prefix.
When matching the APNs to each other, the DRA shall apply the procedures in Annex I.

For the PCRF selection over the Rx reference point, the DRA may additionally match the IP domain Id received in the message from the AF with the PCEF identity stored in the DRA to find the correct PCRF.

NOTE 4:
In order to correlate the PCEF Identity and the domain identity, the DRA uses configured mapping between those identities.
For the PCRF selection over the Rx reference point, when the APN related to the IP-CAN session is dedicated for the purpose of offering services to remote UEs via a ProSe UE-to-network relay UE, the DRA shall use the UE IPv6 prefix alone to find the correct PCRF.

The PCRF routing information stored for an IP-CAN session in the DRA shall be removed after the IP-CAN session is terminated. In case of DRA change (e.g. inter-operator handover), the information about the IP-CAN session stored in the old DRA shall be removed.

The PCRF routing information stored per UE in the DRA may be removed when no more IP-CAN and gateway control sessions are active for the UE.

7.3.3
Capabilities Exchange
In addition to the capabilities exchange procedures defined in IETF RFC 3588 [14], the Redirect DRA and Proxy DRA shall advertise the specific applications it supports (e.g., Gx, Gxx, Rx, Np,S9 and for unsolicited application reporting, Sd) by including the value of the application identifier in the Auth-Application-Id AVP and the value of the 3GPP (10415) in the Vendor-Id AVP of the Vendor-Specific-Application-Id AVP contained in the Capabilities‑Exchange-Request and Capabilities-Exchange-Answer commands.
7.3.4
Redirect DRA
7.3.4.1
Redirecting Diameter Requests

A DRA implemented as a Diameter redirect agent shall redirect the received Diameter request message by carrying out the procedures defined in clause 6.1.7 of IETF RFC 3588 [14]. The Client shall use the value within the Redirect-Host AVP of the redirect response in order to obtain the PCRF identity. The DRA may provide the Redirect-Host-Usage AVP in the redirect response to provide a hint to the Client about how the cached route table entry created from the Redirect-Host AVP is to be used as described in clause 6.13 of IETF RFC 3588 [14].

The two most revelant redirect host usage scenarios for PCC from IETF RFC 3588 [14] are:

-
If the PCRF routing information is per UE-NAI, the DRA shall set the Redirect-Host-Usage AVP to ALL_USER. The DRA client may contact the DRA on IP-CAN session termination.

-
If the PCRF routing information is per IP-CAN session, the DRA shall set the Redirect-Host-Usage AVP to ALL_SESSION. The DRA client shall contact the DRA on IP-CAN session termination.

The DRA may also provide the Redirect-Max-Cache-Time AVP in the redirect response to indicate to the Client the lifetime of the cached route table entry created from the Redirect-Host and Redirect-Host-Usage AVP values as described in clause 6.14 of IETF RFC 3588 [14].
If the DRA is maintaining PCRF routing information per IP-CAN session, the DRA shall be aware of Gx and Gxx Diameter termination requests as defined in 3GPP TS 29.212 [9] in order to detect whether release of DRA bindings is required. Otherwise the DRA clients shall use cached route table entry created from the Redirect-Host, Redirect-Host-Usage and Redirect-Max-Cache-Time AVPs to determine whether DRA interaction is required.
The DRA shall be aware of IP-CAN Session modification requests over Gx which is to update the Ipv4 address of the UE by the PCEF.
If the client is the AF, the DRA (redirect) does not need to maintain Diameter sessions and Diameter Base redirect procedures are applicable. Therefore, an AF should not send an AF session termination request to the DRA.
7.3.4.2
DRA binding removal

If the DRA binding is per IP-CAN session and the IP-CAN session is terminated or if the DRA binding is per UE and the last IP-CAN session is terminated (eg. From an indication by the BBERF/PCEF) the Redirect DRA shall remove the associated DRA binding information and responds with a Diameter redirect answer message.

7.3.5
Proxy DRA

The DRA shall support the functionality of a Diameter proxy agent as defined in RFC 3588 [14].

When the DRA receives a request from a client, it shall check whether it already has selected a PCRF for the UE or the UE’s IP-CAN session; if it does have a PCRF already selected for that UE or UE’s IP-CAN session, it shall proxy the request to the corresponding PCRF. If the request is an IP-CAN session termination or gateway control session termination, the DRA shall check whether PCRF routing information shall be removed as specified in clause 7.3.3. If the DRA does not have a PCRF already selected, it shall follow one of the procedures below:
-
If the request is an IP-CAN session establishment or gateway control session establishment, it shall select a PCRF to handle all sessions for that UE or UE’s IP-CAN session. It shall then proxy the request to the selected PCRF.

-
Otherwise, if the request is not an IP-CAN session establishment or gateway control session establishment, it shall reject the request by returning a DIAMETER_UNABLE_TO_ COMPLY error code.

If a DRA is deployed in a PCRF’s realm, clients of the DRA shall send the first request of a session to the DRA handling the PCRF’s realm. Clients of the DRA shall as well send IP-CAN session termination and gateway control termination requests to the DRA. A client of the DRA shall be capable of sending every message of a session to the DRA. A client of the DRA may be configured to bypass the DRA on session modification messages and AF session termination messages by sending these types of messages directly to the PCRF.

7.3.6
PCRF selection by BBERF/PCEF (non-roaming case)

The PCEF (e.g. P-GW) or BBERF (e.g. Non-3GPP Access, S-GW) shall provide the DRA of the PCRF realm with identity parameters upon the first interaction between the access entity and the PCRF realm.
If the redirect agent is used for DRA, the DRA shall use the redirecting requests procedure as specified in IETF RFC 3588 [14], and include the PCRF identity in the Redirect-Host AVP in the Diameter reply sent to the PCEF or the BBERF.

If proxy agent is used for DRA, the DRA should use the proxy procedure as specified in IETF RFC 3588 [14]. For PA2 solution (described in clause 7.1), only session establishment, session modification with the UE’s Ipv4 address updated and session termination messages shall be sent through the DRA.
The identity parameters from the PCEF or BBERF may comprise the UE’s Ipv4 address in the Framed-IP-Address AVP and/or the UE’s Ipv6 prefix in the Framed-Ipv6-Prefix AVP, PDN information in the Called-Station-Id AVP and user identity in the Subscription-Id AVP.
7.3.7
PCRF selection by AF

If the AF has the realm identification (i.e. FQDN from a UE NAI) and is located in the HPLMN, the AF sends the user identity in the Subscription-Id AVP and PDN information (i.e. APN) if available in the Called-Station-Id AVP in a Diameter request to the DRA which acts as a Diameter agent.
For an AF serving mission critical communication services (e.g. MCPTT), the AF shall derive the realm of the PCRF based on the IP address and, based on the PLMN ID, construct the Diameter destination realm and use that destination realm in the initial request to address the DRA.

NOTE 1:
An AF serving mission critical communication services as defined in 3GPP TS 23.179 [57] can be independent from a PLMN and is not guaranteed to have any realm identification.
If the AF does not have proper knowledge about the user identity and the AF is located in the HPLMN, the AF may use pre-configured information to find the DRA.
Editor’s Note: It is FFS how the AF (e.g. a third party or non-IMS application server) finds the DRA if it does not have the proper knowledge about the user identity. It is FFS whether a pre-configured destination realm will suffice in these cases.

The AF shall provide the DRA of the PCRF realm with identity parameters upon the first interaction between the AF and the PCRF realm.
If redirect agent is used for DRA, the DRA shall use the redirecting requests procedure as specified in IETF RFC 3588 [14], and include the PCRF identity in the Redirect-Host AVP in the Diameter reply sent to the AF.

If proxy agent is used for DRA, the DRA should use the proxy procedure as specified in IETF RFC 3588 [14]. For PA2 solution (described in clause 7.1), only AF session establishment messages shall be sent through the DRA.
The parameters from the AF may comprise the UE IP address in either the Framed-IP-Address AVP or the Framed-Ipv6-Prefix AVP, PDN information in the Called-Station-Id AVP, user identity in the Subscription-Id AVP and domain Identity in the IP-Domain-Id AVP (3GPP TS 23.203 [2]).
NOTE 2:
In case the user identity in the IP‑CAN and the application level identity for the user are of different kinds (e.g. user identity is the IP-CAN is IMSI and application level identity for the user is SIPURI), the DRA needs to maintain, or have access to, the mapping between the identities. Such mapping is not subject to specification within this TS.
7.3.8
PCRF selection in a roaming scenario

In the roaming case, a V-DRA is needed in the visited PLMN when there are more than one PCRFs per realm. The V-DRA will ensure that all the related Diameter sessions for a UE are handled by the same V-PCRF.

The BBERF in the visited access and home routed cases, the PCEF in the case of visited access and the AF when located in the visited PLMN may use pre-configured information (e.g. based on PDN) to find the V-DRA, and then find theV-PCRF. Other possible options are Dynamic peer discovery, or DNS-based.

The V-PCRF can find the H-DRA based on the UE NAI, and then find the H-PCRF by the H-DRA.
The V-PCRF shall provide the H-DRA of the H-PCRF realm with identity parameters upon the first interaction between the V-PCRF and the H-PCRF realm.
If redirect agent is used for H-DRA, the H-DRA shall use the redirecting requests procedure as specified in IETF RFC 3588 [14], and include the H-PCRF identity in the Redirect-Host AVP in the Diameter reply sent to the V-PCRF.

If proxy agent is used for H-DRA, the H-DRA should use the proxy procedure as specified in IETF RFC 3588 [14]. For PA2 solution (described in clause 7.1), only session establishment, session modification with the UE’s Ipv4 address updated and termination messages shall be sent through the H-DRA.
The identity parameters from the V-PCRF may comprise the same parameters sent by the PCEF or the BBERF to the V-PCRF, i.e. the user identity (UE NAI), APN, the UE’s Ipv4 address and/or Ipv6 prefix (3GPP TS 23.203 [2]).
If redirect agent or PA2 is used for H-DRA, and the V-PCRF receives establishment message from the AF in the VPLMN, the V-PCRF may send the message to the H-PCRF directly (e.g. based on the stored information provided by H-DRA during the IP-CAN session establishment).
7.3.9
PCRF selection by TDF for unsolicited application reporting
The TDF uses pre-configured information to find the DRA.

The TDF shall provide the DRA of the PCRF realm with identity parameters upon the first interaction between the TDF and the PCRF realm.
If redirect agent is used for DRA, the DRA shall use the redirecting requests procedure as specified in IETF RFC 3588 [14], and include the PCRF identity in the Redirect-Host AVP in the Diameter reply sent to the TDF.

If proxy agent is used for DRA, the DRA should use the proxy procedure as specified in IETF RFC 3588 [14].

The parameters from the TDF may comprise the UE IP address in either the Framed-IP-Address AVP or the Framed-Ipv6-Prefix AVP and PDN information in the Called-Station-ID AVP.
NOTE:
The TDF located in the HPLMN finds the H-PCRF for the roaming UE with home routed access case. The TDF located in the VPLMN finds the V-PCRF for the roaming UE with visited access case.

7.3.10
PCRF selection by RCAF

For initial Non-Aggregated RUCI reporting, the RCAF shall provide the identity parameters to the DRA of the PCRF realm.
The identity parameters from the RCAF may comprise PDN information in the Called-Station-Id AVP and user identity in the Subscription-Id AVP.

In the roaming with home-routed access case, the RCAF can find the H-DRA based on the IMSI, and then find the H-PCRF by the H-DRA.The RCAF shall provide the H-DRA of the H-PCRF realm with identity parameters upon the first interaction between the RCAF and the H-PCRF realm.
In the roaming with visited case, the RCAF shall provide the V-DRA of the V-PCRF realm with identity parameters upon the first interaction between the RCAF and the V-PCRF realm.
If the redirect agent is used for DRA, the DRA shall use the redirecting requests procedure as specified in IETF RFC 3588 [14], and include the PCRF identity in the Redirect-Host AVP in the Diameter reply sent to the RCAF.

If proxy agent is used for DRA, the DRA should use the proxy procedure as specified in IETF RFC 3588 [14].
For aggregated RUCI reporting, the RCAF shall include the PCRF identity within the Destination-Host AVP for direct PCRF selection. The PCRF identity shall be responded by the PCRF within the PCRF-Address AVP in the initial non-aggregated RUCI reporting, for identifying the same destination PCRF in the subsequent aggregated RUCI reporting.
7.4
DRA flows

7.4.1
Proxy DRA

7.4.1.1
Establishment of Diameter Sessions

7.4.1.1.1
Non-roaming cases

Establishment of Diameter sessions may occur at any of the following cases:

-
Gateway control establishment

-
IP-CAN session establishment

-
AF session establishment
-
For unsolicited application reporting, TDF session establishment

[image: image59.wmf] 

PA2

 

1. 

 

External trigger

 

Legend

:

 

 

       

 

Mandatory

 

               

 

 

Conditional 

 

PCRF

-

1

 

Client 

 

(AF/BBERF/PCEF

/TDF

)

 

 

2. 

 

Rx/Gx/Gxx

/Sd

 

Diameter Request

 

4. Proxy Rx/Gx/Gxx

/Sd

 

Diameter Request

 

3.  DRA binding 

crea

tion/retrieval

 

5. Rx/Gx/Gxx

/Sd

 

Diameter Answer

 

DRA

 

(

proxy

)

 

PCRF

-

2

 

6. Proxy Rx/Gx/Gxx

/Sd

 

Diameter Answer

 

7.  PCRF address storage

 


Figure 7.4.1.1.1.1: Establishment of Diameter sessions using DRA (proxy)

1.
A Client receives an external trigger (e.g. IP-CAN session establishment request) that requires the establishment of a Diameter session with a PCRF.

2.
A Diameter Request (e.g. a Diameter CCR sent by PGW to indicate establishment of an IP-CAN session as defined in clauses 4.5.1, 4a.5.1 of 3GPP TS 29.212 [9]) is sent by the Client and received by a DRA (proxy).
3.
The DRA (proxy) stores the user information (e.g. UE-NAI) and checks whether an active DRA binding exists. If not, the DRA creates a dynamic DRA binding (assignment of a PCRF node per UE or per IP-CAN session).
NOTE 1:
When the AF establishes an Rx session or TDF establishes a TDF session with the DRA, there is already a DRA binding active.
4.
The DRA (proxy) proxies the Diameter Request to the target PCRF. The proxied Diameter Request maintains the same Session-Id AVP value.

5.
PCRF-1 returns a Diameter Answer as defined in clauses 4.5, 4a.5 of 3GPP TS 29.212 [9] to the DRA (proxy).

6.
DRA (proxy) proxies the Diameter Answer to the Client. The proxied Diameter Answer maintains the same Session-Id AVP value.

7.
If PA2 option is implemented, the Client uses the Origin-Host AVP value providedin the Diameter Answer of step 6. This value is the identity of the target PCRF. The client populates the Destination-Host AVP with this address and sends any subsequent Diameter messages directly to this PCRF bypassing the DRA (proxy).
NOTE 2:
Figure 7.4.1.1.1.1 is also applicable when the AF/BBERF/PCEF/TDF in the VPLMN contacts the V-DRA to locate the V-PCRF.
7.4.1.1.2
Roaming cases

Establishment of Diameter sessions may occur at any of the following cases:

-
V-PCRF initiates S9 Diameter session to H-PCRF

-
V-PCRF proxies Rx Diameter session to H-PCRF


[image: image60.wmf] 

1. 

 

Extern

al trigger

 

H

-

PCRF

-

1

 

V

-

PCRF

 

2. 

 

Rx/

S9

 Diameter 

Request

 

4

. Proxy 

Rx/

S9

 Diameter 

R

equest

 

3.  DRA binding

 

creation

/retrieval

 

5

. 

Rx/

S9

 Diameter 

Answer

 

H

-

DRA

 

(

proxy

)

 

H

-

PCRF

-

2

 

6

. Proxy 

Rx/

S9

 Diameter 

Answer

 

7.  PCRF address storage

 

PA2

 

Legend

:

 

 

       

 

Mandatory

 

               

 

 

Conditional 

 


Figure 7.4.1.1.2.1: Establishment of Diameter sessions using DRA (proxy) – Roaming case

1.
V-PCRF receives a trigger to establish a Diameter session to H-PCRF (e.g. S9 session establishment request).

2.
A Diameter Request involving either the Rx or S9 protocol is sent by the V-PCRF and received by a H-DRA (proxy) in the home PLMN.
3.
The H-DRA (proxy) stores the user information (e.g. UE-NAI) and checks whether an active DRA binding exists. If not, the H-DRA creates a dynamic DRA binding (assignment of a PCRF node per UE or per IP-CAN session).
4.
The H-DRA (proxy) proxies the Diameter Request to the target PCRF in the home PLMN. The proxied Diameter Request maintains the same Session-Id AVP value.

5.
H-PCRF-1 returns a Diameter Answer to the H-DRA (proxy).

6.
H-DRA (proxy) proxies the Diameter Answer to the V-PCRF. The proxied Diameter Answer maintains the same Session-Id AVP value.

7.
If PA2 option is implemented, the V-PCRF uses the Origin-Host AVP value provided in the Diameter Answer of step 6. This value is the identity of the target H-PCRF. The V-PCRF populates the Destination-Host AVP with this address and sends any subsequent Diameter messages directly to this H-PCRF bypassing the H-DRA.
7.4.1.2
Modification of Diameter Sessions

7.4.1.2.1
Non-roaming cases

7.4.1.2.1.1
Client-initiated

Modification of Diameter sessions may occur in any of the following cases:

-
Gateway control session modification

-
IP-CAN session modification

-
AF session modification
-
For unsolicited application reporting, TDF session modification
If PA1 option is implemented, only steps 2, 3, 4, 5, 6 are carried out. If PA2 option is implemented, only steps 2a, 5a are carried out.


[image: image61.wmf] 

1. 

 

Exter

nal trigger

 

Legend

:

 

 

       

 

Mandatory

 

               

 

 

Conditional 

 

PCRF

-

1

 

Client

 

(

AF/

BBERF/PCEF

/TDF

)

 

 

2. 

 

Sub

-

sequent 

Rx/Gx/Gxx/Sd 

Diameter Request

 

4. Proxy Rx/Gx/Gxx

/Sd

 

Diameter R

equest

 

3.  

Verify 

DRA 

Binding 

Bin

binding

pdate

 

5. Rx/Gx/Gxx

/Sd

 

Diameter Answer

 

DRA

 

(

proxy

)

 

PCRF

-

2

 

6. P

roxy Rx/Gx/Gxx

/Sd

 

Diameter Answer

 

2a. 

 

Sub

-

sequent 

Rx/Gx/Gxx

/Sd

 

Diameter Request

 

5a. Rx/Gx/Gxx

/Sd

 

Diameter Answer

 

PA2

 

 

PA2

 

 


Figure 7.4.1.2.1.1.1: Modification of Diameter sessions through DRA (proxy)- AF/BBERF/PCEF/TDF interaction
1.
A Client receives an external trigger (e.g. IP-CAN session modification) that requires a subsequent Diameter message to be sent to the PCRF.
2.
A subsequent Diameter Request (e.g. a Diameter CCR sent by PGW to indicate modification of an IP-CAN session) as defined in clauses 4.5.1, 4a.5.1 of 3GPP TS 29.212 [9] or clause 4.4 of 3GPP TS 29.214 [10]) is sent by the Client and received by the DRA (proxy).
2a
If PA2 option is implemented, based on Client configuration and operator policy, the Client communicates directly to the PCRF, bypassing the DRA (proxy), by using the PCRF identity obtained through the Origin-Host AVP (see step 7 in clause 5.2.5.7.1.1). The Client uses the same active Session-Id AVP value on the Diameter Request sent to the PCRF. In such a case steps 2, 3, 4, 5, 6 are not carried out.
3.
After receiving a Diameter Request (Step 2), the DRA (proxy) verifies that there is an active DRA binding for the session identified in the request.
4.
The DRA (proxy) proxies the Diameter Request to the target PCRF.

5.
PCRF-1 returns a Diameter Answer as defined in clauses 4.5, 4a.5 of 3GPP TS 29.212 [9] or clause 4.4 of 3GPP TS 29.214 [10]) to the DRA (proxy).

5a
Upon receiving a Diameter Request (Step 2a), PCRF-1 returns a Diameter Answer directly to the Client, bypassing the DRA (proxy).

6.
DRA (proxy) proxies the Diameter Answer to the Client.
NOTE:
Figure 7.4.1.2.1.1.1 is also applicable when the AF/BBERF/PCEF/TDF in the VPLMN modifies the Diameter session through the V-DRA.
7.4.1.2.1.2
PCRF-initiated

Modification of Diameter sessions occur on PCRF initiated session modifications towards the clients (AF/BBERF/PCEF).

If PA1 option is implemented, only steps 2, 3, 4, 5, 6 are carried out. If PA2 option is implemented, only steps 2a, 5a are carried out.

[image: image62.wmf] 

Legend

:

 

 

       

 

Mandatory

 

               

 

 

Conditional 

 

PCRF

-

1

 

Client

 

(AF/BBERF/PCEF

)

 

 

2. 

 

PCRF initiated 

Rx/Gx/Gxx Diameter 

Request

 

4. Proxy Rx/Gx/Gxx 

Diameter Request

 

3.  Verify DRA Binding 

Binbindingpdate

 

5. Rx/Gx/Gxx Diameter 

Answer

 

DRA

 

(

proxy

)

 

PCRF

-

2

 

6. P

roxy Rx/Gx/Gxx 

Diameter Answer

 

2a. 

 

PCRF initiated 

Rx/Gx/Gxx Diameter 

Request

 

5a. Rx/Gx/Gxx Diameter 

Answer

 

PA2

 

 

PA2

 

 

1.  Trigger

 


Figure 7.4.1.2.1.2.1: Modification of Diameter sessions through DRA (proxy)- PCRF interaction

1.
PCRF receives an internal or external trigger that requires a Diameter message to be sent to the clients (either AF, BBERF, PCEF)

2.
A PCRF-initiated Diameter Request (e.g. a Diameter RAR request sent to the PGW) is sent to the Clients and received by the DRA (proxy).
2a
If PA2 option is implemented, the PCRF communicates directly to the client, bypassing the DRA (proxy). In such a case steps 2, 3, 4, 5, 6 are not carried out.
3.
After receiving a Diameter Request (Step 2), the DRA (proxy) verifies that there is an active DRA binding for the session identified in the request.
4.
The DRA (proxy) proxies the Diameter Request to the Client. The proxied Diameter Request maintains the same Session-Id AVP value.

5.
Clients returns a Diameter Answer as defined in clauses 4.5, 4a.5 of 3GPP TS 29.212 [9] or clause 4.4 of 3GPP TS 29.214 [10]) to the DRA (proxy).

5a
Upon receiving a Diameter Request (Step 2a), Client returns a Diameter Answer directly to the PCRF, bypassing the DRA (proxy).

6.
DRA (proxy) proxies the Diameter Answer to the PCRF.
NOTE:
Figure 7.4.1.2.1.2.1 is also applicable when the V-PCRF modifies the Diameter session through the V-DRA.
7.4.1.2.2
Roaming cases

7.4.1.2.2.1
V-PCRF initiated

In roaming scenarios modification of Diameter sessions may occur at any of the following cases:

(-
V-PCRF S9 Diameter session modification to H-PCRF

-
V-PCRF proxies Rx Diameter session modification to H-PCRF

If PA1 option is implemented, only steps 2, 3, 4, 5, 6 are carried out. If PA2 option is implemented, only steps 2a, 5a are carried out.

[image: image63.wmf] 

Legend

:

 

 

       

 

Mandatory

 

               

 

 

Conditional 

 

H

-

PCRF

-

1

 

V

-

PCRF

 

2. 

 

Rx/S9 

Diameter 

session update

 

4. Proxy Rx/

S9

 Diameter 

session update

 

3.  Ve

rify DRA Binding 

Binbindingpdate

 

5. Rx/

S9

 Diameter 

Answer

 

H

-

DRA

 

(

proxy

)

 

H

-

PCRF

-

2

 

6. P

roxy Rx/

S9

 Diameter 

Answer

 

2a. 

 

Rx/S9 Diameter 

session update

 

5a. Rx/

S9

 Diameter 

Answer

 

PA2

 

 

PA2

 

 

1.  Trigger

 


Figure 7.4.1.2.2.1.1: Modification of Diameter sessions through DRA (proxy) on roaming scenarios – V-PCRF initiated

1.
V-PCRF receives an internal or external trigger that requires a Diameter message to be sent to H-PCRF over the S9 reference point

2.
V-PCRF sends a Diameter session update (e.g. an S9 session modification request) over the S9 reference point that is received by the DRA (proxy) in the home PLMN.
2a
If PA2 option is implemented, the V-PCRF communicates directly to the H-PCRF, bypassing the H-DRA (proxy). In such a case steps 2, 3, 4, 5, 6 are not carried out.
3.
After receiving a Diameter Request (Step 2), the H-DRA (proxy) verifies that there is an active DRA binding for the session identified in the request.
4.
The H-DRA (proxy) proxies the Diameter Request to the H-PCRF. The proxied Diameter Request maintains the same Session-Id AVP value.

5.
H-PCRF returns a Diameter Answer to the H-DRA (proxy) in the home PLMN.

5a
Upon receiving a Diameter Request (Step 2a), Client returns a Diameter Answer directly to the PCRF, bypassing the H-DRA (proxy).

6.
H-DRA (proxy) proxies the Diameter Answer to the PCRF.
7.4.1.2.2.2
H-PCRF initiated

In roaming scenarios modification of Diameter sessions may occur at any of the following cases:

-
H-PCRF S9 Diameter session modification to V-PCRF

If PA1 option is implemented, only steps 2, 3, 4, 5, 6 are carried out. If PA2 option is implemented, only steps 2a, 5a are carried out.


[image: image64.wmf] 

Legend

:

 

 

       

 

Mandatory

 

               

 

 

Conditional 

 

H

-

PCRF

-

1

 

V

-

PCRF

 

2. 

 

S9

 Diameter 

Request

 

4. 

Proxy S9

 Diameter 

Request

 

3.  Verify DRA Binding 

Binbin

dingpdate

 

5. 

S9

Diameter Answer

 

H

-

DRA

 

(

proxy

)

 

H

-

PCRF

-

2

 

6. P

roxy 

S9

 Diameter 

Answer

 

2a. 

 

S9 Diameter 

Request

 

5a. 

S9

 Diameter Answer

 

PA2

 

 

PA2

 

 

1.  Trigger

 


Figure 7.4.1.2.2.2.1: Modification of Diameter sessions through DRA (proxy) on roaming scenarios – H-PCRF initiated

1.
H-PCRF receives an internal or external trigger that requires a Diameter message to be sent to V-PCRF over the S9 reference point.
2.
H-PCRF sends a Diameter session update (e.g. an S9 session modification request) over the S9 reference point that is received by the H-DRA (proxy) in the home PLMN.

2a
If PA2 option is implemented, the H-PCRF communicates directly to the V-PCRF, bypassing the H-DRA (proxy). In such a case steps 2, 3, 4, 5, 6 are not carried out.

3.
After receiving a Diameter Request (Step 2), the H-DRA (proxy) verifies that there is an active DRA binding for the session identified in the request.
4.
The H-DRA (proxy) proxies the Diameter Request to the V-PCRF. The proxied Diameter Request maintains the same Session-Id AVP value.

5.
V-PCRF returns a Diameter Answer to the H-DRA (proxy) in the home PLMN.

5a
Upon receiving a Diameter Request (Step 2a), V-PCRF returns a Diameter Answer directly to the H-PCRF, bypassing the H-DRA (proxy).

6.
H-DRA (proxy) proxies the Diameter Answer to the H-PCRF.
7.4.1.3
Termination of Diameter Sessions
7.4.1.3.1
Non-roaming cases

The procedures required are identical for both PA1 and PA2 options

Termination of Diameter sessions occur at the following cases:

-
Gateway control session termination
-
IP-CAN session termination

-
AF session termination
-
For unsolicited application reporting, TDF session termination

[image: image65.wmf] 

1. 

 

Exter

nal trigger

 

Legend

:

 

 

       

 

Mandatory

 

               

 

 

Conditional 

 

PCRF

-

1

 

Client

 

(

AF/

BBERF/PCEF

/TDF

)

 

 

2

. 

 

Rx/

Gx/Gxx

/S

d

 

Diameter

 

Termination R

equest

 

4

. Proxy 

Rx/

Gx/Gxx

/Sd

 

Diameter

 

T

ermination 

R

equest

 

3

.  

Verify

 

DRA binding

 

5

. 

Rx/

Gx/Gxx

/Sd

 

Dia

meter Answer

 

DRA

 

(

proxy

)

 

PCRF

-

2

 

7

. Proxy 

Rx/

Gx/Gxx

/Sd

 

Diameter Answer

 

6.  Remove DRA binding

 


Figure 7.4.1.3.1.1: Termination of Diameter sessions through DRA (proxy)

1.
A Client receives an external trigger (e.g. an IP-CAN session termination is initiated by the UE or PCRF) that requires the sending of a Diameter Termination Request.

2.
A Diameter Termination Request (e.g., a Diameter CCR sent by PGW to indicate termination of an IP-CAN session) as defined in clauses 4.5, 4a.5 of 3GPP TS 29.212 [9]) is sent by the Client to the DRA (proxy). The message uses the same Session-Id AVP value of the active Diameter session established between the Client and PCRF-1.
3.
The DRA (proxy) verifies that there is an active DRA binding for the IP-CAN session based on the Session-Id AVP in the request.

4.
The DRA (proxy) proxies the Diameter Termination Request to the target PCRF. The proxied Diameter Request maintains the same Session-Id AVP value.

5.
PCRF-1 acknowledges termination of the session. PCRF-1 sends a Diameter Answer, (e.g., as defined in clauses 4.5, 4a.5 of 3GPP TS 29.212 [9]) to DRA (proxy).

6.
The DRA marks the Diameter session terminated. If the DRA binding is per IP-CAN session and all the Diameter sessions (i.e. the Gx session or the Gxx session) of the IP-CAN session are terminated or if the DRA binding is per UE and all the Diameter sessions (i.e. the Gx session or the Gxx session) of that UE are terminated the DRA (proxy) removes the DRA binding.

7.
DRA (proxy) proxies the Diameter Answer to the Client. The proxied Diameter Answer maintains the same Session-Id AVP value.
NOTE 1:
Figure 7.4.1.3.1.1 is also applicable when the AF/BBERF/PCEF/TDF in the VPLMN terminates the Diameter sessions through the V-DRA.
NOTE 2:
AF/TDF is not required to send Diameter session termination request to DRA (PA2).
7.4.1.3.2
Roaming cases

In roaming cases (over S9 reference point) termination of Diameter sessions occur at the following cases:
-
S9 session termination
-
AF session termination

[image: image66.wmf] 

1. 

 

Exter

nal trigger

 

Legend

:

 

 

       

 

Mandatory

 

               

 

 

Conditional 

 

H

-

PCRF

-

1

 

V

-

PCRF

 

2

. 

 

S9

/Rx

 Diameter

 

Termination R

equest

 

4

. Proxy 

S9

/Rx

 Diameter

 

T

ermination 

R

equest

 

3

.  

Verify

 

DRA binding

 

5

. 

S9

/Rx

 Dia

meter 

Answer

 

H

-

DRA

 

(

proxy

)

 

H

-

PCRF

-

2

 

7

. Proxy 

S9

/Rx

 Diameter 

Answer

 

6.  Remove DRA binding

 


Figure 7.4.1.3.2.1: Termination of Diameter sessions through H-DRA (proxy) – Roaming cases

1.
The V-PCRF receives an external trigger (e.g., session termination request from the BBERF or the PCEF) that requires the sending of a Diameter Termination Request.

2.
A Diameter Termination Request (e.g., an S9 termination request) is sent by the V-PCRF and received by the H-DRA (proxy) in the home PLMN. The message uses the same Session-Id AVP value of the active Diameter session established between V-PCRF and H-PCRF-1.
3.
The H-DRA (proxy) verifies that there is an active DRA binding for the IP-CAN session based on the Session-Id AVP in the request.

4.
The H-DRA (proxy) proxies the Diameter Termination Request to the target H-PCRF-1. The proxied Diameter Request maintains the same Session-Id AVP value.

5.
H-PCRF-1 acknowledges termination of the session. H-PCRF-1 sends a Diameter Answer to H-DRA (proxy) in the home PLMN.

6.
The H-DRA marks the Diameter session terminated. If all the Diameter sessions (i.e. the S9 session, the Gxx session, and the Gx session) of that UE are terminated the H-DRA (proxy) removes the DRA binding.

7.
H-DRA (proxy) proxies the Diameter Answer to the V-PCRF. The proxied Diameter Answer maintains the same Session-Id AVP value.
NOTE:
V-PCRF does not need to send Rx Diameter termination messages to proxy H-DRA (PA2 option) since Rx Diameter termination messages do not affect the DRA binding.
7.4.2
Redirect DRA

7.4.2.1
Establishment of Diameter Sessions

7.4.2.1.1
Non-roaming cases
Establishment of Diameter sessions may occur at the following cases:

-
Gateway control session establishment

-
IP-CAN session establishment

-
AF session establishment
-
For unsolicited application reporting, TDF session establishment
The DRA client (AF/BBERF/PCEF/TDF) shall follow the procedure below if an appropriate cached route table entry created from previous DRA (redirect) interactions does not exist. Cached route table entries are created from the Redirect-Host, Redirect-Host-Usage and Redirect-Max-Cache-Time AVPs as described in sections 6.12, 6.13 and 6.14 of IETF RFC 3588 [14].

[image: image67.wmf] 

1. 

 

Exter

nal trigger

 

Legend

:

 

 

       

 

Mandatory

 

               

 

 

Conditional 

 

PCRF

-

1

 

Client

 

 

(

AF/

BBERF/PCEF

/TDF

)

 

 

2. 

 

Rx/

Gx/Gxx

/Sd

 

Diameter

  

Establishment

 

Request

 

4. Diameter 

Answer 

(

redirect

)

 

 

3.  

DRA binding

 

creation/retrieval

 

6

. 

Rx/

Gx/Gxx

/Sd

 

Diameter Answer

 

DRA

 

(

redirect

)

 

PCRF

-

2

 

5. 

Rx/

Gx/Gxx

/Sd

 

Diameter

 

Establishment

 

Re

quest 

 


Figure 7.4.2.1.1.1: Establishment of Diameter session through DRA (redirect)

1.
A Client receives an external trigger (e.g., IP-CAN session establishment request) that requires the establishment of a Diameter session with a PCRF.

2.
A Diameter Establishment request (e.g., a Diameter CCR sent by PGW to indicate establishment of an IP-CAN session as defined in clauses 4.5.1, 4a.5.1 of 3GPP TS 29.212 [9]) with user information (e.g., UE-NAI) is sent by the Client and received by the DRA (redirect).
3.
The DRA (redirect) stores the user information (e.g., UE-NAI) and checks whether an active DRA binding exists. If not the DRA creates a dynamic DRA binding (assignment of a PCRF node per UE or per IP-CAN session); if the DRA (redirect) find there has been a DRA binding for the user, the DRA shall select the PCRF from the binding for the client.
4.
The DRA (redirect) sends a Diameter Answer indicating redirection as defined in IETF RFC 3588 [14]. The target PCRF identity is included in the Redirect-Host AVP.

5.
The Client re-sends the Diameter Establishment Request of step 2 to the target PCRF.

6.
PCRF-1 returns a Diameter Answer, as defined in clauses 4.5, 4a.5 of 3GPP TS 29.212 [9], to the Client.
NOTE:
Figure 7.4.2.1.1.1 is also applicable when the AF/BBERF/PCEF/TDF in the VPLMN contacts the V-DRA to locate the V-PCRF.
7.4.2.1.2
Roaming cases

Establishment of Diameter sessions may occur at the following cases:

-
S9 session 
vailability

-
AF session establishment
The DRA client (AF/BBERF/PCEF) shall follow the procedure below if an appropriate cached route table entry created from previous DRA (redirect) interactions does not exist. Cached route table entries are created from the Redirect-Host, Redirect-Host-Usage and Redirect-Max-Cache-Time AVPs as described in clause 6.12, 6.13 and 6.14 of IETF RFC 3588 [14].

[image: image68.wmf] 

1. 

 

Exter

nal trigger

 

Legend

:

 

 

       

 

Mandatory

 

               

 

 

Conditional 

 

H

-

PCRF

-

1

 

V

-

PCRF

 

2. 

 

Rx/

S9

 Diameter 

Establishment 

Request

 

4. Diameter 

Answer 

(

redirect

)

 

 

3.  

DRA binding

 

creation/retrieval

 

6

. 

Rx/

S

9

 Diameter 

Answer

 

H

-

DRA

 

(

redirect

)

 

H

-

PCRF

-

2

 

5. 

 

Rx/

S9

 Diameter

 

Establishment

 Re

quest 

 


Figure 7.4.2.1.2.1: Establishment of Diameter session through DRA (redirect) – Roaming scenario

1.
The V-PCRF receives an external trigger (e.g., IP-CAN session establishment request) that requires the establishment of a Diameter session with an H-PCRF over the S9 reference point.

2.
A Rx/S9 Diameter Establishment Request with user information (e.g., UE-NAI) is sent by the V-PCRF and received by the H-DRA (redirect) in the home PLMN.
3.
The H-DRA (redirect) stores the user information (e.g., UE-NAI) and checks whether an active DRA binding exists. If not the H-DRA creates a dynamic DRA binding (assignment of a PCRF node per UE); if the DRA (redirect) find there has been a DRA binding for the user, the DRA shall select the PCRF from the binding for the client.
4.
The H-DRA (redirect) sends a Diameter Answer indicating redirection as defined in IETF RFC 3588 [14]. The target PCRF identity is included in the Redirect-Host AVP.

5.
The V-PCRF re-sends the Rx/S9 Diameter Establishment Request of step 2 to the target H-PCRF.
6.
H-PCRF-1 returns a corresponding Diameter Answer to the V-PCRF.

NOTE:
The V-PCRF may proxy the Rx Diameter Establishment Request to the H-PCRF directly (e.g. based on the stored information provided by H-DRA during the S9 Diameter session establishment).
7.4.2.2
Modification of Diameter sessions

The PCEF shall send the Diameter session modification message to the DRA to update the DRA binding information only if the UE’s address(es) is updated and the DRA (redirect) is maintaining PCRF routing information per IP-CAN session. For visited access case, the V-PCRF shall send the Diameter session modification message to the H-DRA to update the DRA binding information only if the UE’s address(es) is updated. The detailed procedure is similar to the Establishment of Diameter sessions, which is described in the clause 7.4.2.1.
7.4.2.3
Termination of Diameter Sessions

7.4.2.3.1
Non-roaming cases

Termination of Diameter sessions that impact the DRA binding occur at the following cases:

-
Gateway control session termination

-
IP-CAN session termination

The DRA client (BBERF/PCEF) shall follow the procedure below if the DRA (redirect) is maintaining PCRF routing information per IP-CAN session or an appropriate cached route table entry created from previous DRA (redirect) interactions does not exist. Cached route table entries are created from the Redirect-Host, Redirect-Host-Usage and Redirect-Max-Cache-Time AVPs as described in clause 6.12, 6.13 and 6.14 of IETF RFC 3588 [14].

[image: image69.wmf] 

1. 

 

Exter

nal trigger

 

Legend

:

 

 

       

 

Mandatory

 

               

 

 

Conditional 

 

PCRF

-

1

 

Client

 

(

BBERF/PCEF

)

 

 

2

 

 

Gx/Gxx Diameter 

T

ermination 

Request

 

6

. 

Gx/Gxx 

Diameter 

Answer

 

4

.  

DRA binding

 

verification/release

 

5

 Diameter Redirect 

Answer

 

DRA

 

(

redirect

)

 

PCRF

-

2

 

3

. 

 

Gx/Gxx Diameter 

T

ermination Request

 


Figure 7.4.2.3.1.1: Termination of Diameter sessions through DRA (redirect)

1.
Client receives an external trigger (e.g. an IP-CAN session termination is initiated by the UE or PCRF) that triggers the client to terminate Diameter session with server (i.e. PCRF)

2
A Diameter Termination Request (e.g., as defined in clauses 4.5.7 (Gx) and 4a.5.3 (Gxx) of 3GPP TS 29.212 [9]) is sent by the Client to the DRA (redirect).

3.
A Diameter Termination Request (e.g., as defined in clauses 4.5.7 (Gx) and 4a.5.3 (Gxx) of 3GPP TS 29.212 [9]) is sent by the Client to PRCF-1. The message uses the same Session-Id AVP value of the active Diameter session established between the Client and PCRF-1.

NOTE:
Steps 2, 3 may be carried out in parallel. Otherwise, the client after step2 may need to wait for the redirect answer before sending the Diameter termination request to the PCRF.
4.
DRA (redirect) verifies that there is an active DRA binding for the IP-CAN session based on the Session-Id AVP and marks the Diameter session terminated. If the DRA binding is per IP-CAN session and all the Diameter sessions (i.e. Gx session or Gxx session) of that IP-CAN session are terminated or if the DRA binding is per UE and all the Diameter sessions (i.e. Gx session or Gxx session) of that UE are terminated the DRA removes the DRA binding.
5
DRA (redirect) acknowledges termination of the session by sending a Diameter redirect answer to the client.

6
PCRF-1 acknowledges termination of session. PCRF-1 sends a Diameter Answer (e.g., as defined in clauses 4.5.7 (Gx) and 4a.5.3 (Gxx) of 3GPP TS 29.212 [9]) to the Client.
NOTE:
Figure 7.4.2.3.1.1 is also applicable when the BBERF/PCEF in the VPLMN terminates the Diameter sessions through the V-DRA.
7.4.2.3.2
Roaming cases

Termination of Diameter sessions occur at the following cases:

-
S9 session termination
The DRA client (AF/BBERF/PCEF) shall follow the procedure below) if the DRA (redirect) is maintaining PCRF routing information per IP-CAN session or an appropriate cached route table entry created from previous DRA (redirect) interactions does not exist. Cached route table entries are created from the Redirect-Host, Redirect-Host-Usage and Redirect-Max-Cache-Time AVPs as described in clause 6.12, 6.13 and 6.14 of IETF RFC 3588 [14].

[image: image70.wmf] 

1. 

 

Exter

nal trigger

 

Legend

:

 

 

       

 

Mandatory

 

               

 

 

Conditional 

 

H

-

PCRF

-

1

 

V

-

PCRF

 

2

 

 

S9

 Diameter 

T

ermination 

Request

 

6

. 

S9

 

Diameter Answer

 

4

.  

DRA binding

 

verification/release

 

5

 Diameter 

Redirect 

Answer

 

H

-

DRA

 

(

redirect

)

 

H

-

PCRF

-

2

 

3

. 

 

S9

 Diameter 

T

ermination Request

 


Figure 7.4.2.3.2.1: Termination of Diameter sessions through DRA (redirect) – Roaming case

1.
V-PCRF receives an external trigger (e.g. session termination request from the BBERF, PCEF) that requires the sending of a Diameter Termination Reqeust.
2
A Diameter Termination Request is sent by the V-PCRF and received by the H-DRA (redirect) in the home PLMN.

3.
A Diameter Termination Request is sent by the V-PCRF to H-PRCF-1. The message uses the same Session-Id AVP value of the active Diameter session established between theV-PCRF and H-PCRF-1.

NOTE:
Steps 2, 3 may be carried out in parallel. Otherwise, the V-PCRF after step2 may need to wait for the redirect answer before sending the Diameter termination request to the H-PCRF
4.
H-DRA (redirect) verifies that there is an active DRA binding for the IP-CAN session based on the Session-Id AVP and marks the Diameter session terminated. If all the Diameter sessions (i.e. S9 session, Gxx session, Gx session) of that UE are terminated the H-DRA removes the DRA binding.
5
H-DRA (redirect) acknowledges termination of the session by sending a Diameter redirect answer to the V-PCRF.

6
H-PCRF-1 acknowledges termination of the session by sending a Diameter answer to the V-PCRF.

NOTE:
Rx Diameter termination messages are not required to be sent to H-DRA (redirect) since such messages do not affect the DRA binding
8
Diameter race condition handling

8.1
Overview

Certain Diameter PCC applications (Gx, Gxx, Sd, S9) allow the server (PCRF for Gx, Gxx, Sd, H-PCRF for S9) to update a session in two ways: unsolicited and solicited. The PCRF can push policy decisions and provision event triggers in an unsolicited fashion using an RAR. It can also install policy decisions in a solicited manner by responding to a CCR sent by the client (BBERF for Gxx, PCEF for Gx, TDF for Sd, V-PCRF for S9).

The client and the server can initiate transactions that modify the state of the session independently (e.g. CCR from the client and RAR from the server) and potentially concurrently. Additionally, there may be Diameter agents in between the client and server (e.g. DRA or in general Diameter relays/proxies) that could cause messages to be delivered out of order. This can lead to race conditions that may result in the wrong information maintained by the client and/or server for a session.

Note that race conditions occur in different ways based on the application. Also, their impact is specific to the application. For example, even though Gx is based on DCCA (RFC 4006), Gx is much more vulnerable to race conditions as Gx allows sessions to be updated based on RARs and CCAs whereas DCCA only allows the server to update the session based on a CCA. The RAR is merely used to solicit the client to send a CCR.

8.2
Procedures for Gx, Gxx, Sd and S9

This clause describes the optional procedures for handling Diameter race conditions in a deterministic manner for the following interfaces: Gx, Gxx, Sd and S9. These procedures apply to the PCEF (Gx), BBERF (Gxx), TDF (Sd) and PCRF (Gx, Gxx, Sd and S9).

In this clause, the terms “client” and “server” are relative to the session context. As an example, for the Gx interface, the client is the PCEF and the server is the PCRF. The term “node” can refer to either a client or a server. The term “transaction” refers to a Diameter request and its associated answer. The term “ongoing transaction” refers to a transaction that has an outstanding answer.

A node that supports the procedures defined in this clause and is configured to comply with them, shall advertise such support by setting the corresponding PendingTransaction feature bit in the Supported-Features AVP during session establishment as defined in 3GPP TS 29.212 [9] for Gx, Gxx and Sd and 3GPP TS 29.215 [22] for S9.

On receipt of a Diameter request for an existing Diameter session, the recipient Diameter node shall check if it has an ongoing transaction on that session:

1.
If there are no ongoing transactions on the session, the node shall process the incoming request normally.

2.
If there is an ongoing transaction on the session and optionally, if the recipient node cannot determine that the incoming request can be safely handled without creating a state mismatch:

a.
The client shall reject the incoming request with a Diameter experimental result code of DIAMETER_PENDING_TRANSACTION as defined in 3GPP TS 29.212 [9].

b.
The server shall either reject the incoming request with a Diameter experimental result code of DIAMETER_PENDING_TRANSACTION or shall wait for one of the following conditions to occur:

i.
The ongoing transaction completes. In this case, the session is updated at the server based on the completion of the ongoing transaction and afterwards, the incoming request (e.g. CCR) is processed normally based on the updated session state.

ii.
The waiting period has exceeded its allotted time. In this case, the server shall reject the incoming request with a Diameter experimental result code of DIAMETER_PENDING_TRANSACTION.

3.
On receipt of a DIAMETER_PENDING_TRANSACTION result code, a client shall retry the request. On the other hand, if a server had rejected a request from the client with a DIAMETER_PENDING_TRANSACTION, the server should not retry the failed request until it receives the re-attempted request from the client. This is to avoid having both the client and server concurrently retry their requests. In all other cases, if the session on the client still needs to be updated, the server shall retry the request.

4.
Nodes should limit the number of times they re-attempt the same request due to receipt of a DIAMETER_PENDING_TRANSACTION.

5.
The only exception to the rules above is a session termination request (CCR with a CC-Request-Type set to TERMINATION_REQUEST) or a request for session release (e.g. RAR with Session-Release-Cause). In both cases, the request should be handled immediately.
Annex A (informative):
Examples of deriving the Maximum Authorized parameters from the SDP parameters
Annex B (normative):
Signalling Flows for IMS

The signalling flows in clause 4 are also applicable for IMS. This Annex adds flows that show interactions with SIP/SDP signalling of the IMS.
B.0
General

The following is applicable for Emergency Services and PSAP call back request:

-
The P-CSCF includes an Emergency indication when service information is sent over Rx as defined in 3GPP TS 29.214 [10].

-
The PCRF only allows Emergency Sessions that are bound to an IP-CAN session established to an Emergency APN.
The following is not applicable for Emergency Services and PSAP call back request:

-
Pre-authorization for a UE terminated IMS session establishment with UE initiated resource reservation.

-
Subscription to notification of Signalling Path Status at IMS Registration, subscription to notification of changes of IP-CAN type at IMS Registration and Provisioning of SIP Signalling flow information at IMS Registration procedures.

B.1
Subscription to Notification of Signalling Path Status at IMS Registration
This clause covers the optional Subscription to Notifications of IMS Signalling Path Status upon an initial successful IMS Registration procedure.


[image: image71.wmf] 

7

. Diameter AAA

 

5. Diameter AAR

 

 

P

-

CSCF

 

 

PCRF

 

1. SIP REGISTER

 

2. SIP REGISTER

 

3. 200 OK

 

4. 200 OK

 

UE

 

6. Session Binding 

 

Legend:

 

 

       Mandatory

 

 

       Optional

 

 

8. 

Subscribe to Bearer Level Event

 


Figure B.1.1: Subscription to Notification of IMS Signaling Path Status at initial IMS Registration

1-4.
The user initiates an initial SIP Registration procedure. The SIP Registration procedure is completed successfully (user has been authenticated and registered within the IMS Core NW).

5.
The P-CSCF requests the establishment of a new Diameter Rx session with the intention to subscribe to the status of the IMS Signaling path. The P-CSCF sends a Diameter AAR command to the PCRF.

6.
The PCRF performs session binding and identifies corresponding PCC Rules related to IMS Signalling.

7.
The PCRF confirms the subscription to IMS Signaling path status and replies with a Diameter AAR command back to the P-CSCF.

8.
If the PCRF had not previously subscribed to the required bearer level events from the IP-CAN for the affected PCC/QoS Rules, then the PCRF shall do so now. The PCRF initiates procedures according to figure 4.3.1.1.1.
B.1a
Subscription to Notification of Change of IP-CAN Type at IMS Registration
This clause covers the optional Subscription to Notifications of change in the type of IP-CAN upon an initial IMS Registration procedure.


[image: image72.wmf] 

4

. Diameter AAA

 

2

. Diameter AAR

 

 

P

-

CSCF

 

 

PCRF

 

1. SIP REGISTER

 

5

. SIP REGISTER

 

UE

 

3

. Session Binding 

 

Legend:

 

 

       Mandatory

 

 

       Optional

 

 

6. 200 OK

 

7. 200 OK

 

8. 

Subscribe to Bearer Level Event

 


Figure B.1.2: Subscription to Notification of change of IP-CAN Type at initial IMS Registration
1.-
The user initiates an initial SIP Registration procedure.

2.
The P-CSCF requests the establishment of a new Diameter Rx session with the intention to subscribe to the notification of IP-CAN Type Change. The P-CSCF sends a Diameter AAR command to the PCRF.

NOTE:
It should be possible for the P-CSCF to request the subscription to notification of IMS Signalling path status also in this step.

3.
The PCRF performs session binding and identifies corresponding PCC Rules related to IMS Signalling.

4.
The PCRF confirms the subscription to notification of change of IP-CAN type and replies with a Diameter AAR command back to the P-CSCF. The PCRF includes in the response the type of IP-CAN currently in use.

5-7.
The SIP Registration procedure is completed successfully (user has been authenticated and registered within the IMS Core NW).

8.
If the PCRF had not previously subscribed to the required bearer level events from the IP-CAN for the affected PCC/QoS Rules, then the PCRF shall do so now. The PCRF initiates procedures according to figure 4.3.1.1.1.

B.1b
Provisioning of SIP signalling flow information at IMS Registration
This clause covers the optional Provisioning of SIP signalling flow information upon an initial successful IMS Registration procedure.


[image: image73.wmf] 

7

. Diameter AAA

 

5. Diameter AAR

 

 

P

-

CSCF

 

 

PCRF

 

1. SIP REGISTER

 

2. SIP REGISTER

 

3. 200 OK

 

4. 200 OK

 

UE

 

6. Session Binding 

 

Legend:

 

 

       Mandatory

 

 

       Optional

 

 

8. 

Provisioning of PCC/QoS Rules

 


Figure B.1.3: Provisioning of SIP Signalling Flow Information at initial IMS Registration

1-4.
The user initiates an initial SIP Registration procedure. The SIP Registration procedure is completed successfully (user has been authenticated and registered within the IMS Core NW).

5.
The P-CSCF requests the establishment of a new Diameter Rx session with the intention to provision the information about the SIP signalling flows established between the UE and the P-CSCF. The P-CSCF sends a Diameter AAR command to the PCRF.

6.
The PCRF performs session binding and identifies corresponding PCC Rules related to IMS Signalling.

7.
The PCRF replies to the P-CSCF with a Diameter AAA.

8.
If the PCRF had not previously provisioned PCC/QoS rules corresponding to the received SIP signalling flows, then the PCRF executes interactions according to figure 4.3.1.1.1. This step implies provisioning of PCC/QoS rules.
B.2
IMS Session Establishment
B.2.1
Provisioning of service information at Originating P-CSCF and PCRF

This clause covers the PCC procedures at the originating P-CSCF and PCRF at IMS session establishment.

In figure B.2.1.1 the P-CSCF derives the provisioning of service information to the PCRF from the SDP offer/answer exchange.


[image: image74.emf]6. Diameter AAR

UE

P-CSCF

2. Define down-link 

connection info

1. SDP offer

3. SDP offer

4. SDP answer

9. SDP answer

5. Define up-link  

connection info

PCRF

10.  PCC/QoS Rules provisioning

7. Store Session 

Information and identify IP-

CAN Session

8. Diameter AAA

11. Diameter RAR

Legend:

Mandatory

Conditional 

12. Diameter RAA

If access 

network 

information 

is requested

13. 

SIP with 

NPLI

NPLI: 

user location and/or UE 

timezone information


Figure B.2.1.1: PCC Procedures for IMS Session Establishment at originating P-CSCF and PCRF

1.
The P-CSCF receives the SDP parameters defined by the originator within an SDP offer in SIP signalling.

2.
The P-CSCF identifies the connection information needed (IP address of the down link IP flow(s), port numbers to be used etc…).

3.
The P-CSCF forwards the SDP offer in SIP signalling.

4.
The P-CSCF gets the negotiated SDP parameters from the terminating side through SIP signalling interaction.

5.
The P-CSCF identifies the connection information needed (IP address of the up-link media IP flow(s), port numbers to be used etc…).

6.
The P-CSCF forwards the derived session information to the PCRF by sending a Diameter AAR over a new Rx Diameter session.

7.
The PCRF stores the received session information, and performs session binding.
8.
The PCRF replies to the P-CSCF with a Diameter AAA.

9.
Upon reception of the acknowledgement from the PCRF, the SDP parameters are passed to the UE in SIP signalling.

10.
The PCRF executes interactions according to figure 4.3.1.1.1 . This step implies provisioning of PCC/QoS rules and is executed in parallel with steps 8 and 9.
11.
If the P-CSCF requested access network information in step 6, the PCRF forwards the access network information received in step 10 in a Diameter RAR.

12.
If step 11 occurs, the P-CSCF acknowledges the receipt of Diameter RAR.

13.
If step 11 occurs, the P-CSCF forwards the access network information as the network provided location information when a suitable SIP message is received.
Optionally, the provisioning of service information may be derived already from the SDP offer to enable that a possible rejection of the service information by the PCRF is obtained by the P-CSCF in time to reject the service with appropriate SIP signalling, or to allow the P-CSCF to request network provided location information for inclusion in the SDP offer. This is described in figure B.2.1.2.


[image: image75.emf]15. Diameter AAA

13. Diameter AAR

UE

P-CSCF

2. Extract down-link 

service information

1. SDPoffer

6. SDPoffer (without NPLI)

11. SDPanswer

17. SDPanswer

12. Extractup-link  

service information

PCRF

16.  Provisioning of PCC/QoS Rules

5. Diameter AAA

3. Diameter AAR

meter AAR

4. Authorize ServiceInformation 

14 . Store Session Information

8. Diameter RAR

Legend:

Mandatory

Conditional 

9. Diameter RAA

If access 

network 

information is 

requested

7.  Provisioning of PCC/QoS Rules

10a.

SDP offer 

(With NPLI)

10b. SIP (With NPLI)

NPLI: 

user location and/or UE 

timezone information


Figure B.2.1.2: PCC Procedures for IMS Session Establishment at originating P-CSCF and PCRF, provisioning of service information derived from SDP offer and answer
1.
The P-CSCF receives the first SDP offer for a new SIP dialogue within a SIP INVITE request.

2.
The P-CSCF extracts service information from the SDP offer (IP address of the down link IP flow(s), port numbers to be used etc…).

3.
The P-CSCF forwards the derived service information to the PCRF by sending a Diameter AAR over a new Rx Diameter session. It indicates that only an authorization check of the service information is requested.

4.
The PCRF checks and authorizes the service information, performs session binding, but does not provision PCC/QoS rules at this stage.

5.
The PCRF replies to the P-CSCF with a Diameter AAA.

6.
If the P-CSCF did not request access network information in step 3, or if the P-CSCF requested access network information but does not require the access network information for inclusion in the SDP offer. The P-CSCF forwards the SDP offer in SIP signalling.
7.
If the P-CSCF requested access network information in step 3, the PCRF executes interactions according to Figure 4.3.1.1.1. This step implies provisioning of PCC/QoS rules.

8.
If the P-CSCF requested access network information in step 3, the PCRF forwards the access network information received in step 7 in a Diameter RAR.

9.
If step 8 occurs, the P-CSCF acknowledges the receipt of Diameter RAR.

10a.
If step 8 occurs, and if the P-CSCF requires the access network information for inclusion in the SDP offer, the P-CSCF forwards the SDP offer and adds the access network information as the network provided location information to the corresponding SIP message.

10b.
If step 8 occurs, and if the P-CSCF does not require the access network information for inclusion in the SDP offer, the P-CSCF forwards the access network information as the network provided location information in a suitable SIP message. This step normally occurs only after step 17.
11.
The P-CSCF receives the negotiated SDP parameters from the terminating side within a SDP answer in SIP signalling.

12.
The P-CSCF extracts service information from the SDP answer (IP address of the up-link media IP flow(s), port numbers to be used etc…).

13.
The P-CSCF forwards the derived service information to the PCRF by sending a Diameter AAR over the existing Rx Diameter session. Access network information is not requested if done in step 7.
14.
The PCRF stores the received session information.
15.
The PCRF replies to the P-CSCF with a Diameter AAA.

16.
The PCRF authorizes the session information. The PCRF executes interactions according to Figure 4.3.1.1.1. This step implies provisioning of PCC/QoS rules and authorized QoS.

17.
Upon successful authorization of the session, the SDP parameters are passed to the UE in SIP signalling. This step is executed in parallel with step 16.
B.2.2
Provisioning of service information at terminating P-CSCF and PCRF

This clause covers the PCC procedures at the terminating P-CSCF and PCRF at IMS session establishment.
In figure B.2.2.1 the P-CSCF derives the provisioning of service information to the PCRF from the SDP offer/answer exchange.

[image: image76.emf]6. Diameter AAR

1. SDP offer

UE

P-CSCF

2. Define up-link  

connection info

3. SDP offer

9. SDP answer 

(without NPLI)

4. SDP answer

5. Define down-link 

connection info

PCRF

8.Diameter AAA

10.  Provisioning of PCC/QoS Rules

7. Store Session 

Information and identify IP-

CAN Session

11. Diameter RAR

Legend:

Mandatory

Conditional 

12. Diameter RAA If access 

network 

information 

is requested

13. SDP answer 

(with NPLI)

NPLI: 

user location and/or UE 

timezone information


Figure B.2.2.1: PCC Procedures for IMS Session Establishment at terminating P-CSCF and PCRF

1.
The P-CSCF receives the SDP parameters defined by the originator.

2.
The P-CSCF identifies the connection information needed (IP address of the up-link IP flow(s), port numbers to be used etc…).
3.
The P-CSCF sends the SDP offer to the UE.

4.
The P-CSCF receives the negotiated SDP parameters from the UE.

5.
The P-CSCF identifies the connection information needed (IP address of the down-link IP flow(s), port numbers to be used etc…).

6.
The P-CSCF forwards the derived service information to the PCRF by sending a Diameter AAR over a new Rx Diameter session.

7.
The PCRF stores the received session information, and performs session binding.
8.
The PCRF sends a Diameter AAA to the P-CSCF.
9.
If the P-CSCF did not request access network information in step 6, upon reception of the acknowledgement from the PCRF, the SDP parameters in the SDP answer are passed to the originator.
10.
The PCRF executes interactions according to clause 4.3.1.1.1. This step implies provisioning of PCC/QoS rules and is executed in parallel with steps 8 and 9.
11.
If the P-CSCF requested access network information in step 6, the PCRF forwards the access network information received in step 10 in a Diameter RAR.

12.
If step 11 occurs, the P-CSCF acknowledges the receipt of Diameter RAR.

13.
If step 11 occurs, the P-CSCF forwards the SDP answer and adds the access network information as the network provided location information to the corresponding SIP message.
Optionally, the provisioning of service information may be derived already from the SDP offer to enable that a possible rejection of the service information by the PCRF is obtained by the P-CSCF in time to reject the service with appropriate SIP signalling or to enable pre-authorization for a UE terminated IMS session establishment with UE initiated resource reservation. This is described in figure B.2.2.2.


[image: image77.emf]10. Diameter AAR

1. SDP offer

UE P-CSCF

2. Extract up-link  

service information

6. SDP offer

13

. SDP answer 

(without NPLI)

8. SDP answer

9. Extract down-link 

service information

PCRF

11.Diameter AAA

12.  Provisioning of PCC/QoS Rules

3. Diameter AAR

5.Diameter AAA

4. Authorize Service Information

7. Provisioning of PCC/QoS Rules

10a . Store Session Information 

14. Diameter RAR

Legend:

Mandatory

Conditional 

15. Diameter RAA

If access 

network 

information is 

requested

16. SDP anwer 

(With NPLI)

NPLI: 

user location and/or UE 

timezone information


Figure B.2.2.2: PCC Procedures for IMS Session Establishment at terminating P-CSCF and PCRF, provisioning of service information derived from SDP offer and answer
1.
The P-CSCF receives the first SDP offer for a new SIP dialogue within SIP signalling, e.g. within a SIP INVITE request.
2.
The P-CSCF extracts the service information from the SDP offer (IP address of the up-link IP flow(s), port numbers to be used etc…).

3.
The P-CSCF forwards the derived session information to the PCRF by sending a Diameter AAR over a new Rx Diameter session. It indicates that the service information that the AF has provided to the PCRF is preliminary and needs to be further negotiated between the two ends.

4.
The PCRF checks and authorizes the session information, performs session binding, but does not provision PCC/QoS Rules at this stage.

5.
The PCRF replies to the P-CSCF with a Diameter AAA.

6.
The P-CSCF sends the SDP offer to the UE.
7.
If the UE initiates a bearer resource modification request, the PCRF provides the PCEF/BBERF with PCC/QoS rules according to figure 4.3.1.1.1 based on the SDP offer.
NOTE:
Step 7 is not applicable for IMS Emergency Sessions.
8.
The P-CSCF receives the negotiated SDP parameters from the UE within an SDP answer in SIP signalling.

9.
The P-CSCF extracts service information from the SDP answer (IP address of the down-link IP flow(s), port numbers to be used etc…).

10.
The P-CSCF forwards the derived service information to the PCRF by sending a Diameter AAR over the existing Rx Diameter session.
10a.
The PCRF stores the received session information.
11.
The PCRF sends a Diameter AAA to the P-CSCF.

12.
The PCRF authorizes the session information. The PCRF executes interactions according to Figure 4.3.1.1.1. This step implies provisioning of PCC/QoS rules and authorized QoS.

13.
If the P-CSCF did not request access network information in step 3 or 10, upon successful authorization of the session the SDP parameters in the SDP answer are passed to the originator. This step is executed in parallel with step 12.
14.
If the P-CSCF requested access network information in step 3 or 10, the PCRF forwards the access network information received in step 12 in a Diameter RAR.

15.
If step 14 occurs, the P-CSCF acknowledges the receipt of Diameter RAR.

16.
If step 14 occurs, the P-CSCF forwards the SDP answer and adds the access network information as the network provided location information to the corresponding SIP message.
B.3
IMS Session Modification
B.3.1
Provisioning of service information

This clause covers the provisioning of service information at IMS session modification both at the originating and terminating side.
In figure B.3.1.1 the P-CSCF derives the provisioning of service information to the PCRF from the SDP offer/answer exchange.

[image: image78.emf]8. Diameter AAA

6. Diameter AAR

P-CSCF PCRF

7. Store Session Info and 

Identify IP-CAN Session

2. extract changes in 

service information 

from SDP offer

1. SDP offer

3. SDP offer

4. SDP answer

5. extract changes in  

service information 

from SDPanswer

9. SDP answer 

(without NPLI)

10. Interactions in Figure 

4.3.1.1.1

11. Diameter RAR

Legend:

Mandatory

Conditional 

12. Diameter RAA

If access 

network 

Information 

is requested

13. SDP answer 

(with NPLI)

NPLI: 

user location and/or UE 

timezone information


Figure B.3.1.1: Provisioning of service information at IMS session modification

1.
The P-CSCF receives the SDP parameters defined by the originator within an SDP offer in SIP signalling.

2.
The P-CSCF identifies the relevant changes in the SDP.

3.
The P-CSCF forwards the SDP offer in SIP signalling.

4.
The P-CSCF gets the negotiated SDP parameters from the terminating side through SIP signalling interaction.

5.
The P-CSCF identifies the relevant changes in the SDP.

6.
The P-CSCF sends a Diameter AAR for an existing Diameter session and includes the derived updated service information.

7.
The PCRF stores the received updated session information and identifies the affected established IP-CAN Session(s).
8.
The PCRF answers with a Diameter AAA.

9.
If the P-CSCF did not request access network information in step 6, the P-CSCF forwards the SDP answer in SIP signalling.

10.
The PCRF executes interactions according to figure 4.3.1.1.1. Due to the updated service information, this step may imply provisioning of PCC/QoS rules or the need to enable or disable IP Flows (see clauses B.3.2 and B.3.3, respectively).
11.
If the P-CSCF requested access network information in step 6, the PCRF forwards the access network information received in step 10 in a Diameter RAR.

12.
If step 11 occurs, the P-CSCF acknowledges the receipt of Diameter RAR.

13.
If step 11 occurs, the P-CSCF forwards the SDP answer and adds the access network information as the network provided location information to the corresponding SIP message.
Optionally, the provisioning of service information may be derived already from the SDP offer to enable that a possible rejection of the service information by the PCRF is obtained by the P-CSCF in time to reject the service with appropriate SIP signalling or to enable pre-authorization for a UE terminated IMS session establishment with UE initiated resource reservation. This is described in figure B.3.1.2.


[image: image79.emf]11. Diameter AAA

10. Diameter AAR

P-CSCF

PCRF

12. 

Provisioning of 

PCC/QoS Rules

2. extract changes in 

service information 

from SDP offer

1. SDP offer

7. SDP offer

8. SDP answer

9 . extract changes in  

service information 

from SDP answer

13. SDP answer 

(without NPLI)

5. Diameter AAA

3. Diameter AAR

4. authorize service 

information

6. 

Provisioning of 

PCC/QoS Rules

14. Diameter RAR

Legend:

Mandatory

Conditional 

15. Diameter RAA

If access 

network 

information 

is requested

16. SDP answer 

(With NPLI)

NPLI: 

user location and/or UE 

timezone information


Figure B.3.1.2: Provisioning of service information derived from SDP offer and answer at IMS session modification

1.
The P-CSCF receives an SDP offer in SIP signalling for an exiting SIP dialogue.

2.
The P-CSCF identifies the relevant changes in the SDP and extracts the corresponding service information.

3.
The P-CSCF forwards the derived service information to the PCRF by sending a Diameter AAR over the existing Rx Diameter session for the corresponding SIP session. It indicates that the service information that the AF has provided to the PCRF is preliminary and needs to be further negotiated between the two ends.

4.
The PCRF checks and authorizes the session information, but does not provision PCC/QoS rules at this stage.

5.
The PCRF replies to the P-CSCF with a Diameter AAA.

6.
If the UE initiates a bearer resource modification request, the PCRF provides the PCEF/BBERF with PCC/QoS rules according to figure 4.3.1.1.1 based on the SDP offer.
NOTE:
Step 6 is not applicable for IMS Emergency Sessions.
7.
The P-CSCF forwards the SDP offer in SIP signalling.

8.
The P-CSCF receives the negotiated SDP parameters within an SDP answer in SIP signalling from the terminating side.

9.
The P-CSCF identifies the relevant changes in the SDP and extracts the corresponding service information.

10.
The P-CSCF sends a Diameter AAR for an existing Diameter session and includes the derived updated service information.

11.
The PCRF answers with a Diameter AAA.

12.
The PCRF interacts with the GW according to figure 4.3.1.1.1. This step may imply provisioning of PCC/QoS rules and authorized QoS. The PCRF may need to enable or disable IP Flows (see clauses B.3.2 and B.3.3, respectively) due to the updated service information.
13.
If the P-CSCF did not request access network information in step 3 or 10, the P-CSCF forwards the SDP answer in SIP signalling. This step is executed in parallel with step 12.
14.
If the P-CSCF requested access network information in step 3 or 10, the PCRF forwards the access network information received in step 12 in a Diameter RAR.

15.
If step 14 occurs, the P-CSCF acknowledges the receipt of Diameter RAR.

16.
If step 14 occurs, the P-CSCF forwards the SDP answer and adds the access network information as the network provided location information to the corresponding SIP message.
B.3.2
Enabling of IP Flows

The PCRF makes a final decision to enable the allocated QoS resource for the authorized IP flows of the media component (s) if the QoS resources are not enabled at the time they are authorized by the PCRF or if the media IP flow(s) previously placed on hold are resumed, i.e. the media IP flow(s) of the media component that was placed on hold at the time of the resource authorization or at a later stage is reactivated (with SDP direction sendrecv, sendonly, recvonly or none direction).

The Enabling of IP Flows procedure is triggered by the P-CSCF receiving any 2xx success response to an INVITE request or a 2xx success response to an UPDATE request within a confirmed dialogue that is not embedded as part of another INVITE Transaction (in both cases a 200 OK response is usually received). When receiving such responses, the PCRF shall take the SDP direction attribute in the latest received SDP (either within the 2xx success or a previous SIP message) into account when deciding, which gates shall be opened:

-
For a unidirectional SDP media component, IP flows in the opposite direction shall not be enabled.

-
For an inactive SDP media component, no IP flows shall be enabled.

Figure B.3.2.1 is applicable to the Mobile Originating (MO) side and the Mobile Terminating (MT) side.


[image: image80.emf] 

5. 2xx Success  

4. Diameter AAA  

2. Diameter AAR  

PCRF  

P - CSCF  

1. 2xx Success  

6. PCR F opens the gates  

3. PCRF updates  flow status of  affected PCC rules   

UE  

Legend:                            Mandatory  


Figure B.3.2.1: Enabling of IP Flows

1.
The P-CSCF receives the 2xx Success message complying with the conditions specified in the paragraphs above.

2.
The P-CSCF sends a Diameter AAR message to the PCRF, requesting that gates shall be opened.

3.
The PCRF approves the enabling of IP flows and PCRF updates flow status of affected PCC rules.

4
The PCRF sends a Diameter AAA to the P-CSCF.

5
The P-CSCF forwards the 2xx Success message.

6.
The PCRF executes interactions according to figure 4.3.1.1.1. This step implies opening the ‘gates’ by updating the flow status of PCC rules.
B.3.3
Disabling of IP Flows

The “Disabling of IP Flows” procedure is used when media IP flow(s) of a session are put on hold (e.g. in case of a media re-negotiation or call hold).

Media is placed on hold as specified in RFC 3264 [12]. Media modified to become inactive (SDP direction attribute) shall also be considered to be put on hold.

If a bidirectional media component is placed on hold by making it unidirectional, the IP flows shall only be disabled in the deactivated direction. If a media component is placed on hold by making it inactive, the IP flows shall be disabled in both directions.

Figure B.3.3.1 presents the “Disabling of IP Flows” procedure at media on hold for both the Mobile Originating (MO) side and the Mobile Terminating (MT) side.


[image: image81.emf] 

4 . Diameter AAA  

5 . SDP answer putting media on hold  

1. SDP answer  putting media  on hold  

2 . Diameter AAR  

P CR F   P - CSCF  

3 .  PCRF updates  flow status of  affected PCC rules  

7 .  PCR F   closes  the related gate  

Legend:                           Mandatory  

UE  


Figure B.3.3.1: Disabling of IP Flows at Media on Hold

1.
The P-CSCF receives an SDP answer putting media on hold within a SIP message. (NOTE)
2.
The P-CSCF sends a Diameter AAR request to the PCRF, requesting that gates shall be closed.

3.
The PCRF updates flow status of affected PCC rules for the media on hold.

4.
The PCRF sends a Diameter AAA message back to the P-CSCF.

5.
The P-CSCF forwards the SDP answer putting media on hold within a SIP message.

6.
The PCRF executes interactions according to figure 4.3.1.1.1. This step implies closing the relevant media IP flow gate(s), leaving the possible related RTCP gate(s) open to keep the connection alive.

NOTE:
This procedure occurs whenever a bidirectional media is made unidirectional or when a media is changed to inactive.

B.3.4
Media Component Removal

Figure B.3.4.1 presents the flows of PCC procedures at the removal of media component(s) from an IMS session which is not being released for both the Mobile Originating (MO) side and the Mobile Terminating (MT) side.


[image: image82.emf] 

1. SDP answer  removing medi a  component  

2 . Diameter AAR  

P CR F   P - CSCF  

3 .  Store Session  Information and Identify  IP - CAN session    

6 .  Remove/Modify PCC/QoS Rules    

5 . SDP answer removing media component  

Legend:                          Mandatory  

4. Diameter AAA  

UE  


Figure B.3.4.1: Revoke authorization for IP resources at media component removal
for both Mobile Originating (MO) and Mobile Terminating (MT) side

1.
A SIP message containing SDP indicating the removal of media component(s) is received by the P-CSCF.

2.
The P-CSCF sends Diameter AAR to the PCRF with modified service information.

3.
The PCRF stores the Session information and identifies the affected IP-CAN Session(s).

4.
The PCRF sends a Diameter AAA message back to the P-CSCF.

5.
The P-CSCF forwards the SDP answer removing a media component.

6.
The PCRF makes a decision on what PCC/QoS rules need to be modified or removed and executes interactions according to figure 4.3.1.1.1.
B.4
IMS Session Termination

B.4.1
Mobile initiated session release / Network initiated session release

Figure B.4.1.1 presents the mobile or network initiated IMS session release without access network information retrieval. The session release may be signalled by a SIP BYE message, or any SIP 3xx redirect response, or any 4xx, 5xx, or 6xx SIP final error response to an initial INVITE request. If any 4xx, 5xx, or 6xx SIP final error response to Re-INVITE or UPDATE request just terminates the transaction, then the session is not released, otherwise if the error response terminates the dialog then the session is released.
Figures B.4.1.2 and B.4.1.3 presents the network initiated and the mobile initiated IMS session release with access network information retrieval, respectively.

[image: image83.emf] 

2.  BYE,   3xx, 4xx, 5xx, or 6xx  

1.  BYE,   3xx, 4xx, 5xx,  or 6xx  

PCRF   P - CSCF  

3.     Removal of PCC /QoS   Rules at AF session release  

UE  


Figure B.4.1.1: IMS session termination without access network information retrieval

1.
SIP BYE message, a SIP 3xx redirect response, or any 4xx, 5xx, or 6xx SIP final error response to an initial INVITE or any 4xx, 5xx, or 6xx SIP final error response to Re-INVITE or UPDATE which terminates the dialog is received by the P-CSCF.

2.
P-CSCF forwards the BYE message, or the SIP 3xx redirect response, or any 4xx, 5xx, or 6xx SIP final error response.

3.
The Interactions in in Figure 4.3.1.2.3.1.1 or Figure 4.3.1.2.3.2.1are applicable.

[image: image84.emf]2. BYE

1. BYE

PCRF P-CSCF

3.

Removal of PCC/QoS Rules at AF session release 

4. 200 OK (BYE)

5. 

200 OK (BYE)

UE


Figure B.4.1.2: network initiated IMS session termination with access network information retrieval

1.
SIP BYE message is received by the P-CSCF.

2.
The P-CSCF forwards the BYE message.

3.
In parallel to step 2, the Interactions in Figure 4.3.1.2.3.1.1 or Figure 4.3.1.2.3.2.1 take place. Within those interactions, the P-CSCF requests and receives the access network information.

4.
The P-CSCF receives the SIP 200 OK (BYE) SIP message.

5.
The P-CSCF forwards the SIP 200 OK (BYE) SIP message. It includes the access networking information obtained in step 3 as the network provided location information.


[image: image85.emf]3. BYE

1. BYE

PCRF P-CSCF

Removal of PCC/QoS Rules at AF session release

2.

UE


Figure B.4.1.3: mobile initiated IMS session termination with access network information retrieval
1.
SIP BYE message is received by the P-CSCF.

2.
The Interactions in Figure 4.3.1.2.3.1.1 or Figure 4.3.1.2.3.2.1 are applicable. Within those interactions, the P-CSCF requests and receives the access network information.

3.
The P-CSCF forwards the BYE message. It includes the access network information obtained in step 2 as the network provided location information.

B.4.2
IP-CAN Bearer Release/Loss

An IP-CAN Bearer Release or Loss event may affect all IP-Flows within an IMS Session. Flows in clause 4.3.2.2.1 (AF located in the HPLMN) or 4.3.2.2.2 (AF located in the VPLMN) apply for case 1. Flows in clause 4.4.2.1.1 (Home Routed case) or 4.4.2.2.1 (Visited Access case) apply for case 2a and case 2b.
B.5
P-CSCF Restoration
This clause is applicable if P-CSCF Restoration is to be performed.

[image: image86.emf] 

UE PCEF V-PCRF H -PCRF P-CSCF

1 .  Diameter AAR

3. Diameter RAR

SIP message

P-CSCF Restoration Procedure

1 a.  Diameter AAR

3a.  Diameter RAR

2a.  Diameter AAA

2. Diameter AAA

4a.  Diameter RAA

4.  Diameter RAA

Roaming 

with local 

breakout 

when the P-

CSCF is 

located in 

the VPLMN

 


Figure B.5.1: P-CSCF Restoration
1.
The P-CSCF sends an AAR command to PCRF to initiate a P-CSCF Restoration procedure, as defined in the 3GPP TS 23.380 [35]. The AAR command contains a Rx-Request-Type AVP with value set to PCSCF_RESTORATION and can contain the IP address of the UE within Framed-IP-Address AVP (if available) or the Framed-Ipv6-Prefix AVP (if available), IMSI (if available) within the Subscription-Id AVP, the IMS APN (if available) within the Called-Station-Id AVP and/or the IP address domain (if available) within the IP-Domain-Id AVP.
For the non-roaming case and the home routed case, the H-PCRF receives the AAR command, i.e. step 1 is executed.

1a.
For the roaming case with visited access, when the P-CSCF is located in the VPLMN, step 1 is executed with the exception that the V-PCRF receives the AAR command and responds with the AAA command
2.
When receiving the AAR command for P-CSCF Restoration from the P-CSCF, the PCRF acknowledges the AAR by sending an AAA command to the P-CSCF.
For the non-roaming case and the home routed case, the H-PCRF sends an AAA command to the P-CSCF, i.e. step 2 is executed.

2a.
For the roaming case with visited access, when the P-CSCF is located in the VPLMN, step 2 is executed with the exception that the V-PCRF sends an AAA command to the P-CSCF.
For the roaming case with local breakout when the P-CSCF is located in the HPLMN, when the H-PCRF receives the AAR command from the P-CSCF, the H-PCRF sends an RAR command to the V-PCRF, i.e. step 1b is executed.

3.
When receiving the AAR command for P-CSCF Restoration, the PCRF finds the corresponding IP-CAN session according to the received information within the AAR command, and sends an RAR command for the corresponding IP-CAN session (IMS PDN connection) to the PCEF for P-CSCF Restoration. The RAR contains a PCSCF-Restoration-Indication AVP with value set to PCSCF_RESTORATION.
For the non-roaming case and the home routed case, when receiving the AAR command from the P-CSCF, the H-PCRF sends the RAR command to the PCEF, i.e. step 3 is executed.

3a,
For the roaming case with visited access, when the P-CSCF is located in the VPLMN, step 3 is executed with the exception that when receiving the AAR command from the P-CSCF, the V-PCRF sends the RAR command to the PCEF.

For the roaming case with local breakout when the P-CSCF is located in the HPLMN, when receiving the RAR command from the H-PCRF, the V-PCRF sends an RAR command to the PCEF, i.e. step 2a is executed.

4.
When receiving the RAR command for P-CSCF Restoration, the PCEF acknowledges the RAR by sending an RAA command to the PCRF and performs the subsequent P-CSCF Restoration procedure as specified in 3GPP TS 23.380 [35].

NOTE 1:
If the IMS PDN disconnection is performed for P-CSCF Restoration, the PCEF sends a CCR command to the PCRF to terminate the corresponding Gx session.
For the non-roaming case and the home routed case, when receiving the RAR command from the H-PCRF, the PCEF acknowledges this message by sending an RAA command to the H-PCRF, i.e. step 4 is executed.

4a.
For the roaming case with visited access, when the P-CSCF is located in the VPLMN, step 4 is executed with the exception that when receiving the RAR command from the V-PCRF, the PCEF acknowledges this message by sending an RAA command to the V-PCRF.

NOTE 2:
This P-CSCF Restoration procedure is applicable to the cases that both the P-CSCF and the PCEF are located in the same PLMN.
Annex C (normative):
NAT Related Procedures

C.1
Support for media traversal of NATs using ICE

The IMS calls out procedures for NAT traversal for media and 
vailabili within IMS. One of the methods supported by IMS for media traversal of NATs is a UE controlled NAT traversal solution based on the IETF Interactive Connectivity Establishment (ICE) protocol, IETF RFC 5245 [15]. When a UE uses the ICE protocol for media traversal of NATs, additional enhancements to the existing PCC procedures are necessary to allow for proper ICE operation.

This annex presents a set of rules that PCC network elements use to build flow descriptors, identify the proper UE IP addresses used by the PCRF for session and bearer binding, and gating control when the ICE procedures are invoked by the UE.

In order for the ICE procedures to work a static, preconfigured PCC rule needs to be in place at the PCEF which allows the UE to perform STUN binding requests prior to offering or answering an SDP.

NOTE 1:
Predefined PCC rules can be created to allow the UE to communicate with the STUN relay much in the same way the UE is allowed to communicate with the IMS network for session management.

NOTE 2:
Given that a STUN relay is a forwarding server under the direction of the UE, necessary precaution needs to be taken by the operator in how it chooses to craft these rules. It is recommended that such predefined rules only guarantee the minimal amount of bandwidth necessary to accomplish the necessary UE to STUN relay communication. Such an approach helps reduce the resources required to support NAT traversal mechanisms. Finally, such an approach allows the preconfigured rule to be over-ridden by dynamic rules which allow for the necessary bandwidth needed by the session.

NOTE 3:
The dynamic PCC rule will need to differentiate between different media traffic between UE and STUN relay (e.g. voice vs. video), which can be identified by the different ports assigned by the residential NAT. Session bindings need to take into account that the relevant terminal IP address may be contained within the ICE candidates contained in the session description, rather than in the normal media description.

NOTE 4:
It is assumed that the NAT device is located between the UE and the PCEF. NAT traversal outside of IMS in FBI services is considered FFS in the current 3GPP stage 2 specifications.

NOTE 5:
When a NAT device is located between the UE and the PCEF, it is assumed that the IP CAN session signalling will contain the IP address assigned by the residential NAT, rather than the UE IP address.

NOTE 6:
It is assumed that NAT devices that assign multiple IP addresses for the UE are outside the scope of release 7.

NOTE 7:
In this release, only one IP address per subscription is supported by session binding at the PCRF. Multiple Ues behind a NAT will use the same IP CAN session and IP address.

C.2
P-CSCF procedures

C.2.1
General

The procedures in clause C.2 are only invoked in the case where the local UE (uplink SDP) has utilized the ICE protocol for media traversal of NATs. The P-CSCF can determine this by inspecting the UE provided SDP (uplink) for the “a=candidate” attribute(s). If such attributes are present this is an indication that the UE has invoked the ICE procedures as defined in IETF RFC 5245 [15] for media traversal of NATs and the P-CSCF shall follow the requirements in clause C.2.

C.2.2
Deriving the Ues IP address

The P-CSCF shall set the Framed-IP-Address AVP or Framed-Ipv6-Prefix AVP to the source IP address of SIP messages received from the UE.

C.2.3
Deriving flow descriptions

In the case where STUN Relay and ICE are used for NAT traversal, the UE is required to place the STUN Relay provided address in the “m=” and “c=” lines of its SDP. Given that these addresses cannot be used by the P-CSCF for building a valid flow description, the P-CSCF will need to determine if a STUN Relay address has been provided in the “m=” and “c=” lines of the UE provided SDP (uplink only). The P-CSCF shall make this determination by inspecting the uplink SDP for “a=candidate” attributes and compare the candidate address with that contained in the “c=” line. If a match is found, the P-CSCF shall then look at the candidate type. If the candidate type is “relay” then the address in the “c=” line is that of a STUN Relay server. In this case, the P-CSCF shall derive the Flow-Description AVP within the service information from the SDP candidate type of “relay” as follows:

Uplink Flow-Description

-
Destination Address and Port: If the P-CSCF knows the destination address and port of the STUN Relay allocation that the UE is sending media to, it should use that information. If the P-CSCF does not know this address and port, it shall wildcard the uplink destination address and port.

-
Source Address and Port: The P-CSCF shall populate the uplink source address with the “rel-addr” address and the uplink source port with the “rel-port” port contained within the “a=candidate” attribute.

Downlink Flow-Description

-
Destination Address and Port: The P-CSCF shall populate the downlink destination address with the “rel-addr” address and the downlink destination port with the “rel-port” port contained within the “a=candidate” attribute.

-
Source Address and Port: If the P-CSCF knows the source address and port of the STUN Relay allocation that the UE is receiving media from, it should use that information. If the P-CSCF does not know this address and port, it shall wildcard the downlink source address and port.

For the other candidate types, the address in the “c=” and “m=” SDP attributes can be used for building the flow descriptor and the P-CSCF shall follow the rules to derive the Flow-Description AVP as described in table 6.2.2 of clause 6.2 of this TS.

C.2.4
Gating control

If both endpoints have indicated support for ICE (both the SDP offer and answer contain SDP attributes of type “a=candidate”) ICE connectivity checks will take place between the two Ues. In order to allow these checks to pass through the PCEF, the P-CSCF shall enable each flow description for each media component upon receipt of the SDP answer.

C.2.5
Bandwidth impacts

ICE has been designed such that connectivity checks are paced inline with RTP data (sent no faster than 20ms) and thus consumes a lesser or equal amount of bandwidth compared to the media itself (given the small packet size of a STUN connectivity check it is expected that the STUN connectivity checks will always have a smaller payload than the media stream itself). Thus, there are no additional requirements on the P-CSCF for bandwidth calculations for a given media flow.

C.3
PCRF procedures

C.3.1
General

The procedures in clause C.3 are only invoked when the following two conditions are met:

1.
Both the local and remote UE have utilized the ICE protocol for media traversal of NATs (see clause C.2.1 for details on how this is determined); and

2.
The IP-CAN which is servicing the IMS session does not support the concept of a default bearer.

C.3.2
Deriving additional flow descriptions

The PCRF may need to develop additional flow descriptions (beyond those provided by the P-CSCF) for a media component based on additional candidate addresses present in the SDP offer/answer exchange. The PCRF shall follow the procedures defined in IETF RFC 5245 [15] for forming candidate pairs based on the data contained within the received codec-data AVP. For each candidate pair created based on the ICE procedures and not already present in the received flow descriptions, the PCRF shall add an uplink and downlink flow description for each media component.

NOTE 1:
The uplink SDP represents the local candidates while the downlink SDP represents the remote candidates.

Following the ICE procedures for forming candidate pairs will result in some flow descriptions which would never be exercised. In particular, while the UE will send connectivity checks (and ultimately its media stream) from its host candidate, from the PCEF perspective, this will appear as being from the server reflexive address. Given this, the PCRF should not form flow descrptions using host candidate addresses and should only form additional flows based on server reflexive addresses and relay addresses.

As candidates are removed from the SDP via subsequent offer/answer exchanges, the PCRF shall update its candidate pair list and shall remove any flow descriptors no longer being used.

NOTE 2:
If the default candidate (the candidate used to populate the “c=” and “m=” lines of both the uplink and downlink SDP) is chosen, then an updated SDP offer/answer will not be done, and any extra flow descriptions not being used by the session will not be removed.

C.3.3
Gating control

For each additional flow description the PCRF adds to a media component (per clause C.3.2), the PCRF shall enable the flow in order to allow connectivity checks to pass.

C.3.4
Bandwidth impacts

Per clause C.2.5 ICE is designed to have minimal impact on bandwidth policy control. However, it is possible that media will begin flowing while the ICE connectivity checks are still in progress. Given the possibility that no session update will be made (the default candidates will be chosen by the ICE protocol), it is not recommended that the PCRF adjust the bandwidth parameters provided by the P-CSCF.
C.4
P_CSCF procedures to support media traversal through hosted NAT without ICE
Both the media flows and the SIP signalling can traverse a NA(P)T device located in the customer premises domain, or “hosted NAT”, as described in Annex F of 3GPP TS 24.229 [5]. The hosted NAT will modify the source IP address and source port of uplink IP packets, and the destination IP address and port of downlink IP packets. The IP addess and port information provided by the UE are thus not appropriate to configure PCC rules.

The UE may use ICE procedures for hosted NAT traversal, and related PCC procedures are described in clauses C.1 to C.3. The present clause provides procedures to cover the case where ICE is not used.

If the P-CSCF determines that the UE is located behind a hosted NAT (using procedures in Annex F of 3GPP TS 24.229 [5]) and that ICE is not used (using procedures in clause C.2.1), the P-CSCF shall, within the service information sent to the PCRF:

-
provide the source IP address of IP packets transporting incoming SIP messages from the UE as destination IP address of downlink media streams;

-
for Ipv4, provide the source IP address of IP packets transporting incoming SIP messages from the UE as source address of uplink media streams;

-
for Ipv6, derive the source address of uplink media streams from the prefix of the source IP address of IP packets transporting incoming SIP messages from the UE;

-
wildcard source ports of uplink media streams; and

-
wildcard destination ports of downlink media streams.

-
provide the port information within SDP sent towards the served UE as source ports of corresponding downlink media streams.

-
provide the port information within SDP sent towards the served UE as destination ports of corresponding uplink media streams.
Annex D (normative):
Access specific procedures for GPRS

D.1
General

The present annex defines IP-CAN specific requirements for General Packet Radio Service (GPRS).

D.2
Binding Mechanisms

Depending on the bearer control mode, bearer binding can be executed either by PCRF, PCEF or both PCRF and PCEF.

-
For “UE-only” IP-CAN bearer establishment mode, the PCRF performs bearer binding.

-
For “UE/NW” IP-CAN bearer establishment mode, the PCRF performs the binding of the PCC rules for user controlled services while the PCEF performs the binding of the PCC rules for the network controlled services.

If the PCEF performs the bearer binding, the PCRF shall follow the procedures as described in clause 5.4 with the exceptions described in this subclause.

If the Bearer Binding function is located at the PCEF, the PCEF shall check the QCI and ARP indicated by the PCC Rule(s) and bind the PCC rule with an IP-CAN bearer that has the same QCI and Evolved ARP (if this is supported by the SGSN).
If there is no suitable PDP-Context to accommodate a PCC rule when PCEF performs the bearer binding, the PCEF shall initiate the establishment of PDP-Contexts as specified in 3GPP TS 23.060 [3].
The PCEF shall not combine PCC rules with different ARP to the same bearer. If the Evolved ARP parameter is not supported by the SGSN, the PCEF shall map the Evolved ARP to Rel-99 ARP as specified in clause B.3.3.3 of 3GPP TS 29.212 [9].
NOTE:
If Evolved ARP is not supported by the SGSN then this enables a modification of the PDP context ARP without impacting the bearer binding after relocation to a SGSN that supports Evolved ARP.
If the Bearer Binding function is located at the PCRF, the PCRF shall compare the TFT(s) of all IP-CAN bearer(s) within the IP-CAN session received via PCEF from the UE with the existing service data flow filter information. The PCRF shall indicate to the PCEF the IP-CAN bearer within the IP-CAN session where the PCC Rules shall be installed, modified or removed. This is done including the Bearer-Identifier AVP together with the associated PCC Rules within the corresponding RAR and/or CCA commands.

-
When the PCRF does not require additional filter information coming from the UE in order to decide on bearer binding, the PCRF shall supply the PCC rules to be installed over the Gx interface to the PCEF within a RAR command.

-
Otherwise, the PCRF shall wait for the PCEF requesting a policy decision for the establishment of a new IP-CAN bearer or the modification of an existing one within a CCR command over the Gx interface.

-
When the PCEF reports the bearer event, it shall include within the CCR command a bearer reference together with the new or modified TFT information, the QCI and associated bitrates for new or modified PDP-Contexts.
D.3
PCC Procedures

D.3.1
IP-CAN Session Modification

D.3.1.1
Network-initiated IP-CAN Session Modification

Network-initiated IP-CAN session modification is executed according to clause 4.3.1.1 with the following differences:

-
The timer in step 4 will also be activated waiting for one of the following cases:

-
If the authorized QoS for an IP-CAN bearer is changed or

-
If one or more Flow Descriptions need to be added, deactivated or removed in any of the PCC rules bound to an IP-CAN bearer

-
If the timer in step 4 expires and the PCRF still requires additional filter information coming from the UE in order to decide on bearer binding for new PCC rules to be installed, all subsequent steps in figure 4.3.1.1.1 shall not be executed, and further reactions of the PCRF are left unspecified. As a possible option, the PCRF could abort the AF session.

-
When the PCRF performs the bearer binding, once the PCC rules are selected, the PCRF identifies the IP-CAN bearer for each of the PCC rules and the authorized QoS. The PCRF may provision PCC Rules and authorized QoS for several IP-CAN Bearers within the same RAR command.

-
For step 9, IP-CAN session signalling, the subsequent steps are executed separately for each IP-CAN bearer under the following conditions:
-
if all PCC rules bound to a PDP context have been removed or deactivated (PDP Context deactivation is applicable)

-
if one or more PDP contexts have to be modified

-
if in UE/NW Bearer Control Mode, the GGSN needs to establish a new PDP context(PDP Context establishment is applicable) if the bearer binding is located at the PCEF.

The GGSN initiates the procedure to Create/Update or Terminate PDP Context Request message to the SGSN. If in the case of updating the PDP Context the authorized QoS for the bearer has changed, the GGSN will modify the UMTS QoS parameters accordingly.

When the procedure in step 9 is completed and requires notifications from the GW, for an IP-CAN Bearer termination in UE-Only Bearer Control Mode, the GGSN sends a Diameter CCR with the Bearer-Identifier and Bearer-Operation AVPs to indicate “Termination”.
D.3.1.2
PCEF-initiated IP-CAN Session Modification

PCEF-initiated IP-CAN Session Modification procedure shall take place according to clauses 4.3.2.1 and 4.3.2.2 except for those procedures initiated by the UE, as described in the clauses below.

D.3.1.2.1
UE-initiated IP-CAN Bearer Establishment or IP-CAN Bearer Modification

This clause is applicable for the establishment of a new IP-CAN Bearer (other than the one which created the IP-CAN session) and for the modification of an already established IP-CAN Bearer. The signalling flows for these cases are as per Figure 4.3.1.2.1.
A bearer-event-initiated Request of PCC Rules occurs when a new bearer is established or when an existing bearer is modified. For GPRS, these are PDP Context Modification(s) or secondary PDP context Activation(s). An IP-CAN Session modification triggers a PCC Rule request only if the PCRF has previously requested a PCC Rule request for the given modification event.


[image: image87.wmf] 

PCRF

 

GW

 

2. 

 

Diameter CCR

 

11. 

 

Diameter CCA

 

9. 

 

PCC Rules Decision,

 

Policy Decision

 

4. 

 

Bind IP CA

N session to 

any existing AF session(s)

 

3. 

 

Store Bearer Information

 

10. 

 

Store PCC Rules 

 

SPR

 

5. 

 

Profile Request

 

6. 

 

Profile Response

 

For GPRS, for each affected other bearer 

where previous authorized QoS is 

modified

 

(because IP Flow(s) were moved to other 

bearer(s))

 

 

1. 

 

IP

-

CAN Bearer 

signall

ing 

 

1

3

. 

 

IP CAN Bearer 

signalling response

 

12. 

 

Installation Modification or 

Removal of PCC Rules, 

Policy Enforcement

 

Legend

:

 

 

       

 

Mandatory

 

               

 

 

Conditional 

 

AF

 

7. 

 

Diameter RAR

 

8. 

 

Diameter RAA

 

If the PCRF needs 

subscription

-

 related 

information and does 

not have it 

 

 

For IP CAN 

session 

modification, 

if 

requested by AF 

at initial 

authorisation 

 

 

14

. 

 

Start of Timer.

 

15. 

 

Interactions in Figure 4.3.1.1.1

 


Figure D.3.1.2.1: UE-initiated IP-CAN Bearer Establishment and Modification.

1.
The GW receives IP-CAN Bearer signalling that is a trigger for a PCC Rule request. For GPRS, the GGSN receives a secondary Establish PDP Context Request or an Update PDP Context Request.

2.
The GW informs the PCRF of the modification of the IP-CAN Session due to the IP-CAN Bearer signalling in step 1, using a Diameter CCR with the CC-Request-Type AVP set to the value UPDATE_REQUEST. The GW reuses the existing Gx DCC session corresponding to the IP-CAN Session.

 If the IP-CAN Bearer signalling in step 1 established a new IP-CAN Bearer, the GW assigns a new bearer identifier to this IP-CAN Bearer. The GW provides information about the new or modified bearer, e.g. requested QoS and TFT filters. If the event that caused the bearer modification applies uniquely to that bearer and PCRF performs the bearer binding, then, the bearer identifier should be provided within the CCR. If no bearer identifier is provided, the event trigger will apply to the IP-CAN session.

3.
The PCRF stores the received information in the Diameter CCR.

4.
The PCRF binds the IP-CAN Session to existing of AF session(s) using the information received from the GW and the Service Information included in the stored PCC rules, which was previously received from the AF(s) , as depicted in figure 4.3.1.1.1.
The PCRF also binds the IP-CAN Bearers within the IP-CAN Session to all matching IP flow(s) of existing AF session(s) using the bearer information received from the GW and the Service Information received from the AF(s). If IP flow(s), which have previously been bound to other bearers, have been bound to the modified bearer, PCC Rules in other bearer(s) may need to be removed. For GPRS, an IP flow may need to be removed if a matching higher priority TFT filter in the newly established PDP context takes precedence over a matching lower priority TFT filter in another PDP context. Furthermore, if IP Flow(s), which have previously been bound to the modified bearer are be bound to other bearer(s), PCC Rules may need to be installed in other bearers. For GPRS, an IP flow may be bound to another PDP context if it was previously bound to the modified PDP context due to a removed higher priority TFT filter, and a lower priority TFT filter in the other PDP context matches the IP flow.

5.
If the PCRF requires subscription-related information and does not have it, the PCRF sends a request to the SPR in order to receive the information.

6.
The SPR replies with the subscription related information containing the information about the allowed service(s) and PCC Rules information.

NOTE:
For steps 5 and 6: The details associated with the Sp reference point are not specified in this Release. The SPR’s relation to existing subscriber databases is not specified in this Release.
7.
For IP CAN session modification, if the AF requested a notification of the corresponding event at the initial authorisation of the AF session, the PCRF shall sent a Diameter RAR with the Specific-Action AVP set to indicate the trigger event that caused the request.

8.
If step 7 happens, the AF replies with a Diameter RAA and may provide updated service information within.

9.
The PCRF selects the new PCC Rule(s) to be installed. The PCRF can also identify existing PCC Rules that need to be modified or removed. The PCC Rules may relate to any of the matching AF sessions identified in step 4 or may exist in the PCRF without matching to any AF session. The PCRF may also make a policy decision by defining an authorized QoS and by deciding whether service flows described in the PCC Rules are to be enabled or disabled. 
For types of IP-CAN, where the PCRF controls IP-CAN Bearers, e.g. GPRS, the PCC Rules may affect the IP-CAN Bearer identified in the CCR of step 2 or any other IP-CAN Bearer identified in step 4.

10.
The PCRF stores the modified PCC Rules.

11.
The PCC Rules are provisioned by the PCRF to the GW using Diameter CCA. The PCRF may also provide authorized QoS. The PCRF identifies the affected IP-CAN Bearer for each of the PCC Rules and the authorized QoS. The PCRF may provision PCC Rules and authorized QoS for several IP-CAN Bearers within the same CCA.

12.
The GW installs the received PCC Rules. The GW also enforces the authorized QoS and enables or disables service flow according to the flow status of the corresponding PCC Rules.

13.
The GW sends a response to the IP-CAN Bearer signalling in step 1.
For GPRS, the GGSN accepts the secondary Establish PDP Context Request or the Update PDP Context Request based on the results of the authorisation policy decision enforcement and sends an Establish PDP Context Response or Update PDP Context Response. If the requested QoS parameters do not correspond to the authorized QoS, the GGSN adjusts (downgrades/upgrades) the requested UMTS QoS parameters to the authorized values.


The PCRF may have decided in step 4 to modify PCC Rules and/or authorized QoS of other IP CAN bearers than the IP-CAN Bearer identified in the CCR of step 2. For each such other IP-CAN Bearer identified in step 4, the GGSN executes the following steps.

14.
The PCRF may start a timer to wait for PDP context modification requests from the UE.

15.
The PCRF interacts with the GW according to figure 4.3.1.1.1.

D.3.1.2.2
UE-initiated IP-CAN Bearer Termination

This clause is applicable if an IP-CAN Bearer is being released while other IP-CAN Bearers and thus the IP-CAN Session are not released.
For the termination of IP-CAN Bearers, three cases are covered:

-
Bearer release that does not cause service data flow(s) within an AF session to be disabled;

-
Bearer release that causes at least one but not all the service data flow(s) within an AF session to be disabled; and

-
Bearer release that causes all the service data flows within an AF session to be disabled.

A Bearer release may not cause a service data flow within this bearer to be disabled if the IP flow can be bound to another bearer. For GPRS, an IP flow can be bound to another PDP context if a lower precedence TFT filter matching the IP flow is installed at the other PDP context.


[image: image88.emf] 

13a.   Diameter AAA  

12a. Diameter AAR  

11. Diameter ASA  

10. Diameter ASR  

GW   PCRF   AF  

2.    Diameter CCR  

If all the  service  data  flows within AF  session are affected  

6.    Diameter CCA  

If not all  the service  data  flows within AF  session are affected  

11a. Diameter RAA  

10a. Diameter RAR  

13. Diameter  STA  

12. Diameter STR  

3.    Identify the IP flows  bound to the session  or bearer.   Re - evaluate binding .  

1 .    Remove IP - CAN  Bearer Request   

7.    Remove PCC Rules   

8.    Remove IP - CAN  Bearer Response   

  I f IP Flow(s)  were moved to  other bearer(s)  

4.    PCC Rules Decision.   Policy Decision  

5.    Store PCC Rules   

9.    Install PCC Rules.  Policy  E nforcement  

  I f IP Flow(s)  were moved to  other bearer(s)    

Legend :                 Ma ndatory                            Conditional   


Figure D.3.1.2.2: UE-Initiated IP-CAN Bearer Termination

1.
The GW receives a Remove IP-CAN Bearer Request that request the deactivation of an IP-CAN Bearer while other IP-CAN Bearers and thus the IP-CAN Session are not released. The form of the Remove IP-CAN Bearer Request depends upon the type of the IP-CAN. For GPRS, the GGSN receives a Delete PDP Context Request.

2.
The GW sends a Diameter CCR message with the CC-Request-Type AVP set to the value UPDATE_REQUEST to the PCRF, indicating the IP-CAN Bearer termination.

3.
The PCRF identifies the IP flows bound to the removed bearer and updates the stored bearer information. The PCRF re-evaluates the binding of IP flows, as IP flows may now be bound to other bearers. For GPRS, an IP flow may be bound to another PDP Context if it was previously bound to the removed PDP context due to a higher priority TFT filter, and a lower priority TFT filter in another PDP context matches the IP flow.

The following steps 4 and 5 are performed for each of the other bearers identified in step 3:

4.
The PCRF selects the PCC Rule(s) to be installed or modified for the affected bearer. The PCRF may also update the policy decision for this bearer.

5.
The PCRF stores the updated PCC Rules for the affected bearer.

6.
The PCRF acknowledges the bearer termination by sending a Diameter CCA message. 
The PCRF provides PCC Rules and possibly updated authorized QoS for each of the other bearers identified in step 3. The PCRF identifies the affected IP-CAN Bearer for each of the PCC Rules and the authorized QoS.

7.
The GW removes those PCC Rules, which have not been moved to other IP CAN bearers by the CCA message and are installed in the IP-CAN bearer, for which a termination has been requested in step 1.

8.
The GW sends a Remove IP-CAN Bearer Response. For GPRS, the GGSN sends the Delete PDP Context Response message.

9.
It the PCRF has provided PCC Rules and possibly updated authorized QoS for other bearers in step 6, the GW installs or modifies the identified PCC Rules. The GW also enforces the authorized QoS and enables or disables service flow according to the flow status of the corresponding PCC Rules.


The following steps 10 to 13 or 10a to 13a apply for the case where at least one IP Flow within an AF session is being disabled, i.e. if the IP Flow is not bound to any other bearer that is still established. The steps shall be performed separately for each ongoing AF session that is affected by the bearer release as explained below.


If all IP flow(s) within the AF session are disabled by the bearer release:

10.
The PCRF indicates the session abort to the AF by sending a Diameter ASR message to the AF.

11.
The AF responds by sending a Diameter ASA message to the PCRF.

12.
The AF sends a Diameter STR message to the PCRF to indicate that the session has been terminated.

13.
The PCRF responds by sending a Diameter STA message to the AF.


If at least one but not all of the IP flow(s) within the AF session are disabled by the bearer release, and the AF has requested notification of bearer removal:

10a.
The PCRF indicates the release of the bearer by sending a Diameter RAR to the AF.

11a.
The AF responds by sending a Diameter RAA to the PCRF.

12a.
The AF may send an AAR to the PCRF to update the session information.

13a.
If step 12a occurs, the PCRF responds by sending a AAA to the AF.
Annex E (normative):
Fixed Broadband Access Interworking with EPC

E.1
General

The present annex defines specific requirements for Fixed Broadband Access Interworking with EPC.
E.2
Definitions and abbreviations
E.2.1
Definitions
UE local IP address is defined as: either the public Ipv4 address and/or Ipv6 address/Ipv6 network prefix assigned to the UE by the BBF domain in the no-NAT case, or the public Ipv4 address assigned by the BBF domain to the NATed RG that is used for this UE.

H(e)NB local IP address is defined as: either the public Ipv4 address and/or Ipv6 address/Ipv6 network prefix assigned to the H(e)NB by the BBF domain in the no-NAT case, or the public Ipv4 address assigned by the BBF domain to the NATed RG that is used for this H(e)NB.
Non-seamless WLAN offload (NSWO) is defined as: a capability of routing specific IP flows over the WLAN access without traversing the EPC as defined in clause 4.1.5 of 3GPP TS 23.402 [21].
Non-seamless WLAN offload APN (NSWO-APN) is defined as: an APN allowing the BPCF to indicate to PCRF that for subscribers of a certain HPLMN the IP-CAN session is related to NSWO traffic.
EPC-routed traffic is defined as: User plane traffic that is routed via a PDN GW in EPC as part of a PDN Connection. EPC-routed traffic applies to non-roaming, roaming with home routed and roaming with visited access cases.
E.2.2
Abbreviations
The following abbreviations are relevant for this annex only:

BBF
Broadband Forum

BPCF
Broadband Policy Control Function
NA(P)T
Network Address (Port) Translation
NSWO
Non-Seamless WLAN offload

NSWO-APN
Non-Seamless WLAN offload APN
RG
Residential Gateway
E.3
Binding Mechanisms
E.3.1
EPC-routed traffic
For EPC- routed traffic, binding mechanisms apply as defined insub clause 5.1 by PCRF, PCEF and BBERF. In addition, if both a Gx and associated S9a session exist for the same IP-CAN session, the PCRF shall generate QoS Rules for all the authorized PCC rules.
E.3.2
NSWO traffic

The binding mechanism includes two steps for the NSWO traffic:

1.
Session binding.

2.
PCC rule authorization.

For NSWO traffic, session binding of AF session to an IP-CAN session is performed by the PCRF for the purpose of policy control in the Fixed Broadband access network.
When the PCRF accepts an AA-Request from the AF over the Rx interface with service information, the PCRF shall perform session binding and associate the described service IP flows within the AF session information (and therefore the applicable PCC rules) to one and only one existing IP-CAN session. This association is done comparing the user IP address received via the Rx interface in either the Framed-IP-Address AVP or the Framed-Ipv6-Prefix AVP with the Ipv4 address or Ipv6 address/Ipv6 prefix received via the S9a or S9 interface. The user identity if present in the Subscription-Id AVP and the PDN information if available in the Called-Station-Id AVP may also assist on this association.

The PCRF determines that the UE has an IP-CAN session if the IP address (Ipv4 or Ipv6) received over the Rx interface matches the Ipv4 address or Ipv6 address/Ipv6 prefix received via one or more of the following interfaces: S9a interface and S9 interface, and if the user identity is used to assist the association, the user identity received over the Rx interface matches the user identity received via one or more of the following interfaces: S9a interface and S9 interface.

NOTE 1:
In case the user identity in the IP-CAN and the application level identity for the user are of different kinds, the PCRF needs to obtain the mapping between the identities. Such mapping is not subject to specification within this TS.

NOTE 2: An Ipv6 address provided over Rx matches an Ipv6 prefix provided over S9a or S9 if the Ipv6 address belongs to the Ipv6 (sub-)network prefix.
As a result from the session binding function, the PCRF identifies what IP-CAN session the current AF session is related with. If the PCRF is not capable of executing the Session Binding, the PCRF shall issue an AA-Answer command to the AF with a negative response.
NOTE:
For roaming cases, the H-PCRF performs session binding of the AF session to an IP-CAN session.

The PCRF derives and authorises PCC rules as described in clause 5.
E.4
PCC Procedures

E.4.1
Introduction
From the network scenarios listed in clause 4.0, in order to support interworking with Fixed Broadband access network, three distinct network scenarios are defined as follows:

 -
the Case 1 (no Gateway Control Session over Gxx reference point) applies to GTP-based S2a or GTP-based S2b, trusted S2c and GTP-based S5/S8 H(e)NB scenarios.
-
the Case 2a (the same Gateway Control Session over Gxx reference point) applies to untrusted S2c.

-
the Case 2b (a Gateway Control Session over Gxx reference point per IP-CAN Session) applies to PMIP-based S2b and PMIP-based S5/S8 H(e)NB scenarios.
NOTE:
No policy interworking solution based on S9a is defined for Fixed Broadband access interworking via S2a in this Release.
Additionally, for case 1, the PCRF checks whether the CoA information is included in the CC-Request command to differentiate the GTP-based S2b case and trusted S2c case. If it is included, the trusted S2c case applies; otherwise, the GTP-based S2b case applies.
E.4.2
IP-CAN Session Establishment

E.4.2.1
IP-CAN Session Establishment for EPC- routed traffic
This procedure is applicable for WLAN and H(e)NB scenarios for EPC-routed traffic. This procedure is same as described in clause 4.1 with the exceptions described in this clause.


[image: image89.emf]R

o

a

m

i

n

g

 

c

a

s

e

(

V

i

s

i

t

e

d

 

A

c

c

e

s

s

,

 

c

a

s

e

 

2

a

 

o

r

 

c

a

s

e

 

2

b

)

4d.Diameter CCR

4e. Store 

information

6a.Diameter CCR

6a.Diameter CCA

6.Diameter CCA

6b.Diameter CCA

R

o

a

m

i

n

g

 

c

a

s

e

(

V

i

s

i

t

e

d

 

A

c

c

e

s

s

,

 

c

a

s

e

 

2

a

 

o

r

 

c

a

s

e

 

2

b

)

4a.Diameter CCR

R

o

a

m

i

n

g

 

c

a

s

e

(

H

o

m

e

 

R

o

u

t

e

d

 

a

n

d

 

c

a

s

e

 

1

)

2b.Diameter TER/RAR

2.Diameter TER/RAR

4.Diameter CCR

BBERF V-PCRF

H-PCRF TDF BPCF SPR OCS PCEF

1. Perform step 1 through step 6: as specified in Figure 4.1.1: IP-CAN session establishment 

2a.Diameter TER/RAR

2c.Diameter TER/RAR

3.Diameter TEA/RAA

3b.Diameter TEA/RAA

3a.Diameter TEA/RAA

3c.Diameter TEA/RAA

4c.Diameter CCR

7. Perform step 7 through step 15: as specified in Figure 4.1.1: IP-CAN session establishment 

5. Store information 

4b. Store 

information

R

o

a

m

i

n

g

 

c

a

s

e

(

H

o

m

e

 

R

o

u

t

e

d

 

a

n

d

 

c

a

s

e

 

1

)

R

o

a

m

i

n

g

 

c

a

s

e

(

V

i

s

i

t

e

d

 

A

c

c

e

s

s

,

 

c

a

s

e

 

2

a

 

o

r

 

c

a

s

e

 

2

b

)

R

o

a

m

i

n

g

 

c

a

s

e

(

H

o

m

e

 

R

o

u

t

e

d

 

a

n

d

 

c

a

s

e

 

1

)

R

o

a

m

i

n

g

 

c

a

s

e

(

V

i

s

i

t

e

d

 

A

c

c

e

s

s

,

 

c

a

s

e

 

2

a

 

o

r

 

c

a

s

e

 

2

b

)

R

o

a

m

i

n

g

 

c

a

s

e

(

H

o

m

e

 

R

o

u

t

e

d

 

a

n

d

 

c

a

s

e

 

1

)

6c.Diameter CCA


Figure E.4.2.1.1 IP-CAN Session Establishment for EPC-routed traffic
1. Step 1 through step 6: as specified in Figure 4.1.1: IP-CAN session establishment are executed towards the H-PCRF (non-roaming case or home routed case) or towards the V-PCRF (roaming case).
For case 2a and case 2b of the WLAN scenario and for case 2b of the H(e)NB scenario, the BBERF provides the data as described in 3GPP TS 29.212 [9], clause E.5.2.


For case 1, the PCEF provides the data as described in 3GPP TS 29.212 [9], clause E.5.1.


For case 1 (visited access), case 2a and case 2b (home routed and visited access) the V-PCRF forwards the data towards the H-PCRF.


NOTE: For the roaming case, the V-PCRF omits the UE Local IP address (WLAN scenario), the H(e)NB Local IP address and the UDP port in the S9 reference point.

2. For GTP/PMIP-based S2b of the WLAN scenario (non-roaming case) the PCRF sends a TER command to BPCF to trigger an S9a session establishment procedure triggered by the Gateway Control session establishment or Indication of IP-CAN session establishment in step 1.
For trusted and untrusted S2c of the WLAN scenario (non roaming case), when there is not an already established S9a session for the user, the PCRF sends a TER command to BPCF to trigger an S9a session establishment procedure triggered by the Gateway Control session establishment in step 1; otherwise, the PCRF may send a RAR command to BPCF to provide the QoS rules to the BPCF.


For H(e)NB scenarion (non roaming case), when there is not an already established S9a session for the H(e)NB Local IP address, the PCRF sends a TER command to BPCF to trigger an S9a session establishment procedure triggered by the Gateway Control session establishment or Indication of IP-CAN session establishment in step 1; otherwise, the PCRF may send a RAR command to BPCF to provide the QoS rules to the BPCF.

The PCRF provides the data as described in 3GPP TS 29.215 [22], clause A.5.1.1
For case 1 (roaming with home routed case), the steps 2a and step 2b are executed instead of step 2.
2a. If there is not an already established S9 session for the user, the H-PCRF sends a TER command to V-PCRF to trigger an S9 session establishment procedure triggered by the Indication of IP-CAN session establishment in step 1; otherwise, the H-PCRF sends a RAR command to V-PCRF to trigger an S9 subsession establishment procedure.


The H-PCRF trigger an S9 session/subsession establishment procedure as described in 3GPP TS 29.215 [22], clause A.6.1.1.1 or A.6.3.1.0.
2b. For GTP-based S2b of the WLAN scenario, the V-PCRF sends a TER command to BPCF to trigger an S9a session establishment procedure.

For trusted S2c of the WLAN scenario, if there is not an already established S9a session for the user, the V-PCRF sends a TER command to BPCF; otherwise, the PCRF may send a RAR command to BPCF to provide the QoS rules to the BPCF.

For the H(e)NB scenario and if there is not an already established S9a session for the H(e)NB local IP address, the V-PCRF sends a TER command to BPCF to trigger an S9a session establishment procedure; otherwise the V-PCRF may send a RAR command to BPCF to provide the QoS rules to the BPCF.

The V-PCRF provides the data as described in 3GPP TS 29.215 [22], clause A.5.1.1.
For case 1 (visited access) and for case 2a and case 2b (visited access or home routed case), step 2c is executed instead of step 2.

2c. For trusted and untrusted S2c of the WLAN scenario when there is not an already established S9a session for the user, the V-PCRF sends a TER command to BPCF to trigger an S9a session establishment procedure; otherwise, the PCRF may send a RAR command to BPCF to provide the QoS rules to the BPCF.


For GTP/PMIP-based S2b of the WLAN scenario the V-PCRF sends a TER command to BPCF to trigger an S9a session establishment procedure.


For case 1 and case 2b of the H(e)NB scenario when there is not an already established S9a session for the H(e)NB local IP address, the V-PCRF sends a TER command to BPCF to trigger an S9a session establishment procedure; otherwise, the V-PCRF may send a RAR command to provide the QoS rules to the BPCF.
The V-PCRF provides the data as described in 3GPP TS 29.215 [22], clause A.5.1.1.

3. The BPCF acknowledges to the PCRF by sending the TEA /RAAcommand.

For case 1(roaming with home routed case), steps 3a and step 3b are executed instead of step 3.

3a. The BPCF acknowledges to the V-PCRF by sending the TEA /RAAcommand.

3b. The V-PCRF acknowledges to the H-PCRF by sending the TEA/RAA command.
For case 1 (visited access) and for case 2a and case 2b (visited access or home routed case), step 3c is executed instead of step 3.
3c. The BPCF acknowledges to the V-PCRF by sending the TEA/RAA command.
4. For the non-roaming case, triggered by step 2, the BPCF initiates an S9a session establishment with the PCRF by sending a CCR to the PCRF with the CC-Request-Type AVP set to the value INITIAL_REQUEST. The BPCF provides the data as described in 3GPP TS 29.215 [22], clause A.5.1.2.

For case 1 (roaming with home routed case), the steps 4a~4c are executed instead of step 4.
4a. Triggered by step 2b, the BPCF initiates an S9a session establishment with the V-PCRF by sending a CCR to the V-PCRF with the CC-Request-Type AVP set to the value INITIAL_REQUEST. The BPCF provides the data as described in 3GPP TS 29.215 [22], clause A.5.1.2.

4b. The V-PCRF determines that the request is for a roaming user and stores the received information.
4c. If there is not an already established S9 session for the user, the V-PCRF sends a CCR to the H-PCRF with the CC-Request-Type AVP set to the value INITIAL_REQUEST. The V-PCRF includes the Subsession-Enforcement-Info AVP within the CCR with a new S9 subsession identifier assigned by the V-PCRF to this IP-CAN session within the Subsession-Id AVP, and the Subsession-Operation AVP set to the value ESTABLISHMENT. If there is an already established S9 session for this roaming user, the V-PCRF sends a CCR to the H-PCRF with the CC-Request-Type AVP set to the value UPDATE_REQUEST. The V-PCRF includes the Subsession-Enforcement-Info AVP within the CCR with a new S9 subsession identifier assigned by the V-PCRF to this IP-CAN session within the Subsession-Id AVP, and the Subsession-Operation AVP set to the value ESTABLISHMENT.
For case 1 (visited access) and for case 2a and case 2b (visited access or home routed case), the steps 4d~step 4e are executed instead of step 4.
4d. Triggered by step 2c, the BPCF initiates an S9a session establishment with the V-PCRF by sending a CCR to the V-PCRF with the CC-Request-Type AVP set to the value INITIAL_REQUEST. The BPCF provides the data as described in 3GPP TS 29.215 [22], clause A.5.1.2.

4e. The V-PCRF determines that the request is for a roaming user and stores the received information.
5. The H-PCRF stores the information received in the CCR.

6. For the non-roaming case, the H-PCRF acknowledges the S9a session establishment by sending a CCA to the BPCF.

For case 1 (roaming with home routed case), steps 6a~6b are executed instead of step 6.

6a. The H-PCRF acknowledges the S9 session establishment/modification by sending a CCA to the V-PCRF.
6b. The V-PCRF acknowledges the S9a session establishment by sending a CCA to the BPCF.


For case 1 (visited access) and for case 2a and case 2b (visited access or home routed case), the step 6c is executed instead of step 6.

6c. The V-PCRF acknowledges the S9a session establishment by sending a CCA to the BPCF.
7. Step 7 through step 15: as specified in Figure 4.1.1: IP-CAN session establishment are executed. Step 13 is only applicable to case 2b of H(e)NB scenario.
E.4.2.2
IP-CAN Session Establishment for NSWO traffic

[image: image90.emf]R

O

A

M

I

N

G

 

C

A

S

E

H-PCRF BPCF

5a. Diameter CCA

4. Perform step 5 through 11: as specified in Figure 

4.1.1: IP-CAN session establishment

3. Store Information

SPR

Legend:

Mandatory

Conditional 

V-PCRF

2b. Store Information

5b. 

Store PCC Rules. 

if available, Policy Decision 

5f. Diameter CCA

2c. Diameter CCR

2.  Diameter CCR

2a. Diameter CCR

5.  Diameter CCA

R

O

A

M

I

N

G

 

C

A

S

E

1.  Trigger

TDF

5c. Diameter CCR

5d. Diameter CCA

5e. TDF session establishment


Figure E.4.2.2.1: IP-CAN Session Establishment for NSWO traffic

1. The broadband access network becomes aware of the IMSI of the 3GPP UE if 3GPP-based access authentication (EAP-AKA/AKA’) is performed. The BPCF also becomes aware of the UE local IP address.
2. For the non-roaming case, the BPCF initiates an S9a* session establishment procedure with the H-PCRF by sending a CCR using the CC-Request-Type AVP set to the value INITIAL_REQUEST. The BPCF shall provide the data as described in 3GPP TS 29.215 [22], clause A.5.1.2.1.
For the roaming case, steps 2a~2c are executed instead of step 2.
2a.The BPCF initiates an S9a* session establishment procedure with the V-PCRF by sending a CCR to the V-PCRF using the CC-Request-Type AVP set to the value INITIAL_REQUEST. The BPCF shall provide the data as described in 3GPP TS 29.215 [22], clause A.5.1.2.1.
2b. The V-PCRF determines that the request is for a roaming user and stores the received information.
2c. If there is not an already established S9 session for this roaming user, the V-PCRF sends a CCR to the H-PCRF with the CC-Request-Type AVP set to the value INITIAL_REQUEST. The V-PCRF includes the Subsession-Enforcement-Info AVP within the CCR with a new S9 subsession identifier assigned by the V-PCRF to this IP-CAN session within the Subsession-Id AVP, and the Subsession-Operation AVP set to the value ESTABLISHMENT.
If there is an already established S9 session for this roaming user, the V-PCRF sends a CCR to the H-PCRF with the CC-Request-Type AVP set to the value UPDATE_REQUEST. The V-PCRF includes the Subsession-Enforcement-Info AVP within the CCR with a new S9 subsession identifier assigned by the V-PCRF to this IP-CAN session within the Subsession-Id AVP, and the Subsession-Operation AVP set to the value ESTABLISHMENT.

3. The H-PCRF stores the information received in the CCR.
4. 
Perform step 5 through 11: as specified in Figure 4.1.1: IP-CAN session establishment. Additionally, an indication on whether policy control for NSWO traffic should be performed for the UE may be retrieved from the SPR. The H-PCRF enables policy control for NSWO traffic for that UE based on operator policies and user profile information that may depend on e.g. NSWO-APN being used by the UE.

5. 
For the non-roaming case, the H-PCRF provisions the PCC Rules to the BPCF using CCA if policy control is enabled.

For the roaming case, steps 5a~5f are executed instead of step 5.
5a. The H-PCRF provisions the PCC Rules if available to the V-PCRF using CCA if policy control is enabled. The H-PCRF includes PCC Rules and ADC Rules if available in the Subsession-Decision AVP of the CCA, along with the S9 subsession identifier as received in step 2c within the Subsession-Id AVP.
5b.
If policy control is enabled in step 5a, the V-PCRF stores the received PCC rules if available. The V-PCRF enforces visited operator policies regarding QoS authorization requested by the H-PCRF as indicated by the roaming agreements.
5c.
The V-PCRF informs the H-PCRF when a request has been denied and may provide the acceptable QoS information for the service.

5d.
The H-PCRF acknowledges the CCR and may additionally include new or modified PCC rules to the V-PCRF. When user profile configuration indicates that Application Detection and Control function is enabled, the H-PCRF may additionally include new or modified PCC rules for application detection and control.

5e. In case of solicited application reporting with a TDF, the V-PCRF initiates a TDF Session Establishment procedure, according to clause 4.6.1, with the selected TDF.
5f.
The V-PCRF provisions PCC rules received from the H-PCRF to the BPCF by using CCA.
E.4.3
IP-CAN Session Termination

E.4.3.1
IP-CAN Session Termination for EPC- routed traffic

[image: image91.emf]5c.Diameter CCA

R

o

a

m

i

n

g

 

c

a

s

e

(

V

i

s

i

t

e

d

 

A

c

c

e

s

s

,

 

c

a

s

e

 

2

a

 

o

r

 

c

a

s

e

 

2

b

)

3c.Diameter RAA

3.Diameter RAA

3a.Diameter RAA

3b.Diameter RAA

2c.Diameter RAR

BBERF V-PCRF

H-PCRF

TDF BPCF SPR OCS

PCEF

6. Perform the step 3 through step 7 :as specified in Figure 4.2.1.1: UE-initiated IP-CAN session termination-AF located in HPLMN

H/V-AF

2.Diameter RAR

2b.Diameter RAR

4a.Diameter CCR

4.Diameter CCR

5.Diameter CCA

2a.Diameter RAR

4b.Diameter CCR

5b.Diameter CCA

5a.Diameter CCA

8. Perform the step 12 through step 15 :as specified in Figure 4.2.1.1: UE-initiated IP-CAN session termination-AF located in 

HPLMN

1. 

Perform the step 1 through step 3: as defined in Figure 4.2.1.1: UE-initiated IP-CAN session termination-AF located in HPLMN,  step 1 

through step 5: as defined in Figure 4.2.2.1: PCEF-initiated IP-CAN session termination-AF located in HPLMN or step 1 through step 8: as 

defined in Figure 4.2.3.1: PCRF-initiated IP-CAN session termination-AF located in HPLMN

7. Perform the step 8 through step 11 :As Specified 

in Figure 4.2.1.1 for AF located in HPLMN or step 2 

through step 9  :as specified in Figure 4.2.1.2 for 

AF located in VPLMN

R

o

a

m

i

n

g

 

c

a

s

e

(

H

o

m

e

 

R

o

u

t

e

d

 

a

n

d

 

c

a

s

e

 

1

)

R

o

a

m

i

n

g

 

c

a

s

e

(

H

o

m

e

 

R

o

u

t

e

d

 

a

n

d

 

c

a

s

e

 

1

)

R

o

a

m

i

n

g

 

c

a

s

e

(

H

o

m

e

 

R

o

u

t

e

d

 

a

n

d

 

c

a

s

e

 

1

)

R

o

a

m

i

n

g

 

c

a

s

e

(

V

i

s

i

t

e

d

 

A

c

c

e

s

s

,

 

c

a

s

e

 

2

a

 

o

r

 

c

a

s

e

 

2

b

)

R

o

a

m

i

n

g

 

c

a

s

e

(

V

i

s

i

t

e

d

 

A

c

c

e

s

s

,

 

c

a

s

e

 

2

a

 

o

r

 

c

a

s

e

 

2

b

)

R

o

a

m

i

n

g

 

c

a

s

e

(

V

i

s

i

t

e

d

 

A

c

c

e

s

s

,

 

c

a

s

e

 

2

a

 

o

r

 

c

a

s

e

 

2

b

)


Figure E.4.3.1.1: IP-CAN Session Termination for EPC-routed traffic

1. Perform the step 1 through step 3: as defined in Figure 4.2.1.1: UE-initiated IP-CAN session termination-AF located in HPLMN, step 1 through step 5: as defined in Figure 4.2.2.1: PCEF-initiated IP-CAN session termination-AF located in HPLMN or step 1 through step 8: as defined in Figure 4.2.3.1: PCRF-initiated IP-CAN session termination-AF located in HPLMN.
2. For GTP/PMIP-based S2b of the WLAN scenario (non-roaming case), triggered by a Gateway control session termination from BBERF(ePDG) or by an Indication of IP-CAN session termination from PCEF, the H-PCRF sends a RAR including the Session-Release-Cause AVP to request that the BPCF terminates the S9a session.
For trusted and untrusted S2c of the WLAN scenario (non-roaming case), triggered by the Indication of IP-CAN session termination, the H-PCRF sends a RAR including the Session-Release-Cause AVP to request that the BPCF terminates the S9a session when the last PDN connection is released for this user; otherwise, the H-PCRF sends a RAR to BPCF to remove all the QoS rules related to the released PDN connection.


For H(e)NB scenario (non-roaming case), triggered by a Gateway control session termination from BBERF(S-GW) or by an indication of IP-CAN session termination, the H-PCRF sends a RAR including the Session-Release-Cause AVP to request that the BPCF terminates the S9a session when last PDN connection is released for this H(e)NB Local IP address; otherwise, the H-PCRF sends a RAR to BPCF to remove all the QoS rules related to the released PDN connection.

For case 1 (roaming with home routed case), the steps 2a and step 2b are executed instead of step 2.
2a. If the subsession being terminated is the last subsession over S9, the H-PCRF sends a RAR including the Session-Release-Cause AVP to the V-PCRF to indicate the termination of the S9 session. Otherwise, the H-PCRF sends a RAR to the V-PCRF including the Subsession-Decision-Info AVP with the Session-Release-Cause AVP to indicate the request for terminating the S9 subsession corresponding to the IP-CAN session.
2b. For GTP-based S2b of the WLAN scenario, the V-PCRF sends a RAR including the Session-Release-Cause AVP to the BPCF to indicate the request for terminating the S9a session.

For trusted S2c of the WLAN scenario, the V-PCRF sends a RAR including the Session-Release-Cause AVP to the BPCF to indicate the request for terminating the S9a session when the last PDN connection is released for this user; otherwise, the V-PCRF sends a RAR to BPCF to remove all the QoS rules related to the released PDN connection.


For the H(e)NB scenario, the V-PCRF sends a RAR including the Session-Release-Cause AVP to the BPCF to indicate the request for terminating the S9a session when the last PDN connection is released for this H(e)NB Local IP address; otherwise, the V-PCRF sends a RAR to BPCF to remove all the QoS rules related to the released PDN connection.
For case 1 (visited access) and for case 2a and case 2b (visited access or home routed case), step 2c is executed instead of step 2.
2c. For GTP/PMIP-based S2b of the WLAN scenario, triggered by a Gateway control session termination from BBERF(ePDG) or by an Indication of IP-CAN session termination, the V-PCRF sends a RAR including the Session-Release-Cause AVP to request that the BPCF terminates the S9a session.
For trusted and untrusted S2c of the WLAN scenario, triggered by the Indication of IP-CAN session termination or S9 session/subsession termination request from the H-PCRF , the V-PCRF sends a RAR including the Session-Release-Cause AVP to request that the BPCF terminates the S9a session when the last PDN connection is released for this user; otherwise, the V-PCRF sends a RAR to BPCF to remove all the QoS rules related to the released PDN connection

For case 1 and case 2b of H(e)NB scenario, triggered by a Gateway control session termination from the BBERF(S-GW) or by an Indication of IP-CAN session termination, the V-PCRF sends a RAR including the Session-Release-Cause AVP to request that the BPCF terminates the S9a session when the last PDN connection is released for this H(e)NB Local IP address; otherwise, the H-PCRF sends a RAR to BPCF to remove all the QoS rules related to the released PDN connection.
3. For the non-roaming case, the BPCF sends a RAA to acknowledge the RAR.
For case 1 (roaming with home routed case), the steps 3a and 3b are executed instead of step 3.

3a. The BPCF sends a RAA to the V-PCRF.

3b. The V-PCRF sends a RAA to the H-PCRF.
For case 1 (visited access) and for case 2a and case 2b (visited access or home routed case), step 2c is executed instead of step 2.
3c. The BPCF sends a RAA to the V-PCRF.
4. For the non-roaming case, the BPCF sends a CCR to the H-PCRF to indicate the S9a session termination if the H-PCRF requests that the BPCF terminates the S9a session in step 2. The BPCF requests the termination of the S9a session using the CC-Request-Type set to the value TERMINATION_REQUEST.
For case 1(roaming with home routed case), the steps 4a and 4b are executed instead of step 4

4a. The BPCF sends a CCR to the V-PCRF to indicate the S9a session termination if the V-PCRF requests that the BPCF terminates the S9a session in step 2b. The BPCF requests the termination of the S9a session using the CC-Request-Type set to the value TERMINATION_REQUEST.

4b. The V-PCRF sends a CCR to the H-PCRF to indicate the S9 session termination if the H-PCRF requests that the V-PCRF terminates the S9 session in step 2a. The V-PCRF requests the termination of the S9 session using the CC-Request-Type set to the value TERMINATION_REQUEST.

For case 1 (visited access) and for case 2a and case 2b (visited access or home routed case), step 4c is executed instead of step 4.
4c. The BPCF sends a CCR to the V-PCRF to indicate the S9a session termination if the V-PCRF request that the BPCF terminates the S9a session in step 2c. The BPCF requests the termination of the S9a session using the CC-Request-Type set to the value TERMINATION_REQUEST.

5. For the non-roaming case, the H-PCRF sends a CCA to acknowledge the CCR.
For case 1 (roaming with home routed case), the steps 5a and 5b are executed instead of step 5

5a. The H-PCRF sends a CCA to acknowledge the CCR.
5b. The V-PCRF sends a CCA to acknowledge the CCR.

For case 1 (visited access) and for case 2a and case 2b (visited access or home routed case), step 5c is executed instead of step 5.
5c. The V-PCRF sends a CCA to acknowledge the CCR.

6. Perform the step 3 through step 7: as defined in Figure 4.2.1.1: UE-initiated IP-CAN session termination-AF located in HPLMN,

7.
Perform the step 8 through step 11 :As Specified in Figure 4.2.1.1 for AF located in HPLMN or step 2 through step 9 :as specified in Figure 4.2.1.2 for AF located in VPLMN

8.
Perform the step 12 through step 15: as defined in Figure 4.2.1.1: UE-initiated IP-CAN session termination-AF located in HPLMN.
E.4.3.2
IP-CAN Session Termination for NSWO traffic
E.4.3.2.1
BPCF-initiated IP-CAN Session Termination for NSWO traffic

[image: image92.emf]R

O

A

M

I

N

G

 

C

A

S

E

H-PCRF BPCF

4a. Diameter CCA

3.Identify AF Session bound 

to the remove IP-CAN 

session

H-AF V-PCRF

2b. TDF session termination

4b. 

Removes the information 

related to the terminated IP-

CAN session 

4c. Diameter CCA

2c. Diameter CCR

2.  Diameter CCR

2a. Diameter CCR

4 Diameter CCA

1.  Trigger

TDF

SPR

Legend:

Mandatory

Conditional 

6.Perform step 8 through 12: as specified in Figure 4.2.1.1: 

UE-Initiated IP-

CAN Session Termination –AF located in the HPLMN

R

O

A

M

I

N

G

 

C

A

S

E

5. TDF session termination


Figure E.4.3.2.1.1: BPCF-initiated IP-CAN Session Termination for NSWO Traffic

1. The Fixed Broadband Access network is aware of the UE is detached from the broadband access network
2. For the non-roaming case, the BPCF sends a CCR to the H-PCRF to indicate the S9a* session termination. The BPCF requests the termination of the S9a* session using the CC-Request-Type set to the value TERMINATION_REQUEST.

For the roaming case, the steps 2a and 2c are executed instead of step 2.
2a. The BPCF sends a CCR to the V-PCRF to indicate the S9a* session termination. The BPCF requests the termination of the S9a* session using the CC-Request-Type set to the value TERMINATION_REQUEST.

2b. If there is an active TDF session between TDF and V-PCRF, the TDF session termination is initiated as defined in clause 4.6.2. For this case, the PCRF described in clause 4.6.2 acts as a V-PCRF.
2c. The V-PCRF sends the CCR to the H-PCRF. If this is the last subsession associated with the S9 session, the V-PCRF sends a CCR to the H-PCRF to request the termination of the S9 session using the CC-Request-Type AVP set to the value TERMINATION_REQUEST. Otherwise, the V-PCRF sends a CCR to the H-PCRF with a CC-Request-Type AVP set to the value UPDATE_REQUEST and a Subsession-Enforcement-Info within which the Subsession-Operation AVP set to value TERMINATION to request the termination of the conresponding S9 subsession.

3. The H-PCRF identifies the AF sessions that are bound to IP flows of the removed IP-CAN Session.
4. For the non-roaming case, the H-PCRF acknowledges the S9a* session termination by sending a CCA to the BPCF.

For the roaming case, steps 4a~4c are executed instead of step 4:
4a. The H-PCRF acknowledges the S9 session or subsession termination by sending a CCA to the V-PCRF.

4b. The V-PCRF removes the information related to the terminated IP-CAN Session.

4c. The V-PCRF acknowledges the S9a* session termination by sending a CCA to theBPCF.
5.  If there is an active TDF session between TDF and H-PCRF, the TDF session termination is initiated as defined in clause 4.6.2.
6. Perform step 8 through step 12: as specified in Figure 4.2.1.1: UE-Initiated IP-CAN Session Termination – AF located in the HPLMN
E.4.3.2.2
PCRF-initiated IP-CAN Session Termination for NSWO traffic

[image: image93.emf]R

O

A

M

I

N

G

 

C

A

S

E

H-PCRF BPCF H-AF V-PCRF

TDF

SPR

Legend:

Mandatory

Conditional 

R

O

A

M

I

N

G

 

C

A

S

E

1. Detection that IP-CAN 

session termination is required

2.Diameter RAR

2a.Diameter RAR

2b.Diameter RAR

3. Remove all the PCC rules

4.Diameter RAA

4a.Diameter RAA

4b.Diameter RAA

5. 

Perform step 2 through 6: as specified in Figure E.4.3.2.1.1:BPCF-initiated IP-CAN session 

Termination for NSWO traffic


Figure E.4.3.2.2.1: PCRF-initiated IP-CAN Session Termination for NSWO Traffic

1.
The H-PCRF detects that the termination of an IP-CAN Session is required.

2.
For the non-roaming case, the H-PCRF sends a RAR including the Session-Release-Cause AVP to request that the BPCF terminates the S9a* session.
For the roaming case, steps 2a~2b are executed instead of step 2:
2a. If the subsession being terminated is the last subsession over S9, the H-PCRF sends a RAR including the Session-Release-Cause AVP to the V-PCRF to indicate the termination of the S9 session. Otherwise, the H-PCRF sends a RAR to the V-PCRF including the Subsession-Decision-Info AVP with the Session-Release-Cause AVP to indicate the request for terminating the S9 subsession corresponding to the IP-CAN session.
2b. The V-PCRF sends a RAR including the Session-Release-Cause AVP to the BPCF.
3.
The Fixed Broadband access network removes all the PCC rules which are applied to the IP-CAN session.

4.
For the non-roaming case, the BPCF sends a RAA to acknowledge the RAR.

For the roaming case, steps 4a~4b are executed instead of step 4:
4a. The BPCF sends a RAA to the V-PCRF.

4b. The V-PCRF sends a RAA to the H-PCRF and acknowledges the request for terminating the S9 session or the S9 subsession corresponding to the IP-CAN session.

5.
Step 2 through 6: as specified in Figure E.4.3.2.1.1:BPCF-initiated IP-CAN session termination for NSWO traffic.
E.4.4
IP-CAN Session Modification

E.4.4.1
IP-CAN Session Modification for EPC-routed traffic

E.4.4.1.1
PCRF-initiated IP-CAN Session Modification
This procedure is applicable both for WLAN and H(e)NB scenario.

[image: image94.emf] 

Roaming Case    

Roaming Case  

BBERF   PCEF   V - PCRF       AF   H - PCRF  

SPR  

  2. Diameter RA R  

4 . Diameter R AA  

Legend :                 Mandatory                       Conditional   

TDF  

1.  Steps 1   to  3   as specified in figure 4.3. 1 .1.1    

5 .  Steps  5   to 1 2   as specified in figure 4.3. 1 .1.1  

OCS   BPCF  

3 .  T ranslates  QoS  rule into access  specific QoS   

  2 a .   Diameter RA R  

  2 c . Diame ter RA R  

4 a . Diameter R AA  

4 b . Diameter R AA  

2b. Perform  authori za tion  


Figure E.4.4.1.1.1: PCRF-Initiated IP-CAN Session Modification for EPC routed traffic

1
Step 1 to 3 as specified in figure 4.3.1.1.1 are executed. The H-PCRF receives an internal or external trigger to update PCC/QoS Rules. External trigger in Step 1 in figure 4.3.1.1.1 only applies to AF session interactions (as described in clause 4.3.1.2) and TDF session interactions (as described in clause 4.3.1.2). For V-PCRF initiated IP-CAN session modification, the V-PCRF always asks the H-PCRF for policy decision.
NOTE:
If the AF/TDF is located in the V-PLMN, upon a request from the AF/TDF, the V-PCRF will proxy the request to the H-PCRF, this may also trigger the PCRF-Initiated IP-CAN Session Modification for EPC routed traffic.
2
The H-PCRF sends a Diameter RAR to update the QoS information to the BPCF.
If the UE is roaming, then steps 2a ~ 2c are executed instead of step 2:

2a.
The H-PCRF sends a Diameter RAR to the V-PCRF to install, modify or remove PCC rules for Visited Access scenario, or QoS Rules for Home Routed scenario.
2b. The V-PCRF performs local authorization of the received PCC rules or QoS rules when necessary.

2c. The V-PCRF sends a Diameter RAR to the BPCF to update the QoS information.

3.
The BPCF shall perform QoS validation and translates the QoS rule as received (i.e. SDF template, QCI, MBR, GBR and ARP) into access specific QoS parameters applicable in the BBF domain (the details of the mapping from 3GPP QoS parameters on S9a to QoS parameters applicable in the BBF domain is out of 3GPP scope).

4.
The BPCF sends RAA to the H-PCRF to acknowledge the RAR and informs the H-PCRF about the outcome of the QoS rule operation. If the BPCF cannot provide the requested QoS by the H-PCRF, the BPCF may respond with the acceptable QoS.

If the UE is roaming, then steps 4a ~ 4b are executed instead of step 4:

4a.
The BPCF sends RAA to the V-PCRF to acknowledge the RAR and informs the V-PCRF about the outcome of the QoS rule operation. If the BPCF cannot provide the QoS requested by the V-PCRF, the BPCF may respond with the acceptable QoS.
4b. The V-PCRF forwards the RAA to the H-PCRF to acknowledge the RAR and informs the H-PCRF about the outcome of the QoS rule operation.

NOTE:
If the H-PCRF is informed that the BPCF cannot provide the requested QoS, the H-PCRF can decide to install, modify or remove QoS rules. In that case, the same flow as described in this clause will be executed.

5.
Step 5 to 12 as specified in figure 4.3.1.1.1 are executed.
E.4.4.1.2
BPCF-initiated IP-CAN Session Modification
This procedure is applicable both for WLAN and H(e)NB scenario.

[image: image95.emf] 

Roaming  Case  

Roaming  Case  

BBERF   PCEF   V - PCRF       AF   H - PCRF  

  2. Diameter  CC R  

3 . Diameter  CC A  

Legend :                 Mandatory                       Conditional   

TDF  

1.  Trigger  

OCS   BPCF  

  2 a . Diameter  CC R  

2b .   Store Information    

  2 c . Diameter  CC R  

3a . Diameter  CC A  

3b . Diameter  CC A  

4 .    PCRF - initiated IP - CAN Session Modification as d escribed  in E.4.4. 1 .1  


Figure E.4.4.1.2.1: BPCF-Initiated IP-CAN Session Modification

1.
The trigger for this procedure is that the Fixed Broadband Access has pre-empted some resources and wants to report a QoS rule failure to the PCRF, or when the Fixed Broadband Access network cannot sustain the bandwidth allocated to a particular traffic class/DSCP aggregate.
2.
The BPCF sends a Diameter CCR to the H-PCRF with the CC-Request-Type AVP set to the value UPDATE_REQUEST to report QoS Rule failure.

When the UE is in roaming case, steps 2a ~ 2c are executed instead of step 2:

2a.
The BPCF sends a Diameter CCR to the V-PCRF with the CC-Request-Type AVP set to the value UPDATE_REQUEST to report QoS Rule failure.

2b. The V-PCRF stores the information received.
2c. For Visited Access scenario, the V-PCRF sends a Diameter CCR to the H-PCRF to report PCC Rule failure. For Home Routed scenario, the V-PCRF sends a Diameter CCR to the H-PCRF to report QoS Rule failure.

3
The H-PCRF sends a Diameter CCA to the BPCF to acknowledge the CCR command.
When the UE is in roaming case, steps 3a ~ 3b are executed instead of step 3:

3a.
The H-PCRF sends a Diameter CCA to the V-PCRF to acknowledge the CCR command.
3b. The V-PCRF forwards the Diameter CCA to the BPCF to acknowledge the CCR.
4.
The PCRF-initiated IP-CAN Session Modification as described in E.4.4.1.1 may take place.
E.4.4.1.3
PCEF-initiated IP-CAN Session Modification
This procedure is applicable both for WLAN and H(e)NB scenario.

[image: image96.emf] 

Roaming Case    

Roaming Case  

BBERF   PCEF   V - PCRF       AF   H - PCRF  

SPR  

  2. Diameter RA R  

4 . Diameter R AA  

Legend :                 Mandatory                       Conditional   

TDF  

1.  Steps 1   to  3   as specified in figure 4.3.2.1.1    

5 .  Steps  4   to 1 8   as specified in figure 4.3.2.1.1  

OCS   BPCF  

3 .  T ranslates QoS rule  into access  specific QoS  

  2 a .   Diameter RA R  

  2 c . Diameter RA R  

4 a . Diameter R AA  

4 b . Diameter R AA  

2b.  Q os   rule  Validation   


Figure E.4.4.1.3.1: PCEF-Initiated IP-CAN Session Modification
1.
Step 1 to 3 (3a-3c for the Visited Access case) is the same as specified in figure 4.3.2.1.1. In step 3 (3c for the Visited Access case), when the UE Local IP address, H(e)NB Local IP address or the UDP port number is changed, the PCEF may provide the updated UE local IP address, the updated H(e)NB IP address, and/or UDP port number if available, to the PCRF; or the PCEF cannot enforce the PCC rule(s) and need report the PCC rule failure to the PCRF.

2.
The H-PCRF sends a Diameter RAR to request that the BPCF installs, modifies or removes QoS Rules (i.e. SDF template, QCI, ARP, MBR and GBR).

When the UE is in roaming case, steps 2a ~ 2c are executed instead of step 2:

2a.
For Visited Access scenario, the H-PCRF may provision the PCC rule(s) to the V-PCRF. For Home Routed scenario the H-PCRF may provision the QoS rule(s) to the V-PCRF. The H-PCRF may provision for WLAN case the UE local IP address, and UDP port number (if available) and for H(e)NB case the H(e)NB local IP address, and UDP port number (if available) to the V-PCRF.
2b. The V-PCRF performs local authorization of the QoS Rules or PCC rules when necessary.
2c. The V-PCRF may provision QoS rules to the BPCF by using RAR command. The V-PCRF may provision for WLAN case the UE local IP address, and UDP port number (if available) and for H(e)NB case the H(e)NB local IP address, and UDP port number (if available) to the BPCF.

3
The BPCF shall perform QoS validation and translates the QoS information as received (i.e. QCI, MBR, GBR and ARP) into access specific QoS parameters applicable in the Fixed Broadband Access.

NOTE
The detail of the mapping from 3GPP QoS parameters on S9a to QoS parameters applicable in the Fixed Broadband Access is out of 3GPP scope.

4.
The BPCF sends RAA to the H-PCRF to acknowledge the RAR command. If the QoS validation for admission control fails, the BPCF may include the acceptable QoS in the Fixed Broadband Access using 3GPP QoS parameters on S9a interface.

When the UE is in roaming case, steps 4a ~ 4b are executed instead of step 2:

4a.
The BPCF sends RAA to the V-PCRF to acknowledge the RAR command.

4b. The V-PCRF forwards the RAA to the H-PCRF to acknowledge the RAR command.

5.
Step 4 to 18 is the same as specified in figure 4.3.2.1.1.
E.4.4.1.4
BBERF-initiated IP-CAN Session Modification
This procedure is applicable for both WLAN and H(e)NB scenario.

[image: image97.emf] 

Roaming  Case  

Roaming Case  

BBERF   PCEF   V - PCRF       AF   H - PCRF  

SPR  

  2. Diameter RA R  

4 . Diameter R AA  

Legend :                 Mandatory                       Conditional   

TDF  

1.  Steps 1   to  3   as specified in figure 4.3.2.1.1    

5 .  Steps  4   to 1 8   as specified in figure 4.3.2.1.1  

OCS   BPCF  

3 .  T ranslates QoS rule into  access specific QoS  

  2 a . Diameter RA R  

2b .  Qos Rule Validation  

  2 c . Diameter RA R  

4 a . Diameter R A A  

4 b . Diameter R AA  


Figure E.4.4.1.4.1: BBERF-Initiated IP-CAN Session Modification

1
Step 1 to 3(3a-3c for the Visited Access case) is the same as specified in figure 4.3.2.1.1.


In step 1, in case 2b and case 2a for WLAN scenario, when the UE Local IP address, and/or the UDP port number is changed, the BBERF (ePDG) may provide these information to the PCRF. For H(e)NB scenario, when the H(e)NB local IP address and the UDP port number is changed, the BBERF (S-GW) may provide these information to the PCRF.

2
The H-PCRF sends a Diameter RAR to request that the BPCF installs, modifies or removes QoS Rules (e.g. SDF template, QCI, ARP, MBR and GBR).

When the UE is in roaming case, steps 2a ~ 2c are executed instead of step 2:

2a.
For Visited Access scenario, the H-PCRF may provision the PCC rule(s) to the V-PCRF. For Home Routed scenario the H-PCRF may provision the QoS rule(s) to the V-PCRF. The H-PCRF provisions for WLAN case the UE local IP address and/or UDP port number, and for H(e)NB case the H(e)NB local IP address and UDP port number to the V-PCRF.

2b. The V-PCRF performs local authorization of the QoS Rules or PCC rules when necessary.
2c. The V-PCRF may provision QoS rules to the BPCF by using RAR command. The V-PCRF provisions for WLAN case the UE local IP address and/or UDP port number, and for H(e)NB case the H(e)NB local IP address and/or UDP port number to the BPCF.

3
The BPCF shall perform QoS validation and translates the QoS information as received (i.e. QCI, MBR, GBR and ARP) into access specific QoS parameters applicable in the Fixed Broadband Access.

NOTE:
The detail of the mapping from 3GPP QoS parameters on S9a to QoS parameters applicable in the Fixed Broadband Access is out of 3GPP scope.

4.
The BPCF sends RAA to the H-PCRF to acknowledge the RAR command. If the QoS validation for admission control fails, the BPCF may include the acceptable QoS in the Fixed Broadband Access using 3GPP QoS parameters on S9a interface.

When the UE is in roaming case, steps 4a ~ 4b are executed instead of step 4:

4a.
The BPCF sends RAA to the V-PCRF to acknowledge the RAR.

4b. The V-PCRF forwards the RAA to the H-PCRF to acknowledge the RAR.
5.
Step 4 to 18 is the same as specified in figure 4.3.2.1.1.
E.4.4.2
IP-CAN Session Modification for NSWO traffic

E.4.4.2.1
PCRF-initiated IP-CAN Session Modification
This procedure is applicable for WLAN scenario.

[image: image98.emf] 

Roaming Case    

Roaming Case  

BBERF   PCEF   V - PCRF       AF   H - PCRF  

SPR  

  2. Diameter RA R  

4 . Diameter  R AA  

Legend :                 Mandatory                       Conditional   

TDF  

1.  Steps 1   to  3   as  specified in figure  4.3. 1 .1.1  

5 .  PCRF notifies the AF  

OCS   BPCF  

3 .  T ranslates QoS  rule into access  specific QoS  

  2 a .   Diameter RA R  

  2 c . Diameter RA R  

4 a . Diameter R AA  

4 b . Diameter R AA  

2b. Perform  authori za tion  


Figure E.4.4.2.1.1.1: PCRF-Initiated IP-CAN Session Modification for NSWO traffic

1
Step 1 to 3 as specified in figure 4.3.1.1.1 are executed. The H-PCRF receives an internal or external trigger to update PCC Rules for NSWO traffic (e.g. upon a request from the AF, a request from the TDF in the non roaming case or due to operator policies). External trigger in Step 1 in figure 4.3.1.1.1 only applies to AF session interactions (as described in clause 4.3.1.2), TDF session interactions (as described in clause 4.3.1.1) and SPR subscription data modification (as described in clause 4.3.1.1).
NOTE:
In the roaming case, the TDF is located in the V-PLMN, upon a request from the TDF, the V-PCRF will proxy the request to the H-PCRF, this may also trigger the PCRF-Initiated IP-CAN Session Modification for NSWO traffic.
2
The H-PCRF sends a Diameter RAR to update the QoS information to the BPCF.
If the UE is roaming, then steps 2a ~ 2c are executed instead of step 2:

2a.:The H-PCRF sends a Diameter RAR to the V-PCRF to install, modify or remove PCC Rules.
2b. The V-PCRF performs local authorization of the received PCC rules when necessary.

2c. The V-PCRF sends a Diameter RAR to the BPCF to update the QoS information.

3
The BPCF shall perform QoS validation and translates the PCC rule as received (i.e. SDF template, QCI, MBR, GBR and ARP) into access specific QoS parameters applicable in the Fixed Broadband Access.

NOTE:
The detail of the mapping from 3GPP QoS parameters on S9a to QoS parameters applicable in the Fixed Broadband Access is out of 3GPP scope.

4.
The BPCF sends RAA to the H-PCRF to acknowledge the RAR and informs the H-PCRF about the outcome of the PCC rule operation. If the BPCF cannot provide the requested QoS from the PCRF, the BPCF may respond with the acceptable QoS.

If the UE is roaming, then steps 4a ~ 4b are executed instead of step 4:

4a.
The BPCF sends RAA to the V-PCRF to acknowledge the RAR and informs the V-PCRF about the outcome of the PCC rule operation. If the BPCF cannot provide the QoS requested by the H-PCRF, the BPCF may respond with the acceptable QoS.

4b. The V-PCRF forwards the RAA to the H-PCRF to acknowledge the RAR and informs the H-PCRF about the outcome of the PCC rule operation.

NOTE:
If the H-PCRF is informed that the BPCF cannot provide the requested QoS, the H-PCRF may decide to install, modify or remove PCC rules. In that case, the same flow as described in this clause will be executed.

5.
If the AF requested it, the PCRF notifies the AF as described in steps 6-11 in clause 4.3.2.1.1.
E.4.4.2.2
BPCF-initiated IP-CAN Session Modification
This procedure is applicable for WLAN scenario.

[image: image99.emf] 

Roaming Case  

Roaming Case  

BBERF   PCEF   V - PCRF       AF   H - PCRF  

SPR  

  2. Diameter  CCR  

3 . Diameter  CCA  

Legend :                 Mandatory                        Conditional   

TDF  

4 .   PCRF - initiated IP - CAN Session Modification as described in  E.4.4.2.1  

OCS   BPCF  

  2 c .   Diameter  CCR  

  2 a . Diameter  CCR  

3 b . Diameter  CCA  

3 a . Diameter  CCA  

1.  Trigger  

2b. Store information  


Figure E.4.4.2.2.1: BPCF-Initiated IP-CAN Session Modification for NSWO traffic
1.
The trigger for this procedure is that the Fixed Broadband Access network has pre-empted some resources and wants to report a PCC rule failure to the PCRF, or when the Fixed Broadband Access network cannot sustain the bandwidth allocated to a particular traffic class/DSCP aggregate.

2.
The BPCF sends a Diameter CCR to the H-PCRF with the CC-Request-Type AVP set to the value UPDATE_REQUEST to report PCC Rule failure.

When the UE is roaming, steps 2a ~ 2c are executed instead of step 2:

2a. The BPCF sends a Diameter CCR to the V-PCRF with the CC-Request-Type AVP set to the value UPDATE_REQUEST to report PCC Rule failure.

2b. The V-PCRF stores the information received.

2c. The V-PCRF sends a Diameter CCR to the H-PCRF within the information received in step 2a to report PCC Rule failure. The V-PCRF shall link the S9a* session to S9 session.
3
The H-PCRF sends a Diameter CCA to the BPCF to acknowledge the CCR command.

When the UE is roaming, steps 3a ~ 3b are executed instead of step 3:

3a.
The H-PCRF sends a Diameter CCA to the BPCF to acknowledge the CCR command..

3b. The V-PCRF sends a Diameter CCA to the BPCF to acknowledge the CCR command.
4.
The PCRF-initiated IP-CAN Session Modification as described in E.4.4.2.1 may take place.
E.5
3GPP HNB Procedures – CS Support

E.5.1
S9a CS Session Establishment

In the following procedure, the PCRF is the V-PCRF for the roaming UE.

[image: image100.emf]BPCF PCRF HNB GW

2.Diameter CCR

4.Diameter CCA

6.Diameter TEA

8.Diameter CCA

1. Trigger 

7.Diameter CCR

5.Diameter TER

3. Store  information 


Figure E.5.1.1: S9a CS session Establishment

1.
The HNB GW receives a trigger to establish an S15 session with the PCRF when the HNB performs the registration to the HNB GW.
2.
The HNB GW initiates an S15 session with the PCRF by sending a CCR to the PCRF with the CC-Request-Type AVP set to the value INITIAL_REQUEST. The HNB GW provides the HNB Local IP address and the UDP source port number of IPSec tunnel if NA(P)T is detected.
3.
The PCRF stores the information received in the CCR.
4.
The PCRF acknowledges the session establishment by sending a CCA message.
5.
The PCRF shall send a TER command to BPCF to trigger an S9a session establishment procedure. The PCRF provides HNB Local IP address and UDP source port number if available. The PCRF shall include the Auth-Session-State AVP set to NO_STATE_MAINTAINED.

NOTE:
When the HNB performs the registration to the HNB GW, there is no PS service handled by the HNB. So there is no already established S9a session for the HNB.
6. The BPCF acknowledges the TER command by sending a TEA command.
7. The BPCF initiates an S9a session establishment with the PCRF by sending a CCR to the PCRF with the CC-Request-Type AVP set to the value INITIAL_REQUEST. The BPCF provides HNB local IP address and UDP source port number if available.

8.
The PCRF acknowledges the S9a Session establishment by sending a CCA to the BPCF.
E.5.2
PCRF initiated S9a CS Session Modification
In the following procedure, the PCRF is the V-PCRF for the roaming UE.

[image: image101.emf]BPCF PCRF HNB GW

2.Diameter CCR

5.Diameter CCA

4.Diameter RAA

1. Trigger 

3.Diameter RAR

6. Complete the procedure for 

CS service 


Figure E.5.2.1: S9a CS session Modification

This procedure is performed when the first UE or a subsequent UE connected to a HNB requesting a CS service.

1.
The HNB GW receives RAB assignment message to request the resource for the CS service.
2.
The HNB GW initiates an S15 session modification with the PCRF by sending a CCR to the PCRF with the CC-Request-Type AVP set to the value UPDATE_REQUEST. The HNB GW provides QoS information derived from the RAB assignment message by the HNB GW.
3.
The PCRF initiates an S9a session modification procedure to the BPCF by sending a RAR to the BPCF. The PCRF provides QoS rule generated by the PCRF based on the QoS information received in step 2.
4.
Fixed Broadband access network performs the admission control based on the QoS rule received in step3 and responds with the outcome of the admission control by sending a RAA to the PCRF.

5.
The PCRF responds with outcome of the admission control to the HNB GW by sending a CCA.
6. The HNB GW complete the procedure for the CS service.
E.5.2a
BPCF initiated S9a CS Session Modification
In the following procedure, the PCRF is the V-PCRF for the roaming UE.

[image: image102.emf]BPCF PCRF HNB GW

2.Diameter CCR

3.Diameter CCA

5.Diameter RAA

1. Trigger 

4.Diameter RAR

6. Complete the procedure for 

CS service 


Figure E.5.2a.1: S9a CS session Modification

This procedure is performed when the first UE or a subsequent UE is connected to a HNB requesting a CS service.

1.
The BPCF receives the report of the QoS rule failure

2.
The BPCF initiates an S9a session modification with the PCRF by sending a CCR to the PCRF with the CC-Request-Type AVP set to the value UPDATE_REQUEST. The BPCF includes QoS-Rule-Report AVP to identify the QoS rules that failed and PCC-Rule-Status AVP set to the value “INACTIVE”.

3.
The PCRF acknowledges to the BPCF by sending a CCA.
4.
The PCRF initiates the S15 session modification procedure by sending a RAR command to the HNB GW and includes the information as defined in clause E.5.3.2.2 of 3GPP TS 29.212 [9].
5.
The HNB GW acknowledges to the PCRF by sending a RAA.

6.
The HNB GW completes the procedure for the CS service.
E.5.3
S9a CS Session Termination
In the following procedure, the PCRF is the V-PCRF for the roaming UE.

[image: image103.emf]BPCF PCRF HNB GW

2.Diameter CCR

3.Diameter CCA

5.Diameter RAA

1. Trigger 

4.Diameter RAR

7.Diameter CCA

6.Diameter CCR


Figure E.5.3.1:  S9a CS Session Termination

1.
The HNB GW initiates deregistration for the HNB or receives a deregistration request from the HNB.
2.
The HNB GW initiates an S15 session termination with the PCRF by sending a CCR to the PCRF with the CC-Request-Type AVP set to the value TERMINATION _REQUEST.
3.
The PCRF acknowledges the session termination by sending a Diameter CCA message to HNB GW.

4.
The PCRF sends a Diameter RAR message to the BPCF including a Session-Release-Cause AVP to indicate request for terminating the S9a session.
NOTE:
When the HNB is deregistered, there is no PS service which can be handled by the HNB any more. So S9a session for the HNB can be terminated.
5.
The BPCF acknowledges the S9a session termination request by sending a Diameter RAA message.
6.
The BPCF initiates the S9a session termination with the PCRF by sending a CCR to the PCRF with the CC-Request-Type AVP set to the value TERMINATION _REQUEST.
7.
The PCRF acknowledges the session termination by sending a Diameter CCA message to BPCF.
E.6
PCRF Addressing
E.6.1
General
PCRF discovery at the DRA shall be done according to clause 7.1 with the additions described in this clause.

For EPC-routed scenario, the DRA keeps status of the assigned PCRF for a certain UE for the applicable reference points, i.e. Gx, Gxx, Rx, Sd (Unsolicited application reporting), S9a and S9 (for roaming cases).
For NSWO scenario, the DRA keeps status of the assigned PCRF for a certain UE for the applicable references points, i.e. Rx, Sd (Unsolicited application reporting) and S9a.The H-DRA keeps status of the assigned H-PCRF for a certain UE for Rx and S9 reference points. The V-DRA keeps the status of the assigned V-PCRF for a certain UE for the S9a and Sd (Unsolicited application reporting) reference points.

The BPCF, as a Diameter client of the DRA, shall support all procedures required to properly interoperate with the DRA in both the proxy and redirect modes.
E.6.2
DRA Definition
For the EPC-routed scenario, DRA is defined as specified in clause 7.2 with the additions described in this clause.

The DRA is a functional element that ensures that all Diameter sessions established over the Gx, S9, Gxx, Rx, S9a and for unsolicited application reporting, the Sd reference points for a certain IP-CAN session reach the same PCRF when multiple and separately addressable PCRFs have been deployed in a Diameter realm.
For the NSWO scenario, the DRA shall ensure that all Diameter sessions established over the Rx, S9a and for unsolicited application reporting, the Sd reference points for a certain UE reach the same PCRF. The H-DRA shall ensure that all Diameter sessions established over Rx and S9 reference points for a certain UE reach the same PCRF. The V-DRA shall ensure that all Diameter sessions established over S9a and for unsolicited application reporting, the Sd reference points for a certain UE reach the same V-PCRF.
E.6.3
DRA Procedure
E.6.3.1
DRA Information Storage

For EPC-routed traffic, DRA Information Storage shall be done according to clause 7.3.1 with the additions described in this subclause. The V-PCRF routing information (i.e. DRA binding) in the V-DRA is created when the S9 session establishment trigger is received.

The V-PCRF routing information stored in the V-DRA shall be removed when the S9 session termination notification is received.

The DRA has information about the user identity (UE NAI), the UE Local IP address/H(e)NB Local IP address, the PDN Id (if available) and the selected PCRF identity for a certain IP-CAN Session or a certain user.
For NSWO traffic, the DRA Information Storage shall be done as specified below:

-
The DRA shall maintain PCRF routing information per UE-NAI and APN.

-
The DRA has information about the user identity (UE NAI), the local UE Ipv4 address and/or the local UE Ipv6 address/prefix, the APN (i.e. NSWO_APN) and the selected PCRF identity for a certain UE.

-
The PCRF routing information stored for an S9a* session in the DRA shall be removed after the S9a* session is terminated.
When both EPC-routed and NSWO traffic exist, the DRA Information Storage shall be done as specified below:

-
The DRA shall maintain PCRF routing information per UE-NAI or per UE-NAI and APN.

-
The DRA has information received for both EPC-Routed and NSWO scenarios and the selected PCRF identity per UE-NAI or per UE-NAI and APN.

-
The PCRF routing information stored per UE in the DRA shall be removed when no more S9a sessions and S9a* sessions are active for the UE.
E.6.3.2
Capabilities Exchange

For EPC-routed traffic, capabilities exchange shall be done according to clause 7.3.2.
For NSWO traffic, the Redirect DRA and Proxy DRA shall advertise the support of S9a*, Rx and Sd (for unsolicited application reporting) applications according to clause 7.3.3.
E.6.3.3
Redirect DRA

Redirect DRA is specified in clause 7.3.4 with the additions described in this subclause.

For EPC-routed traffic, for case 1 (home routed case), the V-DRA shall behave as follows:

-
If the request is an S9 session establishment trigger from the H-PCRF, it shall select a V-PCRF to handle the S9 session for that UE. It shall then send the redirect message including selected V-PCRF to the H-PCRF.

-
If the request is an S9 session termination notification from H-PCRF, the V-DRA shall remove the PCRF routing information (i.e. DRA binding). If the V-DRA does not have a V-PCRF already selected, it shall reject the request.

The S9 session establishment trigger and the S9 session termination notification request shall have the same information of user identity (UE NAI), the UE Local IP address/H(e)NB Local IP address, the PDN Id(if available). The DRA shall remove the DRA binding based on the above information when the DRA receives the S9 session termination notification (i.e. a TER command including DRA- Binding AVP set to the value DRA_BINDING_DELETION).
For NSWO traffic, the DRA is maintaining PCRF routing information per UE-NAI and APN. The DRA shall be aware of the S9a* Diameter termination request as defined in 3GPP TS 29.215 [22] in order to release the DRA binding information.
E.6.3.4
Proxy DRA

Proxy DRA is specified in clause 7.3.5 with the additions described in this subclause.

For EPC-routed traffic for case 1 (home routed case), when the V-DRA receives a message from the H-PCRF, it shall behave as follows:

-
If the message is an S9 session establishment trigger from the H-PCRF, it shall select a V-PCRF to handle the S9 session for that UE. It shall then proxy the request to the selected V-PCRF. The V-DRA indicates that there is a V-DRA deployed in the visited PLMN by including the DRA-Deployment AVP in TEA command.
-
If the message is an S9 session termination notification from the H-PCRF, the V-DRA shall remove the PCRF routing information (i.e. DRA binding). If the V-DRA does not have a V-PCRF already selected, it shall reject the request.

The S9 session establishment trigger and the S9 session termination notification shall have the same information of user identity (UE NAI), the UE Local IP address/H(e)NB Local IP address and the PDN Id (if available). The DRA shall remove the DRA binding based on the above information when the DRA receives the S9 session termination notification (i.e. a TER command including DRA-Binding AVP set to the value DRA_BINDING_DELETION).
Proxy DRA is specified in clause 7.3.5 with the additions described in this subclause.

For NSWO traffic in both non-roaming and roaming cases, when the (V-) DRA receives an S9a* request from the BPCF, it shall behave as follows:

-
If the request is an S9a* session establishment, it shall select a V-PCRF to handle the S9 session for that UE and APN. It shall then proxy the request to the selected V-PCRF.

-
If the request is an S9a* session termination, the (V-) DRA shall remove the PCRF routing information and proxy the request to the (V-) PCRF. If the (V-) DRA does not have a (V-) PCRF already selected, it shall reject the request.

-
If the request is an S9a* session modification, the (V-) DRA shall proxy the request.

The BPCF shall be capable of sending every message of a session to the DRA. The BPCF may be configured to bypass the (V-) DRA on S9a* session modification messages by sending these types of messages directly to the (V-) PCRF.
E.6.3.5
PCRF selection by BPCF

For EPC-routed traffic, when the S9a Session Establishment request is triggered by the (V-) PCRF, the BPCF may use the (V-) PCRF identity provided within the PCRF-Address AVP in the S9a Session Establishment Trigger.
The BPCF may also use the DRA procedures as described in clause 7.3. In order to do so, the BPCF shall provide the DRA of the PCRF realm with identity parameters during the S9a Session Establishment procedure. The identity parameters from the BPCF may comprise the UE Local Ipv4 or UE local Ipv6 address in the UE-Local-IP-Address AVP (WLAN scenario), H(e)NB Local IP address in the HeNB-Local-IP-Address AVP (H(e)NB scenario), PDN information in the Called-Station-Id AVP if available and user identity in the Subscription-Id AVP. The BPCF obtains these data from the S9a Session Establishment Trigger procedure initiated by the (V-) PCRF.

For NSWO traffic, the BPCF finds the PCRF using the DRA procedures as described in clause 7.3. In order to do so, the BPCF shall provide the DRA of the PCRF realm with identity parameters during the S9a* Session Establishment procedure. The identity parameters from the BPCF shall comprise the UE local Ipv4 adress or UE local Ipv6 prefix/address in the UE-Local-IP-Address AVP or UE-Local-IP-Prefix AVP, the APN in the Called-Station-Id AVP and user identity in the Subscription-Id AVP.
For both EPC-routed traffic and NSWO traffic, if the redirect agent is used for DRA, the DRA shall use the redirecting requests procedure as specified in IETF RFC 3588 [14], and include the PCRF identity in the Redirect-Host AVP in the Diameter reply sent to the BPCF .
If proxy agent is used for DRA, the DRA should use the proxy procedure as specified in IETF RFC 3588 [14]. For PA2 solution (described in clause 7.1) only S9a/S9a* session establishment and S9a/S9a* session termination messages shall be sent through the DRA.
For NSWO traffic in a roaming scenario the selected V-PCRF shall belong to the same VPLMN selected during the 3GPP –based authentication procedure. The BPCF uses the VPLMN-Id to find the V-DRA in the VPLMN. The V-PCRF finds the DRA in the HPLMN according to clause 7.3.8.
NOTE:
The BPCF will use the VPLMN-Id to obtain the Destination-Realm AVP used to find the V-DRA and then to find the V-PCRF using Diameter based procedures as described in IETF RFC3588 [14].
E.6.3.6
PCRF selection by AF and TDF in Unsolicited application reporting mode for NSWO traffic

PCRF selection by the AF shall be done according to clause 7.3.7.
PCRF selection by the TDF shall be done according to clause 7.3.9.

NOTE:
The DRA matches the received UE IP address received in either the Framed-IP-Address AVP or the Framed-Ipv6-Prefix AVP in the Rx and Sd reference point with the UE Local IP Address received in UE-Local-IP-Address AVP or UE Local Ipv6 Prefix in the UE-Local-IP-Prefix AVP in the S9a reference point in order to select the same PCRF.
E.6.3.7
PCRF selection in a roaming scenario

For both EPC-routed traffic and NSWO traffic, the V-PCRF uses the DRA procedures as described in clause 7.3.8 to address the H-PCRF. In order to do so, the V-PCRF shall provide the DRA of the H-PCRF realm with identity parameters during the S9 Session Establishment procedure.
For EPC-routed traffic, the identity parameters from the V-PCRF may comprise the UE Local Ipv4 or UE local Ipv6 address in the UE-Local-IP-Address AVP (WLAN scenario), H(e)NB Local IP address in the HeNB-Local-IP-Address AVP (H(e)NB scenario), PDN information in the Called-Station-Id AVP if available and user identity in the Subscription-Id AVP obtained from the S9 Session Establishment Trigger procedure initiated by the H-PCRF.

For NSWO traffic, the identity parameters from the V-PCRF may comprise the UE Local Ipv4 or Ipv6 address in the UE-Local-IP-Address AVP or UE local Ipv6 prefix in the UE-Local-IP-Prefix AVP, PDN information in the Called-Station-Id AVP and user identity in the Subscription-Id AVP obtained from the S9a* Session Establishment procedure.

E.6.3.8
PCRF selection for the HNB CS Service

When the DRA receives a request for a certain S15 Session establishment from the HNB GW, the DRA selects a suitable PCRF for the S15 Session based on the HNB local IP address within the HeNB-Local-IP-Address AVP. When the S15 Session is terminated, the DRA shall remove the information about the S15 Session.
E.6.4
DRA flows
E.6.4.1
General

For the EPC-routed traffic case and for the non-roaming case, the flows for the non-roaming case in the clauses 7.4.1 and 7.4.2 are applied with the following exception:

-
BPCF acts as a client and messages are S9a Diameter messages;

-
The external trigger in the Establishment of Diameter Sessions is an S9a session establishment trigger message from the PCRF

For the EPC-routed traffic case and for the roaming case, the flows for the roaming case in clauses 7.4.1 and 7.4.2 are applied with the following exception:

-
The external trigger in the Establishment of Diameter Sessions is an S9 session establishment trigger message from the PCRF for case 1 and home routed case.
The flows in clauses E.6.4.2 and E.6.4.3 are applicable to the EPC-routed traffic and for case 1 and UE roaming in the home routed scenario.
For the NSWO traffic case and for the non-roaming case, the flows for the non-roaming and roaming case in the clauses 7.4.1 and 7.4.2 are applied with following exception:

-
BPCF acts as a client and messages are S9a* Diameter messages for the non-roaming case.
E.6.4.2
Proxy DRA

E.6.4.2.1
S9 session establishment trigger


[image: image104.wmf] 

1. 

 

Extern

al trigger

 

V

-

PCRF

-

1

 

H

-

PCRF

 

2. 

 

TER

 

4

. Proxy 

TE

R

 

3.  

 

DRA binding

 

creation

 

5

. 

TEA

 

V

-

DRA

 

(

proxy

)

 

V

-

PCRF

-

2

 

6

. Proxy 

TEA

 


Figure E.6.4.2.1.1: S9 session establishment trigger using DRA (proxy) – Roaming case

1.
The H-PCRF receives an external trigger (e.g. IP-CAN session establishment request) and determines that an S9 session shall be established.
2.
A TER command including user identity, PDN ID if available, UE Local IP address/H(e)NB Local IP address and UDP source port number(if NA(P)T is detected) is sent by the H-PCRF and received by a V-DRA (proxy) in the visited PLMN. The Auth-Session-State AVP set to NO_STATE_MAINTAINED shall be included in the TER.
3.
The V-DRA (proxy) stores the user identity and creates a dynamic DRA binding for this user. (assignment of a PCRF node per UE).
4.
The V-DRA (proxy) proxies the TER to the target PCRF in the visited PLMN.

5.
V-PCRF-1 returns a TEA to the V-DRA (proxy).
6.
V-DRA (proxy) proxies the TEA to the H-PCRF and indicates there is a DRA deployed in the visited PLMN by including the DRA-Deployment AVP in the TEA.

NOTE: The H-PCRF is aware that there is a V-DRA (proxy) deployed in the network at this stage.
E.6.4.2.2
S9 session termination notification


[image: image105.wmf] 

V

-

PCRF

-

1

 

H

-

PCRF

 

2

. 

 

TER

 

3

.  DRA binding

 

verification 

and release

 

V

-

DRA

 

(

proxy

)

 

V

-

PCRF

-

2

 

4

. 

TEA

 

1

.  

H

-

PCRF initiated 

S9 

session termi

n

ation

 


Figure E.6.4.2.2.1: S9 session termination notification using V-DRA (proxy) – Roaming cases

1.
The H-PCRF receives an external trigger (e.g.IP-CAN session termination request from the BBERF or the PCEF) and initiates S9 session termination procedure.

2.
If the V-DRA is deployed in the VPLMN as indicated in step 6 of clause E.6.4.2.1, a TER command including DRA-Binding AVP set to the value DRA_BINDING_DELETION is sent by the H-PCRF and received by the V-DRA (proxy) in the visited PLMN. The message includes the same user identity as the S9 session establishment trigger message.
3.
The V-DRA (proxy) verifies that there is an active DRA binding for the user based on the user identity in the request and removes the DRA binding.

4.
V-DRA (proxy) returns the TEA to the H-PCRF.

E.6.4.3
Redirect DRA

E.6.4.3.1
S9 session establishment trigger


[image: image106.wmf] 

1. 

 

Extern

al trigger

 

V

-

PCRF

-

1

 

H

-

PCRF

 

2. 

 

TER

 

4

.

 

TEA(redirect)

 

3.  

 

DRA binding

 

creation

 

5

. 

TE

R

 

V

-

DRA

 

(

Redirect

)

 

V

-

PCRF

-

2

 

6

.

 

TEA

 


Figure E.6.4.3.1.1: S9 session establishment trigger using DRA (Redirect) – Roaming case

1.
The H-PCRF receives an external trigger (e.g. IP-CAN session establishment request) and determines that an S9 session shall be established.
2.
A TER command including user identity, PDN ID if available, UE Local IP address/H(e)NB Local IP address, UDP source port number(if NA(P)T is detected) and the FQDN of Fixed Broadband network where the H(e)NB is connected to if available is sent by the H-PCRF and received by a V-DRA (proxy) in the visited PLMN. The Auth-Session-State AVP set to NO_STATE_MAINTAINED shall be included in the TER.
3.
The V-DRA (redirect) stores the user identity and creates a dynamic DRA binding for this user. (assignment of a PCRF node per UE).
4.
The V-DRA (redirect) sends a TEA command indicating redirection as defined in IETF RFC 3588 [14]. The target V-PCRF identity is included in the Redirect-Host AVP.

NOTE:
The H-PCRF is aware that there is a V-DRA (redirect) deployed in the network at this stage.

5.
The H-PCRF re-sends the TER command of step 2 to the target V-PCRF-1.
6.
The V-PCRF-1 returns a TEA 
vailab.
E.6.4.3.2
S9 session termination notification

The detailed procedure is the same as the S9 session termination notification using DRA (Proxy), which is described in clause E.6.4.2.2.

If the H-PCRF was aware that there is a V-DRA deployed in the V-PLMN (step 4 of S9 session establishment procedure according to clause E.6.4.3.1), this procedure shall apply.
E.7
BPCF Addressing
E.7.1
General

For S9a session establishment trigger procedure initiated by the (V-)PCRF, the PCRF (for non-roaming case) and the V-PCRF (for roaming case) is configured with IP address range mappings { (Ipx..Ipy) ‑> BBF network entry point}. The PCRF (for non-roaming) and the V-PCRF (for roaming cases) selects the correct BBF network entry point based on UE Local IP address for WLAN scenario and based on H(e)NB IP address and FQDN if available for H(e)NB case received from the PCEF/BBERF/HNB GW for H(e)NB scenario. The implementation of a BBF network entry point is out-of-scope for 3GPP e.g. be a BPCF or a DRA.

For case 1 and roaming with home routed, for S9 session establishment trigger procedure initiated by the H-PCRF, H-PCRF finds the VPLMN according to the PLMN Id of the visited network in 3GPP-SGSN-MCC-MNC AVP if received at IP-CAN session establishment received over Gx reference point, and then discovers V-PCRF by V-DRA if there are more than one PCRF deployed in the visited network. The H-PCRF sends the H(e)NB Local IP address within HeNB-Local-IP-Address AVP and optionally the FQDN of BBF access network within the HeNB-BBF-FQDN AVP for H(e)NB scenario or the UE local IP address within the UE-Local-IP-Address AVP for WLAN scenario to the V-PCRF over the S9 reference point.
E.8
Session Linking Function

PCRF and BPCF needs to support session linking function. The PCRF shall be able to perform the linking between multiple Gx and Gateway Control Session to the same S9a session.

When receiving an IP-CAN Session Establishment request or an IP-CAN Session modification request with an UE local IP address or H(e)NB local IP address, the PCRF shall perform the session linking between the S9a session and the corresponding Gx session according to the UE Local IP address/ H(e)NB local IP address.

When receiving a Gateway Control Session Establishment Request or a Gateway Control and QoS Rule Request with UE local IP Address or a H(e)NB local IP Address change, the PCRF shall perform the session linking between the S9a session and the Gateway Control Session according to the UE Local IP address/ H(e)NB local IP address.

If there is not an established S9a session which could be linked to the Gx or Gateway Control Session, the PCRF shall initiate the S9a Session Establishment trigger procedure.

For 3GPP HNB Procedures with CS support, the PCRF shall be able to perform the linking between S15 sessions and the S9a session.

NOTE 1:
There is a single S15 session per HNB for CS calls for all Ues connected to the HNB in order to improve performance. In addition, for CS calls there are no UE specific policies and therefore a single PCRF can handle CS calls for all Ues.
Annex F (normative):
Access specific aspects, Fixed Broadband Access network convergence

F.1
General

This annex defines the enhancement to PCC framework for supporting policy and charging control in the fixed broadband access network in the convergent scenario where the PCRF controls directly the network element(s) in the fixed broadband access without the mediation of a different policy server, such as the Broadband Policy Control Function (BPCF).
Policy and charging control is provided for both Non-seamless WLAN offload traffic from a 3GPP UE and the traffic from fixed devices.
F.2
Definitions and abbreviations
F.2.1
Definitions
The definitions in the following are relevant for this Annex only.

UE local IP address is defined as either the public IP address assigned to the UE by the Broadband Forum domain in the no-NAT case, or the public IP address assigned by the Broadband Forum domain to the NATed RG.

IP-CAN session as defined in clause 3.1 applies with the following clarifications for fixed broadband access. The term UE corresponds to the device that accesses the services provided by the network (i.e. either RG, or 3GPP UE or fixed end-device), the PDN identifies the IP network where the device gets IP connectivity and the UE identity information may be the IMSI, the user-name or the access line identifier (if available). In a Fixed Broadband Access an IP-CAN session corresponds to a Subscriber IP Session defined in Broadband Forum TR-146.

NOTE:
The PDN connection concept and APN are not applicable to a Subscriber IP session for a fixed device.
F.2.2
Abbreviations
The following abbreviations are relevant for this annex only:

BBF
Broadband Forum

BPCF
Broadband Policy Control Function
NSWO
Non-Seamless WLAN offload

NSWO-APN
Non-Seamless WLAN offload APN

RG
Residential Gateway
F.3
Binding Mechanisms
F.3.1
NSWO traffic
The binding mechanism in clause 5 applies, except that the QoS rule generation as described in clause 5.4 and the bearer binding as described in clause 5.4 do not apply since the Fixed Broadband Access network does not support the concept of a bearer and multiple bearers as defined in 3GPP network.

The following exceptions or modifications in PCC rule authorization also apply:

-
The PCRF does not authorize traffic mapping information from the UE.

-
The PCEF in the IP Edge would map the received QoS information over Gx into the relevant data as specified in Broadband Forum.

-
MPS and emergency services are not supported.

-
BCM concept is not applied.
F.3.2
Traffic from fixed devices
The binding mechanism as described in Annex F.3.1 applies to the scenario of traffic from fixed devices with the following exceptions or modifications:

· S9 reference point is not applicable.

-
The Subscriber Identifier used by fixed device at establishment of Subscriber IP session in Fixed Broadband Access network can be the Access Line Identifier (Physical-Access-ID AVP and Logical-Access-ID AVP) or the username (Subscription-Id AVP), for example when the Subscriber IP session is a PPP Session.
NOTE:
In case the Subscriber Identifier in the IP-CAN and the application level identity for the fixed device are of different kinds, the PCRF needs to map between them. Such mapping is not subject to specification within this TS.
F.4
PCC procedures

F.4.1
Introduction

The PCC procedures specified in clause 4.1, 4.2 and 4.3 apply to the Fixed Broadband Access network convergence with the following exceptions or restrictions:

-
Roaming scenarios are not applicable to fixed devices and RGs.

-
Gxx reference point is not used.
-
UE requested bearer resource initiation, modification or termination procedure is not supported.

-
Bearer procedures are not supported in the fixed network.
-
Bearer Control Mode (BCM) concept does not apply.

-
Authorized QoS per bearer and authorized MBR per QCI are not applicable
-
MPS Services and IMS Emergency Services are not supported.
F.4.2
IP-CAN Session Establishment
The PCEF in the IP Edge initiates the IP-CAN session establishment as specified in clause 4.1, with the exceptions as described in Annex F.4.1.The PCRF shall provide parameters to the PCEF in the IP Edge at IP-CAN session establishment as described in 3GPP TS 29.212 [9] Annex G.5.2.
F.4.3
IP-CAN Session Termination
F.4.3.1
UE-Initiated
UE-initiated IP-CAN session termination is not applicable to convergent scenario.
F.4.3.2
PCEF-Initiated

For PCEF-Initiated IP-CAN session termination the procedures described in clause 4.2.2 apply, with the exceptions described in clause F.4.1.

F.4.3.3
PCRF-Initiated

For PCRF-Initiated IP-CAN session termination the procedures described in clause 4.2.3 apply, with the exceptions described in clause F.4.1.
F.4.4
IP-CAN Session Modification
F.4.4.1
PCRF-Initiated IP-CAN Session Modification

For PCRF-Initiated IP-CAN session modification, the procedures described in clause 4.3.1 apply, with the exceptions described in clause F.4.1. The PCRF shall provide parameters to the PCEF in the IP Edge at IP-CAN session modification as described in 3GPP TS 29.212 [9] Annex G.5.4.2.
F.4.4.2
PCEF-Initiated IP-CAN Session Modification

For PCEF-Initiated IP-CAN session modification, the procedures described in clause 4.3.2 apply, with the exceptions described in clause F.4.1. The PCRF shall provide parameters to the PCEF in the IP Edge at IP-CAN session modification as described in 3GPP TS 29.212 [9] Annex G.5.4.1.
F.5
PCRF Addressing

F.5.1
General
PCRF discovery and selection shall be done according to clause 7.1 with the modifications or exceptions described in this subclause.
-
Gxx reference point is not used.

-
The Subscriber Identifier specified in 3GPP TS 23.203 [2] clause S.5.1.2 is used as user identity.

-
For a 3GPP UE, the NSWO-APN is also available.
-
The IPv6 address within the Framed-IPv6-Prefix may be included in the CCR command in the case of bridge-mode RG.
F.5.2
DRA Definition
The DRA definition in clause 7.2 applies with the modifications or exceptions as described in Annex F.5.1.
F.5.3
DRA Procedure
F.5.3.1
Redirect DRA
Redirect DRA is specified in clause 7.3.4 with the adaptions listed in F.5.1.

F.5.3.2
Proxy DRA

Proxy DRA is specified in clause 7.3.5 with the adaptions listed in F.5.1.
F.5.3.3
PCRF selection by AF and TDF in unsolicited application reporting mode

PCRF selection by the AF shall be done according to clause 7.3.7 with the modifications or exceptions as described in Annex F.5.1. PCRF selection by the TDF shall be done according to clause 7.3.9 with the modifications or exceptions as described in Annex F.5.1.
F.5.3.4
PCRF selection in a roaming scenario
The procedures as described in clause 7.3.8 for the V-PCRF to select the H-PCRF apply with the modifications or exceptions as described in Annex F.5.1.
F.5.4
DRA flows
The DRA procedures specified in clause 7.4.1 and 7.4.2 apply to the Fixed Broadband Access network convergence with the following exceptions or restrictions:

-
Gxx reference point is not used.

-
Roaming scenarios are not applicable to fixed devices and RGs.

Annex G (normative):
Diameter overload control mechanism

G.1
General

Support for Diameter overload control by PCC functional elements is optional. Unless otherwise stated, the procedures defined in this Annex assume that a PCC functional element supports the Diameter overload control mechanism.

IETF RFC 7683 [33] specifies the Diameter overload control mechanism. This includes the definition of Diameter overload related AVPs and the Diameter overload related behavior.

To indicate support of the Diameter overload control mechanism, each PCC functional element shall include the OC-Supported-Features AVP in every Diameter request and answer as defined in IETF RFC 7683 [33].

Each PCC functional element (e.g. PCRF, PCEF, AF, etc) shall act asa reacting node and asa reporting node as defined in IETF RFC 7683 [33].
G.2
Reporting Node

When a PCC functional element determines the need to request a reduction in the traffic it is handling due to an overload condition, it shall include the OC-OLR AVP in answer messages, as defined in IETF RFC 7683 [33].

How it determines that it is in an overload situation and the severity of the overload is implementation dependent and based on operator policy.

How it determines the specific contents of the OC-OLR AVP is implementation dependent and based on operator policy.

G.3
Reacting Node

A PCC functional element acting as a reacting node applies the requested traffic reduction received in OC-OLR AVPs in answer messages to corresponding applicable requests, as per IETF RFC 7683 [33].

How it achieves the requested traffic reduction is implementation dependent.

Diameter requests related to priority traffic (e.g. MPS) as described in 3GPP TS 22.153 [58] and emergency have the highest priority. If required by the regional/national regulatory and operator policies, and when the reacting node is able to detect priority traffic, priority traffic shall be exempted from throttling due to Diameter overload control up to the point where requested traffic reduction cannot be achieved without throttling the priority traffic. Relative priority amongst various priority traffic (e.g. MPS) and emergency traffic is subject to regional/national regulatory and operator policies.
G.4
DRA Diameter Overload Behavior

The DRA may optionally incorporate agent behavior specified in IETF RFC 7683 [33].

G.4.1
DRA reacting to Host Reports

The procedures defined in this clause are only applicable to the proxy DRA (PA1 and PA2) as the redirect DRA is not in the path of application answers and as such does not have access to overload reports from other nodes.

The proxy DRA shall use host reports as one of the inputs when making routing decisions for realm-routed requests, i.e. requests that do not contain a Destination-Host AVP. This is needed because entities sending such requests are not aware of the final recipient of the request (e.g. specific PCRF instance).

The following scenarios shall be addressed:

-
No binding exists for the request and the request can result in a new binding (e.g. IP-CAN session establishment); in this case the DRA is selecting the PCRF that will handle the binding. The DRA should use any active and relevant Diameter overload host reports as one of the inputs to the selection of the PCRF. If all PCRFs are in an overload state, the DRA should reduce traffic sent to each of the PCRFs based on the individual host requested traffic reduction. This may result in the DRA rejecting the request.

-
A binding already exists for the request – In this case the DRA should reduce the traffic sent to the overloaded node by the host requested traffic reduction. This may result in the DRA rejecting the request.

NOTE: 
Result-Code when rejecting a request in the above cases need to be used according to the IETF RFC 7683.
How the DRA achieves any requested traffic reduction is implementation dependent and/or based on operator policy.

Annex H (normative):
Access specific procedures for 3GPP EPS

H.1
General

The present annex defines IP-CAN specific requirements for 3GPP Evolved Packet System (EPS).

H.2
Binding Mechanisms

The procedures defined in clause 5.4 apply.

Whenever the PCRF modifies the Authorized QoS of the default bearer, the BBF shall re-evaluate the bearer binding taking into account the default bearer QoS change and any PCC Rule/QoS Rule operation requested by the PCRF.

Annex I (normative):
APN matching procedures

A PCRF or DRA needs to compare APN information received over different interfaces, for instance within the Called-Station-Id AVP, for certain purposes such as session binding. To do so, the PCRF or DRA shall apply the procedures in the present Annex.

The PCRF or DRA shall consider the different possible encoding formats of the APN defined in 3GPP TS 23.003 [37], clause 9.1. When the PCRF or DRA needs to compare an APN only containing the APN Network Identifier, with an APN containing both APN Network Identifier and APN Operator Identifier, the PCRF shall consider those APNs as matching if their APN Network Identifiers match. The PCRF shall perform a case-insensitive comparison for APN Network Identifiers and APN Operator Identifiers.

Annex J (normative):
Diameter message priority mechanism

J.1
General

The support of the Diameter message priority mechanism by PCC functional elements is optional.  PCC functional elements that support Diameter message priority mechanism shall comply with the procedures specified in this annex.
IETF draft-ietf-dime drmp [40] specifies the Diameter message priority mechanism that allows Diameter nodes to indicate the relative priority of Diameter messages. With this information, other Diameter nodes can leverage the relative priority of Diameter messages into routing, resource allocation, and also abatement decisions when overload control is applied. This includes the definition of Diameter message priority related AVPs and the Diameter message priority related behaviour.
J.2
PCC functional element behaviour

A PCC functional entity supporting the Diameter message priority mechanism shall comply with IETF draft-ietf-dime-drmp [40]. When priority is required, a PCC functional element shall include the DRMP AVP indicating a priority level in the requests it sends, and shall prioritise received requests according to the priority level received within the DRMP AVP. It shall prioritise received answers according to the priority level received within the DRMP AVP if present, otherwise according to the priority level of the corresponding request. It shall include the DRMP AVP in the answers to received requests if the priority value of the answer is different from the one of the request. The PCC functional element decisions for a required priority and the priority level value are implementation specific.

Diameter requests related to high priority traffic (e.g. MPS, emergency) shall contain a DRMP AVP with a high priority of which the level is operator dependent.

J.3
Interactions

If the PCC entity supporting the Diameter message priority mechanism receives the request message containing both the Reservation-Priority AVP and DRMP AVP, the PCC entity shall prioritise the request according to priority level received within the DRMP AVP. This procedure also applies for MPS.

NOTE:
Upon receipt of the request message containing the DRMP AVP, the PCRF will use the DRMP AVP to prioritize other PCC interfaces related to that IP-CAN session.
Annex K (informative):
Change history

	Date
	TSG #
	TSG Doc.
	CR
	Rev
	Subject/Comment
	Old
	New

	12-2014
	CT-66
	CP-140922
	0575
	2
	Call flows over Np reference point
	12.5.0
	13.0.0

	12-2014
	CT-66
	CP-140935
	0576
	2
	Double Resource Reuse procedures
	12.5.0
	13.0.0

	03-2015
	CT-67
	CP-150131
	0579
	
	IPCAN Session modification after bearer procedure
	13.0.0
	13.1.0

	03-2015
	CT-67
	CP-150134
	0580
	
	P-CSCF Restoration Procedure for Local Breakout
	13.0.0
	13.1.0

	03-2015
	CT-67
	CP-150136
	0581
	
	Priority Consideration for Diameter Overload Control
	13.0.0
	13.1.0

	03-2015
	CT-67
	CP-150132
	0582
	
	Correction to the Gateway Control Session establishment procedur
	13.0.0
	13.1.0

	03-2015
	CT-67
	CP-150136
	0583
	
	PCRF discovery for the HNB CS Service
	13.0.0
	13.1.0

	03-2015
	CT-67
	CP-150094
	0590
	
	Corrections to the Gateway Control and QoS rules Provision procedure
	13.0.0
	13.1.0

	03-2015
	CT-67
	CP-150125
	0592
	
	Add missing definition of RAN user plane congestion
	13.0.0
	13.1.0

	03-2015
	CT-67
	CP-150125
	0593
	2
	DRA procedure to support PCRF selection by the RCAF
	13.0.0
	13.1.0

	03-2015
	CT-67
	CP-150111
	0595
	
	Remove editor’s Note on access line identifier
	13.0.0
	13.1.0

	03-2015
	CT-67
	CP-150125
	0596
	2
	UE context removal in IP-CAN session termination procedure
	13.0.0
	13.1.0

	06-2015
	CT-68
	CP-150342
	0601
	1
	Remove the user identity from the intermediate spending limt report request
	13.1.0
	13.2.0

	06-2015
	CT-68
	CP-150342
	0604
	2
	Correction to the IP-CAN session termination for spending limit report
	13.1.0
	13.2.0

	06-2015
	CT-68
	CP-150355
	0605
	1
	Alignment with the changes of 29.217
	13.1.0
	13.2.0

	06-2015
	CT-68
	CP-150355
	0606
	1
	Reference update to align with new title of TS 29.217
	13.1.0
	13.2.0

	06-2015
	CT-68
	CP-150361
	0607
	1
	Correction of the editorial errors in Network-iniated IP-CAN session modification
	13.1.0
	13.2.0

	06-2015
	CT-68
	CP-150354
	0609
	2
	QoS change of default bearer
	13.1.0
	13.2.0

	06-2015
	CT-68
	CP-150362
	0610
	3
	APN matching procedures
	13.1.0
	13.2.0

	09-2015
	CT-69
	CP-150480
	0612
	-
	Procedures for monitoring when using the PCC architecture
	13.2.0
	13.3.0

	09-2015
	CT-69
	CP-150482
	0614
	1
	Procedures to allow changing the chargeable party when sponsor connectivity applies
	13.2.0
	13.3.0

	09-2015
	CT-69
	CP-150490
	0616
	2
	Correction on bearer binding on QoS rules
	13.2.0
	13.3.0

	09-2015
	CT-69
	CP-150477
	0617
	5
	Clarification of the PCRF addressing for UPCON
	13.2.0
	13.3.0

	09-2015
	CT-69
	CP-150491
	0618
	2
	PCC signalling update to support NBIFOM
	13.2.0
	13.3.0

	09-2015
	CT-69
	CP-150485
	0619
	2
	Clarification of PCRF address
	13.2.0
	13.3.0

	12-2015
	CT-70
	CP-150649
	0620
	1
	Authorized QCI value in PCRF to avoid SRVCC
	13.3.0
	13.4.0

	12-2015
	CT-70
	CP-150650
	0621
	6
	Update the procedures to support traffic steering control
	13.3.0
	13.4.0

	12-2015
	CT-70
	CP-150809
	0622
	3
	Include the PCC architecture figures in 29.213l
	13.3.0
	13.4.0

	12-2015
	CT-70
	CP-150632
	0625
	2
	Overlapping transactions over Gx
	13.3.0
	13.4.0

	12-2015
	CT-70
	CP-150666
	0626
	-
	RTP/RTCP transport multiplexing
	13.3.0
	13.4.0

	12-2015
	CT-70
	CP-150730
	0629
	6
	Diameter message priority for PCC
	13.3.0
	13.4.0

	12-2015
	CT-70
	CP-150663
	0630
	4
	Support of new bandwidth information over Rx
	13.3.0
	13.4.0

	12-2015
	CT-70
	CP-150631
	0633
	1
	Update the reference of draft-ietf-dime-ovli
	13.3.0
	13.4.0


	Change history

	Date
	TSG #
	TSG Doc.
	CR
	Rev
	Cat
	Subject/Comment
	New

	03-2016
	CT-71
	CP-160107
	0638
	-
	F
	Network-initiated removal of one access from the PDN connection
	13.5.0

	03-2016
	CT-71
	CP-160091
	0639
	2
	B
	ADC rule error handling report initiated by the TSSF
	13.5.0

	03-2016
	CT-71
	CP-160091
	0640
	-
	F
	IPv4 address provisioning
	13.5.0

	03-2016
	CT-71
	CP-160095
	0641
	2
	F
	Update the scope to include the Nt interface
	13.5.0

	03-2016
	CT-71
	CP-160091
	0645
	2
	F
	Update the scope to include the St interface
	13.5.0

	03-2016
	CT-71
	CP-160101
	0646
	-
	D
	Correction of command name for Rx session termination
	13.5.0

	03-2016
	CT-71
	CP-160101
	0648
	2
	F
	Completion of RAN-NAS cause handling procedures
	13.5.0

	03-2016
	CT-71
	CP-160095
	0650
	-
	B
	Support of background data transfer per UE in the PCRF
	13.5.0

	03-2016
	CT-71
	CP-160092
	0651
	1
	B
	UE-to-network relay PCC handling
	13.5.0

	03-2016
	CT-71
	CP-160106
	0652
	-
	D
	Removal of the reference to draft-ietf-dime-ovli
	13.5.0

	03-2016
	CT-71
	CP-160095
	0653
	-
	B
	Rx impact for background data transfer
	13.5.0

	03-2016
	CT-71
	CP-160093
	0654
	2
	B
	Reservation-Priority AVP and DRMP AVP Interaction
	13.5.0

	03-2016
	CT-71
	CP-160101
	0656
	1
	F
	Removal of references to TS 26.236
	13.5.0

	03-2016
	CT-71
	CP-160087
	0657
	1
	B
	The value of ARP for MCPTT emergency
	13.5.0

	03-2016
	CT-71
	CP-160101
	0658
	-
	F
	Correction to eMPS algorithm
	13.5.0

	06-2016
	CT-72
	CP-160270
	0660
	2
	F
	Support of enhanced bandwidth mechanisms in MTSI sessions
	13.6.0

	06-2016
	CT-72
	CP-160265
	0661
	1
	F
	Editorial corrections
	13.6.0

	06-2016
	CT-72
	CP-160278
	0662
	2
	F
	Priority sharing for concurrent sessions
	13.6.0

	06-2016
	CT-72
	CP-160267
	0665
	-
	A
	P-CSCF restoration indication by Rx-Request-Type AVP
	13.6.0

	06-2016
	CT-72
	CP-160251
	0666
	-
	F
	Correction to sponsor status change
	13.6.0

	06-2016
	CT-72
	CP-160265
	0667
	2
	F
	Delete the Editor's Note in subclause of DRA diameter overload behavior
	13.6.0

	06-2016
	CT-72
	CP-160253
	0669
	2
	A
	Diameter requests for priority traffic during overload control mechanism
	13.6.0

	06-2016
	CT-72
	CP-160274
	0659
	2
	B
	Adding the reference to the gate control procedure over Rx interface
	14.0.0

	06-2016
	CT-72
	CP-160265
	0661
	1
	F
	Editorial corrections
	14.0.0


















































































































































































































































































































































































































































































































































































































































































































































































































































































































































































































































































































































































































































































































































































































































































































































































































































































































































































































































































































































































































































































































































































































































































































































_1342423030.doc


SPR







9. 	Install QoS Rules. Policy enforcement







8c. 	Diameter CCR







8. 	Diameter CCA







PCRF







10. 	Establish Gateway Control Session Response







PCEF







If the PCRF needs subscription-related information and does not have it 











5. 	Profile Response







H-PCRF







2. 	Diameter CCR







6. 	PCC Rules Decision,�Policy Decision







Legend:



	       	Mandatory



                	Conditional 







1. 	Establish Gateway Control Session Request







3. Store Information







7. 	Store PCC Rules 







4. 	Profile Request







V-PCRF







BBERF











SPR







8a. 	Diameter CCA







ROAMING CASE







ROAMING CASE







GW























8d. 	Diameter CCA







8b. 	Store PCC Rules. Policy decision







2b. Store Information







8e. 	Diameter CCA







2a. 	Diameter CCR







2c. Diameter CCR












_1429573238.vsd

_1487185255.doc


[image: image1.emf]BBERF PCEF V- PCRF H- PCRF V-AF SPR


. 


For each 


affected 


AF session


(if any)


2.  Diameter ASR


3 .  Diameter ASR


4.  Diameter ASA


5.  Diameter ASA


6.  Diameter STR


7.  Diameter STR


8.  Diameter STA


9.  Diameter STA


Conditional 


Mandatory


Legend:


1  .    Perform  Step  1  through Step  12:  as specified in Figure  4.2.3 .1 .1  : 


PCRF Initiated IP-CAN Session Termination  - AF Located in the HPLMN 


V


I


S


I


T


E


D


 


A


C


C


E


S


S


 


C


A


S


E


10.      Perform Step  17 through Step  20:   as specified in in Figure  4.2.3 .1 .1  : 


PCRF Initiated IP-CAN Session Termination  - AF Located in the HPLMN 


 




















































































































































































































































































































































































































































































_1487185308.doc




[image: image1.emf]BBERF PCEF V-PCRF H-PCRF V-AF SPR



. 



For each 



affected 



AF session



(if any)



2. Diameter ASR



3. Diameter ASR



4. Diameter ASA



5. Diameter ASA



6. Diameter STR



7. Diameter STR



8. Diameter STA



9. Diameter STA



Conditional 



Mandatory



Legend:



1 .   Perform  Step 1 through Step 12: as specified in Figure 4.2.3.1.1 : 



PCRF Initiated IP-CAN Session Termination -AF Located in the HPLMN 



V



I



S



I



T



E



D



 



A



C



C



E



S



S



 



C



A



S



E



10.     Perform Step 17through Step 20:  as specified in in Figure 4.2.3.1.1 : 



PCRF Initiated IP-CAN Session Termination -AF Located in the HPLMN 






_1479540040.vsd








_1506448295.vsd

_1507365374.doc


Gx







Gzn







Gyn







Gy







Gz







Traffic Detection Function (TDF)







Gateway







Policy and Charging Enforcement Function



(PCEF)















Sy







VPLMN







HPLMN







Sd







Gxx







S9







Policy and Charging Rules Function



(H-PCRF)







Sp/Ud







Application Function



(AF)







Rx







Subscription Profile Repository (SPR)/ 



User Data Repository (UDR)







 







Offline Charging System



(OFCS)







AN-Gateway







Bearer Binding and Event Reporting Function



(BBERF)







Policy and Charging Rules Function



(V-PCRF)







Traffic Steering Support



Function



(TSSF)







St







Online Charging System



(OCS)







RAN Congestion



 Awareness Function



     (RCAF)







Np







Service Capability Exposure Function



(SCEF)







Nt












_1509339422.vsd

_1517062912.vsd

_1515651064.vsd

_1507373450.doc


Gzn







Gyn







Online Charging System



(OCS)







Gz







Traffic Detection Function



(TDF)







Policy and Charging Enforcement Function



(PCEF)







Gateway







Sy







Gx







Gxx







AN-Gateway







Bearer Binding and Event Reporting Function



(BBERF)







S9







HPLMN







VPLMN







Policy and Charging Rules Function



(H-PCRF)







Sp/Ud







Application Function



(AF)







Rx







Subscription Profile Repository (SPR)/



User Data Repository (UDR)







 







Offline Charging System



(OFCS)







Policy and Charging Rules Function



(V-PCRF)







Sd







Application Function



(AF)







Rx







Gy







RAN Congestion



 Awareness Function



     (RCAF)







Np







Service Capability Exposure Function



(SCEF)







Nt












_1506448400.vsd
BBERF


PCEF


TDF


V-PCRF


H-PCRF


AF


SPR


OCS


1. Gateway control and QoS Rule request (case 2a & case 2b)


2. IP-CAN Session Modification


3. Diameter CCR


10. Diameter STR


3a. Diameter CCR


VISITED ACCESS
CASE


3b. Store Information


3c. Diameter CCR


4. Profile Request


5. Profile Response


6. Diameter RAR


VISITED ACCESS
CASE


7. Diameter RAA


8. Diameter ASR


9. Diameter ASA


13a. Diameter CCA


13b. PCC/ADC Rules validation


13c. Diameter CCA


13d. TDF session modification


14. TDF session modification


15. Install, modify or remove PCC/ADC Rules


11. Diameter STA


17. IP-CAN Session modification signalling


If a specific action is triggered for the AF session


If all session info for an AF session is decided


Initial/Intermediate/Final Spending limit Report Request


12. PCC/ADC Rule Decision


13. Diameter CCA


18. Resource allocation confirmation (case 1)


TSSF


19. Gateway control & QoS Rule Provision (case 2a & case 2b)


16. St session establishment/modification/termination



_1507365288.doc






Gxx







(OCS)







System 







Policy and Charging Enforcement Function



(PCEF)







Online Charging



















(PCRF)







Policy and Charging Rules Function 











Gx







(OFCS)







System 







Charging 







Offline 



















































Policy and Charging Rules Function (PCRF)











Traffic Steering Support



Function



(TSSF)







Sd







Bearer Binding and Event Reporting Function�(BBERF)







St







Subscription Profile



Repository



(SPR)/



User Data



Repository



(UDR)







Service Capability Exposure Function



     (SCEF)







Nt







Application



 Function 



     (AF)







Gzn







Gyn







Gz







Gateway







AN-Gateway







Traffic



Detection



Function



(TDF)







Np







RAN Congestion Awareness Function 



(RCAF)







Sp/Ud







Rx







Gy







Sy












_1507101490.vsd

_1506448320.vsd

_1506448231.vsd
BBERF


PCEF


TSSF


H-PCRF


H-AF


SPR


14.  Gateway Control and Qos Rules provision (case 2a) Or PCRF-initiated Gateway Control Session Termination (case 2a)



_1506448270.vsd

 (
VISITED 
ACCESS 
CASE
VISITED 
ACCESS 
CASE
PCEF
TSSF
SPR
BBREF
H-
PCRF
TDF
2
. 
Establish IP
-
CAN Session 
Request
3
. 
Diameter CCR
. 
3a.
Diameter CC
R
3b.
Store Information
3c.
Diameter CCR
4.
Store
 Information
. 
5. 
Profile Request
6. 
Profile Response
8
. 
PCC Rules Decision
; 
Policy Decision 
10
. 
ADC Rules Decision
;
Store 
ADC Rules
;
9
. 
Store PCC Rules
1
3
. 
Diameter CCA
1
4
. 
Gateway Control and 
QoS
 Rules Provision 
(
case
 
2
a
 and 
2
b
)
15
.
Install
 PCC Rules
,
Policy Enforcement
1
3
c
. 
Store 
ADC
 Rules
; 
Policy Decision
13a.
Diameter CCA
1
3
d
. 
Diameter 
CCR
1
3
e
Diameter 
CCA
. 
13g.
Diameter
 CCA
 
11
. 
TDF session establishment
.
1
3
b
. 
Policy Enforcement
 
 
. 
16
.
Establish
 IP
-CAN session Response
-
1
3
f
. 
TDF session establishment
.
OCS
. 
.  
7.
Initial
/
intermediate Spending Limit Report Request
V-PCRF
1
. 
Gateway Control Session Establishment  
(
case
 
2
a
 and 
2
b
)
12.
 
St
 
s
ession
 establishment
.
)


_1487185591.doc


[image: image1.emf]RCAF2 PCRF


1. Diameter NRR


2. Diameter NRA


RCAF1


     


UE moves to RCAF2


3. Diameter NRR


4. Diameter NRA


5. Removal of UE context inRCAF1




_1478495643.vsd

RCAF2



PCRF



1. Diameter NRR






_1460961692.vsd
 


6. Diameter AAR


 


UE


 


P


-


C


SCF


 


2. Define down


-


link 


connection info


 


 


1. SDP offer


 


3. SDP offer


 


4. SDP answer


 


9


. SDP answer


 


5. Define up


-


link  


connection info


 


 


PCRF


 


10


. 


PCC/QoS Rules provisioning


 


7. Store Session 


Information and identify IP


-


CAN Session


 


8. Diameter AAA


 


11. Diameter RAR


 


Legend


:


 


 


       


 


Mandatory


 


               


 


 


Conditional 


 


12. Diameter RAA


 



_1476534144.vsd

_1476857465.vsd
RCAF


PCRF


1. Diameter MUR



_1487184436.doc


[image: image1.emf]BBERF PCEF V-PCRF H-PCRF V-AF SPR


. 


For each 


affected 


AF session


(if any)


2. Diameter ASR


3. Diameter ASR


4. Diameter ASA


5. Diameter ASA


6. Diameter STR


7. Diameter STR


8. Diameter STA


9. Diameter STA


Conditional 


Mandatory


Legend:


1 .   Perform  Step 1 through Step 9: as specified in Figure 4.2.2.1.1 : 


PCEF Initiated IP-CAN Session Termination -AF Located in the HPLMN 


V


I


S


I


T


E


D


 


A


C


C


E


S


S


 


C


A


S


E


10.     Perform Step 14through Step 17:  as specified in in Figure 4.2.2.1.1 : 


PCEF Initiated IP-CAN Session Termination -AF Located in the HPLMN 




_1479539925.vsd




_1487179149.doc


[image: image1.emf] 


UE PCEF V-PCRF H-PCRF P-CSCF


1. Diameter AAR


3. Diameter RAR


SIP message


P-CSCF Restoration Procedure


1a. Diameter AAR


3a. Diameter RAR


2a. Diameter AAA


2. Diameter AAA


4a. DiameterRAA


4. Diameter RAA


Roaming 


with local 


breakout 


when the P-


CSCF is 


located in 


the VPLMN




_1475348680.vsd

UE



PCEF



V-PCRF



H-PCRF



P-CSCF



1. Diameter AAR



3. Diameter RAR



SIP message



P-CSCF Restoration Procedure



1a. Diameter AAR



3a. Diameter RAR



2a. Diameter AAA



2. Diameter AAA



Roaming with local breakout when the P-CSCF is located in the VPLMN



4a. Diameter RAA



4. Diameter RAA






_1476535491.vsd

_1462267849.vsd
 


15


. Diameter AAA


 


13


. Diameter AAR


 


UE


 


P


-


CSCF


 


2. 


Extract 


down


-


link 


service information


 


 


1. SDP


 


offer


 


6


. SDP


 


offer (without NPLI)


 


11


. SDP


 


answer


 


17


. SDP


 


answer


 


12


. 


Extract


 


up


-


link  


service information


 


 


PCRF


 


16


. 


Provisioning of PCC/QoS Rules


 


 


5. Diameter AAA


 


3. Diameter AAR


 


meter AAR


 


4. Authorize 


Service


 


Information 


 


14


. Store Session Information


 


8


. Diameter RAR


 


Legend


:


 


 


       


 


Mandatory


 


               


 


 


Conditional 


 


9


. Diameter RAA


 


If access network information is requested


 


 


7


. 


Provisioning of PCC/QoS Rules


 


 


10a.


SDP offer (With NPLI)


 


 


10b


. 


SIP (With NPLI)


 



_1462268030.vsd
 


10


. Diameter AAR


 


1. SDP offer


 


 


UE


 


P


-


CSCF


 


2. Extract up


-


link  


service information


 


 


 


6


. SDP offer


 


13


. SDP answer (without NPLI)


 


8


. SDP answer


 


9


. Extract down


-


link 


service information


 


 


 


 


PCRF


 


11


.Diameter AAA


 


12


. 


Provisioning of PCC/QoS Rules


 


 


 


 


3. Diameter AAR


 


5.Diameter AAA


 


4. Authorize Service Information


 


 


 


 


7


. 


Provisioning of PCC/


QoS Rules


 


 


 


 


10a


. Store Session Information 


 


14. Diameter RAR


 


Legend


:


 


 


   


    


 


Mandatory


 


               


 


 


Conditional 


 


15. Diameter RAA


 


If access network information is requested 


 


 


16. SDP anwer (With NPLI)


 



_1462268225.vsd
 


1


1


. Diameter AAA


 


10


. Diameter AAR


 


 


P-CSCF


 


 


PCRF


 


1


2


. 


Provisioning of 


PCC/QoS Rules


 


2. extract changes in 


service information 


from SDP offer


 


 


1. SDP offer


 


7


. SDP offer


 


8


. SDP answer


 


9


. extract changes in  


service information 


from SDP answer


 


 


13. SDP answer (without NPLI)


5. Diameter AAA


 


3. Diameter AAR


 


4. authorize service 


information


 


6


. 


Provisioning of 


PCC/QoS Rules


 


1


4


. Diameter RAR


 


Legend


:


 


 


       


 


Mandatory


 


               


 


 


Conditional 


 


1


5


. Diameter RAA


 


If access network information is requested 


 


16. SDP answer (With NPLI)


 



_1460961921.vsd
 


6


. Diameter AAR


 


1. SDP offer


 


UE


 


 


P


-


CSCF


 


2. Define up


-


link  


connection info


 


 


 


3


. SDP offer


 


9. SDP answer (without NPLI)


4


. SDP answer


 


5


. Define down


-


link 


connection info


 


 


 


 


PCRF


 



_1460962552.vsd
 


8


. Diameter AAA


 


6


. Diameter AAR


 


 


P-CSCF


 


 


PCRF


 


7


. 


Store S


ession Info and 


Identify IP


-


CAN Session


 


2. extract changes in 


service information 


from SDP offer


 


 


1. SDP offer


 


3. SDP offer


 


4. SDP answer


 


5. extract changes in  


service information 


from SDP


 


answer


 


 


9. SDP answer (without NPLI)


 


10. 


Interactions in Figure 


4.3.1.1.1


 


11. Diameter RAR


 


Legend


:


 


 


       


 


Mandatory


 


               


 


 


Conditional 


 


12. Diameter RAA


 


If access network Information is requested



_1437392746.vsd

 (
PCRF
TDF
1
. Diameter 
CCR
2
. Diameter 
CCA
)


_1430755371.vsd
 


2. BYE


 


1. BYE


 


 


PCRF


 


P


-


CSCF


 


3.


 


 


Removal of PCC/QoS Rules at AF session release 


 


 


4. 200 OK (BYE)


 


5


. 


200 OK (


BYE)


 



_1430755432.doc














P-CSCF







PCRF







UE







1. BYE, 3xx, 4xx, 5xx, or 6xx







2. BYE, 3xx, 4xx, 5xx, or 6xx































3.	 Removal of PCC/QoS Rules at AF session release












_1430731595.vsd

_1430755325.vsd
 


3. BYE


 


1. BYE


 


 


PCRF


 


P


-


CSCF


 


Removal of PCC/QoS Rules at AF session release


2.


 


 


 


 


 


 



_1429573833.vsd

_1389204330.doc


















PA2







DRA



(proxy)























2. 	Rx/Gx/Gxx/Sd Diameter Request











Legend:



	       	Mandatory



                	Conditional 







1. 	External trigger







7.  PCRF address storage















PCRF-1







Client 



(AF/BBERF/PCEF/TDF)



















3.  DRA binding creation/retrieval











PCRF-2































5. Rx/Gx/Gxx/Sd Diameter Answer











4. Proxy Rx/Gx/Gxx/Sd Diameter Request







6. Proxy Rx/Gx/Gxx/Sd Diameter Answer












_1396342523.doc


BPCF







H-PCRF















1. Trigger







TDF







Roaming Case







3b. Diameter CCA







3a. Diameter CCA







OCS







Roaming Case















4.  PCRF-initiated IP-CAN Session Modification as described in E.4.4.1.1







2b. Store Information 







3. Diameter CCA







Legend:



	       	



Mandatory



                	Conditional 







 2c. Diameter CCR







   AF







V-PCRF







PCEF











 2. Diameter CCR















BBERF



























 2a. Diameter CCR
















_1399222672.vsd
 


5c.Diameter CCA


Roaming case
(Visited Access, case 2a or case 2b)


 


 


 


3c.Diameter RAA


 


3.Diameter RAA


3a.Diameter RAA


3b.Diameter RAA


 


2c.Diameter RAR


 


BBERF


V-PCRF


H-PCRF


TDF


BPCF


SPR


OCS


PCEF


6. Perform the step 3 through step 7 :as specified in Figure 4.2.1.1: UE-initiated IP-CAN session termination-AF located in HPLMN


H/V-AF


2.Diameter RAR


2b.Diameter RAR


4a.Diameter CCR


4.Diameter CCR


5.Diameter CCA


2a.Diameter RAR


4b.Diameter CCR


5b.Diameter CCA


5a.Diameter CCA


8. Perform the step 12 through step 15 :as specified in Figure 4.2.1.1: UE-initiated IP-CAN session termination-AF located in HPLMN


1. Perform the step 1 through step 3: as defined in Figure 4.2.1.1: UE-initiated IP-CAN session termination-AF located in HPLMN,  step 1 through step 5: as defined in Figure 4.2.2.1: PCEF-initiated IP-CAN session termination-AF located in HPLMN or step 1 through step 8: as defined in Figure 4.2.3.1: PCRF-initiated IP-CAN session termination-AF located in HPLMN


7. Perform the step 8 through step 11 :As Specified in Figure 4.2.1.1 for AF located in HPLMN or step 2 through step 9  :as specified in Figure 4.2.1.2 for AF located in VPLMN


Roaming case
(Home Routed and case 1)


Roaming case
(Home Routed and case 1)


Roaming case
(Home Routed and case 1)


Roaming case
(Visited Access, case 2a or case 2b)


Roaming case
(Visited Access, case 2a or case 2b)


Roaming case
(Visited Access, case 2a or case 2b)



_1413447823.vsd
 


4d.Diameter CCR


4e. Store information


 


6a.Diameter CCR


 


6a.Diameter CCA


6.Diameter CCA


6b.Diameter CCA


 


Roaming case
(Visited Access, case 2a or case 2b)


 


 


4a.Diameter CCR


 


Roaming case
(Home Routed and case 1)


2b.Diameter TER/RAR


2.Diameter TER/RAR


4.Diameter CCR


BBERF


V-PCRF


H-PCRF


TDF


BPCF


SPR


OCS


PCEF


1. Perform step 1 through step 6: as specified in Figure 4.1.1: IP-CAN session establishment 


2a.Diameter TER/RAR


2c.Diameter TER/RAR


3.Diameter TEA/RAA


3b.Diameter TEA/RAA


3a.Diameter TEA/RAA


3c.Diameter TEA/RAA


4c.Diameter CCR


7. Perform step 7 through step 15: as specified in Figure 4.1.1: IP-CAN session establishment 


5. Store information 


4b. Store information


Roaming case
(Home Routed and case 1)


 


Roaming case
(Visited Access, case 2a or case 2b)


Roaming case
(Visited Access, case 2a or case 2b)


Roaming case
(Home Routed and case 1)


Roaming case
(Visited Access, case 2a or case 2b)


Roaming case
(Home Routed and case 1)


6c.Diameter CCA



_1429509413.vsd
6. Complete the procedure for CS service 


BPCF


PCRF


HNB GW


2.Diameter CCR


3.Diameter CCA


5.Diameter RAA


1. Trigger 


4.Diameter RAR



_1409467525.vsd

_1396361700.doc


BPCF







H-PCRF







3. Translates QoS rule into access specific QoS











1. Steps 1 to 3 as specified in figure 4.3.2.1.1 







TDF







2b. Qos Rule Validation







4b. Diameter RAA







 2c. Diameter RAR







OCS







SPR















4a. Diameter RAA







Roaming Case







4. Diameter RAA







Legend:



	       	



Mandatory



                	Conditional 







 2a. Diameter RAR







   AF







V-PCRF







PCEF











 2. Diameter RAR















BBERF



























Roaming Case







5. Steps 4 to 18 as specified in figure 4.3.2.1.1












_1398082252.vsd

_1396353643.doc


BPCF







H-PCRF







3. Translates QoS rule into access specific QoS











1. Steps 1 to 3 as specified in figure 4.3.1.1.1







TDF







4b. Diameter RAA







2b. Perform authorization







Roaming Case







OCS







SPR















Roaming Case











 2c. Diameter RAR







4. Diameter RAA







Legend:



	       	



Mandatory



                	Conditional 







4a. Diameter RAA







   AF







V-PCRF







PCEF











 2. Diameter RAR















BBERF



























 2a. Diameter RAR







5. PCRF notifies the AF











_1394461462.vsd




_1396354933.doc


BPCF







H-PCRF















1. Trigger







TDF







3a. Diameter CCA







2b. Store information







Roaming Case







OCS







SPR















Roaming Case







 2a. Diameter CCR







3. Diameter CCA







Legend:



	       	



Mandatory



                	Conditional 







3b. Diameter CCA







   AF







V-PCRF







PCEF











 2. Diameter CCR















BBERF



























 2c. Diameter CCR







4. PCRF-initiated IP-CAN Session Modification as described in E.4.4.2.1











_1394461462.vsd




_1396345115.doc


BPCF







H-PCRF







3. Translates QoS rule into access specific QoS











1. Steps 1 to 3 as specified in figure 4.3.2.1.1 







TDF







4b. Diameter RAA







2b. Qos rule  Validation 







Roaming Case







OCS







SPR















Roaming Case











 2c. Diameter RAR







4. Diameter RAA







Legend:



	       	



Mandatory



                	Conditional 







4a. Diameter RAA







   AF







V-PCRF







PCEF











 2. Diameter RAR















BBERF



























 2a. Diameter RAR







5. Steps 4 to 18 as specified in figure 4.3.2.1.1











_1394461462.vsd




_1390283141.vsd
3. Store  information 


BPCF


PCRF


HNB GW


2.Diameter CCR


4.Diameter CCA


6.Diameter TEA


8.Diameter CCA


1. Trigger 


7.Diameter CCR


5.Diameter TER



_1396243017.doc






















V-DRA



(proxy)























2. 	TER















1. 	External trigger



















V-PCRF-1







H-PCRF















3.   DRA binding creation











V-PCRF-2































5. TEA











4. Proxy TER







6. Proxy TEA












_1396243630.doc






















V-DRA



(Redirect)























2. 	TER















1. 	External trigger



















V-PCRF-1







H-PCRF















3.   DRA binding creation











V-PCRF-2































5. TER











4. TEA(redirect)







6. TEA












_1396325967.doc


BPCF







H-PCRF







3. Translates QoS rule into access specific QoS 











1. Steps 1 to 3 as specified in figure 4.3.1.1.1 







TDF







4b. Diameter RAA







2b. Perform authorization







Roaming Case







OCS







SPR















Roaming Case











 2c. Diameter RAR







4. Diameter RAA







Legend:



	       	



Mandatory



                	Conditional 







4a. Diameter RAA







   AF







V-PCRF







PCEF











 2. Diameter RAR















BBERF



























 2a. Diameter RAR







5. Steps 5 to 12 as specified in figure 4.3.1.1.1











_1394461462.vsd




_1396243553.doc






















V-DRA



(proxy)























2. 	TER















1.  H-PCRF initiated S9 session termination



















V-PCRF-1







H-PCRF















3.  DRA binding verification and release











V-PCRF-2











































4. TEA












_1390304443.vsd
BPCF


PCRF


HNB GW


2.Diameter CCR


3.Diameter CCA


5.Diameter RAA


1. Trigger 


4.Diameter RAR


7.Diameter CCA


6.Diameter CCR



_1389204725.doc






















DRA



(redirect)























2. 	Rx/Gx/Gxx/Sd Diameter  Establishment Request











Legend:



	       	Mandatory



                	Conditional 







1. 	External trigger



















PCRF-1







Client 



(AF/BBERF/PCEF/TDF)



















3.  DRA binding creation/retrieval











PCRF-2































6. Rx/Gx/Gxx/Sd Diameter Answer











4. Diameter Answer (redirect) 







5. Rx/Gx/Gxx/Sd Diameter Establishment Request 












_1389368267.vsd
6. Complete the procedure for CS service 


BPCF


PCRF


HNB GW


2.Diameter CCR


5.Diameter CCA


4.Diameter RAA


1. Trigger 


3.Diameter RAR



_1389204449.doc






















DRA



(proxy)























2. 	Rx/Gx/Gxx/Sd Diameter Termination Request











Legend:



	       	Mandatory



                	Conditional 







1. 	External trigger







6.  Remove DRA binding















PCRF-1







Client



(AF/BBERF/PCEF/TDF)



















3.  Verify DRA binding











PCRF-2































5. Rx/Gx/Gxx/Sd Diameter Answer











4. Proxy Rx/Gx/Gxx/Sd Diameter Termination Request







7. Proxy Rx/Gx/Gxx/Sd Diameter Answer












_1365401694.vsd

_1375725629.vsd

_1389204238.doc






PA2



















5a. Rx/Gx/Gxx/Sd Diameter Answer







DRA



(proxy)























2. 	Sub-sequent Rx/Gx/Gxx/Sd Diameter Request











Legend:



	       	Mandatory



                	Conditional 







1. 	External trigger







2a. 	Sub-sequent Rx/Gx/Gxx/Sd Diameter Request















PCRF-1







Client



(AF/BBERF/PCEF/TDF)



















3.  Verify DRA Binding Binbindingpdate







PA2











PCRF-2































5. Rx/Gx/Gxx/Sd Diameter Answer











4. Proxy Rx/Gx/Gxx/Sd Diameter Request







6. Proxy Rx/Gx/Gxx/Sd Diameter Answer












_1374941679.doc


If a specific action is triggered for the AF session  







H-PCRF







11. Diameter RAR











1. Steps 1 to 5 as specified in figure 4.3.2.1.1. 







TDF







8. Diameter STA







6. Diameter STR







7. Diameter STR







13. Diameter RAA







SPR















If all session info for an AF session is deleted







10. Diameter RAR







3. Diameter ASR







Legend:



	       	



Mandatory



                	Conditional 







9. Diameter STA







V-AF







V-PCRF







PCEF











12. Diameter RAA















BBERF



















5. Diameter ASA







4. Diameter ASA







2. Diameter ASR







14. Steps 12 to 18 as specified in figure 4.3.2.1.1. 












_1375648668.doc






















H-DRA



(proxy)























2. 	S9/Rx Diameter Termination Request











Legend:



	       	Mandatory



                	Conditional 







1. 	External trigger







6.  Remove DRA binding















H-PCRF-1







V-PCRF















3.  Verify DRA binding











H-PCRF-2































5. S9/Rx Diameter Answer











4. Proxy S9/Rx Diameter Termination Request







7. Proxy S9/Rx Diameter Answer












_1360011667.doc






6. 	Perform Step 5-8: as specified in Figure 4.5.1.x.1: PCEF-initiated IP-CAN session modification – IP Flow mobility.



















5. 	PCC/QoS Rules decision based on BCM of GW control session and received IP flow mobility routing rules











BBERF



#2















H-PCRF







3. 	Diameter CCR











Legend:



	       	Mandatory











2. 	Indication of IP flow mobility







4. 	Store Information. 















V-PCRF







BBERF



#1



























PCEF































1. 	Perform Step 1-10: as specified in Figure Figure 4.4.1.1 Gateway Control Session Establishment.




























_1360011724.doc






1. 	Perform Step 1-10: as specified in Figure Figure 4.4.1.1 Gateway Control Session Establishment.











8. 	PCC Rules decision and store



















BBERF



#2















H-PCRF







3. Diameter CCR







4. 	Store Information 







Legend:



	       	Mandatory



                	Conditional 







2. 	Indication of IP flow mobility



















V-PCRF







BBERF



#1















9. 	Diameter CCA







6. Diameter CCR







ROAMING CASE



（Visited access）







PCEF



























10. 	Perform Step 9-10: as specified in Figure 4.5.2.x.1: PCEF-initiated IP-CAN session modification – IP flow mobility.







5. 	Evaluates  IP flow  mobility routing rules







7. Store Information




















_1357659927.doc






8. 	Perform Step 9-10b: as specified in Figure 4.5.1.2.1: PCEF-initiated IP-CAN session modification – Handover.











7c. 	Diameter RAA







6. 	Diameter CCA







4. 	PCC/QoS Rules decision based on BCM of GW control session and received IP flow mobility routing rules











BBERF



#2















H-PCRF







2. 	Diameter CCR











Legend:



	       	Mandatory



                	Conditional 







1. 	Indication of IP flow mobility







3. Store Information







5. 	Store PCC/QoS Rules 











V-PCRF







BBERF



#1















7a. 	Diameter RAR











ROAMING CASE







PCEF























7d. 	Diameter RAR







7b. 	Store PCC/QoS Rules. Policy decision











7e. 	Diameter RAR











7. 	Diameter RAR












_1357710344.doc












9a. 	Diameter CCR











7. 	PCC Rules decision and store











BBERF



#2











5. Diameter CCR







H-PCRF







2. Diameter CCR







3. Store Information







Legend:



	       	Mandatory



                	Conditional 







1. 	Indication of IP flow mobility







6. Store Information















V-PCRF







BBERF



#1















8. 	Diameter CCA







9. 	Extract QoS Rules based on PCC rules and IP flow mobility rules







ROAMING CASE



（Visited access）







PCEF



























10. 	Perform Step 11-14: as specified in Figure 4.5.2.2.1: PCEF-initiated IP-CAN session modification – Handover.







4. 	Change in IP flow  mobility routing rules







9b. 	Diameter CCA




















_1283077675.doc






PA2



















5a. Rx/S9 Diameter Answer







H-DRA



(proxy)























2. 	Rx/S9 Diameter session update











Legend:



	       	Mandatory



                	Conditional 







1.  Trigger







2a. 	Rx/S9 Diameter session update















H-PCRF-1







V-PCRF















3.  Verify DRA Binding Binbindingpdate







PA2











H-PCRF-2































5. Rx/S9 Diameter Answer











4. Proxy Rx/S9 Diameter session update







6. Proxy Rx/S9 Diameter Answer












_1295072951.doc


AF











5.	Store Service information Policy Decision







10. Diameter AAA















PCEF















H-PCRF











If the PCRF needs subscription-related information and does not have it 











SPR







1. Trigger











4. Diameter AAR











V-PCRF







 BBERF











Legend:



	       	Mandatory



                	Conditional 











ROAMING CASE







6. Profile Request















3.Store Service Information







2. Diameter AAR







11. 	Interactions in Figure 4.3.1.1.1







8.	Identify affected IP CAN session(s)











9. Diameter AAA







7. Profile Response




















_1312686790.vsd
4. Diameter CCA


BBERF


3a. Diameter CCR


5. Install and update QoS rules


3c. Diameter CCA


 


3b. Store information and update PCC rules


PCEF


V-PCRF


H-PCRF


3d. Store information and Extract QoS rules


2. Diameter CCR


3. Store information and update QoS rules


 


 


1. Trigger to request QoS rules or report event


Roaming case
(Visited access)


 


Legend


:


 


 


       


 


Mandatory


 


               


 


 


Conditional 


 



_1319486556.doc






10. 	Install QoS Rules. Policy enforcement











8. 	Store PCC/QoS Rules. Policy decision







5. Diameter CCR







11. 	Establish Gateway Control Session Response







Target BBERF















H-PCRF







2. 	Diameter CCR















1. 	Establish Gateway Control Session Request







8b. Diameter CC A







6. Store Information, PCC Rules decision and store











4. 	Identify multiple BBERF(s). Derive and Store QoS Rules. Policy decision







V-PCRF







Source BBERF















8a. Diameter CCR











ROAMING CASE



(Visited access)







PCEF



























7. Diameter CC A







3.  Store Information and identify visited access







9. 	Diameter CCA




















_1319561422.doc






13. 	Install QoS Rules Policy enforcement







10b. Diameter CC A







11 Diameter CCA







8. 	PCC Rules decision and store











Target BBERF











6. Diameter CCR







H-PCRF







2. Diameter CCR







3. Store Information







Legend:



	       	Mandatory



                	Conditional 







1. 	Indication of handover







7. Store Information











5. 	Derive primary/non-primary BBERF(s)







V-PCRF







Source BBERF















9. 	Diameter CC A







14. Diameter RAA







ROAMING CASE



（Visited access）







PCEF























10a. Diameter CCR







10. 	Store PCC/QoS Rules. Policy decision







4. 	Identify multiple BBERF(s) 







12. 	Diameter RAR




















_1312688042.doc






8. 	Install QoS Rules. Policy enforcement







7c. 	Diameter CCR







7. 	Diameter CCA







5. 	PCC/QoS Rules decision based on BCM of GW control session







9. 	Establish Gateway Control Session Response







Target BBERF















H-PCRF







2. 	Diameter CCR











Legend:



	       	Mandatory



                	Conditional 







1. 	Establish Gateway Control Session Request







3. Store Information







6. 	Store PCC/QoS Rules 







4. 	Identify multiple BBERF(s) 







V-PCRF







Source BBERF















7a. 	Diameter CCA







ROAMING CASE







ROAMING CASE







PCEF























7d. 	Diameter CCA







7b. 	Store PCC/QoS Rules. Policy decision







2b. Store Information and identify home routed access







7e. 	Diameter CCA







2a. 	Diameter CCR







2c. Diameter CCR












_1318406968.doc






6. Session Binding 







1. SIP REGISTER







UE







Legend:



	       Mandatory



	       Optional







               











3. 200 OK







5. Diameter AAR







7. Diameter AAA











PCRF







2. SIP REGISTER







4. 200 OK











P-CSCF







8. Provisioning of PCC/QoS Rules




















_1312670571.vsd

_1296317367.doc






9. 	Install QoS Rules. Policy enforcement







8c. 	Diameter RAA







7. 	Diameter CCA







5. 	PCC/QoS Rules decision based on BCM of GW control session







10. 	Diameter RAA







Target BBERF















H-PCRF







2. 	Diameter CCR











Legend:



	       	Mandatory



                	Conditional 







1. 	Indication of handover







3. Store Information







6. 	Store PCC/QoS Rules 







4. 	Derive primary/non-primary BBERF(s) 







V-PCRF







Source BBERF















8a. 	Diameter RAR







10a. 	Diameter RAA







ROAMING CASE







PCEF























8d. 	Diameter RAR







8b. 	Store PCC/QoS Rules. Policy decision







10b. 	Diameter RAA







8e. 	Diameter RAR







ROAMING 



CASE







8. 	Diameter RAR












_1286280670.doc






3. Session Binding 







1. SIP REGISTER







UE







Legend:



	       Mandatory



	       Optional







               











7. 200 OK







2. Diameter AAR







4. Diameter AAA











PCRF







5. SIP REGISTER







6. 200 OK











P-CSCF







8. Subscribe to Bearer Level Event




















_1293975126.doc


















Legend:



	       	Mandatory



                	Conditional 







H-DRA



(proxy)























2. 	Rx/S9 Diameter Request











PA2







1. 	External trigger







7.  PCRF address storage















H-PCRF-1







V-PCRF















3.  DRA binding creation/retrieval











H-PCRF-2































5. Rx/S9 Diameter Answer











4. Proxy Rx/S9 Diameter Request







6. Proxy Rx/S9 Diameter Answer












_1295072109.doc
[image: image1.emf] 


2. Diameter AAR  




AF















7. Diameter AAA















PCEF







5. Profile Response











H-PCRF







If the PCRF needs subscription-related information and does not have it 















Legend:



	       	Mandatory



                	Conditional 















3. 	Store Service Information







4. Profile Request







V-PCRF







 BBERF











1. 	Trigger











8. 	Interactions in Figure 4.3.1.1.1























�



















SPR



















6. 	Identify affected IP CAN session(s)












_1288012781.doc






















H-DRA



(redirect)























2. 	Rx/S9 Diameter Establishment Request











Legend:



	       	Mandatory



                	Conditional 







1. 	External trigger



















H-PCRF-1







V-PCRF















3.  DRA binding creation/retrieval











H-PCRF-2































6. Rx/S9 Diameter Answer











4. Diameter Answer (redirect) 







5.  Rx/S9 Diameter Establishment Request 












_1286875543.doc


Legend:



                       Mandatory







5. SDP answer removing media component







6. Remove/Modify PCC/QoS Rules 







3. Store Session Information and Identify IP-CAN session











P-CSCF







PCRF







UE







2. Diameter AAR







1. SDP answer removing media component







7. PCEF closes the related gate























4. Diameter AAA












_1285108645.vsd
BBERF


2b. Store information


5. Install and update QoS rules


2. Diameter CCR


 


Roaming case
(home-routed)


PCEF


V-PCRF


H-PCRF


2a. Diameter CCR


2c. Diameter CCR


4b. Extract QoS rules


3. Store information and update QoS rules


4. Diameter CCA


4a. Diameter CCA


4c. Diameter CCA


Roaming case
(home routed)


1. Trigger to request QoS rules or report event



_1286270078.doc


Legend:



                        Mandatory







7. PCRF closes the related gate







3. PCRF updates flow status of affected PCC rules







P-CSCF







PCRF







UE







2. Diameter AAR







1. SDP answer putting media on hold







5. SDP answer putting media on hold















4. Diameter AAA












_1286280605.doc






6. Session Binding 







1. SIP REGISTER







UE







Legend:



	       Mandatory



	       Optional







               











3. 200 OK







5. Diameter AAR







7. Diameter AAA











PCRF







2. SIP REGISTER







4. 200 OK











P-CSCF







8. Subscribe to Bearer Level Event




















_1286269826.doc


Legend:



                         Mandatory















3. PCRF updates flow status of affected PCC rules 







6. PCRF opens the gates







1. 2xx Success







P-CSCF







PCRF







UE







2. Diameter AAR















4. Diameter AAA







5. 2xx Success












_1283078964.doc






PA2



















5a. S9 Diameter Answer







H-DRA



(proxy)























2. 	S9 Diameter Request











Legend:



	       	Mandatory



                	Conditional 







1.  Trigger







2a. 	S9 Diameter Request















H-PCRF-1







V-PCRF















3.  Verify DRA Binding Binbindingpdate







PA2











H-PCRF-2































5. S9Diameter Answer











4. Proxy S9 Diameter Request







6. Proxy S9 Diameter Answer












_1284887142.doc






















H-DRA



(redirect)























2 	S9 Diameter Termination Request











Legend:



	       	Mandatory



                	Conditional 







1. 	External trigger



















H-PCRF-1







V-PCRF















4.  DRA binding verification/release











H-PCRF-2































5 Diameter Redirect Answer











6. S9 Diameter Answer







3. 	S9 Diameter Termination Request












_1280789801.doc


AF session signalling , possibly with SDI







Signalling path







Gx/Gxx







 Application







IP BS Manager







Access-Specific  BS Manager







 Translation / Mapping function







UE (3)











 Access- Specific BS Manager (5)







Translation / Mapping function (4)







PCEF/BBERF 







IP BS Manager























GW







PCRF (2)







 AF (1)







Service information







Rx







SDI Mapping function







LEGEND







Policy Engine







Authz IP QoS parameters







Access-specific QoS parameters












_1280789988.doc


AF session signalling , possibly with SDI







Signalling path



Only for some IP CAN Types and terminals



Only 







Gx/Gxx







 Application







IP BS Manager







Access-Specific  BS Manager







 Translation / Mapping function







UE











 Access- Specific BS Manager







3) Translation / Mapping function







PCEF/BBERF 











IP BS Manager























GW







PCRF







 AF







Service information







Rx







1) SDI Mapping function







LEGEND







2) Policy Engine







Authorized  IP QoS parameters















Authorized  IP QoS parameters















Authorized  



Access-Specific



QoS parameters







Authorized



Access-specific QoS parameters















Authorized  



Access-Specific



QoS parameters







Authorized  IP QoS parameters












_1280819597.doc






PA2



















5a. Rx/Gx/Gxx Diameter Answer







DRA



(proxy)























2. 	PCRF initiated Rx/Gx/Gxx Diameter Request











Legend:



	       	Mandatory



                	Conditional 







1.  Trigger







2a. 	PCRF initiated Rx/Gx/Gxx Diameter Request















PCRF-1







Client



(AF/BBERF/PCEF)



















3.  Verify DRA Binding Binbindingpdate







PA2











PCRF-2































5. Rx/Gx/Gxx Diameter Answer











4. Proxy Rx/Gx/Gxx Diameter Request







6. Proxy Rx/Gx/Gxx Diameter Answer












_1280904883.doc






















DRA



(redirect)























2 	Gx/Gxx Diameter Termination Request











Legend:



	       	Mandatory



                	Conditional 







1. 	External trigger



















PCRF-1







Client



(BBERF/PCEF)



















4.  DRA binding verification/release











PCRF-2































5 Diameter Redirect Answer











6. Gx/Gxx Diameter Answer







3. 	Gx/Gxx Diameter Termination Request












_1280789875.doc


AF session signalling , possibly with SDI







Signalling path







Gx/Gxx







 Application







IP BS Manager







Access-Specific  BS Manager







 Translation / Mapping function







3) UE











5) Access- Specific BS Manager







4) Translation / Mapping function







PCEF/BBERF 











IP BS Manager























GW







PCRF







 AF







Service information







Rx







1) SDI Mapping function







LEGEND







2) Policy Engine







Authorized  IP QoS parameters







Access-specific QoS parameters







IP QoS parameters







Authorized  IP QoS parameters







IP QoS parameters







Access-Specific



QoS parameters







Authorized  



Access-Specific



QoS parameters







Authorized



Access-specific QoS parameters







Access-Specific



QoS parameters







IP QoS parameters







Authorized  



Access-Specific



QoS parameters







Authorized  IP QoS parameters












_1275910247.vsd
BBERF


1. Remove Gxx Session Request


2. Diameter CCR


Roaming case


5. Remove Gxx Session Reply


PCEF


V-PCRF


H-PCRF


2a. Diameter CCR


2b. Diameter CCR


4b. Identify related IP-CAN session and perform update if needed


3. Identify related IP-CAN session and perform update as needed.


4. Diameter CCA


4a. Diameter CCA


Roaming case


4c. Diameter CCA



_1279117736.vsd
BBERF


1. Remove Gxx Session Request


2. Diameter RAR


4. The BBERF terminates the Gateway control session based on the BBERF-initiated termination procedures.


PCEF


V-PCRF


H-PCRF


2a. Diameter RAR


2b. Diameter RAR


3. Diameter RAA


3b. Diameter RAA


3a. Diameter RAA


Roaming case


Roaming case



_1223986997.doc


AF session signalling , possibly with SDI







Signalling path



Only for some IP CAN Types and terminals



Only 







Gx







 Application







IP BS Manager







Access-Specific  BS Manager







 Translation / Mapping function







UE











 Access- Specific BS Manager







3) Translation / Mapping function







PCEF 











IP BS Manager























GW







PCRF







 AF







Service information







Rx







1) SDI Mapping function







LEGEND







2) Policy Engine







Authorized  IP QoS parameters















Authorized  IP QoS parameters















Authorized  



Access-Specific



QoS parameters







Authorized



Access-specific QoS parameters















Authorized  



Access-Specific



QoS parameters







Authorized  IP QoS parameters












_1275910185.vsd
5. Access specific procedures to modify bearers


BBERF


2. Diameter RAR


Roaming case


PCEF


V-PCRF


H-PCRF


2c. Diameter RAR


2a. Diameter RAR


4. Diameter RAA


4b. Diameter RAA


4a. Diameter RAA


Roaming case


1. Trigger to update QoS rules


2b. Identify the gateway control session and perform authorization


3. Install/Update QoS rules



_1270541146.doc
[image: image1.emf] 


5 . Install ation Modification or Removal  of PCC Rules, P olicy  E nforcement  




SPR







�10. Install PCC Rules,



policy enforcement







For GPRS, for each affected other bearer where previous authorized QoS is modified



(because IP Flow(s) were moved to other bearer(s))











11. 	Diameter CCA







4. 	Bind IP CAN session to any existing AF session(s)











GW







If the PCRF needs subscription- related information and does not have it 











6. 	Profile Response







PCRF







2. 	Diameter CCR







9. 	PCC Rules Decision,�Policy Decision







14. 	Start of Timer.







1. 	IP-CAN Bearer signalling 







3. 	Store Bearer Information







10. 	Store PCC Rules 







5. 	Profile Request







Legend:



	       	Mandatory



                	Conditional 











7. 	Diameter RAR







12. 	Installation Modification or Removal of PCC Rules, Policy Enforcement







AF







For IP CAN session modification, if requested by AF at initial authorisation 











8. 	Diameter RAA







































13. 	IP CAN Bearer signalling response







15. 	Interactions in Figure 4.3.1.1.1












_1083082145.doc


IP BS Manager







Translation /�Mapping







4)







3)







2)







1)







UMTS�QoS�Param.�Per�Applic Type







UE







(SDP)







PDP Contex Activation and Modification







Session�Manager







SGSN







Session�Manager







UMTS BS Manager







SDP Handler







Application




















_1209539289.doc


If not all the service data flows within AF session are affected







4. 	PCC Rules Decision.�Policy Decision







7. 	Remove PCC Rules 







AF







PCRF







GW











If IP Flow(s) were moved to other bearer(s)







6. 	Diameter CCA







1. 	Remove IP-CAN Bearer Request 







2. 	Diameter CCR







10. Diameter ASR







11. Diameter ASA







12a. Diameter AAR







13a.	Diameter AAA







11a. Diameter RAA







If all the service data flows within AF session are affected







10a. Diameter RAR







12. Diameter STR







13. Diameter STA











9. 	Install PCC Rules. Policy Enforcement







5. 	Store PCC Rules 







3. 	Identify the IP flows bound to the session or bearer.



Re-evaluate binding .







8. 	Remove IP-CAN Bearer Response 











If IP Flow(s) were moved to other bearer(s)











Legend:



	       	



Mandatory



                  	 Conditional 












