3GPP TSG-CT WG3 Meeting #117e	C3-214150

# E-Meeting, 18th – 27th August 2021

Source:	Huawei, Nokia, Nokia Shanghai Bell

Title:	Ntsctsf_TimeSynchronization_ConfigUpdateNotify service operation

Spec:	3GPP TS 29.abc

Agenda item:	17.16

Document for:	Decision

1. Introduction

<Introduction part (optional)>

2. Reason for Change

Ntsctsf_TimeSynchronization_ConfigUpdateNotify service operation needs to be specified.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TS 29.abc.

* * * First Change * * * *

5.2.2.8	Ntsctsf_TimeSynchronization_ConfigUpdateNotify

5.2.2.8.1	General

This service operation is used by an TSCTSF to notify the NF Service Consumer of the state of the time synchronization configuration.

The following procedures using the Ntsctsf_TimeSynchronization_ConfigUpdateNotify service operation are supported:

-	notification about the state of the time synchronization configuration.

5.2.2.8.2	Deleting an existing configuration

Figure 5.2.2.8.2-1 illustrates the notification about the state of the time synchronization configuration.

Figure 5.2.2.8.2-1: Notification about the state of the time synchronization configuration

When the TSFCST receives the notification for each DS-TT and NW-TT from the PCF, the TSCTSF shall determine the state of the time synchronization configuration and sends an HTTP POST request with "{configNotifUri}", as previously provided by the NF service consumer within the corresponding configuration, as URI and the TimeSyncExposureConfigNotif data structure as request body, as shown in figure 5.2.2.8.2-1, step 1.

The TimeSyncExposureConfigNotif data structure as request body that shall include:

-	Notification correlation ID provided by the NF service consumer during the subscription within "subsNotifId" attribute; and

-	state of the time synchronization configuration.

Editor's Note:	Error/Redirect responses are FFS.

Editor's Note:	Whether state of the time synchronization configuration is a mandatory parameter is FFS.

* * * End of Changes * * * *