3GPP TSG-CT3 Meeting #122e	C3-223113

# E-Meeting, 12th – 20th May 2022											(revision of C3-22xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.122 | CR | 0584 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to TscQosRequirement and TscQosRequirementRm |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | IIoT |  |  |  |  |  | Date: |  |  | 2022-05-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The "tscaiInputUl" and "tscaiInputDl attributes are used to construct the the TSC Assistance Container according to stage 2 requirement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the descriptions of the "tscaiInputUl" and "tscaiInputDl attributes |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong descriptions lead to misunderstanding. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.14.2.1.9, 5.14.2.1.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR doesn’t impact the OpenAPI file. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * Start of Changes * * * *

5.14.2.1.9	Type: TscQosRequirement

Table 5.14.2.1.9-1: Definition of type TscQosRequirement

| Attribute name | Data type | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- |
| reqGbrDl | BitRate | 0..1 | Requested GBR in downlink. |  |
| reqGbrUl | BitRate | 0..1 | Requested GBR in uplink. |  |
| reqMbrDl | BitRate | 0..1 | Requested MBR in downlink. |  |
| reqMbrUl | BitRate | 0..1 | Requested MBR in uplink. |  |
| maxTscBurstSize | ExtMaxDataBurstVol | 0..1 | Maximum burst size of the TSC traffic in units of Bytes. Minimum = 4096, Maximum = 2000000. |  |
| req5Gsdelay | PacketDelBudget | 0..1 | Requested Delay of the TSC traffic. |  |
| priority | TscPriorityLevel | 0..1 | Unsigned integer indicating the TSC traffic priority in relation to other TSC and non-TSC traffic. |  |
| tscaiTimeDom | Uinteger | 0..1 | Indicates the (g)PTP domain that the (TSN)AF is located in. |  |
| tscaiInputUl | TscaiInputContainer | 0..1 | Transports the input parameters for TSC traffic to construct the TSC Assistance Container in uplink direction. |  |
| tscaiInputDl | TscaiInputContainer | 0..1 | Transports the input parameters for TSC traffic to construct the TSC Assistance Container in downlink direction. |  |

# * * * * Next change * * * *

5.14.2.1.10	Type: TscQosRequirementRm

This data type is defined in the same way as the "TscQosRequirement" data type, but:

-	the removable attributes "reqGbrDl", "reqGbrUl", "reqMbrDl" and "reqMbrUl" are defined with the removable data type "BitRateRm"; the removal attribute "tscaiTimeDom" is defined with the removal data type "UintegerRm", "priority" attribute is defined with the removal data type "TscPriorityLevelRm", "maxTscBurstSize" is defined with the removable data type "ExtMaxDataBurstVolRm"; "req5Gsdelay" is defined with the removable data type "PacketDelBudgetRm" in the OpenAPI.

Table 5.14.2.1.10-1: Definition of type TscQosRequirementRm

| Attribute name | Data type | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- |
| reqGbrDl | BitRateRm | 0..1 | Requested GBR in downlink. |  |
| reqGbrUl | BitRateRm | 0..1 | Requested GBR in uplink. |  |
| reqMbrDl | BitRateRm | 0..1 | Requested MBR in downlink. |  |
| reqMbrUl | BitRateRm | 0..1 | Requested MBR in uplink. |  |
| maxTscBurstSize | ExtMaxDataBurstVolRm | 0..1 | Maximum burst size of the TSC traffic in units of Bytes. Minimum = 4096, Maximum = 2000000. |  |
| req5Gsdelay | PacketDelBudgetRm | 0..1 | Requested Delay of the TSC traffic. |  |
| priority | TscPriorityLevelRm | 0..1 | Unsigned integer indicating the TSC traffic priority in relation to other TSC and non-TSC traffic. |  |
| tscaiTimeDom | UintegerRm | 0..1 | Indicates the (g)PTP domain that the (TSN)AF is located in. |  |
| tscaiInputUl | TscaiInputContainer | 0..1 | Transports the input parameters for TSC traffic to construct the TSC Assistance Container in uplink direction. |  |
| tscaiInputDl | TscaiInputContainer | 0..1 | Transports the input parameters for TSC traffic to construct the TSC Assistance Container in downlink direction. |  |

# * * * * End of change * * * *