3GPP TSG-CT WG3 Meeting #122-e	C3-223155

# E-Meeting, 12th – 20th May 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Pseudo-CR on the error cases update to Npcf_MBSPolicyControl UpdateNotify Service

Spec:	3GPP TS 29.537 V0.2.0

Agenda item:	17.31 (5MBS)

Document for:	Approval

1. Introduction

TS 29.537 has been allocated under the 5MBS work item to define the MBS Policy Control services.

2. Reason for Change

Error cases for Npcf_MBSPolicyControl_UpdateNotify service operation needs to be specified. Error cases as specified in Ts 29.512 are referred and the common error cases applicable to MBS sessions are specified.

3. Conclusions

N/A.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 29.537 V0.2.0.

* * * First Change * * * *

##### 5.2.2.3.2	PCF initiated MBS Session Policy Association Update

Figure 5.2.2.3.2-1: MBS Session Policy Association update

1.	The PCF may decide to provision policies related to an Individual MBS Policy resource in response to e.g. an internal trigger within the PCF. The PCF shall send for this purpose an HTTP POST request to the NF service consumer (e.g. MB-SMF) using the URI"{notificationUri}/update" with the "notificationUri" set to the notification URI received during MBS Session Policy Association establishment procedure as defined in clause 5.2.2.2. The request message body shall contain a MbsPolicyNotif data structure that shall contain:

-	the representation of the updated policies within the "mbsPolicyDecision" attribute; and

-	the identifier of the Individual MBS Policy resource related to the notification, within the "mbsPolicyId" attribute.

2.	In case of a successful update of MBS policies, a "204 No Content" response code shall be returned in the response.

If errors occur when processing the HTTP POST request, the NF service consumer shall send an HTTP error response as specified in clause 6.1.7

Editor's Note:	The complete list of Error cases and the related status codes are FFS.

If the NF service consumer received one or more MBS PCC rules from the PCF, and the validation of all these MBS PCC Rules was unsuccessful, the NF service consumer shall reject with an HTTP "400 Bad Request" status code with the response body including the MbsErrorReport data structure. Within the MbsErrorReport data structure, the NF service consumer shall include the "error" attribute containing the "cause" attribute of the ProblemDetails data structure set to "MBS_PCC_RULE_PERMANENT_FAIL" or "MBS_PCC_RULE_TEMP_FAIL" and the "mbsRuleReports" attribute to report the MBS PCC rule status of the affected MBS PCC rules.

Editor's Note:	The handling of predefined rules is FFS.

* * * Next Change * * * *

6.1.5.2.3.1	POST

This method shall support the request data structures specified in table 6.1.5.2.3.1-1 and the response data structures and response codes specified in table 6.1.5.2.3.1-2.

Table 6.1.5.2.3.1-1: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MbsPolicyNotif | M | 1 | Provision/Update of MBS policies by the PCF. |

Table 6.1.5.2.3.1-2: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The MBS policies are provisioned/updated successfully. |
| array(PartialMbsSuccessReport) |  | O |  | 1..N |  | 200 OK |  | Some of the MBS PCC rules provisioned by the PCF are not installed successfully. |  |
| MbsErrorReport |  | M |  | 1 |  | 400 Bad Request |  | The MBS policies including all the MBS PCC rules provisioned by the PCF are not installed successfully. |  |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative NF service consumer (service) instance where the notification should be sent. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative NF service consumer (service) instance where the notification should be sent. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of TS 29.500 [4] also apply. |  |  |  |  |

Editor's Note:	The handling of predefined rules is FFS.

Table 6.1.5.2.3.1-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | String | M | 1 | An alternative URI representing the end point of an alternative NF consumer (service) instance towards which the notification should be redirected. |
| 3gpp-Sbi-Target-Nf-Id | String | O | 0..1 | Identifier of the target NF (service) instance towards which the notification request is redirected |

Table 6.1.5.2.3.1-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | String | M | 1 | An alternative URI representing the end point of an alternative NF consumer (service) instance towards which the notification should be redirected. |
| 3gpp-Sbi-Target-Nf-Id | String | O | 0..1 | Identifier of the target NF (service) instance towards which the notification request is redirected |

Editor's Note:	There may be updates (e.g. additional scope) depending on the progress of the related stage 2 work.

* * * Next Change * * * *

### 6.1.6	Data Model

#### 6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the  Npcf_MBSPolicyControl service based interface protocol.

Table 6.1.6.1-1: Npcf_MBSPolicyControl specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| MbsPolicyCtxtData | 6.1.6.2.2 | Contains the parameters used to request the creation of an Individual MBS Policy resource. |  |
| MbsPolicyData | 6.1.6.2.4 | Contains the MBS policy data of an Individual MBS Policy resource. |  |
| MbsPolicyDecision | 6.1.6.2.3 | Contains the MBS policies authorized by the PCF. |  |
| MbsPolicyNotif | 6.1.6.2.5 | Represents an MBS policy update notification. |  |
| MbsTermNotif | 6.1.6.2.6 | Represents an MBS policy termination notification. |  |
| PartialMbsSuccessReport | 6.1.6.2.X | Includes the information reported by the NF service consumer when some of the MBS PCC rules are not successfully installed. |  |
| MbsErrorReport | 6.1.6.2.Y | Contains the rule reports. |  |

Table 6.1.6.1-2 specifies data types re-used by the Npcf_MBSPolicyControl service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Npcf_MBSPolicyControl service based interface.

Table 6.1.6.1-2: Npcf_MBSPolicyControl re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| Dnn | TS 29.571 [15] | Identifies a DNN. |  |
| MbsSessionId | TS 29.571 [15] | Represents an MBS Session Identifier. |  |
| RedirectResponse | TS 29.571 [15] | Contains redirection related information. |  |
| Snssai | TS 29.571 [15] | Identifies an S-NSSAI. |  |
| SupportedFeatures | TS 29.571 [15] | Represents the list of supported features. It is used to negotiate the applicability of the optional features. |  |
| Uri | TS 29.571 [15] | Represents a URI. |  |

* * * Next Change * * * *

##### 6.1.6.2.X	Type: PartialMbsSuccessReport

Table 6.1.6.2.X-1: Definition of type PartialMbsSuccessReport

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |

Editor's Note:	The complete list of attributes is FFS.

* * * Next Change * * * *

##### 6.1.6.2.Y	Type: MbsErrorReport

Table 6.1.6.2.Y-1: Definition of type MbsErrorReport

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |

Editor's Note:	The complete list of attributes is FFS.

* * * Next Change * * * *

#### 6.1.7.3	Application Errors

The application errors defined for the Npcf_MBSPolicyControl service are listed in Table 6.1.7.3-1.

Table 6.1.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| MBS_PCC_RULE_PERMANENT_FAIL | 400 Bad Request |  | The HTTP request is rejected because all the MBS PCC rules provisioned by the PCF in the request cannot be installed. It is used to inform the PCF that the request failed and should not be attempted again. |  |
| MBS_PCC_RULE_TEMP_FAIL | 400 Bad Request |  | The HTTP request is rejected because for some reason all the MBS PCC rules provisioned by the PCF in the request cannot be enforced or modified successfully in a network initiated procedure. It is used to inform the PCF that the request could not be satisfied at the time it was received but may be able to satisfy the request in the future. |  |

Editor's Note:	The complete list of application errors is FFS.

* * * End of Changes * * * *