3GPP TSG-CT WG3 Meeting #122-e	C3-223156

# E-Meeting, 12th – 20th May 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Pseudo-CR on the error cases update to Npcf_MBSPoliyAuthorization_Create Service

Spec:	3GPP TS 29.537 V0.2.0

Agenda item:	17.31 (5MBS)

Document for:	Approval

1. Introduction

TS 29.537 has been allocated under the 5MBS work item to define the MBS Policy Authorization services.

2. Reason for Change

Error cases for Npcf_MBSPoliyAuthorization_Create service operation needs to be specified. Error cases as specified in Ts 29.514 are referred and the common error cases applicable to MBS sessions are specified.

3. Conclusions

N/A.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 29.537 V0.2.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[5]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[6]	OpenAPI: "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[7]	3GPP TR 21.900: "Technical Specification Group working methods".

[8]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[9]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[10]	3GPP TS 29.510: "5G System; Network Function Repository Services; Stage 3".

[11]	IETF RFC 7540: "Hypertext Transfer Protocol Version 2 (HTTP/2)".

[12]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[13]	IETF RFC 7807: "Problem Details for HTTP APIs".

[14]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2".

[15]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[16]	3GPP TS 29.532: "5G System; 5G Multicast-Broadcast Session Management Services; Stage 3".

[17]	3GPP TS 29.514: "5G System; Policy Authorization Service; Stage 3".

[XX]	3GPP TS 29.214: "Policy and Charging Control over Rx reference point".

* * * Next Change * * * *

##### 5.3.2.2.2	MBS Application Session Context Establishment

Figure 5.3.2.2.2-1: MBS Application Session Context establishment

1.	In order to request the creation of an MBS Application Session Context, the NF service consumer (e.g. AF, NEF, MBSF) shall send an HTTP POST request to the PCF, as described in step 1of figure 5.3.2.2.2-1, with the request body containing the MbsAppSessionCtxt data structure that shall contain the requested MBS application service requirements.

2.	Upon reception of the HTTP POST request from the NF service consumer, the PCF shall perform MBS application service requirements authorization based on the information received from the NF service consumer and operator policies that are pre-configured at the PCF.

If the MBS application service requirements authorization is successful, the PCF shall create a new "Individual MBS Application Session Context" resource and respond to the NF service consumer with an HTTP "201 Created" status code, including an HTTP Location header field containing the URI of the created "Individual MBS Application Session Context" resource and the response body containing a representation of the created resource within the MbsAppSessionCtxt data structure.

The NF service consumer shall use the URI received in the HTTP Location header in subsequent requests to the PCF targeting the created "Individual MBS Application Session Context" resource.

If errors occur when processing the HTTP POST request, the PCF shall apply the error handling procedures specified in clause 6.2.7.

Editor's Note:	complete list of Error cases and the related status codes are FFS.

If the service information provided in the body of the HTTP POST request is rejected (e.g. not authorized), the PCF shall reject the request with an HTTP "403 Forbidden" status code including the ProblemDetails data structure with the "cause" attribute set to "REQUESTED_MBS_SERVICE_NOT_AUTHORIZED".

If the service information provided in the body of the HTTP POST request is invalid, incorrect or insufficient for the PCF to authorize the MBS policy authorization request (e.g. invalid media type or invalid QoS reference.), the PCF shall reject the request with an HTTP "400 Bad Request" status code including the ProblemDetails data structure with the "cause" attribute set to "INVALID_MBS_SERVICE_INFORMATION".

If the service information provided in the body of the HTTP POST request has IP flow descriptions that cannot be handled by the PCF because the restrictions defined in clause 5.3.8 of 3GPP TS 29.214 [XX] are not observed, the PCF shall indicate in an HTTP "400 Bad Request" response message the cause for the rejection including the "cause" attribute set to "FILTER_RESTRICTIONS".

* * * Next Change * * * *

###### 6.2.3.2.3.1	POST

This method shall support the URI query parameters specified in table 6.2.3.2.3.1-1.

Table 6.2.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.2.3.1-2 and the response data structures and response codes specified in table 6.2.3.2.3.1-3.

Table 6.2.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MbsAppSessionCtxt | M | 1 | Contains the parameters to create an Individual MBS Application Session Context resource. |

Table 6.2.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MbsAppSessionCtxt | M | 1 | 201 Created | Successful case. An Individual MBS Application Session Context resource is successfully created and a representation of the created resource is returned to the NF service consumer. |
| ProblemDetails |  | O |  | 0..1 |  | 400 Bad Request |  | (NOTE 2) |  |
| ProblemDetails |  | O |  | 0..1 |  | 403 Forbidden |  | (NOTE 2) |  |
| NOTE 1: The mandatory HTTP error status code for the HTTP POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: Failure cases are described in subclause 6.2.7. |  |  |  |  |

Table 6.2.3.2.3.1-4: Headers supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/npcf-mbspolicyauth/<apiVersion>/mbs-app-sessions/{mbsAppSessionId} |

* * * Next Change * * * *

#### 6.2.7.3	Application Errors

The application errors defined for the Npcf_MBSPolicyAuthorization service are listed in table 6.2.7.3-1.

Table 6.2.7.3-1: Application errors

| Application Error |  |  |  | HTTP status code |  | Description |
| --- | --- | --- | --- | --- | --- | --- |
| INVALID_MBS_SERVICE_INFORMATION | 400 Bad Request |  | The HTTP request is rejected because the service information is invalid or insufficient for the PCF to perform the requested action, e.g. invalid media type or invalid QoS reference. |  |  |  |
| FILTER_RESTRICTIONS | 400 Bad Request |  | The HTTP request is rejected because the IP flow descriptions cannot be handled by the PCF because the restrictions defined in clause 5.3.8 of 3GPP TS 29.214 [XX] are not observed. |  |  |  |
| REQUESTED_MBS_SERVICE_NOT_AUTHORIZED |  | 403 Forbidden |  |  | The service information provided in the request is rejected because the service is not authorized. |  |

* * * End of Changes * * * *