3GPP TSG-CT WG3 Meeting #122-e	C3-223167

# E-Meeting, 12th – 20th May 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Pseudo-CR on the MBS Policy Decision update to Npcf_MBSPolicyControl Create Service

Spec:	3GPP TS 29.537 V0.2.0

Agenda item:	17.31 (5MBS)

Document for:	Approval

1. Introduction

TS 29.537 has been allocated under the 5MBS work item to define the MBS Policy Control services.

2. Reason for Change

MbsPolicyDecision data structure update for Npcf_MBSPolicyControl_Create service operation.

Stage 2 requirements for policy data are defined in TS 23.247:

### 9.2.2	Npcf_MBSPolicyControl service

#### 9.2.2.1	General

Service description: NF Service Consumer, e.g. MB-SMF can create and manage a MBS Policy Association in the PCF through which the NF Service Consumer receives policy information for a MBS Session.

As part of this service, the PCF may provide the NF Service Consumer, e.g. MB-SMF with policy information about the MBS Session that may contain:

-	MBS Session related policy information.

-	PCC rule information.

-	Policy Control Request Trigger information. When a Policy Control Request Trigger condition is met the NF Service Consumer, e.g. MB_SMF shall contact the PCF and provide information on the Policy Control Request Trigger condition that has been met.

3. Conclusions

N/A.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 29.537 V0.2.0.

* * * First Change * * * *

##### 5.2.2.2.2	MBS Session Policy Association Establishment

Figure 5.2.2.2.2-1: MBS Session Policy Association establishment

1.	In order to request the creation of an MBS Session Policy Association, the NF service consumer (e.g. MB-SMF) shall send an HTTP POST request to the PCF, as described in step 1of figure 5.2.2.2.2-1, with the request body containing the "MbsPolicyCtxtData" data structure that shall contain:

-	the concerned MBS Session Id, within the "mbsSessionId" attribute;

-	the DNN of the MBS session within the "dnn" attribute;

-	the S-NSSAI of the MBS session, within the "snssai" attribute; and

-	the URI towards which MBS policies update notifications should be sent by the PCF, within the "notificationUri" attribute.

Editor's Note:	The complete list of attributes is FFS.

2.	Upon reception of the HTTP POST request from the NF service consumer, the PCF shall perform MBS perform MBS policy authorization based on the information received from the NF service consumer and operator policies that are pre-configured at the PCF. If MBS policy authorization is successful, the PCF shall create a new "Individual MBS Policy" resource, addressed by a URI as defined in clause 6.1.3.2 and containing a PCF created resource identifier. The PCF shall then respond to the NF service consumer with an HTTP 201 Created response, including a Location header field containing the URI of the created resource and the MbsPolicyDecision data structure in the response body.

Editor's Note:	The detailed content of the MbsPolicyData data structure is FFS.

The NF service consumer shall use the URI received in the Location header in subsequent requests to the PCF to refer to the created "Individual MBS Policy" resource.

If errors occur when processing the HTTP POST request, the PCF shall apply the error handling procedures specified in clause 6.1.7.

Editor's Note:	Error / redirection cases and the related status codes are FFS.

* * * Next Change * * * *

#### 5.2.X.m	Provisioning and Enforcement of MBS Policy Decisions

##### 5.2.X.m.1	General

MBS Policy Decisions are provided from the PCF to the NF service consumer (MB-SMF) as part of the following service operations:

-	the Npcf_MBSPolicyControl_Create Service Operation described in subclause 5.2.2.2; and

-	the MBS Policy Association Notification request as part of the Npcf_MBSPolicyControl_UpdateNotify Service Operation as described in subclause 5.2.2.3;

MBS Policy decisions shall be encoded within the MbsPolicyDecision data structure defined in subclause 6.1.6.2.3

MBS Policy decisions may include:

-	MBS PCC Rule(s), as described in subclause 5.2.1.x, encoded within the "mbsPccRules" attribute;

-	QoS policy decision(s), as described in subclause 5.2.1.m, which can be referenced from MBS PCC rule(s), encoded within the "mbsQosInfos" attribute;

-	QoS characteristics for non-standard 5QIs and non-preconfigured 5QIs provided within the "mbsQosChars" attribute;

-	the authorized session AMBR within the "authSessAmbr" attribute

For the Npcf_MbsPolicyControl_Create Service Operation, the MbsPolicyDecision data structure shall contain a full description of all MBS policy decision(s) provided by the PCF for the MBS policy association.

For the Npcf_MBSPolicyControl_UpdateNotify service operation for the MBS Policy Association Notification request and for the Npcf_MBSPolicyControl_Update service operation, the MbsPolicyDecision data structure shall contain a description of the changes to the MBS policy decision(s) with respect to the last provided MBS policy decision(s) for the corresponding MBS policy association.

Editor's Note:	The applicable scenarios and service operations may need to be updated based on stage 2 progress.

* * * Next Change * * * *

##### 5.2.2.x.2	MBS PCC rule

The PCF may perform an operation on a single MBS PCC rule or a group of MBS PCC rules. The impacted MBS PCC rule(s) shall be included in the "mbsPccRules" map attribute within the MbsPolicyDecision data structure with the associated "pccRuleId" as the key of the map. For activating a installing or modifying a dynamic PCF-provisioned MBS PCC rule, the corresponding MbsPccRule data structure shall be provided as the map entry value. For deactivating or removing a MBS PCC rule, the map entry value shall be set to "NULL".

Editor's Note:	 It is FFS is pre-defined MBS PCC rules is defined/used for MBS sessions.

In order to install a new dynamic PCF-provisioned MBS PCC rule, the PCF shall further set other attributes within the PccRule data structure as follows:

-	It may include the precedence of a MBS PCC rule among the other MBS PCC rules of the MBS session, within the "precedence" attribute. Within a MBS session, the PCF shall authorize different precedence values for the MBS PCC rules whose packet filters contained within the "flowInfos" attribute.

-	It shall include flow information within the "flowInfos" attribute.

-	It shall include one reference to the MbsQosInfo data structure within the "refMbsQosInfo" attribute. In this case, a "mbsQosInfos" attribute containing the corresponding QoS data policy decision shall be included in the MbsPolicyDecision data structure, if it has not been previously provided.

* * * Next Change * * * *

###### 6.1.3.2.3.1	POST

This method shall support the URI query parameters specified in table 6.1.3.2.3.1-1.

Table 6.1.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.2.3.1-2 and the response data structures and response codes specified in table 6.1.3.2.3.1-3.

Table 6.1.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MbsPolicyCtxtData | M | 1 | Contains the parameters to create an individual MBS Policy resource. |

Table 6.1.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MbsPolicyDecision | M | 1 | 201 Created | Successful case. An Individual MBS Policy resource is successfully created and a representation of the created resource is returned to the NF service consumer. |
| NOTE: The manadatory HTTP error status code for the POST method listed in Table 5.2.7.1-1 of TS 29.500 [4] also apply. |  |  |  |  |

Editor's Note:	Error / redirection cases and the related status codes are FFS.

Table 6.1.3.2.3.1-4: Headers supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | String | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/npcf-mbspolicycontrol/<apiVersion>/mbs-policies/{mbsPolicyId} |

* * * Next Change * * * *

##### 6.1.6.2.3	Type: MbsPolicyDecision

Table 6.1.6.2.3-1: Definition of type MbsPolicyDecision

| Attribute name |  | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- | --- |
| mbsPccRules |  | map(MbsPccRule) | O | 1..N | A map of MBS PCC rules with the content being the MbsPccRule as described in subclause 6.1.6.2.x. The key used in this map for each entry is the pccRuleId attribute of the corresponding MbsPccRule. |  |
| mbsQosInfos |  | map(MbsQosInfo) | O | 1..N | Map of QoS info policy decisions. The key used in this map for each entry is the qosId attribute of the corresponding MbsQosInfo. (NOTE) |  |
| mbsQosChars |  | map(MbsQosChar) | O | 1..N | Map of QoS characteristics for non-standard 5QIs and non-preconfigured 5QIs. This map uses the 5QI values as keys. (NOTE) |  |
| authSessAmbr (FFS) | Ambr | O | 0..1 | Authorized Session-AMBR. |  |
| NOTE: This attribute shall not be removed if it was provisioned. |  |  |  |  |  |

Editor's Note:	 It is FFS how to convey Session AMBR and other possible MBS Policy information at session level.

* * * Next Change * * * *

# A.2	Npcf_MBSPolicyControl API

openapi: 3.0.0

info:

title: Npcf_MBSPolicyControl API

version: 1.0.0-alpha.2

description: |

MBS Session Policy Control Service

© 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

TS 29.537 V0.2.0; 5G System; Multicast/Broadcast Policy Control Services.

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.537/'

[…]

paths:

/mbs-policies:

post:

summary: Request the creation of a new Individual MBS Policy resource.

operationId: CreateMBSPolicy

tags:

- MBS Policies (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MbsPolicyCtxtData'

responses:

'201':

description: >

Created. An Individual MBS Policy resource is successfully created and

a representation of the created resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/MbsPolicyDecision'

headers:

Location:

description: >

Contains the URI of the newly created Individual MBS Policy resource.

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

[…]

MbsPolicyDecision:

description: >

Identifies the service requirements of an Individual Application MBS Session Context.

type: object

properties:

mbsPccRules:

type: object

additionalProperties:

$ref: '#/components/schemas/MbsPccRule'

minProperties: 1

description: >

A map of MBS PCC rules with the content being the MbsPccRule

The key used in this map for each entry is the pccRuleId attribute of the corresponding

MbsPccRule.

mbsQosInfos:

$ref: '#/components/schemas/MbsQosInfo'

minProperties: 1

description: >

Map of QoS info policy decisions. The key used in this map for each entry is the qosId

attribute of the corresponding MbsQosInfo.

mbsQosChars:

type: object

additionalProperties:

$ref: '#/components/schemas/MbsQosChar'

minProperties: 1

description: >

Map of QoS characteristics for non standard 5QIs. This map uses the 5QI values as keys.

* * * End of Changes * * * *