3GPP TSG-CT WG3 Meeting #123-e	C3-224124

# E-Meeting, 18th – 26th August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.222 | CR | 0251 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to the references for URI structure from TS 29.501 to TS 29.122. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NBI17 |  |  |  |  |  | Date: |  |  | 2022-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It is observed that there is multiple reference to TS 29.501 for URI structure (e.g. clause 4.4. of 3GPP TS 29.501) instead of reference to TS 29.122. This is not aligned with other NBI specifications. To align with other TS, need to make references consistent and with appropriate clause of TS 29.122. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes to: - Update the references to TS 29.122 (instead of TS 29.501) - Made reference to TS 29.500 as void since its not citied anywhere in the specifications. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Inconsistency in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.2.2.3.1, 8.2.4.2.2, 8.2.4.2.6, 8.2.4.2.7, 8.7.4.2.3, 8.8.2.2.3.1, A.2, A.3, A.8, A.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR has impact on the following OpenAPI descriptions defined in this specification: - CAPIF_Discover_Service_API - CAPIF_Publish_Service_API - CAPIF_Logging_API_Invocation_API - CAPIF_Auditing_API |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of changes * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

[3]	Open API: "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[4]	IETF RFC 7230: "Hypertext Transfer Protocol (HTTP/1.1): Message Syntax and Routing".

[5]	IETF RFC 7231: "Hypertext Transfer Protocol (HTTP/1.1): Semantics and Content".

[6]	IETF RFC 7232: "Hypertext Transfer Protocol (HTTP/1.1): Conditional Requests".

[7]	IETF RFC 7233: "Hypertext Transfer Protocol (HTTP/1.1): Range Requests".

[8]	IETF RFC 7234: "Hypertext Transfer Protocol (HTTP/1.1): Caching".

[9]	IETF RFC 7235: "Hypertext Transfer Protocol (HTTP/1.1): Authentication".

[10]	IETF RFC 7540: "Hypertext Transfer Protocol Version 2 (HTTP/2)".

[11]	Void.

[12]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[13]	IETF RFC 6455: "The Websocket Protocol".

[14]	3GPP TS 29.122: "T8 reference point for northbound Application Programming Interfaces (APIs)".

[15]	3GPP TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage 3".

[16]	3GPP TS 33.122: "Security Aspects of Common API Framework for 3GPP Northbound APIs".

[17]	Void.

[18]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[19]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces Stage 3".

[20]	IETF RFC 7239: "Forwarded HTTP Extension".

[21]	Void.

[22]	W3C HTML 4.01 Specification, https://www.w3.org/TR/2018/SPSD-html401-20180327/.

[23]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[24]	IETF RFC 7519: "JSON Web Token (JWT)".

[25]	IETF RFC 7515: "JSON Web Signature (JWS)".

[26]	3GPP TS 29.523: "5G System; Policy Control Event Exposure Service; Stage 3".

[27]	3GPP TR 21.900: "Technical Specification Group working methods".

[28]	3GPP TS 29.510: "5G System; Network Function Repository Services; Stage 3"

[29]	IETF RFC 5280: "Internet X.509 Public Key Infrastructure Certificate and Certificate Revocation List (CRL) Profile".

[30]	3GPP TS 29.572: "5G System; Location Management Services; Stage 3".

* * * * Next changes * * * *

###### 8.1.2.2.3.1	GET

This operation enables to retrieve a list of APIs currently registered in the CAPIF core function, satisfying a number of filter criteria.

Table 8.1.2.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| api-invoker-id | string | M | 1 | It represents the identifier (assigned by the CCF) ofthe API invoker that is sending the request. It may also represent the identifier of the CCF that is sending the request if the request is sent over the CAPIF-6/6e reference point. (NOTE) |  |
| api-name | string | O | 0..1 | Contains the API name as {apiName} part of the URI structure as defined in clause 5.2.4 of 3GPP TS 29.122 [14]. |  |
| api-version | string | O | 0..1 | Contains the API major version conveyed in the URI (e.g. v1). |  |
| comm-type | CommunicationType | O | 0..1 | Communication type used by the API (e.g. REQUEST_RESPONSE). |  |
| protocol | Protocol | O | 0..1 | Protocol used by the API. |  |
| aef-id | string | O | 0..1 | AEF identifier. |  |
| data-format | DataFormat | O | 0..1 | Data format used by the API (e.g. serialization protocol JSON). |  |
| api-cat | string | O | 0..1 | The service API category to which the service API belongs. |  |
| preferred-aef-loc | AefLocation | O | 0..1 | The preferred AEF location. If this parameter is present, the CCF shall try to discover a matched AEF location the service API supports. This parameter is ignored by the CCF if there is no matching record found. |  |
| supported-features | SupportedFeatures | O | 0..1 | To filter irrelevant responses related to unsupported features. |  |
| api-supported-features | SupportedFeatures | C | 0..1 | Features supported by the discovered service API indicated by api-name parameter. This may only be present if the api-name query parameter is present. | ApiSupportedFeatureQuery |
| NOTE: This parameter is not part of API filter criteria so that it is not used in matching APIs published in the CCF. |  |  |  |  |  |

This method shall support the request data structures specified in table 8.1.2.2.3.1-2 and the response data structures and response codes specified in table 8.1.2.2.3.1-3.

Table 8.1.2.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 8.1.2.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DiscoveredAPIs | M | 1 | 200 OK | The response body contains the result of the search over the list of registered APIs. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during resource retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative CAPIF core function. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [14]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during resource retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative CAPIF core function. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [14]. |
| ProblemDetails | O | 0..1 | 414 URI Too Long | Indicates that the server refuses to process the request because the request-target is too long. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [14] also apply. |  |  |  |  |

Table 8.1.2.2.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative CAPIF core function. |

Table 8.1.2.2.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative CAPIF core function. |

* * * * Next changes * * * *

##### 8.2.4.2.2	Type: ServiceAPIDescription

Table 8.2.4.2.2-1: Definition of type ServiceAPIDescription

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| apiName | string | M | 1 | API name, it is set as {apiName} part of the URI structure as defined in clause 5.2.4 of 3GPP TS 29.122 [14]. |  |
| apiId | string | O | 0..1 | API identifier assigned by the CAPIF core function to the published service API. Shall not be present in the HTTP POST request from the API publishing function to the CAPIF core function. Shall be present in the HTTP POST response from the CAPIF core function to the API publishing function and in the HTTP GET response from the CAPIF core function to the API invoker (discovery API). |  |
| aefProfiles | array(AefProfile) | C | 1..N | AEF profile information, which includes the exposed API details (e.g. protocol). For CAPIF-4/4e interface, API publishing function shall provide this attribute to the CAPIF core function in service API publishing. For CAPIF-1/1e interface, the CAPIF core function shall provide this attribute to the API Invoker during service API discovery. (NOTE 2) |  |
| description | string | O | 0..1 | Text description of the API |  |
| supportedFeatures | SupportedFeatures | O | 0..1 | The supported optional features of the CAPIF API. (NOTE 1) |  |
| shareableInfo | ShareableInformation | O | 0..1 | Represents whether the service API and/or the service API category can be published to other CCFs. |  |
| serviceAPICategory | string | C | 0..1 | The service API category to which the service API belongs to. This attribute is only applicable for CAPIF-6/6e interface. (NOTE 2) |  |
| ccfId | string | C | 0..1 | CAPIF core function identifier which can be contacted further for discovering the details of service API information. This attribute is only applicable for CAPIF-6/6e interface and shall be provided with serviceAPICategory. (NOTE 2) |  |
| apiSuppFeats | SupportedFeatures | O | 0..1 | Provided by the consumer to indicate the features supported by the service API. | ApiSupportedFeaturePublishing |
| pubApiPath | PublishedApiPath | C | 0..1 | It contains the published API path within the same CAPIF provider domain. it shall be provided by the CCF when publishing the service API to other CCF via the CAPIF-6 reference point. |  |
| NOTE 1: For CAPIF_Publish_Service_API, the supported features attribute shall be provided in the HTTP POST request and in the response of successful resource creation. In addition, the supportedFeatures attribute may include one or more the supported features as defined in clause 8.2.6. NOTE 2: For CAPIF-6/6e interface, at least one of aefProfiles or serviceAPICategory and the corresponding ccfId shall be provided. |  |  |  |  |  |

* * * * Next changes * * * *

##### 8.2.4.2.6	Type: Resource

Table 8.2.4.2.6-1: Definition of type Resource

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| resourceName | string | M | 1 | Resource name |  |
| commType | CommunicationType | M | 1 | Communication type used by the API resource |  |
| uri | string | M | 1 | Relative URI of the API resource, it is set as {apiSpecificSuffixes} part of the URI structure as defined in clause 5.2.4 of 3GPP TS 29.122 [14]. |  |
| custOpName | string | O | 0..1 | it is set as {custOpName} part of the URI structure for a custom operation associated with a resource as defined in clause 4.4 of 3GPP TS 29.501 [18]. |  |
| operations | array(Operation) | C | 1..N | Supported HTTP methods for the API resource. Only applicable when the protocol in AefProfile indicates HTTP. |  |
| description | string | O | 0..1 | Text description of the API resource. |  |

* * * * Next changes * * * *

##### 8.2.4.2.7	Type: CustomOperation

Table 8.2.4.2.7-1: Definition of type CustomOperation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| commType | CommunicationType | M | 1 | Communication type used by the API resource |  |
| custOpName | string | M | 1 | it is set as {custOpName} part of the URI structure for a custom operation without resource association as defined in clause 4.4 of 3GPP TS 29.501 [18]. |  |
| operations | array(Operation) | C | 1..N | Supported HTTP methods for the API resource. Only applicable when the protocol in AefProfile indicates HTTP. |  |
| description | string | O | 0..1 | Text description of the custom operation. |  |

* * * * Next changes * * * *

##### 8.7.4.2.3	Type: Log

Table 8.7.4.2.3-1: Definition of type Log

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| apiId | string | M | 1 | String identifying the API invoked. |  |
| apiName | string | M | 1 | Name of the API which was invoked, it is set as {apiName} part of the URI structure as defined in clause 5.2.4 of 3GPP TS 29.122 [14]. |  |
| apiVersion | string | M | 1 | Version of the API which was invoked |  |
| resourceName | String | M | 1 | Name of the specific resource invoked |  |
| uri | Uri | M | 1 | Full URI of the API resource as defined in clause 5.2.4 of 3GPP TS 29.122 [14]. |  |
| protocol | Protocol | M | 1 | Protocol invoked. |  |
| operation | Operation | C | 0..1 | Operation that was invoked on the API, only applicable for HTTP protocol. |  |
| result | string | M | 1 | For HTTP protocol, it contains HTTP status code of the invocation |  |
| invocationTime | DateTime | O | 0..1 | Date on which it was invoked |  |
| invocationLatency | DurationMs | O | 0..1 | Latency for the API invocation. |  |
| inputParameters | ANY TYPE (NOTE) | O | 0..1 | List of input parameters |  |
| OutputParameters | ANY TYPE (NOTE) | O | 0..1 | List of output parameters |  |
| srcInterface | InterfaceDescription | O | 0..1 | Interface description of the API invoker. |  |
| destInterface | InterfaceDescription | O | 0..1 | Interface description of the API invoked. |  |
| fwdInterface | string | O | 0..1 | It includes the node identifier (as defined in IETF RFC 7239 [20] of all forwarding entities between the API invoker and the AEF, concatenated with comma and space, e.g. 192.0.2.43:80, unknown:_OBFport, 203.0.113.60 |  |
| NOTE: Any basic data type defined in OpenAPI Specification [3] may be used. |  |  |  |  |  |

* * * * Next changes * * * *

###### 8.8.2.2.3.1	GET

This method shall support the URI query parameters specified in table 8.8.2.2.3.1-1.

Table 8.8.2.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| aef-id | string | O | 0..1 | String identifying the API exposing function |
| api-invoker-id | string | O | 0..1 | String identifying the API invoker which invoked the service API |
| time-range-start | DateTime | O | 0..1 | Start time of the invocation time range |
| time-range-end | DateTime | O | 0..1 | End time of the invocation time range |
| api-id | string | O | 0..1 | String identifying the API invoked. |
| api-name | string | O | 0..1 | API name, it is set as {apiName} part of the URI structure as defined in clause 5.2.4 of 3GPP TS 29.122 [14]. |
| api-version | string | O | 0..1 | Version of the API which was invoked |
| protocol | Protocol | O | 0..1 | Protocol invoked |
| operation | Operation | O | 0..1 | Operation that was invoked on the API |
| result | string | O | 0..1 | HTTP status code of the invocation |
| resource-name | string | O | 0..1 | Name of the specific resource invoked |
| src-interface | InterfaceDescription | O | 0..1 | Interface description of the API invoker. |
| dest-interface | InterfaceDescription | O | 0..1 | Interface description of the API invoked. |
| supported-features | SupportedFeatures | O | 0..1 | To filter irrelevant responses related to unsupported features. |

This method shall support the request data structures specified in table 8.8.2.2.3.1-2 and the response data structures and response codes specified in table 8.8.2.2.3.1-3.

Table 8.8.2.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 8.8.2.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(InvocationLog) | O | 1..N | 200 OK | Result of the query operation along with fetched service API invocation log data. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during resource retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative CAPIF core function. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [14]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during resource retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative CAPIF core function. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [14]. |
| ProblemDetails | O | 0..1 | 414 URI Too Long | Indicates that the server is refusing to service the request because the request-target is too long. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [14] also apply. |  |  |  |  |

Table 8.8.2.2.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative CAPIF core function. |

Table 8.8.2.2.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative CAPIF core function. |

* * * * Next changes * * * *

# A.2	CAPIF_Discover_Service_API

openapi: 3.0.0

info:

title: CAPIF_Discover_Service_API

description: |

API for discovering service APIs.

© 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.2.0"

externalDocs:

description: 3GPP TS 29.222 V17.5.0 Common API Framework for 3GPP Northbound APIs

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.222/

servers:

- url: '{apiRoot}/service-apis/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 7.5 of 3GPP TS 29.222.

paths:

/allServiceAPIs:

get:

description: >

Discover published service APIs and retrieve a collection of APIs according

to certain filter criteria.

parameters:

- name: api-invoker-id

in: query

description: >

String identifying the API invoker assigned by the CAPIF core function.

It also represents the CCF identifier in the CAPIF-6/6e interface.

required: true

schema:

type: string

- name: api-name

in: query

description: >

API name, it is set as {apiName} part of the URI structure as defined

in clause 5.2.4 of 3GPP TS 29.122.

schema:

type: string

- name: api-version

in: query

description: API major version the URI (e.g. v1).

schema:

type: string

- name: comm-type

in: query

description: Communication type used by the API (e.g. REQUEST_RESPONSE).

schema:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/CommunicationType'

- name: protocol

in: query

description: Protocol used by the API.

schema:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/Protocol'

- name: aef-id

in: query

description: AEF identifer.

schema:

type: string

- name: data-format

in: query

description: Data formats used by the API (e.g. serialization protocol JSON used).

schema:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/DataFormat'

- name: api-cat

in: query

description: The service API category to which the service API belongs to.

schema:

type: string

- name: preferred-aef-loc

in: query

description: The preferred AEF location.

content:

application/json:

schema:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/AefLocation'

- name: supported-features

in: query

description: Features supported by the NF consumer for the CAPIF Discover Service API.

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

- name: api-supported-features

in: query

description: >

Features supported by the discovered service API indicated by api-name parameter.

This may only be present if api-name query parameter is present.

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: >

The response body contains the result of the search over the list of registered APIs.

content:

application/json:

schema:

$ref: '#/components/schemas/DiscoveredAPIs'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'414':

$ref: 'TS29122_CommonData.yaml#/components/responses/414'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

schemas:

DiscoveredAPIs:

type: object

description: >

Represents a list of APIs currently registered in the CAPIF core function

and satisfying a number of filter criteria provided by the API consumer.

properties:

serviceAPIDescriptions:

type: array

items:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/ServiceAPIDescription'

minItems: 1

description: >

Description of the service API as published by the service. Each service

API description shall include AEF profiles matching the filter criteria.

* * * * Next changes * * * *

# A.3	CAPIF_Publish_Service_API

openapi: 3.0.0

info:

title: CAPIF_Publish_Service_API

description: |

API for publishing service APIs.

© 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.2.0"

externalDocs:

description: 3GPP TS 29.222 V17.5.0 Common API Framework for 3GPP Northbound APIs

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.222/

servers:

- url: '{apiRoot}/published-apis/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 7.5 of 3GPP TS 29.222.

paths:

# APF published API

/{apfId}/service-apis:

post:

description: Publish a new API.

parameters:

- name: apfId

in: path

required: true

schema:

type: string

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceAPIDescription'

responses:

'201':

description: >

Service API published successfully The URI of the created resource

shall be returned in the "Location" HTTP header.

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceAPIDescription'

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure

{apiRoot}/published-apis/v1/{apfId}/service-apis/{serviceApiId}

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

get:

description: Retrieve all published APIs.

parameters:

- name: apfId

in: path

required: true

schema:

type: string

responses:

'200':

description: Definition of all service API(s) published by the API publishing function.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/ServiceAPIDescription'

minItems: 0

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

# Individual APF published API

/{apfId}/service-apis/{serviceApiId}:

get:

description: Retrieve a published service API.

parameters:

- name: serviceApiId

in: path

required: true

schema:

type: string

- name: apfId

in: path

required: true

schema:

type: string

responses:

'200':

description: >

Definition of individual service API published by the API publishing function.

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceAPIDescription'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

description: Update a published service API.

parameters:

- name: serviceApiId

in: path

required: true

schema:

type: string

- name: apfId

in: path

required: true

schema:

type: string

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceAPIDescription'

responses:

'200':

description: Definition of service API updated successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceAPIDescription'

'204':

description: No Content

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

description: Modify an existing published service API.

operationId: ModifyIndAPFPubAPI

tags:

- Individual APF published API

parameters:

- name: serviceApiId

in: path

required: true

schema:

type: string

- name: apfId

in: path

required: true

schema:

type: string

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/ServiceAPIDescriptionPatch'

responses:

'200':

description: >

The definition of the service API is modified successfully and a

representation of the updated service API is returned in the request body.

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceAPIDescription'

'204':

description: No Content. The definition of the service API is modified successfully.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

description: Unpublish a published service API.

parameters:

- name: serviceApiId

in: path

required: true

schema:

type: string

- name: apfId

in: path

required: true

schema:

type: string

responses:

'204':

description: The individual published service API matching the serviceAPiId is deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

# Components

components:

schemas:

# Data Type for representations

ServiceAPIDescription:

type: object

description: Represents the description of a service API as published by the APF.

properties:

apiName:

type: string

description: API name, it is set as {apiName} part of the URI structure as defined in clause 5.2.4 of 3GPP TS 29.122.

apiId:

type: string

description: >

API identifier assigned by the CAPIF core function to the published service API.

Shall not be present in the HTTP POST request from the API publishing function

to the CAPIF core function. Shall be present in the HTTP POST response from the

CAPIF core function to the API publishing function and in the HTTP GET response

from the CAPIF core function to the API invoker (discovery API).

aefProfiles:

type: array

items:

$ref: '#/components/schemas/AefProfile'

minItems: 1

description: >

AEF profile information, which includes the exposed API details (e.g. protocol).

description:

type: string

description: Text description of the API

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

shareableInfo:

$ref: '#/components/schemas/ShareableInformation'

serviceAPICategory:

type: string

apiSuppFeats:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

pubApiPath:

$ref: '#/components/schemas/PublishedApiPath'

ccfId:

type: string

description: CAPIF core function identifier.

required:

- apiName

InterfaceDescription:

type: object

description: Represents the description of an API's interface.

properties:

ipv4Addr:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Ipv4Addr'

ipv6Addr:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Ipv6Addr'

port:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Port'

securityMethods:

type: array

items:

$ref: '#/components/schemas/SecurityMethod'

minItems: 1

description: >

Security methods supported by the interface, it take precedence over

the security methods provided in AefProfile, for this specific interface.

oneOf:

- required: [ipv4Addr]

- required: [ipv6Addr]

AefProfile:

type: object

description: Represents the AEF profile data.

properties:

aefId:

type: string

description: Identifier of the API exposing function

versions:

type: array

items:

$ref: '#/components/schemas/Version'

minItems: 1

description: API version

protocol:

$ref: '#/components/schemas/Protocol'

dataFormat:

$ref: '#/components/schemas/DataFormat'

securityMethods:

type: array

items:

$ref: '#/components/schemas/SecurityMethod'

minItems: 1

description: Security methods supported by the AEF

domainName:

type: string

description: Domain to which API belongs to

interfaceDescriptions:

type: array

items:

$ref: '#/components/schemas/InterfaceDescription'

minItems: 1

description: Interface details

aefLocation:

$ref: '#/components/schemas/AefLocation'

required:

- aefId

- versions

oneOf:

- required: [domainName]

- required: [interfaceDescriptions]

Resource:

type: object

description: Represents the API resource data.

properties:

resourceName:

type: string

description: Resource name

commType:

$ref: '#/components/schemas/CommunicationType'

uri:

type: string

description: >

Relative URI of the API resource, it is set as {apiSpecificSuffixes} part

of the URI structure as defined in clause 5.2.4 of 3GPP TS 29.122.

custOpName:

type: string

description: >

it is set as {custOpName} part of the URI structure for a custom operation

associated with a resource as defined in clause 4.4 of 3GPP TS 29.501.

operations:

type: array

items:

$ref: '#/components/schemas/Operation'

minItems: 1

description: >

Supported HTTP methods for the API resource. Only applicable when the

protocol in AefProfile indicates HTTP.

description:

type: string

description: Text description of the API resource

required:

- resourceName

- commType

- uri

CustomOperation:

type: object

description: Represents the description of a custom operation.

properties:

commType:

$ref: '#/components/schemas/CommunicationType'

custOpName:

type: string

description: >

it is set as {custOpName} part of the URI structure for a custom operation

without resource association as defined in clause 4.4 of 3GPP TS 29.501.

operations:

type: array

items:

$ref: '#/components/schemas/Operation'

minItems: 1

description: >

Supported HTTP methods for the API resource. Only applicable when the

protocol in AefProfile indicates HTTP.

description:

type: string

description: Text description of the custom operation

required:

- commType

- custOpName

Version:

type: object

description: Represents the API version information.

properties:

apiVersion:

type: string

description: API major version in URI (e.g. v1)

expiry:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

resources:

type: array

items:

$ref: '#/components/schemas/Resource'

minItems: 1

description: Resources supported by the API.

custOperations:

type: array

items:

$ref: '#/components/schemas/CustomOperation'

minItems: 1

description: Custom operations without resource association.

required:

- apiVersion

ShareableInformation:

type: object

description: >

Indicates whether the service API and/or the service API category can be shared

to the list of CAPIF provider domains.

properties:

isShareable:

type: boolean

description: >

Set to "true" indicates that the service API and/or the service API

category can be shared to the list of CAPIF provider domain information.

Otherwise set to "false".

capifProvDoms:

type: array

items:

type: string

minItems: 1

description: >

List of CAPIF provider domains to which the service API information to be shared.

required:

- isShareable

PublishedApiPath:

type: object

description: Represents the published API path within the same CAPIF provider domain.

properties:

ccfIds:

type: array

items:

type: string

minItems: 1

description: A list of CCF identifiers where the service API is already published.

AefLocation:

description: >

The location information (e.g. civic address, GPS coordinates, data center ID)

where the AEF providing the service API is located.

type: object

properties:

civicAddr:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/CivicAddress'

geoArea:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/GeographicArea'

dcId:

type: string

description: >

Identifies the data center where the AEF providing the service API is located.

ServiceAPIDescriptionPatch:

type: object

description: >

Represents the parameters to request the modification of an APF published API resource.

properties:

aefProfiles:

type: array

items:

$ref: '#/components/schemas/AefProfile'

minItems: 1

description:

type: string

description: Text description of the API

shareableInfo:

$ref: '#/components/schemas/ShareableInformation'

serviceAPICategory:

type: string

apiSuppFeats:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

pubApiPath:

$ref: '#/components/schemas/PublishedApiPath'

ccfId:

type: string

description: CAPIF core function identifier.

Protocol:

anyOf:

- type: string

enum:

- HTTP_1_1

- HTTP_2

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Possible values are:

- HTTP_1_1: HTTP version 1.1

- HTTP_2: HTTP version 2

CommunicationType:

anyOf:

- type: string

enum:

- REQUEST_RESPONSE

- SUBSCRIBE_NOTIFY

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Possible values are:

- REQUEST_RESPONSE: The communication is of the type request-response

- SUBSCRIBE_NOTIFY: The communication is of the type subscribe-notify

DataFormat:

anyOf:

- type: string

enum:

- JSON

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Possible values are:

- JSON: JavaScript Object Notation

SecurityMethod:

anyOf:

- type: string

enum:

- PSK

- PKI

- OAUTH

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Possible values are:

- PSK: Security method 1 (Using TLS-PSK) as described in 3GPP TS 33.122

- PKI: Security method 2 (Using PKI) as described in 3GPP TS 33.122

- OAUTH: Security method 3 (TLS with OAuth token) as described in 3GPP TS 33.122

Operation:

anyOf:

- type: string

enum:

- GET

- POST

- PUT

- PATCH

- DELETE

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Possible values are:

- GET: HTTP GET method

- POST: HTTP POST method

- PUT: HTTP PUT method

- PATCH: HTTP PATCH method

- DELETE: HTTP DELETE method

* * * * Next changes * * * *

# A.8	CAPIF_Logging_API_Invocation_API

openapi: 3.0.0

info:

title: CAPIF_Logging_API_Invocation_API

description: |

API for invocation logs.

© 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.2.0"

externalDocs:

description: 3GPP TS 29.222 V17.5.0 Common API Framework for 3GPP Northbound APIs

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.222/

servers:

- url: '{apiRoot}/api-invocation-logs/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 7.5 of 3GPP TS 29.222

paths:

/{aefId}/logs:

post:

description: Creates a new log entry for service API invocations.

parameters:

- name: aefId

in: path

description: Identifier of the API exposing function

required: true

schema:

type: string

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/InvocationLog'

responses:

'201':

description: >

Log of service API invocations provided by API exposing function successfully

stored on the CAPIF core function.

content:

application/json:

schema:

$ref: '#/components/schemas/InvocationLog'

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure

{apiRoot}/api-invocation-logs/v1/{aefId}/logs/{logId}

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/{aefId}/logs/{logId}:

description: Creates a new log entry for service API invocations.

parameters:

- name: aefId

in: path

description: Identifier of the API exposing function

required: true

schema:

type: string

- name: logId

in: path

description: Identifier of individual log entry

required: true

schema:

type: string

components:

schemas:

InvocationLog:

type: object

description: >

Represents a set of Service API invocation logs to be stored in a CAPIF core function.

properties:

aefId:

type: string

description: >

Identity information of the API exposing function requesting logging of

service API invocations

apiInvokerId:

type: string

description: Identity of the API invoker which invoked the service API

logs:

type: array

items:

$ref: '#/components/schemas/Log'

minItems: 1

description: Service API invocation log

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- aefId

- apiInvokerId

- logs

Log:

type: object

description: Represents an individual service API invocation log entry.

properties:

apiId:

type: string

description: String identifying the API invoked.

apiName:

type: string

description: >

Name of the API which was invoked, it is set as {apiName} part of the URI

structure as defined in clause 5.2.4 of 3GPP TS 29.122.

apiVersion:

type: string

description: Version of the API which was invoked

resourceName:

type: string

description: Name of the specific resource invoked

uri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

protocol:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/Protocol'

operation:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/Operation'

result:

type: string

description: For HTTP protocol, it contains HTTP status code of the invocation

invocationTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

invocationLatency:

$ref: '#/components/schemas/DurationMs'

inputParameters:

description: >

List of input parameters. Can be any value - string, number, boolean, array or object.

outputParameters:

description: >

List of output parameters. Can be any value - string, number, boolean, array or object.

srcInterface:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/InterfaceDescription'

destInterface:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/InterfaceDescription'

fwdInterface:

type: string

description: >

It includes the node identifier (as defined in IETF RFC 7239 of all forwarding

entities between the API invoker and the AEF, concatenated with comma and space,

e.g. 192.0.2.43:80, unknown:_OBFport, 203.0.113.60

required:

- apiId

- apiName

- apiVersion

- resourceName

- protocol

- result

DurationMs:

type: integer

description: Unsigned integer identifying a period of time in units of milliseconds.

minimum: 0

* * * * Next changes * * * *

# A.9	CAPIF_Auditing_API

openapi: 3.0.0

info:

title: CAPIF_Auditing_API

description: |

API for auditing.

© 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.2.0"

externalDocs:

description: 3GPP TS 29.222 V17.5.0 Common API Framework for 3GPP Northbound APIs

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.222/

servers:

- url: '{apiRoot}/logs/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 7.5 of 3GPP TS 29.222.

paths:

/apiInvocationLogs:

get:

description: Query and retrieve service API invocation logs stored on the CAPIF core function.

parameters:

- name: aef-id

in: query

description: String identifying the API exposing function.

schema:

type: string

- name: api-invoker-id

in: query

description: String identifying the API invoker which invoked the service API.

schema:

type: string

- name: time-range-start

in: query

description: Start time of the invocation time range.

schema:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

- name: time-range-end

in: query

description: End time of the invocation time range.

schema:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

- name: api-id

in: query

description: String identifying the API invoked.

schema:

type: string

- name: api-name

in: query

description: >

API name, it is set as {apiName} part of the URI structure as defined in

clause 5.2.4 of 3GPP TS 29.122.

schema:

type: string

- name: api-version

in: query

description: Version of the API which was invoked.

schema:

type: string

- name: protocol

in: query

description: Protocol invoked.

schema:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/Protocol'

- name: operation

in: query

description: Operation that was invoked on the API.

schema:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/Operation'

- name: result

in: query

description: Result or output of the invocation.

schema:

type: string

- name: resource-name

in: query

description: Name of the specific resource invoked.

schema:

type: string

- name: src-interface

in: query

description: Interface description of the API invoker.

content:

application/json:

schema:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/InterfaceDescription'

- name: dest-interface

in: query

description: Interface description of the API invoked.

content:

application/json:

schema:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/InterfaceDescription'

- name: supported-features

in: query

description: To filter irrelevant responses related to unsupported features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: >

Result of the query operation along with fetched service API invocation log data.

content:

application/json:

schema:

$ref: 'TS29222_CAPIF_Logging_API_Invocation_API.yaml#/components/schemas/InvocationLog'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'414':

$ref: 'TS29122_CommonData.yaml#/components/responses/414'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

* * * * End of changes * * * *