3GPP TSG-CT WG3 Meeting #123-e	C3-224117r2

# E-Meeting, 18th – 26th August 2022

Source:	Nokia, Nokia Shanghai Bell, Huawei, Ericsson

Title:	Pseudo-CR on the Service Description update for MBS error reports.

Spec:	3GPP TS 29.537 V1.0.1

Agenda item:	17.31 (5MBS)

Document for:	Approval

1. Introduction

TS 29.537 has been allocated under the 5MBS work item to define the MBS Policy Control and Authorization services.

2. Reason for Change

MBS error reports for the failed PCC rules and Policy Decisions in the Npcf_MBSPolicyControl_UpdateNotify service operation needs to be specified.

3. Conclusions

N/A.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 29.537 V1.0.1.

* * * First Change * * * *

### 5.2.4	MBS Policy Error Handling

#### 5.2.4.1	MBS Report

If the installation of one or more MBS PCC rules fails during the procedures as defined in clause 5.2.2.2.2 or 5.2.2.3.2 or the PCF installed or modified one or more MBS PCC rules as part of those procedures but resource allocation for the MBS PCC rule was unsuccessful, the MB-SMF shall include the "mbsReports" attribute for the affected MBS PCC rules to report the failure within the MbsPolicyCtxtDataUpdate data structure. Within each MbsReport instance, the MB-SMF shall identify the failed MBS PCC rule(s) by including the affected MBS PCC rules within the "mbsPccRuleIds" attribute, identify the failed reason code by including a "failureCode" attribute, and shall include rule status within the "mbsPccRuleStatus" attribute with the value as described below.

If the installation of one or more new MBS PCC rules (i.e., rules which were not previously successfully installed) fails, the MB-SMF shall set the "mbsPccRuleStatus" to INACTIVE.

The removal of a MBS PCC rule shall not fail. The MB-SMF shall retain information on the removal and conduct the necessary procedures in the network when it is possible.

If the modification of a currently active MBS PCC rule fails, the MB-SMF shall retain the existing MBS PCC rule as active without any modification unless the reason for the failure has an impact also on the existing MBS PCC rule. The MB-SMF shall report the modification failure to the PCF.

If a MB-PCC rule was successfully installed but can no longer be enforced by the MB-SMF, the MB-SMF shall set the "mbsPccRuleStatus" attribute to INACTIVE.

NOTE:	When the PCF receives "mbsPccRuleStatus" set to INACTIVE, the PCF does not need to request the MB-SMF to remove the inactive MBS PCC rule.

Depending on the value of the "failureCode" attribute, the PCF may decide whether retaining of the old MBS PCC rule, re-installation, modification, removal of the MBS PCC rule or any other action applies.

* * * End of Changes * * * *